| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0160-v2 |
| --- | --- |

| Title: | [AHG9] Lens Optical Correction SEI message implementation and showcase |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Gilles Teniou | Email: | {swenger,teniou}[at]global[dot]tencent[dot]com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

As requested during the last JVET meeting ("No software is currently available and would be helpful to confirm the design"), this contribution describes the implementation of the Lens Optical Correction SEI message. A software patch to the VTM is attached to this document as well as an example of the configuration file. This contribution describes the implementation and explains how to use the syntax. The v2 of this contribution describes a demo illustrating the SEI message features on concrete cases.

# Introduction

During JVET meeting #34 in Rennes, the Lens Optical Correction (Loc) SEI message was first presented during which an implementation of the proposed SEI message was requested.

- Clause 2 is reminder of the justification already presented in JVET-AH0204 with illustrations of the effects and artefacts to be corrected.

- Clause 3 lists the comments raised during the previous meeting and addresses them.

- Clause 4 contains an update of the consequent the proposed syntax and semantic.

- Clause 5 illustrates the radial distortion and transversal chromatic aberration corrections.

# Software description

The attached patch has been developed on top of the commit eb7950b169 in the jvet-tuc branch.

It allows to:

- (Encoder) Read a config file containing the Loc SEI parameters

- (Encoder) Insert the Loc SEI message into the bitstream

- (Decoder) Parse the Loc SEI message from the bitstream

- (Decoder)Apply the radial distortion and/or the Transversal Chromatic Aberration corrections to the decoded video

- (Decoder) Output the corrected video

All the specific code modifications can be well retrieved with the following macro:

#if JVET_AH0204_LENS_CORRECTION_SEI …  #endif.

The activation/deactivation of the code can be done by setting in TypeDef.h:

#define JVET_AH0204_LENS_CORRECTION_SEI 0/1

The optical correction processing of the video is implemented into a class LensOpticalCorrection defined in LensOpticalCorrection.h/cpp as part of the CommonLib library.

The LensOpticalCorrection::process function handles both the radial distortion and the TCA corrections.

- For the TCA correction a conversion to RGB (in BT.709) is done prior to applying the correction. The result is converted back to YUV. This is only meant to illustrate the functionality.

- For the radial distortion correction a bilinear filter is used, but here also, the purpose is to illustrate the feature, implementations using those correcting parameters may use more advanced filters.

# Configuration file

The following illustrates the parameters from the config file:

# ======= sample configuration for Lens Optical Correction SEI ========

SEILocEnable: 1					# Enable Lens Optical Correction SEI

SEILocPersistence: 1			# Set Lens Optical Correction SEI persistence

# **** Focal parameters

SEILocFocalCenterX: -1			# Focal center x position (default -1, image center)

SEILocFocalCenterY: -1			# Focal center y position (default -1, image center)

SEILocFocalLength: 30.5 		# Focal length (in mm)

# **** Radial distorsion parameters

SEILocRdModel: 1 				# 0: poly3, 1: poly5

SEILocRdPoly3k1: -0.010413		# example value for Poly3 Rd model

SEILocRdPoly5k1: -0.227			# example value for Poly5 Rd model

SEILocRdPoly5k2: 0.002			# example value for Poly5 Rd model

# **** Transversal Chromatic Aberration (TCA) parameters

SEILocTcaModel: 1 				# TCA model: -1 = None (default), 0 = Linear, 1 = Poly3

SEILocTcaLinearKr: 1.0004		# kr parameter for Linear TCA model

SEILocTcaLinearKb: 1.0002		# kb parameter for Linear TCA model

SEILocTcaPoly3Br: 0.0000326	# br parameter for Poly3 TCA model

SEILocTcaPoly3Vr: 0.9996400	# vr parameter for Poly3 TCA model

SEILocTcaPoly3Bb: -0.0003242	# bb parameter for Poly3 TCA model

SEILocTcaPoly3Vb: 1.0008367	# vb parameter for Poly3 TCA model

# Command line

On the encoder side, the lens optical correction configuration file is added by adding a -c option:

EncoderApp -c cfg/encoder_lowdelay_vtm.cfg  -i src.yuv -c cfg/sei_vui/lens_optical_correction.cfg

Outputting the lens corrected version is done as follows:

DecoderApp -b str.bin -o output.yuv --SEILocFilename='loc_output.yuv'

# Illustration

# Introduction

Here we have run the 2 types of defined corrections on example videos picked-up on the internet. Screenshots of the original and corrected videos are used to illustrate the corrections.

## Radial distorsion correction

In this 1st example the following video was considered:

Using a Poly5 radial distortion model signaled in a Loc SEI message as follow:

SEILocEnable: 1

SEILocPersistence: 1

SEILocRdModel: 1 # 0: poly3, 1: poly5

SEILocRdPoly5k1: -0.431

SEILocRdPoly5k2: 0.105

The output corrected video is obtained as shown below:

Another example of radial distorsion correction was applied on the video illustrated below:

Still using the Poly5 model, the following parameters were found and inserted into a Loc SEI message

SEILocEnable: 1

SEILocPersistence: 1

SEILocRdModel: 1 # 0: poly3, 1: poly5

SEILocRdPoly5k1: -0.3204

SEILocRdPoly5k2: 0.1052

The decoder interpreting the parameters and applying them to the original video generated the corrected version illustrated below:

## Transversal Chromatic Aberration (TCA) correction

With regards to TCA, the following shot was selected with visible color fringing.

Here the 3rd order polynomial (Poly3) method was used to compensate the chromatic aberrations

SEILocEnable: 1

SEILocPersistence: 1

SEILocTcaModel: 1 # 0: linear 1: poly3

SEILocTcaPoly3Br: -0.001

SEILocTcaPoly3Vr: 1.005

SEILocTcaPoly3Bb: 0.002

SEILocTcaPoly3Vb: 0.996

The results are shown with zooming on a couple of visible areas (original version on the left, corrected version on the right)

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).