| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0162-v1 |
| --- | --- |

| Title: | EE2-4.2: On Shifting Quantization Center |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Haoping Yu, Luhang Xu, Jonathan Gan, Fan Wang, Zhihuang Xie, Dong Wang | Tel: Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-4.2,where shifting quantization center is investigated for both TSRC and RRC. In EE2-4.2a, the shifting quantization center is applied to the transform skip mode. In EE2-4.2b, the amount of quantization-center shifting is inversely proportional to the quantization level of an individual quantizer instead of the quantization indices of DQ. Test EE2-4.1c is a combination test of EE2-4.2a and EE2-4.2b. On top of ECM-13.0, the simulation results are reported as follows.

EE2-4.2a

AI: Nature content {0.00%, -0.02%, 0.01%; 10x.x%, 10x.x%}, Class F {-0.07%, -0.02%,   0.17%; 10x.x%, 10x.x%}, Class TGM {-0.15%, -0.10%, -0.12%; 10x.x%, 10x.x%};

RA: Nature content {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class F {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class TGM {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%};

LDB: Nature content {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class F {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class TGM {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%};

EE2-4.2b

AI: Nature content {-0.02%, -0.05%, -0.07%; 10x.x%, 10x.x%}, Class F {-0.07%, -0.02%, -0.08%; 10x.x%, 10x.x%}, Class TGM {-0.02%, -0.02%, -0.04%; 10x.x%, 10x.x%};

RA: Nature content {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class F {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class TGM {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%};

LDB: Nature content {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class F {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class TGM {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%};

EE2-4.2c

AI: Nature content {-0.02%, -0.05%, -0.13%; 10x.x%, 10x.x%}, Class F {-0.08%, 0.06%, 0.02%; 10x.x%, 10x.x%}, Class TGM {-0.12%, -0.07%, -0.04%; 10x.x%, 10x.x%};

RA: Nature content {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class F {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class TGM {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%};

LDB: Nature content {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class F {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%}, Class TGM {-0.0x%, -0.0x%, -0.0x%; 10x.x%, 10x.x%};

# Introduction

Dependent quantization (DQ) is used to quantize the transformed coefficients and the two quantizers of DQ are shown in Figure 1.

Fig. 1 Two quantizers of DQ

The quantization level k from either Q0 or Q1 is coded into the bitstream using regular residual coding. While DQ can be considered a quantizer with a quantization step size ∆, either Q0 or Q1 can be considered an individual scalar quantizer with a quantization step size 2∆. The quantization index, idxDQ of DQ can be calculated as follows.

idxDQ={2*k-sgnk,    Q1 2*k                  ,    Q0                            (1)

where k is the quantization level of Q0 or Q1, sgn. denotes the signum function

sgnk=(k==0 ?0:k<0 ? -1:1)                                 (2)

ECM-13.0 adopted a method of shifting quantization center for DQ and the shifting amount is inversely proportional to the quantization indices of DQ as follows.

- x=1024-TidxDQ*idxDQ*Δ+TidxDQ*idxDQ'*Δ≫10    if  T>idxDQ>0       (3)

where x is the dequantized coefficient, T is the size of the lookup table,  idxDQ'is the auxiliary quantization index that can be calculated using Eq. (4).

- idxDQ'=idxDQ+(idxDQ>0 ?1:-1)                        (4)

A lookup table Tof size 64 defined in (5), is used in ECM-13.0.

T=[0,63, 31, 21, 15, 12, 10, 9, 7, 7, 6, 5, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1,…,1]       (5)

# Proposed method

The contribution [2] first proposed to also apply the quantization center shifting in the transform skip mode. The same table with ECM-13.0 [1] is used.

Since the quantization level k from Q0 or Q1 instead of the quantization index of DQ is actually coded into a bitstream and the value of quantization level k can be used to better reflect the rate, this contribution proposes that the quantization level from Q0 and Q1 instead of quantization indices of DQ is used to derive the shifting amount.

The same quantization level from quantizer Q0 and Q1 will generate a similar rate while the dequantization value of Q1 is smaller than that of Q0 for the same quantization level k. Therefore, the contribution [2] further proposed that Q1 and Q0 may use different sizes of tables. In the real implementation, a lookup table T0 of size 32 and a lookup table T1 of size 48 are used for Q0 and Q1, respectively.

- x=1024-Tik*Qi-1(k)+TQik*Qi-1(k')≫10    if  Ti>k>0                (6)

where i indicates the Q0 or Q1, x is the dequantized coefficient, Ti is the size of the lookup table, Qi-1(k) is the dequantized value of the quantization level k from quantizer Qi, k'is the auxiliary quantization index that can be calculated using Eq. (7).

- k'=k+(k>0?1:-1)                               (7)

T0[32]=0, 31, 15, 10, 7, 6, 5, 4, 3, 3, 3, 2, 2, 2, 2, 2, 1,1,1,1,1,1,1,1,1,1, 1, 1, 1, 1, 1, 1                (8)

T148=0, 47, 23,15, 11,9, 7, 6, 5, 5,4,4,3, 3, 3, 3, 2, 2, 2, 2, 2,2,2,2, 1,1,1, 1, 1, 1,1……,1,1,1,1,1 (9)

Furthermore, table (8) and (9) can be interleaved to form one table as follows.

T'94=0, 47, 31, 23, 15, 15, 10, 11, 7, 9, 6, 7, 5, 6, 4, 5, 3, 5, 3, 4, 3, 4, 2, 3, 2, 3, 2, 3, 2, 3, 2, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 1,1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 1, 0, 1, 0, 1, 0, 1,0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1          (10)

The dequantization value for quantization level k from Q0 or Q1 can be calculated as follows.

- x=1024-T'idxDQ*idxDQ*Δ+T'idxDQ*idxDQ''*Δ≫10    if  T'>idxDQ>0       (11)

where T' is the size of the lookup table, and idxDQ''is the auxiliary quantization index that can be calculated using Eq. (12).

- idxDQ'=idxDQ+(idxDQ>0 ?2:-2)                         (12)

# Simulation results

The proposed method is implemented on top of the ECM-13.0 and tested based on the common test conditions [3]. The test results are summarized in the following Tables. Table 1 shows the results of EE2-4.2a (applying quantization-center shifting in the transform skip mode). Table 2 shows the results of EE2-4.2b (applying the proposed method for RRC). Table 3 shows the combination results of EE2-4.2a and EE2-4.2b.

Table 1. The results of EE2-4.2a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.03% | 0.00% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class A2 | 0.00% | -0.06% | 0.03% | #DIV/0! | #DIV/0! | 99.3% | 100.0% |
| Class B | 0.00% | -0.10% | 0.02% | #DIV/0! | #DIV/0! | 98.7% | 100.0% |
| Class C | 0.00% | 0.04% | 0.06% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class E | -0.01% | 0.09% | -0.07% | #DIV/0! | #DIV/0! | 100.8% | 100.1% |
| Overall | 0.00% | -0.02% | 0.01% | #DIV/0! | #DIV/0! | 99.7% | 100.0% |
| Class D | 0.00% | 0.09% | -0.13% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class F | -0.07% | -0.02% | 0.17% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class TGM | -0.15% | -0.10% | -0.12% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |

Table 2. The results of EE2-4.2b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.04% | -0.01% | #DIV/0! | #DIV/0! | 100.5% | 100.0% |
| Class A2 | -0.01% | -0.05% | -0.01% | #DIV/0! | #DIV/0! | 100.0% | 99.9% |
| Class B | -0.03% | -0.12% | -0.13% | #DIV/0! | #DIV/0! | 99.0% | 100.0% |
| Class C | 0.01% | -0.07% | -0.08% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class E | -0.01% | 0.07% | -0.05% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Overall | -0.02% | -0.05% | -0.07% | #DIV/0! | #DIV/0! | 99.8% | 100.0% |
| Class D | 0.02% | -0.11% | -0.15% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class F | -0.07% | -0.02% | -0.08% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class TGM | -0.02% | -0.04% | -0.04% | #DIV/0! | #DIV/0! | 100.0% | 100.1% |

Table 3. The results of EE2-4.2c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.03% | -0.11% | #DIV/0! | #DIV/0! | 100.1% | 100.0% |
| Class A2 | -0.02% | -0.05% | -0.05% | #DIV/0! | #DIV/0! | 100.3% | 100.0% |
| Class B | -0.03% | -0.18% | -0.19% | #DIV/0! | #DIV/0! | 96.0% | 100.0% |
| Class C | -0.01% | -0.02% | -0.07% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Class E | -0.03% | 0.03% | -0.20% | #DIV/0! | #DIV/0! | 100.0% | 100.0% |
| Overall | -0.02% | -0.05% | -0.13% | #DIV/0! | #DIV/0! | 98.9% | 100.0% |
| Class D | 0.00% | 0.00% | 0.03% | #DIV/0! | #DIV/0! | 100.2% | 100.0% |
| Class F | -0.08% | 0.06% | 0.02% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | -0.12% | -0.07% | -0.04% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusions

This contribution reports the results of EE2-4.2. Simulation results show a noticeable coding gain for all the classes of test content. It is recommended to adopt the proposed method into ECM.

# References

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- Y. Yu, H. Yu, L. Xu, J. Gan, F. Wang, Z. Xie, D. Wang, “AH12: On Shifting Quantization Center”, JVET-AH0063,  April 2024

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, April, 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).