| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0164-v3 |
| --- | --- |

| Title: | EE2-4.4b: Test 4.4a with CU size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-4.4b. In this test, the multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT with CU size restriction is investigated.

On top of ECM-13.0, simulation results are reported as follows:

AI {-0.08 % Y, 0.00 % U, -0.04 % V, 101.1 % EncT, 100.3 % DecT}

RA {-0.06 % Y, -0.07 % U, -0.17 % V, 100.5 % EncT, 100.3 % DecT}

LDB {0.03 % Y, -0.50 % U, -0.18 % V, 100.2 % EncT, 99.6 % DecT}

Introduction

The multiple transform set selection (MTSS) method for intra blocks coded with LFNST/NSPT was proposed in JVET-AG0062 [1]. The proposed method was further investigated in AH EE2-4.1. AH EE2-4.1 was reported in JVET-AH0056 [2]. The proposed MTSS method allows CUs coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP to select one LFNST/NSPT transform set out of 2 candidate sets. When the current block is coded with DIMD/TIMD/SGPM/MIP/EIP/ITMP in combination with LFNST/NSPT, one more bin is employed to indicate whether the 1st or the 2nd candidate transform set is selected.

In JVET-AH0218 [3] and JVET-AH0237 [4], it is proposed to use the 2nd DIMD IPM derived based on the prediction signal of the current block to derive the 2nd candidate for MIP, EIP and ITMP.

An EE plan for MTSS was proposed in JVET-AH0347 [5]. The MTSS method was further investigated in AI EE2-4.4.

Because OBIC was adopted into ECM-13.0, the MTSS method is extended to OBIC.

In EE2-4.4a [6], the derivation methods for transform sets are summarized below:

|  | IPM to derive the 1st transform set | IPM to derive the 2nd transform set |
| --- | --- | --- |
| DIMD, OBIC, TIMD | The 1st IPM used in current mode | The 2nd IPM used in current mode |
| SGPM, MIP, EIP, ITMP | The 1st IPM derived by HoG of prediction | The 2nd IPM derived by HoG of prediction |

The 1st and 2nd candidate should have different transform sets or different transpose type. If the 2nd candidate is not yet decided by the above process, it is derived by the HoG of neighboring reconstructed pixels.

In EE2-4.4a, no complexity reduction is applied to illustrate the highest gain.

Proposed method

In EE2-4.4b, a CU size restriction over EE2-4.4a is tested. MTSS can be applied to CUs with number of pixels equal or larger than 256.

Test results

EE2-4.4b was implemented on top of the ECM-13.0 software [6] and tested based on the common test condition [7].

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over 20240622_ECM13_4.4b/anchor |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.18% | 0.03% |  | -0.11% |  | 102.1% |  | 100.3% |  | 100.4% |  | 100.1% |  |
| Class A2 | -0.09% | 0.05% |  | 0.08% |  | 100.9% |  | 100.8% |  | 100.0% |  | 100.0% |  |
| Class B | -0.07% | -0.05% |  | -0.08% |  | 100.7% |  | 100.2% |  | 101.7% |  | 100.1% |  |
| Class C | -0.03% | 0.02% |  | 0.05% |  | 100.9% |  | 100.2% |  | 99.6% |  | 100.0% |  |
| Class E | -0.09% | 0.00% |  | -0.12% |  | 101.3% |  | 99.9% |  | 99.3% |  | 100.1% |  |
| Overall | -0.08% | 0.00% |  | -0.04% |  | 101.1% |  | 100.3% |  | 100.4% |  | 100.1% |  |
| Class D | -0.03% | -0.03% |  | 0.17% |  | 101.2% |  | 100.6% |  | 100.2% |  | 100.0% |  |
| Class F | -0.02% | 0.04% |  | 0.10% |  | 100.8% |  | 101.7% |  | 100.1% |  | 100.1% |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over 20240622_ECM13_4.4b/anchor |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.16% | 0.08% |  | -0.35% |  | 100.8% |  | 100.2% |  | 100.2% |  | 99.8% |  |
| Class A2 | -0.05% | -0.08% |  | -0.08% |  | 100.1% |  | 100.2% |  | 99.8% |  | 100.1% |  |
| Class B | -0.03% | -0.08% |  | -0.20% |  | 100.6% |  | 100.8% |  | 100.1% |  | 100.2% |  |
| Class C | -0.03% | -0.16% |  | -0.05% |  | 100.4% |  | 99.8% |  | 99.7% |  | 100.1% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.07% |  | -0.17% |  | 100.5% |  | 100.3% |  | 100.0% |  | 100.1% |  |
| Class D | -0.01% | -0.05% |  | 0.08% |  | 99.9% |  | 99.5% |  | 100.3% |  | 100.1% |  |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over 20240622_ECM13_4.4b/anchor |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% |  | -0.13% |  | -0.31% |  | 100.5% |  | 99.9% |  | 100.0% |  | 99.9% |
| Class C | -0.06% |  | -0.12% |  | -0.31% |  | 100.2% |  | 99.7% |  | 99.8% |  | 100.0% |
| Class E | 0.26% |  | -1.64% |  | 0.21% |  | 99.8% |  | 98.8% |  | 98.2% |  | 100.0% |
| Overall | 0.03% |  | -0.50% |  | -0.18% |  | 100.2% |  | 99.6% |  | 99.5% |  | 100.0% |
| Class D | -0.04% |  | 0.33% |  | 0.10% |  | 100.6% |  | 101.6% |  | 99.6% |  | 100.1% |
| Class F | -0.14% |  | 0.96% |  | 0.50% |  | 99.5% |  | 99.6% |  | 100.1% |  | 99.8% |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |  |

References

- F. Wang, Y. Yue, H. Yu, D. Wang, “Non-EE2: Multiple Transform Set Selection for LFNST/NSPT”, JVET-AG0062, January 2024

- F. Wang, Y. Yue, H. Yu, D. Wang, “EE2-4.1: Multiple Transform Set Selection for LFNST/NSPT”, JVET-AH0056, April 2024

- L. Zhao, K. Zhang, L. Zhang, “EE2-4.1 related: Alternative intra mode derivation for LFNST/NSPT”, JVET-AH0218, April 2024

- M. Coban, M. Karczewicz, V. Seregin, “EE2-4.1 related: Alternative intra mode derivation for LFNST/NSPT kernel selection”, JVET-AH0237, April 2024

- F. Wang, Y. Yu, H. Yu, D. Wang, L. Zhao, K. Zhang, L. Zhang, M. Coban, M. Karczewicz, C. Bonnineau, S. Puri, K. Naser, F. Le Léannec, “EE plan for Multiple Transform Set Selection”, JVET-AH0347, April 2024

- F. Wang, Y. Yu, H. Yu, D. Wang, L. Zhao, K. Zhang, L. Zhang, M. Coban, M. Karczewicz, “EE2-4.4a: Multiple transform set selection for intra LFNST/NSPT without complexity reduction”, JVET-AI0163, July 2024

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).