### Multiview acquisition information SEI message semantics

The multiview acquisition information (MAI) SEI message specifies various parameters of the acquisition environment for the layers that may be present in the current CVS, i.e., the CVS containing the MAI SEI message. Specifically, intrinsic and extrinsic camera parameters are specified. These parameters could be used for processing the decoded views prior to rendering on a 3D display.

When an MAI SEI message is present in any AU of a CVS, an MAI SEI message shall be present for the first AU of the CVS. All MAI SEI messages in a CVS shall have the same content.

When a CVS contains an SDI SEI message with sdi_multiview_info_flag equal to 1 and an MAI SEI message, the CVS shall not contain a CR SEI message. When a CVS does not contain an SDI SEI message with sdi_multiview_info_flag equal to 1 or the CVS does not contain a CR SEI message, the CVS shall not contain an MAI SEI message.

When an AU contains both an SDI SEI message and an MAI SEI message, the SDI SEI message shall precede the MAI SEI message in decoding order.

When a PU contains both a CR SEI message and an MAI SEI message, the CR SEI message shall precede the MAI SEI message in decoding order.

Some of the views for which the MAI is included in an MAI SEI message may not be present in the current CVS.

If the CVS contains an SDI SEI message with sdi_multiview_info_flag equal to 1, in the semantics below, syntax elements and variables with index i refer to the syntax elements and variables that apply to the i-th view in the current CVS specified by the SDI SEI message, i.e., the view with view identifier equal to ViewId[ i ].

Otherwise in the semantics below, syntax elements and variables with index i refer to the syntax elements and variables that apply to any j-th constituent rectangle for which i = cr_rect_group_id[ j ] in any layer to which the MAI SEI message applies.  NumViews is set equal to the greatest value of cr_rect_group_id[ j ] + 1 in any layer to which the MAI SEI message applies.

The extrinsic camera parameters are specified according to a right-handed coordinate system, where the upper left corner of the image is the origin, i.e., the ( 0, 0 ) coordinate, with the other corners of the image having non-negative coordinates. With these specifications, a 3-dimensional world point, wP = [ x y z ] is mapped to a 2-dimensional camera point, cP[ i ] = [ u v 1 ], for the i-th camera according to:

s * cP[ i ] = A[ i ] * R−1[ i ] * ( wP − T[ i ] )	(51)

where A[ i ] denotes the intrinsic camera parameter matrix, R−1[ i ] denotes the inverse of the rotation matrix R[ i ], T[ i ] denotes the translation vector and s (a scalar value) is an arbitrary scale factor chosen to make the third coordinate of cP[ i ] equal to 1. The elements of A[ i ], R[ i ] and T[ i ] are determined according to the syntax elements signalled in this SEI message and as specified below.

intrinsic_param_flag equal to 1 indicates the presence of intrinsic camera parameters. intrinsic_param_flag equal to 0 indicates the absence of intrinsic camera parameters.

extrinsic_param_flag equal to 1 indicates the presence of extrinsic camera parameters. extrinsic_param_flag equal to 0 indicates the absence of extrinsic camera parameters.

num_views_minus1 plus 1 specifies the number of views for which the MAI is included in the MAI SEI message. The value of num_views_minus1 shall be equal to NumViews − 1.

intrinsic_params_equal_flag equal to 1 indicates that the intrinsic camera parameters are equal for all cameras and only one set of intrinsic camera parameters is present. intrinsic_params_equal_flag equal to 0 indicates that the intrinsic camera parameters are different for each camera and that a set of intrinsic camera parameters is present for each camera.

prec_focal_length specifies the exponent of the maximum allowable truncation error for focal_length_x[ i ] and focal_length_y[ i ] as given by 2−prec_focal_length. The value of prec_focal_length shall be in the range of 0 to 31, inclusive.

prec_principal_point specifies the exponent of the maximum allowable truncation error for principal_point_x[ i ] and principal_point_y[ i ] as given by 2−prec_principal_point. The value of prec_principal_point shall be in the range of 0 to 31, inclusive.

prec_skew_factor specifies the exponent of the maximum allowable truncation error for skew factor as given by 2−prec_skew_factor. The value of prec_skew_factor shall be in the range of 0 to 31, inclusive.

sign_focal_length_x[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the horizontal direction is positive. sign_focal_length_x[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_x[ i ] specifies the exponent part of the focal length of the i-th camera in the horizontal direction. The value of exponent_focal_length_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_x[ i ] specifies the mantissa part of the focal length of the i-th camera in the horizontal direction. The length of the mantissa_focal_length_x[ i ] syntax element in units of bits is variable and determined as follows:

- If exponent_focal_length_x[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_x[ i ] + prec_focal_length − 31 ).

sign_focal_length_y[ i ] equal to 0 indicates that the sign of the focal length of the i-th camera in the vertical direction is positive. sign_focal_length_y[ i ] equal to 1 indicates that the sign is negative.

exponent_focal_length_y[ i ] specifies the exponent part of the focal length of the i-th camera in the vertical direction. The value of exponent_focal_length_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified focal length.

mantissa_focal_length_y[ i ] specifies the mantissa part of the focal length of the i-th camera in the vertical direction.

The length of the mantissa_focal_length_y[ i ] syntax element in units of bits is variable and determined as follows:

- If exponent_focal_length_y[ i ] is equal to 0, the length is Max( 0, prec_focal_length − 30 ).

- Otherwise (exponent_focal_length_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_focal_length_y[ i ] + prec_focal_length − 31 ).

sign_principal_point_x[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the horizontal direction is positive. sign_principal_point_x[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_x[ i ] specifies the exponent part of the principal point of the i-th camera in the horizontal direction. The value of exponent_principal_point_x[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_x[ i ] specifies the mantissa part of the principal point of the i-th camera in the horizontal direction. The length of the mantissa_principal_point_x[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_x[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_x[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_x[ i ] + prec_principal_point − 31 ).

sign_principal_point_y[ i ] equal to 0 indicates that the sign of the principal point of the i-th camera in the vertical direction is positive. sign_principal_point_y[ i ] equal to 1 indicates that the sign is negative.

exponent_principal_point_y[ i ] specifies the exponent part of the principal point of the i-th camera in the vertical direction. The value of exponent_principal_point_y[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified principal point.

mantissa_principal_point_y[ i ] specifies the mantissa part of the principal point of the i-th camera in the vertical direction. The length of the mantissa_principal_point_y[ i ] syntax element in units of bits is variable and is determined as follows:

- If exponent_principal_point_y[ i ] is equal to 0, the length is Max( 0, prec_principal_point − 30 ).

- Otherwise (exponent_principal_point_y[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_principal_point_y[ i ] + prec_principal_point − 31 ).

sign_skew_factor[ i ] equal to 0 indicates that the sign of the skew factor of the i-th camera is positive.

sign_skew_factor[ i ] equal to 1 indicates that the sign is negative.

exponent_skew_factor[ i ] specifies the exponent part of the skew factor of the i-th camera. The value of exponent_skew_factor[ i ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified skew factor.

mantissa_skew_factor[ i ] specifies the mantissa part of the skew factor of the i-th camera. The length of the mantissa_skew_factor[ i ] syntax element in units of bits is variable and determined as follows:

- If exponent_skew_factor[ i ] is equal to 0, the length is Max( 0, prec_skew_factor − 30 ).

- Otherwise (exponent_skew_factor[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_skew_factor[ i ] + prec_skew_factor − 31 ).

The intrinsic matrix A[ i ] for i-th camera is represented by

(52)

prec_rotation_param specifies the exponent of the maximum allowable truncation error for rE[ i ][ j ][ k ] (see Equation 50) as given by 2−prec_rotation_param. The value of prec_rotation_param shall be in the range of 0 to 31, inclusive.

prec_translation_param specifies the exponent of the maximum allowable truncation error for tE[ i ][ j ] (see Equation 51) as given by 2−prec_translation_param. The value of prec_translation_param shall be in the range of 0 to 31, inclusive.

sign_r[ i ][ j ][ k ] equal to 0 indicates that the sign of ( j, k ) component of the rotation matrix for the i-th camera is positive. sign_r[ i ][ j ][ k ] equal to 1 indicates that the sign is negative.

exponent_r[ i ][ j ][ k ] specifies the exponent part of ( j, k ) component of the rotation matrix for the i-th camera. The value of exponent_r[ i ][ j ][ k ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified rotation matrix.

mantissa_r[ i ][ j ][ k ] specifies the mantissa part of ( j, k ) component of the rotation matrix for the i-th camera. The length of the mantissa_r[ i ][ j ][ k ] syntax element in units of bits is variable and determined as follows:

- If exponent_r[ i ] is equal to 0, the length is Max( 0, prec_rotation_param − 30 ).

- Otherwise (exponent_r[ i ] is in the range of 0 to 63, exclusive), the length is Max( 0, exponent_r[ i ] + prec_rotation_param − 31 ).

The rotation matrix R[ i ] for i-th camera is represented as follows:

(53)

sign_t[ i ][ j ] equal to 0 indicates that the sign of the j-th component of the translation vector for the i-th camera is positive. sign_t[ i ][ j ] equal to 1 indicates that the sign is negative.

exponent_t[ i ][ j ] specifies the exponent part of the j-th component of the translation vector for the i-th camera. The value of exponent_t[ i ][ j ] shall be in the range of 0 to 62, inclusive. The value 63 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 63 as indicating an unspecified translation vector.

mantissa_t[ i ][ j ] specifies the mantissa part of the j-th component of the translation vector for the i-th camera. The length v of the mantissa_t[ i ][ j ] syntax element in units of bits is variable and is determined as follows:

- If exponent_t[ i ] is equal to 0, the length v is set equal to Max( 0, prec_translation_param − 30 ).

- Otherwise (0 < exponent_t[ i ] < 63), the length v is set equal to Max( 0, exponent_t[ i ] + prec_translation_param − 31 ).

The translation vector T[ i ] for the i-th camera is represented by:

(54)

The association between the camera parameter variables and corresponding syntax elements is specified by Table 16. Each component of the intrinsic and rotation matrices and the translation vector is obtained from the variables specified in Table 16 as the variable x computed as follows:

- If e is in the range of 0 to 63, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

- Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE – The above specification is similar to that found in IEC 60559:1989.

Table 16 – Association between camera parameter variables and syntax elements

| x | s | e | n |
| --- | --- | --- | --- |
| focalLengthX[ i ] | sign_focal_length_x[ i ] | exponent_focal_length_x[ i ] | mantissa_focal_length_x[ i ] |
| focalLengthY[ i ] | sign_focal_length_y[ i ] | exponent_focal_length_y[ i ] | mantissa_focal_length_y[ i ] |
| principalPointX[ i ] | sign_principal_point_x[ i ] | exponent_principal_point_x[ i ] | mantissa_principal_point_x[ i ] |
| principalPointY[ i ] | sign_principal_point_y[ i ] | exponent_principal_point_y[ i ] | mantissa_principal_point_y[ i ] |
| skewFactor[ i ] | sign_skew_factor[ i ] | exponent_skew_factor[ i ] | mantissa_skew_factor[ i ] |
| rE[ i ][ j ][ k ] | sign_r[ i ][ j ][ k ] | exponent_r[ i ][ j ][ k ] | mantissa_r[ i ][ j ][ k ] |
| tE[ i ][ j ] | sign_t[ i ][ j ] | exponent_t[ i ][ j ] | mantissa_t[ i ][ j ] |

### Depth representation information SEI message semantics

The syntax elements in the depth representation information (DRI) SEI message specify various parameters for auxiliary pictures of type AUX_DEPTH or constituent rectangles of type CR_DEPTH for the purpose of processing decoded primary and depth pictures or constituent rectangles prior to rendering on a 3D display, such as view synthesis. Specifically, depth or disparity ranges for depth pictures or constituent rectangles are specified.

Use of this SEI message requires the definition of the following variable:

–	A bit depth for the samples of the luma component, denoted herein by BitDepthY.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i or the CVS does not contain a CR SEI, no picture in the CVS shall be associated with a DRI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of i and a DRI SEI message, the SDI SEI message shall precede the DRI SEI message in decoding order.

When an AU contains both a CR SEI message with cr_rect_type_idc[ i ] equal to 2 for at least value of i and a DRI SEI message, the CR SEI message shall precede the DRI SEI message in decoding order.

When a CVS containing an SDI SEI message with sdi_aux_id[ i ] equal to 2 for at least one value of is present, the DRI SEI message shall be associated with one or more layers that are indicated as depth auxiliary layers by an SDI SEI message.

When a DRI message applies to a picture to which an CR SEI message also applies, the DRI SEI message is associated with the constituent rectangles for which cr_rect_type_idc[ i ] is equal to 2.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the DRI SEI message applies.

When an access unit contains an auxiliary picture that is indicated as a depth auxiliary layer by an SDI SEI message, the pictures is a depth picture.

If the CVS does not contain an SDI SEI message with sdi_aux_id[ targetLayerId ] equal to 2, the associated primary picture is provided via external means.

If CR SEI message applies to the picture, an i-th constituent rectangle is a depth constituent rectangle if cr_rect_type_idc[ i ] is equal to 2. A j-th constituent rectangle is an associated primary constituent rectangle of the i-th constituent rectangle if cr_associated_rect_idc[ i ] is equal to j. If cr_associated_rect_idc[ i ] is undefined, the associated constituent rectangle of the i-th constituent rectangle is provided via external means.

When present, the DRI SEI message may be included in any access unit. It is recommended that, when present, the SEI message is included for the purpose of random access in an access unit in which the coded picture with nuh_layer_id equal to targetLayerId is an IRAP picture.

The information indicated in the SEI message applies to all the pictures with nuh_layer_id equal to targetLayerId from the access unit containing the SEI message up to but excluding the next picture, in decoding order, associated with a DRI SEI message applicable to targetLayerId or to the end of the CLVS of the nuh_layer_id equal to targetLayerId, whichever is earlier in decoding order.

z_near_flag equal to 0 specifies that the syntax elements specifying the nearest depth value are not present in the syntax structure. z_near_flag equal to 1 specifies that the syntax elements specifying the nearest depth value are present in the syntax structure.

z_far_flag equal to 0 specifies that the syntax elements specifying the farthest depth value are not present in the syntax structure. z_far_flag equal to 1 specifies that the syntax elements specifying the farthest depth value are present in the syntax structure.

d_min_flag equal to 0 specifies that the syntax elements specifying the minimum disparity value are not present in the syntax structure. d_min_flag equal to 1 specifies that the syntax elements specifying the minimum disparity value are present in the syntax structure.

d_max_flag equal to 0 specifies that the syntax elements specifying the maximum disparity value are not present in the syntax structure. d_max_flag equal to 1 specifies that the syntax elements specifying the maximum disparity value are present in the syntax structure.

depth_representation_type specifies the representation definition of decoded luma samples of depth pictures or constituent rectangles as specified in Table 17. In Table 17, disparity specifies the horizontal displacement between two texture views and Z value specifies the distance from a camera. The value of depth_representation_type shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this version of this Specification. The values of 4 to 15, inclusive, for depth_representation_type are reserved for future use by ITU-T | ISO/IEC. Although the value of depth_representation_type is required to be in the range of 0 to 3, inclusive, in this version of this Specification, decoders shall also allow values of depth_representation_type in the range of 4 to 15, inclusive, to appear in the syntax. Decoders conforming to this version of this Specification shall ignore the bits that follow a value of depth_representation_type in the range of 4 to 15, inclusive, in the depth representation information SEI message.

The variable maxVal is set equal to ( 1  <<  BitDepthY ) − 1.

Table 17 – Definition of depth_representation_type

| depth_representation_type | Interpretation |
| --- | --- |
| 0 | Each decoded luma sample value of an depth picture represents an inverse or constituent rectangle of Z value that is uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 represents the inverse of ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents the inverse of ZNear (specified below). |
| 1 | Each decoded luma sample value of an depth picture or constituent rectangle represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| 2 | Each decoded luma sample value of an depth picture or constituent rectangle represents a Z value uniformly quantized into the range of 0 to maxVal, inclusive. When z_far_flag is equal to 1, the luma sample value equal to 0 corresponds to ZFar (specified below). When z_near_flag is equal to 1, the luma sample value equal to maxVal represents ZNear (specified below). |
| 3 | Each decoded luma sample value of an depth picture or constituent rectangle represents a nonlinearly mapped disparity, normalized in range from 0 to maxVal, as specified by depth_nonlinear_representation_num_minus1 and depth_nonlinear_representation_model[ i ]. When d_min_flag is equal to 1, the luma sample value equal to 0 represents DMin (specified below). When d_max_flag is equal to 1, the luma sample value equal to maxVal represents DMax (specified below). |
| Other values | Reserved for future use |

disparity_ref_view_id specifies the view identifier for which the disparity values are derived. The value of disparity_ref_view_id shall be in the range of 0 to 1023, inclusive.

NOTE 1 – The view identifier of the i-th view in the current CVS is equal to ViewId[ i ] as specified in the semantics of the SDI SEI message in clause 8.19.2.

NOTE 2 – disparity_ref_view_id is present only if d_min_flag is equal to 1 or d_max_flag is equal to 1 and is useful for depth_representation_type values equal to 1 and 3.

The variables in the x column of Table 18 are derived from the respective variables in the s, e, n and v columns of Table 18 as follows:

–	If the value of e is in the range of 0 to 127, exclusive, x is set equal to ( −1 )s * 2e − 31 * ( 1 + n ÷ 2v ).

–	Otherwise (e is equal to 0), x is set equal to ( −1 )s * 2−( 30 + v ) * n.

NOTE 3 – The above specification is similar to that found in IEC 60559:1989.

Table 18 – Association between depth parameter variables and syntax elements

| x | s | e | n | v |
| --- | --- | --- | --- | --- |
| ZNear | ZNearSign | ZNearExp | ZNearMantissa | ZNearManLen |
| ZFar | ZFarSign | ZFarExp | ZFarMantissa | ZFarManLen |
| DMax | DMaxSign | DMaxExp | DMaxMantissa | DMaxManLen |
| DMin | DMinSign | DMinExp | DMinMantissa | DMinManLen |

The DMin and DMax values, when present, are specified in units of a luma sample width of the associated primary picture of the depth picture.

The units for the ZNear and ZFar values, when present, are identical but unspecified.

depth_nonlinear_representation_num_minus1 plus 2 specifies the number of piece-wise linear segments for mapping of depth values to a scale that is uniformly quantized in terms of disparity. The value of depth_nonlinear_representation_num_minus1 shall be in the range of 0 to 62, inclusive.

depth_nonlinear_representation_model[ i ] for i ranging from 0 to depth_nonlinear_representation_num_minus1 + 2, inclusive, specify the piece-wise linear segments for mapping of decoded luma sample values of an depth picture or constituent rectangle to a scale that is uniformly quantized in terms of disparity. The value of depth_nonlinear_representation_model[ i ] shall be in the range of 0 to 65 535, inclusive. The values of depth_nonlinear_representation_model[ 0 ] and depth_nonlinear_representation_model[ depth_nonlinear_representation_num_minus1 + 2 ] are both inferred to be equal to 0.

NOTE 4 – When depth_representation_type is equal to 3, an depth picture or constituent rectangle contains non-linearly transformed depth samples. The variable DepthLUT[ i ], as specified below, is used to transform decoded depth sample values from the non-linear representation to the linear representation, i.e., uniformly quantized disparity values. The shape of this transform is defined by means of line-segment approximation in two-dimensional linear-disparity-to-non-linear-disparity space. The first ( 0, 0 ) and the last ( maxVal, maxVal ) nodes of the curve are predefined. Positions of additional nodes are transmitted in form of deviations (depth_nonlinear_representation_model[ i ]) from the straight-line curve. These deviations are uniformly distributed along the whole range of 0 to maxVal, inclusive, with spacing depending on the value of depth_nonlinear_representation_num_minus1. The variable DepthLUT[ i ] for i in the range of 0 to maxVal, inclusive, is specified as follows:

for( k = 0; k  <=  depth_nonlinear_representation_num_minus1 + 1; k++ ) {
	pos1 = ( maxVal * k ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev1 = depth_nonlinear_representation_model[ k ]
	pos2 = ( maxVal * ( k + 1 ) ) / (depth_nonlinear_representation_num_minus1 + 2 )
	dev2 = depth_nonlinear_representation_model[ k + 1 ]	(55)

	x1 = pos1 − dev1
	y1 = pos1 + dev1
	x2 = pos2 − dev2
	y2 = pos2 + dev2

	for( x = Max( x1, 0 ); x  <=  Min( x2, maxVal ); x++ )
			DepthLUT[ x ] = Clip3( 0, maxVal, Round( ( ( x − x1 ) * ( y2 − y1 ) ) ÷ ( x2 − x1 ) + y1 ) )
}

When depth_representation_type is equal to 3, DepthLUT[ dS ] for all decoded luma sample values dS of an depth picture or constituent rectangle in the range of 0 to maxVal, inclusive, represents disparity that is uniformly quantized into the range of 0 to maxVal, inclusive.

#### Depth representation information element semantics

The syntax structure specifies the value of an element in the DRI SEI message.

The depth_rep_info_element( OutSign, OutExp, OutMantissa, OutManLen ) syntax structure sets the values of the OutSign, OutExp, OutMantissa and OutManLen variables that represent a floating-point value. When the syntax structure is included in another syntax structure, the variable names OutSign, OutExp, OutMantissa and OutManLen are to be interpreted as being replaced by the variable names used when the syntax structure is included.

da_sign_flag equal to 0 indicates that the sign of the floating-point value is positive. da_sign_flag equal to 1 indicates that the sign is negative. The variable OutSign is set equal to da_sign_flag.

da_exponent specifies the exponent of the floating-point value. The value of da_exponent shall be in the range of 0 to 27 − 2, inclusive. The value 27 − 1 is reserved for future use by ITU-T | ISO/IEC. Decoders shall treat the value 27 − 1 as indicating an unspecified value. The variable OutExp is set equal to da_exponent.

da_mantissa_len_minus1 plus 1 specifies the number of bits in the da_mantissa syntax element. The variable OutManLen is set equal to da_mantissa_len_minus1 + 1.

da_mantissa specifies the mantissa of the floating-point value. The variable OutMantissa is set equal to da_mantissa.

### Alpha channel information SEI message semantics

The alpha channel information (ACI) SEI message provides information about alpha channel sample values and post-processing applied to the decoded alpha planes coded in auxiliary pictures of type AUX_ALPHA or constituent rectangles of type CR_ALPHA and one or more associated primary pictures or constituent rectangles.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i or the CVS does not contain a CR SEI, no picture in the CVS shall be associated with an ACI SEI message.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to 1 for at least one value of i and an ACI SEI message, the SDI SEI message shall precede the ACI SEI message in decoding order.

When an AU contains both a CR SEI message with cr_rect_type_idc[ i ] equal for at least one value of i and an ACI SEI message, the CR SEI message shall precede the ACI SEI message in decoding order.

When an ACI message applies to a picture to which a CR SEI message also applies, the ACI SEI message is associated with the constituent rectangles for which cr_rect_type_idc[ i ] is equal to 1.

When an access unit contains an auxiliary picture picA in a layer, with nuh_layer_id equal to nuhLayerIdA, that is indicated as an alpha auxiliary layer by an SDI SEI message, picA is an alpha picture and the alpha channel sample values of picA persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdA is output.

–	A CLVS containing the auxiliary picture picA ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdA ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the ACI SEI message applies.

If the CVS does not contain an SDI SEI message with sdi_aux_id[ targetLayerId ] equal to 1, the associated primary picture is provided via external means.

If a CR SEI message applies to the picture, an i-th constituent rectangle is an alpha constituent rectangle if cr_rect_type_idc[ i ] is equal to 1. A j-th constituent rectangle is an associated primary constituent rectangle of the i-th constituent rectangle if cr_associated_rect_idc[ i ] is equal to j. If cr_associated_rect_idc[ i ] is undefined, the associated constituent rectangle of the i-th constituent rectangle is provided via external means.

alpha_channel_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous ACI SEI message in output order that applies to the current layer. alpha_channel_cancel_flag equal to 0 indicates that ACI follows.

Let currPic be the picture that the ACI SEI message is associated with. The semantics of ACI SEI message persist for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ACI SEI message is output that follows the current picture in output order.

alpha_channel_use_idc equal to 0 indicates that for alpha blending purposes the decoded samples of the associated primary picture or constituent rectangle should be multiplied by the interpretation sample values of the decoded alpha picture or constituent rectangle in the display process after output from the decoding process. alpha_channel_use_idc equal to 1 indicates that for alpha blending purposes the decoded samples of the associated primary picture or constituent rectangle should not be multiplied by the interpretation sample values of the decoded alpha picture or constituent rectangle in the display process after output from the decoding process. alpha_channel_use_idc equal to 2 indicates that the usage of the alpha picture or constituent rectangle is unspecified. Values greater than 2 for alpha_channel_use_idc are reserved for future use by ITU-T | ISO/IEC. When not present, the value of alpha_channel_use_idc is inferred to be equal to 2. Decoders shall ignore alpha channel information SEI messages in which alpha_channel_use_idc is greater than 2.

alpha_channel_bit_depth_minus8 plus 8 specifies the bit depth of the samples of the luma sample array of the alpha picture or constituent rectangle. alpha_channel_bit_depth_minus8 plus 8 shall be equal to the bit depth of the associated primary picture or constituent rectangle.

alpha_transparent_value specifies the interpretation sample value of a decoded alpha picture or constituent rectangle luma sample for which the associated luma and chroma samples of the primary coded picture or constituent rectangle are considered transparent for purposes of alpha blending. The number of bits used for the representation of the alpha_transparent_value syntax element is alpha_channel_bit_depth_minus8 + 9.

alpha_opaque_value specifies the interpretation sample value of a decoded alpha picture or constituent rectangle luma sample for which the associated luma and chroma samples of the primary coded picture or constituent rectangle are considered opaque for purposes of alpha blending. The number of bits used for the representation of the alpha_opaque_value syntax element is alpha_channel_bit_depth_minus8 + 9.

A value of alpha_opaque_value that is equal to alpha_transparent_value indicates that the alpha coded picture or constituent rectangle is not intended for alpha blending purposes.

NOTE 1 – For alpha blending purposes, alpha_opaque_value can be greater than alpha_transparent_value or it can be less than or equal to alpha_transparent_value.

alpha_channel_incr_flag equal to 0 indicates that the interpretation sample value for each decoded alpha picture or constituent rectangle luma sample value is equal to the decoded alpha picture sample value for purposes of alpha blending. alpha_channel_incr_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the alpha picture or constituent rectangle samples, any alpha picture or constituent rectangle luma sample value that is greater than Min( alpha_opaque_value, alpha_transparent_value ) should be increased by one to obtain the interpretation sample value for the alpha picture or constituent rectangle sample and any alpha picture or constituent rectangle luma sample value that is less than or equal to Min( alpha_opaque_value, alpha_transparent_value ) should be used, without alteration, as the interpretation sample value for the decoded alpha picture or constituent rectangle sample value.

When alpha_transparent_value is equal to alpha_opaque_value or Log2( Abs( alpha_opaque_value − alpha_transparent_value ) ) does not have an integer value, alpha_channel_incr_flag shall be equal to 0.

alpha_channel_clip_flag equal to 0 indicates that no clipping operation is applied to obtain the interpretation sample values of the decoded alpha picture or constituent rectangle. alpha_channel_clip_flag equal to 1 indicates that the interpretation sample values of the decoded alpha picture or constituent rectangle are altered according to the clipping process described by the alpha_channel_clip_type_flag syntax element.

alpha_channel_clip_type_flag equal to 0 indicates that, for purposes of alpha blending, after decoding the alpha picture samples, any alpha picture or constituent rectangle luma sample that is greater than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to Max( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the alpha picture or constituent rectangle luma sample and any alpha picture luma sample that is less or equal than ( alpha_opaque_value + alpha_transparent_value ) / 2 is set equal to Min( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the alpha picture luma sample. alpha_channel_clip_type_flag equal to 1 indicates that, for purposes of alpha blending, after decoding the alpha picture samples, any alpha picture or constituent rectangle luma sample that is greater than Max( alpha_transparent_value, alpha_opaque_value ) is set equal to Max( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the alpha picture or constituent rectangle luma sample and any alpha picture or constituent rectangle luma sample that is less than or equal to Min( alpha_transparent_value, alpha_opaque_value ) is set equal to Min( alpha_transparent_value, alpha_opaque_value ) to obtain the interpretation sample value for the alpha picture or constituent rectangle luma sample.

When both alpha_channel_incr_flag and alpha_channel_clip_flag are equal to one, the clipping operation specified by alpha_channel_clip_type_flag should be applied first, followed by the alteration specified by alpha_channel_incr_flag, to obtain the interpretation sample value for the alpha picture or constituent rectangle luma sample.

Alpha blending composition is ordinarily performed with a background picture B, a foreground picture F, and a decoded alpha picture A, all of the same size. Assume for purposes of example illustration that the chroma resolutions of B and F, if different from the luma resolution, have been upsampled to the same resolution as the luma. Denote corresponding samples of B, F and A by b, f and a, respectively. Denote luma and chroma samples by subscripts Y, Cb and Cr. Each component, e.g., Y, is also assumed for purposes of example illustration to have the same bit depth in each of the pictures B and F. However, different components, e.g., Y and Cb, can have different bit depths in this example. The samples of pictures B and F may alternatively represent green, blue and red component values (see clause 7.3), although the equations use the subscripts Y, Cb and Cr for the three components.

Define the variables alphaRange, alphaFwt and alphaBwt for each luma sample aY of the alpha picture A as follows:

alphaRange = Abs( alpha_opaque_value − alpha_transparent_value )	(56)

alphaFwt = Abs( aY − alpha_transparent_value )		(57)

alphaBwt = Abs( aY − alpha_opaque_value )		(58)

A picture format that is often useful for editing or direct viewing, and that is commonly used, is called pre-multiplied-black video. Pre-multiplied-black video has the characteristic that the decoded picture F will appear the same regardless of whether it is viewed directly without alpha blending composition or is alpha blended with a black background. The use of alpha_channel_use_idc equal to 0 corresponds with source video that is not pre-multiplied-black video, and the use of alpha_channel_use_idc equal to 1 corresponds with source video that is pre-multiplied-black video.

An example of operation of the alpha blending composition process to produce a displayed picture D with sample values d from the pictures B and F is as follows:

–	If alpha_channel_use_idc is equal to 0, the samples d of the displayed picture D are calculated as follows:

dY = ( alphaFwt * fY + alphaBwt * bY + alphaRange / 2 ) / alphaRange	(59)

dCb = ( alphaFwt * fCb + alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(60)

dCr = ( alphaFwt * fCr + alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(61)

–	Otherwise (alpha_channel_use_idc is equal to 1), the samples d of the displayed picture D are calculated as follows:

dY = fY + ( alphaBwt * bY + alphaRange / 2 ) / alphaRange	(62)

dCb = fCb + ( alphaBwt * bCb + alphaRange / 2 ) / alphaRange	(63)

dCr = fCr + ( alphaBwt * bCr + alphaRange / 2 ) / alphaRange	(64)

NOTE 2 – In this case, it is expected that the encoder produces its pre-multipled-black source video picture S with sample values s from some original input picture T with sample values t as expressed by Equations 65 to 67, so that when the decoded picture F is a close approximation of the pre-multipled-black source video picture S, the cascaded effect of Equations 65 to 67 followed by Equations 62 to 64 is approximately the same as expressed in Equations 59 to 61.

sY = ( alphaFwt * tY ) / alphaRange		(65)

sCb = ( alphaFwt * tCb ) / alphaRange		(66)

sCr = ( alphaFwt * tCr ) / alphaRange		(67)

NOTE 3 – In the event that the background picture B is represented using green, blue and red component values (see clause 7.3) in a manner such that the colour black is represented by all three component values being equal to 0, when the background picture B is black, the operation expressed by Equations 62 to 64 becomes simply dY = fY, dCb = fCb, and dCr = fCr. This can help to explain the "pre-multiplied black" term, as the expressions in Equations 65 to 67 are referred to as the pre-multiplication for the black background combination.

For the case with alpha_channel_use_idc equal to 1, somewhat modified processing should be applied when the colour representation domain is different from the use of green, blue, and red colour component values, or with the use of a non-zero black level. Unless the colour black is represented by all three component values bY, bCb, and bCr being equal to 0, Equations 62 to 64 do not simplify to dY = fY, dCb = fCb, and dCr = fCr for pre-multiplied-black video content.

