| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0178-v1 |
| --- | --- |

| Title: | AHG9: On constituent rectangles SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Miska M. Hannuksela | Email: | {jill.boyce, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The constituent rectangles (CR) SEI was proposed in JVET-AH0162 and included in the VSEI TuC in JVET-AH2032. The following VSEI modifications related to constituent rectangles are proposed:

- Modify the inference of the cr_rect_id[ i ] syntax element in the CR SEI.

- Add cr_associated_rect_id[ i ] and cr_rect_group_id[ i ] syntax elements to the CR SEI.

- Add a subpicture definition to VSEI.

- Modify the semantics of the multiview acquisition information, alpha channel information, and depth representation info SEI messages to enable their use with constituent rectangles.

# Proposals

The constituent rectangles (CR) SEI was proposed in JVET-AH0162 and included in the VSEI TuC in JVET-AH2032. Modifications are proposed to the CR SEI message syntax and semantics, and to the alpha channel information, depth representation info, and multiview acquisition information SEI messages to enable their use with constituent rectangles.

## Proposal 1: Modify the inference of the cr_rect_id[ i ] syntax element in the CR SEI

It is proposed to modify the inference of the value of cr_rect_id[ i ] syntax element when not present, to be equal to the previous signalled value cr_rect_id[ i − 1] plus 1 rather than inferring its value to be i.

This proposed modification allows gaps in in the numbering of the rect ID values, for example to have meaningful bit fields in the rect ID values, without requiring signaling of the cr_rect_id[ i ] syntax element for all CRs.

## Proposal 2: Add cr_associated_rect_id[ i ] and cr_rect_group_id[ i ] syntax elements to the CR SEI

It is proposed to add a cr_associated_rect_id[ i ] syntax element to the CR SEI, conditioned upon enable and presence flags, so that a constituent rectangle of a CR_ALPHA or CR_DEPTH type may be associated with a primary constituent rectangle, similar to how auxiliary picture layers may be associated with a primary picture layers in the scalability dimension information (SDI) SEI.

It is also proposed to add a cr_rect_group_id[ i ] syntax element to the CR SEI, conditioned upon a presence flag, to provide the capability to optionally group constituent rectangles together that share the same rect group ID value.

Each constituent rectangle in the CR SEI message must have a unique value of cr_rect_id[ i ], but multiple constituent rectangles may have the same value of cr_rect_group_id[ i ].

The rect group ID value could be used, for example to identify the view ID of a constituent rectangle in a picture with multiple views within a single layer or multi-layer bitstream. In a multi-view bitstream containing both texture CRs and depth CRs, both the texture CR and depth CR for the same view could have the same value of view ID.

Additional use cases for grouping of constituent rectangles can also be supported.

## Proposal 3: Add definition for subpicture to VSEI

The CR SEI message refers to subpictures, which are not defined in VSEI. It is proposed to copy the VVC definition of subpicture to VSEI.

## Proposal 4: Modify the semantics of the multiview acquisition information, alpha channel information, and depth representation info SEI messages to enable their use with constituent rectangles

With proposal 2, the CR SEI message can be used instead of the SDI SEI message to indicate the auxiliary/primary association of CRs.

It is also proposed to modify the semantics of the alpha channel information (ACI), depth representation info (DRI), and multiview acquisition information (MAI) SEI messages in VSEI to enable them to be used with constituent rectangles.

The ACI, DRI, and MAI SEI messages in VSEI currently require the presence of the SDI SEI message. It is proposed to modify this requirement to require either the presence of the SDI SEI message or the CR SEI message.

When an ACI or DRI applies to a picture to which a CR SEI also applies, the ACI or DRI SEI message applies to the constituent rectangles for which cr_rect_type_idc[ i ] is equal to 1 (CR_ALPHA) or 2 (CR_DEPTH), respectively, in the current layer.

The associated primary constituent rectangle is determined by the cr_associated_rect_idc[ i ] syntax element of proposal 2, when present. If not present, the associated constituent rectangle is provided via external means.

In the MAI SEI message in VSEI, camera parameters are sent with an index i, which corresponds to the view with view identifier equal to ViewId[ i ] in the SDI SEI message. In the proposed modifications, the cr_rect_group_id[ i ] of proposal 2 is used as a View ID.

If the MAI applies to a CVS containing multiple layers, each individual layer may have a CR SEI message with different parameters than those in other layers. The camera parameters signalled in the MAI SEI message for index i apply to any j-th constituent rectangle for which i = cr_rect_group_id[ j ] in any layer to which the MAI SEI message applies.

# Syntax and Semantics

## Semantics modification for Proposal 1

cr_rect_id[ i ] indicates the ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits. When not present and i equal to 0, the value of cr_rect_id[ i ] is inferred to be equal to i0. When not present and i greater than 0, the value of cr_rect_id[ i ] is inferred to be equal to cr_rect_id[ i − 1] + 1.

It is a requirement of bitstream conformance that when j not equal to k, cr_rect_id[ j ] shall not be equal to cr_rect_id[ k ].

## Syntax and semantics modification for Proposal 2

| constituent_rectangles( payloadSize ) { | Descriptor |
| --- | --- |
| cr_num_rects_minus1 | u(12) |
| cr_rect_id_present_flag | u(1) |
| cr_associated_rect_id_enabled_flag | u(1) |
| cr_rect_group_id_present_flag | u(1) |
| if ( cr_rect_id_present_flag ¦ ¦ cr_associated_rect_id_enabled_flag ¦ ¦ cr_rect_group_id_present_flag ) |  |
| cr_rect_id_len | u(4) |
| cr_rect_type_enabled_flag | u(1) |
| cr_rect_type_descriptions_enabled_flag | u(1) |
| cr_subpics_partitioning_flag | u(1) |
| if( !cr_subpics_partitioning_flag ) { | u(1) |
| cr_rect_same_size_flag | u(1) |
| if ( cr_rect_same_size_flag ) { |  |
| cr_num_cols_minus1 | ue(v) |
| cr_num_rows_minus1 | ue(v) |
| } else { |  |
| cr_log2_unit_size | u(4) |
| cr_rect_size_len_minus1 | u(4) |
| } |  |
| } |  |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) { |  |
| if ( cr_rect type_enabled_flag ) { |  |
| cr_rect_type_present_flag[ i ] | u(1) |
| if ( cr_rect_type_present_flag[ i ] ) |  |
| cr_rect_type_idc[ i ] | u(8) |
| } |  |
| if ( cr_rect type_idc[ i ] != 255 ) { |  |
| if ( cr_rect id_present_flag ) |  |
| cr_rect_id[ i ] | u(v) |
| if ( cr_associated_rect id_enabled_flag ) { |  |
| cr_associated_rect_id_present_flag[ i ] | u(1) |
| if ( cr_associated_rect_id_present_flag[ i ] ) |  |
| cr_associated_rect_id[ i ] | u(v) |
| } |  |
| if ( cr_group_id_present_flag ) { |  |
| cr_rect_group_id[ i ] | u(v) |
| if ( cr_rect type_description_enabled_flag ) |  |
| cr_rect_type_description_present_flag[ i ] | u(1) |
| } |  |
| if( !cr_subpics_partitioning_flag && !cr_rects_same_size_flag ) { |  |
| cr_rect_top_left_in_units_x[ i ] | u(v) |
| cr_rect_top_left_in_units_y[ i ] | u(v) |
| cr_rect_width_in_units_minus1[ i ] | u(v) |
| cr_rect_height_in_units minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( cr_rect_type_descriptions_enabled_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| cr_bit_equal_to_zero /* equal to 0 */ | f(1) |
| for( i = 0; i <= cr_num_rects_minus1; i++ ) |  |
| if( rect_type_description_present_flag[ i ] ) |  |
| cr_rect_type_description[ i ] | st(v) |
| } |  |
| } |  |

cr_associated_rect_id_enabled_flag equal to 1 specifies that the cr_associated_rect_id_present_flag[ i ] syntax elements are present. cr_associated_rect_id_enabled_flag equal to 0 specifies that the cr_associated_rect_id_present_flag[ i ] syntax elements are not present.

cr_rect_group_id_present_flag equal to 1 specifies that cr_rect_group_id[ i ] syntax elements are present. cr_rect_group_present_flag equal to 0 specifies that_cr_rect_group_id[ i ] syntax elements are not present.

cr_rect_id_len specifies the length of the cr_rect_id[ i ],  cr_associated_rect_id [ i ], and cr_rect_group_id[ i ] syntax elements.

cr_associated_rect_id[ i ] indicates the ID of an associated primary rectangle of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits.

It is a requirement of bitstream conformance that cr_associated_rect_id[ i ] equal cr_rect_id[ j ] for some value of j in 0 .. cr_num_rects_minus1.

When not present, the value of cr_associated_rect_id[ i ] is undefined.

cr_rect_group_id[ i ] indicates the group ID of the i-th rectangle. The length of the syntax element is cr_rect_id_len bits. When not present, the value of cr_rect_group_id[ i ] is inferred to be equal to 0.

## Syntax and semantics modification for Proposal 3

Add to clause 3 the following:

3. X subpicture: A rectangular region of one or more slices within a picture.

## Syntax and semantics modification for Proposal 4

The modified semantics of the multiview acquisition information, alpha channel information, and depth representation info SEI messages for proposal 3 are shown in a change-marked version of VSEI attached to this contribution.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).