| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0187-v1 |
| --- | --- |

| Title: | EE2-3.7: TMVP for chained motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin Zhi Zhang Jian-Liang Lin Vadim Seregin Marta Karczewicz | Tel: Email: | pohanlin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the test results tests EE2-3.7. EE2-3.7 proposes to derive TMVP with MV scaled to reference picture with index 0, and derive CMVP candidates from TMVP candidates.

On top of ECM-13.0 [1], simulation results are reported as follows:

RA {-0.07% Y, -0.09% U, -0.08% V, 99.8% EncT, 100.3% DecT }

LB {-0.02% Y, 0.13% U, 0.03% V, 99.7% EncT, 100.1% DecT }

# Introduction

CMVP candidates are derived as the accumulation of the recursively traced MVs and BVs based on the pre-derived MVs for the inter merge candidate list. For instance, a CMVP candidate, a set of motion vector MVk/m and reference picture RefPick/m can be derived by

MVk/m = MVk(0) + MVk(1) +MVk(2) + … + MVk(m),

RefPick/m = RefPick(m),

where k and m indicate the number of merge index and trace depths of the CMVP, the trace depths set to 1 in current ECM. Figure 1 shows the example of CMVP derivation.

Figure 1 Example of derivation of CMVP candidate

# Proposed method

This contribution proposes to add TMVP candidates with MV scaled to reference picture with index 0. The additional TMVP candidates are alco used to derive CMVP candidates. After TMVP and the related CMVP candidates are derived, ARMC reordering is performed to reduce the candidate number. It also proposes to derive CMVP candidates in BM Merge mode.

# Simulation results

The tables below summarize the coding efficiency and runtime impact of the tests 3.7 in ECM-13.0 following the common test conditions in the Exploration Experiments 2 document [2].

Table 1. EE2-3.7 on top of ECM-13.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  | -0.07% |  | -0.11% | -0.12% |  | 100.1% |  | 100.2% |  | 100.0% |  | 100.0% |  |
| Class A2 |  | -0.07% |  | -0.14% | -0.10% |  | 99.7% |  | 100.2% |  | 100.0% |  | 99.7% |  |
| Class B | -0.06% | -0.06% | -0.15% | 99.6% | 100.4% | 99.8% | 100.1% |
| Class C | -0.09% | -0.09% | 0.04% | 99.8% | 100.3% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.09% | -0.08% | 99.8% | 100.3% | 99.9% | 100.0% |
| Class D | -0.14% | -0.02% | 0.00% | 99.1% | 99.9% | 100.0% | 100.0% |
| Class F | -0.01% | -0.01% | 0.04% | 99.7% | 100.2% | 99.3% | 100.1% |
| Class TGM | -0.07% | -0.04% | -0.02% | 100.1% | 97.9% | 100.3% | 100.0% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.29% | -0.07% | 100.6% | 100.3% | 100.2% | 99.9% |
| Class C | -0.02% | -0.23% | -0.54% | 98.8% | 100.1% | 99.6% | 100.0% |
| Class E | -0.03% | 0.36% | 0.96% | 99.7% | 99.7% | 100.0% | 100.0% |
| Overall | -0.02% | 0.13% | 0.03% | 99.7% | 100.1% | 99.9% | 100.0% |
| Class D | -0.15% | -0.48% | -0.64% | 99.6% | 100.3% | 99.8% | 100.0% |
| Class F | -0.13% | -0.01% | -0.08% | 99.5% | 99.0% | 98.7% | 100.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusions

It is recommended to adopt test3.7 in the next generation of ECM.

# Reference

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AH2024, April 2024

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).