| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI0192-v2 |
| --- | --- |

| Title: | AHG9: On GFV picture order and timing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Kaifa Yang2, Yue Li2, Yiling Xu2 1Bytedance Inc., US 2Shanghai Jiao Tong University, China | Email: | yekui.wang@bytedance.com sekiroyyy@sjtu.edu.cn yueli983@sjtu.edu.cn yl.xu@sjtu.edu.cn |
| Source: | Bytedance Inc., Shanghai Jiao Tong University |  |  |

_____________________________

# Abstract

This contribution proposes the following changes regarding order and timing of GFV-generated pictures (i.e., output pictures derived by the process StoreOutputTensors( ) specified in the semantics of the GFV SEI message):

- When gfv_cnt is greater than 0, an indication that indicates the picture order count value of the GFV-generated picture corresponding to the current GFV SEI message is signalled in the GFV SEI message.

- When gfv_cnt is equal to 0, timing information is present for the CLVS, and the specific timing information of the number of clock ticks corresponding to a difference of picture order count values equal to 1 (denoted as NumTicksPocDiff1Minus1) is not present, a syntax element indicating NumTicksPocDiff1Minus1 is optionally signalled, the presence of which depends on a presence flag.

- Add the following constraint to disallow arbitrary output order of GFV-generated pictures generated from different picture units: For any two pictures picA and picB wherein picA precedes picB in output order, any GFV-generated picture corresponding to a GFV SEI message with a particular gfv_id value and associated with picA shall precede, in output order, any GFV-generated picture corresponding to a GFV SEI message with the particular gfv_id value and associated with picB.

In -v2 of this contribution, the above set of proposed is referred to as option 1, and another option, referred to as option 2, was added.

In option 2, similarly as in the NNPF case, only the output order of GFV-generated pictures is specified, without specifying picture order count and timing information. The proposed changes in option 2 are as follows:

- Add the following constraint to clearly specify the output order of GFV-generated pictures generated from one picture unit: The output order of GFV-generated pictures corresponding to the GFV SEI messages in a picture unit with the same gfv_id value and different gfv_cnt values shall be in increasing order of the gfv_cnt values.

- (same as change 3 in option 1) Add the following constraint to disallow arbitrary output order of GFV-generated pictures generated from different picture units: For any two pictures picA and picB wherein picA precedes picB in output order, any GFV-generated picture corresponding to a GFV SEI message with a particular gfv_id value and associated with picA shall precede, in output order, any GFV-generated picture corresponding to a GFV SEI message with the particular gfv_id value and associated with picB.

The detailed proposed text changes for option 1 are included in an attachment to this contribution. In the attachment in -v2 of this contribution, two typos have been fixed.

# Introduction

When a picture unit contains multiple GFV SEI messages with the same gfv_id value and different gfv_cnt values, the process StoreOutputTensors( ) derives an output picture for (or corresponding to) each of the GFV SEI messages. However, the output order of these output pictures (for simplicity, each output picture derived by the process StoreOutputTensors( ) is referred to as a GFV-generated picture) is not specified, and the timing information, e.g., the output time, of the GFV-generated pictures corresponding to the GFV SEI messages with gfv_cnt greater than 0 is missing even when the timing information for each cropped output picture is present.

Furthermore, currently it is not disallowed that a GFV-generated picture corresponding to a GFV SEI message in one picture unit is between, in output order, two GFV-generated pictures corresponding to GFV SEI messages in another picture unit.

To resolve the above issues, this contribution proposes a set of changes as summarized in the abstract.

# Patent rights declaration(s)

Bytedance Inc. and Shanghai Jiao Tong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).