| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0194-v1 |
| --- | --- |

| Title: | AHG9/AHG16: On chroma key fusion for the generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shiv Gehlot, Guan-Ming Su, Peng Yin, Sean McCarthy, Gary J. Sullivan (Dolby) | Email: | {pyin, smcca, gsull }@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes syntax and semantics for the generative face video SEI message to support use of chroma key for picture fusion. The syntax and semantics are proposed to address comments recorded in the meeting notes of the previous JVET meeting.

# Introduction

The generative face video (GFV) SEI message is included in JVET-AH2032, technologies under consideration for future extensions of VSEI (TuC for VSEI).

JVET-AH0118 describes the use of picture fusion to mitigate the entanglement of face and background [1]. The basic idea is to segment the face (foreground) from background. The background picture is generated by inpainting and sent as an additional picture. The features are generated from foreground picture only and conveyed via the GFV SEI message in the bitstream. At the receiver, fusion is used to merge the face generated from the features conveyed in the GFV SEI message with the background picture. As described in JVET-AH0118, segmentation is performed at both the encoder and receiver.

During discussion of JVET-AH0118, two comments were recorded in the meeting notes of the 34th JVET meeting (JVET-AH_notes_dG.docx):

- It was asked if segmentation needs to be identical at encoder and decoder, and which artifacts that might cause.

- It was asked if it was not also possible the send the segmented person as base picture with a uniform-colour background, as in chroma keying.

The comments point out: 1) if segmentation is different from encoder and decoder, it might cause artifacts; 2) if chroma key background is used, segmentation at decoder can be avoided, which could simplify the receiver and avoid artifacts caused by mismatch of segmentation at the encoder and receiver.

This contribution proposes syntax and semantics for the GFV SEI message to support use of chroma key for picture fusion as illustrated in Figure 1. The proposed changes are modelled on the chroma key information function specified in ITU-T H.263 [3].

Compared to JVET-AH0118, the current proposal avoids segmentation at the receiver and, thus, avoids artifacts that could be caused by mismatched segmentation at the encoder and receiver. For the example shown in Figure 1, the encoder-side segmentation and inpainting NN models referenced in JVET-AH0118 could be used.

Figure 1 Use of chroma key for fusion in the GFV SEI message. In the example shown, the green chroma key background colour corresponds to RGB values of 0, 177, and 64, respectively.

# Proposal

## Generative face video SEI message

Proposed changes to the GFV SEI message are relative to JVET-AH2032 and are highlighted in yellow.

### Generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| … |  |
| } |  |
| gfv_chroma_key_info_present_flag | u(1) |
| if( gfv_chroma_key_info_present_flag ) { |  |
| for( c = 0; c < 3; c++ ) { |  |
| gfv_chroma_key_value_present_flag[ c ] | u(1) |
| if( gfv_chroma_key_value_present_flag[ c ] ) |  |
| gfv_chroma_key_value[ c ] | u(8) |
| } |  |
| for( i = 0; i < 2; i++ ) { |  |
| gfv_chroma_key_thr_present_flag[ i ] | u(1) |
| if( gfv_chroma_key_thr_present_flag[ i ] ) |  |
| gfv_chroma_key_thr_value[ i ] | ue(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| … |  |
| } |  |

### Generative face video SEI message semantics

gfv_chroma_key_info_present_flag equal to 1 indicates that the syntax elements gfv_chroma_key_value_present_flag[ c ] and gfv_chroma_key_thr_present_flag[ i ] are present and the syntax elements and gfv_chroma_key_value[ c ] and gfv_chroma_key_thr_value[ i ] might be present. gfv_chroma_key_info_present_flag equal to 0 specifies that the syntax elements gfv_chroma_key_value_present_flag[ c ], gfv_chroma_key_thr_present_flag[ i ], gfv_chroma_key_value[ c ], and gfv_chroma_key_thr_value[ i ] are not present.

gfv_chroma_key_value_present_flag[ c ] equal to 1 indicates that the syntax element gfv_chroma_key_value[ c ] is present. gfv_chroma_key_present_flag[ c ] equal to 0 indicates that the syntax element gfv_chroma_key_value[ c ] is not present.

The variable ChromaKeyDefaultValueFlag is set equal to !( gfv_chroma_key_value_present_flag[ 0 ]  | |  gfv_chroma_key_value_present_flag[ 1 ]  | |  gfv_chroma_key_value_present_flag[ 2 ] ).

gfv_chroma_key_value[ c ] specifies the chroma key value corresponding to the c-th colour component as follows:

–	If ChromaKeyDefaultValueFlag is equal to 1, the variables GfvChromaKeyValue[ c ] are specified as follows:

–	GfvChromaKeyValue[ 0 ] is set equal to 50

–	GfvChromaKeyValue[ 1 ] is set equal to 220

–	GfvChromaKeyValue[ 2 ] is set equal to 100

–	Otherwise, ChromaKeyDefaultValueFlag is equal to 0, the following applies:

–	if gfv_chroma_key_value_present_flag[ c ] is equal to 1, GfvChromaKeyValue[ c ] is set equal to the value of gfv_chroma_key_value[ c ]

–	otherwise, gfv_chroma_key_value_present_flag[ c ] is equal to 0, GfvChromaKeyValue[ c ] is not specified by this Specification.

gfv_chroma_key_thr_present_flag[ i ] equal to 1 indicates that the syntax element gfv_chroma_thr_value[ i ] is present. gfv_chroma_key_thr_present_flag[ i ] equal to 0 indicates gfv_chroma_key_thr_value[ i ] is not present.

gfv_chroma_key_thr_value[ i ], when present, specifies the i-th chroma key threshold value. When not present, the value of gfv_chroma_key_thr_value[ i ] is inferred as follows:

If i is equal to 0, gfv_chroma_key_thr_value[ 0 ] is set equal to 48

Otherwise, i is equal to 1, gfv_chroma_key_thr_value[ 1 ] is set equal to 75

NOTE – The syntax elements gfv_chroma_key_value_present[ c ], gfv_chroma_key_value[ c ], and gfv_chroma_key_thr_value[ i ] could be used to determine a transparency indicator for fusion of the generated face picture and background picture. For example, a transparency indicator, denoted as alpha[ x ][ y ] , for picture sample value, denoted as I[ c ][ x ][ y ] with bitDepth[ c ], where bitDepth[ 0 ] is equal to BitDepthY, bitDepth[ 1 ] and bitDepth[ 2 ] are equal to BitDepthC, and GfvChromaKey[ c ] values for sample coordinates x, y, and colour components c could be determined as follows:

d[ x ][ y ]= 0
for( c = 0; c < 3; c++ )
	if( gfv_chroma_key_value_present[ c ]  | |  ChromaKeyDefaultValueFlag )
		d[ x ][ y ] += ( I[ c ][ x ][ y ] / ( 1 << (bitDepth[ c ] – 8) ) − GfvChromaKeyValue[ c ] )2	(XX)
if ( ( d[ x ][ y ] < gfv_chroma_key_thr_value[ 0 ] )
	alpha[ x ][ y ] = 0
else if ( ( d[ x ][ y ] > gfv_chroma_key_thr_value[ 1 ] )
	alpha[ x ][ y ] = 1
else
	alpha[ x ][ y ] = ( d[ x ][ y ] - gfv_chroma_key_thr_value[ 0 ] ) ÷ 	 	 ( gfv_chroma_key_thr_value[ 1 ] - gfv_chroma_key_thr_value[ 0 ] )

A value of alpha[ x ][ y ] equal to 0 could indicate transparency. A value of alpha[ x ][ y ] equal to 1 could indicate opacity. Intermediate values of alpha[ x ][ y ] could indicate semitransparency.

# Conclusion

It is recommended to adopt the proposed syntax and semantics to support chroma key background for the generative face video SEI message into the next version of VSEI.

# References

- S. Gehlot, G.-M. Su, P. Yin, S. McCarthy, G. J. Sullivan, “AHG9/AHG16: Showcase for picture fusion for generative face video SEI message”, JVET-AH0118, Rennes, FR, April, 2024.

- S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, and Hendry, “Technologies under consideration for future extensions of VSEI (version 3)”, JVET-AH2032, Rennes, FR, April, 2024.

- ITU-T H.263:Video coding for low bit rate communication, https://www.itu.int/rec/T-REC-H.263-200501-I/en

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).