| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0197-v1 |
| --- | --- |

| Title: | EE2-3.9: Affine candidates derived from temporal collocated pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Che-Wei Kuo, Wei Chen, Xianglin Wang | Email: | machangyue@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2 Test 3.9 on affine candidates derived from temporal collocated pictures. On top of ECM-13.0, simulation results of EE2 Test 3.9 are reported as below:

RA : {xxx%, xxx%, xxx%, xxx%, xxx%},

LDB : {xxx%, xxx%, xxx%, xxx%, xxx%};

# Introduction

Affine candidates derived from temporal collocated pictures were proposed in JVET-AH0208 [1], and included in EE2 [2] as Test 3.9 for further evaluation.

In Test 3.9, it is proposed to add the affine candidates derived from temporal collocated pictures into current affine merge candidate list. The same sampling grid used by regular inter merge mode is reused to scan the predefined positions in the collocated pictures for deriving the proposed affine candidates. Specifically, if the scanned position belongs to one affine coded CU, one new affine candidate is derived by scaling its CPMVs to the current CU based on its position and block-size in the collocated picture. In the current design, the derived new affine candidates are inserted into the existing affine merge list and reordered together with the other affine merge candidates through the ARMC. The number of the final output affine merge candidates after the ARMC is kept unchanged.

# Experimental results

Test 3.9 was implemented on top of ECM-13.0 [3] and tested by following the CTC [4]. The simulation results are summarized in the following tables.

Table 1: Performance of EE2-3.9

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.05% | -0.08% | 0.05% |  |  |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.02% | 0.04% |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |
| Class E | -0.29% | 0.06% | 0.04% |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.42% | -0.69% | -1.33% |  |  |  |  |

# Conclusion

In this contribution, the results of EE2 Test 3.9 on affine candidates derived from temporal collocated pictures are reported. It is recommended to adopt Test 3.9 into ECM.

# References

- C. Ma, X. Xiu, C.-W. Kuo, W. Chen, X.Wang, “Non-EE2: affine candidates derived from temporal collocated picture,” JVET-AH0208, April 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AH2024, April 2024.

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation JVET procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).