| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0198-v1 |
| --- | --- |

| Title: | Non-EE2: On POC conditions of BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ning Yan, Xiaoyu Xiu, Wei Chen, Xianglin Wang | Email: | yanning03@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In ECM-13.0, the bi-directional optical flow (BDOF) tool is only applied to refine the prediction samples of the inter CUs with “true” bi-prediction, i.e., two reference pictures are from two different direction with regard to the current picture. In this contribution, it is proposed to enable the BDOF for the bi-predictive CUs with both reference pictures from the same direction. Simulation results show that the proposed change provides 0.12% BD-rate savings for the LDB configuration with minor enc/dec runtime impacts.

Introduction

In ECM-13.0, BDOF is only applicable to the CUs that satisfy the following POC conditions:

  - The CU is coded using “true” bi-prediction, i.e., one of the two reference pictures is prior to the current picture and the other is after the current picture in display order.

  - The distances (i.e., POC difference) from two reference pictures to the current picture are same

Additionally, besides regular inter CUs, the BDOF is also applied to the affine and SbTMVP subblock-based CUs when the above BDOF condition is satisfied.

Proposal

This contribution proposes to apply the BDOF to the bi-prediction CUs where both reference pictures come from the same direction to the current picture. More specifically, the existing BDOF design in ECM-13.0 is kept exactly unchanged except that the following condition is introduced to additionally enable the BDOF for the CUs:

- The bi-prediction CUs having two different reference pictures (i.e., different POCs) that are either before or after the current picture in display order.

Experimental results

The proposed scheme was implemented on top EMC-13.0 and tested following the CTC [1]. The simulation results are summarized in the following table.

Table 1: Test results of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.01% | 0.00% | 99.9% | 99.2% | 99.9% | 99.9% |
| Class A2 | -0.02% | 0.01% | 0.02% | 100.1% | 100.0% | 100.4% | 100.5% |
| Class B | -0.01% | -0.02% | 0.00% | 100.1% | 100.4% | 100.6% | 100.6% |
| Class C | 0.02% | -0.03% | 0.05% | 100.0% | 99.9% | 99.6% | 99.5% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.02% | 100.0% | 99.9% | 100.1% | 100.2% |
| Class D | -0.03% | -0.03% | 0.12% | 99.9% | 100.0% | 100.0% | 100.0% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.05% | 0.23% | 100.7% | 100.8% | 100.0% | 100.0% |
| Class C | -0.10% | -0.46% | -0.34% | 101.2% | 102.0% | 100.4% | 100.6% |
| Class E | -0.17% | 0.64% | -0.53% | 101.6% | 102.3% | 100.1% | 100.2% |
| Overall | -0.12% | -0.02% | -0.15% | 101.1% | 101.6% | 100.2% | 100.2% |
| Class D | -0.14% | -0.33% | -0.70% | 100.8% | 102.6% | 99.5% | 99.3% |

Conclusion

The contribution proposes to extend BDOF to no “true” bi-prediction CUs. Experimental results show that the proposed method can provide noticeable coding gains with minor enc/dec run-time impact. It is recommended to study the proposed method in the next round of EE.

References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation JVET procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).