| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0202-v1 |
| --- | --- |

| Title: | AHG9: NNPFC SEI modifications to support spatial extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is proposed to support spatial extraction in the NNPFC SEI. It is asserted that spatial extraction can be supported by simply modifying spatial extrapolation text.

# Introduction

Spatial extraction is to extract part of input image to form a smaller image for output. Figure 1 shows an illustration of spatial extraction. It can be used to extract important part of the image, e.g. ROI or content-adaptive resolution adaptation.

Spatial extraction NNPF

Figure 1: Illustration of a spatial extraction NNPF.

# Proposed methods

To support spatial extraction NNPF, there are different options.

Option 1: add a new NNPFC purpose to support spatial extraction.

The modifications to the NNPFC SEI messages include adding a new bit mask for spatial extraction, adding syntax elements to indicate the extracted region.

It is noted that similar modifications are already made for spatial extraction. Thus, to simply the design and corresponding modifications, we can reuse the purpose of spatial extrapolation as follows.

Option 2: reuse spatial extrapolation purpose to support both spatial extraction and spatial extrapolation. The modifications include enabling signalling of negative offsets to indicate extracted region.

For both options, it is suggested having the following straightforward constraints:

1. The output width and height shall be greater than 0

2. Spatial extrapolation and spatial extraction shall not be used together.

# Proposed text

Text modifications of option 2 are shown below based on JVET-AH2006 [1]. The relevant parts that have been added or modified are highlighted in turquoise, and the deleted parts are highlighted in yellow strikethrough in red fonts. There may be some other changes that are editorial in nature and thus not highlighted.

8.28.2.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | use(v) |
| nnpfc_spatial_extrapolation_right_offset | use(v) |
| nnpfc_spatial_extrapolation_top_offset | use(v) |
| nnpfc_spatial_extrapolation_bottom_offset | use(v) |
| } |  |
| ... |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| ... |  |

8.28.2.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application and as specified by the nnpfc_application_purpose_tag_uri.

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

The value of nnpfc_purpose shall be in the range of 0 to 255, inclusive, in bitstreams conforming to this version of this Specification. Values of 256 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 256 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |
| 0x80 | Spatial extrapolation (i.e., generating content outside of the spatial area of the input pictures) or extraction (i.e. extract partial content from the input pictures) |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, ColourizationFlag, and TemporalExtrapolationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, colourization, and temporal extrapolation, respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0
SpatialExtrapolationFlag = ( ( nnpfc_purpose & 0x80 ) > 0 ) ? 1 : 0

...

nnpfc_pic_width_num_minus1 plus 1 and nnpfc_pic_width_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the width of the NNPF-generated pictures relative to CroppedWidth. Both nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 shall be in the range of 0 to 65 535, inclusive.

The value of ( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are not present, the values of nnpfc_pic_width_num_minus1 and nnpfc_pic_width_denom_minus1 are both inferred to be equal to 0.

The variable nnpfcOutputPicWidth, representing the width of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicWidth = Ceil( CroppedWidth *		(77)
		( nnpfc_pic_width_num_minus1 + 1 ) ÷ ( nnpfc_pic_width_denom_minus1 + 1 ) )

When SpatialExtrapolationFlag is equal to 1, nnpfcOutputPicWidth is updated as follows:

nnpfcOutputPicWidth += outSubWidthC * ( nnpfc_spatial_extrapolation_left_offset +	(77)
		nnpfc_spatial_extrapolation_right_offset )

It is a requirement of bitstream conformance that the value of nnpfcOutputPicWidth shall be greater than 0 and nnpfcOutputPicWidth % outSubWidthC shall be equal to 0.

nnpfc_pic_height_num_minus1 plus 1 and nnpfc_pic_height_denom_minus1 plus 1 specify the numerator and denominator, respectively, for the resampling ratio of the height of the NNPF-generated pictures relative to CroppedHeight. Both nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 shall be in the range of 0 to 65 535, inclusive.

The value of ( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) shall be in the range of 1 ÷ 16 to 16, inclusive. When nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are not present, the values of nnpfc_pic_height_num_minus1 and nnpfc_pic_height_denom_minus1 are both inferred to be equal to 0.

The variable nnpfcOutputPicHeight, representing the height of the luma sample arrays of the NNPF-generated pictures, is derived as follows:

nnpfcOutputPicHeight = Ceil( CroppedHeight *		(78)
		( nnpfc_pic_height_num_minus1 + 1 ) ÷ ( nnpfc_pic_height_denom_minus1 + 1 ) )

When SpatialExtrapolationFlag is equal to 1, nnpfcOutputPicHeight is updated as follows:

nnpfcOutputPicHeight += outSubWidthC * ( nnpfc_spatial_extrapolation_top_offset +	(78)
		nnpfc_spatial_extrapolation_bottom_offset )

It is a requirement of bitstream conformance that the value of nnpfcOutputPicHeight shall be greater than 0 and nnpfcOutputPicHeight % outSubHeightC shall be equal to 0.

When ResolutionResamplingFlag is equal to 1, at least one the following conditions shall be true:

–	The value of nnpfcOutputPicWidth is not equal to CroppedWidth.

–	The value of nnpfcOutputPicHeight is not equal to CroppedHeight.

–	SpatialExtrapolationFlag is equal to 1.

...

nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset, and nnpfc_spatial_extrapolation_bottom_offset specify the spatial extrapolation or spatial extraction area. The luma samples with horizontal picture coordinates from outSubWidthC * nnpfc_spatial_extrapolation_left_offset to nnpfcOutputPicWidth − ( outSubWidthC * nnpfc_spatial_extrapolation_right_offset ) and vertical picture coordinates from outSubWidthHeightC * nnpfc_spatial_extrapolation_top_offset to nnpfcOutputPicWidthHeight − ( outSubWidthHeightC * nnpfc_spatial_extrapolation_bottom_offset ) correspond to the spatial area of the input picture. The value of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be in the range of 0 −65 536 to 65 536, inclusive. At least one of nnpfc_spatial_extrapolation_left_offset, nnpfc_spatial_extrapolation_right_offset, nnpfc_spatial_extrapolation_top_offset and nnpfc_spatial_extrapolation_bottom_offset shall be greater than not equal to 0.

One of the following conditions shall be true:

–	The values of nnpfc_spatial_extra_left_offset, nnpfc_spatial_extra_right_offset, nnpfc_spatial_extra_top_offset, and nnpfc_spatial_extra_bottom_offset are all less than or equal to 0.

–	The values of nnpfc_spatial_extra_left_offset, nnpfc_spatial_extra_right_offset, nnpfc_spatial_extra_top_offset, and nnpfc_spatial_extra_bottom_offset are all greater than or equal to 0.

...

# References

[1] J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, S. McCarthy, G. J. Sullivan, Y.-K. Wang, “Additional SEI messages for VSEI version 4 (Draft 2)” JVET output document JVET-AH2006

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).