| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12-19 July 2024 | Document: JVET-AI0203-v1 |
| --- | --- |

| Title: | AHG9: SEI message for signalling AI usage restrictions and context |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arianne T. Hinds Stephan Wenger Gilles Teniou | Tel: Email: | {ahinds, swenger, teniou} @global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

As artificial intelligence applications continue to gain both in sophistication and popularity, concerns from content owners and creators about privacy, ownership, and the creation of deep fakes likewise magnify. This proposal suggests the creation of an SEI message to signal usage restrictions and contextual scenarios for how content may or may not be used by AI applications, including for example, training of AI models.

# Introduction

JVET has recently studied the inclusion of metadata in the form of text strings carried by an SEI message for information such as AI marking, copyright, and other useful indicators as described in JVET-AH0343. However, there still exists a gap in which content creators or content owners can signal how a particular video sequence may be used, if at all, for AI application scenarios. Such scenarios include training of AI models, usage in generative AI, or more broadly, any scenario in which AI plays a role. Likewise, it may be desirable to signal whether a restriction on how content may be used is relevant for commercial purposes, academic purposes, or government use. Finally, it may be helpful to indicate where to find additional information, e.g., “for more information, write to abc@xyz.com.”

This proposal suggests that an SEI be created to signal usage restrictions (or no restrictions at all) along with context information, and possibly an additional “more information” syntax element, for AI applications.

# AI usage restrictions SEI

## General

The primary syntax elements of such an SEI include:

- an extensible binary element to signal the particular restriction or to indicate that there are no restrictions. This element may be interpreted by an accompanying table that indicates how the particular binary restriction is to be interpreted.

- an extensible binary element to signal the context in which the restriction (or no restriction) applies which is likewise interpreted by an accompanying table. This particular syntax element may or may not be present depending on a flag value.

- a string element to provide additional information which may or may not be present depending on flag value.

Other considerations include the possibility that one or more restrictions, each with its own particular context exists and therefore, there is a need to offer multiple instances of restriction and context pairs.

## AI Usage Restrictions SEI Syntax

| AI_usage_restrictions ( payloadSize ) { | Descriptor |
| --- | --- |
| aur_cancel_flag | u(1) |
| if (!aur_cancel_flag) { |  |
| aur_persistence_flag | u(1) |
| aur_num_restrictions_minus1 | u(8) |
| for( i = 0; i <= aur_num_restrictions_minus1; i++ ) |  |
| aur_restriction | u(16) |
| aur_context_present_flag | u(1) |
| if (aur_context_present_flag) |  |
| aur_context | u(16) |
| } |  |
| aur_contact_info_present_flag | u(1) |
| if (aur_contact_info_present_flag) { |  |
| while (!byte_aligned()) |  |
| aur_payload_bit_equal_to_zero | f(1) |
| aur_contact_info | st(v) |
| } |  |
| } |  |
| } |  |

## AI usage restrictions SEI message semantics

The artificial intelligence (AI) usage restrictions SEI message signals restriction and optional context and contact information for usage by AI applications.

aur_cancel_flag equal to 1 specifies that the SEI message cancels the persistence of any previous AI usage restrictions SEI message in output order. aur_cancel_flag equal to 0 specifies that AI usage restriction information follows.

aur_persistence_flag specifies the persistence of the AI usage restrictions SEI message for the current layer.

aur_persistence_flag equal to 0 specifies that the AI usage restrictions SEI message applies to the current decoded picture only.

aur_persistence_flag equal to 1 specifies that the AI usage restrictions SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an ICC profile SEI message is output that follows the current picture in output order.

aur_num_restrictions_minus1 plus one specifies the number of restriction entries that follow.

aur_restriction indicates the restriction or no restriction as specified in Table xx, where (aur_restriction & bitMask ) not equal to zero indicates that the restriction associated with the bitMask value in Table xx exists. When aur_restriction is greater than zero and ( aur_restriction & bitMask ) is equal to zero, the restriction associated with the bitMask value is not applicable. When aur_restriction is equal to zero, no restriction exists.

Table xx – Definition of aur_restriction

| Bitmask | Interpretation |
| --- | --- |
| 0x0001 | Do not use for AI training |
| 0x0002 | Do not use for generative (modification or creation) AI |
| 0x0004 | Do not use in any AI application |
| 0x0008 to 0xFFFF | Reserved for future use of ITU and ISO |

aur_context_present_flag indicates the presence of context information for the value in aur_restriction.

aur_context indicates the context for aur_restriction  as specified in Table xy, where (aur_context & bitMask ) not equal to zero indicates that the context associated with the bitMask value in Table xy applies. When aur_context is greater than 0 and ( aur_restriction & bitMask ) is equal to zero, the context associated with the bitMask value is not applicable.

aur_context shall not be equal to 0.

Table xx – Definition of aur_context

| Bitmask | Interpretation |
| --- | --- |
| 0x0001 | commercial use |
| 0x0002 | non-commercial use |
| 0x0004 | official government use |
| 0x0004 | research and academic use |
| 0x0008 to 0xFFFF | Reserved for future use of ITU and ISO |

aur_contact_info_present_flag indicates the presence of contact information for the AI useage restriction SEI message.

aur_payload_bit_equal_to_zero shall be equal to 0.

aur_contact_info contains a string that should include contact information for an entity through which additional information may be obtained.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).