| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0211-v1 |
| --- | --- |

| Title: | AHG9: On the design of OMI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Chul Keun Kim Junghak Nam Jangwon Lee Jaehyun Lim Seung Hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted that the current design of object mask information (OMI) SEI message may not be efficient in the case when objects in the associated pictures have high motion / movement. In such case, each picture may be associated with different OMI SEI messages and information in those OMI SEI messages may be redundantly repeated.

In this contribution, a new SEI message called object mask base (OMB) SEI message is proposed and some information that are currently signal in OMI SEI message may be moved to the proposed OMB SEI message. The detail design of OMB SEI message and changes to OMI SEI message are as follows:

- An OMB SEI contains information about object mask that does not have to be updated frequently last until it is updated by a new OMB SEI or the CLVS ends.

- OMB SEI message may include information such as the following:

  - Object mask confidence values

  - Object mask depth information

  - Object mask label information.

- Update the signalling in object mask information (OMI) SEI message such that those information that are present in OMB SEI message are not signalled in OMI SEI message or optionally present in OMI in the case it update / override.

- Some information that are present in an OMB SEI message may be updated in OMI SEI message. When the information is not present in an OMI SEI message, it is inferred to be equal to the corresponding information signalled in the immediate OMB SEI message that precedes the OMI SEI message in output order.

- It is constrained that for any OMI SEI message, there shall be at least one OMB SEI message is present and precedes it in output order.

- It is constrained that an object mask that is present in an OMI SEI message shall be present in the OMB that immediately precedes the OMI SEI message in output order.

# Problem description

Object mask information (OMI) SEI message is designed to provide information about object mask on pictures it is associated with. This provides information for object tracking on the decoded pictures. The object mask may need to be updated frequently depending on how much motion the object moves across pictures. It may not be unexpected that each picture is associated with different OMI SEI messages in a bitstream with high motion. This could cost a lot of bits to signal.

In the case where pictures have high motion objects, it is common that a lot of information in the consecutive OMI SEI message in a CLVS would be repeated information. In the case of bitstream with high moving objects, we assert that the information in OMI SEI message that may need frequent updates includes object mask sample value and the bounding box. Other information such as confidence information, mask label, mask depth, etc, may be updated as necessary basis. Therefore, it would be desirable that these information are not repeated redundantly and there is a way to signal them more efficiently.

We also notice that in the current design it is described that information that are not present in an OMI SEI message is inferred or inherited from preceding OMI SEI message in output order. However, in our understanding, it does not say anything about how far back the inference can be. Given that in the current design there are many different information are present (i.e., object mask sample, object mask bound box, object mask depth, object mask confidence value, object mask label, etc), this inference to the previous OMI SEI can get complicated. Furthermore, this would cause decoding system needs to store multiple previous OMI SEI messages, which is not desirable.

# Proposal

To overcome the asserted problems described above, we propose a new SEI message called object mask base (OBM) SEI message. The design of OMB SEI message and its relationship to OMI SEI message is as follows:

- An OMB SEI contains information about object mask that does not have to be updated frequently last until it is updated by a new OMB SEI or the CLVS ends.

- OMB SEI message may include information such as the following:

  - Object mask confidence values

  - Object mask depth information

  - Object mask label information.

- Update the signalling in object mask information (OMI) SEI message such that those information that are present in OMB SEI message are not signalled in OMI SEI message or optionally present in OMI in the case it update / override.

- Some information that are present in an OMB SEI message may be updated in OMI SEI message. When the information is not present in an OMI SEI message, it is inferred to be equal to the corresponding information signalled in the immediate OMB SEI message that precedes the OMI SEI message in output order.

- It is constrained that for any OMI SEI message, there shall be at least one OMB SEI message is present and precedes it in output order.

- It is constrained that an object mask that is present in an OMI SEI message shall be present in the OMB that immediately precedes the OMI SEI message in output order.

# Text implementation

Object mask base SEI message syntax

| object_mask_base( payloadSize ) { | Descriptor |
| --- | --- |
| omb_num_aux_pic_layer | ue(v) |
| omb_mask_id_length_minus1 | ue(v) |
| omb_mask_confidence_info_present_flag | u(1) |
| if( omb_mask_confidence_info_present_flag ) |  |
| omb_mask_confidence_length_minus1 | u(4) |
| omb_mask_depth_info_present_flag | u(1) |
| if( omb_mask_depth_info_present_flag ) |  |
| omb_mask_depth_length_minus1 | u(4) |
| omb_mask_label_info_present_flag | u(1) |
| if( omb_mask_label_info_present_flag ) { |  |
| omb_mask_label_language_present_flag | u(1) |
| if( omb_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omb_bit_equal_to_zero | f(1) |
| omb_mask_label_language | st(v) |
| } |  |
| } |  |
| for( i = 0; i < omb_num_aux_pic_layer; i++ ) { |  |
| omb_num_mask[ i ] | ue(v) |
| for( j = 0; j < omb_num_mask[ i ]; j++ ) { |  |
| omb_mask_id[ i ][ j ] | u(v) |
| if( omb_mask_confidence_info_present_flag ) |  |
| omb_mask_confidence[ i ][ j ] | u(v) |
| if( omb_mask_depth_info_present_flag ) |  |
| omb_mask_depth[ i ][ j ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omb_bit_equal_to_zero | f(1) |
| if( omb_mask_label_info_present_flag ) |  |
| omb_mask_label[ i ][ j ] | st(v) |
| } |  |
| } |  |
| } |  |

Object mask base SEI message semantics

The object mask base (OMB) SEI message provides object mask information of object mask picture in the auxiliary picture layer associated with the primary picture layer, referred to as the current primary picture layer, in which the SEI message is present. If the OMB SEI message is present, it shall be present in a primary picture layer. One primary picture layer can be associated with one or more auxiliary picture layers. For each of the associated auxiliary picture layers containing the object mask picture with nuh_layer_id equal to sdi_layer_id[ i ], the value of sdi_aux_id[ i ] shall be equal to AUX_OBJECT_MASK, for any value of i in range of 0 to sdi_max_layers_minus1, inclusive.

The object mask base SEI message applies to the current picture and all subsequent pictures in the same layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in a PU containing an object mask base SEI message is output that follows the current picture in output order.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i, the OMB SEI message shall be ignored.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i and an OMB SEI message, the SDI SEI message shall precede the OMB SEI message in decoding order.

omb_num_aux_pic_layer indicates the number of auxiliary picture layers associated with the current primary picture layer. It is a requirement of bitstream conformance that the value of omb_num_aux_pic_layer shall be equal to numAuxLayer where the variable numAuxLayeris derived as follows.

Let ombPrimaryLayerId be the value of nuh_layer_id of the NAL unit that contains the SEI message.

numAuxLayer = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ )
	if( sdi_aux_id[ i ] = = AUX_OBJECT_MASK )
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ )			(xx)
			if( sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ]  = =  ombPrimaryLayerId )
				numAuxLayer++;

omb_mask_id_length_minus1 plus 1 specifies the length, in bits, of omb_mask_id[ i ][ j ] syntax elements.

omb_mask_confidence_info_present_flag equal to 1 indicates that omb_mask_confidence[ i ][ j ] syntax elements are present. omb_mask_confidence_info_present_flag equal to 0 indicates that omb_mask_confidence[ i ][ j ]syntax elements are not present.

omb_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omb_mask_confidence[ i ][ j ] syntax elements.

omb_mask_depth_info_present_flag equal to 1 indicates that omb_mask_depth[ i ][ j ] syntax elements are present. omb_mask_depth_info_present_flag equal to 0 indicates that omb_mask_depth[ i ][ j ]syntax elements are not present.

omb_mask_depth_length_minus1 plus 1 specifies the length, in bits, of the omb_mask_depth[ i ][ j ] syntax elements.

It is a requirement of bitstream conformance that the value of omb_num_aux_pic_layer, omb_mask_id_length_minus1, omb_mask_confidence_info_present_flag, omb_mask_confidence_length_minus1, if present, omb_mask_depth_info_present_flag and omb_mask_depth_length_minus1, if present, shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omb_mask_label_info_present_flag equal to 1 indicates that omb_mask_label_language_present_flag and omb_mask_label[ i ][ j ] syntax elements are present. omb_mask_label_info_present_flag equal to 0 indicates that omb_mask_label_language_present_flag and omb_mask_label[ i ][ j ] syntax elements are not present.

omb_mask_label_language_present_flag equal to 1 indicates that omb_mask_label_language syntax element is present. omb_mask_label_language_present_flag equal to 0 indicates that omb_mask_label_language syntax element is not present.

omb_bit_equal_to_zero shall be equal to 0.

omb_mask_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omb_mask_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

omb_num_mask[ i ] specifies the number of object masks in object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. omb_num_mask[ i ] shall be in the range of 0 to (1<<(omb_mask_id_length_minus1 + 1)) – 1, inclusive.

omb_mask_id[ i ][ j ] indicates the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omb_mask_id[ i ][ j ] syntax element is omb_mask_id_length_minus1 + 1 bits.

The variable maskId[ i ][ j ] specifying the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is derived as follows:

for( i = 0; i < omb_num_aux_pic_layer; i++ )
	for( j = 0; j < omb_num_mask_in_pic_update[ i ]; j++ ) 	(xx)
		maskId[ i ][ j ] = omb_mask_id[ i ][ j ] + (1<<(omb_mask_id_length_minus1 + 1))*i

omb_mask_confidence[ i ][ j ] specifies the degree of confidence associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer, in units of 2-( omb_mask_confidence_length_minus1 + 1 ), such that a higher value of omb_mask_confidence[ i ][ j ] indicates a higher degree of confidence. The length of the omb_mask_confidence[ i ][ j ] syntax element is omb_mask_confidence_length_minus1 + 1 bits.

omb_mask_depth[ i ][ j ] specifies the object depth associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. A smaller value of omb_mask_depth indicates a shorter distance to the object. The length of the omb_mask_depth[ i ][ j ] syntax element is omb_mask_depth_length_minus1 + 1 bits.

omb_mask_label[ i ][ j ] specifies the contents of the label associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omb_mask_label[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

…

Object mask information SEI message syntax

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !omi_cancel_flag ) { |  |
| omi_persistence_flag | u(1) |
| omi_num_aux_pic_layer | ue(v) |
| omi_mask_id_length_minus1 | ue(v) |
| omi_mask_sample_value_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if( omi_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_label_language | st(v) |
| } |  |
| } |  |
| for( i = 0; i < omi_num_aux_pic_layer; i++ ) { |  |
| omi_mask_pic_update_flag[ i ] | f(1) |
| if( omi_mask_pic_update_flag[ i ] ) { |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_update_present_flag[ i ] | u(1) |
| omi_num_mask_in_pic_update[ i ] | ue(v) |
| for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) { |  |
| omi_mask_id[ i ][ j ] | u(v) |
| omi_aux_sample_value[ i ][ j ] | u(v) |
| omi_mask_cancel[ i ][ j ] | u(1) |
| if( !omi_mask_cancel[ i ][ j ] ) { |  |
| omi_mask_bounding_box_present_flag[ i ][ j ] | u(1) |
| if( omi_mask_bounding_box_present_flag[ i ][ j ] ) { |  |
| omi_mask_top[ i ][ j ] | u(16) |
| omi_mask_left[ i ][ j ] | u(16) |
| omi_mask_width[ i ][ j ] | u(16) |
| omi_mask_height[ i ][ j ] | u(16) |
| } |  |
| if( omi_mask_confidence_infoupdate_present_flag[ i ] ) |  |
| omi_mask_confidence[ i ][ j ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ][ j ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| if( omi_mask_label_info_present_flag ) |  |
| omi_mask_label[ i ][ j ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Object mask information SEI message semantics

The object mask information (OMI) SEI message provides object mask information of object mask picture in the auxiliary picture layer associated with the primary picture layer, referred to as the current primary picture layer, in which the SEI message is present. If the OMI SEI message is present, it shall be present in a primary picture layer. One primary picture layer can be associated with one or more auxiliary picture layers. For each of the associated auxiliary picture layers containing the object mask picture with nuh_layer_id equal to sdi_layer_id[ i ], the value of sdi_aux_id[ i ] shall be equal to AUX_OBJECT_MASK, for any value of i in range of 0 to sdi_max_layers_minus1, inclusive.

Use of this SEI message requires the definition of the following variables:

–	A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	A conformance cropping window left offset, ConfWinLeftOffset

–	A conformance cropping window top offset, ConfWinTopOffset

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in the same layer, if present, in output order. omi_cancel_flag equal to 0 indicates that object mask information follows.

omi_persistence_flag specifies the persistence of the object mask information provided in this SEI message. omi_persistence_flag equal to 0 specifies that the object mask information applies to the current picture only. omi_persistence_flag equal to 1 specifies that the object mask information applies to the current picture and all subsequent pictures in the same layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in a PU containing an object mask information SEI message is output that follows the current picture in output order.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i, the OMI SEI message shall be ignored.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

There shall be at least one object mask base (OMB) SEI message is present and precedes the OMI SEI message in output order.

omi_num_aux_pic_layer indicates the number of auxiliary picture layers associated with the current primary picture layer. It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer shall be equal to numAuxLayer where the variable numAuxLayeris derived as follows.

Let omiPrimaryLayerId be the value of nuh_layer_id of the NAL unit that contains the SEI message.

numAuxLayer = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ )
	if( sdi_aux_id[ i ] = = AUX_OBJECT_MASK )
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ )			(xx)
			if( sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ]  = =  omiPrimaryLayerId )
				numAuxLayer++;

omi_mask_id_length_minus1 plus 1 specifies the length, in bits, of omi_mask_id[ i ][ j ] syntax elements.

omi_mask_sample_value_length_minus8 plus 8 specifies the length, in bits, of omi_aux_sample_value[ i ][ j ] syntax elements. The value of omi_mask_sample_value_length_minus8 shall be in the range of 0 to 8.

omi_mask_confidence_info_present_flag equal to 1 indicates that omi_mask_confidence[ i ][ j ] syntax elements aremay be present. omi_mask_confidence_info_present_flag equal to 0 indicates that omi_mask_confidence[ i ][ j ]syntax elements are not present.

omi_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_confidence[ i ][ j ] syntax elements.

omi_mask_depth_info_present_flag equal to 1 indicates that omi_mask_depth[ i ][ j ] syntax elements are present. omi_mask_depth_info_present_flag equal to 0 indicates that omi_mask_depth[ i ][ j ]syntax elements are not present.

omi_mask_depth_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_depth[ i ][ j ] syntax elements.

It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer, omi_mask_id_length_minus1, omi_mask_sample_value_length_minus8, omi_mask_confidence_info_present_flag, omi_mask_confidence_length_minus1, if present, omi_mask_depth_info_present_flag and omi_mask_depth_length_minus1, if present, shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_mask_label_info_present_flag equal to 1 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ] syntax elements are present. omi_mask_label_info_present_flag equal to 0 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ] syntax elements are not present.

omi_mask_label_language_present_flag equal to 1 indicates that omi_mask_label_language syntax element is present. omi_mask_label_language_present_flag equal to 0 indicates that omi_mask_label_language syntax element is not present.

omi_bit_equal_to_zero shall be equal to 0.

omi_mask_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omi_mask_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

omi_mask_pic_update_flag[ i ] equal to 1 indicates the object mask information of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer may be updated. omi_mask_pic_update_flag[ i ] equal to 0 indicates there is no change to the mask information of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. When omi_mask_pic_update_flag[ i ] is equal to 0, the persistence mechanism is used, that is the object mask sample value and bounding box information is are inherited from the last OMI SEI message present in the same layer in decoding order which signals the mask information of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer.

omi_mask_confidence_update_present_flag[ i ] equal to 1 indicates that omi_mask_confidence[ i ][ j ] syntax elements are present. omi_mask_confidence_update_present_flag[ i ] equal to 0 indicates that omi_mask_confidence[ i ][ j ]syntax elements are not present. When omi_mask_confidence_info_present_flag is equal to 1, there shall be at least one value of i in the range from 0 to omi_num_aux_pic_layer – 1, inclusive, where the value of omi_mask_confidence_update_present_flag[ i ] is equal to 1.

omi_num_mask_in_pic_update[ i ] specifies the number of object masks in object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. omi_num_mask_in_pic_update [ i ] shall be in the range of 0 to (1<<(omi_mask_id_length_minus1 + 1)) – 1, inclusive.

omi_mask_id[ i ][ j ] indicates the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omi_mask_id[ i ][ j ] syntax element is omi_mask_id_length_minus1 + 1 bits.

The variable maskId[ i ][ j ] specifying the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is derived as follows:

for( i = 0; i < omi_num_aux_pic_layer; i++ )
	for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) 	(xx)
		maskId[ i ][ j ] = omi_mask_id[ i ][ j ] + (1<<(omi_mask_id_length_minus1 + 1))*i

It is constrained that for any maskId[ i ][ j ], there shall be corresponding value of maskId[ x ][ y ] present in the OMB SEI message that precedes the OMI SEI message in output order.

omi_aux_sample_value[ i ][ j ] specifies the value of the samples within the area of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer.

omi_mask_cancel[ i ][ j ] equal to 1 cancels the persistence scope of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture. omi_mask_cancel[ i ][ j ] equal to 0 specifies that the information of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is signalled.

It is a requirement of bitstream conformance that when omi_mask_id[ i ][ j ] with a particular value is parsed for the first time in the current CLVS, the value of the corresponding omi_mask_cancel[ i ][ j ] shall be equal to 0.

omi_mask_bounding_box_present_flag[ i ][ j ] equal to1 specifies that the syntax elements omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ], are present. omi_mask_bounding_box_present_flag[ i ][ j ] equal to 0 indicates syntax elements, omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ], are not present.

omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the j-th object mask in the cropped decoded object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer, relative to the conformance cropping window specified by the active SPS.

The value of omi_mask_left[ i ][ j ] shall be in the range of 0 to ( CroppedWidth / SubWidthC – 1 ), inclusive, CroppedWidth and SubWidthC being associated to the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. When it is not present, the value of omi_mask_left[ i ][ j ] is inferred to be 0.

The value of omi_mask_top[ i ][ j ] shall be in the range of 0 to ( CroppedHeight / SubHeightC – 1 ), inclusive, CroppedHeight  and SubHeightC  being associated to the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. When it is not present, the value of omi_mask_top[ i ][ j ] is inferred to be 0.

The value of omi_mask_width[ i ][ j ] shall be in the range of 0 to ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ] ), inclusive. When it is not present, the value of omi_mask_width [ i ][ j ] is inferred to be ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ] ).

The value of omi_mask_height[ i ][ j ] shall be in the range of 0 to ( CroppedHeight / SubHeightC − omi_mask_top[ i ][ j ] ), inclusive. When it is not present, the value of omi_mask_height [ i ][ j ] is inferred to be ( CroppedHeight / SubWidthC − omi_mask_top[ i ][ j ] ).

The identified object mask is within a bounding box containing luma samples with horizontal coordinates from SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ] ) to SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ] + omi_mask_width[ i ][ j ] ) − 1, inclusive, and vertical coordinates from SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ] ) to SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ] + omi_mask_height[ i ][ j ] ) − 1, inclusive.

Variable pI[ i ] [ x ][ y ] is the decoded value of the sample at the relative sample location (x, y) in the cropped object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The following process is to determine mask area in a auxiliary picture.

for( i = 0; i < omi_num_aux_pic_layer; i++ )
	for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ )
		if( pI[ i ][ x ][ y ]  = =  omi_aux_sample_value [ i ][ j ]
				&&  x  >=  omi_mask_left[ i ][ j ]
				&&  x < omi_mask_left[ i ][ j ] + omi_mask_width[ i ][ j ]	(xx)
				&&  y  >=  omi_mask_top[ i ][ j ]
				&&  y < omi_mask_top[ i ][ j ] + omi_mask_height[ i ][ j ] )
			the sample at location (x, y) is associated with the object mask with the identifier of maskId[ i ][ j ]

omi_mask_confidence[ i ][ j ] specifies the degree of confidence associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ][ j ] indicates a higher degree of confidence. The length of the omi_mask_confidence[ i ][ j ] syntax element is omi_mask_confidence_length_minus1 + 1 bits. When not present, this information is inherited from the last OMB SEI message present in the same layer in decoding order which includes the information for the particular object mask.

omi_mask_depth[ i ][ j ] specifies the object depth associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. A smaller value of omi_mask_depth indicates a shorter distance to the object. The length of the omi_mask_depth[ i ][ j ] syntax element is omi_mask_depth_length_minus1 + 1 bits.

omi_mask_label[ i ][ j ] specifies the contents of the label associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omi_mask_label[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).