| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0214-v1 |
| --- | --- |

| Title: | AHG9: Editorial updates for SPTI and EOI SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Tan Jangwon Lee Chul Keun Kim Junghak Nam Jaehyun Lim Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes several editorial modifications for VSEI v4 draft. The proposed modifications are as follows:

(Related to SPTI SEI message)

- Clarify the first sentence in the first paragraph in the semantics of SPTI SEI message.

- Change syntax element name from spti_max_sublayers_minus_1 to spti_max_sublayers_minus1.

- Clarify the TemporalId in the semantics of spti_max_sublayers_minus_1.

- Update the semantics of spti_sublayer_interval_scale_factor[ i ].

(Related to EOI SEI message)

- Change syntax element name from eoi_privacy_protection_type_idc to eoi_privacy_protection_method_idc.

- Change syntax element name from eoi_privacy_protection_info_type to eoi_protected_privacy_info_type

# On the first paragraph in the semantics of SPTI SEI message

## Problem statement

In our assertion, the first sentence in the first paragraph (shown in cyan highlight below) in the semantics of SPTI SEI message should be actually written into two sentences.

The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, the temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order. The information provided by the SPTI SEI message pertains only for picture(s) starting from the picture in the current layer in the access unit that contains the SPTI SEI message and all subsequent pictures of the current layer in output order based on its persistence. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.]

## Proposed modification

The source picture timing information (SPTI) SEI message indicates the temporal distance between source pictures associated with the corresponding decoded output pictures prior to encoding, e.g., for camera-captured content, t.The temporal distance between source pictures is the difference between the time at which an image sensor was exposed to produce a source picture associated with the current decoded picture and the time at which the image sensor was exposed to produce the source picture associated with a previous decoded picture in output order. The information provided by the SPTI SEI message pertains only for picture(s) starting from the picture in the current layer in the access unit that contains the SPTI SEI message and all subsequent pictures of the current layer in output order based on its persistence. [Ed. Check phrasing of this. “Pertains” is not used in a similar way anywhere in the standard.]

# On the syntax element name of spti_max_sublayers_minus_1

It is proposed to change spti_max_sublayers_minus_1 to spti_max_sublayers_minus1

# On TemporalId in the semantics of spti_max_sublayers_minus_1

## Problem statement

It is asserted that the semantics of spti_max_sublayers_minus_1 may be improved in regard to the mentioning of variable TemporalId. While our understanding that the TemporalId in the sentence “When spti_max_sublayers_minus_1 is not present, it is inferred to be equal to TemporalId.” means the TemporalId of the SPTI SEI message, it would be bette if it is clearly described.

## Proposed modification

spti_max_sublayers_minus_1 plus 1 specifies the maximum number of temporal sublayers for which picture interval scale factor (spti_sublayer_interval_scale_factor[ i ]) and synthesized flag (spti_sublayer_synthesized_picture_flag[ i ]) information is signalled. When spti_max_sublayers_minus_1 is not present, it is inferred to be equal to TemporalId of the SPTI SEI message.

# On semantics of spti_sublayer_interval_scale_factor[ i ]

## Problem statement

In our understanding, the semantics of spti_sublayer_interval_scale_factor[ i ] is about interval between two source pictures (i.e., source picture associated with the current decoded picture and source picture associated with previous decoded picture with TemporalId less than or equal to i). However, the current text is confusing, particularly the part that is shown in cyan highlight below:

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding pictures in the CLVS having TemporalId equal to i relative to the previous output picture with TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture with TemporalId less than or equal to i.

## Proposed modification

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the interval between the source picture associated with the current picture and the source picture associated with the previous output picture with TemporalId less than or equal to i. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture with TemporalId less than or equal to i.

# On the syntax element name of eoi_privacy_protection_type_idc

It is asserted that the name of the syntax element eoi_privacy_protection_type_idc is not intuitive to give impression what the indication is about.

It is recommended to change it to eoi_privacy_protection_method_idc.

# On the syntax element name of eoi_privacy_protected_info_type

It is asserted that the name of the syntax element eoi_privacy_protected_info_type is not intuitive to give impression what the indication is about.

It is recommended to change it to eoi_protected_privacy_info_type.

# Patent rights declaration(s)

LG Electronics Inc. does not have any current or pending patent rights relating to the technology described in this contribution.