| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0217-v2 |
| --- | --- |

| Title: | EE2-related: on the sharp motion compensation filter for bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Ruoyang Yu, Vadim Seregin, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes some fixes to the EE2 Test 3.10 on the sharp motion compensation filter for bi-prediction. In test a, it reports the following result {Y, U, V, EncT, DecT, EncVmPeak,DecVmPeak} under CTC:

RA {-0.08%, -0.01%, 0.04%, 98.9%, 98.5%, 99.9%,99.9%}

LDB {-0.06%, 0.12%, -0.27%, 99.3%, 98.4%, 100.4%,100.4%}

# Introduction

In EE2 Test 3.10, a new luma motion compensation (MC) filter is used for bi-predicted blocks, while the existing MC filter is still used for uni-predicted blocks. The tested filter uses the same number of filter taps and coefficient precision as the existing MC filter for luma, but has sharper filter properties.

However, in the EE software, the filter switch for bi-prediction is not applied in the motion compensation process that is used to generate reference template in LIC, affine TM search, affine merge candidate reordering, et al. In this contribution, it’s proposed to also apply the filter switch in those places.

# Test result

The proposed method was implemented on top of the EE2 Test 3.10a.  The simulation was conducted under common test conditions.   Table 1 shows the test results under CTC.

Table 1 Test result compares to ECM-13.0 anchor

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.06% | -0.02% | 98.8% | 98.2% | 99.9% | 99.9% |
| Class A2 | 0.02% | 0.01% | 0.17% | 99.2% | 99.2% | 100.4% | 100.5% |
| Class B | -0.07% | 0.01% | -0.03% | 98.5% | 98.4% | 99.7% | 99.7% |
| Class C | -0.21% | -0.02% | 0.08% | 99.2% | 98.5% | 99.9% | 99.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.01% | 0.04% | 98.9% | 98.7% | 99.9% | 99.9% |
| Class D | -0.43% | -0.14% | 0.08% | 99.9% | 99.9% | 100.0% | 99.6% |
| Class F | -0.06% | 0.07% | 0.08% | 100.0% | 99.3% | 99.5% | 99.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | -0.25% | -0.21% | 99.9% | 98.4% | 100.0% | 100.0% |
| Class C | -0.14% | 0.25% | -0.57% | 99.1% | 99.1% | 101.2% | 101.3% |
| Class E | 0.07% | 0.57% | 0.05% | 98.4% | 97.4% | 100.2% | 99.9% |
| Overall | -0.06% | 0.12% | -0.27% | 99.3% | 98.4% | 100.4% | 100.4% |
| Class D | 0.00% | -0.10% | -0.26% | 98.3% | 98.0% | 99.3% | 98.4% |
| Class F | -0.07% | -0.23% | -0.22% | 100.0% | 98.5% | 100.0% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution proposes fixes to the EE2 Test 3.10. The simulation results indicate that the proposed fixes improve the bd-rate performance without impacting the runtime. It’s suggested to adopt the proposed method.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).