| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0223-v1 |
| --- | --- |

| Title: | AHG12: Improved implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Charles Bonnineau Saurabh Puri Ismail Marzuki Renan Utida Karam Naser Tangi Poirier Fabrice Le Léannec | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to replace the implicit MTS method of ECM-13.0 by an intra mode and TU size dependent implicit MTS method based on a lookup-table (LUT),

On top of the default MTS implicit method of ECM-13.0, simulation results are reported as follows:

AI {x.xx% Y, x.xx% U, x.xx% V, 100.x% EncT, 100.x% DecT}

# Introduction

Implicit MTS has been introduced in VVC as an alternative tool to explicit MTS. This tool reduces the encoder complexity by inferring the optimal transform type from decoder-side information instead of performing an intensive rate-distortion search in the encoder.

However, implicit MTS remained unchanged in the enhanced compression model (ECM), which introduce new primary transforms to explicit MTS.

It is asserted that the implicit MTS method can benefit from the introduction of the new primary transforms introduced in the ECM.

# Proposed method

In this proposal, the primary transform pair is directly inferred from the intra prediction mode and the TU size of the current block using a lookup-table (LUT).

# Experimental results

The proposed method is implemented on top of the ECM-13.0 [1] and compared with the default implicit MTS method using all intra (AI) configuration.

Table 1: Experimental results of the proposed method over the implicit MTS method of ECM-13.0 (AI)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 Implicit MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.12% | -0.01% | 0.07% | 100% | 102% |
| Class E | -0.13% | 0.04% | -0.05% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.15% | -0.16% | -0.03% | 100% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

It is proposed to replace the current implicit MTS of the ECM-13.0 with the proposed method.

# References

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).