| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0224-v1 |
| --- | --- |

| Title: | EE2-related: Intra Merge Mode extension with BV improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiaye Fu, Zhaoyu Li, Jiaqi Zhang, Chuanmin Jia, Siwei Ma (PKU) | Tel: Email: | jiaye.fu@ucdconnect.ie jqzhang@pku.edu.cn |
| Source: | PKU |  |  |

_____________________________

- Abstract

The block vector-based prediction is highly used for intra blocks in ECM. The Intra merge mode is the method constructing an intra candidate list by the intra modes that are used in intra-coded neighbouring blocks to predict a current coding block. This contribution further proposes using block vectors preidction to replace intra mode for Intra Merge Mode through template cost analysis.

To be more specific, the impact on coding efficiency and runtimes of 1 test over ECM-13.0 is reportedly as follows for {Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}.
The method is implemented on top of EE2-2.5a software

AI: {-0.08%, -0.01%, 0.02%, 101.14%, 98.89%, 100.33%, 100.09%}

Introduction

In JVET-AH0211[1], it is proposed to construct an intra candidate list by the intra modes that are used in intra-coded neighbouring blocks to predict a current coding block.

Block vector-based prediction is currently used in various modes of ECM software. Early versions of ECM only had IBC and IntraTMP as block vector-based predictors for screen content. Later, they were used for camera-captured content, showing significant coding gains compared to conventional directional prediction. In JVET-AG0152 and JVET-AG0146, block vector-based prediction was applied in SGPM and DIMD. In JVET-AH0071 and JVET-AH0274, this method was further applied to TIMD. This approach brings more ideas and potential to intra prediction.

Proposal

This contribution proposes a BV prediction mode replacement for all intra modes derived by Intra Merge Mode.

Specifically, after Intra Merge Mode performs intra merge mode candidate selection, all intra angle modes included in the Intra Merge Mode candidate will undergo template prediction. Simultaneously, all BV prediction modes in the BV List will also undergo the same template prediction. The modes with the least loss will be selected as the prediction modes to replace the current mode included in candidate of Intra Merge Mode. The weights in candidates will be maintained for the final prediction.

Simulation Results

The proposed method is implemented on top of EE2-2.5 [2]. The simulations are run under Common Test Conditions for ECM [3]. The results are shown in the table below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | 0.03% | 0.04% | 101.63% | 97.47% | 100.97% | 100.17% |
| Class A2 | -0.10% | -0.02% | 0.03% | 102.97% | 103.27% | 101.37% | 99.96% |
| Class B | -0.07% | -0.08% | -0.06% | 101.64% | 99.34% | 99.02% | 100.11% |
| Class C | -0.06% | 0.04% | 0.14% | 95.52% | 94.00% | 100.17% | 100.11% |
| Class E | -0.06% | 0.04% | 0.00% | 105.80% | 102.04% | 101.04% | 100.06% |
| Overall | -0.08% | -0.01% | 0.02% | 101.14% | 98.89% | 100.33% | 100.09% |
| Class D | -0.04% | 0.09% | -0.03% | 104.84% | 102.66% | 100.28% | 100.05% |
| Class F | -0.03% | -0.06% | 0.06% | 101.10% | 100.66% | 101.34% | 100.11% |
| Class TGM | -0.01% | -0.02% | -0.04% | 100.84% | 94.72% | 101.43% | 100.23% |

*QP22 of DaylightRoad2 is copied from ECM-13.0

Conclusion

This contribution suggests the extension of OBIC non-adjacent sampling strategy and the inclusion of matrix-based intra prediction in OBIC. The results show that the proposed method can improve the coding efficiency of ECM without complexity increases. It is suggested to further study this method in EE2.

References

[1] Y-J. Cheng, V. Seregin, M. Karczewicz, “Non-EE2: Intra merge mode”, JVET-AH0211, April, 2024.

[2] EE2-2.5, https://vcgit.hhi.fraunhofer.de/ecm/jvet-ah-ee2/ECM/-/tree/TEST_2.5?ref_type=heads

[3] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AF2017, October 2023

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).