| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0249 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0185 (EE2-3.6: Adaptive cost function selection in merge mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Na Zhang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-AI0185 which is about adaptive cost function selection in merge mode. It is confirmed that the available crosscheck results match with those provided in JVET-AI0185. The coding time is not accurate because the cluster is busy.

# Introduction.

In Test 3.6, it is proposed to use the merge index to indicate the cost function that is used to refine the corresponding merge candidate in a merge list.

For bilateral matching merge and affine merge, the two cost functions are set to SATD and SAD. If the merge index is an even number, then SATD is selected, otherwise SAD is selected.

For template matching merge, the two cost functions are set to weighted SAD and SAD. If the merge index is an even number, then weighted SAD is selected to refine the merge candidate, otherwise SAD is selected.

# Experimental Results

JVET-AI0185 [1] was implemented on top of ECM-13.0 [2] and was evaluated following the common test conditions [3]. The crosscheck results are summarized in Table 1.

Table 1: Results of Test 3.6

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.04% | -0.01% | 99.3% | 102.6% | 100.0% | 100.1% |
| Class C | -0.02% | -0.05% | 0.07% | 97.3% | 102.1% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.09% | 0.13% | 0.04% | 101.4% | 101.8% | 99.8% | 100.0% |
| Class F | 0.01% | 0.12% | 0.00% | 98.5% | 102.6% | 99.9% | 100.0% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.08% | -0.11% | -0.21% | 100.8% | 100.0% | 100.0% | 100.0% |
| Class E | -0.06% | 0.04% | 0.12% | 98.0% | 99.5% | 100.4% | 100.0% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.18% | 0.39% | -0.61% | 95.0% | 99.6% | 100.1% | 100.0% |
| Class F |  |  |  |  |  |  |  |

# References

[1] K. Cui, Z. Zhang, Y. Zhang, H. Huang, V. Seregin, M. Karczewicz, “EE2-3.6: Adaptive cost function selection in merge mode,” JVET-AI0185, Jul. 2024.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

[3] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.