| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0252 |
| --- | --- |

| Title: | EE2 related: BV Merge List Improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Ya Chen Milos Radosavljević Fabrice Le Léannec | Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In addition to IntraTMP and IBC, the BV-based prediction is also used in DIMD and SGPM in ECM-13. The EE2-2.2 test further proposes using the BV-based prediction for TIMD. For all these modes, the BV list is generated from all the spatial merge candidates that contain BVs. In contrast to other modes, IntraTMP merge BVs are refined on 5x5 window. This contribution proposes a unified process for generating the BV merge list, including a clustering and refinement process for all the merge BVs.

The following results are obtained by applying the method on top of ECM-13.0:

Proposed Method + EE2-2.2:

AI: -0.07%	-0.08%	-0.08%	EncT xxx.xx%	DecT xxx.xx%

# Introduction

In addition to IBC and IntraTMP modes, the BV-based prediction in ECM is further used in combination with other modes, namely SGPM and DIMD. In EE2-2.2, it is further proposed that the BV-based prediction is also used with TIMD. For all these modes, the merge BV list is derived from the spatial neighboring blocks that contain block vector.

It is observed that the refinement of the merge BV list is currently only performed for IntraTMP. That is, the other modes can be further improved by performing refinement of the merge BVs. However, the further refinement introduces some additional coding complexity. Therefore, a clustering plus refinement method is proposed in this contribution to balance the trade-off between coding gain and complexity.

# Proposed Method:

The contribution proposes a clustering and refinement method for all the BV merge candidates, used for IntraTMP, DIMD, SGPM and TIMD. The method can be described as follows (Figure 1):

- Collecting all BV merge candidates from spatial neighbors.

- Performing ARBVP process to generate additional merge candidates

- Defining a refinement window around each BV from the merge list: This is shown by the dotted red lines in Figure 1

- For all coinciding BVs, the same refinement window is used: as shown in Figure 1. Multiple BVs are coinciding in one area (shown in gray in Figure 1). All these BVs are fused together and a single refinement region is used.

Figure 1: Example of proposed method of the clustering plus refinement of merge BVs

Clearly, the advantage of the clustering process is to reduce the amount of refinement required for each BV, since multiple BVs are merged into a single BV.

# Experimental Results

The method is implemented on top of EE2-2.2 software (EE2-2.2: TIMD with enhanced block vectors deployment). Using the anchor as ECM-13.0 under the common testing conditions, the following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.09% | 0.04% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class A2 | -0.09% | -0.12% | -0.05% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class B | -0.09% | -0.09% | -0.20% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class C | -0.03% | -0.08% | 0.04% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class E | -0.13% | -0.04% | -0.18% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Overall | -0.07% | -0.08% | -0.08% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class D | 0.05% | -0.10% | 0.01% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class F | 0.22% | 0.26% | 0.33% | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

For information, EE2-2.2 performance is:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.00% | -0.02% | 100.4% | 100.8% | #DIV/0! | #DIV/0! |
| Class A2 | -0.07% | 0.03% | -0.05% | 99.6% | 97.3% | #DIV/0! | #DIV/0! |
| Class B | -0.04% | -0.08% | -0.12% | 99.9% | 101.1% | #DIV/0! | #DIV/0! |
| Class C | -0.02% | 0.04% | 0.00% | 100.9% | 104.7% | #DIV/0! | #DIV/0! |
| Class E | -0.06% | 0.04% | -0.11% | 101.1% | 100.8% | #DIV/0! | #DIV/0! |
| Overall | -0.04% | 0.00% | -0.06% | 100.4% | 101.1% | #DIV/0! | #DIV/0! |
| Class D | -0.01% | -0.03% | -0.04% | 100.8% | 103.9% | #DIV/0! | #DIV/0! |
| Class F | 0.00% | 0.19% | 0.10% | 99.1% | 97.9% | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).