| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0256-v3 |
| --- | --- |

| Title: | Subpicture support in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis M. Tourapis, Dimitri Podborski, Seethal Paluri, Jungsun Kim Apple Inc. | Email: | {atourapis, dpodborski, s_paluri, jungsun_kim} @apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

Subpictures were newly introduced in the VVC standard to better enable region of interest and immersive media applications compared to motion constrained tile sets (MCTS). In MCTS, inter prediction constraints were applied to each tile defined for a coded picture so as to remove any dependencies between them. This can have a considerable impact in coding efficiency especially in scenes involving high vertical or horizontal motion. Unlike MCTS, subpicture partitions in VVC are completely independent units that utilize padding around subpicture boundaries to aleviate the MCTS inter prediction limitations. In addition, extraction of subpicture units is said to be simpler than that of MCTS since there is no need to change any bits in the VCL NAL units.

This contribution suggests that subpictures could also be introduced in HEVC by extending its layering functionality. In particular, one could segment a picture into multiple regions, and then code each region as an independent layer using the layering capabilities of HEVC. Metadata, e.g. a new SEI message, could be provided that would describe the relationship of each decoded layer to a final reconstructed image. This concept can also support other multilayer functionalities, such as multiview video and auxiliary data such as depth and alpha.

# Introduction

Recently, immersive applications have necessitated the need for region of interest coding methods in video coding standards. Such techniques can allow services and decoders to only stream, extract, and.or decode information that only correspond to the viewing area of interest in a bitstream and discard all other information. This can reduce the burden to the network as well as to the decoder.

In HEVC [1], motion constrained tile sets (MCTS) were introduced as a mechanism for achieving region of interest (ROI) coding. The existing tiling functionality in HEVC was exploited where a coded picture could be partitioned into rectangular regions, namely tiles, and coded to some extend independently. Unfortunately, tiles in HEVC allow inter prediction to be performed across tiles, which can be an issue for true region of interest decoding applications. For this reason, MCTS imposes a series of constraints during tile encoding, so as to disallow any inter type of prediction to happen across tile boundaries. Loop filtering is also constrained. Such constraints have to be adhered by encoders through certain constraints in their motion estimation and mode decision engines, which can sometimes be challenging for some encoding designs. Regardless, although the desired outcome for independent decoding of each tiles is achieved when such constraints are followed, these constraints can also negatively impact coding efficiency especially in the presence of high motion. Metadata, in the form of supplemental enhancement information (SEI) messages were also provided to distinguish such constrained tiles and to provide the mechanisms for services and decoders to extract, adjust as needed, and reconstruct the regions of interest given a particular viewing experience. It should be also noted that given that MCTS was defined primarily using non-normative SEI messages there was no guarantee that bitstreams would fully adhere to its requirements.

To aleviate such issues, VVC [2] introduce the concept of subpictures. Unlike MCTS, subpictures normatively define regional structures in an image that are fully independent. Motion vectors can point outside a subpicture through padding, considerably improving coding efficiency. The subpicture layout is also defined in the sequence parameter set of a VVC bitstream and the handling process of all subpictures can be said to be much simpler than that of MCTS.

While looking at the characteristics of layered coding in HEVC we realized that the same functionality introduced by VVC subpictures could also be enabled with HEVC layers. Layers in HEVC currently are used to indicate scalability layers, but also multiview and auxiliary layers, such as depth and alpha. However, there is no limitation that would say that layers could not be also used to indicate a region/partition within a larger virtual image. In an example, assuming that we would like to partition a picture into 4 regions (Regions 0-3) and code them independently (Figure 1). Each one of these layers could be assigned to a subpicture and then each subpicture could be assigned to a layer_id and coded using one of HEVC’s multilayer profiles. Metadata could be defined that would allow a decoder to interpret each layer as a subpicture, and allow it to reconstruct the full or partial picture given this information.

Figure 1. Example partitioning of a picture into subpictures and layers.

In a design, ideally subpictures could be of different size, resolution, and even bitdepth, while regions could also partially or even fully overlap. Overlapping could allow seamless blending or even enable ROI SNR quality enhancement if desired. Subpictures could also be assigned with multiple types of data, i.e. a single subpicture may span multiple layer_ids, with each layer carrying different types of data, e.g. left or right views of a stereo signal, depth or alpha information. All these relationships could be easily defined in one or more (if needed) newly introduced SEI messages for HEVC. It is our belief that other than limitations in the number of layers that could be used (i.e. 64 in the case of HEVC given the range of values allowed for the syntax element nuh_layer_id in the NAL unit header syntax) and the lack of an appropriate SEI message and necessary limits, possibly including the definition of a new multi-layer profile for HEVC, HEVC should be fully capable in supporting such concepts and any applications that could benefit from their introduction.

# Proposal

We propose that JVET studies the introduction of a new (or more if deemed needed) SEI message for the support of subpictures in HEVC using multilayered coding. Such subpictures could be used to enable a variety of region of interest applications including immersive media and region of interest enhancement applications using alread available HEVC HW that is capable of multilayered decoding. The group, if agrees in such an investigation, should also possibly study whether new profiles may be needed.

Exemplary syntax of an SEI message that could be used to indicate a set of subpictures and how these can be mapped to layers is provided in Section 3.

# Example SEI message

In this section an exemplary SEI message that allows the definition of subpictures and their association to layers is provided. This SEI message is quite flexible and allows the definition of overlapping subpictures and optional padding (e.g. to create guardbands that could help reduce seaming artifacts or for applications intending to support ROI scalability), flexible partitioning, and different forms of partitioning blending and blending prioritization identification. The SEI message is only provided as an example and the JVET group is welcome, if the application is of interest, to use and refine this example SEI message or define a new one completely from scratch.

| subpicture_info( payloadSize ) { | Descriptor |
| --- | --- |
| spi_use_case | u(6) |
| spi_single_subpicture | u(1) |
| spi_padding_x | ue(v) |
| spi_padding_y | ue(v) |
| if( spi_single_sub_picture == 0 ) { |  |
| // First define the regions |  |
| spi_num_region_columns_minus1 | ue(v) |
| spi_num_region_rows_minus1 | ue(v) |
| spi_uniform_spacing_flag | u(1) |
| if( !spi_uniform_spacing_flag ) { |  |
| for( i = 0; i < spi_num_region_columns_minus1; i++ ) |  |
| spi_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < spi_num_region_rows_minus1; i++ ) |  |
| spi_row_height_minus1[ i ] | ue(v) |
| } |  |
| else { |  |
| spi_nominal_width_minus1 | ue(v) |
| spi_nominal_height_minus1 | ue(v) |
| } |  |
| NumRegionsInPicture = ( spi_num_region_columns_minus1 + 1) * ( spi_num_region_rows_minus1 + 1 ) |  |
| // Group regions into subpictures |  |
| spi_single_region_per_subpicture_flag | u(1) |
| if( !spi_single_region_per_subpicture_flag ) { |  |
| spi_num_subpictures_minus1 | ue(v) |
| for( i = 0; i < spi_num_subpictures_minus1+ 1; i++ ) { |  |
| spi_top_left_region_idx[ i ] | u(v) |
| spi_bottom_right_region_column_offset[ i ] | ue(v) |
| spi_bottom_right_region_row_offset[ i ] | ue(v) |
| } |  |
| } |  |
| else |  |
| spi_num_subpictures_minus1 = NumRegionsInPicture – 1 |  |
| } |  |
| else { |  |
| spi_num_subpictures_minus1 = 0 |  |
| spi_nominal_width_minus1 | ue(v) |
| spi_nominal_height_minus1 | ue(v) |
| } |  |
| // Indicate if a subpicture uses padding |  |
| if (spi_padding_x ¦¦ spi_padding_y) |  |
| for( i = 0; i < spi_num_subpictures_minus1 + 1; i++ ) |  |
| spi_padded_subpicture | u(1) |
| // Label each subpicture |  |
| spi_signalled_subpicture_ids_flag | u(1) |
| if( spi_signalled_subpicture_ids_flag) { |  |
| spi_signalled_subpicture_id_length_minus1 | ue(v) |
| for( i = 0; i < spi_num_subpictures_minus1 + 1; i++ ) { |  |
| spi_subpicture_id[ i ] | u(v) |
| SubpictureIDToIndex[ spi_subpicture_id[ i ] ] = i |  |
| SubpictureIndexToID[ i ] = spi_subpicture_id[ i ] |  |
| } |  |
| } |  |
| else |  |
| for( i = 0; i < spi_num_subpictures_minus1 + 1; i++ ) { |  |
| spi_subpicture_id[ i ] = i |  |
| SubpictureIDToIndex[ i ] = i |  |
| SubpictureIndexToID [ i ] = i |  |
| } |  |
| // Specify the # of layers and assign them to a subpicture id and define their usage |  |
| spi_delta_layer_cnt | ue(v) |
| // number of layers can be same or higher than number of subpictures |  |
| LayerCnt = spi_delta_layer_cnt + spi_num_subpictures_minus1 + 1 |  |
| for( idx = 0; idx < LayerCnt; idx++ ) { |  |
| spi_layer_id[ idx ] | u(6) |
| spi_layer_subpicture_id[ idx ] | u(v) |
| spi_layer_purpose[ idx ] | u(4) |
| spi_overlay_method[ idx ] | u(4) |
| spi_priority_weight[ idx ] | u(8) |
| } |  |
| } |  |

# References

- ISO/IEC 23008-2:2023, “Information technology — High efficiency coding and media delivery in heterogeneous environments - Part 2: High efficiency video coding”

- ISO/IEC 23090-3:2022, “Information technology — Coded representation of immersive media - Part 3: Versatile video coding”

# Patent rights declaration(s)

Apple Inc,. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).