| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0280-v1 |
| --- | --- |

| Title: | No-EE2: Reuse of ALF control information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Nan Hu, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | martak@qti.qualcomm.com hongtaow@qti.qualcomm.com nanh@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In ECM-13.0, for each picture, adaptive loop filter (ALF) control information is signalled at both picture and CTB levels. In this contribution, reuse of ALF control information is proposed. For each picture, when ALF is enabled, a flag is signalled to indicate whether ALF control information reuses the information from a previously encoded picture. On top of ECM-13.0 with common test conditions, simulation results of the proposed method are reported below:

RA: %, %, %, % EncT, % DecT.

# Introduction

In ECM-13.0, a picture may use adaptive loop filter (ALF) derived from the current or previously encoded pictures. Filter usage information and on/off flags are signalled at both picture and coding tree block level. Proposed method

# Proposed method

In this proposal, ALF control information, including on/off decision and ALF APS usage information, from a previously encoded picture is reused. A flag is signalled if ALF is enabled for a picture and ALF control information is reused and not signalled.

At encoder, ALF filter is derived for a group of pictures which share the ALF on/off decision. In the current implementation, the proposed method is only applied to non-reference pictures.

# Simulation results

The proposed method is implemented on top of ECM-13.0. The simulations were performed under common test conditions defined in JVET-AF2017. The simulation results are reported in the table below.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.93% | -0.86% | 100.2% | 99.5% | 104.4% | 107.4% |
| Class C | -0.16% | -0.52% | -0.75% | 99.8% | 100.3% | 103.0% | 104.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.40% | -1.84% | -1.65% | 99.8% | 100.0% | 102.6% | 102.5% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).