| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12-19 July 2024 | Document: JVET-AI0281-v1 |
| --- | --- |

| Title: | AHG12: Context retraining for ECM-13.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. Mahdavinia, P. Nikitin, V. Seregin, M. Karczewicz | Tel: Email: pnikitin@qti.qualcomm.com |  |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results for all CABAC contexts retraining for ECM-13.0. Using ECM-13.0 as the anchor, test results are summarized as follows.

AI:  -0.01%, -0.001% and -0.08% BD-rate gains for Y, U and V, respectively

RA:  -0.05%, 0.08% and -0.09% BD-rate gains for Y, U and V, respectively

LB:  -0.00%, -0.60% and -0.12% BD-rate gains for Y, U and V, respectively

LP:  -0.03%, 0.31% and -0.05% BD-rate gains for Y, U and V, respectively

# Experiment results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.04% | -0.09% | 101.7% | 101.7% | 100.0% | 100.1% |
| Class A2 | 0.01% | 0.01% | -0.07% | 101.6% | 100.7% | 100.3% | 100.0% |
| Class B | 0.01% | -0.04% | -0.17% | 104.0% | 103.7% | 97.2% | 100.0% |
| Class C | -0.02% | 0.12% | 0.05% | 101.9% | 103.7% | 100.0% | 100.0% |
| Class E | -0.02% | 0.03% | -0.10% | 101.4% | 102.9% | 100.0% | 100.0% |
| Overall | -0.01% | 0.01% | -0.08% | 102.3% | 102.7% | 99.3% | 100.0% |
| Class D | -0.01% | -0.08% | -0.09% | 100.4% | 99.6% | 100.0% | 100.0% |
| Class F | -0.08% | 0.16% | -0.06% | 106.8% | 107.3% | 100.0% | 100.0% |
| Class TGM | 0.01% | -0.06% | -0.06% | 109.6% | 118.3% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.08% | -0.15% | 100.8% | 100.0% | 100.0% | 100.0% |
| Class A2 | -0.03% | -0.01% | 0.00% | 100.0% | 98.5% | 100.0% | 100.1% |
| Class B | -0.04% | 0.12% | 0.13% | 102.7% | 105.1% | 100.0% | 99.9% |
| Class C | -0.08% | 0.20% | 0.28% | 100.4% | 100.7% | 100.8% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | 0.08% | 0.09% | 101.2% | 101.6% | 100.2% | 100.0% |
| Class D | -0.05% | -0.27% | 0.07% | 101.5% | 100.0% | 99.8% | 99.9% |
| Class F | -0.06% | -0.01% | -0.09% | 101.8% | 104.6% | 100.0% | 100.1% |
| Class TGM | -0.22% | -0.07% | -0.03% | 101.5% | 104.5% | 101.0% | 100.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.45% | -0.03% | 103.4% | 101.7% | 100.0% | 99.8% |
| Class C | -0.13% | -0.20% | -0.16% | 100.1% | 101.8% | 100.1% | 100.0% |
| Class E | 0.27% | -1.39% | -0.19% | 103.5% | 103.5% | 97.1% | 100.0% |
| Overall | 0.00% | -0.60% | -0.12% | 102.3% | 102.2% | 99.3% | 99.9% |
| Class D | -0.24% | -0.70% | 0.21% | 103.1% | 103.9% | 100.0% | 100.0% |
| Class F | -0.12% | 0.41% | -0.68% | 105.4% | 111.7% | 101.2% | 100.1% |
| Class TGM | -0.12% | 0.07% | 0.05% | 105.4% | 104.4% | 101.9% | 98.3% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.04% | 0.21% | 103.4% | 104.9% | 100.0% | 99.8% |
| Class C | -0.10% | 0.31% | 0.33% | 101.8% | 100.7% | 100.4% | 100.0% |
| Class E | 0.04% | 0.88% | -0.98% | 101.6% | 103.7% | 101.1% | 100.0% |
| Overall | -0.03% | 0.31% | -0.05% | 102.4% | 103.2% | 100.4% | 99.9% |
| Class D | -0.05% | 0.32% | 0.31% | 99.4% | 101.5% | 100.2% | 100.0% |
| Class F | -0.38% | -0.43% | 0.06% | 106.0% | 110.2% | 100.0% | 100.0% |
| Class TGM | -0.14% | -0.13% | -0.12% | 106.1% | 99.6% | 101.5% | 100.1% |

# Cited References

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).