| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0296-v2 |
| --- | --- |

| Title: | AHG9: Questions regarding NNPFC SEI message with temporal extrapolation purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Didier Doyen, Philippe de Lagrange, Fabrice Urban | Tel: Email: | 1stname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VSEI v4, a new purpose has been added to the NNPFC SEI message, related to temporal extrapolation. This feature allows the NNPF to generate one or more extrapolated pictures that follow all input pictures of the NNPF in output order. However, with current draft syntax, it is not possible to generate more than one temporally extrapolated picture per time instance, which may be an important feature for picture extrapolation applications where multiple hypotheses may be generated for the same temporal position.

This contribution suggests considering extending the temporal extrapolation purpose of the NNPFC SEI message to support generating multiple temporally extrapolated picture per time instance.

# Introduction

In VSEI v4, a new purpose has been added to the NNPFC SEI message, related to temporal extrapolation. This feature allows the NNPF to generate one or more extrapolated pictures that follow all input pictures of the NNPF in output order. Example use cases are very low delay computer vision applications (e.g. robotics, autonomous driving) to facilitate anticipatory decision making, or very low-latency applications (e.g. cloud gaming) to anticipate the rendering by emulating several hypotheses of actions.

When this new purpose was proposed [JVET-AG0089], it was commented that it may be relevant to consider generating multiple temporally extrapolated picture instances for a same time instance (or POC) [JVET-AG_notes_dF.docx], but that this option was not supported in current specification:

“It was pointed out that in picture extrapolation applications (navigation, cloud gaming) it is also common that multiple hypotheses are generated for the same temporal position. This would not be supported by the proposed syntax, but could be added and would make another differentiation from the current picture rate upsampling.”

In clause 8.28.1.1 of VSEI v3 or v4 or VSEI TuC text, the following sentence indicates that only one picture per time instance can be output from the NNPF:

### 8.28.1.1 General

Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of NNPF output pictures ListNnpfOutputPics.

First, BitstreamToFilter is decoded, and the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter.

Second, the filtering process for one picture, as specified in subclause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which one or more NNPFs are activated.

The order of the pictures in ListNnpfOutputPics is in output order.

Within ListNnpfOutputPics there shall be no more than one picture pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.

…

This contribution raises this question and suggests finding a solution to address this problem.

# Open questions

Because the possibility of generating several instances of a same temporally extrapolated picture does not seem to be supported by VSEI v3, v4 or VSEI TuC documents, it is questioned whether one (or some) of the following solutions should be considered.

Option 1 – Allowing using multiple NNPFs to generate multiple output pictures at same time instance when purpose is temporal extrapolation

In a first option, multiple NNPFs with temporal extrapolation purpose may be activated and applied to a same decoded picture to generate multiple output temporally extrapolated pictures per time instance.

The following text change in clause 8.28.1.1 of VSEI v3 or v4 or VSEI TuC text is suggested:

### 8.28.1.1 General

Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of NNPF output pictures ListNnpfOutputPics.

First, BitstreamToFilter is decoded, and the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter.

Second, the filtering process for one picture, as specified in subclause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which one or more NNPFs are activated.

The order of the pictures in ListNnpfOutputPics is in output order.

Within ListNnpfOutputPics there shall be no more than one picture resulting from the application of one NNPF or of cascaded NNPFs with a purpose different from temporal extrapolation, pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.

Within ListNnpfOutputPics there may be more than one picture pertaining to any particular output time instance, when several NNPFs with temporal extrapolation purpose are activated and applied to a same cropped decoded picture.

…

Option 1a – Using multiple NNPFC SEIs with the same nnpfc_id, each one allowing generating a different output picture at same time instance

Signaling of more than one NNPFC SEIs with the same nnpfc_id, where different NNPFC SEIs may generate different instances of a same temporally extrapolated output picture, and

Signaling of a new syntax element when nnpfc_purpose is temporal extrapolation, to indicate the id of the temporally extrapolated output picture instance. A syntax element may also be added to indicate the maximum number of instances. This syntax element must be the same for all NNPFC SEIs with the same nnpfc_id.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_extrapolated_pics_max_id | ue(v) |
| nnpfc_extrapolated_pics_id | ue(v) |
| } |  |
| … |  |

One NNPFC SEI may be used as base model for the other NNPFC SEIs potentially used for temporal extrapolation.

One illustration of this option is depicted in Figure 1.

Figure 1. illustration of option 1. Several NNPFC SEIs with same Id are signaled and potentially used to generate several instances of temporally extrapolated picture(s) at same time instance.

Option 1b – Using multiple NNPFC SEIs with different nnpfc_id, each one allowing generating a different output picture at same time instance

Signaling of more than one (N) NNPFC SEIs with different nnpfc_id, and

Activating of the N NNPFC SEIs by N NNPFA SEIs, each one referring to one of the N NNPFC SEIs, and enabling the non-cascaded application of N temporal extrapolation NNPF processes to the same decoded pictures to generate multiple instances of each temporally extrapolated picture, using the N NNPFs.

One illustration of this option is depicted in Figure 2. The main difference with Figure 1 is that Ids of NNPFC0 and NNPFC1 are different, as well as reference Ids of NNPFA0 and NNPFA1.

Figure 2. illustration of option 2. Several NNPFC SEIs with different Ids are signaled and potentially used to generate several instances of temporally extrapolated picture(s) at same time instance.

Option 1c – Signaling of different NNPF models in a same NNPFC SEI

Signaling of different NN models in a same NNPFC SEI, each model targeting one instance of each extrapolated picture. In the example syntax table below, the maximum instance id is indicated (parameter nnpfc_extrapolated_pics_max_id gives the max id number, and therefore the number of instances is deduced as nnpfc_extrapolated_pics_max_id+1).

Corresponding proposed syntax changes are depicted in table below.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| … |  |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_extrapolated_pics_max_id | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| for( i = 1; i <= nnpfc_extrapolated_pics_max_id; i++ ) { |  |
| nnpfc_tag_uri_instance[ i ] | st(v) |
| nnpfc_uri_instance[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| if( TemporalExtrapolationFlag ) { |  |
| for( j = 1; j <= nnpfc_extrapolated_pics_max_id; j++ ) { |  |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ j ][ i ] | b(8) |
| } |  |
| } |  |
| } |  |
| } |  |

When the NNPF is activated, the NNPFs signalled in the NNPFC SEI are applied to any input decoded picture to generate different output versions at same time instance.

Option 2 – Allowing generating multiple output at same time instance from a same NNPF

Another solution is to enable the generation from a same NNPF of different versions of temporally extrapolated picture(s) at same temporal position, by using different inputs to the NNPF. The same NNPF is run several times with different input and generates different output at same temporal position.

For example, in addition to the decoded pictures used as input, additional parameters may be used as input of the NNPF allowing to generate different output pictures from a same NNPF using same input decoded pictures. Additional parameters may be signaled using a new type of auxiliary input data, such as text prompt information indicative of the multiple hypotheses considered for generating different versions of temporally extrapolated pictures, as proposed in JVET-AI0061 for the spatial extrapolation case.

The following text change in clause 8.28.1.1 of VSEI v3 or v4 or VSEI TuC text is suggested:

### 8.28.1.1 General

Input to this process is a bitstream BitstreamToFilter. Output of this process is a list of NNPF output pictures ListNnpfOutputPics.

First, BitstreamToFilter is decoded, and the list CroppedDecodedPictures is set to be the list of the cropped decoded pictures in output order resulted from decoding BitstreamToFilter.

Second, the filtering process for one picture, as specified in subclause 8.28.1.2, is repeatedly invoked, in output order, for each cropped decoded picture that is in CroppedDecodedPictures and for which one or more NNPFs are activated.

The order of the pictures in ListNnpfOutputPics is in output order.

Within ListNnpfOutputPics there shall be no more than one picture resulting from the application of one NNPF or cascaded NNPFs with a purpose different from temporal extrapolation, pertaining to any particular output time instance. When for any particular picture in CroppedDecodedPictures there are multiple NNPFs activated and only one of the NNPFs is allowed to be chosen to be applied although any of the NNPFs may be chosen, the above constraint shall apply regardless of which NNPF is chosen to be applied to the particular picture.

Within ListNnpfOutputPics there may be more than one picture pertaining to any particular output time instance, when one NNPF with temporal extrapolation purpose is activated and applied to a cropped decoded picture multiple times.

…

# Conclusion

This contribution suggests ensuring that the NNPFC / NNPFA SEI messages specification enable generating multiple instances of temporally interpolated picture for a same time instance, as this is a relevant feature for practical use cases requiring low latency.

Some possible options are proposed.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).