| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0302 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0103 (EE2-3.5: Additional chained motion vector prediction candidates) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-AI0103 that proposes an additional chained motion vector prediction candidates in the context of Exploration Experiments 2 (JVET-AF2024). The cross-checker confirms that the software provided by the proponents matches the algorithm description of JVET-AI0103. All the testing results in this cross-check report match those provided by the proponent.

Summary

JVET-AI0103 [1] proposes additional chained motion vector prediction (CMVP) candidates and reports two test results, i.e., EE2-3.5a and EE2-3.5b.

In EE2-3.5a, additional CMVP candidates are inserted after the derivation of pairwise candidates in the pairwise merge candidate list. The maximum size of the pairwise merge candidate list is extended by 6 and 2 for low-delay and non-low-delay configurations. In contrast, the maximum number of pairwise candidates is kept unchanged.

In EE2-3.5b, additional CMVP candidates are inserted before zero padding candidates in the initial merge candidate list. The maximum size of the initial merge candidate list is extended by 4 for low-delay and non-low-delay configurations. In contrast the maximum number of the candidates before the original CMVP candidates, is kept unchanged.

The additional CMVP candidates for 3.5a and 3.5b are derived by checking only the center position of the current block to find traced MVs or BVs, which are different from the derivation of original CMVP candidates.

The cross-checker carefully analyzed the software provided by the proponents [2]. Then, we confirmed that the software implementation perfectly matches the algorithm description of JVET-AI0103.

The cross-checker conducted simulations using the provided software according to the EE2 description [3] and the common test condition for enhanced compression tool testing [4]. The cross-check results are provided in the accompanying Excel sheets. The cross-checker could exactly reproduce the results provided by the proponent and reported in JVET-AI0103. The cross-checker compiled the provided software on a 64-bit Ubuntu environment. The experiments were performed on the cluster PCs.

Conclusion

The cross-checker has verified the description and results reported in JVET-AI0103. It is confirmed that the software provided by the proponent matches the proposal description, and the testing results produced by the cross-checker match the reported results.

Reference

- N. Zhang, K. Zhang, and L. Zhang, “EE2-3.5: Additional chained motion vector prediction candidates,” JVET-AI0103, July 2024.

- https://vcgit.hhi.fraunhofer.de/ecm/jvet-ah-ee2/ECM/-/tree/TEST_3.5

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AH2024, April 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.