| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0317-v0 |
| --- | --- |

| Title: | Preliminary improvements to configurations for LTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Lorenzo Ciccarelli, Manzoor Razaak, Yetish Joshi, Guendalina Cobianchi, Simone Ferrara (V-Nova) 20 Eastbourne Terrace, London, W2 6LG | Tel: Email: | [phone contact number] lorenzo.ciccarelli@v-nova.com, simone.ferrara@v-nova.com |
| Source: | V-Nova |  |  |

_____________________________

# Abstract

The present document provides a report on the progress made by the authors in improving the LCEVC Test Model (LTM).

In previous contributions ([1], [2]) some results on LCEVC performances were presented covering an extended bitrate range. The authors of the present document responded ([3], [4]) explaining that the configurations used in ([1], [2]) were not appropriate for the tested ranges and that therefore any conclusions reached in ([1], [2]) should be considered inconclusive.

At the 14th working group meeting of WG 4 (MPEG 145 – Jan 2024, Online) it was agreed to include the improvement of LTM into the mandates of the AhG on LCEVC [1]. As such, software optimizations by members were encouraged.

At the 15th working group meeting of WG4 (MPEG 146 – April 2024, Rennes (FR)) a workplan for improvement of the LTM was agreed [2]. The intended timeline is reported in Annex A.

This document provides a report of the work carried out by the authors aimed at improving the configuration of the LCEVC Test Model (LTM) since the MPEG 146 meeting in April.

In particular, the authors present results in this paper using a preliminary set of configurations which demonstrate significant improvement in performances, particularly in the higher bitrate range.

## Summary of the work carried out to date

Following the initial investigation described in [3], additional studies on the LCEVC parameters were conducted.

The following test models were used for the tests:

- HEVC: test model HM-16.26 with the related configuration encoder_randomaccess_main10.cfg.

- VVC: test model VTM-19.1 with the related configuration vtm_encoder_randomaccess.cfg.

- LCEVC: test model LTM 5.5 with tested configurations and configurations used in the verification tests.

The sequences used are described in Table 1. The base layers and anchors generated during the initial phase described in [3] were used. In particular, for both base codecs, base layers at 1920x1080 and 960x1080 resolutions were generated along with the anchors in UHD. For the UHD anchors the QPs used are reported in Table 2 for HEVC and in Table 3 for VVC. Base layer 1920x1080 and 960x1080 were generated by using the Lanczos downsampler included in the LTM. Each base sequence was then encoded using HM and VTM with a QP range spanning from 18 and 38 with a step of 2 (11 QPs for each base sequence for each base codec).

Table 1 - SDR sequences

| Index | Class | Sequence name | Resolution | Frame count | Frame rate | Bit depth |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | A1 | Tango2 | 3840×2160p | 294 | 60 | 10 |
| 2 | A1 | FoodMarket4 | 3840×2160p | 300 | 60 | 10 |
| 3 | A1 | Campfire | 3840×2160p | 300 | 30 | 10 |
| 4 | A | Boxe_Logo | 3840×2160p | 596 | 60 | 10 |
| 5 | A2 | DaylightRoad2 | 3840×2160p | 300 | 60 | 10 |
| 6 | A2 | ParkRunning3 | 3840×2160p | 300 | 50 | 10 |
| 7 | A | DrivingPoV_Logo | 3840×2160p | 600 | 60 | 10 |
| 8 | A2 | CatRobot | 3840×2160p | 300 | 60 | 10 |
| 9 | A | LupoPuppet | 3840×2160p | 500 | 50 | 10 |
| 10 | A | Fortnite (Part 1) | 3840×2160p | 600 | 60 | 10 |
| 11 | A | Marathon2 | 3840x2160p | 300 | 30 | 10 |
| 12 | A | MountainBay2 | 3840x2160p | 300 | 30 | 10 |
| 13 | A | NeptuneFountain3 | 3840×2160p | 600 | 60 | 10 |
| 14 | A | TallBuildings2 | 3840×2160p | 300 | 30 | 10 |

Table 2 - QPs used for the HEVC UHD anchors

| Sequence name | QP 1 | QP 2 | QP 3 | QP 4 | QP 5 |
| --- | --- | --- | --- | --- | --- |
| Tango2 | 42 | 37 | 32 | 27 | 22 |
| FoodMarket4 | 42 | 37 | 32 | 27 | 22 |
| Campfire | 42 | 37 | 32 | 27 | 22 |
| CatRobot | 42 | 37 | 32 | 27 | 22 |
| LupoPuppet | 42 | 38 | 34 | 30 | 25 |
| DaylightRoad2 | 42 | 37 | 32 | 27 | 22 |
| ParkRunning3 | 42 | 37 | 32 | 27 | 22 |
| Fortnite (Part 1) | 40 | 37 | 34 | 32 | 27 |
| Boxe_Logo | 42 | 38 | 34 | 30 | 25 |
| DrivingPoV_Logo | 42 | 38 | 32 | 28 | 23 |
| Marathon2 | 42 | 38 | 34 | 30 | 25 |
| MountainBay2 | 46 | 42 | 38 | 34 | 29 |
| NeptuneFountain3 | 39 | 37 | 35 | 32 | 30 |
| TallBuildings2 | 43 | 39 | 35 | 31 | 28 |

Table 3 - QPs used for the VVC UHD anchors

| Sequence name | QP 1 | QP 2 | QP 3 | QP 4 | QP 5 |
| --- | --- | --- | --- | --- | --- |
| Tango2 | 42 | 37 | 32 | 27 | 22 |
| FoodMarket4 | 42 | 37 | 32 | 27 | 22 |
| Campfire | 42 | 37 | 32 | 27 | 22 |
| CatRobot | 42 | 37 | 32 | 27 | 22 |
| LupoPuppet | 42 | 38 | 34 | 30 | 25 |
| DaylightRoad2 | 42 | 37 | 32 | 27 | 22 |
| ParkRunning3 | 42 | 37 | 32 | 27 | 22 |
| Fortnite (Part 1) | 40 | 37 | 34 | 32 | 27 |
| Boxe_Logo | 42 | 38 | 34 | 30 | 25 |
| DrivingPoV_Logo | 42 | 38 | 32 | 28 | 23 |
| Marathon2 | 42 | 38 | 34 | 30 | 25 |
| MountainBay2 | 46 | 42 | 38 | 34 | 29 |
| NeptuneFountain3 | 42 | 39 | 37 | 35 | 33 |
| TallBuildings2 | 46 | 42 | 38 | 34 | 30 |

In this study, only the following parameters of the LTM configurations have been considered:

- the type of transform (transform_type) in the range [0, 1].

- step width for sublayer 2 (step_width_sublayer2) in the range [300, 6000].

- the scaling mode (scaling_mode_level2) in the range of [1, 2]. For info, scaling_mode_level2 equal to 2 enables the down/upscaling process by a factor 2 across  both vertical and horizontal dimensions (quarter resolution), while scaling_mode_level2 equal to 1 enables the down/upscaling by a factor 2 only across the horizontal dimension.

Other configuration parameters were not tested and may be subject to future studies.

- For each sequence and for each combination of transform_type and scaling_mode_level2, the LTM was executed for each base QP on all the range of the step with for sublayer2.

Subsequently, for each sequence the overall convex hull has been calculated to generate a final convex hull. The points from the final convex hull have been selected in a way that the resulting PSNR over bitrate curve was within the same range of the curve of the related anchor.

The points selected are summarized in the Excel file attached to this contribution.

## Results

In Table 4 we report the BD-rate in terms of PSNR between two different configurations of the same software model, namely LTM 5.5 enhancing HM 16.26. In Table 5 we report the BD-rate in terms of PSNR between two different configurations of the same software model, namely LTM 5.5 enhancing VTM 19.1.

The first configuration is based on the one used in [4] (which we call “VT configuration”), whereas the second one is a new configuration based on this study and proposed in this contribution (“New configuration”).

We also report the combined PSNR_YCbCr  which is obtained as follows:

PSNR_YCbCr = 0.75*PSNR_Y + 0.125*PSNR_Cb + 0.125*PSNR_Cr

Table 4 – Preliminary results for LCEVC enhancing HEVC (LCEVC+HEVC)

| Index | Class | Sequence name | BD-Rate PSNR (LTM 5.5 + HM 16.26) New configuration over VT configuration |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | YCrCb |
| 1 | A1 | Tango2 | -0.08% | 0.24% | 0.22% | -0.003% |
| 2 | A1 | FoodMarket4 | 0.47% | 0.59% | 0.69% | 0.513% |
| 3 | A1 | Campfire | 0.15% | -1.23% | -0.10% | -0.054% |
| 4 | A | Boxe_Logo | -5.60% | -20.10% | -5.55% | -7.406% |
| 5 | A2 | DaylightRoad2 | -13.03% | -0.47% | -5.23% | -10.485% |
| 6 | A2 | ParkRunning3 | -2.45% | 0.34% | -0.71% | -1.884% |
| 7 | A | DrivingPoV_Logo | -15.95% | -11.21% | -10.01% | -14.615% |
| 8 | A2 | CatRobot1 | -2.04% | -0.15% | -1.72% | -1.764% |
| 9 | A | TallBuildings | -26.77% | -0.05% | -11.34% | -21.501% |
| 10 | A | LupoPuppet | -0.16% | 0.23% | 0.49% | -0.030% |
| 11 | A | NeptuneFountain | -1.56% | -26.46% | -13.40% | -6.153% |
| 12 | A | FortniteP1 | -7.96% | -25.23% | -18.19% | -11.398% |
| 13 | A | Marathon | -0.57% | -0.16% | -0.54% | -0.515% |
| 14 | A | MountainBay2 | -3.49% | -5.57% | -7.19% | -4.213% |
|  |  | Average | -5.65% | -6.37% | -5.18% | -5.679% |

Table 5 – Preliminary results for LCEVC enhancing VVC (LCEVC+VVC)

| Index | Class | Sequence name | BD-Rate PSNR (LTM 5.5 + VTM 19.1) New configuration over VT configuration |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | YcrCb |
| 1 | A1 | Tango2 | -33.42% | -19.60% | -19.81% | -29.99% |
| 2 | A1 | FoodMarket4 | -34.30% | -18.08% | -18.08% | -30.25% |
| 3 | A1 | Campfire | -39.13% | -27.15% | -27.39% | -36.17% |
| 4 | A | Boxe_Logo | -37.84% | -41.69% | -21.46% | -36.27% |
| 5 | A2 | DaylightRoad2 | -54.13% | -27.38% | -35.83% | -48.50% |
| 6 | A2 | ParkRunning3 | -33.60% | -32.03% | -26.92% | -32.57% |
| 7 | A | DrivingPoV_Logo | -43.88% | -32.61% | -43.25% | -42.39% |
| 8 | A2 | CatRobot1 | -42.65% | -26.57% | -28.72% | -38.90% |
| 9 | A | TallBuildings | -52.81% | -14.51% | -11.97% | -42.92% |
| 10 | A | LupoPuppet | -27.31% | -9.36% | -8.62% | -22.73% |
| 11 | A | NeptuneFountain | -24.53% | -42.42% | -24.10% | -26.71% |
| 12 | A | FortniteP1 | -50.19% | -42.43% | -34.85% | -47.30% |
| 13 | A | Marathon | -29.50% | -18.80% | -18.71% | -26.81% |
| 14 | A | MountainBay2 | -31.89% | -6.16% | -1.98% | -24.94% |
|  |  | Average | -38.23% | -25.63% | -22.98% | -34.75% |

Table 6 reports the BD-Rate PSNR-Y of LTM enhancing HM over HM for two different LTM configurations (New and VT). Table 7 reports the BD-Rate PSNR-Y of LTM enhancing VTM over VTM for two different LTM configurations (New and VT). The bitrate range has been extended to cover bitrates above the ones used in the Verification Tests [4].

Table 6 – LTM+HM over HM – BD-Rate PSNR-Y

| Index | Class | Sequence name | BD-Rate PSNR-Y |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LTM+HM (New over VT) | LTM+HM (VT) over HM | LTM+HM (New) over HM |
| 1 | A1 | Tango2 | -0.08% | -10.99% | -10.89% |
| 2 | A1 | FoodMarket4 | 0.47% | -12.43% | -10.37% |
| 3 | A1 | Campfire | 0.15% | -22.56% | -20.28% |
| 4 | A | Boxe_Logo | -5.60% | 29.74% | 26.99% |
| 5 | A2 | DaylightRoad2 | -13.03% | 16.22% | 7.86% |
| 6 | A2 | ParkRunning3 | -2.45% | -19.57% | -20.48% |
| 7 | A | DrivingPoV_Logo | -15.95% | 40.95% | 25.76% |
| 8 | A2 | CatRobot1 | -2.04% | 3.89% | 6.58% |
| 9 | A | TallBuildings | -26.77% | 49.06% | 35.08% |
| 10 | A | LupoPuppet | -0.16% | -14.13% | -14.32% |
| 11 | A | NeptuneFountain | -1.56% | 28.19% | 25.12% |
| 12 | A | FortniteP1 | -7.96% | 14.43% | 10.08% |
| 13 | A | Marathon | -0.57% | -2.67% | -0.06% |
| 14 | A | MountainBay2 | -3.49% | -5.13% | -6.99% |
|  |  | Average | -5.65% | 6.78% | 3.86% |

Table 7 – LTM+VTM over VTM – BD-Rate PSNR-Y

| Index | Class | Sequence name | BD-Rate PSNR-Y |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LTM+VTM (New over VT) | LTM+VTM (VT) over VTM | LTM+VTM (New) over VTM |
| 1 | A1 | Tango2 | -33.42% | 52.70% | 3.42% |
| 2 | A1 | FoodMarket4 | -34.30% | 55.15% | 4.18% |
| 3 | A1 | Campfire | -39.13% | 52.69% | -3.76% |
| 4 | A | Boxe_Logo | -37.84% | 143.22% | 52.73% |
| 5 | A2 | DaylightRoad2 | -54.13% | 126.15% | 27.37% |
| 6 | A2 | ParkRunning3 | -33.60% | 41.18% | -1.38% |
| 7 | A | DrivingPoV_Logo | -43.88% | 146.71% | 48.32% |
| 8 | A2 | CatRobot1 | -42.65% | 104.95% | 25.71% |
| 9 | A | TallBuildings | -52.81% | 172.65% | 51.64% |
| 10 | A | LupoPuppet | -27.31% | 32.29% | -3.76% |
| 11 | A | NeptuneFountain | -24.53% | 82.96% | 36.23% |
| 12 | A | FortniteP1 | -50.19% | 132.91% | 26.86% |
| 13 | A | Marathon | -29.50% | 52.23% | 10.45% |
| 14 | A | MountainBay2 | -31.89% | 47.81% | 3.54% |
|  |  | Average | -38.23% | 88.83% | 20.11% |

Table 8 reports the BD-Rate PSNR (divided in different components) along with VMAF and VMAF-NEG of LTM enhancing HM over HM using the New configuration. Table 9 reports the BD-Rate PSNR (divided in different components) along with VMAF and VMAF-NEG of LTM enhancing VTM over VTM ing the new configuration.

Table 8 - BD-Rate LTM+HM (New) over HM

| Index | Class | Sequence name |  | BD-Rate LTM+HM (New) over HM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PSNR-Y | PSNR-U | PSNR-V | YCbCr | VMAF | VMAF-NEG |
| 1 | A1 | Tango2 | -10.89% | 5.72% | 8.47% | -6.22% | -19.90% | -16.21% |
| 2 | A1 | FoodMarket4 | -10.37% | 7.70% | 6.28% | -6.85% | -17.34% | -15.88% |
| 3 | A1 | Campfire | -20.28% | 63.24% | 4.93% | -9.46% | -27.13% | -23.53% |
| 4 | A | Boxe_Logo | 26.99% | 83.13% | 15.70% | 30.91% | 8.18% | 21.13% |
| 5 | A2 | DaylightRoad2 | 7.86% | 10.80% | 29.57% | 10.20% | -2.79% | 0.18% |
| 6 | A2 | ParkRunning3 | -20.48% | 221.05% | 146.55% | -7.10% | -35.21% | -32.82% |
| 7 | A | DrivingPoV_Logo | 25.76% | 38.68% | 38.82% | 27.45% | 0.49% | 11.02% |
| 8 | A2 | CatRobot1 | 6.58% | 30.82% | 42.85% | 11.88% | -12.25% | -6.56% |
| 9 | A | TallBuildings | 35.08% | 14.51% | 32.04% | 34.14% | 1.60% | 10.54% |
| 10 | A | LupoPuppet | -14.32% | 9.70% | 7.33% | -8.87% | -16.48% | -15.18% |
| 11 | A | NeptuneFountain | 25.12% | 178.02% | 16.23% | 29.12% | -0.15% | 19.46% |
| 12 | A | FortniteP1 | 10.08% | 9.43% | 2.82% | 9.51% | -5.46% | -0.99% |
| 13 | A | Marathon | -0.06% | 43.41% | 44.90% | 9.57% | -14.21% | -11.46% |
| 14 | A | MountainBay2 | -6.99% | 10.61% | 2.48% | -4.45% | -12.87% | -11.68% |
|  |  | Average | 3.86% | 51.92% | 28.50% | 8.56% | -10.97% | -5.14% |

Table 9 - BD-Rate LTM+VTM (New) over VTM

| Index | Class | Sequence name |  | BD-Rate LTM+VTM (New) over VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PSNR-Y | PSNR-U | PSNR-V | YCbCr | VMAF | VMAF-NEG |
| 1 | A1 | Tango2 | 3.42% | 1.19% | 10.29% | 3.71% | -12.31% | -4.15% |
| 2 | A1 | FoodMarket4 | 4.18% | 4.59% | 5.86% | 4.34% | -8.32% | -0.93% |
| 3 | A1 | Campfire | -3.76% | 63.99% | 19.17% | 6.49% | -16.50% | -10.03% |
| 4 | A | Boxe_Logo | 52.73% | 124.37% | 28.67% | 56.71% | 24.03% | 44.57% |
| 5 | A2 | DaylightRoad2 | 27.37% | 9.02% | 30.43% | 25.27% | 1.71% | 11.44% |
| 6 | A2 | ParkRunning3 | -1.38% | 122.07% | 58.19% | 7.51% | -14.22% | -11.64% |
| 7 | A | DrivingPoV_Logo | 48.32% | 41.08% | 73.19% | 48.87% | 6.41% | 28.50% |
| 8 | A2 | CatRobot1 | 25.71% | 38.71% | 53.88% | 29.93% | 0.30% | 11.94% |
| 9 | A | TallBuildings | 51.64% | 5.80% | -10.22% | 35.49% | 3.40% | 16.95% |
| 10 | A | LupoPuppet | -3.76% | -0.35% | 2.52% | -2.55% | -13.88% | -11.01% |
| 11 | A | NeptuneFountain | 36.23% | 95.22% | -0.13% | 37.56% | -3.65% | 30.33% |
| 12 | A | FortniteP1 | 26.86% | 24.50% | 14.06% | 25.54% | -0.53% | 7.07% |
| 13 | A | Marathon | 10.45% | 38.74% | 42.23% | 17.90% | -5.74% | -1.30% |
| 14 | A | MountainBay2 | 3.54% | -6.47% | -12.48% | 0.33% | -1.77% | 1.58% |
|  |  | Average | 20.11% | 40.18% | 22.55% | 21.22% | -2.93% | 8.09% |

In the Annex A and B some plot showing the comparison between the 2 configurations are reported.

In order to provide an indication of the percentage of enhancement, Table 10 and Table 11 report respectively the distribution of the data point for LTM+HM and LTM+VTM around different range of enhancement percentage. The full breakdown is reported in the XLS attached to this contribution.

Table 10 – Enhancement percentage across LTM+HM data points

| Range % | Number of data points |
| --- | --- |
| Up to 3% | 34 |
| From 3% to 7% | 11 |
| From 7% to 12% | 7 |
| Above 12% | 4 |

Table 11 – Enhancement percentage distribution across LTM+VTM data points

| Range % | Number of data points |
| --- | --- |
| Up to 3% | 22 |
| From 3% to 7% | 17 |
| From 7% to 12% | 11 |
| Above 12% | 2 |

The results show a general improvement of the New configuration when compared with the VT configuration, particularly across an extended bitrate range. In particular, the improvement between the two configurations in terms of BD-Rate PSNR is as follows:

- LTM+HM over HM: 	Y: -5.65%; 	U: -6.37%; 	V: -5.18%; 	YCrCb: -5.679%

- LTM+VTM over VTM: Y: -38.23%; 	U: -25.63%; 	V: -22.98%; 	YCrCb: -34.75%

Note that the study of the configurations for LTM focused on PSNR as the primary metric. Although it is expected that these gains are also reflected in visual improvements, the study did not verify the visual quality of the encoded sequences.

IMPORTANT CLARIFICATION: By way of clarifying, the performances of the LTM are not linked to the capability of the LCEVC format and standard tools, but rather to how these are used in a given implementation. For example, with other commercial implementations – [5] – we can see that the performances of the format can be even higher than what demonstrated in [4].

## Next steps

This document provides a report of the work carried out by the authors aimed at improving the configuration of the LCEVC Test Model (LTM). The results are still preliminary configurations as only part of the configuration parameters have been studied.

In the next phase, the authors expect to review some additional configuration parameters to determine further improvements to the configurations and aim at providing encoding parameter formulas for automatic configuration of the LTM (per metric) as per [6].

# Patent rights declaration(s)

V-Nova may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Referenced publications

[1] “Evaluation results of Low Complexity Enhancement Video Codec (LCEVC) with HM and VTM on 4K content”, JVET-AF0075, Hannover, October 2023.

[2] “Informal Subjective Evaluation of Low Complexity Enhancement Video Codec (LCEVC) with VVC on SDR UHD (4K) Content”, JVET-AG0071, Online, January 2024.

[3] “Considerations on LCEVC performances in relation to input document JVET-AF0075/m65560”, JVET-AF0292, Hannover, October 2023

[4] “Response to JEVT-AG0071/m65960 on Informal Subjective Evaluation of Low Complexity Enhancement Video Codec (LCEVC) with VVC on SDR UHD (4K) Content”, JVET-AG0224, Online, January 2024

[5] Report of 14th meeting - ISO/IEC JTC 1/SC 29/WG 04 N0439 – Online, January 2024

[6] “LTM improvement plan” ISO/IEC JTC 1/SC 29/WG 04 N0513 – Rennes, April 2024

[7] “Progress Report on Improvements to LTM” ISO/IEC JTC 1/SC 29/WG m67786 – Rennes, April 2024

[8] “Verification Test Report on the Compression Performance of Low Complexity Enhancement Video Coding” ISO/IEC JTC 1/SC 29/WG 4 N0029 (m20173) – Online, April 2021

ANNEX A (Verification test configuration vs optimized configuration using HM as base codec)

ANNEX B (Verification test configuration vs optimized configuration using VTM as base codec)