| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0322-v1 |
| --- | --- |

| Title: | AHG9: On Neural Network Post Filter with Multiple Temporal Extrapolations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

The current Neural-Network Post-Filter allows temporal extrapolation purpose. It is asserted that currently NNPF can only return one possible temporal extrapolation result. It is proposed to allow NNPF to return multiple temporal extrapolation results.

# Introduction

The VSEI V4 draft 2 [1] includes a Neural-Network Post-Filter Characteristics SEI.

The Neural network post filter characteristics (NNPFC) SEI includes temporal extrapolation purpose which allows generating one or more future pictures.

Following is proposed related to temporal extrapolation NNPF:

## It is asserted that currently NNPF can only return one possible temporal extrapolation result. It is asserted that it is useful to allow more than one temporal extrapolation results to be returned by the NNPF. Multiple temporal extrapolation results can provide alternative possible scenarios that may occur as time (on video playback timeline) increases. To enable support for multiple temporal extrapolations, it is proposed to define a syntax element which allows specifying desired number of multiple temporal extrapolations expected to be generated (or requested) from NNPF when performing temporal extrapolation. A modification is also proposed to the StoreOutputTensors( outputTensor ) to allow the NNPF to return multiple temporal extrapolation results.

# Proposal

It is proposed to allow NNPF to return multiple temporal extrapolation results.

## The proposed approach is shown below with modifications on top of JVET-AH2006 [1].

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) { |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| nnpfc_num_temporal_extrapolations_minus1 | ue(v) |
| } |  |
| … |  |
| } |  |
| nnpfc_component_last_flag | u(1) |
| … |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

nnpfc_num_temporal_extrapolations_minus1 plus one indicates the number of alternative temporal extrapolations expected to be generated (or requested) by the NNPF.

The variable NumTemporalExtrapolations, specifying the number of temporal extrapolations present in the output tensor of the NNPF is derived as follows:

NumTemporalExtrapolations = nnpfc_num_temporal_extrapolations_minus1 + 1

…

The process StoreOutputTensors( ), for deriving sample values in the sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, for the NNPF-generated pictures, from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numPicsInOutputTensor; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
	 if(TemporalExtrapolationFlag)
    for( j = 0; j < NumTemporalExtrapolations; j++ ) 
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfcOutputPicHeight  &&  xY < nnpfcOutputPicWidth )
					if( !nnpfc_component_last_flag )
					 if(TemporalExtrapolationFlag ) 
						 FilteredYPic[ i ][ j ][ xY ][yY ] = outputTensor[ 0 ][ i ][ j ][ 0 ][ yP ][ xP ]
					 else
						 FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]					else
					if( TemporalExtrapolationFlag) 
						 FilteredYPic[ i ][ j ][ xY ][yY ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 0 ]
					 else						 
						 FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )				(97)
	 if(TemporalExtrapolationFlag)
    for( j = 0; j < NumTemporalExtrapolations; j++ ) 
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfcOutputPicHeight / outSubHeightC  &&
						xSrc < nnpfcOutputPicWidth / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
				if(TemporalExtrapolationFlag) {
						 FilteredCbPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ 0 ][ yP ][ xP ]
						 FilteredCrPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ 1 ][ yP ][ xP ]
					} else {
						 FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						 FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					 }
					} else {
				if(TemporalExtrapolationFlag ) {
						 FilteredCbPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 0 ]
						 FilteredCrPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 1 ]					 } else {
						 FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						 FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					 }
				}
			}
	else if( nnpfc_out_order_idc  = =  2 ) 
	if(TemporalExtrapolationFlag)
    for( j = 0; j < NumTemporalExtrapolations; j++ ) 
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfcOutputPicHeight  &&  xY < nnpfcOutputPicWidth )
					if( !nnpfc_component_last_flag ) {
				if(TemporalExtrapolationFlag) {
						 FilteredYPic[ i ][ j ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ j ][ 0 ][ yP ][ xP ]
						 FilteredCbPic[ i ][ j ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ j ][ 1 ][ yPc ][ xPc ]
						 FilteredCrPic[ i ][ j ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ j ][ 2 ][ yPc ][ xPc ]					} else {
						 FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						 FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						 FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					 }
					} else {
					if(TemporalExtrapolationFlag) {
						 FilteredYPic[ i ][ j ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 0 ]
						 FilteredCbPic[ i ][ j ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ j ][ yPc ][ xPc ][ 1 ]
						 FilteredCrPic[ i ][ j ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ j ][ yPc ][ xPc ][ 2 ]					} else {
						 FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						 FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						 FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					 }
				}
			}
	else if( nnpfc_out_order_idc  = =  3 ) 
	 if(TemporalExtrapolationFlag)
    for( j = 0; j < NumTemporalExtrapolations; j++ ) 
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfcOutputPicHeight / 2  &&
						xSrc < nnpfcOutputPicWidth / 2 )
					if( !nnpfc_component_last_flag ) {
						 if(TemporalExtrapolationFlag) {
						 FilteredYPic[ i ][ j ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ j ][ 0 ][ yP ][ xP ]
						 FilteredYPic[ i ][ j ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ j ][ 1 ][ yP ][ xP ]
						 FilteredYPic[ i ][ j ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ j ][ 2 ][ yP ][ xP ]
						 FilteredYPic[ i ][ j ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ j ][ 3 ][ yP ][ xP ]
						 FilteredCbPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ 4 ][ yP ][ xP ]
						 FilteredCrPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ 5 ][ yP ][ xP ]					} else {
						 FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						 FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						 FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						 FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						 FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						 FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					 }
					} else {
						if(TemporalExtrapolationFlag) {
						 FilteredYPic[ i ][ j ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 0 ]
						 FilteredYPic[ i ][ j ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 1 ]
						 FilteredYPic[ i ][ j ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 2 ]
						 FilteredYPic[ i ][ j ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 3 ]
						 FilteredCbPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 4 ]
						 FilteredCrPic[ i ][ j ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ j ][ yP ][ xP ][ 5 ]					} else {
						 FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						 FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						 FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						 FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						 FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						 FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					 }
				}
			}
}

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AH2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, SEI messages for VSEI version 4 (Draft 2), May 2024

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.