| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 10–17 July 2024 | Document: JVET-AI0334-v1 |
| --- | --- |

| Title: | AHG8: Report of VCM-GTS1: study of ground truth data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating (Sony), Rose Nguyen (Canon), Se Yoon Jeong (ETRI), Hyomin Choi (InterDigital), Ivan V. Bajić (SFU) | Email: | steve.keating@sony.com |
| Source: |  |  |  |

_____________________________

# Abstract

This document summarizes the study of SFU-HW ground truth data performed between the 145th and 146th MPEG meetings (34th and 35th JVET meetings).  As a continuation of the GTS1 study two sequences’ data have been revised.

This document is taken directly from the MPEG document m68542 for sharing with JVET in the interests of being able to have a common CTC for both groups.

# Scope of study

The study was to revise two sequences: Traffic and PartyScene.  The revisions are due to concerns raised about these sequences’ data at the April 2024 MPEG WG4 meeting.

# Revised sequences

In the previous phase of the study 4 sequences were revised, in this phase two of those sequences were further revised to address concerns raised.  The revised data was cross-checked in each case, sometimes this involved further stages of revision.

Table 1. Sequences for ground truth data revision

| Test | Sequence | Testers | Contact(s) | Cross-checker |
| --- | --- | --- | --- | --- |
| GTS.1 | Traffic | Sony | Steve.Keating@sony.com | Se Yoon Jeong (ETRI) Rose Nguyen (Canon) |
| GTS.2 | PartyScene | Sony | Steve.Keating@sony.com | Se Yoon Jeong (ETRI) Rose Nguyen (Canon) |

Traffic (before & after, top-right detail)

Some of the boxes for the cars on the overpass have been removed to improve consistency.  A few early images of the sequence are shown above: before (left) and after (right) revision.

PartyScene (before & after)

The box for the soft toy on the left has been removed from all frames, the box for the girl on the left has been removed for some early frames where it was judged that the object was not clear.
[ground truth boxes are shown in green, Detectron2 output boxes are shown in red dashed]

## Objective results

The table below shows the mAP values for the source image sequences with the original, previous revised and re-revised ground truth data:

| Sequence | mAP orig | mAP revised April24 | mAP revised July24 |
| --- | --- | --- | --- |
| Traffic_2560x1600_30 | 44.21 | 82.69 | 83.32 |
| PartyScene_832x480_50 | 66.78 | 80.90 | 82.90 |

This shows little change for Traffic and a small increase for PartyScene due to the removal of the additional bounding boxes.

## RD Curves

The RD curves (VCM-RSv0.9 Random Access mode) using the April version (in red) and revised version (in purple) are shown below:

Traffic

PartyScene

It can be seen for this sequence that the mAP is generally higher due to the removal of the additional bounding box for the soft toy.

# Conclusions

Two sequences’ ground truth data have been revised in response to the concerns raised at the April 2024 meeting.

# Availability

Updated version 3.2 data of object detection labels is available in YOLO format at the following URL:

https://data.mendeley.com/datasets/367kty6nw7/3

# License

Copyright 2024 - © Simon Fraser University (SFU) Multimedia Lab

The dataset is provided under the Creative Commons license BY 4.0 (CC BY 4.0).

URL: https://creativecommons.org/licenses/by/4.0/

If you use this dataset in your work, please refer to it as SFU-HW-Objects-v3.2, cite the following document:

H. Choi, E. Hosseini, S. R. Alvar, R. A. Cohen, and I.V. Bajić, “A dataset of labelled objects on raw video sequences,” Data in Brief, vol.34, article no. 106701, Feb. 2021. [DOI]

and provide the URL: https://doi.org/10.1016/j.dib.2020.106701

Note that the dataset contains only the object detection labels, not the video sequences themselves. The sequence license agreements are based on  [5] and [6], and are summarized in the following table:

| Class | Sequence name | Source/Owner/Copyright | Can be used for VCM activity? |
| --- | --- | --- | --- |
| B | Kimono | Tokyo Institute of Technology, Nakajima Laboratory / C3 | Yes |
| B | ParkScene | Tokyo Institute of Technology, Nakajima Laboratory / C3 | Yes |
| B | Cactus | EBU/RAI / C4 | Yes |
| B | BasketballDrive | NTT DOCOMO Inc. / C5 | Yes |
| C | BasketballDrill | NTT DOCOMO Inc. / C5 | Yes |
| C | PartyScene | NTT DOCOMO Inc. / C5 | Yes |
| C | RaceHorses | NTT DOCOMO Inc. / C5 | Yes |
| D | BasketballPass | NTT DOCOMO Inc. / C5 | Yes |
| D | BlowingBubbles | NTT DOCOMO Inc. / C5 | Yes |
| D | RaceHorses | NTT DOCOMO Inc. / C5 | Yes |
| E | KristenAndSara | Vidyo, Inc. / C6 | Yes |
| E | Johnny | Vidyo, Inc. / C6 | Yes |
| E | FourPeople | Vidyo, Inc. / C6 | Yes |

C3:

Individuals and organizations extracting sequence from this archive agree that the sequences and all intellectual property rights therein remain the property of Nakajima Laboratory of Tokyo Institute of Technology. This material may only be used for the purpose of developing, testing and promulgating technology standards. The material cannot be distributed with charge. Nakajima Laboratory of Tokyo Institute of Technology makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose.

C4:

These sequences and all intellectual property rights therein remain the property of the RAI. These sequences may only be used for the purpose of developing, testing and promulgating technology standards. RAI and EBU Technical make no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose.

C5:

These sequences and all intellectual property rights therein remain the property of the NTT DOCOMO, INC. These sequences may only be used for the purpose of developing, testing and promulgating technology standards. NTT DOCOMO, INC. makes no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose.

C6:

Vidyo donates the sequences to the public domain.

# References

| [1] | S. Liu, H. Zhang, C. Rosewarne. "Common test conditions for video coding for machines" ISO/IEC JTC 1/SC 29/WG 04 N0419, October 2023, Hannover, DE. |
| --- | --- |
| [2] | “VCM-GTS1: Study of ground truth data” ISO/IEC JTC 1/SC 29/WG 04 N0507, April 2024, Rennes. |
| [3] | S Y Jeong, J Lee, Y Kim. “[VCM] VCM-GTS(m67333) cross-check results and comments“ ISO/IEC JTC 1/SC 29/WG 4 m68115, April 2024, Rennes. |
| [4] | S. Keating, R. Nguyen, H. Choi, I.V. Bajić. “[VCM] report of VCM-GTS1: study of ground truth data” ISO/IEC JTC 1/SC 29/WG 4 m67333, April 2024, Rennes. |
| [5] | S. Keating, R. Nguyen, H. Choi, I.V. Bajić. “[VCM] report of VCM-GTS1: study of ground truth data” ISO/IEC JTC 1/SC 29/WG 4 m66223, January 2023, Online. |
| [6] | H. Choi, E. Hosseini, S.R. Alvar, R.A. Cohen, I.V. Bajić (SFU), A. Karabutov, Yin Zhao, E. Alshina (Huawei) "[VCM] Object labelled dataset on raw video sequences” ISO/IEC JTC1/SC 29/WG 11 m54737, July 2020, Geneva, CH. |
| [7] | JCT-VC, "Joint call for proposals on video compression technology”, ISO/IEC JTC 1/SC 29/WG 11 document N11113, Jan. 2010. |
| [8] | W. Wang, J. Boyce, D. Hong, A Abbas (Vidyo), “New class E videoconferencing source content”, JCTVC-H0389, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG 11 8th Meeting: San José, CA, USA, 1-10 Feb. 2012. |