| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI2027-v1 |
| --- | --- |

| Title: | Draft Common Test Conditions (CTC) for gaming applications |  |  |
| --- | --- | --- | --- |
| Status: | Output document of JVET |  |  |
| Purpose: | Common test conditions including the description of gaming sequences and auxiliary data |  |  |
| Author(s) or Contact(s): | J. Sauer, Huawei R. Chernyak, Tencent S. Puri, InterDigital S. Thiebaud, InterDigital | Email: | johannes.sauer@huawei.com chernyak@global.tencent.com saurabh.puri@interdigital.com sylvain.thiebaud@interdigital.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC) and configurations to be used in the context of gaming content coding experiments after the 35th JVET meeting. In addition, this document includes the description of sequences as candidate sets G1/G2/G3 and documentation of the auxiliary data formats associated with the candidate sets G1/G3.

The common test conditions are recommended for use in technical contributions to 36th JVET meeting relating to gaming content coding.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and facilitate the comparison of the outcome of experiments.

This document defines 4 test conditions for gaming video content, reflecting intra-only, random-access, and low-delay settings:

- Intra, 10-bit

- Random-access, 10-bit

- Low delay, 10-bit

- Low delay, P slices only, 10-bit (optional)

A subset of these test conditions might be used for a particular experiment. However, as these test conditions are for gaming video coding experiments, results for low delay configurations shall be provided.

The following sections define anchors, test sequences, quantization parameter values, encoder configuration files, evaluation metrics, and compile-time options to be used.

Anyone bringing input contributions to JVET meetings should provide a set of results that is as complete as possible and uses the common test conditions that apply to the proposal.

# Test sequences

## Class G1 and G2

Table 1 defines the set of test sequences to be used for intra, random-access, and low-delay conditions. The test sequences are split into three classes, class G1, class G2 and class G3. Class G1 and G3 provide auxiliary data associated to the gaming sequence extracted from the respective game engines. The difference between theses classes is that rendering was done for SDR in class G1 and for HDR in class G3. The details on auxiliary data for classes G1/G3 are provided below in section 2.2.

When encoding the test sequences, frames from the start frame until the end frame (included) shall be encoded for all sequences and test cases described below, with the exception of the intra configuration. For the intra configuration, the first of every 8 frames shall be encoded starting with the first frame (indicated by the start frame in Table 1: Test sequences) in the sequence for all sequences, and bit rates shall be calculated using the sequence frame rate (as defined by frame rate in the table) divided by eight.

The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the CTC for the given configuration

- “-” indicates that the test sequence is not requested in the CTC or the given configuration

Original versions of the test sequences in Table 1 are available as follows:

- Test sequences and auxiliary data are available on JVET FTP at:

  - ftp://jvet-ul1@ftp.ient.rwth-aachen.de/draftCTC_gaming_AI2027

Test sequences are only available to qualified participants. Qualified participants may contact the JCT-VC/JVET chairs for login information. Specific license information is provided with each test sequence.

Table 1: Test sequences

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 8 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 8 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 8 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
| G3 (HDR) | Level1_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | Darktree_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | ARPG2_HDR | 600 | 300 | 60 | 8 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 8 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 8 | O | O | O |

Transfer function is PQ for HDR class G3.

MD5 hashes and file names for each sequence are listed in Annex A. The hashes shall be used to verify that the correct version of the test sequence is used.

Notes:

- If visuals tests are requested:

  - BaoleiYard: it is recommended to encode the full 10s (600 frames), but use only the last 9s for visual tests.

## Auxiliary information for Classes G1 and G3

This paragraph describes the mandatory auxiliary information associated with the gaming content of Classes G1 and G3. Two kinds of information are provided: some “camera parameters" and some maps, including a depth map.

Depth maps are provided in floating point 32-bits, in the screen space, range [0,1]. These maps depend on the way the depth is coded in the game engine. The closest 3D points to the virtual camera are usually coded 0 and the furthest points coded 1. This mapping is called “Normal” mapping in this document. Conversely, some depth maps may code the closest point 1 and the furthest 0, this mapping is called “Inverted”. The way the map is coded by the game engine is indicated in Table 2 for each sequence.

Table 2. G1/G3 test sequences parameters

| Sequence name | Depth map Type | Depth map coding |  |
| --- | --- | --- | --- |
| Level1_SDR, Level1_HDR | Type 1 | Normal/Inverted |  |
| Darktree_SDR, Darktree_HDR | Type 1 | Normal/Inverted |  |
| ARPG2_SDR, ARPG2_HDR | Type 2 | Normal/Inverted |  |
| DesertTown3_SDR, DesertTown3_HDR | Type 2 | Normal/Inverted |  |
| SunTemple3_SDR, SunTemple3_HDR | Type 2 | Normal/Inverted |  |

When a 3D point of the 3D game scene is projected by the game engine’s virtual camera, the result of the projection is provided in Normalize Device Coordinates NDC, in the range [-1,1]. As described above, the depth map information is provided in the screen space, in the range [0,1].

When the depth map coding parameter Table 2 is “Normal”, the relationship between the ZNDC depth in Normalized Device Coordinate and the depth map in screen space provided by the game engine ZBufEngine is:

ZbufEngine=zNDC+12

And when the depth coding is “Inverted”:

ZbufEngine=1-zNDC2

Whatever the sequence, the relationship between the 3D depth Z and ZNDC is given by:

ZNDC= aZ+c

The 2 coefficients a and c depend on the depth map Type Table 2, associated with each gaming sequence.

The near clipping plane ZN and the far clipping plane ZF respectively represent the minimum (closest distance to the camera) and the maximum (furthest distance) depth Z rendered by the game engine. ZN and ZF are provided as camera parameters auxiliary information, as presented paragraph 2.2.3. They are used to compute the 2 coefficients a and c, depending on the depth map Type Table 2.

### Coefficients for Depth maps from Unity game engine

For sequences of Type 1, the 2 coefficients are:

a=-2ZFZNZF-ZN,  and c=ZF+ZNZF-ZN

### Coefficients for Depth maps from Unreal game engine

For sequences of Type 2, the 2 coefficients are:

a=2ZN,  and c=-1

### Camera Parameters as auxiliary information

Some parameters are mandatory for gaming contents of Class G1. These mandatory parameters are:

- The Projection matrix “projectionMatrix”

This 4x4 32-bits floating point matrix is used to project a 3D point relative to the camera coordinate system to NDC domain.

- The World to Camera Matrix “worldToCameraMatrix”

This 4x4 32-bits floating point matrix transform a 3D point relative to the 3D world coordinate system to a 3D point relative to the camera coordinate system. This matrix may also refer as the "view matrix”.

- The Near clipping plane “nearClipPlane”

The near clipping plane is a 32-bits floating point defining the closest distance to the camera used for the rendering. This parameter corresponds to ZN in the equation of depth mapping.

- The Far clipping plane “farClipPlane”

The far clipping plane is a 32-bits floating point defining the furthest distance to the camera used for the rendering. This parameter corresponds to ZF in the equation of depth mapping.

Drawings in Annex E illustrate these parameters.

These parameters are provided in a text file, as JSON objects. A JSON object per frame must be provided, the frame number being indicated as an integer, by the mandatory element “frame”.

This format is documented in Annex D.

### Maps as auxiliary information

A 32-bits floating point depth map is provided as auxiliary information associated with the gaming sequences of Classes G1 and G3. The resolution of the map is the same as the gaming content resolution.

The relationship between this game engine map ZBufEngine and the real 3D depth is given paragraph 2.2

32-bits floating point motion vectors maps are also provided in the same resolution as the gaming content. A map is provided for each component of the motion vector mv, representing the displacement of the current pixel pc relative to the pixel in the previous frame pp, expressed in pixel:

mv=pc-pp

These maps are provided in EXR files. When these 3 maps are provided as a unique EXR file per frame, the depth is coded in the red channel, the horizontal component of the motion vector in the green channel and the vertical component in the blue channel.

The file name convention for these files is as follows:

“{SeqName}_motion_depth_frame{X}.exr”,

where {SeqName} should be replaced by the name of the seqeunce and {X} by the franme number to which the auxiliary information corresponds.

# Anchor

## Software

The enhanced compression model (ECM) is provided as an anchor. Corresponding configuration files are provided, as described in section 6.

It is recommended to use the most recently tagged version of the ECM and VTM-11.0ecm reference software. Tagged versions are listed at Tags · ECM / ECM · GitLab (fraunhofer.de). The ECM and VTM versions that are used should be reported along with results.

## Configuration files

The following sections define encoder configuration files to be used for each test case. Parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system

- FrameRate to reflect the frame rate of a given sequence as per Table 1

- SourceWidth to reflect the width of the source video sequence

- SourceHeight to reflect the height of the source video sequence

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1

- IntraPeriod to reflect the intra refresh period in the random-access test cases. The intra refresh period is dependent on the frame rate of the source and rounded to multiples of the GOP size in use as shown in Table 3.

- QP to reflect the quantization parameter values defined in section 4.

- InputBitDepth to reflect the bit depth of a given sequence as per Table 1

Table 3. Intra period for random access test case

| Frame rate (fps) | IntraPeriod |
| --- | --- |
| 20 | 32 |
| 24 | 32 |
| 30 | 32 |
| 50 | 64 |
| 60 | 64 |
| 100 | 96 |

The following configuration files are provided in the cfg/ folder of the ECM software package (available at https://vcgit.hhi.fraunhofer.de/ecm/ECM).

Test condition configuration files:

- “All Intra” (AI): encoder_intra_ecm.cfg

- “Random-access” (RA): encoder_randomaccess_ecm.cfg

- “Low-delay B” (LB): encoder_lowdelay_ecm.cfg

- “Low-delay P” (LP, optional): encoder_lowdelay_P_ecm.cfg

Different class-dependent encoder optimizations are enabled for different configurations. Per-class configuration files are provided in the cfg/per-class/ folder and their usage depending on the test configuration is detailed in Table 4. For example, classA.cfg sets CTU size to 256., classB_lowdelay.cfg and classB_randomaccess.cfg set different MTT depths. For example, when invoking the ECM encoder, the class configuration file should be provided after the main test condition configuration file such as to correctly override settings, e.g:

- EncoderApp -c cfg/encoder_intra_ecm.cfg -c cfg/per-class/classA.cfg

- EncoderApp -c cfg/encoder_randomaccess_ecm.cfg -c cfg/per-class/classB_randomaccess.cfg

- EncoderApp -c cfg/encoder_lowdelay_ecm.cfg -c cfg/per-class/classB_lowdelay.cfg

Table 4. Per-sequence configuration files depending on the test configuration

| Sequence | AI | RA | LB/LP |
| --- | --- | --- | --- |
| Level1_SDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| Darktree_SDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| ARPG2_SDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| DesertTown3_SDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| SunTemple3_SDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| GTAV |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| Minecraft |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| JianlingTemple |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| BaoleiYard | classA.cfg | classA_randomaccess.cfg |  |
| Level1_HDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| Darktree_HDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| ARPG2_HDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| DesertTown3_HDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |
| SunTemple3_HDR |  | classB_randomaccess.cfg | classB_lowdelay.cfg |

Sequence-specific parameters are to be found in the cfg/per-sequence-gaming/ for gaming content, respectively.

## Parallel encoding/decoding

Parallel encoding/decoding as described in JVET-B0036 may be applied for RA configurations. If parallel encoding and decoding is used, this method should be applied for both versions, the reference and the version under test.

When temporal pre-filtering is enabled (TemporalFilter configuration option), the encoder may use additional frames before the beginning and after the end of an encoded segment. The used number of frames is specified in the parameters. These additional frames need to be available to the encoder to match sequential coding.

Encoded bitstream segments can be concatenated using the “parcat” tool included in the reference software. PSNR can be computed by comparing the complete decoded YUV file with the encoder input file (possibly with bit depth scaling applied to it).

Alternatively, PSNR values output by the encoder for each frame can be parsed and averaged across all bitstream segments (frames that are coded in multiple bitstream segments should be included only once in the average). In this case, the average should be computed using precise 64-bit PSNR values. The PrintHexPSNR configuration option enables precise PSNR values output. An example output is shown below.

started @ Thu Jul 19 12:00:36 2018
POC    0 TId: 0 ( I-SLICE, QP 31 )      99744 bits [Y 34.2229 dB    U 39.9935 dB    V 40.4950 dB] [xY 40411c86a4885c06 xU 4043ff297914448c xV 40443f5abbf92dc0] [ET     5 ] [L0 ] [L1 ]
POC    1 TId: 0 ( P-SLICE, QP 40 )        752 bits [Y 32.0300 dB    U 39.9398 dB    V 40.5244 dB] [xY 404003d68968f64a xU 4043f84b7d49cd2e xV 40444320b56fc3c1] [ET     0 ] [L0 0 ] [L1 ]

	Total Frames |   Bitrate     Y-PSNR    U-PSNR    V-PSNR    YUV-PSNR 
	        2    a    3014.8800   33.1264   39.9666   40.5097   34.3586

 finished @ Thu Jul 19 12:00:43 2018

Such hexadecimal strings can easily be converted back to double-precision floating points values using C code such as:

#include <inttypes.h>

double hex2double(const char *str)
{
  double val;
  sscanf(str, "%" SCNx64, (uint64_t *)&val);
  return val;
}

or Python code such as:

import struct
import binascii

def hex2double(str):
  return struct.unpack('>d', binascii.unhexlify(str))[0]

or Ruby code such as:

def hexToDouble(hex)

["#{hex}"].pack('H16').unpack('G').first

## Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/CommonLib folder of the reference software. The default settings provided in the source code should be used.

# Test Conditions

## Temporal Prediction

The three test conditions are further defined as follows:

- All Intra: No prediction between different pictures.

- Random Access: Not more than 64 frames of structural delay, e.g. 32 picture “group of pictures (GOP)” and random-access intervals of 1.6 seconds or less. A random-access interval of 1.6 seconds or less shall be defined as 32 pictures or less for a video sequence with a frame rate of 20, 24, 25, or 30 frames per second, 64 pictures or less for a video sequence with a frame rate of 50 or 60 frames per second, and 96 pictures or less for a video sequence with a frame rat of 100 frames per second.

- Low Delay: No picture reordering between decoder processing and output.

- Low Delay, P Slice Only: No picture reordering between decoder processing and output; no use of bi-directional prediction.

## Bit depth

The input and output bit-depth of the codec shall be 10-bit regardless of the bit-depth of the input sequence.  For the 8-bit sequences in Table 1, each 8-bit source sample x should be converted prior to encoding to a 10-bit value 4*x.

Note: This behavior is built into the anchor software and no external conversion program is required when generating the anchor.

## Quantization parameter values

For each video sequence five quantization parameter values are to be used: 22, 27, 32, 37 and 42. These values define the initial QP values that are specified as the input QP of the anchor software. Adaptation of QP to lambda and refinement for other frames in the GOP in the configuration files may change the used QP value on per-frame basis.

# Reported Metrics

The attached SDR Excel sheets contain a reporting template in which bitrate, PSNR, coding time, peak memory usage and BD-rate results are reported for the tested configuration against the anchor.

For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE), 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results. This behavior is built into the reference encoder and no external conversion program is required.

For the purpose of reporting encoding and decoding running times, the anchor and test should be simulated on the same platform, e.g. similar CPU cores, to have reliable time comparison. The Excel summary sheet contains fields to report attributes of the testing environment, including CPU type, compiler, and decoder configuration. Filling in these fields when making a report is highly encouraged.

For the purpose of reporting peak memory usage of the encoder and the decoder, the “VmPeak” value in the encoding and decoding log file shall be used. In the case of random access configuration, where video segments can be encoded in parallel as described in Section 9, the maximum “VmPeak” value among all of the segments shall be reported in the Excel sheets.

In the attached Excel sheets, BD-rate is calculated using both piece-wise cubic and cubic interpolations, and the piece-wise cubic interpolation is used in the results summary.

The attached HDR Excel sheets contain a reporting template, where results reported for G3 sequence class shall include the following metrics:

- PSNR for Y′, Cb and Cr

- wPSNR for Y’, Cb and Cr

- deltaE100

- PSNRL100

Results reported for H2 sequence class shall include the following metrics:

- PSNR for Y’, Cb and Cr

The metrics deltaE100 and PSNR-L100 shall be calculated using the HDRLib functionality available in the ECM software. More details about enabling the HDRLib functionality in the ECM are provided in Annex D and information for calculating wPSNR, deltaE100, and PSNR-L100 metrics is provided in Annex E.

# Annex A. Test sequence checksums

For each test sequence included in the common test conditions, the MD5 hash is listed below. The list can be copied into a text file and hashes can be checked using the md5sum tool:

md5sum -c <filename>

| Class | Sequence name | md5sum |
| --- | --- | --- |
| G1 (SDR) | Level1_SDR_1920x1080_60fps_10bit_420.yuv | 314468a1ce25a2132d442ee3bda5991a |
|  | Darktree_SDR_1920x1080_60fps_10bit_420.yuv | e105da995a5d2880ef7797ccd17330d2 |
|  | ARPG2_SDR_1920x1080_60fps_10bit_420.yuv | 17fcbd5c795db9beb90fc3147ce14b24 |
|  | DesertTown3_SDR_1920x1080_60fps_10bit_420.yuv | 749520e52c42514fc39479dd4d6bc14a |
|  | SunTemple3_SDR_1920x1080_60fps_10bit_420.yuv | 5220b5fa56104cde49a651907833a9fc |
| G2 | GTAV_1920x1080_60fps_8bit_420_start91.yuv | 5491b95f8cd6c312ddb68eac990107a1 |
|  | Minecraft_1920x1080_60fps_8bit_420_start600.yuv | 4f1055d558284e789fcae7686f3419bc |
|  | JianlingTemple_1920x1080_60fps_8bit_420.yuv | 408e5ed2c441a2fe71a6653615c9489d |
|  | BaoleiYard_4096x2160_60fps_8bit_420.yuv | 9ee5cf01da50eaf5a286e6128684d481 |
| G3 (HDR) | Level1_HDR_1920x1080_60fps_10bit_420.yuv | xxxxxxxxxxxxxxxxxx |
|  | Darktree_HDR_1920x1080_60fps_10bit_420.yuv | xxxxxxxxxxxxxxxxxx |
|  | ARPG2_HDR_1920x1080_60fps_10bit_420.yuv | 6eabdb7f0827afa5b0ecb95208b069c1 |
|  | DesertTown3_HDR_1920x1080_60fps_10bit_420.yuv | 8b91f2c5630b592d8bc0e30c6e33b758 |
|  | SunTemple3_HDR_1920x1080_60fps_10bit_420.yuv | e9766660f098c63a5874605a89bf6ccf |

# Annex B: Text file format providing the Camera Parameters for Class G1

An example of camera parameters text file is provided here. A JSON object is provided per frame, one JSON object per text line.

To make reading easier, here, the floating point values have been truncated, some carriage return and tabulation have been added.

The first line is informative, indicating a version number, and the list of the provided elements.

Then, for each frame a json object containing these 5 JSON elements must be provided:

- JSON element “frame”: Integer indicating the frame number.

- JSON objects “projectionMatrix” and “worldToCameraMatrix”: objects containing the elements representing the 4x4 32-bits floating point coefficients of the matrices.

- JSON element “nearClipPlane”: float representing the near clipping plane ZN.

- JSON element “farClipPlane”: float representing the far clipping plane ZF.

# Annex C: Drawing illustrating the game engine camera parameters

Camera parameters matrices

[WorldToCamera]

Pw(Xw,Yw,Zw)

Relative to the 3D coordinate system

Image

Pi(xi,yi,ziNDC)

Depth Z

Camera

coordinate system

[Projection]

Pc(Xc,Yc,Zc)

Relative to the camera coordinate system

3D world

coordinate system

ZN, ZF,

FocalLength…

NDC : Normalized Device Coordinate

ZNDC = 2*ZBufEngine – 1 (when ZN coded 0)

3D clipping planes projection to Normalized Device Coordinates

Xcam

Ycam

Zcam

Camera

coordinate system

3D world

coordinate system

Far plane

Near plane

ZF

ZN

Viewing volume delimited by the field of view and the clipping planes

Viewing volume in Normalized Device Coordinates NDC

(-1,-1,-1)

(1,1,1)

NDC mapping

# Annex D: Enabling HDRLib functionality in the ECM software

Enabling the HDRLib functionality in the ECM software is accomplished by doing the following:

- Create a local copy of the ECM software.

- Create a local copy of the HDRTools v0.19.1 or newer utility available at https://gitlab.com/standards/HDRTools.  More recent versions are encouraged where applicable.

- Copy the HDRTools/common directory from the HDRTools utility into the source/Lib/HDRLib directory of the ECM software.

Example: On a Linux system, this could be accomplished with the command:

ln -s <HDRTools_dir>/HDRTools/common <ECM_dir>/souce/Lib/HDRLib

- Append “-DEXTENSION_HDRTOOLS=on” when building the ECM software using CMake.

Example: If the ECM is normally built using the command:

cmake -DCMAKE_BUILD_TYPE=Release

The HDRLib functionality is enabled by using the command:

cmake -DCMAKE_BUILD_TYPE=Release -DEXTENSION_HDRTOOLS=on

The class specific parameters and sequence specific parameters provided with the ECM will then enable and configure the metric calculation parameters appropriately.

# Annex E: Objective distortion metrics for HDR quality evaluations

The common test conditions described in this document requires a number of metrics to be reported for evaluation. In this Annex, further information about the wPSNR, deltaE100, and PSNR-L100 metrics are provided.

- C.1. wPSNR

The wPSNR, or weighted PSNR, metric is calculated from the weighted mean squared error of the pixel values. An implementation of the metric is provided in the ECM software, and that implementation shall be used for calculation of the metric using the weighting functions provided below. It is noted that the HDRMetrics tool is capable of computing the weighted PSNR metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the ECM software.

As an informative description of the metric follows.

The wPSNR metric is calculated as:

wPSNR=10*logX2wMSE,

where X is the maximum pixel value for the specific bit depth and wMSE is given as

wMSE= all pixels i in blockwilumaxorig,i*(xorig, i-xdec,i)2,

where wilumaxorig,iis a weight that is a function of the luma value corresponding to the pixel xorig,i, xorig,i is the original value at location i, and xdec,i is the reconstructed value at location i.

The calculation of the weight is computed as:

XL = bitDepth >= 10 ? luma(xorig,i) >> (bitDepth - 10) : luma(xorig,i) << (10 - bitDepth);

yi = 0.015*xL − 1.5 − 6;

yi = yi < −3 ? −3 : (yi>6 ? 6 : yi);

wi(luma(xorig,i)) = pow(2.0, yi÷3.0);

where bitDepth is the bit depth of xorig.

The metric is calculated individually for a single luma or chroma channel and then used to compute a Bjøntegaard Delta-Rate and Delta-PSNR rate.

Note: The metric is defined for all bit-depths of xorig.  However, it is not anticipated that bit-depths less than 10-bits would be used in a practical HDR application.

- C.2. deltaE100-based metric

An implementation of the deltaE100 distortion metric is provided in the ECM software when enabled with the HDRLib functionality, and that implementation shall be used for calculation of the metric. It is noted that the HDRMetrics tool is also capable of computing the deltaE100 distortion metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the ECM software.

An informative description of the metric follows.

The original and test material must first be converted to linear-light 4:4:4 RGB EXR. For example, if the material is in the YCbCr BT.2100 4:2:0 PQ 10 bit format, it must be converted to a 4:4:4 RGB BT.2100 OpenEXR file using configuration provided with the HDRTools software.

Subsequently, the following steps should be applied for each (R,G,B) sample within the content to be compared, i.e. original source (content 1) and test material (content 2) :

- Convert the content to the XYZ colour space

- Convert the content from the XYZ to Lab space using the following equations. Computations are performed using double floating point precision.

  - invYn = 1.0 ÷ Yn, with Yn = 100

  - invXn = invYn ÷ 0.95047

  - invZn = invYn ÷ 1.08883

  - yLab = convToLab( y * invYn )

  - L = 116.0 * yLab − 16.0

  - a = 500.0 * (convToLab(x * invXn) − yLab )

  - b = 200.0 * ( yLab − convToLab (z * invZn) )

with convToLab(x) is defined as

if x >= 0.008856

convToLab(x) = x(1÷3)

else

convToLab(x) = 7.78704 * x + 0.137931

- The deltaE100 distance DE between two samples (L1,a1,b1) and (L2,a2,b2) is then computed as follows:

cRef = sqrt( a1 * a1 + b1 * b1 )

cIn  = sqrt( a2 * a2 + b2 * b2 )

cm = (cRef + cIn) ÷ 2.0

g = 0.5 * ( 1.0 − sqrt( cm7.0 ÷ ( cm7.0 + 257.0 ) ) )

aPRef = ( 1.0 + g ) * a1

aPIn  = ( 1.0 + g ) * a2

cPRef = sqrt( aPRef * aPRef + b1 * b1 )

cPIn  = sqrt( aPIn * aPIn + b2 * b2 )

hPRef = arctan( b1, aPRef )

hPIn  = arctan( b2, aPIn )

deltaLp = L1 − L2

deltaCp = cPRef − cPIn

deltaHp = 2.0 * sqrt( cPRef * cPIn ) * sin( (hPRef − hPIn) ÷ 2.0 )

lpm = ( L1 + L2 ) ÷ 2.0

cpm = ( cPRef + cPIn ) ÷ 2.0

hpm = ( hPRef + hPIn ) ÷ 2.0

rC = 2.0 * sqrt( cpm7.0 ÷ ( cpm7.0 + 257.0 ) )

dTheta = DEG30 * exp(−((hpm − DEG275) ÷ DEG25) * ((hpm − DEG275) ÷ DEG25))

rT = −sin( 2.0 * dTheta ) * rC

t = 1.0 − 0.17 * cos(hpm − DEG30) + 0.24 * cos(2.0 * hpm) +

0.32 * cos(3.0 * hpm + DEG6) − 0.20 * cos(4.0 * hpm − DEG63)

sH = 1.0 + ( 0.015 * cpm * t )

sC = 1.0 + ( 0.045 * cpm )

sL = 1.0 + ( 0.015 * (lpm − 50) * (lpm-50) ÷ sqrt(20 + (lpm − 50) * (lpm − 50)) )

deltaLpSL = deltaLp ÷ sL

deltaCpSC = deltaCp ÷ sC

deltaHpSH = deltaHp ÷ sH

deltaE100 = sqrt( deltaLpSL * deltaLpSL + deltaCpSC * deltaCpSC +

deltaHpSH * deltaHpSH + rT * deltaCpSC * deltaHpSH )

with

DEG275 = 4.7996554429844

DEG30 = 0.523598775598299

DEG6 = 0.1047197551196598

DEG63 = 1.099557428756428

DEG25 = 0.436332

After this process the deltaE100 values for each frame are averaged within the “Distortion” specified window. Finally, a PSNR based value is derived as:

PSNR_DE = 10 * log10( PeakValue ÷ deltaE100 )

where PeakValue is set to 10,000 for sequences in the PQ format and 1,000 for sequences in the HLG format.

- C.3. PSNR-L100

An implementation of the PSNR-L100 distortion metric is provided in the ECM software when enabled with the HDRLib functionality, and that implementation shall be used for calculation of the metric. It is noted that the HDRMetrics tool is also capable of computing the PSNR-L100 distortion metric. If the HDRMetric package is used, it shall be configured to provide the same result as the implementation of the metric provided in the ECM software.

An informative description of the metric follows:

PSNR-L100 represents the distortion in the lightness domain of the CIELab colour space. The derivation of Lab values from the linear representation of the signal is similar as given in the description of deltaE100. The mean absolute error (MAE) in the L domain is used to compute the PSNR-L100 as follows:

PSNR-L100 = 10 * log10( PeakValue ÷ MAE )

where PeakValue is set to 1,000 for sequences in the PQ format and 1,000 for sequences in the HLG format.

The HDRTools utility and ECM software are maintained separately and not released according to the same schedule. Thus, it is possible that a version of the utilities newer than v0.19.1 may not be compatible with a given version of the ECM software (and, equivalently, a newer version of the ECM software may not be compatible with a given version of the HDRTools utility). It is recommended to use the most recent version of the HDRTools utility when possible. Otherwise, if there are compatibility issues between the most recent version of the HDRTools and the ECM software, it is then recommended to use v0.19.1.

For some versions of HDRMetrics, this is accomplished by using the following settings; -p EnableWTPSNR=1 -p Enable JVETPSNR=1 -p WeightTableFile=”hdrTable.txt”

As defined above, reporting the deltaE100, PSNR-L100 and wPSNR metrics is not required for HLG content. This example is provided solely for completeness.