| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 37th Meeting, Geneva, CH, 14–22 January 2025 | Document: JVET-AK_notes_d1 |
| --- | --- |

| Title: | Meeting Report of the 37th Meeting of the Joint Video Experts Team (JVET), Geneva, 14–22 January 2025 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its thirty-seventh meeting during 14–22 January 2025 meeting at the ITU headquarters facilities in Geneva, Switzerland. The meeting was held as a hybrid meeting, where remote participation was provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the eighteenth meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 0930 CET on Tuesday 14 January 2025. Meeting sessions were held on all days including the weekend days of Saturday and Sunday 18 and 19 January 2025, until the meeting was closed at approximately XXXX hours CET on Wednesday 22 January 2025. Approximately XXX people attended the JVET meeting (XXX in person and XXX remotely), and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 17 AHG reports, 2 EE summary reports, X BoG reports, and X incoming liaison document(s) were discussed. The meeting took place in a collocated fashion with a meeting of SG21 – one of the two parent bodies of the JVET, under whose auspices this JVET meeting was held. Various SC29 Working Groups and Advisory Groups were also meeting in Geneva with partial temporal overlap – where WG 5 is representing the Joint Video Experts Team and its activities from the perspective of the SC 29 parent body. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the thirty-sixth JVET meeting in producing the following documents:

a) JVET documents

- JVET-AJ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AJ1006 HEVC extensions and corrections

- JVET-AJ1011 HEVC white paper, also issued as AG 3 N 174

- JVET-AJ1012 Overview of IT systems used in JVET

- JVET-AJ2003 Guidelines for VTM-based software development

- JVET-AJ2005 Additions and corrections for VVC version 4 (Draft 10), also issued as WG 5 preliminary DAM N 330

- JVET-AJ2006 Additional SEI messages for VSEI version 4 (Draft 4), also issued as WG 5 preliminary DAM N 319

- JVET-AJ2007 Guidelines for NNVC software development

- JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2), also issued as WG 5 DIS N 322

- JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AJ2019 Description of algorithms version 9 and software version 11 in neural network-based video coding (NNVC)

- JVET-AJ2020 Film grain synthesis technology for video applications ed. 2 (Draft 1)

- JVET-AJ2021 Verification test plan for VVC multilayer coding (update 5)

- JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4)

- JVET-AJ2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AJ2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AJ2025 Algorithm description of Enhanced Compression Model 15 (ECM 15)

- JVET-AJ2026 Testing of video coding technology beyond conditions of exploration experiments

- JVET-AJ2027 Common test conditions for gaming applications

- JVET-AJ2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7), also issued as WG 5 CDTR N 323

- JVET-AJ2032 Technologies under consideration for future extensions of VSEI (version 6)

- JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding

- JVET-AJ2037 Report on subjective quality testing of the FGC SEI message, also issued as AG 5 N 140

b) documents produced as WG 5 documents only:

WG 5 N 318 Draft disposition of comments received on ISO/IEC 23002-7:202x/CDAM1

WG 5 N 320 Request for ISO/IEC 23008-2:202x (6th ed.) Amd.1 Additional profiles and SEI messages

WG 5 N 321 Disposition of comments received on ISO/IEC CD 23090-16:202x

WG 5 N 324 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 325 Liaison statement to DVB on film grain synthesis

WG 5 N 326 Liaison statement to ARIB, ATSC, SBTVD, and SCTE on film grain synthesis evaluation

WG 5 N 327 Liaison statement to ATSC on multi-layer coding

WG 5 N 329 Draft disposition of comments received on ISO/IEC 23090-3:202x/CDAM1

As main results, the JVET produced XX output documents from the current meeting (update after meeting):

- JVET-AJ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AJ1006 HEVC extensions and corrections

- JVET-AJ1011 HEVC white paper, also issued as AG 3 N 174

- JVET-AJ1012 Overview of IT systems used in JVET

- JVET-AJ2003 Guidelines for VTM-based software development

- JVET-AJ2005 Additions and corrections for VVC version 4 (Draft 10), also issued as WG 5 preliminary DAM N 330

- JVET-AJ2006 Additional SEI messages for VSEI version 4 (Draft 4), also issued as WG 5 preliminary DAM N 319

- JVET-AJ2007 Guidelines for NNVC software development

- JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2), also issued as WG 5 DIS N 322

- JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AJ2019 Description of algorithms version 9 and software version 11 in neural network-based video coding (NNVC)

- JVET-AJ2020 Film grain synthesis technology for video applications ed. 2 (Draft 1)

- JVET-AJ2021 Verification test plan for VVC multilayer coding (update 5)

- JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4)

- JVET-AJ2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AJ2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AJ2025 Algorithm description of Enhanced Compression Model 15 (ECM 15)

- JVET-AJ2026 Testing of video coding technology beyond conditions of exploration experiments

- JVET-AJ2027 Common test conditions for gaming applications

- JVET-AJ2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7), also issued as WG 5 CDTR N 323

- JVET-AJ2032 Technologies under consideration for future extensions of VSEI (version 6)

- JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding

- JVET-AJ2037 Report on subjective quality testing of the FGC SEI message, also issued as AG 5 N 140

- The following X documents were produced as WG 5 or SG21 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 275 Disposition of comments received on ISO/IEC DIS 14496-10:202x

WG 5 N 277 Request for ISO/IEC 23002-7:2023/Amd.1

WG 5 N 280 Disposition of comments received on ISO/IEC 23008-2:2023 (5th ed.) DAM 1

WG 5 N 282 Request for ISO/IEC 23090-3:2023/Amd.1

WG 5 N 285 Request for ISO/IEC 23090-16:202x

WG 5 N 287 Disposition of comments received on ISO/IEC DIS 23091-3:202X

WG 5 N 295, included in SG16-TD237/WP3, Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 296, included in SG16-TD237/WP3, Liaison statement to 3GPP SA 4 on feasibility study on film grain synthesis

WG 5 N 297, included in SG16-TD237/WP3, Liaison statement to ITU-R WP 6B on use cases of VVC multilayer profiles for broadcasting applications

WG 5 N 298, included in SG16-TD237/WP3, Liaison statement to ARIB, ATSC, DVB, SBTVD, and SCTE on film grain synthesis investigations

WG 5 N 299 List of AHGs established at the 15th WG 5 meeting

- The following X draft revised ITU-T Recommendations were forwarded by JVET and Q6/21 for ITU-T Consent:

TD276/Plen ITU-T H.264 (V15) "Advanced video coding for generic audiovisual services" (Rev.)

TD278/Plen ITU-T H.266.2 (V2) "Reference software for ITU-T H.266 versatile video coding" (Rev.)

TD279/Plen ITU-T H.265 (V10) "High efficiency video coding" (Rev.)

TD283/Plen ITU-T H.273 (V4) "Coding-independent code points for video signal type identification" (Rev.)

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, X Exploration Experiments (EE) were defined. The next eight JVET meetings were planned for 26 March – 4 April 2025 under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting; during 26 June – 4 July 2025 under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, KR; during 2 – 10 October 2025 under ITU-T SG21 auspices in Geneva, CH; during 14 – 23 January 2026 under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting; during 24 April – 1 May 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES; during July 2026 under ITU-T SG21 auspices, date and location t.b.d.; during October 2026 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during January 2027 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. It was noted that the previous sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ were shut down, but the most recent version of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its thirty-seventh meeting during 14–22 January 2025 meeting at the ITU headquarters facilities in Geneva, Switzerland. The meeting was held as a hybrid meeting, where remote participation was provided on best-effort basis for experts who were unable to travel.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, consisting of two parts, (B1) in-person attendees as recorded by a sign-in sheet circulated in meeting rooms, (B2) remote attendees as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2025_01_AK_Geneva/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the thirty-sixth JVET meeting in producing the following output documents:

a) JVET documents

- JVET-AJ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

- JVET-AJ1006 HEVC extensions and corrections

- JVET-AJ1011 HEVC white paper, also issued as AG 3 N 174

- JVET-AJ1012 Overview of IT systems used in JVET

- JVET-AJ2003 Guidelines for VTM-based software development

- JVET-AJ2005 Additions and corrections for VVC version 4 (Draft 10), also issued as WG 5 preliminary DAM N 330

- JVET-AJ2006 Additional SEI messages for VSEI version 4 (Draft 4), also issued as WG 5 preliminary DAM N 319

- JVET-AJ2007 Guidelines for NNVC software development

- JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2), also issued as WG 5 DIS N 322

- JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology

- JVET-AJ2019 Description of algorithms version 9 and software version 11 in neural network-based video coding (NNVC)

- JVET-AJ2020 Film grain synthesis technology for video applications ed. 2 (Draft 1)

- JVET-AJ2021 Verification test plan for VVC multilayer coding (update 5)

- JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4)

- JVET-AJ2023 Exploration experiment on neural network-based video coding (EE1)

- JVET-AJ2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

- JVET-AJ2025 Algorithm description of Enhanced Compression Model 15 (ECM 15)

- JVET-AJ2026 Testing of video coding technology beyond conditions of exploration experiments

- JVET-AJ2027 Common test conditions for gaming applications

- JVET-AJ2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7), also issued as WG 5 CDTR N 323

- JVET-AJ2032 Technologies under consideration for future extensions of VSEI (version 6)

- JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding

- JVET-AJ2037 Report on subjective quality testing of the FGC SEI message, also issued as AG 5 N 140

b) documents produced as WG 5 documents only:

WG 5 N 318 Draft disposition of comments received on ISO/IEC 23002-7:202x/CDAM1

WG 5 N 320 Request for ISO/IEC 23008-2:202x (6th ed.) Amd.1 Additional profiles and SEI messages

WG 5 N 321 Disposition of comments received on ISO/IEC CD 23090-16:202x

WG 5 N 324 Liaison statement to ISO/IEC JTC 1/SC 29/WG 1 (JPEG) on JPEG AI and explorations on video coding

WG 5 N 325 Liaison statement to DVB on film grain synthesis

WG 5 N 326 Liaison statement to ARIB, ATSC, SBTVD, and SCTE on film grain synthesis evaluation

WG 5 N 327 Liaison statement to ATSC on multi-layer coding

WG 5 N 329 Draft disposition of comments received on ISO/IEC 23090-3:202x/CDAM1

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the CEST timezone (local time in Geneva), except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 7 January 2025. Any documents uploaded after 1159 hours Paris/Geneva time on Wednesday 8 January 2025 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AK0239 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AK0XXX (a proposal on …), uploaded 01-XX,

- … .

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AK0XXX (a document describing …), uploaded 01-XX,

- … .

All cross-verification reports at this meeting were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AK0046, JVET-AK0051, JVET-AK0116, JVET-AK0120, JVET-AK0143, JVET-AK0144, JVET-AK0145, and JVET-AK0163.

The following cross-verification reports were still missing by the end of the meeting, but were uploaded later: JVET-AK0XXX, …. (not applicable for current meeting, kept for future use) The following reports had not become available yet three weeks after the end of the meeting: JVET-AK0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did apply to documents JVET-AK0077, JVET-AK0078, and JVET-AK0220 which were empty of results and were flagged as late in the list above, based on the time of the first reasonable document upload.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AJ1000, the Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP JVET-AJ1004, the HEVC extensions and corrections (draft 1) JVET-AJ1006, the White paper on HEVC JVET-AJ1011 (not delivered yet by beginning of the 37th meeting), the Overview of IT systems used in JVET JVET-AJ1012, the Guidelines for VTM-based software development JVET-AJ2003, the Additions and corrections for VVC version 4 (Draft 10) JVET-AJ2005, the Additional SEI messages for VSEI version 4 (Draft 4) JVET-AJ2006, the Guidelines for NNVC software development JVET-AJ2007, the Reference software for versatile video coding 2nd edition (Draft 2) JVET-AJ2009, the Common test conditions and evaluation procedures for neural network-based video coding technology JVET-AJ2016, the Description of algorithms version 9 and software version 11 in neural network-based video coding (NNVC) JVET-AJ2019, the Film grain synthesis technology for video applications ed. 2 (Draft 1) JVET-AJ2020 (not delivered yet by beginning of the 37th meeting), the Verification test plan for VVC multilayer coding (update 5) JVET-AJ2021, the Plan for subjective quality testing of the FGC SEI message (update 4) JVET-AJ2022, the Description of the EE on Neural Network-based Video Coding JVET-AJ2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AJ2024, the Algorithm description of Enhanced Compression Model 15 (ECM 15) JVET-AJ2025, the Testing of video coding technology beyond conditions of exploration experiments JVET-AJ2026, the Common test conditions for gaming applications JVET-AJ2027, the Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7) JVET-AJ2030, the Technologies under consideration for future extensions of VSEI (version 6) JVET-AJ2032, the Test conditions and evaluation procedures for generative face video coding JVET-AJ2035, and the Report on subjective quality testing of the FGC SEI message JVET-AJ2037 (not delivered yet by beginning of the 37th meeting), had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible. The software implementations of VTM version 23.5 and 23.6, ECM version 15.0, and NNVC version 11.0 were also approved.

Only minor editorial issues were found in the meeting report JVET-AJ1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site. On-site participants furthermore were required to register via the ITU-T website in order to get access to the headquarter facilities.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Adoption of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiments on neural-network-based video coding

Report of exploration experiments on enhanced compression beyond VVC capability

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were expected to be 0900-2000 CET with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was anticipated that some activities (such as breakout sessions) could be held at short notice.

## ISO and IEC Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that the previous document sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ were shut down, but the most recent version of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) will shortly be provided via the ITU ftp site, providing a “documents” subfolder in the directory of the respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1277 (as of 13 January 2025). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CIIP: Combined inter/intra prediction

- CIPF: CABAC initialization from the previous frame

- CL-RAS: Cross-layer random-access skip

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for neural network-based filter.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (initially for VVC, later expanded)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for neural network-based filter.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code.

- operation point: A temporal subset of an OLS.

- PDPC: Position-dependent (intra) prediction combination.

- PERP: Padded equirectangular projection (a 360° projection format).

- PH: Picture header.

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format).

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading (type of picture).

- RASL: Random-access skipped leading (type of picture).

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROI: Region of interest.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPL: Reference picture list.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SADL: Small adhoc deep learning library

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SCIPU: Smallest chroma intra prediction unit.

- SD: Slice data; alternatively, standard-definition.

- SDH: Sign data hiding.

- SDT: Signal-dependent transform.

- SE: Syntax element.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIF: Switchable (motion) interpolation filter.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPO: SEI processing order.

- SPS: Sequence parameter set (as in AVC and HEVC).

- SR: Super resolution .

- STMVP: Spatial-temporal motion vector prediction.

- STRP: Short-term reference picture.

- STSA: Step-wise temporal sublayer access.

- TBA/TBD/TBP: To be announced/determined/presented/public.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TIMD: Template-based intra mode derivation

- TM: Template matching.

- TMVP: Temporal motion vector prediction.

- TS: Transform skip.

- TSRC: Transform skip residual coding.

- TT: Ternary tree.

- UCBDS: Unrestricted center-biased diamond search.

- UGC: User-generated content.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VCM: Video coding for machines.

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VQA: Visual quality assessment.

- VT: Verification testing.

- VTM: VVC Test Model.

- VUI: Video usability information.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WGs of ISO/IEC JTC 1/SC 29).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Standards, TRs, supplements and technical papers approval and publication status (update)

- MPEG-2 | H.262 (coding specification is common text)

  - ITU-T H.262 V3 was approved in 2012-02; Amd.1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012, Cor 4:2011 may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) FDIS closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15.

    - H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13 (after the current meeting)

    - ISO/IEC 14496-10 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, pending FDIS ballot

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004, including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V7 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (shutter interval information SEI message and miscellaneous corrections), published 2020-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24.

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM1 issued 31st meeting 2023-07, DAM issued with new profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, pending FDIS ballot

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V4 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

  - ISO/IEC 23090-3:2024/CDAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, ready for action at the current meeting

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04.

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:202x (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, ready for action at the current meeting (Roughly corresponding H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12.

  - ISO/IEC 23002-7:202x (Ed. 3) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, ready for action at the current meeting

- CICP (twin text)

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:202x (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, pending FDIS ballot.

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128.

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07.

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08

- Usage of video signal type code points (twin text)

  - H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T provisional name H.Sup-FGST

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC 23888-3 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation further deferred due to meeting timing, ready for action at the current meeting

  - ITU-T provisional name H.Sup-MACVC

- The following freely available standards are published here in ISO/IEC:
https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of the time of the current meeting:

  - ISO/IEC 13818-4:2004 Conformance for MPEG-2

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2

  - Various amendments of ISO/IEC 14496-4:2004 Conformance for AVC

  - Various amendments of ISO/IEC 14496-5:2001 Reference software for AVC

  - ISO/IEC 14496-10:2022 (Ed. 10) AVC

  - ISO/IEC 23090-16:2022 (Ed. 1) Reference software for VVC

  - ISO/IEC 23091-2:2021 (Ed. 2) Video CICP

- The following standards that have been intended by JVET to be publicly available were not available at https://standards.iso.org/ittf/PubliclyAvailableStandards/index.html as of the time of the current meeting. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 23002-7:2024 (Ed. 3) – VSEI

  - ISO/IEC 23008-2:2023 (Ed. 5) HEVC, published 2023-10-30

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04.

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that it was never requested, e.g. for those that were produced by JCT-3V. This topic was left TBD until the next meeting – perhaps it would be best to compile a list of all relevant software and conformance parts of AVC, HEVC, MPEG-2 aka H.262, CICP, and request these in bulk.

## Draft standards progression status (update)

- VVC ISO/IEC 23090-3:2024/CDAM 1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, ready for action at the current meeting.

- VVC reference software ISO/IEC 23090-16:202x (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, ready for action at the current meeting. (Roughly corresponding H.266.2 V2 already approved and published in ITU-T.)

- VSEI ISO/IEC 23002-7:202x (Ed. 3) / CDAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, ready for action at the current meeting.

- Film grain synthesis technology for video applications – JVET draft 4 and the ISO/IEC 23002-9 CDTR were issued at the 29th meeting (JVET-AC2020) (a request to start work on the TR had been made at the 25th meeting), and the CDTR consultation period ended 2023-07-09. A DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24. ITU-T provisional name H.Sup-FGST. ITU-T approval delayed to January 2025 due to the delay on the ISO/IEC side.

- Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation further deferred due to meeting timing, ready for action at the current meeting. ITU-T provisional name H.Sup-MACVC.

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the following parts:

  - For the ongoing work items, when they become finalized

  - ISO/IEC 23008-2:2020/Amd.1:2021 – HEVC FDAM issued 20th meeting (October 2020), public availability not yet requested but may not be necessary as it becomes included in next edition

## Opening remarks

Remarks during the opening session of the meeting Tuesday 14 January at 0930 CET were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was slightly lower than in last meeting (approximately 205 vs. 220 by the time of opening the meeting) – parallel sessions were announced to be necessary.

  - Evening sessions may also be necessary, typically expecting to meet until around 2000.

  - Parallel sessions starting the afternoon of the first day (EE and HLS), and probably continuing until the weekend.

  - Friday 1400-1830: Joint ITU/ISO workshop on “Future video coding – advanced signal processing, AI and standards” – no JVET meeting in parallel with that, but sessions may resume thereafter. Separate registration is necessary for the workshop.

- In the context of registration for JVET in the ITU website, it was found that some experts (also some who had participated in-person in the previous meeting) were neither listed as WG 5 nor as ITU members. In most cases this could be clarified, confirming that the process of becoming member of one of the parent bodies was experiencing delays. It will be necessary to enforce this check for qualification more severely in the future, also for online participants.

- It was noted that the JVET document template has a new header with WP3/21 as ITU parent body. Roughly half of the uploaded inputs obviously used own edits of old documents with the old header.

- Plans for subsequent hybrid meetings (with best-effort remote access) were reviewed:June/July 2025 (Daejeon), Oct. 2025 (Geneva), April 2026 (Santa Eulària), June/July 2026 (Geneva, dates to be clarified).

- March/April 2025 and January 2026 meetings will be virtual. Considering the rigid time constraints of virtual meetings, and considering the increased number of input documents since the last virtual meeting in January 2024, it may be necessary to extend them by one or two days, or meet over weekends.

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, anyway (both for hybrid and virtual)

- Significant workload can be expected at this meeting for AHG17 activities – possible need of having a hybrid AHG meeting in Aachen during second week of March

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing. In the meeting room C, it is also possible to show them on a separate projection screen in parallel with another presentation.

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AJ1000 were reviewed. Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, on-site attendance will be recorded via the traditional sign-in sheet. Participants were asked to correct their affiliation and email in case these changed.

- All cross-check documents were, but number of late non-cross-check documents seemed to have slightly increased relative to the last meeting. Several contributions did not report any results initially, and were also flagged as late. There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a few documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions.

- Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

- Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs (as that feature of the DMS site has not been working properly).

- Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- The practice introduced in the last meeting, submitting WG 5 N-numbered output documents only in cases of standards text submitted for ballot, DoCs, standards/parts requests, and meeting reports has turned out to be useful. It is basically the same practice that had always been exercised on the ITU side.

- Document JVET-AJ1012 was produced, which includes the changes in the sites for hosting the JVET test sequences, as well as the new location of JCT-VC and JCT-3V documents. Only the newest versions of the JCT-VC and JCT-3V documents are available from the links in the JVET site, but it is still planned to provide all approved versions in the future (although this may take some time). During or after the current meeting, It is further planned to set up a second source for JVET, JCT-VC and JCT-3V docs in the ITU ftp directories of each meeting. This will include all versions of documents with original upload times.

- The following ballot results had become available through the SC 29 secretariat (kept for future use).

  - …

- DAM/DIS for VVC and VSEI could be done at the current meeting, but no ballot results could become available in time for the March/April meeting. The FDAM/FDIS could be in July 2025 at the earliest. This might be achieved with a very long editing period (potentially reaching into the next meeting). Another option would be to target the 4th editions for October 2025, both for ITU-T and ISO/IEC, and send them to ballot early in April, such that still improvements might be made during the March/April meeting.

- The CDTR on machine analysis was submitted, but it would be necessary that the VSEI v4 Amd. reaches DAM/DIS stage before it is promoted to TR (as SEI messages in VSEI v4 are referred to in the draft of the TR).

- DIS of VVC software was submitted for ballot – matching the ITU edition that was issued in April. A next edition could be targeted for October, considering implementations of new SEI messages.

- The primary goals of the meeting were:

  - Film grain TR to be submitted for ITU approval – which version? V1 with corrections/improvements, to be taken from JVET-AJ2020

  - New versions of VVC and VSEI (JVET-AK2005/2006, <preliminary> DAM texts, not for ITU consent yet)

  - In the last meeting, a new edition of HEVC software was considered to be possible at the current meeting. This would however depend on the implementation status of the software for the new multiview profiles (provided they would be complete).

  - Any action items on reference software JM?

  - Lot of activity in AHG17 – planning of viewing sessions needed.

  - Any viewing for film grain and multi-layer?

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

  - Any liaison communication (through ITU)?

- New training materials for NNVC – extended BVI set.

- It was suggested that the VVC white paper could benefit from an update, and generating a separate one on VSEI would be useful – first draft in the current meeting?

- Joint meetings were expected with MPEG AG 5 (on matters involving visual quality assessment), with MPEG WG 2 Requirements and ITU-T VCEG on future video standardization. Joint meetings will be held in ITU premises, not in Crowne Plaza Hotel where other MPEG WGs will meet. (see section 7.3).

- JVET cannot meet during plenaries of the ITU parent body (and will need to finish by Wednesday next week around lunchtime). Due to time constraints, JVET sessions will be held during MPEG WGs’ information exchange meetings on Monday morning and Wednesday morning.

- As a follow-up to liaison communication after the April meeting, parent bodies should conduct discussion about future JVET management structures. SC 29 had requested all MPEG WGs to appoint convener support teams by January 2025 at latest.

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.12.

## Scheduling of discussions

- The times of the meeting sessions followed the needs of the onsite meeting arrangements, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were expected to be in the range of 0900-2000 CET with coffee breaks and lunch breaks as appropriate. Sessions were announced in the JVET calendar and the ITU schedule system as far as possible in advance, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timezone of the meeting venue, except as otherwise noted:

- Tue. 14 Jan., 1st day

  - Morning session:

    - 0930–1030 Opening remarks, review of practices, agenda, IPR policy reminder

    - 1030–1315 Reports of AHGs 1-11

  - Afternoon sessions:

    - 1430–1545 Reports of AHGs 12-17

    - 1600–2000 EE1/EE2 review (room C)

    - 1530–XXXX HLS (room A, chaired by Jill Boyce)

- Wed. 15 Jan., 2nd day

  - Morning sessions:

    - …

  - Afternoon sessions:

    - …

- Thu. 16 Jan., 3rd day

  - Morning sessions:

    - …

  - Afternoon sessions:

    - …

- Fri. 17 Jan., 4th day

  - Morning sessions:

    - …

  - 1400–1830 Workshop on “Future video coding – advanced signal processing, AI and standards” – announcement/registration on the ITU website

  - Evening sessions:

    - …

- …

- Wed. 22 Jan., 8th day

  - 0900–XXX Wrapup plenary:

    - Remaining revisits

    - EE descriptions, DoCRs

    - BoG reports

    - Establishment of AHGs

    - Review and approval of output docs

    - Software timelines

    - Approval of WG 5 meeting recommendations

    - Future planning, a.o.b.

## Contribution topic overview (update categories and numbers)

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (16) (section 3)

- Project development (section 4)

  - AHG1: Development, deployment and advertisement of standards (6)

  - AHG2: Text development and errata reporting (2)

  - AHG3: Test conditions (0)

  - AHG3: Software development (2)

  - AHG4: Subjective quality testing and verification testing (2)

  - AHG4: Test and training material (3)

  - AHG4: Codec performance with alternative test material and non-CTC conditions (0)

  - AHG5: Conformance test development (0)

  - AHG7: ECM tool assessment (2)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (7)

  - AHG10: Encoding algorithm optimization (3)

  - AHG13: Film grain synthesis (0)

  - Implementation studies (3)

  - Profile/tier/level specification (3)

  - Gaming content compression (5)

  - Generative face video (1)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (26) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (94) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - SEI messages in VSEIv4 (32) (section 6.1)

  - SEI messages in TuC doc (29) (section 6.2)

  - SEI messages on other topics (4) (section 6.3)

  - Non-SEI HLS aspects (0) (section 6.4)

- Joint meetings, plenary discussions, BoG reports (2) liaison (3), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (16)

These reports were discussed during 1030–1315 and 1430–1545 on Tuesday 14 Jan. 2025 (chaired by JRO).

JVET-AK0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice chair)]

The reflector used for discussions by the JVET and all of its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

The number of subscribers (on the day before the beginning of the current meeting) was 1277 (compared to 1270 by the time of the previous meeting). Furthermore, the previous lists of joint teams (which were still kept open as archives) had the following number of subscribers:

- JCT-VC – 1158 subscribers

- JCT-3V – 680 subscribers

- JVT-experts – 2072 subscribers

It is likely that many subscriptions of these three reflectors have become obsolete, as no emails are being sent over them, and therefore automatic removal of participants causing permanent bounces through invalid email addresses (as with the main JVET reflector) cannot be executed by the postorius management system.

- Goals and activity

The work of the JVET overall had proceeded well in the interim period with higher number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and most output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET ftp site (http://wftp3.itu.int/av-arch/jvet-site/2024_11_AJ_Kemer/). It is noted that the previous document sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ were shut down, but JCT-VC and JCT-3V documents can now be accessed directly via the JVET site. During or shortly after the 37th meeting, all documents of JCT-VC, JCT-3V and JVET will also be made available from the ITU-based ftp site, where sub-folders ‘./documents/’ are planned to be created in the folders of the respective meetings. This will provide the files with the original time stamps when the original versions of documents were uploaded.

The list of output documents produced since the last meeting included the following, particularly:

- JVET-AJ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Posted 2025-01-06]

- JVET-AJ1006 HEVC extensions and corrections [Posted 2024-12-02]

- JVET-AJ1011 HEVC white paper, also issued as AG 3 N 174 [Posted 2025-01-XX]

- JVET-AJ1012 Overview of IT systems used in JVET [Posted 2025-01-06]

- JVET-AJ2003 Guidelines for VTM-based software development [Posted 2025-01-12]

- JVET-AJ2005 Additions and corrections for VVC version 4 (Draft 10), also issued as WG 5 preliminary DAM N 330 [Posted 2024-11-26, last update 2024-12-30]

- JVET-AJ2006 Additional SEI messages for VSEI version 4 (Draft 4), also issued as WG 5 preliminary DAM N 319 [Posted 2025-12-07, last update 2024-12-18]

- JVET-AJ2007 Guidelines for NNVC software development [Posted 2024-12-04]

- JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2), also issued as WG 5 DIS N 322 [Posted 2024-12-30]

- JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology [Posted 2024-12-04]

- JVET-AJ2019 Description of algorithms version 9 and software version 11 in neural network-based video coding (NNVC) [Posted 2025-01-08]

- JVET-AJ2020 Film grain synthesis technology for video applications ed. 2 (Draft 1) [Posted 2025-01-XX]

- JVET-AJ2021 Verification test plan for VVC multilayer coding (update 5) [Posted 2025-01-13]

- JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4) [Posted 2025-01-13]

- JVET-AJ2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2024-11-08, last update 2024-12-12]

- JVET-AJ2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [Posted 2024-11-08, last update 2024-12-11]

- JVET-AJ2025 Algorithm description of Enhanced Compression Model 15 (ECM 15) [Posted 2025-01-09]

- JVET-AJ2026 Testing of video coding technology beyond conditions of exploration experiments [Posted 2024-12-03]

- JVET-AJ2027 Common test conditions for gaming applications [Posted 2024-11-22]

- JVET-AJ2030 Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7), also issued as WG 5 CDTR N 323 [Posted 2024-12-31]

- JVET-AJ2032 Technologies under consideration for future extensions of VSEI (version 6) [Posted 2024-12-06, last update 2024-12-07]

- JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [Posted 2024-11-26, last update 2024-12-09]

- JVET-AJ2037 Report on subjective quality testing of the FGC SEI message, also issued as AG 5 N 140 [Posted 2025-01-XX]

The seventeen ad hoc groups had made progress, and reports from those activities had been submitted. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability were conducted.

The arrangements for the 37th meeting had been announced in the JVET reflector, in the JVET logistics document (https://www.itu.int/wftp3/av-arch/jvet-site/2025_01_AK_Geneva/JVET-AK_Logistics.docx), and in the WG 5 calling notice (N 331) and agenda (N 332) for the 18th WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Roughly 205 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 37h meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2025_01_AK_Geneva/JVET-AK_notes_d0.docx.

- Recommendations

- The AHG recommends its continuation.

- The AHG recommends communication between JVET’s parent bodies about appointment of an extended management support team.

JVET-AK0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

Ad hoc group activity

  - Output documents produced

    - JVET-AJ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP

This document contains a list of reported errata items for VVC, VSEI, HEVC, AVC, and Video CICP, for tracking purposes. Some of the items have been confirmed by the JVET and have been agreed to require fixing. This document also provides publication status backgrounds of these standards.

Incorporated items at the JVET-AJ meeting:

- For HEVC:

  - Removed three items that have been addressed by the following integrations in JVET-AJ1006:

    - JVET-AJ0213 On Multiview Main 10 profiles

    - Bug fix: The variable subBitstream was used without being defined for the Multiview Extended or Multiview Extended 10 profiles.

    - Bug fix: Change the general_profile_idc value for the multiview format range extension profiles from 10 to 14, to differ from that for the multiview format range extension profiles.

- For VVC:

  - Spec ticket #1650

    - JVET-AJ2005 Additions and corrections for VVC version 4 (Draft 10)

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Changes to be integrated or checked:

- Elements from JVET-AG1004 (errata), and bug fixes from JVET-AH0002 for tickets #1609 (NoBackwardPredFlag derivation ambiguity), #1617 (Not initialized NumCtusInSlice[ 0 ] to 0), #1624 (Incorrect indexing in computation of motion vector offset), #1628 (Derivation of ModeTypeCondition should say “one or more”), #1630 (Missing equations for applying AmvrShift), #1631 (Should “Motion vector storing process for geometric partitioning mode” store HpelIfIdx? BBross: HpekIfIdx should always be written since it might be accessed later so it is definitely missing in "Motion vector storing process for geometric partitioning mode".), #1632 (Incorrect indexing used for choosing matrix intra sample prediction), #1634 (Matrices QStateTransTable,levelScale,AlfFixFiltCoeff,AlfClassToFiltMap are incorrectly transposed).

Changes that have been integrated for the 36th JVET meeting in November 2024:

- JVET-AJ0048 On SeiProcessingOrderSeiList and SpoProcessSeiList

- JVET-AJ0049 NNPFC and NNPFA interface text in VVC

- JVET-AJ0129 Miscellaneous comments on the SPO SEI message

- JVET-AJ0151 Move digitally signed content SEI messages to VSEI draft

- JVET-AJ0207 Move GFV and GFVE to VSEI draft

- JVET-AJ0114 On signalling input pictures shift in NNPFA SEI message (the aspect regarding the establishment of input picture lists for NNPF)

    - JVET-AJ2006 Additional SEI messages for VSEI version 4 (Draft 4)

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including encoder optimization information, source picture timing information , object mask information, modality information, text description information, generative face video, generative face video enhancement, digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages and updates to the neural-network post-filter characteristics SEI message.

Editors’ notes and changes yet to be integrated:

- All automatically generated numbering and cross-reference fields will need to be updated after the changes are integrated into the basis text.Check the use of “pertain”; the word is not used in a similar way anywhere in the existing standard.payloadType values are to be specified the corresponding VVC, HEVC or AVC draft text. The use of prefix SEI NAL unit was enabled for all new SEI messages, whereas the use of suffix SEI NAL unit was enabled for encoder optimization information and object mask information SEI messages, since their content may be determined while the encoder encodes the associated picture.

- (JVET-AH0121 item 5) check/fix the ChromaFormatIdc derivation for the use of the colour transform information SEI message as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027 (to carefully check and communicate offline for correctness, e.g., w.r.t. JVET-AH0047).

- (JVET-AH0121 item 6) check/fix the interface variable derivation for the use of the NNPFC and NNPFA SEI messages as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027 (to carefully check and communicate offline for correctness, e.g., w.r.t. JVET-AH0047).

To be integrated:

  - General editorial improvementsJVET-AJ0185 Editorial updates for NNPFC, SPO and TDI SEI messages

    - Item 2

Changes that have been integrated:

A description of the contributions reflected in this document relative to JVET-AI2006:

- General editorial improvements

  - JVET-AJ0050 Editorial changes to VSEI draft text

  - JVET-AJ0065 Comments On VSEI Version 4

  - JVET-AJ0070 Editorial and technical problems in the draft of VSEI v4 and their solutions

  - JVET-AJ0120 Miscellaneous modifications for SEI messages in the VSEI draft

  - JVET-AJ0185 Editorial updates for NNPFC, SPO and TDI SEI messages

    - Item 1, 3, 4, 5, and 6.

- SEI Processing Order SEI

  - JVET-AJ0045 Handling of processing chains - operations

  - JVET-AJ0046 Handling of processing chains - constraints and special cases

  - JVET-AJ0047 Merging NNPF filtering process into processing chain handling

  - JVET-AJ0048 On SeiProcessingOrderSeiList and SpoProcessSeiList

  - JVET-AJ0128 Adding functionality to po_processing_degree_flag[ i ] of the SPO SEI message

  - JVET-AJ0129 Miscellaneous comments on the SPO SEI message

  - JVET-AJ0105 On signalling of complexity information in SEI processing order SEI message

- Neural Network Post Filter

  - JVET-AJ0049 NNPFC and NNPFA interface text in VVC

  - JVET-AJ0073 A clean-up for the spatial resampling NNPF design

  - JVET-AJ0058 Generalizing text prompt usage in the NNPFC SEI message

  - JVET-AJ0131 On text prompts for NNPF

  - JVET-AJ0234 On Spatial Extrapolation for NNPF

  - JVET-AJ0104 On signalling of prompt in NNFPA SEI message

  - JVET-AJ0114 On signalling input pictures shift in NNPFA SEI message

- Encoder optimization information SEI

  - JVET-AJ0063 On Encoder Optimization Information SEI

  - JVET-AJ0183 Signaling source and added pictures in EOI SEI message

- Source picture timing SEI

  - JVET-AJ0308 Leading pictures and the SPTI SEI message

  - JVET-AJ0252 On SPTI SEI message

- Text description information SEI

  - JVET-AJ0184 On signaling of cancellation, persistency and id in TDI SEI messages

  - JVET-AJ0241 Text description SEI purpose for encoder description

- Generative Face Video and Generative Face Video Enhancement SEI message

  - JVET-AJ0207 Move GFV and GFVE to VSEI draft

  - JVET-AJ0051 On the GFV SEI message

  - JVET-AJ0101 Miscellaneous modifications for SEI messages in the TuC for futhre exensions of VSEI (GFV aspects)

  - JVET-AJ0108 On signalling of instance count in generative face video SEI message

  - JVET-AJ0135 Refined Methodology for Pupil Position SEI Message for Generaive Face Video

- Digitally signed content SEIs

  - Move to VSEI draft (see notes for JVET-AJ0151)

- Neural Network Post Filter

  - JVET-AJ0142 On potential conflicts between NNPFs and other post-processings

Related input contributions

Input contributions were noted as relevant to the work of this ad hoc group:

- (none)

Remaining VVC bug tickets

Closed since JVET-AJ0002 was reported:

- #1618 [Multilayer Profiles] Potential mismatch of VTM22.0 & specification related to derivation process for merge motion vector difference.

- #1637 The wording of H.266 B.3 seems to require byte streams to start with at least three zeros.

Carried over (strikethough means addressed in the indicated errata report and can be closed):

- #1609 NoBackwardPredFlag derivation ambiguity (Addressed in JVET-AI1004)

- #1617 Not initialized NumCtusInSlice[0] to 0. (Addressed in JVET-AI1004)

- #1624 Incorrect indexing in computation of motion vector offset. (Addressed in JVET-AI1004)

- #1627 "Decoding process for palette mode" does not say what to do with output samples. (Discussed in JVET-AI1004)

- #1628 Derivation of ModeTypeCondition should say "one or more". (Addressed in JVET-AI1004)

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType. (Discussed in JVET-AI1004)

- #1630 Missing equations for applying AmvrShift. (Discussed in JVET-AI1004)

- #1631 Should "Motion vector storing process for geometric partitioning mode" store HpelIfIdx? (addressed in JVET-AI1004)

- #1632 Incorrect indexing used for choosing matrix intra sample prediction. (Addressed in JVET-AI1004)

- #1634 Matrices QStateTransTable, levelScale, AlfFixFiltCoeff, AlfClassToFiltMap are incorrectly transposed. (Addressed in JVET-AI1004)

- #1635 Incorrect inference for tu_y_coded_flag. (Discussed in JVET-AI1004)

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

New (since JVET-AJ0002 was reported)

- #1650 Incorrect indexing for the h location component in ALF process. (Discussed in JVET-AJ1004)

- #1651 References to various non-existent syntax elements relating to sps_partition_constraints_override_enabled_flag.

Remaining HEVC bug tickets

Closed since JVET-AJ0002 was reported:

- (none)

Carried over:

- #1427 (8-155) and (8-157) do not seem to be used

- #1491 Duplicate invocation of 9.3.4.3 arithmetic decoding process

- #1498 Typos in the Table 9-43

- #1500 Typo in equation (8-69),(8-70)

- #1504 Small typos in profile_tier_level syntax in tabular form (7.3.3)

- #1505 Misleading bitstream requirement related to EOB NAL unit

- #1507 Duplicate row entries for CU QP delta syntax elements in Table 9-48

- #1520 Some smaller errors in the multiview spec

- #1522 Offset issue in clause 8.5.4.3

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

New (since JVET-AI0002 was reported):

- (none)

Recommendations

The AHG recommends to:

- Approve JVET-AJ1004, JVET-AJ2005 and JVET-AJ2006 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2 related inputs and act on them if found to be necessary.

It was agreed to do fixes to HEVC in the next editions (version 11 in ITU targeted for October), and in the ISO amendment on top of edition 6 requested in last meeting (other items are related to new SEI messages in VSEI). It was agreed to issue CDAM at this meeting (short editing period) and DAM in April. This may require leaving out interfaces with some VSEI parts, which could be added at DAM.

This would also have the advantage of keeping timing of all three standards (HEVC, VVC, VSEI) aligned.

Edition 6 FDIS at this moment is aligned with ITU H.265 version 10 that was approved after the April meeting. Both have some errors in the multi-view part.

JVET-AK0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

The software model versions prior to the start of the meeting were:

- VTM 23.6 (Nov. 2024)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.24 (March 2023)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

- Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

Between the 31st and 32nd meetings, the GitLab server was affected by a huge number of spam issues, which were generated using newly registered accounts. Administrator user account confirmation was enabled to protect the server from spam postings. Proponents should plan for some time for server admins to confirm new accounts.

- VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued on the GitLab server. VTM version 23.5 was tagged on Nov. 8, 2024. VTM 23.7 is expected during the 37th JVET meeting.

VTM 23.5 was tagged on Nov. 8, 2024. Changes include:

- Fix Scalable Nesting SEI parsing

- Fix Multilayer decoding with wraparound

- Fix invalid vector indexing

- Explicit configuration of inter-layer reference pictures

- FIX input file with rescaling (multilayer scalable) when using "SourceScalingRatioHor/Ver" encoder option

- Fix GCC 14 warning: use of uninitialized variable

- Fix uninitialized and cleaning

- JVET-AH2006: Text Description Information SEI message

- Make RdCost members non-`static` to use correct weighting tables for each layer in multilayer encoding

- JVET-AI0073: On the handling of a processing chain specified by an SPO SEI message

- Override SPS using --UpscaledOutputWidth and --UpscaledOutputHeight when upscaling output...

- JVET-AI0071: Indication of NNPFs and processing chains for human viewing and/or machine consumption

- JVET-AI0180: Adding original source picture dimensions to EOI SEI

- Move macro definition into appropriate location

- Add NNPFC SEI message support of spatial extrapolation in JVET-AH0174 and JVET-AI0202.

- add MS-SSIM2 for multilayer with reference layer (scalable)

- JVET-AI0153: Object Mask Information SEI

- optimization of RdCost::getWeightedMSE() in case of constant weight

- Workaround for VLCReader::xReadString for Visual C++

- JVET-AI0059: Define persistence scope for text description information SEI to be purpose specific

- JVET-AI0061-PROPOSAL2: Ensure spatial extrapolation works correctly (Option 1)

- Fix #1643 on different pps.conformanceWindows when using --UpscaledOutput=1 with RPR

- Clean up unregistered user data SEI

- Fix #1647 on adding HMVP candidates to AMVP list (multilayer with >2 layers)

- JVET-AG0322: Modality Information SEI

- JVET AI0061 Proposal 1: signal a NNPFC prompt for spatial extrapolation

- Fix #1646 on the condition of using tempMV for SbTMVP

- Fix #1645 on sbtmvp enabling condition in CU-level in case of multilayer profile.

- Fix #1649 on rprConstraintsActiveFlag

- Fix a regression in RPL parsing in Multilayer case

- Fix #1648 on sps_chroma_horizontal/vertical_collocated_flag usage in case of multilayer

- Define default value for Picture::m_numSubPic

- Initialize m_numSubPic for unavailable pictures.

VTM 23.6 was tagged on Nov. 11, 2024. Changes only include removal of macros from previous meeting cycles.

VTM 23.7 is expected to be tagged during the 37th JVET meeting. Changes are expected to include:

- Take bit depth into account when applying temporal prefilter

- JVET-AJ0067: AHG10: Suggested updates of settings for scalable VVC coding

- JVET-AJ0225: Changes to RDO for enhancement layer

- JVET-AJ0129: Add more SEI types to the list of allowed SEI types in SPO SEI

- JVET-AJ0241: Text description SEI purpose for encoder description

- Fix GCC 14 warning

- JVET-AJ0207: GFV SEI message

- JVET-AJ0063: Proposal 1,2,4,5

- Port ET with millisecond precision (from ECM MR 803)

- JVET-AJ0151: Digitally Signed Content SEI messages

- fix ScalingList for multilayer, when used for only one layer that is not the...

- JVET-AJ0131: Add nnpfc_inband_prompt_flag to gate NNPF text prompt

- Fix for decoder when DSC SEI is not present

- JVET-AJ0128: Adding functionality to po_processing_degree_flag[i] of the SPO SEI message

- JVET-AJ0234: Allow for patch-wise operation when using spatial extrapolation

In addition, there are pending MRs on the fixes of multi-layer coding. It may be good to also have some tests to verify the fixes.

  - CTC Performance

VTM 23.6 shows identical performance compared to VTM 23.4 in SDR CTC.

For the HDR CTCs, coding performance of VTM 23.6 and VTM 23.4 are identical. Encoding/decoding run times are very close.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-23.5 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

With a bug fix of temporal filtering (merged shortly after VTM 23.6), VTM shows good improvement with option --InternalBitDepth=12. Note that the runtime is not reliable.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.6-d25697cc5 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR |
| Class A1 | -0.52% | -0.84% | -0.61% | Not reliable |  | 101% |
| Class A2 | -0.37% | 0.39% | 0.68% |  |  | 100% |
| Class B | -0.21% | 0.29% | 0.22% |  |  | 100% |
| Class C | -0.13% | 0.73% | 0.67% |  |  | 100% |
| Class E | -0.24% | 0.29% | -0.02% |  |  | 100% |
| Overall | -0.28% | 0.22% | 0.22% |  |  | 100% |
| Class D | 0.08% | 0.95% | 0.73% |  |  | 100% |
| Class F | -0.14% | -0.03% | 0.19% |  |  | 100% |
| Class TGM | -0.34% | 0.21% | 0.19% |  |  | 100% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-23.6-d25697cc5 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR |
| Class A1 | -0.85% | -1.14% | -0.32% | Not reliable |  | 100% |
| Class A2 | -0.55% | -0.31% | -0.64% |  |  | 100% |
| Class B | -0.40% | 0.33% | 0.01% |  |  | 100% |
| Class C | -0.36% | 1.42% | 0.93% |  |  | 100% |
| Class E |  |  |  |  |  |  |
| Overall (Ref) | -0.51% | 0.20% | 0.06% |  |  | 100% |
| Class D | -0.19% | 0.74% | 0.41% |  |  | 100% |
| Class F | -0.47% | 0.17% | 0.30% |  |  | 100% |
| Class TGM | -0.46% | 0.25% | 0.33% |  |  | 100% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-23.6-d25697cc5 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.52% | -0.80% | -1.01% | Not reliable |  | 105% |
| Class C | -0.35% | 0.57% | 0.35% |  |  | 99% |
| Class E | -0.64% | -0.75% | -0.67% |  |  | 100% |
| Overall (Ref) | -0.49% | -0.33% | -0.48% |  |  | 102% |
| Class D | -0.26% | 1.16% | 0.53% |  |  | 100% |
| Class F | -0.35% | -0.24% | -0.18% |  |  | 101% |
| Class TGM | -0.29% | 0.06% | 0.30% |  |  | 99% |
|  | Low delay P Main10 |  |  |  |  |  |
|  | Over VTM-23.6-d25697cc5 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.44% | -0.86% | -1.32% | Not reliable |  | 89% |
| Class C | -0.31% | 0.68% | 0.19% |  |  | 101% |
| Class E | -0.90% | -0.79% | -1.16% |  |  | 100% |
| Overall (Ref) | -0.51% | -0.33% | -0.78% |  |  | 96% |
| Class D | -0.27% | 0.29% | -0.18% |  |  | 100% |
| Class F | -0.30% | -0.73% | 0.14% |  |  | 100% |
| Class TGM | -0.34% | 0.08% | 0.14% |  |  | 97% |

For the high bit depth CTCs, VTM 23.6 also shows no changes in coding performance compared to VTM 23.4.

  - Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

- Missing HLS features (see sections below)

  - Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

  - SEI TuC software

Per the decision during the 32nd JVET meeting, an SEI TuC repository was created based on VTM-22.2. The repository is located at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

VTM-22.2-TuC-2.0 was tagged on July 17, 2024. Changes include:

- Remove unused variables

- JVET-AG0044: Copyright information SEI message

- JVET-AG0045: AI information SEI message

- JVET-AF0141: EXIF, JFIF and XMP metadata SEI messages

VTM-22.2-TuC-3.0 was tagged on August 2nd, 2024. Changes include:

- JVET-AH0162: Constituent Rectangles SEI

- JVET-AH0161: Packed Regions Info SEI

- JVET-AH0164: Quality metric SEI

VTM-22.2-TuC-4.0 was expected to be tagged during the 36th JVET meeting. The release was delayed due to pending merge request. It is now expected to be tagged during the 37th JVET meeting. The following merge requests were submitted:

- JVET-AI0099-SW: Implementation of adaptive film grain models in different regions (No change in TuC software. Experiment was run using external software. To be closed.)

- JVET-AI0195: GFV SEI message (comments pending)

- JVET-AI0182: Bitdepth range information SEI (review pending)

- Fix typos in constituent rectangles implementation (review pending)

- JVET-AI0194: GFV chroma key (based on JVET-AI0195 – review pending)

- JVET_AI0184_0189_0190_0191_GFV_SEI (based on JVET-AI0195 – review pending)

- JVET-AI0127: Digitally Signed Content SEI messages (review pending)

- JVET-AI0181: Support for Display Overlays Information (DOI) SEI (minor comments pending)

The MRs for JVET-AI0195, JVET-AI0194, JVET-AI0184 and JVET-AI0127 were closed after adoption of the related technologies into VTM and the merge requests for VTM are available.

- HM related activities

There had not been any further developments to HM during this meeting cycle.

The following MRs are pending [with status indicated]:

- Implement phase indication SEI message (JVET-AE0101) [waiting review]

- Port the Y4M support [one issue remains]

- Mark the current picture as short-term ref (for SCM) [need SCC expert reviewer]

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Otherwise the HEVC bug tracker lists:

- 44 tickets for “HM”, (most of which were created before 2019; 1 created in last year),

- 1 ticket for “HM RExt” (created in 2020),

- 9 tickets for “HM SCC” (most of which were created before 2018),

- 1 ticket for “RExt Text” (created in 2015),

- 1 ticket for “SCC Text” (created in 2016),

- 8 tickets for text (6 created before 2021; 2 created in the last year),

- 2 tickets for encoder description (created in 2020 and 2014)

Help to address these tickets would be appreciated.

- 360Lib related activities

The latest 360Lib software can be found at https://vcgit.hhi.fraunhofer.de/jvet/360lib/-/tags/360Lib-13.6

The following table is for the projection formats comparison using VTM-23.0 according to 360-degree video CTC (JVET-U2012) compared to that using VTM-22.0 (VTM-22.0 as anchor).

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | 0.00% | -0.01% | 0.05% | 0.00% | 0.00% | 0.05% |
| Class S2 | 0.08% | 0.16% | 0.34% | 0.08% | 0.15% | 0.34% |
| Overall | 0.03% | 0.06% | 0.17% | 0.03% | 0.06% | 0.16% |

The following table compares generalized cubemap (GCMP) coding and padded equi-rectangular projection (PERP) coding using VTM-23.0 (PERP as anchor).

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.52% | -5.47% | -6.09% | -11.50% | -5.41% | -6.03% |
| Class S2 | -3.65% | 0.98% | 1.48% | -3.62% | 1.08% | 1.56% |
| Overall | -8.37% | -2.89% | -3.06% | -8.35% | -2.82% | -3.00% |

The following tables are for PERP and GCMP coding comparison between VTM-23.0 and HM-16.22 (HM as anchor), respectively.

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -30.91% | -38.82% | -41.24% | -30.91% | -38.88% | -41.24% |
| Class S2 | -36.84% | -37.23% | -39.43% | -36.82% | -37.27% | -39.49% |
| Overall | -33.28% | -38.19% | -40.51% | -33.28% | -38.23% | -40.54% |

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -34.64% | -40.04% | -42.14% | -34.59% | -40.02% | -42.09% |
| Class S2 | -38.34% | -38.18% | -40.06% | -38.32% | -38.19% | -40.11% |
| Overall | -36.12% | -39.30% | -41.31% | -36.08% | -39.29% | -41.30% |

- SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

- SHM related activities

There had not been any further developments to SHVC’s SHM during this meeting cycle.

- HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

One merge request is was merged:

- JVET-AE0295: MV Main 10 profile support (waiting for proponent response)

The next release will also include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

- HDRTools related activities

There had not been any further developments to HDRTools during this meeting cycle.

- JM, JSVM, JMVM related activities

There had not been any further developments to JM, JSVM or JMVM during this meeting cycle.

For the previous release of JM only basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch was reported privately, but needs more investigation.

- Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

- CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-Y2010		VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018	VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100	HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009	SHVC test conditions

JVET-AC1015	SCM test conditions

JVET-AE1013		3DV test conditions

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

- Guidelines for reference software development

An updated version of VTM-based software guidelines is proposed in JVET-AJ0187.

- Recommendations

The AHG recommends to:

- Continue to develop reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bit-streams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

It was noted that no merge request was received on the new MV-HEVC profiles in combination with RExt tools, though there was an input report in the last meeting. This also has impact on development of conformance for these profiles.

It was commented that the origin of this 12-bit change was a normative ECM proposal, which also fixed the MCTF bug in ECM.

Coffee break until 11:35

JVET-AK0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

Activities

  - Verification tests for VVC multilayer coding

The verification test plan has been updated to output JVET-AJ2021 of the previous JVET meeting.

  - Subjective quality testing of FGC SEI message

A report on subjective quality testing of FGC SEI message has been produced as output document JVET-AJ2037. The report includes the results and findings from expert viewing activities at the 35th and 36th JVET meetings.

The plan for subjective quality testing has been updated as output JVET-AJ2022 of the previous JVET meeting.

  - Test sequences

The server providing JVET test sequences has been updated. The new site is https://vqa.lfb.rwth-aachen.de. A mirror of this site is available at https://datacloud.hhi.fraunhofer.de with the same login credentials. The directory structure of the previous ftp server has been maintained. The test sequences used for CfP/CTC are available in directory “/ctc”.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG). Members of JVET may contact the JVET chair for login information. It is suggested to update the password for the site after each meeting with the Zoom password of that meeting.

Related contributions

| JVET-AK0066 | AHG4/AHG17: 8K HDR sequences as a response to the call for new HDR test materials for future video coding | Y. Li (CMG), Q. Zhang, J. Ning (ABP) |
| --- | --- | --- |
| JVET-AK0282 | AHG4/AHG17: Additional HDR sequences for non-CTC testing | A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV) |

Recommendations

The AHG recommends:

- To review the input contributions related to AHG4 during the meeting.

- To progress the work in AHG17 towards setting up a test set suitable for performance assessment of video compression technology beyond the JVET CTC.

- To continue to discuss and to update the non-finalized categories of the verification test plan for multilayer VVC and the subjective quality test plan for FGC, including those which have not been addressed yet.

- To collect volunteers to conduct further verification tests and subjective quality tests.

- To collect volunteers to actively contribute to the verification test development.

- To review the set of available test sequences for the verification tests as well as subjective quality tests and potentially collect more test sequences with a variety of content.

- To continue to collect new test sequences available for JVET with licensing statement.

JVET-AK0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

- Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 36th and 37th meetings.

- Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15

  - ISO/IEC FDIS 23090-15:202x 2nd edition text output of 2023-01, preparation delayed to 2023-09, FDIS ballot opened 2024-04-08, FDIS ballot closed 2024-06-03, published 2024-07-04

  - H.266.1 V2 Consent 2023-07, Last Call began 2023-08-16, Approved 2023-09-13, pre-published 2023-10-06, published 2023-10-19

- Status on bitstream submission

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - Three streams have been re-generated

  - No changes between 36th and 37th meeting.

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 1 bitstream of 1 identified category has been re-generated

  - 128 bitstreams of 57 identified categories have been cross-checked and uploaded.

  - No changes between 36th and 37th meeting.

- additional conformance bitstreams for VVC Multilayer:

  - 3 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 7 total bitstreams have been provided, cross-checked, and made available

  - No changes between 36th and 37th meeting.

- conformance bitstreams for new HEVC Multiview profiles

  - 2 HEVC Multiview Extended 10 bitstreams have been provided and cross-checked.

  - 2 HEVC Multiview Extended bitstreams have been provided and cross-checked.

  - No changes between 36th and 37th meeting.

- Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.

VVC activities:

The new version of RAP_B (RAP_B_2) reported at the 36th meeting is available at ​https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC2ndEd_regenerated/under_test. No change in the other bitstreams and/or packages.

VVC operation range extensions activities:

No change in bitstreams and/or packages, 2nd edition packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC2ndEd/

VVC Multilayer activities:

No change in bitstreams and/or packages.

HEVC Multiview supporting extended bit depth activities:

No change in bitstreams and/or packages, the 4 packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/under_test/.

No update on the implementation of the HEVC Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles) in JVET-AH1006 and in WG5 N0281. No update on conformance streams for the new HEVC Multiview Main 10 profile in JVET-AJ0213 added at the 36th meeting and in output document AJ1006.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

- Contributions

No contributions.

- Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

- Recommendations

The AHG recommends the following:

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Continue the generation, cross-checking, and documentation of the conformance streams for the HEVC Multiview profiles supporting extended bit depth and the new HEVC Multiview Main 10 profile

- Prepare for a new edition of VVC conformance with the new multi-layer streams and the corrections of the existing streams.

It was reported that software for generating standalone bitstreams for profiles with bitdepth >10 should work OK, but was not merged yet (see AHG3 report). This should be the next step. For bitstreams using combinations of layers with different bit depth, more updates of software are necessary.

On the last bullet item, and considering that the document JVETAI2028 is stable, it was concluded that a new edition of VVC conformance should be started (request in ISO at the current meeting, CD in April, DIS and ITU consent in October).

JVET-AK0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

- Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-14.1:

Fix issue #74 TM tools off for JVET-AI0103 [MR 726]

Fix Clang 16.0.6 compiling issues [MR 724]

Set GeoInerIbc in config file explicitly to enable it for SCC [MR 725]

Fix: compilation when macro JVET_AI0136_ADAPTIVE_DUAL_TREE is set to 0
[MR 726]

Fix: wrong memory access in case of DebugBitstream feature usage [MR 728]

Correction related to adaptive dual treee (JVET_AI0136) to make feature DebugBitstream work [MR 727]

Fix DebugBitstream issue caused by copying values for adaptive clipping [MR 730]

Fix: RPR encoder crash when JVET_AH0314_LIC_INHERITANCE_FOR_MRG=1 [MR 729]

Fix useless copies of TempCtx objects to Ctx dest [MR 731]

Fix RPR w.r.t. temporal prediction of partitioning information [MR 734]

Fix: 2 valgrind erros at encoder side [MR 735]

Fix DebugBitstream issue caused by PDP [MR 733]

Fix in addBIOAvgN_SSE for non-AVX2 with JVET_AI0046_HIGH_PRECISION_BDOF_SAMPLE [MR 736]

Add check whether writing to bitstream succeeded [MR 738]

Fix a bug of tracing blocks stats on decoder [MR 740]

The following changes were integrated into ECM-14.2:

Fix: always build hash info for I slices [MR 801]

The following changes were integrated into ECM-15.0:

JVET-AJ0096: SATD-based reordering for Intra/Inter (Test3.10a/b) [MR 752]

JVET-AJ0081: Chroma TMRL (Test 2.18) [MR 742]

JVET-AJ0161: OBMC extension with intra prediction (Test 3.3b) [MR 748, 798]

JVET-AJ0274: GPM-affine with TM and Regression-based GPM with TM (Test 3.12) [MR 753]

JVET-AJ0172: Unified reference area of IBC and IntraTMP (Test 2.16) [MR 743]

JVET-AJ0061: TIMD Merge Mode (Test 2.5) [MR 750, 769]

JVET-AJ0226: Using CABAC cost for MTT split modes selection [MR 758]

JVET-AJ0268: Combination of DIMD with 2x2 edge operator applied to small blocks (Tests 2.11) and adaptive HoG for DIMD (Test 2.10a) (Test 2.13a) [MR 760, 763]

JVET-AJ0126: Enhanced inter AMVP (Test 3.2b) [MR 751]

JVET-AJ0175: NSPT for non-regular modes (Test 4.1) [MR 749]

JVET-AJ0057: High-level intra tool control (Test 2.14) [MR 744]

JVET-AJ0188: Coding Info based Classification for ALF (Test 5.4) [MR 754, 781]

JVET-AJ0082: Multi-model EIP (Test 2.19a) [MR 745, 786]

JVET-AJ0107: GPM mode extension (Test 3.4d) [MR 741, 776, 783]

JVET-AJ0097: BDOF for lowdelay pictures (Test 3.5) [MR 755]

JVET-AJ0085: Subblock merge mode extension (Test 3.7a) [MR 770]

JVET-AJ0112: Regression-based SGPM blending (Test 2.15a*) [MR 762, 785]

JVET-AJ0158: Sub-block inter mode improvement (Test 3.9d) [MR 746, 775, 778, 795]

JVET-AJ0260: SBT corner mode (Test 4.3a) [MR 771]

JVET-AJ0249: Neural network-based intra prediction with DIMD mode derivation (Test 2.20/21) [MR 764, 791]

JVET-AJ0146: Additional TIMD mode with different cost metric (Test 2.6) [MR 773, 780]

JVET-AJ0257: Improved Implicit MTS (Test 4.2) [MR 779]

JVET-AJ0079: Disabling timd-isp and timd-mrl combinations in a non-normative way (Test 2.7b) [MR 761]

JVET-AJ0237: 12-bit internal bit depth modifications for ECM [MR 757]

DebugBitstream usage without –DebugPOC [MR 772]

Fix ClassF.cfg lowdelay [MR 765, 774]

Fix for resolving enc/dec mismatch from JVET-AG0098(getAmvpSbTMVP) [MR 777]

Fix memory leak in allocated g_pdpFiltersMip [MR 782]

Fix the compiling issues of clang 16.0.6 [MR 784]

Fix for bvFilter copying of FIBC [MR 788]

Fix for IPM storage of SGPM [MR 789]

Fix for SeparateTree [MR 790]

Fix KEEP_PRED_AND_RESI_SIGNALS [MR 792]

The following changes were integrated into VTM-11ecm anchor:

JVET-AJ0226: Using CABAC cost for MTT split modes selection [MR 768]

Update class F low delay config [MR 793]

ECM-14.1 was tagged on November 7, 2024.

ECM-15.0, ECM-14.2, and VTM-11ecm15.0 were tagged on December 11, 2024.

  - CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AF2017 are summarized.

ECM-15.0 performance over ECM-14.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -1.24% | -1.09% | -1.26% | 107.0% | 105.6% | 117.8% | 109.1% |
| Class A2 | -0.72% | -0.69% | -0.68% | 107.7% | 110.4% | 117.0% | 109.4% |
| Class B | -0.82% | -0.60% | -0.57% | 108.4% | 115.4% | 110.7% | 106.1% |
| Class C | -0.64% | -0.64% | -0.72% | 103.8% | 118.7% | 105.6% | 104.2% |
| Class E | -1.23% | -1.33% | -1.14% | 108.2% | 116.6% | 105.0% | 104.7% |
| Overall | -0.90% | -0.83% | -0.83% | 107.0% | 113.8% | 110.7% | 106.5% |
| Class D | -0.64% | -0.56% | -0.76% | 104.1% | 126.0% | 104.1% | 103.5% |
| Class F | -0.93% | -0.97% | -0.89% | 106.5% | 115.9% | 102.6% | 104.6% |
| Class TGM | -1.54% | -1.73% | -1.67% | 106.6% | 115.6% | 101.0% | 105.3% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.73% | -0.89% | -0.83% | 104.3% | 103.5% | 101.4% | 104.3% |
| Class A2 | -0.84% | -0.66% | -0.78% | 103.7% | 103.4% | 101.0% | 104.5% |
| Class B | -0.74% | -0.74% | -0.66% | 104.1% | 106.9% | 101.2% | 103.6% |
| Class C | -0.89% | -0.69% | -0.50% | 106.3% | 110.0% | 103.0% | 102.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.79% | -0.74% | -0.68% | 104.7% | 106.3% | 101.7% | 103.7% |
| Class D | -0.85% | -0.43% | -0.72% | 106.4% | 110.7% | 104.2% | 103.1% |
| Class F | -0.88% | -0.92% | -0.93% | 106.4% | 115.5% | 101.7% | 103.3% |
| Class TGM | -1.23% | -1.13% | -1.18% | 106.3% | 111.4% | 100.4% | 103.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.79% | -0.57% | -0.62% | 110.5% | 109.0% | 102.1% | 102.7% |
| Class C | -1.05% | -0.12% | -0.98% | 111.2% | 113.8% | 104.1% | 102.2% |
| Class E | -1.14% | 0.61% | -1.08% | 113.2% | 107.2% | 101.4% | 102.0% |
| Overall | -0.96% | -0.13% | -0.86% | 111.4% | 110.1% | 102.6% | 102.4% |
| Class D | -1.22% | -0.03% | -0.22% | 112.9% | 111.8% | 104.2% | 103.2% |
| Class F | -1.10% | -1.35% | -0.21% | 108.2% | 110.7% | 102.4% | 103.2% |
| Class TGM | -1.19% | -1.15% | -1.14% | 106.7% | 116.1% | 100.7% | 103.5% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-14.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.73% | -0.49% | -0.70% | 107.6% | 109.4% | 101.4% | 105.1% |
| Class C | -0.82% | -0.46% | -0.57% | 105.7% | 106.3% | 106.8% | 104.3% |
| Class E | -1.01% | -2.31% | -1.10% | 106.6% | 105.0% | 102.5% | 104.7% |
| Overall | -0.83% | -0.93% | -0.76% | 106.7% | 107.2% | 103.4% | 104.7% |
| Class D | -1.32% | -0.73% | -1.17% | 104.9% | 100.4% | 105.4% | 103.5% |
| Class F | -0.92% | -0.66% | -0.94% | 106.3% | 114.3% | 102.0% | 104.6% |
| Class TGM | -0.94% | -0.88% | -1.15% | 107.0% | 113.0% | 102.3% | 104.6% |

The below tables show ECM-15.0 performance comparing to VTM-11.0ecm15.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -14.31% | -15.16% | -26.55% | 1136.4% | 528.0% |  |  |
| Class A2 | -20.82% | -23.47% | -27.99% | 1131.2% | 568.7% |  |  |
| Class B | -14.38% | -21.88% | -19.73% | 1052.3% | 601.4% |  |  |
| Class C | -14.30% | -11.21% | -12.34% | 1007.8% | 551.3% |  |  |
| Class E | -18.63% | -21.83% | -20.07% | 995.7% | 623.0% |  |  |
| Overall | -16.13% | -18.65% | -20.66% | 1058.7% | 575.3% |  |  |
| Class D | -12.16% | -8.19% | -8.92% | 990.4% | 585.2% |  |  |
| Class F | -29.93% | -33.46% | -33.65% | 744.5% | 672.5% |  |  |
| Class TGM | -43.12% | -48.77% | -48.02% | 576.4% | 704.0% |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -26.92% | -23.37% | -35.46% | 1148.5% | 1017.6% |  |  |
| Class A2 | -30.10% | -33.29% | -38.74% | 1103.2% | 1216.1% |  |  |
| Class B | -24.46% | -31.49% | -28.80% | 933.8% | 1077.1% |  |  |
| Class C | -26.33% | -21.66% | -22.37% | 1003.4% | 1179.8% |  |  |
| Class E |  |  |  |  |  |  |  |
| Overall | -26.58% | -27.61% | -30.41% | 1025.7% | 1117.9% |  |  |
| Class D | -27.14% | -22.16% | -23.42% | 949.0% | 1295.2% |  |  |
| Class F | -32.70% | -34.86% | -35.64% | 870.5% | 827.1% |  |  |
| Class TGM | -42.33% | -47.83% | -47.63% | 735.6% | 656.0% |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -21.69% | -35.54% | -31.98% | 964.9% | 896.7% |  |  |
| Class C | -24.13% | -24.52% | -26.26% | 917.1% | 969.2% |  |  |
| Class E | -21.40% | -25.57% | -24.70% | 861.1% | 592.9% |  |  |
| Overall | -22.43% | -29.37% | -28.26% | 922.1% | 829.8% |  |  |
| Class D | -25.45% | -25.45% | -26.27% | 933.8% | 1110.6% |  |  |
| Class F | -30.25% | -38.19% | -37.94% | 824.3% | 729.5% |  |  |
| Class TGM | -40.56% | -50.17% | -50.03% | 740.4% | 622.4% |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm15.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -19.47% | -44.58% | -41.77% | 836.4% | 875.7% |  |  |
| Class C | -22.13% | -33.99% | -34.62% | 774.1% | 895.5% |  |  |
| Class E | -19.91% | -35.54% | -35.94% | 778.7% | 627.7% |  |  |
| Overall | -20.47% | -38.79% | -37.93% | 800.7% | 811.8% |  |  |
| Class D | -24.78% | -36.48% | -37.19% | 783.5% | 1014.5% |  |  |
| Class F | -28.36% | -43.60% | -44.08% | 815.3% | 742.8% |  |  |
| Class TGM | -38.65% | -52.57% | -52.14% | 790.4% | 613.0% |  |  |

- ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1& A2) | 9 | 15 |  |
| class B | 4 | 6 | 5 |
| class C | 2 | 3 | 2 |
| class D | 2 | 2 | 2 |
| class E | 3 |  | 3 |
| class F | 4 | 6 | 5 |
| class TGM | 3 | 8 | 6 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

- Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.

JVET-AK0007 JVET AHG report: ECM tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, X. Li, J. Pardo, H. Wang (vice chairs)]

- AHG7 Conference call on 12/11/2024

An AHG7 conference call was held on 12/11/2024. Two agenda items were discussed: 1) ECM software implementation on tool controls for the group 5; 2) Complexity analysis of the tools in the group 5. Most items were fully discussed during the call. Several follow-up items were offline addressed. The details including the complexity assessment may be found in the conference call report JVET-AK0048.

- Group off tests

  - Test settings and crosschecking

The following five groups of tools were defined. Among them the group 5 was newly introduced for tools with large memory access during the 36th JVET meeting. However, the implementation of the control flags for the group 5 have not been completed before the testing. Therefore, the first four groups below were tested by AHG7 during this meeting cycle. There are input documents on tools of group 5.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

- Group 5: Tools with large memory access

The testers and crosscheckers are summarized in the table below.

| Tests | Testers | Crosscheckers |
| --- | --- | --- |
| Group 1 off | Johan Pardo (johan.esprit.pardo@huawei.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Ryo Ishimoto (ishimoto.ryo@mail.sharp) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Xiang Li (xlxiangli@google.com) | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) |
| Group 1-4 off | Lien-Fei Chen (lienfei.chen@global.tencent.com), Xiang Li (xlxiangli@google.com) | Hongtao Wang (hongtaow@qti.qualcomm.com) |

ECM-15.0 and a commit before it were used in the AHG7 tool off tests, as summarized in the table below. The cfg files used are included in the ECM software package.

| Tests | Testers | Crosscheckers |
| --- | --- | --- |
| Group 1 off | ECM-15.0 | ECM-15.0 |
| Group 2 off | ECM-15.0 | ECM-15.0 |
| Group 3 off | ECM-15.0 | ECM-15.0 |
| Group 4 off | ECM-15.0 | ECM-15.0pre-1e54f849f |
| Group 1-4 off | ECM-15.0 | ECM-15 |

  - Group 1 off

Group 1 includes inter template matching tools. The offgroup1.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-15 (698389d6) |  |  |  |  |  | Over VTM-11ecm15 (0113b3c0) |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102.0% | 100.6% | 100.0% | -14.32% | -15.16% | -26.55% | 2101.4% | 1153.2% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101.6% | 100.0% | 100.0% | -20.82% | -23.47% | -27.99% | 2044.5% | 1190.0% |
| Class B | 0.00% | 0.00% | 0.00% | 101.8% | 101.6% | 100.9% | -14.39% | -21.89% | -19.73% | 1894.9% | 1240.7% |
| Class C | -0.01% | -0.01% | -0.01% | 101.1% | 102.1% | 99.1% | -14.31% | -11.22% | -12.35% | 1865.3% | 1166.0% |
| Class E | -0.02% | -0.02% | -0.02% | 101.7% | 101.9% | 100.0% | -18.65% | -21.84% | -20.08% | 1787.6% | 1349.0% |
| Overall | -0.01% | -0.01% | -0.01% | 101.6% | 101.3% | 100.1% | -16.14% | -18.65% | -20.66% | 1926.8% | 1217.3% |
| Class D | -0.04% | -0.04% | -0.04% | 101.2% | 99.6% | 99.9% | -12.20% | -8.23% | -8.96% | 1828.5% | 1214.9% |
| Class F | -0.02% | -0.02% | -0.02% | 102.8% | 103.0% | 99.7% | -29.94% | -33.47% | -33.66% | 1286.5% | 1255.4% |
| Class TGM | -0.01% | 0.00% | 0.00% | 101.8% | 101.6% | 102.4% | -43.12% | -48.77% | -48.02% | 1055.6% | 1249.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-15 (698389d6) |  |  |  |  |  | Over VTM-11ecm15 (0113b3c0) |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.94% | 4.49% | 4.84% | 88.5% | 73.8% | 100.1% | -23.28% | -20.09% | -32.18% | 1816.9% | 1704.0% |
| Class A2 | 5.82% | 5.67% | 5.74% | 86.6% | 66.0% | 99% | -26.04% | -29.56% | -35.21% | 1761.3% | 1810.5% |
| Class B | 4.63% | 4.57% | 4.70% | 80.6% | 67.4% | 99.9% | -20.93% | -28.34% | -25.41% | 1414.1% | 1634.1% |
| Class C | 4.90% | 4.95% | 5.24% | 77.1% | 63.5% | 100.0% | -22.67% | -17.78% | -18.29% | 1419.1% | 1635.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 5.00% | 4.87% | 5.08% | 82.3% | 67.3% | 100% | -22.88% | -24.12% | -26.83% | 1555.0% | 1682.2% |
| Class D | 3.97% | 3.95% | 4.25% | 75.5% | 60.1% | 100.0% | -24.19% | -19.04% | -20.09% | 1320.3% | 1738.3% |
| Class F | 3.71% | 3.69% | 3.80% | 88.2% | 75.6% | 99.9% | -30.11% | -32.34% | -33.09% | 1297.2% | 1177.9% |
| Class TGM | 4.63% | 4.48% | 4.53% | 89.2% | 80.5% | 100.3% | -39.67% | -45.45% | -45.21% | 1164.4% | 1130.0% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-15 (698389d6) |  |  |  |  |  | Over VTM-11ecm15 (0113b3c0) |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 5.75% | 6.11% | 6.04% | 70.6% | 59.0% | 100% | -17.15% | -31.62% | -28.02% | 1248.8% | 1243.1% |
| Class C | 5.84% | 6.03% | 6.53% | 70.8% | 54.7% | 100% | -19.69% | -19.98% | -21.41% | 1155.8% | 1202.9% |
| Class E | 5.94% | 5.77% | 6.12% | 77.4% | 71.6% | 100% | -16.73% | -21.34% | -20.15% | 1188.1% | 1129.0% |
| Overall (Ref) | 5.82% | 6.00% | 6.22% | 72.3% | 60.4% | 100% | -17.90% | -25.17% | -23.85% | 1201.9% | 1200.3% |
| Class D | 4.95% | 4.91% | 5.30% | 69.5% | 54.2% | 100% | -21.74% | -21.87% | -22.32% | 1182.9% | 1262.1% |
| Class F | 5.51% | 5.17% | 4.83% | 82.8% | 66.3% | 100% | -26.31% | -34.83% | -34.85% | 1183.0% | 998.5% |
| Class TGM | 6.61% | 6.23% | 6.34% | 88.8% | 73.5% | 99% | -36.60% | -46.90% | -46.69% | 1121.2% | 915.0% |

  - Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The offgroup2.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM15 |  |  |  |  |  | Over VTM11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.27% | 0.63% | 94.1% | 95.5% | 100% | -14.17% | -14.94% | -26.04% | 1068.1% | 544.9% |
| Class A2 | 0.26% | 0.43% | 0.48% | 93.4% | 97.4% | 101% | -20.61% | -23.17% | -27.65% | 1040.6% | 577.0% |
| Class B | 0.20% | 0.44% | 0.45% | 92.7% | 96.2% | 100% | -14.21% | -21.55% | -19.35% | 984.1% | 613.7% |
| Class C | 0.16% | 0.38% | 0.41% | 91.5% | 94.1% | 99% | -14.16% | -10.88% | -11.99% | 963.0% | 548.7% |
| Class E | 0.32% | 0.58% | 0.86% | 92.9% | 96.3% | 98% | -18.37% | -21.38% | -19.37% | 930.5% | 640.9% |
| Overall | 0.22% | 0.42% | 0.54% | 92.8% | 95.8% | 99% | -15.95% | -18.32% | -20.21% | 992.8% | 585.1% |
| Class D | 0.12% | 0.23% | 0.44% | 91.1% | 92.1% | 99% | -12.05% | -7.99% | -8.52% | 944.6% | 585.0% |
| Class F | 0.62% | 0.59% | 0.56% | 92.8% | 98.1% | 100% | -29.52% | -33.08% | -33.23% | 576.9% | 625.9% |
| Class TGM | 0.41% | 0.54% | 0.48% | 93.3% | 99.9% | 100% | -42.88% | -48.49% | -47.76% | 496.4% | 664.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM15 |  |  |  |  |  | Over VTM11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 2.47% | 1.18% | 1.96% | 87.7% | 98.4% | 100% | -25.12% | -22.38% | -34.22% | 984.7% | 1151.3% |
| Class A2 | 1.44% | 0.80% | 1.57% | 86.7% | 100.6% | 100% | -29.08% | -32.72% | -37.74% | 945.5% | 1417.7% |
| Class B | 1.46% | 1.07% | 1.16% | 86.6% | 97.2% | 99% | -23.34% | -30.72% | -27.88% | 812.7% | 1197.8% |
| Class C | 1.04% | 1.14% | 1.01% | 85.7% | 95.0% | 99% | -25.56% | -20.79% | -21.61% | 881.0% | 1253.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.55% | 1.06% | 1.36% | 86.6% | 97.5% | 100% | -25.44% | -26.81% | -29.45% | 889.4% | 1244.0% |
| Class D | 0.50% | -0.11% | 0.25% | 84.7% | 94.9% | 99% | -26.77% | -22.26% | -23.24% | 850.3% | 1485.7% |
| Class F | 1.97% | 2.11% | 1.84% | 88.2% | 95.8% | 99% | -31.37% | -33.50% | -34.46% | 668.5% | 736.9% |
| Class TGM | 3.47% | 5.39% | 5.63% | 90.5% | 98.6% | 99% | -40.46% | -45.30% | -45.01% | 608.5% | 685.5% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM15 |  |  |  |  |  | Over VTM11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.84% | 3.78% | 3.49% | 80.8% | 94.2% | 99% | -20.24% | -33.19% | -29.66% | 788.2% | 1016.4% |
| Class C | 1.52% | 2.53% | 3.79% | 81.4% | 90.3% | 98% | -23.03% | -22.61% | -23.43% | 781.3% | 1042.6% |
| Class E | 1.27% | 3.17% | 3.22% | 83.8% | 94.1% | 101% | -20.40% | -23.25% | -22.27% | 732.3% | 721.4% |
| Overall (Ref) | 1.59% | 3.21% | 3.52% | 81.7% | 92.9% | 99% | -21.21% | -27.18% | -25.73% | 771.6% | 940.8% |
| Class D | 0.73% | 3.31% | 3.53% | 79.4% | 92.4% | 98% | -24.88% | -22.98% | -23.61% | 793.0% | 1268.1% |
| Class F | 2.46% | 3.06% | 3.30% | 85.9% | 95.1% | 97% | -28.56% | -36.45% | -35.99% | 644.3% | 711.4% |
| Class TGM | 2.43% | 7.51% | 8.57% | 89.1% | 100.2% | 101% | -39.24% | -46.80% | -46.40% | 588.2% | 651.5% |

  - Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The offgroup3.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM15 |  |  |  |  |  | Over VTM11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.01% | 3.88% | 6.28% | 72.4% | 90.9% | 100% | -13.45% | -12.15% | -22.35% | 774.4% | 487.7% |
| Class A2 | 2.29% | 6.17% | 6.09% | 71.9% | 80.8% | 100% | -18.95% | -19.12% | -24.05% | 749.0% | 457.9% |
| Class B | 2.21% | 4.85% | 5.03% | 70.1% | 79.3% | 99% | -12.52% | -18.48% | -15.92% | 698.3% | 489.5% |
| Class C | 1.77% | 2.48% | 2.73% | 70.2% | 79.9% | 99% | -12.81% | -9.09% | -10.01% | 706.8% | 474.1% |
| Class E | 3.06% | 4.71% | 4.20% | 70.5% | 79.6% | 102% | -16.14% | -18.32% | -16.75% | 676.5% | 522.8% |
| Overall | 2.07% | 4.36% | 4.77% | 70.8% | 81.6% | 100% | -14.41% | -15.42% | -17.17% | 716.9% | 485.7% |
| Class D | 1.17% | 1.96% | 2.23% | 71.3% | 81.4% | 100% | -11.13% | -6.43% | -6.90% | 694.5% | 543.8% |
| Class F | 6.05% | 6.86% | 7.14% | 71.3% | 69.8% | 100% | -25.91% | -29.31% | -29.28% | 437.9% | 421.5% |
| Class TGM | 11.53% | 13.48% | 13.88% | 75.8% | 57.5% | 100% | -36.70% | -42.03% | -40.99% | 389.2% | 349.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM15 |  |  |  |  |  | Over VTM11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.98% | 2.60% | 6.53% | 92.0% | 96.9% | 100% | -26.21% | -21.45% | -32.08% | 1010.1% | 1106.4% |
| Class A2 | 1.39% | 3.73% | 3.70% | 93.5% | 97.7% | 99% | -29.13% | -30.88% | -36.51% | 999.4% | 1317.1% |
| Class B | 1.16% | 4.42% | 4.81% | 92.0% | 98.7% | 100% | -23.59% | -28.52% | -25.42% | 842.5% | 1179.6% |
| Class C | 0.81% | 1.42% | 1.76% | 92.0% | 95.3% | 100% | -25.73% | -20.54% | -20.98% | 892.3% | 1270.4% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.08% | 3.12% | 4.12% | 92.3% | 97.2% | 100% | -25.79% | -25.45% | -27.79% | 917.9% | 1214.3% |
| Class D | 0.64% | 1.46% | 2.05% | 93.1% | 94.0% | 100% | -26.68% | -20.97% | -21.76% | 822.8% | 1284.3% |
| Class F | 4.05% | 4.77% | 4.93% | 89.7% | 93.1% | 100% | -30.19% | -32.15% | -32.84% | 672.2% | 689.6% |
| Class TGM | 7.21% | 8.87% | 9.34% | 92.6% | 91.7% | 100% | -38.19% | -43.26% | -42.80% | 627.2% | 617.7% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM15 |  |  |  |  |  | Over VTM11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.43% | 1.61% | 1.06% | 95.2% | 99.6% | 100% | -21.35% | -34.64% | -31.33% | 880.5% | 1013.0% |
| Class C | 0.54% | 0.16% | 1.35% | 94.3% | 97.5% | 100% | -23.72% | -24.38% | -25.22% | 864.3% | 1126.4% |
| Class E | 0.66% | 2.69% | 2.54% | 96.2% | 96.0% | 101% | -20.87% | -23.66% | -22.84% | 814.6% | 724.1% |
| Overall (Ref) | 0.52% | 1.40% | 1.53% | 95.1% | 98.0% | 100% | -22.02% | -28.47% | -27.17% | 858.2% | 965.0% |
| Class D | 0.26% | 0.54% | 0.39% | 93.2% | 98.1% | 100% | -25.25% | -25.10% | -25.89% | 859.2% | 1245.6% |
| Class F | 3.48% | 4.91% | 4.95% | 92.5% | 103.1% | 100% | -27.95% | -35.49% | -35.07% | 683.8% | 683.5% |
| Class TGM | 5.10% | 6.23% | 6.51% | 92.1% | 97.7% | 100% | -37.55% | -47.22% | -46.92% | 624.9% | 575.7% |

  - Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The offgroup4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-15 |  |  |  |  |  | Over VTM-11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.67% | 6.72% | 9.34% | 86.7% | 89.5% | 100% | -12.89% | -9.94% | -20.12% | 702.2% | 323.1% |
| Class A2 | 2.56% | 6.88% | 7.95% | 82.5% | 90.3% | 101% | -18.89% | -18.56% | -22.87% | 674.0% | 361.0% |
| Class B | 1.09% | 7.21% | 6.08% | 84.7% | 90.8% | 98% | -13.46% | -16.94% | -15.11% | 814.9% | 525.8% |
| Class C | 1.19% | 3.56% | 3.56% | 79.7% | 85.1% | 100% | -13.30% | -8.16% | -9.31% | 791.1% | 442.2% |
| Class E | 1.58% | 7.01% | 5.79% | 85.8% | 87.1% | 100% | -17.35% | -16.76% | -15.56% | 783.3% | 520.9% |
| Overall | 1.54% | 6.23% | 6.33% | 83.7% | 88.6% | 100% | -14.88% | -14.06% | -16.03% | 760.1% | 437.5% |
| Class D | 1.11% | 3.34% | 3.24% | 86.6% | 87.0% | 100% | -11.19% | -5.16% | -5.97% | 835.4% | 491.9% |
| Class F | 1.49% | 4.43% | 4.32% | 88.7% | 90.7% | 100% | -28.89% | -30.70% | -30.88% | 482.5% | 471.8% |
| Class TGM | 2.27% | 4.09% | 4.16% | 87.5% | 93.2% | 100% | -41.88% | -46.76% | -45.96% | 418.4% | 573.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-15 |  |  |  |  |  | Over VTM-11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.65% | 3.43% | 5.95% | 92.1% | 99.1% | 100% | -25.72% | -20.84% | -32.13% | 734.9% | 659.1% |
| Class A2 | 1.20% | 3.49% | 3.74% | 93.8% | 100.3% | 99% | -29.26% | -31.03% | -36.53% | 719.8% | 833.1% |
| Class B | 0.99% | 6.08% | 5.50% | 89.4% | 93.8% | 100% | -23.72% | -27.50% | -25.05% | 760.8% | 942.4% |
| Class C | 1.13% | 2.08% | 2.13% | 91.9% | 96.5% | 100% | -25.49% | -20.04% | -20.70% | 845.4% | 1037.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.20% | 3.97% | 4.34% | 91.5% | 96.8% | 100% | -25.70% | -24.88% | -27.60% | 768.5% | 878.2% |
| Class D | 0.95% | 1.78% | 2.16% | 93.2% | 96.9% | 99% | -26.44% | -20.81% | -21.79% | 850.8% | 1139.7% |
| Class F | 1.33% | 3.25% | 3.39% | 92.0% | 95.8% | 99% | -31.87% | -32.96% | -33.63% | 656.3% | 616.2% |
| Class TGM | 0.87% | 1.72% | 1.66% | 90.8% | 94.6% | 100% | -41.87% | -46.96% | -46.79% | 568.9% | 543.6% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-15 |  |  |  |  |  | Over VTM-11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.05% | 1.63% | 1.62% | 98.4% | 99.8% | 100% | -20.86% | -34.56% | -31.01% | 875.2% | 835.4% |
| Class C | 1.29% | 1.03% | 1.11% | 93.2% | 95.8% | 99% | -23.15% | -23.76% | -25.42% | 796.9% | 862.8% |
| Class E | 0.97% | 3.29% | 2.38% | 91.4% | 91.8% | 99% | -20.63% | -23.35% | -22.93% | 758.4% | 568.2% |
| Overall (Ref) | 1.11% | 1.84% | 1.64% | 94.9% | 96.4% | 99% | -21.57% | -28.15% | -27.13% | 818.4% | 766.9% |
| Class D | 1.08% | 0.71% | 1.33% | 95.3% | 92.5% | 98% | -24.60% | -24.97% | -25.34% | 821.3% | 1011.7% |
| Class F | 1.31% | 2.48% | 1.29% | 95.0% | 96.7% | 99% | -29.33% | -36.87% | -37.18% | 604.8% | 519.1% |
| Class TGM | 0.54% | 1.33% | 1.40% | 100.0% | 102.9% | 102% | -40.26% | -49.53% | -49.38% | 637.6% | 509.1% |

  - Group 1-4 off

In this test, all the tools in the group 1-4 are switched off. The offgroup1-4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-15 |  |  |  |  |  | Over VTM-11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 3.09% | 10.53% | 15.85% | 56.6% | 72.6% | 100% | -11.66% | -6.92% | -15.90% | 458.2% | 262.2% |
| Class A2 | 5.64% | 13.95% | 14.47% | 52.9% | 62.8% | 102% | -16.37% | -13.52% | -18.49% | 432.1% | 251.1% |
| Class B | 3.98% | 12.74% | 11.55% | 52.9% | 68.5% | 98% | -11.01% | -12.96% | -10.92% | 509.0% | 396.4% |
| Class C | 3.46% | 6.60% | 6.93% | 50.7% | 62.8% | 99% | -11.38% | -5.53% | -6.42% | 504.0% | 326.0% |
| Class E | 6.09% | 12.35% | 11.25% | 55.5% | 65.5% | 102% | -13.68% | -12.68% | -11.20% | 506.9% | 391.9% |
| Overall | 4.34% | 11.14% | 11.68% | 53.4% | 66.4% | 100% | -12.54% | -10.35% | -12.06% | 485.3% | 327.7% |
| Class D | 2.70% | 5.43% | 5.78% | 52.2% | 64.1% | 99% | -9.80% | -3.26% | -3.68% | 503.3% | 362.2% |
| Class F | 12.66% | 17.04% | 17.55% | 58.8% | 60.3% | 100% | -21.56% | -23.28% | -22.99% | 319.9% | 313.8% |
| Class TGM | 17.96% | 22.00% | 22.94% | 61.7% | 47.5% | 100% | -33.16% | -37.76% | -36.42% | 295.1% | 292.6% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-15 |  |  |  |  |  | Over VTM-11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 10.79% | 12.44% | 21.02% | 62.6% | 69.4% | 100% | -19.03% | -14.14% | -23.27% | 499.2% | 461.7% |
| Class A2 | 11.05% | 15.76% | 16.55% | 62.3% | 63.2% | 100% | -22.37% | -23.05% | -28.72% | 478.5% | 524.8% |
| Class B | 8.87% | 16.97% | 16.38% | 57.2% | 63.1% | 99% | -17.71% | -20.06% | -17.24% | 487.2% | 634.6% |
| Class C | 8.42% | 10.47% | 11.35% | 55.5% | 59.4% | 99% | -20.07% | -13.46% | -13.54% | 510.3% | 638.7% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 9.57% | 14.09% | 16.00% | 58.8% | 63.3% | 99% | -19.54% | -17.71% | -19.76% | 493.9% | 574.3% |
| Class D | 6.50% | 8.99% | 10.11% | 54.5% | 57.3% | 97% | -22.30% | -15.10% | -15.56% | 497.4% | 673.6% |
| Class F | 14.30% | 17.10% | 17.50% | 63.7% | 65.7% | 99% | -23.41% | -24.51% | -25.08% | 454.5% | 422.3% |
| Class TGM | 19.13% | 24.03% | 24.77% | 67.0% | 68.1% | 99% | -31.46% | -35.60% | -35.04% | 419.7% | 391.3% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-15 |  |  |  |  |  | Over VTM-11ecm15 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 9.42% | 13.50% | 13.61% | 49.2% | 53.8% | 99% | -14.27% | -27.11% | -23.06% | 438.0% | 450.7% |
| Class C | 9.25% | 10.66% | 12.52% | 47.2% | 51.1% | 98% | -17.15% | -16.56% | -17.01% | 403.4% | 460.7% |
| Class E | 9.25% | 15.64% | 15.88% | 53.8% | 62.7% | 99% | -14.10% | -14.33% | -12.65% | 446.9% | 387.9% |
| Overall (Ref) | 9.32% | 13.09% | 13.82% | 49.6% | 55.0% | 99% | -15.19% | -20.40% | -18.44% | 428.3% | 437.3% |
| Class D | 7.39% | 10.36% | 11.67% | 48.5% | 50.3% | 97% | -19.85% | -17.89% | -17.52% | 418.1% | 549.6% |
| Class F | 14.02% | 16.41% | 16.95% | 60.6% | 62.5% | 98% | -20.58% | -28.62% | -27.85% | 385.7% | 335.5% |
| Class TGM | 17.74% | 25.52% | 27.60% | 69.2% | 61.1% | 100% | -30.10% | -37.91% | -37.17% | 441.2% | 302.3% |

  - Summary

The tool-off results on top of the recent ECM versions are summarized below for BD-PSNR-Y. Note that SCC results are not included.

- Issues

  - Resolved issues

- #83, ECM-14.1 different intra frame coding results for two successive segments (AHG17 encoding task)

  - Open issues

- #87, ECM-15.0 decoder crashes when "InterLFNST=0"

- #80, Compilation Issue ECM14.0 when disabling GPM reordering

- #78, Encoder crash in RA with --LMChroma=0

- #77, Encoder crash in RA with --DepQuant=0 --NumSignPred=0

- #71, Encode/decode mismatch when using single tree

- #66, Difference in encoding results of ECM-12.0 in AhG7 group 2 tool off test

- #65, Different encoding results of ECM12 in AHG7 group1-4 off tests

- #64, Encode/decode mismatch and decoder crash when inter-CCCM is disable

- #53, Decoding mismatch was observed when AMVR is off

- Input contributions

| JVET-AK0047 | m70721 | 2024-12-09 14:16:13 | 2024-12-09 14:16:38 | 2024-12-12 16:56:00 | AHG7: Complexity and memory footprint assessment of PDP, Neural Network-based Intra Prediction, MTS and LFNST/NSPT, and ALF in ECM-15 | G. Boisson, T. Dumas, F. Galpin, S. Puri, K. Naser, C. Bonnineau (InterDigital) |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AK0048 | m70734 | 2024-12-10 22:34:32 | 2024-12-11 19:13:02 | 2024-12-11 19:13:02 | Report of AHG7 conference call | X. Li |
| JVET-AK0049 | m70736 | 2024-12-11 12:19:58 | 2024-12-11 12:30:13 | 2024-12-11 17:10:52 | AHG7: Analysis of MTS memory footprint and complexity | J. Gan (OPPO) |
| JVET-AK0067 | m70793 | 2025-01-07 7:47:49 | 2025-01-07 9:03:27 | 2025-01-07 9:03:27 | AHG7: Tool-off test results of LFNST extension and NSPT | M. Koo, J. Zhao, J. Lim, S. Kim (LGE) |
| JVET-AK0092 | m70819 | 2025-01-07 10:18:35 |  |  | AHG7: Inter LDB tool assessments for each ECM version | R. Ishimoto, Z. Fan, T. Chujoh, T. Ikai (Sharp) |
| JVET-AK0138 | m70867 | 2025-01-07 16:29:27 | 2025-01-07 17:01:08 | 2025-01-07 17:01:08 | AHG7: Intra tool assessments between ECM14 and ECM15 | C.-Y Teng, K.-W Liang, Y.-H Lin, Y.-C Yang, T.Ikai (Sharp) |
| JVET-AK0215 | m70968 | 2025-01-07 23:03:34 |  |  | Non-EE2: MTS with reduced memory | B. Ray, V. Seregin, M. Karczewicz (Qualcomm) |
| JVET-AK0228 | m70986 | 2025-01-08 4:58:08 |  |  | AHG7: Tool-off test results of ALF fixed filters | N. Hu, V. Seregin, M. Karczewicz (Qualcomm) |

- Recommendations

- Continue and improve tool assessment

- Resolve identified software issues related to the tool assessment

It was commented that reporting encoding/decoding run time cannot be the only criterion. Criteria such as memory access and bandwidth, number of maximum processing cycles, processing complexity, block decoding dependencies, number of context coded bins, pipeline and parallelization problems should be put more in focus of the AHG, in particular to develop reporting criteria that would allow more precise assessment of tools. Something like that was developed during HEVC and VVC development. Enforce mandates of AHG, conference call in interim period, expecting input contributions to the next meeting. This would later be relevant for a CfP, and some ECM tools may be good examples for this study.

JVET-AK0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

- Activities

The AHG 8 used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. There were no email exchanges in the main reflector between the last and this meeting, but active email discussions (50+ emails) among co-chairs, editors and proponents on dense QP result generation and CDTR preparation. There are 7 input contributions related to AHG 8 mandates submitted to this meeting. They are listed in Section 3.

  - Software and Common Test Conditions

AHG 8 related software and documents can be accessed at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects: common test conditions, reporting templates with anchor results, evaluation scripts and task networks are available in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc, and software implementation examples are hosted in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm.

For this meeting cycle, common test conditions remain unchanged as described in output document JVET-AI2031. Some software implementation examples are available in the repository:

| Branch name | Description |
| --- | --- |
| JVET-AB0275 | Region-of-Interest-based adaptive QP |
| JVET-AC0086 | Foreground/background separation |
| JVET-AD0122 | Temporal QP offsets |
| JVET-AE0143 | Spatial resampling (Experimental) |
| JVET-AG0212 | Post-processing filter (Removed by proponent) |
| JVET-AH0130 | Spatial resampling |
| JVET-AH0157 | Combined pre- and post-processing |
| JVET-AJ0178 | Lightweight post-processing |
| JVET-AJ0181 | Combined software JVET-AB0275, JVET-AC0086, JVET-AJ0178 |
| JVET-AJ0254 | Pre-analysis based temporal resampling |

Following the discussion in the last (36th) JVET meeting, dense QP experiments were conducted on JVET-AJ0181 to produce results for all QPs in the range from 16 to 58. The results are reported in JVET-AK0122 and crosschecked in JVET-AK0272 and JVET-AK0289.

  - Technical Report

The seventh draft of the technical report (TR) JVET-AJ2030 “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 7)” and was produced, including the following additions compared with draft 6:

- Text on chroma processing (to clause 8) JVET-AJ0168

- Post-processing algorithm (to annex A.3) JVET-AJ0178

- Combinations to new annex (to annex B) JVET-AJ0181, JVET-AJ0232, JVET-AJ0233

- Editorial change on RPR and aggressive downsampling (to clause 7.4) JVET-AJ0232

- Editorial change on useful combinations in the annex (to clause 5.1) JVET-AJ0233

- Non-monotonicity issues and curve fitting (to clause 6) JVET-AJ0235

- Temporal subsampling (to annex A.4) JVET-AJ0254

- Description of MIOU (to clause 6) JVET-AJ0254

CDTR was generated based on JVET-AJ2030 and submitted for ballot (comments).

- Input contributions

There are 7 input contributions related to AHG 8 mandates. They are listed below.

| Report |  |  |
| --- | --- | --- |
| JVET-AK0008 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) |
| Proposals |  |  |
| JVET-AK0094 | AHG8: On combination of adaptive temporal resampling, pre-processing, post-processing and ROI-based adaptive QP algorithm for machine vision | S. Wang, J. Chen, Y. Ye, B. Li (Alibaba), S. Wang (CityUHK) |
| JVET-AK0122 | AHG8: Dense QP results for joint software | C. Hollmann, J. Ström (Ericsson) |
| JVET-AK0140 | AHG9/AHG8: Showcase for Packed regions information SEI | J. Boyce, H. Zhang, M. M. Hannuksela (Nokia) |
| JVET-AK0141 | AHG8: Add Packed regions info SEI to TR on coding video for machine consumption | J. Boyce, H. Zhang, M. M. Hannuksela (Nokia) |
| Crosschecks |  |  |
| JVET-AK0272 | Cross-check of JVET-AK0122 on SFU-HW dataset (AHG8: Dense QP results for joint software) | S. Wang (Alibaba) |
| JVET-AK0289 | Cross-check of JVET-AK0122 on TVD dataset (AHG8: Dense QP results for joint software) | M. Xu (Tencent) |

- Recommendations

The AHG recommends to:

- Review all input contributions.

- Continue improving draft TR based on CDTR feedback and other inputs, towards TR finalization.

- Continue investigating non-normative technologies and their suitability for machine analysis applications.

- Continue refining test conditions, evaluation and reporting procedures.

A mismatch was observed in the dense QP results, which was resolved according to the newest upload of JVET-AK0122.

Should be presented to WG 4, but no joint meeting necessary at this time.

Align timeline with VSEI v4, which could be DTR in April, final approval (also ITU) in October

JVET-AK0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, Y.-K. Wang (co-chairs), J. Boyce, T. Chujoh, S. Deshpande, C. Fogg, M. M. Hannuksela, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger (vice-chairs)]

Related contributions

A total of 65 contributions are identified relating to the mandates of AHG9. Some contributions relate to more than one mandate and/or SEI message. Some contributions also relate to the work of AHG13.

The following is a list of contributions related to the mandates of AHG9.

  - Study the SEI messages in VSEI, VVC, HEVC and AVC (5)

    - JVET-AK2005 Additions and corrections for VVC version 4 (4)

JVET-AK0053 AHG9: NNPF interface text in VVC [Y.-K. Wang, J. Xu, L. Zhang (Bytedance), K. Yang, Y. Li, Y. Xu (SJTU)]

JVET-AK0080 AHG9: Comments on the GFV and GFVE SEI messages [M. M. Hannuksela (Nokia), J. Chen, B. Chen, Y. Ye (Alibaba)]

Also relates to mandate to study JVET-AK2006 (GFV SEI messages)

JVET-AK0160 AHG9: On VVC interface of object mask information and annotated regions SEI messages [J. Chen, B. Chen, Y. Ye (Alibaba), M. M. Hannuksela (Nokia)]

JVET-AK0197 AHG13: on picture size for the interpretation of the FGC SEI message [P. de Lagrange, E. François (InterDigital)]

    - Additions for HEVC (1)

JVET-AK0107 AHG9: Modality Information SEI for HEVC [J. Gao, H.-B. Teo, C.-S. Lim, K. Abe, V. Drugeon (Panasonic)]

Also relates to mandate to study JVET-AK2006 (modularity information SEI message)

  - Study JVET-AK2006 Additional SEI messages for VSEI version 4 (32)

    - NNPF SEI messages (9)

JVET-AK0054 AHG9: Semantics of the NNPFC and NNPFA SEI messages [Y.-K. Wang, J. Xu, L. Zhang (Bytedance), K. Yang, Y. Li, Y. Xu (SJTU)]

JVET-AK0070 AHG9: On seed parameter for NNPF [T. Chujoh, Z. Fan, R. Ishimoto, T. Ikai (Sharp), L. Jin, H. Watanabe (Waseda Univ.)]

JVET-AK0072 AHG9: Comments on NNPF SEI messages [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AK0074 AHG9: Handling output variability of an NNPF [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AK0147 AHG9: On NNPF for Tone Mapping with Colour Volume Information [C.-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0152 AHG9: On Spatial Extrapolation for NNPF [S. Deshpande (Sharp), M. M. Hannuksela (Nokia)]

JVET-AK0167 AHG9: On Enabling the usage of auxiliary layers for NNPF [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AK0188 AHG9: On NNPF filtering control strength value adjustment [J. Xu, Y.-K. Wang (Bytedance)]

    - SEI processing order and processing order nesting SEI messages (5)

JVET-AK0055 AHG9: Semantics of the SPO SEI message [Y.-K. Wang, J. Xu, L. Zhang (Bytedance), K. Yang, Y. Li, Y. Xu (SJTU)]

JVET-AK0112 AHG9: On the SPO SEI message [Y. Gao, P. Wu, Y. Bai, S. Xie, M. Jia, W. Niu, C. Huang (ZTE)]

JVET-AK0156 AHG9: On SEI processing order SEI message [C.H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0165 AHG9: On SPO sub-chain signaling [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AK0205 AHG9: On SEI messages in spoPropertySeiList [H. Tan, J. Lee, J. Nam, C. Kim, J. Lim, S. Kim (LGE)]

    - Encoder optimization information SEI message (2)

JVET-AK0075 AHG9: Additional information on object-based optimization for the encoder optimization information SEI message [M. Pedzisz, M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AK0155 AHG9: On Encoder optimization information SEI message [C.H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

    - Source picture timing information SEI message (1)

JVET-AK0168 [m70903 [AHG9: On SPTI SEI message: Robustness and source constant framerate [Y. Sanchez, T. M. Borges, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

    - Modality information SEI message (1)

JVET-AK0107 AHG9: Modality Information SEI for HEVC [J. Gao, H.-B. Teo, C.-S. Lim, K. Abe, V. Drugeon (Panasonic)]

Also relates to mandate to study JVET-AK2005

    - Generative face video SEI messages (8)

JVET-AK0080 AHG9: Comments on the GFV and GFVE SEI messages [M. M. Hannuksela (Nokia), J. Chen, B. Chen, Y. Ye (Alibaba)]

Also relates to mandate to study JVET-AK2005

JVET-AK0124 AHG9: On timing information and order of pictures in GFV SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0127 AHG9: On miscellaneous aspects of GFV and DSCI SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Also relates to DSC SEI messages

JVET-AK0128 AHG9: Editorial updates for GFV SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0154 AHG9: Comments on Generative Face Video SEI  [A. C. Sidiya, S. Deshpande (Sharp)]

JVET-AK0164 AHG9: Supplementation of value range definition and editorial bugs fixes on the GFVE pupil position SEI messages [F. Ma, A. Trioux, F. Yang (Xidian Univ.), B. Li, F. Xing, Z. Wang (Hisense)]

JVET-AK0238 AHG9: Semantics and syntax fixes for generative face video SEI message [J. Chen, B. Chen, Y. Ye (Alibaba)]

JVET-AK0239 AHG9: On generative face video enhancement SEI message [J. Chen, B. Chen, Y. Ye (Alibaba)]

    - Digitally signed content messages (8)

JVET-AK0109 AHG9: On signaling and constraint for RefDigest in digitally signed content SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0110 AHG9: On reference message digest for verification in digitally signed content SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0125 AHG9: On presence and persistency of digitally signed content SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0126 AHG9: On signaling the mapping between verification substreams and layer and temporal sublayer for DSCI SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0127 AHG9: On miscellaneous aspects of GFV and DSCI SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Also relates to GFV SEI messages

JVET-AK0194 AHG9: Digitally Signed Content SEI messages for AVC and HEVC  [K. Suehring, T. Hinz, Y. Sanchez, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

JVET-AK0206 AHG9: On digitally signed content SEI messages  [Sean McCarthy, Christof Fersch, Iraj Sodagar (Dolby)]

JVET-AK0287 AHG9: Multilayer digitally signed content authentication SEI messages [J. Boyce, M. M. Hannuksela (Nokia)]

  - Study JVET-AK2032 TuC for VSEI (26)

    - NNPF SEI messages (1)

JVET-AK0073 AHG9: Enabling multiple instances of NNPF extrapolated pictures[M. M. Hannuksela, F. Cricri (Nokia)]

    - Film grain regions characteristics SEI message (2)

JVET-AK0058 [AHG9/AHG13] Comments on Film grain regions characteristics SEI message [S. Xie, P. Wu, Y. Gao, Y. Bai, C. Huang (ZTE)]

JVET-AK0211 AHG9/AHG13: Proposed Film Grain Region SEI message for version 5 of VSEI [S. Wenger, G. Teniou, A.T. Hinds, (Tencent), P. de Lagrange, E. François, D. Doyen, (InterDigital)]

    - Constituent rectangle SEI messages (4)

JVET-AK0115 AHG9: On supporting 4:4:4 color format in constituent rectangles and enhanced colour format information SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

Also relates to ECFI SEI message

JVET-AK0209 AHG9: On constituent rectangles SEI message [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0210 AHG9: On constituent rectangle grouping for 4:4:4 color format support in constituent rectangle SEI message [H. Tan, J. Nam, C. Kim, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0278 AHG9: Constituent Rectangles / Enhanced Colour Format Information SEI extension to support 4:2:2 source content coded in bitstream with 4:2:0 chroma format [S. Keating, M. Ikeda (Sony)]

Also relates to ECFI SEI message

    - Quality metrics SEI message (3)

JVET-AK0159 AHG9: On miscellaneous aspects of quality metrics SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0207 AHG9: On signalling of average quality in quality metrics SEI message [H. Tan, J. Nam, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0208 AHG9: On the inclusion of quality metric SEI message in SEI processing order SEI message [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

    - Lens optical correction SEI messages (2)

JVET-AK0179 AHG9: On Lens Optical Correction SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AK0204 AHG9: Proposed Lens Optical Correction SEI message for version 5 of VSEI [S. Wenger, G. Teniou, A. T. Hinds (Tencent)]

    - Display overlays SEI message (6)

JVET-AK0099 AHG9: On bit depth alignment between target and coded pictures in DOI SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AK0100 AHG9: DOI SEI support for target picture with alpha channel [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AK0157 AHG9: On the number of DOI SEI message in an AU [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0158 AHG9: On reconstruction of target display picture in DOI SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0190 AHG9: On the display overlays information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AK0265 AHG9: Display overlay sets for DOI SEI [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

    - Bitdepth range information SEI message (1)

JVET-AK0189 AHG9: On the bitdepth range information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

    - AI usage restrictions SEI message (2)

JVET-AK0114 AHG9: Updates and suggestion on AI usage restrictions SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0281 AHG9: On AI usage restrictions SEI message [M. M. Hannuksela, F. Cricri (Nokia)]

    - Enhanced colour format information SEI message (3)

JVET-AK0115 AHG9: On supporting 4:4:4 color format in constituent rectangles and enhanced colour format information SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

Also relates to CR SEI message

JVET-AK0278 AHG9: Constituent Rectangles / Enhanced Colour Format Information SEI extension to support 4:2:2 source content coded in bitstream with 4:2:0 chroma format [S. Keating, M. Ikeda (Sony)]

Also relates to CR SEI message

JVET-AK0286 AHG9: Simplified enhanced colour format information SEI [J. Boyce, M. M. Hannuksela (Nokia)]

    - Photosensitive content information SEI message (1)

JVET-AK0153 AHG9: On Photosensitive Content Information Signaling [S. Deshpande (Sharp)]

    - Progression of specifications (1)

JVET-AK0178 AHG9: Proposed plan for initiating version 5 of VSEI [A. T. Hinds, S. Wenger (Tencent), P. de Lagrange, E. François (InterDigital)]

  - Collect software and showcase information for SEI messages (2)

JVET-AK0140 AHG9/AHG8: Showcase for Packed regions information SEI [J. Boyce, H. Zhang, M. M. Hannuksela (Nokia)]

JVET-AK0258 AHG9/AHG11: Demo of real-time NNPF inference [J. Funnell (Nokia), M. Santamaria, F. Cricri, M. M. Hannuksela, M. Pedzisz, R. Yang, S. Schwarz]

  - Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (4)

JVET-AK0136 AHG9: Target Colour Volume SEI message [C.H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0142 AHG9: Display rectangles SEI [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)

JVET-AK0191 AHG9: Examples of danmaku applications [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AK0203 AHG9: On signalling segmentation and object tracking information [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. No emails with [AHG9] were exchanged.

Recommendations

The AHG recommends to:

- Review all related contributions; and

- Continue SEI messages studies.

Considering extensions of NNPF, it should be assessed which elements are mature enough for v4, which are going to TuC, or even more study needed.

It was commented that it would be premature to request for a version 5 of VSEI, as v4 has not yet reached DAM/DIS stage, and ISO rules would forbid going for another amendment already. As for now, TuC is sufficient to document potential candidates.

JVET-AK0010 JVET AHG report: Encoding algorithm optimization (AHG10) [P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs)]

- Activity

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

Teleconferences were held on December 4th and December 19th, and a few related organizational e-mails were sent to the JVET reflector, as well as a few support requests and answers to reproduce the results from the previous meeting cycle that were used for multilayer viewing tests during the Kemer meeting.

- Related contributions

Two contributions were identified relating to AHG10 and summarized in the following sections.

  - General

    - JVET-AK0052 - AHG10: Teleconference on encoder optimization

This document summarizes discussions during teleconferences on encoder optimization.

The remaining work related to VVC multi-layer coding was discussed, including support for multi-view (suggested tests, test sequences, expected software tools improvements and if specific configurations and encoder optimizations for visual testing would be acceptable.

  - Common test conditions

    - JVET-AK0181 - AHG10: Suggested upgrade of CTC for random access for VTM

This contribution suggests upgrading the VTM settings for random access to better handle a larger span of bitrates:

- Enabling GOP-based RPR as it can be beneficial to encode at a lower resolution for lower bitrates

- Enabling DMVR encoder control to improve subjective quality in some situations also especially at lower bitrates

Minor BD-rate gains (around -0.3%) are reported using the CTC, but more gain (up to -1.6%) when testing higher QPs, and more gain at higher resolutions. Visual quality improvement is reported, especially at low bitrates.

- Recommendation

The AHG recommends that the related input contributions are reviewed, and to further continue the study of encoding algorithm optimizations in JVET.

JVET-AK0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu, A. Segall (co-chairs), J. Li, R.-L. Liao, D. Rusanovskyy, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs)]

Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for email. Five emails were exchanged on the reflector related to the AHG mandates.

  - Common Test Conditions

    - Anchor Encoding

Anchor for the NN-based video coding activity made available on Dec. 17 though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/tree/master/Anchor%20performance?ref_type=heads

also distributed by AhG14 in JVET-AK0014.

  - New training materials

JVET group received new training materials are response for the call issued during 33rd meeting. In total 78 new video scenes can be potentially added to NNVC training data set.

| JVET-AK0255 | [AHG11] Response to Call for training materials for neural network-based video coding tool development | Fan Zhang, David Bull, Jakub Nawala, Yuxuan Jiang, Xiaoqing Zhu, Joel Sole, Elena Alshina |
| --- | --- | --- |

  - Dataset access

It has been reported by multiple proponents trying to retrain models from scratch that the original datasets used to train the current models have changed since their first use. New training materials come with permission to copy them to the JVET hosting. Group should discuss if hosting for training data is possible.

For sequences which are not hosted by JVET changes need to tracked regularly and md5sum updated if needed.

  - Interaction with ECM

Neural network based Intra (not identical, but similar to one in NNVC) was adopted to ECM showing compression performance benefits 0.2% BD-rate in random access configuration (relatively to VTM gain of NN-Intra in NNVC set was more than 1%).

  - EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available at this meeting as:

| JVET-AK0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, R. Chang, Z. Xie (EE coordinators) |
| --- | --- | --- |

  - Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on December 02 and December 20, 2024. In those teleconferences, the following topics were discussed:

- NN-based filter (LOP4) joint training report and LOP4 inference cross-check

- NNVC11.0 software integration status and anchors performance,

- EE1 description finalization

Combination of two proposals on LOP and VLOP filters, ‘Neural network-based in-loop filters using early cropping’ from JVET-AJ054 and ‘Reduced complexity input feature extraction for LOP & VLOP’ from JVET-AJ0066, were jointly trained together, verified though the training cross-check and adoption to NNVC-11.0 was confirmed during joined teleconference of AhG11/14.

As was agreed earlier EE1 proponents design their tests keeping complexity in kMAC/pxl not exceeding competing tool in NNVC and keeping the number of channels as multiple 16. If not possible to meet both requirements at the same time then two sub-tests are conducted (this is the main reason for multiple sub-tests in EE1).

  - Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital.

In NNVC-11, LOP and VLOP filters performance improved 0.2% (in random access test).

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR | Total | Filter | Intra | SR |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 (LOP4) | -7.6% | -14.3% | -13.2% | 1.2 | 36 | 21.6 | 16.8 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-10.0 (LOP3) | -7.4% | -13.6% | -11.6% | 1.2 | 33 | 21.7 | 16.9 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-9.1(LOP3) | -7.3% | -13.1% | -11.3% | 1.2 | 81 | 24.8 | 16.9 | 7.9 | 0 | 1.7 | 0.21 | 1.5 | 0 |
| NNVC-8.0(LOP2) | -6.9% | -13.2% | -12.1% | 1.2 | 73 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-7.1(LOP2) | -6.9% | -13.2% | -12.1% | 1.3 | 86 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-9.1(LOP2CA) | -8.2% | -16.5% | -15.5% | 2.5 | 69 | 25.5 | 17.6 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 (HOP5) | -14.2% | -19.5% | -19.9% | 2.6 | 1135 | 471 | 466 | 4.8 | 0 | 2.7 | 1.4 | 1.3 | 0 |
| NNVC-9.1(HOP4) | -14.1% | -19.2% | -19.6% | 2.9 | 1447 | 484 | 476 | 7.9 | 0 | 3.0 | 1.44 | 1.5 | 0 |
| NNVC-8.0(HOP3) | -13.7% | -13.9% | -14.5% | 2.5 | 1092 | 474 | 466 | 7.9 | 0 | 2.9 | 1.40 | 1.5 | 0 |
| NNVC-7.0(HOP2) | -13.6% | -12.5% | -14.2% | 4.1 | 2071 | 485 | 477 | 7.9 | 0 | 3.0 | 1.50 | 1.5 | 0 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 (VLOP3) | -5.8% | -6.6% | -5.7% | 1.2 | 7 | 9.9 | 5.10 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| NNVC-10.0 (VLOP2) | -5.6% | -7.6% | -6.4% | 1.1 | 15 | 10 | 5.16 | 4.8 | 0 | 1.3 | 0.06 | 1.3 | 0 |
| NNVC-9.1(VLOP) | -5.3% | -5.4% | -5.2% | 1.2 | 40 | 13 | 5.12 | 7.9 | 0 | 1.5 | 0.02 | 1.5 | 0 |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 aLOP3 | -8.2% | -16.7% | -15.5% | 2.2 | 34 | 21.8 | 16.9 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-11.0 NNSR | -8.5% | -12.2% | -10.9% |  |  | 26.3 | 16.8 | 4.8 | 4.7 | 1.4 | 0.05 | 1.3 | 0.05 |
| NNVC-8.0 RPR | -7.5% | -10.9% | -9.7% |  |  | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-8.0 NNSR | -7.8% | -11.8% | -10.5% |  |  | 45.3 | 17.1 | 7.9 | 20.3 | 1.8 | 0.21 | 1.5 | 0.1 |

More details and analysis for tools and tools combination is expected in AhG14 report.

Input contributions

There are 29 input contributions related to the AHG mandates. The list of input contributions is provided below.

| Reporting (2) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AK0023 | EE1: Summary report of exploration experiment on neural network-based video coding |  |  |  | E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, R. Chang, Z. Xie (EE coordinators) |  |  |  |
| JVET-AK0050 | [AHG11] [AHG14] Teleconference on NNVC |  |  |  | E. Alshina, F. Galpin |  |
| NNVC training materials (1) |  |  |  |  |  |  |
| JVET-AK0255 | [AHG11] Response to Call for training materials for neural network-based video coding tool development |  |  |  | Fan Zhang, David Bull, Jakub Nawala, Yuxuan Jiang, Xiaoqing Zhu, Joel Sole, Elena Alshina |  |
| EE1 contributions (8) |  |  |  |  |  |  |
| JVET-AK0077 | EE1-2.1: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding | Q. Qin, C. Jung (Xidian Univ.) |  |  |  |  |  |
| JVET-AK0078 | EE1-2.2: Transformer-Based Reference Frame Synthesis for VVC Inter Coding | Q. Qin, C. Jung (Xidian Univ.) |  |  |  |  |  |
| JVET-AK0093 | EE1-1.5: Adaptive skip of LOP filtering based on boundary strength partitions | H. Kwon, J. Seo, H. Ko (HYU), D. Kim, S.-C. Lim (ETRI) |  |  |  |  |  |
| JVET-AK0106 | EE1-1.1: Partial Convolution and Over-Parameterization | J. Chi, A. Li, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion) |  |  |  |  |  |
| JVET-AK0139 | EE1-1.3: Multiscale blocks in LOP4 and VLOP3 filters | R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, M. M. Hannuksela (Nokia), Y. Li, D. Rusanovskyy, S. Eadie, M. Karczewicz, J. Wang, L. Kerofsky (Qualcomm), J. Chi, A. Li, C. Zhu, L. Lei, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion) |  |  |  |  |  |
| JVET-AK0150 | EE1-1.2: LOP with improved Attention and residual groups | Y. Li, D. Rusanovskyy, S. Eadie, M. Karczewicz, J. Wang, L. Kerofsky (Qualcomm) |  |  |  |  |  |
| JVET-AK0195 | EE1-1.4: Cross-component enhanced LOP filter | Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang (Bytedance) |  |  |  |  |  |
| JVET-AK0235 | EE1-4.1: A Neural Network Downscaling Filter for RPR | Y.-Q. Zhu, W.-X. He, X. Li, J.-D. Y, Q. Liu (HUST), Z. Lv (vivo) |  |  |  |  |  |
| New NNVC (AhG11 or EE1 related) contributions (8) |  |  |  |  |  |  |
| JVET-AK0071 | AHG11: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding |  | P. Li, C. Jung, Q. Qin (Xidian Univ.) |  |  |
| JVET-AK0079 | EE1-Related: Combination Test of NNVC Tools and TRFS |  | Q. Qin, C. Jung (Xidian Univ.) |  |  |
| JVET-AK0119 | AHG11: NNLF LOP3 improvement with parallel 1x3/3x1 Backbone |  | T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam) |  |  |
| JVET-AK0146 | [AHG11] A Hybrid Framework Integrating End-to-End Learned Image Codec with Conventional Codec |  | N. Zou, A. Hallapuro, F. Cricri, H. Zhang, M. M. Hannuksela (Nokia) |  |  |
| JVET-AK0175 | AhG11: Dimension-wise decomposed multiplier for content-adaptive loop-filter |  | Z. Xu, J. Konieczny (TCL) |  |  |
| JVET-AK0177 | AHG 11: Neural Network Coded Reference Frame for Intra Coding |  | Fabian Brand, Timofey Solovyev, Elena Alshina (Huawei) |  |  |
| JVET-AK0242 | AHG11: Enhancing super-resolution with residual in NNVC |  | T. Yang, X. Li, W.-X. He, Y.-Q. Zhu, Q. Liu (HUST), Z.-Y. Lv (vivo) |  |  |
| JVET-AK0258 | AHG9/AHG11: Demo of real-time NNPF inference |  | J. Funnell (Nokia), M. Santamaria, F. Cricri, M. M. Hannuksela, M. Pedzisz, R. Yang, S. Schwarz |  |  |
| Cross-checks (6 all not yet uploaded at the time report was prepared) |  |  |  |  |  |  |  |  |
| JVET-AK0166 | Crosscheck of JVET-AK0139 (EE1-1.3 : multiscale blocks in LOP4 and VLOP3 filters) MISSED |  |  | T. Dumas (InterDigital) |  |  |
| JVET-AK0216 | Crosscheck of JVET-AK0195 (EE1-1.4: Cross-component enhanced LOP filter) MISSED |  |  | Y. Li (Qualcomm) |  |  |
| JVET-AK0241 | Crosscheck of JVET-AK0093 (EE1-1.5: Adaptive skip of LOP filtering based on boundary strength partitions) |  |  | J. Pardo, E. Alshina (Huawei) |  |  |
| JVET-AK0247 | Crosscheck of JVET-AK0235 (EE1-4.1: A Neural Network Downscaling Filter for RPR) |  |  | Z. Li, J. Zhang(PKU) |  |  |
| JVET-AK0263 | Crosscheck of JVET-AK0150 (EE1-1.2: LOP with improved Attention and residual groups) |  |  | Z.Ameur, T.Dumas, F.Galpin (Interdigital), |  |  |
| JVET-AK0292 | Crosscheck of JVET-AK0106 (EE1-1.1 Partial Convolution and Over-Parameterization) |  |  | J. Wang (Qualcomm) |  |  |

Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue collecting training materials for neural network-based video coding tool development and investigate training stability.

- Consider an up-date of NNVC training set (with re-training of existing NNVC tools)

JVET-AK0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

- Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AJ2024). The combined improvements of the ECM-15.0 over VTM-11.0ecm15.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.31% | -15.16% | -26.55% | 1136.4% | 528.0% |
| Class A2 | -20.82% | -23.47% | -27.99% | 1131.2% | 568.7% |
| Class B | -14.38% | -21.88% | -19.73% | 1052.3% | 601.4% |
| Class C | -14.30% | -11.21% | -12.34% | 1007.8% | 551.3% |
| Class E | -18.63% | -21.83% | -20.07% | 995.7% | 623.0% |
| Overall | -16.13% | -18.65% | -20.66% | 1058.7% | 575.3% |
| Class D | -12.16% | -8.19% | -8.92% | 990.4% | 585.2% |
| Class F | -29.93% | -33.46% | -33.65% | 744.5% | 672.5% |
| Class TGM | -43.12% | -48.77% | -48.02% | 576.4% | 704.0% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -26.85% | -23.26% | -35.35% | 1016.7% | 1018.4% |
| Class A2 | -30.05% | -33.22% | -38.66% | 1001.2% | 1201.5% |
| Class B | -24.46% | -31.49% | -28.80% | 933.8% | 1077.1% |
| Class C | -26.33% | -21.66% | -22.37% | 1003.4% | 1179.8% |
| Class E |  |  |  |  |  |
| Overall | -26.55% | -27.57% | -30.37% | 981.8% | 1115.4% |
| Class D | -27.14% | -22.16% | -23.42% | 949.0% | 1295.2% |
| Class F | -32.70% | -34.86% | -35.64% | 870.5% | 827.1% |
| Class TGM | -42.33% | -47.83% | -47.63% | 735.6% | 656.0% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -21.69% | -35.54% | -31.98% | 964.9% | 896.7% |
| Class C | -24.13% | -24.52% | -26.26% | 917.1% | 969.2% |
| Class E | -21.40% | -25.57% | -24.70% | 861.1% | 592.9% |
| Overall | -22.43% | -29.37% | -28.26% | 922.1% | 829.8% |
| Class D | -25.45% | -25.45% | -26.27% | 933.8% | 1110.6% |
| Class F | -30.25% | -38.19% | -37.94% | 824.3% | 729.5% |
| Class TGM | -40.56% | -50.17% | -50.03% | 740.4% | 622.4% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-14.0’s {-25.94%, -26.96%, -29.84%} to ECM-15.0’s {-26.55%, -27.57%, -30.37%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-14.0’s { -41.56%, -47.16%, -46.90%} to ECM-15.0’s { -42.33%, -47.83%, -47.63%}.

- Contributions

In addition to 39 EE2 contributions, 35 (comparing to 50 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

  - Intra (8)

JVET-AK0089, "Non-EE2: Improvement on MPM", G. Wang, C. Zhou, Z. Lv (vivo)

JVET-AK0090, "Non-EE2: IntraTMP complexity reduction", Z. Fan, T. Chujoh, T. Ikai (Sharp)

JVET-AK0171, "Non-EE2: Picture-level mirroring and rotation", D. Mieloch, M. Lorkiewicz, J. Stankowski (PUT)

JVET-AK0173, "Non-EE2: Adaptive picture-level vertical mirroring", D. Mieloch, M. Lorkiewicz, A. Dziembowski, J. Stankowski, D. Karwowski (PUT)

JVET-AK0202, "AHG12: On cross-component intra prediction", Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AK0223, "Non-EE2: CCP merge mode with adjustment", Y. Wang, K. Zhang, W. Yin, Z. Deng, N. hang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)

JVET-AK0234, "Non-EE2: Subblock-based CCCM", F. Pu, N. C. Thuong, P. Yin, S. McCarthy (Dolby)

JVET-AK0262, "EE2-related: On EIP filter shapes", K. Panusopone, M. He, S. Hong, L. Wang, J. Lainema (Nokia)

  - Inter (12)

JVET-AK0096, "Non-EE2: Additional inter merge candidates", N. Zhang, K. Zhang, Z. Deng, L. Zhao, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)

JVET-AK0103, "Non-EE2: Template matching based jointly reordering for GPM split modes and partition indexes", C. Ma, X. Xiu, X. Wang (Kwai)

JVET-AK0108, "Non-EE2: CMVP extension for constructed affine merge candidates", C. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)

JVET-AK0111, "Non-EE2: On interpolation filter for template matching", Z. Dai, R.-L. Liao, J. Chen, X. Li, Y. Ye (Alibaba)

JVET-AK0133, "Non-EE2: Improvements on the BV-based prediction in SGPM", J. Huo, Y. Fei, L. Wang, Y. Ma, F. Yang (Xidian Univ.)

JVET-AK0162, "AHG12: On affine motion compensation", H. Huang, R. Yu, Z. Zhang, Y. Zhang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AK0186, "Non-EE2: Subblock-based spatial MVP ", Z. Deng, K. Zhang, N. Zhang, L. Zhao, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)

JVET-AK0192, "Non-EE2: On Sample-Based BDOF", M. Salehifar, Y. He, K. Zhang, L. Zhao, Y. Wang, N. Zhang, Z. Deng, W. Yin, L. Zhang (Bytedance)

JVET-AK0199, "Non-EE2: MV refinement for TMVP", Z. Zhang, J.-L. Lin, Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AK0213, "AHG12: Extended BDOF usage for MV refinement", R. Yu, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AK0225, "Non-EE2: Extension on spatial and temporal merge candidates", J.-L. Lin, P.-H. Lin, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AK0254, "Case study of candidates in AMVP candidate list", Seori Park, Hyunki Jeong, Byeungwoo Jeon (SKKU)

  - In Loop Filters (7)

JVET-AK0113, "AHG12: On CCALF", L. Xu, Y. Yu, H. Yu, N. Song, D. Wang (OPPO)

JVET-AK0117, "Non-EE2: Neural network based loop filtering for ECM", Y. Du, A. Li, J. Liu, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion)

JVET-AK0132, "AHG12: Boundary strength adjustment for DBV mode", J. Huo, J. Fan, Y. Fei, J.Liu, Y. Ma, F. Yang (Xidian Univ.)

JVET-AK0184, "EE2-related: Additional results for NN-based ILF in ALF", D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

JVET-AK0200, "Non-EE2: CCSAO with reused CTU control and extended edge classifier", C.-W. Kuo, X. Xiu, X. Wang (Kwai)

JVET-AK0224, "Non-EE2: Cross-Chroma Adaptive Loop-Filter", W. Yin, K. Zhang, H. Liu, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)

JVET-AK0237, "Non-EE2: CCALF with dual-chroma inputs", C. Ma, X. Xiu, C.-W. Kuo, X. Wang (Kwai)

  - Transform (2)

JVET-AK0214, "Non-EE2: IntraNN NSPT set", G. Verba, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)

JVET-AK0215, "Non-EE2: MTS with reduced memory ", B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

  - Entropy Coding (3)

JVET-AK0135, "AhG12: CABAC contexts retraining	F. Galpin (Interdigital)

JVET-AK0174, "Non-EE2: Optimization of probability estimation in CABAC", D. Karwowski, D. Mieloch, M. Lorkiewicz (PUT)

JVET-AK0279, "Non-EE2: On coefficient level binarization in Transform Skip", M. Abdoli, R. G. Youvalari, F. Plowman, A. Tissier (Xiaomi)

  - Partitioning (1)

JVET-AK0219, "Non-EE2: Chroma partition prediction in separate tree condition", P.-H. Lin, J.-L. Lin, V. Seregin, M. Karczewicz (Qualcomm)

  - CTC and Requirements (2)

JVET-AK0201, "On additional operation point for the exploration model", T. Solovyev, J. Pardo, J. Sauer, Z. Li, E. Alshina (Huawei)

JVET-AK0232, “On real-time test condition for limited uplink/storage use case and MTT depth suggestions on common test condition”, T. Ikai (Sharp)

- Recommendations

The AHG recommends to:

- To review all the related contributions.

JVET-AK0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), A. Duenas, D. Grois, Y. He, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

- Discussion

This period was slow and limited to of the second edition of the technical report.  The output document was addressed as was discussion on additional topics.  A gitlab project was created on git.mpeg.expert to track issues and edits to the technical report [1].

A discussion was held about other issues.  The group determined that additional updates and/or fixes should be included in an input document.  Examples are the order of precedence and cropping order.

Finally, it is reiterated that the following five items should be in the second edition:

- Updated TR text to fix errors and clumsy wording

- Updated tools for the TR

- Video test sequences specific to film grain to be added to the TR

- Metrics – discussion and/or survey of available techniques or metrics

- Subjective testing of Film Grain

The following are worthy of discussion for the second edition:

- Conformance – this is interesting due to the process tied around an SEI message

- Analysis – preprocessing analysis (parameter estimation)

These are additional topics for consideration and discussion:

- The possible creation of Engineering Guidelines (EG) or Recommended Practice(RP) relating to this topic – This raises the question of what would be useful information for such guiding documents?

- Future parameters and signaling – not a v2 topic

- Related contributions

Five contributions related to AHG13 were identified as of 01/13/2025.

- One contribution was the AHG report:

  - JVET-AK0013 JVET AHG report: Film grain technologies (AHG13)

- Four other contributions were uploaded at the time of the report drafting:

  - JVET-AK0058 AHG9/AHG13: Comments on Film grain regions characteristics SEI message

  - JVET-AK0169 AHG13: film grain synthesis improvement using frequency shaping

  - JVET-AK0197 AHG13: on picture size for the interpretation of the FGC SEI message

  - JVET-AK0211 AHG9/AHG13: Proposed Film Grain Region SEI message for version 5 of VSEI

  - Contributions

There were four contributions uploaded other than the AHG report.

    - JVET-AK0058 AHG9/AHG13: Comments on Film grain regions characteristics SEI message

While the model adaptation method proposed in JVET-AH0166 [1] was adopted into the VSEI TuC doc, i.e. JVET-AJ2032 [2], some model-adaptive modifications to the related syntax and semantics in the Film Grain Regions Characteristics (FGR) SEI message are proposed in this contribution to ensure the standard usage of adaptive film grain technology.

    - JVET-AK0169 AHG13: film grain synthesis improvement using frequency shaping

This contribution reports about the implementation of a soft frequency window instead of a rectangular frequency cutoff, when using film grain synthesis based on the FGC SEI message in frequency-filtering mode. This is said to visually improve the synthetic grain.

    - JVET-AK0197 AHG13: on picture size for the interpretation of the FGC SEI message

This contribution proposes to amend the VVC specification to refer to the SPS instead of the PPS to define picture resolution for the interpretation of the film grain characteristics SEI message, the intent being to describe film grain characteristics at source picture resolution independently of encoder decisions to perform reduced-resolution coding.

It is also proposed to indicate that unspecified cropping and padding can be applied as needed.

    - JVET-AK0211 AHG9/AHG13: Proposed Film Grain Region SEI message for version 5 of VSEI

This contribution proposes to include the Film Grain Region (FGR) SEI message into the working draft of Version 5 of VSEI, should it be established at this meeting. The FGR SEI message is meant to be available for HEVC and VVC.

- Recommendations

The AHG recommends:

- Request opening a second edition for the TR;

- the related input contributions are reviewed;

- any liaisons are reviewed;

- testing of FGS be discussed;

- continued conformance discussion;

- SEI message extensions;

- discussion topics for an EG/RP; and

- continue the study of film grain technologies in JVET.

The text of JVET-AJ2020 is under preparation, could become basis of the ITU output.

JVET-AK0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, Y. Li, Y. Li, M. Santamaria, J. N. Shingala, Z. Xie (vice-chairs)]

- Software development

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, enabling deblocking in the RDO, JVET-AH0054 with improved MCTF, JVET-AI0124 for reference picture alignment.

NNVC-11.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc is used for NNVC performance evaluation.

  - Software changes

  - NNVC-11.0

Several commits were merged in the NNVC repository. The following changes were integrated:

| Merge request | Contribution | Description |
| --- | --- | --- |
| 280/284/286 | JVET-AJ0066 | inference for reduced inputs |
| 272 | JVET-AJ0056 | wavelet loss NNSR |
| 283 | JVET-AJ0054 | training scripts fix |
| 281 | JVET-AJ0054 | adding models |
| 273 | JVET-AJ0054 | scripts + NNVC code |
| 269 | JVET-AJ0124 | QP per block |
| 271 | JVET-AJ0066 | split block for HOP5 |
| 285/274 | JVET-AJ0125 | SADL update |
| 277 | fix | memory error |
| 279 | fix | memory error |
| 275 | fix | gitlab CI |
| 276 | fix | xLOP backward compatiblity |
| 270 | fix | dual tree data extraction |
| 265 | fix | backward compatible SPS |

In SADL, the following changes were integrated:

| Merge request | Contribution | Description |
| --- | --- | --- |
| 126 |  | more info on function calls |
| 125 | JVET-AJ0054 related | conv2D to support pad 0 and improve performance (no skip border anymore) |
| 121/137 |  | speed up for LOP |
| 115 | JVET-AJ0361 | add code to get layers anaysis |
| 118 | JVET-AJ0125 | info in model |
| 112 | JVET-AJ0122 | sparse packed |
| 111 | JVET-AJ0106 | batchnorm experimental |
| 131 | fix | gcc 13 bug |
| 119 | fix | MAC count for sparse packed |
| 117 | fix | allow multiple load of models |
| 116 | fix | correct stack overflow |
| 113 | fix | fix large kernel mismatch |
| 128/136 | fix | correct CI and build |

  - Software version

NNVC-11.0 was tagged December 10th, 2024 (bit accurate with 11rc)

NNVC-11.0rc was tagged December 3rd, 2024

NNVC-10.0 was tagged August 9th, 2024

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

See configurations section for naming convention.

Note: For InterDigital results, the encoding times might not be correct since encoding was done before a bit accurate update of the models. Decoding time are using the last version of NNVC-11.0, using an homogenous cluster with some speed variation.

  - Comparison to VTM

    - NNVC-10.0 VTM vs NNVC-11.0 VTM

VTM configuration of NNVC is not changed between version 10.0 and 11.0.

Note: 1 redundant bit in SPS is removed between the 2 versions (see MR 265).

    - NNVC-11.0 VTM vs NNVC-11.0 anchor

The NNVC-11.0 anchor includes LOP.4 filter and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.89% | -13.05% | -14.28% | -9.43% | -14.76% | -16.00% | 119% | 3884% |
| Class A2 | -7.88% | -14.40% | -9.69% | -8.36% | -15.17% | -9.53% | 117% | 3665% |
| Class B | -7.29% | -15.44% | -14.36% | -7.96% | -17.45% | -16.84% | 120% | 3744% |
| Class C | -6.91% | -13.77% | -13.51% | -7.78% | -14.78% | -15.09% | 117% | 3336% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.63% | -14.31% | -13.19% | -8.29% | -15.74% | -14.74% | 118% | 3642% |
| Class D | -7.13% | -13.31% | -12.30% | -6.87% | -14.56% | -13.06% | 113% | 3052% |
| Class F | -4.02% | -8.28% | -7.29% | -5.03% | -10.94% | -10.87% | 122% | 1712% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -5.85% | -11.54% | -11.01% | -6.96% | -11.74% | -13.13% | 107% | 3709% |
| Class C | -5.60% | -11.57% | -10.75% | -7.02% | -10.21% | -9.09% | 102% | 3169% |
| Class E | -6.52% | -4.30% | -7.75% | -8.50% | -1.92% | -6.25% | 109% | 3579% |
| Overall | -5.94% | -9.74% | -10.11% | -7.36% | -8.78% | -10.07% | 106% | 3488% |
| Class D | -6.33% | -9.37% | -8.05% | -7.09% | -4.97% | -6.54% | 98% | 3003% |
| Class F | -3.33% | -5.41% | -5.64% | -5.52% | -5.27% | -7.43% | 107% | 1819% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.32% | -14.44% | -13.28% | -7.35% | -16.25% | -16.87% | 111% | 3619% |
| Class C | -6.00% | -13.66% | -12.66% | -7.21% | -12.92% | -13.31% | 103% | 3033% |
| Class E | -7.36% | -5.63% | -8.77% | -9.47% | -4.59% | -7.92% | 113% | 3656% |
| Overall | -6.47% | -11.98% | -11.95% | -7.83% | -12.23% | -13.45% | 109% | 3421% |
| Class D | -6.61% | -11.17% | -9.28% | -7.42% | -7.90% | -8.60% | 99% | 2774% |
| Class F | -3.64% | -6.93% | -6.80% | -5.49% | -8.16% | -11.72% | 108% | 1679% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.19% | -15.70% | -16.76% | -9.37% | -18.57% | -18.15% | 171% | 2838% |
| Class A2 | -7.96% | -15.42% | -12.22% | -8.53% | -16.09% | -11.91% | 170% | 2466% |
| Class B | -7.86% | -15.36% | -16.39% | -8.27% | -17.73% | -18.36% | 172% | 2431% |
| Class C | -7.71% | -14.17% | -14.72% | -8.57% | -17.34% | -17.80% | 154% | 1875% |
| Class E | -11.22% | -17.07% | -18.08% | -11.90% | -17.26% | -19.04% | 166% | 2611% |
| Overall | -8.63% | -15.45% | -15.67% | -9.17% | -17.43% | -17.24% | 166% | 2389% |
| Class D | -7.61% | -12.34% | -13.39% | -7.97% | -16.35% | -17.00% | 146% | 1679% |
| Class F | -5.39% | -9.85% | -9.70% | -5.81% | -12.75% | -13.41% | 136% | 1955% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Note: Results from Interdigital, crosschecked by Qualcomm (AI/RA).

    - NNVC-11.0 VTM mode vs NNVC-11.0 HOP.5

The NNVC-11.0 where LOP.4 filter is replaced by HOP.5 and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -15.60% | -13.69% | -21.60% | -17.26% | -19.06% | -23.79% | 276% | 123461% |
| Class A2 | -15.13% | -20.72% | -26.68% | -14.49% | -20.24% | -23.74% | 257% | 112524% |
| Class B | -13.10% | -24.21% | -17.36% | -11.95% | -22.58% | -16.70% | 272% | 117302% |
| Class C | -13.92% | -17.30% | -16.86% | -12.52% | -14.58% | -14.20% | 215% | 103138% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -14.22% | -19.57% | -19.94% | -13.67% | -19.28% | -18.86% | 253% | 113563% |
| Class D | -14.83% | -17.41% | -17.34% | -11.03% | -12.96% | -11.40% | 208% | 98862% |
| Class F | -8.99% | -11.35% | -10.48% | -9.24% | -12.20% | -10.80% | 371% | 53532% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -9.82% | -12.00% | -6.93% | -9.30% | -8.96% | -4.82% | 291% | #NUM! |
| Class C | -11.65% | -9.46% | -6.96% | -10.48% | -4.78% | -0.10% | 232% | 109681% |
| Class E | -10.33% | -10.68% | -5.57% | -10.01% | -10.57% | -4.81% | 588% | 101925% |
| Overall | -10.56% | -10.82% | -6.60% | -9.87% | -7.97% | -3.24% | 321% | #NUM! |
| Class D | -12.84% | -7.60% | -0.50% | -10.53% | 1.16% | 14.54% | 223% | 109822% |
| Class F | -9.00% | -9.70% | -8.81% | -10.13% | -10.56% | -8.72% | 411% | 66305% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -11.07% | -15.39% | -9.62% | -10.31% | -12.22% | -7.20% | 358% | #NUM! |
| Class C | -12.58% | -11.57% | -9.68% | -10.96% | -6.13% | -2.93% | 275% | 105881% |
| Class E | -11.64% | -10.69% | -4.73% | -11.55% | -10.77% | -3.90% | 748% | 111964% |
| Overall | -11.72% | -12.94% | -8.42% | -10.84% | -9.83% | -4.95% | 394% | #NUM! |
| Class D | -13.93% | -10.90% | -5.70% | -11.51% | -1.74% | 9.66% | 265% | 101795% |
| Class F | -9.78% | -12.06% | -11.22% | -10.72% | -13.43% | -12.18% | 484% | 65005% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13.53% | -10.72% | -17.20% | -15.00% | -16.58% | -19.47% | 346% | 103002% |
| Class A2 | -13.56% | -17.53% | -21.27% | -14.30% | -18.54% | -18.26% | 252% | 82417% |
| Class B | -12.13% | -16.96% | -14.32% | -12.03% | -17.06% | -14.79% | 241% | 77815% |
| Class C | -13.07% | -14.27% | -16.16% | -12.79% | -13.89% | -15.46% | 188% | 52191% |
| Class E | -16.66% | -19.95% | -18.84% | -16.38% | -18.82% | -19.54% | 253% | 88218% |
| Overall | -13.56% | -15.91% | -17.12% | -13.80% | -16.81% | -17.09% | 246% | 76922% |
| Class D | -12.19% | -12.68% | -15.79% | -11.24% | -12.70% | -14.68% | 172% | 47123% |
| Class F | -9.96% | -12.44% | -10.41% | -9.68% | -12.47% | -10.99% | 168% | 65649% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-11.0 VTM mode vs NNVC-10.0 VLOP

The NNVC-11.0 where LOP.4 filter is replaced by VLOP3 and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.18% | -5.57% | -5.74% | -7.66% | -6.78% | -6.23% | 116% | 1811% |
| Class A2 | -5.92% | -6.40% | -4.16% | -6.51% | -7.01% | -3.96% | 115% | 1643% |
| Class B | -5.45% | -7.56% | -6.50% | -6.25% | -8.21% | -7.34% | 118% | 1689% |
| Class C | -4.95% | -6.49% | -5.83% | -6.01% | -7.79% | -6.45% | 116% | 1504% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -5.75% | -6.65% | -5.70% | -6.52% | -7.57% | -6.20% | 116% | 1651% |
| Class D | -4.87% | -5.48% | -4.91% | -5.46% | -6.03% | -5.38% | 112% | 1396% |
| Class F | -2.93% | -3.48% | -3.22% | -3.86% | -5.59% | -4.53% | 118% | 773% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -3.93% | -6.35% | -3.35% | -5.26% | -8.10% | -7.25% | 106% | 1608% |
| Class C | -3.46% | -5.60% | -4.05% | -5.42% | -9.16% | -5.37% | 105% | 1414% |
| Class E | -4.05% | -1.92% | -1.25% | -6.47% | -4.57% | -2.74% | 107% | 1401% |
| Overall | -3.80% | -4.99% | -3.06% | -5.61% | -7.57% | -5.50% | 106% | 1488% |
| Class D | -3.93% | -4.00% | -1.90% | -5.24% | -9.44% | -4.95% | 103% | 1364% |
| Class F | -1.89% | -1.37% | -1.46% | -3.53% | -4.22% | -3.77% | 107% | 803% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -4.11% | -7.42% | -4.07% | -5.08% | -8.88% | -8.13% | 109% | 1489% |
| Class C | -3.75% | -6.75% | -4.65% | -5.32% | -9.92% | -5.75% | 105% | 1314% |
| Class E | -4.60% | -2.57% | 0.10% | -7.08% | -5.17% | -1.87% | 108% | 1448% |
| Overall | -4.11% | -5.98% | -3.22% | -5.66% | -8.30% | -5.77% | 108% | 1418% |
| Class D | -4.16% | -5.15% | -2.02% | -5.21% | -9.32% | -6.05% | 104% | 1243% |
| Class F | -1.96% | -1.86% | -2.16% | -3.57% | -5.44% | -6.06% | 108% | 751% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.24% | -8.99% | -8.35% | -8.36% | -10.77% | -9.22% | 166% | 1475% |
| Class A2 | -6.80% | -8.35% | -6.85% | -7.16% | -8.55% | -6.21% | 168% | 1333% |
| Class B | -6.73% | -8.80% | -8.72% | -7.11% | -9.70% | -9.50% | 167% | 1339% |
| Class C | -6.25% | -7.86% | -7.59% | -7.16% | -9.37% | -8.65% | 156% | 1109% |
| Class E | -9.34% | -10.06% | -9.98% | -10.08% | -9.77% | -9.18% | 166% | 1400% |
| Overall | -7.32% | -8.76% | -8.30% | -7.83% | -9.63% | -8.67% | 165% | 1314% |
| Class D | -6.10% | -6.89% | -6.85% | -6.61% | -8.78% | -8.50% | 144% | 1041% |
| Class F | -4.31% | -5.60% | -5.58% | -4.75% | -7.00% | -6.89% | 134% | 1008% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Note: Results from Interdigital, crosschecked by Qualcomm (RA/AI)

    - NNVC-11.0 VTM vs NNVC-11.0 NNSR

Note: only class A sequences are changed, LDx configuration are thus omitted.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.07% | -10.17% | -10.56% | -19.18% | -21.21% | -20.88% |  |  |
| Class A2 | -8.86% | -6.49% | -1.95% | -17.40% | -15.13% | -14.57% |  |  |
| Class B | -7.29% | -15.44% | -14.36% | -7.96% | -17.45% | -16.84% |  |  |
| Class C | -6.91% | -13.77% | -13.51% | -7.78% | -14.78% | -15.09% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.46% | -12.15% | -10.89% | -12.04% | -17.02% | -16.73% |  |  |
| Class D | -7.13% | -13.31% | -12.30% | -6.87% | -14.56% | -13.06% |  |  |
| Class F | -4.02% | -8.28% | -7.29% | -5.03% | -10.94% | -10.87% |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.98% | -9.96% | -11.73% | -13.82% | -13.69% | -13.78% |  |  |
| Class A2 | -11.00% | -4.54% | -1.25% | -17.09% | -6.78% | -2.52% |  |  |
| Class B | -7.86% | -15.36% | -16.39% | -8.27% | -17.73% | -18.36% |  |  |
| Class C | -7.71% | -14.17% | -14.72% | -8.57% | -17.34% | -17.80% |  |  |
| Class E | -11.22% | -17.07% | -18.08% | -11.90% | -17.26% | -19.04% |  |  |
| Overall | -9.43% | -12.68% | -13.00% | -11.34% | -15.07% | -14.95% |  |  |
| Class D | -7.61% | -12.34% | -13.39% | -7.97% | -16.35% | -17.00% |  |  |
| Class F | -5.39% | -9.85% | -9.70% | -5.81% | -12.75% | -13.41% |  |  |

Note: Results from HUTS, crosschecked by xxx.

  - Comparison to NNVC-11.0 anchor

    - NNVC-10 anchor vs NNVC-11.0 anchor

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.14% | -1.20% | -1.87% | 0.00% | -1.34% | -1.49% | 102% | 114% |
| Class A2 | -0.24% | -1.88% | -1.32% | -0.07% | -1.92% | -1.01% | 102% | 112% |
| Class B | -0.34% | -0.75% | -2.09% | -0.28% | -1.45% | -1.99% | 102% | 111% |
| Class C | -0.33% | 0.00% | -1.67% | -0.17% | -0.07% | -2.14% | 102% | 108% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.28% | -0.86% | -1.78% | -0.15% | -1.15% | -1.73% | 102% | 111% |
| Class D | -0.46% | -0.72% | -1.40% | -0.14% | -1.62% | -1.06% | 101% | 101% |
| Class F | -0.12% | -0.44% | -0.96% | 0.06% | 0.38% | -1.42% | 103% | 113% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.40% | -1.34% | -2.79% | -0.46% | -1.96% | 0.03% | 106% | 121% |
| Class C | -0.32% | 0.26% | -1.75% | -0.02% | -0.15% | -0.92% | 103% | 110% |
| Class E | -0.60% | 0.71% | -3.87% | -0.48% | 1.63% | -1.07% | 102% | 112% |
| Overall | -0.42% | -0.29% | -2.72% | -0.32% | -0.46% | -0.56% | 104% | 115% |
| Class D | -0.48% | 0.33% | -0.41% | -0.15% | 0.04% | 2.64% | 101% | 108% |
| Class F | -0.19% | 1.21% | -3.04% | 0.07% | 1.53% | -0.77% | 104% | 116% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.41% | -1.50% | -3.21% | -0.51% | -2.89% | -0.80% | 97% | 108% |
| Class C | -0.37% | 0.34% | -1.61% | -0.01% | 0.02% | -1.67% | 94% | 102% |
| Class E | -0.54% | 0.95% | -4.45% | -0.11% | 1.37% | 0.39% | 96% | 103% |
| Overall | -0.43% | -0.28% | -2.99% | -0.24% | -0.86% | -0.79% | 96% | 105% |
| Class D | -0.36% | 0.85% | 0.73% | -0.07% | 1.09% | 4.08% | 94% | 100% |
| Class F | -0.22% | 0.77% | -3.27% | 0.08% | 1.77% | -1.80% | 94% | 100% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.05% | -1.88% | -2.31% | -0.01% | -2.32% | -2.26% | 101% | 113% |
| Class A2 | -0.14% | -1.87% | -1.37% | -0.07% | -2.11% | -1.46% | 100% | 116% |
| Class B | -0.19% | -1.41% | -2.32% | -0.16% | -2.03% | -2.44% | 100% | 112% |
| Class C | -0.27% | -0.86% | -1.98% | -0.20% | -1.58% | -2.60% | 99% | 115% |
| Class E | -0.40% | -1.29% | -2.22% | -0.17% | -1.36% | -2.25% | 98% | 110% |
| Overall | -0.19% | -1.42% | -2.07% | -0.13% | -1.88% | -2.25% | 100% | 113% |
| Class D | -0.37% | -0.74% | -1.60% | -0.27% | -1.86% | -2.31% | 97% | 108% |
| Class F | -0.24% | -0.43% | -1.15% | -0.18% | -0.52% | -1.94% | 103% | 113% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Note: Results from InterDigital, crosschecked by xxx.

  - NNVC-11.0 vs NNVC-11.0 HOP5

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.35% | -1.03% | -8.61% | -8.61% | -5.52% | -9.43% | 232% | 3179% |
| Class A2 | -7.85% | -7.27% | -18.72% | -6.64% | -5.85% | -15.80% | 219% | 3070% |
| Class B | -6.29% | -10.73% | -3.66% | -4.32% | -6.65% | 0.16% | 226% | 3133% |
| Class C | -7.55% | -4.04% | -3.85% | -5.13% | 0.33% | 1.20% | 184% | 3092% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.15% | -6.32% | -7.71% | -5.86% | -4.40% | -4.67% | 214% | 3118% |
| Class D | -8.31% | -4.78% | -5.96% | -4.44% | 1.98% | 1.84% | 184% | 3239% |
| Class F | -5.26% | -3.37% | -3.45% | -4.48% | -1.48% | 0.17% | 303% | 3127% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -4.23% | -0.53% | 4.35% | -2.53% | 3.32% | 9.29% | 271% | #NUM! |
| Class C | -6.42% | 2.52% | 4.20% | -3.74% | 6.39% | 10.14% | 228% | 3461% |
| Class E | -4.07% | -6.15% | 2.40% | -1.61% | -8.13% | 1.77% | 542% | 2848% |
| Overall | -4.92% | -0.92% | 3.81% | -2.70% | 1.48% | 7.70% | 304% | #NUM! |
| Class D | -6.95% | 1.51% | 8.19% | -3.71% | 6.34% | 22.55% | 227% | 3657% |
| Class F | -5.91% | -4.45% | -3.40% | -4.90% | -5.39% | -1.33% | 385% | 3646% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -5.10% | -1.19% | 3.88% | -3.20% | 4.82% | 11.08% | 324% | #NUM! |
| Class C | -7.02% | 2.51% | 3.22% | -4.07% | 7.91% | 11.49% | 267% | 3491% |
| Class E | -4.62% | -4.93% | 4.43% | -2.23% | -6.10% | 4.33% | 664% | 3062% |
| Overall | -5.62% | -0.89% | 3.80% | -3.25% | 3.12% | 9.53% | 363% | #NUM! |
| Class D | -7.84% | 0.30% | 3.90% | -4.42% | 7.44% | 18.18% | 268% | 3670% |
| Class F | -6.43% | -5.48% | -4.78% | -5.56% | -5.55% | -0.50% | 447% | 3872% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.76% | 6.00% | -0.40% | -6.14% | 2.71% | -1.60% | 203% | 3629% |
| Class A2 | -6.10% | -1.81% | -10.06% | -6.25% | -2.09% | -7.04% | 148% | 3342% |
| Class B | -4.66% | -1.69% | 2.57% | -4.07% | 0.95% | 4.58% | 140% | 3201% |
| Class C | -5.86% | 0.04% | -1.69% | -4.60% | 4.53% | 2.95% | 122% | 2783% |
| Class E | -6.15% | -3.46% | -0.87% | -4.99% | -1.85% | -0.58% | 152% | 3379% |
| Overall | -5.43% | -0.34% | -1.55% | -5.05% | 1.06% | 0.39% | 148% | 3220% |
| Class D | -4.98% | -0.38% | -2.93% | -3.57% | 4.63% | 2.55% | 117% | 2806% |
| Class F | -4.92% | -2.91% | -0.75% | -4.10% | 0.39% | 3.08% | 123% | 3358% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

  - NNVC-11.0 vs NNVC-11.0 VLOP

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.88% | 9.08% | 10.00% | 1.96% | 10.11% | 11.74% | 97% | 47% |
| Class A2 | 2.14% | 9.96% | 6.25% | 2.00% | 10.40% | 6.23% | 98% | 45% |
| Class B | 2.00% | 9.53% | 9.27% | 1.85% | 11.67% | 11.62% | 98% | 45% |
| Class C | 2.11% | 8.60% | 9.03% | 1.91% | 8.58% | 10.47% | 99% | 45% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 2.03% | 9.28% | 8.75% | 1.92% | 10.28% | 10.26% | 98% | 45% |
| Class D | 2.44% | 9.11% | 8.66% | 1.50% | 10.26% | 9.13% | 99% | 46% |
| Class F | 1.17% | 5.49% | 4.47% | 1.24% | 6.42% | 7.53% | 97% | 45% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 2.05% | 5.96% | 8.56% | 1.84% | 4.36% | 6.98% | 99% | 43% |
| Class C | 2.28% | 6.82% | 7.66% | 1.75% | 1.28% | 4.50% | 104% | 45% |
| Class E | 2.64% | 2.49% | 7.27% | 2.17% | -2.59% | 4.22% | 99% | 39% |
| Overall | 2.27% | 5.38% | 7.94% | 1.89% | 1.59% | 5.46% | 100% | 43% |
| Class D | 2.57% | 5.76% | 7.05% | 2.01% | -3.04% | 2.51% | 105% | 45% |
| Class F | 1.50% | 4.33% | 4.47% | 2.12% | 1.20% | 4.10% | 101% | 44% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 2.36% | 8.27% | 10.58% | 2.46% | 8.96% | 10.61% | 99% | 41% |
| Class C | 2.40% | 8.05% | 9.34% | 2.05% | 3.44% | 8.79% | 102% | 43% |
| Class E | 2.96% | 3.20% | 9.79% | 2.57% | -0.49% | 6.82% | 96% | 40% |
| Overall | 2.53% | 6.93% | 9.97% | 2.35% | 4.76% | 9.06% | 99% | 41% |
| Class D | 2.62% | 7.10% | 8.13% | 2.40% | 0.58% | 3.11% | 105% | 45% |
| Class F | 1.76% | 5.52% | 5.01% | 2.06% | 3.28% | 6.59% | 100% | 45% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.04% | 8.43% | 10.37% | 1.10% | 10.43% | 11.25% | 98% | 52% |
| Class A2 | 1.26% | 8.85% | 6.31% | 1.48% | 9.53% | 6.54% | 99% | 54% |
| Class B | 1.24% | 7.89% | 9.24% | 1.24% | 10.09% | 10.98% | 98% | 55% |
| Class C | 1.61% | 7.61% | 8.62% | 1.53% | 10.19% | 11.56% | 101% | 59% |
| Class E | 2.13% | 8.61% | 10.19% | 2.00% | 9.37% | 12.42% | 100% | 54% |
| Overall | 1.44% | 8.20% | 8.96% | 1.45% | 9.96% | 10.65% | 99% | 55% |
| Class D | 1.65% | 6.40% | 7.88% | 1.47% | 9.31% | 10.72% | 99% | 62% |
| Class F | 1.17% | 4.99% | 4.74% | 1.12% | 7.10% | 7.88% | 99% | 52% |

  - NNVC-11.0 vs NNVC-11.0+NNSR

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.24% | 3.32% | 4.31% |  |  |  |  |  |
| Class A2 | -0.85% | 8.35% | 7.86% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.82% | 2.33% | 2.43% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |  |  |  |
| Class H | #VALUE! | #VALUE! | #VALUE! |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | PSNR Overlap |
| Class A1 | -1.86% | 6.35% | 5.82% | -4.71% | 5.66% | 5.37% | 131% | #NUM! | #DIV/0! | 97% |
| Class A2 | -3.19% | 11.65% | 11.54% | -9.14% | 9.88% | 9.54% | 95% | #NUM! | #DIV/0! | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% | #DIV/0! | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% | #DIV/0! | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% | #DIV/0! | 100% |
| Overall | -0.84% | 3.00% | 2.89% | -2.31% | 2.59% | 2.49% | 104% | #NUM! | #DIV/0! | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% | #DIV/0! | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% | #DIV/0! | 100% |

Results from HUTS, crosschecked by xxx

- Contributions

We have 3 contributions for AhG14 and 1 telco report.

| JVET-AK0134 | AhG14: SADL update |  | F. Galpin (Interdigital) |
| --- | --- | --- | --- |
| JVET-AK0151 | AhG14: On the impact of explicit full SIMD implementation for 1x3 and 3x1 DW convolution | Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm), |  |
| JVET-AK0257 | AHG14: The extension of SADL library | N. Fu, W. Bao, Z. Chen (Wuhan Univ.) |  |

- Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration |
| --- | --- | --- |
| NNVC VTM mode | none | encoder_xxx_vtm.cfg |
| NNVC Anchor/EE1 | Intra Pred + LOP.4 | encoder_xxx_nnvc.cfg |
| NNVC. HOP | Intra Pred + HOP.5 | encoder_xxx_nnvc.cfg + nn-based/HOP5.cfg |
| NNVC. VLOP | Intra Pred + VLOP.3 | encoder_xxx_vtm.cfg + nn-based/vlop3.cfg |
| SR | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg |
| PF | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |
| ALOP | Intra pred+adaptive LOP | encoder_xxx_vtm.cfg +nn-based/intra.cfg+xxx |

Deprecated options:

| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg |
| --- | --- | --- |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg |
| set0+rdo | Loop filter set #0 + Rdo | nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Temporal filter | nn-based/NnlfOption_2.cfg + --NnlfSet1Multiframe=1 |

- Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.

JVET-AK0015 JVET AHG report: Gaming content compression (AHG15) [S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang (vice chairs)]

Work in between meetings

- Anchor generation

- Software repositories (at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gcc)

  - Support of reading depth and motion maps (JVET-AJ0300)

  - Update to latest ECM version

- Added repository for scripts for verification of auxiliary information for gaming (JVET-AJ0137)

- Bug reported in ECM (https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues/84)

  - Encoder crash for RA on BaoleiYard, QPs 22, 27, 32, 37 and 42.

Test results

Test results were obtained according to JVET-AJ2027. The list of sequences/classes is copied for convenience below:

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 10 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
| G3 (HDR) | Level1_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | Darktree_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | ARPG2_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 10 | O | O | O |

For class G3 the HDR versions of Level1 and Darktree are still missing.

  - VTM-11.0ecm15.0 vs ECM-15.0

    - SDR

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -11.33% | -23.56% | -18.28% | 895.65% | 348.38% | #REF! |
| Class G2 | -18.36% | -22.51% | -26.64% | 1240.13% | 691.59% | #REF! |
| Overall | -14.45% | -23.09% | -21.99% | 1035.02% | 472.50% | #REF! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -27.08% | -42.28% | -34.12% | 671.18% | 1193.40% | #REF! |
| Class G2* | -23.98% | -35.97% | -37.25% | 869.45% | 1515.22% | #REF! |
| Overall | -25.92% | -39.91% | -35.29% | 739.59% | 1305.18% | #REF! |

* ECM crashed for BaoleiYard. Results are averaged without BaoleiYard

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -20.63% | -52.33% | -41.62% | 686.55% | 848.99% | #REF! |
| Class G2* | -18.83% | -55.86% | -55.24% | 1035.63% | 1397.60% | #REF! |
| Overall | -20.11% | -53.34% | -45.51% | 772.11% | 978.93% | #REF! |

* JianlingTemple did not finish in time for ECM. It has been excluded from averages.

|  | Low delay P Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -18.94% | -58.18% | -49.65% | 592.71% | 856.06% | #REF! |
| Class G2 | -15.62% | -54.08% | -54.41% | 923.03% | 1205.41% | #REF! |
| Overall | -17.70% | -56.64% | -51.43% | 699.82% | 973.28% | #REF! |

    - HDR

There are still no HDR versions for Level1 and Darktree. Due to a configuration error AI was run without using HDR config, hence results for AI are omitted.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm15.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -40.39% | -26.60% | -27.79% | -57.87% | -56.17% | -27.99% | -58.41% | -56.40% | 774% | 1435% |

|  | Low Delay B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm15.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -36.84% | -20.85% | -20.09% | -62.36% | -57.80% | -20.36% | -63.00% | -57.71% | 914% | 1602% |

|  | Low Delay P |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11.0ecm15.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -36.67% | -20.56% | -19.78% | -62.15% | -56.97% | -20.02% | -62.71% | -56.94% | 888% | 1158% |

Input contributions

- AHG15: Analysis of ECM 12-bit internal bit depth in gaming content (JVET-AK0082)

- AHG15: Analysis of high-level control of intra prediction methods in gaming content 
(JVET-AK0083)

- AHG15: On the compression of depthmaps from auxiliary data (JVET-AK0170)

Recommendations

The AHG recommends to:

- Review input documents on gaming content compression

- Identify if/how auxiliary information can be used for coding of gaming content

- Further work on unification of auxiliary data format

JVET-AK0016 JVET AHG report: Generative face video compression (AHG16) [Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs)]

- AHG activities

Regarding the mandate on developing and maintaining the GFVC software, the AHG16 GFVC software tool and accompanying usage instructions and exemplar configurations for experimentation are maintained in the GIT repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc. During this AHG period, the AHG16 software was updated by replacing the existing CFTE, FOMM, FV2V, and DAC models with respective multi-resolution models as provided in JVET-AJ0052. Implementation of the GFV SEI message following the syntax defined in the VSEI version 4 draft 4 in JVET-AI2006 has been merged into VTM-23.6.

Regarding testing of higher resolution content, GFVC test conditions and evaluation procedures as defined in JVET-AJ2035 have been updated to include additionally 33 test sequences at 512x512 resolution.

Regarding coordination with AHG9 to develop the GFV SEI message, the GFV and GFVE SEI messages have been integrated into VSEI version 4 draft 4 JVET-AJ2006. At this meeting, 8 input contributions addressing various high-level syntax aspects of the GFV and GFVE SEI messages have been received.

- Related contributions

The following two input contributions to this meeting are related to the activities of AHG16:

- JVET-AK0068, AHG16: GFVC Extension of the VVC Standard [L. Liu, C. Jung (Xidian Univ.)]

- JVET-AK0069, AHG16: QP-Adaptive GFVC [W. Kang, L. Liu, C. Jung (Xidian Univ.)]

The following input contributions are related to the high-level syntax aspects of the generative face video (GFV) and/or generative face video enhancement (GFVE) SEI messages:

- JVET-AK0080, AHG9: Comments on the GFV and GFVE SEI messages [M. M. Hannuksela (Nokia), J. Chen, B. Chen, Y. Ye (Alibaba)]

- JVET-AK0124, AHG9: On timing information and order of pictures in GFV SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

- JVET-AK0127, AHG9: AHG9: On miscellaneous aspects of GFV and DSCI SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

- JVET-AK0128, AHG9: Editorial updates for GFV SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

- JVET-AK0154, AHG: Comments on Generative Face Video SEI [A. C. Sidiya, S. Deshpande (Sharp)]

- JVET-AK0164, AHG9: Supplementation of value range definition and editorial bugs fixes on the GFVE pupil position SEI messages [F. Ma, A. Trioux, F. Yang (Xidian Univ.), B. Li, F. Xing, Z. Wang (Hisense)]

- JVET-AK0238, AHG9: Semantics and syntax fixes for generative face video SEI message [J. Chen, B. Chen, Y. Ye (Alibaba)]

- JVET-AK0239, AHG9: On generative face video enhancement SEI message [J. Chen, B. Chen, Y. Ye (Alibaba)]

- Recommendations

The AHG recommends to:

- Review related contributions, including those on the high-level syntax aspects of the GFV and GFVE SEI messages;

- To continue AHG16 to study GFVC-related topics.

JVET-AK0017 JVET AHG report: Testing of video coding technology beyond CTC (AHG17) [M. Wien (chair), Y. Ye, V. Baroncini, E. Alshina (vice chairs)]

Activities

Two AHG calls were held on 2024-11-20 and 2024-12-18, respectively. The reports of these meetings are available in JVET-AK0042.

  - Identification of test sequences and preparation of bitstreams

The set of sequences to be explored was determined including sequences discussed during breakout activities at the last JVET meeting and new content. The sequences were assigned to volunteers for the encoding task and the corresponding RD data and bitstreams were collected.

The discussion and the results of this activity are reported in JVET-AK0042.

The set of sequences consists of nine classes including SDR_RA_UHD/4K, SDR_RA_HD, SDR_LD_HD, HDR_RA_UHD, HDR_RA_8Kcrop, Gaming_LD_HD. And UGC_RA, making a total of 24 UHD/4K sequences and 27 sequences at HD resolution. For distribution of the encoding tasks, the intended QP range was split into interleaved subset for the VTM and the ECM, respectively. The sets contain one overlapping QP each for crosschecking purposes. The overall encoding effort comprised a total of 1152 rate points with 504 point at UHD/4K and 648 points at HD resolution (384 ECM bitstreams and 768 VTM bitstreams).

| Resolution | QP set Name | Values |
| --- | --- | --- |
| HD | VTM QP set 1 | QP=20:4:48 |
| HD | VTM QP set 2 | QP=22:4:50, 36 |
| HD | ECM QP set 1 | QP=22:8:46 |
| HD | ECM QP set 2 | QP=26:8:50, 38 |
| UHD | VTM QP set 1 | QP=26:4:50 |
| UHD | VTM QP set 2 | QP=28:4:52, 38 |
| UHD | ECM QP set 1 | QP=28:8:52 |
| UHD | ECM QP set 2 | QP=32:8:48, 36 |

Before the meeting, 363 out of 384 ECM bitstreams and 732 out of 768 were uploaded (about 95% for each of the test models.

Input contributions JVET-AK0041, JVET-AK0043, JVET-AK0044, and JVET-AK0045 have been covered in the AHG phase.

Related contributions

| JVET-AK0041 | AHG17: Materials for assisting sequence selection for Non-CTC testing | J. Pardo, J. Sauer (Huawei) |
| --- | --- | --- |
| JVET-AK0042 | AHG17: AhG meeting notes | M. Wien |
| JVET-AK0043 | AHG17: Additional sequences for testing of video coding technology beyond CTC | R.-L. Liao, Y. Ye (Alibaba) |
| JVET-AK0044 | AHG17: Cropped HDR sequences for non-CTC testing | A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV) |
| JVET-AK0045 | AHG17: Test sequences from YouTube | X. Li, Y. Wang, N. Birkbeck, B. Adsumilli (Google) |
| JVET-AK0066 | AHG4/AHG17: 8K HDR sequences as a response to the call for new HDR test materials for future video coding | Y. Li (CMG), Q. Zhang, J. Ning (ABP) |
| JVET-AK0196 | AHG17: Additional sequences for testing of video coding technology beyond CTC | J. Pardo, Y. Sun, T. Solovyev, E. Alshina (Huawei) |
| JVET-AK0218 | AHG17: UGC test sequences from Huawei | Y. Zhao, Y. Sun, P. Liu, Y. Lu, C. Wang, E. Alshina (Huawei) |
| JVET-AK0221 | [AHG17] JVET CTC and non-CTC sequences performance analysis | E. Alshina, J. Pardo, J. Sauer, T. Solovyev (Huawei) |
| JVET-AK0246 | Crosscheck of JVET-AK0196 (AHG17: Additional sequences for testing of video coding technology beyond CTC) | Z. Xiang (Tencent) |
| JVET-AK0282 | AHG4/AHG17: Additional HDR sequences for non-CTC testing | A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV) |

Recommendations

The AHG recommends:

- To review the remaining input contributions related to AHG17 during the meeting.

- To review the provided bitstreams and the corresponding rate-distortion results in a BoG activity in order to assert correctness of the completed encoding tasks.

- To perform informal viewing sessions together with SC 29/AG 5 for checking the characteristics of the provided test sequences, with a focus on new or previously rarely used material, and the potential range of QPs suitable for conducting formal subjective tests.

- To prepare a proposed list of candidate sequences for each category for further consideration for testing.

At maximum roughly 60 sequences in total (51 from the coordinated activity, 6 from 0196 and 0218, 4 additional from an update of 0045)

- As first step, parties who have encoded the sequences shall share the proposed lowest and highest rate points for ECM in an update of the spread sheet by Wednesday morning, decoding those points

- First viewing with a smaller group of people for pre-selection (Wed. afternoon), also to investigate appropriateness of “voting methodology” about the correctness of the suggested rate points and suitability of the sequences.

- BoG to decide on reduction of the set (Thu.)

- First round of “real” test on Friday morning (after larger number of displays would be available)

# Project development (37)

## AHG1: Development, deployment and advertisement of standards (1)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0178 AHG9: Proposed plan for initiating version 5 of VSEI [A. T. Hinds, S. Wenger (Tencent), P. de Lagrange, E. François (InterDigital)]

## AHG2: Text development and errata reporting (0)

This section is kept as a template for future use.

## AHG3: Test conditions (0)

This section is kept as a template for future use.

## AHG3: Software development (0)

This section is kept as a template for future use.

## AHG4: Subjective quality testing and verification testing (0)

This section is kept as a template for future use.

## AHG4: Test and training material (0)

This section is kept as a template for future use.

Proposals on new test materials are also included in 4.17 and 5.1.4

## Codec performance under non-CTC conditions (5)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0057 [AHG3]?: NAL unit loss software [S. Wenger, A. Hinds, G. Teniou (Tencent)]

JVET-AK0193 Low latency Wi-Fi transmission simulation and suggestions on codec requirements [S. Ikonin, X. Ma, I. Gribushin, M. Sychev, V. Khamidullin, R. Shabaev, E. Alshina (Huawei)]

JVET-AK0201 On additional operation point for the exploration model [T. Solovyev, J. Pardo, J. Sauer, Z. Li, E. Alshina (Huawei)]

JVET-AK0232 On real-time test condition for limited uplink/storage use case and MTT depth suggestions on common test condition [T. Ikai (Sharp)]

JVET-AK0236 Use cases and requirement of next generation video coding standard for broadcasting services [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

## AHG5: Conformance test development (0)

This section is kept as a template for future use.

## AHG7: ECM tool assessment (6)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0047 AHG7: Complexity and memory footprint assessment of PDP, Neural Network-based Intra Prediction, MTS and LFNST/NSPT, and ALF in ECM-15 [G. Boisson, T. Dumas, F. Galpin, S. Puri, K. Naser, C. Bonnineau (InterDigital)]

JVET-AK0048 Report of AHG7 conference call [X. Li]

JVET-AK0049 AHG7: Analysis of MTS memory footprint and complexity [J. Gan (OPPO)]

JVET-AK0067 AHG7: Tool-off test results of LFNST extension and NSPT [M. Koo, J. Zhao, J. Lim, S. Kim (LGE)]

JVET-AK0092 AHG7: Inter LDB tool assessments for each ECM version [R. Ishimoto, Z. Fan, T. Chujoh, T. Ikai (Sharp)]

JVET-AK0138 AHG7: Intra tool assessments between ECM14 and ECM15 [C.-Y. Teng, K.-W. Liang, Y.-H. Lin, Y.-C. Yang, T. Ikai (Sharp)]

JVET-AK0228 AHG7: Tool-off test results of ALF fixed filters [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-AK0319 Proposing an Ad Hoc Group on Enhanced Complexity Analysis  [A. Stein, S. Ferrara (V-Nova)] [late]

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (3+1)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0094 AHG8: On combination of adaptive temporal resampling, pre-processing, post-processing and ROI-based adaptive QP algorithm for machine vision [S. Wang, J. Chen, Y. Ye, B. Li (Alibaba), S. Wang (CityUHK)]

JVET-AK0302 Crosscheck of JVET-AK0094 (AHG8: On combination of adaptive temporal resampling, pre-processing, post-processing and ROI-based adaptive QP algorithm for machine vision) [H. Zhang (Nokia)] [late]

JVET-AK0122 AHG8: Dense QP results for joint software [C. Hollmann, J. Ström (Ericsson)]

JVET-AK0272 Cross-check of JVET-AK0122 on SFU-HW dataset (AHG8: Dense QP results for joint software) [S. Wang (Alibaba)] [late]

JVET-AK0289 Cross-check of JVET-AK0122 on TVD dataset (AHG8: Dense QP results for joint software) [M. Xu (Tencent)] [late] [miss]

JVET-AK0140 AHG9/AHG8: Showcase for Packed regions information SEI [J. Boyce, H. Zhang, M. M. Hannuksela (Nokia)]

JVET-AK0141 AHG8: Add Packed regions info SEI to TR on coding video for machine consumption [J. Boyce, H. Zhang, M. M. Hannuksela (Nokia)]

## AHG10: Encoding algorithm optimization (2)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0052 AHG10: Teleconference on encoder optimization for multi-layer coding [P. de Lagrange]

JVET-AK0181 AHG10: Suggested upgrade of CTC for random access for VTM [K. Andersson, P. Wennersten, J. Ström, W. Ahmad, V. Shchukin, D. Liu, J. Enhorn (Ericsson)]

## AHG13: Film grain synthesis (2+2)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0058 [AHG9/AHG13] Comments on Film grain regions characteristics SEI message [S. Xie, P. Wu, Y. Gao, Y. Bai, C. Huang (ZTE)]

JVET-AK0169 AHG13: Film grain synthesis improvement using frequency shaping [P. de Lagrange (InterDigital)] [late]

JVET-AK0197 AHG13: on picture size for the interpretation of the FGC SEI message [P. de Lagrange, E. François (InterDigital)]

JVET-AK0211 AHG9/AHG13: Proposed Film Grain Region SEI message for version 5 of VSEI [S. Wenger, G. Teniou, A. T. Hinds (Tencent)]

## Implementation studies (2)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0258 AHG9: Demo of real-time NNPF inference for VVC decoding on consumer laptop [J. Funnell, M. Santamaria, F. Cricri, M. M. Hannuksela, M. Pedzisz, R. Yang, S. Schwarz (Nokia)] [late]

JVET-AK0274 Development of VVC live encoder with content layering capability [S. Nemoto, S. Iwamura (NHK)] [late] [miss]

## Profile/tier/level specification (0)

This section is kept as a template for future use.

## Gaming content compression (AHG15) (4)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0081 AHG6: wPSNR log output for gaming content in ECM [X. Liang, K. Choi (Kyung Hee Univ.)]

JVET-AK0082 AHG15: Analysis of ECM 12-bit internal bit depth in gaming content [X. Liang, K. Choi (Kyung Hee Univ.), S. Lee (Atins)]

JVET-AK0083 AHG15: Analysis of high-level control of intra prediction methods in gaming content [X. Liang, K. Choi (Kyung Hee Univ.), S. Lee (Atins)]

JVET-AK0170 AHG15: On the compression of depth maps from auxiliary data [J. Sauer, T. Solovyev, E. Alshina (Huawei)]

## Generative face video (AHG16) (2)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0068 AHG16: GFVC Extension of the VVC Standard [L. Liu, C. Jung (Xidian Univ.)]

JVET-AK0069 AHG16: QP-Adaptive GFVC [W. Kang, L. Liu, C. Jung (Xidian Univ.)]

## Testing for future standardization (AHG17) (10)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

Some aspects discussed under 4.7 could also be relevant here.

JVET-AK0041 AHG17: Materials for assisting sequence selection for non-CTC testing [J. Pardo, J. Sauer (Huawei)]

JVET-AK0042 AHG17: AhG meeting notes [M. Wien]

JVET-AK0043 AHG17: Additional sequences for testing of video coding technology beyond CTC [R.-L. Liao, Y. Ye (Alibaba)]

JVET-AK0044 AHG17: Cropped HDR sequences for non-CTC testing [A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV)]

JVET-AK0045 AHG17: Test sequences from YouTube [X. Li, Y. Wang, N. Birkbeck, B. Adsumilli (Google)]

JVET-AK0066 AHG4/AHG17: 8K HDR sequences as a response to the call for new HDR test materials for future video coding [Y. Li (CMG), Q. Zhang, J. Ning (ABP)]

JVET-AK0180 On User Generated Content and non-pristine source videos [J. Samuelsson-Allendes (Sharp)]

JVET-AK0196 AHG17: Additional sequences for testing of video coding technology beyond CTC [J. Pardo (Huawei)]

JVET-AK0246 Crosscheck of JVET-AK0196 (AHG17: Additional sequences for testing of video coding technology beyond CTC) [Z. Xiang (Tencent)] [late]

JVET-AK0218 AHG17: UGC test sequences from Huawei [Y. Zhao, Y. Sun, P. Liu, Y. Lu, C. Wang, E. Alshina (Huawei)]

JVET-AK0221 [AHG17] JVET CTC and non-CTC sequences performance analysis [E. Alshina, J. Pardo, J. Sauer, T. Solovyev (Huawei)]

JVET-AK0282 AHG4/AHG17: Additional HDR sequences for non-CTC testing [A. Filippov, J. Konieczny, Z. Zhi, V. Rufitskiy, H. Qin, T. Dong, X. Tang (TCL), S. Shen, Y. Qin, Y. Wang, X. Wang, Y. Guan (NERC-DTV)] [late]

# Low-level tool technology proposals (XX)

## AHG11/AHG14: Neural network-based video coding (23)

### Summary and BoG reports

Contributions in this area were discussed at 1600–1815 on Tuesday 14 Jan. 2025 (chaired by JRO).

JVET-AK0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, R. Chang, Z. Xie (EE coordinators)]

This report summarizes the activities of the Exploration Experiment 1 (EE1) performed between the 36th and 37th JVET meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, evaluate their complexity aspects, and clarify training procedure. Two teleconferences have been conducted during this meeting circle.

NNVC has three major operation points with NN-intra always enabled and NN-based in-loop filters with different level of complexity: High Operational Point (HOP, 471kMAC/pixel), Low Operational Point (LOP, 21.6kMAC/pixel) and Very Low Operational Point (VLOP, 9.9kMAC/pixel). Compared with VVC, NNVC HOP5 provides {14.2%, 19.5%, 19.9%}, NNVC LOP4 provides {7.6%, 14.3%, 13.2%} and NNVC VLOP3 provides {5.8%, 6.6%, 5.7%} bitrate saving for {Y, Cb, Cr} channels in the RA configuration.  Observed run time increase relatively to VVC is 2.6, 1.2, 1.2 (encoding)  and 1136, 36,  17 (decoding) for HOP, LOP and VLOP configurations respectively. EE1 tests were targeting improving the performance complexity trade-off of all three operation points of neural-network based filter and super-resolution.

Introduction

It was decided that the code base for the test should be NNVC-11.0. The anchor for all tests is default configuration of NNVC-11.0 (NN-Intra as adopted from JVET-AI0130 and LOP-4 filter are enabled). Anchor performance results were provided by AhG 14 and have been used by all proponents.

For tests competing with technologies in NNVC-11.0 it was agreed to configure the proposed solution targeting close to existing NNVC tool complexity, but not exceeding it:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- Number of Parameters EE1 test   Number of Parameters NNVC (if possible).

The minor violation of this requirements is allowed for the final model, if the reason is keeping the number of channels a multiple of 16 (or at least 8), since it is very helpful for neural network algorithms implementation on wide range of platforms. Reported results should follow the constraints for evaluation purpose but the final model might violate the constraints.

NN architecture provided in this test description should not be changed, beside minor adjustment for parameters (such as channels number) in order to meet recommendation above.

Exact parameters settings were announced by proponents by 2nd AhG11/14 teleconference on December 20.

Table 1 provides performance and complexity number of several tools’ combinations in NNVC relatively to VVC anchor.

Table 1 Performance of NNVC tools combinations relatively to VVC

| Test | Random Access |  |  |  |  | All Intra |  |  |  |  | Total kMAC/pxl | Total Param (Mprm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |  |  |
| NNIntra+LOP | -7.6% | -14.3% | -13.2% | 1.2 | 36 | -8.6% | -15.4% | -15.7% | 1.7 | 24 | 21.6 | 1.5 |
| NNIntra+HOP | -14.2% | -19.6% | -20% | 2.5 | 1135 | -13.7% | -15.9% | -17.1% | 2.5 | 769 | 471 | 2.7 |
| NNIntra+VLOP | -5.8% | -6.6% | -5.7% | 1.2 | 17 | -7.3% | -8.8% | -8.3% | 1.7 | 13 | 9.9 | 1.4 |
| NNIntra+LOP+SR | -8.5% | -12.1% | -10.9% |  |  | -9.4% | -12.7% | -13.0% |  |  | 26.4 | 1.5 |

List of tests

This round of EE1 tests includes:

- EE1-1: LOP and VLOP in-loop filter

  - EE1-1.1 – Partial Convolution and Over-Parameterization

    - Report: JVET-AK0106 (UESTC), (Transsion), cross-check: JVET-AK0292 (Qualcomm)

  - EE1-1.2 – LOP with improved Attention and residual group

    - Report: JVET-AK0150 (Qualcomm), cross-check JVET-AK0263 (Interdigital), JVET-AK0300 (Bytedance)

  - EE1-1.3 - Multiscale blocks in LOP4 and VLOP3 filters

    - Report: JVET-AK0139 (UESTC), (Transsion), (Qualcomm), (Nokia), cross-check: JVET-AK0166 (Interdigital)

  - EE1-1.4 – Cross-component enhanced LOP filter

    - Report: JVET-AK0195 (Bytedance)  cross-check: JVET-AK0216 (Qualcomm), JVET-AKxxxx (Ittiam)

  - EE1-1.5 - Adaptive skip of LOP filtering based on boundary strength partitions

    - Report: JVET-AK0093 (HYU, ETRI) cross-check: JVET-AK0241 (Huawei)

  - EE1-1.6: Reduced complexity input feature extraction – withdrawn

- EE1-2: NN-inter prediction

  - EE1-2.1 RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding

    - Report: JVET-AK0077 (Xidian Uni) cross-check: ??? (Nokia)

  - EE1-2.2 –  Transformer-Based Reference Frame Synthesis for VVC Inter Coding

    - Report: JVET-AK0078  cross-check: ???

- EE1-4: NN-based super resolution

  - EE1-4.1 – A Neural Network Downscaling Filter for RPR

    - Report: JVET-AK0235 (VIVO), cross-check: JVET-AK0247 (PKU)

Test results summary

Details of each test can be found in attached presentation.

LOP and VLOP filter modifications

In NNVC-11 LOP4 and VLOP3 filters architectures are unified. Just number of channels and back bone blocks are different to match two different level of complexity.

Figure 1. LOP4 and VLOP3 filters in NNVC-11

Proponents of tests EE1-1.1-3 provide their solutions for both LOP and VLOP keeping filters unified:

- EE1-1.1 – Partial Convolution and Over-Parameterization, (UESTC), (Transsion)

  - Complexity reduction proposal. Splits input tensor in channel dimension into two parts, one part goes though convolution, another part skips convolution, so decoding run time reduced 23-27%.

- EE1-1.2 – LOP with improved Attention and residual groups (Qualcomm)

  - Performance improvement proposal. For each pair of Back Bone Blocks skip connection, PReLu and attention block are added. Attention block includes: depth-wise convolution blocks, MaxPool operation, HardSigmoid and bi-linear interpolation. For operations not yet in SADL proponent provides SIMD implementation (subject for confirmation by AhG14). Peak memory usage increases 3 (block level processing). BD-rate reduction 0.3…0.6%.

- EE1-1.3 – Multiscale blocks in LOP4 and VLOP3 filters (UESTC), (Transsion), (Qualcomm), (Nokia)

  - Additionally to EE1-1.1 and EE1-1.2 multi-scale blocks are added (includes EE1-1.1 & EE1-1.2). No memory usage increment compared to EE1-1.2. Froma partially available results peromance benefits are not clear.

Tests EE1-1.4 and EE1-1.5 provide solution and test results only for LOP.

- EE1-1.4 – Cross-component enhanced LOP filter (Bytedance)

  - After the most of NN-based filter operation Luma tensor is added as auxiliary input to the Chroma processing (only ‘tail’ operations of NN-filter use extra input). Approximately 1% Chroma BD-rate gain.

- EE1-1.5 - Adaptive skip of LOP filtering based on boundary strength partitions (HYU, ETRI)

  - NN-filtering operations are by-passed based on number of block boundaries and BS. Decoding run time reduction 35% (LDB) and 23% (RA), almost no drop and gain in Chroma. Non-normative solution is slightly worse.

Table 2 EE1-1: LOP in-loop filter

|  | Random Access |  |  |  |  | All Intra |  |  |  |  | kMAC/pxl |  |  | Num Param, M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Total | Filter | Intra | Total | Filter | Intra |
| NNVC-LOP4 | 0.0% | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% |  |  | 21.6 | 16.8 | 4.8 | 1.5 | 0.248 | 1.3 |
| EE1-1.1.1 |  |  |  |  |  | 0.0% | 0.5% | 0.4% | 101% | 77% | 21.4 | 16.6 | 4.8 | 1.5 | 0.246 | 1.3 |
| EE1-1.1.2 |  |  |  |  |  | 0.0% | -0.1% | -0.3% | 99% | 73% | 21.5 | 16.7 | 4.8 | 1.5 | 0.205 | 1.3 |
| EE1-1.2 | -0.6% | 0.2% | -0.3% |  |  | -0.6% | 0.1% | 0.0% | 99% | 100% | 21.4 | 16.6 | 4.8 | 1.5 | 0.247 | 1.3 |
| EE1-1.3.2 |  |  |  |  |  |  |  |  |  |  | 21.6 | 16.8 | 4.8 | 1.5 | 0.267 | 1.3 |
| EE1-1.4 | 0.0% | -1.5% | -1.3% | 100% | 98% | 0.0% | -0.7% | -0.5% | 100% | 99% | 21.6 | 16.8 | 4.8 | 1.5 | 0.247 | 1.3 |
| EE1-1.5.1.1 | 0.1% | -0.2% | -0.1% | 100% | 77% | 0.0% | -1.5% | -1.0% | 101% | 65% | 21.6 | 16.8 | 4.8 | 1.5 | 0.248 | 1.3 |
| EE1-1.5.2 | 0.2% | -0.1% | 0.0% | 100% | 76% | 0.2% | -1.2% | -0.5% | 101% | 63% | 21.6 | 16.8 | 4.8 | 1.5 | 0.248 | 1.3 |

Table 3 EE1-1: VLOP in-loop filter

|  | Random Access |  |  |  |  | All Intra |  |  |  |  | kMAC/pxl |  |  | Num Param, M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Total | Filter | Intra | Total | Filter | Intra |
| NNVC-VLOP3 | 0.0% | 0.0% | 0.0% |  |  | 0.0% | 0.0% | 0.0% |  |  | 9.9 | 5.1 | 4.8 | 1.4 | 0.067 | 1.3 |
| EE1-1.1 |  |  |  |  |  | 0.2% | 0.7% | 0.8% | 100% | 84% | 9.9 | 5.1 | 4.8 | 1.4 | 0.070 | 1.3 |
| EE1-1.2 | -0.3% | 0.6% | 0.8% |  |  | -0.4% | 0.5% | 0.4% | 100% | 104% | 9.8 | 5.0 | 4.8 | 1.4 | 0.069 | 1.3 |
| EE1-1.3.2 | 0.1% | 0.9% | 1.0% |  |  | -0.1% | 0.6% | 0.9% | 97% | 112% |  | 5.1 |  |  | 0.071 |  |

1-1.1.x could be seen beneficial in AI due to complexity reduction, but preliminary RA results (incomplete) indicate significant losses. Applies to both LOP and VLOP.

1-1.2 shows interesting gain, but training crosscheck JVET-AK0263 not successful so far (larger deviation than usually seen (in particular different luma/chroma balance), and preliminary results indicate even loss in performance in case of VLOP). Reason not clear at this moment; might be caused by using different hyperparameters. It was also commented that perhaps only retraining the last iteration might not be sufficient as there are more significant changes relative to the original LOP design. Another training crosscheck JVET-AK0300 is less deviating for LOP with results available so far. Revisit for further clarification and finalization of training cross-check.

1-1.3 has more complete results in the powerpoint presentation than in the table (results below are from subtest 1.3.2):

For VLOP, worse performance than 1-1.2 (where the latter does not have successful training crosscheck yet). For LOP, less gain in luma, but significantly larger in chroma due to the concept it is understandable that it provides another luma/chroma balance. Training of floating point model was conducted and gave better results than with proponents.

In the original upload, only subtest 1-1.3.2 was reported, in which reparameterization is only applied to luma. Furthermore, the results above are for floating point model, and a new version reports different results with quantized model, with better results for luma, but worse for chroma.

Furthermore, results on 1-1.3.3 are reported, which are better for VLOP and LOP at least for AI and RA. VLOP applies reparameterization also on chroma.

It was asked if the number of channels is multiple of 16 for both luma and chroma.

The following information was requested by proponents of 1-1.2 and 1-1.3:

- Exact architecture description for LOP and VLOP, number of channels for each element of the pipeline

- Hyper parameters used for training (by proponents and cross checker)

- Training time compared to current LOP and VLOP

- Possible reasons for deviations of training cross-check

Revisit after clarifying the questions above, and after crosscheck (also training) has been completed for anything that is requested for adoption.

EE1-1.4 (only relevant for LOP) shows some benefit for chroma, and minor complexity reduction. In the investigated architecture, a latency problem between luma and chroma was resolved. Not applicable to VLOP due to different cropping strategy (earlier cropping only possible in luma).

Number of channels in chroma is increased to 144, but operations are saved due to 1x1 convolution in later stage.

Revisit after completion of training cross-check.

Further study on the aspect of using this in VLOP is recommended.

It was commented that any of the proposals so far would cause larger deviation between LOP and VLOP, where it would however be questionable if a harmonization between the two is really relevant in terms of implementation.

EE1-1.5 is not changing the model, but skips its usage by blockwise decision, which reduces decoding time. Most effective for screen content class F. Worst case complexity would not be reduced, and logic would need to be added if it was normative.

It is interesting to see that NN filter is not very beneficial in quite some cases. It was commented that potentially using it during training would even provide benefit in performance. It was however also commented that decoding runtime is not a serious bottleneck in the context of the exploration.

Decision(SW/non-CTC): Adopt EE1-1.5.2 encoder-only option.

HOP filter modifications

There are no tests for HOP in this round of EE1.

NN-Inter

Two versions of NN-based reference frame generation are studied:

- EE1-2.1 URFS Unified Reference Frame Synthesis (649 kMAC/pxl, 1.6M param)

- EE1-2.2 TRFS Unified Reference Frame Synthesis (261 kMAC/pxl, 2.0 M param)

Training performed on Vimeo 90K (15K video scenes) triplet and BVI-DVC (200 video scenes). Training on Vimeo 90K triplet resulted in 4..5% higher  BD-rate.

Variant with transformers (based on partially available data) has 1.5% lower compression performance (likely training set is not big enough for 2M parameter model).

Performance of both methods in presence of NNVC tools is 12% lower than in absence NNVC tools (test on top of VTM).

No decoder run time reported due to decoder crash. It was commented that this may be due to a modification of GOP structure since NNVC4. Encoder runtimes also appear unreasonable in comparison to NNVC anchor.

Too early for taking action. Training crosscheck would be desirable.

Revisit after more complete results are available.

NN-based super resolution (NNSR)

Existing in NNVC-11.0 NNSR operated as a combination of adaptive resolution coding (scaling factors 1.5 and 2 both supported) and NN-based post-filter for up-sampling.

In tests EE1-4.1:

- EE1-4.1.1 scaling ratio is only 2, NN-based down-sampling has been added.

- EE1-4.1.2 scaling ratio is only 2, NN-based down-sampling and up-sampling filters are trained together,

- EE1-4.1.3 scaling ratio 1.5 added

| Test | Random Access |  |  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| EE1-4.1.1 | -0.01% | -0.10% | -0.09% | ?? | ?? | -0.04% | -0.13% | -0.11% | ?? | ?? |
| EE1-4.1.2 | 0.00% | -0.11% | -0.08% | ?? | ?? | -0.05% | -0.14% | -0.08% | ?? | ?? |
| EE1-4.1.3 | 0.76% | 12.45% | 8.52% | ?? | ?? | 0.03% | 10.15% | 7.28% | ?? | ?? |

Minor gain, mostly in Chroma. EE1-4.1.3 (scaling ratio 1.5) has a big loss likely due to a bug.

Further study recommended.

JVET-AK0050 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### EE1 contributions: Neural network-based video coding (10)

Contributions in this area were discussed in the context of the EE summary report JVET-AK0023.

JVET-AK0077 EE1-2.1: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding [Q. Qin, C. Jung (Xidian Univ.)] [late]

Initial version rejected as “placeholder”

JVET-AK0315 Crosscheck of JVET-AK0077 (EE1-2.1: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) [L. Murn (Nokia)] [late]

JVET-AK0078 EE1-2.2: Transformer-Based Reference Frame Synthesis for VVC Inter Coding [Q. Qin, C. Jung (Xidian Univ.)] [late]

Initial version rejected as “placeholder”

JVET-AK0316 Crosscheck of JVET-AK0078 (EE1-2.2: Transformer-Based Reference Frame Synthesis for VVC Inter Coding) [L. Murn (Nokia)] [late]

JVET-AK0093 EE1-1.5: Adaptive skip of LOP filtering based on boundary strength partitions [H. Kwon, J. Seo, H. Ko (HYU), D. Kim, S.-C. Lim (ETRI)]

JVET-AK0241 Crosscheck of JVET-AK0093 (EE1-1.5: Adaptive skip of LOP filtering based on boundary strength partitions) [J. Pardo, E. Alshina (Huawei)] [late] [miss]

JVET-AK0106 EE1-1.1: Partial Convolution and Over-Parameterization [J. Chi, A. Li, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion)]

JVET-AK0292 Crosscheck of JVET-AJ0106 (EE1-1.1 Partial Convolution and Over-Parameterization) [J. Wang (Qualcomm)] [late] [miss]

JVET-AK0139 EE1-1.3: Multiscale blocks in LOP4 and VLOP3 filters [R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, M. M. Hannuksela (Nokia), Y. Li, D. Rusanovskyy, S. Eadie, M. Karczewicz, J. Wang, L. Kerofsky (Qualcomm), J. Chi, A. Li, C. Zhu, L. Lei, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion)]

JVET-AK0166 Crosscheck of JVET-AK0139 (EE1-1.3: multiscale blocks in LOP4 and VLOP3 filters) [T. Dumas (InterDigital)] [late] [miss]

JVET-AK0150 EE1-1.2: LOP with improved Attention and residual groups [Y. Li, D. Rusanovskyy, S. Eadie, M. Karczewicz, J. Wang, L. Kerofsky (Qualcomm)]

JVET-AK0263 Crosscheck of JVET-AK0150 (EE1-1.2: LOP with improved Attention and residual groups) [Z. Ameur, T. Dumas, F. Galpin (InterDigital)] [late] [miss]

JVET-AK0300 Crosscheck of JVET-AK0150 (EE1-1.2: LOP with improved attention and residual groups) [Y. Li (Bytedance)] [late] [miss]

JVET-AK0182 EE1-1.6: Reduced complexity input feature extraction [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)] [late] [miss]

JVET-AK0195 EE1-1.4: Cross-component enhanced LOP filter [Y. Li, J. Li, C. Lin, K. Zhang, L. Zhang (Bytedance)]

JVET-AK0216 Crosscheck of JVET-AK0195 (EE1-1.4: Cross-component enhanced LOP filter) [Y. Li (Qualcomm)] [late] [miss]

JVET-AK0301 Crosscheck of JVET-AK0195 (EE1-1.4: Cross-component enhanced LOP filter) [J. N. Shingala, A. Shyam (Ittiam)] [late]

JVET-AK0235 EE1-4.1: A Neural Network Downscaling Filter for RPR [Y.-Q. Zhu, W.-X. He, X. Li, J.-D. Y, Q. Liu (HUST), Z. Lv (vivo)] [late]

JVET-AK0247 Crosscheck of JVET-AK0235 (EE1-4.1: A Neural Network Downscaling Filter for RPR) [Z. Li, J. Zhang (PKU)] [late] [miss]

JVET-AK0212 EE2-2.1: OBMC modifications [R. Yu, H. Huang, C.-C. Chen, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0256 Crosscheck of JVET-AK0212 (EE2-2.1: OBMC modifications) [Y. Kidani, K. Kawamura (KDDI)] [late]

JVET-AK0290 Crosscheck of Test 2.1a and b in JVET-AK0212 (EE2-2.1: OBMC modifications) [Z. Deng (Bytedance)] [late] [miss]

### EE1 related and beyond-EE contributions: Neural network-based video coding (9)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0079 EE1-Related: Combination Test of NNVC Tools and TRFS [Q. Qin, C. Jung (Xidian Univ.)]

JVET-AK0071 AHG11: Lightweight Multiscale Reference Frame Generation for VVC Inter Coding [P. Li, C. Jung, Q. Qin (Xidian Univ.)]

JVET-AK0119 AHG11: NNLF LOP3 improvement with parallel 1x3/3x1 Backbone [T. Shao, P. Yin, S. McCarthy (Dolby), J. N. Shingala, A. Shyam, A. Suneja, S. Badya (Ittiam)]

JVET-AK0146 [AHG11] A Hybrid Framework Integrating End-to-End Learned Image Codec with Conventional Codec [N. Zou, A. Hallapuro, F. Cricri, H. Zhang, M. M. Hannuksela (Nokia)]

JVET-AK0175 AhG11: Dimension-wise decomposed multiplier for content-adaptive loop-filter [Z. Xu, J. Konieczny (TCL)]

JVET-AK0177 AHG 11: Neural Network Coded Reference Frame for Intra Coding [F. Brand, T. Solovyev, E. Alshina (Huawei)]

JVET-AK0242 AHG11: Enhancing super-resolution with residual in NNVC [T. Yang, X. Li, W.-X. He, Y.-Q. Zhu, Q. Liu (HUST), Z.-Y. Lv (vivo)] [late]

JVET-AK0311 AHG11/AHG14: Content-adaptive LOP4 filter [M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, D. Bugdayci Sansli, A. Hallapuro, J. Lainema, H. Zhang (Nokia)] [late]

JVET-AK0312 AHG11: Conditional loop-filter [M. Santamaria, F. Cricri (Nokia)] [late]

### SADL and NNVC implementation, CTC (4)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0134 AhG14: SADL update [F. Galpin (InterDigital)] [late]

JVET-AK0151 AhG14: On the impact of explicit full SIMD implementation for 1x3 and 3x1 DW convolution [Y. Li, D. Rusanovskyy, M. Karczewicz (Qualcomm)]

JVET-AK0255 [AHG11] Response to Call for training materials for neural network-based video coding tool development [F. Zhang, D. Bull, J. Nawala, Y. Jiang, X. Zhu, J. Sole, E. Alshina] [late]

JVET-AK0257 AHG14: The extension of SADL library [N. Fu, W. Bao, Z. Chen (Wuhan Univ.)] [late]

## AHG6/AHG12: Enhanced compression beyond VVC capability (74)

### Summary and BoG reports

Contributions in this area were discussed at 1825–2010 on Tuesday 14 Jan. 2025 (chaired by JRO) and at 0900–XXXX on Wednesday 15 Jan. 2025 (chaired by JRO).

JVET-AK0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

- List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1a | Extended OBMC for IBC/IntraTMP coded blocks | Y. Kidani (KDDI) X. Li (Alibaba) | R. Yu (Qualcomm) D. Kim (ETRI) |
| 1.1b | Test 1.1a + extended OBMC for CCP coded blocks | X. Li (Alibaba) |  |
| 1.2a | Improvement on non-MPM | G. Wang (vivo) | J. Fu (PKU) |
| 1.2b | Most dominant intra prediction mode | M. Hong (LGE) | M. Coban (Qualcomm) |
| 1.2c | Test 1.2b + excluding improbable intra modes | M. Hong (LGE) | M. Coban (Qualcomm) |
| 1.2d | Test 1.2a + Test 1.2c | G. Wang (vivo), M. Hong (LGE) |  |
| 1.3 | IntraTMP merge candidates enrichment | M. Radosavljević (InterDigital) | D. Ruiz Coll, J.-K. Lee (Ofinno) |
| 1.4 | IntraTMP signalling improvement | F. Le Léannec (InterDigital) | withdrawn |
| 1.5 | Test 1.3 + Test 1.4 | F. Le Léannec (InterDigital) | withdrawn |
| 1.6 | Extended IntraTMP merge candidate list with an improved fusion mode | D. Ruiz Coll (Ofinno) | W. Lim, S.-C. Lim (ETRI) |
| 1.7 | Block vector guided DIMD | L. Zhang (OPPO) | N. Yan (Kwai) |
| 1.8 | Non-adjacent DIMD fusion modes for TMRL intra mode candidate list construction | V. Rufitskiy, A. Filippov (TCL) | S. Blasi (Nokia) |
| 1.9 | Weighted OBIC | P. Andrivon (Ofinno) | F.Le Léannec (InterDigital) |
| 1.10a | Bilateral filtering for intra prediction | W. Yin (Bytedance) | V. Shchukin (Ericsson) C. Ma (Kwai) |
| 1.10b | Test 1.10a + 6-tap cubic interpolation filter for TIMD | W. Yin (Bytedance) | V. Shchukin (Ericsson) C. Ma (Kwai) |
| 1.11 | 8-tap interpolation filter for angular intra prediction | T. Dong (TCL) | S.H. Park (KNU) |
| 1.12 | On MPM with matrix-based position dependent intra prediction | Z. Sun (OPPO) | X. Li (Alibaba) |
| 1.13 | Multiple filter taps for EIP | C. Zhou (vivo) | Z. Deng (Bytedance) |
| 1.14 | Block vector guided EIP | Z. Xie (OPPO) | Z. Zhang (Alibaba) |
| 1.15 | Test 1.13 + Test 1.14 | C. Zhou (vivo) Z. Xie (OPPO) | Z. Zhang (Alibaba) |
| 1.16a | Adaptive intra filter for template prediction | Z. Zhang (Alibaba) C. Zhou (vivo) | Z. Xie (OPPO) |
| 1.16b | Template-based filter selection for intra prediction | Z. Zhang (Alibaba) C. Zhou (vivo) | Z. Xie (OPPO) |
| 1.16c | Test 1.16a+Test 1.16b | Z. Zhang (Alibaba) C. Zhou (vivo) | Z. Xie (OPPO) |
| 1.17 | Test 1.10a + Test 1.11b + Test 1.16b | W. Yin (Bytedance) T. Dong (TCL) Z. Zhang (Alibaba) C. Zhou (vivo) | Z. Xie (OPPO) |
| 1.18 | Test 1.7 + Test 1.9 | L. Zhang (OPPO) P.Andrivon (Ofinno) | F.Le Léannec (InterDigital) |
| 1.19 | Test 1.8 + Test1.9 | V.Rufitskiy (TCL) L. Zhang (OPPO) | R. G. Youvalari (Xiaomi) |
| 2 Inter prediction |  |  |  |
| 2.1a | OBMC modification for GPM blocks | R. Yu (Qualcomm) | Z. Deng (Bytedance) |
| 2.1b | OBMC operation order for block and subblock boundaries | R. Yu (Qualcomm) | Z. Deng (Bytedance) |
| 2.1c | Test 2.1a + Test 2.1b | R. Yu (Qualcomm) | Y. Kidani (KDDI) |
| 2.2a | Additional temporal affine candidates | L. Zhao (Bytedance) | Y. Zhang (Qualcomm) |
| 2.2b | Additional synthetic affine candidates | L. Zhao (Bytedance) | Y. Zhang (Qualcomm) |
| 2.2c | Test 2.2a + Test 2.2b | L. Zhao (Bytedance) | Y. Zhang (Qualcomm) |
| 2.3 | Regression-based GPM with intra and inter prediction | K. Jia (Alibaba) | Y. Wang (Bytedance) C. Zhou (vivo) |
| 2.4 | GPM extension | H. Zhang (OPPO) | Y. Ahn (LGE) |
| 2.5 | TMVP candidate selection | S. Hong (Nokia) | R. G. Youvalari (Xiaomi) |
| 2.6 | Template matching padding | N. Neumann (RWTH) | X. Li (Google) |
| 3 Transform and coefficients coding |  |  |  |
| 3.1a | Multiple kernel set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection | M. Coban (Qualcomm), F. Wang (OPPO), C. Bonnineau (Interdigital), L. Zhao (Bytedance) | M. Hong (LGE) |
| 3.1b | Test 3.1a with different types of transform sets | M. Coban (Qualcomm), F. Wang (OPPO), C. Bonnineau (Interdigital), L. Zhao (Bytedance) | M. Hong (LGE) |
| 3.2 | LFNST/NSPT set derivation for CCP coded block | H. Huang (OPPO) | M. Hong (LGE) |
| 3.3a | Advanced SBT | G. Laroche (Canon) | F. Le Léannec (InterDigital) |
| 3.3b | Test 3.3a with position granularity set to 4 instead to 1 | G. Laroche (Canon) |  |
| 3.3c | Test 3.3a not applied after SBT | G. Laroche (Canon) |  |
| 3.3d | Test 3.3a with only the position is inferred | G. Laroche (Canon) |  |
| 3.4 | Last significant coefficient position signalling with secondary prefix | F. Le Léannec (InterDigital) | P. Nikitin (Qualcomm) C. Nguyen (Dolby) |
| 3.5a | Coefficient significant and gtX flags coding with probability adaptation | M. Balcilar (InterDigital) | P. Nikitin (Qualcomm) |
| 3.5b | Coefficient significant and gtX flags coding with increased CABAC engine’s range resolution | M. Balcilar (InterDigital) | P. Nikitin (Qualcomm) |
| 3.6a | Test 3.4 + Test 3.5a | F. Le Léannec (InterDigital) | P. Nikitin (Qualcomm) |
| 3.6b | Test 3.4 + Test 3.5b | F. Le Léannec (InterDigital) | P. Nikitin (Qualcomm) |
| 3.7 | Improved LFNST/NSPT kernel set selection for SGPM | S. Puri (InterDigital) | I. Zupancic (Nokia) |
| 3.8a | Implicit MTS extension | M. Karczewicz (Qualcomm) | M. Salehifar (Bytedance) |
| 3.8b | Test 3.8a reusing existing LUT | M. Karczewicz (Qualcomm) | M. Salehifar (Bytedance) |
| 4 In-loop filtering |  |  |  |
| 4.1 | Restrictions on ALF coefficient values | V. Shchukin (Ericsson), P. Bordes (InterDigital) | I. Jumakulyyev (Nokia) |
| 4.2 | Using Laplacian information in ALF | I. Jumakulyyev (Nokia) | V.Shchukin (Ericsson) V. Rufitskiy (TCL) |
| 4.3a | Boundary-aware offset refinement for ALF | W. Yin (Bytedance) | N. Hu (Qualcomm) |
| 4.3b | Test 4.3a + boundary-aware offset refinement for DBF | W. Yin (Bytedance) | N. Hu (Qualcomm) |
| 4.4 | Improvement to ALF scaling factors | N. Hu (Qualcomm) | withdrawn |
| 4.5a | ALF joint optimization (encoder only) | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 4.5b | Encoder only ALF joint optimization tested in VTM | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 4.5c | Increase of the number of ALF APSs | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 4.5d | Test 4.5a + Test 4.5c | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 4.5e | Test 4.5a + reuse of ALF control information | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 4.5f | Test 4.5d + reuse of ALF control information | H. Wang (Qualcomm) | W. Yin (Bytedance) |
| 4.6a | Huffman coding for CCALF + simulated annealing for CCALF | K. Takada (Sharp) | W. Yin (Bytedance) |
| 4.6b | Test 4.1 + Test 4.6a | K. Takada (Sharp), V. Shchukin (Ericsson) | W. Yin (Bytedance) |
| 4.7 | Temporal ALF | L. Xu (OPPO) | W. Yin (Bytedance) |
| 4.8.a1 | NN-based ILF with ALF | D. Rusanovskyy (Qualcomm) | F.Galpin (InterDigital ) |
| 4.8.a2 | NN-based ILF with ALF (Complexity constrained) | D. Rusanovskyy (Qualcomm) | F.Galpin (InterDigital ) |
| 4.8b | NN-based ILF with VLOP model from NNVC | D. Rusanovskyy (Qualcomm) | F.Galpin (InterDigital) |
| 4.9a | ALF-CCCM | P. Astola (Nokia) | R. G. Youvalari (Xiaomi) |
| 4.9b | ALF-CCCM without the use of reference pictures | P. Astola (Nokia) | R. G. Youvalari (Xiaomi) |

Description of tests

  - Intra prediction

Test 1.1: Extended OBMC for non-inter blocks (JVET-AK0076)

In ECM, OBMC is applied to inter coded blocks. In the tests, OBMC is extended to non-inter coded blocks. The extended OBMC method is applied to sub-blocks on top and left boundaries of the current block. A predictor is generated using the neighbouring block information and blended with the prediction samples of the current block.

In Test 1.1a, OBMC is extended to IBC/IntraTMP coded blocks. If a neighbouring block is IBC/IntraTMP coded, the block vector information of the neighbouring block is used for generating the predictor. If a neighbouring block is intra mode (neither IBC nor IntraTMP) coded, an intra prediction mode is derived by applying DIMD-liked method to the neighbouring reconstructed samples of the current block for generating the predictor.

In Test 1.1b, OBMC is additionally extended to CCP coded blocks. The OBMC is further applied when the current block is coded by a CCP mode and the neighbouring block is coded by a non-CCP mode. An intra prediction mode derived by applying DIMD-liked method is used to generate the predictor for Cb and Cr, respectively.

Test 1.1a: Extended OBMC for IBC/IntraTMP coded blocks

Test 1.1b: Test 1.1a + extended OBMC for CCP coded blocks

Test 1.2a: Improvement on non-MPM (JVET-AK0086)

In ECM, intra modes are coded using primary MPM list (6 modes), secondary MPM list (22 modes) and the remaining modes (45 modes).

In the test, the non-MPM list are constructed as follows:

- The first non-planar mode in the MPM list is selected as the starting intra-picture mode.

- From the starting intra mode, non-MPM list is filled in turns with one forward mode and then one backward with the same step, where the step increases by one in each turn. Mode is skipped if it has already been included in the primary or secondary MPM lists.

- If a mode satisfies the PDP mode condition, it is swapped with the previous filled non-PDP mode.

Test 1.2a: Improvement on non-MPM

Test 1.2bc: Intra mode coding based on HoG of neighbouring templates (JVET-AK0059)

In ECM, in DIMD prediction up to 7 intra modes are derived from reconstructed neighbouring samples. These directional predictors are then combined with a non-directional predictor (either planar or block vector-based) using weights calculated from the histogram of gradients (HoG).

Test 1.2b targets intra mode signalling where up to 8 intra prediction modes are derived based on the HoG from neighbouring templates. The HoG computation follows the same process as in DIMD, but template area differs. If the block size is less than 256, a 3-line template area is used, otherwise, template area is extended to 4 lines. Up to 8 intra prediction modes are derived by the HoG based on predefined template areas as follows:

- Up to 4 intra prediction modes are derived from above and left template area

- Up to 2 intra prediction modes are derived from left template area

- Up to 2 intra prediction modes are derived from above template area

- Neighbouring modes of derived modes (1) ~ (3) are added until up to 8 intra prediction modes are identified.

After constructing up to 8 intra prediction modes, they are ordered by computing their template cost. The intra prediction mode with the minimum template cost is signalled before MPM information at each CU when block size is equal or less than 1024, and to avoid redundancy, this mode is excluded from MPM, secondary MPM, and non-MPM.

In Test 1.2c, on top of Test 1.2b, some improbable intra modes are excluded. After computing the HoG, 20 improbable intra modes with small HoG amplitude are excluded from MPM, SMPM, and non-MPM list. If the signalling using the smallest template cost mode is applied, the HoG which was utilized to derive that mode is reused to exclude the intra prediction modes. If the signalling is not applied due to block size restriction, the HoG of DIMD is used to exclude the intra prediction modes.

When both the signalling and the excluding improbable intra modes are applied, non-MPM mode size reduces from 45 (=67 – 22) to 24 (=67 – 22 – 1(smallest template cost mode) – 20(excluding intra prediction modes)).

Test 1.2b: Most dominant intra prediction mode

Test 1.2c: Test 1.2b + excluding improbable intra modes

Test 1.3: IntraTMP merge candidates enrichment (JVET-AK0220)

This test evaluates modifications for BV merge list generation, where new BV candidates are added.

The distance for non-adjacent spatial merge candidates is increased from 4 to 5, where up to 5 rows/columns of spatial neighboring blocks (in terms of the CU height/width) of the current CU can be used in merge BV candidate construction. This is achieved for camera captured content only. The spatial neighbouring blocks of the current block are identified using 13 adjacent blocks (as in OBIC). Total number of BV candidates is eventually limited to 36.

In ECM, merge BVs are collected from local and non-local PUs coded in IBC or Intra TMP mode. In this test BV merge list is generated from all the spatial merge candidates that contain BVs (BV-based prediction is currently used in several modes and submodes in ECM software).

When constructing merge BV candidates, BVs from HMVP list are used. HMVP candidates are added after spatial (local and non-local) candidates and after ARBVP candidate construction. Up to 25 candidates from HMVP list are added to the merge candidates.

The complexity due to newly introduced above aspects is reduced by following elements:

- Block vector refinement employs a sparse search withing refinement area, followed by a last refinement step around best found positions.

- A distance-based criteria is used to discard some BV candidates close to already existing candidates in merge list under construction.

- Top-left template is no more considered in IntraTMP template cost computations.

Test 1.6: Extended IntraTMP merge candidate list with an improved fusion mode (JVET-AK0161)

In ECM, in addition to the regular IntraTMP mode, a TMP fusion mode can blend up to 5 candidate reference blocks according to weight parameters. Two different fusion models are available to derive the weight parameters of the fusion mode, which are based on the template SAD cost and the Gaussian solver methods. An intra_tmp_fusion_weight_type flag is signalled to the decoder, indicating the TMP fusion model used to derivate the weight parameters in the blending process.

For the neighbour blocks encoded as regular IntraTMP (non-fusion) mode, if the IntraTMP index is not the index 0, the first BV candidate in the IntraTMP list is also included in the IntraTMP merge and AR-BVP list.

In the test, the adjacent and non-adjacent neighbour blocks are checked in the IntraTMP merge and AR-BVP list construction. Currently, if those blocks were encoded using the IntraTMP fusion mode, only the first BV fusion candidate is selected to be added to these lists. The reminder fusion BV candidates, up to 5 adjacent BVs in the IntraTMP refined list, are not checked. In the test, the entire set of fusion BV candidates when a neighbouring block is coded in IntraTMP fusion mode are evaluated.

This test also modifies IntraTMP fusion model signalling. Instead of signalling the fusion model with a flag, the fusion model for IntraTMP groups 1 and 2 is derived on the decoder side based on the template matching cost of the fused templates, which are calculated using the respective weight parameters.

To alleviate the complexity impact, sparse searching in regions R4 and R5 of IntraTMP search range is skipped if the top-left position of those regions in not valid, and if the bottom-right position of those regions is valid, the whole validation process is bypassed for such regions. Merge candidates with the template cost above a threshold (set equal to the template cost of the last candidate in the IntraTMP sparse list) are skipped and not considered for the IntraTMP and the AR-BVP list construction.

Test 1.6: Extended IntraTMP merge candidate list with an improved fusion mode

Test 1.7: Block vector guided DIMD (JVET-AK0060)

In this test, neighbour reconstructed samples pointed at by BVs, instead of the spatial adjacent samples, are used to build HoG for DIMD as shown in the next figure. Up to 5 block vectors are obtained by the same searching process as in IntraTMP. With each block vector, a reference area in the current picture is determined and its samples are used to derive the intra modes and corresponding amplitudes. To reduce complexity, overlapping reference areas are clustered.

Two intra modes with the highest amplitudes are selected from the HoG and the prediction of the current block is the blending of those 2 predictors and a non-directional predictor. The non-directional predictor is the blending of up to 5 BV-based predictors obtained using the block vectors.

The BV guided DIMD mode is signalled as a sub-mode of DIMD after OBIC. At encoder, to reduce complexity, the mode is checked by a full RD only for block sizes larger than 128 with width and height greater than 4, otherwise, it is compared with other intra modes by the SATD.

Test 1.7: Block vector guided DIMD

Test 1.8: Non-adjacent DIMD for TMRL (JVET-AK0104)

In the test, DIMD is applied to non-adjacent reference lines, where HoG is calculated in the area determined by a reference suing 2x2 edge operator. Intra prediction modes obtained using the HoG and fusion candidates are added to the list of intra prediction modes for TMRL lines. DC mode is not included into TMRL list.

Each candidate added to a TMRL candidate list consists of a pair of

- an intra-prediction mode (or a set of modes if DIMD fusion is used in the case of non-adjacent DIMD applied to TMRL), and

- non-adjacent reference lines (reference line index).

To compare template matching costs, which is estimated by using the HAD metric in TMRL, each non-adjacent reference line used in TMRL is projected onto the adjacent reference line of a current block according to the angular mode direction of a TMRL. If the fusion process is enabled for a candidate from the TMRL list, a projection of each mode is obtained and, then, these projections are combined according to the weights assigned to each angular mode and reference line belonging to this set.

Test 1.9: Weighted OBIC (JVET-AK0056)

In ECM, OBIC derived intra prediction modes are based on the sample-wise occurrences of the neighbouring intra prediction modes. Similarly to DIMD, a histogram of occurrence (HoC) is built and up to five most represented angular modes are selected. For neighbouring blocks using more than one intra prediction mode to generate a predictor (e.g. TIMD, DIMD…), each corresponding intra mode is used with the same level of contribution in the HoC construction. The selected modes are blended with planar mode or a block-vector based mode to generate the OBIC predictor.

In the test, instead of using the same level in HoC construction, the occurrence value is weighted for the case when multiple intra prediction modes are used from a neighbour block, weights (ki) and the location dependency state (used on the fusion process) are reused to scale the relevant mode occurrence as follows.

HoC(Mi) += ki . w. h

In case a HoC entry is built from an intra mode with different location dependency states, the most represented location dependency state is selected.

Additionally, each contributor to the HoC is weighted by a scaling factor depending on the distance between the current block and the considered neighbouring block where the distance weight is selected from {4, 2, 1, 0}.

Test 1.18: Combination of Test 1.7 and Test 1.9 (JVET-AK0283)

Test 1.18: Test 1.7 + Test 1.9

Test 1.19: Combination of Test 1.7 and Test 1.8 (JVET-AK0284)

Test 1.19: Test 1.7 + Test 1.8

Test 1.10: Bilateral filtering for intra prediction (JVET-AK0118)

In Test 1.10a, intra prediction samples are further filtered by bilateral filter to reduce the noise, bilateral filter is kept the same as BIF used in the loop-filtering stage. The available above and left neighbouring reconstructed samples are used in the bilateral filtering process for the current block.

In Test 1.10b, on top of Test 1.10a, a 6-tap cubic interpolation filter is always applied for the blocks coded with TIMD mode.

Test 1.10a: Bilateral filtering for intra prediction

Test 1.10b: Test 1.10a + 6-tap cubic interpolation filter for TIMD

Test 1.11: 8-tap interpolation filter for angular intra prediction (JVET-AK0087)

In the test, 8-taps interpolation filter is used for intra prediction reference samples if the following conditions are met:

- Intra mode prediction angle has sufficient angular distance from horizontal or vertical modes. The threshold to determine that is dependent on the block size. The threshold is different for normal (66 modes) and high (131 modes in TIMD) angular mode precision.

- Block size is smaller than a given threshold, the threshold is different for normal (66 modes) and high (131 modes in TIMD) angular mode precision.

Otherwise, the existing 6-taps interpolation filter is used.

Test 1.11a: 8-tap filter with the norm 8192.

Test 1.11b: 8-tap filter with the norm 256 (reduced precision).

Test 1.16: On intra interpolation filter (JVET-AK0102)

In the test, intra interpolation filter is selected based on TM cost as follows.

In Test 1.16a, instead of always choosing 4-tap DCTIF interpolation filter for template prediction, the MDIS condition is also applied in intra filter selection for the template prediction. The 6-tap DCTIF is always applied for TMRL mode.

In Test 1.16b, the intra interpolation filter is determined by using TM cost of the current coding block. That is, 6-tap DCTIF, 4- and 6-tap smoothing interpolation filters are used to perform intra prediction on the template area, and the one results in the minimum TM cost is selected as the intra interpolation filter for the prediction of the current block. If the 6/4-tap smoothing interpolation filter is selected, [1, 2, 1]/4 reference filter is disabled to avoid applying smoothing filter twice for intra prediction.

Test 1.16a: Adaptive intra filter for template prediction

Test 1.16b: Template-based filter selection for intra prediction

Test 1.16c: Test 1.16a + Test 1.16b

Test 1.17: Combination of Test 1.10a, Test 1.11b, and Test 1.16b (JVET-AK0148)

Test 1.17: Test 1.10a + Test 1.11b + Test 1.16b

Test 1.12: On MPM with matrix-based position dependent intra prediction (JVET-AK0061)

In ECM, matrix-based position-dependent intra-prediction (PDP) replaces modes 0, 1, (2+2k) for blocks up to 16x16, and 0, 1, (2+4k) for other blocks.

In the test, some of the non-PDP intra prediction modes (IPM) in the MPM/Non-MPM list are converted to PDP modes as follows:

PDPMode=IPM-1≫1×2+2, W≤16 &and &&&H≤16IPM+1≫2×4+2, otherwise

If PDPMode is not added to the MPM/Non-MPM list, PDPmode±offset will be further considered for MPM/Non-MPM list, where offset is equal to 2 for blocks with width and heights less or equal to 16 or 4, otherwise.

Additionally, PDP prediction, instead of the conventional angular prediction, is used to calculate the template cost for MPM mode sorting for PDP modes.

Test 1.12a: Converting non-PDP to PDP in the MPM and non-MPM lists

Test 1.12b: Using PDP prediction for MPM sorting

Test 1.12c: Test 1.12a + Test 1.12b

Test 1.13: Multiple filter taps for EIP (JVET-AK0084)

In ECM, EIP filter length is 15. In the test, a 9-tap filter is introduced and an index indicating which filter length is used is signalled. For EIP mode using 9-tap filter, the prediction is calculated as follows:

predx,y=i=07(ci×tx-offsetXi, y-offsetYi)+c8×2bitdepth-1

9-tap filter is applied to multi-model EIP for blocks with area larger than or equal to 64, and smaller than or equal 256.

Test 1.13: Multiple filter lengths for EIP

Test 1.14: Block vector guided EIP (JVET-AK0062)

In the test, BV is used to determine the reference area for calculating EIP filter parameters instead of directly using the adjacent spatial reference area and the mode is signalled with a flag as a sub-mode of EIP.

BV is derived from the sparse searching process of the IntraTMP in the current ECM.

Only square EIP filter shape is allowed in this mode.

Test 1.14: Block vector guided EIP

Test 1.15: Combination of Test 1.13 and Test 1.14 (JVET-AK0149)

Test 1.15: Test 1.13 + Test 1.14

1.1a is straightforward and matching according to cross-checkers. Tradeoff with encoding time still in reasonable range. 1.1b is not attractive.

Decision: Adopt JVET-AK0076 test 1.1a

Among the 1.2x tests, 1.2c has most attractive tradeoff. Implementation is straightforward and results confirmed by cross-checker.

Decision: Adopt JVET-AK0059 test 1.2c

1.3 and 1.6 are having rather small gain but increase complexity by additional processing at encoder and decoder. No action on these.

1.7 and 1.8 provide some compression gain, but also require significantly more encoder/decoder processing. Run times reported are not in the range of reasonable tradeoff, and reported to be even higher by cross-checkers. Further study (EE) recommended as some experts found the ideas interesting, but the proposals would not be taken into consideration without significant decrease of complexity/runtime, without significantly affecting the compression benefit.

1.9 was supported by several experts, as it is simple to implement and has small gain without increasing run time.

Decision: Adopt JVET-AK0056 test 1.9.

For 1.10a, it was commented by one expert that some misalignment with requests from previous meeting notes (processing order with PDPC etc.) was found in the new implementation, and conditions under which it is applied (certain modes, certain block shapes) appear a bit arbitrary to him. Generally, the tradeoff would be still acceptable, even though a similar concept had been rejected during VVC development, as it was assumed that the latency in the intra prediction loop would be critical. This might not be so relevant in an exploration, however.

Several experts supported the adoption.

Decision: Adopt JVET-AK0118 test 1.10a

Test 1.11b shows a very good tradeoff (in comparison with some other proposals), providing compression gain without significant complexity increase

Decision: Adopt JVET-AK0087 test 1.11b

Test 1.16b standalone does not have enough compression benefit justifying the additional complexity. The combination 1.17 shows would not change this assessment.

Several independent experts supported adoption of test 1.12c which has attractive tradeoff. Even though the element of 1.12b has some complexity increase, its standalone performance would still in the margin of acceptable tradeoff.

It was however commented that due to the PDP conversion of conventional modes, a significant complexity increase could be expected. However, due to the fact that the current ECM encoder does only check even positions of the non-MPM list in the first pass, converting the odd modes to PDP is not noticed in terms of run time.

Even though it might not be practical in a standard, in terms of exploration the method is assessed to be acceptable.

Decision: Adopt JVET-AK0061 test 1.12c.

### EE2 contributions: Enhanced compression beyond VVC capability (39)

There was no presentation or discussion about specific proposals in this category.

For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AK0056 EE2-1.9: Weighted OBIC [P. Andrivon, M. Blestel, N. Zouidi (Ofinno)]

JVET-AK0275 Cross-check of JVET-AK0056 EE2-1.9: Weighted OBIC [F. Le Léannec (InterDigital)] [late]

JVET-AK0059 EE2-1.2: Intra mode coding based on HoG of neighboring templates [M. Hong, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0306 Crosscheck of JVET-AK0059 (EE2-1.2: Intra mode coding based on HoG of neighboring templates) [M. Coban (Qualcomm)] [late] [miss]

JVET-AK0060 EE2-1.7: Block vector guided DIMD [L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AK0267 Crosscheck of JVET-AK0060 (EE2-1.7: Block vector guided DIMD) [N. Yan (Kwai)] [late] [miss]

JVET-AK0061 EE2-1.12: On MPM with matrix-based position dependent intra prediction [Z. Sun, Y. Yu, H. Yu, L. Xu, D. Wang (OPPO)]

JVET-AK0231 Crosscheck of JVET-AK0061 (EE2-1.12: On MPM with matrix-based position dependent intra prediction) [X. Li (Alibaba)] [late] [miss]

JVET-AK0062 EE2-1.14: block vector guided EIP [Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AK0299 Crosscheck of JVET-AK0062 (EE2-1.14: block vector guided EIP) [Z. Zhang (Alibaba)] [late] [miss]

JVET-AK0063 EE2-2.4: GPM extension [H. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AK0266 Crosscheck of JVET-AK0063 (EE2-2.4: GPM extension) [Y. Ahn (LGE)] [late] [miss]

JVET-AK0064 EE2-3.2: LFNST/NSPT set derivation for CCP coded block [H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AK0280 Crosscheck of JVET-AK0064 (EE2-3.2: LFNST/NSPT set derivation for CCP coded block) [M. Hong (LGE)] [late] [miss]

JVET-AK0307 Crosscheck of JVET-AK0064 (EE2-3.2: LFNST/NSPT set derivation for CCP coded block) [M. Coban (Qualcomm)] [late] [miss]

JVET-AK0065 EE2-4.7: Temporal ALF [L. Xu, Y. Yu, H. Yu, N. Song, D. Wang (OPPO)]

JVET-AK0230 Crosscheck of JVET-AK0065(EE2-4.7: Temporal ALF) [W. Yin (Bytedance)] [late] [miss]

JVET-AK0076 EE2-1.1: Extended OBMC for non-inter blocks [Y. Kidani, H. Katou, K. Kawamura (KDDI), X. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AK0269 Crosscheck of Test 1.1a in JVET-AK0076 (EE2-1.1: Extended OBMC for non-inter blocks) [D. Kim, S.-C. Lim (ETRI)] [late]

JVET-AK0293 Crosscheck of Test 1.1a in JVET-AK0076 (EE2-1.1: Extended OBMC for non-inter blocks) [R. Yu (Qualcomm)] [late] [miss]

JVET-AK0084 EE2-1.13: Multiple filter taps for EIP [Z. Lv, C. Zhou, G. Wang (vivo)] [late]

JVET-AK0291 Crosscheck of JVET-AK0084 (EE2-1.13: Multiple filter taps for EIP) [Z. Deng (Bytedance)] [late] [miss]

JVET-AK0085 EE2-2.6: Template Matching Padding [N. Neumann (Nokia), M. Wien (RWTH Aachen Univ.)]

JVET-AK0244 Crosscheck JVET-AK0085 on EE2-2.6 Template Matching Padding [X. Li (Google)] [late] [miss]

JVET-AK0086 EE2-1.2a: Improvement on non-MPM [G. Wang, C. Zhou, Z. Lv (vivo)]

JVET-AK0245 Crosscheck of JVET-AK0086 (EE2-1.2a: Improvement on non-MPM) [J. Fu (PKU)] [late]

JVET-AK0087 EE2-1.11: 8-tap interpolation filter for angular intra prediction [T. Dong, A. Filippov, V. Rufitskiy, J. Konieczny, H. Qin, J. Zhang (TCL)]

JVET-AK0303 Crosscheck of JVET-AK0087 (EE2-1.11: 8-tap interpolation filter for angular intra prediction) [S. Park (KNU)] [late] [miss]

JVET-AK0318 Crosscheck of JVET-AK0087 (EE2-1.11b: 8-tap interpolation filter for angular intra prediction) [G. Verba (Qualcomm)] [late] [miss]

JVET-AK0088 EE2-4.9: ALF-CCCM [P. Astola, I. Jumakulyyev, D. Bugdayci Sansli, J. Lainema (Nokia)]

JVET-AK0248 Crosscheck of JVET-AK0088 (EE2-4.9: ALF-CCCM) [R. G. Youvalari (Xiaomi)] [late] [miss]

JVET-AK0091 EE2-4.2: Using Laplacian information in ALF [I. Jumakulyyev, D. Bugdayci Sansli, I. Zupancic, J. Lainema (Nokia)]

JVET-AK0129 Crosscheck of JVET-AK0091 (EE2-4.2: Using Laplacian information in ALF) [V. Shchukin (Ericsson)] [late]

JVET-AK0314 Crosscheck of JVET-AK0091 (EE2-4.2: Using Laplacian information in ALF) [V. Rufitskiy (TCL)] [late] [miss]

JVET-AK0095 EE2-2.2: Enhanced derivation of affine merge candidates [L. Zhao, K. Zhang, Z. Deng, N. Zhang, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0304 Crosscheck of JVET-AK0095 (EE2-2.2: Enhanced derivation of affine merge candidates) [Y. Zhang (Qualcomm)] [late]

JVET-AK0097 EE2-3.4: Last significant coefficient position signaling with secondary prefix [F. Le Léannec, M. Balcilar, C. Salmon-Legagneur, F. Galpin (InterDigital)]

JVET-AK0259 Cross-check of JVET-AK0097 (EE2-3.4: Last significant coefficient position signaling with secondary prefix) [P. Nikitin (Qualcomm)] [late] [miss]

JVET-AK0298 Crosscheck of JVET-AK0097 (EE2-3.4: Last significant coefficient position signaling with secondary prefix) T. N. Canh (Dolby Labs) [late]

JVET-AK0098 EE2-3.6a/3.6b: combinations of tests 3.4 and 3.5a/3.5b [F. Le Léannec, M. Balcilar, C. Salmon-Legagneur, F. Galpin (InterDigital)]

JVET-AK0261 Cross-check of JVET-AK0098 (EE2-3.6a/3.6b: combinations of tests 3.4 and 3.5a/3.5b) P. Nikitin (Qualcomm)

JVET-AK0101 EE2-2.3: Regression-based GPM with intra and inter prediction [K. Jia, J. Chen, X. Li, R.-L. Liao, Y. Ye (Alibaba)]

JVET-AK0268 Crosscheck of JVET-AK0101 (EE2-2.3: Regression-based GPM with intra and inter prediction) [Y. Wang (Bytedance)] [late]

JVET-AK0294 Crosscheck of JVET-AK0101 (EE2-2.3: Regression-based GPM with intra and inter prediction) [C. Zhou (vivo)] [late] [miss]

JVET-AK0102 EE2-1.16: On intra interpolation filter [Z. Zhang, J. Chen, X. Li, R.-L. Liao, Y. Ye (Alibaba), C. Zhou, Z. Lv, G. Wang (vivo)]

JVET-AK0309 Crosscheck of JVET-AK0102 (EE2-1.16: On intra interpolation filter) [Z. Xie (OPPO)] [late] [miss]

JVET-AK0104 EE2-1.8: Non-adjacent DIMD for TMRL [V. Rufitskiy, A. Filippov, T. Dong, H. Qin, J. Konieczny, K. Ding (TCL)]

JVET-AK0297 Crosscheck of JVET-AK0104 (EE2-1.8: Non-adjacent DIMD for TMRL) [S. Blasi (Nokia)] [late] [miss]

JVET-AK0105 EE2-3.5: High Resolution Arithmetic Engine [M. Balcilar, F. Le Leannec, C. Salmon-Legagneur, F. Galpin (InterDigital)]

JVET-AK0260 Cross-check of JVET-AK0105 (EE2-3.5: High Resolution Arithmetic Engine) [P. Nikitin (Qualcomm)] [late] [miss]

JVET-AK0118 EE2-1.10: Bilateral Filtering for Intra Prediction [W. Yin, K. Zhang, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0130 Crosscheck of JVET-AK0118 (EE2-1.10: Bilateral Filtering for Intra Prediction) [V. Shchukin (Ericsson)] [late] [miss]

JVET-AK0295 Crosscheck of JVET-AK0118 (EE2-1.10: Bilateral Filtering for Intra Prediction) [C. Ma (Kwai)] [late] [miss]

JVET-AK0121 EE2-4.3: Boundary-Aware Offset Refinement for In-Loop-Filters [W. Yin, K. Zhang, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0229 Crosscheck of JVET-AK0121 (EE2-4.3: Boundary-Aware Offset Refinement for In-Loop-Filters) [N. Hu (Qualcomm)] [late]

JVET-AK0123 EE2-4.1: Restrictions on ALF coefficient values [V. Shchukin, P. Wennersten, J. Ström (Ericsson)]

JVET-AK0172 Crosscheck of JVET-AK0123 (EE2-4.1: Restrictions on ALF coefficient values) [I. Jumakulyyev (Nokia)] [late] [miss]

JVET-AK0131 EE2-3.3: Advanced SBT with direction and position inference [G. Laroche, P. Onno (Canon)]

JVET-AK0276 Cross-check of JVET-AK0131 EE2-3.3: Advanced SBT with direction and position inference [F. Le Léannec (InterDigital)] [late]

JVET-AK0148 EE2-1.17: Combination of EE2-1.16, EE2-1.11 and EE2-1.10 [Z. Zhang, J. Chen, X. Li, R.-L. Liao, Y. Ye (Alibaba), C. Zhou, Z. Lv, G. Wang (vivo), T. Dong, A. Filippov, J. Konieczny, V. Rufitskiy, H. Qin, K. Ding (TCL), W. Yin, K. Zhang, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0296 Crosscheck of JVET-AK0148 (EE2-1.17: Combination of EE2-1.16, EE2-1.11 and EE2-1.10) [C. Ma (Kwai)] [late] [miss]

JVET-AK0310 Crosscheck of JVET-AK0148 (EE2-1.17: Combination of EE2-1.16, EE2-1.11 and EE2-1.10) [Z. Xie (OPPO)] [late] [miss]

JVET-AK0149 EE2-1.15: a combination of EE2-1.13 and EE2-1.14 [Z. Lv, C. Zhou, G. Wang (vivo), Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AK0240 Crosscheck of JVET-AK0149 (EE2-1.15: a combination of EE2-1.13 and EE2-1.14) [Z. Zhang (Alibaba)] [late] [miss]

JVET-AK0161 EE2-1.6: Extended IntraTMP merge candidate list with an improved fusion mode [D. Ruiz Coll, J.-K. Lee (Ofinno)]

JVET-AK0271 Crosscheck of JVET-AK0161 (EE2-1.6: Extended IntraTMP merge candidate list with an improved fusion mode) [W. Lim, S.-C. Lim (ETRI)] [late]

JVET-AK0176 EE2-4.5: Reuse of ALF control information [M. Karczewicz, H. Wang, N. Hu, V. Seregin (Qualcomm)]

JVET-AK0226 Crosscheck of JVET-AK0176 (EE2-4.5: Reuse of ALF Control Information) [W. Yin (Bytedance)] [late] [miss]

JVET-AK0183 EE2-4.8: Integration of NN-based ILF in ALF [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)]

JVET-AK0264 Crosscheck of JVET-AK0183 (EE2-4.8: Integration of NN-based ILF in ALF) [F. Galpin, T. Poirier (InterDigital)] [late] [miss]

JVET-AK0185 EE2-2.5: TMVP Candidate Selection [S. Hong, L. Wang, K. Panusopone (Nokia)]

JVET-AK0249 Crosscheck of JVET-AK0185 (EE2-2.5: TMVP Candidate Selection) [R. G. Youvalari (Xiaomi)] [late] [miss]

JVET-AK0187 EE2-3.8: Implicit MTS extension [P. Garus, M. Karczewicz, M. Coban, V. Seregin, H. Wang, P. Nikitin (Qualcomm)]

JVET-AK0285 Crosscheck of JVET-AK0187 (EE2-3.8: Implicit MTS extension) [M. Salehifar (Bytedance)] [late] [miss]

JVET-AK0198 EE2-4.6: Huffman coding and simulated annealing for CCALF [K. Takada, S. Deshpande (Sharp), V. Shchukin (Ericsson)]

JVET-AK0227 Crosscheck of JVET-AK0198 (EE2-4.6: Huffman Coding and Simulated Annealing for CCALF) [W. Yin (Bytedance)] [late] [miss]

JVET-AK0217 EE2-3.1: Multiple transform set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection [M. Coban, M. Karczewicz, P. Nikitin, P. Garus, B. Ray, V. Seregin (Qualcomm), F. Wang, Y. Yu, H. Yu, D. Wang (OPPO), C. Bonnineau, K. Naser, S. Puri, F. Le Léannec (InterDigital), L. Zhao, K. Zhang, L. Zhang (Bytedance)]

JVET-AK0233 Crosscheck of JVET-AK0217 (EE2-3.1: Multiple transform set selection for intra LFNST/NSPT with subsampled DIMD for transform set selection) [M. Hong (LGE)] [late] [miss]

JVET-AK0220 EE2-1.3: Intra TMP improvements [M. Radosavljević, T. Dumas, K. Naser, F. Le Léannec (InterDigital)] [late]

Initial version rejected as”placeholder”

JVET-AK0243 Crosscheck of JVET-AK0220 (EE2-1.3: Intra TMP improvements) [D. Ruiz Coll, J.-K. Lee (Ofinno)] [late] [miss]

JVET-AK0222 EE2-3.7: Improved LFNST/NSPT kernel set selection for SGPM [S. Puri, C. Bonnineau, K. Naser, F. Le Léannec (InterDigital)]

JVET-AK0273 Crosscheck of JVET-AK0222 (EE2-3.7: Improved LFNST/NSPT kernel set selection for SGPM) [I. Zupancic (Nokia)] [late] [miss]

JVET-AK0283 EE2-1.18: Combination of EE2-1.7 and EE2-1.9 [P. Andrivon, M. Blestel, N. Zouidi (Ofinno), L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)] [late]

JVET-AK0277 Cross-check of EE2-1.18: EE2-1.7 + EE2-1.9 [F. Le Léannec (InterDigital)] [late] [miss]

JVET-AK0284 EE2-1.19: Combination of EE2-1.7 and EE2-1.8 [V. Rufitskiy, A. Filippov, T. Dong, H. Qin, J. Konieczny, K. Ding (TCL), L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)] [late] [miss]

### EE2 related contributions (2)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0184 EE2-related: Additional results for NN-based ILF in ALF [D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)] [late]

JVET-AK0317 EE2-related: Simplification of EE2-3.3 Advanced SBT with direction and position inference [Y. Zhang, J. Ye, H. Wang, V. Seregin, M. Karczewicz (Qualcomm)] [late]

### ECM modifications and software improvements beyond EE2 (33)

#### Intra and CIIP (5)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0089 Non-EE2: Improvement on MPM [G. Wang, C. Zhou, Z. Lv (vivo)] [late]

JVET-AK0305 Crosscheck of JVET-AK0089 (Non-EE2: Improvement on MPM)) [J. Fu (PKU)] [late]

JVET-AK0202 AHG12: On cross-component intra prediction [Y.-J. Chang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0223 Non-EE2: CCP merge mode with adjustment [Y. Wang, K. Zhang, W. Yin, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0234 Non-EE2: Subblock-based CCCM [F. Pu, N. C. Thuong, P. Yin, S. McCarthy (Dolby)]

JVET-AK0262 EE2-related: On EIP filter shapes [K. Panusopone, M. He, S. Hong, L. Wang, J. Lainema (Nokia)] [late]

JVET-AK0288 Crosscheck of JVET-AK0262 (EE2-related: On EIP filter shapes) [Y. Yu, L. Xu (OPPO)] [late] [miss]

#### Inter (12)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0090 Non-EE2: IntraTMP complexity reduction [Z. Fan, T. Chujoh, T. Ikai (Sharp)]

JVET-AK0250 Crosscheck of JVET-AK0090 (Non-EE2: IntraTMP complexity reduction) [F. Wang (OPPO)] [late] [miss]

JVET-AK0096 Non-EE2: Additional inter merge candidates [N. Zhang, K. Zhang, Z. Deng, L. Zhao, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0108 Non-EE2: CMVP extension for constructed affine merge candidates [C. Li, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-AK0111 Non-EE2: On interpolation filter for template matching [Z. Dai, R.-L. Liao, J. Chen, X. Li, Y. Ye (Alibaba)]

JVET-AK0251 Crosscheck of JVET-AK0111 (Non-EE2: On interpolation filter for template matching) ]L. Xu (OPPO)] [late] [miss]

JVET-AK0137 Non-EE2: Improvements on inter AMVP candidate list construction [S. Park, H. Jeong, B. Jeon (SKKU)] [late]

JVET-AK0270 Crosscheck of JVET-AK0137 (Non-EE2: Improvements on inter AMVP candidate list construction) [D. Kim, S.-C. Lim (ETRI)] [late]

JVET-AK0162 AHG12: On affine motion compensation [H. Huang, R. Yu, Z. Zhang, Y. Zhang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0186 Non-EE2: Subblock-based spatial MVP [Z. Deng, K. Zhang, N. Zhang, L. Zhao, Y. Wang, W. Yin, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0192 Non-EE2: On Sample-Based BDOF [M. Salehifar, Y. He, K. Zhang, L. Zhao, Y. Wang, N. Zhang, Z. Deng, W. Yin, L. Zhang (Bytedance)]

JVET-AK0199 Non-EE2: MV refinement for TMVP [Z. Zhang, J.-L. Lin, Y. Zhang, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0213 AHG12: Extended BDOF usage for MV refinement [R. Yu, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0225 Non-EE2: Extension on spatial and temporal merge candidates [J.-L. Lin, P.-H. Lin, Z. Zhang, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JVET-AK0254 Case study of candidates in AMVP candidate list [S. Park, H. Jeong, B. Jeon (SKKU)] [late]

#### GPM (2)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0103 Non-EE2: Template matching based jointly reordering for GPM split modes and partition indexes [C. Ma, X. Xiu, X. Wang (Kwai)]

JVET-AK0133 Non-EE2: Improvements on the BV-based prediction in SGPM [J. Huo, Y. Fei, L. Wang, Y. Ma, F. Yang (Xidian Univ.)]

JVET-AK0253 Crosscheck of JVET-AK0133 (Non-EE2: Improvements on the BV-based prediction in SGPM) [X. Li (Alibaba)] [late] [miss]

#### In-Loop Filters (6)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0113 AHG12: On CCALF [L. Xu, Y. Yu, H. Yu, N. Song, D. Wang (OPPO)]

JVET-AK0308 Crosscheck of JVET-AK0113(AHG12: On CCALF) [W. Yin (Bytedance)] [late] [miss]

JVET-AK0117 Non-EE2: Neural network-based loop filtering for ECM [Y. Du, A. Li, J. Liu, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu (Transsion)]

JVET-AK0132 AHG12: Boundary strength adjustment for DBV mode [J. Huo, J. Fan, Y. Fei, J.Liu, Y. Ma, F. Yang (Xidian Univ.)]

JVET-AK0252 Crosscheck of JVET-AK0132 (AHG12: Boundary strength adjustment for DBV mode) [X. Li (Alibaba)] [late] [miss]

JVET-AK0200 Non-EE2: CCSAO with reused CTU control and extended edge classifier [C.-W. Kuo, X. Xiu, X. Wang (Kwai)]

JVET-AK0224 Non-EE2: Cross-Chroma Adaptive Loop-Filter [W. Yin, K. Zhang, H. Liu, Y. Wang, Z. Deng, N. Zhang, L. Zhao, M. Salehifar, L. Zhang (Bytedance)]

JVET-AK0237 Non-EE2: CCALF with dual-chroma inputs [C. Ma, X. Xiu, C.-W. Kuo, X. Wang (Kwai)]

#### Entropy coding, transforms, quantization, and transform coefficient coding (5)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0135 AhG12: CABAC contexts retraining [F. Galpin (InterDigital)] [late]

JVET-AK0174 Non-EE2: Optimization of probability estimation in CABAC [D. Karwowski, D. Mieloch, M. Lorkiewicz (PUT)]

JVET-AK0214 Non-EE2: IntraNN NSPT set [G. Verba, M. Coban, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0215 Non-EE2: MTS with reduced memory [B. Ray, V. Seregin, M. Karczewicz (Qualcomm)]

JVET-AK0279 Non-EE2: On coefficient level binarization in Transform Skip [M. Abdoli, R. G. Youvalari, F. Plowman, A. Tissier (Xiaomi)] [late]

#### Other (3)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0171 Non-EE2: Picture-level mirroring and rotation [D. Mieloch, M. Lorkiewicz, J. Stankowski (PUT)]

JVET-AK0173 Non-EE2: Adaptive picture-level vertical mirroring [D. Mieloch, M. Lorkiewicz, A. Dziembowski, J. Stankowski, D. Karwowski (PUT)]

JVET-AK0219 Non-EE2: Chroma partition prediction in separate tree condition [P.-H. Lin, J.-L. Lin, V. Seregin, M. Karczewicz (Qualcomm)]

### CTC for EE2/ECM and general ECM improvements (0)

This section is kept as a template for future use.

See also discussion under 4.7 and 4.15.

# High-level syntax (HLS) and related proposals (XX)

## AHG9: Aspects on SEI messages in VSEIv4 and related (34)

### General comments (1)

Contributions in this area were discussed at 1810–1830 on Tuesday 14 Jan. 2025 (chaired by Y.-K. Wang).

JVET-AK0160 AHG9: On VVC interface of object mask information and annotated regions SEI messages [J. Chen, B. Chen, Y. Ye (Alibaba), M. M. Hannuksela (Nokia)]

Chaired by Ye-Kui Wang on 14 January 2025 at 18:10.

This contribution proposes the following items:

- It is proposed to add the interface of OMI SEI message in VVC spec., which is currently missing.

Two alternative options are proposed. The second option needs change to VSEI and VVC text.

- It is proposed to remove the interface variables ConfWinLeftOffset and ConfWinTopOffset from the OMI and AR SEI messages and clarify that the coordinates indicated by these SEI messages are relative to the top-left luma sample of the cropped decode picture. The changes proposed in this item are asserted to be editorial.

This part relates to option 2.

Regarding AR SEI message JVET-AE0054 is a previous related proposal but was only for AR SEI. That was marked for further study.

It was agreed that we need interface text (first part).

A comment was made that the change related to including conformance cropping (using cropped decoded picture) makes things clear and consistent (e.g. for AR) and so option 2 is preferred.

It was asked if the change to AR part proposed is editorial and it was confirmed by author to be the case.

Two non-proponents supported option 2.

Decision: Adopt option 2.

Regarding AR SEI message JVET-AE0054 is a previous related proposal but was only for AR SEI. That was marked for further study.

It was agreed that we need interface text (first part).

A comment was made that the change related to including conformance cropping (using cropped decoded picture) makes things clear and consistent (e.g. for AR) and so option 2 is preferred.

It was asked if the change to AR part proposed is editorial and it was confirmed by author to be the case.

Two non-proponents supported option 2.

Decision: Adopt option 2.

### SEI processing order and processing order nesting SEI message aspects (5)

Contributions in this area were discussed at 1530–1915 on Tuesday 14 Jan. 2025 (chaired by J. Boyce).

JVET-AK0055 AHG9: Semantics of the SPO SEI message [Y.-K. Wang, J. Xu, L. Zhang (Bytedance), K. Yang, Y. Li, Y. Xu (SJTU)]

This contribution proposes the following changes to the semantics of the SPO SEI message:

- Add a constraint to require that the value of po_sei_importance_flag[ i ] shall be equal to 0 when po_sei_payload_type[ i ] is equal to any value in spoPropertySeiList, i.e., the i-th type of SEI message indicates a property.

Justification: It is asserted that an SEI message type that only indicates a property should never determine that a processing chain must be ignored when the post processor does not recognize the SEI message type.

A participant questioned if this change was necessary, and it doesn’t seem harmful to let the encoder have this flexibility.

A participant noted that there are two flags to which this might apply.

No action.

- Add a constraint to require that a processing chain shall not include both an ERP SEI message type and a GCMP SEI message type.

Justification: It is asserted that it does not make sense for a processing chain to include an RWP SEI message but not an ERP or GCMP SEI message.

A participant questioned if there was a restriction against having both ERP and GCMP SEI messages in the same scope. If we do have such a constraint, it would be good to be consistent.

It was questioned whether it would be more appropriate for such a constraint to be present in in the video coding spec (VVC, HEVC, AVC) rather than in VSEI. The proponent suggests that if a video coding spec refers to both ERP and GCMP, the constraint could be applied in VSEI.

Revisit.

- Add a constraint to require that, when a processing chain includes an RWP SEI message type, it shall also include either an ERP or GCMP SEI message type.

Justification: It is asserted that it does not make sense for a processing chain to include an RWP SEI message type but not either an ERP or GCMP SEI message type.

It was suggested that the wording should not preclude use of a different projection format defined in the future with RWP. It is possible to change constraints in the future if/when a different projection format is defined, but we need to remember to do that.

It was suggested that it would be desirable to impose such constraints in a similar way to how the constraints are imposed in the absence of SPO.

It was noted that sometimes an SEI message has the same name in more than one video coding spec but has a different definition.

Decision: Adopt #3

- Due to that certain order in omnidirectional processing at the decoder side needs to be followed, the following constraints are proposed:

  - When a processing chain includes both an RWP SEI message type and an ERP SEI message type, the indicated processing order for the RWP SEI message type shall be later than that for the ERP SEI message type.

  - When a processing chain includes both an RWP SEI message type and a GCMP SEI message type, the indicated processing order for the RWP SEI message type shall be later than that for the GCMP SEI message type.

  - When a processing chain includes both an ERP SEI message type and a frame packing arrangement (FPA) SEI message type, the indicated processing order for the ERP SEI message type shall be later than that for the FPA SEI message type.

  - When a processing chain includes both a GCMP SEI message type and an FPA SEI message type, the indicated processing order for the GCMP SEI message type shall be later than that for the FPA SEI message type.

  - When a processing chain includes an ERP SEI message type, an FPA SEI message type, and an RWP SEI message type, the indicated processing order for the ERP SEI message type shall be later than that for the FPA SEI message type, and the indicated processing order for the FPA SEI message type shall be later than that for the RWP SEI message type.

  - When a processing chain includes a GCMP SEI message type, an FPA SEI message type, and an RWP SEI message type, the indicated processing order for the GCMP SEI message type shall be later than that for the FPA SEI message type, and the indicated processing order for the FPA SEI message type shall be later than that for the RWP SEI message type.

It was suggested that the abstract wording is not consistent with the language in the spec attachment.

Decision: Adopt #4 as described in the spec attachment. A new version of this contribution to be uploaded which fixes the abstract to be consistent with the spec text.

- Many SEI messages include a persistence cancel flag in the syntax. For example, the NNPFA SEI message syntax includes the nnpfa_cancel_flag, which, when equal to 1, cancels the persistence of the target NNPF established by any previous NNPFA SEI message with the same nnpfa_target_id. Currently, it is allowed for the SEI prefix indication of an SEI message type in the SPO SEI message to have the persistence cancel flag equal to 1. However, it is asserted that that does not make sense.

It is therefore proposed that, when an SPO SEI message includes an SEI message type and the SEI prefix indication for the SEI message type includes the persistence cancel flag, it is required that the value of the persistence cancel flag included in the SEI prefix indication for this SEI message type shall be equal to 0. Specifically, the following constraints are proposed:

The detailed proposed text changes, marked relative to JVET-AJ2006-v3, are included in an attachment to this contribution.

It was suggested that a note may be able to used in place of the amount of text, although a note may be difficult to provide correct language for.

No action.

JVET-AK0112 AHG9: On the SPO SEI message [Y. Gao, P. Wu, Y. Bai, S. Xie, M. Jia, W. Niu, C. Huang (ZTE)]

Chaired by S. Deshpande on 14 January 2025 at 18:25.

It is asserted that the current design of poSeiList derivation related to sub-chains in JVET-AJ2006 has the following problem:

- When there is no SEI message in the processing chain specified by this SPO SEI message such that po_sei_importance_flag[ i ] is equal to 0 and po_sei_processing_degree_flag[ i ] is equal to 1, the lists PoSeiList derived from the derivation process described in JVET-AI2006 and JVET-AJ2006 are inconsistent in some cases.

- When there is more than one SEI message in a sub-chain such that po_sei_importance_flag[ i ] is equal to 1 and po_sei_processing_degree_flag[ i ] is equal to 0, the sub chain processing result derived from JVET-AJ2006 may not be consistent with the initial intention of the content provider.

This contribution proposes the following modifications to address the above asserted problems:

Change the Interpretation of the value combination of po_importance_flag[ i ]=1 and po_processing_degree_flag[ i ] =0 to the following:

“ The i-th SEI message type belongs to a sub-chain. when the decoding system cannot interpret or does not support the i-th SEI message type ,the derivation of PoSeiList associated with this sub-chain is terminated.”

No comments or conflicting opinions were voiced regarding the two cases which show different behavior of JVET-AI2006 and JVET-AJ2006.

It was commented that JVET-AK0165 also addresses the nested sub-chains aspect (similar to this proposal), but the proposed solution is different than that in JVET-AK0112.

It was decided to make the decision after discussing that proposal. See notes for that below.

It is asserted that the current design of poSeiList derivation related to sub-chains in JVET-AJ2006 has the following problem:

- When there is no SEI message in the processing chain specified by this SPO SEI message such that po_sei_importance_flag[ i ] is equal to 0 and po_sei_processing_degree_flag[ i ] is equal to 1, the lists PoSeiList derived from the derivation process described in JVET-AI2006 and JVET-AJ2006 are inconsistent in some cases.

- When there is more than one SEI message in a sub-chain such that po_sei_importance_flag[ i ] is equal to 1 and po_sei_processing_degree_flag[ i ] is equal to 0, the sub chain processing result derived from JVET-AJ2006 may not be consistent with the initial intention of the content provider.

This contribution proposes the following modifications to address the above asserted problems:

Change the Interpretation of the value combination of po_importance_flag[ i ]=1 and po_processing_degree_flag[ i ] =0 to the following:

“ The i-th SEI message type belongs to a sub-chain. when the decoding system cannot interpret or does not support the i-th SEI message type ,the derivation of PoSeiList associated with this sub-chain is terminated.”

No comments or conflicting opinions were voiced regarding the two cases which show different behavior of JVET-AI2006 and JVET-AJ2006.

It was commented that JVET-AK0165 also addresses the nested sub-chains aspect (similar to this proposal), but the proposed solution is different than that in JVET-AK0112.

It was decided to make the decision after discussing that proposal. See notes for that below.

JVET-AK0156 AHG9: On SEI processing order SEI message [C. H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

This contribution proposes to enhance the SEI processing order (SPO) SEI message with information related to potential optimization for energy savings purposes.

When energy consideration comes to play, such information can for example be used by a receiver to decide whether or not to apply the processing chain specified by the SPO SEI message.

It was noted that the po_reserved_zero_2bits should be in the range of 0 to 3, not 0 to 4.

One possible use case is that two SPO chains are present, with one SPO chain for normal operation and another SPO chain for energy efficient operation.

A question was raised about how the encoder would know what post-processing (applied after the SPO processing chain is applied) would be applied, which would be needed to indicate energy savings.

A question was raised about example use cases for bit number 3 of energy savings can be expected when using the content for machine analysis other than reducing the sample rate via spatial or temporal resampling. Cropping was suggested as an operation.

A question was raised as to what other processing chain that a chain marked as energy savings would be relative to. The proponent suggested that it would be relative to not applying the SPO at all.

It was suggested that it is not clear how an encoder can know if energy savings would occur for post-processing following the SPO operations, and that a receiver would know that it could trust that information.

If more than one SPO chain indicates energy savings, it would be unclear what the energy savings was relative to.

A participant noted that energy management will likely become important in the future.

Further study encouraged, including providing some use case examples involving SEI messages included in VSEI or the TuC.

JVET-AK0165 AHG9: On SPO sub-chain signaling [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

Chaired by S. Deshpande on 14 January 2025 at 16:40.

This contribution proposes modifications on the signaling of sub-chains from the SPO. The following options are proposed:

- Change the interpretation of state po_sei_importance_flag[ i ] = 1, po_sei_processing_degree_flag[ i ] = 1, when within a sub-chain such that the concept of critical sub-chains is created, allowing to have dependency for further processes only.

- Constrain the usage of state po_sei_importance_flag[ i ] = 0, po_sei_processing_degree_flag[ i ] = 1, when outside of a sub-chain.

- Use, as an alternative to the two separate flags, a single syntax element in the form of an idc, to simplify the derivation of PoSeiList and the interpretation of the processing states.

Item 3 is editorial and was discussed at previous meeting.

Regarding item 1 of this proposal: It was asked what is the behaviour when sub-chain is “closed” with flags 0,1. It was answered that the change relates to closing a sub-chain with flags 1,1.

It was commented that in JVET-AK0112 for example on page 4 of JVET-AK0165, the SEI with i=0 will be processed if SEI i=1 can not be processed, but the behavior is different in the JVET-AK0165 proposed solution. It was commented by proponents that the signaling should be different to support that example.

It was asked if the JVET-AK0165 proposal does not allow “critical” sub-chain of length 1.  That seemed to be the case.

Regarding item 2 of this proposal, it proposes to add a constraint.  It was commented that this can be decided after the decision regarding JVET-AK0112 and item 1 of this proposal.

Proponents of JVET-AK0112 commented that it is important to support the originally discussed use cases (e.g. broadcast use case). It was commented by proponents of JVET-AK0165 that the current VSEI draft does not support nesting and their proposed approach is trying to bring it back. It was asserted by JVET-AK0165 proponents that the proposal in JVET-AK0112 cannot support sub-chain with more than 2 important stages.

It was commented that we may need to decide which of these two things are important:

-to support sub-chains of length 1 or

-to support nested sub-chains with more than 2 important stages

It was commented that initial version of JVET-AJ0128 had retained the nesting functionality, but later revisions lost that functionality. So it was requested to also consider JVET-AJ0128 v1 and the proponent of that document volunteered to update the solution in it to show changes with respect to current VSEI text if the group was interested in.

There was a general agreement to support nested functionality.

An off-line activity was planned among JVET-AK0112, JVET-AK0165 and JVET-AJ0128-v1 proponents and other interested parties. The activity will be co-ordinated by M. M. Hannuksela.

Revisit item 1

Regarding item 2: It is asserted that 0, 1 can be used to close the sub-chain currently irrespective of if sub-chain is opened or not. There was support for the proposed approach of adding a constraint from a non-proponent. But some more time to review was requested by a participant.

Revisit item 2 and item 3

Regarding item 1 of this proposal: It was asked what is the behaviour when sub-chain is “closed” with flags 0,1. It was answered that the change relates to closing a sub-chain with flags 1,1.

It was commented that in JVET-AK0112 for example on page 4 of JVET-AK0165, the SEI with i=0 will be processed if SEI i=1 can not be processed, but the behavior is different in the JVET-AK0165 proposed solution. It was commented by proponents that the signaling should be different to support that example.

It was asked if the JVET-AK0165 proposal does not allow “critical” sub-chain of length 1.  That seemed to be the case.

Regarding item 2 of this proposal, it proposes to add a constraint.  It was commented that this can be decided after the decision regarding JVET-AK0112 and item 1 of this proposal.

Proponents of JVET-AK0112 commented that it is important to support the originally discussed use cases (e.g. broadcast use case). It was commented by proponents of JVET-AK0165 that the current VSEI draft does not support nesting and their proposed approach is trying to bring it back. It was asserted by JVET-AK0165 proponents that the proposal in JVET-AK0112 cannot support sub-chain with more than 2 important stages.

It was commented that we may need to decide which of these two things are important:

-to support sub-chains of length 1 or

-to support nested sub-chains with more than 2 important stages

It was commented that initial version of JVET-AJ0128 had retained the nesting functionality, but later revisions lost that functionality. So it was requested to also consider JVET-AJ0128 v1 and the proponent of that document volunteered to update the solution in it to show changes with respect to current VSEI text if the group was interested in.

There was a general agreement to support nested functionality.

An off-line activity was planned among JVET-AK0112, JVET-AK0165 and JVET-AJ0128-v1 proponents and other interested parties. The activity will be co-ordinated by M. M. Hannuksela.

Revisit item 1

Regarding item 2: It is asserted that 0, 1 can be used to close the sub-chain currently irrespective of if sub-chain is opened or not. There was support for the proposed approach of adding a constraint from a non-proponent. But some more time to review was requested by a participant.

Revisit item 2 and item 3

JVET-AK0205 AHG9: On SEI messages in spoPropertySeiList [H. Tan, J. Lee, J. Nam, C. Kim, J. Lim, S. Kim (LGE)]

It is asserted that the design of categorizing the SEI types into the list of SeiProcessingOrderSeiList, SpoProcessSeiList, and spoPropertySeiList is not clear on several aspects such as the meaning of having property SEI messages in a certain position (i.e., processing order) within a processing chain, why property SEI messages can be the first SEI message in a processing chain (i.e., what the different between that case with simply having the SEI message not included in an SPO SEI message), and finally, the process that may be associated with property SEI message (e.g., frame packing arrangement SEI message has associated unpacking process but the SEI is not included in SpoProcessSeiList).

This contribution includes several items to clarify the asserted unclear aspects above as follows:

- To clarify the impact of SEI message in a processing chain that is part of spoPropertySeiList, do one of the following options:

  - Option 1:

    - Add a constraint such that SEI messages that are included in spoPropertySeiList shall not be the first SEI message in any processing chain.

    - When an SEI message that is included in spoPropertySeiList is the i-th SEI in a processing chain, it indicates property of the associated picture after the invocation of post-processing processes implied by up to the (i – 1)-th SEI message in the processing chain.

  - Option 2:

    - When an SEI message that is included in spoPropertySeiList is the first SEI in a processing chain, it indicates property of the associated picture prior to the application of any post-processing process in the processing chain.

    - Otherwise (i.e., an SEI message that is included in spoPropertySeiList is the i-th SEI in a processing chain), it indicates property of the associated picture after the invocation of post-processing processes implied by up to the (i – 1)-th SEI message in the processing chain.

- Add text that describes when the i-th SEI message in a processing chain is included in spoPropertySeiList and the SEI message may be associated with a particular process, such process is not part of processing chain.

It was suggested that Item 1 Option 2 is an expression of our current intent, which could potentially be done with an editorial change to the semantics, rather than the proposed language in the processing chain.

Revisit Item 1 after language is provided to express the intent in the semantics.

For item 2, it was suggested by a participant that the suggested language conflicts with existing language.

It was suggested that an associated process is difficult to describe.

It seems difficult to write a proper note.

No action on item 2.

### NNPF modifications (9)

Contributions in this area were discussed at 1915–1950 on Tuesday 14 Jan. 2025 (chaired by J . Boyce ).

JVET-AK0053 AHG9: NNPF interface text in VVC [Y.-K. Wang, J. Xu, L. Zhang (Bytedance), K. Yang, Y. Li, Y. Xu (SJTU)]

It is asserted that, in the current NNPF interface text in the VVC spec, some aspects are not clearly specified, including:

- The NNPF process may also be applied to an inserted (interpolated or extrapolated) picture, in addition to be applied to a cropped decoded output (CDO) picture or a processed version of a CDO picture. However, this aspect is not clearly specified. Furthermore, in this case, it is not clear what the current picture denoted by the variable currPic is.

- For NNPFs, an input picture may be unavailable in the list of candidate input pictures but is padded based on pictures in that list or padded to be zero-valued. However, when the list of candidate input pictures is not a list of CDO pictures, this aspect is not clearly specified.

- The phrases like “currPic is the last picture of the bitstream in output order”, “a picture prevPic that is the last cropped decoded output picture in output order among all cropped decoded output pictures”, and “SliceQpY of the first slice of inputPic[ i ]” are vague and unclear when the candidate input picture is a processed version of a CDO picture and more unclear when the candidate input picture is an inserted picture.

This contribution proposes changes to the NNPF interface text in the VVC spec to address the above asserted issues.

The detailed proposed text changes, marked relative to JVET-AJ2005-v3, are included in an attachment to this contribution.

It was suggested by a participant that the contribution is editorial in nature as it expresses the current intent.

It was noted that previous guidance had been to localize changes when possible.

It was noted that other SEI messages have a different way of wording handling of cropped decoded pictures vs coded pictures, but that NNPF is more complicated, so the other SEI messages probably do not require this type of change.

It was suggested to move the definitions from D.2.1 to D.12.11, to be local to NNPF interface text.

Decision: Adopt with moving the definitions from D.2.1 to D.12.11.

JVET-AK0054 AHG9: Semantics of the NNPFC and NNPFA SEI messages [Y.-K. Wang, J. Xu, L. Zhang (Bytedance), K. Yang, Y. Li, Y. Xu (SJTU)]

JVET-AK0070 AHG9: On seed parameter for NNPF [T. Chujoh, Z. Fan, R. Ishimoto, T. Ikai (Sharp), L. Jin, H. Watanabe (Waseda Univ.)]

TBP

JVET-AK0072 AHG9: Comments on NNPF SEI messages [M. M. Hannuksela, F. Cricri (Nokia)]

TBP

JVET-AK0073 AHG9: Enabling multiple instances of NNPF extrapolated pictures [M. M. Hannuksela, F. Cricri (Nokia)]

TBP

JVET-AK0074 AHG9: Handling output variability of an NNPF [M. M. Hannuksela, F. Cricri (Nokia)]

TBP

JVET-AK0147 AHG9: On NNPF for Tone Mapping with Colour Volume Information [C.-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0152 AHG9: On Spatial Extrapolation for NNPF [S. Deshpande (Sharp), M. M. Hannuksela (Nokia)]

TBP

JVET-AK0167 AHG9: On Enabling the usage of auxiliary layers for NNPF [T. M. Borges, Y. Sanchez, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

TBP

JVET-AK0188 AHG9: On NNPF filtering control strength value adjustment [J. Xu, Y.-K. Wang (Bytedance)]

### Generative face video SEI messages (8)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0080 AHG9: Comments on the GFV and GFVE SEI messages [M. M. Hannuksela (Nokia), J. Chen, B. Chen, Y. Ye (Alibaba)]

TBP

JVET-AK0124 AHG9: On timing information and order of pictures in GFV SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0127 AHG9: On miscellaneous aspects of GFV and DSCI SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0128 AHG9: Editorial updates for GFV SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0154 AHG9: Comments on Generative Face Video SEI [A. C. Sidiya, S. Deshpande (Sharp)]

JVET-AK0164 AHG9: Supplementation of value range definition and editorial bugs fixes on the GFVE pupil position SEI messages [F. Ma, A. Trioux, F. Yang (Xidian Univ.), B. Li, F. Xing, Z. Wang (Hisense)]

JVET-AK0238 AHG9: Semantics fixes for generative face video (GFV) SEI message [J. Chen, B. Chen, Y. Ye (Alibaba)] [late]

JVET-AK0239 AHG9: On generative face video enhancement (GFVE) SEI message [J. Chen, B. Chen, Y. Ye (Alibaba)] [late]

### Digitally signed content SEI messages (8)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0109 AHG9: On signaling and constraint for RefDigest in digitally signed content SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0110 AHG9: On reference message digest for verification in digitally signed content SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0125 AHG9: On presence and persistency of digitally signed content SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0126 AHG9: On signaling the mapping between verification substreams and layer and temporal sublayer for DSCI SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0127 AHG9: On miscellaneous aspects of GFV and DSCI SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AK0194 AHG9: Digitally Signed Content SEI messages for AVC and HEVC [K. Sühring, T. Hinz, Y. Sanchez, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI)]

JVET-AK0206 AHG9: On digitally signed content SEI messages [S. McCarthy, C. Fersch, I. Sodagar (Dolby)]

JVET-AK0287 AHG9: Multilayer digitally signed content authentication SEI messages [J. Boyce, M. M. Hannuksela (Nokia)] [late]

### Other (3)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0075 AHG9: Additional information on object-based optimization for the encoder optimization information SEI message [M. Pedzisz, M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AK0155 AHG9: On Encoder optimization information SEI message [C. H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0168 AHG9: On SPTI SEI message: Robustness and source constant framerate [Y. Sanchez, T. M. Borges, R. Skupin, C. Hellge, T. Schierl (Fraunhofer HHI)]

## AHG9: Aspects on SEI messages in TuC and related (23)

### Constituent rectangles SEI (4)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0115 AHG9: On supporting 4:4:4 color format in constituent rectangles and enhanced colour format information SEI messages [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0209 AHG9: On constituent rectangles SEI message [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0210 AHG9: On constituent rectangle grouping for 4:4:4 color format support in constituent rectangle SEI message [H. Tan, J. Nam, C. Kim, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0278 AHG9: Constituent Rectangles / Enhanced Colour Format Information SEI extension to support 4:2:2 source content coded in bitstream with 4:2:0 chroma format [S. Keating, M. Ikeda (Sony)] [late] [miss]

### Display overlay SEI (6)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0099 AHG9: On bit depth alignment between target and coded pictures in DOI SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AK0100 AHG9: DOI SEI support for target picture with alpha channel [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

JVET-AK0157 AHG9: On the number of DOI SEI message in an AU [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0158 AHG9: On reconstruction of target display picture in DOI SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0190 AHG9: On the display overlays information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AK0265 AHG9: Display overlay sets for DOI SEI [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)] [late]

### Quality metrics SEI (3)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0159 AHG9: On miscellaneous aspects of quality metrics SEI message [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0207 AHG9: On signalling of average quality in quality metrics SEI message [H. Tan, J. Nam, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

JVET-AK0208 AHG9: On the inclusion of quality metric SEI message in SEI processing order SEI message [H. Tan, C. Kim, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

### Other (10)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0140 AHG9/AHG8: Showcase for Packed regions information SEI [J. Boyce, H. Zhang, M. M. Hannuksela (Nokia)]

JVET-AK0189 AHG9: On the bitdepth range information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AK0114 AHG9: Updates and suggestion on AI usage restrictions SEI message [C. Kim, H. Tan, J. Nam, J. Lee, J. Lim, S. Kim (LGE)]

JVET-AK0179 AHG9: On Lens Optical Correction SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AK0204 AHG9: Proposed Lens Optical Correction SEI message for version 5 of VSEI [S. Wenger, G. Teniou, A. T. Hinds (Tencent)]

JVET-AK0153 AHG9: On Photosensitive Content Information Signaling [S. Deshpande (Sharp)]

JVET-AK0058 [AHG9/AHG13] Comments on Film grain regions characteristics SEI message [S. Xie, P. Wu, Y. Gao, Y. Bai, C. Huang (ZTE)]

JVET-AK0211 AHG9/AHG13: Proposed Film Grain Region SEI message for version 5 of VSEI [S. Wenger, G. Teniou, A. T. Hinds (Tencent)]

JVET-AK0281 AHG9: On AI usage restrictions SEI message [M. M. Hannuksela, F. Cricri (Nokia)] [late]

JVET-AK0286 AHG9: Simplified enhanced colour format information SEI [J. Boyce, M. M. Hannuksela (Nokia)] [late]

## AHG9: Other SEI topics (6)

Contributions in this area were discussed at XXXX–XXXX on XXday XX Jan. 2025 (chaired by XXX).

JVET-AK0107 AHG9: Modality Information SEI for HEVC [J. Gao, H.-B. Teo, C.-S. Lim, K. Abe, V. Drugeon (Panasonic)]

JVET-AK0136 AHG9: Target Colour Volume SEI message [C. H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0147JVET-AK0147 AHG9: On NNPF for Tone Mapping with Colour Volume Information [C.-H. Demarty, E. François, F. Aumont, O. Le Meur (InterDigital)]

JVET-AK0142 AHG9: Display rectangles SEI [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

JVET-AK0191 AHG9: Examples of danmaku applications [J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

JVET-AK0203 AHG9: On signalling segmentation and object tracking information [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

## Non-SEI HLS aspects (0)

Kept as template for future use.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

The following topics in JVET plenary XXday X Jan. XXXX–XXXX:

- Scheduling for remaining week (further detail on scheduling is recorded in section 2.12).

- Joint meetings involving JVET were scheduled as follows:

- …

Further detail about joint sessions with other groups is provided in the subsection 7.3.

Break-out group reports are discussed in section 7.4.

General plenary wrap-up discussions are recorded under sections 8, 9, and 10.

## Information sharing meetings

(kept for future use in meetings under SC 29 auspices)

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 15 July 0900–1200, Wednesday 17 July 0900–1000, and Friday 19 July 1400–1600.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

### Joint session XXXX-XXXX XXday X Jan. on XXXX: MPEG WG X / XXXX, MPEG AG 5 / Quality Assessment, MPEG WG 5 / JVET and VCEG (ITU-T Q6/21)

(These notes were recorded by XXX)

Chaired by XXX (XXX Convenor/Rapporteur), … .

## BoGs (X)

The following break-out groups were established at this meeting to conduct discussion and develop recommendations on particular subjects.

## Liaison communications (X) (update)

m70439 Liaison statement from SC 29/WG 1 to WG 5 on JPEG AI [WG 1 via SC 29 Secretariat]

The liaison response WG 5 N 324 was reviewed in JVET on Thursday 7 Nov. at 1445-1500. The draft reply was also presented in the MPEG AG 3 Communication meeting Thursday 7 Nov. at 1500-1800.

# Project planning

## Software timeline (update)

ECM 15.0 software (including all adoptions) was planned to be available 3 weeks after the meeting (29 November).

The NNVC 11.0 codebase software was planned to be available 3 weeks after the meeting (29 November).

Extensions on top of VTM23.5 software will be released as appropriate (e.g., integration and updates of SEI messages incuded in JVET-AJ2006 by the current meeting).

Updates on top of HM18.0 and HTM16.3 software were not planned, but might be released after merging pending requests, as appropriate.

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning (update)

An EE on neural network-based video coding was established, as recorded in output document JVET-AJ2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AJ2024.

Initial versions of these documents were presented and approved.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-T2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow is described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Review of AHG plans was conducted during the plenary on XXday X Jan. 2025 at XXXX–XXXX.

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AJ2005 and JVET-AJ2006). - Collect reports of errata for the VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports and produce the JVET-AJ1004 errata output collection. - Coordinate with the test model software development AhG to address issues relating to mismatches between software and text. - Collect and consider errata reports on the texts. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target conducting tests for VVC multi-layer features, and update the test plan accordingly. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing. - Coordinate with AHG15 on investigating sequences with gaming content, and make such sequences available for study. - Coordinate with AHG17 on investigating sequences outside of CTC, and making arrangements for viewing at the next meeting. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | N |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study the draft of additional conformance bitstreams for VVC multilayer configurations JVET-AI2028, and investigate the need for future improvements of conformance testing specifications. - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AI1008, and further develop related conformance bitstreams. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-15.0 software version (and potential updates), corresponding VTM anchor, and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| ECM tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing. - Prepare reporting of tool assessment results. - Coordinate with AHG17 to collect simulation results on non-CTC sequences (e.g., those used in previous verification tests), and identify a set of non-CTC sequences that would be appropriate for additional testing. - Develop methodology of more reliable runtime measurement | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, X. Li, J. Pardo, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Generate anchors according to the common test conditions JVET-AI2031. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development, and continue to migrate the software basis used in AHG8 to newest VTM version. - Coordinate experiments on optimization of encoders and receiving systems for machine analysis of coded video content, including combinations of proposed technologies. - Maintain the software implementation examples and develop tool combination examples in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose improvements to JVET-AJ2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potential of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Investigate the impact of using different machine task models in the evaluation of the compression performance of tools optimized for machine analysis tasks. - Coordinate with WG 4 VCM AHG on aspects such as unified common test conditions, evaluation metrics, test and training materials, usage of SEI messages, and on studying characteristics and requirements of machine analysis tasks, etc. Provide WG 4 VCM with AHG software and scripts to experiment non-normative tools and tool combination examples, and provide bitstreams generated by running a dense set of QP points for each sequence. | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) | Y (tel., 2 weeks notice, joint with WG 4 AHG) |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AJ2006 and identify any issues and propose solutions as appropriate. - Study JVET-AJ2032 and propose improvements. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages, including the study of SEI messages defined in HEVC and AVC for potential use in the VVC context. - Study the alignments of the same SEI messages in different standards. - Coordinate with AHG8 and WG 4 to study mechanisms for signalling metadata in the context of machine analysis of coded video content. - Coordinate with AHG3 for software support of SEI messages, both for HM and VTM. | S. McCarthy, Y.-K. Wang (co-chairs), J. Boyce, T. Chujoh, S. Deshpande, C. Fogg, M. M. Hannuksela, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, or higher objective quality, and coordinate such efforts with AHG3 and AHG6. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | P. de Lagrange, A. Duenas, R. Sjöberg, A. Tourapis (AHG chairs) | Y (tel., 2 weeks notice) |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Update the test conditions for NN-based video coding in JVET-AJ2016, including list, location and md5sums for training set. Generate and distribute anchor encoding, and develop supporting software as needed. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. - Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology. - Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also considering the impact of sparse models. - Finalize and discuss the EE on neural network-based video coding. - Promote the call for training materials, distribute it, and actively communicate with content owners. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. If possible, prepare encodings with combinations of tools included in the NNVC software for visual quality assessment at the next meeting. - Coordinate with AHG14 on items related to NNVC software development and studying the impact of training set extension. | E. Alshina, F. Galpin, S. Liu, A. Segall (co-chairs), J. Li, R.-L. Liao, D. Rusanovskyy, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) | Y (tel., 2 weeks notice), first on Nov. 28, second on Dec. 20 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM15 algorithm description JVET-AJ2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Support AHG6 in generating anchors according to the test conditions in JVET-AI2017. - Analyse the results of exploration experiments described in JVET-AJ2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss and propose refinements to the draft of the TR 2nd ed. JVET-AJ2020. - Study alternative film grain models and their associated documentation. - Discuss and enumerate updates, improvements, and additions for the second edition of the technical report. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the plan for subjective quality testing of the FGC SEI message JVET-AJ2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), A. Duenas, D. Grois, Y. He, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-11.0 software version (and potential updates), and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AJ2016. Study the impact of the addition of new dataset on the already integrated models. - Investigate and bridge coding performance of NNVC VTM anchor compared to the latest VTM version and/or VTM anchor in ECM software. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (HOP/LOP/VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AJ2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. - Coordinate with AHG11 on items related to NNVC activities. | F. Galpin (chair), R. Chang, Y. Li, Y. Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Nov. 28, second on Dec. 20 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AJ2027, and provide results at the next meeting. - Develop and maintain software elements for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for transporting them in the video bitstream. - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Study conversion of depth maps into integer representation that could be input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang (vice chairs) | Y (tel., 2 weeks notice) |
| Generative face video compression (AHG16) (jvet@lists.rwth-aachen.de) - Maintain GFVC software tools, associated configuration files, and software usage documentation. - Study the extension of GFVC software capability to handle video content with higher resolutions. - Identify and study additional test content, including content with higher resolutions, suitable for use in GFVC performance evaluation. - Study GFVC performance under test conditions defined in JVET-AJ2035, as well as performance on additional test content, and wider bitrate ranges. - Coordinate with AHG9 on further development of the GFV and GFVE SEI messages in JVET-AJ2006. | Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs) | N |
| Testing of video coding technology beyond CTC (AHG17) (jvet@lists.rwth-aachen.de) - Identify test sequences extending the represented range of applications (i.e. beyond those defined in common test conditions documents JVET-AB2010, JVET-AC2011 and JVET-AI2017) and investigate their suitability for use in visual quality assessments. - Prepare encoded bitstreams from the identified test sequences using VTM23.5 and ECM14.1 - Make preparations for viewing at the 37th JVET meeting in coordination with AHG4 and AG 5. - Develop the output document JVET-AJ2026, starting from the BoG report JVET-AJ0378. - Investigate potential need of arranging a face-to-face AHG meeting (with remote participation for group discussions) between the 37th and 38th JVET meetings. | M. Wien (chair), Y. Ye, V. Baroncini, E. Alshina (vice chairs) | Y (tel., 2 weeks notice, first on Nov. 20, second on Dec. 18) |

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

JVET-AJ1000 Meeting Report of the 37th JVET Meeting [J.-R. Ohm] [WG 5 N XXX] (2025-02-19)

Initial versions of the meeting notes (d0 … dX) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] [WG 5 N 103]

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis] (2024-06-28)

Primary editor: G. J. Sullivan.

JVET-AJ1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan] (2024-12-31, near next meeting)

Primary editor: Y.-K. Wang.

This includes changes from new bug tickets.

Remains valid – not updated: JVET-AH1005 Technology under consideration for future editions of CICP [E. Thomas, A. Tourapis] [WG 5 N 289)] (2024-06-28)

From JVET-AH0154 and JVET-AH0217.

JVET-AJ1006 HEVC extensions and corrections (draft 1) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis ] (2024-12-20)

Request for amendment WG 5 N 320 was reviewed Friday 8 Nov. 1030.

Primary editor: Y.-K. Wang.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Remains valid: JVET-AI1008 Conformance testing for HEVC multiview extended and monochrome profiles [I. Moccagatta, S. Paluri, A. Tourapis, Y.-K. Wang]

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174] (2024-12-06)

JVET-AJ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien] (2024-12-31)

Requires update for new test sequence download server, and new JCT-VC/-3V sites

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Remains valid – not updated: JVET-AH1016 AVC with extensions and corrections (draft 3) [B. Bross, T. Ikai, G. J. Sullivan, A. Tourapis, Y.-K. Wang] (2024-06-28)

Primary editor: B. Bross.

No output: JVET-Axx1017 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

No output: JVET-Axx2001

Remains valid – not updated: JVET-AH2002 Algorithm description for Versatile Video Coding and Test Model 22 (VTM 22) [Y. Ye, A. Browne, S. Kim] [WG 5 N 284] (2024-07-05)

Primary editor: Y. Ye.

New elements from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was suggested that editorial improvements submitted as input to the next meeting would be welcome.

JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring] (2024-11-22)

Update from JVET-AJ0187

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

JVET-AJ2005 Additions and corrections for VVC version 4 (Draft 10) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] [WG 5 preliminary DAM N 330)] (2024-12-06)

Also issue Draft DoC WG 5 N 329 (with editing period 2024-12-06)

Agreed changes are alignment of code points with JVET-AJ2006, text improvements, text for digitally signed content SEI messages as per JVET-AJ0151v3. Not yet to be sent for DAM ballot.

Primary editor: G. J. Sullivan.

JVET-AJ2006 Additional SEI messages for VSEI version 4 (Draft 4) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] [WG 5 preliminary DAM N 319)] (2024-12-06)

Also issue Draft DoC WG 5 N 321 (with editing period 2024-12-06)

New elements from notes elsewhere in this report:

- General editorial improvements

  - …

- SEI Processing Order SEI

  - …

- Neural Network Post Filter

  - …

- Encoder optimization information SEI

  - …

- Text description information SEI

  - …

- Source picture timing SEI

  - …

- Generative Face video SEI

  - …

- Digitally signed content SEIs

  - …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Primary editor: J. Boyce.

JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li] (2024-11-22)

To be updated from JVET-AJ0193.

Remains valid – not updated: JVET-X2008 Conformance testing for versatile video coding (Draft 7) [J. Boyce, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan]

Number might be re-used in future.

JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)] (2024-12-06)

DoCR on CD WG 5 N 321 was reviewed Friday 8 Nov. 1020.

Primary editor: F. Bossen.

Remains valid – not updated JVET-AB2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

Remains valid – not updated: JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall] (2024-11-22)

Developed from JVET-AJ0362

Also include the new requirement that an analysis as conducted in JVET-AJ0361 shall be submitted with proposals.

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye] (2024-08-30)

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

JVET-AJ2019 Description of algorithms version 9 and software version 11 in neural network-based video coding (NNVC) [F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang] (2024-12-13)

New elements in text and software from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AJ2020 Film grain synthesis technology for video applications ed. 2 (Draft 1) [D. Grois, Y. He, W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis] (2024-12-31)

Start with corrections on first edition, and first thoughts on other content to be added (see JVET-AJ0013)

JVET-AJ2021 Verification test plan for VVC multilayer coding (update 5) [O. Chubach, P. de Lagrange, M. Wien] (2024-12-31)

See discussion under JVET-AJ0366.

JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4) [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien] (2024-12-31)

Remove items except for future tests in category 3.

JVET-AJ2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, F. Galpin, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, R. Chang, Z. Xie (EE coordinators)] (2024-11-29)

An initial draft of this document was reviewed and approved at 0940-1000 on Friday 8 Nov.

This round of EE1 tests includes:

- …

JVET-AJ2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)] (2024-12-06)

An initial draft of this document was reviewed and approved at 1000-1015 on Friday 8 Nov.

This round of EE2 tests will include:

- …

JVET-AJ2025 Algorithm description of Enhanced Compression Model 15 (ECM 15) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] (2024-12-31)

New elements from notes elsewhere in this report:

- Intra prediction:

  - …

- Inter prediction:

  - …

- Transforms and coefficient coding:

  - …

- In-loop filtering:

  - …

- Software/CTC:

  - …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AJ2026 Testing of video coding technology beyond conditions of exploration experiments [M. Wien, Y. Ye, V. Baroncini, E. Alshina] (2024-11-29)

Developed from BoG JVET-AJ0378, to be finalzed in AHG telco

JVET-AJ2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud] (2024-11-22)

Developed from JVET-AJ0136.

Remains valid – not updated: JVET-AI2028 Additional conformance bitstreams for VVC multilayer configurations [S. Iwamura, P. de Lagrange, I. Moccagatta] (2024-08-02)

Remains valid – not updated: JVET-AH2029 Visual quality comparison of ECM/VTM encoding [V. Baroncini, J.-R. Ohm, M. Wien] [AG 5 N 118] (2024-07-05)

JVET-AJ2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 7) [S. Liu, J. Chen, J. Ström] [WG 5 CDTR 23888-3, N 323)] (2024-12-31)

Primary editor: S. Liu. New elements from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann] (2024-08-16)

JVET-AJ2032 Technologies under consideration for future extensions of VSEI (version 6) [S. McCarthy, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, H. Tan, Y.-K. Wang] (2024-12-06)

New elements from notes elsewhere in this report:

- General editorial improvements

  - …

- Constituent rectangles SEI

  - …

- Display overlays SEI

  - …

- Image Format Metadata SEI

  - …

- Packed regions information SEI

  - …

- Bitdepth range information SEI

  - …

- AI usage restrictions SEI

  - …

- Lens optical correction SEI

  - …

- Quality metrics SEI

  - …

- Photosenstive content information SEI

  - …

- Graphics rendering information SEI

  - …

- Neural Network Post Filter

  - …

- Derived chroma formats

  - …

It was agreed during approval of this document that the following removals are also to be implemented to the TuC:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was agreed that MPI is not further changed, and will be removed upon completion of a corresponding MIV profile.

Remains valid – not updated: JVET-AF2033 Report of verification test on VVC multi-layer coding: Content layering [S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 105)] (2023-12-22)

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)] (2024-07-26)

Developed from JVET-AI0266

JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen] (2024-11-29)

Add higher resolution sequences as proposed in JVET-AJ0209.

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)] (2024-02-09)

The draft of WG 5 recommendations was reviewed and approved in JVET at 1055-1125 on Friday 19 July.

JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)] (2024-12-31)

The draft of WG 5 recommendations was reviewed and approved in JVET at 1230-1255 on Friday 8 November.

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting

Future meeting plans were established according to the following guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG21 meeting – a total of 8-9 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Thursday or Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8–9 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans (to be confirmed) were established as follows:

- During Wed. 26 March – Fri. 4 April 2025, 38th meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting,

- During Thu. 26 June – Fri. 4 July 2025, 39th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Daejeon, – KR,

- During 2 – 10 October 2025, 40th meeting under ITU-T SG21 auspices in Geneva, CH,

- During 14 – 23 January 2026, 41st meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting,

- During 24 April – 1 May 2026, 42nd meeting under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES,

- During July 2026, 43rd meeting under ITU-T SG21 auspices, date and location t.b.d.

- During October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

A face-to-face meeting of AHG17 (with best-effort online access to discussions) may be conducted during March 10-14, 2025 in Aachen, Germany (estimated duration of 3 days), with the purpose of identifying non-CTC test cases for subjective investigation. This is to be decided by the 37th JVET meeting.

The agreed document deadline for the 38th JVET meeting was planned to be XXday X March 2025.

(update thanks)

TCL was thanked for offering new HDR test material, and Bytedance was thanked for offering test material showing user generated content.

TCL and Vestel were thanked for providing 4K displays, Fraunhofer HHI was thanked for providing play-out equipment used in the experts viewing in Kemer. Adam Wieckowski was thanked for helping with the test setup. The experts who volunteered to participate in the viewing were also thanked.

Marius Preda was thanked for maintaining the document site jvet-experts.org, and for re-installing access to the documents of JCT-VC and JCT-3V. Institut Mines-Télécom was thanked for hosting the sites.

Fraunhofer HHI and RWTH Aachen University were thanked for hosting the new JVET content sites.

Ali Begen was thanked for planning, preparing and hosting the 36th JVET meeting in Kemer. Mustafa Bay, Alev Yavuz, Basak Erel and Merve Dağlı of Dekon Congress and Tourism, and the staff of Mirage Park Resort Hotel were thanked for the excellent support during the meeting. Further thanks were expressed to the silver sponsors Ofinno, Perculus and Unified Streaming, bronze sponsors Dolby, Ericsson, Nokia, and Qualcomm, and supporters Kuru Kahveci Mehmet Efendi, Ozyegin University, TCL and Vestel.

The 37th JVET meeting was closed at approximately XXX hours CET on Wednesday 22 January 2025.

# Annex A to JVET report:
List of documents

(Dates and times in the table below are in Paris/Geneva time (1 hr. ahead of UTC).

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the thirty-seventh meeting of the JVET, according to a sign-in sheet circulated in the JVET meeting rooms (approximately XXX people in total), were as follows:

- …

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the thirty-sieventh meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

- …

# Annex C to JVET report:
Recommendations of the 18th meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG 21

ISO/IEC JTC 1/SC 29/WG 5 N XXX