| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 40th Meeting: Geneva, CH, 3–12 October 2025 | Document: JVET-AN_notes_dK |
| --- | --- |

| Title: | Meeting Report of the 40th Meeting of the Joint Video Experts Team (JVET), Geneva, 3 – 12 October 2025 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contacts): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its fortieth meeting during 3 – 12 October 2025 at the ITU headquarters facilities in Geneva, Switzerland. The meeting was held as a physical meeting with remote participation, provided on best-effort basis for experts who were unable to travel.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twenty-first meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 1400 hours CEST (UTC+2) on Friday 3 October 2025. Meeting sessions were held on all days including the weekend days of Saturday and Sunday 4, 5, 11 and 12 October 2025, until the meeting was closed at approximately 1445 hours CEST on Sunday 12 October 2025. Approximately 574 people attended the JVET meeting (295 in person and 279 remotely), and approximately 247 input documents (not counting crosschecks, reports, and summary documents), 18 AHG reports, 2 EE summary reports, 1 BoG report(s), and 1 incoming liaison document(s) were discussed. The meeting took place in a collocated fashion with a meeting of SG21 – one of the two parent bodies of the JVET, under whose auspices this JVET meeting was held. Various SC29 Working Groups and Advisory Groups were also meeting in Geneva with partial temporal overlap – where WG 5 is representing the Joint Video Experts Team and its activities from the perspective of the SC 29 parent body. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development. In the latter context, the evaluation of responses received on the Joint Call for Evidence on video compression with capability beyond VVC was a major activity at his meeting.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the thirty-ninth JVET meeting in producing the following documents:

a) JVET documents

JVET-AM1005 Future CICP extensions (Draft 1)

JVET-AM1006 HEVC additional profiles and SEI messages (Draft 4)

JVET-AM1008 Conformance testing for HEVC multiview extended and monochrome profiles

JVET-AM1017 Support for additional VSEI messages in AVC (Draft 3), also issued as WG 5 DAM N 363

JVET-AM1018 HEVC with extensions and corrections (Draft 2)

JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13)

JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7)

JVET-AM2019 Description of algorithms version 12 and software version 14 in neural network-based video coding (NNVC)

JVET-AM2021 Verification test plan for VVC multilayer coding (update 6)

JVET-AM2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AM2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AM2025 Algorithm description of Enhanced Compression Model 18 (ECM 18)

JVET-AM2026 Joint Call for Evidence on video compression with capability beyond VVC, also issued as WG 5 N 367

JVET-AL2032 Technologies under consideration for future extensions of VSEI (version 9)

JVET-AL2038 White paper on VSEI, also issued as AG 3 N 209

b) documents produced as WG 5 documents only:

WG 5 N 362 Disposition of comments received on ISO/IEC 14496-10:202x (11th ed.) CDAM 1

WG 5 N 364 Disposition of comments received on CD ISO/IEC 23090-15:202x (3rd ed.)

WG 5 N 365 Text of DIS ISO/IEC 23090-15:202x (3rd ed.) Conformance testing for versatile video

WG 5 N 366 Disposition of comments received on DIS ISO/IEC 23090-16:202x (2nd ed.) Reference software for versatile video coding

As main results, the JVET produced 19 output documents from the current meeting:

JVET-AN1005 Future CICP extensions (Draft 2)

JVET-AN1016 AVC with extensions and corrections (Draft 4)

JVET-AN1017 Support for additional VSEI messages in AVC (Draft 4)

JVET-AN1018 HEVC with extensions and corrections (Draft 3), also forwarded for consent as SG21-TD210/PLEN ITU-T H.265 (V11) "High efficiency video coding" (Rev.)

JVET-AN1019 VSEI with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD212/PLEN ITU-T H.274 (V4) "Versatile supplemental enhancement information messages for coded video bitstreams" (Rev.)

JVET-AN2001 VVC with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD211/PLEN ITU-T H.266 (V4) "Versatile video coding" (Rev.)

JVET-AN2008 Conformance testing for versatile video coding (Draft 8), also forwarded for consent as SG21-TD214/PLEN ITU-T H.266.1 (V3) " Conformance specification for ITU-T H.266 versatile video coding" (Rev.)

JVET-AN2019 Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC)

JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4)

JVET-AN2021 Verification test plan for VVC multilayer coding (update 7)

JVET-AN2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AN2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19)

JVET-AN2026 Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC

JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC, also issued as AG 5 N 180

JVET-AN2032 Technologies under consideration for future extensions of VSEI (version 10)

JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience

JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools

JVET-AN2041 Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026

- The following 6 documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 373 Disposition of comments received on ISO/IEC 23002-7:2024/DAM 1

WG 5 N 374 Text of FDIS ISO/IEC 23002-7:202x (4th ed.) Versatile supplemental enhancement information messages for coded video bitstreams

WG 5 N 375 Disposition of comments received on ISO/IEC 23008-2:2025/DAM 1

WG 5 N 376 Text of FDIS ISO/IEC 23008-2:202x (7th ed.) High efficiency video coding

WG 5 N 377 Disposition of comments received on ISO/IEC 23090-3:2024/DAM 1

WG 5 N 378 Text of FDIS ISO/IEC 23090-3:202x (4th ed.) Versatile video coding

For the organization and planning of its future work, the JVET established 19 “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, 2 Exploration Experiments (EE) were defined. The next ten JVET meetings were planned for 14 – 23 January 2026 under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting (with contribution deadline of 7 January 2026, and no plan to meet during the weekend 17 – 18 January); during 24 April – 1 May 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES; during 7 – 15 July 2026 under ITU-T SG21 auspices in Geneva, CH; during 17 – 23 October 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN; during 13 – 22 January 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU; during 17 – 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN; during January 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; and during April 2028 under ITU-T SG21 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. The most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively. JVET documents are also provided via http://wftp3.itu.int/av-arch/jvet-site/, but as this is cannot currently be maintained as a mirror with the jvet-experts.org site, documents are typically only made available with a delay of two or three meeting cycles, to avoid that late uploads of output documents would be missing.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its fortieth meeting during 3 – 12 October 2025 at the ITU headquarters facilities in Geneva, Switzerland. The meeting was held as a physical meeting with remote participation, provided on best-effort basis for experts who were unable to travel.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

As requested by the parent bodies, JVET had issued a call for evidence on video compression with capability beyond existing standards, with responses evaluated during the current meeting.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants, consisting of two parts, (B1) in-person attendees as recorded by a sign-in sheet circulated in meeting rooms, (B2) remote attendees as recorded by the teleconferencing tool used for the meeting

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2025_10_AN_Geneva/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the thirty-ninth JVET meeting in producing the following output documents:

a) JVET documents

JVET-AM1005 Future CICP extensions (Draft 1)

JVET-AM1006 HEVC additional profiles and SEI messages (Draft 4)

JVET-AM1008 Conformance testing for HEVC multiview extended and monochrome profiles

JVET-AM1017 Support for additional VSEI messages in AVC (Draft 3), also issued as WG 5 DAM N 363

JVET-AM1018 HEVC with extensions and corrections (Draft 2)

JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13)

JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7)

JVET-AM2019 Description of algorithms version 12 and software version 14 in neural network-based video coding (NNVC)

JVET-AM2021 Verification test plan for VVC multilayer coding (update 6)

JVET-AM2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AM2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AM2025 Algorithm description of Enhanced Compression Model 18 (ECM 18)

JVET-AM2026 Joint Call for Evidence on video compression with capability beyond VVC, also issued as WG 5 N 367

JVET-AL2032 Technologies under consideration for future extensions of VSEI (version 9)

JVET-AL2038 White paper on VSEI, also issued as AG 3 N 209

b) documents produced as WG 5 documents only:

WG 5 N 362 Disposition of comments received on ISO/IEC 14496-10:202x (11th ed.) CDAM 1

WG 5 N 364 Disposition of comments received on CD ISO/IEC 23090-15:202x (3rd ed.)

WG 5 N 365 Text of DIS ISO/IEC 23090-15:202x (3rd ed.) Conformance testing for versatile video

WG 5 N 366 Disposition of comments received on DIS ISO/IEC 23090-16:202x (2nd ed.) Reference software for versatile video coding

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the CET timezone (local time in Geneva), except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair, or other experts that were appointed to chair parallel sessions. It is indicated who took the notes in review of a given category (or individual documents în a category), and by which day and time the review was conducted. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 26 September 2025. Any documents uploaded after 1159 hours Paris/Geneva time on Saturday 27 September 2025 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AN0270 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AN0070 (a proposal on future CICP semantics change), uploaded 09-30,

- JVET-AN0270 (a proposal on CCLM clipping), uploaded 09-28,

- JVET-AN0275 (a proposal on combining EE2 elements), uploaded 10-01,

- JVET-AN0286 (a proposal on TMRL modification), uploaded 09-29,

- JVET-AN0287 (a proposal on PDPC modification), uploaded 09-30,

- JVET-AN0293 (a proposal on new multiview profiles), uploaded 10-01,

- JVET-AN0299 (a proposal on MPM modification), uploaded 09-29,

- JVET-AN0310 (a proposal on modifying PRI SEI), uploaded 10-01,

- JVET-AN0319 (a proposal on alternative flow computation in DRF), uploaded 10-02,

- JVET-AN0324 (a proposal on quantization center shifting), uploaded 10-03,

- JVET-AN0340 (a proposal on block-based activation of NNLF), uploaded 10-03,

- JVET-AN0342 (a proposal on NNPF activation SEI), uploaded 10-04,

- JVET-AN0357 (a proposal on modifying CMI SEI), uploaded 10-05,

- JVET-AN0365 (a proposal on multi-purpose SEI), uploaded 10-08,

- JVET-AN0366 (a proposal on purpose in DOI SEI), uploaded 10-08,

- JVET-AN0367 (a proposal on modifying PRI SEI), uploaded 10-08,

- JVET-AN0373 (a proposal on modified architecture of NNLF), uploaded 10-10.

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AN0223 (an investigation about bin to bit ratio), uploaded 10-03,

- JVET-AN0257 (an information about relation of CPU/GPU runtime), uploaded 10-04,

- JVET-AN0271 (an information about optimized ECM software implementation), uploaded 09-28,

- JVET-AN0272 (an information about ECM encodings in CfE), uploaded 09-28,

- JVET-AN0292 (an information on HEVC multiview software development), uploaded 09-28,

- JVET-AN0298 (a suggestion on conditions for multi-layer VVC testing), uploaded 10-04,

- JVET-AN0301 (comments on possible CfP procedures), uploaded 10-01,

- JVET-AN0303 (additional information on a CfE submission), uploaded 10-01,

- JVET-AN0317 (an analysis of IntraTMP complexity), uploaded 10-02,

- JVET-AN0318 (a review of CfE submissions), uploaded 10-02,

- JVET-AN0330 (comments on possible CfP test conditions), uploaded 10-03,

- JVET-AN0341 (a discussion on measuring quality in different colour spaces), uploaded 10-03,

- JVET-AN0348 (a discussion on measuring quality in case of RPR usage), uploaded 10-11,

- JVET-AN0360 (an assesment of CfE submissions), uploaded 10-06,

- JVET-AN0362 (an assesment of CfE submissions regarding RPR), uploaded 10-07.

All cross-verification reports at this meeting (except for JVET-AN0082, JVET-AN165, JVET-AN166, and JVET-AN243) were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AN0051, JVET-AN0159, JVET-AN0179, JVET-AN0268, JVET-AN0273, JVET-AN0285, JVET-AN0290, JVET-AN0320, JVET-AN0326, JVET-AN0355, JVET-AN0371, JVET-AN0372.

The following cross-verification report(s) were still missing by the end of the meeting, but were uploaded later: JVET-AN0XXX, … (sentence kept for future use). The following report(s) had not become available yet three weeks after the end of the meeting: JVET-AN0055, JVET-AN0359, JVET-AN0369. These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did not apply to any contributions. (did apply to document(s) JVET-AN0XXX, being initially empty of results and flagged as late in the list above, based on the time of the first reasonable document upload | sentence kept for future use).

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AM1000, the Future CICP extensions (draft 1) JVET-AM1005, the HEVC additional profiles and SEI messages (draft 4) JVET-AM1006, the Conformance testing for HEVC multiview extended and monochrome profiles JVET-AM1008, the Support for additional SEI messages in AVC (draft 3) JVET-AM1017, the HEVC with extensions and corrections (draft 2)  JVET-AM1018, the Additions and corrections for VVC version 4 (Draft 13) JVET-AM2005, the Additional SEI messages for VSEI version 4 (Draft 7) JVET-AM2006, the Description of algorithms version 12 and software version 14 in neural network-based video coding (NNVC) JVET-AM2019, the Verification test plan for VVC multilayer coding (update 6) JVET-AM2021, the Description of the EE on Neural Network-based Video Coding JVET-AM2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AM2024, the Algorithm description of Enhanced Compression Model 18 (ECM 18) JVET-AM2025, the Joint Call for Evidence on video compression with capability beyond VVC JVET-AM2026, the Technologies under consideration for future extensions of VSEI (version 9) JVET-AM2032, and the White paper on VSEI JVET-AM2038, had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible (kept for future use). The software implementations of VTM version 23.11, ECM version 18.0, and NNVC version 14.0 were also approved.

It was further noted that some editorial modifications were necessary on the WG 5 versions of standard deliverables, to be compliant with the ISO/IEC rules.

Only minor editorial issues were found in the meeting report JVET-AM1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annexes B1 and B2 of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiment on neural-network-based video coding (EE1)

Report of exploration experiment on enhanced compression beyond VVC capability (EE2)

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on software development in exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on gaming content compression

Consideration of contributions on generative face video compression

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on assessment of implementation complexity of video coding technology

Consideration of contributions submitted as responses to the Joint Call for Evidence on video compression with capability beyond VVC, assessment of results collected in the context of the CfE, and planning of next steps towards a possible Call for Proposals on future video coding standardization

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of the face-to-face meeting, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were expected to be 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were anticipated to be necessary. Sessions were announced in the online JVET calendar in advance as far as possible, but it was agreed that some activities (such as breakout sessions) could be held at short notice.

## ISO, IEC and ITU-T Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

Participants were also reminded of the UN Codes of Conduct, applying to work under ITU-T, found at https://www.un.org/management/sites/www.un.org.management/files/un-system-model-code-conduct.pdf

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1348 (as of 2 Oct. 2025). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – video coding standard developed by JVT, formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BIF: Bilateral filter

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardization

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted for study of coding technology

- CfE: Call for Evidence

- CfP: Call for Proposals

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CICP: Coding-independent code points (Rec. ITU-T H.273 | ISO/IEC 23091-2)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CNN: Convolutional neural network

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EE: Exploration experiment – a coordinated experiment conducted for study of coding technology prior during exploration work prior to establishment of a formal project (EEn for some number n is exploration experiment number n)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase and test model algorithm for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FGRC: Film grain regions characteristics

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- GS: Gaussian splatting

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a software codebase and video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for NNLF.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC 3D video extensions)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (originally joint video exploration team, initially for VVC, later expanded in scope)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- kMAC: thousands of multiply-accumulate operations – used as a complexity metric

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LB-CCP: Local-boosting CCP

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for NNLF.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MM-CCCM: Multi-model CCCM

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MOS: Mean opinion score (a metric for subjective quality evaluation)

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MSE: Mean-squared error, a distortion metric

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNLF: Neural network loop filter

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PSNR: Peak signal-to-noise ratio, a fidelity metric derived from MSE

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SAD: Sum of abolute differences, a measurement of distortion

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SCM: Screen content model – a software codebase and test model algorithm for the HEVC screen content coding extensions

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SGPM: Spatial geometric partition mode

- SH: Slice header

- SHM: Scalable HM – a software codebase and test model algorithm for SHVC

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TALF: Temporal adaptive loop filter

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMRL: Template-based multiple reference line prediction

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- TuC: Technology under consideration

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VLOP: Very low-complexity operating point for NNLF

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model – a software codebase and test model algorithm for the VVC standard

- VUI: Video usability information (part of an SPS, similar to an SEI message that applies to a video sequence)

- VVC: Versatile Video Coding, video coding standard developed by JVET, formalized by ITU-T as Rec. ITU-T H.266 and by ISO/IEC as ISO/IEC 29090-3

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29)

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in MPEG-2 and AVC:

  - MB: Macroblock – a 16x16 array for the luma component and corresponding 8x8 arrays for the chroma components (basically the same as a CTU in later standards, but with a fixed size)

  - MB pair: A pair of vertically contiguous macroblocks in a frame that is coupled for use in macroblock-adaptive frame/field decoding

  - Sub-MB: One quarter of the samples of a macroblock, i.e., an 8x8 luma block and two corresponding chroma blocks of which one corner is located at a corner of the macroblock for a picture that has three sample arrays or an 8x8 luma block of which one corner is located at a corner of the macroblock for a monochrome picture or a picture that is coded using three separate colour planes.

  - Sub-MB partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a sub-macroblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a sub-macroblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU)

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions)

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape

  - PB: Prediction block, a luma or chroma block of a PU

  - PU: Prediction unit, has the same size as a CU in the VVC context

  - TB: Transform block, a luma or chroma block of a TU

  - TU: Transform unit, has the same size as a CU in the VVC context

## Standards, TRs, supplements and technical papers approval and publication status

- MPEG-2 | H.262 (the video coding specification is common text and not publicly available without cost per ISO special request)

  - ITU-T H.262 V3 was approved in 2012-02; its Amd 1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012 (publication availability unclear, as these do not appear to be directly available from the ISO website) may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) final ballot closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13 (ITU work programme link)

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15

    - ITU-T H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

    - ISO/IEC 14496-10:2025 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; published 2025-07-10

  - Amendment to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, to close 2025-12-31, no action at current meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004 (Ed. 2), including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V6 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (with shutter interval information SEI message and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - ITU-T H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - Amendment to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, ready for action at current meeting (ISO Projects link)

  - ITU-T H.265 (V11) to forward for consent at current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V3 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

    - H.266 (2023) Erratum 1 published 2025-05 to correct errors in a few figures

  - ISO/IEC 23090-3:2024/DAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, ready for action at the current meeting (ISO Projects link)

  - ITU-T H.266 (V4) to forward for consent at the current meeting (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, to close 2025-12-22, no action in ISO/IEC at current meeting (ISO Projects link)

    - ITU-T H.266.1 (V3) to forward for consent at current meeting (ITU work prgramme link)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - ITU-T H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:202x (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, closed 2025-06-01, approved to proceed to publication without an FDIS at 29th meeting 2025-07, public availability requested 2025-07. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12

  - ISO/IEC 23002-7:202x (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, ready for action at current meeting (ISO Projects link)

  - ITU-T H.274 (V4) to forward for consent at the current meeting (ITU work programme link)

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - ITU-T H-Series Supplement H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - ITU-T H-Series Supplement H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08

- Usage of video signal type code points (twin text)

  - ITU-T H-Series Supplement H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - ITU-T H-Series Supplement HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement H.Sup21 V1 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication 2025-03-26, published 2025-05-21

  - ISO/IEC TR 23002-9 (Ed. 2) Edition 2 planned but not yet in formal work programme of ISO/IEC

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC TR 23888-3 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, to close 2025-11-03, no action at current meeting (ISO Projects link)

  - ITU-T provisional name H.Sup.MACVC (ITU work programme link)

- Freely available ISO/IEC standards (without cost) in the ISO Webstore and IEC Webstore include

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2 (confirmed available 2025-10-31)

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed available 2025-10-31)

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2 (confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-4:2004 specify conformance for AVC (base text confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) specify reference software for AVC (base text confirmed available 2025-10-31)

  - ISO/IEC 23090-16:2022 (Ed. 1) Reference software for VVC (soon to be withdrawn)

- Intended to be freely available (without cost) but unavailable (without cost) standards and technical reports include the following. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 14496-10:2025 (Ed. 11) AVC, published 2025-07-10 (public availability request to be confirmed)

  - ISO/IEC 23002-7:2024 (Ed. 3) VSEI, published 2024-10-30 (public availability requested 2023-07)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2025-03 (public availability requested 2024-04)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02, to be reiterated at the current meeting)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (public availability not requested 2017-01, 2017-04, 2017-07, 2017-10, 2018-01, to be requested at the current meeting)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10, to be reiterated at the current meeting)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15 (public availability to be requested at the current meeting)

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04

  - ISO/IEC 23090-16:202x (Ed. 2) Reference software for VVC, not yet published, public availability requested 2025-07

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25 (public availability requested 2024-04, to be reiterated at the current meeting)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06, public availability requested 2025-07

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08, public availability requested 2025-07

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23, public availability requested 2025-07

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20, public availability requested 2025-07

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24 (not requested 2024-04), public availability requested 2025-07

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that public availability for some was never requested, e.g. for those that were produced by JCT-3V.

## Draft standards progression status for active work items

- AVC ISO/IEC 14496-10:2025/DAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, to close 2025-12-31, no action at current meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting (ITU work programme link)

- HEVC ISO/IEC 23008-2:2025/DAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, ready for action at current meeting (ISO Projects link)

  - ITU-T H.265 (V11) to forward for consent at current meeting (ITU work programme link)

- VVC ISO/IEC 23090-3:2024/DAM 1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, ready for action at the current meeting (ISO Projects link)

  - ITU-T H.266 (V4) to forward for consent at the current meeting (ITU work programme link)

- VVC conformance ISO/IEC DIS 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, pending closure 2025-06-30 during the current meeting, DIS issued at 39th meeting 2025-07, to close 2025-12-22, no action in ISO/IEC at current meeting (ISO Projects link)

  - ITU-T H.266.1 (V3) to forward for consent at current meeting (ITU work programme link)

- VSEI ISO/IEC 23002-7:2024 (Ed. 3)/DAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, ready for action at current meeting (ISO Projects link)

  - ITU-T H.274 (V4) to forward for consent at the current meeting (ITU work programme link)

- ISO/IEC TR 23002-9 (Ed. 2) Film grain synthesis technology for video applications – Edition 2 planned but not yet in formal work programme of ISO/IEC

- H.Sup.MACVC | ISO/IEC TR 23888-3 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC TR 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, to close 2025-11-03, no action at current meeting (ISO Projects link)

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Friday 3 October at 1430-1510 CEST were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was higher than for last meeting (approximately 230 vs. 200 by the time of opening the meeting) – parallel sessions were announced to be necessary.

  - Start of parallel sessions (HLS and EE review) Saturday morning

  - Expert viewing of CfE submissions to be conducted Saturday, Sunday and Monday

  - JVET will not meet during the WP3/21 plenary on Monday 6 Oct. 1430-1545 CEST. JVET will meet during the MPEG information exchange session on Tuesday 7 Oct. (0900-1200 CEST), and may also meet during the session on Thursday 9 Oct. (0900-1000 CEST), both of them held in CICG. It is planned to approve WG 5 recommendations before the MPEG information exchange on Saturday 11 Oct. (1400-1600 CEST, in ITU/Popov) where those should be presented. JVET will however continue working until Sunday 12 Oct. evening (depending on workload).

  - SG21 workshop on Friday 1400-1830 “Embodied AI and Media Technology Standards” (https://www.itu.int/en/ITU-T/Workshops-and-Seminars/2025/1010/Pages/default.aspx), to be held in room C  JVET can only meet in smaller rooms (e.g., breakouts) during that time.

- Future non-virtual meetings will be held as hybrid meetings with best-effort remote access (when under ISO/IEC SC 29 auspices) or as physical meetings with remote participation (when under ITU-T SG21 auspices). In terms of practical arrangements, this is identical.

- Plans for subsequent non-virtual meetings were reviewed: April 2026 (Santa Eulària), July 2026 (Geneva), October 2026 (Hangzhou), January 2027 (Brisbane), April 2027 (likely Geneva), July 2027 (Tampere), October 2027 (Shenzhen), January 2028 (likely Geneva).

- The January 2026 meeting remains planned to be virtual.

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, or plan for AHG meeting days prior to the regular meeting (e.g., for analysis of CfE/CfP submissions)

- Significant workload was expected at this meeting for AHG17 activities – to evaluate responses to the the Call for Evidence and plan subsequent steps. Before starting discussion, the viewing of CfE submissions needs to be conducted (Sat.-Mon.). Up to 336 volunteers are needed for viewing sessions (but experts can participate in multiple sessions). Review of EE1/EE2 needs to be done in parallel. Joint meetings with parent bodies are also expected next week on this topic.

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing. In the meeting room C, it is also possible to show them on a separate projection screen in parallel with another presentation.

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AM1000 were reviewed (which was finalized during early September, but sufficiently mature drafts had been available before in the ITU ftp site). Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, on-site attendance will again be recorded via the traditional sign-in sheet, in addition to the list of web registrations made via ITU. Participants were asked to correct their affiliation and email in cases where these changed (also in the ITU account). Remote attendance (of those not present in person) will be recorded via the zoom reports. It is therefore important to follow the conventions of naming as mentioned before. Participants who cannot be correctly identified will not appear in the attendance sheet.

- There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a few documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions. As for now, the JVET chair took action of correcting those cases in the document registry.

- Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

- Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs (as that feature of the DMS site has not been working properly).

- Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- Still, only the newest versions of the JCT-VC and JCT-3V documents are available from the links in the JVET site, but a second source for JVET, JCT-VC and JCT-3V documents was set up in the ITU ftp directories of each meeting. This includes all versions of documents with original upload times. JVET documents can only be made available with a delay of at least one meeting cycle, as often newer versions (in particular of output documents) are still uploaded later

- The following ballot results had become available through the SC 29 secretariat:

  - Summary of Voting on ISO/IEC 23008-2:2025/DAmd 1 (Ed 6) m74713

  - Summary of Voting on ISO/IEC 23002-7:2024/DAmd.1 (Ed. 3) m74822

  - Summary of Voting on ISO IEC 23090-3:2024/DAmd.1 (Ed.3) m74824

- It was planned to issue FDIS of new editions for HEVC, VVC and VSEI at this meeting, and also submit them for ITU-T consent on the next versions of H.265, H.266 and H.274.

- It was originally planned to submit the supplement on machine analysis for ITU-T approval. As ballot comments will only arrive in November, which might make it necessary second DTR would need to be issued, the ITU version should be deferred until July 2026.

- The AVC amendment (support for additional VSEI messages) is currently under DAM ballot which will close before the January 2026 meeting. An FDIS on the next edition could be planned for January or April, and the next H.264 version be submitted in April 2026.

- The 3rd edition of VVC conformance is currently under DIS ballot. Submission to consent on next version of H.266.1 should be done at the current meeting.

- Plans for generating the next versions of reference software for AVC, HEVC and VVC (which would need to support new SEI messages from VSEIv4) need to be developed. Earliest target date for ITU-T consent would be April 2026 – should requests be made for ISO? For AVC, it might also be useful to generate a new part of ISO/IEC 14496.

- The primary goals of the meeting were:

  - Generate next versions/editions of VSEI, VVC and HEVC

  - VVC conformance for ITU consent

  - Evaluation of CfE results, and further steps

  - TuC on VSEI and CICP

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

  - Liaison communication – if any

  - VVC white paper update (remove VSEI, some update of the 2021 version)

- Joint meetings were expected with MPEG AG 5 (on matters involving visual quality assessment), and with MPEG WG 2 Requirements and ITU-T VCEG on future video standardization (see section 7.3). Another potential joint meeting may be necessary on Gaussian splatting (SEI proposal) with WGs 4 and 7.

- As a follow-up to communication after previous meetings, parent bodies need to conduct further discussion about future JVET management structures.

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.12.

## Scheduling of discussions

- The times of the meeting sessions followed the needs of the onsite meeting arrangements, with highest priority given to the aim of achieving the goals of the meeting. Typical meeting hours were in the range of 0900-2000 CEST with coffee breaks and lunch breaks as appropriate, however some early morning or late-night sessions were noted to potentially be necessary. Sessions were announced in the JVET calendar as far as possible in advance, although it was acknowledged that some activities (such as breakout sessions) might be held at short notice.

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete. Times are recorded in the local timezone of the meeting venue, except as otherwise noted:

Coffee breaks were regularly scheduled at 1030 and 1630.

- Fri. 3 Oct., 1st day

  - Afternoon session:

    - 1400–1510 Opening remarks, review of practices, agenda, IPR policy reminder

    - 1510–1850 Reports of AHGs 1-18

- Sat. 4 Oct., 2nd day

  - Sessions with coffee/lunch break as appropriate:

    - 0900–1930 Experts viewing of CfE submissions (compression performance category)

    - 0900–2000 Review all EE1 5.1.x categories, 4.1, 4.2, 4.3 (Popov room)

    - 0900–1930 HLS track:6.1, 6.2 (Rm. A, chaired by Jill Boyce)

- Sun. 5 Oct., 3rd day

  - Sessions with coffee/lunch break as appropriate:

    - 0900–1930 Experts viewing of CfE submissions (encoding runtime category)

    - 0900–1930 Review EE2 5.2.1, 5.2.3, 5.2.4.1 categories (Popov room)

    - 0900–1930 HLS track: 6.2, 6.3 (Rm. A, chaired by Jill Boyce)

- Mon. 6 Oct., 4th day

  - 0900–1830 Experts viewing of CfE submissions (encoding runtime category and functionality category)

  - Morning sessions:

    - 0930–1400 Review 5.2.4 remaining (Room C)

    - 0900–1300 HLS track: 6.5, 6.2 revisits, 6.3 (CICG 14, chaired by Jill Boyce)

  - Afternoon sessions (no session during WP3/21 plenary):

    - 1600–1700 Review 4.4  (Room C)

    - 1600–2000 HLS track: Remaining 6.x (CICG 14, chaired by Jill Boyce)

    - 1800–2000 Review remaining 5.2.4, 5.2.5 (Room C, chaired by Y. Ye)

- Tue. 7 Oct., 5th day

  - 0900–1230 MPEG information sharing session

  - Morning sessions:

    - 0900–1300 Review 4.16.1 CfE contributions (Room C)

  - Afternoon sessions:

    - 1415–2100 CfE viewing results, 4.16.1/4.16.2 continued (Room C)

    - 1800–2100 HLS:  TBD

- Wed. 8 Oct., 6th day

  - Morning sessions:

    - 0830–1055 Joint with VCEG, WG 2 and AG 5: CfE results and next steps (Room C)

    - 1115–1245 JVET plenary: Coordination, further planning, open issues from tracks

  - Afternoon sessions:

    - 1400–1700 4.17 ULD (incl. results of CfE viewing)

    - 1700–2000 BoG on complexity to review 4.8 Tool assessment, discuss potential extension w.r.t. encoder complexity, and discuss structure/content of an output document  (X. Li, Y. Zhao)

    - 1400–2000 HLS: TBD

- Thu. 9 Oct., 7th day

  - 0900–1000 MPEG information sharing session

  - Morning sessions:

    - 1015–1300 Report BoG on complexity, remaining 4.17

    - 0900–1400 HLS: TBD

  - Afternoon sessions:

    - 1400–2000 4.15, 4.11, 4.14, 4.6, 4.16.3 and discuss next steps necessary for CfP preparation

    - 1400–2000 HLS: TBD

- Fri. 10 Oct., 8th day

  - Morning sessions:

    - 0900–1300 Planning of output docs and AHGs, review 4.5, 6.6, 4.18, EE Revisits

    - 1400–2000 HLS: TBD

  - Afternoon sessions:

    - No JVET main session, 1400-1830 ITU workshop in room C

    - 1400–2000 HLS: TBD

- Sat. 11 Oct., 9th day

  - 0900–1310 JVET plenary (room C):

    - Establishment of AHGs

    - Review of WG 5 meeting recommendations

    - Review of DoCRs HEVC, VVC, VSEI

    - Initial report HLS adoptions

    - Liaison planning

    - Schedule planning

    - Review 4.13

  - 1400–1550 MPEG information sharing session (and closing sessions of other MPEG WGs thereafter)

  - 1700-XXXX HLS (Room A), revisits, DoC review from 1800

- Sun. 10 Oct., 10th day

  - 0930–XXXX JVET plenary:

    - 0930-1125CfP preparation: Report/discussion about viewing of new sequences, edits on top of CfE doc, initial version of spreadsheet

    - 1125-1235 Remaining doc review (4.5, 5.1.3, 6.6 et al.)

    - 1235-1300 Remaining revisits

    - 1300-1340 Liaison review

    - 1340-1420 Final approval of output docs (incl. EE review), and delivery dates

    - 1420-1435 Updates of WG 5 recommendations

    - Software timeline

    - Future planning, a.o.b.

    - Closing of meeting

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (18) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (1)

  - AHG2: Text development and errata reporting (1)

  - AHG3: Software development (1)

  - AHG3: Test conditions and metrics (2)

  - AHG4: Subjective quality testing and verification testing (1)

  - AHG4: Test and training material (2)

  - AHG5: Conformance test development (0)

  - AHG7: ECM tool assessment (7)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

  - AHG10: Encoding algorithm optimization (0)

  - AHG13: Film grain technology (1)

  - Implementation studies (0)

  - Profile/tier/level specification (1)

  - AHG15: Gaming content compression (2)

  - AHG16: Generative face video (1)

  - AHG17: CfE on video coding technology (20)

  - AHG18: Ultra-low latency and error resilience (8)

  - CICP (1)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (27) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (69) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - Aspects of SEI messages in VSEI, VVC, HEVC and AVC (4) (section 6.1)

  - Aspects of SEI messages in VSEI v4 (33) (section 6.2)

  - SEI messages in TuC for VSEI (55) (section 6.3)

  - SEI messages on other topics (7) (section 6.4)

  - SEI software and showcases (1) (section 6.5)

  - Non-SEI HLS aspects (2) (section 6.6)

- Joint meetings, plenary discussions, BoG reports (1) liaison (1), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (18)

These reports were discussed during 1510–1850 on Friday 3 Oct. 2025 (chaired by JRO).

JVET-AN0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice chair)]

The reflector used for discussions by the JVET and all of its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

The number of subscribers (on the day before the beginning of the current meeting) was 1348 (compared to 1315 by the time of the previous meeting). Furthermore, the previous lists of joint teams (which were still kept open as archives) had the following number of subscribers:

- JCT-VC – 1158 subscribers

- JCT-3V – 680 subscribers

- JVT-experts – 2077 subscribers

It is likely that many subscriptions of these latter three reflectors would have become obsolete, as no emails are being sent over them any more, and therefore automatic removal of addresses causing permanent bounces by being invalid (as it happens with the main JVET reflector) cannot be executed by the reflector management system.

  - Goals and activity

The work of the JVET overall had proceeded well in the interim period with increased number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET ftp site (http://wftp3.itu.int/av-arch/jvet-site/2025_06_AM_Daejeon/). It is noted that the previous document sites http://phenix.int-evry.fr/jvet/, http://phenix.int-evry.fr/jct/, and http://phenix.int-evry.fr/jct3v/ were shut down, but JCT-VC and JCT-3V documents can be accessed directly via the JVET site. All documents of JCT-VC, JCT-3V and JVET are also available from the ITU-based ftp site, where sub-folders ‘./documents/’ were created in the folders of the respective meetings (it is noted that, for practicality reasons, for JVET this is only done after all meeting documents including output documents are completely available in all versions). All those files come with the original time stamps by which the original versions of documents were uploaded.

The list of output documents produced since the last meeting included the following:

- JVET-AM1005 Future CICP extensions (Draft 1) [Posted 2025-08-01]

- JVET-AM1006 HEVC additional profiles and SEI messages (Draft 4) [Posted 2025-07-23]

- JVET-AM1008 Conformance testing for HEVC multiview extended and monochrome profiles [Posted 2025-07-25]

- JVET-AM1017 Support for additional VSEI messages in AVC (Draft 3), also issued as WG 5 DAM N 363 [Posted 2025-08-01, last update 2025-08-08]

- JVET-AM1018 HEVC with extensions and corrections (Draft 2) [Posted 2025-07-23]

- JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [Posted 2025-10-03]

- JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7) [Posted 2025-07-23]

- JVET-AM2019 Description of algorithms version 12 and software version 14 in neural network-based video coding (NNVC) [Posted 2025-07-28, last update 2025-08-02]

- JVET-AM2021 Verification test plan for VVC multilayer coding (update 6) [Posted 2025-09-15]

- JVET-AM2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2025-07-03, last update 2025-08-04]

- JVET-AM2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [Posted 2025-07-03, last update 2025-08-08]

- JVET-AM2025 Algorithm description of Enhanced Compression Model 18 (ECM 18) [Posted 2025-09-24]

- JVET-AM2026 Joint Call for Evidence on video compression with capability beyond VVC, also issued as WG 5 N 367 [Posted 2025-07-05, last update 2025-08-09]

- JVET-AL2032 Technologies under consideration for future extensions of VSEI (version 9) [Posted 2025-08-27, last update 2025-09-08]

- JVET-AL2038 White paper on VSEI, also issued as AG 3 N 209 [Posted 2025-07-25]

It is noted that the following output document from the 38th JVET meeting also became available:

- JVET-AL2020 Film grain synthesis technology for video applications ed. 2 (Draft 3)

The eighteen ad hoc groups had made progress, and reports from those activities had been submitted. Numerous teleconferences had been held by AHGs. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability were conducted, and 5 submissions were received as responses to the Joint Call for Evidence on video compression with capability beyond VVC.

The arrangements for the 40th meeting had been announced in the JVET reflector, in the JVET logistics document (https://www.itu.int/wftp3/av-arch/jvet-site/2025_10_AN_Geneva/JVET-AN_Logistics.docx), and in the WG 5 calling notice (N 369) and agenda (N 370) for the 21st WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

More than 230 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, maintenance, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC and JCT-3V topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 40th meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2025_10_AN_Geneva/JVET-AN_notes_d0.docx.

Further discussion needs to be conducted with the parent bodies about future JVET management structures, considering that substantially higher workload could be expected in upcoming standardization activities.

Recommendations

- The AHG recommended its continuation.

JVET-AN0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

  - Output documents produced

    - JVET-AM1006 HEVC additional profiles and SEI messages (draft 4) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis]

This document contains the draft text for changes to the High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 | ISO/IEC 23008-2). The changes include the support of a new profile, namely the Multiview Main 10 profile, and the support of 12 new SEI messages, namely the SEI processing order, processing order nesting, encoder optimization information, source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, and digitally signed content verification, generative face video, generative face video enhancement, AI usage restrictions, and packed regions information SEI messages, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7). The changes also include some extensions and corrections to the previous version of the HEVC standard.

Changes that have been integrated for the 39th JVET meeting in June-July 2025:

- JVET-AM0048 AHG9/AHG2: Miscellaneous changes for HEVC

- JVET-AM0121 AHG9: On the SEI processing order SEI message

- JVET-AM0324 Correction to the Alpha Channel Information SEI message processing order

- Changed all instances of "BtDepth" to "BitDepthVal".

- Changed "generative face video enhancement" to "generative face enhancement", to avoid one SEI message name etc. being a subset of another SEI message name etc.

- JVET-AM0118 AHG9: Digital signing of selected SEI messages

- JVET-AM0166 AHG9: Miscellaneous aspects in VSEI v4. Item 4 – Constraint for PON SEI message on prefix and suffix SEI messages.

    - JVET-AM1017 Support for additional SEI messages in AVC (draft 3) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang]

This document contains the draft text for changes to the Advanced Video Coding (AVC) standard (Rec. ITU-T H.264 | ISO/IEC 14496-10). The changes include the support of five new SEI messages, namely the source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7).

Changes that have been integrated for the 39th JVET meeting in June 2025:

- DoCR US-003: Update SEI message lists in subclauses F.13.2, G.13.2, H.13.2

- Changed all instances of "BtDepth" to "BitDepthVal".

- Changed "generative face video enhancement" to "generative face enhancement", to avoid one SEI message name etc. being a subset of another SEI message name etc.

- Fixes for formatting and ISO/IEC terminology

    - JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang]

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3). The changes include the support of 15 new SEI messages, namely 1) SEI processing order, 2) processing order nesting, 3) encoder optimization information, 4) source picture timing information, 5) object mask information, 6) modality information, 7) text description information, 8) digitally signed content initialization, 9) digitally signed content selection, 10) digitally signed content verification, 11) generative face video, 12) generative face video enhancement, 13) AI usage restrictions, 14) packed regions information, and 15) image format metadata, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7). The changes also include updates to the interface text for support of the two neural-network post-filter (NNPF) SEI messages as well as some corrections to the previous version of the VVC standard.

Changes that have been integrated for the 39th JVET meeting in June-July 2025:

- JVET-AM0118 AHG9: Digital signing of selected SEI messages

- JVET-AM0121 AHG9: On the SEI processing order SEI message

- Changed all instances of "BtDepth" to "BitDepthVal".

- Changed "generative face video enhancement" to "generative face enhancement", to avoid one SEI message name etc. being a subset of another SEI message name etc.

- JVET-AM0174 AHG9: On general SEI payload constraints.

JVET-AM0166 AHG9: Miscellaneous aspects in VSEI v4. Item 4 – Constraint for PON SEI message on prefix and suffix SEI messages.

    - JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang]

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify 15 additional SEI messages, namely 1) SEI processing order, 2) processing order nesting, 3) encoder optimization information, 4) source picture timing information, 5) object mask information, 6) modality information, 7) text description information, 8) digitally signed content initialization, 9) digitally signed content selection, 10) digitally signed content verification, 11) generative face video, 12) generative face enhancement, 13) AI usage restrictions, 14) packed regions information, and 15) image format metadata. The changes also include updates to some of the existing SEI messages, including the two neural-network post-filter (NNPF) SEI messages, as well as some corrections to the previous version of the VSEI standard.

Editors’ notes and changes yet to be integrated:

- All automatically generated numbering and cross-reference fields will need to be updated after the changes are integrated into the basis text. Check the use of “pertain”; the word is not used in a similar way anywhere in the existing standard.

- payloadType values are to be specified the corresponding VVC, HEVC or AVC draft text. The use of prefix SEI NAL unit was enabled for all new SEI messages, whereas the use of suffix SEI NAL unit was enabled for encoder optimization information and object mask information SEI messages, since their content may be determined while the encoder encodes the associated picture.

Changes that have been integrated:

A description of the contributions reflected in this document relative to JVET-AL2006:

Multiple SEI messages

- JVET-AM0047 AHG9: Miscellaneous changes for VSEI

- Changed all instances of "BtDepth" to "BitDepthVal".

- JVET-AM0052 AHG9: Editorial changes for VSEI

- JVET-AM0165 AHG9: On value range and reserved values for syntax elements in VSEI v4

- JVET-AM0166 AHG9: Miscellaneous aspects in VSEI v4 . Item 4 needs to be implemented in the VVC and HEVC interface spec draft.

- JVET-AM0189 AHG9: On semantics of persistence flag of SEI messages in VSEI v4 and v3

- JVET-AM0191 AHG9: Editorial updates for VSEI v4

SEI processing order (SPO) and processing order nesting (PON)

- JVET-AM0121 AHG9: On the SEI processing order SEI message

- JVET-AM0292 AHG9/AHG2: On the semantics of po_num_bits_in_prefix_indication_minus1[ i ] and po_sei_prefix_data_bit[ i ][ j ]

- JVET-AM0321 AHG9: On SEI message types indicated by an SPO SEI message

- JVET-AM0081 AHG9: On the PON dependency constraint

- JVET-AM0082 AHG9: On the PON nested FGC SEI message

AI usage restrictions (AUR)

- JVET-AM0117 AHG9: On the AI usage restrictions SEI message

- JVET-AM0112 AHG9: On AI usage restrictions SEI message

- JVET-AM0152 AHG9: AI usage restrictions for entities other than decoded pictures

Digitally signed content (DSC)

- JVET-AM0118 AHG9: Digital signing of selected SEI messages

- JVET-AM0119 AHG9: On start and end flags of digitally signed

- JVET-AM0164 AHG9: On DSC SEI

- JVET-AM0190 AHG9: On implicit association mode in digitally-signed content SEI messages in VSEI v4

- JVET-AM0210 AHG9: On miscellaneous aspects in digitally-signed content SEI messages in VSEI v4 draft

Generative face video (GFV) and generative face enhancement (GFE)

- JVET-AM0334 AHG9: Common specification text for GFV SEI message

- Changed "generative face video enhancement" to "generative face enhancement", and "gfve" to "gfe", to avoid one SEI message name etc. being a subset of another SEI message name etc.

Packed regions info (PRI)

- JVET-AM0211 AHG9: On the packed regions information SEI message

- JVET-AM0075 AHG9: On packed regions information SEI message

- Related input contributions

Input contributions noted as relevant to the work of this ad hoc group:

- (none)

- Remaining VVC spec tickets

Closed since JVET-AM0002 was reported:

- (none)

Carried over (strikethough means addressed in the indicated errata report and can be closed):

- #1609 NoBackwardPredFlag derivation ambiguity (Addressed in JVET-AI1004)

- #1617 Not initialized NumCtusInSlice[0] to 0. (Addressed in JVET-AI1004)

- #1624 Incorrect indexing in computation of motion vector offset. (Addressed in JVET-AI1004)

- #1627 "Decoding process for palette mode" does not say what to do with output samples. (Discussed in JVET-AI1004)

- #1628 Derivation of ModeTypeCondition should say "one or more". (Addressed in JVET-AI1004)

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType. (Discussed in JVET-AI1004)

- #1630 Missing equations for applying AmvrShift. (Discussed in JVET-AI1004)

- #1631 Should "Motion vector storing process for geometric partitioning mode" store HpelIfIdx? (addressed in JVET-AI1004)

- #1632 Incorrect indexing used for choosing matrix intra sample prediction. (Addressed in JVET-AI1004)

- #1634 Matrices QStateTransTable, levelScale, AlfFixFiltCoeff, AlfClassToFiltMap are incorrectly transposed. (Addressed in JVET-AI1004)

- #1635 Incorrect inference for tu_y_coded_flag. (Discussed in JVET-AI1004)

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

- #1650 Incorrect indexing for the h location component in ALF process. (Discussed in JVET-AJ1004)

- #1651 References to various non-existent syntax elements relating to sps_partition_constraints_override_enabled_flag.

- #1652 Incorrect heading style in JVET-S2001-v17.

New (since JVET-AM0002 was reported)

- (none)

- Remaining HEVC spec tickets

Closed since JVET-AM0002 was reported:

- (none)

Carried over:

- #1427 (8-155) and (8-157) do not seem to be used

- #1491 Duplicate invocation of 9.3.4.3 arithmetic decoding process

- #1498 Typos in the Table 9-43

- #1500 Typo in equation (8-69),(8-70)

- #1504 Small typos in profile_tier_level syntax in tabular form (7.3.3)

- #1505 Misleading bitstream requirement related to EOB NAL unit

- #1507 Duplicate row entries for CU QP delta syntax elements in Table 9-48

- #1520 Some smaller errors in the multiview spec

- #1522 Offset issue in clause 8.5.4.3

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Filed as VVC ticket but applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

New (since JVET-AM0002 was reported):

- (none)

Recommendations

The AHG recommended to:

- Approve JVET-AM1004, JVET-AM1017, JVET-AL2005, and JVET-AM2006 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2-related inputs and act on them if found to be necessary.

It is noted that currently, no need is seen for an update of bug collection doc JVET-AL1004.

JVET-AN0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

The software model versions prior to the start of the meeting were:

- VTM 23.11 (July 2025)

- VTM-22.2-TuC4.0 (Feb. 2025)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.24 (March 2023)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

- Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

Between the 31st and 32nd meetings, the GitLab server was affected by a huge number of spam issues, which were generated using newly registered accounts. Administrator user account confirmation was enabled to protect the server from spam postings. Proponents should plan for some time for server admins to confirm new accounts.

Between the 37th and 38th JVET meeting extensive renewal work was performed on the build server infrastructure. To finish these works, changes to the build configurations are required for all software projects using build pipelines. The necessary information will be communicated to the software coordinators when available.

- VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued on the GitLab server. VTM version 23.11 was tagged on July. 4, 2025. VTM 23.12 is expected during the 40th JVET meeting. VTM 23.11 was not followed up with a macro cleanup version. Macros will be cleaned up after VTM 23.12.

VTM 23.11 was tagged on July 4, 2025. Changes include:

- JVET-AJ0183: Implementation of syntax element eoi_src_pic_flag in Encoder Optimization Information SEI message

- JVET-AK0287: Signal flags in DSCI SEI to indicate substream dependency

- JVET-AL0123/AL0310: Signalling changes to the EOI SEI message

- JVET-AK0140: Packed Regions Info SEI

- JVET-AL0301: Specify constraints on syntax elements related to matrix_coeffs

- JVET-AJ0060: multilayer adaptation of Packed regions information SEI message

- Introduce cfg parameter to remove dependency on GOP size when determining `pps_init_qp_minus26`

- JVET-AL0324, AL0070: updates of PRI SEI message

- JVET-AL0339: Spatial Resolution for the FGC SEI message

- JVET-AL0075: Add nnpfa_selected_input_flag to NNPFA SEI

- JVET-AK0326 and JVET-AI0070: Updated NNPF SEI messages

- JVET-AL0117: On association of NAL units to DSC verification substreams

- Faster temporal prefilter

- Faster CRC32C computation

- JVET-AL0056: Constraint for quant threshold delta in EOI SEI message

- JVET-AL0067: OMI SEI constraints

- Improved implementation of hash search

- Fix incorrect constant in ISP search

- WG03 N01464: Implementation of attenuation maps as new green metadata type

- JVET-AK0114: Implementation of AI-Usage Restrictions SEI message

- JVET-AJ0105: Complexity Info in SPO SEI message

- JVET-AM0280: Decoder memory printout

- Update for high performance cfg files

- add self-contained cfg for CfE

- JVET-AM0200: reduced run time configurations

- Update version number to 23.11

VTM 23.12 was expected to be tagged during the 40th JVET meeting. Changes were expected to include:

- Fix memory leak when RPR is used

- Fix a link error of parcat when JVET_J0090_MEMORY_BANDWITH_MEASURE is on

- Initialize pointer to VPS of a pilot Slice object

- Correctly set input chroma format when reading a y4m file

- Avoid recomputing LMCS parameters when using debugBitstream by copying APS info (same as AFL APSs)

- Fix trace output of coded block flag (CBF)

- JVET-AM0227: Fix CABAC statistics collection

- JVET-AJ0258: Image Format Metadata SEI

- New enum for splits - removes magic numbers from CtxSplit

- Increase minimum required CMake version

- DSC: fix detection of last picture

- JVET-AK0206: Digitally signed content ID

- remove dead code in EncModeCtrl.h

- DSC: Align DSCI syntax with spec

- JVET-AL0222: Digitally signed content intended start and end flags

- JVET-AL0066: Add sample tolerance value to OMI SEI

- JVET-AM0164: move dsci_key_source_uri and increase length of dscv_signature_length_in_octets_minus1

- JVET-AL0058: Change aur_context to fixed length

- JVET-AM0334: Update to GFV SEI chroma key syntax

- JVET-AK0281: AUR SEI message in SPO SEI message

- JVET-AM0117: aur_sei_exclusion_flag

  - CTC Performance

The VTM 23.11 shows some improvements compared to VTM 23.9/23.10 due to MR2840, which enables DeblockingFilterOffsetInPPS, and faster code with reduced memory usage due to improvements in temporal prefilter and hash ME.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 90% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class TGM | 0.00% | 0.00% | 0.00% | 98% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-23.9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Class A2 | -0.01% | -0.01% | -0.01% | 98% | 103% |
| Class B | -0.01% | -0.01% | -0.01% | 97% | 99% |
| Class C | -0.02% | -0.02% | -0.02% | 96% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 97% | 99% |
| Class D | -0.06% | -0.07% | -0.07% | 97% | 95% |
| Class F | -0.03% | -0.03% | -0.03% | 96% | 95% |
| Class TGM | -0.01% | -0.01% | -0.01% | 99% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-23.9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 98% | 99% |
| Class C | -0.02% | -0.02% | -0.02% | 98% | 100% |
| Class E | -0.11% | -0.11% | -0.10% | 95% | 104% |
| Overall | -0.04% | -0.04% | -0.04% | 97% | 100% |
| Class D | -0.06% | -0.05% | -0.05% | 97% | 100% |
| Class F | -0.04% | -0.04% | -0.04% | 96% | 101% |
| Class TGM | -0.01% | -0.01% | -0.01% | 97% | 98% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-23.9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 96% | 97% |
| Class C | -0.01% | -0.01% | -0.01% | 98% | 103% |
| Class E | -0.09% | -0.09% | -0.08% | 94% | 101% |
| Overall | -0.03% | -0.03% | -0.03% | 96% | 100% |
| Class D | -0.04% | -0.03% | -0.03% | 97% | 95% |
| Class F | -0.02% | -0.03% | -0.03% | 96% | 98% |
| Class TGM | -0.01% | -0.01% | -0.01% | 95% | 94% |

For the HDR CTCs, coding performance of VTM 23.11 compared to VTM 23.10 are reported in the table below. Slight variations are observed in RA. Encoding run time seems to be slightly reduced. Decoding run time is almost not changed.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM23.10 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.03% | -0.03% | -0.03% | -0.03% | -0.03% | -0.03% | -0.03% | -0.03% | 97% | 97% |
| Class H2 |  |  |  |  |  | -0.01% | -0.01% | -0.01% | 96% | 101% |
| Overall | -0.03% | -0.03% | -0.03% | -0.03% | -0.03% | -0.02% | -0.02% | -0.02% | 96% | 99% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM23.10 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class H2 |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 101% |

Coding performance of VTM 23.11 compared to HM 18.0 are reported in the table below.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -39.49% | -37.66% | -36.75% | -54.96% | -48.52% | -33.76% | -49.45% | -40.94% | 255% | 79% |
| Class H2 |  |  |  |  |  | -31.99% | -57.73% | -63.35% | 220% | 79% |
| Overall | -39.49% | -37.66% | -36.75% | -54.96% | -48.52% | -33.12% | -52.46% | -49.09% | 241% | 79% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.44% | -27.31% | -26.79% | -57.73% | -52.77% | -24.00% | -52.92% | -45.07% | 1511% | 103% |
| Class H2 |  |  |  |  |  | -21.76% | -47.23% | -50.60% | 1225% | 96% |
| Overall | -41.44% | -27.31% | -26.79% | -57.73% | -52.77% | -23.18% | -50.85% | -47.08% | 1400% | 100% |

  - Issues in VTM affecting conformance

The following issues in the VTM master branch may affect conformance:

- Missing HLS features (see sections below)

Several merge requests for proposals from the 36th or 37th JVET meeting, mostly related to SEI messages in VSEI are still pending review and/or waiting for resolution of comments and rebasing.

  - Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

  - SEI TuC software

Per the decision during the 32nd JVET meeting, an SEI TuC repository was created based on VTM-22.2. The repository is located at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

It should be noted that some of the technologies for which merge requests were submitted to TuC, got moved into the working draft. The merge requests were closed, when appropriate merge requests for VTM were available.

VTM-22.2-TuC-5.0 is expected to be tagged during the 40th JVET meeting.

The following changes were merged into the TuC branch:

- JVET-AG0328: FGR SEI message

The following merge requests were submitted and are pending:

- JVET-AM0086: PHOTOSENSITIVE CONTENT ADDITIONAL INFORMATION

- JVET-AL0219

- JVET-AK0142: "AHG9: Display Rectangles SEI"

- JVET-AI0340: Implementation of AI-restrictions usage SEI message

- HM related activities

There was no new HM version tagged during this meeting cycle.

The following MRs were merged:

- JVET-AE0101: implement phase indication SEI message

- Update auto-build from VTM build definition

- Update copyright headers to include 2025

- JVET-AK0107: Modality Information SEI

- JVET-AK0194: Digitally Signed Content SEI messages

- Fix building on ARM

- Enable build on macOS/arm

- Modify semantics of IntraPeriod to enable autosetting based on frame rate

- Fix yuv output when bitdepth is changed between sequence

- Add missing initialization in TComPPS and TComScalingList (fix asan/msan errors)

The following MRs are pending [with status indicated]:

- JVET-AL0059: Packed regions information SEI (pending reviewing)

- JVET-AL0148: implementation of GFV and GFVE SEI messages (pending review)

- Avoid reading SEI content when payloadSize = 0 (one issue remains)

- JVET-AL0062: AI usage restrictions SEI message (pending review)

- JVET-AK2006: Add support for SPTI SEI message (pending review/rebasing)

- JVET-AL0061: Import encoder optimization information SEI Message form VTM (pending review)

- JVET-AL0148: implementation of GFV and GFVE SEI messages (pending review)

- Port the Y4M support [one issue remains]

- Mark the current picture as short-term ref (for SCM) [need SCC expert reviewer]

A new version of HM is expected to be tagged during or after the 40th JVET meeting.

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimization in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Otherwise the HEVC bug tracker lists:

- 44 tickets for “HM”, (most of which were created before 2019; 1 created in last year),

- 1 ticket for “HM RExt” (created in 2020),

- 9 tickets for “HM SCC” (most of which were created before 2018),

- 1 ticket for “RExt Text” (created in 2015),

- 1 ticket for “SCC Text” (created in 2016),

- 8 tickets for text (6 created before 2021; 2 created in the last year),

- 2 tickets for encoder description (created in 2020 and 2014)

Help to address these tickets would be appreciated.

- 360Lib related activities

The latest 360Lib software can be found at https://vcgit.hhi.fraunhofer.de/jvet/360lib/-/tags/360Lib-13.6

The following table is for the projection formats comparison using VTM-23.0 according to 360-degree video CTC (JVET-U2012) compared to that using VTM-22.0 (VTM-22.0 as anchor).

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | 0.00% | -0.01% | 0.05% | 0.00% | 0.00% | 0.05% |
| Class S2 | 0.08% | 0.16% | 0.34% | 0.08% | 0.15% | 0.34% |
| Overall | 0.03% | 0.06% | 0.17% | 0.03% | 0.06% | 0.16% |

The following table compares generalized cubemap (GCMP) coding and padded equi-rectangular projection (PERP) coding using VTM-23.0 (PERP as anchor).

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.52% | -5.47% | -6.09% | -11.50% | -5.41% | -6.03% |
| Class S2 | -3.65% | 0.98% | 1.48% | -3.62% | 1.08% | 1.56% |
| Overall | -8.37% | -2.89% | -3.06% | -8.35% | -2.82% | -3.00% |

The following tables are for PERP and GCMP coding comparison between VTM-23.0 and HM-16.22 (HM as anchor), respectively.

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -30.91% | -38.82% | -41.24% | -30.91% | -38.88% | -41.24% |
| Class S2 | -36.84% | -37.23% | -39.43% | -36.82% | -37.27% | -39.49% |
| Overall | -33.28% | -38.19% | -40.51% | -33.28% | -38.23% | -40.54% |

|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class S1 | -34.64% | -40.04% | -42.14% | -34.59% | -40.02% | -42.09% |
| Class S2 | -38.34% | -38.18% | -40.06% | -38.32% | -38.19% | -40.11% |
| Overall | -36.12% | -39.30% | -41.31% | -36.08% | -39.29% | -41.30% |

- SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

- SHM related activities

There had not been any further developments to SHVC’s SHM during this meeting cycle.

- HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

One merge request is was merged:

- JVET-AE0295: MV Main 10 profile support (waiting for proponent response)

The next release will also include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

- HDRTools related activities

Two new versions (0.25 and 0.26) of HDRTools have been tagged.

Version 0.25 includes the following changes:

- Update HDRMontage to support temporal interleaving

- Fixes to TIFF files with Alpha

- support for absolute crop

- Compute metrics with files of different bitdepth

Version 0.26 includes the following changes:

- fixes computation of WPSNR metric to match VTM software when EnableJVETPSNR=1

- fixes build on recent gcc versions (13 and above)

- JM, JSVM, JMVM related activities

There had not been any further developments to JSVM or JMVM during this meeting cycle.

There was no new JM version tagged during this meeting cycle, although merge requests were submitted for SEI messages in VSEI.

The following MRs are pending [with status indicated]:

- JVET-AK0107: Modality Information SEI (pending review)

- JVET-AK2006: NNPFC and NNPFA SEI message (pending review)

- JVET-AL0062: AI usage restrictions SEI message (pending review)

- JVET-AK2006: SPTI SEI Message (pending review)

- JVET-AL0061: Encoder Optimization Information SEI Message (pending review)

- JVET-AE0101: implement phase indication SEI message (pending review)

- JVET-AL0148: Add support for Generative Face Video (GFV) and Generative Face Video Enhancement (GFVE) SEI messages(pending review)

For the previous release of JM only basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch was reported privately, but needs more investigation.

- Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain.

https://hevc.hhi.fraunhofer.de/trac/hevc

New account creation was disabled due to a large amount of spam registration. Extensive maintenance of the Trac bug tracker would be required to keep it secure. It should be considered to move bug tracking to GitLab, although no automatic conversion of existing bugs is available.

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Contributors were requested to please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

- CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-AL2010		VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018		VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100		HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009		SHVC test conditions

JVET-AC1015		SCM test conditions

JVET-AE1013		3DV test conditions

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

- Guidelines for reference software development

Current versions of the software guidelines are:

- JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

- JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

- Related proposals

JVET-AN0292 proposes to add multi-view functionality to HM. Mutli-view HEVC is usually supported by HTM. It is asserted that range extension support in HTM is very limited but required for the new multi-view profiles. It is asserted that it was easier to port multi-view functionality to HM instead if range extension to HTM.

- Recommendations

The AHG recommends to:

- Continue to develop reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

Numerous merge requests for adding new SEI messages to JM and HM had not yet been integrated due to lack of time of software coordinators. This should be used later as basis for issuing new versions of AVC and HEVC reference software.

Further discussion on JVET-AN0292 about harmonization of HM and HTM – see section 4.3. It was mentioned that also document JVET-AN0048 reports activities on harmonizing HM and SHM

JVET-AN0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

- Preparations for the Joint Call for Evidence on video compression with capability beyond VVC

AHG4 supported the activities of AHG17 and prepared for the on-site testing activity.

- Test sequences

The set of JVET test sequences is hosted at https://vqa.lfb.rwth-aachen.de. A mirror of this site is available at https://datacloud.hhi.fraunhofer.de with the same login credentials. The directory structure of the previous ftp server has been maintained. The test sequences used for CfP/CTC are available in directory “/ctc”.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG). Members of JVET may contact the JVET chair for login information. The password for the site is updated after each meeting with the Zoom password for that meeting. The tentative schedule for the changes is as follows:

- After the April meeting, password changes on 1st of June

- After the July meeting, password changes on 1st of September

- After the October meeting, password changes on 1st of December.

- After the January meeting, password changes on 1st of March.

- Related contributions

(listing document directly related to AHG4 as well as documents related to testing activities and test material)

| JVET-AN0048 | AHG4: teleconference on VVC multilayer testing | P. de Lagrange (InterDigital) |
| --- | --- | --- |
| JVET-AN0053 | AHG4/AHG15/AHG17: 4K gaming sequences featuring “Black Myth: Wukong” | J. Sauer, Y. Zhao, Y. Sun, J. Zhou, E. Alshina (Huawei) |
| JVET-AN0177 | AHG4/AHG17: Response to the call for new HDR materials for future video coding development | J. Wang, J. Zhang, L. Yu (ZJU) |
| JVET-AN0298 | AHG4: proposed updates for VVC multi-layer verification test plan | P. de Lagrange (InterDigital) |

It is noted that JVET-AN0048 includes new test sequences which were explored in the context of the VVC ML testing activity, some of which are available both in SDR and HDR.

- Recommendations

The AHG recommended:

- To conduct the viewing tests for the Joint Call for Evidence on video compression with capability beyond VVC with the test points recommended by AHG17.

- To continue to discuss and to update the non-finalized categories of the verification test plan for multilayer VVC and the subjective quality test plan for FGC, including those which have not been addressed yet.

- To collect volunteers to conduct further verification tests and subjective quality tests.

- To collect volunteers to actively contribute to the verification test development.

- To review the set of available test sequences for the verification tests as well as subjective quality tests and potentially collect more test sequences with a variety of content.

- To continue to collect new test sequences available for JVET with licensing statement.

JVET-AN0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

- Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 39th and the 40th meetings.

- Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15

  - ISO/IEC FDIS 23090-15:202x 2nd edition text output of 2023-01, preparation delayed to 2023-09, FDIS ballot opened 2024-04-08, FDIS ballot closed 2024-06-03, published 2024-07-04

  - H.266.1 V2 Consent 2023-07, Last Call began 2023-08-16, Approved 2023-09-13, pre-published 2023-10-06, published 2023-10-19

- VVCv3 conformance:

  - CD: 2025-04-15

  - DIS: 2025-07-25

  - FDIS: 2026-02-20

  - IS: 2026-06-30

  - H.266.1 V3 Consent 2025-10

- Status on bitstream submission

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - No changes between 39th and the 40th meeting.

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 128 bitstreams of 57 identified categories have been provided, cross-checked and made available

  - No changes between the 39th and 40th meeting.

- additional conformance bitstreams for VVC Multilayer:

  - 3 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 7 total bitstreams have been provided, cross-checked, and made available

  - No changes between the 39th and 40th meeting.

- conformance bitstreams for new HEVC Multiview profiles

  - 2 HEVC Multiview Extended 10 bitstreams have been provided, cross-checked, and made available, no changes between 39th and 40th meeting

  - 2 HEVC Multiview Extended bitstreams have been provided, cross-checked, and made available, no changes between 39th and 40th meeting

  - 2 HEVC Multiview Main 10 bitstreams have been provided, cross-checked and made available, no changes between 39th and 40th meeting.

- Activities and Discussion

The AHG activities were on schedule with the preliminary timeline shown above.

VVC activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

VVC operation range extensions activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

VVC Multilayer activities:

The streams are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

HEVC Multiview supporting extended bit depth activities:

The 6 packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/  No change in bitstreams and/or packages.

It has been reported that code is available to extend the HM encoder and decoder to support all the HEVC multiview profiles, including the Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles in JVET-AH1006 and in WG5 N0281). No update on the generation of corresponding conformance streams.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

- Contributions

JVET-AN0292 Multiview HEVC Reference SW implementation status [S. Choi, S. Paluri, D. Podborski, A. Tourapis (Apple)].

JVET-AN0293 Multiview 4:4:4 profiles for HEVC [S. Choi, S. Paluri, D. Podborski, E. Asbun, A. Tourapis (Apple)].

- Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

- Recommendations

The AHG recommends the following:

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and continue the generation, cross-checking, and documentation of the conformance streams for the HEVC Multiview profiles supporting extended bit depth, the HEVC Multiview Main 10 profile, and the HEVC Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles).

- Study DIS of VVC conformance 3rd edition and suggest improvements to JVET-AL2028, as appropriate.

It ws reminded to deliver the 3rd edition of VVC conformance for ITU consent by Oct.15.

JVET-AN0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-18.0:

- JVET-AM0056: Predictive Transform Coefficient Coding (PTCC) [MR 933]

- JVET-AM0106: Chained mv candidates in AMVP and merge (Test 2.1b) [MR 932]

- JVET-AM0163: Block prediction with cubic interpolation filter in TIMD (Test 1.7a) [MR 934]

- JVET-AM0215: Regression-based GPM intra-inter prediction modification (Test 2.2a) [MR 936]

- JVET-AM0068 for fixing encoding side of JVET-AE0102 and JVET-AG0100 [MR 935]

- JVET-AM0104: EE2-related: On reference sample filtering for TIMD [MR 938]

- JVET-AM0063: On ALF-CCCM (Test 4.3c) [MR 937]

- JVET-AM0157: Harmonization of SGPM-BV and LIC (Test 1.8) [MR 946]

- JVET-AM0216: Fixes for 12-bit internal bit depth in ECM [MR 947]

- JVET-AM0231: NNLF encoder optimizations (Test 4.5c) [MR 944]

- JVET-AM0307: combination of EE2-1.2c, EE2-1.6a and EE2-1.3 (Test 1.12) [MR 941]

- JVET-AM0209: Chroma ALF and CCALF with reused CTB control (Test 4.2) [MR 945]

- JVET-AM0221: TMRL tool-off bug fix [MR 950]

- JVET-AM0295: External memory bandwidth evaluation [MR 949]

- Add self-contained cfg for CfE [MR 940]

Fixes:

Fix parameters rewriting in SPS for AE0159, AG0112, AK0095 (#111) [MR 931]

Fix #22 and #77: solve encoder crash when --DepQuant=0 [MR 942]

The following changes were integrated into VTM-11.0ecm anchor:

JVET-AM0280: encoder and decoder memory print for VTM11_ANC [MR 939]

ECM-18.0 and VTM-11.0ecm18.0 were tagged on August 4, 2025.

  - CTC Performance

In this section, ECM test results following ECM CTC configuration described in JVET-AI2017 are summarized.

ECM-18.0 performance over ECM-17.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | 0.13% | 0.29% | 103.6% | 104.6% | 101.4% | 101.6% |
| Class A2 | -0.28% | 0.12% | 0.11% | 103.2% | 102.2% | 101.5% | 101.9% |
| Class B | -0.20% | 0.06% | 0.07% | 102.8% | 103.5% | 102.7% | 101.5% |
| Class C | -0.21% | -0.11% | -0.13% | 101.3% | 104.2% | 101.6% | 100.8% |
| Class E | -0.21% | -0.36% | -0.24% | 100.2% | 102.9% | 97.9% | 100.6% |
| Overall | -0.19% | -0.02% | 0.02% | 102.2% | 103.5% | 101.2% | 101.3% |
| Class D | -0.10% | 0.13% | -0.07% | 103.1% | 107.1% | 101.0% | 100.3% |
| Class F | -0.66% | -0.56% | -0.42% | 103.3% | 104.8% | 101.8% | 101.0% |
| Class TGM | -0.62% | -0.49% | -0.47% | 98.2% | 102.9% | 100.3% | 101.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | -0.49% | -0.38% | 100.0% | 99.9% | 100.8% | 100.6% |
| Class A2 | -0.22% | -0.32% | -0.51% | 95.6% | 95.2% | 100.7% | 101.1% |
| Class B | -0.16% | -0.92% | -0.53% | 99.1% | 100.5% | 100.6% | 101.0% |
| Class C | -0.07% | -0.84% | -0.72% | 97.9% | 97.8% | 101.0% | 100.7% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.69% | -0.55% | 98.3% | 98.6% | 100.7% | 100.9% |
| Class D | -0.03% | -0.81% | -1.32% | 101.2% | 102.4% | 101.2% | 100.3% |
| Class F | -0.48% | -1.06% | -0.63% | 103.4% | 104.8% | 101.0% | 101.0% |
| Class TGM | -0.50% | -0.66% | -0.87% | 95.8% | 100.1% | 101.2% | 101.0% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -1.20% | -0.44% | 98.2% | 96.1% | 100.6% | 100.7% |
| Class C | -0.10% | -0.85% | -0.44% | 99.1% | 100.6% | 101.0% | 100.5% |
| Class E | -0.23% | 0.46% | 0.32% | 98.8% | 100.5% | 100.7% | 100.5% |
| Overall | -0.13% | -0.67% | -0.25% | 98.6% | 98.7% | 100.8% | 100.6% |
| Class D | 0.20% | -1.86% | -0.92% | 101.4% | 99.8% | 101.3% | 100.2% |
| Class F | -0.74% | -0.58% | -0.58% | 100.4% | 97.6% | 100.2% | 100.9% |
| Class TGM | -0.49% | -0.88% | -1.09% | 97.1% | 100.1% | 101.3% | 100.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-17.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | -1.16% | -0.92% | 95.9% | 95.1% | 100.6% | 100.8% |
| Class C | -0.11% | -0.97% | -0.81% | 99.3% | 101.7% | 101.0% | 100.7% |
| Class E | -0.02% | -0.99% | -0.08% | 101.9% | 102.4% | 100.7% | 100.5% |
| Overall | -0.12% | -1.05% | -0.68% | 98.5% | 99.1% | 100.7% | 100.7% |
| Class D | -0.05% | -0.87% | -2.16% | 98.7% | 98.7% | 100.9% | 100.3% |
| Class F | -0.64% | -0.95% | -0.89% | 104.3% | 98.3% | 100.8% | 100.6% |
| Class TGM | -0.29% | -0.75% | -1.02% | 96.1% | 94.3% | 99.7% | 100.8% |

The below tables show ECM-18.0 performance comparing to VTM-11.0ecm18.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -14.82% | -16.49% | -27.44% | 1202.2% | 577.9% | 236.6% | 325.9% |
| Class A2 | -21.47% | -24.85% | -29.07% | 1119.5% | 624.0% | 237.6% | 325.3% |
| Class B | -15.06% | -23.15% | -21.18% | 1086.0% | 627.5% | 181.0% | 355.2% |
| Class C | -15.19% | -12.08% | -13.23% | 1026.3% | 583.6% | 121.7% | 394.4% |
| Class E | -19.58% | -23.35% | -21.51% | 992.5% | 615.0% | 139.3% | 381.4% |
| Overall | -16.87% | -19.90% | -21.83% | 1080.0% | 606.4% | 173.6% | 357.4% |
| Class D | -12.92% | -8.80% | -9.64% | 1013.4% | 663.9% | 109.3% | 414.2% |
| Class F | -30.84% | -34.56% | -34.60% | 688.3% | 658.9% | 125.9% | 374.2% |
| Class TGM | -43.72% | -49.31% | -48.60% | 513.1% | 658.7% | 149.7% | 351.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -27.52% | -25.12% | -37.22% | 1205.3% | 1195.4% | 153.0% | 247.5% |
| Class A2 | -30.81% | -34.98% | -40.50% | 1103.1% | 1346.4% | 152.7% | 246.9% |
| Class B | -25.42% | -34.33% | -31.10% | 999.4% | 1197.0% | 144.5% | 267.5% |
| Class C | -27.12% | -24.29% | -25.03% | 1064.9% | 1294.1% | 116.8% | 321.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -27.37% | -29.94% | -32.58% | 1076.3% | 1250.9% | 139.7% | 272.1% |
| Class D | -27.81% | -24.52% | -25.81% | 1003.1% | 1417.7% | 104.9% | 360.3% |
| Class F | -33.60% | -36.80% | -37.37% | 851.2% | 853.5% | 121.7% | 296.3% |
| Class TGM | -43.07% | -48.90% | -48.87% | 693.7% | 676.2% | 125.7% | 281.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -22.61% | -40.61% | -35.58% | 976.7% | 1030.5% | 166.8% | 266.9% |
| Class C | -25.00% | -28.31% | -30.38% | 937.3% | 1074.8% | 125.2% | 316.1% |
| Class E | -22.74% | -29.72% | -28.28% | 965.0% | 712.7% | 139.7% | 296.8% |
| Overall | -23.44% | -33.79% | -32.02% | 960.5% | 953.0% | 145.0% | 290.0% |
| Class D | -26.46% | -29.97% | -31.37% | 938.9% | 1175.1% | 110.5% | 352.6% |
| Class F | -31.37% | -41.12% | -41.47% | 859.7% | 808.4% | 129.6% | 295.0% |
| Class TGM | -41.36% | -51.49% | -51.48% | 691.5% | 643.3% | 125.2% | 281.6% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -20.59% | -48.71% | -44.40% | 846.1% | 954.0% | 171.8% | 293.3% |
| Class C | -23.15% | -37.36% | -38.79% | 797.4% | 1112.7% | 126.4% | 344.7% |
| Class E | -20.95% | -38.22% | -37.17% | 859.0% | 674.0% | 143.6% | 318.3% |
| Overall | -21.53% | -42.30% | -40.72% | 832.7% | 920.7% | 148.3% | 315.9% |
| Class D | -25.84% | -40.19% | -41.26% | 768.1% | 1094.3% | 110.8% | 374.4% |
| Class F | -29.58% | -46.43% | -46.91% | 811.4% | 771.7% | 130.6% | 319.2% |
| Class TGM | -39.54% | -53.82% | -53.53% | 718.9% | 626.2% | 126.0% | 294.8% |

- ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1 & A2) | 9 | 16 |  |
| class B | 4 | 6 | 5 |
| class C | 2 | 3 | 3 |
| class D | 2 | 2 | 2 |
| class E | 3 |  | 3 |
| class F | 4 | 6 | 5 |
| class TGM | 3 | 8 | 6 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

- Recommendations

The AHG recommended to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.

JVET-AN0007 JVET AHG report: ECM tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice chairs)]

- Conference call on Complexity Analysis

An input document was uploaded as JVET-N0043. The call was on August 19, 2025 Tuesday UTC 14:00-15:29. The report was uploaded as JVET-AN0044.

- Group off tests

  - Test settings and crosschecking

The following five groups of tools were defined.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighbouring reconstructed samples than VVC

- Group 5: Tools with large memory access

The testers and crosscheckers are planed in the table below.

| Tests | Testers / Crosscheckers | Testers / Crosscheckers |
| --- | --- | --- |
| Group 1 off | Johan Pardo (johan.esprit.pardo1@huawei.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Ryo Ishimoto (ishimoto.ryo@mail.sharp) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) | Xiang Li (xlxiangli@google.com) |
| Group 5 off | Lien-Fei Chen (lienfei.chen@global.tencent.com) | Jani Lainema (jani.lainema@nokia.com) |
| Group 1-5 off | Xiang Li (xlxiangli@google.com) | Hongtao Wang (hongtaow@qti.qualcomm.com) |

ECM-18.0 was used in the AHG7 tool off tests. The cfg files used are included in the ECM software package.

  - Group 1 off

Group 1 includes inter template matching tools. The offgroup1.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11ecm18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101.2% | 100.1% | 100.2% | -14.82% | -16.49% | -27.45% | 1073.6% | 587.2% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99.5% | 98.5% | 100.2% | -21.47% | -24.85% | -29.07% | 1010.9% | 627.9% |
| Class B | 0.00% | 0.00% | 0.00% | 99.4% | 98.3% | 100.0% | -15.06% | -23.15% | -21.18% | 1009.7% | 696.3% |
| Class C | -0.01% | -0.01% | -0.01% | 99.9% | 98.4% | 101.8% | -15.20% | -12.09% | -13.23% | 966.1% | 661.7% |
| Class E | -0.01% | -0.01% | -0.01% | 99.6% | 97.0% | 101.0% | -19.60% | -23.36% | -21.52% | 938.1% | 732.0% |
| Overall | -0.01% | -0.01% | -0.01% | 99.9% | 98.4% | 100.6% | -16.88% | -19.90% | -21.83% | 998.0% | 663.3% |
| Class D | -0.04% | -0.04% | -0.04% | 100.0% | 99.2% | 99.9% | -12.96% | -8.83% | -9.67% | 923.6% | 667.2% |
| Class F | -0.01% | -0.01% | -0.01% | 99.0% | 98.3% | 102.9% | -30.85% | -34.57% | -34.61% | 642.0% | 711.2% |
| Class TGM | -0.01% | 0.00% | 0.00% | 99.2% | 96.6% | 104.7% | -43.72% | -49.31% | -48.60% | 515.3% | 734.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11ecm18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.92% | 4.80% | 5.01% | 81.5% | 68.6% | 100.0% | -23.92% | -21.75% | -33.92% | 969.7% | 951.7% |
| Class A2 | 5.74% | 5.65% | 6.00% | 81.0% | 62.4% | 100% | -26.84% | -31.37% | -36.91% | 922.5% | 1056.1% |
| Class B | 4.58% | 4.67% | 4.66% | 77.1% | 61.8% | 99.9% | -21.97% | -31.21% | -27.84% | 767.8% | 965.4% |
| Class C | 5.01% | 5.28% | 5.62% | 72.3% | 57.8% | 100.0% | -23.43% | -20.28% | -20.78% | 752.6% | 1023.7% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.99% | 5.05% | 5.25% | 77.4% | 62.1% | 100% | -23.72% | -26.44% | -28.99% | 830.2% | 995.5% |
| Class D | 4.08% | 4.12% | 4.28% | 72.1% | 55.7% | 100.0% | -24.81% | -21.34% | -22.53% | 689.5% | 1034.3% |
| Class F | 3.76% | 3.86% | 3.96% | 85.5% | 71.6% | 100.0% | -31.00% | -34.27% | -34.79% | 714.4% | 786.3% |
| Class TGM | 4.58% | 4.42% | 4.43% | 86.2% | 73.8% | 100.0% | -40.47% | -46.62% | -46.57% | 620.9% | 666.5% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11ecm18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 5.80% | 6.38% | 6.42% | 69.8% | 55.4% | 100% | -18.09% | -36.84% | -31.52% | 704.1% | 798.9% |
| Class C | 5.72% | 5.83% | 6.11% | 67.2% | 50.1% | 100% | -20.71% | -24.18% | -26.16% | 652.3% | 782.8% |
| Class E | 6.02% | 6.49% | 7.09% | 75.6% | 64.4% | 102% | -18.09% | -25.21% | -23.18% | 717.4% | 730.9% |
| Overall (Ref) | 5.83% | 6.23% | 6.49% | 70.3% | 55.6% | 101% | -18.96% | -29.71% | -27.65% | 689.6% | 776.1% |
| Class D | 5.10% | 5.60% | 5.73% | 68.2% | 51.3% | 100% | -22.70% | -25.97% | -27.52% | 643.3% | 814.3% |
| Class F | 5.73% | 5.31% | 4.65% | 82.1% | 64.2% | 100% | -27.37% | -37.93% | -38.40% | 692.6% | 691.6% |
| Class TGM | 6.56% | 6.41% | 6.56% | 85.1% | 65.6% | 100% | -37.47% | -48.19% | -48.07% | 637.5% | 573.2% |

  - Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The offgroup2.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11.0ecm18.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.58% | 0.62% | 94.9% | 96.9% | 100% | -14.66% | -16.07% | -26.98% | 1121.5% | 546.9% |
| Class A2 | 0.23% | 0.42% | 0.48% | 94.1% | 97.1% | 100% | -21.29% | -24.55% | -28.75% | 1074.0% | 581.5% |
| Class B | 0.18% | 0.48% | 0.59% | 93.4% | 96.2% | 98% | -14.90% | -22.79% | -20.71% | 1029.5% | 591.6% |
| Class C | 0.15% | 0.30% | 0.43% | 91.9% | 95.2% | 99% | -15.06% | -11.82% | -12.85% | 982.5% | 532.0% |
| Class E | 0.26% | 0.72% | 0.53% | 93.9% | 96.3% | 102% | -19.37% | -22.83% | -21.09% | 977.3% | 616.0% |
| Overall | 0.19% | 0.49% | 0.53% | 93.5% | 96.3% | 99% | -16.71% | -19.53% | -21.41% | 1031.8% | 572.5% |
| Class D | 0.11% | 0.34% | 0.38% | 91.1% | 94.8% | 99% | -12.83% | -8.49% | -9.30% | 939.3% | 508.6% |
| Class F | 0.51% | 0.50% | 0.50% | 93.7% | 98.5% | 100% | -30.50% | -34.25% | -34.32% | 607.0% | 560.3% |
| Class TGM | 0.36% | 0.38% | 0.34% | 94.4% | 100.9% | 100% | -43.50% | -49.12% | -48.43% | 503.6% | 632.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11.0ecm18.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 2.39% | 1.56% | 2.24% | 85.6% | 96.4% | 100% | -25.78% | -23.92% | -35.84% | 1001.1% | 1294.6% |
| Class A2 | 1.44% | 1.26% | 1.76% | 84.9% | 97.2% | 100% | -29.80% | -34.15% | -39.41% | 953.0% | 1578.7% |
| Class B | 1.48% | 1.24% | 1.33% | 85.8% | 96.2% | 99% | -24.31% | -33.45% | -30.12% | 841.3% | 1442.6% |
| Class C | 1.09% | 1.19% | 1.01% | 85.2% | 94.3% | 99% | -26.33% | -23.40% | -24.28% | 887.7% | 1620.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.55% | 1.29% | 1.51% | 85.4% | 95.9% | 99% | -26.24% | -29.00% | -31.56% | 905.9% | 1482.6% |
| Class D | 0.54% | 0.94% | 0.58% | 84.7% | 94.7% | 99% | -27.42% | -23.82% | -25.33% | 821.5% | 1735.4% |
| Class F | 2.00% | 2.19% | 1.93% | 88.6% | 96.0% | 98% | -32.27% | -35.42% | -36.11% | 712.6% | 980.5% |
| Class TGM | 3.27% | 4.82% | 5.06% | 91.0% | 99.2% | 100% | -41.33% | -46.72% | -46.57% | 637.5% | 872.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11.0ecm18.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.88% | 3.55% | 3.17% | 80.6% | 92.6% | 100% | -21.15% | -38.58% | -33.52% | 798.4% | 1000.7% |
| Class C | 1.59% | 2.39% | 2.79% | 81.4% | 90.4% | 99% | -23.86% | -26.70% | -28.46% | 774.5% | 1069.6% |
| Class E | 1.30% | 1.64% | 3.41% | 84.1% | 94.8% | 99% | -21.73% | -28.53% | -25.69% | 744.7% | 691.0% |
| Overall (Ref) | 1.64% | 2.69% | 3.10% | 81.7% | 92.4% | 99% | -22.20% | -32.11% | -29.88% | 776.7% | 932.7% |
| Class D | 0.66% | 2.66% | 2.96% | 78.1% | 93.7% | 98% | -25.95% | -28.16% | -29.28% | 742.7% | 1216.4% |
| Class F | 2.69% | 2.24% | 1.47% | 86.2% | 96.1% | 99% | -29.54% | -39.95% | -40.44% | 698.3% | 708.0% |
| Class TGM | 2.61% | 6.24% | 7.16% | 89.7% | 97.1% | 100% | -39.93% | -48.83% | -48.63% | 639.5% | 592.9% |

  - Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The offgroup3.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM18 |  |  |  |  |  | Over VTM11ecm18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.23% | 4.24% | 6.53% | 71.3% | 90.0% | 100% | -13.77% | -13.27% | -23.17% | 822.0% | 487.9% |
| Class A2 | 2.70% | 7.11% | 6.74% | 71.3% | 81.3% | 100% | -19.29% | -19.95% | -24.82% | 793.6% | 477.5% |
| Class B | 2.74% | 5.57% | 5.78% | 68.2% | 78.5% | 101% | -12.77% | -19.31% | -16.88% | 739.4% | 506.4% |
| Class C | 2.59% | 3.02% | 3.44% | 68.7% | 74.9% | 99% | -13.05% | -9.54% | -10.34% | 705.9% | 471.7% |
| Class E | 3.78% | 4.88% | 4.57% | 69.4% | 78.2% | 100% | -16.54% | -19.79% | -17.97% | 700.8% | 523.0% |
| Overall | 2.62% | 4.92% | 5.34% | 69.5% | 79.9% | 100% | -14.71% | -16.32% | -17.98% | 747.0% | 493.2% |
| Class D | 1.64% | 2.09% | 2.52% | 69.1% | 79.4% | 100% | -11.50% | -6.92% | -7.37% | 690.6% | 565.2% |
| Class F | 7.24% | 8.08% | 8.07% | 71.9% | 67.5% | 99% | -26.07% | -29.76% | -29.75% | 454.9% | 436.7% |
| Class TGM | 12.49% | 14.41% | 14.86% | 77.1% | 56.4% | 100% | -36.83% | -42.19% | -41.18% | 404.4% | 367.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM18 |  |  |  |  |  | Over VTM11ecm18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.05% | 2.33% | 5.96% | 93.1% | 99.5% | 100% | -26.76% | -23.43% | -34.25% | 1060.9% | 1138.3% |
| Class A2 | 1.65% | 3.93% | 3.86% | 93.6% | 98.6% | 100% | -29.67% | -32.54% | -38.26% | 1029.2% | 1361.4% |
| Class B | 1.41% | 4.90% | 4.61% | 91.2% | 98.8% | 100% | -24.37% | -31.11% | -27.96% | 864.6% | 1202.3% |
| Class C | 1.04% | 1.87% | 2.02% | 92.4% | 98.6% | 100% | -26.35% | -22.84% | -23.46% | 950.5% | 1354.3% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.29% | 3.39% | 4.04% | 92.4% | 98.9% | 100% | -26.44% | -27.66% | -30.08% | 956.5% | 1258.5% |
| Class D | 0.86% | 1.64% | 2.31% | 92.3% | 99.5% | 100% | -27.19% | -23.26% | -23.98% | 886.5% | 1480.0% |
| Class F | 4.82% | 5.39% | 5.45% | 90.3% | 90.8% | 99% | -30.64% | -33.80% | -34.31% | 730.9% | 764.0% |
| Class TGM | 7.64% | 9.30% | 9.74% | 94.0% | 92.8% | 100% | -38.74% | -44.21% | -43.96% | 650.3% | 659.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM18 |  |  |  |  |  | Over VTM11ecm18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.52% | 1.95% | 1.56% | 94.2% | 98.6% | 100% | -22.21% | -39.56% | -34.74% | 925.1% | 1114.7% |
| Class C | 0.51% | 1.13% | 1.02% | 94.0% | 100.2% | 100% | -24.62% | -27.49% | -29.67% | 875.7% | 1142.5% |
| Class E | 1.13% | 2.72% | 1.98% | 95.3% | 96.5% | 100% | -21.84% | -27.81% | -26.71% | 859.8% | 737.1% |
| Overall (Ref) | 0.67% | 1.87% | 1.48% | 94.4% | 98.6% | 100% | -22.92% | -32.60% | -31.04% | 891.9% | 1013.5% |
| Class D | 0.23% | 0.68% | 0.77% | 96.2% | 104.6% | 100% | -26.30% | -29.57% | -31.01% | 915.7% | 1286.4% |
| Class F | 3.94% | 4.77% | 5.33% | 90.4% | 98.7% | 100% | -28.82% | -38.69% | -38.82% | 762.5% | 752.1% |
| Class TGM | 5.13% | 6.25% | 6.77% | 93.0% | 96.2% | 100% | -38.37% | -48.66% | -48.45% | 661.4% | 632.6% |

  - Group 4 off

Group 4 includes tools that require more processing on the neighbouring reconstructed samples than VVC.

The offgroup4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-18-0 |  |  |  |  |  | Over VTM-11-ECM18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.45% | 5.47% | 5.99% | 84.8% | 90.5% | 100% | -13.58% | -12.20% | -23.24% | 1017.5% | 530.1% |
| Class A2 | 1.67% | 5.38% | 5.95% | 82.6% | 92.8% | 100% | -20.20% | -21.14% | -25.30% | 931.7% | 575.5% |
| Class B | 0.99% | 5.16% | 5.19% | 85.9% | 94.3% | 98% | -14.23% | -19.65% | -17.33% | 963.0% | 633.7% |
| Class C | 0.97% | 3.27% | 3.37% | 86.0% | 93.0% | 99% | -14.38% | -9.34% | -10.42% | 921.9% | 579.9% |
| Class E | 1.51% | 6.42% | 5.34% | 86.9% | 96.1% | 103% | -18.38% | -18.83% | -17.49% | 921.1% | 658.5% |
| Overall | 1.26% | 5.04% | 5.07% | 85.4% | 93.4% | 100% | -15.84% | -16.23% | -18.13% | 950.2% | 597.3% |
| Class D | 0.90% | 2.54% | 2.97% | 85.4% | 90.1% | 100% | -12.15% | -6.50% | -6.94% | 884.3% | 606.3% |
| Class F | 1.31% | 4.49% | 4.75% | 92.1% | 95.2% | 99% | -29.95% | -31.80% | -31.68% | 608.3% | 625.3% |
| Class TGM | 2.19% | 4.16% | 4.30% | 93.7% | 97.2% | 100% | -42.55% | -47.30% | -46.51% | 496.3% | 645.6% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18-0 |  |  |  |  |  | Over VTM-11-ECM18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.85% | 2.98% | 5.01% | 94.4% | 99.7% | 100% | -26.19% | -22.96% | -34.45% | 1142.4% | 1197.4% |
| Class A2 | 1.30% | 3.57% | 3.81% | 95.8% | 100.1% | 100% | -29.90% | -32.74% | -38.29% | 1110.5% | 1445.2% |
| Class B | 0.99% | 3.53% | 3.25% | 95.3% | 99.4% | 100% | -24.69% | -32.10% | -28.98% | 972.1% | 1280.8% |
| Class C | 1.08% | 1.86% | 1.84% | 96.0% | 98.7% | 100% | -26.33% | -22.88% | -23.63% | 1067.2% | 1438.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.25% | 2.98% | 3.34% | 95.4% | 99.4% | 100% | -26.47% | -27.94% | -30.51% | 1057.1% | 1335.2% |
| Class D | 0.95% | 2.20% | 2.68% | 95.7% | 98.0% | 100% | -27.11% | -22.88% | -23.72% | 1008.8% | 1565.0% |
| Class F | 1.33% | 2.99% | 2.96% | 100.9% | 98.8% | 99% | -32.79% | -35.10% | -35.63% | 828.7% | 876.4% |
| Class TGM | 0.87% | 1.67% | 1.59% | 98.7% | 99.3% | 99% | -42.61% | -48.07% | -48.11% | 695.5% | 723.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18-0 |  |  |  |  |  | Over VTM-11-ECM18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.03% | 1.55% | 0.93% | 98.0% | 99.3% | 100% | -21.81% | -39.76% | -35.12% | 1003.0% | 1132.0% |
| Class C | 1.28% | 0.87% | 0.44% | 97.0% | 97.8% | 99% | -24.05% | -27.73% | -30.15% | 974.1% | 1161.2% |
| Class E | 1.07% | 3.91% | 2.22% | 98.5% | 99.5% | 100% | -21.90% | -26.95% | -26.69% | 925.4% | 742.4% |
| Overall (Ref) | 1.12% | 1.91% | 1.09% | 97.8% | 98.8% | 100% | -22.58% | -32.55% | -31.35% | 973.5% | 1027.4% |
| Class D | 1.03% | 0.31% | 0.34% | 98.7% | 100.4% | 99% | -25.65% | -29.79% | -31.33% | 992.3% | 1344.2% |
| Class F | 1.53% | 1.60% | 1.22% | 98.0% | 99.6% | 100% | -30.33% | -40.36% | -40.73% | 825.7% | 776.4% |
| Class TGM | 0.49% | 1.20% | 1.34% | 98.2% | 97.9% | 99% | -41.06% | -50.85% | -50.85% | 709.4% | 624.4% |

  - Group 5 off

Group 5 includes tools which need large memory access.

The offgroup5.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11.0ecm18.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.47% | 4.72% | 4.60% | 77.9% | 85.2% | 90% | -10.95% | -12.55% | -24.14% | 798.6% | 432.1% |
| Class A2 | 3.66% | 3.23% | 3.79% | 80.0% | 89.5% | 90% | -18.57% | -22.50% | -26.54% | 791.0% | 505.5% |
| Class B | 4.32% | 4.01% | 4.55% | 79.6% | 89.0% | 79% | -11.38% | -20.19% | -17.64% | 788.7% | 539.6% |
| Class C | 6.22% | 4.42% | 5.15% | 79.6% | 92.1% | 92% | -10.08% | -8.32% | -8.97% | 760.8% | 552.4% |
| Class E | 6.11% | 5.48% | 4.39% | 80.8% | 88.8% | 87% | -14.67% | -19.30% | -18.10% | 742.6% | 569.8% |
| Overall | 4.95% | 4.34% | 4.54% | 79.6% | 89.1% | 87% | -12.77% | -16.52% | -18.35% | 776.6% | 521.8% |
| Class D | 5.08% | 4.39% | 5.66% | 78.3% | 90.8% | 98% | -8.50% | -4.76% | -4.50% | 714.4% | 568.2% |
| Class F | 4.27% | 2.78% | 3.13% | 88.0% | 89.6% | 88% | -27.70% | -32.52% | -32.35% | 563.6% | 572.3% |
| Class TGM | 0.58% | 0.57% | 0.56% | 89.1% | 89.6% | 83% | -43.38% | -49.03% | -48.34% | 458.9% | 597.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11.0ecm18.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 3.87% | 2.78% | 3.56% | 93.6% | 91.3% | 96% | -24.74% | -22.97% | -35.19% | 1060.2% | 1033.2% |
| Class A2 | 3.40% | 2.25% | 2.81% | 94.2% | 92.6% | 96% | -28.47% | -33.54% | -38.82% | 1028.3% | 1295.5% |
| Class B | 4.16% | 2.93% | 3.42% | 92.8% | 92.4% | 84% | -22.33% | -32.45% | -28.79% | 889.2% | 1161.6% |
| Class C | 5.28% | 3.49% | 4.00% | 95.8% | 94.6% | 92% | -23.25% | -21.68% | -22.03% | 964.1% | 1396.7% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.25% | 2.91% | 3.48% | 94.0% | 92.8% | 91% | -24.28% | -27.90% | -30.27% | 968.9% | 1218.1% |
| Class D | 4.77% | 3.65% | 4.45% | 94.5% | 94.0% | 96% | -24.32% | -21.71% | -22.36% | 876.8% | 1501.9% |
| Class F | 3.28% | 2.55% | 2.41% | 88.0% | 91.5% | 89% | -31.27% | -35.06% | -35.68% | 706.3% | 761.0% |
| Class TGM | 0.76% | 0.80% | 0.75% | 87.1% | 83.0% | 88% | -42.60% | -48.49% | -48.50% | 598.4% | 580.8% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  | Over VTM-11.0ecm18.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 3.40% | 1.30% | 1.45% | 94.3% | 88.7% | 82% | -20.00% | -39.88% | -34.81% | 908.5% | 1005.3% |
| Class C | 4.97% | 1.67% | 2.16% | 96.2% | 90.5% | 90% | -21.30% | -27.16% | -28.93% | 885.0% | 1117.4% |
| Class E | 4.36% | 3.38% | 1.22% | 89.5% | 91.1% | 91% | -19.36% | -27.37% | -27.37% | 789.0% | 734.0% |
| Overall (Ref) | 4.16% | 1.94% | 1.63% | 93.7% | 89.9% | 87% | -20.27% | -32.51% | -30.99% | 869.4% | 962.6% |
| Class D | 4.38% | 2.34% | 2.65% | 94.8% | 92.0% | 96% | -23.21% | -28.38% | -29.67% | 862.5% | 1279.0% |
| Class F | 3.76% | 1.32% | 1.05% | 87.7% | 90.7% | 87% | -28.72% | -40.49% | -40.80% | 704.3% | 740.8% |
| Class TGM | 1.03% | 1.18% | 1.01% | 86.4% | 86.1% | 85% | -40.72% | -50.92% | -50.99% | 606.3% | 580.9% |

  - Group 1-5 off

In this test, all the tools in the group 1-5 are switched off. The offgroup1-5.cfg was used in addition to ECM CTC settings.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-18 |  |  |  |  |  | Over VTM-11-ECM18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 9.60% | 19.75% | 25.21% | 36.7% | 41.4% | 89% | -6.58% | -0.96% | -10.54% | 357.6% | 172.5% |
| Class A2 | 11.28% | 22.49% | 23.37% | 36.9% | 37.8% | 90% | -12.57% | -9.15% | -14.04% | 343.0% | 173.4% |
| Class B | 10.31% | 20.87% | 20.31% | 35.5% | 35.3% | 85% | -6.30% | -8.65% | -5.84% | 378.9% | 211.1% |
| Class C | 11.48% | 13.50% | 14.84% | 35.5% | 36.1% | 96% | -5.69% | -0.58% | -0.72% | 385.4% | 223.5% |
| Class E | 15.48% | 22.79% | 19.60% | 36.3% | 39.1% | 96% | -7.15% | -6.83% | -6.34% | 382.1% | 251.6% |
| Overall | 11.47% | 19.64% | 20.30% | 36.1% | 37.5% | 91% | -7.40% | -5.35% | -6.93% | 371.0% | 206.0% |
| Class D | 8.99% | 11.66% | 13.20% | 35.7% | 38.8% | 97% | -5.11% | 1.81% | 2.33% | 372.9% | 266.2% |
| Class F | 19.14% | 23.57% | 24.48% | 48.9% | 37.2% | 94% | -17.91% | -20.18% | -19.45% | 284.2% | 229.5% |
| Class TGM | 20.17% | 24.86% | 26.00% | 54.8% | 38.1% | 89% | -32.66% | -37.03% | -35.67% | 267.6% | 240.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18 |  |  |  |  |  | Over VTM-11-ECM18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 15.97% | 16.45% | 24.88% | 54.7% | 57.3% | 95% | -16.00% | -13.08% | -22.98% | 514.0% | 453.0% |
| Class A2 | 15.75% | 19.84% | 21.07% | 55.0% | 54.9% | 94% | -19.96% | -22.47% | -28.11% | 492.6% | 519.8% |
| Class B | 14.33% | 20.85% | 20.86% | 50.6% | 56.0% | 82% | -14.70% | -20.84% | -16.82% | 487.8% | 625.6% |
| Class C | 14.94% | 15.60% | 16.82% | 48.0% | 50.1% | 90% | -16.17% | -12.53% | -12.40% | 514.6% | 658.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 15.10% | 18.37% | 20.63% | 51.6% | 54.4% | 89% | -16.40% | -17.40% | -19.13% | 501.0% | 572.8% |
| Class D | 12.05% | 13.85% | 15.71% | 45.9% | 48.4% | 95% | -18.97% | -13.99% | -13.95% | 475.6% | 684.8% |
| Class F | 18.99% | 21.46% | 21.97% | 52.0% | 57.1% | 89% | -21.17% | -23.80% | -24.05% | 432.0% | 442.6% |
| Class TGM | 20.67% | 25.30% | 26.03% | 55.4% | 52.5% | 89% | -31.43% | -36.28% | -35.92% | 394.0% | 355.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-18 |  |  |  |  |  | Over VTM-11-ECM18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 13.30% | 14.18% | 15.48% | 44.0% | 40.7% | 79% | -12.30% | -32.36% | -25.88% | 438.7% | 406.0% |
| Class C | 14.57% | 12.60% | 13.95% | 44.7% | 39.7% | 89% | -14.17% | -19.49% | -20.81% | 420.6% | 438.0% |
| Class E | 14.20% | 18.71% | 15.92% | 47.6% | 57.2% | 84% | -11.77% | -16.98% | -16.74% | 447.6% | 407.0% |
| Overall (Ref) | 13.95% | 14.78% | 15.08% | 45.1% | 44.0% | 83% | -12.79% | -24.23% | -21.91% | 434.7% | 416.6% |
| Class D | 11.98% | 11.24% | 13.23% | 40.8% | 40.5% | 94% | -17.58% | -22.21% | -22.39% | 408.1% | 483.2% |
| Class F | 18.53% | 16.60% | 17.60% | 50.4% | 48.9% | 87% | -18.72% | -31.81% | -31.34% | 399.0% | 342.7% |
| Class TGM | 19.22% | 25.26% | 26.76% | 57.0% | 45.4% | 84% | -30.13% | -39.69% | -39.31% | 416.3% | 273.0% |

  - Summary

The tool-off results on top of the recent ECM versions are summarized below for BD-PSNR-Y. Note that SCC results are not included.

- Issues

  - Resolved issues

- #113, Decoder mismatch and encoder crash in RA for the current master tip (d3d7d951) with AHG7 group3 off

- #112, Current master tip (d3d7d95150) decoding mismatch with AHG7 group1-5 off

- #87, ECM-15.0 decoder crashes when "InterLFNST=0"

- #77, Encoder crash in RA with --DepQuant=0 --NumSignPred=0

  - Open issues

- #91, MaxTU setting

- #80, Compilation Issue ECM14.0 when disabling GPM reordering

- #78, Encoder crash in RA with --LMChroma=0

- #71, Encode/decode mismatch when using single tree

- #66, Difference in encoding results of ECM-12.0 in AhG7 group 2 tool off test

- #65, Different encoding results of ECM12 in AHG7 group1-4 off tests

- #64, Encode/decode mismatch and decoder crash when inter-CCCM is disable

- #53, Decoding mismatch was observed when AMVR is off

- Input contributions

| JVET-AN0043 | m73918 | 2025-07-29 21:41:16 | 2025-07-31 23:57:14 | 2025-07-31 23:57:14 | AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 | X. Li (Google) |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AN0044 | m73927 | 2025-08-19 0:10:35 | 2025-08-19 19:59:14 | 2025-08-19 19:59:14 | Report of AHG7 conference call on Complexity Analysis | X. Li |
| JVET-AN0119 | m74144 | 2025-09-26 8:03:15 | 2025-09-26 11:30:54 | 2025-09-26 11:30:54 | AHG7: Analysis on group-off tests using Valgrind | R. Ishimoto, F. Zheming, T. Ikai (Sharp) |
| JVET-AN0203 | m74246 | 2025-09-26 14:58:02 | 2025-09-26 18:04:43 | 2025-09-26 18:04:43 | AHG7: On RGPM moved into group-2 | P. Bordes, K. Reuzé, E. Francois, K. Naser, F. Le Léannec (InterDigital) |
| JVET-AN0223 | m74273 | 2025-09-26 17:02:23 |  |  | AHG7: On bin to bit ratio in ECM | T. N. Canh, P. Yin, S. McCarthy (Dolby), J. N. Shingala (Ittiam) |
| JVET-AN0253 | m74304 | 2025-09-26 21:55:26 | 2025-09-26 22:06:04 | 2025-09-26 22:06:04 | AHG7: On Memory Bandwidth Measurement | X. Li (Google), Y. Kim (Samsung), J. Pardo (Huawei), R. Ishimoto (Sharp), L.-F. Chen (Tencent) |
| JVET-AN0317 | m74735 | 2025-10-02 19:25:25 | 2025-10-02 19:29:42 | 2025-10-02 19:29:42 | AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 – Update on IntraTMP | K. Naser, T. Dumas, M. Radosavljevic (InterDigital) |

- Recommendations

- Continue and improve tool assessment

- Resolve identified software issues related to the tool assessment

- Review all the input documents

Review of AHG7 related input documents should be conducted early – possible BoG to develop an out document on tool complexity criteria from JVET-AN0044 and other documents.

JVET-AN0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

The AHG 8 used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. Between the last and this meeting, there was no AHG8 email sent to the main reflector. There is one AHG8/AHG9 input contribution to this meeting. The current focus is on finalization of TR (v1).

  - Software and Common Test Conditions

AHG 8 related software and documents can be accessed at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects: common test conditions, reporting templates with anchor results, evaluation scripts and task networks are available in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc, and software implementation examples are hosted in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm. For this meeting cycle, common test conditions remain unchanged as described in output document JVET-AI2031.

  - Technical Report

There was no update on the Draft Technical Report (DTR), i.e., JVET-AL2030 “Optimization of encoders and receiving systems for machine analysis of coded video content (draft 9)”.

- Input contributions

There are two input contributions related to AHG 8 mandates (by the time this report is uploaded) including this report.

- JVET-AN0008	JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8)

- JVET-AN0208 	AHG8/AHG9: Depth-aware optimization for Encoder optimization information SEI message

- Recommendations

The AHG recommends to:

- Review input contributions.

- Continue improving the on-going TR document based on feedback towards finalization.

It was initially assessed that the input contribution on EOI SEI does not have impact on the way it is referred in the TR.

JVET-AN0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

A total of 89 contributions are identified relating to the mandates of AHG9. Some contributions relate to more than one mandate and/or SEI message. Some contributions also relate to the work of AHG8 and AHG11.

The following is a list of contributions related to the mandates of AHG9.

    - Study the SEI messages in VSEI, VVC, HEVC and AVC (4)

JVET-AN0071 AHG9: On HEVC Omnidirectional viewport SEI in HEVC [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0072 AHG9: On 3DRD, MAI and Alternative Depth information SEI messages in HEVC and VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0076 AHG9: On floating point signalling in SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0076 also relates to JVET-AM2006 (VSEI) and in JVET-AM2032 TuC for VSEI

JVET-AN0107 AHG9: On scalable-nested DSCI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0107 also relates to the DSC SEI messages in JVET-AM2032 TuC for VSEI

    - Study JVET-AM1006 HEVC (2)

JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AN0059 also relates to JVET-AM1017 (AVC), JVET-AM2005 (VVC), and JVET-AM2006 (VSEI)

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the NNPF, SPO, and DSC SEI messages

    - Study JVET-AM1017 AVC (2)

JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AN0059 also relates to JVET-AM1006 (HEVC), JVET-AM2005 (VVC) ), and JVET-AM2006 (VSEI)

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the NNPF, SPO, and DSC SEI messages

    - Study JVET-AM2005 VVC (3)

JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AN0059 also relates to JVET-AM1006 (HEVC), JVET-AM1017 (AVC), and JVET-AM2006 (VSEI)

JVET-AN0162 AHG9: On subpicture sub-bitstream extraction in VVC [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the NNPF, SPO, and DSC SEI messages

    - Study JVET-AM2006 VSEI (32)

    - Editorial (4)

JVET-AN0067 AHG9: Comments on Neural-network Post-filter in VSEI V4 and V3 [S. Deshpande (Sharp)]

JVET-AN0067 also relates to the NNPF SEI messages

JVET-AN0133 AHG9: Miscellaneous editorial changes for the VSEI v4 draft [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0156 AHG9: Text fixes and cleanup for GFV and GFE SEI messages [J. Chen, Y. Ye, B. Chen (Alibaba)]

JVET-AN0156 also relates to the GFV SEI messages

JVET-AN0186 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

    - Specification of syntax functions and descriptors (3)

JVET-AN0134 AHG9: On the SEI message payload extension in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0076 AHG9: On floating point signalling in SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0076 also relates to VSEI, VVC, HEVC and AVC

JVET-AN0242 AHG9: Signalling of floating-point values in VSEI and graphics rendering information SEI [V. Zakharchenko, J. Boyce (Nokia)]

JVET-AN0242 also relates to the GRI messages

    - Film grain characteristics SEI message (1)

JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

    - NNPF SEI messages (6)

JVET-AN0058 AHG9: On the NNPF SEI messages in VSEI version 4 [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AN0063 AHG9: On the NNPF SEI messages in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0064 AHG9: On the resampling constraint in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0064 also relates to the PRI SEI message

JVET-AN0067 AHG9: Comments on Neural-network Post-filter in VSEI V4 and V3 [S. Deshpande (Sharp)]

JVET-AN0067 also relates to editorial aspects of JVET-AM2006

JVET-AN0140 AHG9: On the image format metadata SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the SPO and DSC SEI messages and interface text in JVET-AM1006, JVET-AM1017, and JVET-AM2006

    - SEI processing order and processing order nesting SEI messages (5)

JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AN0059 also relates to the JVET-AM1006 (HEVC), JVET-AM1017 (AVC), and JVET-AM2005 (VVC)

JVET-AN0065 AHG9: On the SPO and PON SEI message in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0181 AHG9: On process SEI messages in processing chain in SPO SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 also relates to the EOI and SPTI SEI messages

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the NNPF and DSC SEI messages and interface text in JVET-AM1006, JVET-AM1017, and JVET-AM2006

    - Encoder optimization information SEI message (1)

JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 also relates to the SPO, PON, and SPTI SEI messages

    - Source picture timing information SEI message (1)

JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 also relates to the SPO. PON, and EOI SEI messages

    - Object mask information SEI message (3)

JVET-AN0060 AHG9: On the OMI SEI message in VSEI version 4 [M. M. Hannuksela, J. Boyce (Nokia)]

JVET-AN0135 AHG9: On the object mask information SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0161 AHG9: On OMI SEI message [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

    - Modality information SEI message (2)

JVET-AN0073 AHG9: On MI SEI message in VSEI version 4 [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0138 AHG9: On the modality information SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

    - Text description information SEI message (2)

JVET-AN0061 AHG9: On the text description information SEI message in VSEI version 4 [M. M. Hannuksela (Nokia)]

JVET-AN0139 AHG9: On the text description information SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

    - Generative face video SEI messages (2)

JVET-AN0141 AHG9: On the generative face video SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0156 AHG9: Text fixes and cleanup for GFV and GFE SEI messages [J. Chen, Y. Ye, B. Chen (Alibaba)]

JVET-AN0156 also relates to editorial aspects for JVET-AM2006

    - Digitally signed content messages (2)

JVET-AN0182 AHG9: On DSC SEI messages in VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the NNPF and SPO SEI messages and interface text in JVET-AM1006, JVET-AM1017, and JVET-AM2006

    - AI usage restrictions SEI message (2)

JVET-AN0062 AHG9: On the AI usage restrictions SEI message in VSEI version 4 [M. M. Hannuksela, F. Cricri, K. Kammachi Sreedhar (Nokia)]

JVET-AN0136 AHG9: On the AI usage restrictions SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

    - Packed regions information SEI message (4)

JVET-AN0064 AHG9: On the resampling constraint in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0064 also relates to the NNPFC SEI message

JVET-AN0180 AHG9: On PRI SEI message in processing chain [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0183 AHG9: On PRI SEI message in VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0310 AHG9: On PRI ID [Y. Li (SJTU), Y.-K. Wang (Bytedance), K. Yang, Y. Xu (SJTU)]

    - Image format metadata SEI message (1)

JVET-AN0137 AHG9: On the image format metadata SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

    - Study JVET-AM2032 TuC for VSEI (48)

    - Scalability dimension information SEI message (2)

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the FGRC, GRI, and PSI SEI messages

JVET-AN0265 AHG9: On confidence auxiliary layer design [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

    - Shutter interval information SEI message (1)

JVET-AN0145 AHG9: On the shutter interval information extension in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

    - NNPF SEI messages (3)

JVET-AN0068 AHG9: On Neural-network Post-filter Information Signalling and Semantics [S. Deshpande (Sharp)]

JVET-AN0176 AHG9: On multi-purpose NNPFs [C-H. Demarty, E. François, A. Ak (InterDigital)]

JVET-AN0264 AHG9: Support for implicit representations with the Gaussian splatting information SEI message [S. Lee, S. Sasse, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AN0264 also relates to the Gaussian splatting information SEI message

    - SEI processing order and processing order nesting SEI messages (1)

JVET-AN0108 AHG9: On the SPO SEI message extension complexity information [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

    - Encoder optimization information SEI message (1)

JVET-AN0208 AHG8/AHG9: Depth-aware optimization for Encoder optimization information SEI message [G. Teniou, S. Wenger, A. Hinds, J. Ricard]

    - Digitally signed content messages (5)

JVET-AN0078 AHG9: Digitally signed content (DSC): Comments on TuC [I. Sodagar, C. Fersch, S. McCarthy (Dolby)]

JVET-AN0107 AHG9: On scalable-nested DSCI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0107 also relates to the study of SEI messages in VSEI, VVC, HEVC and AVC

JVET-AN0187 AHG9: On DSC SEI messages for subpicture-based signing in TuC of VSEI [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0188 AHG9: On signing parameter sets in DSC SEI messages for subpictures in TuC of VSEI [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0210 AHG9: On subpicture signing in the DSC SEI messages [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

    - Film grain regions characteristics SEI message (3)

JVET-AN0069 AHG9/AHG13: Updates for the film grain regions SEI message [E. Francois, F. Urban, P. de Lagrange, D. Doyen (InterDigital), G. Teniou (Tencent)]

JVET-AN0146 AHG9: On the film grain regions characteristics SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the SDI, GRI, and PSI SEI messages

    - Constituent rectangles SEI messages (4)

JVET-AN0102 AHG9: On the CR SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0116 AHG9: On the colour mapping information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0116 also relates to the BRI, ECFI, and CMI SEI messages

JVET-AN0149 AHG9: On the constituent rectangles SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages [K. Abe, T. Nishi (Panasonic)]

JVET-AN0160 also relates to the DOI and DR SEI messages

    - Quality metrics SEI message (3)

JVET-AN0066 AHG9: On the QM SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0246 AHG9: Metric value range for quality metrics SEI [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

JVET-AN0247 AHG9: Quality metrics SEI for concatenated encoding [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

    - Lens optical correction SEI message (1)

JVET-AN0074 AHG9: On Lens Optical Correction SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

    - Display overlays information SEI message (7)

JVET-AN0104 AHG9: On the DOI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0105 AHG9: On applying display overlay on CR target picture [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0117 AHG9: On display overlays information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0148 AHG9: On the display overlays information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages [K. Abe, T. Nishi (Panasonic)]

JVET-AN0160 also relates to the CR and DR SEI messages

JVET-AN0207 AHG9: Overlay purpose indicator for Display overlays information SEI message [G. Teniou, S. Wenger, A. Hinds]

JVET-AN0248 AHG9: Overlay set purpose indicator for Display overlays information SEI message [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

    - Bitdepth range information SEI message (2)

JVET-AN0116 AHG9: On the colour mapping information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0116 also relates to the CR, ECFI, and CMI SEI messages

JVET-AN0147 AHG9: On the bitdepth range information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

    - Graphics rendering information SEI message (4)

JVET-AN0144 AHG9: On the graphics rendering information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0230 AHG9: on the GRI SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the SDI, FGRC, and PSI SEI messages

JVET-AN0242 AHG9: Signalling of floating-point values in VSEI and graphics rendering information SEI [V. Zakharchenko, J. Boyce (Nokia)]

JVET-AN0242 also relates to specification of syntax functions and descriptors

    - Enhanced colour information SEI message (2)

JVET-AN0116 AHG9: On the colour mapping information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0116 also relates to the CR, BRI, and CMI SEI messages

JVET-AN0143 AHG9: On the enhanced colour format information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

    - Display rectangles SEI message (2)

JVET-AN0124 AHG9: Additional features in display rectangles SEI message [R. Imichi, Y. Tokumo, T. Ikai (Sharp)]

JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages [K. Abe, T. Nishi (Panasonic)]

JVET-AN0160 also relates to the CR and DOI SEI messages

    - Danmu information SEI message (5)

JVET-AN0115 AHG9: On the danmu Information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0232 AHG9: on the Danmu SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0259 AHG9: Enable the regional display in the Danmu information SEI message [S. Xie, P. Wu, W. Niu, Y. Gao, C. Huang (ZTE)]

JVET-AN0261 AHG9: Comments on the Danmu information SEI message [S. Xie, P. Wu, W. Niu, Y. Gao, C. Huang (ZTE)]

JVET-AN0262 AHG9: On danmu information SEI message [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

    - Colour mapping information SEI message (4)

JVET-AN0116 AHG9: On the colour mapping information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0116 also relates to the CR, BRI, and ECFI SEI messages

JVET-AN0231 AHG9: on the CMI SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0244 AHG9: Prespecified colour maps for the colour mapping info SEI message [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

JVET-AN0245 AHG9: On colour mapping info SEI message [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

    - Auxiliary sampling alignment information SEI message (2)

JVET-AN0233 AHG9: On ASAI SEI Message applied to primary and auxiliary layers [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the SDI, FGRC, GRI, and PSI SEI messages

    - Localization and mapping information SEI message (4)

JVET-AN0075 AHG9: On Localization and Mapping SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0150 AHG9: On the localization and mapping SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AN0167 AHG9: On localization and mapping SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0260 AHG9: Suggested modifications for the LAM SEI [V. Zakharchenko, J. Boyce (Nokia)]

    - Collect software and showcase information for SEI messages (2)

JVET-AN0122 AHG9/AHG11: Demo of real-time NNPF inference with banding reduction [S. Schwarz, H. Sethi, J. Funnell, M. Santamaria, R. Yang, F. Cricri, M.M. Hannuksela (Nokia)]

JVET-AN0206 AHG9: Gaussian Splat compression experiment platform [J. Ricard, G. Teniou, S. Wenger, A. Hinds]

JVET-AN0206 also relates to identifying potential needs for additional SEI messages

    - Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (7)

    - Sample grouping SEI message (1)

JVET-AN0077 AHG9: Sample Grouping SEI message [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

    - Gaussian splatting information SEI message (4)

JVET-AN0100 AHG9: Gaussian splatting information SEI message [Y. He, J. Jung, L. Kerofsky, G. Van der Auwera, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang, L. Zhang (Bytedance Inc.)]

JVET-AN0205 AHG9: Gaussian Splat Information SEI message [J. Ricard, G. Teniou, S. Wenger, A. Hinds]

JVET-AN0206 AHG9: Gaussian Splat compression experiment platform [J. Ricard, G. Teniou, S. Wenger, A. Hinds]

JVET-AN0206 also relates to software and showcase information for SEI messages

JVET-AN0264 AHG9: Support for implicit representations with the Gaussian splatting information SEI message [S. Lee, S. Sasse, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AN0264 also relates to NNPF SEI messages

    - Spatial and bit depth restoration information SEI message (1)

JVET-AN0226 AHG9: Spatial and bit depth restoration SEI message [X. Xu, M. Xu, S. Liu (Tencent)]

    - Temporal resampling information SEI message (1)

JVET-AN0227 AHG9: Temporal resampling information SEI message [X. Xu, S. Liu (Tencent)]

    - Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. No emails with [AHG9] were exchanged.

    - Recommendations

The AHG recommends to:

- Review all related contributions; and

- Continue SEI messages studies.

It was emphasized again that,

“VSEI v4 is intended for FDIS and ITU-T consent at this meeting. There is a preference for design stability. Bug fixes and editorial improvements are OK. We intend to be cautious about adoptions that change the design. Small items with clear benefits may be considered.”

JVET-AN0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs)]

Following contributions were identified relating to AHG10 and summarized in the following sections.

    - JVET-AN0052: [AHG17] [ Performance of GOP based RPR under CfE test conditions

This document presents results for enabling GOP based RPR encoder control for VTM (option ‘--GOPBasedRPR=1’) on CfE test conditions. VVC standard supports RPR functionality, which allows changing the scaling factor for video during coding. When GOP based RPR is enabled the first picture in each GOP is down-sampled and up-sampled before coding. Decision on scaling factor is based on PSNR computed between pristine and distorted by down-/up-sampling. Scaling factor decided this way is used for the whole GOP. The BD-rate compared to corresponding anchor configuration is on average is reported to be about -5% for random access and -2% for low-delay configurations. The document proposes to enable GOP based RPR for CfE/CfP anchor.

    - JVET-AN0189: AHG17/AHG10: On perceptual coding for next-generation video coding standard

This document suggests considering perceptually oriented coding tools in the development of next-generation video coding standard. In the development of previous standards (e.g., VVC, HEVC), the CTC are mostly based on PSNR metric. This paradigm of developing a standard may have two limitations that perceptual tuning capability of the standard may not be well tested and verified, and CTC which emphasizes on PSNR may not encourage the development of normative perceptual optimization tools. The document recommends investigating in objective metrics with better correlation with human vision during the preparation of CfP, in AhG17 and perhaps with the help from AG5. The document also suggest that a call for objective quality metrics for video coding quality assessment may be issued to solicit more reliable metrics beyond what we have been familiar with.

    - JVET-AN0190: AHG17/AHG10: On test model simulating hardware encoder

This document recommends developing the next-generation video coding standard of a native design for hardware-friendly encoding. During previous standardization, hardware decoder complexity was emphasized more than hardware encoder complexity, partly because the standards specify the decoding process. A test model developed with an encoding configuration that mimics some key characteristics of hardware encoder (besides a configuration for simulating software encoder) may facilitate more accurate evaluation on the performance of coding tools in the context of hardware encoder implementation and solicit hardware encoding friendly coding technologies. The document also encourages to have experts with hardware encoder design expertise participating in the new standard and improving its hardware friendliness with their valuable comments and suggestions.

    - Recommendation

The AHG recommended that the related input contributions are reviewed, and to further continue the study of encoding algorithm optimizations in JVET.

Review of the related documents in the context of CfE.

JVET-AN0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs)]

    - Common Test Conditions

    - Anchor Encoding

Anchor for the NN-based video coding activity made available though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC-14-VTM_vs_NNVC-14.xlsm

also distributed by AhG14 in JVET-AN0014 and intermediate report JVET-AN0042

    - NNVC tools performance evaluation and bit-exact reproducibility of NN-based methods on various platforms

Several of CfE responses include elements of NNVC (as well as not yet in NNVC new algorithms).

JVET-AN0085 (test 3) reports 2.8% (RA) and 1.9% (LDB) gain from LOP6 added on top of ‘ECMx5’.

JVET-AN0198 reports 9.4% (SDR UHD, SDR HD, UGC) gain over ‘ECM default’ from just NN-based two technologies (CV-based reference frame and transformers-based filter).

JVET-AN0212 reports 9.0% (RA) gain over VTM23.11 with RPR enabled from LOP6 and NN-Intra from NNVC (enabled together of RPR).

JVET-AN0267 (sub 2) reports 3.2% (RA) gain over NextSoftware2 configured at 0.2 of VTM encoder speed, using VLOP-like in-loop filter.

Submission which includes elements from default configuration of NNVC (NN-Intra and LOP filter) typically were successfully decoded by cross-checker and md5sum of reconstructed YUV matched. But not default NNVC elements for various reasons caused problem during decoding or miss-match. From one hand side it is reported that with enough efforts robust decoding of NNVC tools is possible, but check of bit-exact reproducibility on different platform must become a part of evaluation process. Up-date of NNVC CTC can be recommended. A joint effort with AhG14 is recommended to address this topic.

    - EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available at this meeting as:

| JVET-AN0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators) |
| --- | --- | --- |

EE1 related contributions and cross-checks are summarized in the EE1 summary report.

    - Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on July 28 and August, 22. In those teleconferences, the following topics were discussed:

- NNVC14.0 and NNVC-14.1 software integration status

- NNVC14.0 anchor performance

- New RPR/NNSR “CTC” for reporting results

- Implementation of JVET-AM0175 DRF (deep reference frame)

- Hybrid plus E2E AI video codec framework development status report

- JVET-AM2019 finalization

- EE1 tests complexity parameters

- Potential NNVC-based CfE response

| JVET-AN0042 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- |

    - Performance Evaluation

The performance and complexity of NN-based tools available in NNVC software is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital, except content adaptive tools that are from Nokia.

There were no changes in default configuration on NNVC software. The main reason for slight reduction of performance and decreases of speed for NNVC-14 (compared to numbers reported for earlier version of NNVC) is a change of software base to VTM23.11, which is faster and slightly better performing. Note that NNVC-14 is compared to new anchor VTM-23.11 whereas previous versions were compared to VTM-11.0 based version. Overall, NNVC-14 is about 30% faster than NNVC-13 for a slight decrease of performance (less than 0.1%).

Performance of VLOP filter improved after attention module (JVET-AM0135) was added into design.

New NNVC tool – Deep Reference Frame (JVET-AM0175) was added into NNVC tool box.

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR or DRF | Total | Filter | Intra | SR or DRF |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 27 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-13.0 (LOP6) | -8.2% | -14.9% | -13.5% | 1.1 | 28 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-12.0 (LOP5) | -8.2% | -15.3% | -13.5% | 1.2 | 35 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-11.0 (LOP4) | -7.6% | -14.3% | -13.2% | 1.2 | 36 | 21.6 | 16.8 | 4.8 | 0 | 1.5 | 0.24 | 1.3 | 0 |
| NNVC-10.0 (LOP3) | -7.4% | -13.6% | -11.6% | 1.2 | 33 | 21.7 | 16.9 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NNVC-9.1(LOP3) | -7.3% | -13.1% | -11.3% | 1.2 | 81 | 24.8 | 16.9 | 7.9 | 0 | 1.7 | 0.21 | 1.5 | 0 |
| NNVC-8.0(LOP2) | -6.9% | -13.2% | -12.1% | 1.2 | 73 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NNVC-7.1(LOP2) | -6.9% | -13.2% | -12.1% | 1.3 | 86 | 25.0 | 17.1 | 7.9 | 0 | 1.6 | 0.05 | 1.5 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (HOP5) | -14.1% | -19.8% | -20.1% | 3.4 | 1180 | 471 | 466 | 4.8 | 0 | 2.7 | 1.41 | 1.3 | 0 |
| NNVC-10.0 (HOP5) | -14.2% | -19.5% | -19.9% | 2.6 | 1135 | 471 | 466 | 4.8 | 0 | 2.7 | 1.41 | 1.3 | 0 |
| NNVC-9.1(HOP4) | -14.1% | -19.2% | -19.6% | 2.9 | 1447 | 484 | 476 | 7.9 | 0 | 3.0 | 1.44 | 1.5 | 0 |
| NNVC-8.0(HOP3) | -13.7% | -13.9% | -14.5% | 2.5 | 1092 | 474 | 466 | 7.9 | 0 | 2.9 | 1.42 | 1.5 | 0 |
| NNVC-7.0(HOP2) | -13.6% | -12.5% | -14.2% | 4.1 | 2071 | 485 | 477 | 7.9 | 0 | 3.0 | 1.50 | 1.5 | 0 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (VLOP4) | -6.1% | -7.5% | -6.0% | 1.1 | 14 | 9.9 | 5.09 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| NNVC-11.0 (VLOP3) | -5.8% | -6.6% | -5.7% | 1.2 | 7 | 9.9 | 5.12 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| NNVC-10.0 (VLOP2) | -5.6% | -7.6% | -6.4% | 1.1 | 15 | 10 | 5.16 | 4.8 | 0 | 1.3 | 0.06 | 1.3 | 0 |
| NNVC-9.1(VLOP) | -5.3% | -5.4% | -5.2% | 1.2 | 40 | 13 | 5.12 | 7.9 | 0 | 1.5 | 0.02 | 1.5 | 0 |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC 14.0 caLOP4-decp | -8.6% | -15.7% | -16.3% | 2.3 | 25 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC 11.0 caLOP4 | -8.5% | -18.0% | -17.1% | 2.3 | 36 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC-10.0 caLOP3 | -8.2% | -16.8% | -15.5% | 2.1 | 33 | 22.1 | 17.3 | 4.8 | 0 | 1.5 | 0.21 | 1.3 | 0 |
| NN-Intra & VLOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-10.0 caVLOP | -6.3% | -7.8% | -6.6% | 2.2 | 34 | 10.2 | 5.4 | 4.8 | 0 | 1.3 | 0.02 | 1.3 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 NNSR | -8.6% | -13.7% | -12.5% | 1.1 | 24 | 26.1 | 16.8 | 4.8 | 4.7 | 1.6 | 0.05 | 1.3 | 0.2 |
| NNVC-14.0 RPR | -8.6% | -13.7% | -12.6% | 1.1 | 24 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-8.0 NNSR | -7.8% | -11.8% | -10.5% |  |  | 45.3 | 17.1 | 7.9 | 20.3 | 1.8 | 0.21 | 1.5 | 0.1 |
| NN-Intra & LOP filter & DRF (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 DRF | -9.8% | -15.9% | -14.2% | 1.7 | 604 | 441.4 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |

More details and analysis for tools and tools combination is expected in AhG14 report.

    - End-to-end AI coded reference picture

In between the two meetings intensive debugging process of end-to-end AI coded reference picture implementation into NNVC was conducted. It appears that two different ways of implementation are possible:

- So-called multi-layer (fully reuses syntax and referencing mechanism of multi-layer VVC, including all high-level syntax needed for base layer independent decoding and rendering)

- So-called ‘single layer’ (as it was proposed in JVET-AL0196), in which end-to-end AI coded picture is intended to be used only as reference, so there is no high-level syntax needed for base layer independent decoding and rendering is required.

Two approaches converged and achieved the status that performance difference now is only high-level syntax of base layer. Decision about which approach to chose can be done only after discussion during the JVET meeting. It probably makes sense to keep both since they have different functionality and address different use cases, but align them in order to reduce deployment cost.

Related contributors need to be discussed together

| JVET-AN0202 | [AHG11/AHG14] Comparison of Multi-Layer and Single-Layer Interfaces for Hybrid End-to-End Video Coding Frameworks | N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A. B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia) |
| --- | --- | --- |
| JVET-AN0216 | [AHG11] Update on Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec | F. Urban, Y. Chen, F. Galpin, P. de Lagrange (InterDigital) |
| JVET-AN0224 | [AHG11] A Hybrid Framework Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A.B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AN0225 | [AHG11][AHG14]: Further Improvements on Hybrid Multilayer Framework for End-to-End Learned Intra Frame | M. Aderdor, T. Solovyev, E. Alshina (Huawei) |

    - NNVC implementation aspects

Some trial implementation of NN-based filters on various devices are discussed in two contributions.

| JVET-AN0122 | AHG9/AHG11: Demo of real-time NNPF inference with banding reduction | S. Schwarz, H. Sethi, J. Funnell, M. Santamaria, R. Yang, F. Cricri, M.M. Hannuksela (Nokia) |
| --- | --- | --- |
| JVET-AN0257 | [AHG11] CPU and GPU time comparison for NNVC models | A. Karabutov, E.Alshina (Huawei) |

    - Test conditions

Some practical suggestions on NNVC tools testing are done following two contributions

| JVET-AN0047 | EE1-related: Automated environment information collection | N. Le, M. Santamaria, H. Zhang, F. Cricri (Nokia) |
| --- | --- | --- |
| JVET-AN0301 | AhG17 Sharing experience of conducting JPEG AI Call for Proposals | E.Alshina |

    - New design elements proposal

During NNVC-based CfE response preparation several encoders only modifications which substantially reduce decoder complexity have been found. Also, four new proposals targeting NNVC performance improvement are submitted.

| JVET-AN0172 | AHG11: Proposal of normative TDO method as an alternative option in NNVC | H. Kwon, H. Ko (Hanyang Univ.), D. Kim, S.-C. Lim (ETRI) |
| --- | --- | --- |
| JVET-AN0215 | AhG11: Filter restriction depending on hierarchical depth | F. Galpin, E. François (InterDigital) |
| JVET-AN0255 | AHG11: VLOP3 with new backbone block based on Spatial-Channel Mixing (SCM) | H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI) |
| JVET-AN0285 | AHG11: In-loop filter with dynamic feature guidance for NNVC | X.-T. Xie, W.-X. He, T. Yang, Y.-Q. Zhu, J.-S. Gong, Q.-M. Wang, Q. Liu (HUST), Z.-Y. Lv (vivo) |
| JVET-AN0209 | AhG 11/AHG 14: Harmonization of Deep Reference Frame (DRF) with RPR in NNVC14.1 | N. Bhaskar, T. Solovyev, E. Alshina (Huawei) |
| JVET-AN0319 | EE1-related: FlowWarp Operator for DRF Integer Inference Optimization | N. Fu, X. Chen, L. Qin, W. Zhang, Z. Chen (Wuhan Univ.) |

    - Recommendations

The AHG recommended:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Discuss and make decision on ‘multi-layer’ and ‘single layer’ implementations of hybrid codec using End-to-End AI coded pictures.

- Discuss device interoperability and decoder complexity of NNVC elements in CfE submissions.

- Recommend promising technologies for next EE1 round.

It was commented that, at this exploration stage of study, it may be less important how high-level enabling such functionality is achieved.

It was also commented that the term “hybrid” has a much wider use in describing an architecture that mixes conventional and NN-based technology.

JVET-AN0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

    - Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AM2024). The combined improvements of the ECM-18.0 over VTM-11.0ecm18.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.82% | -16.49% | -27.44% | 1202.2% | 577.9% |
| Class A2 | -21.47% | -24.85% | -29.07% | 1119.5% | 624.0% |
| Class B | -15.06% | -23.15% | -21.18% | 1086.0% | 627.5% |
| Class C | -15.19% | -12.08% | -13.23% | 1026.3% | 583.6% |
| Class E | -19.58% | -23.35% | -21.51% | 992.5% | 615.0% |
| Overall | -16.87% | -19.90% | -21.83% | 1080.0% | 606.4% |
| Class D | -12.92% | -8.80% | -9.64% | 1013.4% | 663.9% |
| Class F | -30.84% | -34.56% | -34.60% | 688.3% | 658.9% |
| Class TGM | -43.72% | -49.31% | -48.60% | 513.1% | 658.7% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -27.52% | -25.12% | -37.22% | 1205.3% | 1195.4% |
| Class A2 | -30.81% | -34.98% | -40.50% | 1103.1% | 1346.4% |
| Class B | -25.42% | -34.33% | -31.10% | 999.4% | 1197.0% |
| Class C | -27.12% | -24.29% | -25.03% | 1064.9% | 1294.1% |
| Class E |  |  |  |  |  |
| Overall | -27.37% | -29.94% | -32.58% | 1076.3% | 1250.9% |
| Class D | -27.81% | -24.52% | -25.81% | 1003.1% | 1417.7% |
| Class F | -33.60% | -36.80% | -37.37% | 851.2% | 853.5% |
| Class TGM | -43.07% | -48.90% | -48.87% | 693.7% | 676.2% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -22.61% | -40.61% | -35.58% | 976.7% | 1030.5% |
| Class C | -25.00% | -28.31% | -30.38% | 937.3% | 1074.8% |
| Class E | -22.74% | -29.72% | -28.28% | 965.0% | 712.7% |
| Overall | -23.44% | -33.79% | -32.02% | 960.5% | 953.0% |
| Class D | -26.46% | -29.97% | -31.37% | 938.9% | 1175.1% |
| Class F | -31.37% | -41.12% | -41.47% | 859.7% | 808.4% |
| Class TGM | -41.36% | -51.49% | -51.48% | 691.5% | 643.3% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-17.0’s {-27.27%, -29.46%, -32.22%} to ECM-18.0’s {-27.37%, -29.94%, -32.58%}. For SCC sequences (class TGM) the rate reduction for RA configuration increased from ECM-16.1’s {-42.78%, -48.57%, -48.43%} to ECM-17.0’s { -43.07%, -48.90%, -48.87%}.

    - Contributions

In addition to 24 EE2 contributions, 42 (comparing to 41 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

    - Intra (16)

JVET-AN0110, "Non-EE2: Enhancement of Non-CCP Reordering List", Z. Liu, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AN0123, "Non-EE2: Improvement on chroma MPM", Z. Li, W. Niu, X. Zeng, M. Jia, Y. Wang, C. Huang (ZTE)

JVET-AN0126, "Non-EE2: MPDIP clipping", K.-Y. Kim, J.-H. Son, J.-Y. Kim, J.-S. Kwak (WILUS)

JVET-AN0131, "Non-EE2: On interpolation filter for SGPM	J. Lee, G. Moon, K. Kim, J.-G. Kim (KAU), J. Lee, S.-C. Lim (ETRI)

JVET-AN0132, "Non-EE2: Enhancement of MDIP mode selection", K. Kim, G. Moon, J. Lee, J.-G. Kim (KAU), J. Lee, S.-C. Lim (ETRI)

JVET-AN0152, "Non-EE2: Multiple Candidate Selection for MDIP", L. Wang, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)

JVET-AN0154, "Non-EE2: Position-dependent Weighted Prediction Extension", Y. Wang, X. Zeng, Z. Li, M. Jia, C. Huang (ZTE)

JVET-AN0170, "Non-EE2: Additional OBIC candidates in MPM list", Y. Kim, S. Lee, S. Noh, H. Choi (HNU), W. Lim, S.-C. Lim (ETRI)

JVET-AN0192, "Non-EE2: Modification of reconstructed area for EIP", W. Niu, S. Xie, Z. Li, M. Jia, Y. Bai, C. Huang (ZTE)

JVET-AN0194, "Non-EE2: Extension on TMRL mode", Y. Liu, Y. Huo, Z. Zhang (Transsion)

JVET-AN0219, "AHG12 IntraTMP with DMVR", K. Naser, P. Bordes, F. Le Léannec, A. Robert (InterDigital)

JVET-AN0221, "AHG12: Reference sample generation for intra prediction", T. N. Canh, P. Yin, S. McCarthy (Dolby)

JVET-AN0234, "Non-EE2: On filtering condition for angular modes", Z. Zhu, Y. Yin, H. Xu, S. Wan (NWPU), S. Xie, X. Zeng, C. Huang (ZTE)

JVET-AN0286, "Non-EE2: TMRL Angle Offset Refinement", Gagan Rath, Karam Naser, Fabrice Le Leannec, Thierry Dumas (InterDigital)

JVET-AN0287, "Non-EE2: Combined Angular- and Gradient-PDPC", Gosala Kulupana, Saverio Blasi, Nicolas Neumann, Jani Lainema (Nokia)

JVET-AN0299, "Non-EE2: Improvement for candidates on MPM", W. Niu, Z. Li, S. Xie, M. Jia, Y. Bai, C. Huang (ZTE)

    - Inter (7)

JVET-AN0111, "Non-EE2: additional candidates for regular inter AMVP candidate list", C. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AN0155, "Non-EE2: On improving adaptive GPM blending", Z. Zhang, Y. Huo, Y. Liu (Transsion)

JVET-AN0191, "Non-EE2: Binarization Improvement of GPM", X. Wang, J. Chen, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)

JVET-AN0200, "Non-EE2: On improving CIIP Fusion", Y. Wang, X. Zeng, M. Jia, Z. Li, C. Huang (ZTE)

JVET-AN0218, "Non-EE2: Extension of Sub-Block Merge Mode Availability Conditions", Y. Liu, Y. Huo, Z. Zhang (Transsion)

JVET-AN0266, "Pairwise Merge Candidates with Motion Vector Scaling", S. Hong, L. Wang, K. Panusopone, D. Rusanovskyy (Nokia)

JVET-AN0228, "Independent CTUs for TM-padding", N. Neumann (Nokia)

    - Cross Component Prediction (1)

JVET-AN0270, "Non-EE2: CCCM with clipping", L.-C. Xu, C.-F. Liao, Y.-K. Lu, F. Liang (SYSU)

    - In Loop Filters (7)

JVET-AN0086, "AHG12: Bug Fix for ALF Coefficient Calculation", R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)

JVET-AN0113, "Non-EE2: Additional candidates for merge mode in ALF-CCCM", H. Zhang, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AN0114, "Non-EE2: Local boosting CCP for ALF-CCCM", H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AN0130, "Non-EE2: Improvement on Frame-level Inheritance Mode in ALF-CCCM", M. Jia, Y. Bai, X. Zeng, Z. Li, C. Huang (ZTE)

JVET-AN0163, "AHG12: On improving the bi-filtering modes in TALF", P. Astola, D. Buğdayci Sansli, J. Lainema (Nokia)

JVET-AN0164, "Non-EE2: ALF coefficient coding group extension", I. Jumakulyyev, D. Buğdayci Sansli, J. Lainema (Nokia)

JVET-AN0269, "AHG12: BIF with before DBF samples", K. Takada, S. Deshpande (Sharp)

    - Coefficients Quantization and Coding (6)

JVET-AN0087, "AHG12: Budget Control for Transform Skip Residual Coding", R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)

JVET-AN0088, "AHG12: Context Modeling for sig_coeff_flag in TSRC", R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)

JVET-AN0109, "EE2-4.1-related: On Linked Sign Prediction", Y. Zhang, L. Xu, Y. Yu, J. Gan, H. Yu, D. Wang (OPPO)

JVET-AN0112, "Non-EE2: Coding of a coefficient level in TSRC", Y. Yu, L. Xu, J. Gan, H. Yu, D. Wang (OPPO)

JVET-AN0125, "Non-EE2: On CABAC bin budget constraint", K.-Y. Kim, J.-H. Son, J.-Y. Kim, J.-S. Kwak (WILUS)

JVET-AN0175, "EE2-related: Unbiased Simple Quantizer (Related to EE2-4.2i)", M. Balcilar, M. Blestel, P. Andrivon (Ofinno)

    - Entropy Coding (1)

JVET-AN0214, "AhG12: CABAC contexts retraining", F. Galpin (InterDigital)

    - Transform (2)

JVET-AN0169, "Non-EE2: On inter multiple transform set selection and advanced SBT", J. Huo, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)

JVET-AN0179, "EE2-2.1-related: Multiple Transform Set Selection for TMRL blend", W. Feng, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)

    - Partitioning (1)

JVET-AN0211, "AhG12: Second reference frame for temporal partitioning prediction", G. Laroche, P. Onno, B. Galmiche (Canon)

    - Other (1)

JVET-AN0089, "AHG12/AHG18: On GDR test condition for video compression beyond VVC", T. Chujoh, Y. Kidani, K. Kawamura (KDDI)

    - Recommendations

The AHG recommended:

- To review all the related contributions.

JVET-AN0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

- Discussion

This period was reportedly slow, with no meetings or discussions held for this AHG.

From the previous meetings, the following five items should be in the second edition:

- Updated Technical Report (TR) text to fix errors and clumsy wording

- Updated tools for the TR

- Video test sequences specific to film grain to be added to the TR

- Metrics – discussion and/or survey of available techniques or metrics

- Subjective testing of Film Grain modeling methods

The following topics are worthy of discussion for the second edition:

- Conformance – this is interesting due to Film Grain modeling being associated with SEI messages

- Film Grain Analysis methods– preprocessing analysis (parameter estimation)

The following are additional topics that could also be considered and discussed at likely lower priority:

- EG/RP documents providing guidance or usage recommendations for film grain technologies– The group is still considering what could be useful information for the industry and the scope of such documents

- Future parameters and signalling – not a v2 topic

- Related contributions

Five contributions related to AHG13 were identified as of 06/26/2025.

- One contribution was the AHG report:

  - JVET-AN0013 JVET AHG report: Film grain technologies (AHG13)

- Four other contributions were uploaded at the time of the report drafting:

  - JVET-AN0069 AHG9/AHG13: Updates for the film grain regions SEI message

  - JVET-AN0146 AHG9: On the film grain regions characteristics SEI message in TuC of VSEI

  - JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4

  - JVET-AN0237 AHG13: Film grain analysis improvement

  - Contributions

There were four contributions registered other than the AHG report.  All four were uploaded as of 10/02/25.

    - JVET-AN0069 AHG9/AHG13: Updates for the film grain regions SEI message

This contribution proposes updates for the film grain regions (FGR) SEI message (included in the technologies under consideration for future extensions of VSEI).

The Film Grain Regions Characteristics (FGR) SEI message supports three FG synthesis modes: per picture, per picture modulated by an alpha map, per region. In the region mode, the film grain model and  film grain parameters may be changed per region. Snapshots of the region-based and alpha-map modes are provided below for illustration.

The current film grain regions (FGR) SEI message syntax can be split into different parts, corresponding to information on:

- The resolution at which the grain should be applied

- The FG mode, defined by

  - the number of signaled FG models (1 or 2)

  - the spatial mode (picture-level, region-level, picture-level with alpha-belnding) used for the FG synthesis

- Colour description information

- FG parameters

- Regions parameters.

    - JVET-AN0146 AHG9: On the film grain regions characteristics SEI message in TuC of VSEI

This contribution proposes the following changes for the film grain regions characteristics (FGR) SEI message:

- Remove the ChromaFormatIdc condition in the interface text.

- Add two constraints for fgr_matrix_coeffs.

- Change the semantics of fgr_comp_model_present_flag[ m ][ c ] when it is equal to 0 to reflect the actual process.

- Add a sensibility constraint for fgr_intensity_interval_upper_bound[ m ][ c ][ i ] and fgr_intensity_interval_lower_bound[ m ][ c ][ i ].

- Add inference for fgr_comp_model_value[ m ][ c ][ i ][ 0 ] when it is not present.

    - JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4

This contribution proposes the following changes for the film grain characteristics (FGC) SEI message:

- Move the payload extension inside the if( ) loop for fg_characteristics_cancel_flag.

- Remove the ChromaFormatIdc condition in the VSEI interface text

- Add two notes on the presence of fg_matrix_coeffs

- Change the semantics of fg_comp_model_present_flag[ c ] when it is equal to 0 to reflect the actual process.

- Add a sensibility constraint for fg_intensity_interval_upper_bound[ c ][ i ] and fg_intensity_interval_lower_bound[ c ][ i ].

- Add inference for fg_comp_model_value[ c ][ i ][ 0 ] when it is not present.

- Various editorial changes.

    - JVET-AN0237 AHG13: Film grain analysis improvement

This work presents several enhancements to the film grain analysis process. They include:

- Improved cut-off frequency estimation where cut-off frequencies are now estimated by accumulating data over frames.

- Implementing software to limit the number of different cut-off frequencies (different cut-off pairs) within the frame

Experiments were carried out on different film grain sequences, and the visual evaluation demonstrated that the estimated film grain parameters result in a higher fidelity representation of the film grain compared to the source.

- Recommendations

The AHG recommended:

- Continue editing the second edition for the TR;

- the related input contributions (non-AHG9) to be reviewed;

- any liaisons to be reviewed;

- to continue conformance discussion;

- to consider any newly proposed SEI message extensions;

- to discuss the possibility of creating as appropriate documents providing Engineering Guideline (EG) or recommended practices (RP) relating to film grain technologies and their applications; and

- to continue the study of film grain technologies in JVET.

A potential for issuing a new version JVET-AN2020 based on JVET-AN0237 was noted.

JVET-AN0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs)]

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM in the NNVC branch.

NNVC software is based on latest VTM VTM-23.11 at the time of software integration.

Legacy version (up to NNVC-13.0) can be found in branch VTM-11.0_nnvc and contains history of development of tools integrated in NNVC-14.0.

NNVC-14.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc is used for NNVC performance evaluation.

  - Software changes

    - Current NNVC

Several commits were merged in the NNVC repository. The following changes were integrated:

| MR | Comments |
| --- | --- |
|  | new official branch NNVC (default branch) |
|  | VTM-11.0_nnvc dis deprecated: no merge is possible |
| 332 | JVET-AM0135 VLOP4 integration |
| 321 | adaptive nnlf training scripts have been merged |
| 329 | cherry picking VTM 23.10 |
| 337 | JVET-AM0170 sadl v14 update |
| 347 | sadl v14.1 (fix + converter update) |
| 325 | nnpf models |
| 338 | NNSR porting |
| 334 | cherry picking VTM 23.11 |
| 340 | JVET-AM0050 merging of branch hybrid_codec_e2e_dev |
| 345/349/354 | set nnsr/rpr policy for official results |
| 346 | fix RPR memory leak |
| 342 | inter tools JVET-AM0175 training |
| 343 | inter tools JVET-AM0175 models |
| 341 | inter tools JVET-AM0175 nnvc code |
| 356/358/357 | clean up DRF inference code |
| 351 | fix single layer default mode for hybrid |
| 344/350 | add scale information in log for rpr/nnsr mode |
| 352 | fix vtm mode build |
| 360 | nnpf nnvc sei code |
| 352 | fix build macro off |
| 362 | fix hybrid e2e chunk |
| 365 | mv zero for hybrid mode |

Additionally, several fixes were done on VTM default mode, allowing a running time closer to vanilla VTM, several fixes in hybrid mode, etc.

In SADL v14, the following changes were integrated:

| SADLa |  |  |  |
| --- | --- | --- | --- |
| Status | MR | Comments |
| DONE | 153 | JVET-AM0178 new layer pad |
| DONE | 154 | JVET-AM0178 speed-up conv2d 2x2 |
| DONE | 155 | JVET-AM0136 speed up lop6 |
| DONE | 156 | fix macro |
| DONE | 158 | fix build no saturation |
| DONE | 160 | fix speed up lop6 |
| DONE | 161 | add option to converter for DRF |
| DONE | 162 | fix build clang |

Additionally, several bug fixes were integrated, notably on the float inference regression in v14.

A branch allowing dataset extension has been created to ease the process of dataset creation ( see https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-6.1_aom_dataset?ref_type=heads )

A new branch “NNVC_cfe” used to generate results of CfE has been created. It contains the following non-normative changes:

- Choose NNLF activation per hierarchical depth (see JVET-AN0215)

- Set RPR policy to VTM mode

- Allows configuration file-based TDO parameters

During integration of new VTM and CfE tests, several issues were found and need to be addressed:

- Interaction of RPR and DRF

- Possible issues using HOP5 with high QP

- Possible interaction of NNSR and FGS

- NNSR policy for LDB not well handled

- NNSR/RPR policy to clean up

  - Software overview

Between the 2 meetings, a joint effort was made to support new features such as hybrid end-to-end coding and also to rebase the NNVC software on the newest VTM 23.11. The result is now the official NNVC software.

All NN tools of NNVC-13 and adoptions have been ported:

- NNLF: NN based Loop-Filter, support for latest VLOP, LOP and HOP, CALOP

- NNSR: NN based super-resolution

- NNIP: NN based Intra Prediction

- NNPF: NN based Post-Filter

- DRF: NN based Deep Reference Frame

A new framework for Hybrid end-to-end coding of intra slice has been integrated.

The master branch is kept synchronized with the upstream repository (VTM).

    - Status of the repository

The following diagram shows the status of the repository during the integration process.

The following diagram shows the current status of the repository.

  - NNVC-14

  - Software version

NNVC-14.1 was tagged 28th August, 2025

NNVC-14.0 was tagged July 28th, 2025 (contains VLOP4, DRF, hybrid e2e)

NNVC-13 was tagged April 28th, 2025 (contains LOP6)

NNVC-12 was tagged February 6th, 2025 (containing final LOP5 model)

NNVC-12rc1 was tagged February 5th, 2025 (containing new intermediate LOP5 model)

NNVC-11.0 was tagged December 10th, 2024 (bit accurate with 11rc)

NNVC-11.0rc was tagged December 3rd, 2024

NNVC-10.0 was tagged August 9th, 2024

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

Overall, the new VTM-23.11 base allowed a speedup of about 30% of the encoding runtime for minor decrease in performance.

Moreover, the reduced runtime configurations allow to target an encoding runtime between 116% and 26% compared to VTM in RA configuration, for gains between {-8.01%, -14.92%, -13.53%} and {0.38%,-5.60%,-2.98%} for NNVC anchor.

Using more complex tools, i.e. HOP5 and DRF on top of NNVC-14 allows gains of {-15.60%,-20.55%,-20.70%} for an encoding complexity of 378% for RA configuration.

Note: See the configurations section for the naming convention.

  - Comparison to VTM

    - NNVC-13.0 VTM vs NNVC-14.0 VTM

Note: NNVC-14 is now based on VTM-23.11.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 13 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.13% | 0.17% | 0.03% | -0.39% | 0.16% | -0.08% | 67% | 94% |
| Class A2 | 0.02% | 0.24% | 0.48% | -0.21% | 0.03% | 0.35% | 70% | 95% |
| Class B | -0.04% | 0.11% | 0.13% | -0.31% | 0.16% | 0.21% | 71% | 99% |
| Class C | -0.35% | 0.15% | 0.18% | -0.41% | 0.33% | 0.23% | 72% | 103% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.13% | 0.16% | 0.19% | -0.33% | 0.18% | 0.18% | 70% | 98% |
| Class D | -0.74% | -0.30% | -0.07% | -0.46% | 0.57% | 1.07% | 78% | 103% |
| Class F | 0.38% | 0.49% | 0.84% | 0.28% | 0.41% | 0.69% | 73% | 102% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 13 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.03% | 0.29% | -0.28% | -0.41% | 0.30% | -0.66% | 70% | 92% |
| Class C | -0.12% | -0.01% | 0.33% | -0.31% | 0.00% | 1.62% | 74% | 95% |
| Class E | -0.46% | 0.22% | 0.59% | -0.93% | 0.59% | -0.21% | 70% | 97% |
| Overall | -0.17% | 0.18% | 0.14% | -0.51% | 0.27% | 0.21% | 71% | 94% |
| Class D | -0.23% | 0.11% | -0.53% | -0.47% | 0.10% | -0.55% | 79% | 96% |
| Class F | 0.34% | 0.50% | 0.40% | 0.02% | 0.90% | 0.21% | 71% | 94% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 13 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.67% | 0.11% | -0.29% | 0.76% | -0.15% | 66% | 95% |
| Class C | -0.02% | 0.17% | 0.97% | -0.23% | 0.71% | 2.09% | 71% | 97% |
| Class E | -0.19% | 0.60% | 0.34% | -0.72% | 0.79% | -0.50% | 68% | 98% |
| Overall | -0.04% | 0.49% | 0.45% | -0.38% | 0.75% | 0.51% | 68% | 96% |
| Class D | -0.03% | 0.13% | -0.08% | -0.65% | 0.09% | -0.06% | 77% | 98% |
| Class F | 0.43% | 0.76% | 0.20% | 0.30% | 1.10% | 0.23% | 70% | 95% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 13 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.62% | -0.66% | 0.42% | -0.59% | -0.41% | 0.78% | 79% | 97% |
| Class A2 | -0.60% | -0.50% | -0.50% | -0.58% | -0.42% | -0.40% | 83% | 97% |
| Class B | -0.69% | -0.41% | -0.35% | -0.85% | -0.19% | -0.20% | 84% | 100% |
| Class C | -0.43% | -0.63% | -0.53% | -0.83% | -0.26% | -0.26% | 88% | 102% |
| Class E | -0.36% | 0.12% | 0.38% | -0.60% | 0.27% | 0.30% | 83% | 100% |
| Overall | -0.55% | -0.43% | -0.17% | -0.72% | -0.20% | 0.00% | 84% | 99% |
| Class D | -0.40% | -0.47% | -0.17% | -0.98% | -0.41% | 0.35% | 90% | 107% |
| Class F | -0.12% | -0.69% | -1.05% | -0.08% | -0.39% | -0.25% | 85% | 101% |

Notes: results by InterDigital, crosschecked by xxx.

    - NNVC-14.0 VTM vs NNVC-14.0 anchor

Anchor: NNVC in VTM mode.

Test: NNLF LOP6 + NNIP.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.98% | -13.88% | -15.28% | -8.82% | -16.00% | -17.48% | 117% | 2264% |
| Class A2 | -8.31% | -14.98% | -9.57% | -8.42% | -15.43% | -9.57% | 116% | 2197% |
| Class B | -7.71% | -15.96% | -14.70% | -7.80% | -18.20% | -17.56% | 117% | 2965% |
| Class C | -7.43% | -14.34% | -13.72% | -7.62% | -14.96% | -15.20% | 114% | 3103% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.01% | -14.92% | -13.53% | -8.08% | -16.34% | -15.31% | 116% | 2678% |
| Class D | -7.94% | -13.68% | -12.50% | -6.81% | -15.31% | -13.82% | 108% | 3133% |
| Class F | -4.36% | -8.60% | -7.33% | -5.40% | -11.14% | -11.46% | 122% | 1541% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -5.90% | -10.86% | -10.00% | -6.14% | -10.75% | -12.09% | 109% | 2903% |
| Class C | -6.03% | -11.92% | -9.96% | -6.73% | -10.60% | -8.78% | 103% | 3206% |
| Class E | -6.48% | -4.86% | -8.42% | -7.68% | -4.57% | -6.24% | 115% | 2892% |
| Overall | -6.09% | -9.71% | -9.59% | -6.72% | -9.16% | -9.52% | 109% | 2998% |
| Class D | -6.80% | -10.02% | -6.71% | -6.35% | -6.01% | -3.24% | 101% | 3098% |
| Class F | -3.61% | -6.13% | -5.18% | -5.59% | -7.08% | -8.54% | 113% | 1761% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -6.47% | -13.30% | -12.04% | -6.51% | -14.04% | -14.97% | 115% | 2838% |
| Class C | -6.45% | -14.40% | -12.76% | -6.93% | -14.41% | -13.72% | 106% | 3111% |
| Class E | -7.40% | -6.56% | -8.74% | -8.54% | -7.28% | -8.30% | 122% | 3165% |
| Overall | -6.70% | -11.98% | -11.45% | -7.16% | -12.47% | -12.89% | 113% | 3007% |
| Class D | -7.32% | -11.59% | -9.54% | -6.39% | -8.50% | -8.43% | 101% | 3186% |
| Class F | -3.86% | -7.62% | -5.46% | -5.49% | -10.41% | -11.80% | 118% | 1774% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.32% | -15.82% | -17.29% | -9.08% | -19.30% | -19.74% | 165% | 2617% |
| Class A2 | -7.74% | -15.36% | -11.33% | -8.26% | -16.09% | -11.47% | 164% | 2210% |
| Class B | -7.67% | -15.11% | -15.80% | -7.80% | -17.73% | -18.38% | 163% | 2206% |
| Class C | -7.79% | -13.37% | -13.30% | -7.99% | -16.94% | -17.03% | 150% | 1750% |
| Class E | -11.32% | -16.94% | -18.34% | -11.54% | -17.52% | -19.77% | 159% | 2514% |
| Overall | -8.59% | -15.19% | -15.17% | -8.76% | -17.51% | -17.39% | 160% | 2204% |
| Class D | -7.68% | -11.58% | -12.23% | -7.11% | -16.07% | -16.69% | 144% | 1566% |
| Class F | -5.54% | -9.46% | -8.35% | -5.99% | -12.65% | -12.64% | 133% | 1871% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-14 VTM mode vs NVC-14 reduced-runtime1

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.01% | -12.69% | -13.98% | -7.89% | -14.85% | -16.32% | 71% | 2297% |
| Class A2 | -7.01% | -13.32% | -7.91% | -7.18% | -13.79% | -7.95% | 72% | 2227% |
| Class B | -6.62% | -14.65% | -13.22% | -6.90% | -17.00% | -16.07% | 72% | 3041% |
| Class C | -6.04% | -12.58% | -11.80% | -6.43% | -13.38% | -13.46% | 67% | 3205% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.82% | -13.44% | -11.93% | -7.03% | -14.96% | -13.80% | 70% | 2739% |
| Class D | -6.85% | -12.12% | -11.08% | -5.92% | -14.05% | -12.89% | 70% | 3149% |
| Class F | -3.22% | -7.29% | -5.73% | -4.43% | -9.99% | -10.11% | 90% | 1610% |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.32% | -15.82% | -17.30% | -9.08% | -19.30% | -19.75% | 157% | 2622% |
| Class A2 | -7.74% | -15.36% | -11.34% | -8.26% | -16.10% | -11.47% | 161% | 2204% |
| Class B | -7.67% | -15.11% | -15.80% | -7.80% | -17.73% | -18.38% | 159% | 2194% |
| Class C | -7.80% | -13.37% | -13.31% | -8.00% | -16.95% | -17.04% | 150% | 1763% |
| Class E | -11.32% | -16.94% | -18.34% | -11.54% | -17.52% | -19.77% | 155% | 2442% |
| Overall | -8.59% | -15.19% | -15.18% | -8.76% | -17.51% | -17.39% | 156% | 2193% |
| Class D | -7.68% | -11.58% | -12.23% | -7.11% | -16.07% | -16.69% | 144% | 1590% |
| Class F | -5.54% | -9.46% | -8.35% | -6.00% | -12.65% | -12.65% | 134% | 1909% |

    - NNVC-14 VTM mode vs NVC-14 reduced-runtime2

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -5.29% | -11.22% | -11.25% | -4.91% | -13.32% | -13.61% | 46% | 2275% |
| Class A2 | -4.44% | -11.84% | -6.09% | -4.37% | -12.07% | -5.90% | 47% | 2206% |
| Class B | -4.17% | -13.58% | -12.22% | -4.35% | -15.75% | -14.83% | 46% | 3028% |
| Class C | -4.19% | -11.73% | -10.72% | -4.17% | -12.16% | -11.91% | 42% | 3189% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.45% | -12.27% | -10.40% | -4.42% | -13.57% | -12.02% | 45% | 2722% |
| Class D | -5.56% | -11.76% | -10.49% | -4.23% | -13.59% | -11.55% | 41% | 3127% |
| Class F | 0.49% | -5.53% | -4.14% | -0.73% | -8.25% | -8.16% | 57% | 1623% |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.91% | -14.51% | -16.38% | -6.90% | -18.38% | -19.21% | 81% | 2551% |
| Class A2 | -5.45% | -14.66% | -10.56% | -6.17% | -15.42% | -10.83% | 75% | 2126% |
| Class B | -5.01% | -14.43% | -15.02% | -5.48% | -17.07% | -17.71% | 61% | 2114% |
| Class C | -4.95% | -11.19% | -11.91% | -5.20% | -15.48% | -16.32% | 56% | 1655% |
| Class E | -7.88% | -15.72% | -17.06% | -8.38% | -16.47% | -18.69% | 56% | 2355% |
| Overall | -5.87% | -13.98% | -14.15% | -6.25% | -16.56% | -16.67% | 64% | 2105% |
| Class D | -5.20% | -8.48% | -10.32% | -4.46% | -14.14% | -15.47% | 53% | 1508% |
| Class F | -1.14% | -8.29% | -7.10% | -1.78% | -11.56% | -11.28% | 54% | 1840% |

    - NNVC-14 VTM mode vs NVC-14 reduced-runtime3

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.10% | -4.68% | -4.11% | 1.10% | -6.25% | -6.81% | 26% | 2425% |
| Class A2 | 2.67% | -3.82% | 3.10% | 3.69% | -4.01% | 3.55% | 27% | 2321% |
| Class B | 0.23% | -6.45% | -4.53% | 0.64% | -9.27% | -7.77% | 26% | 3084% |
| Class C | -0.93% | -6.57% | -4.75% | -0.77% | -7.46% | -5.96% | 23% | 3201% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.38% | -5.60% | -2.98% | 0.97% | -7.13% | -4.83% | 26% | 2805% |
| Class D | -3.16% | -7.41% | -5.39% | -1.91% | -9.44% | -7.39% | 23% | 3122% |
| Class F | 4.15% | 0.28% | 2.45% | 2.81% | -3.24% | -3.03% | 36% | 1692% |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -6.71% | -11.43% | -13.80% | -6.73% | -15.35% | -17.60% | 63% | 2542% |
| Class A2 | -5.18% | -11.29% | -7.75% | -6.03% | -12.49% | -8.11% | 52% | 2120% |
| Class B | -4.81% | -11.73% | -11.36% | -5.32% | -14.63% | -14.51% | 49% | 2109% |
| Class C | -4.72% | -7.90% | -7.69% | -5.04% | -12.93% | -12.87% | 41% | 1664% |
| Class E | -7.66% | -14.28% | -14.01% | -8.16% | -15.10% | -15.83% | 46% | 2361% |
| Overall | -5.64% | -11.18% | -10.79% | -6.08% | -14.10% | -13.81% | 49% | 2105% |
| Class D | -5.21% | -5.81% | -7.32% | -4.57% | -11.91% | -12.81% | 38% | 1507% |
| Class F | -2.23% | -6.12% | -4.09% | -3.17% | -10.53% | -9.57% | 39% | 1839% |

    - NNVC-14.0 VTM mode vs NNVC-14.0 HOP.5

Anchor: NNVC in VTM mode.

Test: NNLF HOP5 + NNIP.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -15.53% | -13.87% | -21.64% | -17.14% | -19.25% | -23.78% | 388% | 108236% |
| Class A2 | -15.05% | -20.80% | -26.89% | -14.34% | -20.07% | -23.88% | 352% | 95863% |
| Class B | -12.91% | -24.46% | -17.47% | -11.62% | -22.83% | -16.86% | 364% | 130086% |
| Class C | -13.69% | -17.83% | -17.25% | -12.17% | -14.96% | -14.32% | 274% | 130180% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -14.07% | -19.84% | -20.13% | -13.42% | -19.46% | -18.97% | 340% | 117985% |
| Class D | -14.51% | -18.02% | -17.88% | -10.74% | -13.01% | -12.24% | 252% | 132762% |
| Class F | -8.95% | -11.38% | -10.73% | -9.18% | -12.10% | -11.14% | 521% | 66119% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -9.69% | -12.14% | -6.10% | -8.85% | -9.20% | -3.21% | 352% | 123537% |
| Class C | -11.44% | -9.53% | -6.80% | -10.13% | -3.89% | -1.06% | 264% | 133154% |
| Class E | -10.17% | -11.01% | -6.26% | -9.51% | -11.40% | -5.28% | 742% | 113394% |
| Overall | -10.40% | -10.99% | -6.37% | -9.44% | -7.98% | -3.01% | 385% | 123979% |
| Class D | -12.63% | -7.30% | -0.62% | -9.94% | 1.90% | 13.67% | 242% | 128697% |
| Class F | -8.84% | -9.84% | -8.70% | -9.60% | -11.08% | -8.80% | 520% | 77947% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -10.89% | -15.13% | -8.90% | -9.89% | -12.30% | -5.92% | 469% | 122040% |
| Class C | -12.41% | -11.74% | -10.29% | -10.72% | -6.08% | -4.37% | 330% | 128334% |
| Class E | -11.18% | -11.14% | -5.88% | -10.52% | -11.92% | -4.51% | 1015% | 122535% |
| Overall | -11.47% | -13.00% | -8.61% | -10.32% | -10.13% | -5.05% | 506% | 124229% |
| Class D | -13.80% | -11.49% | -4.14% | -11.02% | -3.04% | 11.42% | 292% | 131792% |
| Class F | -9.63% | -11.73% | -10.41% | -10.37% | -13.35% | -11.39% | 638% | 77712% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -12.88% | -10.05% | -17.45% | -14.41% | -16.24% | -20.05% | 411% | 110327% |
| Class A2 | -12.95% | -17.02% | -20.64% | -13.73% | -18.12% | -17.78% | 284% | 86418% |
| Class B | -11.41% | -16.49% | -13.84% | -11.14% | -16.79% | -14.46% | 261% | 82910% |
| Class C | -12.56% | -13.64% | -15.46% | -11.89% | -13.61% | -15.18% | 199% | 58776% |
| Class E | -16.18% | -19.94% | -18.98% | -15.64% | -18.81% | -19.54% | 279% | 99486% |
| Overall | -12.96% | -15.45% | -16.79% | -13.04% | -16.55% | -16.95% | 272% | 83613% |
| Class D | -11.68% | -11.95% | -15.27% | -10.08% | -11.78% | -14.64% | 182% | 52002% |
| Class F | -9.64% | -11.73% | -9.09% | -9.32% | -11.98% | -10.21% | 184% | 73454% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC 14 VTM mode vs NNVC 14 HOP5+DRF

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -16.37% | -14.26% | -21.96% | -18.25% | -20.17% | -24.63% | 432% | 160427% |
| Class A2 | -16.62% | -21.67% | -27.30% | -16.52% | -21.69% | -25.14% | 390% | 142191% |
| Class B | -14.21% | -24.92% | -17.78% | -13.45% | -24.11% | -18.27% | 408% | 202265% |
| Class C | -15.99% | -18.97% | -18.47% | -15.66% | -17.68% | -17.22% | 304% | 202655% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -15.60% | -20.55% | -20.70% | -15.61% | -21.12% | -20.64% | 378% | 180055% |
| Class D | -16.43% | -17.66% | -17.55% | -13.77% | -15.35% | -14.83% | 281% | 192075% |
| Class F | -9.72% | -11.77% | -10.79% | -10.48% | -13.06% | -12.03% | 604% | 191335% |

    - NNVC-14.0 VTM mode vs NNVC-14.0 VLOP

Anchor: NNVC in VTM mode.

Test: NNLF VLOP4 + NNIP.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.86% | -7.33% | -6.00% | -9.05% | -8.85% | -7.10% | 114% | 1197% |
| Class A2 | -6.21% | -7.35% | -4.75% | -6.84% | -8.31% | -4.86% | 113% | 1099% |
| Class B | -5.68% | -8.06% | -6.66% | -6.45% | -9.46% | -8.32% | 114% | 1500% |
| Class C | -5.10% | -7.12% | -6.21% | -6.03% | -8.74% | -7.53% | 112% | 1608% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.07% | -7.52% | -6.03% | -6.94% | -8.91% | -7.17% | 113% | 1373% |
| Class D | -5.05% | -5.83% | -5.26% | -5.38% | -7.63% | -7.39% | 109% | 1563% |
| Class F | -3.03% | -3.82% | -3.89% | -4.05% | -5.53% | -5.67% | 117% | 839% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -3.97% | -7.94% | -2.95% | -5.03% | -11.17% | -9.68% | 106% | 1466% |
| Class C | -3.49% | -7.06% | -4.55% | -5.29% | -10.79% | -8.58% | 102% | 1594% |
| Class E | -4.35% | -5.72% | -2.68% | -6.50% | -8.27% | -4.74% | 107% | 1757% |
| Overall | -3.90% | -7.09% | -3.42% | -5.49% | -10.32% | -8.08% | 105% | 1578% |
| Class D | -3.95% | -4.79% | -1.90% | -5.26% | -10.71% | -7.71% | 101% | 1503% |
| Class F | -1.80% | -2.44% | -0.91% | -3.41% | -4.96% | -3.91% | 107% | 945% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -4.29% | -8.94% | -3.47% | -5.17% | -11.74% | -11.05% | 110% | 1421% |
| Class C | -3.90% | -7.79% | -4.94% | -5.38% | -11.77% | -8.36% | 104% | 1546% |
| Class E | -5.11% | -5.65% | -0.92% | -7.31% | -8.37% | -3.61% | 110% | 1880% |
| Overall | -4.37% | -7.73% | -3.32% | -5.77% | -10.91% | -8.29% | 108% | 1568% |
| Class D | -4.38% | -5.77% | -2.06% | -5.08% | -11.32% | -7.77% | 101% | 1565% |
| Class F | -2.08% | -2.52% | -1.21% | -3.73% | -6.40% | -6.86% | 110% | 910% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.16% | -9.18% | -8.86% | -8.68% | -11.78% | -9.92% | 161% | 1391% |
| Class A2 | -6.61% | -8.95% | -7.11% | -7.09% | -9.62% | -6.78% | 162% | 1246% |
| Class B | -6.39% | -8.65% | -8.77% | -6.66% | -10.33% | -10.01% | 161% | 1272% |
| Class C | -6.09% | -7.77% | -7.35% | -6.62% | -10.24% | -9.12% | 150% | 1085% |
| Class E | -9.32% | -10.60% | -11.23% | -9.87% | -10.83% | -10.86% | 157% | 1417% |
| Overall | -7.14% | -8.92% | -8.60% | -7.59% | -10.52% | -9.40% | 158% | 1265% |
| Class D | -5.97% | -6.23% | -6.79% | -5.91% | -9.04% | -9.13% | 144% | 990% |
| Class F | -4.26% | -5.00% | -4.83% | -4.76% | -6.27% | -6.63% | 133% | 1029% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-14.0 VTM mode vs NNVC-14.0 CALOP

Anchor: NNVC in VTM mode.

Test: NNLF CALOP4 + NNIP.

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-v14-VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.05% | -12.79% | -18.48% | -12.08% | -14.43% | -18.25% | 273% | 2231% |
| Class A2 | -9.54% | -11.90% | -12.32% | -11.77% | -19.38% | -16.18% | 228% | 1981% |
| Class B | -8.56% | -20.62% | -19.18% | -6.08% | -21.47% | -20.10% | 237% | 2739% |
| Class C | -6.95% | -14.56% | -14.13% | -7.14% | -16.74% | -16.43% | 224% | 2927% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.63% | -15.69% | -16.32% | -8.70% | -18.38% | -17.97% | 239% | 2508% |
| Class D | -7.03% | -13.26% | -12.46% | -6.83% | -14.86% | -14.24% | 217% | 2260% |
| Class F | -3.85% | -8.68% | -7.87% | -4.73% | -10.87% | -11.50% | 252% | 1643% |

Note: Results from Nokia, crosschecked by xxx.

  - Comparison to NNVC-14 anchor

    - NNVC-13 anchor vs NNVC-14.0 anchor

Anchor: NNLF LOP6 + NNIP (NNVC 13)

Test: NNLF LOP6 + NNIP (NNVC 14)

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 13 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.22% | 0.15% | 0.07% | 0.22% | -0.03% | -0.15% | 70% | 93% |
| Class A2 | 0.11% | 0.19% | 0.34% | -0.04% | 0.13% | 0.29% | 73% | 93% |
| Class B | 0.16% | 0.23% | 0.10% | 0.07% | 0.30% | 0.07% | 73% | 93% |
| Class C | -0.17% | 0.12% | 0.01% | 0.08% | 0.46% | -0.09% | 73% | 95% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.07% | 0.18% | 0.12% | 0.08% | 0.24% | 0.03% | 72% | 94% |
| Class D | -0.50% | -0.41% | -0.20% | -0.15% | 0.53% | 0.59% | 78% | 95% |
| Class F | 0.44% | 0.53% | 0.58% | 0.31% | 0.76% | 0.29% | 74% | 94% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 13 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.10% | 0.10% | 0.31% | -0.04% | -0.22% | -0.16% | 72% | 93% |
| Class C | -0.05% | 0.03% | 0.53% | -0.04% | 0.05% | 1.07% | 74% | 95% |
| Class E | -0.08% | -0.07% | -0.09% | -0.23% | 0.16% | -0.46% | 73% | 92% |
| Overall | 0.00% | 0.04% | 0.29% | -0.09% | -0.04% | 0.17% | 73% | 93% |
| Class D | -0.06% | -0.26% | 0.24% | 0.09% | 1.21% | -0.67% | 80% | 96% |
| Class F | 0.42% | 0.66% | 0.76% | 0.21% | 1.19% | 0.93% | 74% | 94% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 13 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.16% | 0.63% | 0.73% | 0.09% | 0.52% | 0.77% | 68% | 94% |
| Class C | 0.03% | 0.49% | 0.64% | -0.04% | 0.51% | 1.08% | 72% | 96% |
| Class E | -0.08% | 0.22% | 1.13% | -0.19% | 0.34% | 1.15% | 71% | 93% |
| Overall | 0.06% | 0.48% | 0.80% | -0.02% | 0.47% | 0.97% | 70% | 94% |
| Class D | 0.00% | 0.42% | 0.54% | 0.20% | 1.10% | 0.48% | 77% | 97% |
| Class F | 0.48% | 1.04% | 1.17% | 0.73% | 1.35% | 1.31% | 73% | 94% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 13 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.03% | -0.29% | -0.08% | 0.00% | -0.28% | -0.10% | 82% | 94% |
| Class A2 | 0.01% | -0.01% | 0.14% | 0.02% | -0.11% | 0.09% | 85% | 95% |
| Class B | 0.02% | 0.06% | 0.16% | 0.03% | 0.13% | 0.08% | 86% | 96% |
| Class C | 0.12% | 0.13% | 0.23% | 0.20% | 0.25% | 0.34% | 88% | 96% |
| Class E | 0.18% | 0.23% | 0.04% | 0.24% | 0.12% | 0.00% | 84% | 95% |
| Overall | 0.06% | 0.03% | 0.11% | 0.10% | 0.05% | 0.10% | 86% | 96% |
| Class D | 0.20% | 0.25% | 0.29% | 0.30% | 0.25% | 0.90% | 90% | 97% |
| Class F | 0.17% | 0.23% | 0.12% | 0.22% | 0.42% | 0.52% | 85% | 96% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-14 anchor vs NNVC-14.0 VLOP

Anchor: NNLF LOP6 + NNIP

Test: NNLF VLOP4 + NNIP

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.23% | 7.97% | 11.02% | -0.25% | 9.07% | 12.83% | 97% | 53% |
| Class A2 | 2.30% | 9.60% | 5.48% | 1.73% | 9.19% | 5.30% | 97% | 50% |
| Class B | 2.22% | 9.65% | 9.59% | 1.46% | 11.24% | 11.56% | 97% | 51% |
| Class C | 2.53% | 8.59% | 8.88% | 1.72% | 7.67% | 9.35% | 99% | 52% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 2.12% | 9.02% | 8.86% | 1.24% | 9.45% | 9.97% | 98% | 51% |
| Class D | 3.16% | 9.18% | 8.52% | 1.54% | 9.39% | 7.81% | 100% | 50% |
| Class F | 1.43% | 5.50% | 3.92% | 1.44% | 6.79% | 6.97% | 96% | 54% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 2.07% | 3.39% | 7.85% | 1.19% | -0.44% | 2.94% | 97% | 51% |
| Class C | 2.72% | 5.61% | 6.26% | 1.56% | -0.13% | 0.70% | 99% | 50% |
| Class E | 2.28% | -0.72% | 6.34% | 1.28% | -3.52% | 2.06% | 93% | 61% |
| Overall | 2.34% | 3.10% | 6.94% | 1.34% | -1.11% | 1.97% | 97% | 53% |
| Class D | 3.07% | 5.98% | 5.36% | 1.18% | -2.96% | -3.06% | 100% | 49% |
| Class F | 1.90% | 3.99% | 4.49% | 2.34% | 2.35% | 5.22% | 95% | 54% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 2.35% | 5.17% | 9.71% | 1.45% | 2.73% | 4.84% | 96% | 50% |
| Class C | 2.74% | 7.74% | 9.16% | 1.68% | 3.14% | 6.41% | 98% | 50% |
| Class E | 2.47% | 1.06% | 8.61% | 1.34% | -1.00% | 5.26% | 91% | 59% |
| Overall | 2.51% | 5.00% | 9.26% | 1.50% | 1.93% | 5.47% | 95% | 52% |
| Class D | 3.17% | 6.96% | 8.41% | 1.41% | -0.90% | 2.03% | 99% | 49% |
| Class F | 1.88% | 5.62% | 4.55% | 1.89% | 4.74% | 5.67% | 94% | 51% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.31% | 8.44% | 10.47% | 0.45% | 10.20% | 12.54% | 98% | 53% |
| Class A2 | 1.24% | 8.14% | 5.01% | 1.27% | 8.35% | 5.46% | 99% | 56% |
| Class B | 1.40% | 7.84% | 8.50% | 1.23% | 9.43% | 10.48% | 99% | 58% |
| Class C | 1.86% | 6.78% | 7.14% | 1.49% | 8.72% | 10.04% | 100% | 62% |
| Class E | 2.27% | 7.85% | 9.08% | 1.85% | 8.48% | 11.40% | 99% | 56% |
| Overall | 1.61% | 7.75% | 8.04% | 1.27% | 9.06% | 10.04% | 99% | 57% |
| Class D | 1.86% | 6.34% | 6.62% | 1.29% | 8.77% | 9.61% | 100% | 63% |
| Class F | 1.39% | 5.20% | 4.01% | 1.31% | 7.79% | 7.26% | 100% | 55% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-14 anchor vs NNVC-14.0 HOP

Anchor: NNLF LOP6 + NNIP

Test: NNLF HOP5 + NNIP

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.18% | -0.31% | -7.66% | -9.12% | -4.37% | -7.82% | 332% | 4780% |
| Class A2 | -7.34% | -6.68% | -19.04% | -6.42% | -5.28% | -15.93% | 303% | 4363% |
| Class B | -5.66% | -10.48% | -3.42% | -4.14% | -6.02% | 0.88% | 311% | 4387% |
| Class C | -6.77% | -4.05% | -4.08% | -4.91% | 0.06% | 1.20% | 241% | 4196% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.60% | -5.97% | -7.57% | -5.80% | -3.92% | -4.14% | 293% | 4405% |
| Class D | -7.16% | -5.11% | -6.32% | -4.20% | 2.93% | 1.80% | 232% | 4237% |
| Class F | -4.89% | -3.07% | -3.57% | -4.03% | -1.03% | 0.65% | 427% | 4291% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -4.04% | -1.41% | 4.14% | -2.90% | 1.73% | 9.75% | 323% | 4255% |
| Class C | -5.77% | 2.78% | 3.51% | -3.67% | 7.75% | 8.50% | 255% | 4153% |
| Class E | -3.94% | -6.23% | 2.22% | -1.92% | -7.04% | 1.16% | 646% | 3921% |
| Overall | -4.59% | -1.22% | 3.45% | -2.91% | 1.54% | 7.19% | 355% | 4136% |
| Class D | -6.26% | 2.61% | 6.79% | -3.83% | 8.87% | 18.16% | 241% | 4155% |
| Class F | -5.46% | -3.88% | -3.78% | -4.25% | -4.27% | -0.32% | 459% | 4426% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -4.74% | -2.05% | 3.31% | -3.62% | 1.92% | 10.14% | 409% | 4301% |
| Class C | -6.39% | 3.09% | 2.72% | -4.07% | 9.63% | 10.55% | 311% | 4125% |
| Class E | -4.08% | -4.73% | 3.16% | -2.13% | -4.98% | 4.32% | 835% | 3872% |
| Overall | -5.13% | -1.01% | 3.08% | -3.40% | 2.76% | 8.82% | 446% | 4131% |
| Class D | -7.00% | -0.02% | 5.80% | -4.94% | 6.37% | 21.29% | 288% | 4136% |
| Class F | -6.06% | -4.35% | -5.15% | -5.17% | -3.15% | 0.49% | 541% | 4380% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main 10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.90% | 6.91% | 0.02% | -5.82% | 4.15% | -0.24% | 250% | 4215% |
| Class A2 | -5.68% | -1.06% | -10.21% | -5.93% | -1.34% | -6.83% | 174% | 3910% |
| Class B | -4.11% | -1.36% | 2.50% | -3.62% | 1.33% | 5.06% | 160% | 3758% |
| Class C | -5.23% | -0.16% | -2.47% | -4.24% | 4.37% | 2.47% | 133% | 3358% |
| Class E | -5.52% | -3.58% | -0.70% | -4.57% | -1.47% | 0.38% | 176% | 3957% |
| Overall | -4.82% | -0.04% | -1.67% | -4.67% | 1.56% | 0.84% | 170% | 3793% |
| Class D | -4.36% | -0.34% | -3.72% | -3.22% | 5.48% | 2.14% | 127% | 3322% |
| Class F | -4.44% | -2.50% | -0.75% | -3.53% | 0.83% | 3.13% | 138% | 3926% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-14.0 vs NNVC-14.0 RPR

Anchor: NNLF LOP6 + NNIP

Test: NNLF LOP6 + NNIP + RPR multi-ratio

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.57% | 1.80% | 1.26% | -4.43% | -3.73% | -3.80% | 90% | 72% |
| Class A2 | -1.13% | 4.69% | 3.60% | -2.63% | -2.14% | -1.90% | 93% | 83% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.54% | 1.30% | 0.97% | -1.41% | -1.17% | -1.14% | 96% | 90% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.45% | 5.46% | 5.96% | -2.10% | -1.93% | -1.49% | 108% | 57% |
| Class A2 | -1.42% | 14.91% | 14.54% | -10.14% | -8.54% | -8.53% | 84% | 49% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.48% | 3.40% | 3.42% | -2.04% | -1.74% | -1.67% | 98% | 81% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-14 RPR anchor vs NNVC-14.0 NNSR

Anchor: NNLF LOP6 + NNIP + RPR multi-ratio

Test: NNLF LOP6 + NNIP + NNSR multi-ratio

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 RPR |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.17% | 0.37% | 0.54% | 0.00% | 0.00% | 0.00% | 104% | 100% |
| Class A2 | -0.07% | -0.59% | -0.32% | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.05% | -0.04% | 0.04% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | All Intra Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 RPR |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.33% | 0.92% | 0.06% | 0.00% | 0.00% | 0.00% | 104% | 100% |
| Class A2 | -1.92% | -2.89% | -2.67% | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.38% | -0.33% | -0.43% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Note: Results from InterDigital, crosschecked by xxx.

    - NNVC-14 anchor vs NNVC-14.0 DRF

Anchor: NNLF LOP6 + NNIP

Test: NNLF LOP6 + NNIP + DRF

|  | Random access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 14 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.00% | -0.82% | -0.60% | -1.32% | -1.77% | -1.56% | 155% | 2275% |
| Class A2 | -1.98% | -1.14% | -1.05% | -2.63% | -2.29% | -2.09% | 145% | 2105% |
| Class B | -1.58% | -0.83% | -0.05% | -2.11% | -1.99% | -1.69% | 151% | 2409% |
| Class C | -2.96% | -1.87% | -1.71% | -4.09% | -3.62% | -3.68% | 133% | 2176% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -1.91% | -1.17% | -0.81% | -2.58% | -2.44% | -2.27% | 145% | 2256% |
| Class D | -3.01% | 0.25% | 0.51% | -3.57% | -2.79% | -3.12% | 130% | 2040% |
| Class F | -0.89% | -0.65% | -0.27% | -1.44% | -1.30% | -1.00% | 183% | 6383% |

Note: Results from InterDigital, crosschecked by xxx.

- Contributions

10 contributions were identified for AhG14, and 1 telco report.

| JVET-AN0014 | JVET AHG report: NNVC software development (AHG14) | F. Galpin (chair), R. Chang, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) |
| --- | --- | --- |
| JVET-AN0042 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| JVET-AN0054 | AHG14: Extension of the QP range for TDO Lambda in NNVC S/W | H. Kwon, H. Ko (Hanyang Univ.), D. Kim, S.-C. Lim (ETRI) |
| JVET-AN0128 | AhG14: Improvement of SIMD implementation with expanded SIMD operators in SADL for LOP with Overlapped Feature Integration | J. Chi, A. Li, Y. Du, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion) |
| JVET-AN0196 | AHG14: Operator Improvements in the SADL library | W. Zhang, N. Fu, L. Qin, X. Chen, Z. Chen (Wuhan Univ.) |
| JVET-AN0197 | AHG14: The extension of SADL library | L. Qin, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.) |
| JVET-AN0202 | [AHG11/AHG14] Comparison of Multi-Layer and Single-Layer Interfaces for Hybrid End-to-End Video Coding Frameworks | N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A. B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AN0209 | AhG 11/AHG 14: Harmonization of Deep Reference Frame (DRF) with RPR in NNVC14.1 | N. Bhaskar, T. Solovyev, E. Alshina (Huawei) |
| JVET-AN0213 | AhG14: SADL update | F. Galpin (InterDigital) |
| JVET-AN0222 | [AHG14] New NN elements for SADL | A. Karabutov, E. Alshina (Huawei) |
| JVET-AN0225 | [AHG11][AHG14]: Further Improvements on Hybrid Multilayer Framework for End-to-End Learned Intra Frame | M. Aderdor, T. Solovyev, E. Alshina (Huawei) |
| JVET-AN0216 | [AHG11] Update on Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec | F. Urban, Y. Chen, F. Galpin, P. de Lagrange (InterDigital) |

- Configurations

The following configurations are used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration |
| --- | --- | --- |
| NNVC VTM mode | None | encoder_xxx_vtm.cfg |
| NNVC Anchor/EE1 | Intra Pred + LOP.6 | encoder_xxx_nnvc.cfg |
| NNVC. HOP | Intra Pred + HOP.5 | encoder_xxx_nnvc.cfg + nn-based/HOP5.cfg |
| NNVC. VLOP | Intra Pred + VLOP.4 | encoder_xxx_vtm.cfg + nn-based/vlop4.cfg |
| NNSR | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| RPR | Anchor RPR | nn-based/rpr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| NNPF | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |
| CALOP | Intra pred+adaptive LOP | encoder_randomaccess_nnvc.cfg+nn-based/intra.cfg+nn-based/calop.cfg |
| NNVC DRF | NNVC anchor + DRF | encoder_xxx_nnvc.cfg + drf.cfg |

- Recommendations

The AHG recommended:

- Review all input contributions.

- Keep synchronized with VTM upstream

- Merge the NNVC_cfe branch into NNVC branch

- Continue to develop NNVC software.

- Continue to develop hybrid end-to-end framework.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

It was agreed to encourage people to submit merge requests fixing identified bugs.

It was commented that rather than integrating the complete CfE branch (related to submission JVET-AN0212) the normal process should be followed, i.e. making decisions on the single elements (for which contributions exist), and potentially to investigate them in EEs.

JVET-AN0015 JVET AHG report: Gaming content compression (AHG15) [S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs)]

- Software repositories

Software developed by AHG15 consists of forks of VTM/ECM that have support for reading auxiliary data that is provided for sequences of classes G1/G3. A third repository contains scripts for the verification of the auxiliary information. The repositories can be found at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gcc.

- Test results

Test results were obtained according to JVET-AJ2027. The list of sequences/classes is copied for convenience below:

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 10 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
| G3 (HDR) | Level1_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | Darktree_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | ARPG2_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 10 | O | O | O |

For class G3 the HDR versions of Level1 and Darktree are still missing.

  - VTM-11.0ecm17.0 vs ECM-17.0

    - SDR

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -11.9% | -24.8% | -19.6% | 1015% | 589% | #DIV/0! |
| Class G2 | -18.9% | -23.6% | -27.8% | 1201% | 765% | #DIV/0! |
| Overall | -15.0% | -24.3% | -23.2% | 1094% | 662% | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -27.9% | -46.1% | -37.2% | 778% | 1278% | #DIV/0! |
| Class G2 | -25.4% | -36.9% | -37.7% | 960% | 1409% | #DIV/0! |
| Overall | -26.8% | -42.0% | -37.4% | 854% | 1335% | #DIV/0! |

* ECM decoder crashed for BaoleiYard in RA.

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -21.2% | -54.7% | -43.7% | 990% | 1501% | #DIV/0! |
| Class G2 | -18.8% | -50.5% | -49.4% | 942% | 1395% | #DIV/0! |
| Overall | -20.3% | -53.1% | -45.8% | 972% | 1460% | #DIV/0! |

|  | Low delay P Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over Anchor |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -19.6% | -60.3% | -51.0% | 819% | 1166% | #DIV/0! |
| Class G2 | -16.6% | -57.3% | -57.0% | 978% | 1417% | #DIV/0! |
| Overall | -18.5% | -59.2% | -53.2% | 875% | 1254% | #DIV/0! |

    - HDR

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -36.9% | -17.3% |  |  |  | -15.6% | -58.0% | -55.4% | 1088% | 655% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -53.7% | -32.9% |  |  |  | -33.9% | -71.8% | -70.6% | 872% | 1666% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -50.6% | -27.8% |  |  |  | -26.0% | -75.6% | -72.9% | 883% | 1710% |
|  | Low Delay P |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -50.5% | -27.1% |  |  |  | -25.5% | -76.0% | -72.2% | 828% | 1366% |

Weighted PSNR metrics were not collected.

- Input contributions

Contributions listed below have been identified relating to the mandates of AHG15.

| Report |  |  |
| --- | --- | --- |
| JVET-AN0053 | AHG4/AHG15/AHG17: 4K gaming sequences featuring “Black Myth: Wukong” | J. Sauer, Y. Zhao, Y. Sun, J. Zhou, E. Alshina (Huawei) |
| JVET-AN0174 | AHG15: Compression of gaming content using auxiliary data | J. Sauer, Z. Li (Huawei) |
| JVET-AN0263 | AHG15: Transformer based depthmaps reconstruction for gaming content | V. Zakharchenko (Nokia) |

- Recommendations

The AHG recommended to:

- Review input documents on gaming content compression

- Identify if/how auxiliary information can be used for coding of gaming content

JVET-AN0016 JVET AHG report: Generative face video compression (AHG16) [Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs)]

- Activities

Regarding the mandate on developing and maintaining the GFVC software, the AHG16 GFVC software tool and accompanying usage instructions and exemplar configurations for experimentation are maintained in the GIT repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc. No changes were made to the AHG16 software during this ad hoc period. Merge requests to add support for GFV and GFVE SEI messages to HEVC and AVC (as in JVET-AL0148) were submitted to HM and JM software before the 39th meeting, and are currently pending integration. At this meeting, one contribution on colour calibration post-processing has been received.

Regarding coordination with AHG9 to develop the generative face video SEI messages, at this meeting, two contributions related to editorial improvements and bug fixes have been received.

- Related contributions

The following input contribution to this meeting is related to the activities of AHG16:

- JVET-AN0057, AHG16: Colour calibration post-processing for generative face video coding [S. Yin, Z. Zhang, S. Wang (CityUHK), B. Chen, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

Additionally, the following input contribution is related to the high-level syntax aspects of the generative face video SEI messages:

- JVET-AN0141, AHG9: On the generative face video SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

- JVET-AN0156, AHG9: Text fixes and cleanup for GFV and GFE SEI messages [J. Chen, Y. Ye, B. Chen (Alibaba)]

- Recommendations

The AHG recommended to:

- Review related contributions.

JVET-AN0017 JVET AHG report: Testing of video coding technology beyond CTC (AHG17) [J.-R. Ohm, M. Wien, F. Bossen (co-chairs), M. Abdoli, E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, L. Li, P. Nikitin, D. Rusanovskyy (vice chairs)]

- Activities

The draft CfE document JVET-AM2026 was finalized and uploaded to the JVET document site. The test sequences for all categories defined in the CfE document were collected in a dedicated folder on the JVET content server.

Four online AHG calls were held on 2025-07-29, 2025-08-21, 2025-09-11, and 2025-09-29, respectively. The reports of these meetings are available in JVET-AN0041.

- Preparation of bitstreams according to the CfE conditions

The categories of the draft CfE include:

- SDR RA UHD/4K: Representing the use case of distribution of standard dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration.

- SDR RA HD: Representing the use case of distribution of standard dynamic range HD video content e.g. in a streaming scenario, using a random-access configuration.

- SDR LB HD: Representing the use case of conversational and other low delay applications at HD resolution, correspondingly using a low-delay configuration.

- HDR RA 4K: Representing the use case of distribution of high dynamic range UHD/4K video content e.g. in a streaming scenario, using a random-access configuration.

- HDR RA Cropped 8K: Representing the use case of distribution of high dynamic range 8K video content e.g. in a streaming scenario, using a random-access configuration. In order to reduce the encoding workload for assessment of this category and allow investigation on 4K displays, cropped regions of 3840×2160 resolution are used.

- Gaming LB HD: Representing the use case of online gaming with a low-delay configuration.

- UGC RA: Representing the use case of user generated content at 1080×1920 or 1920×1080 resolution using a random-access configuration.

For all categories, test sequences and corresponding target rates are specified in JVET-AM2026. VTM anchor bitstreams as well as ECM bitstreams matching the target bitrates for study of improved compression performance were generated. Furthermore, for the study of reduced complexity encoding, bitstreams for the following encoder configurations were generated:

- VTM RT1: VTM with reduced runtime setting at about 65% of the runtime of the default VTM configuration;

- VTM RT2: VTM with reduced runtime setting at about 50% of the runtime of the default VTM configuration;

- VTM RT3: VTM with reduced runtime setting at about 25% of the runtime of the default VTM configuration;

- ECM 2x: ECM with a reduced runtime setting at about a factor of 2 of the runtime of the default VTM configuration.

- ECM 5x: ECM with a reduced runtime setting at about a factor of 5 of the runtime of the default VTM configuration.

- Planning for on-site viewing sessions

For conduction of the viewing tests, the AHG recommended all proposals submitted for the compression efficiency test case (CP) to be evaluated. For the runtime test case (RT), a selected subset was recommended in the last AHG. With this selection, 686 test points (“processed video sequences”, PVS) are to be evaluated for the CP case and 1088 test points are to be evaluated for the RT case.

A test including 12 test sessions for CP and 21 test sessions for RT was designed. The sessions separate PVS in HDR PQ, HDR HLG, SDR UHD, and SDR HD 50/60fps for smooth playout purposes. The suggested allocation of the test efforts is as follows:

| CP | Start time | duration | est. End |  |
| --- | --- | --- | --- | --- |
| Sat | 09:00 | 137 | 11:17 | CP1 |
| Sat | 11:30 | 125 | 13:35 | CP2 |
| Sat | 14:00 | 133 | 16:13 | CP3 |
| Sat | 16:30 | 163 | 19:13 | CP4 |
| RT | Start time | duration | est. End |  |
| Sun | 09:00 | 129 | 11:09 | RT1 |
| Sun | 11:30 | 127 | 13:37 | RT2 |
| Sun | 14:00 | 125 | 16:05 | RT3 |
| Sun | 16:30 | 125 | 18:35 | RT4 |
| RT | Start time | duration | est. End |  |
| Mon | 09:00 | 129 | 11:09 | RT5 |
| Mon | 11:30 | 119 | 13:29 | RT6 |
| Mon | 13:30 | 145 | 15:55 | RT7 |

For each session, 24 volunteers are requested to participate. A email requesting for registration for the sessions will be sent out at the beginning of the JVET meeting.

- Related contributions

| JVET-AN0041 | AHG17: AhG meeting notes | M. Wien |
| --- | --- | --- |
| JVET-AN0052 | [AHG17] [ Performance of GOP based RPR under CfE test conditions | Y. Sun, Y. Zhao, J. Sauer, J. Pardo, P. Jia, T. Solovyev, E. Alshina (Huawei) |
| JVET-AN0053 | AHG4/AHG15/AHG17: 4K gaming sequences featuring “Black Myth: Wukong” | J. Sauer, Y. Zhao, Y. Sun, J. Zhou, E. Alshina (Huawei) |
| JVET-AN0080 | AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience | S. Ikonin, V. Khamidullin, I. Gribushin, M. Sychev, K. Malyshev, A. Dzugaev, A. Bovsha, X. Ma, E. Alshina (Huawei) |
| JVET-AN0085 | Response to the joint CfE on video compression with capability beyond VVC | M. Abdoli, P. Nikitin, F. Plowman, A. Tissier, R. G. Youvalari, M.-L. Champel (Xiaomi), Y.-J. Chang, C.-C. Chen, M. Coban, K. Cui, P. Garus, N. Hu, M. Karczewicz, J.-L. Lin, P.-H. Lin, X. Meng, B. Ray, V. Seregin, Y. Shao, G. Verba, H. Wang, E. Ye, R. Yu, Y. Zhang, Z. Zhang, W. Zhu (Qualcomm), F. Le Léannec, K. Naser (InterDigital), F. Wang (OPPO), Y. Kidani, H. Kato, T. Chujoh, K. Kawamura (KDDI), Z. Deng, K. Zhang, L. Zhang (Bytedance), A. Filippov, J. Konieczny, V. Rufitskiy, H. Qin, T. Dong, Z. Xu, K. Ding, C. Hollmann, I. Zupancic, D. Li (TCL), X. Xiu, X. Wang (Kwai) |
| JVET-AN0157 | AHG17: Valgrind complexity analysis on CfE encoder run time targets | S. Hong, Y. Tokumo, T. Ikai (Sharp) |
| JVET-AN0177 | AHG4/AHG17: Response to the call for new HDR materials for future video coding development | J. Wang, J. Zhang, L. Yu (ZJU) |
| JVET-AN0189 | AHG17/AHG10: On perceptual coding for next-generation video coding standard | Y. Zhao, A. Karabutov, T. Guo, P. Jia, E. Alshina (Huawei) |
| JVET-AN0190 | AHG17/AHG10: On test model simulating hardware encoder | Y. Zhao, J. Mao, R. Zhao, X. Ma, T. Solovyev, E. Alshina (Huawei) |
| JVET-AN0198 | [AHG17] Wuhan University’s Response in Joint Call for Evidence on Video Compression with Capability beyond VVC | W. Zhang, L. Qin, X. Chen, N. Fu, H. Qu, W. Ma, J. Zhang, Z. Chen (Wuhan Univ.) |
| JVET-AN0212 | AHG17: Description of NNVC-based CfE response | D. Kim, S.-C. Lim (ETRI), T. Solovyev, J. Sauer, J. Pardo, P. Jia, A. Karabutov, E. Alshina (Huawei), H. Kwon, H. Ko (HYU), F. Galpin, T. Dumas, E. François (InterDigital), Y. Li, M. Karczewicz (Qualcomm), Z. Xiang, R. Chernyak, S. Liu (Tencent) |
| JVET-AN0243 | Crosscheck of JVET-AN0052 (AHG17: Performance of GOP based RPR under CfE test conditions) | Z. Xiang (Tencent) |
| JVET-AN0267 | Ericsson, Fraunhofer HHI and Nokia responses to Joint CfE on Video Compression with Capability beyond VVC | W. Ahmad, K. Andersson, M. Damghanian, V. Gritsenko, L. Litwic, D. Liu, M. Pettersson, V. Shchukin, R. Sjöberg, N. Stegmaier, J. Ström, N. Svensson, P. Wennersten (Ericsson), C. Bartnik, J. Brandenburg, B. Bross, V. George, J. Güther, G. Hege, A. Henkel, T. Hinz, G. Lazarov, C. Lehmann, W. Lim, Y. Liu, S. de Luxán Hernández, D. Marpe, V. Menon, T. Nguyen, J. Pfaff, S. Puttkammer, T. Schierl, H. Schwarz, B. Stallenberger, C. Stoffers, K. Sühring, A. Wieckowski, T. Wiegand, M. Winken (Fraunhofer HHI), P. Astola, S. Blasi, D. Buğdayci Sansli, C. Feldmann, D. Fortin, J. Funnell, S. Hong, I. Jumakulyyev, J. Lainema, N. Neumann, J. Ridge, D. Rusanovskyy, S. Schwarz (Nokia) |
| JVET-AN0272 | AHG17: ECM comparison points for CfE | K. Andersson, E. François, S. Hong, Y. Kidani, F. Le Léannec, X. Li, R.-L. Liao, Z. Lyu, F. Pu, V. Seregin, H.-B. Teo, A. Tissier, G. Verba, F. Wang, J.-R. Ohm, M. Wien, F. Bossen, M. Abdoli, E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, L. Li, P. Nikitin, D. Rusanovskyy |
| JVET-AN0301 | AhG17 Sharing experience of conducting JPEG AI Call for Proposals | E. Alshina (Huawei) |
| JVET-AN0303 | External memory bandwidth evaluation for CfE response JVET-AN0267 | C. Feldmann (Nokia) |
| JVET-AN0318 | AHG17 – Review of CfE responses | K. Naser, F. Le Léannec, F. Galpin, E. François (InterDigital) |
| JVET-AN0330 | Proposed modification of CfE test conditions | E. Alshina (Huawei) |

It is noted that multiple documents have partial allocation to AHG17 and other AHGs. This includes JVET-AN0053, JVET-AN0157, JVET-AN0177, JVET-AN0189, JVET-AN0190.

- Expression of thanks

All volunteers for the efforts of bitstream generation and reporting of metrics were thanked. Frank Bossen was thanked for crosschecking bitstreams and CSV files. Mathias Wien was thanked for preparing the viewing tests.

- Recommendations

The AHG recommended:

- To conduct subjective tests assessing the proposed rate points and coding conditions in the draft CfE.

- To study the results of the visual tests and conclude on the findings.

- To review the remaining input contributions related to AHG17 during the meeting.

It was suggested to add a viewing session related to the CfE contribution on ULD/ER functionality (last session Monday).

First presentation of results planned for Tuesday (late, e.g. 1900).

JVET-AN0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström (vice-chairs)]

- Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). Few organizational e-mails were sent to reflector.

  - Teleconference

The teleconference was held on August 21st with 27 participants attending.

The following topics were discussed:

- JVET-AN0046: On test conditions for ultra-low latency and packet loss resilience with full QP range

  - Network bandwidth point selection automation script

  - LDP/LDB simulations in full QP range

  - Spatial Scalability

- AHG18 software, MRs status

  - MR2: Full VTM

  - MR3: GDR fix

- JVET-AN0045: Codec software for ultra-low latency and packet loss resilience (related to JVET-AM0218)

  - Software overview

  - Transmission priority model

The meeting minutes are available in JVET-AN0049.

  - Simulation software

Adoption of last JVET meeting (JVET-AM0203, full VTM support) merged to ‘ull-master’ branch: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM/-/tree/ull-master. JVET-AM0195 (GDR) is cross-check pending.

- Related contributions

A total of 12 contributions are identified relating to the mandates of AHG18. They are listed below. Some contributions also relate to the work of AHG12.

| Report |  |  |
| --- | --- | --- |
| JVET-AN0018 | JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) | S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström (vice-chairs) |
| JVET-AN0049 | AHG18: Teleconference on ultra-low latency and packet loss resilience | S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, J. Ström, X. Ma, C. Kim, S. Puri, S. Fößel (vice-chairs) |
| Test condition and evaluation methodology proposals |  |  |
| JVET-AN0046 | AHG18: On test conditions for ultra-low latency and packet loss resilience with full QP range | S. Ikonin, I. Gribushin, V. Khamidullin, B. Shevchenko, E. Alshina (Huawei) |
| JVET-AN0079 | AHG18: Proposed methodology and test conditions for ultra-low latency and packet loss resilience performance evaluation | S. Ikonin, I. Gribushin, X. Ma, E. Alshina (Huawei), S. Deshpande (Sharp) |
| JVET-AN0089 | AHG12/AHG18: On GDR test condition for video compression beyond VVC | T. Chujoh, Y. Kidani, K. Kawamura (KDDI) |
| Other proposals |  |  |
| JVET-AN0045 | AHG18: Codec software for ultra-low latency and packet loss resilience | S. Ikonin, V. Khamidullin, K. Malyshev, I. Gribushin, X. Ma, E. Alshina (Huawei) |
| JVET-AN0080 | AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience | S. Ikonin, V. Khamidullin, I. Gribushin, M. Sychev, K. Malyshev, A. Dzugaev, A. Bovsha, X. Ma, E. Alshina (Huawei) |
| JVET-AN0081 | AHG18: Software fixes for robust decoding in unicast scenario | K. Malyshev, V. Khamidullin, M. Sychev, S. Ikonin, E. Alshina (Huawei) |
| JVET-AN0204 | AHG18: Request on inclusion ultra-low latency and packet loss resilience category into CfP | S. Ikonin, X. Ma, E. Alshina (Huawei), S. Wenger (Tencent) |
| Crosscheck |  |  |
| JVET-AN0082 | AhG18: Crosscheck report for JVET-AM0195 | V. Zakharchenko (Nokia) |
| Informational |  |  |
| JVET-AN0050 | AHG18 : An Introduction to 5G Latency/Throughput Measurement and Real-Time Monitoring for ULL Application Scenarios | B. Jeon (TTA), S. M. Kim (LGUplus) |
| JVET-AN0254 | AHG18: Packet marking mechanism for unicast scenario with channel feedback | V. Zakharchenko (Nokia) |

- Recommendations

The AHG recommended:

- Review all input contributions.

- Discuss test conditions and evaluation methodology, taking into account CfE evaluation results.

- Collect test cases and specific requirements.

- Continue development of simulation software.

- Continue the study of ultra-low latency and packet loss resilience technologies in JVET.

# Project development (48)

## AHG1: Deployment and advertisement of standards (1)

Contributions in this area were discussed during 1910–1920 on Saturday 4 Oct. 2025 (chaired by JRO).

JVET-AN0021 Deployment status of the VVC standard [G. J. Sullivan]

This contribution contains a survey of deployed products and services, publicly available software source code, related tools, and formal specifications supporting the VVC standard (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Revision marking is included to show changes relative to JVET-AL0021-v1 of April 2025.

- Fluendo received an award grant in July 2024 from the SPIRIT EU project for its STREAM (Scalable Telepresence with Real-time EnhAnced Multimedia) project for collaborative telepresence. The project provides support in the GStreamer system for alpha channel usage with VVC, based on the NHK multilayer extension of the Fraunhofer HHI VVenC software, and was released to Github.

- Amlogic released an S905X5 Armv9 processor SoC with dual-stream 4K 10 bit @60 fps VVC capability in September 2023. SEI Robotics included the Amlogic processor in its AI-SR Android TV box at that time. Ugoos used it for its AM9 Android TV box in July 2025.

- SBTVD Forum in Brazil, following its phase 2 testing conducted during July–December 2021, announced in January 2022 the decision for VVC to be used for the main video stream for the upcoming next-generation TV 3.0 broadcasting standard in Brazil, both for over-the-air and over-the-Internet transmission. Experimental broadcasting began in Rio de Janeiro and São Paulo in 2025, and President of Brazil Luiz Inácio signed a presidential decree on 27 August 2025 establishing Brazil’s DTV+ service (previously known as TV 3.0) based on ATSC 3.0 with VVC as Brazil’s next-generation broadcast standard.

## AHG2: Text development and errata reporting (1)

Contributions in this area were discussed during 1920–1930 on Saturday 4 Oct. 2025 (chaired by JRO).

See also sections 6.1 and 6.2.1.

JVET-AN0258 Reference Guide to the Structural Concepts in the Versatile Video Coding (VVC) Standard [G. J. Sullivan (Dolby Labs)]

As JVET contemplates the creation of a new generation of video coding standard with capabilities beyond VVC, it is appropriate to make sure the high-level design and capabilities of VVC itself are well understood and appreciated.

This document contains an outline, primarily in a tabular format, of the structure of the Versatile Video Coding (VVC) standard (ITU-T H.266 and ISO/IEC 23090-3). It is primarily drafted to provide a reference guide to basic descriptions of the structural elements of the standard without requiring the reader to read through a substantial amount of less structured explanatory prose, as would ordinarily be found in a journal paper on the subject (such as the IEEE Trans. CSVT high-level syntax paper of October 2021), and without being burdened with all the extra information that would be found in the standard itself. Some historical background information is also provided about the features.

The author drafted this document as an informal guide for colleagues and eventual publication in some form, and it might still eventually become a conference or journal paper, although its format is a bit unusual for such a publication. It is provided here as tutorial information and could also evolve into some group publication such as an updated white paper or technical report, or an informative annex for the standard.

As an editorial convention, since VVC is the focus of this document and is the most recent ITU-T/ISO/IEC video coding standard, it is referred to in the document in the present tense, while other standards are referred to in the past tense as historical background. In cases where this document says a particular feature was introduced or first used in a particular standard, that means only that this feature was not in previous ITU-T and ISO/IEC standards, not that there was no knowledge of the technique in the community before that time.

It was commented that the paper could be referenced in the VVC white paper, and potentially some of the paragraphs might be used there.

## AHG3: Software development (1)

Contributions in this area were discussed during 1935–2000 on Saturday 4 Oct. 2025 (chaired by JRO).

JVET-AN0292 Multiview HEVC Reference SW implementation status [S. Choi, S. Paluri, D. Podborski, A. M. Tourapis (Apple)] [late]

This contribution discusses the current development status of a reference software implementation for the currently defined multiview profiles of the HEVC standard. It is reported that the HM reference software was extended to include support for all multiview profiles, including multiview monochrome profiles, in both the encoder and the decoder. The main limitation of the implementation is that currently the software cannot be configured to support layers of different chroma format, bitdepth, or resolution capabilities. Other functionalities, such as motion compensated temporal filtering and film grain analysis and insertion may also be of limited use when multiview functionalities are desired. Such support is expected to be added in subsequent versions. It is proposed that the provided code is integrated with the official HM reference software.

It was requested that the software should be provided in gitlab, and in a style that allows to identify the changes relative to the current HM.

Generally, the integration of existing and new multiview profiles in HM (transferring the old ones from HTM) is an excellent step forward.

It was also requested to start generating bitstreams for the remaining multiview profiles in JVET-AM1008, and provide a list of bitstreams until the next meeting. Also for the cases where different chroma format, bitdepth, resolution, or profile IDC is needed in the different layers, these might be generated separately, and also be helpful in the software development fully supporting such cases.

## AHG3: Test conditions and metrics (2)

Contributions in this area were discussed during 1610–1700 on Monday 6 Oct. 2025 (chaired by JRO).

JVET-AN0341 Influence of colour transformation on codec performance [M. Domanski, A. Grzelka, O. Stankiewicz (Poznan Univ. Tech.)] [late]

Recently, AI-based techniques have been used quite frequently in video coding experiments. Software modules often operate on RGB representations, while others, including input and output modules, operate on YCBCR components. In this study, the quality losses resulting from colour-space transformations are examined. It is reportedly clear that these losses must be considered when developing software for prospective new standards.

It was reported that PSNR in YCbCr and RGB can be quite different after encoding in luma/chroma.

This may partially be caused by encoder settings for luma/chroma, but also by the fact that the transformation is non-orthogonal.

It was commented performing PSNR based comparison in a unified colour is important.

It was also commented that PSNR YCbCr to some extent better reflects the colour perception of human visual system.

JVET-AN0348 AHG10: On reporting metrics for RPR [K. Andersson (Ericsson)] [late]

When RPR is used in the VVC reference software, PSNR1 and PSNR2 are reported, and those are only described in a RPR functionality confirmation test condition for reference picture resampling in JVET-Q2015, but the software is also reporting PSNR in this case. To avoid confusion between PSNR, PSNR1 and PSNR2 and from the point of view of using RPR as a coding tool it makes sense to report PSNR with respect to source resolution. It is also suggested to update the software manual for further clarification.

It was commented that PSNR1 and PSNR2 were originally designed to investigate the benefit of RPR elements by performing unrealistically frequent switching in the CTC, used during RPR development. Currently, when RPR is used with the adaptive method, log files report both PSNR1 per frame, where framess may have different coded resolutions, and further PSNR2 in original resolution (potentially after upsampling, and optionally using an own, non-normative upsampler).

It is generally agreed that clarification is useful.

Actions to modify in log file:

- Change order of printout when the option is used to output the PSNR at original resolution generated after using a non-normative upsampling in case where coding was performed in reduced resolution

Also some clarification needs to be added to the software manual.

## AHG4: Subjective quality testing and verification testing (1)

Contributions in this area were discussed during 1015–1045 on Friday 10 Oct. 2025, and 1210–1235 on Sunday 12 Oct. 2025 (chaired by JRO).

JVET-AN0048 AHG4: teleconference on VVC multilayer testing [P. de Lagrange (InterDigital)]

This document summarizes discussions during teleconferences on visual assessment for verification testing of VVC multi-layer, conducted by JVET AHG4 between the 39th meeting (Daejeon, KR, 26 June – 4 July 2025) and the 40th meeting (Geneva, CH, 3–12 October 2025). Offline continuation work is also reported.

Was presented in JVET as preliminary report about verification testing with quality ladder using spatial scalability.

Target MOS approx. 4/6/8.

Planned to test single-layer VTM versus multi-layer VTM with spatial scalability, 2x upsampling for UHD, 1.5x for HD. Same bitrate base/enhancement layer in UHD, 2/3 vs. 1/3 base/enhancement in HD.

Enhancement layer rate matched with single layer.

DCR test, expert viewing sessions planned for Saturday morning.

JVET-AN0298 AHG4: proposed updates for VVC multi-layer verification test plan [P. de Lagrange (InterDigital)] [late]

This contribution proposes the updates to the category 2 (quality ladder) of verification test plan for VVC multilayer coding (JVET-AM2021), following the work reported in JVET-AN0048, and provides modified test plan text.

Final selection of sequences, rate points largely defined with some updates to be made according to viewing results.

JVET-AN0377 Report on dry-run expert viewing for the verification tests for VVC multilayer coding [M. Wien, C. Lehmann, P. de Lagrange]

Initial version presented Sunday 12 Oct.

Some adjustment may be necessary on rate points in various test cases, and also bug fixing is still necessary on SHM (HDR, MCTF) to include SHVC in the test.

To be sorted out in the AHG if (after SHM fix would be done), whether the verification test with non-expert viewers can be arranged until the next meeting (as currently in the test plan).

## AHG4: Test and training material (2)

Contributions in this area were discussed during 1625–1710 on Thursday 9 Oct. 2025 (chaired by JRO).

JVET-AN0053 AHG4/AHG15/AHG17: 4K gaming sequences featuring “Black Myth: Wukong” [J. Sauer, Y. Zhao, Y. Sun, J. Zhou, E. Alshina (Huawei)]

In this proposal, five 4K 60fps gaming sequences of the AAA game “Black Myth: Wukong” are described and contributed to JVET. This game has been released mid 2024 and the contributors suggested it is a good example of current leading-edge gaming content. The contributors encouraged study of these sequence in the context of gaming content compression in AHG15, and recommended them as candidate test sequences in developing new-generation video coding standard.

Compression ECM17 over VTM 9.0% for AI, 16.0%.

It was commented that these sequences have been produced by much more recent gaming engines than the current test material. It was also commented that when participating in real games, usually some graphics content would be overlayed which is not the case in those sequences.

Sequences might also be downsampled to HD if needed.

No auxiliary data / depth are available. Not available in 444 (converted from 422 to 420).

A viewing session should be organized for Friday afternoon.

Follow-up on Sunday 12 Oct. 0930, after viewing had been conducted – comments made during viewing:

- Gaming sequences were generally well received, being different in characteristics

- Wukong1 was most preferred by various experts

- Wukong2 and Wukong4 were also mentioned to be interesting

- Wukong5 was least preferred

- No comments were made on Wukong3, Mathias thinks it may be a bit more difficult to watch

- Challenging in terms of rate: Wukong4, Wukong1, Wukong3, Wukong2, Wukong5 is significantly less challenging

It was concluded that encoding (VTM and ECM) should be conducted on all of them, UHD and HD (subsampled) both in LDB (ECM only for HD at this moment). Baolei Yard might also be considered (the only UHD in gaming CTC). It was suggested to consider about the correct chroma location when doing downscaling.

It was commented that proposals for new test materials should be submitted early enough to be considered in interim AHG17 meeting(s). This should include information about VTM performance with QP steps of 2 in the QP range between 26 and 50. The test coordinator should be consulted to clarify upload and other aspects.

JVET-AN0177 AHG4/AHG17: Response to the call for new HDR materials for future video coding development [J. Wang, J. Zhang, L. Yu (ZJU)]

This contribution provides a response from ZJU to the call for new HDR materials for future video coding development which was proposed at the 35th JVET meeting in Sapporo. The contributor provided six 4K HDR sequences captured by a DJI Ronin 4D (6K) cinema camera that supports high dynamic range and ProRes RAW recoding.

VTM coded results are shown below (blue are existing ones, orange are proposed)

It was commented that, except for MuseumFountain, qualitatively the sequences seem to be not significant different in terms of being challenging for encoding than existing ones.

Proponents will send bitstreams of the VTM encoding to M. Wien, such that they can be included in the viewing session on Friday.

Follow-up on Sunday 12 Oct. 0955, after viewing had been conducted – comments made during viewing:

- Several sequences had parts which were out of focus (amusement park, west lake,

- Some chroma aberrations in Amusement Park 1, gate towers and light show

- Some overexposure may be present in Geese and light show

- Some more sequences were proposed in m74033 (WG 5 document), which were also shown but no compressed versions

- Museum fountain could be interesting, but focus is on the building. The other version from m74033 (museum fountain 2) could be more interesting, as it has less focus problems. There might be distortion or reflection on the floor in both sequences. Both are challenging in terms of rate.

- Gate towers is interesting, but very low rate.

It was also commented that possibly some post processing had been applied, 4:2:0 conversion might also have been applied without luma adjustment. Proponents of sequences should be asked about that, and if that was the case, potentially different versions could be provided.

It was concluded that encoding (VTM and ECM) should be conducted on gate towers and museum fountain 2. Licensing conditions for the latter should be confirmed.

## AHG5: Conformance test development (0)

This section is kept as a template for future use.

## AHG7: ECM tool assessment (7)

Contributions in this area were discussed in BoG JVET-AN0364 except as noted otherwise.

JVET-AN0043 AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 [X. Li (Google)]

JVET-AN0044 Report of AHG7 conference call on Complexity Analysis [X. Li]

JVET-AN0119 AHG7: Analysis on group-off tests using Valgrind [R. Ishimoto, F. Zheming, T. Ikai (Sharp)]

JVET-AN0203 AHG7: On RGPM moved into group-2 [P. Bordes, K. Reuzé, E. François, K. Naser, F. Le Léannec (InterDigital)]

Was initially discussed in BoG JVET-AN0364 which recommended adoption for the following reason.

About JVET-AN0203: There is no change in performance in CTC, but for proper grouping of RGPM in group 2 instead of group 1, a change of low-level conditions is necessary.

Decision by JVET plenary: Adopt JVET-AN0203.

JVET-AN0369 Crosscheck of JVET-AN0203 (AHG7: On RGPM moved into group-2) [P. Nikitin (Xiaomi)] [late] [miss]

JVET-AN0223 AHG7: On bin to bit ratio in ECM [T. N. Canh, P. Yin, S. McCarthy (Dolby), J. N. Shingala (Ittiam)] [late]

JVET-AN0253 AHG7: On Memory Bandwidth Measurement [X. Li (Google), Y. Kim (Samsung), J. Pardo (Huawei), R. Ishimoto (Sharp), L.-F. Chen (Tencent)]

JVET-AN0317 AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 – Update on IntraTMP [K. Naser, T. Dumas, M. Radosavljevic (InterDigital)] [late]

JVET-AN0378 Suggested Draft of Complexity Reporting Template for Coding Algorithms and Tools [X. Li]

This document was requested to be generated during the review of the BoG report JVET-AN0364. A review was conducted Sunday 12 Oct. 1050-1120 (chaired by JRO). The following comments were made:

- The questionnaire in 3.1 should not only cover NN, but learning based approaches

- 4.3 should be further clarified on its intent.

- For some aspects on tool level, numbers should be provided (e.g. size of lookup table, etc.)

- “overall memory usage” means the peak reported in software.

- For NN, information about precision (FP, int, bitdepth) should be added

- Some other aspects might also be asked, such as size of receptive field

- Clarification that numbers shall include processing for chroma, but relate to count per luma sample.

- Binary size of encoder and decoder should be asked, also external libraries that may necessary, and whether external data files would be needed.

- The granularity of tool analysis needs clarification, and needs to some extent be left to proponents. At the time of CfP, it could be covering building blocks with their worst case complexity, but in CE it might be at much finer granularity of dedicated tools.

- Examples should be given for VTM, not ECM as in the current version. Trying to fill the questionnaire for VVC/VTM could be a good exercise to find answers to the previous bullet point.

JVET-AN2040 should have a long editing period, to be further refined in joint telco(s) of AHG7/AHG19.

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (0)

This section is kept as a template for future use.

## AHG10: Encoding algorithm optimization (0+4)

This section is kept as a template for future use.

JVET-AN0189 AHG17/AHG10: On perceptual coding for next-generation video coding standard [Y. Zhao, A. Karabutov, T. Guo, P. Jia, E. Alshina (Huawei)]

See section 4.16.1.

JVET-AN0190 AHG17/AHG10: On test model simulating hardware encoder [Y. Zhao, J. Mao, R. Zhao, X. Ma, T. Solovyev, E. Alshina (Huawei)]

See section 4.16.1.

JVET-AN0298 AHG4: proposed updates for VVC multi-layer verification test plan [P. de Lagrange (InterDigital)] [late]

See section 4.5.

JVET-AN0348 AHG10: On reporting metrics for RPR [K. Andersson (Ericsson)] [late]

See section 4.4.

## AHG13: Film grain technology (1)

Contributions in this area were discussed during 1420–1450 on Thursday 9 Oct. 2025 (chaired by JRO).

JVET-AN0237 AHG13: Film grain analysis improvement [M. Radosavljević, F. Lefebvre, P. de Lagrange, Z. Ameur (InterDigital)]

This work presents several enhancements to the film grain analysis process. They include:

- Unified block size (16x16 blocks) used to estimate scaling function and to estimate cut-off frequencies (to speed-up calculations).

- Cut-off frequencies are now estimated by accumulating data over frames.

- Improved cut-off frequencies estimation algorithm.

- The number of different cut-off frequencies (different cut-off pairs) within the frame is limited.

Experiments were carried out on different film grain sequences, and the visual evaluation demonstrated that the estimated film grain parameters result in a higher fidelity representation of the film grain compared to the source.

(note that the sequence of the four changes is different in a version that was uploaded after presentation – the third and fourth bullet points are switched.)

The major change is in the improved cut-off frequency estimation algorithm. Based on a threshold, the cut-off may me switched off in cases where the estimation is likely wrong. In that case, the weight of the highest band is used up to the maximum of the frequency scale.

It was commented by one expert that in case of inputs that had been coded (even if at high rate) it might be necessary to adjust the threshold values. In the reported case however, the processing was done using the grained original and denoised sequences.

Cases were demonstrated where improvements over the previous estimation algorithm (which was already described in the latest draft of ed. 2 of the FG TR) were clearly visible.

Decision: Adopt JVET-AN0237, description to be added in JVET-AN2020, and software in VTM and HM.

## Implementation studies (0)

This section is kept as a template for future use.

## Profile/tier/level specification (1)

Contributions in this area were discussed during 1250–1310 on Friday 11 Oct. 2025 (chaired by JRO).

JVET-AN0293 Multiview 4:4:4 profiles for HEVC [S. Choi, S. Paluri, D. Podborski, E. Asbun, A. M. Tourapis (Apple)] [late]

This contribution proposes the creation of two additional multiview profiles in the HEVC specification, the Multiview 4:4:4 10 and the Multiview 4:4:4 12 profiles. It is asserted that such profiles can be quite useful for not only professional applications and content creators but also for consumer applications, such as stereoscopic screen casting and video-based 3D gaming. It is noted that the reference software implementation in contribution JVET-AN0292 also includes support for multiview 4:4:4 coding as previously requested by the group.

Main application would be for screen content coding, such as connecting devices and displays, e.g. for gaming. Based on range extension tools, where existing 4:4:4 decoders could easily be adapted.

It was suggested to consider also the possible definition of an 8 bit profile.

Merge request for software was already submitted, not merged yet.

General support is expressed. It is expected that a draft could be started after the merging of multiview and RExt tools in HM is finalized and verified.

Additional crosschecks of bitstreams are solicited.

## AHG15: Gaming content compression (2+1)

Contributions in this area were discussed during 1450–1600 on Thursday 9 Oct. 2025 (chaired by JRO).

JVET-AN0053 AHG4/AHG15/AHG17: 4K gaming sequences featuring “Black Myth: Wukong” [J. Sauer, Y. Zhao, Y. Sun, J. Zhou, E. Alshina (Huawei)]

See section 4.6.

JVET-AN0174 AHG15: Compression of gaming content using auxiliary data [J. Sauer, Z. Li (Huawei)]

A method was presented for the compression of gaming content that uses depth maps from the auxiliary data provided for class G1 of the gaming CTC (JVET-AJ2027) in order to generate additional frames that are used for prediction. The depth map is encoded and included into the bit-stream. For some sequences we achieve gains of up to 10% Y-PSNR BD-Rate vs VTM were reported, where both anchor and test utilized the reduced runtime 3 configuration.

Multi-layer coding where intermediate “virtual” layers are used for the generated frames used for the prediction.

Average gain is -0.7%, due to large loss in darktree sequence. In that sequence, the amount of data for depth map is extremely large, and loss is 17%.

The camera motion is known from the available metadata of rendering and depth maps, which is then used to generate a globally compensated picture for prediction. Camera parameters are also transmitted per picture which cannot be done by SEI message as the decoding process depend on them.

It was commented that in principle this has some similarity with 3D-HEVC, but the proposed approach does not conduct picture compression dependent on depth locally.

It was suggested to make depth and auxiliary information available along with those CfE sequences that have them. They could also be used for encoder optimization. The information about the formats of auxiliary data is available in the CTC document JVET-AJ2027.

The depth maps were originally floating point and quantized to 10 bit using znear/zfar. Processing for the new reference frame is done in FP.

Further study was recommended.

JVET-AN0263 AHG15: Transformer based depthmaps reconstruction for gaming content [V. Zakharchenko (Nokia)]

Sequences in classes G1/G3 of the JVET-AJ2027 Common Test Conditions (CTC) for gaming applications are accompanied with auxiliary data, including depth map. This contribution provides a study of derived depth maps precision for gaming content. It is observed that the depth map information can be matched to the results of the preliminary study of compressing the auxiliary depth maps as reported in JVET-AK0170. It is suggested to port the solution to NNVC code base for further study and evaluation.

The model has 21 million parameters, 85 Mbyte, 384 layers

Was the accuracy of the generated depth maps investigated in comparison to the precise ones investigated? A good criterion could be the reconstruction quality of a video generated from it, using a similar approach as in JVET-AN0174.

Might be applied at the encoder end or at the decoder end to avoid transmitting depth maps.

Original camera parameters were used. It would however also be possible to estimate them.

Depth maps were generated in full video resolution (HD). Downsampled depth maps might be generated to reduce complexity.

Multiple frames (three past frames) were used to get temporal consistency.

It was agreed to include a branch in the AHG15 software repository to make the software available to interested experts.

The AHG should investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available.

It was also commented that the usage of 3D models might be useful to improve the prediction in the gaming sequences which have other motion characteristics than camera captured content.

## AHG16: Generative face video (1)

Contributions in this area were discussed during 1400–1420 on Thursday 9 Oct. 2025 (chaired by JRO).

See also section 6.2.14.

JVET-AN0057 AHG16: Colour calibration post-processing for generative face video coding [S. Yin, Z. Zhang, S. Wang (CityUHK), B. Chen, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

This contribution proposes to add a colour calibration process to the generated face pictures as an optional post-processing to solve the occasional colour shift issue observed in some generated pictures. The colour calibration process utilizes the distribution parameters from the decoded key frames to adjust the pixel values of the generated inter frames, so as to mitigate the colour shifting in some generated frames and improve the temporal coherence of the reconstructed video. The colour calibration post-processing is implemented on top of all existing multi-resolution GFVC algorithms in AHG 16 software and the experimental results reportedly show that both objective and subjective quality can be improved without introducing additional bit consumption or any other normative change.

Based on DISTS and LPIPS BD metrics, CFTE and FOMM which are prone to colour deviation are improved in most cases (sometimes slight loss in LPIPS, larger gain in DISTS). Also the examples shown in the presentation (which showed sequences where the problem of colour deviation was more prominently occurring) showed that the post processing is able to bring the colour impression of generated frames closer to the key frame).

Decision (SW): Adopt JVET-AN0057 to the AHG16 branch of software.

## AHG17: CfE on video coding technology beyond VVC (20)

### General (4+2)

Contributions in this area were discussed during 1915–2100 on Tuesday 7 Oct. 2025 (chaired by XXX).

JVET-AN0041 AHG17: AhG meeting notes [M. Wien]

JVET-AN0052 [AHG17] [ Performance of GOP based RPR under CfE test conditions [Y. Sun, Y. Zhao, J. Sauer, J. Pardo, P. Jia, T. Solovyev, E. Alshina (Huawei)]

This document reports test results for GOP based RPR enabled in VTM software under CfE test conditions. Objective metrics results as well as visual examples are provided. Video examples are attached as part of the package.

It was requested in the presentation of the submission to re-issue the CfE with anchors using RPR, and evidence would be questionable as this had not been used.

Other experts disagreed with the statement that there would be no evidence, as various submissions did not use it.

It was commented that GOP-based RPR is a non-normative tool, and the upsampling step is not normative.

Several experts disagreed on using anchors with RPR, others agreed.

One expert commented that for CTC RPR should not be used, and better look into rate points where RPR does not give benefit.

For a CfP, requiring submission of higher rate point(s) would be useful, not to be investigated visually.

It was suggested that in a CfP two VTM encodings (one using RPR, one without RPR) could be used. As the anchor, the one without RPR should be defined, which marks the lowest quality. This was agreed.

It was commented that in typical bit rate ladders at least over a sequence at a given target rate is kept constant, whereas RPR can change.

It was commented that RPR might have more impact in the encoder run time category, and also reduce the decoder run time (or energy consumption)

The set of test sequences may need some alignment

JVET-AN0055 Crosscheck of JVET-AN0052 on Performance of GOP based RPR under CfE test conditions [X. Li (Google)] [late] [miss]

JVET-AN0243 Crosscheck of JVET-AN0052 (AHG17: Performance of GOP based RPR under CfE test conditions) [Z. Xiang (Tencent)]

JVET-AN0294 Crosscheck of JVET-AN0052 (Performance of GOP based RPR under CfE test conditions) [L. Li, J. Park, K. Choi] [late]

JVET-AN0053 AHG4/AHG15/AHG17: 4K gaming sequences featuring “Black Myth: Wukong” [J. Sauer, Y. Zhao, Y. Sun, J. Zhou, E. Alshina (Huawei)]

See section 4.6.

JVET-AN0157 AHG17: Valgrind complexity analysis on CfE encoder run time targets [S. Hong, Y. Tokumo, T. Ikai (Sharp)]

In standard development, decoder execution time has long been recognized as unreliable due to various noise factors. As part of AHG7's mandate, there is a request to “Develop methodology of more reliable runtime measurement.”. Experimental results indicate a strong correlation between decoder execution time and instruction references. This contribution asserts that the Valgrind based operation count is more reliable measure. Although the Valgrind decoding is roughly 60 times than usual decoding, this contribution suggests using Valgrind operation count can be beneficial to obtain a more reliable measurement of decoding operation in specific classes.

It was commented that the decoder run time is also highly dependent on memory accesses, cache misses, and in case of multi-core how many instructions can be performed in parallel.

It was asked why for instruction counting not better performance counter would be used, as this does not increase run time.

JVET-AN0177 AHG4/AHG17: Response to the call for new HDR materials for future video coding development [J. Wang, J. Zhang, L. Yu (ZJU)]

See section 4.6.

JVET-AN0189 AHG17/AHG10: On perceptual coding for next-generation video coding standard [Y. Zhao, A. Karabutov, T. Guo, P. Jia, E. Alshina (Huawei)]

NNVC and ECM have been developed with MSE-oriented optimization and evaluated with PSNR-BD-rate. It is suggested to consider perceptually oriented coding tools in the development of next-generation video coding standard.

In a future standards development, usage of more perceptual metrics than MS-SSIM is probably welcome in particular for evaluation. It might be dangerous to use for optimization. Ways must also be found to agree on a solution where results are persistent.

Also contributions on tools with perceptual benefit and perceptual encoder optimization would be welcome.

It was commented that the visual quality also highly depends on target displays.

It was commented that for generative approaches, also metrics like MSSIM and VMAF might not work.

Often (see example HDR) metrics contradict each other.

In the context of CfE responses, the correlation between visual and results and MS-SSIM/PSNR should be investigated.

Further study in AHG4 and AHG17.

JVET-AN0190 AHG17/AHG10: On test model simulating hardware encoder [Y. Zhao, J. Mao, R. Zhao, X. Ma, T. Solovyev, E. Alshina (Huawei)]

Hardware encoders have become increasingly important for video applications, and they are deployed on billions of mobile devices with constrained chip area size and power consumption. It is recommended to develop the next-generation video coding standard of a native design for hardware-friendly encoding. A test model developed with an encoding configuration that mimics some key characteristics of hardware encoder C model (besides a configuration for simulating software encoder) may facilitate more accurate evaluation on the performance of coding tools in the context of hardware encoder implementation and solicit hardware encoding friendly coding technologies.

Several experts supported the idea of putting more focus on hardware-friendly implementation. It was however commented that investigation of hardware complexity of encoders is more complicated than for decoders, as the degrees of freedom are much higher.

Examples might be limitation on number of RDO decisions needed, granularity of partitioning, memory access limitations.

Smartphones (or mobile devices in general) are mentioned as important. Also hardware transcoding is used a lot by content/service providers.

JVET-AN0118 presents a software module for counting RDO decision in ECM.

It was concluded to install a new AHG on hardware implementation complexity (Y. Zhao, L. Li, X. Li, …)

### CfE submissions and evaluation (11)

Contributions in this area were discussed during 0910–1330 and during 1415–1845  on Tuesday 7 Oct. 2025 (chaired by JRO).

JVET-AN0080 AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience [S. Ikonin, V. Khamidullin, I. Gribushin, M. Sychev, K. Malyshev, A. Dzugaev, A. Bovsha, X. Ma, E. Alshina (Huawei)]

This contribution describes the coding technology beyond VVC that can be used for ultra-low latency and packet loss resilient decoding scenarios. The described technology re-uses concept of data partitioning of H.263/4, which separates prediction and residual information into different sub-streams. Additionally, entropy coding of residual coefficients is performed using multiple independent entropy coded chunks. A lost aware NN-based filter is further applied to reduce subjective impact of lost residuals and propagation if these errors via inter frame prediction. The evaluation methodology following discussion and contributions of AHG18 is also proposed. According to this methodology the described solution provides average 2.8 dB improvement on top of scalable VTM anchor in broadcast scenario, and freeze rate reduction from 14.7% to 0.4% over single layer VTM anchor in unicast scenario.

NN filter not yet implemented in SADL.

Overhead is counted in NAL units.

It was commented that it might be appropriate to also use channel conditions of other networks.

Data partitioning and entropy coding would need to be normative. NN-based filter might not need to be defined normatively, as it does not change the output in the case of uncorrupted bitstream.

It was commented that a key question would be if all decoders would need to be burdened by tools that are only relevant for some applications.

In case of unicast, the feedback to encoder is at frame level, not packet level.

What is the granularity of data partitioning? Block (CU) level. However, partitioned information from multiple CUs is transported in separate NAL units. The positions of lost CUs are assumed to be known for further decoding operations.

The opinion is expressed that the testing method needs improvement.

JVET-AN0085 Response to the joint CfE on video compression with capability beyond VVC [M. Abdoli, P. Nikitin, F. Plowman, A. Tissier, R. G. Youvalari, M.-L. Champel (Xiaomi), Y.-J. Chang, C.-C. Chen, M. Coban, K. Cui, P. Garus, N. Hu, M. Karczewicz, J.-L. Lin, P.-H. Lin, X. Meng, B. Ray, V. Seregin, Y. Shao, G. Verba, H. Wang, E. Ye, R. Yu, Y. Zhang, Z. Zhang, W. Zhu (Qualcomm), F. Le Léannec, K. Naser (InterDigital), F. Wang (OPPO), Y. Kidani, H. Kato, T. Chujoh, K. Kawamura (KDDI), Z. Deng, K. Zhang, L. Zhang (Bytedance), A. Filippov, J. Konieczny, V. Rufitskiy, H. Qin, T. Dong, Z. Xu, K. Ding, C. Hollmann, I. Zupancic, D. Li (TCL), X. Xiu, X. Wang (Kwai)]

This contribution responds to the Call for Evidence (CfE) on video compression beyond the capability of Versatile Video Coding (VVC). To this end, Enhanced Compression Model (ECM) 18.0 was used to compare against the CfE anchor based on VVC Test Model (VTM) 23.11.

Three configurations are presented in this contribution, one for the evidence on improved compression and two for the evidence on improved compression with runtime-constrained encoding.

For Test 1, the following BD rates are reported relative to VTM 23.11: -25.15%, -30.29%, -33.72% (Y, U, V) for RA SDR, -21.49%, -50.44%, -58.76% (Y, U, V) for RA HDR and -20.31%,-37.13%,-32.96% (Y, U, V) for LDB..

For Test 2, the following BD rates are reported relative to VTM 23.11: -22.89%, -27.51%, -31.03% (Y, U, V) for RA SDR, -19.72%, -48.32%, -56.72% (Y, U, V) for RA HDR and -18.80%,-35.00%,-30.93% (Y, U, V) for LDB.

For Test 3, the following BD rates are reported relative to VTM 23.11: -25.00%, -33.87%, -36.26% (Y, U, V) for RA SDR, -21.95%, -50.41%, -58.57% (Y, U, V) for RA HDR and -20.32%,-38.35%,-33.75% (Y, U, V) for LDB.

Results show clear compression efficiency improvements over VVC, achieved by means of tools beyond those in VVC.

The response encodings are obtained by running the ECM-18.0 software in its default settings for unconstrained complexity test (Test 1). For constrained complexity tests, restricted partitioning configuration similar to AHG 17 is utilized in Test 2 and the LOP6 filter developed in JVET’s exploration experiment 1 is added on top in Test 3.

More specifically, partitioning parameters as follows are employed:

- MaxMTTHierarchyDepth: 2

- MaxMTTHierarchyDepthISliceL: 2

- MaxMTTHierarchyDepthISliceC: 2

- MaxMTTHierarchyDepthByTid: 221111

For the purpose of rate matching, as instructed in the CfE document, a static quantization parameter (QP) setting is applied, where a one-time change of the quantization parameter from value QP to value QP+1 is used in order to meet the defined target bit rates.

Test 1 (ECM default)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.11-default |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | VmPeakDec |
| SDR_RA_UHD | -30.42% | -34.75% | -38.29% | 1926.3% | 1560.0% | 341.1% | 369.1% |
| SDR_RA_HD | -21.22% | -29.84% | -35.58% | 1696.7% | 1248.7% | 341.1% | 372.0% |
| UGC_RA_HD | -24.26% | -27.60% | -29.44% | 1685.9% | 1020.0% | 350.4% | 369.7% |
| HDR_RA_UHD | -20.81% | -61.93% | -69.55% | 1523.8% | 1109.9% | 288.6% | 338.9% |
| HDR_RA_8Kcrop | -22.16% | -38.96% | -47.96% | 2043.5% | 1250.8% | 314.1% | 367.9% |
| Overall SDR | -25.15% | -30.29% | -33.72% | 1754.6% | 1220.2% | 345.0% | 370.2% |
| Overall HDR | -21.49% | -50.44% | -58.76% | 1764.6% | 1178.3% | 301.1% | 353.1% |
| Overall | -23.82% | -37.62% | -42.82% | 1758.2% | 1204.8% | 328.3% | 363.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-23.11-default |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak | VmPeakDec |
| SDR_LD_HD | -21.36% | -31.98% | -30.17% | 1445.8% | 1491.9% | 449.2% | 359.6% |
| Gamin_LD_HD | -19.27% | -42.28% | -35.76% | 1261.9% | 1137.3% | 462.5% | 398.4% |
| Overall | -20.31% | -37.13% | -32.96% | 1350.7% | 1302.6% | 455.8% | 378.5% |

Test 2 (ECM 5x)

|  |  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.11-default |  |  |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | VmPeak | VmPeakDec |  |
| SDR_RA_UHD |  | -28.25% | -32.39% | -35.79% | 883.0% | 1213.9% | 321.2% | 355.3% |  |
| SDR_RA_HD |  | -18.96% | -26.71% | -32.75% | 684.9% | 1216.2% | 340.6% | 372.0% |  |
| UGC_RA_HD |  | -21.93% | -24.78% | -26.70% | 795.5% | 993.9% | 349.9% | 368.6% |  |
| HDR_RA_UHD |  | -18.89% | -60.13% | -68.10% | 787.1% | 1116.8% | 288.4% | 339.8% |  |
| HDR_RA_8Kcrop |  | -20.56% | -36.51% | -45.34% | 1017.6% | 1051.2% | 315.1% | 368.1% |  |
| Overall SDR |  | -22.89% | -27.51% | -31.03% | 785.2% | 1114.8% | 338.9% | 365.7% |  |
| Overall HDR |  | -19.72% | -48.32% | -56.72% | 895.0% | 1083.5% | 301.5% | 353.7% |  |
| Overall |  | -21.74% | -35.07% | -40.37% | 823.5% | 1103.3% | 324.8% | 361.3% |  |
|  |  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  |  | Over VTM-23.11-default |  |  |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | VmPeak | VmPeakDec |  |
| SDR_LD_HD |  | -19.83% | -30.07% | -28.54% | 732.1% | 1038.5% | 460.7% | 395.2% |  |
| Gamin_LD_HD |  | -17.78% | -39.94% | -33.32% | 744.9% | 1137.0% | 461.4% | 399.1% |  |
| Overall |  | -18.80% | -35.00% | -30.93% | 738.4% | 1086.6% | 461.0% | 397.1% |  |

Test 3 (ECM 5x + LOP6)

|  |  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.11-default |  |  |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | VmPeak | VmPeakDec |  |
| SDR_RA_UHD |  | -30.54% | -37.74% | -39.45% | 998.0% | 3206.4% | 449.7% | 540.0% |  |
| SDR_RA_HD |  | -20.82% | -33.08% | -37.48% | 756.8% | 2668.0% | 448.6% | 511.5% |  |
| UGC_RA_HD |  | -24.09% | -31.81% | -33.32% | 907.5% | 2447.0% | 454.8% | 513.2% |  |
| HDR_RA_UHD |  | -20.88% | -61.42% | -69.31% | 967.7% | 2818.6% | 446.6% | 538.5% |  |
| HDR_RA_8Kcrop |  | -23.02% | -39.40% | -47.83% | 1140.9% | 4207.7% | 445.7% | 535.6% |  |
| Overall SDR |  | -25.00% | -33.87% | -36.26% | 885.3% | 2709.6% | 451.6% | 520.2% |  |
| Overall HDR |  | -21.95% | -50.41% | -58.57% | 1050.7% | 3443.8% | 446.2% | 537.0% |  |
| Overall |  | -23.89% | -39.88% | -44.38% | 942.2% | 2956.4% | 449.6% | 526.3% |  |
|  |  | Low delay B Main 10 |  |  |  |  |  |  |  |
|  |  | Over VTM-23.11-default |  |  |  |  |  |  |  |
|  |  | Y | U | V | EncT | DecT | VmPeak | VmPeakDec |  |
| SDR_LD_HD |  | -22.17% | -33.88% | -32.02% | 833.8% | 3441.6% | 546.7% | 522.6% |  |
| Gamin_LD_HD |  | -18.46% | -42.82% | -35.48% | 841.2% | 1955.3% | 547.5% | 531.4% |  |
| Overall |  | -20.32% | -38.35% | -33.75% | 837.5% | 2594.1% | 547.1% | 527.0% |  |

Run times of test 2 and test 3 are de facto larger than 5x VTM anchor.

It was commented that there were so many versions that it was difficult to verify.

Generally, results are somewhat better than the ECM5x generated by AHG17 (which also has larger run time than 5x VTM anchor).

JVET-AN0198 [AHG17] Wuhan University’s Response in Joint Call for Evidence on Video Compression with Capability beyond VVC [W. Zhang, L. Qin, X. Chen, N. Fu, H. Qu, W. Ma, J. Zhang, Z. Chen (Wuhan Univ.)]

This contribution describes Wuhan University’s response to the Joint Call for Evidence on video compression with capability beyond VVC. The goal of this response is to provide the evidence on the compression efficiency improvement over VVC Main-10 Profile. This response is based on ECM 17.0 and introduces neural network-based tools. This response covers three test categories: SDR RA UHD/4K, SDR RA HD, UGC RA. Relative to VTM 23.11 anchor, the response achieves an average BD-rate reduction of -37.37%/-40.48%/-42.93% for SDR RA UHD/4K category, -28.04%/-37.65%/-41.96% for SDR RA HD category, and -31.11%/-32.27%/-32.98% for UGC RA category on Y/U/V components, respectively.

NN-based tools: CV-based Deep Reference Frame, TTA-enhanced Neural Network Loop Filter (based on JVET-AH0205), both integerized in 32 bit.

Comparison of the proposal with VTM 23.11 anchor in terms of both coding performance and encoding/decoding complexity for SDR RA UHD/4K category

| Sequence |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| SDR RA UHD/4K | CrowdRun | -29.19% | -30.99% | -26.67% | 4844.7% | 3906276.2% |
|  | DrivingPOV3 | -35.98% | -34.59% | -56.16% | 10176.6% | 4046930.5% |
|  | FireDance | -39.21% | -38.84% | -35.50% | 6576.3% | 4105405.5% |
|  | HallwayScene | -45.10% | -57.51% | -53.38% | 9466.7% | 4225048.1% |
| Average |  | -37.37% | -40.48% | -42.93% | 7443.3% | 4069294.3% |

Comparison of the proposal with VTM 23.11 anchor in terms of both coding performance and encoding/decoding complexity for SDR RA HD category

| Sequence |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| SDR RA HD | DucksTakeOff | -20.31% | -64.46% | -76.01% | 3811.6% | 3807375.6% |
|  | DrivingPOV4 | -32.83% | -29.64% | -36.73% | 9174.7% | 4010982.4% |
|  | Seeking | -28.08% | -28.95% | -13.78% | 5864.2% | 4585305.9% |
|  | Umbrella | -30.95% | -27.54% | -41.31% | 4675.3% | 3989596.9% |
| Average |  | -28.04% | -37.65% | -41.96% | 5564.5% | 4088306.5% |

Comparison of the proposal with VTM 23.11 anchor in terms of both coding performance and encoding/decoding complexity for UGC RA category

| Sequence |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| UGC RA | Camellia | -33.78% | -23.20% | -26.09% | 7495.9% | 3135669.0% |
|  | Hobby-w5xz-backpack | -32.43% | -23.01% | -35.81% | 11167.0% | 4541791.9% |
|  | NightLandscapeRunning | -30.98% | -41.94% | -35.53% | 7788.4% | 3889414.2% |
|  | Sports-76a2-iceball | -40.16% | -26.56% | -34.40% | 15393.8% | 4547589.4% |
|  | VerticalVideo-3709-snow | -26.13% | -45.22% | -34.39% | 11725.6% | 4126232.7% |
|  | VerticalVideo-3d96-walk | -23.21% | -33.71% | -31.67% | 11712.5% | 4123926.6% |
| Average |  | -31.11% | -32.27% | -32.98% | 10549.3% | 4030397.9% |

Comparison of ECM 17.0 + CV-DRF with ECM 17.0 anchor in terms of both coding performance and encoding/decoding complexity for Class D

| Sequence |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Class D | BasketballPass | -3.57% | -4.11% | -3.09% | 117.5% | 77961.8% |
|  | BQSquare | -5.04% | -0.75% | -1.96% | 177.0% | 122508.6% |
|  | BlowingBubbles | -2.94% | -1.46% | -1.56% | 129.7% | 88721.6% |
|  | RaceHorses | -3.49% | -4.94% | -5.04% | 110.9% | 62709.1% |
| Average |  | -3.76% | -2.82% | -2.91% | 131.5% | 85379.2% |

Comparison of ECM 17.0 + CV-DRF + TTA-NNLF with ECM 17.0 anchor in terms of both coding performance and encoding/decoding complexity for Class D

| Sequence |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Class D | BasketballPass | -10.16% | -8.75% | -7.10% | 139.5% | 101860.9% |
|  | BQSquare | -11.59% | -10.77% | -17.49% | 222.5% | 147853.3% |
|  | BlowingBubbles | -7.39% | -2.10% | -1.28% | 158.1% | 104911.3% |
|  | RaceHorses | -8.45% | -10.83% | -11.93% | 127.9% | 78152.9% |
| Average |  | -9.40% | -8.11% | -9.45% | 158.3% | 105414.7% |

Complexity unrealistic at this moment (decoding of submitted bitstreams not yet finished, YUV files used in visual testing)

It was asked what the visual quality is and if artifacts appear that are different from conventional compression methods? Proponents think it is visually better, however only MSE used for optimization.

It was commented that it will be interesting to see visual results and compare to objective metrics.

Training material was not including sequences from CfE set.

Low delay was not run, as the DRF interface was only developed for RA, and network would have to be re-trained.

About 2000 kMAC/pix for DRF, 500 kMAC/pix for LF

No HDR results, not enough computing resources, and might require re-training.

DRF uses another version of optical flow than DRF in NNVC (CV=Cost Volume based)

It was commented that further investigation might reduce complexity.

JVET-AN0212 AHG17: Description of NNVC-based CfE response [D. Kim, S.-C. Lim (ETRI), T. Solovyev, J. Sauer, J. Pardo, P. Jia, A. Karabutov, E. Alshina (Huawei), H. Kwon, H. Ko (HYU), F. Galpin, T. Dumas, E. François (InterDigital), Y. Li, M. Karczewicz (Qualcomm), Z. Xiang, R. Chernyak, S. Liu (Tencent)]

This contribution summarizes and analyzes results obtained in the Call for Evidence (CfE) responses which were built as combinations of tools available in the recent NNVC tool-box. Responses are provided in the constrained encoder run time category. The goal of those responses is to demonstrate compression performance vs encoder complexity trade-off provided by learnable technologies. This response also serves as sanity check for technologies trained off-line, since at CfE performance is checked under test conditions substantially different from NNVC training and testing conditions. For target run time ‘0.2’ (which is twice faster than HM at CfE rates) performance of ‘VTM default’ was achieved, while at target run time ‘1’ 11% gain over VTM was demonstrated. Normative changes to the anchor are limited to just tool tools for all (except just one) of operation points.

Development principles of those CfE responses are as follows:

- Only tools available in NNVC-14.1 were used,

- SADL implementation for NN-based tools, int 16 quantized models,

- No multi-pass coding (content adaptive NN-based filters are not used),

- No pre-/post-processing other than MCTF.

Several encoder-only techniques such as time-distortion optimization (TDO) and filter restriction depending on hierarchical depth, were developed during this CfE response preparation which allow drastically reduce decoder run-time of NNVC tools with minimal performance effect.

Two types of NNVC bitstreams were generated as responses in encoder-constrained category:

- ‘5’ and ‘1’ not including RPR, actual encoder run time is ‘5.7’ and ‘1.1’ respectively.

- ‘0.2’, ‘0.5’, ‘1’, ‘2’ and ‘5’ with RPR enabled, actual encoder run time is ‘0.23’, ‘0.51’, ‘1.1’, ‘2.0’ and ‘5.4’ respectively.

Uses NN-based LF (LOP/HOP), NN Intra Pred, DRF from NNVC14

As encoder-only tools, TDO (JVET-AN0054) and NNLF usage restriction (JVET-AN0215) are used.

The graph above is average over all sequences from all categories (regardless of LB or RA)

What are expectations on visual quality? Only partial visual inspection was done, e.g. to make sure that no visual artifacts appear. RPR has good benefit on visual quality.

Why is no lowest run time point provided for the non-RPR version? Mainly because also RPR contributes to encoder run time reduction.

Which percentage of pixels is actually filtered? Can roughly be deduced from the columns TDO and max hier depth NNLF in the subsequent table:

Different constraints are applied to RPR and non-RPR cases. For LB, gain of RPR is smaller as A classes are not included where RPR gives most gain.

Complexity:

JVET-AN0267 Ericsson, Fraunhofer HHI and Nokia responses to Joint CfE on Video Compression with Capability beyond VVC [W. Ahmad, K. Andersson, M. Damghanian, V. Gritsenko, L. Litwic, D. Liu, M. Pettersson, V. Shchukin, R. Sjöberg, N. Stegmaier, J. Ström, N. Svensson, P. Wennersten (Ericsson), C. Bartnik, J. Brandenburg, B. Bross, V. George, J. Güther, G. Hege, A. Henkel, T. Hinz, G. Lazarov, C. Lehmann, W. Lim, Y. Liu, S. de Luxán Hernández, D. Marpe, V. Menon, T. Nguyen, J. Pfaff, S. Puttkammer, T. Schierl, H. Schwarz, B. Stallenberger, C. Stoffers, K. Sühring, A. Wieckowski, T. Wiegand, M. Winken (Fraunhofer HHI), P. Astola, S. Blasi, D. Buğdayci Sansli, C. Feldmann, D. Fortin, J. Funnell, S. Hong, I. Jumakulyyev, J. Lainema, N. Neumann, J. Ridge, D. Rusanovskyy, S. Schwarz (Nokia)]

This contribution provides a short description of the responses to the joint call for evidence from Ericsson, Fraunhofer HHI, and Nokia. Two responses have been submitted with the only difference of one having a Neural Network-based Loop Filter (NNLF) applied. NextSoftware2, i.e. the software used to generate the bitstreams, is based on VTM-20.0 with some VVC tools disabled and selected tools from ECM added. Bitstreams have been submitted for all test cases for evidence on improved compression without runtime-constrained encoding as well as for three runtime targets.

For all random-access (RA) and low-delay B (LB) test cases, the average YUV PSNR-based BD-rates and runtimes compared to the VTM anchor are:

NextSoftware2 (Submission 1)

RA: -13% YUV,   21% (Enc), 135% (Dec)		LB:    1% YUV,   20% (Enc), 103% (Dec)

RA: -22% YUV, 100% (Enc), 330% (Dec)		LB: -15% YUV,   92% (Enc), 337% (Dec)

RA: -24% YUV, 164% (Enc), 329% (Dec)		LB: -18% YUV, 154% (Enc), 345% (Dec)

RA: -25% YUV 322% (Enc), 331% (Dec)		LB: -20% YUV, 348% (Enc), 356% (Dec)

NextSoftware2 + NNLF (Submission 2)

RA: -16% YUV,  22% (Enc), 418% (Dec)		LB:   -3% YUV,   20% (Enc), 475% (Dec)

RA: -23% YUV, 99% (Enc), 613% (Dec)		LB: -16% YUV,   91% (Enc), 740% (Dec)

RA: -25% YUV, 163% (Enc), 614% (Dec)		LB: -19% YUV, 155% (Enc), 750% (Dec)

RA: -27% YUV, 317% (Enc), 618% (Dec)		LB: -21% YUV, 351% (Enc), 760% (Dec)

This document provides evidence that NextSoftware2 achieves relevant, objective compression gains over VVC at a wide range of encoder runtimes from 20% VTM to 350% VTM reference software runtime.

|  | NextSoftware2 | Sub1 | Sub2 |
| --- | --- | --- | --- |
| Software base | VTM-20.0 | x | x |
| Tools | Some VVC tools disabled | x | x |
|  | Selected ECM tools added (no template matching tools) | x | x |
|  | GOP-wise RPR from ECM with normative upscaling | x | x |
|  | VLOP-like NN-based LF | no | x |
| Optimizations | Clean implementation for selected ECM toolset including SIMD optimizations as applied in ECM | x | x |
| Functionality | Commonly used functionality of ECM kept and fixed, including: |  |  |
|  | Chunk-wise parallel encoding | x | x |
|  | RPR | x | x |
|  | Resume encoding based on an encoded bitstream | x | x |
|  | Tracing | x | x |
| Configurations | RA and LB with same intra period, GOP size and QP hierarchy as in ECM | x | x |
| Encoding runtime | Compression Efficiency: ~160% (RA) ~150% (LB) | x | x |
| targets relative to | Constrained (20% VTM): ~20% (RA) ~20% (LB) | x | x |
| VTM (average) | Constrained (100% VTM): ~100% (RA) ~90% (LB) | x | x |
|  | Constrained (500% VTM): ~320% (RA) ~350% (LB) | x | x |

The VTM base also has some non-normative (encoder optimization) changes.

ECM RPR is used which uses other upsampling filters.

Criteria for tool selection was benefit runtime vs. efficiency, which turned out to be different in some cases in ECM and in Next software.

How much gain is due to RPR? See document JVET-AN0362.

Software might be made available.

Why was a point selected for the “unconstrained” compression that has relatively low encoding and decoding run time? It was answered this was needed to be a compromise.

“normative upsampling” for RPR would conceptually mean that this is the conformance point.

It was commented that in a future standard development it might be useful to also have an encoder configuration which allows investigating the benefit of tools with constrained encoder runtime.

Could the software optimization also be applied to the VTM anchors? The next software would not be bitstream compatible to VVC.

Was optimization of tool selection made based on CTC or CfE? Both, and more. However, results under CTC are not available.

NNLF saves close to 3%.

It was suggested that for a CfP it would be useful to also request for performance at higher rate points, which might not be used for visual evaluation but getting additional information on performance by objective metrics.

SADL was used for VLOP implementation.

It was commented that the gain in Chroma is relatively larger. Therefore, a 6:1:1 weighting might be inappropriate.

JVET-AN0271 Software optimization and complexity reduction of ECM [Y.-J. Chang, C.-C. Chen, M. Coban, K. Cui, P. Garus, N. Hu, M. Karczewicz, P.-H. Lin, X. Meng, B. Ray, V. Seregin, Y. Shao, G. Verba, H. Wang, E. Ye, R. Yu, Y. Zhang, Z. Zhang, W. Zhu (Qualcomm) [late]

This contribution presents a reimplemented software based on ECM-18.0 and VTM-23.8. The software was optimized for speed and trade-offs between compression efficiency and runtime were improved.

Simulation results in RA configuration under CTC and CFE test conditions are summarized as follows.

CTC: -23.38% (Y), -24.17% (U), -26.92% (V); 681.1% (EncT), 742.1% (DecT),

CFE: -20.52% (Y), -33.15% (U), -38.72% (V); 1238.3% (EncT), 830.9% (DecT),

(Though this is not the software used in JVET-AN0085, it was commented by the proponent and a non-proponent that this contribution is more relevant for discussion within the context of CfE, together with contributions in section 4.16.2)

It was announced that the software could be made available – this should be done as a branch (with contribution number) of AHG12 software.

Macros to disable certain tools were also included in the software, as well as high-level flags.

Same config file as ECM is used.

Bitstream compatibility with ECM is not given.

JVET-AN0272 AHG17: ECM comparison points for CfE [K. Andersson, E. François, S. Hong, Y. Kidani, F. Le Léannec, X. Li, R.-L. Liao, Z. Lyu, F. Pu, V. Seregin, H.-B. Teo, A. Tissier, G. Verba, F. Wang, J.-R. Ohm, M. Wien, F. Bossen, M. Abdoli, E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, L. Li, P. Nikitin, D. Rusanovskyy] [late]

This contribution presents a summary of the ECM comparison point generation for the Call-for-Evidence tests on video compression with capability beyond VVC, for information – no need for detailed presentation at the meeting was apparent.

JVET-AN0303 External memory bandwidth evaluation for CfE response JVET-AN0267 [C. Feldmann (Nokia)] [late]

This document provides NextSoftware2, VTM and ECM evaluation results of decoder external memory access operations with various cache configurations. NextSoftware2 is the software proposed in the joint CfE response of Ericsson, Fraunhofer HHI and Nokia (JVET-AN0267). This evaluation uses Valgrind as in JVET-AM0295 which measures external memory access operations for the whole decoder.

When measured using valgrind, ECM presents an increase in memory access operations compared to VTM of 707% to 2142% depending on the cache size. NextSoftware2 shows a much lower overhead between 184% and 827%.

The measurement for ECM includes all numbers, including group 1 (TM tools). It was commented that it would be interesting to compare against ECM with group 1 off (as by AHG7 study).

JVET-AN0318 AHG17 – Review of CfE responses [K. Naser, F. Le Léannec, F. Galpin, E. François (InterDigital)] [late]

This contribution provides an overall review of the CfE responses in this meeting cycle. The 4 sets of responses reported in JVET-AN0085, JVET-AN0198, JVET-AN0212 and JVET-AN0267 are reviewed and their performance trends are analyzed. This review can be useful in discussing the performance of these responses and for the further work of defining a potential CfP.

The analysis of the different responses to the CfE may lead to the two following main comments.

- Many of the UHD contents turn out to be easy to encode. It may be necessary in view of a potential CfP to consolidate the test set with more challenging UHD test content.

- In some CfE responses, Adaptive Resolution Coding was used and shows quite good performance. This good performance is also strongly linked to the limited encoding complexity of the test content. Considering ARC for CfP would require taking care of also using it for the anchors.

It was commented that it has always been the case that higher resolution sequences can be more efficiently compressed, and challenging test material needs to be carefully selected. This is also the case for 8K. Also RPR typically works better at higher resolutions.

JVET-AN0360 AHG17: Information on coding efficiency and complexity [L. Li, W. Choi, M. Park, M. Budagavi, K. Choi (Samsung)] [late]

This contribution collected and illustrated coding efficiency vs. computational complexity for both encoder and decoder. The trade-off points are from responses of Call for Evidence (CfE) and ECM comparison points (ECM, ECM2x, ECM5x). Additional trade-off points outside scope of CfE response are also included in section 3. Section 4 shows trade-off points over RPR enabled VTM 23.11.

Following observation is made:

- More submissions in runtime constraint category than compression efficiency category. However, only encoder runtime is considered, and for some points, decoder runtime is off the chart.

- Majority of submissions disabled TM, and almost half of them enabled RPR.

- Enabling RPR not only brings coding efficiency but also decreases encoding time and decoding time. Since RPR hides runtime complexity, it is difficult compare between submissions.

- Submissions that use TM seem to have significantly increased decoding time. Decoding complexity implications of TM needs to be studied carefully.

Points of coding efficiency vs EncT

Points of coding efficiency vs DecT

It is also assessed that BD gains are reduced by approx. 5% Some conclusions drawn by authors:

- More submissions in runtime constraint category than compression efficiency category. However, only encoder runtime is considered, and for some points, decoder runtime is off the chart.

- Majority of submissions disabled TM, and almost half of them enabled RPR.

- Enabling RPR not only brings coding efficiency but also decreases encoding time and decoding time. Since RPR hides runtime complexity, it is difficult compare between submissions.

- Submissions that use TM seem to have significantly increased decoding time. Decoding complexity implications of TM needs to be studied carefully.

JVET-AN0361 AHG17: Report on CfE testing [M. Wien, F. Bossen, Ch. Lehmann, V. Baroncini]

Initial results presented Tue 7 Oct. 1425.

Test on Compression capability

Examples from SDR UHD

Examples SDR HD

LB

HDR 4K

HDR 8K

Gaming

It was commented that gaming content is somewhat more difficult to view, and for gaming DCR might have been better to know how it should look like.

UGC

From the compression efficiency test case, the following initial conclusions can be drawn:

- Technically, all submissions are based on VTM, ECM, and/or NNVC, or variants thereof with modifications of encoder or some modification of coding tools (no new coding tools were included)

- Majority of cases (roughly 2/3) where at least one submission has non-overlapping confidence interval with VTM

- In a number of cases, submissions are better in quality than the ECM16 comparison point

- Some proposals show clear improvement in compression versus VTM with substantially less encoder/decoder complexity than ECM

- In various cases (including challenging ones), NN based technology showed clear visual benefit (more than PSNR wise)

- Clearly larger difference for 8K HDR, and less difference for gaming (average rate saving per category?)

From the encoding run time test case, the following initial conclusions can be drawn:

- Significant number of cases (roughly 1/2 for 1x, more for 0.2x) where at least one submission has non-overlapping confidence interval with VTM

- Even if the encoder of a submission is faster than VTM encoder, the decoder may be slower (where one submission comes with only 35% increase).

JVET-AN0362 Information on the impact of GOP-based RPR on JVET-AN0267 in context of CfE repsonses [C. Lehmann, A. Wieckowski, B. Bross] [late]

This document reports results without GOP-based RPR for the first submission of the NextSoftware2 described in JVET-AN0267. The results are put in context of other CfE responses (JVET-AN0085, JVET-AN0212, JVET-AN0198) and additional information on VTM with GOP-based RPR (JVET-AN0052) as well as ECM improvements (JVET-AN0271).

Results reporting JVET-AN0267 with RPR are rate-matched. In JVET-AN0052 the closest QP point with lower rate was used. It might be concluded that RPR benefits more from RPR than JVET-AN0267.

### Considerations towards CfP (5)

Contributions in this area were discussed in the joint  meeting and during 1600–2000 on Thursday 9 Oct. 2025 (chaired by JRO).

JVET-AN0056 On Next Generation Video Codec Call for Proposal Timing [S. Wenger (Tencent)]

See notes in section 7.3.1.

JVET-AN0204 AHG18: Request on inclusion ultra-low latency and packet loss resilience category into CfP [S. Ikonin, X. Ma, E. Alshina (Huawei), S. Wenger (Tencent)]

See notes in section 7.3.1.

JVET-AN0330 Proposed modification of CfE test conditions E. [Alshina (Huawei)] [late]

See notes in section 7.3.1.

JVET-AN0249 Suggestions for a potential upcoming CfP [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

This input contribution discusses a potential upcoming Call for Proposals (CfP) for a video coding standard with capabilities beyond VVC. Three items are discussed and advocated for in the proposal:

- Not include general preprocessing and postprocessing in the scope and in CfP responses

- Include compression-level dependent preprocessing and postprocessing in the scope and in CfP responses

- Make CfP subjective test results non-anonymous with possibility to opt-out

The following proposal are made with the intention of addressing the three items above:

- In the coding conditions text of the CfP:

  - Disallow (or strongly discourage) the use of preprocessing and postprocessing that does not depend on the level of compression (e.g., QP)

  - Explicitly allow (or encourage) use of preprocessing and postprocessing that depends on the level of compression (e.g., QP)

- In the subjective test:

  - Ask the question “How close is the processed video to the original?” rather than “What is the level of the visual quality of the video?”

- For submissions to the CfP:

  - Provide submitters with results of the subjective test prior to the meeting in which all results are anonymised except their own submission and the anchors.

  - Provide two options for submitters:

    - Have their results removed from the subjective test and not published

    - Have their results published with the subjective test in a non-anonymous manner

The following text is proposed for aspect 1 (pre/post processing):

- Use of preprocessing and/or postprocessing that does not depend on the level of compression (e.g., QP) is discouraged.

- Use of preprocessing and/or postprocessing that depends on the level of compression (e.g., QP) is encouraged.

The first bullet is stronger than the following bullet from CfE

- Use of preprocessing and/or postprocessing shall be described.

The second bullet may be practically covered by the following bullet in CfE:

- Sequences are expected to be encoded at full input resolution for all rate points. If coding at reduced resolution is part of the algorithmic concept, it shall be described.

It was commented on the first bullet that, in a DCR test, methods that would like to improve the appearance of the output, would be deviating from the original.

It was commented on the second bullet that it is not difficult to make anything QP dependent.

It was suggested to encourage, if pre or post processing is applied, also to report results where the same is applied to the anchor.

This could be extended to some sentence of the body of the text:

“If any methods are applied in preprocessing, postprocessing or encoder optimization, it is encouraged to provide also results when applying equivalent methods to the anchor. Such information may also be requested during evaluation at the meeting to be provided.”

It was agreed to include such sentence, but otherwise keep the formulations at this moment as they were in the CfE.

It was suggested to check the formulation on low delay conditions, if it is clear enough that the sentence “Overall structural delay shall not be larger than that of the respective anchor” shall also include pre and post processing.

Aspect 2 (subjective test) is obsolete as DCR (impairment) will be used.

Aspect 3 (making results anonymous optionally):

A comment was made that it might be better to keep the current practice. Results would not be included in public report, if not desired by proponents, and anonymous documentation in intermediate presentations in JVET is hardly interpretable by people outside JVET.

A comment was made by the test coordinator that the option of not including results in the initial presentation may be difficult to implement.

It was commented that the “binary” decision might enforce unnecessary decisions to opt out.

Several other experts also expressed that the current practice should be retained.

No action taken on aspect 3.

JVET-AN0301 AhG17 Sharing experience of conducting JPEG AI Call for Proposals  [E. Alshina (Huawei)] [late]

Documents shares information about organization call for proposals on learnable image codec technologies in JPEG. Hopefully this experience can be taken into account and help to resolve several issues observed during Call for Evidence on video compression with capability beyond VVC.

JPEG AI call for proposals was conducted using multiple steps.

1.	Call for proposal was issued. Document describes evaluation methodology, anchors, training set, but doesn’t specify test set. Only CfP responses for which decoding on different devices (namely on GPU and on CPU) result in matching reconstruction were seriously considered, the rest to submissions were taken into account just as information. Among 10 submissions only 5 met this criterion.

2.	Registered proponents uploaded decoder package (which includes all model parameters) and then write access was closed (up-date of decoder or model parameter no longer was no longer possible).

3.	Several days after decoders freeze evaluation set, selected by experts, not participating in CfP, was specified and announced. This might be too much for video coding standard CfP, it should be enough to built secret test set (announced after decoder freeze) which consist of one video sequence per each class (HDR/SDR, UHD/HD, random access and low-delay coded). Those secret test sequences can be selected by visual test coordinator(s) taking into account how good they are for visual evaluation.

4.	Proponents were given limited time to encode secret testing set and match target rates. Different credentials were given to uploaded streams, again write access was closed after deadline. This really helps to resolve a mess with multiple versions of uploaded streams. Limited time for rate matching naturally limits encoder complexity.

5.	After both streams and decoders are submitted and frozen, the cross-checkers for each response was assigned by chairs (randomly assigned). Cross-checkers were mandated to decode streams using CPU and GPU and check difference in reconstruction. Distributed cross-check reduced loading; device interoperability cross-check increases the quality of submissions.

6.	Submission which failed in a cross-check are considered for information only.

The following is proposed:

- Close write access after decoder submission deadline.

- Conduct CfP evaluation using (at least partially) secret test set, which is announced after decoder freeze.

- Close write access after streams submission deadline.

- Randomly assign cross-checker to reduce loading. Mandate cross-checker to verify device interoperability.

- Consider implementation and testing algorithms under consideration on mobile device in parallel with standard development as one of the most realistic ways of complexity evaluation.

It was commented that, if SADL was used for NN implementation, no mismatch typically occurs.

The following was agreed :

An additional set of sequences would be beneficial, likely to be selected by the test coordinator and the JVET chair after delivery of decoder binaries and bitstreams.

In the CfP a statement needs to be added that it is mandatory to report which material was used to train NN or other trained approaches.

The aspect about kMAC, number of parameters, number of layers, etc. should be part of the complexity questionnaire.

It was pointed out that the additional set would need to be large enough to unveil relevant information. It needs more consideration about practicality, or potentially not include it in subjective testing.

During 1900-2000 on Friday 10 October, a subsequent discussion was held about possible other changes to CfP conditions and test set.

Adding higher rate points also for proposals appears useful to also be able investigating the gain in the higher quality range (based on objective metrics, as quality may be close to transparent). For this, it would not be required to have precise rate matching, to be useful e.g. for additional BD rate computation in the higher rate range.

During the joint meeting, some concern had been raised about inappropriateness of certain sequences.

On possible aspects to improve the test set, the following comments were made:

- Hallway scene has a very low rate, but on the other hand seems good for visual testing, also has noise. Several experts supported keeping it.

- Fashion lady has a very low rate, due to the fact that fine details are rare (except for some areas); on the other hand, it was commented that it has local gradients which is also important to be properly reconstructed in HDR

- It might be useful to find a replacement for one of the Gregory sequences (should be replaced by another sequence taken directly from a smartphone camera without compression). If a replacement is found, the scarf sequence which needs higher bitrate should be kept.

- Gaming sequences are all produced with outdated rendering. Level1 (priority) and GTAV could be candidates for replacement, the latter being too simple. GTAV was however mentioned to be relevant for low latency testing (could be kept in a dedicated category when the functionality of ultra low latency / error resilience would be included in CfP).

- Office walk might be a candidate for removal (not optimum for viewing), but a good replacement would need to be found.

- The lowest rate for Beatriz may be too low, otherwise it is a good test sequence.

- Some 8K sequences (except Waterfall Forest and Chandelier) might better be downsampled to 4K. Chandelier has relatively low rate. On the other hand, concern is raised this would make the 8K category almost empty. No replacement should be made if no better material would be available.

- It might not be necessary to have 6 sequences in UGC.

- Several experts commented that higher bit rate sequences would be desirable.

Comments and agreed changes on sequences and conditions were collected during the session on Friday 10 October in an edited version JVET-AN2026 of the previous CfE, with the intent to consider those in generating an initial version  of the CfP document during the AHG period.

## AHG18 Ultra-low latency and packet loss resilience (8+2)

Contributions in this area were discussed during 1400–1700 on Wednesday 8 Oct. 2025, and  during 1110–1300 on Thursday 9 Oct. 2025 (chaired by JRO).

JVET-AN0045 AHG18: Codec software for ultra-low latency and packet loss resilience [S. Ikonin, V. Khamidullin, K. Malyshev, I. Gribushin, X. Ma, E. Alshina (Huawei)]

This contribution includes the software related to document JVET-AM0218 “AHG18: Solution beyond scalable coding for ultra-low latency and packet loss resilience” presented and 39th JVET meeting in Daejeon, Korea. The video demonstration related to the contribution was also presented during the demo session. The presented ULL coding technology was comprehensively discussed with experts, and many experts expressed the interest about the details. After further polishing and in order to complement AHG18 research activity the software is provided with this contribution. The software is provided as a git patch on top of VTM-based simulation software used in AhG18. It is suggested to include the software into exploration experiments in context of AhG18 work.

It was commented that this investigation does not necessarily need an exploration experiment, could well be done in an AHG study.

It was also commented that more clarification about the needs and use cases of applications would be required. There are other cases than the mobile transmission scenario that is focused here. A larger variety of error conditions matching

It was asked if it is not necessary to investigate with longer sequences, as network losses can be considered as a statistical process. Currently, focus is on simulating problematic cases.

In a possible CfP, it would be relevant that the exact pattern of errors would not be known by the time of submission.

JVET-AN0082 AhG18: Crosscheck report for JVET-AM0195 [V. Zakharchenko (Nokia)]

Cross-check related to a contribution of last meeting – it was confirmed that no issue was found in terms of results reported. The bug fix on GDR (non-normative) is asserted appropriate.

JVET-AN0339 Crosscheck of JVET-AM0195 (AHG18: Gradual Decoding Refresh under ultra-low latency test scenario) [E. Mora (VITEC)] [late]

Results are confirmed. It was commented that in cases of affine, GPM and SbTMVP candidates some more improvements might be desirable.

Decision (SW): Merge software from JVET-AM0195 to AHG18 branch.

JVET-AN0046 AHG18: On test conditions for ultra-low latency and packet loss resilience with full QP range [S. Ikonin, I. Gribushin, V. Khamidullin, B. Shevchenko, E. Alshina (Huawei)]

In 39th JVET meeting several comments and suggestions concerning ULL CTC were discussed, mainly based on contributions JVET-AM0109, JVET-AM0202, JVET-AM0204, JVET-AM0222, JVET-AM0325. This contribution follows up this discussion and addresses the comments in aspects of LDP/LDB configuration, broader QP range, spatial scalability, new packet trace files and automatic bandwidth rate points selection for matching channel capacity to video bitrate.

Was already presented in AHG meeting, but discussed again.

The basic problem that this contribution tries to address is the fact that it is tried to simulate cases where errors occur, and if the same network model would be used for QP22 and QP40, the latter would be less affected by errors due to lower rate. It is noted that a similar effect is used when using spatial scalability where the base layer has lower rate and is prioritized

One comment was made that it is difficult to judge the appropriateness of the error patterns suggested for the different QP values. It was also pointed out that it might be useful to look into similar investigations made in other bodies such as 3GPP.

Further study in AHG.

The basic problem that this contribution tries to address is the fact that it is tried to simulate cases where errors occur, and if the same network model would be used for QP22 and QP40, the latter would be less affected by errors due to lower rate. It is noted that a similar effect is used when using spatial scalability where the base layer has lower rate and is prioritized

One comment was made that it is difficult to judge the appropriateness of the error patterns suggested for the different QP values. It was also pointed out that it might be useful to look into similar investigations made in other bodies such as 3GPP.

The contribution also includes the software patch for spatial scalability which requires further confirmation by an independent party.

JVET-AN0049 AHG18: Teleconference on ultra-low latency and packet loss resilience [S. Ikonin, S. Wenger, V. Zakharchenko (co-chairs), S. Deshpande, J. Ström, X. Ma, C. Kim, S. Puri, S. Fößel (vice-chairs)]

No need for presentation.

JVET-AN0050 AHG18: An Introduction to 5G Latency/Throughput Measurement and Real-Time Monitoring for ULL Application Scenarios [B. Jeon (TTA), S. M. Kim (LGUplus)]

This contribution reports experimental results of latency and throughput measurements in a 5G mobile network. It also describes a method for monitoring latency in real time. The method is based on measuring the inter-arrival time of periodically transmitted packets. This approach can be applied to systems that need to transmit video for real-time safety over mobile networks. It is also considered advantageous for supporting ultra-low latency application scenarios in MPEG.

No review was conducted. This document is for information, and the contributor informed the JVET chair that he is unavailable for presentation this time. He will re-submit to the next meeting.

JVET-AN0079 AHG18: Proposed methodology and test conditions for ultra-low latency and packet loss resilience performance evaluation [S. Ikonin, I. Gribushin, X. Ma, E. Alshina (Huawei), S. Deshpande (Sharp)]

In the past several JVET meetings, several documents discussing aspects related to AHG18 were discussed, the related techniques and test conditions were discussed comprehensively. This contribution proposes an evaluation methodology and suggests test conditions based on the discussion targeting further AHG18 work. Comparing with the current AHG18 test condition, which only contains Wi-Fi connection case, the 5G network constraints are additionally proposed, full VTM tool set is enabled, with a wide bitrate range, LDB and spatial scalability testing conditions are further supported. The contribution also suggests the objective quality metrics calculation methodology, accompanied with an excel template for quality plots construction and summary results collecting.

It was suggested to follow a methodology where the error patterns are randomly selected after bitstreams have been generated. It is also suggested to use several different error patterns and compute the average performance (provided that a reasonable objective metric exists).

Some concern was raised on the aspect of encoder configuration. It was agreed to use reduced runtime 3 for the unicast case (as it simplifies the feedback reaction), and use the default VTM settings for spatial scalability.

At this moment, the section 4 about metrics is left is left open. In the future, various metrics might be used, such as variation of errors over a frame, finding of extremely distorted regions, etc. Some investigation about potential existing metrics might be useful.

On the aspects of possible other feedback mechanisms, and usage of  GDR rather than  IRAP for refresh, see further notes under JVET-AN0089 and JVET-AN0254.

In an update of the document, the current GDR implementation is mentioned as an alternative with IRAP, and the aspect of metrics is left open.

Formulation for section 4: “One metric that may optionally be reported is PSNR. Reports using other metrics to demonstrate the benefit are welcome.”

Furthermore, the Excel sheet should be modified such that quality over rate for a given latency can be plotted, as well as quality over latency for a given rate point.

It was agreed to use the modified JVET-AN0079 as basis for the first version of CTC for ULL/ER

1610-XXXX Presentation of CfE results by M. Wien:

All at QP 22 (single layer or scalable enhancement layer), delay

Broadcast scenario (scalability)

Crowd run (left) is a sequence where also the base layer had a freeze.

Unicast scenario:

There seems to be at least some coherence with PSNR values by tendency, where often PSNR curves are closer than MOS curves. A scatter plot would be helpful.

One participant reports that he found it difficult to decide whether frame freeze at good quality or a smoothed reconstruction was better. It might be useful not testing unicast and broadcast (scalability) scenario in the same session.

It was suggested that a better methodology might be to make an A/B comparison.

Another option might be to run a DCR test where the uncorrupted decoded video is compared against the concealed version and measure impairment.

Generally, it appears useful for visual testing not going into the higher QP range where larger compression errors occur, as the goal is to judge the benefit of an error resilience/concealment method.

In general, it appears possible to test visual benefit of error resilience/error concealment also with relatively short sequences under the assumption that just the behaviour in corrupted cases is included. It might be useful to test different error patterns to guarantee that enough variability of error cases is given.

JVET-AN0080 AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience [S. Ikonin, V. Khamidullin, I. Gribushin, M. Sychev, K. Malyshev, A. Dzugaev, A. Bovsha, X. Ma, E. Alshina (Huawei)]

See section 4.16.2.

Subsequent documents were reviewed on Thursday 9 Oct.

JVET-AN0081 AHG18: Software fixes for robust decoding in unicast scenario [K. Malyshev, V. Khamidullin, M. Sychev, S. Ikonin, E. Alshina (Huawei)]

The ultra-low latency simulation scenario of AHG18 considers decoding robustness under possible network packet losses. These may cause the unavailability of the referenced APS NAL units and breaking the decoding process. To ensure robust decoding four aspects are considered, and specific modifications are proposed: signalling in the slice header to provide a robust CTU-level decoding process; new logic to skip the ALF process when the referenced APS NAL unit is unavailable; functionality to obtain standalone APS NAL units apart of VCL NAL unit; and the new command line option BitsForPoc for encoder to enable the long reference case. These modifications were used in JVET-AN0080 “AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience” for anchor and test generation.

F. Bossen suggests to implement aspect 4 also in VTM main branch. It was suggested to call it BitsForPocLsb.

Decision (SW): Adopt JVET-AN0081. Include all 4 aspects in AHG18 software branch, aspect 1 disabled by default. Include aspect 4 in VTM main branch.

JVET-AN0089 AHG12/AHG18: On GDR test condition for video compression beyond VVC [T. Chujoh, Y. Kidani, K. Kawamura (KDDI)]

This contribution proposes to study a Gradual Decoder Refresh (GDR) test condition for video compression beyond VVC. GDR is well-known for its practical method of realizing ultra-low latency video transmission and has been adopted into VVC. For video compression beyond VVC, GDR has been studied using the early version of ECM, but GDR does not work in the latest version of ECM, i.e., ECM-18.0. Therefore, there appears to be insufficient discussion on the GDR test condition for video compression beyond VVC in the current JVET ad-hoc groups. To study the feasibility of GDR for video compression beyond VVC, this contribution reports the experimental results where GDR can be activated on ECM-18.0 with specific constraints and discusses the test conditions of GDR.

Multiple slices would need to be used to further reduce the latency. It was commented that a similar test condition should be implemented in VTM, and comparison should also be performed against VTM.

It should be tested if ECM still works with multiple slices.

It was asked which value was used for PH recovery POC count? Same as VVC.

Software is not yet fully implemented.

It was commented that in the current implementation of GDR in VTM (non-normative aspects), only configuration with one reference picture is supported.

Further study is highly desirable on GDR, as this is crucial for low delay. Periodic IDR would introduce buffering delay, which could introduce additional data loss.

JVET-AN0204 AHG18: Request on inclusion ultra-low latency and packet loss resilience category into CfP [S. Ikonin, X. Ma, E. Alshina (Huawei), S. Wenger (Tencent)]

See section 4.16.3.

JVET-AN0254 AHG18: Packet marking mechanism for unicast scenario with channel feedback [V. Zakharchenko (Nokia)]

During activities of the AhG18 it has been emphasized that the usage of the feedback protocol is one of the important features of the error mitigation in the ultra low latency scenario. This contribution provides a study of an integration of the L4S protocol into the simulation software to enable channel feedback.

The advantage in comparison with the current feedback mechanism is that it will be known to the encoder which parts of the picture are unavailable. The modification of encoder software to take action on this might be more difficult to implement, but the actual processing might be less.

It was commented that introducing network-specific packetization mechanisms in the reference software would be undesirable. This is rather an aspect at systems level.

It was commented that it would be useful to get report about the benefit before taking action.

It was commented that the protocols proposed are not yet commonly deployed, and some further developments e.g. in IETF could be expected.

It was commented that the information about single packets lost might not be very useful, as typically the remaining part of a slice is lost if a single packet loss occurs.

Another option would be to include priority mechanisms and scalability also in the current unicast scenario, such that action of re-encoding only is necessary in case of base layer losses.

Further study on these aspects.

## CICP (1)

Contributions in this area were discussed during 1140–1200 on Friday 10 Oct. 2025 (chaired by JRO).

JVET-AN0070 CICP enum for generic subsampling and sample location [L. Barnes, D. Podborski, A. Tourapis (Apple)] [late]

At the 34th JVET meeting, Apple presented a proposal (JVET-AH0217)) for adding codec agnostic signalling of subsampling and chroma location to T-REC-H.273. The proposed text got added to the Future CICP extensions document (JVET-AM1005) at the 39th JVET meeting. This contribution proposes some minor editorial changes to the text to better align it with the HEIF 3ed FDAM2 draft.

The changes are assessed to be appropriate.

Decision: Include the changes proposed in JVET-AN0070 in JVET-AN1005.

# Low-level tool technology proposals (98)

## AHG11/AHG14: Neural network-based video coding (27)

### Summary and BoG reports

Contributions in this area were discussed during 0900–1050 on Saturday 4 Oct. 2025 (chaired by JRO).

JVET-AN0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)]

Code base for the EE1 tests was NNVC14.0, anchor is default configuration of NNVC-14.0 (NN-Intra and LOP6 filter enabled). NNVC common test conditions, results and complexity reporting template were be used.

For proposals in all categories, proponents used AhG11 training set, which consists of DIV2K, BVI-DVC, TVD. NN-based Inter tools were trained using Vimeo 90K triplet. Comparison is done between tests which use the same sub-set of training data.

For tests competing with technologies in NNVC it was agreed to configure the proposed solution targeting the complexity close to existing NNVC tool, but not exceeding it:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- the number of channels (both input and output) in neural network modules which are modified must be keep multiple of 16,

- Number of Parameters EE1 test  Number of Parameters NNVC (if possible).

If it is not possible to respect both constrains 1) and 2) at the same time then two sub tests need to be performed: first test respecting the constrain 1) only – in order to prove the value of the proposed change, and the second test respecting constrain 2) with minor violation of 1)   - in order to provide software and hardware friendly design.

According to the EE description only tests with results provided to quantized (int 16) model are considered for adoption to NNVC, float point model results are provided just for information.

Exact parameters settings were announced by proponents by 2nd AhG11/14 teleconference on August 28.

Inference cross-check is required for all EE1 tests. Candidates for adoption to NNVC are required to undergo training cross-check, implementation must be compatible with SADL.

This round of EE1 tests includes:

EE1-1: LOP in-loop filter

EE1-2: VLOP in-loop filter

EE1-3: NN-inter prediction

EE1-4: NN-based super-resolution

Anchor for ALL tests is NNVC-14.0 with NN-Intra and LOP6 enabled (default configuration).

EE1-1: LOP in-loop filter

- EE1-1.1 – LOP with Overlapped Feature Integration  JVET-AN0127 (UESTC, Transsion) (crosscheck JVET-AN0281)

Key changes in head block of LOP filter are additional operations over Luma reconstruction and prediction input:

- Boundary Cropping: Crop 1 pixel from the left/right columns and top/bottom rows of the original block, retaining the central (W-2)×(H-2) region.

- Mirror Symmetric Padding: Restore the original block size by padding mirrored values from the cropped boundaries.

- 2×2 DCT Transformation: Apply 2x2 DCT-II to the padded block.

The changes introduced by proposed solution (on a right) relatively to LOP6 (on a left) are highlighted in the diagram below:

| ecExtY 1,144,144]; PredExtY 1,144,144; …. DCT; DCT. 4,72,72; 4,72,72. CONCAT(d1, d2,d3, d4, d4, d5).  d1+ d2+ d3+ d4+ d4+ d5 = d7,72,72 . CONV, 11, d7, d6. PReLU. dwCONV, 31, d6, d6, 2.  d6,36,72 . dwCONV, 13, d6, d6, 2.  d6,36,36 . CONV, 11, d6, 2C. PReLU. 2C,36,36 . | RecExtY 1,144,144]; RecExtY 1,144,144; PredExtY 1,144,144; PredExtY 1,144,144; …. DCT; Overlap DCT; DCT; Overlap DCT. 4,72,72; [4,72,72; 4,72,72; [4,72,72. CONV 33, 16, d1, 2, groups=2. CONCAT(d1, d2d3, d4, d4, d5). d1+ d2+ d3+ d4+ d4+ d5,72,72. CONV, 11, d6, C. PReLU. CONV, 13, C, C, 2. C,72,36. CONV, 31, C, C, 2. C,36,36. CONV, 11, C, C. PReLU. C,36,36. |
| --- | --- |

Ablation study:

- EE1-1.1.1: retraining LOP6 w/o change of architecture, use changed learning rate.

- EE1-1.1.2: only change architecture w/o overlapped feature.

- EE1-1.1.3.1: DCT II added to Sub-test 1.1.2 (full proposed package).

- EE1-1.1.3.2: DCT II added to Sub-test 1.1.2 (full proposed package), use changed learning rate.

Complexity measured in kMAC/pxl remains close to the LOP6 (16.57 kMAC/pxl), number of parameters are not reported, training time (only stage 3) is 7 days. PSNR drop for SlideShow is 0.4% (all-intra cfg test).

Encoder run time is unreliable, decoder run time claimed to be reliable, indicates reduction of decoding time (10%) due to SIMD acceleration for the group convolution (not yet available in SADL, but proposed in JVET-AN0128) and aligning channels number with multiple of 16.  Cross-checker reported status of cross-check via e-mail:

- for inference:

  - successful inference xcheck of EE1-1.1.1, EE1-1.1.2, EE1-1.1.3.1 and EE1-1.1.3.2 under AI configuration, all results matched; and

  - all tests under RA configuration are still under-going and results are not available.

- for training:

  - as requested by proponents, only training of EE1-1.1.3.2 was cross-checked

    - successful training xcheck of EE1-1.1.3.2 under AI configuration, {Y: 0.01%, U: -0.16%, V: 0.12%} compared with JVET-AN0127.

    - RA configuration are still under-going, RA results are expected to be obtained within 5 days.

- EE1-1.2 –Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels  JVET-AN0083 ((training cross-check JVET-AN0335 )

Re-parametrization is only training modification. Three variants of over-parametrized modules were used during training, but in the inference, model returned to the original network structure. Training time increase 10h – 190 h.

Normative changes are in the chroma branch, to provide BBB(x) with richer input information, the proposed method increases the number of input channels of BBB(1) and gradually reduce the number of input channels of BBB(0) accordingly. The number of input channels of the five BBB(x)s in the chroma branch gradually decreases from 128 to 64.

SIMD acceleration for the depth-wise separable convolution at the boundary was provided. This acceleration is also applicable to the anchor. Run time reported relatively to accelerated NNVC-14.0.

Computational complexity reduced 16.60 -> 16.56 kMAC/pxl.

Training was successfully cross-checked (two runs of training ended up in very close performance).

EE1 LOP in-loop filter modifications

|  | All intra cfg |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| NNVC-LOP6 | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% |
| EE1-1.1.1 | 0.0% | 0.1% | -0.0% |  | 100% | 0.1% | -0.2% | -0.4% |  | 100% |
| EE1-1.1.2 | -0.1% | -0.1% | -1.0% |  | 92% | -0.1% | -0.3% | -1.1% |  | 90% |
| EE1-1.1.3.1 | 0.0% | 0.1% | -0.0% |  | 91% | 0.1% | -0.2% | -0.4% |  | 89% |
| EE1-1.1.3.2 | -0.1% | -0.6% | -0.7% |  | 92% | -0.2% | -0.9% | -0.9% |  | 88% |
| EE1-1.2 | -0.1% | -0.6% | -0.1% | 99% | 102% | -0.1% | -0.7% | 0.2% | 100% | 103% |

It was commented that gain of both 1.1 and 1.2 is relatively low (within the 0.1% margin of training uncertainty). Basically, both could be combined, but then re-training in combination would be necessary.

For 1.1, training cross-check is not finalized yet. It shows an interesting decoding runtime reduction, which however is due to SIMD optimization in SADL which might likewise be applied to the  anchor.

For 1.2, both inference and training cross-check are finalized. A concern was raised how much of the gain comes from over-parameterization using the new training strategy, and how much gain is due to the architectural change.

1.2 should be further investigated in an EE on the aspect of benefit of the architectural change. It appears that the overparameterization could stabilize the training of LOP6 also without architectural change (which was not tested so far).

It was commented that the reported gain of both 1.1 and 1.2 appears too small to justify architectural changes. Proponents might consider if a combination could be beneficial to be investigated in an EE.

1.1.1 is retraining of LOP6 without architectural change.

Further consideration of variant 1.1.3.2 was planned after completion of training cross-check and potential confirmation of the slightly higher gain (to be compared against 1.1.1 which does not apply an architectural change).

Further discussion Friday 10 Oct. 12:00

In an update of JVET-AN0023, more results are reported that after completion of training cross-check, the results on AI were within the usual margin, but in RA a larger deviation was observed.

Cross-check JVET-AN0281 not yet available.

Further study was planned in EE1.

EE1-2: VLOP in-loop filter

- EE1-2.2 – Decomposed Content-Adaptive VLOP JVET-AN0153 (registered but not uploaded cross-check JVET-AN0282)

The comparison for this test is caVLOP4 (content-adaptive VLOP4).

Claimed performance benefits over caVLOP4 are as follows.

EE1 cVLOP in-loop filter modifications

|  | Random Access |  |  |  |  | kMAC/pxl |  |  | Num Param, M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Total | Filter | Intra | Total | Filter | Intra |
| NNVC-caVLOP4 | 0.0% | 0.0% | 0.0% | 100% | 100% | 9.9 | 5.2 | 4.8 | 1.4 | 0.074 | 1.3 |
| EE1-2.2 | -0.2% | -0.2% | -0.4% | 100% | 100% | 9.9 | 5.2 | 4.8 | 1.4 | 0.074 | 1.3 |

Performance of ‘VLOP’ family in-loop filters in NNVC vs VTM (NNVC all tools off)

| vs fixed NNLF | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | DecT |
| caVLOP4 (JVET-AL0169) | -0.6% |  | -4.9% | -4.3% | 196% | 102% |
| dcaVLOP4 (JVET-K0311) | -1.0% |  | -4.4% | -4.6% | 195% | 99% |

Comments from cross-checker: inference cross-check matched, training is still ongoing.

Gain for the content-adaptive VLOP4 appears interesting. However, the relatively large increase in decoding time is of concern, which appears larger than with the current CA-VLOP1 implementation. This may be an implementation issue, or may be caused by the decomposition approach. One reason may be that the model needs to be re-compiled and re-loaded each time it is changed.

Further consideration on Friday 10 Oct. after completion of cross-check. The proponents were asked to investigate why the decoding runtime is more significantly increased than for the existing CA-VLOP1 implementation. In the crosscheck, the gain is confirmed. After further investigation, it was found that the decoder runtime is accurate and does not significantly increase (in the initial report, an error was made in the reference to compare run time)..

Decision: Adopt JVET-AN0153 test EE1-2.2.

EE1-3: NN-inter prediction

Anchor for tests in this category is NNVC-14.0 with NN-Intra and LOP6 enabled (default configuration).

Deep Reference Frame (DRF) was adopted to NNVC at JVET 39th meeting. It is disabled by default.

Results for DRF in NNVC are provided as reference. As was agreed during NNVC teleconference adopted version of DRF is not enabled in LDB cfg (model was not trained for those purposes).

- EE1-3.1 – Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AN0193 (training and inference cross-check JVET-AN0304)

Significant reduction of model size and number of multiplications by using light version of DRF and structural re-parameterization. Training uses sophisticated loss: CharbonnierLoss + TernaryLoss. Training set is Vimeo-90K triplet. Training takes approx. 80 hours.

Cross-checker performed re-training, after re-training deviation from proponent’s results in average is {0.15%, -0.53%,0.64%} BD-rate for {Y, U, V} components, but bigger difference (0.3%) is observed for classes A2 , 0.14% for A1 and B, for other classes within usual margin.

- EE1-3.2 – RA/LDB Reference Frame Synthesis for VVC Inter Coding  JVET-AN0084 (cross-check JVET-AN0306 ongoing)

Training set is Vimeo-90K triplet, Loss is L1. Training uses video data compressed with QPs 7,12,17,22,27,32,37,42). Training takes approx. 120 hours. Decoding is still in progress (partial data available).

Cross-check status has been reported by cross-checker via e-mail:

- Inference crosscheck failed for LDB (decoder crash)

- Inference crosscheck for RA was not bit exact (they have floating point operations in their inference code), there are also a crash in Class A

- Training crosscheck for RA model was done – Chroma results varied by 1.21%. Luma was okay.

- Inference crosscheck of float models did not show the same results. Maybe the models they provided were wrong.

- EE1-3.5 – Retrained DRF in NNVC14.0  JVET-AN0195 (training and inference cross-check JVET-AN0305)

During porting of adopted DRF version to NNVC, colour conversion process was modified compared to original version, so inference was different from training. Retraining was needed to align training and inference. Training takes approx. 50 hours.

Cross-checker performed re-training, after re-training deviation from proponent’s results is negligible (below 0.1%).

EE1 DRF family of tools in NNVC

|  |  | Random Access |  |  |  |  |  | Low-delay-B |  |  |  |  |  | kMAC/pxl |  |  | Num Param, M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | DecT | Y |  | U | V | EncT | DecT | Total |  | NN-Inter | Total |  | NN-Inter |
| NNVC-LOP6 | 0.0% |  | 0.0% | 0.0% | 100% | 100% | 0.0% |  | 0.0% | 0.0% | 100% | 100% | 21 |  | 0 | 1.5 |  | 0 |
| NNVC-DRF | -1.9% |  | -1.2% | -0.8% | 145% | 2256% | NA |  |  |  |  |  | 525 |  | 504 | 5.3 |  | 3.8 |
| EE1-3.1 | -1.3% |  | -0.6% | -0.8% | 111% | 776% | NA |  |  |  |  |  | 70 |  | 69 | 4.4 |  | 2.9 |
| EE1-3.2 | -2.5% |  | -0.5% | -0.9% | 135% | - | -1.9% |  | 0.5% | 3.0% | 158% | - | 608 |  | 487 | 4.3 |  | 2.8 |
| EE1-3.5 | -2.2% |  | -1.5% | -2.2% | 149% | 2474% | NA |  |  |  |  |  | 525 |  | 504 | 5.3 |  | 3.8 |

EE1-3.1 would be interesting as additional operation point with lower complexity of DRF.

It was reported by proponents of EE1-3.1 that they are currently are trying to identify the problems leading to partial deviation in training crosscheck. The cross-check was not finalized by the end of the meeting, and will be finalized in further EE study.

EE1-3.2 is immature (not to be continued in an EE)

Decision: Adopt JVET-AN0195 EE1-3.5 (re-training the current DRF model).

EE1-4: NN-based super-resolution

Per agreement during the second NNVC teleconference test conditions for NN-based super-resolution tests are aligned with RPR in NNVC:

NNSR testing conditions are:

- Multiple scaling ratios are used (x1.5 and x2)

- Reduced resolution is allowed for QP>=32 (base QP)

- Enabled only for 4K classes

- QP offset for chroma is the same for A1 and A2 (no per sequence adaptation).

EE1-4.1   NNSR with new backbone block based on Spatial-Channel Mixing (SCM) JVET-AN0201 (registered, but not yet available cross-check JVET-AN0277)

The key element is a new backbone block for NNSR that performs spatial mixing followed by channel mixing. No run time provided. Some (0.1…0.4%) BD-rate gain in Chroma compared to NNSR in NNVC-14.0.

Two variants are provided with C_Y1=32 (kMAC/pxl 4.05, Nparam 17k) and C_Y1=16 (kMAC/pxl 3 Nparam 12k).

- EE1-4.2 Cross-component enhanced NNSR JVET-AN0238 (cross-check JVET-AN0256)

Proposed changes enhance feature interaction between luma and chroma components. Some (0.1…0.4%) BD-rate gain in Chroma compared to NNSR in NNVC-14.0. Increase of kMAC/pxl compared to NNSR in NNVC-14.0

Anchor for tests in this category is NNVC-14.0 with NN-Intra and LOP6 enabled (default configuration).

Results (generated by AhG14) for RPR and NN-SR in NNVC are provided as reference.

Cross-check of inference – match, after re-training by cross-checked  Chroma BD-rate drop 0.1..0.2%.

|  | Random Access |  |  |  |  | All Intra |  |  |  |  | kMAC/pxl |  | Num Param, M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Total | NN-SR | Total | NN-SR |
| NNVC-LOP6 | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% | 21 | 0 | 1.5 | 0 |
| NNVC-RPR | -0.5% | 1.3% | 1.0% | 96% | 90% | -0.5% | 3.4% | 3.4% | 98% | 81% | 21 | 0 | 1.5 | 0 |
| NNVC-NNSR | -0.6% | 1.3% | 1.0% | 97% | 90% | -0.8% | 3.1% | 3.0% | 100% | 81% | 26 | 4.7 | 1.7 | 0.02 |
| EE1-4.1.1 | -0.6% | 1.2% | 0.9% |  |  | -0.8% | 2.7% | 2.7% |  |  | 25 | 4.0 | 1.6 | 0.02 |
| EE1-4.1.2 | -0.6% | 1.2% | 0.9% |  |  | -0.8% | 2.7% | 2.7% |  |  | 24 | 3 | 1.6 | 0.01 |
| EE1-4.2 | -0.6% | 1.2% | 0.8% |  |  | -0.8% | 2.7% | 2.6% |  |  | 26 | 5.2 | 1.5 | 0.03 |

Run times are reported to be similar to current NNVC-NNSR.

EE1-4.1.2 could be interesting, having significant complexity reduction compared to current NNSR without impact on gain. After completion of cross-check JVET-AN0277, it was found that the significant gains in class A1 that had been reported by proponents could not be confirmed, the cross-check reports slight losses. For other classes, the method did not change much in terms of compression results. Further investigation in an EE was planned.

EE1-4.2 is not attractive, no benefit in compression, slight complexity increase compared to current NNSR.

JVET-AN0042 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

### EE1 contributions: Neural network-based video coding (8)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AN0023. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AN0083 EE1-1.2: Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels [J. Han, C. Jung, Q. Qin (Xidian Univ.)]

JVET-AN0335 Cross-check of JVET-AN0083 (EE1-1.2: backbone block enhancement of LOP in-loop filter with over-parameterized training and variable channels) [T. Dumas (InterDigital)] [late]

JVET-AN0084 EE1-3.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding [Q. Qin, C. Jung (Xidian Univ.)]

JVET-AN0306 Crosscheck of JVET-AN0084 (EE1-3.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) [N. Bhaskar (Huawei)] [late]

JVET-AN0127 EE1-1.1: LOP with Overlapped Feature Integration [J. Chi, A. Li, Y. Du, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)]

JVET-AN0281 Crosscheck of JVET-AN0127 (EE1-1.1: LOP with Overlapped Feature Integration) [Z. Xu (TCL)] [late]

JVET-AN0153 EE1-2.2: Decomposed Content-Adaptive VLOP4 [Z. Xu, J. Konieczny, A. Filippov, C. Hollmann, V. Rufitskiy, T. Dong, H. Qin (TCL)]

JVET-AN0282 Crosscheck of JVET-AN0153 (EE1-2.2: Decomposed Content-Adaptive VLOP4) [J. Chi, A. Li, Y. Du, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)] [late]

JVET-AN0193 EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization [W. Zhang, N. Fu, X. Chen, W. Ma, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AN0304 Crosscheck of JVET-AN0193 (EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization) [N. Bhaskar (Huawei)] [late]

JVET-AN0195 EE1-3.5: Retrained DRF in NNVC-14.0 [X. Chen, N. Fu, W. Zhang, W. Ma, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AN0305 Crosscheck of JVET-AN0195 (EE1-3.5: Retrained DRF in NNVC-14.0) [N. Bhaskar (Huawei)] [late]

JVET-AN0201 EE1-4.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) [H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)]

JVET-AN0277 Crosscheck of JVET-AN0201 (EE1-4.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM)) [T. Yang, W.-X. He, Y.-Q. Zhu, X.-T. Xie, J.-S. Gong, Q.-M. Wang, Q. Liu (HUST), Z.-Y. Lv (vivo)] [late]

JVET-AN0238 EE1-4.2: Cross-component enhanced NNSR [T. Yang, W.-X. He, Y.-Q. Zhu, X.-T. Xie, J.-S. Gong, Q.-M. Wang, Q. Liu (HUST), Z.-Y. Lv (vivo)]

JVET-AN0256 Crosscheck of JVET-AM0238 (EE1-4.1: Cross-component enhanced NNSR) [H. Cho, S. Bahk, T. Lee, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)] [late]

### EE1 related and beyond-EE contributions: Neural network-based video coding (12)

Contributions in this area were discussed during 1110–1245 and 1430–1800 on Saturday 4 Oct. 2025 (chaired by JRO).

JVET-AN0054 AHG14: Extension of the QP range for TDO Lambda in NNVC S/W [H. Kwon, H. Ko (Hanyang Univ.), D. Kim, S.-C. Lim (ETRI)]

This contribution proposes an NNVC S/W modification of the time-distortion optimization (TDO) Lambda configuration to support a wider range of QP values, which can be used for Call for Evidence (CfE) condition, where relatively higher QP ranges are needed. The TDO in the NNVC S/W skips the neural network-based loop filtering if the number of boundary strength (BS) partitions is less than a BS partition threshold. However, the BS partition threshold can have different optimal values for each slice. Thus, the threshold is determined by calculating the trade-off between distortion and the number of filtered blocks. To determine the optimal threshold value at the encoder, the proper Lambda value corresponding to the slice QP should be used. However, in the NNVC-14.1 S/W, the same Lambda is applied for QP values above 42, which restricts the use of QP-dependent optimized Lambda values, especially at high QP values. To address this issue, this contribution makes the QP range more flexible for TDO Lambda configuration in NNVC.

This makes the current non-normative TDO more flexible, by adding parameters that are currently hard coded in software to the parameter file. In CTC, nothing would change, as TDO is disabled.

Decision (SW): Adopt JVET-AN0054 (non-CTC). Also needs update of the description in JVET-AN2019.

JVET-AN0172 AHG11: Proposal of normative TDO method as an alternative option in NNVC [H. Kwon, H. Ko (Hanyang Univ.), D. Kim, S.-C. Lim (ETRI)]

This contribution proposes the adoption of a normative time-distortion optimization (TDO) method as an alternative option in the NNVC S/W, with the aim of minimizing the drop in coding performance. The TDO method skips NN-based filtering according to boundary strength (BS) partitions. The number of BS partitions is determined from the BS map, which is the input of the NN-based filter, and filtering is skipped if the number of BS partitions is lower than the BS partition threshold. Unlike the non-normative TDO method, the BS partition threshold is signalled in the slice header, which can reduce signalling overhead caused by block-level filtering on/off flags. The experimental results {Y, Cb, Cr | EncT, DecT} of the normative TDO method on top of NNVC-14.1, are reported as follows:

RA: {Y: 0.05% Cb: -0.10% Cr: -0.09% | EncT: 101% DecT: 78%},

LDB: {Y: -0.13% Cb: -1.19% Cr: -0.75% | EncT: 100% DecT: 70%}

It was commented that by disabling the option of block-level on/off control (and replacing it by a threshold based on BS) might have an impact that an encoder could no longer control potential visual artifacts locally.

It was asked how the threshold is determined? Similar as in non-normative method.

It was agreed to investigate this in an EE, but report also results for VLOP and HOP.

JVET-AN0209 AhG 11/AHG 14: Harmonization of Deep Reference Frame (DRF) with RPR in NNVC14.1 [N. Bhaskar, T. Solovyev, E. Alshina (Huawei)]

In NNVC14.1, it was observed that when the tools Deep Reference Frame (DRF) and RPR were enabled in combination, the encoder crashed for some of the test sequences. Several bugs in the DRFG software were detected and fixed. These bugs occurred in relation to the DRF frames when RPR is applied. The fixes are available as a merge request. This input contribution explains details of the modifications needed to enable simultaneous functionalities DRF and RPR in NNVC.

This contribution resolves the problem that DRF so far requires pictures of identical size to generate the new reference. RPR filters are used for re-sizing.

It is noted that this was not included in the CfE response JVET-AN0212, as it would be a normative change of the NNVC software.

Decision: Adopt JVET-AN0209 (non-CTC). Software coordinators will investigate appropriateness of the change by the time of merge request.

JVET-AN0215 AhG11: Filter restriction depending on hierarchical depth [F. Galpin, E. François (InterDigital)]

In NNVC software, several neural-network based loop-filter (NNLF) are available, targeting different complexity and gains. Moreover, additional algorithms on encoder side are also available to control the complexity at decoder, namely TDO (Time Distortion Optimization) based on different criteria (distortion, boundary strength etc.) and targeting block level control. An additional criterion is proposed based the hierarchical depth of the frame to encode, targeting both encoder and decoder complexity. For NNVC 14 anchor (using LOP6 NNLF), it allows to vary encoder complexity from x1.1 to x1.2 while decoder complexity vary from x3 to x27 wrt VTM, while gains vary from {-4.6%,-11.3%,-9.5} to {-8%,-14.9%,-13.5%} for RA configuration. For LDB configuration encoder complexity varies from x1 to x1.1 and decoder x9 to x30 with gains varying from {-4.5%,-11.3%,-11%} to {-6.1%,-9.7%, -9.6%}. The method has been used in CfE responses using NNVC in order to control the encoder and decoder complexity tradeoffs.

The decision of using the filter is basically dependent on the position in a GOP, using the depth parameter of VTM.

Decision (SW): Adopt JVET-AN0215 (non-CTC). Also needs description in JVET-AN2019.

JVET-AN0202 [AHG11/AHG14] Comparison of Multi-Layer and Single-Layer Interfaces for Hybrid End-to-End Video Coding Frameworks [N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A. B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia)]

This contribution compares two hybrid E2E video coding frameworks that integrate end-to-end learned intra-frame compression (LIC) methods with conventional compression techniques. Among the two hybrid frameworks, one adopts a single-layer interface, whereas the other employs a multi-layer interface. With these two hybrid frameworks, under the Random-Access configuration, the resulting BD-rates over NNVC-7.1 VTM (with NN tools off) anchor and NNVC-14 VTM (with NN tools off) anchor are respectively reported to be as follows:

The framework with single-layer interface compared to NNVC-7.1 VTM (with NN tools off):

Overall  	-0,45 % (Y), -3,11 % (Cb), -1,31 % (Cr)

The framework with multi-layer interface compared to NNVC-7.1 VTM (with NN tools off):

Overall  	4,47 % (Y), 0,26 % (Cb), 1,79 % (Cr)

The framework with single-layer interface compared to NNVC-14 VTM (with NN tools off):

Overall  	0,37 % (Y), -2,44 % (Cb), -0,62 % (Cr)

The framework with multi-layer interface compared to NNVC-14 VTM (with NN tools off):

Overall  	5,33 % (Y), 0,96 % (Cb), 2,51 % (Cr)

It was commented that the difference is not only due to the overhead of the multi-layer stream, but likely also caused by different approach of RDO for the single-layer and multi-layer investigated in this contribution.

The multi-layer approach also allows a residual on the intra picture. A more fair comparison might be to configure the encoder such that the residual is skipped.

It was commented that there were bug fixes in referencing between base and enhancement layer between 14.0 and 14.1 (and some later fixes from JVET-AN0225, which may not have ben merged yet).

JVET-AN0216 [AHG11] Update on Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec [F. Urban, Y. Chen, F. Galpin, P. de Lagrange (InterDigital)]

In this contribution, results are presented using the framework integrated into the NNVC software to support an external base layer, proposed for hybrid codecs that combine End-to-End Learned (E2E) image compression methods with conventional compression methods such as VVC in JVET-AL0243. The framework is based on high level modifications of VVC to allow external picture coding in a base layer. The multi-layer capability of VVC can then be used transparently to allow the use of such pictures inside VVC, while allowing the encoder to do low-level RD choices in the enhancement layer. The main advantage of this approach is that it allows the use of an external picture without defining explicitly the external codec, while allowing the use of a standard VVC decoder on the enhancement layer.  The framework is demonstrated with intra pictures encoded with VTM as external base layer using the NNVC software. The syntax overhead of the external base layer compared to single layer is minor.

The contribution investigates the amount of syntax overhead by testing single layer VVC versus multi-layer VVC with IDR pictures as base layer. The estimated rate increase appears to be less than 0.2%.

It is noted that there is a small difference in the reconstructed I pictures due to different usage of ALF in single and multi layer cases, which could effect some very small gains in some cases even if the rate is increased.

The overhead is typically larger for the smaller resolution classes. For class A1 the overhead is larger than A2, caused by the smaller intra period of campfire.

JVET-AN0225 [AHG11][AHG14]: Further Improvements on Hybrid Multilayer Framework for End-to-End Learned Intra Frame [M. Aderdor, T. Solovyev, E. Alshina (Huawei)]

This contribution reports the recent progress in the development of the hybrid codec exploration branch of NNVC, which allows usage of an end-to-end coded I-frame in order to improve compression performance. The first version of the multi-layer framework did not provide the expected compression benefits compared to the previous proposal of an end-to-end coded I-frame, which uses a single layer in the VTM. In this contribution, the difference is narrowed down only to high level syntax associated with multi-layer. This contribution describes the details of modifications and proposes harmonization between ‘multi-layer’ and ‘single layer’ frameworks. Further software changes are proposed to improve performance further.

The presented bug fixes and proposals increase overall PSNR BD rate of multilayer hybrid VTM DCVC-FM vs. VTM 23.11 from:

AI: 		-4.75%	(Y) 	-11.24% (U)	-10.60% (V)

RA: 	5.69% (Y)		1.00% (U)		3.25% (V)

to:

AI: 		-5.31%	(Y)		-11.58% (U)	-10.80% (V)

RA:	-2.23%	(Y)		-7.06%	(U)		-4.43% (V)

Some bug fixes were implemented in the context of this contribution on top of NNVC 14.1, caused by inconsistencies in RPL, and problems occurring when GOP size was not identical with intra period.

It was asked to include software with the bug fixes in an update of the contribution.

Further changes proposed are enabling LMCS in E2E base layer (shifting some chroma gain to luma, which however may depend on the training of the E2E network), and enabling MCTF (which was not used before, but not disabled for technical reason). This provides a gain of approximately 2% (most comes from MCTF).

As an additional result, an average overhead of 0.4% is reported for the multi-layer approach in AI (on average over all classes, by tendency larger for smaller resolution).

Run time reported in the xls is unreliable.

The usage of LMCS is asserted to be useful for flexibility in experimentation with AI based codecs for intra pictures. Enabling MCTF is just an issue of config file.

Residual in enhancement layer was not skipped, but probably only used for high rate.

Decision (SW): Implement LMCS as a configurable option from JVET-AN0225 in the NNVC multi-layer interface with external base layer.

Complexity around 500 kMAC/pix.

JVET-AN0224 [AHG11] A Hybrid Framework Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. Hallapuro, F. Cricri, H. Zhang, A.B. Koyuncu, J. Ahonen, M. M. Hannuksela (Nokia)]

This contribution proposes a hybrid framework that integrates end-to-end learned intra-frame compression (LIC) methods with conventional compression techniques. The framework involves using LIC-coded intra frames and VTM-coded inter frames. Furthermore, for each intra frame, the encoder decides whether to code it with LIC or with VTM. Two sets of results are provided, depending on whether the LIC was optimized by means of perceptual finetuning (PFT). With this hybrid framework, under the Random-Access configuration, the resulting BD-rates over NNVC-7.1 VTM (with NN tools off) anchor are reported to be as follows:

The system optimized with MSE and rate losses (no perceptual finetuning):

Overall  	-0,92 % (Y), -3,94 % (Cb), -2,30 % (Cr)

Additionally, the Random-Access simulation results are evaluated with 8 perceptual metrics, the resulting perceptual BD-rates over NNVC-7.1 VTM (with NN tools off) anchor are reported to be as follows:

|  | AVG | msssim Torch | vif | fsim | nlpd | iw-ssim | vmaf | psnrHVS | lpips |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W/O PFT | -6,04% | -6,99 % | -5,76 % | -6,08 % | -5,21 % | -6,74 % | -5,52 % | -4,16 % | -5,98 % |
| W/ PFT | -10,10 % | -13,29 % | -8,12 % | -8,72 % | -7,67 % | -11,93 % | -6,15 % | -4,88 % | -14,81 % |

The frame-level RDO is either using MSE (where it gives 0.9% luma gain compared to 0.4% without switching from JVET-AN0202). With perceptual metrics, the gain becomes 6%, but it is not known how much benefit the switching gives on perceptual metrics. PFT becomes worse for PSNR, but better for perceptual metrics (where the switching also includes MSSIM criteria).

For training, both MSE and MSSIM were used. 5 models for different QP were used, trained for the intra QP settings of RA.

Gain is zero for AI, as CTC uses different QPs

Exact numbers about usage of VVC vs E2E are not provided. Reportedly, highly depends on QP.

Complexity is around 1000 kMAC/pixel.

Overall discussion:

- The multi-layer framework that was included by last meeting had various problem in implementation that were solved at least partially.

- It is still assessed that the multi-layer interface still provides most flexibility to investigate with different E2E codecs, and use adaptation mechanisms globally and locally.

- The coding gain reported so far may not justify the impact on complexity. On key aspect might be the better optimization of adaptation, RDO, etc.

- More coordination appears necessary to further investigate and enhance the interface. A dedicated AHG11/14 telco (4 weeks after the meeting) could be useful.

JVET-AN0255 AHG11: VLOP3 with new backbone block based on Spatial-Channel Mixing (SCM) [H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)]

This contribution proposes a new backbone block based on Spatial-Channel Mixing (SCM) for VLOP3. In the proposed method, the backbone block uses a depth-wise 3×3 convolution for spatial mixing and 1×1 pointwise convolutions that first expand and then reduce the channel dimension for channel mixing. These methods have been implemented based on VLOP3 of NNVC-14.1 using SADL float.

The BD-Rate results over VLOP3 of NNVC-14.1 for float are as follows:

- AI: {0.08%	-0.89%,	-1.10%}, DecT: 85%

- RA: {0.08%, -1.32%, -1.29%}, DecT: 77%

- LDB: {0.02%, -3.75%, -3.90%}, DecT: 78%

Complexity over the VLOP3 of NNVC-14.1 is as follows:

- Parameter Counts: 67.9k  68.2k (proposed)

- MAC per Pixel: 5.11k  5.04k (proposed)

It was asked why the decoding run time is significantly increased? Some operations are performed at lower resolution.

Training procedure is identical, floating point implementation is used, and compared to VLOP3 float..

Decoding runtime decrease is attractive.

It is believed by proponents that the benefit could still be realised with VLOP4

It was agreed to investigate this in an EE. Integer implementation on top of VLOP4 should be tested.

JVET-AN0319 EE1-related: FlowWarp Operator for DRF Integer Inference Optimization [N. Fu, X. Chen, L. Qin, W. Zhang, Z. Chen (Wuhan Univ.)] [late]

This contribution introduces a FlowWarp operator, designed to mitigate the performance degradation observed in integer (int16) SADL inference of the NN inter tool Deep Reference Frame (DRF). Unlike the GridSample-based warp currently used in DRF, where pixel coordinates are first normalized to [−1,1] to construct a sampling grid and then de-normalized internally during sampling, FlowWarp operates directly in the pixel domain. By removing the normalization/de-normalization round-trip, it reduces extra arithmetic and tensor overhead and, under integer inference, avoids additional quantization and rounding errors, while preserving the intended warping behaviour of the existing implementation.

Integrated the proposed FlowWarp operator into the JVET-AN0195 method (EE1-3.5: Retrained DRF in NNVC-14.0) with the JVET-AN0195 anchor and an int16 SADL implementation, the coding gains are as follows:

- Class A1: 0.00% (Y), -0.04% (U), -0.01% (V) EncT: 98% DecT: 102%

- Class A2: -0.05% (Y), 0.08% (U), 0.03% (V) EncT: 100% DecT: 106%

- Class B: -0.08% (Y), 0.01% (U), 0.03% (V) EncT: 102% DecT: 102%

- Class C: -0.17% (Y), 0.21% (U), 0.07% (V) EncT: 101% DecT: 101%

- Overall: -0.08% (Y), 0.07% (U), 0.03% (V) EncT: 100% DecT: 102%

- Class D: -0.70% (Y), 0.44% (U), 0.03% (V) EncT: 101% DecT: 101%

- Class F: -0.05% (Y), -0.03% (U), 0.01% (V) EncT: 103% DecT: 104%

Integrated the proposed FlowWarp operator into the JVET-AN0193 method (EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization) with the JVET-AN0193 anchor and an int16 SADL implementation, the coding gains are as follows:

- Class A1: -0.02% (Y), 0.08% (U), -0.03% (V) EncT: 99% DecT: 97%

- Class A2: -0.06% (Y), 0.07% (U), 0.04% (V) EncT: 95% DecT: 93%

- Class B: -0.07% (Y), 0.03% (U), 0.07% (V) EncT: 112% DecT: 109%

- Class C: -0.09% (Y), 0.10% (U), 0.00% (V) EncT: 112% DecT: 110%

- Overall: -0.06% (Y), 0.07% (U), 0.02% (V) EncT: 106% DecT: 103%

- Class D: -0.18% (Y), 0.24% (U), 0.28% (V) EncT: 105% DecT: 105%

- Class F: -0.04% (Y), 0.06% (U), 0.09% (V) EncT: 107% DecT: 105%

This is only used in inference by changing normalization. Model parameters of EE1-3.5 are not changed.

It was commented that it is interesting to start integerization in Pytorch rather than doing it subsequently in SADL. However, it requires usage of an additional customization of Pytorch which might cause problems to users.

It was agreed to investigate this in an EE, and also to investigate if further improvement is possible by retraining the model (where the integerization could already be applied in training)

JVET-AN0376 Crosscheck of JVET-AN0319 (EE1-related: FlowWarp Operator for DRF Integer Inference Optimization) [N. Bhaskar (Huawei)] [late]

JVET-AN0340 AHG11: Neural network in-loop filter usage based on activation map [M. Santamaria, D. Buğdayci Sansli, F. Cricri (Nokia)] [late]

This contribution presents a method to derive the block-level activation of the NNLF in NNVC. For each block to be filtered in an inter coded slice, an activation map is generated using boundary strength values of the block itself and activation information of its reference block(s). The activation map is generated at the decoder side reportedly reduces signalling of per-block activation information. For LOP6 the proposal is reported to achieve average BD-rates of {-0.02%, -0.10%, -0.07%} RA, {-0.25%, -1.34%, -1.37%} LDB, {-0.22%, -1.21%, -1.26%} LDP configurations, with decoding times of 96%, 91% and 90%, respectively. For VLOP4 the proposal is reported to achieve average BD-rates {-0.01%, 0.00%, -0.02%} RA, {-0.15%, -0.17%, -0.40%} LDB, {-0.06%, -0.18%, -0.57%} LDP with decoding times 97%, 90%, 90%, respectively. It is recommended to study this tool in the next EE1 round.

Conceptually, some similarity with JVET-AN0172, but using more criteria than BS. The usage of activation information requires additional memory

It was agreed to investigate this in an EE, and also to report about an encoder-only solution (without determining the activation map at the decoder). It might also be considered if a combination with JVET-AN0172 is possible.

JVET-AN0373 AHG11: Efficient Depthwise Separable Backbone Network for Neural Network in-Loop Filter [Y. Shi, J. Wang, Y. Zuo, P. Wang, X. Chen, H. Zhu (BJUT), B. Li, P. Han, F. Xing, Z. Wang (Hisense)] [late]

A split-PReLU depthwise–pointwise residual block (SPR-DW) is proposed for the LOP in-loop filter, featuring dual 1×1 channel reductions, depthwise 3×3 spatial modeling, and a separate PReLU sequence to stabilize bottleneck activations. Evaluations against the NNVC-13.0 anchor show average BD-rate deltas of {AI: 0.10% (Y), 0.02% (U), -0.42% (V)}.

Very late contribution No presenter available Sunday 12Oct. 1130.

### SADL and NNVC implementation, CTC (7)

Contributions in this area were discussed during 1805–1910 on Saturday 4 Oct. 2025 (chaired by JRO).

JVET-AN0047 EE1-related: Automated environment information collection [N. Le, M. Santamaria, H. Zhang, F. Cricri (Nokia)]

Currently, technical contributions on neural network technology for video coding provide limited information on the development and test environments, which may hinder reproduction and debugging processes by crosscheckers. To address this, the contributor proposes to employ a common Python script to automatically collect comprehensive details of these environments, and the contribution provides an initial draft version of such a script, to be eventually further improved. Experts are invited to review the script and offer recommendations for enhancements.

It was asked what the benefit would be? Could this be used to identify reasons for divergence in crosscheck, for example? Yes, it might be useful to identify how different the environments are.

It was commented that also the ONNX version is relevant to be matching to generate the same model. It was reported that this was already included.

It was commented that some information collected by the script may be restricted due to privacy.

Decision (SW): It was agreed to include the script from JVET-AN0047 in the NNVC CTC repository as a tool that can be used (but not mandatory) to investigate potential problems in the computing environment.

JVET-AN0128 AhG14: Improvement of SIMD implementation with expanded SIMD operators in SADL for LOP with Overlapped Feature Integration [J. Chi, A. Li, Y. Du, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)]

In this document, the results from the expanded SIMD acceleration implementation in the SADL library are presented. Specifically, this implementation adds SIMD support for the 3x3 group convolution operator with group count = 2 and input channel counts = 16/4—an operator used in the current JVET-AN0127 EE1-1.1 LOP with Attention models of NNVC. The simulation results demonstrate that the proposed expanded SIMD implementation improves decoding time without any loss in video quality. It is suggested to update the current SIMD implementation in SADL and integrate this new 3x3 group convolution SIMD operator.

Decision (SW): Adopt JVET-AN0128

JVET-AN0196 AHG14: Operator Improvements in the SADL library [W. Zhang, N. Fu, L. Qin, X. Chen, Z. Chen (Wuhan Univ.)]

This contribution presents the operator improvements in the Small AdHoc Deep Learning (SADL) library from Wuhan University. These improvements include layers: Resize, GridSample, Concat, Slice, Flatten, ReduceMean, Add, and Relu.

It was commented that the limitation to batch dimension 1 existing in many other SADL elements than the slice operator. It is not wrong changing this in slice, but might be useful for other operators as well (it is noted that for slice no warning message is output, as is the case for other operators).

Decision (SW): Adopt JVET-AN0196

JVET-AN0197 AHG14: The extension of SADL library [L. Qin, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.)]

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Wuhan University. These extensions include four new layers: Gather, GatherElements, Floor, and Sub.

It was asked which networks would need the newly proposed layers? Future versions of DRF may be using Gather and Floor.

Proponents are asked to further study and issue merge requests when the new layers are practically needed in the NNVC exploration. It may be too much burden to implement all possible Pytorch layers to SADL. The software coordinator is asked to set priorities of merge requests.

JVET-AN0213 AhG14: SADL update [F. Galpin (InterDigital)]

- Changes details

Changes are available in the repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/sadl.

- New in version v14

  - Improvements

  - MR154: SIMD speed-up for conv2D 2x2

  - MR155: speed up targeting LOP

  - New

  - MR153: new feature Pad

  - Fixes

  - MR156: fix issue for sparse pack for float

  - MR158: fix saturation off

  - MR161: converter special case for DRF

  - MR162: new compilers fixes

  - Regression

In float mode, LOP6 model has inference issue (see issue #62). The issue was solved in MR 180.

- New proposed in version v15

Several MR in the queue:

- 8 new layers:

  - New Floor layer

  - New Gather layer

  - New GatherElements layer

  - New Sub layer

  - New Clip layer

  - New BitShift layer

  - New Depth2Space layer

  - New Abs layer

- Extension/speed-up:

  - SIMD Int32 ReLU

  - Extend concatenation

  - SIMD int32 add

  - Extend gridsample

  - Extend resize

  - Extension SIMD convolution 3x3 group=2 and depth=4/16 (for EE1 1.1)

- Fixes:

  - Flatten alignment with onnx

  - SIMD version of BiasAdd

  - Batch > 1

  - SIMD add scaling

  - ReduceMean

All changes that were implemented or planned were agreed.

Issuing bug reports in case of problems found is encouraged.

JVET-AN0222 [AHG14] New NN elements for SADL [A. Karabutov, E. Alshina (Huawei)]

This contribution presents the extensions in the Small AdHoc Deep Learning (SADL) library from Huawei Technologies with the following new elements:

- BitShift,

- Clip,

- Depth2Space,

- Abs.

The bitshift operation follows the method of ONNX and C++ (arithmetic shift).

Decision (SW): Adopt JVET-AN0222

JVET-AN0257 [AHG11] CPU and GPU time comparison for NNVC models [A. Karabutov, E.Alshina (Huawei), F. Galpin (InterDigital)] [late]

This contribution provides information about the execution time of neural networks across different devices. Two devices were tested: Huawei Mate50 Pro (CPU and NPU) and a high-end PC with an RTX 3090 GPU and CPU AMD Ryzen 9 5950X. LOP6 model from NNVC was used as the model under test. Several frameworks were used for the model inferencing: SADL, PyTorch and SNPE. The results showed that execution time on GPU can be faster than inference by SADL on CPU for the typical size of the processing block up to 15 times.

The current implementation of SADL allows you to control the operations executed by NN elements, but it is not optimized for running on GPUs, on which neural network solutions can demonstrate significant acceleration. To evaluate the complexity of algorithms on real devices, it is proposed to use the algorithm from JVET-AM0073 (including tables that allow the mapping).

Tile size and overlap can also be considered in the calculation, as well as bit depth of integer implementation.

It was commented that guaranteed real-time capability can likely not be deduced by this approach (or at least only for similar hardware architectures).

Further study on other elements of NNVC is recommended, e.g. to find coherence with kMAC/sample, CPU time, etc.

## AHG6/AHG12: Enhanced compression beyond VVC capability (69)

### Summary and BoG reports

Contributions in this area were discussed during 0905–1250 and during 1435–1535  on Sunday 5 Oct. 2025 (chaired by JRO).

JVET-AN0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1a | Restrictions for TT splitting (normative way) with split_cu_flag context change | G.Wang (vivo) | Z.Deng(Bytedance) Y.Liu(Transsion) Z.Sun(OPPO) |
| 1.1b | Restrictions for TT splitting (non-normative way) | G.Wang (vivo) | Z.Deng(Bytedance) Z.Sun(OPPO) |
| 1.1c | Temporal partitioning prediction optimization | G.Wang (vivo) | Z.Sun(OPPO) |
| 1.1d | Test 1.1a + Test 1.1c | G.Wang (vivo) | withdraw |
| 1.1e | Test 1.1b + Test 1.1c | G.Wang (vivo) | withdraw |
| 1.1f | Restrictions for TT splitting (normative way) | G.Wang (vivo) | Y.Liu(Transsion) |
| 1.1g | split_cu_flag context change | G.Wang (vivo) | Y.Liu(Transsion) |
| 2 Intra prediction |  |  |  |
| 2.1 | TMRL blend | S. Blasi (Nokia) | V. Rufitskiy (TCL) |
| 2.3a | Switchable interpolation filter for TIMD side reference | T. Dong (TCL) | S. Lee, Y. Kim, S. Noh, J. Bang, H. Choi (HNU) |
| 2.3b | Switchable interpolation filter for TIMD template. | T. Dong (TCL) | M. Abdoli (Xiaomi) |
| 2.3c | Test 2.3a + Test 2.3b | T. Dong (TCL) | S. Lee, Y. Kim, S. Noh, J. Bang, H. Choi (HNU) |
| 2.4 | Longer tap interpolation filtering | V. Rufitskiy (TCL) | M. Abdoli (Xiaomi) |
| 2.5 | Combination of Test 2.1, Test 2.3b, and Test 2.4 | V. Rufitskiy (TCL) | P. Andrivon (Ofinno) |
| 2.6a | Adaptive subsampling filter selection for CCLM/intra-CCCM/ALF-CCCM | Y. Kidani (KDDI) | D. Buğdayci Sansli (Nokia) P.Bordes (InterDigital) |
| 2.6b | Adaptive subsampling filter selection for inter-CCCM | Y. Kidani (KDDI) | D. Buğdayci Sansli (Nokia) |
| 2.6c | Test 2.6a + Test 2.6b | Y. Kidani (KDDI) | D. Buğdayci Sansli (Nokia) |
| 2.7 | Reducing candidate modes in decoder derived CCP | S. Wan(NWPU) S. Xie(ZTE) | P. Bordes (InterDigital) |
| 2.8 | Enhanced CCP Fusion mode | P. Bordes (InterDigital) | Y.Kidani (KDDI) N.Zouidi (Ofinno) |
| 3 Inter prediction |  |  |  |
| 3.1 | Generated merge candidates | D. Buğdayci Sansli (Nokia) | K. Naser (InterDigital) R. G. Youvalari (Xiaomi) |
| 3.2a | Joint reordering of GPM with affine prediction | L. Zhang (OPPO) | Y. Wang (Bytedance) |
| 3.2b | Modifications to the affine merge list | L. Zhang (OPPO) | Y. Wang (Bytedance) |
| 3.2c | Test 3.2a + Test 3.2b | L. Zhang (OPPO) | Y. Wang (Bytedance) |
| 3.3 | Joint reordering of GPM with intra prediction | Z. Sun (OPPO) | N.Zouidi (Ofinno) X.Li (Alibaba) |
| 3.4a | Test 3.2a + Test 3.3 | Z. Sun (OPPO) | X. Li (Alibaba) C. Ma (Kwai) |
| 3.4b | Test 3.2a + Test 3.2b + Test 3.3 | Z. Sun (OPPO) | X. Li (Alibaba) C. Ma (Kwai) |
| 4 Transform and coefficients coding |  |  |  |
| 4.1 | Linked sign prediction | Y. Zhang (OPPO) | C. Hollmann (TCL) |
| 4.2a | Shifting quantization center for TSRC under non-CTC (RDOQ on) | Y. Yu (OPPO) M. Le Pendu (InterDigital) | P. Nikitin (Xiaomi) M. Balcilar (Ofinno) B. Ray (Qualcomm) |
| 4.2b | Shifting quantization center for RRC under non-CTC (DQ disabled, RDOQ on) | Y. Yu (OPPO) M. Le Pendu (InterDigital) | P. Nikitin (Xiaomi) M. Balcilar (Ofinno) B. Ray (Qualcomm) |
| 4.2c | Test 4.2a + Test 4.2b | Y. Yu (OPPO) M. Le Pendu (InterDigital) | P. Nikitin (Xiaomi) M. Balcilar (Ofinno) B. Ray (Qualcomm) |
| 4.2d | Shifting quantization center for TSRC under CTC | Y. Yu (OPPO) M. Le Pendu (InterDigital) | P. Nikitin (Xiaomi) |
| 4.2e | Shifting quantization center for RRC under CTC | Y. Yu (OPPO) M. Le Pendu (InterDigital) | P. Nikitin (Xiaomi) |
| 4.2f | Test 4.2d + Test 4.2e | Y. Yu (OPPO) M. Le Pendu (InterDigital) | P. Nikitin (Xiaomi) |
| 4.2g | Shifting quantization center for RRC under non-CTC (DQ disabled, RDOQ disabled) | M. Le Pendu (InterDigital) Y. Yu (OPPO) | M. Balcilar (Ofinno) |
| 4.2h | Shifting quantization center for TSRC under non-CTC (DQ disabled, RDOQ disabled) | M. Le Pendu (InterDigital) Y. Yu (OPPO) | M. Balcilar (Ofinno) |
| 4.2i | Test 4.2g + Test 3.2h | M. Le Pendu (InterDigital) Y. Yu (OPPO) | M. Balcilar (Ofinno) |
| 4.3a | Residual sign prediction restriction | C. Hollmann (TCL) |  |
| 4.3b | RSP context modelling | C. Hollmann (TCL) |  |
| 4.3c | Test 4.3a with retraining of existing contexts | C. Hollmann (TCL) |  |
| 4.3d | Test 4.3a + Test 4.3b | C. Hollmann (TCL) |  |
| 5 In-loop filtering |  |  |  |
| 5.1a | Regularization of ALF-CCCM | Z. Xie (OPPO) | M. Jia (ZTE) |
| 5.1b | Regularization for ALF-CCCM enabled in I slice | Z. Xie (OPPO) | M. Jia (ZTE) |
| 5.2a | Non-downsampled ALF-CCCM | L. Xu (OPPO) |  |
| 5.2b | Reuse of CU partition in ALF-CCCM | L. Xu (OPPO) |  |
| 5.2c | Test 5.2a + Test 5.2b | L. Xu (OPPO) |  |
| 5.3a | Remove single model of ALF-CCCM | N. Song (OPPO) |  |
| 5.3b | Separate “bad window” condition for Cb and Cr components | N. Song (OPPO) |  |
| 5.3c | Test 5.3a + Test 5.3b | N. Song (OPPO) |  |
| 5.4 | Additional models for ALF-CCCM | F. Wang (OPPO) |  |
| 5.5a | Test 5.1 + Test 5.2 | L. Xu Z. Xie (OPPO) | P.Astola (Nokia) |
| 5.5b | Test 5.1 + Test 5.2 + Test 5.4 | L. Xu Z. Xie F. Wang (OPPO) | P.Astola (Nokia) |
| 5.5c | Test 5.1 + Test 5.2 + Test 5.3 + Test 5.4 | N. Song L. Xu Z. Xie F. Wang (OPPO) | P.Astola (Nokia) |
| 5.6 | Reuse of TALF control information | Y. Bai (ZTE) | I.Jumakulyyev (Nokia) Z.Xie (OPPO) |

Partitioning

Test 1.1: On partitioning optimization (JVET-AN0121)

In multi-type tree partitioning, different splitting patterns may result in the same coding block structure. As shown in part (a) of the figure below, a binary tree split in vertical direction followed by a ternary tree split in horizontal direction may have the same coding block structure as a ternary tree split in horizontal direction followed by a binary tree split in vertical direction. A similar case is shown in part (b) of the figure.

In the tests, the duplicated partitioning splitting is disabled in a normative way and in encoder only. Additionally, context derivation for split_cu_flag was changed where instead of using width for the above neighbour block and height of the left neighbour block, the area of those blocks is utilized in comparing the current block area with the neighbour blocks area.

In the second part of the test, a prediction method for (No split) mode is introduced. For each CU, if the current depth is smaller than the temporal minimum QT depth minus 1, then (No split) is disallowed. Other split predictions are not changed. ECM has a similar prediction scheme for splitting modes (QT, BT, TT).

Test 1.1a: Restrictions for TT splitting (normative way) with split_cu_flag context change

Test 1.1b: Restrictions for TT splitting (non-normative way).

Test 1.1c: Temporal partitioning prediction optimization.

Test 1.1f: Restrictions for TT splitting (normative way).

Test 1.1g: split_cu_flag context change.

Results (also in runtime) for tests a and b confirmed by crosscheckers (at least partial results available), also confirm reduction in encoding time. Some crosscheckers expressed support for test 1.1a.

It was commented that 1.1g (which is a subset of 1.1a) has slightly better gain and also gives runtime reduction. It is a very small modification of the context definition, not adding context. It is also confirmed by crosschecker (crosscheck doc not registered yet) that also the encoding time is lower at least in RA and LB, and slightly higher gain in LB. After availability of full crosscheck results, the values in terms of gain and run time are confirmed.

Decision: Adopt JVET-AN0121 test 1.1g.

Intra prediction

Test 2.1: TMRL blend (JVET-AN0171)

In the test, a new mode is introduced for blending TMRL predictors. In TMRL, a template search is performed to determine pairs of intra mode and MRL index. In this mode, a template search is performed to determine up to three pairs of intra mode and MRL index. Correspondingly up to three predictors are then blended together to compute a prediction for the current block. Up to two predictors are obtained using directional intra prediction modes. A third non-directional predictor can also be used, obtained using PLANAR or BV prediction.

To differentiate TMRL blend from conventional TMRL, a different template size and a different list of MRL indexes are used in TMRL blend than in conventional TMRL. The template size is adaptively set to 2 or 4 depending on the block size, similarly to the template size used in TIMD. Template costs are computed in such a way that template samples closer to the current block are weighted more than template samples located further away from the current block.

A flag is coded to indicate the mode.

Test 2.3: Interpolation filter unification for unwrapping and TIMD template generation (JVET-AN0120)

In Test 2.3a, a set of candidate interpolation filters (4-tap or 6-tap) is defined for generating side reference (shown in the next figure) samples along the prediction direction. The filter switch is done based on the block size.

In Test 2.3b, switchable interpolation filters (4-tap or 8-tap) are applied for template generation of TIMD. The filter switch is done based on the block size.

Test 2.3a: Switchable interpolation filter for TIMD side reference.

Test 2.3b: Switchable interpolation filter for TIMD template.

Test 2.3c: Test 2.3a + Test 2.3b.

Test 2.4: Longer tap interpolation filtering (JVET-AN0158)

For each predictor a component used in DIMD, an interpolation filter is selected, and the selection of an interpolation filter is performed based on the DIMD blending parameters specified for a block.

The length of the interpolation filters is selected according to the next table.

| Weight | [1,7] | [8,24] | [25,31] | [32,48] | >48 |
| --- | --- | --- | --- | --- | --- |
| Filter length | 16 | 16 | 16 | 8 | 8 |

Similarly, when TIMD uses blending, an interpolation filter of 8-, 12-, or 16-taps is selected based on the number of modes used in the blending classified into the following categories:

- angular modes;

- DC mode;

- non-angular modes other than DC mode.

The filter length (number of taps) is determined from the number of modes in the categories as follows.

| Number of angular modes | 2 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- |
| DC mode | any | 1 | 0 | any |
| Non-angular modes other than DC mode | any | 0 | 0 | more than 0 |
| Filter length | 12 | 16 | 8 | 12 |

When the main reference side has length of 8 or smaller then the number of taps of the selected interpolation filter is capped to not exceed 10-tap.

Test 2.4: Switchable interpolation filtering for DIMD and TIMD predictors.

Test 2.5: Combination of TMRL, TIMD, and DIMD interpolation filtering (JVET-AN0158)

In this combination test, TMRL blend is combined with TIMD interpolation filters such that Test 2.3b is used for filtering in the prediction derivation for TIMD template, while Test 2.4 filters are used in the TIMD and DIMD prediction derivation.

Test 2.5: Test 2.1 + Test 2.3b + Test 2.4.

Test 2.6:  Adaptive subsampling filter selection for CCLM/CCCM (JVET-AN0275)

Many 4:2:0 colour format videos are generated from 4:4:4 colour format videos using different subsampling filters. For example, {1,1; 1,1} and {1, 2, 1; 1, 2 ,1} filters are used, which are known as MPEG1 and MPEG2 subsampling filters for 4:2:0 format, respectively. ECM uses only MPEG2 filter for CCLM and CCCM.

To align the subsampling filters between the generation process of 4:2:0 colour format videos and CCLM and CCCM processes, this test introduces an adaptive subsampling filter selection method for CCLM and CCCM. MPEG1 and MPEG2 subsampling filters are adaptively selected for the luma subsampling processes of CCLM and CCCM (including intra-CCCM and ALF-CCCM) by reusing the existing SPS flags in ECM, which specify the video colour format and spatial allocation of luma and chroma samples (i.e., sps_chroma_format_idc, sps_chroma_horizontal_collocated_flag and sps_chroma_vertical_collocated_flag). The values of the existing SPS flags for every test sequence are automatically determined by the encoder-side method based on the analysis of a first picture characteristic.

Similarly, the adaptive filter selection method is also introduced into the filter derivation and application processes of inter-CCCM, where the prediction chroma sample value of inter-CCCM can be obtained using the following formula:

predChromaVal = c0 L0+ c1L2 + c2L3 + c3L5 + c4 nonlinear((L0+L2+L3+L5+2) >> 2) + c5 B,

in addition to the following existing inter-CCCM formula:

predChromaVal = c0 L0+ c1L1 + c2L2 + c3L3 + c4L4 + c5L5 + c6 nonlinear((L0+L3+1) >> 1) + c7 B,

where nonlinear is CCCM’s nonlinear operator and B is bias.

Test 2.6a: Adaptive subsampling filter selection for CCLM/intra-CCCM/ALF-CCCM.

Test 2.6b: Adaptive subsampling filter selection for inter-CCCM.

Test 2.6c: Test 2.6a + Test 2.6b.

Test 2.6d: Test 2.6a for Gaming_LD_HD class with CTC QPs.

Test 2.7: Reduced candidate modes in DDCCP (JVET-AN0220)

In the test, cross-component models with local boost are removed from the DDCCP candidate list, the candidate list is changed from {CCLM, CCCM, CCCM with LB-CCP, MM-CCCM, MM-CCCM with LB-CCP, GL-CCCM} to {CCLM, CCCM, MM CCCM, GL-CCCM}.

The next table summarizes the reduction for the number of modes.

|  | Number of non-fusion modes | Number of fusion modes | Total Number |
| --- | --- | --- | --- |
| ECM DDCCP | 6 | 15 | 21 |
| Test | 4 | 6 | 10 |
| Reduction | 2 | 9 | 11 |

Test 2.7: Reduced candidate modes in decoder derived CCP.

Test 2.8: Enhanced CCP fusion mode (JVET-AN0168)

In the test, for the decoder-derived CCP merge or CCP merge fusion modes, two CCP predictions using the regressing-based GPM (RGPM) method, which derives the blending weights from the template, are combined. The blending weights are derived jointly for the two chroma components using the same method as in RGPM and SGPM modes. The maximum number of fusion candidates is signalled in SPS (8 in AI and for inter slices, otherwise 12).

Test 2.8a: Enhanced CCP fusion mode.

Test 2.8b: Test 2.7 + Test 2.8a (JVET-AN0173).

For 2.1, an expert comments that the gain is higher in class E than in other classes (lowest in class A). Also, some encoder optimization is performed that could also be applied in anchor. No additional TM is used, but additional processing similar to DIMD is necessary. Among 2.1…2.4, 2.1 has worst tradeoff.

It was commented by crosscheckers that gains of 2.3b and 2.4 are believed be additive, however no results are available on such a combination. Both are straightforward changes using longer interpolation filters. Standalone, 2.4 has a better tradeoff.

Decision: Adopt JVET-AN0158 test 2.4.

On Sunday 12 Oct. Proponents of 2.3b reported additional results on combination with 2.4 (cross-checked in JVET-AN0375) Coding gain 0.01% in AI over 2.4 with no runtime increase, however standalone the gain was 0.02%. No results in RA available yet, but for RA 2.3b standalone had no gain, or very small loss in chroma. No action taken.

Test 2.6 shows significant gain only for two sequences (one from class A, and one from class F), which is natural as the other sequences obviously used the chroma subsampling currently implemented.

It is straightforward to use the chroma subsampling (horizontal and/or vertical shift) information from SPS in CCCM/CCLM/ALF-CCCM (if available), as investigated in test 2.6a. An automatic detection algorithm is also provided.

In 2.6b, a different model is used for inter CCCM. However, inter CCCM does not use downsampling, and the new model does not show benefit

Results are confirmed by cross-check.

Decision: Adopt JVET-AN0090 test 2.6a, including the detection method (the latter disabled by default). In CTC, the per-sequence config can be used to signal (update necessary for cat robot and slide show)

Also some of the sequences in CfE would have additional gain for ECM (see info in JVET-AN0090). Proponents provided updated configuration files in version 4 of JVET-AN0090. It is further noted that for CCLM in VTM no change would be useful, as it only supports vertical shift.

2.7 removes the two local boost modes showing reduction of encoding and decoding time with marginal loss in compression.

2.8a reduces the number of candidates from 12 to 8, which also gives similar reduction in encoding and decoding runtime.

2.8a has better tradeoff than 2.7, and also the combination 2.8b has worse tradeoff than 2.8a.

Decision: Adopt JVET-AN0168 test 2.8a.

Inter prediction

Test 3.1: Generated merge candidates (JVET-AM0059)

In this test, new merge candidates are generated from the existing ones in the list. After the initial merge list is constructed, new candidates are added to the list before the pair-wise average candidates are generated and ARMC is applied.

Candidate generation first creates two separate lists of uni-predictors from existing candidates. If a uni-prediction candidate is found, it is taken as it is and if a bi-prediction candidate is found, motion information from each prediction direction is taken to the related list.

The template cost of each candidate in the list is calculated, the lists are sorted in ascending cost order. From the lowest cost predictors, new uni and bi-predicted merge candidates are generated and added to the list before ARMC stage. The actual number of newly generated candidates depends on the found unique uni-predictors, where upper limit is set to the maximum number of merge candidates.

Test 3.1: Generated merge candidates.

Test 3.2:  Joint reordering of GPM with affine prediction (JVET-AN0091)

In ECM, when the jointly reordering GPM method is applied, a candidate list is constructed with each candidate containing one split mode and the MVs of the two GPM partitions. This candidate list is reordered using a template-based scheme. Only the selected candidate index, instead of the split mode and the motion information of the two partitions, needs to be signalled to the decoder to indicate the split mode and motion vector pair.

In Test 3.2a, a jointly reordering GPM with affine mode is introduced, where a candidate list is constructed with each candidate containing one split mode, two partition indices and two affine mode indicators (isAffine0, isAffine1), the usage of the mode is indicated by flag.

For each GPM partition, the partition index is used to get motion information from regular merge or affine merge lists, and the partition is predicted by affine or non-affine motion compensation. The candidate list is reordered using a template-based scheme, the selected candidate index is signalled to the decoder. Besides, the samples of the current template are mapped to the original domain during the construction of the jointly reordering GPM candidate list when the LMCS is enabled.

To reduce the complexity, the tested mode is only used when the block size is not equal to 128 or when there is at least one adjacent/non-adjacent coded block with affine prediction. When constructing the candidate list, the number of available affine merge candidates is reduced to 15, and the candidate list size is reduced to 10 when the current slice satisfied the LDB condition; otherwise, the candidate list size is set to be 16. No additional RDO is added on the encoder side.

In Test 3.2b, the construction of affine candidates is modified. For each control point, while checking the corresponding neighbouring positions, all the available motion information with a different reference picture is stored and then used to construct the possible affine candidates (temporally derived affine candidates for GPM). This process is carried out by traversing the possible CPMVs combinations. The neighbouring positions and CPMVs combination are the same as in ECM.

Test 3.2a: Joint reordering of GPM with affine prediction.

Test 3.2b: Affine merge list modifications.

Test 3.2c: Test 3.2a + Test 3.2b.

Test 3.3: Joint reordering of GPM with intra prediction (JVET-AN0092)

In the test, intra prediction modes are added as additional candidates into the joint reordering of split modes and partition indices, where the regular GPM modes together with intra prediction modes are reordered jointly, and there up to 6 intra-prediction candidate modes are added to the final list. A flag is introduced to indicate whether the scheme is applied.

The construction of intra prediction candidate modes is the same as in intra MPM list. To reduce the overall complexity for both encoding and decoding, the size of candidate list is set to 16 for slice not satisfying low delay condition; otherwise, the candidate list size is set to 10. The total RDO processes in ECM remain unchanged.

Test 3.3: Joint reordering of GPM with intra prediction.

Test 3.4: Combination for GPM reordering tests (JVET-AN0093)

Test 3.4a: Test 3.2a + Test 3.3.

Test 3.4b: Test 3.2a + Test 3.2b + Test 3.3.

Test 3.1: Results confirmed by cross-checkers, and technology supported by them. Tradeoff is within an acceptable margin.

Decision: Adopt JVET-AN0236 test 3.1.

Results confirmed by cross-checkers, and support was expressed for the combination 3.4a which has the best overall tradeoff. It is noted that 3.2b which standalone provides more interesting gain for LB, loses this advantage when combined with 3.2a and 3.3 (in 3.4b).

Decision: Adopt JVET-AN0093 test 3.4a.

Transform and coefficient coding

Test 4.1: Linked sign prediction (JVET-AN0094)

In the test, a linked sign prediction method is introduced, where the residual contribution from non-sign predicted coefficients is added to the residual contribution of sign predicted coefficient when assessing boundary continuity, the method has three steps:

- Non sign predicted coefficients linking process

- Residual hypothesis of the linked coefficient group

- Sign correction of non sign predicted coefficients.

In the non sign predicted coefficient linking process, those coefficients are classified into two groups: inside (type A) and outside (type B) of the sign predicted coefficient area, shown in the next figure.

If neither type A nor type B coefficients exist, then the method is not used. If only type A or B coefficients exists, these non sign predicted coefficients are linked to the sign predicted coefficient with the smallest coefficient magnitude. Together, these coefficients constitute a linked coefficient group. The linked coefficients are used together to calculate two hypotheses for their combined contribution to the residual.

If both type A and B coefficients exist, type A coefficients are linked to the sign predicted coefficient with the second-smallest coefficient magnitude. Type B coefficients are linked to the sign predicted coefficient with the smallest coefficient magnitude. Each linked group will be used to calculate two hypotheses respectively for their combined contribution to the residual.

An example of linked coefficients grouping is shown in the next figure, where LSP denotes a selected sign predicted coefficient in the linked group.

Similar to the current sign prediction method, each linked group is used to calculate two hypotheses (for positive and negative sign) of its contribution to the residual as follows:

rlx,y= rsl x,y+i=0n-1ri x,y,      for positive hypothesis

or

rlx,y= -rsl x,y-i=0n-1ri x,y,     for negative hypothesis

where rlx,y represents the combined residual contribution at the x-th and y-th position of the current block for one hypothesis of the linked group; rsl x,y represents the residual contribution at the x-th and y-th position of the current block from the sign predicted coefficient assuming it has a positive sign; ri x,y represents the residual contribution at the x-th and y-th position of the current block from the i-th non sign predicted coefficient with signs applied as signalled by their corresponding bypass coded sign bits; and n represents the number of non-SP coefficients in the LCG.

The signs for both type A and B coefficients remain bypass coded. However, when they are included into a linked group, the meaning of these signalled sign bits may be modified.

If the sign predicted coefficient associated with the group is correct, then the signs of its linked non sign predicted coefficients are the same as indicated by coded sign bits. Otherwise, the signs of its linked non sign predicted coefficients are interpreted as the reverse of what was signalled by their sign bits

Test 4.1: Linked sign prediction.

Test 4.3: Residual sign prediction restriction (JVET-AN0129)

In Test 4.3a, sign prediction is not applied to blocks that have only one non-zero coefficient in the sign prediction area.

Int Test 4.3b, the number of contexts used for the sign prediction flags is increased from 8 to 16 by adding a dependency on the transform type.

In Test 4.3c, a restriction from Test 4.3a has been applied and the initial values for the original context models have been retrained using the CabacTraining application. Initial values for other contexts have not been retrained.

Test 4.3a: Sign prediction restriction.

Test 4.3b: Increased contexts for sign prediction.

Test 4.3c: Test 4.3a with sign prediction context retraining.

Test 4.3d: Test 4.3a + Test 4.3b.

Test 4.2: Shifting quantization center (JVET-AN0095, JVET-AN0096, JVET-AN0097)

A fix for RDOQ implementation with LFNST and NSPT was submitted to ECM-18.0 in MR 960. In ECM, when RDOQ is used for a block with LFNST or NSPT, only up to 16 or 8 coefficients (depending on the block size) are encoded in RDOQ, instead of using the number of output coefficients of LFNST or NSPT for the current block.

In non-CTC configuration, when DQ and SignPred are disabled, the test results for the fix are as follows.

AI: Y -0.52% / U -0.21% / V -0.20%

RA: Y -0.35% / U -0.22% / V -0.14%

LDB: Y -0.25% / U 0.03% / V -0.09%

In the 4.2 tests, (*) denotes the results when the fix is applied.

In ECM for RRC, the shifting amount in the dequantized coefficients is inversely proportional to the quantization indices of DQ as follows.

x=1024-TidxDQ*Q-1idxDQ+TidxDQ*Q-1idxDQ'≫10    if  T>idxDQ>0

where x is the dequantized coefficient,  T is the size of the lookup table, Q-1idxDQ is the dequantized value of the idxDQ-th of DQ,  idxDQ'is the auxiliary quantization index calculated as

idxDQ'=idxDQ+(idxDQ>0 ?1:-1)

A lookup table of T is defined as follows.

T=[0,63, 31, 21, 15, 12, 10, 9, 7, 7, 6, 5, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1,…,1]

In the test, the shift to dequantized values is applied when DQ is disabled (non-CTC configuration). The shift is applied as follows.

x=1024-Tk*Q-1k+Tk*Q-1k'≫10    if  T>k>0

where x is the dequantized coefficient,  T is the size of the lookup table, Q-1k is the dequantized value of the quantization level k, and k’ is the auxiliary quantization level that is calculated as

k'=k+(k>0 ?1:-1)

Test 4.2a: Shifting of quantization center applied to TSRC (JVET-AN0095).

Test 4.2b: Shifting of quantization center applied to RRC (JVET-AN0095).

Test 4.2c: Test 4.2a + Test 4.2b (JVET-AN0095).

Test 4.2g: Shifting of quantization center applied to RRC without RDOQ (JVET-AN0097).

Test 4.2h: Shifting of quantization center applied to TSRC without RDOQ JVET-AN0097).

Test 4.2i: Test 4.2h + Test 4.2h (JVET-AN0097).

Test 4.2d: Shifting of quantization center applied to TSRC in CTC (JVET-AN0096)

A modification of the quantization center shift is tested with enabled DQ (used only in RRC in the CTC settings). Since the quantization level k from Q0 or Q1 (instead of the DQ quantization index) is coded into a bitstream and the value of quantization level k can be used to better reflect the rate, this test uses the quantization level from Q0 and Q1 instead of quantization indices of DQ to derive the shifting amount.

The same quantization level from quantizer Q0 and Q1 will generate a similar rate while the dequantization value of Q1 is smaller than that of Q0 for the same quantization level k. Therefore, Q1 and Q0 are further modified to use different table sizes, a lookup table T0 of size 32 and a lookup table T1 of size 48 are used for Q0 and Q1, respectively.

x=1024-Tik*Qi-1(k)+TQik*Qi-1(k')≫10    if  Ti>k>0

where i indicates the Q0 or Q1, x is the dequantized coefficient, Ti is the size of the lookup table, Qi-1(k) is the dequantized value of the quantization level k from quantizer Qi, k'is the auxiliary quantization index calculated as

k'=k+(k>0?1:-1)

T0[32]=0, 31, 15, 10, 7, 6, 5, 4, 3, 3, 3, 2, 2, 2, 2, 2, 1,1,1,1,1,1,1,1,1,1, 1, 1, 1, 1, 1, 1

T148=0, 47, 23,15, 11,9, 7, 6, 5, 5,4,4,3, 3, 3, 3, 2, 2, 2, 2, 2,2,2,2, 1,1,1, 1, 1, 1,1…,1,1,1,1,1

Furthermore, those tables can be interleaved to form one table as follows.

T'94=0, 47, 31, 23, 15, 15, 10, 11, 7, 9, 6, 7, 5, 6, 4, 5, 3, 5, 3, 4, 3, 4, 2, 3, 2, 3, 2, 3, 2, 3, 2, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 2, 1, 1,1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 1, 0, 1, 0, 1, 0, 1,0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1, 0, 1

The dequantization value for quantization level k from Q0 or Q1 is calculated as follows.

x=1024-T'idxDQ*idxDQ*Δ+T'idxDQ*idxDQ''*Δ≫10    if  T'>idxDQ>0

where T' is the size of the lookup table, and idxDQ''is the auxiliary quantization index that is calculated as

idxDQ''=idxDQ+(idxDQ>0 ?2:-2)

Test 4.2e: Modified shifting of quantization center applied to RRC (JVET-AN0096)

Test 4.2f: Test 4.2d + Test 4.2e (JVET-AN0096)

In summary, there are the following aspects:

- RDOQ fix MR 960 (encoder only)

- Quantization center shift for TSRC with DQ enabled and disabled.

- Quantization center shift for RRC with DQ disabled.

- Modified quantization center shift for RRC with DQ enabled.

Test 4.1 does not have interesting tradeoff, encoding runtime increase too large.

Test 4.3a could be attractive in reducing the number of sign predictions, but this has some more significant loss in LB without large decrease in encoding/decoding time. It was reported by proponents that this is mainly caused by class E. Retraining reduces the chroma losses, but mainly by better gain in other classes.

Test 4.3b has good tradeoff for LB, but increasing the number of contexts does not improve in RA, and also in AI, gain is rather small. When combining tests a and b, RA even shows some loss

Overall, no relevant benefit in tests 4.1 and 4.3.

Test 4.2 investigates shifting of quantization center. This shows benefit using the shift for RRC (4.2b*) but not for TSRC (4.2a*) when DQ and sign prediction are disabled (non-CTC), and some further gain if RDOQ is also disabled (4.2g). Under CTC, application to TSRC gives some gain mainly for screen content (4.2d), and results in 4.2a* and 4.2c* also show that it has almost no impact in the non-CTC case when used for TSRC as well. Another variant is tested using a modified shifting for RRC which shows also some gain for non-screen content (4.2e). It is however not known how this modified shifting would work in the non-CTC case, and how this modified shifting would work for TSRC. Therefore, it is not to be considered at this moment, but further results may be provided in the future.

It was confirmed that sign bit hiding is enabled in all cases.

The normative change related to RRC would be to remove the constraint not using the quantization center when DQ is disabled (non-CTC case). This is asserted to be beneficial according to test 4.2b*.

The normative change related to TSRC would be to enable the shift of quantization center (same as for RRC). This would also affect the CTC case (would be beneficial for screen content, test 4.2d).

In terms of normative changes, test 4.2c* includes both aspects.

Decision: Adopt JVET-AN0095 test 4.2c*. Also encoder modifications are necessary for CTC and non-CTC cases.

In-loop filtering

Test 5.1: On regularization of ALF-CCCM (JVET-AN0098)

A regularization method of EIP is applied to derive of ALF-CCCM coefficients, a regularization parameter is determined by the number of input samples. For the cases where the number of input samples is smaller than {16, 32, 64, 256, or 1024}, the regularization parameter is set to {192, 160, 128, 96, or 64}, respectively. Otherwise, the regularization parameter is always set to 32.

If there is a sample whose correction exceeds the threshold, the corresponding samples using the same ALF-CCCM model will not be filtered. In the test, the threshold is modified from 32 to 16 and 28 for the single-model filter and the multi-model filter, respectively.

The tested method is not applied to large blocks, which regression window is larger than 64x64, in ALF-CCCM.

Test 5.1a: Regularization of ALF-CCCM.

Test 5.1b: Regularization of ALF-CCCM enabled in I-slices.

Test 5.2: On ALF-CCCM (JVET-AN0099)

In ALF-CCCM method, the output samples of luma SAO are used as inputs to the CCCM filtering. To obtain a correction signal, the SAO chroma samples are subtracted from the CCCM output samples. The correction is weighted by 0.5 and added to the ALF chroma output to improve chroma reconstruction samples.

For each CTU, the encoder’s RDO decides the best cross-component model from eight possible models, they differ in the number of samples, non-linear term, and biases. When applying multi-models, if any sample’s correction exceeds the threshold with either model, the corresponding samples using the same ALF-CCCM model will not be filtered.

Two aspects are introduced in this test:

- Two new model types are introduced, which are non-downsampled models used in intra CCCM prediction and inter CCCM prediction. A slice level syntax element is signalled to indicate the application of non-downsampled models. A CTU level flag is further signalled to indicate whether the current CTU uses the downsampled ALF-CCCM. The filtering process of the two new model types is summarized in the following equations.

New model 0 (same to intra CCCM):

C=i=05αi∙(Li-offsetLuma)+i=69αi∙(((Li-4-offsetLuma)2+β)≫bitDepth)+α10∙β

New model 1 (same to inter CCCM):

C=i=05αi∙(Li-offsetLuma)+α6∙((L6-offsetLuma)2+β)≫bitDepth)+α7∙β

- The CU partition is introduced into ALF-CCCM. A slice level syntax element is signalled to indicate the application of CU partition. A CTU level flag is further signalled to indicate whether the current CTU is processed according to the CU partition instead of the existing 8 ALF-CCCM block sizes {4x4, 8x2, 2x8, 8x8, 16x16, 32x32, 64x64, 128x128}.

Test 5.2a: Non-downsampled ALF-CCCM.

Test 5.2b: CU partitioning in ALF-CCCM.

Test 5.2c: Test 5.2a + Test 5.2b.

Test 5.3: Updated multi-models usage strategy for ALF-CCCM (JVET-AN0101)

In ALF-CCCM, a single CCCM model is calculated for each block, and then samples within the block are filtered by using this model. If any sample in either the Cb or Cr component of the block has a correction exceeding a threshold, the block will use multi-models instead. When filtered by the multi-models, if there is a sample in either the Cb or Cr component whose correction exceeds the threshold, the corresponding samples that use the same model as this sample will not be filtered.

In Test 5.3a, single model is removed, and only multi-models are applied in a block.

In Test 5.3b, the correction threshold condition is treated separately for the Cb and Cr components. ECM disables the model for both components together.

Test 5.3a: Remove single model of ALF-CCCM.

Test 5.3b: Separate correction condition for Cb and Cr components.

Test 5.3c: Test 5.3a + Test 5.3b.

Test 5.4: On ALF-CCCM model (JVET-AN0103)

In the test, nonlinear terms with the chroma samples are added at additional positions for model 0, model 3, model 4 and model 5, as shown in yellow in the next table.

Test 5.4: Additional models for ALF-CCCM.

Test 5.5: Combination tests for ALF-CCCM (JVET-AN0106)

Test 5.5a: Test 5.1a + Test 5.2c.

Test 5.5b: Test 5.1a + Test 5.2c + Test 5.4.

Test 5.5c: Test 5.1a + Test 5.2c + Test 5.3a +Test 5.4.

Test 5.5c*: Test 5.5c with encoder optimization.

Test 5.5d: Test 5.1a + Test 5.2c + Test 5.3a.

Test 5.6: Reuse of TALF control information (JVET-AN0199)

In this test, slice and CTB-level TALF control information of the current picture is stored for a potential reuse in subsequent pictures. A flag is introduced to indicate whether the TALF control information of the current picture reuses that of the previously encoded picture.

At the encoder side, a similarity is evaluated by comparing the ALF CTB on/off status of the two pictures. The similarity score is defined as the percentage of CTBs with the same ALF on/off status relative to the total number of CTBs. If the similarity score falls below a threshold of 0.2, the TALF control information from the previous picture is not reused.

Test 5.5: Reuse of TALF control information.

Answers to questions:

- The regularization of 5.1a is used for both cases of single model and multi model.

- Decoding time is increased in 5.3a, as the option of single model is removed, and always two models need to be computed.

5.1…5.5 are related to different modifications in ALT-CCCM (and combinations thereof). 5.1.a and 5.3a each have a reasonable standalone tradeoff. However, a combination of 5.1a and 5.3a was not tested. 5.5d is a combination additionally including 5.2c (combination of 5.2a and 5.2b). Gains of all three together can at least for RA be considered as additive (as for RA 5.2c has almost no gain overall with loss in luma and small gain in chroma). Therefore, it might be concluded that the gain only comes from 5.1a and 5.3a, such that their gains would be additive in combination. Further, the runtime is also approximately the addition of the individual encoder runtimes of all three proposals (2% in total), whereas the addition of individual runtimes only from 5.1a and 5.3a would only be slightly more than 1%.

It was further remarked that the combination 5.2c has worse performance than 5.2a in RA, and for LB perhaps small gain, but a different balance balance between luma and chroma (which likely comes from 5.2b). Also the standalone tradeoff of either 5.2a or 5.2b is not reasonable in RA.

Decision: Adopt JVET-AN0098 Test 5.1a.

Decision: Adopt JVET-AN0101 Test 5.3a.

Test 5.6 does not provide gain in RA, and small loss in chroma. The runtime reduction is unreliable – according to proponents, there should be no change in runtime.

### EE2 contributions: Enhanced compression beyond VVC capability (24)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AN0024. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AN0090 EE2-2.6: Adaptive subsampling filter selection for CCLM/CCCM [Y. Kidani, H. Kato, K. Kawamura (KDDI)]

JVET-AN0251 Crosscheck of JVET-AN0090 (EE2-2.6: Adaptive subsampling filter selection for CCLM/CCCM) [P. Bordes (InterDigital)] [late]

JVET-AN0278 Crosscheck report for JVET-AN0090 (EE2-2.6: Adaptive subsampling filter selection for CCLM/CCCM) [D. Buğdayci Sansli (Nokia)] [late]

JVET-AN0091 EE2-3.2: Joint reordering of GPM with affine prediction [L. Zhang, Y. Yu, Z. Sun, H. Yu, D. Wang (OPPO)]

JVET-AN0315 Crosscheck of JVET-AN0091 (EE2-3.2: Joint reordering of GPM with affine prediction) [Y. Wang (Bytedance)] [late]

JVET-AN0092 EE2-3.3: Joint reordering of GPM with intra prediction [Z. Sun, Y. Yu, L. Xu, H. Yu, D. Wang (OPPO)]

JVET-AN0165 Crosscheck of JVET-AN0092 (EE2-3.3: Joint reordering of GPM with intra prediction) [N. Zouidi (Ofinno)]

JVET-AN0276 Crosscheck of JVET-AN0092 (EE2-3.3: Joint reordering of GPM with intra prediction) [X. Li (Alibaba)] [late]

JVET-AN0093 EE2-3.4: Combination of EE2-3.2 and EE2-3.3 [L. Zhang, Z. Sun, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AN0323 Crosscheck of JVET-AN0093 (EE2-3.4: Combination of EE2-3.2 and EE2-3.3) [X. Li (Alibaba)] [late]

JVET-AN0332 Crosscheck of JVET-AN0093 (EE2-3.4: Combination of EE2-3.2 and EE2-3.3) [C. Ma (Kwai)] [late]

JVET-AN0094 EE2-4.1: Linked Sign Prediction [Y. Zhang, L. Xu, Y. Yu, J. Gan, H. Yu, D. Wang (OPPO)]

JVET-AN0312 Crosscheck of JVET-AN0094 (EE2-4.1: Linked Sign Prediction) [C. Hollmann (TCL), L. Xu (SYSU)] [late]

JVET-AN0095 EE2-4.2a, b, c: Shifting Quantization Center for non-CTC [Y. Yu, H. Yu, J. Gan, L. Xu, H. Huang, F. Wang, Z. Xie, D. Wang (OPPO), M. Pendu, F. Le Léannec, K. Naser (InterDigital)]

JVET-AN0239 Crosscheck of JVET-AN0095 (EE2-4.2a, b, c: Shifting Quantization Center for non-CTC) [P. Nikitin (Xiaomi)] [late]

JVET-AN0291 Crosscheck of JVET-AN0095 (EE2-4.2a*, b*, c*: Shifting Quantization Center for non-CTC) [M. Balcilar (Ofinno)] [late]

JVET-AN0311 Crosscheck of JVET-AN0095 (EE2-4.2a*, b*, c*: Shifting Quantization Center for non-CTC) [B. Ray (Qualcomm)] [late]

JVET-AN0096 EE2-4.2d, e, f: Shifting Quantization Center for CTC [Y. Yu, H. Yu, J. Gan, L. Xu, H. Huang, F. Wang, Z. Xie, D. Wang (OPPO), M. Pendu, F. Le Léannec, K. Naser (InterDigital)]

JVET-AN0240 Crosscheck of JVET-AN0096 (EE2-4.2d, e, f: Shifting Quantization Center for CTC) [P. Nikitin (Xiaomi)] [late]

JVET-AN0097 EE2-4.2g, h, i: Shifting Quantization Center for non-CTC [M. Pendu, F. Le Léannec, K. Naser (InterDigital), Y. Yu, H. Yu, J. Gan, L. Xu, H. Huang, F. Wang, Z. Xie, D. Wang (OPPO)]

JVET-AN0250 Crosscheck of JVET-AN0097 (EE2-4.2g, h, i: Shifting Quantization Center for non-CTC) [M. Balcilar (Ofinno)] [late]

JVET-AN0098 EE2-5.1: On regularization of ALF-CCCM [Z. Xie, N. Song, L. Xu, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AN0295 Crosscheck of JVET-AN0098 (EE2-5.1: On regularization of ALF-CCCM) [M. Jia (ZTE)] [late]

JVET-AN0099 EE2-5.2: On ALF-CCCM [L. Xu, N. Song, Z. Xie, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AN0327 Crosscheck of JVET-AN0099 (EE2-5.2: On ALF-CCCM) [W. Yin (Bytedance)] [late]

JVET-AN0101 EE2-5.3: Updated multi-models usage strategy for ALF-CCCM [N. Song, L. Xu, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AN0333 Crosscheck of JVET-AN0101 (EE2-5.3: Updated multi-models usage strategy for ALF-CCCM) [C. Ma (Kwai)] [late]

JVET-AN0103 EE2-5.4: On ALF-CCCM Model [F. Wang, N. Song, Z. Xie, L. Xu, H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AN0334 Crosscheck of JVET-AN0103 (EE2-5.4: On ALF-CCCM Model) [C. Ma (Kwai)] [late]

JVET-AN0106 EE2-5.5: The combinations of EE2-5.1, EE2-5.2, EE2-5.3 and EE2-5.4 [N. Song, L. Xu, F. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AN0322 Crosscheck of JVET-AN106 (EE2-5.5: The combinations of EE2-5.1, EE2-5.2, EE2-5.3 and EE2-5.4) [P. Astola (Nokia)] [late]

JVET-AN0120 EE2-2.3: Interpolation filter unification for unwrapping and TIMD template generation [T. Dong, V. Rufitskiy, A. Filippov (TCL)]

JVET-AN0288 Crosscheck of JVET-AN0120 (EE2-2.3b: Interpolation filter unification for unwrapping and TIMD template generation) [M. Abdoli (Xiaomi)] [late]

JVET-AN0296 Crosscheck of JVET-AN0120 (EE2-2.3a/ab: Interpolation filter unification for unwrapping and TIMD template generation) [S. Lee, Y. Kim, S. Noh, J. Bang, H. Choi (HNU)] [late]

JVET-AN0121 EE2-1.1: On partitioning optimization [G. Wang, C. Zhou, Z. Lv (vivo)]

JVET-AN0274 Crosscheck of JVET-AN0121 (EE2-1.1: On partitioning optimization) [Z. Deng (Bytedance)] [late]

JVET-AN0328 Crosscheck of JVET-AN0121 (EE2-1.1a: On partitioning optimization) [Y. Liu, Y. Huo, Z. Zhang (Transsion)] [late]

JVET-AN0344 Crosscheck of JVET-AN0121 (EE2-1.1: On partitioning optimization) [Z. Sun (OPPO)] [late]

JVET-AN0353 Crosscheck of JVET-AN0121 (EE2-1.1f: On partitioning optimization) [Y. Liu, Y. Huo, Z. Zhang (Transsion)] [late]

JVET-AN0374 Crosscheck of JVET-AN0121 (EE2-1.1g: On partitioning optimization) [Z. Zhang, Y. Huo, Y. Liu (Transsion)] [late]

JVET-AN0129 EE2-4.3: Residual Sign Prediction restriction [C. Hollmann, A. Filippov (TCL)]

JVET-AN0338 Crosscheck of JVET-AN0129 (EE2-4.3: Residual Sign Prediction restriction) [Y. Zhang (OPPO)] [late]

JVET-AN0158 EE2-2.4: Longer tap interpolation filtering [V. Rufitskiy, A. Filippov, T. Dong (TCL)]

JVET-AN0289 Crosscheck of JVET-AN0158 (EE2-2.4: Longer tap interpolation filtering) [M. Abdoli (Xiaomi)] [late]

JVET-AN0168 EE2-2.8a: Enhanced CCP Fusion mode [P. Bordes, F. Galpin, K. Naser, F. Le Léannec (InterDigital)]

JVET-AN0171 EE2-2.1: TMRL blend [S. Blasi, G. Kulupana, D. Buğdayci Sansli, J. Lainema (Nokia), K. Naser, G. Rath, F. Le Leannec, T. Dumas (InterDigital)]

JVET-AN0302 Crosscheck of JVET-AN0171 (EE2-2.1: TMRL blend) [V. Rufitskiy (TCL), Y. Lu (SYSU)] [late]

JVET-AN0173 EE2-2.8b: Combination of EE2-2.7 and EE2-2.8a [P. Bordes, F. Galpin, K. Naser, F. Le Léannec (InterDigital), S. Wan, Y. Yin, Z. Zhu (NWPU), S. Xie, X. Zeng, C. Huang (ZTE)]

JVET-AN0166 Crosscheck of JVET-AN0168 and JVET-AN0173 (EE2-2.8: Enhanced CCP Fusion mode) [N. Zouidi (Ofinno)]

JVET-AN0329 Crosscheck of JVET-AN0168 and JVET-AN0173 (EE2-2.8a, b: Enhanced CCP Fusion mode) [Y. Kidani (KDDI)] [late]

JVET-AN0199 EE2-5.6: Reuse of TALF control information [Y. Bai, M. Jia, W. Niu, S. Xie, C. Huang (ZTE)]

JVET-AN0300 Cross-check of JVET-AN0199 (EE2-5.6: Reuse of TALF control information) [I. Jumakulyyev (Nokia)] [late]

JVET-AN0345 Crosscheck of JVET-AN0199 (EE2-5.6: Reuse of TALF control information) [Z. Xie, L. Xu (OPPO)] [late]

JVET-AN0220 EE2-2.7: Reducing Candidate Modes in DDCCP [S. Wan, Y. Yin, Z. Zhu (NWPU), S. Xie, X. Zeng, C. Huang (ZTE)]

JVET-AN0252 Crosscheck of JVET-AN0220 (EE2-2.7: Reducing Candidate Modes in DDCCP) [P. Bordes (InterDigital)] [late]

JVET-AN0236 EE2-3.1: Generated Merge Candidates [D. Buğdayci Sansli, J. Lainema (Nokia)]

JVET-AN0241 Crosscheck of JVET-AN0236 (EE2-3.1: Generated Merge Candidates) [R. G. Youvalari (Xiaomi)] [late]

JVET-AN0314 crosscheck of JVET-AN0236 EE2-3.1: Generated Merge Candidates [K. Naser (InterDigital)] [late]

JVET-AN0275 EE2-2.5a: Combination of tests EE2-2.1, EE2-2.3b, and EE2-2.4 [V. Rufitskiy, T. Dong, A. Filippov (TCL), S. Blasi, G. Kulupana, D. Buğdayci Sansli, J. Lainema (Nokia), K. Naser, G. Rath, F. Le Leannec, T. Dumas (InterDigital)] [late]

JVET-AN0308 Crosscheck of JVET-AN0275 (EE2-2.5a: Combination of tests EE2-2.1, EE2-2.3b, and EE2-2.4) [P. Andrivon (Ofinno)] [late]

JVET-AN0375 Crosscheck of Combination of tests EE2-2.3b and EE2-2.4 [G. Verba (Qualcomm)] [late]

### EE2 related contributions (3)

Contributions in this area were discussed during 1535–1615 on Sunday 5 Oct. 2025 (chaired by JRO).

JVET-AN0109 EE2-4.1-related: On Linked Sign Prediction [Y. Zhang, L. Xu, Y. Yu, J. Gan, H. Yu, D. Wang (OPPO)]

This contribution proposes a new linked sign prediction method related to the EE2-4.1. In particular, restrictions to the linked sign prediction (LSP) algorithm are proposed, where only the SP coefficient with the lowest magnitude can be converted to LSP coefficient, and the LSP method will only be applied when the number of those non-SP coefficients inside the SP area (type A coefficients) is less than or equal to the number of predicted signs in the current block. The proposed method is implemented on top of ECM-18.0 and experimental results following the CTC are summarized as follows:

AI: -0.01 % (Y)	 0.06% (U)  0.10% (V)	100.6% (EncT) 100.3% (DecT)

RA: -x.xx% (Y)	 x.xx% (U)  x.xx% (V)   xx% (EncT) xx% (DecT)

LD: -x.xx% (Y)	 x.xx% (U)  x.xx% (V)   xx% (EncT) xx% (DecT)

Overall, no better tradeoff compared to EE2-4.1.

JVET-AN0307 Crosscheck of JVET-AN0109 (EE2-4.1-related: On Linked Sign Prediction) [C. Hollmann (TCL)] [late]

JVET-AN0175 EE2-related: Unbiased Simple Quantizer (Related to EE2-4.2i) [M. Balcilar, M. Blestel, P. Andrivon (Ofinno)]

In JVET-AM0311, it was proposed to apply quantization center shifting when the encoder is supposed to use only Uniform Reconstruction Quantizer (URQ). This setting is non-CTC where the encoder should be configurated with both DQ and RDOQ disabled. In this contribution, it is claimed that URQ quantizer is biased and re-arranging dequantization points decreases the quantization error. Even though Test-4.2i decreases the error by shifting reconstruction points, this contribution shows that further improvement is possible. The experimental results over Test-4.2i are summarized as follows:

AI   :  -0.26% (Y)	 -0.51%(U)	   -0.71%(V)	100.0%(EncT)	100.4% (DecT)

RA  :  -1.29% (Y)	  -2.23 % (U)  -2.29% (V)   100.4% (EncT)	100.7% (DecT)

LDB : -0.79%(Y)     -0.18 % (U)  -0.60% (V)    99.9% (EncT)      100.7% (DecT)

It is proposed to shift the dequantization points such that the distance between 0 and +/-1 is higher than for the higher points.

It was commented that the shift could also be applied at the encoder side.

It was asked if beta would be signalled? No, it is hard coded, same values as currently used for the different slice types.

It was pointed out that signalling beta in slice header might not be costly.

It was agreed to investigate this in an EE along with JVET-AN0324.

JVET-AN0316 Crosscheck of JVET-AN0175 (EE2-related: Unbiased Simple Quantizer) [M. Le Pendu, F. Le Léannec (InterDigital)] [late]

JVET-AN0324 EE2-4.2i-related: Unbiased Simple Quantizer with Quantization Center Shifting [M. Le Pendu, F. Le Léannec (InterDigital)] [late]

In JVET-AN0097, the test EE2-4.2i proposed to apply quantization center shifting when the encoder only uses Uniform Reconstruction Quantizer (URQ). This setting is non-CTC where the encoder should be configured with both Dependent Quantization (DQ) and Rate Distortion Optimized Quantization (RDOQ) disabled. In JVET-AN0175 another adjustment method of the dequantization points was proposed to compensate for the bias of the URQ quantizer in the same non-CTC setting. In this contribution, it is claimed that the URQ quantizer bias can be similarly compensated using the quantization center shifting method with modified weight tables. The experimental results over EE2-4.2i are summarized as follows:

AI   :  -0.26% (Y)	  -0.55%(U)  -0.67%(V) 	99.8%(EncT)	100.3% (DecT)

RA  :  XX% (Y)  XX %(U)  XX% (V)   XXX.X% (EncT)	XXX.X% (DecT)

LDB:  XX% (Y)  XX %(U)  XX% (V)   XXX.X% (EncT)	XXX.X% (DecT)

Similar concept as JVET-AN0175, but different way of implementation. Results are not identical (but gains are almost identical) due to different rounding.

It was agreed to investigate this in an EE.

### ECM modifications and software improvements beyond EE2 (39)

#### Intra and CIIP (18)

Contributions in this area were discussed during 1615–1930 on Sunday 5 Oct. 2025 and during 0930–1035 on Monday 6 Oct. 2025 (chaired by JRO).

JVET-AN0110 Non-EE2: Enhancement of Non-CCP Reordering List [Z. Liu, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO)]

This contribution proposes to extend the non-CCP mode in Intra chroma coding by including additional chroma DIMD modes in the mode candidate list. On top of the ECM-18.0 software, simulation results {Y, U, V, EncT, DecT} for all the test sequences in the CTC are reported below:

AI: -0.02%	0.03%	0.02%, EncT 99.7%, DecT 100.1%

RA: x.xx% x.xx% x.xx%, EncT xx.x%, DecT, xx.x%

Not enough interest expressed due to small gain, and it is not a simplification, additional processing necessary.

JVET-AN0336 Cross-check of JVET-AN0110 (Non-EE2: Enhancement of Non-CCP Reordering List) [Z. Lyu (vivo)] [late]

JVET-AN0123 AHG12: Improvement on chroma MPM [Z. Li, W. Niu, X. Zeng, M. Jia, Y. Wang, C. Huang (ZTE)]

This proposal proposes to modify the chroma MPM list construction, specifically, to modify the MPM list construction for non-CCP mode. The simulation results on top of ECM-18.0 are summarized as follows:

AI : {-0.02%, -0.01%, 0.02%, 99.6%, 100.0%}

Not enough interest expressed due to small gain, and it is not a simplification, additional processing necessary.

JVET-AN0346 Crosscheck of JVET-AN0123 (Non-EE2: Improvement on chroma MPM) [Z. Xie, Z. Liu (OPPO)] [late]

JVET-AN0126 Non-EE2: MPDIP clipping [K.-Y. Kim, J.-H. Son, J.-Y. Kim, J.-S. Kwak (WILUS)]

This contribution proposes a modification to the MPDIP mode by clipping the predicted sample values to the range of the reference samples. This approach aligns with the existing behaviour in the EIP mode. On top of ECM-18.0, the simulation results are reported as follows:

AI: {-0.01%, 0.02%, 0.09%, 99.8%, 100.1%}

RA: {*-0.01%, *0.05%, *0.00%, XXX%, XXX%} 
(*The overall result in RA was obtained by copying several anchor results.)

Not enough interest expressed due to small gain, and it is not a simplification, additional processing necessary to determine clipping values.

JVET-AN0131 Non-EE2: On interpolation filter for SGPM [J. Lee, G. Moon, K. Kim, J.-G. Kim (KAU), J. Lee, S.-C. Lim (ETRI)]

This contribution proposes a method to replace the interpolation filter for Spatial Geometric Partitioning Mode (SGPM). In ECM-18.0, SGPM forms candidates from combinations of the partition, block vectors, and angular modes. Each candidate produces the final predictor by blending two predicted blocks. However, when a smoothing filter is applied for prediction, the resulting prediction can become excessively smoothed. In the proposed method, the 8-tap cubic interpolation filter is used for the angular mode in SGPM. The proposed method was implemented on top of ECM-18.0 software, and the experimental results are summarized as follows:

- AI: -0.02 %, 0.04 %, 0.04 %, 100.2% (EncT), 100.1% (DecT)

- RA: x.x%, x.x%, x.x%, x% (EncT), x% (DecT)

Avoiding the switching between filters is assessed to be an advantage, and even comes with a small coding gain. By using always an 8-tap filter, encoding time should not be impacted, and decoding time might slightly increase.

It was agreed to investigate this in an EE.

JVET-AN0132 Non-EE2: Enhancement of MDIP mode selection [K. Kim, G. Moon, J. Lee, J.-G. Kim (KAU), J. Lee, S.-C. Lim (ETRI)]

This contribution proposes a modification of the Most Dominant Intra Prediction (MDIP) process in ECM-18.0. In ECM-18.0, MDIP derives 8 directional intra-prediction modes from a histogram of gradients (HoG) of neighbouring templates and selects the mode with the lowest template cost. However, this process does not evaluate non-directional candidates, and for modes predicted with Matrix-based Position-Dependent Intra Prediction (MPDIP), the template cost is still compared using the corresponding conventional angular prediction instead of MPDIP itself, which can limit accurate mode selection. To address these issues, the proposed method adds DC mode to the MDIP candidate list and applies MPDIP to template prediction in order to reorder the candidates. The proposed method was implemented on top of ECM-18.0 software, and the experimental results are summarized as follows:

- AI: -0.02%, 0.04%, 0.04%, 100.3% (EncT), 100.0% (DecT)

- RA: x.x%, x.x%, x.x%, x% (EncT), x% (DecT)

The number of candidates is increased by one by adding DC mode.

The usage of MPDIP prediction for template cost calculation likely increases processing complexity.

Preliminary results for RA show losses in some classes, and also for AI most gain comes from class E.

Not sufficient gain to justify investigation – Further study was recommended, e.g. why is loss occurring in RA?

JVET-AN0152 Non-EE2: Multiple Candidate Selection for MDIP [L. Wang, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

In ECM, the MDIP mode is selected as the one with the lowest template cost among eight intra prediction modes derived from HoG, and its usage is indicated by a signalling flag. This contribution proposes a Second Most Dominant Intra Prediction mode, which selects an intra mode with the second lowest template cost from eight intra prediction modes. And a flag is used to indicate which mode is selected.

On top of ECM18.0, simulation results of the proposed method are reported as follows:

For AI configuration: -0.03%, 0.08%, 0.06%, with xx% EncT, xx% DecT.

For some classes, the encoder run time is increased. This may be due to the additional RD check necessary whether the new flag should use the best or the second best mode. The decoder run time should not be affected.

It was agreed to investigate this in an EE. The encoder run time should be brought to reasonable tradeoff to justify against the small coding gain, the number of RD checks should not be increased.

JVET-AN0154 Non-EE2: Position-dependent Weighted Prediction Extension [Y. Wang, X. Zeng, Z. Li, M. Jia, C. Huang (ZTE)]

This contribution proposes a position-dependent weighted prediction extension method. The method expends adjustment range and makes weight adjustment more relevant to position. The proposed method was implemented on top of ECM-18.0 software. The simulation results are summarized as follows,

AI: -0.02%/0.00%/0.04% Y/U/V, 99.9%/99.6% EncT/DecT.

Most gain coming from class E, also D/F/TGM have higher gain than the larger classes.

The gain is not high. Results confirmed and proposal supported by cross-checker.

It was agreed to investigate this in an EE. It should be analysed if it requires more or less processing than the current method. The latter could be the case if more multiplications by zero occur.

JVET-AN0313 Crosscheck of JVET-AN0154 (Non-EE2: Position-dependent Weighted Prediction Extension) [S. Blasi (Nokia)] [late]

JVET-AN0170 Non-EE2: Additional OBIC candidates in MPM list [Y. Kim, S. Lee, S. Noh, H. Choi (HNU), W. Lim, S.-C. Lim (ETRI)]

In this contribution, a modification to MPM list construction is proposed. The MPM list provides a set of highly probable intra prediction modes for a coding unit, reducing signalling overhead and improving compression efficiency. However, the current MPM list relies only on DIMD, neighbouring, and non-adjacent modes, which limits prediction diversity. Therefore, in the proposed method, additional Occurrence-Based Intra Coding (OBIC) modes are included in the MPM list. Moreover, OBIC modes already encompass non-adjacent spatial regions, restriction of search range for non-adjacent modes is applied.

On top of ECM-18.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

- AI : -0.01%, 0.00 %, 0.06 %    {101.7%, 102.7%}

- RA : x.x%, x.x%, x.x%    {xxx%, xxx%}

Tradeoff is far from attractive – further study.

JVET-AN0192 Non-EE2: Modification of reconstructed area for EIP [W. Niu, S. Xie, Z. Li, M. Jia, Y. Bai, C. Huang (ZTE)]

This contribution proposes a replacement method of reconstructed areas for EIP mode. Smaller reconstructed areas are used to replace the existing reconstructed areas to reduce the complexity of ECM. The proposed method was implemented on top of ECM-18.0 software. The simulation results are summarized as follows,

AI: 0.00%/0.04%/0.05% Y/U/V, 99.3%/99.7% EncT/DecT

The area for deriving coefficients is reduced approximately by half.

Results confirmed by cross-checker, also expresses support for proposal.

Otherwise, no significant interest expressed.

JVET-AN0280 Crosscheck of JVET-AN0192 (Non-EE2: Modification of reconstructed area for EIP) [L. Xu (OPPO)] [late]

JVET-AN0194 Non-EE2: Extension on TMRL mode [Y. Liu, Y. Huo, Z. Zhang (Transsion)]

This contribution introduces two modifications to the Template-based Multiple Reference Line (TMRL) mode. The first modification is two distinct reference line sets for different coding unit (CU) sizes. The second is the use of differently sized templates for different CU sizes. The performance of the proposed method, evaluated against ECM-18.0, is summarized as follows {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

AI { -0.02%, 0.02%, 0.09%, 87.9%, 100.1%, 99.7%, 100.0%}

RA { xxx%, xxx%, xxx%, xxx%, xxx%, xxx%, xxx%}

Overall, the method in both aspects would increase the average complexity (using higher number of reference lines for larger blocks, and larger sets of templates for larger blocks as well). For reference line 1 always a template size of 1 is used.

Worst case complexity might not be affected.

Preliminary results in RA indicate loss in class C.

Encoding time is unreliable, an independent expert reports that he found roughly 100 %.

Not sufficient gain to justify complexity increase – Further study was recommended, e.g. why is loss occurring in RA, what is the benefit of the two aspects, and what is complexity impact in terms of processing on average and in worst case?

JVET-AN0200 Non-EE2: On improving CIIP Fusion [Y. Wang, X. Zeng, M. Jia, Z. Li, C. Huang (ZTE)] [late]

This contribution proposes a CIIP position-dependent weighted prediction method, that is not limited to fixed weights and makes the weight adjustment more relevant to position. The proposed method was implemented on top of ECM-18.0 software. The simulation results are summarized as follows,

LDB: -0.04%/0.18%/0.07% Y/U/V, 99.9%/99.8% EncT/DecT.

RA (preliminary results): around -0.01% luma gain.

Weight derivation is sometimes changed from subblock level to pixel level.

Not enough interest expressed due to small gain, and it is not a simplification, additional processing at pixel level necessary.

JVET-AN0354 Crosscheck of JVET-AN0200 (Non-EE2: On improving CIIP Fusion) [Z. Zhang (Alibaba)] [late]

JVET-AN0219 AHG12 IntraTMP with DMVR [K. Naser, P. Bordes, F. Le Léannec, A. Robert (InterDigital)]

This contribution is about enabling DMVR (decoder side motion vector refinement) mode with IntraTMP coded blocks. Specifically, 2 block vectors from the IntraTMP list are selected and a block matching refinement is performed. The resulting predictors are blended using constant weights. The following results are obtained:

AI 	-0.05%	0.00%	0.03% 100.1%	100.1%

Basically this is applying a weighted bi prediction in the main mode in IntraTMP (only in case of full pixel). No new mode is introduced. Search is performed by block matching starting from the current vector, using a 3x3 search range, 2 iterations. The blending is done 3/4 and 1/4.

This is not the worst case of IntraTMP modes, which may be the reason that the decoding time is not largely increased. A report on the frequency of usage would be helpful.

It was agreed to investigate this in an EE.

JVET-AN0221 AHG12: Reference sample generation for intra prediction [T. N. Canh, P. Yin, S. McCarthy (Dolby)]

This contribution proposes a reference sample generation (RSG) method to generate missing reference samples in intra prediction. For intra coding block in inter frames, RSG replaces simple repeated padding of missing reference samples with available collocated reference samples. On top of ECM-18.0, the BD-Rate achieved of RSG is as follows:

RA (Y/ Cb/ Cr):   { 0.01%, -0.01%, 0.00%} {xx.xx%, xx.xx%}

LDB (Y/ Cb/ Cr): { -0.09%, 0.25%, -0.03%} {xx.xx%, xx.xx%}

Five rate points are copied from ECM-18.0 anchor in RA.

In RA, high loss in class A1, one sequence (Campfire)

Impact on runtime should be small, as only copying colocated samples.

Comments were made that it appears strange an encoder had selected intra, but still appropriate samples are found in a temporal reference.

It was commented that rather than just copying reference samples it would be appropriate to apply LMCS.

It was commented that there may be problems when GDR or RPR is used.

It was commented that usage of motion compensated samples might give better results, but that would also increase complexity.

Several experts expressed interest to study this in an EE.

It was agreed to investigate this in an EE, including study of applying LMCS, and reporting about possible interference with GDR and RPR. Also LDP results to be reported.

JVET-AN0234 Non-EE2: On filtering condition for angular modes [Z. Zhu, Y. Yin, H. Xu, S. Wan (NWPU), S. Xie, X. Zeng, C. Huang (ZTE)]

In ECM-18.0, the decision on whether to apply reference smoothing filter is determined based on the MDIS condition. However, in the current ECM version, the existing condition may not be appropriate because of current prediction methods. This contribution proposes to disable usage of {1, 2, 1}-filtered reference samples in some larger blocks than those currently allowed, thereby avoiding switching between the cubic interpolation filter and the gaussian interpolation filter. On top of ECM-18.0, the overall coding performance for AI configuration are reported as below:

when the block size condition is modified from 32 to 64:

Natural: -0.01%, 0.02%, 0.04%, with 100.1% EncT, 99.7% DecT.

Class F: -0.06%, -0.14%, -0.24%, with xxx% EncT, xxx% DecT.

when the block size condition is modified from 32 to 256:

Natural: -0.01%, 0.04%, 0.04%, with 100.0% EncT, 100.0% DecT.

Class F: -0.10%, 0.08%, -0.17%, with 100.0% EncT, 98.9% DecT.

Almost no change in coding efficiency for camera captured content is reported, although there is some gain in class F.

Decoding time seems unreliable.

Even though it is not completely avoiding switching, it could be a complexity advantage. It could be tested in an EE what happens if the condition is modified for all blocks.

It was agreed to investigate this in an EE, and also to test a case where the cubic filter is used for all block sizes (and no boundary sample filtering is applied), and also report results for the TGM sequences.

JVET-AN0279 Crosscheck of JVET-AN0234 (Non-EE2: On filtering condition for angular modes) [L. Xu (OPPO)] [late]

JVET-AN0270 Non-EE2: CCCM with clipping [L.-C. Xu, C.-F. Liao, Y.-K. Lu, F. Liang (SYSU)] [late]

The CCCM method in ECM clips the chroma prediction value. Usually, the clipping is based on the effective bit depth of the pixel, limiting the pixel range to 0~2bitdepth−1. This proposal suggests limiting the output of the prediction value to the chroma value range of the reference samples instead of the entire sample bit depth range. On top of ECM-18.0, simulation results of the proposed method are reported as below:

AI:

Overall: {-0.03%, 0.00%, 0.01%};

Class F: {-0.03%, -0.36%, -0.47%};

Class TGM: {-0.14%, -0.21%, -0.18%};

Chroma losses are observed in some sequences (campfire, park running), which leads to the fact that overall no gain in chroma is resulting.

Was this applied to all CCCM modes? Yes. Also tried for CCLM? No.

Clipping ranges are determined from the reference area for each CU. This is likely the reason for decoder run time increase of 0.4% on average. Would it be possible to do it at coarser granularity, e.g. CTU level?

It was agreed to investigate this in an EE, and also to study the possibility of less frequent derivation of ranges, and to study the reason for losses for some sequences.

JVET-AN0286 Non-EE2: TMRL Angle Offset Refinement [G. Rath, K. Naser, F. Le Léannec, T. Dumas (InterDigital)] [late]

This contribution proposes refining prediction angles for TMRL modes with farther MRL indices. It is asserted that, for these indices, more information is available between adjacent prediction directions than the near ones. It is therefore proposed to refine the TMRL angles to improve the prediction quality. The following results (Y, Cb, Cr, EncT, DecT) are obtained on top of ECM-18 CTC.

AI 	-0.03%, 0.04%, 0.02%, 99.8%, 100.5%

RA -0.01%, 0.05%, 0.03%, 99.9%, 100.5%

It was reported that other offset values had been tested, but the reported values were found optimum.

Several experts expressed interest.

It was agreed to study this in an EE.

JVET-AN0287 Non-EE2: Combined Angular- and Gradient-PDPC [G. Kulupana, S. Blasi, N. Neumann, J. Lainema (Nokia)] [late]

This contribution proposes to combine the angular-PDPC component with a gradient-PDPC component obtained for intra-prediction angles that currently support angular-PDPC. Several combination criteria and the associated weights are experimented.

The impact on coding efficiency and runtimes of the proposed method over ECM-18.0 are reportedly {for Y, U, V, EncT, DecT }:

Test1 : AI {-0.02%, 0.05%, 0.06%, 100.1%, 100.0%}, RA {-0.01%, 0.08%, 0.03%, 99.8%, 99.9%}

Test2 : AI {-0.02%, 0.02%, 0.07%, 100.0%, 100.0%}, RA {-0.01%, 0.08%, 0.03%, xx.xx%, xx.xx%}

This replaces angular PDPC.

It was commented that test 2 appears more simple and (luma only) has slightly better performance.

Was not applied on the TIMD part.

It was commented that gain does not appear consistent over the classes.

Several experts expressed interest.

It was agreed to study this in an EE.

JVET-AN0359 crosscheck of JVET-AN0287 Non-EE2: Combined Angular- and Gradient-PDPC [K. Naser (InterDigital)] [late] [miss]

JVET-AN0299 Non-EE2: Improvement for candidates on MPM [W. Niu, Z. Li, S. Xie, M. Jia, Y. Bai, C. Huang (ZTE)] [late]

This contribution proposes an improvement method of candidates for MPM. A new candidate that is related to DIMD and MDIP mode is added to MPM list. And the MDIP derivation process is also improved by adding some default intra mode. The proposed method was implemented on top of ECM-18.0 software. The simulation results are summarized as follows,

AI: -0.02%/0.05%/0.06% Y/U/V, 99.6%/99.9% EncT/DecT,

Results confirmed by cross-checker, but did not find reduction in run time.

It was commented that the first element (adding average of DIMD and MDIP to MPM list) appears more complicated than the second element (modifying derivation of MDIP adding more intra modes). It was also commented that the two elements may be contradicting, as it makes MDIP closer to the default MPM modes.

It was agreed to study this in an EE. Also study the benefit of the elements separately (using non-modified MDIP in the averaging of the first elements).

JVET-AN0309 Crosscheck of JVET-AN0299 (Non-EE2: Improvement for candidates on MPM) [L. Xu (OPPO)] [late]

#### Inter (3)

Contributions in this area were discussed during 1035–1115 on Monday 6 Oct. 2025 (chaired by JRO).

JVET-AN0111 Non-EE2: additional candidates for regular inter AMVP candidate list [C. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

In this contribution, additional spatial adjacent candidates, additional temporal collocated candidates, and newly added pairwise-average candidates are introduced to the regular inter AMVP candidate list. The test results on top of ECM-18.0 are summarized as follows:

Test a: add 4 pairwise-average candidates + additional spatial candidates + additional temporal candidates

RA: Overall: { -0.03%(Y), -0.05%(U), 0.01%(V), 101.9%(EncT), 101.0%(DecT)}.

LDB: Overall: { -0.07%(Y), 0.09%(U), 0.10%(V), 101.3%(EncT), 101.6%(DecT)}.

Test b: add 2 pairwise-average candidates + additional spatial candidates + additional temporal candidates

RA: Overall: { -0.02%(Y), 0.02%(U), -0.06%(V), 100.7%(EncT), 100.0%(DecT)}.

LDB: Overall: { -0.06%(Y), 0.16%(U), 0.04%(V), 101.2%(EncT), 100.3%(DecT)}.

The additional run time is probably caused by additional re-ordering (10 additional candidates, 4 pairwise, 6 spatial, and replacing the temporal).

What is the benefit of the different elements? Most of the gain comes by pairwise-average candidates.

It was agreed to study this in an EE, and to study the different aspects separately (benefit of changing the temporal, benefit of adding pairwise, and adding spatial). Both cases with 2 or 4 pairwise candidates should be tested.

JVET-AN0356 Crosscheck of JVET-AN0111 (Non-EE2: additional candidates for regular inter AMVP candidate list) S. Hong (Nokia) [late]

JVET-AN0218 Non-EE2: Extension of Sub-Block Merge Mode Availability Conditions [Y. Liu, Y. Huo, Z. Zhang (Transsion)]

This contribution proposes an extension of the sub-block merge mode. The availability conditions of the sub-block merge mode are expanded to incorporate additional adjacent neighbouring blocks and non-adjacent neighbouring blocks for mode availability checking. The performance of the proposed method over ECM-18.0 is reported as follows {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

Test a: without the limitation of CU size

RA { -0.01%, -0.03%, 0.00%, 93.7%, 100.5%, 100.0%, 100.0%}

LDB* { -0.05%, 0.18%, -0.12%, 93.8%, 100.0%, 100.0%, 99.8%}

Test b: with the limitation of CU size larger than 32 samples

RA  class D { -0.09%, 0.04%, -0.08%, 91.5%, 99.8%, 100.0%, 100.0%}

LDB class D { -0.24%, 0.23%, -0.52%, 91.7%, 102.3%, 100.0%, 100.0%}

Encoding and decoding times are unreliable. According to proponents, there should only be minor increase (however might be increase due to additional RD checks in 4x4 blocks).

The results of test b seem to indicate that the method is not effective for 4x4 blocks, as those are probably used frequently in class D.

It was agreed to study this in an EE.

JVET-AN0266 Pairwise Merge Candidates with Motion Vector Scaling [S. Hong, L. Wang, K. Panusopone, D. Rusanovskyy (Nokia)]

This contribution proposes an extension to pairwise candidate derivation for merge candidates in ECM-18.0. Unlike the current design, which only allows pairwise candidates with the same reference index, the proposed method enables pairwise candidates across different reference indices by applying motion vector scaling. Simulation results based on ECM-18.0 are reported as follows:

LB: { -0.05%, 0.06%, 0.10%, 99.8%, 98.6%}.

RA: { 0.0%, 0.02%, 0.03%, 98.4%, 98.9%}.

Slide deck should be uploaded.

Results confirmed by cross-checker. Reduction in run time appears unrealistic, but should also not largely increase.

It was agreed to investigate this in an EE.

JVET-AN0358 Crosscheck of JVET-AN0266 (Pairwise Merge Candidates with Motion Vector Scaling) [C. Wang (OPPO)] [late]

#### GPM (3)

Contributions in this area were discussed during 1115–1145 on Monday 6 Oct. 2025 (chaired by JRO).

JVET-AN0142 Non-EE2: GPM without blending for screen content [J.-Y. Kim, J.-H. Son, K. Kim, J.-S. Kwak (WILUS)]

For screen content videos, a blending width of 1/4τ is consistently used for SGPM-coded blocks. However, the adaptive blending method remains enabled for the GPM-coded blocks. This contribution proposes the GPM-coded blocks in without blending for screen content, consistent with the SGPM without blending. Based on ECM-18.0, corresponding simulation results are reported.

RA configuration:

Class F: -0.02%, 0.04%, 0.05%; 99.7% enc time, 100.0% dec time

Class TGM: -0.12%, -0.12%, -0.08%; 99.2% enc time, 100.1% dec time

LDB configuration:

Class F: -0.18%, -0.03%, 0.04%; 99.9% enc time, 100.2% dec time

Class TGM: -0.20%, -0.30%, -0.02%; 99.2% enc time, 98.7% dec time

It was commented that for mixed content (as some sequences in class F) it may not be helpful to turn off the blending. Otherwise, it is definitely useful for graphics content.

It was agreed to investigate this in an EE.

JVET-AN0155 Non-EE2: On improving adaptive GPM blending [Z. Zhang, Y. Huo, Y. Liu (Transsion)]

This contribution presents improvements of adaptive GPM blending method. The selection of the blending width index (geoBldIdx) is optimized based on block size, prioritizing likely values for smaller blocks to improve coding efficiency. This low-complexity extension, requiring only minimal code additions, achieves the following results on top of ECM-18.0:

RA: xxx (Y), xxx (U), xxx (V),xxx(EncT), xxx (DecT)

LB: -0.05% (Y), 0.32% (U), -0.06% (V), 90.8% (EncT), 100.4% (DecT).

The encoding time is unreliable, while the decoding time is reliable.

Preliminary results indicate less gain in RA, but gains in LB are interesting.

The decoding time would not be expected to increase according to crosschecker (code change is simple), except for the fact that GPM would be used more frequently.

It was agreed to study this in an EE.

JVET-AN0283 Crosscheck of JVET-AN0155 (Non-EE2: On improving adaptive GPM blending) [M. Abdoli (Xiaomi)] [late]

JVET-AN0191 Non-EE2: Binarization Improvement of GPM [X. Wang, J. Chen, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)]

This contribution presents a modified index binarization table for Geometric Partitioning Mode   (GPM). Specifically, it merges the last three index groups in the original table. In addition, for screen content, the coding of the first four indices is changed from fixed-length to variable-length,, drawing on the coding method originally designed for camera-captured content. Experimental results of the proposed binarization scheme, implemented on top of the ECM-18.0, are reported as follows:

LB: {-0.04%, 0.19%, -0.05%, xxx% ,99.9%}

LB: Class F {-0.01%, 0.03%,0.09%,xxx%, 101.7%}

Class TGM {-0.19%, -0.16%, 0.09%, xxx%, 99.7%}.

RA: {-xx%, xxx%, -xxx%,xxx%, xxx%}

RA: Class F  {0.02%, 0.22%, 0.04%,xxx%,100.4%}

Class TGM {-0.09%, -0.11%, -0.07%, xxx%, 100.5%}.

It was commented that it is not desirable to use different binarization in the same tool for different types of content. This also does not give relevant insight into compression capability in the context of an exploration.

JVET-AN0321 Crosscheck of JVET-AN0191 (Non-EE2: Binarization Improvement of GPM) [K. Jia (Alibaba)] [late]

#### In-Loop Filters (7)

Contributions in this area were discussed during 1200–1345 on Monday 6 Oct. 2025 (chaired by JRO).

JVET-AN0086 AHG12: Bug Fix for ALF Coefficient Calculation [R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

The contribution reports a reliability issue related to the solved ALF coefficients at low QP for screen content. The issue is identified to be related to the amplification of precision errors—originating from using the float data type—when the covariance matrix for coefficient calculation becomes ill-conditioned.

Corresponding fixes are proposed and implemented on top of ECM-18.0. Specifically, replacing the float data type with int64_t for storing covariance matrix elements yields the following results:

All-Intra:

Class F:			 { 0.00%,  0.01%,  0.00%, xx.x%, xx.x%}

Class TGM:	 { 0.00%,  -0.02%,  -0.01%, xx.x%, xx.x%}

All-Intra low QP (2, 7, 12, 17, 64-th):

Class F:			 { -0.09%,  0.00%,  0.00%, xx.x%, xx.x%}

Class TGM:	 { -0.74%,  0.00%,  0.00%, xx.x%, xx.x%}

The results of using the residual check are as follows:

All-Intra:

Class F:			 { 0.02%,  0.00%,  0.00%, xx.x%, xx.x%}

Class TGM:	 { 0.00%,  -0.09%,  -0.08%, xx.x%, xx.x%}

All-Intra low QP (2, 7, 12, 17, 64-th):

Class F:			 { -0.09%,  -0.01%,  -0.01%, xx.x%, xx.x%}

Class TGM:	 { -0.72%,  -0.01%,  -0.01%, xx.x%, xx.x%}

The effect happens if the correlation between original and reconstruction comes close to 1.

It was commented that solution 1 would be consuming a lot of memory.

Solution 2 might be OK, but has a 0.02% loss for class F in CTC (only slide show).

Confirmed by cross-checker.

Decision (SW): Adopt JVET-AN0086, disabled by default. It is left to the discretion of the software coordinator if solution 1 or solution 2 is better.

JVET-AN0350 Crosscheck of JVET-AN0086 (AHG12: Bug Fix for ALF Coefficient Calculation) [Z. Zhu, S. Wan (NWPU)] [late]

JVET-AN0113 Non-EE2: Additional candidates for merge mode in ALF-CCCM [H. Zhang, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)]

This contribution proposes to check more spatial candidates into the ALF-CCCM merge list so that a CTU may inherit both the block size and the model type from more spatial adjacent and non-adjacent CTUs. On top of the ECM-18.0 software, simulation results {Y, U, V, EncT, DecT} for all the test sequences in the CTC are reported below:

RA: 0.00%, 0.01%, 0.02%, EncT 100.2%, DecT 100.0%

LB: -0.06%, 0.21%, 0.04%, EncT 100.1%, DecT 100.1%

Why is it causing chroma loss whereas ALF-CCCM is targeting chroma gain?

It was asked if this also is useful with multi-model? Yes, only the model type is inherited.

More candidates are checked, but the merge list size is kept constant.

Support was expressed to study this in an EE. It was also suggested to the following configuration of additional candidates, which might save memory (not storing parameters over 2 CTU rows)

JVET-AN0331 Crosscheck of JVET-AN0113 (Non-EE2: Additional candidates for merge mode in ALF-CCCM) [H.-J. Jhu (Kwai)] [late]

JVET-AN0114 Non-EE2: Local boosting CCP for ALF-CCCM [H. Huang, Y. Yu, H. Yu, D. Wang (OPPO)]

In this contribution, the 3×3 low-pass filter from Local-Boosting Cross-Component Prediction (LB-CCP) method is applied to ALF-CCCM. For each CTU, a flag is signaled to indicate whether the filtering is applied or not. On top of ECM-18.0, the simulation results are summarized as below:

LB: -0.05% (Y), 0.10% (U), 0.12% (V), xx.x% (EncT), xx.x% (DecT)

RA: 0.00%(Y), 0.01%(U), 0.04%(V), xx.x% (EncT), xx.x% (DecT)

Most gain from low resolution classes (sometimes also loss in chroma), and almost no gain (or sometimes loss) in RA.

Encoder run time of current implementation increases by 1%.

Further study for more consistent gain over different classes, and also consistent gain in RA.

JVET-AN0343 Crosscheck of JVET-AN0114 (Non-EE2: Local boosting CCP for ALF-CCCM) [Z. Deng (Bytedance)] [late]

JVET-AN0130 Non-EE2: Improvement on Frame-level Inheritance Mode in ALF-CCCM [M. Jia, Y. Bai, X. Zeng, Z. Li, C. Huang (ZTE)]

This contribution introduces an improvement to the frame-level Inheritance mode in ALF-CCCM. Specifically, the maximum number of reference frames that can be inherited has been increased from 1 to 2. The proposed method shows coding gains as follows:

LB: -0.08% (Y), 0.11% (U), 0.09% (V), 99.5% (EncT), 100.5% (DecT)

RA: xx% (Y), xx% (U), xx% (V), xx% (EncT), xx% (DecT)

Preliminary results indicate only small gain in RA for classes B and C.

The decoding runtime is likely increased by more usage of CCCM.

Support was expressed to study this in an EE.

JVET-AN0347 Crosscheck of JVET-AN0130 (Non-EE2: Improvement on Frame-level Inheritance Mode in ALF-CCCM) [Z. Xie (OPPO)] [late]

JVET-AN0163 AHG12: On improving the bi-filtering modes in TALF [P. Astola, D. Buğdayci Sansli, J. Lainema (Nokia)]

In ECM-18.0 the bi-filtering modes of temporal ALF use as input the average of the samples in the two reference pictures. This contribution proposes removing the computation of the average from the centermost 3x3 filter inputs and instead uses the samples from the two reference pictures independently. Filter size is kept same at 13 inputs.

The impact on coding efficiency and runtimes of the proposed method over ECM-18.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

RA 	{0.00%, -0.03%, -0.02%, 100.1%, 100.0%, 100.0%, 100.1%}

LDB 	{-0.05%, 0.14%, 0.03%, 100.0%, 98.7%, 100.0%, 100.0%}

A number of additions and shifts are saved in TALF bi-filtering mode. The benefit in compression comes by the fact that the input to TALF that comes from different reference frames can be processed independently in the derivation of the coefficients.

The cross-checker confirmed the results, and also confirmed the simplification.

It was agreed to study this in an EE.

JVET-AN0337 Crosscheck of JVET-AN0163 (AHG12: On improving the bi-filtering modes in TALF) [N. Song (OPPO)] [late]

JVET-AN0164 Non-EE2: ALF coefficient coding group extension [I. Jumakulyyev, D. Buğdayci Sansli, J. Lainema (Nokia)]

In this contribution, ALF coefficient coding group extension is proposed to improve ALF coefficient coding efficiency. The method proposes to extend the coding groups of coefficients.

The coding efficiency and runtimes of the proposed method over ECM-18.0 is reported as below {for Y, U, V, EncT, DecT}:

AI {0.00%, -0.01%, 0.00%, 99.9%, 100.0%}

RA { -0.02%, 0.01%, -0.03%, 100.1%, 100.0%}

LDB { -0.06%, 0.06%, 0.02%, 99.9%, 99.9%}

It was commented that some of the gain may be due to retraining, which according to proponents is however necessary to get the full benefit of the method.

Is applied to both luma and chroma.

The BVI-D dataset was used for training.

It was agreed to investigate this in an EE, and also to compare against an anchor that is retrained without the group extension approach.

JVET-AN0297 Crosscheck of JVET-AN0164 (Non-EE2: ALF coefficient coding group extension) [V. Shchukin (Ericsson)] [late]

JVET-AN0269 AHG12: BIF with before DBF samples [K. Takada, S. Deshpande (Sharp)]

The bilateral filter (BIF) is carried out in the sample adaptive offset (SAO) stage. The bilateral filter (BIF), SAO and CC-SAO are using samples from deblocking as input. Each filter creates an offset per sample, and these are added to the input sample and then clipped, before proceeding to ALF. This contribution proposes the BIF using the samples before the deblocking filter (DBF). The outputs of the proposed BIF are blended with that out the original BIF. The proposal is implemented on the top of ECM-18.0. The BD-rate PSNR of proposed approach compared to ECM-18.0 is as follows:

•	{-0.04 %, 0.16 %, 0.04 %} for {Y, Cb, Cr} on LDB condition

Almost no change of compression in AI, loss 0.01%in RA due to larger loss in on A2 sequence.

The bilateral filter as building block is unchanged.

Based on RDO made at slice level, it is decided whether the original or the new branch is used.

It was commented that it might also be interesting to investigate the performance of BIF before deblocking in general, rather than adding an additional branch.

It was commented that the results indicate larger losses (in particular in chroma) for some sequences.

Generally, the additional complexity introduced by the new building block appears too large to be justified by the relatively small coding gain.

Further study was recommended.

#### Entropy coding, transforms, quantization, and transform coefficient coding (6)

Contributions in this area were discussed during 1810–1950 on Monday 6 Oct. 2025 (chaired by Y. Ye).

JVET-AN0087 AHG12: Budget Control for Transform Skip Residual Coding [R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

In TSRC, assigning higher context budget priority to syntax elements with greater coding efficiency has been shown to improve performance, as demonstrated in JVET-AM0062. Following this principle, this contribution proposes to increase the priority of coeff_sign_flag, which also exhibits relatively high coding efficiency.

The tests in this proposal are conducted on top of one method described in JVET-AM0062 and the bug fix described in JVET-AN0086.

Simulation results based on ECM-18.0 are reported as belowClass F:			 {-0.10%, -0.02%, -0.04%, xx.x%, xx.x%}

Class TGM:	 {-0.23%, -0.28%, -0.32%, xx.x%, xx.x%}\

RA:

Class F:			 {-0.09%, 0.02%, -0.28%, xx.x%, xx.x%}

Class TGM:	 {-0.29%, -0.29%, -0.35%, xx.x%, xx.x%}

Low QP AI (2, 7, 12, 17, 64-th):

Class F:			 {-0.14%, 0.27%, 0.13%, xx.x%, xx.x%}

Class TGM:	 {-1.24%, -0.53%, -0.54%, xx.x%, xx.x%}

JVET-AN0086 proposed an ALF-related modification to improve the ALF coefficient derivation process which could result in coding performance gain at very low QP range (2, 7, 12, 17). Option 2 of what was proposed in JVET-AN0086 had been just adopted. However, the results reported in this contribution used option 1 of what was proposed in JVET-AN0086. It was reported by the proponent that the performance impact due to JVET-AN0086 (both options) is negligible as that contribution mainly affects low QP coding performance.

This proposal is based on the concept of prioritizing context-coded bins for residual coding as proposed in JVET-AM0062 (which was an EE test at meeting, but wasn’t adopted at the last meeting due to it coding performance loss at low QP range for classes F and TGM). However, the technology in this contribution isn’t otherwise related to JVET-AM0062.

The anchor was modified to include the encoder modification in JVET-AN0086. Compared to ECM-18+ encoder modification in JVET-AN0086, the following results are shown:

CTC QP:

ClassF: -0.10%, -0.03%, -0.05%

ClassTGM: -0.23%, -0.26%, -0.53%

lowQP:

ClassF: -0.05%, 0.27%, 0.14%

ClassTGM: -0.51%, -0.52%, -0.53%requested to upload a revision that includes the above data.

It was commented that coding results should have been provided for natural content as well, as TSRC is also used in natural content.

JVET-AN0112 is a related contribution, which uses alternative method to solve the same problem.

It was commented that given the performance loss at low QP range had been assertedly solved, it might be interesting to study this proposal in an EE. Several experts supported this.

Results for all sequences (including natural content) and low QP coding results should be provided.

JVET-AN0351 Crosscheck of JVET-AN0087 (AHG12: Budget Control for Transform Skip Residual Coding) [Z. Zhu, S. Wan (NWPU)] [late]

JVET-AN0088 AHG12: Context Modeling for sig_coeff_flag in TSRC [R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

This contribution proposes to modify the context modeling of sig_coeff_flag. Specifically, the number of contexts for sig_coeff_flag in Transform Skip mode is increased to improve context modeling and overall compression efficiency.

On top of ECM-18.0, simulation results of the proposed method are reported as below:

All-Intra:

Class F:			 {-0.04%, -0.06%, -0.26%, xx.x%, xx.x%}

Class TGM:	 {-0.07%, -0.08%, -0.12%, xx.x%, xx.x%}

Six additional contexts are added by the proposed method.

The proposed method is applied to all slice types, however, only AI results are provided. It was commented that to put a proposal that affects inter coding into EE, merely having AI results isn’t enough.

Futher study recommended, including studying the performance impact on RA and LDB configurations, and also on natural content.

JVET-AN0112 Non-EE2: Coding of a coefficient level in TSRC [Y. Yu, L. Xu, J. Gan, H. Yu, D. Wang (OPPO)]

This contribution proposes to always code a significant flag with context to indicate if the coefficient level is zero or not for all coefficient levels within all non-zero subblocks of a TSRC block. In order to guarantee all significant flags coded with context and all context coded bin consumption still follows the current TSRC CABAC bin budget, this contribution proposes to reserve as many context bins as the total number of positions of all non-zero subblocks. The pre-defined CABAC bin budget for a TSRC block is updated accordingly by subtracting the number of reserved context bins and then used as a new CABAC bin budget to control the coding of other context coded syntax. In addition, all significant flags within a subblock are coded together in a separate coding pass before coding any other context coded syntax elements for a coefficient level so that there is no interleaving of context coded bins and bypass coded bins. This contribution also proposes that an adaptive Rice parameter may be used to binarize the remaining of the coefficient levels and Rice parameter is signalled at slice level if needed. For CTC application, adaptive Rice is disabled and Rice parameter 0 is used. On top of ECM-18.0, the simulation results are reported as follows.

AI: Natural content:       { 0.00%, 0.00%, 0.03%, 100.1%	, 99.8%}

Class F:	                    {-0.12%, -0.15%, -0.27%, 99.1%, 98.1%}

Class TGM:                {-0.21%, -0.26%, -0.32%, 99.2%, 100.5%}

RA: Natural content:      { -0.01%,,0.01%,,-0.04%, 100.4%, 99.9%}

Class F:	                    {-0.06%, -0.02%, -0.02%, 100.5%, 99.3%}

Class TGM:	              {-0.16%, -0.16%, -0.08%, 100.3%, 100.6%}

LDB: Natural content:    { 0.00%, -0.03%, 0.02%, 100.5%, 100.3%}

Class F:	                     {0.08%, -0.02%, -0.53%, 101.0%, 101.0%}

Class TGM:	        {-0.17%, -0.07%, -0.20%, 101.0%, 98.2%}

LowQP AI: Natural content:  { -0.03%, -0.07%, -0.07%, 100.6%, 100.2%}

Class F:	                     { -0.18%, -0.14%, -0.18%, 99.6%, 100.3%}

Class TGM:                { -1.03%, -0.95%, -0.90%, 101.2%, 100.4%}

This proposal is related to JVET-AM0062, which was an EE test at the last meeting.

To solve the performance loss at low QP, adaptive Rice parameter derivation (which is a method supported in VVC v2 but with small modification of the set of allowed parameters) at slice level is used, invoked by encoder-side screen content detection algorithm.

It was asserted by the proponent that if in combination with the encoder modification in JVET-AN0086, this proposal could potentially achieve more gain than reported for low QP. proposal would increase the number of TSRC coefficient coding passes 1 pass in order to code significance flags in a separate pass, which are all context coded. Then, this number is subtracted from the total budget.

Cross checker confirmed the reported results.

Several experts expressed support for this method.

The method proposed in this contribution can be combined and harmonized with JVET-AN0087 and conducted as one EE test for the next round of investigation.

It was commented that JVET-AN0086 was adopted as an encoder software change but turned off by default for CTC.

It was commented that the adaptive Rice parameter derivation in RRC could potentially be used in TSRC to achieve some of the gains reported here. However, that would be a different proposal.

It was agreed to investigate this in an EE, as one EE test together with JVET-AN0087. For low QP results, provide test results should be provided with the JVET-AN0086 encoder change turned on and turned off.

JVET-AN0284 Crosscheck of JVET-AN0112 (Non-EE2: Coding of a coefficient level in TSRC) [M. Abdoli (Xiaomi)] [late]

JVET-AN0125 Non-EE2: On CABAC bin budget constraint [K.-Y. Kim, J.-H. Son, J.-Y. Kim, J.-S. Kwak (WILUS)]

This contribution proposes a solution to the issue of exceeding the bin budget limit. Specifically, the threshold for checking the remaining bin budget is increased from 4 to 8. Accordingly, before parsing any coefficients during regular residual coding, the decoder checks whether the number of remaining bin budget is less than 8 (instead of 4). The overall bin budget control mechanism remains unchanged. On top of ECM-18.0, the simulation results are reported as follows:

AI: {0.00%, 0.03%, 0.05%, 100.0%, 100.0%}

RA: {0.00%, 0.08%, 0.01%, 100.1%, 100.2%}

LDB: {-0.03%, 0.19%, 0.30%, 99.9%, 100.4%}

LDP: {0.01%, -0.16%, -0.09%, 100.0%, 100.4%}

The proposal proposes to change the interval of checking the remaining bin budget from 4 coefficients in ECM to 8 coefficients.

Cross checker reports matched results.

It was asked why the proposal, which practically reduces number of context coded bins, can achieve a small gai in LDB. The reason is unknown (perhaps small fluctuation). 
It was commented that exactly the same modification had been proposed before, and no action was taken because nothing is broken. The small chance of exceeding context coded bins is irrelevant at this stage of the exploration.

No action at this moment.

JVET-AN0325 Crosscheck of JVET-AN0125 (Non-EE2: On CABAC bin budget constraint) [P. Nikitin (Xiaomi)] [late]

JVET-AN0169 Non-EE2: On inter multiple transform set selection and advanced SBT [J. Huo, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

As is known to all, the 1st and 2nd candidates of multiple transform set selection (MTSS) should have different transform sets or different transpose types. In ECM 18.0, for intra MTSS, if the 2nd candidate fails to meet this criterion, it will be modified. However, for inter MTSS, no measures ensure this criterion. This contribution proposes that for inter MTSS, if the 1st and 2nd candidates have the same or similar transform sets with identical transpose types, the 2nd candidate should be modified. Specifically, for NSPT applied blocks, the 1st IPM is used to select kernels from set0 (regular intra mode kernels) instead of the 2nd IPM, aligning with the operation in intra MTSS. It is also proposed to fix a bug in ASBT. On top of ECM-18.0, the simulation results are summarized as follows:

RA:

Class A1:0.01%(Y), -0.22%(U), 0.04%(V), xx.x% (EncT), xx.x% (DecT).

Class B: -0.01% (Y), 0.03% (U), 0.02% (V), xx.x% (EncT), xx.x% (DecT).

Class C: -0.05% (Y), 0.04% (U), -0.03% (V), 100.2% (EncT), 100.1% (DecT).

LDB: -0.09% (Y), 0.15% (U), 0.00% (V), xx.x% (EncT), xx.x% (DecT).

For the proposed modification in ASBT, ECM-18.0 is not broken, and therefore the proposed element should be considered a proposed improvement, rather than a bug. It was commented that the suggested modification seems sensible and straightforward.

The two elements in this contribution are not inter-dependent.

A cross checker confirmed the results and supported to study this in an EE.

It was agreed to investigate this in an EE, studying the two elements separately.

JVET-AN0349 Crosscheck of JVET-AN0169 (Non-EE2: On inter multiple transform set selection and advanced SBT) [Y. Yin, S. Wan (NWPU)] [late]

JVET-AN0214 AhG12: CABAC contexts retraining [F. Galpin (InterDigital)]

This contribution proposes to update context initialization parameters for all slice types. All CABAC parameters (initial probabilities parameters, window sizes, adaptive weights and rate offsets) are retrained and updated. It is reported that on top of ECM-18.0, the overall coding performance impact for {Y, U, V} is {-0.04%,0.05%, 0.08%} {-0.07% -0.07% -0.07%} {-0.07% 0.26% 0.70%} in AI, RA and LDB configurations respectively.

Last time CABAC contexts were retrained was for ECM-15.0.

Results for Class TGM were not available.

More results were presented Friday 10 Oct. after class TGM results became available. Even though class TGM (which was included in training) shows slight loss, and class F is also not improving, several experts supported to adopt, as the overall gain is attractive, and the last retraining was done several cycles ago..

Decision: Adopt JVET-AN0214.

#### Other (2)

Contributions in this area were discussed during 1950–2005 on Monday 6 Oct. 2025 (chaired by Y. Ye).

JVET-AN0211 AhG12: Second reference frame for temporal partitioning prediction [G. Laroche, P. Onno, B. Galmiche (Canon)]

This contribution presents the results of an additional reference frame for the temporal partitioning prediction for random access case. In the adopted temporal partitioning prediction, JVET-AH0135, the split modes allowances, the partitioning depths and the split syntax elements ordering are derived for each block according to the partitioning parameters of the current frame and parameters obtained from a temporal area. In this contribution, a second temporal area is considered under some conditions to derive a second set of temporal partitioning parameters.

Compared to ECM-18.0, the average BD rate gains and runtimes reported in this contribution are as follows:

0.04%	0.03%	0.06%	97.9%	99.6% for the RA configuration.

Additionally, results in the scope of an ECM 5x configuration used for the CfE are also provided for the CTC sequences.

It was commented that this proposed method is a normative change that is only shown to save encoding time. Although there is some run time reduiton, there is no evidence that the proposed method can be used to achieve other kinds of tradeoff, such as achieving some performance gain.

No action at this moment.

JVET-AN0228 Independent CTUs for TM-padding [N. Neumann (Nokia)]

The TM-padding (JVET-AK0085) has been adopted into ECM as an advanced reference picture padding algorithm, which is especially useful for low-delay applications. The proposed method introduces an independency of individual CTU segments. It is reported that the performance of the proposed method against ECM-18.0 anchor is measured as:

{Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

RA   {-0.01%,   0.01%, 0.02%,    99.7%, 100.2%,   99.7%, 100.0%}

LDB {-0.04%, -0.04%, 0.16%,  100.3%,   99.0%, 100.3%, 100.0%}

Compared to the TM-padding method in ECM-18, the proposed method introduces a constraint:

| Allowed candidate | Disallowed candidate |
| --- | --- |

It was commented that the proposed modification is based on CTU-independence principle, which is generally desirable, and achieves some gain.

It was agreed to investigate this in an EE.

### CTC for EE2/ECM and general ECM improvements (3)

Contributions in this area were discussed during 2005–2055 on Monday 6 Oct. 2025 (chaired by Y. Ye).

JVET-AN0118 AHG6: ECM software extension for RDO count information [Y. Tokumo, S. Hong, T. Ikai (Sharp)]

This contribution proposes ECM software extension for RDO count information. The proposed software outputs three types of RDO count, i.e. rdoCountRdCost, rdoCountBestMode, and rdoCountModeCost for all the RD cost, tracing best mode and tracing mode cost calculation, respectively. Experimental results show that encoding runtime and RDO count are correlated and RDO count seems to be useful for developing as this is a reliable complexity metric; that is, exactly same values are obtained irrespective of environments. It is recommended to adopt the proposal to the next version of ECM software.

The proponents suggest that they do not intend to require RDO count report for all proposals. It is instead meant for experts to utilize this evaluation method for their tool development.

The source code based on ECM-18.0is available at JVET ECM fork repository, https://vcgit.hhi.fraunhofer.de/tokumo/ECM/-/tree/ECM-18.0-rdoCount.

It was commented that if someone writes a separate RDO branch of code within the ECM, the code in this contribution might not be able to catch that RDO. RDO count is currently not normalized by CU size.

Cross checker commented that the proposed software change seems straightforward, and would not put burden on future tool/software development.

Several experts commented that more careful study should be conducted on this proposed software, and the activity should be moved to AHG7 for further inspection of the code, as well as RDO complexity in terms of block sizes.

Further study in the conext of AHG7, e.g. RDO complexity impact in relation to block size, etc..

JVET-AN0352 Crosscheck of JVET-AN0118 (AHG6: ECM software extension for RDO count information) [B. Wang, R. Chernyak (Tencent)] [late]

JVET-AN0151 AHG6: Additional config files for ECM 2x and ECM 5x [T. Ikai, K.-W. Liang (Sharp)]

This contribution proposes the inclusion of additional configuration files for different operation points, referred to as ECM 2x and ECM 5x, utilizing the conditions outlined in JVET-AL0245 and JVET-AM0332.

It is asserted that making these configurations easily accessible would be beneficial for both experts and various purposes. VTM also has alternative configurations in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/blob/master/cfg/alternative-addon/reduced_runtime[1, 2, 3].cfg as in JVET-AM0200.

The same configurations in this contribution were used in the CfE for 5x and 2x runtime coding. Currently, these configurations were stored only in the form of email messages.

Multiple experts suggested that these configuration files should be stored together with ECM software, in the same way as what was done for the VTM reduced runtime configurations 1, 2 and 3.

Decision (sw. config.):  store these configuration files together with ECM software.

JVET-AN0217 AHG6: Software Implementation for Runtime Reporting [S.-Y. Lim, J. Gao, J.-Y. Thong, H.-B. Teo, C.-S. Lim, K. Abe (Panasonic)]

This contribution presents a software implementation for ECM-18.0 that provides separate runtime measurement for different processes with the goal to enable a more accurate assessment of traditional coding tools' complexity. The implementation reports the runtime of the neural network-based tool as well as auxiliary functions such as MD5 checksum and all objective metric calculations. Since NN components running on CPUs can dominate total runtime and mask the performance of non-NN tools, and auxiliary functions are not part of the actual encoding/decoding process, separating their runtime provides a fairer and more accurate assessment of traditional coding tool’s complexity. Therefore, it’s proposed to add the runtime measurement codes to ECM to measure the following:

- Runtime of NN-based tool including NN Loop Filter and optionally NN Intra.

- Runtime of MD5 checksum calculation.

- Runtime of all objective metric calculation, wherein the objective metric calculation includes PSNR metric, MS-SSIM metric, HDR metric, etc.

So far, AHG7 has not been separating out NN tools and conducting coordinated study around only NN-tool complexity.

It was commented that it was not clear the benefits of having these additional timing data being reported, and further, it would burden all participants with updating their scripts.

Perhaps, if and when CfP is drafted, some finer granularity reporting of runtime might be needed. However, it is premature to go into that discussion at this moment.

No action at this moment.

# High-level syntax (HLS) and related proposals (100)

## AHG9: Aspects of SEI messages in VSEI, VVC, HEVC and AVC (4)

Contributions in this area were discussed during 0900–0955 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

JVET-AN0071 AHG9: On HEVC Omnidirectional viewport SEI in HEVC [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

It is asserted the polynomial describing the sample location remapping process in the HEVC Omnidirectional viewport SEI message semantics should begin at power one and not power zero. In addition, the technical description used when the number of coefficients is zero is recognized as a subset of the general polynomial case and the description may be simplified. Two options to address this issue are presented. This change is proposed for future HEVC errata such as JVET-AM1018.  A delta text relative to JVET-AM1018 is also included.

- (technical) Change exponent in equation D-61 from j to j+1

- (additional editorial) Change exponent in equation D61 and modify semantics when coefficients are not signalled to infer a polynomial and use equation D61 in all cases.

- (technical) A third option is mentioned.  A single sentence is added stating that the signalled DC coefficient is replaced by zero before applying equation D61.

It is noted that the proposed change is about the fisheye message, rather than the omnidirectional viewpoint.

It was suggested to check with the original contributor of the fisheye message.

Further discussion requested.

Further discussion 6 Oct 2025.

-v3 version of contribution includes a new option.

Agreed to the recommended solution in v3.

JVET-AN0072 AHG9: On 3DRD, MAI and Alternative Depth information SEI messages in HEVC and VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

Several editorial changes in describing floating point values in 3DRD SEI message of HEVC, DA structure and MAI Alternate depth information messages included in both HEVC and VSEI are proposed. This change is proposed for HEVC errata such as JVET-AM1018 and VSEI.  A delta text relative to JVET-AM1018 is also included.

1) (editorial). Transpose table G.2, of 3D references display SEI (HEVC only), for consistency between floating point descriptions i.e. Table G.4, G.5.

2) (editorial) Remove notes mentioning similarity to withdrawn specification IEC 60559:1989 used in 3DRA, DA, MAI, and Alternate Depth information messages.

3) (editorial) Clarify MAI semantics syntax elements in association table.

a) Introducing variables for length of different mantissa elements of the MAI message

b) Add an additional column to table G.5 to describe the length of mantissa elements of MAI floating point syntax.

4) (editorial) Renaming syntax elements corresponding to MAI floating point elements in MAI and Alternate Depth Information SEI messages.

Delegate all editorial improvements to the editors.

JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0185 also relates to the NNPF, SPO, and DSC SEI messages

This contribution proposes miscellaneous modifications that are asserted to provide improvement to some aspects of the AVC / HEVC / VVC interface text for VSEI v4. The proposed modifications are as follows:

(Related to NNPFC SEI messages)

1	Modify a constraint on repetition of NNPFC SEI message.

(Related to SPO SEI messages)

2	Add constraints addressing cubemap projection SEI message in the AVC / HEVC interface text.

3	Add missing payloadType values of SeiProcessingOrderSeiList in the AVC / HEVC interface text.

(Related to DSC SEI messagess)

4	Add additional nal_unit_type values for NonVclDigitallySignedNalUnitsList.

Agreed to item 1 as a sensibility constraint.

Agreed to item 2, 3.

Regarding item 4, it is noted that in multi-layer AVC it depends on the profile if some NAL unit types are VCL or non-VCL. It may be desirable to not require the verifier to consider the profile.

Further discussion requested on item 4.

Further discussed on 5 Oct 2025.

Adopt item 4 with revised language regarding modifications to AVC interface of NAL unit type for types 20 and 21 to always be considered as VCL NAL unit for the purposes of signing, subject to review of text.

Further discussed on 9 Oct 2025, adoption confirmed.

JVET-AN0363 Interface text for SEI message extensions [J. Boyce (Nokia), Y. Sanchez, K. Sühring (Fraunhofer HHI)]

Chaired by S. Deshpande on 9 October 2025.

NNPFA and FGC SEI messages in  use an SEI extension mechanism that is supported in  and VVC but not in AVC.  US-017 and US-020 include the following:

| US-017 | The text mentions the possibility of extending SEI message payloads. However, some referencing specifications may not have full flexibility in this regard. AVC can reference VSEI for enabling the inclusion of only those SEI messages for which the SEI payloads are specified in VSEI but the syntax has not been extended; while HEVC and VVC can reference VSEI for enabling the inclusion of SEI messages for which the SEI payloads are specified in VSEI regardless of whether the syntax has been extended or not. This issue may not be clear to those who use the AVC standard. | Add a NOTE in 6.1 to describe the limitations of using this text with AVC. |  |
| --- | --- | --- | --- |
| US-020 | A variable called SeiExtensionBitsPresentFlag is set to a value, but this variable is never used. | If the variable has no purpose, remove it. |  |

It is proposed to add interface text in VSEI to describe that the value of SeiExtensionBitsPresentFlag is derived in the NNPFC and FGC SEI messages. SeiExtensionBitsPresentFlag is initialized to 0, and set to 1 if an extension is present.

In the AVC interface text, a constraint is imposed that SeiExtensionBitsPresentFlag shall be equal to 0.

Agreed, with the removal of the initialization of SeiExtensionBitsPresentFlag.

### Ballot comments

Ballot comments were discussed during 1530 - 1700 on Thursday 9 Oct. 2025 (chaired by J. Boyce), and f

urther discussed 11 Oct 2025.

For comment **-039, the ISO secretariat wants any external normative reference to include “shall” language.

We need to update the forward to indicate the new SEI message added in this edition.

#### VSEI

| US-007 |  | D.2.1 | The name “ai_usage_restrictions( )” may imply a requirement to conform to the expressed restriction preferences. | Consider another name, such as ai_usage_restriction_preferences( ). |
| --- | --- | --- | --- | --- |

Suggested response: It was clarified that the name does not imply a requirement.

After further discussion, the name was changed to “AI usage restriction request”.

Table 29 – Definition of eoi_privacy_protection_method_idc

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Blurring; area in the picture that shows personal information is blurred to make it unidentifiable or less readily identifiable. |
| 0x02 | Replacing; area in the picture that shows personal information is replaced with something different from the original to make it unidentifiable or less readily identifiable. |
| 0x04 | Masking; area in the picture that shows personal information is masked to make it so that it cannot be identified unidentifiable or less readily identifiable. |
| 0x08 | PixelationPixelization; pixel size in area in the picture that shows personal information is pixelated modified to make it undiscernible unidentifiable or less readily identifiable. |

The above modifications are agreed to address comment US-027.

JVET-AN0368 AHG9: On semantics of ifm_type_id for IFM SEI [A. Hinds, S. Wenger, G. Teniou (Tencent)]

It is reported that the reference to TIFF 6.0 for the IFM SEI does not conform to the ITU rules for citing normative references in the current ballot text for VSEI v4, and should be removed. The corresponding semantics for type ‘0’ of the IFM SEI accordingly should be updated to remove the reference to TIFF 6.0 and instead cite the EXIF specification by CIPA, which is now approved to be used as a normative reference through the ITU A.4 and A.5 processes.

| **-039 | ISO 20473:2007, Optics and photonics – Spectral bands. — NIST FIPS PUB 180-4, Secure Hash Standard (SHS). — ISO/IEC 21617-1:2024, Information technology - JPEG Trust - Part 1: Core Foundation. — Rec. ITU-T T.871 ¦ ISO/IEC 10918-5 (in force), Information technology - Digital compression and coding of continuous-tone still images: JPEG File Interchange Format (JFIF) — Exchangeable image file format for digital still cameras: Exif Version 3.0, Camera & Imaging Products Association, May 2023. — ISO 16684-1: Graphic Technology – Extensible metadata platform (XMP) specification– Part 1 Data model, serialization and core properties. — ISO 16684-3: Graphic Technology – Extensible metadata platform (XMP) specification – Part 3 JSON-LD serialization of XMP. — ISO 15076-1 Image technology colour management — Architecture, profile format and data structure — Part 1: Based on ICC.1:2010 — ISO 20677 Image technology colour management — Extensions to architecture, profile format and data structure | ISO/IEC Directives, Part 2, 2021, 15.5.3: ‘Only references cited in the text in such a way that some or all of their content constitutes requirements of the document shall be listed in the Normative references clause.’ | Check ISO 20473:2007, Optics and photonics – Spectral bands can be moved to informative. JPEG trust was moved to informative. NIST FIPS PUB 180-4, Secure Hash Standard (SHS) is normative: When a DSCI SEI message is present in an AU, the calculation of NumVerificationSubstreams message digests, one for each verification substream, is initialized according to the specification in NIST FIPS PUB 180-4 for the specified dsci_hash_method_type JFIF is normative. When ifm_type_id[ i ] is equal to 2, the i-th image format metadata payload contains bytes from one or more JFIF extension markers, without the APP0 marker segment identifiers themselves, as specified in ITU-T Recommendation T.871 ¦ International Standard ISO/IEC 10918-5. |
| --- | --- | --- | --- |

ITU publishes a list of

Organizations recognized according to Recommendation ITU-T A.5 (linked here).

https://www.itu.int/net4/ITU-T/lists/sdo.aspx

The group Agreed to the modifications in this contribution.

It was commented that SeiExtensionBitsPresentFlag = 0 may not be needed for AVC (minor editorial issue).

It was suggested to consider adding SeiExtensionEnabledFlag to other places where needed, it was asked if anyone is aware of such messages.

#### VVC

Initial review of VVC DoC.

| US-011 | D.12.11 | There are three terms defined at the beginning D.12.11. These are formatted partly as if they were in Clause 3, but also partly not. For example, they have a colon after each term and no line break. They are phrased as sentences, which is contrary to ISO/IEC style. | Rewrite these as ordinary sentences or put them into clause 3 and format them accordingly. |  |
| --- | --- | --- | --- | --- |
| US-013 | D.12.11 | NOTE 3 does not appear to be merely an optional observation. It appears to express a prescriptive requirement. | Rewrite this to be prescriptive or clarify it so it appears less prescriptive. |  |

For US-011 and US-013, check to see if HEVC had a similar comment, and if so, try to align the resolution.

For US-013, it was suggested to try to make the note less prescriptive.

## AHG9: Aspects related to VSEI version 4 (33)

VSEI v4 is intended for FDIS and ITU-T consent at this meeting. There is a preference for design stability. Bug fixes and editorial improvements are OK. We intend to be cautious about adoptions that change the design. Small items with clear benefits may be considered.

### Editorial updates (3)

Contributions in this area were discussed during 0955–1205 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

JVET-AN0067 AHG9: Comments on Neural-network Post-filter in VSEI V4 and V3 [S. Deshpande (Sharp)]

JVET-AN0067 also relates to the NNPF SEI messages

Following asserted bug-fix is proposed related to neural-network post-filter in VSEI V4 and V3:

- Proposal 1: It is asserted that for certain value of y that can occur, the function InpSampleVal( y, x, picHeight, picWidth, croppedPic, cIdx ) does not specify what the sampleVal should be when nnpfc_padding_type is equal to 3. An else clause is proposed to be added to provide a valid value for sampleVal under those conditions.

Additionally following bug-fix and typo-fix are proposed related to neural-network post-filter in VSEI V4:

- Item 1: When TemporalExtrapolationFlag is equal to 1, equation 79 for computation of numPicsInOutputTensor should use the signalled syntax element nnpfc_extrapolated_pics_minus1.

- Item 2: The missing descriptor of u(1) is specified for the syntax element nnpfa_no_foll_clvs_flag in neural-network post-filter activation SEI message

- Item 3: In Figure 12, on the right side the value “16” should be changed to the value “15” to correspond to the value on the left side. (Also applies to VSEI V3)

For proposal 1, it was suggested to use wrapping instead of clipping for the x component.

Adopt proposal 1 modified to use wrapping for the x component. A -v2 version of the document to be uploaded with this modification.

Delegate all editorial improvements to the editors.

JVET-AN0133 AHG9: Miscellaneous editorial changes for the VSEI v4 draft [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes various asserted purely editorial changes to the current VSEI v4 draft, which are briefly summaried as follows:

- Various asserted typos in the colour transform SEI message.

- Change "decoders shall ignore ..." to "decoders conforming to this version of this document shall ignore ...".

- Various other editorial changes in the GCMP, NNPFA, EOI, SPTI, TDI, GFE etc. SEI messages.

Delegate all editorial improvements to the editors.

JVET-AN0186 AHG9: Editorial updates for VSEI v4 [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes several editorial modifications for the VSEI v4 draft and the HEVC interface text. The proposed modifications are as follows:

(Related to OMI SEI messages)

1	Clarify the units of the syntax elements for specifying a position and a size of masks.

(Related to SPTI SEI messages)

2	Change the wording in the constraint on a frame-field SEI message.

(Related to PRI SEI messages)

3	Remove the unnecessary sentence in the semantics of pri_persistence_flag.

Item 1 clarifies that the units are in chroma samples rather than luma samples, which is already reflected in the equations.

Agreed item 1, item 2.

It was suggested that the sentence about ordering of SEI messages is general and can be included in a different spec location about output order. Delegate to the editors.

### Specification of syntax functions and descriptors (3)

Contributions in this area were discussed during 1020–1200 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

JVET-AN0134 AHG9: On the SEI message payload extension in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the SEI message payload extension in VSEI:

1)	Add a note to clarify the differences in referencing VSEI by different standards.

2)	Editorial change of the definition of more_data_in_payload( ).

3)	Add definition of payload_extension_present( ).

Regarding item 1, the note seems to be directed to specification editors more than to implementers.

It is noted that other standards such as systems might have the same issue with extensions.

It was suggested that “MPEG Rec. ITU-T H.264 | ISO/IEC 14496-10 can only reference VSEI for enabling” would be better to move the “only”, for this wording:  “ MPEG Rec. ITU-T H.264 | ISO/IEC 14496-10 can reference VSEI only for enabling”

Agreed to add the note. Exact wording is delegated to editors.

Agreed to items 2 and 3.

JVET-AN0076 AHG9: On floating point signalling in SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0076 also relates to VSEI, VVC, HEVC and AVC.

Chaired by S. Deshpande on 04 October 2025 from 11:00-11:15.

Several SEI messages include floating point values.  Differences in naming of syntax elements and presentation of semantics are used.  Common methods are noted.  It may not be practical to modify all floating-point descriptions in already standardized SEI messages but a recommended procedure for unified presentation of floating-point values in draft and future SEI messages is provided. HEVC and VSEI have notes that mention the withdrawn specification IEC 60559:1989.

1) (Editorial) Information describing several conventions related to floating point syntax and semantics collected form reviewing floating point elements in current SEI messages.  Proposals in separate contributions, areas mentioned here for harmonization.

a) Suggested syntax naming convention.

b) Suggested syntax order convention. (One technical change for MI message is noted see JVET-AN0073 for the formal proposal.)

c) Semantics table convention including content and structure (see JVET-AN0072 for proposals for table alignments of current HEVC and VSEI messages)

2) (Editorial) Propose use of a floating-point helper function to section 5.8 of VSEI common floating-point construction.  Use this helper function in current TuC and future SEI messages.  This similar to the function defined in the Alternative depth information SEI message but that function is only used withing the single SEI message.

3) (Editorial) Propose to removal or modify mentions of withdrawn specification version IEC 60559:1989.

4) (Future) In addition to aligning existing SEI messages, these conventions are proposed for future use.

a) Syntax naming

b) Syntax order

c) Common structure for semantic association table.

d) Use of external floating point helper function.

V2 had one document number change only and was presented.

Regarding reference to ISO 60559, it was commented that the referencing rules require referencing a particular version of the standard.

There was a general agreement that the word “similar” in the NOTE 3 is unclear. See further discussion jointly with JVET-AN0242

JVET-AN0242 AHG9: Signalling of floating-point values in VSEI and graphics rendering information SEI [V. Zakharchenko, J. Boyce (Nokia)]

JVET-AN0242 also relates to the GRI messages

Chaired by S. Deshpande on 04 October 2025 from 11:15-11:45.

This contribution proposes to add to the VSEI TuC a definition of a descriptor type, fl(n), to specify representation of a floating-point value with a single syntax element, rather than using separate syntax elements for the sign, exponent, and mantissa. The fl(v) version of the descriptor may be used to support different lengths/precisions of floating-point numbers, with the length determined by other syntax elements. It is proposed to restrict the allowable lengths of the floating-point syntax element to those that are expected to be commonly implemented in hardware. A table associates each allowable syntax element length with an exponent length and mantissa length.

It is proposed to modify the graphics rendering information SEI message in the TuC to use this descriptor for syntax elements representing floating-point values. It is also suggested to consider its use for other SEI messages in the TuC that contain syntax elements representing floating-point values, including the lens optical correction SEI and the localization and mappingProponents thought that no change should be made to V4 SEIs, and thus the proposal is only for TuC.In the proposed table the fl(8) is additionally defined by proponents compared to ISO 60559:2020.

It was asked if we could copy and paste from ISO 60559:2020 in our speficiation. It was commented that normative  referencing is better than copying and pasting. It was also suggested to add an informative note in our specification in addition to a normative reference.

For the fl(x) where x is not defined by the normative reference, a particular w, t may be  possibly defined in our specification. However it does not prevent syntax elements in SEIs to define their own separate syntax elements which have different w, t.

It was commented that a more flexible floating point descriptor with different values for w, t could be defined.

- General discussion of the above two proposals (JVET-AN0076, JVET-AN0242):agreed that making some changes for the future SEIs is reasonable.

- Regarding already defined SEIs, no change should be made to SEIs other than possibly the modality SEI (which is yet to be finalized in v4). After further discussion even modality SEI is kept as it is.

- For the future (including SEIs in Tuc) the choices include defining:

  - Fl(s,w,t) s= sign

  - Fl(w,t)

  - Fl(n) – referencing ISO 60559:2020

Additional parameter could be base 10 or base 2.

Another option could be to define both Fl(w, t), and Fl(n). The end goal of these is to define w, t.

Regarding defining a common descriptor for floating point syntax for future VSEI specification only: it was agreed to do this, while still allowing a flexibility for some SEIs to define their own separate floating point syntax elements.

It was commented to not put multiple options in the TuC, but rather to further study and choose one in the future. It was suggested and agreed that if we define Fl(n), we should not define n values in our specification other than those defined in the normative reference.

V3 was discussed on 5 October from 17:40-17:xx.

It was commented that it may be useful to add a sign column to the table. In this case, the sign is always one bit.

It was asked what is the advantage of defining both flf(w,t) and fl(n). It was commented that doing it gives more flexibility and simplified notation (fl(n)) in some cases.

It was asked if the referred to ISO/IEC document is expected to have longevity.

It was commented that some consideration of bias should be taken into account.

Agreed to add the yellow highlighted text from V3 into TuC.

About using the fl(n) notation for graphics rendering information SEI, the proponents mentioned that the current u(v) definition in TuC for that SEI was underspecified and this is somewhat of a “bug-fix”/ clarification for it.

Agreed to use fl(n) as the descriptor for various syntax elements in graphic rendering information SEI.

### Film grain characteristics SEI message (1)

Contributions in this area were discussed during 1205–1300 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

1)	Move the payload extension inside the if( ) loop for fg_characteristics_cancel_flag.

2)	Remove the ChromaFormatIdc condition in the VSEI interface text

3)	Add two notes on the presence of fg_matrix_coeffs

4)	Change the semantics of fg_comp_model_present_flag[ c ] when it is equal to 0 to reflect the actual process.

5)	Add a sensibility constraint for fg_intensity_interval_upper_bound[ c ][ i ] and fg_intensity_interval_lower_bound[ c ][ i ].

6)	Add inference for fg_comp_model_value[ c ][ i ][ 0 ] when it is not present.

7)	Various editorial changes.

Agreed to item 1.

For item 2, there is a problem with misalignment between the VVC and VSEI interfaces. An alternate suggestion is to change the inference of fg_bit_depth_chroma_minus8 to not use BitDepthC, but to infer it to be equal to the luma bit depth.

It was noted that there is an issue with the bit depth of the FGC SEI – there is a bit depth of the coded picture and a bit depth that is used for the model, which may differ. This is not addressed in this contribution.

Agreed to item 2.

Further discussion requested foritem 3.

Item 4 proposes that the fg_comp_model_present_flag[ c ] not be used to indicate whether or not the film grain modeling of colour components take place. A proponent suggested that this was not the design intent.

OK to add the cyan text, but don’t remove the yellow highlighted text.

It was suggested that the existing wording “film grain is not modelled on the c-th colour component” be changed to allow more flexibility to the decoder.

Delegate the editors adding the presence text, without removing the wording about modelling the colour components.

Regarding item 5, a concern was raised about backwards compatibility.suggested to add a note.

No action on item 5.

Regarding item 6, it was suggested that the inference value for fg_comp_model_value[ ][ ][ ] be 0 rather than 1 as proposed.

Agreed on item 6 with an inferred value of 0.

Delegate editorial changes in item 7 to the editors.

Further discussion on 6 Oct. 6, 2025.

No action necessary for item 3.

Item 1 applies to VSEI-v4 and HEVC.

Items 4, 6, and 7 apply to VSEI-v4, HEVC, and AVC.

### SEI processing order and processing order nesting SEI messages (4)

Contributions in this area were discussed during 1430–1540 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AN0059 also relates to the JVET-AM1006 (HEVC), JVET-AM1017 (AVC), and JVET-AM2005 (VVC).

Chaired by S. Deshpande on 04 October 2025 from 14:00-14:50.

This contribution proposes the following changes related to the SEI processing order (SPO) and processing order nesting (PON) SEI messages:

- The requirement that the nnpfc_for_human_viewing_idc and nnpfc_for_machine_analysis_idc shall be equal to po_for_human_viewing_idc and po_for_machine_analysis_idc, respectively, when the NNPFA SEI message has the greatest value of po_sei_processing_order[ i ] is asserted to miss the case when the NNPF is the last process but is followed by property SEI message type(s) in a processing chain. It is proposed that the requirement applies when the NNPFA SEI message has the greatest value of po_sei_processing_order[ i ] among all processes of a processing chain.

Decision: Agreed

- The semantics of the PON SEI message include a number of payload-type-specific constraints ensuring that the semantics and effect of an SEI message that is not a PON-nested SEI message shall not depend on any PON-nested SEI message. Among those payload-type-specific constraints, there is a constraint that when an NNPFC SEI message with nnpfc_id nnpfId is a PON-nested SEI message, any associated NNPFA SEI messages with nnpfa_target_id equal to nnpfId shall also be PON-nested SEI messages.

It is proposed to relax the constraint to concern the same value of the base flag, i.e., when an NNPFC SEI message with nnpfc_id nnpfId and nnpfc_base_flag nnpfBaseFlag is a PON-nested SEI message, any associated NNPFA SEI messages with nnpfa_target_id equal to nnpfId and nnpfa_target_base_flag equal to nnpfBaseFlag shall also be PON-nested SEI messages.

Additionally, it is proposed to limit the scope of the constraint to SEI messages within the same CLVS.

The proposal relaxes a constraint for an assertedly reasonable use case.

There was some support to relaxing a few constraints at this meeting. There was also specific support for the proposed changes.

Decision: Agreed

- It is proposed to add a constraint requiring that when a PON-nested NNPFC SEI message with nnpfc_base flag equal to 1 is present, all other NNPFC SEI messages with the same value of nnpfc_id shall also be PON-nested.

It was asked if in the proposed text the word “associated” should be removed. Proponent agreed to this change. There was support for adding this constraint.

Decision: Agreed

- Because the same constraint appears in both VSEI and VVC, it is proposed to remove the following from VVC, clause D.12.13: "When a generalized cubemap project SEI message is present with gcmp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated generalized cubemap project SEI message in the same CLVS that is a PON-nested SEI message."

Decision: Agreed to remove the constraint from VVC to avoid duplication.

- AVC and HEVC include the following constraint: "The SEI messages contained in a PON SEI message are referred to as PON-nested SEI messages. When a cubemap projection SEI message is present with cmp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated cubemap projection SEI message in the same {CVS|CLVS} that is a PON-nested SEI message."

It is proposed to clarify "an associated cubemap projection SEI message" in AVC and HEVC with phrasing that is similar to a NOTE that appears in VSEI:

When a cubemap projection SEI message is present with cmp_persistence_flag equal to 1 that is not a PON-nested SEI message and is associated with an SEI message type indicated by an SEI processing order SEI message seiMsgSpo, there shall not be an associated cubemap projection SEI message in the same {CVS|CLVS} that is a PON-nested SEI message associated with seiMsgSpo.

Decision: Agreed as a clarification of existing intent.

JVET-AN0065 AHG9: On the SPO and PON SEI message in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm), Y.-K. Wang (ByteDance), H. Tan (LG Electronics Inc.), Y. Sanchez (Fraunhofer HHI), M. M. Hannuksela (Nokia)]

This contribution proposes the following bug fix and constraints to the SPO and PON SEI message in VSEI v4.

- A bug fix on the multiple processing chains constraint

- Constrain the SPO SEI message content space

- Remove a redundant condition from the PON-nested SEI message dependency constraint for consistency

Agreed on item 1

For item 2, it was commented that the design intent was to allow flexibility to modify the prefix indication and data. No action.

For item 3, it was noted that the two SEI messages for the proposed changes both have an ID value.

Further discussion requested on item 3.

Further discussed 5 Oct 2025, chaired by S. Deshpande.

There were multiple co-authors who preferred option 2.

Agreed to option 2 with the final wording in -v3 to be uploaded.

JVET-AN0181 AHG9: On process SEI messages in processing chain in SPO SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution asserts that a display orientation a packed regions information (PRI), and neural-network post-filter activation (NNPFA) SEI messages that follows region-wise packing SEI message in processing order have a problem that may breaks a projected picture. The following is proposed to address the problem:

1	On a RWP SEI message and a display orientation SEI message in a processing chain

–	Disallow the presence of both a RWP SEI message and a DOI SEI message in a processing chain.

2	On a RWP SEI message and a PRI SEI message in a processing chain

–	Option 1: Disallow the presence of both the RWP SEI message and the PRI SEI message in a processing chain.

–	Option 2: Disallow the PRI SEI message from following the RWP SEI message in the processing order.

3	On a RWP SEI message and a NNPFA SEI message in a processing chain

–	Disallow an NNPFA SEI message that is associated with an NNPF including spatial extrapolation from following a RWP SEI message in processing order

In the v2 of this document, some editorial errors were fixed.

Regarding option 3, it was suggested that the combination of NNPF SEI with spatial extrapolation and RWP SEI might be useful and shouldn’t be disallowed.

It was suggested that a note could be added to indicate that some combinations of SEI messages in the same processing chain could lead to odd behaviour.

Discussed after language for a note was available in a v3 of the document, on October 7, chaired by S. Deshpande.

Agreed for v4.

JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 also relates to the EOI and SPTI SEI messages

This contribution proposes miscellaneous modifications that are asserted to provide improvement to some aspects of VSEI v4 draft. The proposed modifications are as follows:

(Related to SPO SEI messages)

1	Clarify that a processing order applies to the PRI SEI message only in the current CLVS.

(Related to PON SEI messages)

2	Modify the constraints on the presence of PON-nested and non-PON-nested SEI messages.

(Related to EOI SEI messages)

3	Change the constraint on eoi_orig_pic_width_minus1 and eoi_orig_pic_height_minus1 values.

(Related to SPTI SEI messages)

4	On signalling of the spti_max_sublayers_minus1 value

–	Option 1: Add the constraint on the upper limit of spti_max_sublayers_minus1 in HEVC / VVC interface document.

–	Option 2: In addtion to the changes of Option 1, change the descriptor of spti_max_sublayers_minus1 from u(3) to ue(v).

In the v2 of this document, the title of clause 2 was modified.

For item 1, a question was raised of where in the text the proposed language would go.

It is noted that PRI SEI has a CVS scope while SPO SEI has a CLVS scope, which may require some change.

It was suggested that the language be modified.

No action on item 2.

### NNPF SEI extensions (5)

Contributions in this area were discussed during 1540–1800 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

General discussion 1910 – 1920 on Tuesday 7 Oct 2025 (chaired by J. Boyce).

Agreed to align the HEVC NNPF interface text with VVC, by integrating adopted VVC NNPF interface text changes from the JVET-AK, JVET-AL, and JVET-AM meetings.

JVET-AN0058 AHG9: On the NNPF SEI messages in VSEI version 4 [M. M. Hannuksela, F. Cricri (Nokia)]

Chaired by S. Deshpande on 04 October 2025 from 17:25-17:50.

The following changes are proposed to neural-network post-filter SEI messages in VSEI version 4:

- It is proposed to make the presence of the u(v)-coded syntax element nnpfc_reserved_metadata_extension conditional so that its length is greater than 0.

It is asserted that u(0) is not defined.

Decision: Agreed

- Temporally extrapolated pictures are handled similarly to temporally interpolated pictures in most places of the NNPF SEI message semantics. It is asserted that in two instances, the NNPFC SEI message semantics discusses only interpolated pictures, when the semantics should refer to both interpolated pictures and temporally extrapolated pictures. These two instances are proposed to be fixed to refer to both interpolated pictures and temporally extrapolated pictures.

Decision: Agreed

- It is asserted that nnpfa_prompt_update_flag is allowed to be equal to 1 in an NNPFA SEI message even when text prompt auxiliary input is not enabled in the respective NNPFC SEI message. However, nnpfa_seed_update_flag is disallowed to be equal to 1 in an NNPFA SEI message when seed auxiliary input is not enabled in the respective NNPFC SEI message. It is proposed to require nnpfa_prompt_update_flag to be equal to 0, when text prompt auxiliary input is not in use.

Decision: Agreed

- When an NNPFA SEI message is not included in a PON SEI message, it is proposed that nnpfa_num_input_pic_shift shall be equal to 0 rather than shall not be present. It is remarked that when nnpfa_num_input_pic_shift is not present, it is inferred to be equal to 0.

It was suggested to instead add a gating flag for nnpfa_num_input_pic_shift and code the syntax element with a minus1.

It was commented that nnpfa_num_pic_shift syntax element was added at the end for supporting the case where it may not be signalled. But the proposal changes that usage.

Decision: Agreed to add a gating flag before nnpfa_num_input_pic_shift and to code the syntax element as nnpfa_num_input_pic_shift_minus1.

- There exists one instance of nnpfc_extrapolated_pics, which should be corrected to nnpfc_extrapolated_pics_minus1.

Delegated to the editors.

- Editorial suggestions.

Delegated to the editors.

JVET-AN0063 AHG9: On the NNPF SEI messages in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes the following bug fixes and constraints on the NNPFC and NNPFA SEI messages in VSEI v4.

- Constrain NNPFC metadata extension syntax elements, nnpfc_scan_type_idc, nnpfc_for_human_viewing_idc and nnpfc_for_machine_analysis_idc, shall be the same as the base NNPFC.

- Bug fix on NNPFC and NNPFA SEI messages order constraint

- Editorial change on nnpfa_output_flag[ i ] semantic

It was suggested by a participant that it is desirable to allow nnpfc_for_human_viewing_idc and nnpfc_for_machine_analysis_idc to differ for updates and for the base.

No action on item 1.

Agreed on item 2.

On item 3, agreed on moving of the constraint, but not to change the inference value.

JVET-AN0064 AHG9: On the resampling constraint in VSEI v4 [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0064 also relates to the PRI SEI message

JVET-AM0094 proposed to constrain the resampling display overlay ratio to be in the range of 1/16 to 16, and it was concluded such constraint is not needed. This contribution proposes to remove the similar resampling constraint on NNPFC specified in VSEI v3 and PRI SEI messages specified in VSEI v4.

It was noted that making this change could have backwards compatibility issues with VSEI v3. No action for NNPF.

No action for PRI.

JVET-AN0140 AHG9: On the neural-network post-filter characteristics SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the neural-network post-filter characteristics (NNPFC) SEI message:

- Change the condition for the inference for nnpfc_matrix_coeffs.

- Change the inference for VUI-related syntax elements.

- Add a constraint for nnpfc_separate_colour_description_present_flag.

- Complete the note on nnpfc_matrix_coeffs.

- Add a note to notify decoders the need to keep multiple NNPFC SEI messages.

- Add a flag to tell whether NNPFC SEI message reordering exists or not.

- Various editorial NNPFC-related changes.

For item 1, it isn’t clear if a change is necessary.

Discussed item 2 on October 6th from 09:30-09:45 as per the alternative text in v2 of the document, chaired by S. Deshpande. Agreed.

Item 3 is not necessary.

Agreed to item 4.

Regarding items 5 and 6, it is noted that the NNC signalling of a NN allows updates of just indicate deltas in the network.

No action on items 5 and 6.

Delegate editorial changes to the editors.

JVET-AN0342 AHG9: On the neural-network post-filter activation SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance) [late]

This contribution proposes (option 1) adding a NOTE on a case when the NNPFC persistence scope may be smaller than that of the associated NNPFA or (option 2) changing the semantics of nnpfa_persistence_flag to simply the case.

The proposed option 2 may cause a backwards compatibility issue with version 3.

Further discussion was requested, and conducted on 6 Oct 2025.

Agreed to add the note in Option 1.

### Encoder optimization information SEI message (0+1)

Contributions in this area were discussed during 1800–1815 on Saturday 4 Oct. 2025 (chaired by J. Boyce).

JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 also relates to the SPO, PON, and SPTI SEI messages

(Related to EOI SEI messages)

3	Change the constraint on eoi_orig_pic_width_minus1 and eoi_orig_pic_height_minus1 values.

It was noted that the picture input to an encoder has the cropped picture width and height. It was suggested to not change the constraint to apply to coded picture width and height.

The contribution also proposes derivation of the CroppedWidth and CroppedHeight for the AVC interface which was missing.

Agreed to add the derivation of CroppedWidth and CroppedHeight for the AVC interface.

### Object mask information SEI message (3)

Contributions in this area were discussed during 1815–2000 on Saturday 4 Oct. 2025 and during 0945-1015 on Sunday 5 Oct 2025 (chaired by J. Boyce).

JVET-AN0060 AHG9: On the OMI SEI message in VSEI version 4 [M. M. Hannuksela, J. Boyce (Nokia)]

Chaired by S. Deshpande on 05 October 2025 from 09:45-10:00.

The following asserted bug fixes are proposed to the object mask information (OMI) SEI message:

- In three instances, the for loop end condition is proposed to be fixed to use less than or equal to, i.e., "i <= omi_num_aux_pic_layer_minus1".

Already handled during the discussion JVET-AN0135, JVET-AN0161.

- It is proposed that the derivation of OmiAuxSampleRangeDeltaMax[ i ] and the constraint related to consecutive tolerance value ranges are changed so that omi_aux_sample_value[ i ][ j ] + OmiAuxSampleRangeDeltaMax[ i ] indicates the greatest luma sample value that belongs to the mask area.

It is asserted that the range of exactly 1 sample value cannot be specified due to the calculation equations. Also it is asserted that the ranges can currently overlap.

It was commented that this was a mistake added during editing.

Further editorial simplification of the equation is possible and delegated to the editors.

Decision: Agreed.

- It is asserted that a requirement for omi_mask_id_new_object_flag[ i ][ j ] to be equal to 1 could be understood to refer to decoding order, since it uses "is parsed for the first time" in its phrasing. The requirement for omi_mask_id_new_object_flag[ i ][ j ] to be equal to 1 is proposed to be phrased to refer to output order.

This is related to item 11 in JVET-AN0135.

It was commented that this approach and item 11 in JVET-AN0135 resolve the same issue.

Based on a non-proponent support, the language from item 11 of JVET-AN0135 is agreed (possibly further improved by the editors).

- It is proposed to be clarified that omi_mask_depth[ i ][ j ] less than omi_mask_depth[ m ][ n ] indicates that the distance to the the j-th object of the i-th object mask auxiliary layer is less than the distance to the the n-th object of the m-th object mask auxiliary layer.

Decision: Agreed.

- It is proposed to add: When omi_mask_depth[ i ][ j ] is not present, the object depth associated with the j-th object mask of the i-th object mask auxiliary layer is unknown or unspecified.

It was commented that there is an existing constraint that make sure the asserted problem never happensIn addition to the changes proposed above, the accompanying specification text document proposes other changes that are asserted to be editorial.

Delegated these to the editors.

JVET-AN0135 AHG9: On the object mask information SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

- Fix a for( ) loop error in the syntax table.

- Change the constraint for object_mask_info( ).

- Change the signalling position of omi_tolerance_present_flag in the syntax table.

- Change the condition to signal omi_aux_sample_tolerance.

- Switch the positions of omi_mask_top and omi_mask_left.

- Clarify the PU association of an OMI SEI message.

- Clarify the semantics of omi_mask_label_language.

- Change the semantics for the omi_mask_pic_update_flag.

- Change the range for omi_num_mask_in_pic.

- Change the range specification for the omi_aux_sample_value constraint.

- Clarify the order for the omi_mask_id_new_object_flag constraint.

- Fix a for( ) loop error in the MaskId derivation process.

- Specify the processing order to derive MaskId.

- Change the condition of the same MaskId.

- Fix some typos about SubHeightC.

- Change the confidence derivation equation.

- Change the omi_depth semantics.

- Various other editorial changes.

Agreed on item 1, 2.

On item 3, it was suggested to not move the omi_tolerance_present_flag but to add it to the constraint to be same for all SEI messages in a CLVS.

Agreed to above suggestion for item 3.

It was questioned if the current inference for omi_tolerance_present_flag is appropriate if this new condition is added.

Agreed on item 4.

For item 5, don’t change the order but fix the semantics. It was noted that other SEI messages that use top/left should be checked, which include annotated regions and region-wise packing.

Delegate to editors the semantics fix.

Item 6: It was suggested to modify the proposed language to: in a PU in a primary layer

Delegate items 6, 7, and 8 to editors.

Agreed to item 9, 10.

JVET-AN0060 item 3 is related to item 11. item 11 was discussed when JVET-AN0060 is presented. See notes under item 3 of JVET-AN0060.

Agreed to item 12.

Regarding item 13, an additional suggestion is to add a constraint on the value of omi_mask_pic_update_flag[ ] to require it to 1 for the first OMI SEI in a CLVS in output order.

Some description of the processing order is seen to be helpful, but it should be based on output order rather than decoding order.

Agreed to add a description of the processing to follow output order and to add a constraint on the value of omi_mask_pic_update_flag[ ] as described above. Exact language delegated to editors.

Regarding item 14, an alternate suggestion is to add a bullet with an additional condition:

omi_persistence_flag of omiA is equal to 1 or omi_persistence_flag of omiB is equal to 0

Discussed item 14 again on October 6 from 19:55-20:00 chaired by S. Deshpande. A note is suggested to be added and other spec text changes made for omi_persistenc_flag. Agreed with version 2 for item 17.Further discussion requested item 14.

Delegate items 15 to editors.

Regarding item 16, the current design does not allow signalling a confidence of 1, just near to it. For 8-bit numbers, the contribution would divide by 255 instead by 256, allowing a value of 1.

It was noted that the proposed equation needs another parenthesis.

No action on item 16.

Regarding item 17, an alternate suggestion is to add a note clarifying what the signalled value of depth means.

Further discussed requested for item 17 to review proposed language for a note.

The attachment includes proposed text of a note regarding semantics of omi_persistence_flag. No action on the note.

Delegate editorial modifications in item 18 to the editors.

Further discussed 5 Oct 2025.

The following language for a note was provided:

NOTE: Values of omi_mask_depth[ i ][ j ] represent a depth order between objects and do not have to be proportional to physical distance of objects.

Agreed to add the note for item 17.

Further discussion on 6 Oct 2025.

JVET-AN0161 AHG9: On OMI SEI message [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to OMI SEI messages in v4 draft. The proposed modifications are as follows:

  - On the loop over auxiliary layers.

  - On constraint for omi_mask_sample_value_length_minus8 in VVC interface text.

  - On constraint for auxiliary sample values and tolerance values.

  - On constraint for value range of auxiliary sample values in each auxiliary layer.

  - On the number of bits for signalling tolerance values.

Item 1 agreed, see notes for JVET-AN0135 and JVET-AN0060.

Item 3 agreed, see notes for JVET-AN0060.

Agreed to Item 2.

For item 4, it is suggested that this constraint would reduce flexibility to assign the highest value allowed by the bit depth. No action.

Item 5 is for bitrate savings. It is considered late for v4 to make this change. No action.

### AI usage restrictions SEI message (2)

Contributions in this area were discussed during 0900–0945 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0062 AHG9: On the AI usage restrictions SEI message in VSEI version 4 [M. M. Hannuksela, F. Cricri, K. Kammachi Sreedhar (Nokia)]

Chaired by S. Deshpande on 05 October 2025 from 09:25-09:45.

This document proposes the following changes to the AI usage restrictions SEI message:

- It is asserted that it would be better to indicate whether restrictions apply to generative AI only separately from the category of AI application indicated by aur_restriction[ i ]. Therefore, the following changes are proposed:

  - Addition of aur_generative_flag[ i ]. When equal to 1, it specifies that the AI application category defined by aur_restriction[ i ] is constrained to include generative AI only. aur_generative_flag[ i ] equal to 0 does not constrain the AI application category defined by aur_restriction[ i ].

  - Removal of "unusable for generative AI" entry from the values of aur_restriction[ i ].

  - Removal of the constraint "For any i and j values with i not equal to j, aur_restriction[ i ] shall not be equal to aur_restriction[ j ]", because entries i and j can have the same value of aur_restriction and different values of aur_generative_flag, aur_context, and aur_sei_exclusion_flag.

It is asserted that unusable for generative AI overlaps with unusable for AI training and unusable for AI inference.

It was commented that the genesis of the current table entries is motivated by regulatory reason.

Generative AI cases discussed:

-Not allowed to modify the content but you can train on it.

-You can’t train with the content but you can change the content.

It was asked if more entries can be added to the table (although entries are ue(v) coded) instead of adding a new flag.

It was commented that adding a new flag at this late stage should be considered very carefully as now the aur_restriction[i] needs to be interpreted together with a new flag.

It was suggested and agreed to move forward by adding more entries to the aur_restriction[i] table instead of adding a new flag.

Discussed the text for the exact table entries wording based on V3 of the document on October 6 from 19:30-19:45 chaired by S. Deshpande.

It was commented that with new entries added to the aur_restriction[i] table, the allowed range for aur_num_restrictions_minus1 should be correspondingly updated.

Agreed as per V3 of the document.

- Some options are proposed to editorially rephrase the introduction paragraph of the AI usage restriction SEI message to clarify that it does not impose a conformance requirement for decoding systems.

Delegated to the editors, the option A was preferred by proponents and non-proponents.

Further discussed on 11 Oct 2025, as related to the ballot comment US-035.

It was suggested to replace in the semantics “unusable” with “should not be used”.

The following note was previously proposed:

NOTE 1 – Even though receiving systems are expected to obey the indications provided by the AI usage restrictions SEI message, no conformance requirements for receiving systems are imposed by this document for the interpretation of the AI usage restrictions SEI message.

Alternate wording was suggested:

NOTE 1 – Although receiving systems are requested to obey the indications provided by the AI usage restrictions SEI message, no conformance requirements for receiving systems are imposed by this document for the interpretation of the AI usage restrictions SEI message.

It was also suggested to rename the SEI message the AU usage restrictions request SEI.

Agreed to modify the AUR SEI as indicated above.

Additional editorial changes are proposed, which includes converting note 2 about processing order to body text.

JVET-AN0136 AHG9: On the AI usage restrictions SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance), A. Hinds (Tencent)]

This contribution proposes the following changes for the AI usage restrictions (AUR) SEI message:

- Change the range constraint for aur_num_restrictions_minus1.

- Change the value 0 restriction for aur_context[ i ].

- Modify the semantics of aur_sei_exclusion_flag[ i ] to restrict AI processes.

In -v2 of this document, an alternative text change for item 3 is described.

For item 1, it was suggested to also add language about the decoder for future compatibility: Decoders conforming to this version of this document shall allow any value of aur_num_restrictions_minus1 in the range of 0 to 255, inclusive.

Agreed for item 1, with additional language about decoder, as described above.

Agreed for item 2.

Agreed for item 3, with the exact language delegated to the editors.

### Digitally signed content SEI messages (1)

Contributions in this area were discussed during 1015–1200 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0182 AHG9: On DSC SEI messages in VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE), K. Sühring, Y. Sanchez (Fraunhofer HHI), M. M. Hannuksela (Nokia)]

This contribution proposes modifications that are asserted to provide improvement to DSC SEI messages in v4 draft. The proposed modifications are as follows:

1	On DSCI SEI messages when SEI signing is enabled.

–	Option 1: disallow any DSCI SEI messages to be signed (by any verification system).

–	Option 2: disallow the DSCI SEI message to be signed by its own verification system.

2	On DSCV SEI messages when SEI signing is enabled.

–	Option 1: disallow any DSCV SEI messages to be signed (by any verification system).

–	Option 2: disallow the DSCV SEI message to be signed by its own verification system.

3	On signing SEI NAL unit containing only DSCS SEI messages.

4	On DSCS SEI messages with substream Id 0 that do not precede all verification inclusion NAL units.

5	On DSCS SEI messages with substream Id not equal to 0 that do not precede all verification inclusion NAL units

6	On the presence of more than one DSCS SEI message when implicit mode and SEI signing are enabled.

7	On the presence of DSCS SEI messages in a PU SEI signing is enabled.

8	On the presence of DSCS SEI message that is not the last verification inclusion NAL unit in a PU when implicit association mode and SEI signing are enabled

In the v2 of this document, option 2 of item 2 is updated to clarify the intension. In addition, some typos are fixed.

For item 1 Option 1, it was suggested to clarify what an associated DSCS SEI message is, e.g. has the same value of dsci_id.

Agreed for Item 1, option 1, with exact language delegated to the editors.

Agreed for Item 2 Option 1.

For item 3, it was suggested that the proposal adds some complexity to the verifier. No action.

Further discussion requested for item 4.

For item 5, it was suggested that the existing language is sufficient. It was suggested that a note may be helpful.

Further discussion requested for item 5.

For item 6, it was suggested that if this bullet were removed, a DSCS SEI message in a suffix SEI NAL unit could only be used for the implicitly associated substream ID. It may be possible rephrase the bullet rather than to remove it.

Discussed item 6 again after offline discussion. A note is added. Agreed as per document v3 (in v4 zip file) to be uploaded.

For item 7, alternative wording was suggested.

Discussed item 7 again after offline discussion. Agreed with the “after offline discussion” text. Agreed as per document v3 (in v4 zip file) to be uploaded.

Agreed on Item 8.

Discussion on October 6 from 19:45-19:55 was chaired by S. Deshpande.

Further discussion 6 Oct 2025 regarding items 4, 5, 8.

A note was added to describe the recursion if an SEI message is not the first one with a particular ID value.

An additional bullet of conditions was added so that the different inference methods are separated.

A version -v3 of the contribution to be uploaded to reflect the above in a new item 9.

Agreed.

### Packed regions information SEI message (4)

Contributions in this area were discussed during 1200–1300 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0180 AHG9: On PRI SEI message in processing chain [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes allowing a PRI SEI message to specify a property or process in the processing chain.

The summary of the proposed modifications are as follows:

–	Introduce a list of payloadTypes ‘currSpoProcessSeiList’ for each processing chain.

–	‘currSpoProcessSeiList’ includes the PRI SEI type when the PRI SEI message is used as a process, and other SEI types that are used in the processing chain.

–	‘SpoProcessSeiList’ in current text are changed to ‘currSpoProcessSeiList’.

In the v2 of this document, the proposed modifications are changed for simplification.

It was noted that because the PRI type is included in both property and process lists, does not mean that it cannot be used to indicate property.

It was noted that with this proposal, special handling of the processing chain would be required for PRI SEI.

Further discussed on 10 Oct. 2025.

v3 to be uploaded.

Agreed on option 2 in -v3 contribution which removes a constraint on the value of pri_target_pic_params_present_flag, and adds a note.

JVET-AN0183 AHG9: On PRI SEI message in VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to PRI SEI messages in v4 draft. The proposed modifications are as follows:

1	On constraint for PRI SEI message when present in PON SEI message.

2	On repetition of PRI SEI message in an AU and in a PU.

3	On the presence of PRI SEI message when its regions refer to multiple layers.

4	On layers included in PRI SEI message and OLS.

JVET-AN0065 item 3 relates to removing the constraints.

Based on JVET-AN0065, no action on item 3.

For item 2, it was suggested that for a single layer PRI, there doesn’t need to be a constraint, because each layer with a PRI SEI could different contents. For single layer PRI, there is an inference of the layer ID.

It was suggested that repetition of PRI SEI messages should be allowed.

It was suggested there is an issue with the current text because the PRI SEI applies to an AU.

Some support was expressed for item 4.

Further discussion requested for items 2 and 4.

Further discussion on 6 Oct. 6, 2025.

An example was discussed with having the PRI SEI apply to multiple layers but not to be at the AU level. This general approach was endorsed. Specific language to be provided for this approach.

Further discussed 10 Oct. 2025.

See notes in JVET-AN0367 for item 2.

Agreed for item 4, to add a constraint on unique region values in HEVC and VVC.

JVET-AN0310 AHG9: On PRI ID [Y. Li (SJTU), Y.-K. Wang (Bytedance), K. Yang, Y. Xu (SJTU)] [late]

This contribution proposes to add an ID to the PRI SEI message, such that multiple PRI SEI messages can be associated with an AU for different purposes, e.g., for support of different target pictures. Two options are provided, the first targeting VSEI v4, and the second targeting VSEI TuC.

A suggested use case is that some users would be interested in some player but not others.

It was noted that display rectangles SEI in the TuC also addresses the use case described above.

JVET-AN0367 AHG9: On design of PRI SEI message [H. Tan, J. Lee (LGE), Y.-K Wang (Bytedance), J. Boyce (Nokia)] [late]

Chaired by S. Deshpande on 9 October 2025.

This contribution proposes modifications to clarify the design of Packed Regions Information (PRI) SEI message. The following changes are proposed:

1. Specify a constraint such that if there are more than one PRI SEI message with a particular value of pri_id in an AU, both the following apply:

a. PRI SEI messages in the same PU shall have the same content.

b. When two PRI SEI messages are present in different PUs withing the same AU, their list of associated layers shall be mutually exclusive.

2. Specify a constraint such that PRI SEI shall be present in PU belong to the lowest layer from the layers which contains regions in the SEI.

3. Specify a constraint such that for two PRI SEI messages, priSeiA and priSeiB, in different AUs in the CVS, if they have the same list of associated layers then the latter cancel the former. Otherwise, if they have some intersection of list of associated layers then there must be PRI SEI message with cancel flag equal to 1 in the layer where priSeiA is present and is present in between priSeiA and priSeiB.

It was asked what if priSeiA message had persistence flag equal to 0. In this case another cancel would not be needed. It was confirmed that text should be clarified to apply only to persistence flag equal to 1.

A comment was made that some additional edits like this related to cancel flag and persistence flag values may (or may not) be needed.

4. Regarding persistence, pri_persistence_flag equal to 1 specifies that the packed regions information SEI message in the current CLVS applies to pictures in the the current AU and persists for pictures in all subsequent AUs in output order in the current CVS until an AU, when present, that follows the current AU in output order and contains a packed regions information SEI message in the same CLVS.

Text implementation of the proposed aspects above is available in the attachment of this contribution.

This proposal applies to v4 and is asserted to be necessary for correct behaviour.

Some typos in the proposed text were fixed.

Some editorial wording is needed for SPO related text that is proposed. After further discussion SPO relatex text change is not needed.

A version 2 of the proposed text changes with the modifications describe above will be uploaded. Agreed.

### Source picture timing information SEI message (0+1)

Contributions in this area were discussed during 1545–1510 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0184 also relates to the SPO. PON, and EOI SEI messages

4	On signalling of the spti_max_sublayers_minus1 value

–	Option 1: Add the constraint on the upper limit of spti_max_sublayers_minus1 in HEVC / VVC interface document.

–	Option 2: In addtion to the changes of Option 1, change the descriptor of spti_max_sublayers_minus1 from u(3) to ue(v).

It was noted that most bitstreams won’t use all of the allowable temporal sublayers, so this proposal doesn’t guarantee that the info signaled in the SPTI all is useful.

Agreed for item 4 option 1.

### Modality information SEI message (2)

Contributions in this area were discussed during 1510–1600 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0073 AHG9: On MI SEI message in VSEI version 4 [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

Examples using the MI syntax for various wavelength values are presented.  It is asserted the current syntax has multiple representations of a single value wavelength and the precision is insufficient.  Several proposals are described to address these limits and harmonize with other floating point uses.

1)	(Editorial) Use units of nanometres rather than metres for these wavelength values.

2)	(Technical) Reverse order of mantissa and exponent syntax elements for consistency with other floating point uses i.e. Multiview acquisition information, 3D reference displays information, Depth representation information element, etc.

3)	(Technical) Reduce the number of bits for the exponents while keeping the range from IR to UV, 1nm to 1mm = 106 nm.

4)	(Technical) Eliminating the need for minus 15 in exponent elements, rename syntax names and modify semantics to support this.

5)	(Technical) Increase the number of bits allocated for mantissa for greater precision.

6)	(Technical) Modify semantics for computing reconstructed floating point values using other changes to be unique, increase precision, and consistent with the other floating point representation formats.

7)	(Editorial) Introduce an associate table between floating-point values and syntax elements to be consistent with other floating-point use in SEI messages.

Agreed for item 2, reversing order of syntax elements for mantissa and exponent.

Regarding item 1,  ISO 20473-2007 uses meters in its specification. It specifies from 1 nm to 10^6 nm, which is 1 mm.

It is noted that the exponent and mantissa are base 10.

JVET-AN0138 AHG9: On the modality information SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the modality information (MI) SEI message:

- Remove inference for mi_modality_type.

- Specify range constraints for wavelength values according to mi_modality_type.

- Change the coding lengths for wavelength mantissa and exponent.

Inference not needed, since mi_modality_type is always present, except for when cancel flag is set.

Agreed to item 1.

It is noted that the proposed constraint would be problematic for content that has wavelengths that span more than one range. Discussed  item 2 on October 6th.

It is preferred to keep the syntax stable at this stage.

It was suggested to modify the wording in the semantics about optical radiation to indicate more flexibility.

Discussed on October 6th from 09:20-09:30 to consider proposed modified semantics wording from V2 of the document Chaired by S. Deshpande.

Agreed item 2 and rewording as per V2.

### Text description information SEI message (2)

Contributions in this area were discussed during 1600–1615 on Sunday 5 Oct. 2025 (chaired by XXX).

JVET-AN0061 AHG9: On the text description information SEI message in VSEI version 4 [M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande on 05 October 2025 from 04:00-04:07

The following changes are proposed for the text description information (TDI) SEI message:

- It is asserted that the syntax table accidentally uses gating by tdi_cancel_by_purpose_flag, which does not exist in the syntax. Instead, the gating is proposed to be based on tdi_purpose_cancel_flag.

Editorial fix agreed.

- It is asserted that the impact of tdi_purpose_cancel_flag in a subsequent TDI SEI message in output order is not specified for the persistence of the current the TDI SEI message. It is proposed to add a bullet that the persistence of the current TDI SEI message ends when a picture in the current layer associated with a TDI SEI message with the same value of tdi_descr_purpose and tdi_purpose_cancel_flag equal to 1 is output that follows the current picture in output order.

Reviewed together with item 1 in JVET-AN0139.

Agreed.

JVET-AN0139 AHG9: On the text description information SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

Chaired by S. Deshpande on 05 October 2025 from 04:07-04:12

This contribution proposes the following changes for the text description information (TDI) SEI message:

- Change the persistence scope to include the case of tdi_id_cancel_flag being equal to 1.

This item was discussed together with item 2 of JVET-AN0061. See notes above.

- Change the name of tdi_persistence_flag to tdi_id_persistence_flag.

No change, as the persistence has id and purpose.

### Generative face video SEI messages (2)

Contributions in this area were discussed during 1615–1710 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0141 AHG9: On the generative face video SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the generative face video (GFV) SEI message.

- Specify a range costraint for gfv_chroma_key_purpose_idc.

- Require gfv_base_pic_flag equal to 1 when a GFV SEI message is the first GFV SEI message in output order.

- Various editorial changes.

Related to item 1, it is suggested to add a row in the table to indicate values 3 .. 25 are reserved values.

Agreed to item 1, delegate the editors if the table should be updated.

Agreed to item 2.

Delegate the editorial fixes to the editors.

JVET-AN0156 AHG9: Text fixes and cleanup for GFV and GFE SEI messages [J. Chen, Y. Ye, B. Chen (Alibaba)]

JVET-AN0156 also relates to editorial aspects for JVET-AM2006

This contribution proposes bug fixes, text cleanup and general editorial improvements to the current text of generative face video (GFV) and generative face enhancement (GFE) SEI messages. The proposed text was implemented on top of JVET-AM2006 and attached to this contribution.

The proposed changes include:

- Add missing inference to gfe_matrix_element_sign_flag[ i ][ j ][ k ] and gfe_matrix_delta_element_sign_flag[ i ][ j ][ k ];

- Remove two conditions asserted to be redundant in the GFV SEI message;

- Suggest to rename “generative face enhancement (GFE)” SEI message to “generative enhancement face video (GEFV)” SEI message;

- Text cleanups and editorial improvements;

- Text bug and typo fixes.

Agreed to items 1, 2, 5.

Delegate to editors item 3, 4.

### Image format metadata SEI message (1)

Contributions in this area were discussed during 1710–1720 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0137 AHG9: On the image format metadata SEI message in VSEI v4 [J. Xu, Y.-K. Wang (Bytedance), A. Hinds (Tencent)]

This contribution proposes the following changes for the image format metadata (IFM) SEI message:

- Change the range specification for ifm_num_metadata_payloads_minus1

- Specify a range constraint for ifm_type_id[ i ].

- Change the range specification for ifm_payload_len_minus1[ i ].

- Specify a constraint on the value of ifm_payload_len_minus1 when ifm_type_id[ i ] is equal to 5.

- Change the condition of ICCmajorVer and ICCminorVer derivation.

In -v2 of this document, an alternative text change for item 4 is described.

Agreed items 1, 2, 3,

Agreed item 4, with the first wording.

Agreed item 5.

## AHG9: Aspects on new and extended SEI messages in TuC for VSEI (55)

### Scalability dimension information SEI message (2)

Contributions in this area were discussed during 1810–1900 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the FGRC, GRI, and PSI SEI messages

This proposal introduces SDI SEI message dependency constraints for the SEI messages listed in the VSEI TuC that currently lack explicit dependency signalling:

- GRI SEI messages

- PSI SEI messages

- ASAI SEI messages

- FGRC SEI messages

Agreed for item 1, 2.

Further discussion requested for item 3.

For item 4, it was suggested to reword the first constraint to consider an inference value of . fgr_alpha_channel_adapation_flag equal when not present.

It was suggested to reword the second constraint : when the CVS does not contain an SEI with aux_type equal to 1 for at least one value of i, the adaptation flag shall be equal to 0.

Further discussion requested item 4.

Further discussion on 11 Oct 2025.

A constraint is proposed as follows:

When an ASAI SEI message is present and it indicates various parameters for auxiliary pictures of type AUX_DEPTH, the following applies:

- When a CVS does not contain an SDI SEI message with sdi_aux_id[i] greater than 0 equal to 2 for at least one value of i, no picture in the CVS shall be associated with an ASAI SEI message.

- When an AU contains both an SDI SEI message with sdi_aux_id[i] greater than 0 equal to 2 for at least one value of i and an ASAI SEI message, the SDI SEI message shall precede the ASAI SEI message in decoding order.

The language “it indicates various parameters for auxiliary pictures of type AUX_DEPTH” is too imprecise to be in a constraint. A constraint needs to be more precise and refer to syntax elements, variables, or presence of syntax elements or syntax structures.

JVET-AN0265 AHG9: On confidence auxiliary layer design [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

In this contribution, it is asserted that the current SDI SEI message does not support hierarchical reference chains between confidence and depth layers based on the changes in the VSEI TuC document. This contribution proposes to modify the syntax to enable a confidence auxiliary layer to reference an auxiliary depth layer as well as precising the interpretation of the confidence image sample values.

It was noted that the SDI extension design in the TuC includes a described layer idx value to indicate an auxiliary layer.

The proposal first item proposes renaming and modifying the semantics of sdi_associated_primary_layer_idx[ ][ ] to allow it to reference an auxiliary layer. A concern was raised about backwards compatibility. It was suggested that it might be possible to have wording that addresses backwards compatibility of bitstreams and decoders.

If there was more than one associated primary layer, there could be ambiguity as to which layer was the described layer of the confidence info.

A suggestion was made the wording of the interpretation of the sample value should just describe the min and max values as 0 and 2^n – 1, and not try to calculate a number.

Additional discussion chaired by S. Deshpande.

For item 1, there is non-proponent support, however the exact language including the backwards compatibility aspects need to be specified. Further discussion requested..

For item 2, the wording of the interpretation of the sample values was changed to apply to all aux type values and clarifies that the 2^bitdepth– 1 represents the maximum value.

Agreed to item 2.

Further discussed 10 Oct 2025, chaired by J. Boyce.

The version v2 implements the following modifications after a fist presentation in session:

- The relaxation of the syntax element is limited to the new types of auxiliary layers (types 4 and 5).

- Further text fixes for updated syntax element names.

- The interpretation of the sample values of a confidence image is expressed in terms of least and most confidence.

- The interpretation of the sample values of a confidence image is expressed for all types of associated auxiliary layers and not specifically for auxiliary depth layers.

Agreed to the changes proposed in the section 4.2 of the v2 version.

### Shutter interval information SEI message (1)

Contributions in this area were discussed during 0725–0730 on Sunday 5 Oct. 2025 (chaired by J. Boyce).

JVET-AN0145 AHG9: On the shutter interval information extension in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes to add a gated flag for the newly introduced syntax elements in the extension to the shutter interval information (SII) SEI message.

Agreed.

### NNPF SEI messages (3)

Contributions in this area were discussed during 1930–2000 on Sunday 5 Oct. 2025 and 1030-1120 on Monday 6 Oct 2025 (chaired by J. Boyce).

JVET-AN0068 AHG9: On Neural-network Post-filter Information Signalling and Semantics [S. Deshpande (Sharp)]

Following modifications are proposed related to NNPF signalling and semantics in VSEI Technologies under Consideration (TuC) document:

- Proposal 1: It is asserted that there are issues in Neural-network post-filter characteristics SEI in VSEI TuC related to the signalling of text prompt for temporal extrapolation (nnpfc_prompt[ i ]) and its semantics. Two alternative options are proposed.

  - Option 1: It is proposed to bug-fix the asserted issues and to separate the signalling of the text prompt information for temporal extrapolation purpose compared with the signalling of the text prompt information for spatial extrapolation purpose which is already in VSEI V4.

  - Option 2: Bug-fixes are proposed for the asserted issues related to the text prompt signalling for temporal extrapolation purpose.

- Proposal 2: In Neural-network post-filter characteristics SEI message an asserted bug-fix is proposed for the semantics of nnpfc_mul_intefaces_flag equal to 1, to be consistent with the derivation of NNPfInferenceProcess.

- Proposal 3: In Neural-network post-filter activation SEI message it is proposed to update a constraint as follows: When nnpfc_inband_prompt_flag is equal to 0 in the NNPFC SEI message with nnpfc_id equal to nnpfa_target_id, the value of nnpfa_prompt_update_flag[ i ], if present, shall be equal to 0. This also applies to V4.

Option 1 proposes more changes than option 2.  Option 1 allows separation of temporal extrapolation spatial extrapolation. It was suggested that a single prompt could be worded to specifying both an instruction for both temporal extrapolation and for spatial extrapolation.

It was suggested to modify the option 2 loop to reference nnpfc_num_alt_instances_minus1 rather than the variable nnpfNumInferences.

Agreed for proposal 1 option 2, with the modification to loop described above.

Agreed for proposal 2.Regarding proposal 3, it was suggested that fix also applies to VSEI v4, and was considered to be the original design intent.

Agreed for proposal 3 for TuC and for VSEI v4.

JVET-AN0176 AHG9: On multi-purpose NNPFs [C.-H. Demarty, E. François, A. Ak (InterDigital)]

This contribution builds upon contribution JVET-AM-0173 on multi-purpose NNPFs, which proposed to avoid the need to define multiple NNPFs and NNPF cascades, by enabling the adaptive use of NNPF models that include several purposes. Such a functionality was adopted in the TuC during last meeting.

One advantage of using multi-purpose NNPFs is that a unique NNPF structure can be used for all combinations of purposes and that some NNPF’s parameters remain identical and do not need to be redefined from one purpose to another.

This contribution therefore proposes to allow not to re-send information regarding the NNPF structure, nor to re-send parameter values that do not need to be updated upon an update of the NNPF, whether it comes from an update of purpose or not.

In proposal 2, many syntax elements are not signalled in some cases. There may need to be some additional semantics language to infer the values of the syntax elements that are not signalled. Special care is needed for syntax elements that are already sometimes not present and already have inference language.

For proposal 1, it was suggested that the proposed use of the nnpf_mode_idc is not usual.

A question was raised about backwards compatibility. What would a prior version decoder do if it received such an SEI message?Proposal 1 would save a significant amount of bits if the NN was sent inband.

Some concern was raised about the semantics language with the new proposed mode is sufficient to describe the usage.

Further discussion requested.

Further discussed at 1810 on Thursday 9 Oct. 2025 chaired by Y.-K. Wang.

The added inference of the skipped syntax elements seems OK.

It was commented that another, bigger concern was on violation of the constraint that the NNPFC purpose for a particular NNPFC ID does not change within a CLVS, and it was further commented that this might imposes a backward compatibility issue.

In the TuC there is already an approach for supporting multi-purpose NNPFs by including NNPFC purpose in the input tensor through a new auxiliary input type. It was argued that in some sense the multi-purpose NNPF support in the TuC is sort of already not aligning with the constraint that the NNPFC purpose for a particular NNPF ID does not change within a CLVS.

It was commented that the multi-purpose support in the TuC, there is no the backward compatibility issue because the signalling was through a new auxiliary input type, for which v4 decoders would ignore the NNPF entirely.

It was commented that the TuC solution can avoid sending NNR of mode idc 1. This proposal can avoid sending NNR of mode idc 0.

For the proposal to work, the NNPFC purpose would need to be updated.

Also see notes under JVET-AN0365.

JVET-AN0365 AHG9: On backward compatibility of multi-purpose NNPFs [M. M: Hannuksela, F. Cricri (Nokia), H. Tan (LGE)] [late]

It is asserted that JVET-AN0176 violates the following constraint in VSEI v3 and v4:

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

This contribution proposes the following options to overcome the asserted problem:

- When nnpfc_mode_idc is equal to 2, signal nnpfc_updated_purpose. Derive nnpfcPurpose to be equal to nnpfc_updated_purpose (if present) and equal to nnpfc_purpose (otherwise). In the NNPFC SEI message syntax and semantics, replace nnpfc_purpose with nnpfcPurpose.

- Relax the constraint to apply only when nnpfc_mode_idc is equal to 0 or 1.

- When nnpfc_mode_idc is equal to 2, signal nnpfc_ref_id, which identifies the NNPFC SEI message that defines the neural network information that is reused in this NNPFC SEI message.

Discussed at 1845 on Thursday 9 Oct. 2025 chaired by Y.-K. Wang.

On option 1: It was commented that although the nnfpc_purpose syntax element is not updated, the effective NNPF purpose is updated. It was commented that it is important that the effective NNPF purpose is only determined by the first syntax element in the NNPFC SEI message.

On option 2: It was commented that relaxing the constraint of NNPF purpose not changing within a CLVS to only of modes 0 and 1, but not applicable to the new mode is still somewhat against the design principle we had.

On option 3: It was commented that a similar proposal was the same technical benefits was discussed earlier and no action was taken. It was commented that the old decision was due to at the time there was no good use case, while now some use cases have been brought up.

It was commented that it is important to keep our design principles, and we already have some ways of avoiding sending the NNR bitstream and some other information in the NNPFC SEI message.

Proponents emphasized that option 3 does not have a backward compatibility issue and it enables good bit saving, and it improves the current multi-purpose design in the TuC.

It was commented that the NNPF design is becoming increasingly complicated in the context of the TuC, If this becomes part of a future version of the standard, imaging in the future that implementations of versions 3 or 4 also exist thus end up with implementing both the new version and an old version.

### Constituent rectangles SEI message (3)

Contributions in this area were discussed during 1120–1210 on Monday 6 Oct. 2025 (chaired by J. Boyce).

JVET-AN0102 AHG9: On the CR SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

- Various editorial changes

- Add a condition to avoid signalling cr_rect_id when the rectangle is empty and remove all constraints to avoid referencing an empty constituent rectangle in TuC.

- Two options are proposed to handle the issue that the rectangle ID may be identical across layers.

  - Add syntax element of the layer index of the associated rectangle

  - Update the constraint to ensure the CR ID numbering space is common across layers

Agreed on item 1.

Regarding item 2, the ID is inferred for empty rectangles. It was commented that this change might reduce flexibility. No action on item 2.

For item 3, there is potential interaction with inference across layers.

Further discussion requested for item 3.

Further discussion on 8 Oct 2025.

Agreed on item 3 option A.

JVET-AN0149 AHG9: On the constituent rectangles SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the constituent rectangles (CR) SEI message:

- Specify the persistence scope of the SEI message.

- Avoid signalling a layer of all CR_EMPTY rectangles.

Agreed to item 1, 2.

JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages [K. Abe, T. Nishi (Panasonic)]

JVET-AN0160 also relates to the DOI and DR SEI messages

This contribution proposes improvements to the syntax design of the display rectangles SEI (DR SEI), and corrections of typos to the display overlays information SEI (DOI SEI) and the constituent rectangles SEI (CR SEI) in the VSEI TuC draft of JVET-AM2032. The proposed modifications are as follows:

3. Corrections of typos in CR SEI

- Fix typos in semantics of cr_num_rects_in_layer_minus1.

Item 3 is purely editorial.

Delegate item 3 to editors.

### Display rectangles SEI message (2)

Contributions in this area were discussed during 1210–1300 on Monday 6 Oct. 2025 (chaired by J. Boyce).

JVET-AN0124 AHG9: Additional features in display rectangles SEI message [R. Imichi, Y. Tokumo, T. Ikai (Sharp)]

The display rectangles (DR) SEI message is specified in technologies under consideration for future extensions of VSEI (version 9) in JVET-AM2032.

It is asserted that the current display rectangles (DR) SEI message has three problems:

- There is no functionality to fill the i-th rectangle with a gradient.

- The decoded picture can be centered relative to the i-th display rectangle without additional syntax, but there is no functionality to do that.

- Some editorial improvements should be conducted.

Slide deck to be uploaded in new version.

It was suggested to make the semantics more compact, to use variables to indicate the different directions.

Decision: Agreed to items 1 and 2.

Delegated Item 3 to editors.

JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages [K. Abe, T. Nishi (Panasonic)]

JVET-AN0160 also relates to the CR and DOI SEI messages

This contribution proposes improvements to the syntax design of the display rectangles SEI (DR SEI), and corrections of typos to the display overlays information SEI (DOI SEI) and the constituent rectangles SEI (CR SEI) in the VSEI TuC draft of JVET-AM2032. The proposed modifications are as follows:

- Improvements to the syntax design of DR SEI

- Add an unspecified filling method to solve the issue of needing to signal filling parameters even if dr_fill_method_present_flag is equal to 0.

- Fix typos using an incorrect value 8 in dr_fill_method_idc.

It was suggested to not add a new method_idc value, but to modify the semantics of the dr_fill_method_present_flag so that if not present, the fill method is unspecified.

Should also remove the inference of dr_fill_method_idc[ i ].

It was suggested that it is preferable to have a default method that is defined rather than unspecified.

For item 3, see the notes under JVET-AN0124.

This was discussed again on 10 October 2025, chaired by S. Deshpande.V3 version of the document with additional co-authors was discussed. A new option 2 for item 1 was added.

Decision: Agreed to option 2 from V3 version of the document.

### SEI processing order and processing order nesting SEI messages (1)

Contributions in this area were discussed during 1430–1615 on Monday 6 Oct. 2025 (chaired by J. Boyce).

JVET-AN0108 AHG9: On the SPO SEI message extension complexity information [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes semantic constraints on SPO SEI message extension syntax elements, po_root_complexity_info_present_flag and po_key_processes_complexity_info_present_flag, in TuC.

Agreed to the -v2 version.

### Encoder optimization information SEI message (1)

Contributions in this area were discussed during 1615–1715 on Monday 6 Oct. 2025 (chaired by J. Boyce).

JVET-AN0208 AHG8/AHG9: Depth-aware optimization for Encoder optimization information SEI message [G. Teniou, S. Wenger, A. Hinds, J. Ricard (Tencent)]

This document is a revision of the proposal for a new eoi_type associated to depth-aware encoding optimization. The comments made during the previous meeting are addressed and an updated version of the depth-aware information is proposed together with use case examples to illustrate the added value of the depth-aware encoder optimization information.

It was suggested that the EOI message doesn’t currently have a foreground/background optimization indication, although one of the eoi_object_based_idc value indicates areas outside detected objects have higher QP.

If the depth map is not provided in the bitstream, it is more difficult for the receiver to use this information.

It as suggested to provide a depth threshold to differentiate between foreground and background.

It is noted that source content may have focus at different depths.

It was suggested to include depth information using the eoi_object_based_idc, which is addressed in section 6 of the document.

It is noted that new syntax elements are proposed to be in the middle of the structure, so that the decoder is not able to skip them. The syntax elements would need to be moved to the extension.

It was suggested to add as a eoi_object_based_idc bit mask value, but to not include the syntax elements.

Agreed for the eoi_object_based_id mask approach, subject to review of the text that moves the syntax elements to the extension.

Further discussed 10 Oct. 2025.

The text has been confirmed, except that the if condition in the next syntax elements should be compared to 0x20.

### Digitally signed content messages (6)

Contributions in this area were discussed during 1715–1730 on Monday 6 Oct. 2025 and 1755 – 2040 Tuesday 7 Oct. 2025 and 1400 – 1430 Thursday 9 Oct. 2025 (chaired by J. Boyce).

General discussion.

Design principles for subpicture:

- Each verification substream and the substreams that it is dependent on, if extracted, would be decodable

- It should be possible to verify that an entire picture is complete, whether or not the picture contains subpictures

- Optionally sets of one or more subpictures may also additionally be able to be verified

  - Options:

    - all subpictures in a set signed together

    - separately signal the set association with the subpictures

- Question: Can an individual subpicture be included in more than one set of subpictures?

- Question: Is it necessary to include parameter sets in the signing of the subpictures/sets of subpictures?

Contribution proposes using a scalable nesting SEI message to associate subpictures.

It was noted that it was proposed in JVET-AM0164 to use scalable nesting SEI for the DSCV SEIs without requiring additional syntax changes.

JVET-AN0078 AHG9: Digitally signed content (DSC): On subpicture signing [I. Sodagar, C. Fersch, S. McCarthy (Dolby)]

This contribution provides an analysis on the 4 various options in TuC for selective subpicture/NAL signalling in DSC and provide a few possible paths forward.

Two set of use cases were previously provided:

  - JVT-AL00186: signing and verification of content for a variety of services that may take advantage of subpicture extraction, including VR streaming, multi-view streaming (i.e. side-by-side content streams) and ROI-based services.

  - JVET-AM0205: signing and verification of the content by excluding or including one or more ROI for overlay application using single-stream streaming and/or playback.

Another possible use case can be:

  - Another use case is reducing the hash calculation with the trade of off light level of authentication. In this case, selective NAL association enables limiting the number of hash calculations per PU for the purpose of reducing computational complexity. This approach is similar to pattern encryption used for encrypted content.

The use case for replacement of an ROI was suggested to be contrary to the purpose of the SEI.

It was questioned with option 4 in the TuC if is required that all subpicture we included in a substream. The original proponent of option 4 stated that it was the design intention to require that, although it might not be explicitly required in the current draft text.

JVET-AN0107 AHG9: On scalable-nested DSCI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

JVET-AN0107 also relates to the study of SEI messages in VSEI, VVC, HEVC and AVC

This contribution proposes to utilize the scalable nesting SEI message to support content verification of extracted subpictures in TuC.

The proposed changes of the SN SEI message and scalable nested DSCI SEI message are as follows.

- When the scalable nesting SEI message contains an SEI message that has payloadType equal to 220 (digitally signed content initialization), the value of sn_subpic_flag shall be equal to 1.

- dsci_vss_implicit_association_mode_flag shall be equal to 1 in a scalable nested DSCI SEI message.

- There shall be no DSCS SEI message associated with the scalable nested DSCI SEI message, and the associated verification system applies to all subpictures indicated by the scalable nesting SEI message.

- SPS and PPS are not associated with any verification substream associated with the scalable nested DSCI SEI message

Proposes to include both the DSCI and DSCV in the scalable nesting SEI.

This solution allows signing of individual subpictures, but not of signing of a group of subpictures, because the scalable nesting SEI message applies to each subpicture individually and the subpicture extraction process can only extract a single subpicture and not a group of subpictures.

It was suggested that the proposed constraint in item 2 is unnecessary if a different ID value is used.

JVET-AN0187 AHG9: On DSC SEI messages for subpicture-based signing in TuC of VSEI [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution presents an analysis of four options related to digitally signed content SEI messages for subpicture/region-based signing, which are currently included in the TuC of VSEI (JVET-AM2032). The analysis shows that the four options provide the same functionality, with differences only in the signalling method. A comparison of syntax elements and bit overhead under typical subpicture-based bitstream conditions indicates that Option 4 requires the least number of additional bits. Therefore, it is proposed to merge the four existing options into Option 4.

JVET-AN0210 and JVET-AN0078 is related.

Options 1-3 were proposed by a different proponent in JVET-AL0186. Option was previously proposed by the proponent in JVET-AM0193.

Some bitrate estimates were provided for the different options under some suggested test conditions.

JVET-AN0188 AHG9: On signing parameter sets in DSC SEI messages for subpictures in TuC of VSEI [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes to include SPS and PPS NAL units for subpicture-based signing in DSC SEI messages in TuC of VSEI.

The summary of the proposed modifications are as follows:

–	In the VVC interface text, add a constraint that a verification substream shall not be associated with more than one subpicture.

–	In the VVC interface text and VSEI document, add interface text to address SPS / PPS NAL units, that are set for extracted subpicture sub-bitstream according to C.7 of VVC document.

–	In the VSEI document, modify the verification process of the bitstream signature by adding a process of replacing the original SPS / PPS NAL units to SPS / PPS NAL units that are set for each extracted subpicture sub-bitstream according to C.7.

–	In C.7 of the VVC document, add a sentence saying that when subpicture sub-bitstream extraction is done, dsci_subpicture_mode_flag is set to 0.

In the v2 of this document, the modifications on the first bullet above and the related text were made.

Should the SPS and PPS be included in the hash calculation for the signing of an individual subpicture or sets of subpictures?

It is noted that the sub-bitstream extraction process in VVC works to extract an individual subpicture and not a set of subpictures, and it involves rewriting of parameter sets.

It is noted that it is not possible to rely on the signing of the SPS and PPS in the signing of the complete picture.

This contribution provides a solution for including parameter sets in the signing for subpictures.

JVET-AN0210 AHG9: On subpicture signing in the DSC SEI messages [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

The TuC for future extensions of VSEI (JVET-AM2032-v2) contains four options for subpicture/region-based signing with the digitally signed content SEI messages. At the meeting in Daejeon, it was encouraged to bring contributions to the Geneva meeting to converge on a single solution.

This contribution characterizes the four options and proposes to converge to a single solution consisting of option 3 with the addition of the extension mechanism from option 4. This will remove other options from the TuC.

In case it is decided to keep any of options 1, 2 or 4 in the TuC, it is suggested to update the syntax and semantics for these options by:

- Adding the extension mechanism to options 1 and 2.

- Adding VVC specification text to option 4 to disallow signing of SPS and PPS for subpictures and to specify that the value of dsci_subpicture_mode_flag shall be equal to 0 if sps_subpic_info_present_flag is equal to 0.

It is noted that for use of the scalable nesting SEI message to indicate the association of a set of subpictures, the subpicture ID from the slice header must be determined, which includes parsing the slice header, SPS, and PPS, which can be a burden for the verifier.

It was suggested that sub-bitstream extraction would have already have taken place before verification, so perhaps is not a burden for the verifier.

It is asserted in the contribution that the proposal avoids the need to determine the subpicture ID.

It was noted that option 4 does not allow multiple verification substreams for the same NAL unit, if the same DSC ID is used.

JVET-AN0370AHG9: Digitally signed content DSC): On subpicture signing [
I. Sodagar (Dolby), M. Pettersson (Ericsson), J. Lee (LGE)]

Reviewed at 14030 on 9 Oct 2025, chaired by J. Boyce.

Subpicture authentication constraints:

- There is always another authentication stream for the entire picture, requiring that every PU to be included in one verification substream.

- Optionally the bitream may have an authentication stream that signals authentication of one or more subpictures togethers in one verification substream.

- The nesting scalable SEI is used when extracting a single subpicture, i.e. the authentication signalling cannot be used for combined authentication of a collection of subpictures together.

Options to consider:

- For simplicity/efficiency, option 4 (DSCS applicable to NAL units following the DSCS) with following additions:

  - Skip flag in DSCS to allow skip

  - Exclusion of SPS PPS in authentication.

  - Signalling which subpictures belong to which verification substream in DSCI.

Or:

- Use Option 1 with either:

  - add a loop in DSCS to signal one or more subpicture + constraint that the signalling shall stay the same in different PUs.

Or

  - Add a loop at DSCI assigning subpictures to a verification substream.

It was noted that parsing of headers and parameters sets is needed in order to determine the subpicture ID of a NAL unit.

It was suggested that not signing the SPS and PPS could mean that if they were replaced by an attack the bitstream could become undecodable.

A question was raised of if we already have SEI messages that apply to part of a PU. The DU SEI is associated with a portion of a PU.

A question was raised about backwards compatibility of these proposals.

Agreed to replace the 4 options in the TuC with Option A of this contribution.

### Film grain regions characteristics SEI message (3)

Contributions in this area were discussed during 1730–1750 on Monday 6 Oct. 2025 (chaired by J. Boyce).

JVET-AN0069 AHG9/AHG13: Updates for the film grain regions SEI message [E. François, F. Urban, P. de Lagrange, D. Doyen (InterDigital), G. Teniou (Tencent)]

This contribution proposes updates for the film grain regions (FGR) SEI message (included in the technologies under consideration for future extensions of VSEI).

Issue 1 – blending equations of synthesized grain with alpha channel

It is questioned why there is a division by 255, etc. rather than by 256. It is noted that the existing film grain characteristics SEI has this design.

Issue 2 simplifies indication of one vs. two models.

Issue 3 proposes reordering the syntax elements and avoids signalling unnecessary syntax elements for region based parameters when operating in frame based mode.

Agreed for issues 1, 2, 3.

JVET-AN0146 AHG9: On the film grain regions characteristics SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the film grain regions characteristics (FGR) SEI message:

- Remove the ChromaFormatIdc condition in the interface text.

- Add two constraints for fgr_matrix_coeffs.

- Change the semantics of fgr_comp_model_present_flag[ m ][ c ] when it is equal to 0 to reflect the actual process.

- Add a sensibility constraint for fgr_intensity_interval_upper_bound[ m ][ c ][ i ] and fgr_intensity_interval_lower_bound[ m ][ c ][ i ].

- Add inference for fgr_comp_model_value[ m ][ c ][ i ][ 0 ] when it is not present.

In JVET-AN0229, the same issues 1 were already agreed for FGC SEI.

Issues 2, 3, and 5would align FGR with FGC.

Issue 4 is a sensibility constraint.

Agreed for issue 1, 2, 3, 4, 5.

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the SDI, GRI, and PSI SEI messages

The -v2 version of the contribution updates item 4.

Agreed on item 4 in -v2.

### Quality metrics SEI message (3)

Contributions in this area were discussed during 1420–1430 on Wednesday 8 Oct. 2025 (chaired by J. Boyce).

JVET-AN0066 AHG9: On the QM SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes to the quality metrics (QM) SEI message in TuC.

- Propose relaxing the processing order constraint between the QM SEI message and the corresponding post-processing SEI message to enhance flexibility

- Propose constraint on non-PON QM SEI message

- Editorial changes on the text relevant to qm_three_component_flag[ i ]

For item 1, it was unclear why the additional flexibility in position of the QM SEI is useful given that the encoder can control the order. No action.

Agreed on item 2.

Item 3: missing parenthesis delegated to editor.

No action on qm_three_component_flag[ i ] aspect.

JVET-AN0246 AHG9: Metric value range for quality metrics SEI [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

Chaired by S. Deshpande on 8 October 2025.

The quality metrics SEI message in the TuC allocates a range of qm_metric_type[ i ] values to unspecified quality metric types, for which some additional information is signaled to provide some descriptive information about the unspecified metric type. It is proposed to add signalling of a max quality metric value and number of fractional bits for unspecified quality metric types.

It was commented that this design was motivated by AG5 chair’s suggestion/ comment.

It was asked if a minimum value should also be signaled. The assumption is 0 is the minimum value.

Agreed.

JVET-AN0247 AHG9: Quality metrics SEI for concatenated encoding [J. Boyce, T. Biatek, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande on 8 October 2025.

Concatenated encoding is frequently performed on video content before it reaches an end user. It is proposed to add to the quality metrics SEI message in the TuC an encoder count syntax element to indicate a count of the number of encoding processes applied for which the quality metric values apply. For full reference quality metrics, when the encoder count, n, is greater than 1, the full reference quality metric calculation uses as reference the input picture to the n-th encoding process, rather than the original source picture.

It was commented that the transcoder may change the resolution or other parameters.

It was asked how to interpret the quality if resolution was changed and if the resolution of an earlier stage would matter.

In the proposal you can only go back one stage in one SEI message. Although multiple SEIs could be added one for each stage.

It was commented that this may have some benefit for testing and debugging purpose. But it was asked why much detailed information may not be available for those purposes.

It was asked how this may help if it was a scalable bitstream. There is no signalling specific to a scalable case.

Further study.

### Lens optical correction SEI message (1)

Contributions in this area were discussed during 1525–1530 on Wednesday 8 Oct. 2025 (chaired by J. Boyce).

JVET-AN0074 AHG9: On Lens Optical Correction SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

The Lens Optical Correction SEI message uses floating point values in one radial distortion model.  The naming of syntax elements currently follows the convention proposed for floating point syntax elements in JVET-AN0076.  Several other SEI messages include tables describing the association between syntax elements and floating-point values and this proposes to include a similar association table in the LOC semantics.  There are two editorial proposals.

- (Editorial) Add association table relating syntax to floating point values for consistency with the use of floating point in other SEI messages.

- (Editorial) Remove note mentioning the similarity of the floating-point description to that of IEEE 754.

Version 2 of this document uses the recently proposed fl(16) format and removes separate syntax element references.  The range of fl(16) is asserted to be sufficient for application examples.

Version 3 of this document removes all material not related to the proposal not related to support the fl(v) process introduced in JVET-AN0242-v3 and includes additional text from the TuC describing the equation X-4.

It is proposed to change the signalling of the loc_rd_angle_poly coefficients to 16 bit float.

Agreed to change the signalling of the loc_rd_angle_poly coefficients to 16 bit float.

### Display overlays information SEI message (8)

Contributions in this area were discussed during 1430–1500 on Wednesday 8 Oct. 2025 (chaired by J. Boyce).

JVET-AN0104 AHG9: On the DOI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes on DOI SEI message in TuC.

- Editorial changes

- Overlay texture derivation bug fix

- Add the derivation of display rectangle overlay texture

Item 1 has identified a bug in the doi_partition_type_idc[ ]. It was suggested to reorder the values of doi_partition_type_idc[ ] so that explicit is the highest value. A new version of the document to be uploaded that implements that.

Item also has a change to layer ID constraint.

Agreed to item 1, subject to a review of the revised text described above.

Delegate item 2 to editors.

Agreed to item 3.

Further discussed on 10 Oct. 2025.

Text for item 1 confirmed.

JVET-AN0105 AHG9: On applying display overlay on CR target picture [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes the following changes on DOI SEI message in TuC.

- Change doi_partition_type_idc data length from u(2) to u(3) to accommodate more overlay types

- Propose to support CR chroma enhanced target picture as a new type of overlay component for DOI SEI message

The contribution proposes applying an overlay to a target picture formed by the CR SEI. It would require that the SEI messages be applied in a particular order. It was suggested that the use case seems somewhat niche.

Increasing the doi_partition_type_idc data length can be considered if there is a need for more types.

JVET-AN0117 AHG9: On display overlays information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes miscellaneous modifications that are asserted to provide improvement to some aspects of DOI SEI messages in the TuC for future extensions of VSEI. The proposed modifications are as follows:

- On signalling of the width and height of alpha partition

- On consistent layer indexing for primary and alpha partitions

- On referencing doi_nuh_layer_id through doi_layer_idx

- On a unified constraint for layer-ID ordering under doi_layer_order_constrained_flag

For item 1, inference language is missing for determining the width and height of the alpha partition when same size flag is 1. Sometimes the width and heigh parameters are not present because there is no alpha partition.

Agreed to item 1.

Agreed to item 2, item 3, item 4.

Further discussed on 10 Oct 2025.

Agreement confirmed based on -v4 version of contribution. Text to be editorially modified to add a condition layer.

JVET-AN0148 AHG9: On the display overlays information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the display overlays information (DOI) SEI message:

- Clarify the constraints for the case when doi_partition_type_idc is equal to 1.

Avoid referencing a CR_EMPTY rectangle for doi_alpha_partition_idx.indicated that because of the actions on JVET-AN0149, the last proposed bullet can be removed.The proposed language: “The value of cr_rect_type_idc of the constituent rectangle specified by doi_alpha_partition_idx[ i ]” is suggested to be value. In the CR SEI syntax table, there are two array indices for cr_rect_type_idc, cr_rect_type_idc[ lIdx ][ i ], which are missing from the proposed text.

The constraint is considered to be needed, but the exact language requires some further review.

Further discussion requested.

Further discussed on 10 Oct 2025.

Agreed to Item 1 in -v2.

Item 2 includes editorial changes and also adds a constraint that any referenced CR is not an empty CR.

Agreed to item 2 in -v2.

JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages [K. Abe, T. Nishi (Panasonic), Thibaud Biatek (Nokia)]

JVET-AN0160 also relates to the CR and DR SEI messages

Delegate item 2 to the editors.

JVET-AN0207 AHG9: Overlay purpose indicator for Display overlays information SEI message [G. Teniou, S. Wenger, A. Hinds (Tencent)]

See notes under JVET-AN0366.

JVET-AN0248 AHG9: Overlay set purpose indicator for Display overlays information SEI message [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

See notes under JVET-AN0366.

JVET-AN0366 AHG9: Purpose indicators for Display Overlays Information SEI message [G. Teniou, S. Wenger, A. Hinds (Tencent), T. Biatek, J. Boyce, M. Hannusksela (Nokia)] [late]

Chaired by S. Deshpande on 8 October 2025.

This document is a merge of JVET-AN0207 and JVET-AN0248. It contains an update from the last meeting during which useful comments were made on the purpose indication of the overlays. They are addressed individually, and the proposed syntax is updated on top the of the one in the TuC. It also proposes to enable purpose indicators to be switchable and to be specified either for overlay sets or for individual overlays.

It was asked if only sets should have purpose signalled or should both the sets and overlays should have purpose signalled.

Some typos were fixed.

Agreed

### Bitdepth range information SEI message (1)

Contributions in this area were discussed during 1545–1630 on Wednesday 8 Oct. 2025 (chaired by J. Boyce).

JVET-AN0147 AHG9: On the bitdepth range information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the bitdepth range information (BRI) SEI message:

- Specify the persistence scope of the SEI message.

- Clarify the constraints for the case when bri_partition_type_idc is equal to 1.

Agreed to item 1.

Item 2 is being reworded because of the action on JVET-AN0149, to the following:

When bri_partition_type_idc is equal to 1, it is a requirement of bitstream conformance that both of the following conditions shall be true:

–	There is a constituent rectangles SEI message crSeiMsg preceding the BRI SEI message in decoding order.

–	There is an entry of cr_num_rects_in_layer_minus1[ lIdx ] in crSeiMsg for which cr_layer_id[ lIdx ] is equal to bri_nuh_layer_id[ i ].

It is suggested that this language isn’t clear that the constraint must be imposed for all values of i in the range of 0 .. brin_num_ranges_minus1, inclusive.

Further discussion requested.

Further discussed 10 Oct. 2025.

A -v2 version of the contribution was uploaded.

In -v2, per the discussion during the first group review and the agreed item 2 of JVET-AN0149, the revised text for item 2 is provided.

Agreed to the revised constraint text for item 2 in -v2.

### Colour mapping information SEI message (5)

Contributions in this area were discussed during 1630–2000 on Wednesday 8 Oct. 2025 (chaired by J. Boyce).

JVET-AN0116 AHG9: On the colour mapping information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

JVET-AN0116 also relates to the CR, BRI, and ECFI SEI messages

This contribution proposes clarifications to the Colour Mapping Information (CMI) SEI message, and accompanying clarifications to interacting SEI messages (including BRI, CR, and ECFI SEI messages), to ensure consistent interoperation when alpha or depth is defined per layer or per region. The proposed items include:

- Item 1: Constraint on composite pictures from multi-layer

- Item 2: On clarifying the application of CMI SEI message across regions

Item 1 addresses a use case where the CMI SEI message follows another SEI message which forms a target picture. is noted that the current CMI SEI design does not support operating on a target picture, but operates on a decoded picture. Before considering the specific technical proposals in this contribution would need to justify the use case and address how to support the CMI SEI operating with a target picture as its input.

Several other SEI messages in the TuC involve multiple layers.

What is the interaction with the SEI processing order SEI? SEI messages that form a target picture are considered to be process SEIs.The SPO SEI operates on a CLVS. This contribution address multiple layers.

Further study encouraged on how to handle processing order for a chain where some stages involve multiple layers and others are single layer.

Item 2 proposes syntax in the CR SEI to indicate the colour mapping SEI be used. It was questioned if that was the appropriate place to add this to CR SEI.

JVET-AN0231 AHG9: on the CMI SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution introduces the following proposed updates to the Colour Mapping Information (CMI) SEI message as currently defined in the VSEI TuC:

- (Editorial) Correct cmi_lut_present_flag to cmi_function_present_flag

- (Editorial) Correct the interpretation of LUT piecewise functions

- Propose new syntax element (cmi_output_colour_space_idc) to explicitly signal the target output colour space.

- Propose new syntax element (cmi_purpose_idc) to indicate the intended use of the colour mapping

- Propose new syntax element to define the numerical precision (cmi_poly_coeff_precision_flag) and scaling factor (cmi_poly_coeff_scaling_factor) used in polynomial-based colour mapping functions.

- Propose to limit the upper bound anchor point

Agreed to item 1.

For item 2, agreed to remove the Clip3 operator.

The anchorPointFoundFlag is actually needed.

The modification of the pseudocode is delegate to the editors to consider if it is an improvement.

Item 3 proposes output of colour mapping in different colour spaces than RGB, including LAB. It is noted that LAB is not usually directly displayed, and that it can have negative values.

It was questioned why YCbCr output would be needed, given that conversion to RGB typically happens before display.

Item 4 proposes a colour map purpose, with entries for human visualization, AI/training or machine, and accessibility.

Some references were provided for AI, but it was unknown if they operated on a conversion from 1 colour component to 3 colour components.

It is noted that some specific colour maps are designed for visibility with colour blindness.

Content creators intent for the human or machine is available in other SEI messages, such as EOI.

The purpose of accessibility was suggested to be worth considering.

Item 5 proposes syntax elements to allow reducing the number of bits used for signalling the polynomial coefficients.

It is noted that JVET-AN0242 would allow flexible floating point precision values.

Further study for precision of coefficient signalling.

In item 6 it was proposed to add a range for the cmi_num_points_minus2, which is coded as ue(v).

Agreed on item 6, to impose a range constraint on cmi_num_points_minus2, while maintaining ue(v) signalling.

JVET-AN0244 AHG9: Prespecified colour maps for the colour mapping info SEI message [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

Chaired by G. Teniou on 8 October 2025. It is proposed to prespecify colour maps for the Jet, Turbo, and Viridis colour maps in the colour mapping info SEI message. A table is proposed to describe the interpretation of cmi_colour_map_id, with either piecewise linear LUT parameters or polynomial coefficients provided for each specified entry in the table.

It was asked what is the particular ability of Jet, Turbo and Viridis colour maps. This was answered using example colour plots for these three colour maps.

It was asked if the usage of the words “Jet, Turbo and Viridis” are trademarked.

It was asked if 256 number in a table should be 255.

A few typos were found in the proposal which should be fixed editorially.

It was commented that one of the syntax elements (cmi_function_nominal_bitdepth_minus8) for which inference is proposed is actually proposed as a new syntax element in JVET-AN0245. So for the purpose of this discussion that syntax element should be ignored.

Final decision was done after discussion of JVET-AN0357.

Agreed with additionally the modification from JVET-AN0357 to be applied to replace the Jet colourmap.

JVET-AN0245 AHG9: On colour mapping info SEI message [J. Boyce, M. M. Hannuksela, T. Biatek (Nokia)]

Chaired by S. Deshpande on 8 October 2025.

Several modifications to the colour mapping info SEI message in the TuC are proposed, related to the polynomial signalling functionality, summarized as follows.

- Replace cmi_poly_order_minus1 with cmi_poly_order_minus2

Currently two way of signalling a line. Agreed

Avoid right shifting of negative numbers – it was commented by several experts that it is not an issue. No action was taken on this.

- Add an interface variable for decoded picture bit depth

The variable is decBitDepth which is the bit depth of the decode luma sample.

Agreed to replace decBitDepth with BitDepthY

- Signal a nominal bit depth for signalled functions

It is asserted that a prespecified map has a nominal bit depth, decoded picture has a bit depth, display has a bit depth. It was commented that there are three bit depths: Input bitdepth, output bitdepth, decoded bitdepth (BitDepthY).

In the first review it was noted that the equations need to be modified, also some possible missing bracket was noted. Initially, conditional agreement was reached, subject to review of the text. Further discussed with the text review aspect for this item on 10 October. It was finally agreed to add the item 3 text in TuC, but further study is encouraged to improve the equations related to item 3.

JVET-AN0357 AHG9: On prespecified colour maps for the colour mapping information SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)] [late]

Chaired by G. Teniou on 8 October 2025.

This contribution relates to the suggestion of adding prespecified colour maps to the colour mapping information SEI message. It was initially intended to be a crosscheck of the values proposed in JVET-AN0244. However, it is asserted that the values proposed in JVET-AN0244 for the Jet colour map do not correspond to the conventional implementation of the Jet colour map and produces significant visual differences. This contribution can be summarized as follows:

- An independent implementation of the Turbo colour map confirms the correctness of the values proposed in JVET-AN0244

- An independent implementation of the Viridis colour map confirms the correctness of the values proposed in JVET-AN0244

- An independent implementation of the Jet colour map as proposed in JVET-AN0244 shows visual differences compared to the conventional implementation of the Jet colour map

- New prespecified values for the Jet colour map are proposed

For the Jet colour map, it is proposed to increase the number of anchor points of the piece-wise linear LUT.

Agreed to use the parameters for the Jet colour map.

### Auxiliary sampling alignment information SEI message (1)

Contributions in this area were discussed during 1700–XXXX on Saturday 11 Oct. 2025 (chaired by J. Boyce).

JVET-AN0233 AHG9: On ASAI SEI Message applied to primary and auxiliary layers [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

Feedback from JVET-AM0083 and JVET-AL0224 highlights ambiguity in geometric alignment and the relationships between multiple auxiliary and primary pictures, especially for advanced use cases like stereo imaging, subsampling, and gaming.

An ASAI SEI message is associated with an ASAI identifier asai_id

A use case was suggested where a confidence auxiliary picture type describes a depth auxiliary picture.

It was noted that the adoption of JVET-AN0265 to modify the SDI SEI to allow an auxiliary picture to be associated with another auxiliary picture applies only to the new aux types in the TuC and not to the legacy types, such as alpha and depth. So the current ASAI design does not allow indication of an associated depth auxiliary picture.

It is noted that there is an editorial bug referencing the SFI SEI, which should be ASAI SEI.

### Localization and mapping information SEI message (4)

Contributions in this area were discussed during 0900–1035 on Thursday 9 Oct. 2025 (chaired by J. Boyce).

JVET-AN0075 AHG9: On Localization and Mapping SEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

The localization and mapping SEI message defines the position and orientation of a camera used to capture video along with additional application specific information.  The following modifications are proposed.

1) (Editorial) Typo fixes.

2) (Editorial) Syntax renaming for consistency with floating point convention. Unneeded if floating point change.

3) (Editorial) Introduce an associate table between floating-point values and syntax elements to be consistent with other floating-point use in SEI messages. Unneeded if floating point change.

4) (Editorial) Remove notes referencing similarity of floating point to external specification. Unneeded if floating point change.

5) (Technical) Addition of a distortion model asserted to be used in automotive applications.  Floating point signalling is modified.

6) (Technical) Two options for lam_content_features_presence_flag.

a) Provide meta-data on driving side.  Change flag to indicator and including addition of automotive application data specifically signalling if the captured video was in Right-Hand traffic or Left-Hand Traffic environment. This is asserted to benefit applications such as sign reading.

b) Represent flag with 1 bit rather than 2 as is currently done.

7) (Technical) Improve signalling of unit quaternion, Two options.

a) Signal only 3 components and 1-bit sign rather than 4 floating point components.

b) Signal only 3 components and modify the semantics so the first component is positive, and the sign is used for the fourth, unsignalled, component.

8) Reduce the number of bits used for Euler angle signalling

9) Modify floating point signalling of quaternion components.

10) Modify floating point signalling of rotation components.

The v2 version modifies the range of lam_camera_model to account for additional camera model.

The v3 version of this document replaces individual sign, mantissa, exponent syntax elements with use of fl(n) and  the number of bits needed to signal angular variables is reduced.  This change is introduced for all floating-point elements, proposed radial distortion coefficients, existing quaternion components, and existing rotation matrix components. )

Version 4 of this document improves the floating-point language and aligns with the LOC updates for the distortion model floating point description.  Highlighting is added to indicate changes more clearly.

Delegate editorial improvements to editors. Some changes no longer needed because of floating point change.

Item 5 proposes an additional value of lam_camera_model.

It would be good to add reserved values to the table. The syntax element is coded as u(7) so there are many remaining values. proposed model is asserted to be a commonly used automotive fisheye model.

Agreed to item 5 add the additional value of lam_camera_model.

Item 6 proposes an indication of the side of the road. This seems very specific and not within the current scope of the message.

The current design has a syntax element lam_content_features_presence_flag which is u(2). Decision to change to u(1). In the future, if additional values are needed, it can be made an idc.

Agreed to modify the loop of quaternion values to send 3 instead of 4 values, as the 4th value can be derived. (Alternative preferred 2 option of item 7).

Item 8 proposed changing yaw, pitch, and roll rotation parameters from i(32) to i(16).

It was questioned why the yaw, pitch, and roll rotation parameters are coded as fixed point integers rather than floating point values.

It is noted that JVET-AN0260 proposes using fl(16). Consider this item when JVET-AN0260 is discussed.

Agreed to change signalling of quaternions and the rotation to use the fl(32) instead of separate sign, exponent, and mantissa syntax elements.

JVET-AN0150 AHG9: On the localization and mapping SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes the following changes for the localization and mapping (LAM) SEI message:

- Change the persistence scope for the intrinsic camera parameters.

- Change the persistence scope for the extrinsic camera parameters.

- Specify the persistence scope for the content feature

JVET-AN0167 item 2 is related to items 1 and 2.

It was questioned why there are separate persistence and presence flags in the current design.

See notes for JVET-AN0167.

JVET-AN0167 AHG9: On localization and mapping SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes miscellaneous modifications that are asserted to provide improvement to some aspects of LAM SEI messages in the TuC for future extensions of VSEI. The proposed modifications are as follows:

- On rotation representation – constraints and decoder behaviour options

- On revising the persistence mechanism

It is noted that the GRI SEI message uses a single presence flag, but derives a variable that is used in the inference to distinguish between a value that persists vs. a value that is not applicable.

It is noted that the parameters controlled by the presence and persistence flags are considered likely to change infrequently.suggested that the persistence flag should apply to an individual camera id. The semantics for the persistence flag needs to be updated to apply to an individual camera.

Agreed to option 2, subject to review of text with modified persistence language to apply to the camera id.noted that the cancel flag in the current design cancels all camera_id values. It was questioned if that was the desired behaviour. Further study on this aspect is encouraged.

Proposal on quaternion is considered unnecessary given the adoption in JVET-AN0075.

It is recognized that the problem described is indeed a problem. Further study on the best solution is encouraged. There is concern that it is difficult to impose strict requirements on floating point numbers.

Further discussion was held on 10 Oct. 2025.

The text was confirmed.

JVET-AN0260 AHG9: Suggested modifications for the LAM SEI [V. Zakharchenko, J. Boyce (Nokia)]

Chaired by S. Deshpande on 9 October 2025.

This document provides a suggestion on improvement of the location and mapping SEI message proposed in JVET-AM0181and JVET-AM0103. While the original proposal has been designed to support the 3D model creation from 2D video streams this document suggests to account for location and mapping in graphic environments to assist correct placement of virtual objects in scenes captured with 2D cameras.

This contribution describes the following suggested modifications:

- Specifies the axis system orientation: Agreed

- Uses fl(v) coding for signalling of floating-point values, as proposed in JVET-AN0242: A previous decision on adding fl(n) has been noted elsewhere. This modifies the current syntax rto additionaly make the length variable. It was asked what precision is really needed and if we really need the flexibility of two different precisions as this is signalling. It was asked what the applications need. Currently the syntax elements were 32 bits. The proposal allows both 32 and 16.

Agreed to use fl(32) only currently. It was requested to show some information/ use case/ application which needs different precision in signalling.

- Clarification of the rotation process using the Euler angles, including type of rotation, rotation axis order, indication of rotation order and interpretation of the sign of the Euler angle.

It was asked if integer signalling should be used instead of floating or fixe point signalling for Euler angles. Same comment as above about using fl(v) instead of a fixed value. Support for adding the proposed flag lam_euler_rotation_ccw_flag.

Regarding using integer signalling (another proposal) it was commented that we need to handle floating values anyway and that it is not a burden.

Agreed to add the flag, use the new for loop with i < 3 (instead of 2) and fl(32) instead of fl(v), and semantics as proposed. Variable names could editorially changed.

- Clarification of the measurement units and uses interface for variables for the picture width and height. Agreed

- Clarification of the translation and depth information. No change at this time. Consider some additional information for possible reasoning to change this.

- Additional item proposal to change lam_rd_poly3_k1, lam_rd_poly5_k1 and lam_rd_poly5_k2 from i(25) to fl(v). Agreed instead to change those to fl(16).

### Danmu information SEI message (6)

Contributions in this area were discussed during 1045–1600 on Thursday 9 Oct. 2025 and 0900–1050 on Friday 10 Oct. 2025 (chaired by J. Boyce).

JVET-AN0115 AHG9: On the danmu Information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes changes that are asserted improving various aspects in the design of Danmu Information SEI message in technologies under consideration for future extensions of VSEI. The proposed items include:

- Item 1: On inferred value for fixed duration

- Item 2: On signalling global fixed duration

- Item 3: On danmu indexing rule

- Item 4: On signalling source video URI to link danmu with its source video

- Item 5: On single-loop signalling for danmu properties and text

- Item 6: On changing di_lang_idx[i] coding from ue(v) to u(v)

- Item 7: On changing di_num_languages_minus1 from ue(v) to u(8)

- Item 8: On signalling of common danmu display attributes

- Item 9: On signalling creation time of DI SEI message

JVET-AN0232 is related to item 1.

Agreed on item 2. It may be desirable to include a note to indicate that the system determines the duration.

No action on item 3.

The use case for item 4 is not understood. No action.

For item 5, it allows easier processing but has a bitrate overhead because of frequent byte alignment for text.

It was suggested that an entry point offset might be an alternative to allow for sequential processing.

It is noted that the encoder has the flexibility to limit how many danmus are represented in each SEI message.

Further discussion requested for item 5.

Further discussion of item 6, 7 after JVET-AN0262.

For item 8, JVET-AN0261 has an alternate solution to address the same issue. Further discussion requested after JVET-AN0261.

There was support for the concept of having some default parameters because it is common for the damnus to use the defaults.

The inference language is problematic.

For item 9, the use case was not clear. No action.

Further discussed 10 Oct. 2025.

Further study item 1 as part of larger questions regarding timestamps and durations.

Further study on item 5. The proposal allows sequential processing of each Danmu without having to do processing in two passes.

Further study item 6 and 7.

It was suggested that there is no need to signal a default language as the inferred value could be the index 0.

For item 8, agreed to add default value signalling and inference for the di_display_type_idc[ i ] and di_colour_idc[ i ], with improved inference language. JVET-AN0261 is also related.

JVET-AN0232 AHG9: on the Danmu SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

During the JVET AM meeting, feedback was received on JVET-AM0182 regarding the signalling of Danmu rendering parameters. Specifically, concerns were raised about (1) the ability to signal font size for individual Danmu messages, (2) the need to support unspecified font types and the signalling of specific font families, and (3) the control of Danmu transparency levels.

In response, this proposal introduces syntax elements to specify Danmu font size, font type, and transparency level, along with a clarification on the use of start and end PoC values for long-duration Danmu timing. Specifically, it:

- Proposes syntax elements to specify Danmu font size.

- Proposes syntax elements to specify Danmu font type

- Proposes syntax elements to specify Danmu transparency level

- Clarification on Start and End Danmu PoC for long danmu timing

It was questioned if it makes sense to signal a particular point size because this will be impacted by the display and user control.

It was suggested that rather than signalling a point size, to signal a small/medium/large indication. Further study encouraged for a size indication that is relative in nature and not describing a particular point or pixel size. It was noted that CSS provides 8 size values.

It is proposed to signal a font type per Danmu. The proposed syntax has no option to disable the feature and avoid sending font information per Danmu.

A question was raised about what the intended playback behaviour is when the font type is not supported.

It was suggested to remove the font_type_idc[ ] and only support OFF.

It was suggested to have the syntax elements signaled per danmu to correspond more closely to the OFF parameters.

Further study encouraged on font type to dramatically simplify the proposed changes.

Transparency is seen to be primarily viewer controlled than creator controlled.

Delegate the exact language of the note in section 2.4 to the editors.

JVET-AN0259 AHG9: Enable the regional display in the Danmu information SEI message [S. Xie, P. Wu, W. Niu, Y. Gao, C. Huang (ZTE)]

This contribution proposes to support the regional display of Danmu messages to improve the design and flexibility of the Danmu Information (DI) SEI message in the current VSEI TuC. In detail, the proposed modification is to add the region indication message into the DI SEI message.

Example videos were shown that show a face region that avoids having a Danmu displayed, but it is not aligned to rectangles, as is proposed in this contribution.

It was commented that the face region can move, which might cause problems.

It was remarked that client applications can do a face detection and subtitle detection to avoid displaying a danmu over those areas.

It was questioned if this feature is useful for an archival application.

Multiple regions width and height are signalled, and whether or not display is allowed. There is no restriction against overlapping regions.

JVET-AN0261 AHG9: Comments on the Danmu information SEI message [S. Xie, P. Wu, W. Niu, Y. Gao, C. Huang (ZTE)]

This contribution proposes to add a syntax element to control the presentation of Danmu display features to improve the design and flexibility of the Danmu Information (DI) SEI message in the current VSEI TuC.

It was suggested to have default values explicitly signaled for the di_display_type_idc[ i ] and di_colour_idc[ i ], which are inferred when the presence flag indicates that they are not present.

See notes under JVET-AN0115.

JVET-AN0262 AHG9: On danmu information SEI message [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

This contribution analyzes the current danmu information SEI message format by comparing it with commercial implementations of danmu formats. It is asserted that discrepancies exist with certain DI SEI parameters such as media timestamp, presentation duration, language information, danmu identifier and RGB colour format. Based on this analysis, the contribution suggests modifications to address the asserted discrepancies.

From the parser of the danmu file present in the DanmakuFlame at Sample/src/main/java/com/sample/BiliDanmukuParser.java, the documentation describes the following properties of a danmu object:

- p[0]: Media timestamp

- p[1]: Type (1: right-to-left scrolling | 6: left-to-right scrolling | 5: top-fixed | 4: bottom-fixed | 7: advanced | 8: script-based)

- p[2]: Font size

- p[3]: Colour

- p[4]: Timestamp (world-clock time)

- p[5]: Bullet screen pool ID

- p[6]: User hash

- p[7]: Bullet screen ID

In order to make the danmu information SEI message usable by common danmu services, the following changes are proposed which are all independent from each other and can be decided individually:

- Add signalling of the media timestamp associated with each danmu.

- Add a unique identifier to each danmu.

- Rename di_fixed_duration in di_fixed_presentation_duration and adapt semantic accordingly.

- Remove the language information.

- Refine the colour parameter to reference the sRGB colour space.

The current design does not have a Danmu ID.

It is noted that the current design has u(24) for RGB, and indicates the order of R, G, B, but doesn’t specify how to interpret the values. The contribution proposes indicating that it is sRGB. The IEC 61966-2-1 is referenced.

Further study encouraged on the media timestamp and duration, related to items 1 and 3.

The Danmu ID is proposed to be signalled as u(32). It was suggested to add a presence flag to gate it.

Agreed to item 2 signalling of di_identifier[ i ], with a higher level presence flag added once in the SEI message. A new version of the contribution to be uploaded to reflect this change.

Item 4 proposes removing the language information. It was suggested that an alternative would be to remove the minus 1 from di_num_languages_minus1. Further study on language aspect.

Agreed to item 5.

### Graphics rendering information SEI message (3)

Contributions in this area were discussed during 1130–1205 on Friday 10 Oct. 2025 (chaired by J. Boyce).

JVET-AN0144 AHG9: On the graphics rendering information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes to change the persistence scope specification for the graphics inforamtion in the graphics rendering information (GRI) SEI message.

Agreed.

JVET-AN0230 AHG9: on the GRI SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

- add a constraint to gri_near_plane and gri_far_plane

- Editorial changes:

  - Miss match gri_game_engine_params_present_flag and gri_game_engine_parameters_present_flag. Prefer to use gri_game_engine_params_present_flag to minimize text change.

  - When gri_game_engine_parameters_present_flag equal 1, there should be a gri_right_hand_flag, not gri_left_hand_flag

  - gri_projection_coeff[ i ][ j ] shall be associated with gri_projection_matrix_present_flag, not gri_world_to_cam_matrix_present_flag.

  - When gri_depth_params_present_flag equal to 1, change gri_disparity_flag to gri_depth_inverse_flag

  - Change gri_disparity_flag  to gri_depth_inverse_flag

  - gri_right_handed_flag no zero value signalled (second “equal to 1” shall be equal to 0)

  - change gri_data_format_idc to gri_data_precision_format_idc

  - change gri_focal_plane to gri_focal_length

  - other miscellaneous fix highlighted inline.

The group agree to item 1.

Editorial changes were delegated to the editors.

JVET-AN0235 AHG9: on the SDI SEI message dependency [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AN0235 also relates to the SDI, FGRC, and PSI SEI messages

This proposal introduces SDI SEI message dependency constraints for the SEI messages listed in the VSEI TuC that currently lack explicit dependency signalling:

- GRI SEI messages

- PSI SEI messages

- ASAI SEI messages

- FGRC SEI messages

Two constraints are proposed:

- When an AU contains both an SDI SEI message and a GRI SEI message, the SDI SEI message shall precede the GRI SEI message in decoding order.

- It is a requirement of bitstream conformance that, when a GRI SEI message applies to auxiliary layers of type AUX_DEPTH, an SDI SEI message with sdi_aux_id[i] equal to 2 for at least one value of i shall be present in the same CVS.

The first constraint is seen as overly restrictive, because the GRI may not refer to auxiliary depth layers.

In the wording of the second proposed constraint, it isn’t clear what is meant by “applies to auxiliary layers of type AUX_DEPTH”.

Some type of constraint related to the SDI SEI message may be needed when the GRI SEI is describing characteristics of a depth auxiliary layer. Further study encouraged to properly express the constraint, for example by referring to syntax elements in the GRI SEI message.

### Enhanced colour information SEI message (1)

Contributions in this area were discussed during 1205–1210 on Friday 10 Oct. 2025 (chaired by J. Boyce).

JVET-AN0143 AHG9: On the enhanced colour format information SEI message in TuC of VSEI [J. Xu, Y.-K. Wang (Bytedance)]

This contribution proposes to specify the persistence scope for the enhanced colour format information (ECFI) SEI message.

Agreed.

## Identify potential needs for additional SEI messages, including study of AVC and HEVC SEI messages for use in VVC (7)

### Gaussian splatting information SEI message (4)

Contributions in this area were discussed during 1400–1700 on Friday 10 Oct. 2025 (chaired by J. Boyce).

JVET-AN0100 AHG9: Gaussian splatting information SEI message [Y. He, J. Jung, A. Akhtar, L. Kerofsky, G. van der Auwera, A. Akhtar, M. Karczewicz (Qualcomm), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

This contribution proposes a Gaussian Splatting Information (GSI) SEI message to support video based Gaussian Splat Coding (GSC) currently being jointly explored by MPEG WG4 and WG7. The proposed GSI SEI message extends the previous work in m73127/JVET-AM0212 to achieve efficient and lightweight representation of 3DGS. Additional showcase and test results are provided for both static and dynamic scenes. This contribution is concurrently submitted to WG4 as m74340.

The proponent asserts that an SEI message approach would enable fast standardization and deployment.

The INRIA format is named after the first paper in the field, and is a term known in the larger research community, and not just within MPEG.

In the proposal, width and heights are the same for all attributes.

In the showcase, 61 attributes are used.

Bit depth is signalled for each attribute component.

The gsi_frame_packing_idc syntax element is unnecessary.

Attributes are rectangular regions packed into pictures in multiple layers.

The approach could be used for static or dynamic gaussian splats, using still picture or video, respectively.

For video coding, RPR hasn’t been considered, e.g. the coded picture size varies within a sequence.

The SEI message carries the information needed to reconstruct an INRIA format representation or any other format that have attributes in rectangular containers.

An asserted goal of the proposed approach is to be flexible to support future formats.

The Showcase uses HEVC HM 16.21+DCM8.8, but doesn’t use screen content coding tools. The approach hasn’t yet been tested with the screen content coding tools.

The showcase used separate 2 single layer bitstreams, although the syntax describes a multi-layer approach.

In the INRIA format, all attributes are floating point.

An encoder is able to select the bit precision for each attribute component, which may be scaled to be coded in a picture which is at a particular bit depth, which may differ from the attribute component bit precision.

In the showcase, the position had 16 bit precision, which was split into two 8 bit numbers – MSB and LSB and coded in a 10-bit 4:4:4 picture.

The showcase was tested using the CTC for Gaussian splats in document MDS25512, WG 07
Explorations Draft CTC for Gaussian Splat Coding WG 07 MPEG 3D Graphics Coding and Haptics Coding.

At different rate points, the video codec QP was adapted, and the effective precision of each attribute component was adapted.

A question was raised if the coded picture could be downsampled to reduce pixel rate and bitrate. For some attributes that would not be appropriate.

Full reference metrics were shown that use as a reference a number of rendered views from the uncompressed INRIA format representation.

A comparison was made with G-PCC. There is some activity in WG4/WG7 for V-PCC, but it is a newer activity and no comparison was made with it.

A question was raised if V-PCC could be used with the same coded picture as in this proposal.

Experimental results were shown for both static and dynamic Gaussian splats.

In the showcase, each attribute was 754x754 resolution.

A comparison was made with GSCODEC-STUDIO_mpeg152, which is a video based approach that is being used as an anchor in the Gaussian splat CTC for 2 meetings.

Both the proposal and GSCODEC-STUDIO_mpeg152 has significant coding gain relative to G-PCC for dynamic content.

It was questioned if the proposed SEI message approach could have an pre-processor that configured to create the same video(s) as used in this GSCODEC-STUDIO_mpeg152 anchor, in which case only the signalling would differ.  The answer to this question could help assess how flexible the proposed approach is, and its potential robustness to evolving research on best representations.

The proponents asserted that the number of bits of the SEI messages are negligible relative to the total bitrate for the showcase. It would be useful to have such a breakdown provided.

The software was not currently made available.

JVET-AN0205 AHG9: Gaussian Splat Information SEI message [J. Ricard, G. Teniou, S. Wenger, A. Hinds (Tencent)]

This contribution proposes a Gaussian Splat Information (GSI) Supplemental Enhancement Information (SEI) message that standardizes how 3D Gaussian Splatting (3DGS) attributes are stored in one or multiple coded videos, enabling interoperable decoding and reconstruction. The main advantage of this approach is high degrees of flexibility in how 3DGS components are stored in video planes to accommodate different types of 3DGS data.

The GSI SEI is per video and may persist across AUs. Each instance declares:

- Enable/persistence flags

- General 3DGS parameters

  - Block size: size of the block used to pack the 3DGS components into video (Ex. 640x640)

  - Colour and spherical harmonics colour space: None, BT601, BT709 or BT 2020

  - 3DGS bit depths for the geometry and attribute coordinates ( Ex. {18,12}, {32,32}, …)

  - Most significant bits (MSB) and Least significant bits (LSB) bit depth allocate for geometry coordinates (Ex. {8,8}, {10,8}, …)

- Flag indicates if the geometry coordinates have been signed log transformed.

- Video parameters

  - Format of the video used to store the 3DGS data (Ex, YUV444, YUV420, YUV400)

  - Packing: way to pack the 3DGS components into the video frames:

    - PLANAR: components mapped to planes/channels/blocks or

    - TEMPORAL: components mapped to frames/time

  - Bit depth of the video

  - Component map: Ordered list of 3DGS attributes present in the associated video/layer:

    - Most significant bits of geometry components: x,y,z

    - Less significant bits of geometry components: x_add, y_add, z_add

    - Scale: scale_0, scale_1, scale_2

    - Rotation: rot_0, rot_1, rot_2, rot_3

    - Opacity: opacity

    - Colour: f_dc_0, f_dc_1, f_dc_2

    - Spherical harmonics: f_rest_0, f_rest_1, …; f_rest_44

    - Empty plane: zero

  - Quantization descriptors: mode (linear, gaussian, …), and per-component min/max (or offset/scale) needed to de-quantize.

Data may be packed in a planar and/or temporal manner.

They have a separate SEI message per video, that describes the attributes within the video.

The approach aims at having multiple separate video bitstreams, which a systems layer would be expected to synchronize.

It was suggested that multi-layer codecs are not widely supported in implementations. It is a design goal for implementability on current phones.

It was noted that 3GPP SA4 has defined media capabilities for augmented reality where they have limited the number of decoder instances and MV-HEVC to support only 2 independently coded layers.

In the showcase, results are shown using between 1 and 4 videos. It was questioned what the sample rate range is for the showcase.

Effective bit depth precision is used only for position. It is separated into MSB and LSB.

Two quantization methods may be used – linear and non-linear using Gaussian.

At a high level, this contribution is similar to the approach in JVET-AN0100, with some differences in features and signalling approaches.

JVET-AN0206 AHG9: Gaussian Splat compression experiment platform [J. Ricard, G. Teniou, S. Wenger, A. Hinds (Tencent)]

JVET-AN0206 also relates to software and showcase information for SEI messages

This contribution presents the use of the Test Model Candidate reference software (TMCV) to compress 3DGS objects with one or several videos with metadata stored in Gaussian Splat information SEI messages. Some experimental results are presented in the second part of this contribution.

Some comparisons were made of different configurations of the proposal, aiming at different bitrate points, and switched between 4:2:0 and 4:4:4.

Comparisons were made with several solutions documented in m74809, [GSC][JEE 6.6 & JEE 6.7] Draft performance aggregation of existing MPEG GSC solutions.

It was asserted by the proponent that an SEI message approach is implementable and deployable on legacy hardware and software implementations.

The software was not currently made available. The proponents stated that it would be made available if added to the TuC.  The software includes support for the entire workflow and is in Python. In the workflow, multiple bitstream files are concatenated into a single file. The input and output are an INRIA format .ply file.

JVET-AN0264 AHG9: Support for implicit representations with the Gaussian splatting information SEI message [S. Lee, S. Sasse, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (Fraunhofer HHI)]

JVET-AN0264 also relates to NNPF SEI messages

This contribution proposes a novel method for Gaussian Splat Coding (GSC), through the usage of an intermediate representation (implicit representation) using optimized feature planes using SEI messages. It is proposed to extend the SEI message proposed at the previous meeting, i.e., SEI message Gaussian Splat Information (GSI) SEI, and extend the existing NNPFC SEI message to support the reverse mapping to the original Gaussian splat representation at the decoder side.

Revision 1 of this contribution adds

- Results with improved RD performance due to an enhanced optimization of feature planes; results now include additional sequences.

- An alternative compact syntax for the GSI SEI message proposal.

- Fixes of issues of the initial verbose GSI SEI message proposal.

- A mapping from regions in the GSI SEI message to the array FeaturePlaneRegion[][] in the NNPF SEI message.

- Output tensor derivation is also added in this version.

In the following, the approach taken by the authors to encode 3DGS is explained in detail, which consist of:

- Performing a factorization the 3DGS original attributes into feature planes;

- Mapping the features planes, which consist of multiple channels (planes stacks), together with the positions of the splats onto packed pictures;

- Encoding the packed pictures and required NNs into a video bitstream;

- At the decoder side, for each attribute, unpack the planes stacks and feed them to neural networks (NN) to output back the 3DGS attributes.

This contributions performs a factorization the original attributes (such as in an INRIA format) to feature planes in order to reduce the data. NN model information is signalled so that the receiver may invert the factorization process.

This factorization information could be used on top of one of the other two contributions approaches to signal packing of attribute components regions into pictures, but using the feature planes rather than attributes.

Instead of representing the rotation for the N splats directly (e.g., as an N×4 tensor of quaternions), a factorized representation based on triplanes is used. Let the set of plane coordinate combinations be p={xy,xz,yz}. This representation consists of three feature planes, Pxy,Pxz, and Pyz, each with a spatial resolution of Mw×Mh and C feature channels. Thus, each plane is a tensor Pp∈RMw×Mh×C.

When directly mapping the Gaussian splats onto pictures (i.e., attributes consisting of 59 dimensions overall), since some of the attributes (position) use more than one sample per dimension, 62 samples are required per gaussian splat. That means that for a particular level, the maximum number of gaussian splats that is supported would be at most the MaxLumaPictureSize [ level ] divided by 62.

The proposal allows reduction of sample rate for the video decoder, which may enable use of existing video codec HW for a larger number of splats. However, a NN is required at the receiver.

In the experiments, picture positions were coded losslessly.

In the experiments, information packed into UV planes that is unrelated to the Y component. This allows the most efficient use of sample rate may impact the coding efficiency because the Y, U, and V planes share a block portioning and motion vector.

It would be useful to have data on the sample rate and bit rate impact of the NN for factorization to feature planes. It was suggested that a comparison could be made with JVET-AN0206 and JVET-AN0100 experimental results for a rough estimate of the impact of the NN-based factorization to feature planes.

It would be useful to have estimates of the impact on encoder and decoder end complexity of the factorization such as kMAC/pixel estimates, and the size of the NN model. In the experiments, a 2 layer model was used with floating point weights.

Summary:

The proposed solutions are capable of enabling a system where a pre-process is performed to a Gaussian splat representation such as the INRIA format to pack attributes into pictures that are input to a video codec, and for a receiving system to invert the packing process to reconstruct the Gaussian splat representation such as the INRIA format.

Two of the contributions had slightly different sets of features and syntax, but were similar in spirit. Showcases had implemented software that performed the workflow of INRIA format in to INRIA format out, and was indicated by the proponents that the software could be made available quickly if work on this topic was launched.

One contribution also proposed factorization of Gaussian splat attributes into feature planes to reduce the amount of data. In addition to the packing information that the other two contributions signaled, this approach signalled a NN to invert the factorization, so that it also enabled reconstruction in the original Gaussian splat representation format. It was indicated by the proponents that the software could be made available if work on this topic was launched.

### Other (3)

Contributions in this area were discussed during 1215–1920 on Friday 10 Oct. 2025 (chaired by J. Boyce).

JVET-AN0077 AHG9: Sample Grouping SEI message [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes an additional SEI message to describe the grouping of captured image samples which are interleaved. A primary application is supporting compression of raw Bayer colour filter array samples which have colour components interleaved.

The SG SEI message signals the following elements:

- Indication of the horizontal and vertical interleaving steps sizes.

- Optional padding to ensure groups are contained within coding unit boundaries.

- Parameters describing dynamic range mapping.

The slide deck to be uploaded.

The proposed approach is to pre-process the Bayer image to make 4 regions in a single picture.

It was questioned why different interleaving deltas is required? It is not needed for Bayer, but perhaps could make the SEI message applicable to other use cases such as lenslet.

It was suggested to investigate what is currently being done in WG4 for lenslet coding, to understand their technical approach.

Range reduction/expansion is proposed for use because the Bayer samples may be 16-bit, and the preference is to code with a 10-bit codec.

The description of how the equations for range reduction/expansion are applied is unclear in the contribution.

The proposed name “sample grouping” is an overloaded term, and perhaps a different name could be used.

It was questioned if any existing SEI messages could be used or extended to support this functionality.

It was also suggested an SEI message with no payload with semantics for derived variables could be used. That approach would not allow range mapping capability, although other tone mapping mechanism exist.

Further discussion requested after text for the SEI message with no payload is available.

Further discussion on 11 Oct 2025.

The contribution was updated to support an SEI message with no payload, and the recommended postprocess can be determined by the presence of the SEI message.

The table parameters are using a syntax element style but are not syntax elements and should be renamed to use the mixed case as is done for variables.

Agreed to add to TuC, with modification of the naming.

JVET-AN0226 AHG9: Spatial and bit depth restoration SEI message [X. Xu, M. Xu, S. Liu (Tencent)]

This document presents the spatial and bit depth restoration SEI message, which provides information necessary to specify the intended spatial and sample bit depth information of the video, after decoding, for subsequent down-stream tasks. The proposed spatial resampling and bit depth shift information SEI message provides support on the decoder-side to recover task-specific video resolution and sample bit depth after decoding.

It was noted that the EOI SEI message provides signalling of the original source picture.

The use case for the spatial indication was unclear. A machine task at the decoder would likely know its preferred resolution. No action on this aspect.

For the bit depth aspect, it is noted that the bitdepth range info SEI message may be able to address the use case, but has more signalling with additional capabilities.

No action.

JVET-AN0227 AHG9: Temporal resampling information SEI message [X. Xu, S. Liu (Tencent)]

This document presents the temporal resampling information SEI message, which provides information necessary to specify the skipped frames by the encoder’s decision and their relationships with the decoded frames. The proposed temporal resampling information SEI message provides support on the receiving system end to recover frames that existed before encoding, for subsequent down-stream tasks.

It was noted that the EOI SEI message has temporal resampling information, but lacks an indication of recommendation of interpolation vs. extrapolation.

It was noted the VCM has some indication of skipped pictures, and has an indication of interpolation vs. extrapolation.

It was suggested that if such functionality is considered useful, it would be possible to extend the EOI SEI.

It would be useful to better understand the benefit of having that recommendation of interpolation vs. extrapolation.

Further study in the context of an extension to the EOI SEI.

## SEI Software and showcases (1)

Contributions in this area were discussed during 0900–0930 on Tuesday 7 Oct. 2025 (chaired by J. Boyce).

JVET-AN0122 AHG9/AHG11: Demo of real-time NNPF inference with banding reduction [S. Schwarz, H. Sethi, J. Funnell, M. Santamaria, R. Yang, F. Cricri, M.M. Hannuksela (Nokia)]

In this demo, Nokia showcases the application of a neural network post-processing filter (NNPF) for visual quality enhancement, specifically banding artefact reduction, for various video codecs (H.264, H.265, VVC, and AV1). The demo verifies the positive effect of NNPF on banding artefacts, even without specific weight updates (i.e., no content adaptation), and for various codec architectures, as well as the possibility to run NNPF inference (complexity block-wise: 34 kMAC/pixel, number of parameters 109068) for 1080p video in real-time on current consumer hardware.

Noted.

## Non-SEI HLS aspects (2)

Contributions in this area were discussed at the JVET plenary level during 1015–1115 on Friday 10 Oct. 2025 (chaired by JRO).

JVET-AN0162 AHG9: On subpicture sub-bitstream extraction in VVC [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to text for subpicture sub-bitstream extraction process in VVC. The proposed modifications are as follows:

1	On collocation property of subpictures for subpicture sub-bitstream extraction.

2	On the input target subpicture index.

In v2 of this contribution, additional text to explain the problem was added.

It is emphasized in aspect 1 that in case of multi-layer bitstreams, it is important to clarify that subpictures are colocated.

It was commented that in case of dependent layers, there is already such a constraint. In case of independent layers, it would give up some flexibility which might be desirable for some applications.

It was requested to bring clear evidence of a case where this problem is relevant (preferably demonstrating by software).

It was commented that an alternative could be to add a note, once it would be clarified under which cases this would be a problem.

Further study in aspect, could be included in document 1004 when the problem is better clarified.

On aspect 2, it was commented that the proposed semantics change might be problematic if layer 0 does not have subpictures, and layer 1 has subpictures e.g. for the benefit of region of interest enhancement.

No action was taken on aspect 2.

JVET-AN0178 VVC multilayer extension for an external base layer [F. Urban, Y. Chen, F. Galpin, E. François, P. de Lagrange, P. Bordes (InterDigital)]

Multilayer coding with an external base layer is a functionality that is available in SHVC but not yet in VVC. It is proposed to add this functionality to VVC.

Uses cases such as enhancing an HEVC or AVC legacy bitstream with a VVC enhancement layer are considered. To this end, it is proposed to follow a syntax similar to SHVC, adding an extension to the VPS to describe the format of the external base layer image, and adapt the decoding process to load the external pictures in the DPB.

In terms of reference software, it is noted that an implementation is already available in the NNVC software.

Was presented in JVET main track Friday 10 Oct. 1045 (chaired by JRO).

Inheritance of motion information from base layer would need to be switched off.

Signalling to be done similar as in SHVC, a change in VPS is proposed. This would require definition of a new profile.

No conformance could be defined in this context.

One expert reports that in terms of hardware, implementers of her company don’t see problems to extend chips that have multiple standards implemented.

Several experts expressed support.

More evidence would be requested about market needs to go into defining new VVC profile(s).

It should also be clarified if it would be necessary/beneficial to define at least a minimum level of conformance for the enhancement layer.

It was commented that this might be beneficial in a transition phase towards a new standard (one reference might be found in JVET-AK0236)

Also information would be beneficial about compression advantage, e.g. using a VVC enhancement layer on top of HEVC instead of using SHVC.

Conformance was defined in SHVC, and might be similarly done here.

Further study on those aspects.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

The following topics were discussed in JVET plenary Wednesday 8 Oct. 1115–1245:

- Liaison communication

  - No incoming liaison statements had been received until late in the meeting when a late letter was made available from JPEG. See further notes in 7.5.

- Scheduling for the remaining week was discussed (further detail on scheduling is recorded in section 2.12)

- Joint meetings

  - After the decision to conduct review of CfP responses in January 2027, the joint meeting on this topic on Friday was cancelled.

- Review status from main track

  - All EE1/EE2 reviewed, few revisits

  - Many 4.x categories not reviewed yet, planned for today and tomorrow

  - JVET main will not meet during ITU workshop on Friday afternoon, and during the MPEG information sharing sessions Thursday 9-10, Saturday afternoon

- Review status from HLS track and discussion on potential open issues

  - First pass on all contributions related to VSEIv4, VVC, HEVC, nothing critical, waiting for some input on revisits after offline activities

  - Initial review of ballot comments VSEI, review of DoCRs on Saturday morning

  - Integration into JVET-AN2006 started, needs to be integrated into H.274, FDIS can have editing period. Same for HEVC (integration already started), VVC not yet touched (to be clarified with Gary)

  - AVC related changes in JVET-AN1017 with editing period

  - Around 40% first pass on TuC and new messages

  - There are four contributions on support for Gaussian splat coding. As in parallel GS is also a topic of defining compression standards currently in WG 4 and WG 7, and they are still in the process of assessment of suitable technology, it would be premature to define such SEI messages in JVET. The contributions can be reviewed in terms of identifying syntactic correctness, to potentially help proponents to submit in a more mature shape for consideration in another WG. If proponents are not getting appropriate opportunity of presenting their idea in the other WGs, or having it assessed/compared with other technology that is investigated there, they should inform the JVET chair.

It was suggested to send a liaison to JPEG, to be reviewed on Sunday.

An initial review of modifications to be implemented in HEVC, VVC, VSEI and TuC was conducted in JVET at 1245 on Friday 11 Oct. No objections were raised. Final approval to be done on Sunday.

It was commented that for some SEI messages in TuC software implementation may be missing.

## MPEG information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Tuesday 7 Oct. 0900–1230 (the JVET meeting continued during that session, where only the JVET chair gave a brief report about achievements so far), Thursday 9 Oct. 0900–1000, and Saturday 11 Oct. 1400–1550.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

### Joint session 0830-1255 Wednesday 8 Oct. on next generation video standardization CfE / CfP: MPEG WG 2 / Requirements, MPEG WG 5 / JVET, MPEG AG 5 Visual Quality Assessment, and VCEG (ITU-T Q6/21)

This joint session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Gary Sullivan (VCEG Rapporteur and SC 29 Chair), Mathias Wien (AG 5 Convenor), and Jörn Ostermann (AG 2 Convenor), on behalf of Igor Curcio (WG 2 Convenor).

A joint meeting of Q6/21 with JVET, MPEG WG 2 Requirements and MPEG AG 5 Visual quality assessment was held on Wednesday 8 October 2025 to discuss the potential development of a next-generation video coding standard. A joint “Call for Evidence” (CfE) for potential development of such a standard had been issued from the interim JVET meeting of July 2025 (as JVET-AM2026). This section describes the discussions held in this joint meeting.

Notes were taken by G. J. Sullivan.

Submissions responding to the CfE

Submissions responding to the CfE were summarized as follows:

- All submissions were based on the VTM, ECM or NNVC software codebases developed in JVET.

- For the compression efficiency test case (CP), the evaluated cases were described as:

  - ECM + 4 submissions + anchor

  - All but one submission in all test categories

  - No additional coding tools were submitted that had not been previously studied in JVET (e.g. in the ECM of NNVC studies)

- For the runtime test case (RT), schemes were targeting 0.2× to 5× VTM encoder runtimes, and the responses were described as:

  - ECM_2x + 15 submission variants + anchor

  - All submissions responded to all test categories

  - Evaluated on subset of test sequences

- For the functionality test (FC) (error concealment) test case, the responses were described as:

  - One submission was submitted with subset of test sequences

JVET document JVET-AN0361 reported visual results:

- Many test cases ~76/120 had at least one submission that had a non-overlapping confidence interval with the VVC VTM.

- Some show improved compression with substantially less encoder complexity.

- Many cases had neural network technology showing a clear visual benefit (beyond the measured PSNR benefit)

- One submission with 0.2× encoder runtime had visual benefit and ~1.4× decoder runtime.

- The test methodology used absolute category rating (ACR) with a hidden reference. There were 873 viewers, 2 131 test cases and 50 146 votes.

- Testing was generally according to ITU-R BT.500, and few outliers were found.

JVET-AN0056 (C334) On Next Generation Video Codec Call for Proposal Timing [S. Wenger (Tencent)])

The proponent suggested that the relevant groups should target a CfP issue date no earlier than the July 2026 meeting, with a CfP evaluation no earlier than the spring 2027 meeting, following the WP3/21 meeting cadence. The proponent perceived no immediate business need and expressed a desire for time for industry to clarify the direction of intended work.

A participant commented that there is competition and advocated that we should provide a competitive standard in response. Others said they did not see urgency and readiness for the type of content that is highest priority.

It was commented that July 2026 vs April 2027 seem to be the two primary requested timelines for CfP response deadlines, and it was suggested that January 2027 is a reasonable compromise time. Not proceeding too quickly was said to be justified by the need to study template methods and neural network-based methods, which need time for study in preparation of a good CfP proposal.

JVET-AN0204 AHG18: Request on inclusion ultra-low latency and packet loss resilience category into CfP [S. Ikonin, X. Ma, E. Alshina (Huawei), S. Wenger (Tencent)]

This contribution suggests to include ultra-low latency and packet loss resilience as a specific category for the future Call for Proposals

Considering the requirements mentioned in sections 6 “End-to-end delay” and 9 “Packet loss resilience” of Annex B of the April 2025 JVET and Q6/21 meeting report (TD 142/WP3), also issued as MPEG WG 2/N 448 “Draft of use cases and requirements for potential next-generation video coding standard beyond VVC capability”, and progress of JVET AHG18 work in part of test conditions design, simulation platform preparation, and evidence generation, the proponents suggested to include ultra-low latency and packet loss resilience as a specific category of a Call for Proposals document.

Based on contributions JVET-AL0051 and JVET-AN0079, a marked-up version of JVET-AM2026 “Joint Call for Evidence on video compression with capability beyond VVC” covering ultra-low latency and packet loss resilience was provided, adding the following:

- Test cases related to ultra-low latency and packet loss resilience

The subset of categories listed in section 2 are considered as test cases. Namely:

- Gaming LB HD

- SDR LB HD

- SDR RA UHD/4k coded in LDB configuration

Submitters are strongly encouraged (but not required) to submit results for all test cases. However, submitters are required to provide results for all sequences in a given test case.

  - Test cases

Two main scenarios are considered for evaluation.

- Unicast scenario. Feedback channel is available with feedback time well above 10ms . The encoder is allowed to adjust the reference picture list based on the feedback.

- Broadcast scenario. No feedback channel except maybe for statistics; feedback channel delay in the seconds range.

- Clean channel performance verification. To verify BD-rate overhead of assessed methods the clean channel performance should be reported.

During the unicast and broadcast test the reconstruction quality is evaluated against different latency restrictions, e.g.: 16, 24, 32 and 50ms.

  - Network channel model

Network channel model emulates the transmission and obtains packet losses and delays based on pre-recorded packet trace from NS-3 network simulator. The trace files are available in AHG18 simulation software and contribution JVET-AN0079.

For every sequence and bitrate the corresponding bandwidth point is specified to approximately match the network channel capacity to the average bitrate of the coded sequence. The bandwidth point consists of packet trace file and start-line are specified, same way as for example in JVET-AN0079.

Reconstruction performance is evaluated in different last mile connection types:

- Wi-Fi connection. The Wi-Fi connection emulating office or home usage scenario

- 5G connection. The 5G connection emulating office or home usage scenario using.

Reconstruction performance is evaluated in regular channel model and channel model with transmission priorities.

- Regular transmission model. Packets of every frame are transmitted one-by-one (without any priority mechanism) and retransmitted during their lifetime. Packet lifetime is a common mechanism used in Wi-Fi and 5G to control retransmission logic. The sending buffer of the router keeps each specific packet in the buffer until lifetime is over. During this time router keeps retransmitting the packet in case of transmission fail. The packet lifetime is set equal to default value of 512 ms (as in Wi-Fi and 5G).

- Transmission priority model. Modern transmission protocols have various priority mechanisms (JVET-AM0188, JVET-AM0201). Such mechanisms may be very useful for ULL video transmission scenarios. An approximated transmission priority model is used. The model operates by traffic ID (from 0 to 7) which may correspond to different layers, NAL unit types, data partitions, or any other categories depending on information importance.

  - Anchors

- Single layer VTM. Regular VTM with regular LDB configuration is used.

  - Reported Metrics

Proposals on other metrics suitable for error resilient decoding are highly encouraged.

  - Software

The simulation software of AGH18 located at: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM. is used for latency constrained transmission emulation and anchors generation.

It was commented that there has been active work in this area, but JVET had not yet looked into all of the results of the CfE results in that area. The artefacts experienced in such scenarios are more difficult to evaluate, and it was mentioned that using traditional methods with short test sequence viewing may not be appropriate when such phenomena as frame freezes can occur.

JVET-AN0330 Proposed modification of CfE test conditions E. [Alshina (Huawei)] [late]

This (late) contribution proposed revising CfE test conditions with more challenging sequences and updated parameters to better align NGVC with industry needs, despite delaying standardization.

During CfE response preparation, the proponent said (based on JVET-AN0052, which discusses GOP-based RPR, reporting a 5-8% gain overall and more in particular cases, with runtime reductions for encoding and decoding) that for two-thirds of test cases under CfE test conditions, a substantial compression gain with visual quality benefits as well as encoder and decoder speed-up can be demonstrated without changing the standard, using reference software. The proponent asserted that the Call for Evidence cannot be conclusive, since it was asserted that the anchor was significantly penalized for the majority of test cases.

It was commented that it was well known that variable resolution has an effect, and that the CfE and prior CfPs have discussed this issue, and it was noted that proponents had provided information about their use of reference picture reasampling (RPR, as had been requested in the CfE). Some proposals that did not use RPR did show substantial benefits. It had been planned to make some provisions about this for the CfP.

The remaining one-third of test cases in CfE test conditions (for example, all gaming sequences) were said to be more challenging for coding and could be used for demonstrating the evidence of needs and possibilities for the next generation video codec.

The proponent suggested to modify the CfE test conditions, including more challenging sequences, enabling all methods of using the current video coding standard which improve compression, select new target rates and conduct another CfE. They suggested having a call for test materials issued to speed up collecting video data reflecting modern use cases.

The proposed modification of test conditions might delay the start of a next generation video coding standardization project, but the proponent said this would make the next generation video codec better aligned with industry requirements. The proponent asserted that a next-generation video coding standard should be defined by business requirements of different industry players. The proponent further said that developments in JVET should focus not only on improving compression efficiency but also on providing benefits with constrained complexity, and enabling functionalities such as ultra-low delay and error resilience. The first attempt of establishing testing with a constrained encoder was done as part of the prior CfE. The proponent said this attempt was successful and encouraged JVET to continue NGVC development with limited encoder complexity.

No other participants supported the idea of re-running the CfE.

#### General discussion of the CfE and next steps was held in the joint meeting. Historically we have typically had 3 years for the “collaborative phase” of developing a first version of a video coding standard, and typically we do not identify some “clear winner” from a CfP.

It was commented that we need to improve the availability of UHD test content.

It was commented that July 2026 and April 2027 seemed to be the two primary requested timelines for CfP response deadlines, and it was suggested that January 2027 is a reasonable compromise time. Not proceeding too quickly was said to be justified by the need to study template methods and neural network-based methods, which need time for study in preparation of a good CfP proposal.

It was commented that we should take action to respond to the fact that evidence of technology advances was shown in the CfE.

A three-year collaborative phase seemed generally agreed among participants as the planned horizon. After discussion, planning to have the CfP response evaluation in January 2027 was agreed. The submission deadline for bitstreams would be early November 2026.

It was further agreed to plan to issue a first draft CfP in January 2026, and for a final CfP to be issued in July 2026 with a CfP response evaluation in January 2027.

## BoGs (1)

JVET-AN0364 BoG report on Tool Complexity Analysis [X. Li, Y. Zhao]

Agenda

- Review documents of AHG7: ECM tool assessment (4.8)

- Potential extension to encoder complexity

- Structure/content of an output document

AHG7: ECM tool assessment (6)

JVET-AN0043 AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 [X. Li (Google)]

Per the suggestion on performing the study on detailed criteria of tool analysis during the JVET-AM meeting, a preliminary tool analysis is conducted in this contribution. It is emphasized that the contribution is to provide examples of using the criteria in JVET-AM0042 rather than analyzing certain tools.

No need for presentation

JVET-AN0044 Report of AHG7 conference call on Complexity Analysis [X. Li]

This document contains the report of the AHG7 conference call between 39th and 40th JVET meeting. The call was on August 19, 2025 Tuesday UTC 14:00-15:29.

Following items were recommended:

- Use the tables in 2.2.1 as the starting points of complexity analysis. Note that additional input from experts is needed for some items.

- Produce an output document on the criteria of the complexity analysis in the 40th JVET meeting

- Document how the complexity of a tool could be scaled at encoder and decoder

To report to JVET

JVET-AN0119 AHG7: Analysis on group-off tests using Valgrind [R. Ishimoto, F. Zheming, T. Ikai (Sharp)]

This contribution provides evaluation results of external memory access concerning the group-off tests of ECM-18.0. The measurements were conducted using Valgrind, focusing on instruction reference (Ir) and LL cache data read misses (DLmr) across the entire decoding process in class B, C, and D of RA under the CTC. This allows us to assess the complexity of each group-off test from the perspective of instructions and external memory access.

For different configurations and different complexity aspects (instruction/cache miss), different groups shows different complexity balance.

It was commented that performance counter also provides cache miss info but is much faster.

It was commented that using valgrind to decode may be very slow.

Further study is encouraged.

JVET-AN0203 AHG7: On RGPM moved into group-2 [P. Bordes, K. Reuzé, E. François, K. Naser, F. Le Léannec (InterDigital)]

This contribution reports results of putting Regression-based Geometric Partition Mode (RGPM) in the group-2 coding tool list of AHG-7, instead of group-1. To do so, when TM tools are disabled (TMToolsEnableFlag is false), then RGPM uses TM for deriving the blending matrices only, but not for candidates re-ordering nor MV refinement.

The tests reportedly provide {Y, U, V} BD-reduction in RA with group-1-off as follows:

RA: -0.xx%, -0.xx%, -0.xx%, xx.x% (EncT), xx.x% (DecT)

It was reported that the control of RGPM in ECM leads to an improper grouping RGPM in AHG7 tool off tests.

ECM changes (normative) were proposed to allow test RGPM in group2 which looks more appropriate for RGPM.

An expert supported having separate controls to have more accurate tests.

It was commented normative changes to ECM are beyond the mandate of AHG7.

The crosschecker of the proposal supported the proposal.

It is recommended to report back to JVET and request a decision. If the proposal is agreed, RGPM should be tested in group 2.

JVET-AN0223 AHG7: On bin to bit ratio in ECM [T. N. Canh, P. Yin, S. McCarthy (Dolby), J. N. Shingala (Ittiam)] [late]

This contribution analyzes CABAC bin statistics across ECM 18.0 and G1 to G5 group tools off tests. The study reports unweighted and weighted peak bin to bit ration (bin2bit), and CABAC/EP bin ratio for AI/RA configuration. Additionally, the frequency of context switching is studied.

Different groups show different ratios. Transform tools may have more impact on the ratios. The throughput of group2 seems the worst. Group2 also shows higher context switching frequency (the frequency is calculated by checking whether the current bin and the previous bin have the same context).

ECM uses much more contexts than VTM.

Propose to report information bin2bit ratio in CfP response.

It was commented that more contexts are because more models/slice types.

It was commented that some contexts may not be used and it may be helpful to report this information.

It was commented that group level report may not be helpful. It was further commented that the overall comparison between ECM and VTM may be more interested. It was replied that such information was provided last meeting in JVET-AM0227

It was commented that context switching information may be useful when analyzing some tools.

It was commented that the ECM software still crashes for AGH7 group 2 and 4 off tests when bin2bit ratio is calculated. It was suggested to file an ECM software issue.

It was commented that the context information is helpful in CfP evaluation. It was further commented that the context switching frequency statistics collection may be too much burden for now.

It was asked how to use bin2bit. It was replied that the bin2bit ratio indicate CABAC throughput.

It was commented AVC has a level constraint on bin2bit ratio.

It was asked what is the relationship between context switching frequency and bit2bin ratio. It was commented that context switching frequency does not correlate the throughput much. It was replied that it may have connection.

It was commented that if too many contexts are used, the context switching may impact the throughput.

It was commented that reporting number of contexts makes more sense.

Recommendation: To report number of contexts (memory needed for the contexts).

JVET-AN0253 AHG7: On Memory Bandwidth Measurement [X. Li (Google), Y. Kim (Samsung), J. Pardo (Huawei), R. Ishimoto (Sharp), L.-F. Chen (Tencent)]

This is a joint study on memory bandwidth measurement. It is reported that the memory bandwidth measurement method JVET_J0090_MEMORY_BANDWITH_MEASURE in the VTM software provides more reasonable results than Valgrind (Cachegrind) in the context of measuring the memory bandwidth consumption of the motion compensation module in the VTM software.

It is anticipated that LDB case has more chances than LDP in cache miss. However, the results from Valgrind are contradictory to this anticipation (i.e., by the last level cache miss results reported by Valgrind). J0090 method seems more reliable.

Further work on testing with a commercial memory bandwidth consumption tool is encouraged, to verify the consistency with the analysis in this proposal (especially J0090).

An expert commented that it may not be needed to report memory bandwidth data for CfP submissions. Better to analyze such data in future CEs.

An expert suggested that such analysis could be extended to other tools. The proponent suggests considering the extension after confirming the correctness of data by hardware company.

An expert commented that the J0090 method may not be accurate, according to the experience in VVC standardization period.

It is commented that stress test in LDP and LDB may be helpful. It is commented the worst-case data may be more relevant than the CTC average data (e.g., by forcing small blocks).

Offline discussion with hardware experts is encouraged.

Recommendation: tentatively use the J0090 method for memory bandwidth measurement in AhG 7 study.

JVET-AN0317 AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 – Update on IntraTMP [K. Naser, T. Dumas, M. Radosavljevic (InterDigital)] [late]

In JVET-AN0043, preliminary evaluation of multiple coding tools is provided. It is asserted that the evaluation of IntraTMP is based on early version of it. This contribution provides an update of IntraTMP evaluation using the latest version of ECM.

This document provides updated information on the exemplar tool analysis for IntraTMP.

It is reported that IntraTMP has unexpected behaviour with Bvg-EIP and Bvg-DIMD. It is not a bug affecting ECM performance, but may affect the tool off test. It was suggested to file an ECM software issue.

It was asked about the detailed number of template matching. Further study on the details is encouraged.

The proponent of the Bvg-EIP and Bvg-DIMD confirmed that the design intention is to group these two tools with intraTMP in the AHG7 test.

Potential extension to encoder complexity

Discussed additional items related to hardware encoding complexity to those in table 1.3 (hardware perspective) in JVET-AM0042.

- Coding tree decision complexity: MTT coding tree depth and maximum number of split modes tested in encoding for any coding tree node

  - It was asked about why MTT depth is to be collected. Higher MTT depth generally corresponds to more encoding resource / cycles for encoding a coding tree node. It was commented that total number of RDO of a coding tree would be better, and some technology may not use MTT.

- Coding unit mode decision complexity: Maximum number of RDO checks in CU size ranges

  - Several CU size ranges are suggested: cuW * cuH ∈ [1,16], (16, 64], (64, 256], (256, 1024], and above 1024, since smaller CUs has more tight cycle budget and often implemented with smaller number of RDO checks than bigger CUs.

  - It was commented number of rd check is useful. It was suggested that reporting separate numbers for inter and intra modes could be more informative.

  - It was commented the dependency RD number should be also counted, which impacts the parallel processing

- RDO rate estimation complexity: VTM-like precious rate estimation that based on probability in CABAC context models or a proxy function that does not access CABAC context models

  - It was asked the rate estimation details. Detailed description on the proxy function may not be needed to report.

  - It was commented that hardware encoder may not be capable to do precise rate estimation like VTM.

Further study on hardware encoding complexity perspectives is needed. It is encouraged to submit new proposals related to hardware encoding complexity dimensions and measurements in the next JVET meeting.

Add Mandates to AHG

- Rd number counting in a software patch

  - It was commented that the definition of RDO shall be clarified. Several experts suggested to count the forward transform number as RDO number. Some tools which conduct multiple times of inverse transforms are considered already reflected in decoding complexity measurement.

  - It was commented that normally hardware may not have early stop.

- Function interface for rate estimation

  - Simpler rate estimation methods are encouraged to be development.

Structure/Content of an output document

- JVET-H1003 Description of video coding technology proposal by XYZ

  - Complexity analysis

    - Encoding time and measurement methodology

    - Decoding time and measurement methodology and comparison vs. anchor bitstreams decoded by HM16.16

    - Description of computing platform used to determine encoding and decoding times reported in sections 4.1 and 4.2

    - Expected memory usage of encoder

    - Expected memory usage of decoder

    - Complexity characteristics of encoder motion estimation and motion segmentation selection

    - Complexity characteristics of decoder motion compensation

    - Complexity characteristics of encoder intra-frame prediction type selection

    - Complexity characteristics of decoder intra-frame prediction operation

    - Complexity characteristics of encoder transforms and transform type selection

    - Complexity characteristics of decoder inverse transform operation

    - Complexity characteristics of encoder quantization and quantization type selection

    - Complexity characteristics of decoder inverse quantization

    - Complexity characteristics of encoder side motion data refinement

    - Complexity characteristics of decoder side motion data refinement

    - Complexity characteristics of encoder in-loop filtering type selection

    - Complexity characteristics of decoder in-loop filtering operation

    - Complexity characteristics of encoder entropy coding type selection

    - Complexity characteristics of decoder entropy decoding operation

    - Complexity characteristics of encoder quantization and quantization type selection

    - Complexity characteristics of 360° video specific tools (if any)

    - Complexity characteristics of HDR specific tools (if any)

    - Degree of capability for encoder parallel processing

    - Degree of capability for decoder parallel processing

- Recommendation: Description/Analysis of major tools using the table in JVET-AN0044

  - Coding tools with gains above a threshold

  - Encoding only methods with gains or runtime saving above a threshold

  - Decoding simplification methods with runtime/resources saving above a threshold

Recommendations

Document review

- Review JVET-AN0203 and make the decision on the normative ECM software changes. Two individual supports were recorded in the BoG (see decision below)

- JVET-AN0223: Report number of contexts and the memory needed for the context storage (agreed in JVET)

- JVET-AN0253: Tentatively use the J0090 method for memory bandwidth measurement in AhG 7 study (agreed in JVET)

Potential extension to encoder complexity

Structure/Content of an output document

- Use the table in JVET-AN0044 to report tool complexity for the following cases

  - Coding tools with gains above a threshold

  - Encoding only methods with gains or runtime saving above a threshold

  - Decoding simplification methods with runtime/resources saving above a threshold

Issue an output document “Draft of complexity reporting template for coding algorithms and tools”

X. Li was asked to draft a first version on the basis of JVET-AN0044, JVET-H1003, and JVET-AN0364, to be further discussed in BoG or JVET later this week. See further notes under JVET-AN0378.

It is noted that only part of the aspects collected might be included in a questionnaire to be submitted with CfP responses, and other aspects might be used later in CEs.

About JVET-AN0203: There is no change in performance in CTC, but for proper grouping of RGPM in group 2 instead of group 1, a change of low-level conditions is necessary.

Decision: Adopt JVET-AN0203.

## Liaison communications (1)

A late incoming liaison letter from JPEG (SC 29/WG 1) was received as m74904 / WG 1 N 101272 / SC 29 N 22963 (registered in the MPEG DMS on 10 October), and JPEG had sent a similar letter to SG21 as WG 1 N 101271 / SC 29 N 22962 / SG21-TD230/Gen. It was planned to send a response.

A draft reply was reviewed in JVET on Sunday 12 Oct. during 1300 – 1345. This was intended to be conveyed by a parent body, but it was not registered as a WG 5 output document or as an SG21 document, so it was not ultimately conveyed as the planned output, and the reply reporting will need to be resolved at the January 2026 meeting.

(The following text is kept as a template for future use)

The liaison document WG 5 N XXX was reviewed in JVET on XXday XX July at XXXX-XXXX. The draft reply was also presented in the MPEG AG 3 Communication meeting Thursday 03 July during 1500-1800.

# Project planning

## Software timeline

ECM 19.0 software (including all adoptions) was planned to be available 3 weeks after the meeting (3 Nov.).

The NNVC 15.0 codebase software was planned to be available 3 weeks after the meeting (3 Nov. 3), including all elements needed for CTC and EE1. Additional integration and harmonization with VTM may be deferred for version 15.1.

VTM23.13 software was recently released. Additional versions will be released as appropriate (e.g. for integration and updates of SEI messages included in JVET-AN1019 and JVET-AN2001).

Updates on top of HM18.0 and JM19.1 software will be released as appropriate (e.g., integration and updates of SEI messages included in JVET-AN1018 and JVET-AN1017).

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning

An EE on neural network-based video coding was established, as recorded in output document JVET-AN2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AN2024.

Initial versions of these documents were presented and approved (see section 10).

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). In cases where combinations of different elements are planned to be tested, mutual cross-checking of the individual elements by other parties of the combination is discouraged. The combination must be cross-checked by an independent party. The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-T2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow was previously described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

However, it was noted that the link doesn’t seem to exist anymore.

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs had been asked to send draft mandates to JRO before 0830 on 11 Oct., preferably copy from the table below and sending with changemarks or yellow highlight of changes.

Review of AHG plans was conducted during the plenary on Saturday 11 Oct. 2025 at 0910–1035.

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents. - Conduct consultation with parent bodies on future JVET management structures. | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AN1016, JVET-AN1017, JVET-AN1018, JVET-AN1019, and JVET-AN2001). - Collect reports of errata for VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports. - Coordinate with AHG3 to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target conducting tests for VVC multi-layer features, and update the test plan accordingly. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing, as well as studying objective metrics in that context. - Coordinate with AHG17 on investigating sequences and making arrangements for viewing at the next meeting. - Coordinate with AHG18 on investigating visual impact of data losses. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study DIS of VVC conformance 3rd edition and suggest improvements to JVET-AL2028, as appropriate. - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and further develop related conformance bitstreams. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-19.0 software version (and potential updates), corresponding VTM anchor, and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| Tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Study JVET-AN2040 and suggest improvements. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing. - Prepare reporting of tool assessment results. - Develop methodology of more reliable runtime measurement. - Collaborate with AHG19 on encoding complexity assessment. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development and experiments on optimization of encoders and receiving systems for machine analysis of coded video content, including combinations of proposed technologies. - Maintain the software implementation examples and develop tool combination examples in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose potential improvements to JVET-AL2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potential of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Investigate the impact of using different machine task models in the evaluation of the compression performance of tools optimized for machine analysis tasks. | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AN1017, identify any issues and propose solutions as appropriate. - Study JVET-AN2032, including study of SEI messages with different options, when those are present, and propose improvements. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages. - Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context. - Study the alignment of the same SEI messages in different standards. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. | J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3, AHG6, and AHG17. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Study potential improvements of the NNVC CTC document JVET-AJ2016. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. Promote the call for training materials, distribute it, and actively communicate with content owners. - Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology. - Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also investigate bit-exact reproducibility of NN-based methods on various platforms in coordination with AhG14. - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. - Coordinate with AHG14 on items related to NNVC software development, in particular, evaluate the interface in the NNVC for end-to-end optimized AI coded reference pictures. | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) | Y (tel., 2 weeks notice), first on Nov. 18, second on December 16 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM19 algorithm description JVET-AN2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Establish a repository to make other codebases that implement enhanced compression available to interested JVET members. - Support AHG6 in generating anchors according to the test conditions in JVET-AI2017. - Analyse the results of exploration experiments described in JVET-AN2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss and propose refinements to the draft of the TR 2nd ed. JVET-AN2020. - Study alternative film grain models and their associated documentation. - Discuss and enumerate updates, improvements, and additions for the second edition of the technical report. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the plan for subjective quality testing of the FGC SEI message JVET-AJ2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-15.0 software version (and potential updates), based on updated VTM with adopted contributions and hybrid framework, and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AJ2016. Study the impact of the addition of new dataset on the already integrated models. - Continue to bridge the gap between NNVC and most recent VTM as necessary. - Continue to develop missing functionalities for hybrid end-to-end framework exploration. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AN2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. | F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Nov. 18, second on Dec. 16 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AJ2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Develop and maintain software for estimation of depth maps for gaming sequences, and evaluate the effect of the estimated maps against the original depth maps on coding efficiency where they are available using established accuracy metrics - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs) | N |
| Generative face video compression (AHG16) (jvet@lists.rwth-aachen.de) - Maintain GFVC software tools, associated configuration files, and software usage documentation. - Study the extension of GFVC software capability to handle video content with higher resolutions. - Identify and study additional test content, including content with higher resolutions, suitable for use in GFVC performance evaluation. - Study GFVC performance under test conditions defined in JVET-AJ2035, as well as performance on additional test content, and wider bitrate ranges. | Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs) | N |
| Preparation of Call for Proposals (AHG17) (jvet@lists.rwth-aachen.de) - Review the set of categories, test sequences and rate points based on the results of the CfE, and propose updates for the test set to be used in the CfP. - Identify test sequences extending the represented range of applications and investigate the suitability for use in visual quality assessment. - Develop proposed text for a draft CfP document, considering JVET-H1002 and JVET-AN2026. - Coordinate the generation of VTM anchors and other bitstreams for the test cases defined in the updated test set, and make them available according to the timeline. - Prepare logistics and plans for tentatively conducting an AHG meeting after the 41st JVET meeting, and issue an announcement JVET-AN2041. | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs) | Y (tel., 2 weeks notice) |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Study JVET-AN2039 common test conditions and propose improvements as appropriate. - Investigate creation of practical simulation software based on VTM, including network transmission aspects, including consideration of network functionality that can aid timely video-coding based error correction, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. - Coordinate with AHG4 on investigating visual impact of data losses. | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Hardware implementation complexity (AHG19) (jvet@lists.rwth-aachen.de) - Investigate hardware encoding complexity constraints in typical applications (e.g., mobile devices and hardware transcoding), and identify challenges and evaluate coding tools from hardware encoding implementation perspectives. - Solicit and develop hardware encoding complexity measurements (e.g., maximum number of RDO decisions per CU / CTU), and use them to analyze the performance of test models (e.g., VTM) and coding tools. - Consider the possibility of designing, developing and maintaining a hardware-encoding-mimicking simulation framework on top of test models, e.g., constraining RDO number per CU, constraining coding tree decision complexity (e.g., maximum tree depth, per-node split modes), developing rate estimation function interface, low-complexity CU/TU-level bit estimation function, etc. - Collaborate with AHG7 on aspects of hardware encoding and decoding complexity perspectives, and in improving JVET-AN2040. | Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs) | Y (tel., 2 weeks notice) |

It was suggested to discontinue AHG8 by the time of finalization of the TR (likely April 2026), and discontinue AHG16 by the next meeting. For both AHGs, it is necessary to find a persistent home for the software.

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N 379) in order to make it easy to reference.

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N 379, as noted in section 9.

JVET-AN1000 Meeting Report of the 40th JVET Meeting [J.-R. Ohm] [WG 5 N 372] (2025-11-09)

Initial versions of the meeting notes (d0 … d9) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne, K. Sharman, R. Sjöberg, G. J. Sullivan] [WG 5 N 103]

Primary editor: C. Rosewarne

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

Remains valid – not updated: JVET-AL1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan]

Primary editor: Y.-K. Wang.

JVET-AN1005 Future CICP extensions (Draft 2) [J. Boyce, E. Thomas, A. Tourapis] (2025-10-26)

Primary editor: J. Boyce.

Including update from JVET-AN0070.

Remains valid – not updated JVET-AM1006 HEVC additional profiles and SEI messages (Draft 4) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-08-01)

Primary editor: Y.-K. Wang.

A DoC WG 5 N 375 on the DAM was reviewed and approved on Saturday 11 Oct. at 1125-1140.

It was requested in the WG 5 recommendations to integrate the amendment into a new edition, and an integrated version was issued as WG 5 N 376 FDIS ISO/IEC 23008-2 HEVC 7th ed. (with editing period 2026-01-09). The latter is based on JVET-AM1006 plus the additional modifications documented under JVET-AN1018.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Remains valid – not updated: JVET-AM1008 Conformance testing for HEVC multiview extended and monochrome profiles [I. Moccagatta, T. Fu, S. Paluri, A. Tourapis]

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

Remains valid – not updated: JVET-AJ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien]

Update of bug tracking system description expected in next meeting.

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

JVET-AN1016 AVC with extensions and corrections (Draft 4) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-01-09)

Primary editor: K. Sühring.

To prepare the next integrated edition of H.264, including changes from JVET-AN1017.

JVET-AN1017 Support for additional SEI messages in AVC (Draft 4) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2025-10-26)

Primary editor: K. Sühring

Changes are editorial improvements, interface changes, constraints, and bug fixes agreed at this meeting, including elements from:

- JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces

- JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text

- JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4

- JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft : add the derivation of CroppedWidth and CroppedHeight for the AVC interface.

- JVET-AN0363 Interface text for SEI message extensions [J. Boyce (Nokia), Y. Sanchez, K. Sühring (Fraunhofer HHI)]

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-10-17)

Primary editor: Y.-K. Wang.

Changes are editorial improvements, interface changes, constraints, and bug fixes agreed at this meeting, including elements from:

- JVET-AN0071 AHG9: On HEVC Omnidirectional viewport SEI in HEVC

- JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text

- JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4

- Align the HEVC NNPF interface text with VVC, by integrating adopted VVC NNPF interface text changes from the JVET-AK, JVET-AL, and JVET-AM meetings.

- JVET-AN0363 Interface text for SEI message extensions [J. Boyce (Nokia), Y. Sanchez, K. Sühring (Fraunhofer HHI)]

- JVET-AN0183 AHG9: On PRI SEI message in VSEI v4 draft

- JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft

- JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces

- JVET-AN0072 AHG9: On 3DRD, MAI and Alternative Depth information SEI messages in HEVC and VSEI

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

This was submitted to ITU consent on H.265 11th ed.

JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-10-17)

Primary editor: J. Boyce.

Changes include mostly small editorial improvements and responses to ballot comments: additions of constraints, clarifications, interface changes, updating of reference handling, and bug fixes. No new features were added. This includes elements from:

Editorial

- JVET-AN0072 AHG9: On 3DRD, MAI and Alternative Depth information SEI messages in HEVC and VSEI

- JVET-AN0133 AHG9: Miscellaneous editorial changes for the VSEI v4 draft

- JVET-AN0186 AHG9: Editorial updates for VSEI v4

Multiple SEI messages

- Syntax descriptors and functions

- JVET-AN0134 AHG9: On the SEI message payload extension in VSEI v4

- EOI

- Response to ballot comment US-027

IFM

- JVET-AN0368 AHG9: On semantics of ifm_type_id for IFM SEI

NNPF

- JVET-AN0067 AHG9: Comments on Neural-network Post-filter in VSEI V4 and V3

- JVET-AN0058 AHG9: On the NNPF SEI messages in VSEI version 4

- JVET-AN0063 AHG9: On the NNPF SEI messages in VSEI v4

- JVET-AN0140 AHG9: On the neural-network post-filter characteristics SEI message in VSEI v4

- JVET-AN0342 AHG9: On the neural-network post-filter activation SEI message in VSEI v4

FGC

- JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4

SEI processing order (SPO) and processing order nesting (PON)

- JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text

- JVET-AN0065 AHG9: On the SPO and PON SEI message in VSEI v4

- JVET-AN0181 AHG9: On process SEI messages in processing chain in SPO SEI message

Object mask information (OMI)

- JVET-AN0060 AHG9: On the OMI SEI message in VSEI version 4

- JVET-AN0135 AHG9: On the object mask information SEI message in VSEI v4

- JVET-AN0161 AHG9: On OMI SEI message

AI usage restrictions request (AURR)

- JVET-AN0062 AHG9: On the AI usage restrictions SEI message in VSEI version 4

- JVET-AN0136 AHG9: On the AI usage restrictions SEI message in VSEI v4

Digitally signed content (DSC)

- JVET-AN0182 AHG9: On DSC SEI messages in VSEI v4 draft

Packed regions info (PRI)

- JVET-AN0180 AHG9: On PRI SEI message in processing chain

- JVET-AN0367 AHG9: On design of PRI SEI message

Modality information (MI)

- JVET-AN0073 AHG9: On MI SEI message in VSEI version 4

- JVET-AN0138 AHG9: On the modality information SEI message in VSEI v4

Text descriptions info (TDI)

- JVET-AN0061 AHG9: On the text description information SEI message in VSEI version 4

Generative face video (GFV)

- JVET-AN0141 AHG9: On the generative face video SEI message in VSEI v4 [J. Xu, Y.-K. Wang

- JVET-AN0156 AHG9: Text fixes and cleanup for GFV and GFE SEI messages

Image format medata (IFM)

- JVET-AN0137 AHG9: On the image format metadata SEI message in VSEI v4

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

This was submitted to ITU consent on H.274 4th ed.

No output: JVET-Axx1020 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

Links to test sequences need to be updated due to the change of the content server.

JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan] (2025-10-17)

Primary editor: Y.-K. Wang.

Changes are editorial improvements, interface changes, constraints, and bug fixes agreed at this meeting, including elements from:

- JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text

- JVET-AN0363 Interface text for SEI message extensions [J. Boyce (Nokia), Y. Sanchez, K. Sühring (Fraunhofer HHI)]

- JVET-AN0161 AHG9: On OMI SEI message

- JVET-AN0183 AHG9: On PRI SEI message in VSEI v4 draft

- JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft

- JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

This was submitted to ITU consent on H.266 4th ed.

Remains valid – not updated: JVET-AH2002 Algorithm description for Versatile Video Coding and Test Model 22 (VTM 22) [Y. Ye, A. Browne, S. Kim] [WG 5 N 284]

Primary editor: Y. Ye.

New elements from notes elsewhere in this report (kept for future use):

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was suggested that editorial improvements submitted as input to the next meeting would be welcome.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated: JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] (2025-08-01)

Primary editor: G. J. Sullivan.

A DoC WG 5 N 377 on the DAM was reviewed and approved on Saturday 11 Oct. at 1140-1155.

It was requested in the WG 5 recommendations to integrate the amendment into a new edition, and an integrated version was issued as WG 5 N 378 FDIS ISO/IEC 23090-3 VVC 4th ed. (with editing period 2026-01-09). The latter is based on JVET-AM2005 plus the additional modifications documented under JVET-AN2001.

Remains valid – not updated: JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-08-01)

Primary editor: J. Boyce.

A DoC WG 5 N 373 on the DAM was reviewed and approved on Saturday 11 Oct. at 1210-1245 (with some finalization in HLS track thereafter, which included a name change of AI usage restrictions later agreed by JVET, and otherwise purely editorial issues).

It was commented that for referencing some external specs in DSC, it will be necessary to fill form A.5 for justification in the ITU-T submission.

It was requested in the WG 5 recommendations to integrate the amendment into a new edition, and an integrated version was issued as WG 5 N 374 FDIS ISO/IEC 23002-7 VSEI 4th ed. (with editing period 2025-09-01). The latter is based on JVET-AM2006 plus the additional modifications documented under JVET-AN1019.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

JVET-AN2008 Conformance testing for versatile video coding (Draft 8) [I. Moccagatta, F. Bossen, S. Iwamura, P. de Lagrange]

This integrates the additional bitstreams from JVET-AL2028, and corrected bitstreams according to JVET-AM0005.

This was submitted to ITU consent on H.266.1 3rd ed.

An equivalent version is currently under ballot as ISO/IEC DIS 23090-15 3rd ed.

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

A request for next edition(s) of reference software(s) could be issued in the next meeting, and also CD(s) could be produced by the same meeting, as SC 29 is meeting beginning of February and could approve such projects without issuing a ballot. Issuing CD(s) by the January meeting (if mature enough) could avoid that an ISO version would lag behind an ITU version which might be submitted for consent in July 2026. Another consideration could be to start the process for ISO early enough in 2026, such that FDIS and ITU consent could be synchronized in April 2027.

Remains valid – not updated: JVET-AL2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

Add alternative configuration from JVET-AL0055 and clarification about intra picture period (see discussion under JVET-AL0114). Also links to test sequences are updated due to the change of the content server.

Remains valid – not updated: JVET-AC2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall]

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

Links to test sequences need to be updated due to the change of the content server.

It was also commented that it might be good to add conditions relating to HDR, even though currently no HDR tests are conducted in CTC.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AN2019 Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC) [F. Galpin, Yue Li, Yun Li, D. Rusanovskyy, T. Shao, J. Ström, L. Wang] (2026-01-09)

New elements in text and software from notes elsewhere in this report:

- JVET-AN0153 test EE1-2.2.

- JVET-AN0195 EE1-3.5 (re-training the current DRF model).

- JVET-AN0054 (non-CTC). Also needs update of the description in JVET-AN2019.

- JVET-AN0209 (non-CTC). Software coordinators were to investigate appropriateness by time of merge request.

- JVET-AN0215 (non-CTC). Also needs description in JVET-AN2019.

- JVET-AN0225 (non-CTC). LMCS as configurable option in NNVC multi-layer interface for external base layer.

- JVET-AN0047 (SW) include script for environment information collection in NNVC CTC repository (usage not mandatory).

- For changes in SADL, see section 5.1.4.

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4) [W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis] (2026-01-09)

Update of analysis algorithm from JVET-AN0237.

JVET-AN2021 Verification test plan for VVC multilayer coding (update 7) [O. Chubach, P. de Lagrange, M. Wien] (2025-11-14)

Developed from JVET-AN0298.

Remains valid – not updated: JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4) [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien]

Number might be re-used (activity discontinued) as there has been no activity for 3 meeting cycles.

JVET-AN2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)] (2025-10-26)

An initial draft of this document was reviewed and approved at 1405-1415 on Sunday 12 October.

This round of EE1 tests includes:

EE1-1: LOP in-loop filter

  - EE1-1: LOP with Overlapped Feature Integration  JVET-AN0127 (UESTC, Transsion)

    - EE1-1.1.1 – only training modification, filter architecture LOP6

    - EE1-1.1.2 – training modification and proposed LOP architecture

  - EE1-1.2 –Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels  JVET-AN0083 (

    - EE1-1.2.1 Only over-parametrization aspect

    - EE1-1.2.2 potential combination with EE1-1.1

EE1-2: VLOP in-loop filter

  - EE1-2.1 – VLOP with new backbone block based on Spatial-Channel Mixing JVET-AN0255

EE1-3: Normative TDO methods

  - EE1-3.1 Normative TDO method as an alternative option in NNVC JVET-AN0172

    - EE1-3.1.1, the proposed normative TDO for VLOP, LOP and HOP will be tested.

    - EE1-3.1.2, the proposed normative TDO for VLOP, LOP and HOP will be tested with sub-tests using the modified TDO lambda sets.

  - EE1-3.2 Neural network in-loop filter usage based on activation map JVET-AN0340

    - EE1-3.2.1, study the proposed normative activation map usage for NNLF

    - EE1-3.2.2, study syntax modifications of EE1-3.2.1.

EE1-4: NN-Inter

  - EE1-4.1 – Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AN0193

  - EE1-4.2 FlowWarp Operator for DRF Integer Inference Optimization JVET-AN0319

EE1-5 NN-based super-resolution complexity reduction

  - EE1-5.1 NNSR with new backbone block based on Spatial-Channel Mixing (SCM) JVET-AN0201.

EE1-6 Framework for externally coded pictures

  - EE1-6.1 Multi-layer framework (3 sub-tests)

  - EE1-6.2 Single-layer-like framework (3 sub-tests)

JVET-AN2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)] (2025-11-10)

An initial draft of this document was reviewed and approved at 1415-1420 on Sunday 12 October.

This round of EE2 tests will include:

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Reference sample generation for intra prediction | T. N. Canh (Dolby) | N. Neumann (Nokia) |
| 1.2 | Combined angular- and gradient-PDPC | G. Kulupana (Nokia) |  |
| 1.3 | Block prediction using the 8-tap cubic interpolation filter in SGPM | J. Lee (KAU) J. Lee (ETRI) | Y. Bai (ZTE) |
| 1.4 | Position-dependent weighted prediction extension | Y. Wang (ZTE) | S. Blasi (Nokia) |
| 1.5a | Test the threshold for block size larger than 32 | S.Wan (NWPU) S. Xie (ZTE) |  |
| 1.5b | No reference sample smoothing for all block sizes | S.Wan (NWPU) S. Xie (ZTE) |  |
| 1.6a | Adding DIMD and MDIP average mode to MPM | W. Niu (ZTE) |  |
| 1.6b | MDIP mode modification | W. Niu (ZTE) |  |
| 1.6c | Test 1.6a + Test 1.6b | W. Niu (ZTE) |  |
| 1.7 | Multiple candidate selection for MDIP | L. Wang (Xidian Univ) |  |
| 1.8 | IntraTMP with DMVR | K. Naser (InterDigital) |  |
| 1.9 | TMRL angle offset refinement | G. Rath (InterDigital) | D. Ruiz (Ofinno) |
| 1.10 | CCCM with clipping | L.-C. Xu (SYSU) |  |
| 2 Inter prediction |  |  |  |
| 2.1 | GPM without blending for screen content | J. Kim (WILUS) |  |
| 2.2 | On improving adaptive GPM blending | Z. Zhang (Transsion) |  |
| 2.3a | Sub-block merge mode availability extension with enlarged block set and without CU size limitation | Y. Liu (Transsion) | F. Wang (OPPO) |
| 2.3b | Sub-block merge mode availability extension with enlarged block set and CU size limitation | Y. Liu (Transsion) | F. Wang (OPPO) |
| 2.4a | Additional spatial adjacent candidates | C. Wang (OPPO) |  |
| 2.4b | Modified temporal collocated candidates | C. Wang (OPPO) |  |
| 2.4c | Number of pairwise-average candidates | C. Wang (OPPO) |  |
| 2.4d | Test 2.4a + Test 2.4b + Test 2.4c | C. Wang (OPPO) |  |
| 2.5 | Pairwise merge candidates with motion vector scaling | S. Hong (Nokia) |  |
| 3 Transform and coefficients coding |  |  |  |
| 3.1 | Finding optimal simple quantizer borders and reconstruction points | M. Balcilar (Ofinno) M. Le Pendu (InterDigital) |  |
| 3.2a | Modified coding of a coefficient level in TSRC | R. Xu (Xidian University) Y. Yu (OPPO) |  |
| 3.2b | Test 3.2a with low QPs | R. Xu (Xidian University) Y. Yu (OPPO) |  |
| 3.2c | Test 3.2b with JVET-AN0086 encoding change enabled | R. Xu (Xidian University) Y. Yu (OPPO) |  |
| 3.3a | Modified multiple transform set selection for inter LFNST/NSPT | J. Huo (Xidian University) |  |
| 3.3b | Modified Advanced SBT | J. Huo (Xidian University) |  |
| 3.3c | Test 3.3a + Test 3.3b | J. Huo (Xidian University) |  |
| 4 In-loop filtering |  |  |  |
| 4.1 | On improving the bi-filtering modes in TALF | P. Astola (Nokia) |  |
| 4.2 | ALF coefficient coding group extension | I. Jumakulyyev (Nokia) |  |
| 4.3 | Modified merge candidate list in ALF-CCCM | H. Zhang (OPPO) |  |
| 4.4 | Improvement on frame-level inheritance mode in ALF-CCCM | M. Jia (ZTE) |  |
| 5 Other |  |  |  |
| 5.1 | Independent CTUs for TM-padding | N. Neumann (Nokia) |  |

JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] (2026-01-09)

New elements from notes elsewhere in this report:

- JVET-AN0121 test 1.1g.

- JVET-AN0158 test 2.4.

- JVET-AN0090 test 2.6a, including the detection method (the latter disabled by default).

- JVET-AN0168 test 2.8a.

- JVET-AN0236 test 3.1.

- JVET-AN0093 test 3.4a.

- JVET-AN0095 test 4.2c*. Also encoder modifications for CTC and non-CTC cases.

- JVET-AN0098 Test 5.1a.

- JVET-AN0101 Test 5.3a.

- JVET-AN0086 (SW), disabled by default. The software coordinator was to decide if solution 1 or 2 is better.

- JVET-AN0214 CABAC context retraining.

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AN2026 Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, F. Bossen] (2025-10-13)

See notes at the end of section 4.16.3.

Remains valid – not updated: JVET-AJ2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud]

Remains valid – not updated: JVET-AL2028 Additions and corrections for VVC conformance (Draft 1) [S. Iwamura, P. de Lagrange, I. Moccagatta]

JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [F. Bossen, J.-R. Ohm, M. Wien] [AG 5 N 180] (2025-12-19)

Remains valid – not updated: JVET-AL2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 9) [S. Liu, J. Chen, J. Ström] [WG 5 DTR N 354]

Primary editor: S. Liu.

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AN2032 Technologies under consideration for future extensions of VSEI (version 10) [S. McCarthy, J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, H. Tan, Y.-K. Wang] (2025-12-05)

New elements from notes elsewhere in this report (this includes new features, editorial improvements, additions of constraints, clarifications, and bug fixes of existing SEI messages in TuC; addition of floating point descriptor types in two cases; extensions to existing SEI messages in VSEI v4):

Syntax descriptors

- JVET-AN0242 AHG9: Signalling of floating-point values in VSEI and graphics rendering information SEI

Encoder optimization info (EOI)

- JVET-AN0208 AHG8/AHG9: Depth-aware optimization for Encoder optimization information SEI message

Scalability dimension info (SDI) SEI

- JVET-AN0235 AHG9: on the SDI SEI message dependency

- JVET-AN0265 AHG9: On confidence auxiliary layer design

Shutter interval

- JVET-AN0145 AHG9: On the shutter interval information extension in TuC of VSEI

- Alpha channel info (ACI)

NNPF

- JVET-AN0068 AHG9: On Neural-network Post-filter Information Signalling and Semantics

Constituent rectangles (CR)

- JVET-AN0102 AHG9: On the CR SEI message

- JVET-AN0149 AHG9: On the constituent rectangles SEI message in TuC of VSEI

- JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages

Display rectangles (DR)

- JVET-AN0124 AHG9: Additional features in display rectangles SEI message

- JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages

SEI processing order (SPO) and processing order nesting (PON)

- JVET-AN0108 AHG9: On the SPO SEI message extension complexity

Digitally signed content (DSC)

- JVET-AN0370 AHG9: Digitally signed content DSC): On subpicture signing

Film grain regions characteristics (FGRC)

- JVET-AN0069 AHG9/AHG13: Updates for the film grain regions SEI message

- JVET-AN0146 AHG9: On the film grain regions characteristics SEI message in TuC of VSEI

Lens optical correction (LOC)

- JVET-AN0074 AHG9: On Lens Optical Correction SEI

Display overlays info (DOI)

- JVET-AN0104 AHG9: On the DOI SEI message

- JVET-AN0117 AHG9: On display overlays information SEI message

- JVET-AN0148 AHG9: On the display overlays information SEI message in TuC of VSEI

- JVET-AN0366 AHG9: Purpose indicators for Display Overlays Information SEI message

Quality metric QM

- JVET-AN0066 AHG9: On the QM SEI message

- JVET-AN0246 AHG9: Metric value range for quality metrics SEI

Bitdepth range info (BRI)

- JVET-AN0147 AHG9: On the bitdepth range information SEI message in TuC of VSEI

Colour mapping info (CMI)

- JVET-AN0231 AHG9: on the CMI SEI message

- JVET-AN0244 AHG9: Prespecified colour maps for the colour mapping info SEI message

- JVET-AN0245 AHG9: On colour mapping info SEI message

- JVET-AN0357 AHG9: On prespecified colour maps for the colour mapping information SEI message

Localization and mapping (LAM) SEI:

- JVET-AN0075 AHG9: On Localization and Mapping SEI

- JVET-AN0167 AHG9: On localization and mapping SEI message

- JVET-AN0260 AHG9: Suggested modifications for the LAM SEI

Danmu information

- JVET-AN0115 AHG9: On the danmu Information SEI message

Graphics rendering info (GRI)

- JVET-AN0144 AHG9: On the graphics rendering information SEI message in TuC of VSEI

- JVET-AN0230 AHG9: on the GRI SEI message

- Enhanced colour information

- JVET-AN0143 AHG9: On the enhanced colour format information SEI message in TuC of VSEI

Referred to editors for consideration:

- JVET-AN0160 AHG9: On DR SEI, DOI SEI, and CR SEI messages

- JVET-AN0232 AHG9: on the Danmu SEI message

- JVET-AN0262 AHG9: On danmu information SEI message

New messages:

- JVET-AN0077 AHG9: Sample Grouping SEI message (name TBD – for not use Bayer information SEI)

Remains valid – not updated: JVET-AF2033 Report of verification test on VVC multi-layer coding: Content layering [S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 105)]

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)]

Remains valid – not updated: JVET-AM2038 White paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] [AG 3 N209]

Was presented to AG 3, and target to release the final version by October.

JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [S. Ikonin, S. Deshpande, V. Zakharchenko] (2025-10-31)

JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools [X. Li, E. Alshina, I. Moccagatta] (2025-12-01)

Developed from JVET-AN0378

JVET-AN2041 Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026 [J.-R. Ohm, M. Wien] (2025-10-27)

Meeting will be hybrid.

Experts requesting for Schengen visa might want to contact J.-R. Ohm and M. Wien as soon as possible.

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at 1030–1100 on Saturday 11 October. In an update discussed on Sunday, the following changes (which had not yet been included in the version that was presented for information in the MPEG WG’s information exchange meeting on Saturday) were approved:

- The following recommendation was deemed necessary to be added: “WG 5 notes that action appears to not yet have been taken on some previous requests for public availability of ISO/IEC deliverables without cost, and kindly requests that ISO/IEC 23008-5:2017 (Ed. 2), ISO/IEC 23008-5:2017/Amd.1:2017, ISO/IEC 23008-8:2018 (Ed. 2), ISO/IEC 23008-8:2018/Amd1:2019, and ISO/IEC 23091-2:2025 (Ed. 3) be made freely available, considering that equivalent publications are freely available elsewhere from ITU-T as editions of ITU-T H.265.2, ITU-T H.265.1, and ITU-T H.273.” Experts were also asked to inform J.-R. Ohm and G. J. Sullivan if relevant standards were missing from this list, as far as they would be expected to be made publicly available.

- The responsible person for delivering 23090-3 is to be changed to Y.K. Wang.

- Some additions are also necessary to the resolutions of thanks, relating to multi-layer testing and offers of new test materials.

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG21 meeting – a total of 8-9 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Thursday or Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8–9 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined, or AHG meetings might be scheduled prior to the meeting. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans were established as follows:

- During 14 – 23 January 2026, 41st meeting under ISO/IEC JTC 1/‌SC 29 auspices, to be conducted as teleconference meeting, not meeting during the weekend 17 – 18 January,

- During 24 April – 1 May 2026, 42nd meeting under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES, to be conducted as hybrid meeting,

- During 7 – 15 July 2026, 43rd meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 17 – 23 October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN, to be conducted as hybrid meeting,

- During 13 – 22 January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU, to be conducted as hybrid meeting,

- During 17 – 28 April 2027, 46th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 7 – 16 July 2027, 47th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI, to be conducted as hybrid meeting,

- During 20 – 29 October 2027, 48th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN, to be conducted as hybrid meeting,

- During January 2028, 49th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

- During April 2028, 50th meeting under ITU-T SG21 auspices, date and location t.b.d.

The agreed document deadline for the 41st JVET meeting was planned to be Wednesday 7 Jan. 2026.

Bytedance, Ericsson, ETRI, Fraunhofer HHI, Hanyang University, Huawei, InterDigital, KDDI, Kwai, Nokia, OPPO, Qualcomm, TCL, Tencent, Wuhan University, and Xiaomi were thanked for submitting responses to the Joint Call for Evidence on video compression with capability beyond VVC.

TCL was thanked for providing 4K displays used in the CfE viewing. Bossen Technologies, Fraunhofer HHI and RWTH Aachen were thanked for providing play-out and testing equipment. Mathias Wien was thanked for organizing and supervising the test, and Christian Lehmann was thanked for helping with the test setup. Christian Lehmann and Vittorio Baroncini were thanked for assistance in preparing and conducting the viewing. The numerous experts who volunteered to participate in the viewings of CfE submissions and for the dry-run of multi-layer verification tests were also thanked.

Alibaba, Bossen Technologies, Bytedance, Dolby, Ericsson, Google, Huawei, InterDigital, KDDI, Nokia, OPPO, Panasonic, Qualcomm, Samsung, Sharp, Tencent, vivo, and Xiaomi were thanked for generating VTM and ECM bitstreams that were used in the context of the Call for Evidence.

Frank Bossen and Mathias Wien were thanked for decoding the bitstreams, for cross-checking results in the preparation of the CfE evaluation, and for preparing the summary of results.

Huawei and Zhejiang University were thanked for offering new video test sequences in the categories of gaming content and 4-UHD/HDR, respectively.

Marius Preda was thanked for the service of managing and maintaining the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

ITU was thanked for the excellent hosting of the 40th JVET meeting in Geneva. Simão Campos and Stefano Polidori were thanked for the help in preparation and organisation, and Kelya-Axelle Beke was thanked for maintaining the registrations. The following members of ITU staff were thanked for their dedication and daily help in the technical setup of meeting facilities: Gent Bajrami, Paul Marko, Gvantsa Nikolishvili, and Marc Antoine Zanou.

The 40th JVET meeting was closed at approximately 1445 hours CEST on Sunday 12 October 2025.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (2 hr. ahead of UTC). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

| JVET number | MPEG number | Created | First upload | Last upload | Title | Authors |
| --- | --- | --- | --- | --- | --- | --- |
| JVET-AN0001 | m74321 | 2025-09-27 07:21:24 | 2025-10-02 18:48:50 | 2025-10-03 21:29:33 | JVET AHG report: Project Management (AHG1) | J.-R. Ohm (chair) G. J. Sullivan (vice chair) |
| JVET-AN0002 | m74322 | 2025-09-27 07:22:11 | 2025-10-03 13:38:02 | 2025-10-03 13:54:38 | JVET AHG report: Draft text and test model algorithm description editing (AHG2) | B. Bross C. Rosewarne (co-chairs) F. Bossen A. Browne S. Kim S. Liu J.-R. Ohm G. J. Sullivan A. Tourapis Y.-K. Wang Y. Ye (vice chairs) |
| JVET-AN0003 | m74323 | 2025-09-27 07:22:41 | 2025-10-03 14:12:00 | 2025-10-03 14:12:00 | JVET AHG report: Test model software development (AHG3) | F. Bossen X. Li K. Sühring (co-chairs) E. François Y. He K. Sharman V. Seregin A. Tourapis (vice chairs) |
| JVET-AN0004 | m74324 | 2025-09-27 07:22:59 | 2025-10-03 14:45:23 | 2025-10-03 14:45:23 | JVET AHG report: Test material and visual assessment (AHG4) | V. Baroncini T. Suzuki M. Wien (co-chairs) W. Husak S. Iwamura P. de Lagrange S. Liu X. Meng S. Puri A. Segall S. Wenger (vice-chairs) |
| JVET-AN0005 | m74325 | 2025-09-27 07:23:20 | 2025-10-03 13:24:03 | 2025-10-03 20:34:34 | JVET AHG report: Conformance testing (AHG5) | I. Moccagatta (chair) F. Bossen T. Ikai S. Iwamura H.-J. Jhu K. Kawamura P. de Lagrange S. Paluri K. Sühring Y. Yu (vice chairs) |
| JVET-AN0006 | m74326 | 2025-09-27 07:23:41 | 2025-10-03 09:16:15 | 2025-10-03 09:16:15 | JVET AHG report: ECM software development (AHG6) | V. Seregin (chair) J. Chen R. Chernyak F. Le Léannec K. Zhang (vice-chairs) |
| JVET-AN0007 | m74327 | 2025-09-27 07:24:06 | 2025-10-02 15:42:59 | 2025-10-03 13:59:44 | JVET AHG report: ECM tool assessment (AHG7) | X. Li (chair) L.-F. Chen Z. Deng J. Gan E. François R. Ishimoto H.-J. Jhu J. Lainema X. Li J. Pardo A. Stein H. Wang (vice chairs) |
| JVET-AN0008 | m74328 | 2025-09-27 07:25:03 | 2025-10-02 23:08:06 | 2025-10-02 23:08:06 | JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) | S. Liu J. Ström S. Wang M. Zhou (AHG chairs) |
| JVET-AN0009 | m74329 | 2025-09-27 07:25:31 | 2025-10-02 12:14:00 | 2025-10-03 17:24:14 | JVET AHG report: SEI message studies (AHG9) | S. McCarthy J. Boyce Y.-K. Wang (co-chairs) T. Chujoh S. Deshpande M. M. Hannuksela P. de Lagrange G. J. Sullivan H. Tan A. Tourapis S. Wenger P. Wu (vice-chairs) |
| JVET-AN0010 | m74330 | 2025-09-27 07:25:57 | 2025-10-02 21:17:15 | 2025-10-02 21:17:15 | JVET AHG report: Encoding algorithm optimization (AHG10) | K. Andersson P. de Lagrange A. Duenas (co-chairs) T. Ikai T. Solovyev A. Tourapis (vice chairs) |
| JVET-AN0011 | m74331 | 2025-09-27 07:26:16 | 2025-10-02 16:17:49 | 2025-10-03 16:16:11 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina F. Galpin S. Liu (co-chairs) J. Li Y. Li R.-L. Liao M. Santamaria T. Shao M. Wien P. Wu (vice chairs) |
| JVET-AN0012 | m74332 | 2025-09-27 07:26:36 | 2025-10-03 09:52:29 | 2025-10-03 09:52:29 | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) | M. Karczewicz Y. Ye L. Zhang (co-chairs) B. Bross R. Chernyak X. Li K. Naser Y. Yu (vice-chairs) |
| JVET-AN0013 | m74333 | 2025-09-27 07:27:15 | 2025-10-03 14:36:08 | 2025-10-03 14:36:08 | JVET AHG report: Film grain technologies (AHG13) | W. Husak P. de Lagrange (co-chairs) A. Duenas X. Meng M. Radosavljević A. Segall G. Teniou A. Tourapis (vice-chairs) |
| JVET-AN0014 | m73914 | 2025-07-28 14:25:58 | 2025-07-28 15:54:11 | 2025-10-03 14:01:07 | JVET AHG report: NNVC software development (AHG14) | F. Galpin (chair) R. Chang Yue Li Yun Li M. Santamaria J. N. Shingala Z. Xie (vice chairs) |
| JVET-AN0015 | m74334 | 2025-09-27 07:27:58 | 2025-10-01 10:30:42 | 2025-10-06 18:18:20 | JVET AHG report: Gaming content compression (AHG15) | S. Puri J. Sauer (co-chairs) R. Chernyak A. Duenas L. Wang V. Zakharchenko (vice chairs) |
| JVET-AN0016 | m74335 | 2025-09-27 07:28:17 | 2025-10-02 22:13:14 | 2025-10-02 22:13:14 | JVET AHG report: Generative face video compression (AHG16) | Y. Ye (chair) H.-B. Teo Z. Lyu S. McCarthy S. Wang (vice chairs) |
| JVET-AN0017 | m74336 | 2025-09-27 07:28:37 | 2025-10-03 14:45:43 | 2025-10-03 17:54:42 | JVET AHG report: Testing of video coding technology beyond CTC (AHG17) | J.-R. Ohm M. Wien F. Bossen (co-chairs) M. Abdoli E. Alshin V. Baroncini J. Chen R. Chernyak Z. Deng P. de Lagrange L. Li P. Nikitin D. Rusanovskyy (vice chairs) |
| JVET-AN0018 | m74337 | 2025-09-27 07:29:03 | 2025-10-03 14:25:04 | 2025-10-03 18:03:23 | JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) | S. Ikonin S. Wenger V. Zakharchenko (co-chairs) S. Deshpande S. Fößel C. Kim X. Ma S. Puri J. Ström (vice-chairs) |
| JVET-AN0021 | m74309 | 2025-09-26 22:50:54 | 2025-09-26 23:09:51 | 2025-09-26 23:09:51 | Deployment status of the VVC standard | G. J. Sullivan |
| JVET-AN0023 | m74338 | 2025-09-27 07:30:42 | 2025-10-03 12:40:17 | 2025-10-10 08:34:57 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina R. Chang F. Galpin Yue Li Yun Li M. Santamaria T. Shao J. Ström Z. Xie (EE coordinators) |
| JVET-AN0024 | m74339 | 2025-09-27 07:31:06 | 2025-10-04 22:46:55 | 2025-10-04 22:46:55 | EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability | V. Seregin D. Buğdayci Sansli J. Chen R. Chernyak K. Naser J. Ström F. Wang M. Winken X. Xiu K. Zhang (EE coordinators) |
| JVET-AN0041 | m73908 | 2025-07-15 09:03:55 | 2025-07-30 11:34:14 | 2025-09-30 08:46:15 | AHG17: AhG meeting notes | M. Wien |
| JVET-AN0042 | m73915 | 2025-07-28 14:40:13 | 2025-08-04 11:53:11 | 2025-08-04 11:53:11 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina F. Galpin |
| JVET-AN0043 | m73918 | 2025-07-29 21:41:16 | 2025-07-31 23:57:14 | 2025-07-31 23:57:14 | AHG7: Preliminary tool analysis with the criteria in JVET-AM0042 | X. Li (Google) |
| JVET-AN0044 | m73927 | 2025-08-19 00:10:35 | 2025-08-19 19:59:14 | 2025-08-19 19:59:14 | Report of AHG7 conference call on Complexity Analysis | X. Li |
| JVET-AN0045 | m73928 | 2025-08-20 04:49:10 | 2025-08-20 05:04:04 | 2025-08-20 05:04:04 | AHG18: Codec software for ultra-low latency and packet loss resilience | S. Ikonin V. Khamidullin K. Malyshev I. Gribushin X. Ma E. Alshina (Huawei) |
| JVET-AN0046 | m73929 | 2025-08-21 02:19:53 | 2025-08-21 04:13:40 | 2025-08-21 22:11:23 | AHG18: On test conditions for ultra-low latency and packet loss resilience with full QP range | S. Ikonin I. Gribushin V. Khamidullin B. Shevchenko E. Alshina (Huawei) |
| JVET-AN0047 | m73930 | 2025-08-22 13:21:26 | 2025-08-22 13:31:38 | 2025-08-22 15:45:49 | EE1-related: Automated environment information collection | N. Le M. Santamaria H. Zhang F. Cricri (Nokia) |
| JVET-AN0048 | m73931 | 2025-08-25 13:13:42 | 2025-08-29 15:01:22 | 2025-10-11 09:09:17 | AHG4: teleconference on VVC multilayer testing | P. de Lagrange (InterDigital) |
| JVET-AN0049 | m73936 | 2025-08-26 19:13:14 | 2025-08-26 19:16:08 | 2025-08-26 19:16:08 | AHG18: Teleconference on ultra-low latency and packet loss resilience | S. Ikonin S. Wenger V. Zakharchenko (co-chairs) S. Deshpande J. Ström X. Ma C. Kim S. Puri S. Fößel (vice-chairs) |
| JVET-AN0050 | m73955 | 2025-09-08 07:04:17 | 2025-09-18 03:23:48 | 2025-09-22 02:36:48 | AHG18 : An Introduction to 5G Latency/Throughput Measurement and Real-Time Monitoring for ULL Application Scenarios | B. Jeon (TTA) S. M. Kim (LGUplus) |
| JVET-AN0051 | m73960 | 2025-09-09 17:26:51 |  |  | Withdrawn |  |
| JVET-AN0052 | m73970 | 2025-09-11 12:57:50 | 2025-09-11 13:02:49 | 2025-10-06 22:35:37 | [AHG17] Performance of GOP based RPR under CfE test conditions | Y. Sun Y. Zhao J. Sauer J. Pardo P. Jia T. Solovyev E. Alshina (Huawei) |
| JVET-AN0053 | m74018 | 2025-09-23 11:33:38 | 2025-09-23 20:02:20 | 2025-09-23 20:02:20 | AHG4/AHG15/AHG17: 4K gaming sequences featuring â€œBlack Myth: Wukongâ€ | J. Sauer Y. Zhao Y. Sun J. Zhou E. Alshina (Huawei) |
| JVET-AN0054 | m74028 | 2025-09-24 08:58:19 | 2025-09-26 11:43:18 | 2025-10-03 18:57:56 | AHG14: Extension of the QP range for TDO Lambda in NNVC S/W | H. Kwon H. Ko (Hanyang Univ.) D. Kim S.-C. Lim (ETRI) |
| JVET-AN0055 | m74036 | 2025-09-24 17:56:06 |  |  | Crosscheck of JVET-AN0052 on Performance of GOP based RPR under CfE test conditions | X. Li (Google) |
| JVET-AN0056 | m74039 | 2025-09-24 23:57:20 | 2025-09-25 19:32:08 | 2025-09-25 19:32:08 | On Next Generation Video Codec Call for Proposal Timing | S. Wenger (Tencent) |
| JVET-AN0057 | m74040 | 2025-09-25 03:46:17 | 2025-09-25 03:48:25 | 2025-09-25 03:48:25 | AHG16: Colour calibration post-processing for generative face video coding | S. Yin Z. Zhang S. Wang (CityUHK) B. Chen R.-L. Liao J. Chen Y. Ye (Alibaba) |
| JVET-AN0058 | m74054 | 2025-09-25 09:49:15 | 2025-09-26 19:30:38 | 2025-09-26 19:30:38 | AHG9: On the NNPF SEI messages in VSEI version 4 | M. M. Hannuksela F. Cricri (Nokia) |
| JVET-AN0059 | m74055 | 2025-09-25 09:51:55 | 2025-09-26 19:31:02 | 2025-09-26 19:31:02 | AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text | M. M. Hannuksela F. Cricri (Nokia) |
| JVET-AN0060 | m74056 | 2025-09-25 09:53:05 | 2025-09-26 19:31:44 | 2025-09-26 19:31:44 | AHG9: On the OMI SEI message in VSEI version 4 | M. M. Hannuksela J. Boyce (Nokia) |
| JVET-AN0061 | m74057 | 2025-09-25 09:54:19 | 2025-09-26 19:32:10 | 2025-09-26 19:32:10 | AHG9: On the text description information SEI message in VSEI version 4 | M. M. Hannuksela (Nokia) |
| JVET-AN0062 | m74058 | 2025-09-25 09:55:52 | 2025-09-26 19:32:59 | 2025-10-06 12:45:44 | AHG9: On the AI usage restrictions SEI message in VSEI version 4 | M. M. Hannuksela F. Cricri K. Kammachi Sreedhar (Nokia) H. Tan (LGE) S. Wenger (Tencent) |
| JVET-AN0063 | m74067 | 2025-09-25 17:42:03 | 2025-09-25 22:49:36 | 2025-09-25 22:49:36 | AHG9: On the NNPF SEI messages in VSEI v4 | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0064 | m74068 | 2025-09-25 17:42:32 | 2025-09-25 22:49:52 | 2025-09-25 22:49:52 | AHG9: On the resampling constraint in VSEI v4 | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0065 | m74069 | 2025-09-25 17:43:01 | 2025-09-25 22:50:11 | 2025-10-05 18:10:42 | AHG9: On the SPO and PON SEI message in VSEI v4 | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) Y.-K. Wang (ByteDance) H. Tan (LG Electronics Inc.) Y. Sanchez (Fraunhofer HHI) M. M. Hannuksela (Nokia) |
| JVET-AN0066 | m74070 | 2025-09-25 17:49:06 | 2025-09-25 22:50:26 | 2025-09-28 19:19:02 | AHG9: On the QM SEI message | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0067 | m74072 | 2025-09-25 18:39:55 | 2025-09-26 23:42:20 | 2025-10-04 14:55:39 | AHG9: Comments on Neural-network Post-filter in VSEI V4 and V3 | S. Deshpande (Sharp) |
| JVET-AN0068 | m74073 | 2025-09-25 18:42:34 | 2025-09-26 23:42:46 | 2025-09-26 23:42:46 | AHG9: On Neural-network Post-filter Information Signalling and Semantics | S. Deshpande (Sharp) |
| JVET-AN0069 | m74074 | 2025-09-25 18:53:56 | 2025-09-26 16:15:50 | 2025-09-26 16:38:09 | AHG9/AHG13: Updates for the film grain regions SEI message | E. François F. Urban P. de Lagrange D. Doyen (InterDigital) G. Teniou (Tencent) |
| JVET-AN0070 | m74076 | 2025-09-25 19:41:09 | 2025-09-30 01:14:26 | 2025-10-01 23:30:44 | CICP enum for generic subsampling and sample location | L. Barnes D. Podborski A. Tourapis (Apple) |
| JVET-AN0071 | m74078 | 2025-09-25 20:31:58 | 2025-09-26 17:58:43 | 2025-10-06 15:26:52 | AHG9: On HEVC Omnidirectional viewport SEI in HEVC | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0072 | m74079 | 2025-09-25 20:37:56 | 2025-09-26 17:59:42 | 2025-09-26 17:59:42 | AHG9: On 3DRD, MAI and Alternative Depth information SEI messages in HEVC and VSEI | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0073 | m74080 | 2025-09-25 20:39:56 | 2025-09-26 18:00:09 | 2025-09-26 18:00:09 | AHG9: On MI SEI message in VSEI version 4 | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0074 | m74081 | 2025-09-25 20:41:31 | 2025-09-26 18:00:33 | 2025-10-07 10:48:55 | AHG9: On Lens Optical Correction SEI | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0075 | m74082 | 2025-09-25 20:42:53 | 2025-09-26 18:01:07 | 2025-10-07 11:41:28 | AHG9: On Localization and Mapping SEI | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0076 | m74083 | 2025-09-25 20:44:10 | 2025-09-26 18:01:29 | 2025-09-26 18:01:29 | AHG9: On floating point signalling in SEI | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0077 | m74084 | 2025-09-25 20:46:07 | 2025-09-26 18:02:02 | 2025-10-11 16:57:10 | AHG9: Sample Grouping SEI message | L. Kerofsky Y. He S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0078 | m74085 | 2025-09-25 22:41:15 | 2025-09-26 19:28:56 | 2025-09-26 19:28:56 | AHG9: Digitally signed content (DSC): On subpicture signing | I. Sodagar C. Fersch S. McCarthy (Dolby) |
| JVET-AN0079 | m74086 | 2025-09-25 22:46:03 | 2025-09-27 01:20:17 | 2025-10-08 13:59:29 | AHG18: Proposed methodology and test conditions for ultra-low latency and packet loss resilience performance evaluation | S. Ikonin I. Gribushin X. Ma E. Alshina (Huawei) S. Deshpande (Sharp) |
| JVET-AN0080 | m74087 | 2025-09-25 22:46:19 | 2025-09-27 00:58:15 | 2025-10-12 16:33:09 | AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience | S. Ikonin V. Khamidullin I. Gribushin M. Sychev K. Malyshev A. Dzugaev A. Bovsha X. Ma E. Alshina (Huawei) |
| JVET-AN0081 | m74091 | 2025-09-26 01:05:40 | 2025-09-26 21:23:16 | 2025-10-08 15:16:27 | AHG18: Software fixes for robust decoding in unicast scenario | K. Malyshev V. Khamidullin M. Sychev S. Ikonin E. Alshina (Huawei) |
| JVET-AN0082 | m74092 | 2025-09-26 01:21:42 | 2025-09-26 01:25:09 | 2025-09-26 01:25:09 | AhG18: Crosscheck report for JVET-AM0195 | V. Zakharchenko (Nokia) |
| JVET-AN0083 | m74097 | 2025-09-26 02:32:25 | 2025-09-26 15:25:53 | 2025-10-01 02:04:32 | EE1-1.2: Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels | J. Han C. Jung Q. Qin (Xidian Univ.) |
| JVET-AN0084 | m74098 | 2025-09-26 02:37:49 | 2025-09-26 15:26:35 | 2025-10-04 04:58:38 | EE1-3.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding | Q. Qin C. Jung (Xidian Univ.) |
| JVET-AN0085 | m74103 | 2025-09-26 03:40:46 | 2025-09-26 17:55:29 | 2025-10-03 11:18:29 | Response to the joint CfE on video compression with capability beyond VVC | M. Abdoli P. Nikitin F. Plowman A. Tissier R. G. Youvalari M.-L. Champel (Xiaomi) Y.-J. Chang C.-C. Chen M. Coban K. Cui P. Garus N. Hu M. Karczewicz J.-L. Lin P.-H. Lin X. Meng B. Ray V. Seregin Y. Shao G. Verba H. Wang E. Ye R. Yu Y. Zhang Z. Zhang W. Zhu (Qualcomm) F. Le Léannec K. Naser (InterDigital) F. Wang (OPPO) Y. Kidani H. Kato T. Chujoh K. Kawamura (KDDI) Z. Deng K. Zhang L. Zhang (Bytedance) A. Filippov J. Konieczny V. Rufitskiy H. Qin T. Dong Z. Xu K. Ding C. Hollmann I. Zupancic D. Li (TCL) X. Xiu X. Wang (Kwai) |
| JVET-AN0086 | m74104 | 2025-09-26 03:48:08 | 2025-09-26 16:26:37 | 2025-10-05 13:48:48 | AHG12: Bug Fix for ALF Coefficient Calculation | R. Xu W. Zhang F. Yang (Xidian Univ.) B. Li F. Xing P. Han Z. Wang W. Song (Hisense) |
| JVET-AN0087 | m74105 | 2025-09-26 03:49:07 | 2025-09-26 16:26:59 | 2025-10-07 08:49:26 | AHG12: Budget Control for Transform Skip Residual Coding | R. Xu W. Zhang F. Yang (Xidian Univ.) B. Li F. Xing P. Han Z. Wang W. Song (Hisense) |
| JVET-AN0088 | m74106 | 2025-09-26 03:49:35 | 2025-09-26 16:27:15 | 2025-10-05 09:53:38 | AHG12: Context Modeling for sig_coeff_flag in TSRC | R. Xu W. Zhang F. Yang (Xidian Univ.) B. Li F. Xing P. Han Z. Wang W. Song (Hisense) |
| JVET-AN0089 | m74109 | 2025-09-26 04:48:05 | 2025-09-26 10:48:29 | 2025-10-07 11:29:43 | AHG12/AHG18: On GDR test condition for video compression beyond VVC | T. Chujoh, Y. Kidani K. Kawamura (KDDI) |
| JVET-AN0090 | m74112 | 2025-09-26 05:38:53 | 2025-09-27 00:00:50 | 2025-10-09 16:19:16 | EE2-2.6: Adaptive subsampling filter selection for CCLM/CCCM | Y. Kidani H. Kato K. Kawamura (KDDI) |
| JVET-AN0091 | m74114 | 2025-09-26 06:09:24 | 2025-09-26 17:21:07 | 2025-10-04 14:22:33 | EE2-3.2: Joint reordering of GPM with affine prediction | L. Zhang Y. Yu Z. Sun H. Yu D. Wang (OPPO) |
| JVET-AN0092 | m74115 | 2025-09-26 06:09:56 | 2025-09-26 17:15:56 | 2025-10-04 14:19:27 | EE2-3.3: Joint reordering of GPM with intra prediction | Z. Sun Y. Yu L. Xu H. Yu D. Wang (OPPO) |
| JVET-AN0093 | m74116 | 2025-09-26 06:10:33 | 2025-09-26 17:21:30 | 2025-10-04 14:22:53 | EE2-3.4: Combination of EE2-3.2 and EE2-3.3 | L. Zhang Z. Sun Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0094 | m74117 | 2025-09-26 06:13:50 | 2025-09-26 17:59:58 | 2025-10-04 16:42:10 | EE2-4.1: Linked Sign Prediction | Y. Zhang L. Xu Y. Yu J. Gan H. Yu D. Wang (OPPO) |
| JVET-AN0095 | m74118 | 2025-09-26 06:14:26 | 2025-09-26 18:42:31 | 2025-09-26 18:42:31 | EE2-4.2a, b, c: Shifting Quantization Center for non-CTC | Y. Yu H. Yu J. Gan L. Xu H. Huang F. Wang Z. Xie D. Wang (OPPO) M. Pendu F. Le Léannec K. Naser (InterDigital) |
| JVET-AN0096 | m74119 | 2025-09-26 06:14:53 | 2025-09-26 18:40:47 | 2025-09-26 18:40:47 | EE2-4.2d, e, f: Shifting Quantization Center for CTC | Y. Yu H. Yu J. Gan L. Xu H. Huang F. Wang Z. Xie D. Wang (OPPO) M. Pendu F. Le Léannec K. Naser (InterDigital) |
| JVET-AN0097 | m74120 | 2025-09-26 06:15:23 | 2025-09-26 18:41:23 | 2025-09-26 18:41:23 | EE2-4.2g, h, i: Shifting Quantization Center for non-CTC | M. Pendu F. Le Léannec K. Naser (InterDigital) Y. Yu H. Yu J. Gan L. Xu H. Huang F. Wang Z. Xie D. Wang (OPPO) |
| JVET-AN0098 | m74121 | 2025-09-26 06:16:10 | 2025-09-26 17:29:08 | 2025-10-04 14:24:46 | EE2-5.1: On regularization of ALF-CCCM | Z. Xie N. Song L. Xu F. Wang Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0099 | m74122 | 2025-09-26 06:17:03 | 2025-09-26 18:18:33 | 2025-10-04 14:17:26 | EE2-5.2: On ALF-CCCM | L. Xu N. Song Z. Xie F. Wang Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0100 | m74123 | 2025-09-26 06:18:52 | 2025-09-26 07:20:22 | 2025-09-30 19:04:38 | AHG9: Gaussian splatting information SEI message | Y. He J. Jung L. Kerofsky G. van der Auwera A. Akhtar M. Karczewicz (Qualcomm) J. Xu Y.-K. Wang L. Zhang (Bytedance) |
| JVET-AN0101 | m74124 | 2025-09-26 06:19:51 | 2025-09-26 17:32:53 | 2025-10-04 14:27:20 | EE2-5.3: Updated multi-models usage strategy for ALF-CCCM | N. Song L. Xu Z. Xie Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0102 | m74125 | 2025-09-26 06:20:54 | 2025-09-26 07:21:37 | 2025-10-05 18:40:24 | AHG9: On the CR SEI message | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0103 | m74126 | 2025-09-26 06:21:12 | 2025-09-26 17:31:44 | 2025-10-04 15:46:54 | EE2-5.4: On ALF-CCCM Model | F. Wang N. Song Z. Xie L. Xu H. Huang Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0104 | m74127 | 2025-09-26 06:21:18 | 2025-09-26 07:22:06 | 2025-10-09 01:12:39 | AHG9: On the DOI SEI message | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0105 | m74128 | 2025-09-26 06:21:38 | 2025-09-26 07:22:50 | 2025-09-26 07:22:50 | AHG9: On applying display overlay on CR target picture | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0106 | m74129 | 2025-09-26 06:21:42 | 2025-09-26 17:34:18 | 2025-10-04 19:14:20 | EE2-5.5: The combinations of EE2-5.1, EE2-5.2, EE2-5.3 and EE2-5.4 | N. Song L. Xu F. Wang Z. Xie Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0107 | m74130 | 2025-09-26 06:22:07 | 2025-09-26 07:23:08 | 2025-10-09 13:28:54 | AHG9: On scalable-nested DSCI SEI message | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0108 | m74131 | 2025-09-26 06:22:24 | 2025-09-26 07:23:23 | 2025-10-06 12:54:01 | AHG9: On the SPO SEI message extension complexity information | Y. He L. Kerofsky S. Zhao M. Karczewicz (Qualcomm) |
| JVET-AN0109 | m74132 | 2025-09-26 06:25:46 | 2025-09-26 22:22:06 | 2025-10-05 10:41:57 | EE2-4.1-related: On Linked Sign Prediction | Y. Zhang L. Xu Y. Yu J. Gan H. Yu D. Wang (OPPO) |
| JVET-AN0110 | m74133 | 2025-09-26 06:27:59 | 2025-09-26 17:17:40 | 2025-10-05 09:16:11 | Non-EE2: Enhancement of Non-CCP Reordering List | Z. Liu L. Zhang Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0111 | m74134 | 2025-09-26 06:29:28 | 2025-09-26 17:23:30 | 2025-10-05 13:48:31 | Non-EE2: additional candidates for regular inter AMVP candidate list | C. Wang Z. Xie Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0112 | m74135 | 2025-09-26 06:29:57 | 2025-09-26 18:35:14 | 2025-10-05 13:51:39 | Non-EE2: Coding of a coefficient level in TSRC | Y. Yu L. Xu J. Gan H. Yu D. Wang (OPPO) |
| JVET-AN0113 | m74136 | 2025-09-26 06:30:25 | 2025-09-26 17:26:23 | 2025-10-05 09:59:18 | Non-EE2: Additional candidates for merge mode in ALF-CCCM | H. Zhang N. Song Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0114 | m74137 | 2025-09-26 06:31:02 | 2025-09-26 18:16:29 | 2025-10-05 13:46:54 | Non-EE2: Local boosting CCP for ALF-CCCM | H. Huang Y. Yu H. Yu D. Wang (OPPO) |
| JVET-AN0115 | m74138 | 2025-09-26 06:37:47 | 2025-09-26 16:56:03 | 2025-10-11 12:03:04 | AHG9: On the danmu Information SEI message | C. Kim H. Tan J. Lee J. Nam J. Lim, S. Kim (LGE) S. Xie P. Wu (ZTE) |
| JVET-AN0116 | m74139 | 2025-09-26 06:38:17 | 2025-09-26 16:59:30 | 2025-10-05 17:40:52 | AHG9: On the colour mapping information SEI message | C. Kim H. Tan J. Lee J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0117 | m74140 | 2025-09-26 06:38:46 | 2025-09-26 17:01:23 | 2025-10-10 08:21:43 | AHG9: On display overlays information SEI message | C. Kim H. Tan J. Lee J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0118 | m74141 | 2025-09-26 06:45:21 | 2025-09-26 11:00:29 | 2025-10-05 18:32:19 | AHG6: ECM software extension for RDO count information | Y. Tokumo S. Hong T. Ikai (Sharp) |
| JVET-AN0119 | m74144 | 2025-09-26 08:03:15 | 2025-09-26 11:30:54 | 2025-10-08 17:19:02 | AHG7: Analysis on group-off tests using Valgrind | R. Ishimoto F. Zheming T. Ikai (Sharp) |
| JVET-AN0120 | m74145 | 2025-09-26 08:10:00 | 2025-09-26 16:49:00 | 2025-10-12 11:42:14 | EE2-2.3: Interpolation filter unification for unwrapping and TIMD template generation | T. Dong V. Rufitskiy A. Filippov (TCL) |
| JVET-AN0121 | m74146 | 2025-09-26 08:17:03 | 2025-09-26 11:47:35 | 2025-10-05 08:39:48 | EE2-1.1: On partitioning optimization | G. Wang C. Zhou Z. Lv (vivo) |
| JVET-AN0122 | m74147 | 2025-09-26 08:17:26 | 2025-09-26 15:17:48 | 2025-09-26 15:17:48 | AHG9/AHG11: Demo of real-time NNPF inference with banding reduction | S. Schwarz H. Sethi J. Funnell M. Santamaria R. Yang F. Cricri M.M. Hannuksela (Nokia) |
| JVET-AN0123 | m74148 | 2025-09-26 08:17:36 | 2025-09-27 00:44:53 | 2025-10-05 11:11:54 | Non-EE2: Improvement on chroma MPM | Z. Li W. Niu X. Zeng M. Jia Y. Wang C. Huang (ZTE) |
| JVET-AN0124 | m74149 | 2025-09-26 08:21:24 | 2025-09-26 10:38:43 | 2025-10-09 13:42:32 | AHG9: Additional features in display rectangles SEI message | R. Imichi Y. Tokumo T. Ikai (Sharp) |
| JVET-AN0125 | m74150 | 2025-09-26 08:24:47 | 2025-09-26 13:09:54 | 2025-10-06 12:15:33 | Non-EE2: On CABAC bin budget constraint | K.-Y. Kim J.-H. Son J.-Y. Kim J.-S. Kwak (WILUS) |
| JVET-AN0126 | m74151 | 2025-09-26 08:25:36 | 2025-09-26 13:10:12 | 2025-10-05 15:38:54 | Non-EE2: MPDIP clipping | K.-Y. Kim J.-H. Son J.-Y. Kim J.-S. Kwak (WILUS) |
| JVET-AN0127 | m74152 | 2025-09-26 08:26:35 | 2025-09-26 09:15:10 | 2025-10-03 14:59:09 | EE1-1.1: LOP with Overlapped Feature Integration | J. Chi A. Li Y. Du C. Zhu L. Luo H. Guo (UESTC) Y. Huo Y. Liu Z. Zhang (Transsion) |
| JVET-AN0128 | m74154 | 2025-09-26 08:36:00 | 2025-09-26 09:41:18 | 2025-10-03 15:07:30 | AhG14: Improvement of SIMD implementation with expanded SIMD operators in SADL for LOP with Overlapped Feature Integration | J. Chi A. Li Y. Du C. Zhu L. Luo H. Guo (UESTC) Y. Huo Y. Liu Z. Zhang (Transsion) |
| JVET-AN0129 | m74156 | 2025-09-26 08:54:30 | 2025-09-26 14:31:57 | 2025-09-30 17:03:26 | EE2-4.3: Residual Sign Prediction restriction | C. Hollmann A. Filippov (TCL) |
| JVET-AN0130 | m74157 | 2025-09-26 08:57:19 | 2025-09-26 18:34:35 | 2025-10-04 21:23:35 | Non-EE2: Improvement on Frame-level Inheritance Mode in ALF-CCCM | M. Jia Y. Bai X. Zeng Z. Li C. Huang (ZTE) |
| JVET-AN0131 | m74158 | 2025-09-26 08:58:07 | 2025-09-26 17:50:33 | 2025-10-05 12:13:52 | Non-EE2: On interpolation filter for SGPM | J. Lee G. Moon K. Kim J.-G. Kim (KAU) J. Lee S.-C. Lim (ETRI) |
| JVET-AN0132 | m74159 | 2025-09-26 08:58:11 | 2025-09-26 18:14:47 | 2025-10-05 11:43:42 | Non-EE2: Enhancement of MDIP mode selection | K. Kim G. Moon J. Lee J.-G. Kim (KAU) J. Lee S.-C. Lim (ETRI) |
| JVET-AN0133 | m74160 | 2025-09-26 09:03:55 | 2025-09-27 02:01:51 | 2025-09-27 02:01:51 | AHG9: Miscellaneous editorial changes for the VSEI v4 draft | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0134 | m74161 | 2025-09-26 09:04:09 | 2025-09-27 02:02:54 | 2025-09-27 02:02:54 | AHG9: On the SEI message payload extension in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0135 | m74162 | 2025-09-26 09:04:22 | 2025-09-27 02:04:56 | 2025-10-06 18:04:52 | AHG9: On the object mask information SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) M. M. Hannuksela (Nokia) J. Chen (Alibaba) H. Tan (LGE) |
| JVET-AN0136 | m74163 | 2025-09-26 09:04:33 | 2025-09-27 02:05:44 | 2025-10-05 09:06:12 | AHG9: On the AI usage restrictions SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) A. Hinds (Tencent) |
| JVET-AN0137 | m74164 | 2025-09-26 09:04:46 | 2025-09-27 02:06:37 | 2025-10-05 09:26:04 | AHG9: On the image format metadata SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) A. Hinds (Tencent) |
| JVET-AN0138 | m74165 | 2025-09-26 09:04:58 | 2025-09-27 02:07:21 | 2025-10-06 08:50:10 | AHG9: On the modality information SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) H. B. Teo (Panasonic) L. Kerofsky (Qualcomm) V. Zakharchenko (Nokia) |
| JVET-AN0139 | m74166 | 2025-09-26 09:05:09 | 2025-09-27 02:08:00 | 2025-09-27 02:08:00 | AHG9: On the text description information SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0140 | m74167 | 2025-09-26 09:05:19 | 2025-09-27 02:10:08 | 2025-10-05 19:59:39 | AHG9: On the neural-network post-filter characteristics SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) M. M. Hannuksela (Nokia) |
| JVET-AN0141 | m74168 | 2025-09-26 09:05:29 | 2025-09-27 02:11:20 | 2025-09-27 02:11:20 | AHG9: On the generative face video SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0142 | m74169 | 2025-09-26 09:05:40 | 2025-09-27 01:04:35 | 2025-10-05 10:08:38 | Non-EE2: GPM without blending for screen content | J.-Y. Kim J.-H. Son K. Kim J.-S. Kwak (WILUS) |
| JVET-AN0143 | m74170 | 2025-09-26 09:05:41 | 2025-09-27 02:12:14 | 2025-09-27 02:12:14 | AHG9: On the enhanced colour format information SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0144 | m74171 | 2025-09-26 09:05:55 | 2025-09-27 02:13:21 | 2025-09-27 02:13:21 | AHG9: On the graphics rendering information SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0145 | m74172 | 2025-09-26 09:06:07 | 2025-09-27 02:14:11 | 2025-09-27 02:14:11 | AHG9: On the shutter interval information extension in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0146 | m74173 | 2025-09-26 09:06:19 | 2025-09-27 02:15:10 | 2025-09-27 02:15:10 | AHG9: On the film grain regions characteristics SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0147 | m74174 | 2025-09-26 09:06:31 | 2025-09-27 03:34:16 | 2025-10-09 03:24:42 | AHG9: On the bitdepth range information SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0148 | m74175 | 2025-09-26 09:06:57 | 2025-09-27 03:37:10 | 2025-10-09 03:25:04 | AHG9: On the display overlays information SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0149 | m74176 | 2025-09-26 09:07:08 | 2025-09-27 02:17:34 | 2025-09-27 02:17:34 | AHG9: On the constituent rectangles SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0150 | m74177 | 2025-09-26 09:07:19 | 2025-09-27 02:18:30 | 2025-09-27 02:18:30 | AHG9: On the localization and mapping SEI message in TuC of VSEI | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0151 | m74178 | 2025-09-26 09:10:48 | 2025-09-26 09:47:29 | 2025-09-26 09:47:29 | AHG6: Additional config files for ECM 2x and ECM 5x | T. Ikai K.-W. Liang (Sharp) |
| JVET-AN0152 | m74179 | 2025-09-26 09:24:27 | 2025-09-26 16:31:15 | 2025-10-05 10:02:56 | Non-EE2: Multiple Candidate Selection for MDIP | L. Wang W. Zhang F. Yang (Xidian Univ.) B. Li F. Xing P. Han Z. Wang W. Song (Hisense) |
| JVET-AN0153 | m74180 | 2025-09-26 09:34:51 | 2025-09-26 17:22:15 | 2025-10-01 09:58:50 | EE1-2.2: Decomposed Content-Adaptive VLOP4 | Z. Xu J. Konieczny A. Filippov C. Hollmann V. Rufitskiy T. Dong H. Qin (TCL) |
| JVET-AN0154 | m74181 | 2025-09-26 09:44:16 | 2025-09-27 10:30:13 | 2025-10-05 12:09:46 | Non-EE2: Position-dependent Weighted Prediction Extension | Y. Wang X. Zeng Z. Li M. Jia C. Huang (ZTE) |
| JVET-AN0155 | m74182 | 2025-09-26 09:46:47 | 2025-09-26 10:01:44 | 2025-10-06 10:47:50 | Non-EE2: On improving adaptive GPM blending | Z. Zhang Y. Huo Y. Liu (Transsion) |
| JVET-AN0156 | m74185 | 2025-09-26 09:58:57 | 2025-09-26 15:38:28 | 2025-09-26 15:38:28 | AHG9: Text fixes and cleanup for GFV and GFE SEI messages | J. Chen Y. Ye B. Chen (Alibaba) |
| JVET-AN0157 | m74186 | 2025-09-26 10:05:47 | 2025-09-26 14:46:04 | 2025-09-26 14:46:04 | AHG17: Valgrind complexity analysis on CfE encoder run time targets | S. Hong Y. Tokumo T. Ikai (Sharp) |
| JVET-AN0158 | m74187 | 2025-09-26 10:06:03 | 2025-09-26 23:53:22 | 2025-10-04 18:48:45 | EE2-2.4: Longer tap interpolation filtering | V. Rufitskiy A. Filippov T. Dong (TCL) |
| JVET-AN0159 | m74189 | 2025-09-26 10:20:24 |  |  | Withdrawn |  |
| JVET-AN0160 | m74191 | 2025-09-26 10:34:17 | 2025-09-26 11:08:54 | 2025-10-07 10:17:39 | AHG9: On DR SEI, DOI SEI, and CR SEI messages | K. Abe T. Nishi (Panasonic) T. Biatek (Nokia) |
| JVET-AN0161 | m74192 | 2025-09-26 10:39:32 | 2025-09-27 00:37:53 | 2025-09-27 00:37:53 | AHG9: On OMI SEI message | H. Tan J. Lee C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0162 | m74193 | 2025-09-26 10:40:13 | 2025-09-27 00:38:20 | 2025-10-11 08:22:04 | AHG9: On subpicture sub-bitstream extraction in VVC | H. Tan J. Lee C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0163 | m74194 | 2025-09-26 10:43:39 | 2025-09-26 11:30:23 | 2025-10-05 16:41:49 | AHG12: On improving the bi-filtering modes in TALF | P. Astola D. Buğdayci Sansli J. Lainema (Nokia) |
| JVET-AN0164 | m74196 | 2025-09-26 10:56:56 | 2025-09-26 17:52:02 | 2025-09-26 17:52:02 | Non-EE2: ALF coefficient coding group extension | I. Jumakulyyev D. Buğdayci Sansli J. Lainema (Nokia) |
| JVET-AN0165 | m74199 | 2025-09-26 11:12:13 | 2025-09-26 18:37:19 | 2025-10-04 10:49:08 | Crosscheck of JVET-AN0092 (EE2-3.3: Joint reordering of GPM with intra prediction) | N. Zouidi (Ofinno) |
| JVET-AN0166 | m74201 | 2025-09-26 11:17:44 | 2025-09-26 18:30:19 | 2025-10-02 15:59:47 | Crosscheck of JVET-AN0168 and JVET-AN0173 (EE2-2.8a, b: Enhanced CCP Fusion mode) | N. Zouidi (Ofinno) |
| JVET-AN0167 | m74202 | 2025-09-26 11:23:56 | 2025-09-26 17:03:32 | 2025-10-10 13:59:57 | AHG9: On localization and mapping SEI message | C. Kim H. Tan J. Lee J. Nam J. Lim S. Kim (LGE) J. Xu (Bytedance) |
| JVET-AN0168 | m74203 | 2025-09-26 11:27:35 | 2025-09-26 17:56:33 | 2025-09-26 17:56:33 | EE2-2.8a: Enhanced CCP Fusion mode | P. Bordes F. Galpin K. Naser F. Le Léannec (InterDigital) |
| JVET-AN0169 | m74204 | 2025-09-26 11:33:34 | 2025-09-26 17:01:25 | 2025-10-05 09:42:42 | Non-EE2: On inter multiple transform set selection and advanced SBT | J. Huo W. Zhang F. Yang (Xidian Univ.) B. Li F. Xing P. Han Z. Wang W. Song (Hisense) |
| JVET-AN0170 | m74206 | 2025-09-26 11:40:37 | 2025-09-26 15:51:14 | 2025-10-05 15:52:37 | Non-EE2: Additional OBIC candidates in MPM list | Y. Kim S. Lee S. Noh H. Choi (HNU) W. Lim S.-C. Lim (ETRI) |
| JVET-AN0171 | m74208 | 2025-09-26 11:43:20 | 2025-09-26 12:05:15 | 2025-10-04 15:15:28 | EE2-2.1: TMRL blend | S. Blasi G. Kulupana D. Buğdayci Sansli J. Lainema (Nokia) K. Naser G. Rath F. Le Leannec T. Dumas (InterDigital) |
| JVET-AN0172 | m74210 | 2025-09-26 11:44:12 | 2025-09-26 11:55:39 | 2025-09-26 11:55:39 | AHG11: Proposal of normative TDO method as an alternative option in NNVC | H. Kwon H. Ko (Hanyang Univ.) D. Kim S.-C. Lim (ETRI) |
| JVET-AN0173 | m74211 | 2025-09-26 11:47:12 | 2025-09-26 18:00:35 | 2025-09-26 18:00:35 | EE2-2.8b: Combination of EE2-2.7 and EE2-2.8a | P. Bordes F. Galpin K. Naser F. Le Léannec (InterDigital) S. Wan Y. Yin Z. Zhu (NWPU) S. Xie X. Zeng C. Huang (ZTE) |
| JVET-AN0174 | m74213 | 2025-09-26 11:53:08 | 2025-09-26 12:24:36 | 2025-09-26 12:24:36 | AHG15: Compression of gaming content using auxiliary data | J. Sauer Z. Li (Huawei) |
| JVET-AN0175 | m74214 | 2025-09-26 11:57:49 | 2025-09-26 16:17:19 | 2025-10-01 17:47:37 | EE2-related: Unbiased Simple Quantizer (Related to EE2-4.2i) | M. Balcilar M. Blestel P. Andrivon (Ofinno) |
| JVET-AN0176 | m74215 | 2025-09-26 12:14:05 | 2025-09-26 18:02:58 | 2025-10-08 15:13:48 | AHG9: On multi-purpose NNPFs | C.-H. Demarty E. François A. Ak (InterDigital) |
| JVET-AN0177 | m74216 | 2025-09-26 12:16:41 | 2025-09-26 17:56:25 | 2025-10-06 11:07:39 | AHG4/AHG17: Response to the call for new HDR materials for future video coding development | J. Wang J. Zhang L. Yu (ZJU) |
| JVET-AN0178 | m74217 | 2025-09-26 12:44:29 | 2025-09-26 12:55:17 | 2025-09-26 12:55:17 | VVC multilayer extension for an external base layer | F. Urban Y. Che F. Galpin E. François P. de Lagrange P. Bordes (InterDigital) |
| JVET-AN0179 | m74218 | 2025-09-26 12:57:03 |  |  | Withdrawn |  |
| JVET-AN0180 | m74219 | 2025-09-26 13:10:07 | 2025-09-26 23:57:08 | 2025-10-11 05:05:56 | AHG9: On PRI SEI message in processing chain | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0181 | m74220 | 2025-09-26 13:10:19 | 2025-09-26 23:57:42 | 2025-10-08 09:01:52 | AHG9: On process SEI messages in processing chain in SPO SEI message | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) Y.-K. Wang (Bytedance) M. M. Hannuksela (Nokia) |
| JVET-AN0182 | m74221 | 2025-09-26 13:10:31 | 2025-09-26 23:58:12 | 2025-10-07 09:21:23 | AHG9: On DSC SEI messages in VSEI v4 draft | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) K. Sühring Y. Sanchez (Fraunhofer HHI) M. M. Hannuksela (Nokia) |
| JVET-AN0183 | m74222 | 2025-09-26 13:10:40 | 2025-09-27 01:12:19 | 2025-09-27 01:12:19 | AHG9: On PRI SEI message in VSEI v4 draft | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0184 | m74223 | 2025-09-26 13:10:51 | 2025-09-26 23:58:42 | 2025-10-02 02:19:19 | AHG9: Miscellaneous on VSEI v4 draft | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0185 | m74224 | 2025-09-26 13:10:57 | 2025-09-27 01:12:55 | 2025-10-09 05:08:08 | AHG9: Miscellaneous on VSEI codec interfaces | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0186 | m74225 | 2025-09-26 13:11:03 | 2025-09-26 23:59:17 | 2025-09-26 23:59:17 | AHG9: Editorial updates for VSEI v4 | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0187 | m74226 | 2025-09-26 13:11:18 | 2025-09-26 23:59:42 | 2025-09-26 23:59:42 | AHG9: On DSC SEI messages for subpicture-based signing in TuC of VSEI | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0188 | m74227 | 2025-09-26 13:11:30 | 2025-09-27 00:00:16 | 2025-10-03 11:34:45 | AHG9: On signing parameter sets in DSC SEI messages for subpictures in TuC of VSEI | J. Lee H. Tan C. Kim J. Nam J. Lim S. Kim (LGE) |
| JVET-AN0189 | m74228 | 2025-09-26 13:29:37 | 2025-09-26 14:15:28 | 2025-09-26 14:15:28 | AHG17/AHG10: On perceptual coding for next-generation video coding standard | Y. Zhao A. Karabutov T. Guo P. Jia E. Alshina (Huawei) |
| JVET-AN0190 | m74229 | 2025-09-26 13:30:33 | 2025-09-26 16:10:29 | 2025-10-07 12:17:14 | AHG17/AHG10: On test model simulating hardware encoder | Y. Zhao J. Mao R. Zhao X. Ma T. Solovyev E. Alshina (Huawei) |
| JVET-AN0191 | m74230 | 2025-09-26 13:38:28 | 2025-09-26 13:58:49 | 2025-10-05 11:32:49 | Non-EE2: Binarization Improvement of GPM | X. Wang J. Chen C. Zhu L. Luo H. Guo (UESTC) Y. Huo Y. Liu Z. Zhang (Transsion) |
| JVET-AN0192 | m74231 | 2025-09-26 14:01:08 | 2025-09-27 04:40:15 | 2025-10-05 12:11:30 | Non-EE2: Modification of reconstructed area for EIP | W. Niu S. Xie Z. Li M. Jia Y. Bai C. Huang (ZTE) |
| JVET-AN0193 | m74232 | 2025-09-26 14:17:01 | 2025-09-26 15:04:25 | 2025-09-26 15:04:25 | EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization | W. Zhang N. Fu X. Chen W. Ma J. Zhang Z. Chen (Wuhan Univ.) |
| JVET-AN0194 | m74233 | 2025-09-26 14:18:38 | 2025-09-26 14:38:30 | 2025-10-05 10:15:34 | Non-EE2: Extension on TMRL mode | Y. Liu Y. Huo Z. Zhang (Transsion) |
| JVET-AN0195 | m74234 | 2025-09-26 14:19:05 | 2025-09-26 15:05:03 | 2025-09-30 14:53:21 | EE1-3.5: Retrained DRF in NNVC-14.0 | X. Chen N. Fu W. Zhang W. Ma J. Zhang Z. Chen (Wuhan Univ.) |
| JVET-AN0196 | m74235 | 2025-09-26 14:21:53 | 2025-09-26 15:05:46 | 2025-09-26 15:05:46 | AHG14: Operator Improvements in the SADL library | W. Zhang N. Fu L. Qin X. Chen Z. Chen (Wuhan Univ.) |
| JVET-AN0197 | m74236 | 2025-09-26 14:23:56 | 2025-09-26 15:06:10 | 2025-09-26 15:06:10 | AHG14: The extension of SADL library | L. Qin W. Zhang N. Fu Z. Chen (Wuhan Univ.) |
| JVET-AN0198 | m74238 | 2025-09-26 14:28:58 | 2025-09-26 15:06:36 | 2025-10-08 18:26:44 | [AHG17] Wuhan University’s Response in Joint Call for Evidence on Video Compression with Capability beyond VVC | W. Zhang L. Qin X. Chen N. Fu H. Qu W. Ma J. Zhang Z. Chen (Wuhan Univ.) |
| JVET-AN0199 | m74239 | 2025-09-26 14:30:50 | 2025-09-27 03:23:48 | 2025-10-03 10:42:27 | EE2-5.6: Reuse of TALF control information | Y. Bai M. Jia W. Niu S. Xie C. Huang (ZTE) |
| JVET-AN0200 | m74240 | 2025-09-26 14:30:58 | 2025-09-27 12:36:03 | 2025-10-05 12:10:46 | Non-EE2: On improving CIIP Fusion | Y. Wang X. Zeng M. Jia Z. Li C. Huang (ZTE) |
| JVET-AN0201 | m74244 | 2025-09-26 14:51:48 | 2025-09-26 19:22:45 | 2025-10-03 23:53:27 | EE1-4.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) | H. Cho S. Bahk H. Y. Kim (KHU) D. Kim S.-C. Lim (ETRI) |
| JVET-AN0202 | m74245 | 2025-09-26 14:57:05 | 2025-09-26 15:02:11 | 2025-10-02 09:58:25 | [AHG11/AHG14] Comparison of Multi-Layer and Single-Layer Interfaces for Hybrid End-to-End Video Coding Frameworks | N. Zou A. Hallapuro F. Cricri H. Zhang A. B. Koyuncu J. Ahonen M. M. Hannuksela (Nokia) |
| JVET-AN0203 | m74246 | 2025-09-26 14:58:02 | 2025-09-26 18:04:43 | 2025-10-09 10:23:09 | AHG7: On RGPM moved into group-2 | P. Bordes K. Reuzé E. François K. Naser F. Le Léannec (InterDigital) |
| JVET-AN0204 | m74247 | 2025-09-26 14:59:28 | 2025-09-27 01:04:15 | 2025-09-27 01:04:15 | AHG18: Request on inclusion ultra-low latency and packet loss resilience category into CfP | S. Ikonin X. Ma E. Alshina (Huawei) S. Wenger (Tencent) |
| JVET-AN0205 | m74249 | 2025-09-26 15:16:52 | 2025-09-26 21:43:47 | 2025-10-10 15:34:02 | AHG9: Gaussian Splat Information SEI message | J. Ricard G. Teniou S. Wenger A. Hinds (Tencent) |
| JVET-AN0206 | m74250 | 2025-09-26 15:18:47 | 2025-09-26 21:46:29 | 2025-10-10 15:34:23 | AHG9: Gaussian Splat compression experiment platform | J. Ricard G. Teniou S. Wenger A. Hinds (Tencent) |
| JVET-AN0207 | m74252 | 2025-09-26 15:20:45 | 2025-09-26 21:45:39 | 2025-09-27 11:04:12 | AHG9: Overlay purpose indicator for Display overlays information SEI message | G. Teniou S. Wenger A. Hinds (Tencent) |
| JVET-AN0208 | m74253 | 2025-09-26 15:23:04 | 2025-09-26 21:46:05 | 2025-10-10 18:19:29 | AHG8/AHG9: Depth-aware optimization for Encoder optimization information SEI message | G. Teniou S. Wenger A. Hinds J. Ricard (Tencent) |
| JVET-AN0209 | m74255 | 2025-09-26 15:26:57 | 2025-09-26 17:10:33 | 2025-10-04 10:58:49 | AhG 11/AHG 14: Harmonization of Deep Reference Frame (DRF) with RPR in NNVC14.1 | N. Bhaskar T. Solovyev E. Alshina (Huawei) |
| JVET-AN0210 | m74256 | 2025-09-26 15:28:03 | 2025-09-26 15:38:40 | 2025-09-26 15:38:40 | AHG9: On subpicture signing in the DSC SEI messages | M. Pettersson R. Sjöberg M. Damghanian (Ericsson) |
| JVET-AN0211 | m74257 | 2025-09-26 15:29:39 | 2025-09-26 16:10:44 | 2025-10-06 17:18:24 | AhG12: Second reference frame for temporal partitioning prediction | G. Laroche P. Onno B. Galmiche (Canon) |
| JVET-AN0212 | m74258 | 2025-09-26 15:37:25 | 2025-09-26 17:08:14 | 2025-10-06 22:22:29 | AHG17: Description of NNVC-based CfE response | D. Kim S.-C. Lim (ETRI) T. Solovyev J. Sauer J. Pardo P. Jia A. Karabutov E. Alshina (Huawei) H. Kwon H. Ko (HYU) F. Galpin T. Dumas E. François (InterDigital) Y. Li M. Karczewicz (Qualcomm) Z. Xiang R. Chernyak S. Liu (Tencent) |
| JVET-AN0213 | m74259 | 2025-09-26 15:42:07 | 2025-09-26 15:43:48 | 2025-10-03 14:54:36 | AhG14: SADL update | F. Galpin (InterDigital) |
| JVET-AN0214 | m74260 | 2025-09-26 15:44:49 | 2025-09-26 15:48:31 | 2025-10-10 12:10:25 | AhG12: CABAC contexts retraining | F. Galpin (InterDigital) |
| JVET-AN0215 | m74261 | 2025-09-26 15:49:19 | 2025-09-26 15:53:08 | 2025-10-02 12:24:37 | AhG11: Filter restriction depending on hierarchical depth | F. Galpin E. François (InterDigital) |
| JVET-AN0216 | m74262 | 2025-09-26 15:54:00 | 2025-09-26 16:04:30 | 2025-10-04 14:37:08 | [AHG11] Update on Multilayer framework for supporting a hybrid codec using End-to-End Learned Image Codec and Conventional Video Codec | F. Urban Y. Chen F. Galpin P. de Lagrange (InterDigital) |
| JVET-AN0217 | m74263 | 2025-09-26 16:01:47 | 2025-09-26 16:09:11 | 2025-10-06 16:13:27 | AHG6: Software Implementation for Runtime Reporting | S.-Y. Lim J. Gao J.-Y. Thong H.-B. Teo C.-S. Lim K. Abe (Panasonic) |
| JVET-AN0218 | m74264 | 2025-09-26 16:04:02 | 2025-09-26 16:18:56 | 2025-10-06 09:30:01 | Non-EE2: Extension of Sub-Block Merge Mode Availability Conditions | Y. Liu Y. Huo Z. Zhang (Transsion) |
| JVET-AN0219 | m74269 | 2025-09-26 16:55:49 | 2025-09-26 16:58:32 | 2025-10-05 12:23:37 | AHG12 IntraTMP with DMVR | K. Naser P. Bordes F. Le Léannec A. Robert (InterDigital) |
| JVET-AN0220 | m74270 | 2025-09-26 17:00:16 | 2025-09-26 17:07:45 | 2025-10-04 20:14:45 | EE2-2.7: Reducing Candidate Modes in DDCCP | S. Wan Y. Yin Z. Zhu (NWPU) S. Xie X. Zeng C. Huang (ZTE) |
| JVET-AN0221 | m74271 | 2025-09-26 17:01:37 | 2025-09-27 03:57:12 | 2025-10-05 09:24:58 | AHG12: Reference sample generation for intra prediction | T. N. Canh P. Yin S. McCarthy (Dolby) |
| JVET-AN0222 | m74272 | 2025-09-26 17:02:07 | 2025-09-26 20:50:14 | 2025-09-26 20:50:14 | [AHG14] New NN elements for SADL | A. Karabutov E. Alshina (Huawei) |
| JVET-AN0223 | m74273 | 2025-09-26 17:02:23 | 2025-10-03 21:51:38 | 2025-10-06 20:14:18 | AHG7: On bin to bit ratio in ECM | T. N. Canh P. Yin S. McCarthy (Dolby) J. N. Shingala (Ittiam) |
| JVET-AN0224 | m74274 | 2025-09-26 17:04:28 | 2025-09-26 18:36:48 | 2025-10-02 10:01:58 | [AHG11] A Hybrid Framework Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou A. Hallapuro F. Cricri H. Zhang A. B. Koyuncu J. Ahonen M. M. Hannuksela (Nokia) |
| JVET-AN0225 | m74275 | 2025-09-26 17:59:11 | 2025-09-26 23:05:06 | 2025-10-04 17:43:25 | [AHG11][AHG14]: Further Improvements on Hybrid Multilayer Framework for End-to-End Learned Intra Frame | M. Aderdor T. Solovyev E. Alshina (Huawei) |
| JVET-AN0226 | m74276 | 2025-09-26 18:00:22 | 2025-09-26 18:08:27 | 2025-10-10 12:14:49 | AHG9: Spatial and bit depth restoration SEI message | X. Xu M. Xu S. Liu (Tencent) |
| JVET-AN0227 | m74277 | 2025-09-26 18:02:03 | 2025-09-26 18:08:01 | 2025-10-10 12:15:10 | AHG9: Temporal resampling information SEI message | X. Xu S. Liu (Tencent) |
| JVET-AN0228 | m74278 | 2025-09-26 18:14:47 | 2025-09-26 18:18:27 | 2025-09-26 18:18:27 | Independent CTUs for TM-padding | N. Neumann (Nokia) |
| JVET-AN0229 | m74279 | 2025-09-26 18:15:06 | 2025-09-27 02:00:43 | 2025-09-27 02:00:43 | AHG9: On the film grain characteristics SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0230 | m74280 | 2025-09-26 18:15:13 | 2025-09-26 19:03:09 | 2025-10-05 11:13:55 | AHG9: on the GRI SEI message | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AN0231 | m74281 | 2025-09-26 18:18:32 | 2025-09-26 19:01:03 | 2025-09-26 19:01:03 | AHG9: on the CMI SEI message | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AN0232 | m74282 | 2025-09-26 18:20:16 | 2025-09-26 19:02:26 | 2025-09-26 19:02:26 | AHG9: on the Danmu SEI message | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AN0233 | m74283 | 2025-09-26 18:21:36 | 2025-09-26 18:58:58 | 2025-10-11 06:35:34 | AHG9: On ASAI SEI Message applied to primary and auxiliary layers | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AN0234 | m74284 | 2025-09-26 18:22:34 | 2025-09-26 18:41:54 | 2025-10-05 14:10:03 | Non-EE2: On filtering condition for angular modes | Z. Zhu Y. Yin H. Xu S. Wan (NWPU) S. Xie X. Zeng C. Huang (ZTE) |
| JVET-AN0235 | m74285 | 2025-09-26 18:22:59 | 2025-09-26 19:00:08 | 2025-10-11 06:51:22 | AHG9: on the SDI SEI message dependency | S. Zhao Y. He L. Kerofsky M. Karczewicz (Qualcomm) |
| JVET-AN0236 | m74286 | 2025-09-26 18:27:21 | 2025-09-26 19:29:06 | 2025-10-03 17:25:40 | EE2-3.1: Generated Merge Candidates | D. Buğdayci Sansli J. Lainema (Nokia) |
| JVET-AN0237 | m74287 | 2025-09-26 18:43:38 | 2025-09-27 00:03:12 | 2025-10-02 22:57:52 | AHG13: Film grain analysis improvement | M. Radosavljević F. Lefebvre P. de Lagrange Z. Ameur (InterDigital) |
| JVET-AN0238 | m74288 | 2025-09-26 18:46:42 | 2025-09-26 19:02:55 | 2025-09-26 19:02:55 | EE1-4.2: Cross-component enhanced NNSR | T. Yang W.-X. He Y.-Q. Zhu X.-T. Xie J.-S. Gong Q.-M. Wang Q. Liu (HUST) Z.-Y. Lv (vivo) |
| JVET-AN0239 | m74289 | 2025-09-26 19:10:13 | 2025-10-03 18:11:43 | 2025-10-03 18:11:43 | Crosscheck of JVET-AN0095 (EE2-4.2a, b, c: Shifting Quantization Center for non-CTC) | P. Nikitin (Xiaomi) |
| JVET-AN0240 | m74290 | 2025-09-26 19:11:36 | 2025-10-04 16:03:50 | 2025-10-04 16:03:50 | Crosscheck of JVET-AN0096 (EE2-4.2d, e, f: Shifting Quantization Center for CTC) | P. Nikitin (Xiaomi) |
| JVET-AN0241 | m74291 | 2025-09-26 19:14:13 | 2025-10-05 09:19:22 | 2025-10-05 09:19:22 | Crosscheck of JVET-AN0236 (EE2-3.1: Generated Merge Candidates) | R. G. Youvalari (Xiaomi) |
| JVET-AN0242 | m74292 | 2025-09-26 19:36:57 | 2025-09-26 20:12:00 | 2025-10-04 22:19:54 | AHG9: Signalling of floating-point values in VSEI and graphics rendering information SEI | V. Zakharchenko J. Boyce (Nokia) |
| JVET-AN0243 | m74293 | 2025-09-26 19:43:24 | 2025-09-26 23:44:39 | 2025-09-26 23:44:39 | Crosscheck of JVET-AN0052 (AHG17: Performance of GOP based RPR under CfE test conditions) | Z. Xiang (Tencent) |
| JVET-AN0244 | m74294 | 2025-09-26 20:19:38 | 2025-09-26 21:07:04 | 2025-09-26 21:07:04 | AHG9: Prespecified colour maps for the colour mapping info SEI message | J. Boyce M. M. Hannuksela T. Biatek (Nokia) |
| JVET-AN0245 | m74295 | 2025-09-26 20:19:42 | 2025-09-26 21:09:24 | 2025-09-26 21:09:24 | AHG9: On colour mapping info SEI message | J. Boyce M. M. Hannuksela T. Biatek (Nokia) |
| JVET-AN0246 | m74296 | 2025-09-26 20:19:45 | 2025-09-26 21:54:29 | 2025-09-26 21:54:29 | AHG9: Metric value range for quality metrics SEI | J. Boyce M. M. Hannuksela T. Biatek (Nokia) |
| JVET-AN0247 | m74297 | 2025-09-26 20:19:48 | 2025-09-26 22:44:33 | 2025-10-08 19:41:12 | AHG9: Quality metrics SEI for concatenated encoding | J. Boyce M. M. Hannuksela T. Biatek (Nokia) |
| JVET-AN0248 | m74298 | 2025-09-26 20:19:51 | 2025-09-26 21:04:29 | 2025-09-26 21:04:29 | AHG9: Overlay set purpose indicator for Display overlays information SEI message | T. Biatek J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AN0249 | m74299 | 2025-09-26 20:22:23 | 2025-09-26 22:27:26 | 2025-09-26 22:27:26 | Suggestions for a potential upcoming CfP | J. Samuelsson-Allendes S. Deshpande (Sharp) |
| JVET-AN0250 | m74301 | 2025-09-26 20:41:55 | 2025-10-01 20:09:15 | 2025-10-01 20:09:15 | Crosscheck of JVET-AN0097 (EE2-4.2g, h, i: Shifting Quantization Center for non-CTC) | M. Balcilar (Ofinno) |
| JVET-AN0251 | m74302 | 2025-09-26 21:24:16 | 2025-10-01 14:23:03 | 2025-10-01 14:23:03 | Crosscheck of JVET-AN0090 (EE2-2.6: Adaptive subsampling filter selection for CCLM/CCCM ) | P. Bordes F. Galpin (InterDigital) |
| JVET-AN0252 | m74303 | 2025-09-26 21:36:01 | 2025-09-30 15:58:38 | 2025-09-30 15:58:38 | Crosscheck of JVET-AN0220 (EE2-2.7: Reducing Candidate Modes in DDCCP) | P. Bordes (InterDigital) |
| JVET-AN0253 | m74304 | 2025-09-26 21:55:26 | 2025-09-26 22:06:04 | 2025-09-26 22:06:04 | AHG7: On Memory Bandwidth Measurement | X. Li (Google) Y. Kim (Samsung) J. Pardo (Huawei) R. Ishimoto (Sharp) L.-F. Chen (Tencent) |
| JVET-AN0254 | m74305 | 2025-09-26 22:06:37 | 2025-09-26 22:09:27 | 2025-10-07 15:03:04 | AHG18: Packet marking mechanism for unicast scenario with channel feedback | V. Zakharchenko (Nokia) |
| JVET-AN0255 | m74306 | 2025-09-26 22:16:18 | 2025-09-26 22:22:48 | 2025-10-04 12:16:11 | AHG11: VLOP3 with new backbone block based on Spatial-Channel Mixing (SCM) | H. Cho S. Bahk H. Y. Kim (KHU) D. Kim S.-C. Lim (ETRI) |
| JVET-AN0256 | m74307 | 2025-09-26 22:28:44 | 2025-10-03 23:14:28 | 2025-10-04 07:55:01 | Crosscheck of JVET-AN0238 (EE1-4.2: Cross-component enhanced NNSR) | H. Cho S. Bahk T. Lee H. Y. Kim (KHU) D. Kim S.-C. Lim (ETRI) |
| JVET-AN0257 | m74308 | 2025-09-26 22:33:26 | 2025-10-04 10:04:29 | 2025-10-04 18:05:08 | [AHG11] CPU and GPU time comparison for NNVC models | A. Karabutov E.Alshina (Huawei) Franck Galpin (InterDigital) |
| JVET-AN0258 | m74310 | 2025-09-26 23:17:20 | 2025-09-26 23:27:24 | 2025-09-26 23:27:24 | Reference Guide to the Structural Concepts in the Versatile Video Coding (VVC) Standard | G. J. Sullivan (Dolby Labs) |
| JVET-AN0259 | m74311 | 2025-09-26 23:22:42 | 2025-09-26 23:24:55 | 2025-10-09 15:31:20 | AHG9: Enable the regional display in the Danmu information SEI message | S. Xie P. Wu W. Niu Y. Gao C. Huang (ZTE) |
| JVET-AN0260 | m74312 | 2025-09-26 23:26:32 | 2025-09-26 23:28:04 | 2025-09-26 23:28:04 | AHG9: Suggested modifications for the LAM SEI | V. Zakharchenko J. Boyce (Nokia) |
| JVET-AN0261 | m74313 | 2025-09-26 23:27:50 | 2025-09-26 23:29:32 | 2025-10-08 14:38:22 | AHG9: Comments on the Danmu information SEI message | S. Xie P. Wu W. Niu Y. Gao C. Huang (ZTE) |
| JVET-AN0262 | m74314 | 2025-09-26 23:30:40 | 2025-09-26 23:34:40 | 2025-10-10 15:36:25 | AHG9: On danmu information SEI message | E. Thomas E. Potetsianakis E. Alexiou M.-L. Champel (Xiaomi) |
| JVET-AN0263 | m74315 | 2025-09-26 23:37:58 | 2025-09-26 23:43:35 | 2025-10-06 15:52:24 | AHG15: Transformer based depthmaps reconstruction for gaming content | V. Zakharchenko (Nokia) |
| JVET-AN0264 | m74316 | 2025-09-26 23:53:12 | 2025-09-27 00:31:33 | 2025-10-03 11:01:43 | AHG9: Support for implicit representations with the Gaussian splatting information SEI message | S. Lee S. Sasse Y. Sanchez R. Skupin T. M. Borges C. Hellge T. Schierl (Fraunhofer HHI) |
| JVET-AN0265 | m74318 | 2025-09-27 01:27:34 | 2025-09-27 01:47:16 | 2025-10-11 15:33:26 | AHG9: On confidence auxiliary layer design | E. Thomas E. Potetsianakis E. Alexiou M.-L. Champel (Xiaomi) |
| JVET-AN0266 | m74319 | 2025-09-27 03:15:42 | 2025-09-27 04:43:45 | 2025-10-06 12:21:15 | Pairwise Merge Candidates with Motion Vector Scaling | S. Hong L. Wang K. Panusopone D. Rusanovskyy (Nokia) |
| JVET-AN0267 | m74317 | 2025-09-27 00:29:53 | 2025-09-27 00:37:48 | 2025-10-07 10:28:21 | Ericsson, Fraunhofer HHI and Nokia responses to Joint CfE on Video Compression with Capability beyond VVC | W. Ahmad K. Andersson M. Damghanian V. Gritsenko L. Litwic D. Liu M. Pettersson V. Shchukin R. Sjöberg N. Stegmaier J. Ström N. Svensson P. Wennersten (Ericsson) C. Bartnik J. Brandenburg B. Bross V. George J. Güther G. Hege A. Henkel T. Hinz G. Lazarov C. Lehmann W. Lim Y. Liu S. de Luxán Hernández D. Marpe V. Menon T. Nguyen J. Pfaff S. Puttkammer T. Schierl H. Schwarz B. Stallenberger C. Stoffers K. Sühring A. Wieckowski T. Wiegand M. Winken (Fraunhofer HHI) P. Astola S. Blasi D. Buğdayci Sansli C. Feldmann D. Fortin J. Funnell S. Hong I. Jumakulyyev J. Lainema N. Neumann J. Ridge D. Rusanovskyy S. Schwarz (Nokia) |
| JVET-AN0268 | m74320 | 2025-09-27 03:22:54 |  |  | Withdrawn |  |
| JVET-AN0269 | m74341 | 2025-09-27 09:00:33 | 2025-09-27 09:07:27 | 2025-10-01 13:05:12 | AHG12: BIF with before DBF samples | K. Takada S. Deshpande (Sharp) |
| JVET-AN0270 | m74342 | 2025-09-27 09:59:45 | 2025-09-28 19:05:32 | 2025-10-05 16:24:04 | Non-EE2: CCCM with clipping | L.-C. Xu C.-F. Liao Y.-K. Lu F. Liang (SYSU) |
| JVET-AN0271 | m74351 | 2025-09-28 20:53:57 | 2025-09-28 23:04:41 | 2025-10-07 19:18:26 | Software optimization and complexity reduction of ECM | Y.-J. Chang C.-C. Chen M. Coban K. Cui P. Garus N. Hu M. Karczewicz P.-H. Lin X. Meng B. Ray V. Seregin Y. Shao G. Verba H. Wang E. Ye R. Yu Y. Zhang Z. Zhang W. Zhu (Qualcomm) |
| JVET-AN0272 | m74352 | 2025-09-28 20:54:40 | 2025-09-28 23:11:55 | 2025-10-03 18:33:18 | AHG17: ECM comparison points for CfE | K. Andersson E. François S. Hong Y. Kidani F. Le Léannec X. Li R.-L. Liao Z. Lyu F. Pu V. Seregin H.-B. Teo A. Tissier G. Verba F. Wang J.-R. Ohm M. Wien F. Bossen M. Abdoli E. Alshina V. Baroncini J. Chen R. Chernyak Z. Deng P. de Lagrange L. Li P. Nikitin D. Rusanovskyy |
| JVET-AN0273 | m74353 | 2025-09-28 20:54:46 |  |  | Withdrawn |  |
| JVET-AN0274 | m74376 | 2025-09-29 06:10:24 | 2025-09-29 06:16:55 | 2025-10-04 08:59:40 | Crosscheck of JVET-AN0121 (EE2-1.1: On partitioning optimization) | Z. Deng (Bytedance) |
| JVET-AN0275 | m74391 | 2025-09-29 07:48:34 | 2025-10-01 06:30:39 | 2025-10-04 19:03:29 | EE2-2.5a: Combination of tests EE2-2.1, EE2-2.3b, and EE2-2.4 | V. Rufitskiy T. Dong A. Filippov (TCL) S. Blasi G. Kulupana D. Buğdayci Sansli J. Lainema (Nokia) K. Naser G. Rath F. Le Leannec T. Dumas (InterDigital) |
| JVET-AN0276 | m74392 | 2025-09-29 08:08:32 | 2025-10-05 05:27:10 | 2025-10-05 05:27:10 | Crosscheck of JVET-AN0092 (EE2-3.3: Joint reordering of GPM with intra prediction) | X. Li (Alibaba) |
| JVET-AN0277 | m74404 | 2025-09-29 09:19:55 | 2025-10-04 09:12:11 | 2025-10-06 10:18:27 | Crosscheck of JVET-AN0201 (EE1-4.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM)) | T. Yang W.-X. He Y.-Q. Zhu X.-T. Xie J.-S. Gong Q.-M. Wang Q. Liu (HUST) Z.-Y. Lv (vivo) |
| JVET-AN0278 | m74407 | 2025-09-29 09:42:33 | 2025-10-01 20:31:36 | 2025-10-01 20:31:36 | Crosscheck report for JVET-AN0090 (EE2-2.6: Adaptive subsampling filter selection for CCLM/CCCM) | D. Buğdayci Sansli (Nokia) |
| JVET-AN0279 | m74409 | 2025-09-29 09:56:45 | 2025-10-04 06:26:46 | 2025-10-04 06:26:46 | Crosscheck of JVET-AN0234 (Non-EE2: On filtering condition for angular modes) | L. Xu (OPPO) |
| JVET-AN0280 | m74410 | 2025-09-29 09:57:16 | 2025-10-04 06:27:14 | 2025-10-04 06:27:14 | Crosscheck of JVET-AN0192 (Non-EE2: Modification of reconstructed area for EIP) | L. Xu (OPPO) |
| JVET-AN0281 | m74427 | 2025-09-29 11:03:04 | 2025-10-10 18:00:43 | 2025-10-11 17:09:18 | Crosscheck of JVET-AN0127 (EE1-1.1: LOP with Overlapped Feature Integration) | Z. Xu (TCL) |
| JVET-AN0282 | m74429 | 2025-09-29 11:19:16 | 2025-10-10 10:03:08 | 2025-10-10 10:03:08 | Crosscheck of JVET-AN0153 (EE1-2.2: Decomposed Content-Adaptive VLOP4) | J. Chi A. Li Y. Du C. Zhu L. Luo H. Guo (UESTC) Y. Huo Y. Liu Z. Zhang (Transsion) |
| JVET-AN0283 | m74430 | 2025-09-29 11:25:12 | 2025-10-06 11:26:01 | 2025-10-06 11:26:01 | Crosscheck of JVET-AN0155 (Non-EE2: On improving adaptive GPM blending) | M. Abdoli (Xiaomi) |
| JVET-AN0284 | m74431 | 2025-09-29 11:27:25 | 2025-10-06 09:16:20 | 2025-10-06 09:16:20 | Crosscheck of JVET-AN0112 (Non-EE2: Coding of a coefficient level in TSRC) | M. Abdoli (Xiaomi) |
| JVET-AN0285 | m74492 | 2025-09-29 15:48:54 |  |  | Withdrawn |  |
| JVET-AN0286 | m74502 | 2025-09-29 16:29:46 | 2025-09-29 16:51:57 | 2025-10-05 20:53:41 | Non-EE2: TMRL Angle Offset Refinement | G. Rath K. Naser F. Le Léannec T. Dumas (InterDigital) |
| JVET-AN0287 | m74506 | 2025-09-29 16:37:04 | 2025-09-29 16:40:55 | 2025-10-02 01:14:38 | Non-EE2: Combined Angular- and Gradient-PDPC | G. Kulupana S. Blasi N. Neumann J. Lainema (Nokia) |
| JVET-AN0288 | m74526 | 2025-09-29 17:46:18 | 2025-10-03 11:10:59 | 2025-10-05 08:30:11 | Crosscheck of JVET-AN0120 (EE2-2.3b: Interpolation filter unification for unwrapping and TIMD template generation) | M. Abdoli (Xiaomi) |
| JVET-AN0289 | m74527 | 2025-09-29 17:46:51 | 2025-10-03 11:12:04 | 2025-10-05 08:30:22 | Crosscheck of JVET-AN0158 (EE2-2.4: Longer tap interpolation filtering) | M. Abdoli (Xiaomi) |
| JVET-AN0290 | m74572 | 2025-09-29 22:46:44 |  |  | Withdrawn |  |
| JVET-AN0291 | m74573 | 2025-09-29 22:46:45 | 2025-10-01 23:00:28 | 2025-10-01 23:00:28 | Crosscheck of JVET-AN0095 (EE2-4.2a*, b*, c*: Shifting Quantization Center for non-CTC) | M. Balcilar (Ofinno) |
| JVET-AN0292 | m74589 | 2025-09-30 01:28:26 | 2025-10-01 02:22:29 | 2025-10-01 23:31:18 | Multiview HEVC Reference SW implementation status | S. Choi S. Paluri D. Podborski A. M. Tourapis (Apple) |
| JVET-AN0293 | m74590 | 2025-09-30 01:29:46 | 2025-10-01 02:22:51 | 2025-10-01 23:31:37 | Multiview 4:4:4 profiles for HEVC | S. Choi S. Paluri D. Podborski E. Asbun A. M. Tourapis (Apple) |
| JVET-AN0294 | m74591 | 2025-09-30 01:40:47 | 2025-10-12 13:35:11 | 2025-10-12 13:35:11 | Crosscheck of JVET-AN0052 (Performance of GOP based RPR under CfE test conditions) | L. Li J. Park K. Choi (Samsung) |
| JVET-AN0295 | m74596 | 2025-09-30 02:46:17 | 2025-10-04 22:38:14 | 2025-10-04 22:38:14 | Crosscheck of JVET-AN0098 (EE2-5.1: On regularization of ALF-CCCM) | M. Jia (ZTE) |
| JVET-AN0296 | m74606 | 2025-09-30 07:44:48 | 2025-10-01 09:55:26 | 2025-10-04 15:13:43 | Crosscheck of JVET-AN0120 (EE2-2.3a/ab: Interpolation filter unification for unwrapping and TIMD template generation) | S. Lee Y. Kim S. Noh J. Bang H. Choi (HNU) |
| JVET-AN0297 | m74610 | 2025-09-30 08:41:13 | 2025-10-06 08:25:40 | 2025-10-06 08:25:40 | Crosscheck of JVET-AN0164 (Non-EE2: ALF coefficient coding group extension) | V. Shchukin (Ericsson) |
| JVET-AN0298 | m74624 | 2025-09-30 14:50:21 | 2025-10-11 03:03:53 | 2025-10-12 12:38:32 | AHG4: proposed updates for VVC multi-layer verification test plan | P. de Lagrange (InterDigital) |
| JVET-AN0299 | m74628 | 2025-09-30 16:00:57 | 2025-09-30 16:06:11 | 2025-10-05 12:06:19 | Non-EE2: Improvement for candidates on MPM | W. Niu Z. Li S. Xie M. Jia Y. Bai C. Huang (ZTE) |
| JVET-AN0300 | m74634 | 2025-09-30 17:07:56 | 2025-10-03 11:00:31 | 2025-10-03 11:00:31 | Cross-check of JVET-AN0199 (EE2-5.6: Reuse of TALF control information) | I. Jumakulyyev (Nokia) |
| JVET-AN0301 | m74660 | 2025-10-01 01:28:05 | 2025-10-01 01:32:14 | 2025-10-09 17:54:53 | AhG17 Sharing experience of conducting JPEG AI Call for Proposals | E. Alshina (Huawei) |
| JVET-AN0302 | m74671 | 2025-10-01 08:19:36 | 2025-10-05 08:20:32 | 2025-10-05 08:20:32 | Crosscheck of JVET-AN0171 (EE2-2.1: TMRL blend) | V. Rufitskiy (TCL) Y. Lu (SYSU) |
| JVET-AN0303 | m74672 | 2025-10-01 09:46:50 | 2025-10-01 09:48:45 | 2025-10-01 09:48:45 | External memory bandwidth evaluation for CfE response JVET-AN0267 | C. Feldmann (Nokia) |
| JVET-AN0304 | m74679 | 2025-10-01 13:46:58 | 2025-10-02 16:51:09 | 2025-10-12 13:45:43 | Crosscheck of JVET-AN0193 (EE1-3.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization) | N. Bhaskar (Huawei) |
| JVET-AN0305 | m74680 | 2025-10-01 13:48:46 | 2025-10-02 21:08:27 | 2025-10-10 16:39:47 | Crosscheck of JVET-AN0195 (EE1-3.5: Retrained DRF in NNVC-14.0) | N. Bhaskar (Huawei) |
| JVET-AN0306 | m74681 | 2025-10-01 13:55:24 | 2025-10-04 08:13:38 | 2025-10-04 08:13:38 | Crosscheck of JVET-AN0084 (EE1-3.2: RA/LDB Unified Reference Frame Synthesis for VVC Inter Coding) | N. Bhaskar (Huawei) |
| JVET-AN0307 | m74682 | 2025-10-01 14:14:35 | 2025-10-05 15:15:22 | 2025-10-10 10:25:50 | Crosscheck of JVET-AN0109 (EE2-4.1-related: On Linked Sign Prediction) | C. Hollmann (TCL) |
| JVET-AN0308 | m74683 | 2025-10-01 14:34:29 | 2025-10-04 22:50:32 | 2025-10-04 22:50:32 | Crosscheck of JVET-AN0275 (EE2-2.5a: Combination of tests EE2-2.1, EE2-2.3b, and EE2-2.4) | P. Andrivon (Ofinno) |
| JVET-AN0309 | m74685 | 2025-10-01 15:45:36 | 2025-10-04 06:26:16 | 2025-10-04 06:26:16 | Crosscheck of JVET-AN0299 (Non-EE2: Improvement for candidates on MPM) | L. Xu (OPPO) |
| JVET-AN0310 | m74689 | 2025-10-01 20:11:29 | 2025-10-01 20:16:09 | 2025-10-01 20:16:09 | AHG9: On PRI ID | Y. Li (SJTU) Y.-K. Wang (Bytedance) K. Yang Y. Xu (SJTU) |
| JVET-AN0311 | m74695 | 2025-10-01 21:38:26 | 2025-10-05 10:00:19 | 2025-10-05 10:00:19 | Crosscheck of JVET-AN0095 (EE2-4.2a*, b*, c*: Shifting Quantization Center for non-CTC) | B. Ray (Qualcomm) |
| JVET-AN0312 | m74708 | 2025-10-02 10:04:10 | 2025-10-02 10:56:11 | 2025-10-11 09:30:20 | Crosscheck of JVET-AN0094 (EE2-4.1: Linked Sign Prediction) | C. Hollmann (TCL) L. Xu (SYSU) |
| JVET-AN0313 | m74710 | 2025-10-02 10:10:24 | 2025-10-03 15:21:07 | 2025-10-04 10:16:48 | Crosscheck of JVET-AN0154 (Non-EE2: Position-dependent Weighted Prediction Extension) | S. Blasi (Nokia) |
| JVET-AN0314 | m74715 | 2025-10-02 12:50:58 | 2025-10-03 09:48:13 | 2025-10-03 17:22:07 | crosscheck of JVET-AN0236 EE2-3.1: Generated Merge Candidates | K. Naser (InterDigital) |
| JVET-AN0315 | m74730 | 2025-10-02 13:02:42 | 2025-10-03 12:20:44 | 2025-10-03 12:20:44 | Crosscheck of JVET-AN0091 (EE2-3.2: Joint reordering of GPM with affine prediction) | Y. Wang (Bytedance) |
| JVET-AN0316 | m74734 | 2025-10-02 17:25:23 | 2025-10-02 18:02:52 | 2025-10-02 18:02:52 | Crosscheck of JVET-AN0175 (EE2-related: Unbiased Simple Quantizer) | M. Le Pendu F. Le Léannec (InterDigital) |
| JVET-AN0317 | m74735 | 2025-10-02 19:25:25 | 2025-10-02 19:29:42 | 2025-10-06 11:16:59 | AHG7: Preliminary tool analysis with the criteria in JVET-AM0042: Update on IntraTMP | K. Naser T. Dumas M. Radosavljević (InterDigital) |
| JVET-AN0318 | m74736 | 2025-10-02 19:45:14 | 2025-10-02 19:49:04 | 2025-10-07 14:19:46 | AHG17: Review of CfE responses | K. Naser F. Le Léannec F. Galpin E. François (InterDigital) |
| JVET-AN0319 | m74737 | 2025-10-02 20:14:03 | 2025-10-02 20:20:23 | 2025-10-03 09:34:16 | EE1-related: FlowWarp Operator for DRF Integer Inference Optimization | N. Fu X. Chen L. Qin W. Zhang Z. Chen (Wuhan Univ.) |
| JVET-AN0320 | m74738 | 2025-10-02 20:14:09 |  |  | Withdrawn |  |
| JVET-AN0321 | m74740 | 2025-10-03 05:22:16 | 2025-10-05 12:44:23 | 2025-10-05 12:44:23 | Crosscheck of JVET-AN0191 (Non-EE2: Binarization Improvement of GPM) | K. Jia (Alibaba) |
| JVET-AN0322 | m74741 | 2025-10-03 07:19:29 | 2025-10-04 21:15:47 | 2025-10-05 08:22:02 | Crosscheck of JVET-AN106 (EE2-5.5: The combinations of EE2-5.1, EE2-5.2, EE2-5.3 and EE2-5.4) | P. Astola (Nokia) |
| JVET-AN0323 | m74743 | 2025-10-03 08:35:55 | 2025-10-05 05:28:24 | 2025-10-05 05:28:24 | Crosscheck of JVET-AN0093 (EE2-3.4: Combination of EE2-3.2 and EE2-3.3) | X. Li (Alibaba) |
| JVET-AN0324 | m74745 | 2025-10-03 11:03:12 | 2025-10-03 11:20:20 | 2025-10-04 16:26:24 | EE2-4.2i-related: Unbiased Simple Quantizer with Quantization Center Shifting | M. Le Pendu F. Le Léannec (InterDigital) |
| JVET-AN0325 | m74746 | 2025-10-03 12:02:10 | 2025-10-06 10:17:46 | 2025-10-06 10:17:46 | Crosscheck of JVET-AN0125 (Non-EE2: On CABAC bin budget constraint) | P. Nikitin (Xiaomi) |
| JVET-AN0326 | m74747 | 2025-10-03 12:02:15 |  |  | Withdrawn |  |
| JVET-AN0327 | m74749 | 2025-10-03 13:10:27 | 2025-10-06 13:42:54 | 2025-10-06 13:42:54 | Crosscheck of JVET-AN0099 (EE2-5.2: On ALF-CCCM) | W. Yin (Bytedance) |
| JVET-AN0328 | m74750 | 2025-10-03 13:39:07 | 2025-10-03 13:47:57 | 2025-10-03 13:47:57 | Crosscheck of JVET-AN0121 (EE2-1.1a: On partitioning optimization) | Y. Liu Y. Huo Z.Zhang (Transsion) |
| JVET-AN0329 | m74752 | 2025-10-03 14:15:29 | 2025-10-04 12:53:41 | 2025-10-04 12:53:41 | Crosscheck of JVET-AN0168 and JVET-AN0173 (EE2-2.8a, b: Enhanced CCP Fusion mode) | Y. Kidani (KDDI) |
| JVET-AN0330 | m74753 | 2025-10-03 14:16:11 | 2025-10-03 14:19:02 | 2025-10-03 14:19:02 | Proposed modification of CfE test conditions | E. Alshina (Huawei) |
| JVET-AN0331 | m74754 | 2025-10-03 14:17:29 | 2025-10-05 00:27:47 | 2025-10-05 00:27:47 | Crosscheck of JVET-AN0113 (Non-EE2: Additional candidates for merge mode in ALF-CCCM) | H.-J. Jhu (Kwai) |
| JVET-AN0332 | m74755 | 2025-10-03 14:18:57 | 2025-10-08 07:51:30 | 2025-10-08 07:51:30 | Crosscheck of JVET-AN0093 (EE2-3.4: Combination of EE2-3.2 and EE2-3.3) | C. Ma (Kwai) |
| JVET-AN0333 | m74756 | 2025-10-03 14:23:08 | 2025-10-08 08:39:48 | 2025-10-08 08:39:48 | Crosscheck of JVET-AN0101 (EE2-5.3: Updated multi-models usage strategy for ALF-CCCM) | C. Ma (Kwai) |
| JVET-AN0334 | m74757 | 2025-10-03 14:24:35 | 2025-10-08 08:50:34 | 2025-10-08 08:50:34 | Crosscheck of JVET-AN0103 (EE2-5.4: On ALF-CCCM Model) | C. Ma (Kwai) |
| JVET-AN0335 | m74759 | 2025-10-03 15:01:13 | 2025-10-03 17:06:32 | 2025-10-09 21:16:28 | Cross-check of JVET-AN0083 (EE1-1.2: backbone block enhancement of LOP in-loop filter with over-parameterized training and variable channels) | T. Dumas (InterDigital) |
| JVET-AN0336 | m74760 | 2025-10-03 15:16:26 | 2025-10-03 15:29:54 | 2025-10-03 15:29:54 | Cross-check of JVET-AN0110 (Non-EE2: Enhancement of Non-CCP Reordering List) | Z. Lyu (vivo) |
| JVET-AN0337 | m74764 | 2025-10-03 17:59:05 | 2025-10-05 10:00:09 | 2025-10-05 10:00:09 | Crosscheck of JVET-AN0163 (AHG12: On improving the bi-filtering modes in TALF) | N. Song (OPPO) |
| JVET-AN0338 | m74767 | 2025-10-03 21:12:02 | 2025-10-11 11:17:09 | 2025-10-11 11:17:09 | Crosscheck of JVET-AN0129 (EE2-4.3: Residual Sign Prediction restriction) | Y. Zhang (OPPO) |
| JVET-AN0339 | m74768 | 2025-10-03 21:21:12 | 2025-10-03 21:27:14 | 2025-10-03 21:27:14 | Crosscheck of JVET-AM0195 (AHG18: Gradual Decoding Refresh under ultra-low latency test scenario) | E. Mora (VITEC) |
| JVET-AN0340 | m74769 | 2025-10-03 21:30:46 | 2025-10-03 21:35:57 | 2025-10-03 21:35:57 | AHG11: Neural network in-loop filter usage based on activation map | M. Santamaria D. Buğdayci Sansli F. Cricri (Nokia) |
| JVET-AN0341 | m74770 | 2025-10-03 21:34:16 | 2025-10-03 21:36:43 | 2025-10-07 10:27:13 | Influence of colour transformation on codec performance | M. Domanski A. Grzelka O. Stankiewicz (Poznan Univ. Tech.) |
| JVET-AN0342 | m74771 | 2025-10-03 23:08:01 | 2025-10-04 03:32:47 | 2025-10-04 03:32:47 | AHG9: On the neural-network post-filter activation SEI message in VSEI v4 | J. Xu Y.-K. Wang (Bytedance) |
| JVET-AN0343 | m74772 | 2025-10-04 06:23:25 | 2025-10-04 18:05:07 | 2025-10-04 18:05:07 | Crosscheck of JVET-AN0114 (Non-EE2: Local boosting CCP for ALF-CCCM) | Z. Deng (Bytedance) |
| JVET-AN0344 | m74775 | 2025-10-04 09:54:18 | 2025-10-04 10:32:45 | 2025-10-09 11:44:36 | Crosscheck of JVET-AN0121 (EE2-1.1: On partitioning optimization) | Z. Sun (OPPO) |
| JVET-AN0345 | m74776 | 2025-10-04 10:19:12 | 2025-10-07 16:32:20 | 2025-10-07 16:32:20 | Crosscheck of JVET-AN0199 (EE2-5.6: Reuse of TALF control information) | Z. Xie L. Xu (OPPO) |
| JVET-AN0346 | m74777 | 2025-10-04 10:21:13 | 2025-10-07 16:32:44 | 2025-10-07 16:32:44 | Crosscheck of JVET-AN0123 (Non-EE2: Improvement on chroma MPM) | Z. Xie Z. Liu (OPPO) |
| JVET-AN0347 | m74778 | 2025-10-04 10:22:31 | 2025-10-07 16:33:05 | 2025-10-07 16:33:05 | Crosscheck of JVET-AN0130 (Non-EE2: Improvement on Frame-level Inheritance Mode in ALF-CCCM) | Z. Xie (OPPO) |
| JVET-AN0348 | m74779 | 2025-10-04 13:28:49 | 2025-10-04 13:36:54 | 2025-10-04 13:36:54 | AHG10: On reporting metrics for RPR | K. Andersson (Ericsson) |
| JVET-AN0349 | m74781 | 2025-10-04 20:18:09 | 2025-10-05 14:28:33 | 2025-10-05 14:29:27 | Crosscheck of JVET-AN0169 (Non-EE2: On inter multiple transform set selection and advanced SBT) | Y. Yin S. Wan (NWPU) |
| JVET-AN0350 | m74782 | 2025-10-04 20:27:40 | 2025-10-05 12:01:49 | 2025-10-05 12:01:49 | Crosscheck of JVET-AN0086 (AHG12: Bug Fix for ALF Coefficient Calculation) | Z. Zhu S. Wan (NWPU) |
| JVET-AN0351 | m74783 | 2025-10-04 20:29:01 | 2025-10-05 12:03:17 | 2025-10-05 12:03:17 | Crosscheck of JVET-AN0087 (AHG12: Budget Control for Transform Skip Residual Coding) | Z. Zhu S. Wan (NWPU) |
| JVET-AN0352 | m74784 | 2025-10-04 20:37:44 | 2025-10-06 15:32:17 | 2025-10-06 15:32:17 | Crosscheck of JVET-AN0118 (AHG6: ECM software extension for RDO count information) | B. Wang (Tencent) |
| JVET-AN0353 | m74786 | 2025-10-05 09:07:45 | 2025-10-05 09:19:40 | 2025-10-05 14:36:53 | Crosscheck of JVET-AN0121 (EE2-1.1f: On partitioning optimization) | Y. Liu Y. Huo Z. Zhang (Transsion) |
| JVET-AN0354 | m74790 | 2025-10-05 10:47:07 | 2025-10-05 11:14:56 | 2025-10-05 11:14:56 | Crosscheck of JVET-AN0200 (Non-EE2: On improving CIIP Fusion) | Z. Zhang (Alibaba) |
| JVET-AN0355 | m74791 | 2025-10-05 14:15:53 |  |  | Withdrawn |  |
| JVET-AN0356 | m74794 | 2025-10-05 18:14:52 | 2025-10-05 19:26:38 | 2025-10-05 19:26:38 | Crosscheck of JVET-AN0111 (Non-EE2: additional candidates for regular inter AMVP candidate list) | S. Hong (Nokia) |
| JVET-AN0357 | m74797 | 2025-10-05 21:38:38 | 2025-10-05 21:45:28 | 2025-10-05 21:45:28 | AHG9: On prespecified colour maps for the colour mapping information SEI message | J. Samuelsson-Allendes S. Deshpande (Sharp) |
| JVET-AN0358 | m74800 | 2025-10-06 08:05:19 | 2025-10-06 08:21:21 | 2025-10-06 08:21:21 | Crosscheck of JVET-AN0266 (Pairwise Merge Candidates with Motion Vector Scaling) | C. Wang (OPPO) |
| JVET-AN0359 | m74802 | 2025-10-06 09:38:42 |  |  | crosscheck of JVET-AN0287 Non-EE2: Combined Angular- and Gradient-PDPC | K. Naser (InterDigital) |
| JVET-AN0360 | m74807 | 2025-10-06 15:32:58 | 2025-10-06 15:38:58 | 2025-10-07 11:22:29 | AHG17: Information on coding efficiency and complexity | L. Li W. Choi M. Park M. Budagavi K. Choi (Samsung) |
| JVET-AN0361 | m74820 | 2025-10-07 09:23:11 | 2025-10-08 08:36:21 | 2025-10-08 08:36:21 | AHG17: Report on CfE testing | M. Wien F. Bossen C. Lehmann V. Baroncini |
| JVET-AN0362 | m74821 | 2025-10-07 09:31:36 | 2025-10-07 11:30:26 | 2025-10-07 11:30:26 | Information on the impact of GOP-based RPR on JVET-AN0267 in context of CfE repsonses | C. Lehmann A. Wieckowski B. Bross |
| JVET-AN0363 | m74845 | 2025-10-08 09:59:52 | 2025-10-08 13:04:22 | 2025-10-08 13:04:22 | Interface text for SEI message extensions | J. Boyce (Nokia) Y. Sanchez K. Sühring (Fraunhofer HHI) |
| JVET-AN0364 | m74847 | 2025-10-08 11:46:24 | 2025-10-09 09:10:16 | 2025-10-09 09:10:16 | BoG report on Tool Complexity Analysis | X. Li Y. Zhao |
| JVET-AN0365 | m74853 | 2025-10-08 14:25:58 | 2025-10-08 14:32:09 | 2025-10-08 16:21:06 | AHG9: On backward compatibility of multi-purpose NNPFs | M. M: Hannuksela F. Cricri (Nokia) H. Tan (LGE) |
| JVET-AN0366 | m74854 | 2025-10-08 15:25:49 | 2025-10-08 15:27:22 | 2025-10-09 12:07:15 | AHG9: Purpose indicators for Display Overlays Information SEI message | G. Teniou S. Wenger A. Hinds (Tencent) T. Biatek J. Boyce M. M. Hannuksela (Nokia) |
| JVET-AN0367 | m74858 | 2025-10-08 19:34:16 | 2025-10-08 21:45:55 | 2025-10-10 14:53:45 | AHG9: On design of PRI SEI message | H. Tan J. Lee (LGE) Y.-K Wang J. Xu (Bytedance) J. Boyce (Nokia) |
| JVET-AN0368 | m74860 | 2025-10-09 01:17:35 | 2025-10-09 01:20:04 | 2025-10-09 01:20:04 | AHG9: On semantics of ifm_type_id for IFM SEI | A. Hinds S. Wenger G. Teniou (Tencent) |
| JVET-AN0369 | m74870 | 2025-10-09 12:15:19 |  |  | Crosscheck of JVET-AN0203 (AHG7: On RGPM moved into group-2) | P. Nikitin (Xiaomi) |
| JVET-AN0370 | m74871 | 2025-10-09 14:11:30 | 2025-10-09 14:14:12 | 2025-10-09 16:31:31 | AHG9: Digitally signed content DSC): On subpicture signing | I. Sodagar (Dolby) M. Pettersson (Ericsson) J. Lee (LGE) |
| JVET-AN0371 | m74873 | 2025-10-09 16:10:12 |  |  | Withdrawn |  |
| JVET-AN0372 | m74875 | 2025-10-09 17:11:54 |  |  | Withdrawn |  |
| JVET-AN0373 | m74884 | 2025-10-10 05:18:38 | 2025-10-10 05:22:53 | 2025-10-10 10:07:26 | AHG11: Efficient Depthwise Separable Backbone Network for Neural Network in-Loop Filter | Y. Shi J. Wang Y. Zuo P. Wang X. Chen H. Zhu (BJUT) B. Li P. Han F. Xing Z. Wang (Hisense) |
| JVET-AN0374 | m74886 | 2025-10-10 09:21:37 | 2025-10-10 09:24:19 | 2025-10-10 09:36:05 | Crosscheck of JVET-AN0121 (EE2-1.1g: On partitioning optimization) | Z. Zhang Y. Huo Y. Liu (Transsion) |
| JVET-AN0375 | m74903 | 2025-10-10 11:56:30 | 2025-10-10 12:26:47 | 2025-10-10 12:26:47 | Crosscheck of JVET-AN0120 (EE2-2.3d: Combination of tests EE2-2.3b and EE2-2.4) | G. Verba (Qualcomm) |
| JVET-AN0376 | m74913 | 2025-10-10 16:41:35 | 2025-10-10 17:01:09 | 2025-10-10 17:01:09 | Crosscheck of JVET-AN0319 (EE1-related: FlowWarp Operator for DRF Integer Inference Optimization) | N. Bhaskar (Huawei) |
| JVET-AN0377 | m74931 | 2025-10-11 14:18:24 | 2025-10-12 12:43:18 | 2025-10-12 12:57:17 | Report on dry-run expert viewing for the verification tests for VVC multilayer coding | M. Wien C. Lehmann P. de Lagrange |
| JVET-AN0378 | m74933 | 2025-10-12 11:24:28 | 2025-10-12 11:29:11 | 2025-10-12 11:29:11 | Suggested Draft of Complexity Reporting Template for Coding Algorithms and Tools | X. Li |
| JVET-AN1000 | m74934 | 2025-10-13 11:29:13 |  |  | Meeting Report of the 40th JVET Meeting | J.-R. Ohm |
| JVET-AN1005 | m74935 | 2025-10-13 11:30:10 | 2025-11-03 23:37:19 | 2025-11-03 23:37:19 | Future CICP extensions (Draft 2) | J. Boyce E. Thomas A. Tourapis |
| JVET-AN1016 | m74936 | 2025-10-13 11:31:28 |  |  | AVC with extensions and corrections (Draft 4) | K. Sühring J. Boyce G. J. Sullivan Y.-K. Wang |
| JVET-AN1017 | m74937 | 2025-10-13 11:32:32 | 2025-10-26 22:55:18 | 2025-10-26 22:55:18 | Support for additional SEI messages in AVC (Draft 4) | K. Sühring J. Boyce G. J. Sullivan Y.-K. Wang |
| JVET-AN1018 | m74938 | 2025-10-13 11:33:29 | 2025-10-17 20:51:10 | 2025-12-13 00:08:27 | HEVC with extensions and corrections (Draft 3) | Y.-K. Wang B. Bross S. Deshpande G. J. Sullivan A. Tourapis |
| JVET-AN1019 | m74939 | 2025-10-13 11:34:08 | 2025-10-17 21:54:17 | 2025-10-17 21:54:17 | VSEI with extensions and corrections (Draft 1) | J. Boyce J. Chen S. Deshpande M. M. Hannuksela S. McCarthy G. J. Sullivan H. Tan Y.-K. Wang |
| JVET-AN2001 | m74940 | 2025-10-13 11:34:56 | 2025-10-17 20:55:35 | 2025-12-15 23:53:45 | VVC with extensions and corrections (Draft 1) | Y.-K. Wang B. Bross M. M. Hannuksela G. J. Sullivan |
| JVET-AN2008 | m74941 | 2025-10-13 11:37:49 | 2026-01-01 12:25:05 | 2026-01-01 12:25:05 | Conformance testing for versatile video coding (Draft 8) | I. Moccagatta F. Bossen S. Iwamura P. de Lagrange |
| JVET-AN2019 | m74942 | 2025-10-13 11:40:50 | 2025-12-13 09:56:45 | 2025-12-13 09:56:45 | Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC) | F. Galpin Yue Li Yun Li D. Rusanovskyy T. Shao J. Ström L. Wang |
| JVET-AN2020 | m74943 | 2025-10-13 11:41:39 |  |  | Film grain synthesis technology for video applications ed. 2 (Draft 4) | W. Husak P. de Lagrange A. Norkin M. Radosavljević A. Tourapis |
| JVET-AN2021 | m74944 | 2025-10-13 11:42:16 |  |  | Verification test plan for VVC multilayer coding (update 7) | O. Chubach P. de Lagrange M. Wien |
| JVET-AN2023 | m74930 | 2025-10-11 12:57:53 | 2025-10-11 12:59:02 | 2025-10-26 16:12:39 | Exploration experiment on neural network-based video coding (EE1) | E. Alshina R. Chang F. Galpin Y. Li Y. Li M. Santamaria T. Shao J. Ström Z. Xie |
| JVET-AN2024 | m74932 | 2025-10-12 09:21:17 | 2025-10-12 09:48:59 | 2025-10-12 09:48:59 | Exploration experiment on enhanced compression beyond VVC capability (EE2) | V. Seregin D. Buğdayci Sansli J. Chen R. Chernyak K. Naser J. Ström F. Wang M. Winken X. Xiu K. Zhang |
| JVET-AN2025 | m74945 | 2025-10-13 11:43:46 | 2025-12-15 23:38:43 | 2025-12-15 23:38:43 | Algorithm description of Enhanced Compression Model 19 (ECM 19) | M. Coban R.-L. Liao K. Naser J. Ström L. Zhang |
| JVET-AN2026 | m74946 | 2025-10-13 11:48:50 | 2025-10-13 14:32:28 | 2025-10-13 14:32:28 | Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC | J.-R. Ohm M. Wien F. Bossen |
| JVET-AN2029 | m74947 | 2025-10-13 11:56:27 | 2025-12-20 10:41:43 | 2025-12-20 10:41:43 | Results of the Joint Call for Evidence on video compression with capability beyond VVC | V. Baroncini F. Bossen J.-R. Ohm M. Wien |
| JVET-AN2032 | m74948 | 2025-10-13 12:03:15 | 2025-11-19 20:04:03 | 2025-12-03 21:21:35 | Technologies under consideration for future extensions of VSEI (version 10) | S. McCarthy J. Boyce J. Chen S. Deshpande M. M. Hannuksela H. Tan Y.-K. Wang |
| JVET-AN2039 | m74949 | 2025-10-13 12:04:03 | 2025-11-01 02:35:44 | 2025-12-24 20:32:30 | Common test conditions for technology targeting ultra low latency and packet loss resilience | S. Ikonin S. Deshpande V. Zakharchenko |
| JVET-AN2040 | m74950 | 2025-10-13 12:04:36 | 2025-12-02 18:54:59 | 2025-12-02 18:54:59 | Draft of complexity reporting template for coding algorithms and tools | X. Li E. Alshina I. Moccagatta |
| JVET-AN2041 | m74951 | 2025-10-13 12:05:08 | 2025-10-23 10:39:30 | 2025-10-23 10:39:30 | Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026 | J.-R. Ohm M. Wien |

# Annex B1 to JVET report:
List of meeting participants attending in person

The participants who were personally present at the meeting site of the fortieth meeting of the JVET, according to a sign-in sheet circulated in the JVET meeting rooms (approximately 295 people in total), were as follows:

- Mohsen Abdoli (Xiaomi – FR)

- Kiyofumi Abe (Panasonic – JP)

- Mouad Aderdor (Huawei – DE)

- Elena Alshina (Huawei – DE)

- Alireza Aminlou (Nokia – FI)

- Kenneth Andersson (Ericsson – SE)

- Pierre Andrivon (Ofinno – US)

- Jeeva Raj Arumugam (Ittiam – IN)

- Yaxian Bai (ZTE – CN)

- Muhammet Balcilar (Ofinno – US)

- Giovanni Ballocca (Sisvel – IT)

- Manu Bansal (Dolby – US)

- Thibaud Biatek (Nokia – FI)

- Saverio Blasi (Nokia – UK)

- Médéric Blestel (Ofinno – US)

- Frank Bossen (Bossen Technologies – CA)

- Jill Boyce (Nokia – US)

- Jens Brandenburg (FhG HHI – DE)

- Benjamin Bross (FhG HHI – DE)

- Madhukar Budagavi (Samsung – US)

- Döne Buğdayci Sansli (Nokia – FI)

- Janko Calic (BBC – UK)

- Alessia Casella (Sisvel – IT)

- Bolin Chen (Alibaba – CN)

- Guan-Hao Chen (MediaTek – US)

- Jie Chen (Alibaba – CN)

- Lulin Chen (MediaTek – US)

- Yi-Wen Chen (MediaTek – US)

- Roman Chernyak (Tencent – US)

- Hyung Dong Cho (KHU – KR)

- Kwang Pyo Choi (Samsung – KR)

- Woong Il Choi (Samsung – KR)

- Keiichi Chono (NEC – JP)

- Tzu-Der Chuang (MediaTek – US)

- Olena Chubach (MediaTek – US)

- Takeshi Chujoh (KDDI – JP)

- Lorenzo Ciccarelli (V-Nova – UK)

- Tim Claßen (RWTH Aachen – DE)

- Philippe de Lagrange (InterDigital – FR)

- Claire-Hélène Demarty (InterDigital – FR)

- Franck Denoual (Canon – FR)

- Sachin Deshpande (Sharp – US)

- Weihang Ding (Huawei – CN)

- Quockhanh Dinh (Samsung – KR)

- Marek Domanski (PUT – PL)

- Tianyu Dong (TCL – CN)

- Virginie Drugeon (Panasonic – DE)

- Zheming Fan  (Sharp – JP)

- Christian Feldmann (Nokia – DE)

- Simone Ferrara (V-Nova – UK)

- Christof Fersch (Dolby – DE)

- Alexey Filippov (TCL – CN)

- Siegfried Fößel (FhG IIS – DE)

- Denis Fortin (Nokia – FI)

- Edouard François (InterDigital – FR)

- Per Fröjdh (Ericsson – SE)

- Nianxiang Fu (Wuhan Univ. – CN)

- John Funnell (Nokia – FI)

- Neeraj Gadgil (Samsung – IN)

- Franck Galpin (InterDigital – FR)

- Jonathan Gan (OPPO – AU)

- Jingying Gao (Panasonic – SG)

- Ramin Ghaznavi Youvalari (Xiaomi – FI)

- Vladimir Gritsenko (Ericsson – SE)

- Xiaojun Gu (Huawei – CN)

- Miska M. Hannuksela (Nokia – FI)

- Yong He (Qualcomm – US)

- Viktor Herrmann (Fraunhofer HHI – DE)

- Christopher Hollmann (TCL – CN)

- Myungoh Hong (LGE – KR)

- Seungwook Hong (Nokia – US)

- Sujun Hong (Sharp – JP)

- Reetu Hooda (Qualcomm – US)

- Hang Huang (OPPO – CN)

- Junnan Huang (Huawei – CN)

- Yu-Wen Huang (MediaTek – US)

- Junyan Huo (Xidian Univ. – CN)

- Yongkai Huo (Transsion – CN)

- Walt Husak (Dolby – US)

- Tomohiro Ikai (Sharp – JP)

- Masaru Ikeda (Sony – JP)

- Sergey Ikonin (Huawei – CN)

- Hitomi Imamura (Canon – JP)

- Ruri Imichi (Sharp – JP)

- Takaaki Ishikawa (Canon – JP)

- Ryo Ishimoto (Sharp – JP)

- Shunsuke Iwamura (NHK) – JP)

- Howon Jang (HNU – KR)

- Menghu Jia (ZTE – CN)

- Rajan Joshi (Samsung – US)

- Ikram Jumakulyyev (Nokia – DE)

- Joel Jung (Qualcomm – US)

- Maike Kaiser (RMU – DE)

- Kazuhiro Kamada (Mitsubishi – JP)

- Jung-Yang Kao (ITRI – US)

- Alexander Karabutov (Huawei – DE)

- Marta Karczewicz (Qualcomm – US)

- Kei Kawamura (KDDI – JP)

- Steve Keating (Sony – UK)

- Joachim Keinert (FhG IIS – DE)

- Louis Kerofsky (Qualcomm – US)

- Vyacheslav Khamidullin (Huawei – CN)

- Yoshitaka Kidani (KDDI – JP)

- Donghyun Kim (ETRI – KR)

- Hui Yong Kim (KHU – KR)

- Jongho Kim (ETRI – KR)

- Jungsun Kim (Apple – US)

- Kyuri Kim (KAU – KR)

- Mariya Kim (DE)

- Seonjae Kim (Dong-A Univ. – KR)

- Seung-Hwan Kim (LGE – US)

- Young Tae Kim (MSI – KR)

- Krasimir Kolarov (Apple – US)

- Jacek Konieczny (TCL – PL)

- Yiwei Kou (Huawei – DE)

- Philipp Kreowsky (FhG HHI – DE)

- Gosala Kulupana (Nokia – UK)

- Toshihiko Kusakabe (Panasonic – JP)

- Jani Lainema (Nokia – FI)

- Guillaume Laroche (Canon – FR)

- Nam Le (Nokia – FI)

- Fabrice Le Léannec (InterDigital – FR)

- Olivier Le Meur (InterDigital – FR)

- Jinho Lee (ETRI – KR)

- Jong Min Lee (KAU – KR)

- Jung-Kyung Lee (Ofinno – US)

- Soeun Lee (HNU – KR)

- Taesik Lee (Dong-A Univ. – KR)

- Yen-Wei Lee (Acer – US)

- Christian Lehmann (FhG HHI – DE)

- Shawmin Lei (MediaTek – US)

- Julien Lémotheux (ETSI – FR)

- Ling Li (Samsung – KR)

- Ming Li (OPPO – CN)

- Xiang Li (Google – US)

- Yiming Li (Tencent – CN)

- Zhengang Li (ZTE – CN)

- Kai-Wen Liang (Sharp – JP)

- Ru-Ling Liao (Alibaba – CN)

- Eugene Lim (SJU – KR)

- Jaehyun Lim (LGE – KR)

- Sung-Chang Lim (ETRI – KR)

- Sungwon Lim (KT – KR)

- Woong Lim (ETRI – KR)

- Youngkwon Lim (Samsung – KR)

- Ching-Chieh Lin (ITRI – US)

- Yi Hao Lin (Sharp – CN)

- Lukasz Litwic (Ericsson – SE)

- Ao Liu (Huawei – CN)

- Du Liu (Ericsson – SE)

- Po-Kai Liu (ITRI – US)

- Yutian Liu (Transsion – CN)

- Zhikai Liu (OPPO – CN)

- Zhuoyi Lyu (vivo – CN)

- Xiang Ma (Huawei – CN)

- Detlev Marpe (Fraunhofer HHI – DE)

- Cédric Mateu (ITU – CH)

- Ville-Veikko Mattila (Nokia – FI)

- Sean McCarthy (Dolby – US)

- Dominik Mehlem (RWTH Aachen – DE)

- Akira Minezawa (Mitsubishi – JP)

- Ken Miyamoto (Mitsubishi – JP)

- Gihwa Moon (KAU – KR)

- Elie Mora (Vitec – FR)

- Linus Müller (FhG IIS – DE)

- Luka Murn (Nokia – UK)

- Prathap M. Nair (Dolby – US)

- Hiroya Nakamura (JVC Kenwood – JP)

- Junghak Nam (LGE – KR)

- Karam Naser (InterDigital – FR)

- Shimpei Nemoto (NHK – JP)

- Nicolas Neumann (Nokia – DE)

- Tung Nguyen (FhG HHI – DE)

- Thuong Nguyen Canh (Dolby – US)

- Pavel Nikitin (Xiaomi – DE)

- Yuma Nishikawa (Panasonic – JP)

- Weihong Niu (ZTE – CN)

- Seungmin Noh (HNU – KR)

- Andrey Norkin (Netflix – US)

- Sejin Oh (Dolby – US)

- Jens-Rainer Ohm (RWTH Aachen – DE)

- Patrice Onno (Canon – FR)

- Nicolai Otto (MainConcept – DE)

- Krit Panusopone (Nokia – US)

- Johan Pardo (Huawei – DE)

- Jaeseung Park (SJU – KR)

- Jeeyoon Park (Samsung – KR)

- Min Woo Park (Samsung – KR)

- Naeri Park (LGE – KR)

- Sangcheol Park (MSI – KR)

- Seoyeon Park (EWU – KR)

- Martin Pettersson (Ericsson – SE)

- Jonathan Pfaff (FhG HHI – DE)

- Pierrick Philippe (Orange – FR)

- Yinji Piao (Samsung – KR)

- Sophie Pientka (FhG HHI – DE)

- Hongdong Qin (TCL – CN)

- Mohamad Raad (LIU – LB)

- Fabien Racapé (InterDigital – FR)

- Julien Ricard (Tencent – US)

- Justin Ridge (Nokia – FI)

- Christopher Rosewarne (Canon – AU)

- Vasily Rufitskiy (TCL – CN)

- Damian Ruiz Coll (Ofinno – US)

- Dmytro Rusanovskyy (Nokia – US)

- Changwoo Ryu (MSI – KR)

- Takahiro Saito (Canon – JP)

- Yuto Sakai (Canon – JP)

- Jonatan Samuelsson-Allendes (Sharp – US)

- Yago Sanchez de la Fuente (FhG HHI – DE)

- Maria Santamaria (Nokia – FI)

- Simon Sasse (FhG HHI – DE)

- Manuel Schneckenburger (Huawei – DE)

- Heiko Schwarz (FhG HHI – DE)

- Sebastian Schwarz (Nokia – DE)

- Andrew Segall (Amazon – US)

- Juyeon Seo (HNU – KR)

- Vadim Seregin (Qualcomm – US)

- Vladislav Shchukin (Ericsson – SE)

- Masato Shima (Canon – JP)

- Seoyoung Shin (KT – KR)

- Jay Nitin Shingala (Ittiam – IN)

- Robert Skupin (FhG HHI – DE)

- Iraj Sodagar (Dolby – US)

- Timofey Solovyev (Huawei – RU)

- Jianwen Song (Canon – AU)

- Noah Stegmaier (Ericsson – SE)

- Alan Stein (V-Nova – US)

- Jacob Ström (Ericsson – SE)

- Karsten Sühring (FhG HHI – DE)

- Toshiyasu Sugio (Panasonic – JP)

- Gary J. Sullivan (Dolby – US)

- Yule Sun (Huawei – CN)

- Zexing Sun (OPPO – CN)

- Teruhiko Suzuki (Sony – JP)

- Niclas Svensson (Ericsson – SE)

- Ryoya Taguchi (NEC – JP)

- Keiichiro Takada (Sharp – JP)

- Hendry Tan (LGE – US)

- Chih-Yu Teng (Sharp – CN)

- Hamed Rezazadegan Tavakoli (Nokia – FI)

- Gilles Teniou (Tencent – US)

- Han Boon Teo (Panasonic – SG)

- Emmanuel Thomas (Xiaomi – NL)

- Yasuaki Tokumo (Sharp – JP)

- Alexandros Tourapis (Apple – US)

- Anthony Trioux (Xidian Univ. – CN)

- Chenxi Tu (Huawei – CN)

- Kyohei Unno (NHK – JP)

- Geert Van der Auwera (Qualcomm – US)

- Gleb Verba (Qualcomm – US)

- Shuai Wan (NWPU – CN)

- Chen Wang (OPPO – CN)

- Dong Wang (OPPO – CN)

- Fan Wang (OPPO – CN)

- Feng Wang (OPPO – CN)

- Hanming Wang (MIIT – CN)

- Huifen Wang (CTC – CN)

- Sheng-Po Wang (ITRI – US)

- Xianglin Wang (Kwai – US)

- Yonghua Wang (ZTE – CN)

- Stephan Wenger (Tencent – US)

- Philippe Wetzel (Vitec – FR)

- Adam Wieckowski (FhG HHI – DE)

- Thomas Wiegand (FhG HHI – DE)

- Mathias Wien (RWTH Aachen – DE)

- Ping Wu (ZTE – UK)

- Shaowei Xie (ZTE – CN)

- Zhihuang Xie (OPPO – CN)

- Xiaoyu Xiu (Kwai – US)

- Luhang Xu (OPPO – CN)

- Xiaozhong Xu (Tencent – CN)

- Seok-hyun Yang (SJU – KR)

- Tian Yang (HUST – CN)

- Yan Ye (Alibaba – US)

- Peng Yin (Dolby – US)

- Haoping Yu (OPPO – CN)

- Yue Yu (OPPO – CN)

- Hui Yuan OPPO – CN)

- Vladyslav Zakharchenko (Nokia – US)

- Honglei Zhang (Nokia – FI)

- Hongli Zhang (OPPO – CN)

- Junxi Zhang (Wuhan Univ. – CN)

- Lai Zhang (OPPO – CN)

- Wei Zhang (Xidian Univ.  – CN)

- Wenhao Zhang (YoJaJa STD – CN)

- Yuhuai Zhang (OPPO – CN)

- Zuhai Zhang (Transsion – CN)

- Shuai Zhao (Qualcomm – US)

- Yin Zhao (Huawei – CN)

- Jianhua Zheng (Peng Cheng Lab – CN)

- Jiantong Zhou (Huawei – CN)

- Dan Zou (OPPO – CN)

- Nannan Zou (Nokia – FI)

- Wenjie Zou (Xidian Univ. – CN)

# Annex B2 to JVET report:
List of meeting participants attending remotely

The remote participants of the fortieth meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately 279 people in total, not including those who had attended the meeting in person at least part-time (see annex B1), and not including those who attended only the joint sessions with other groups), were as follows:

- Waqas Ahmad (Ericsson – SE)

- Jukka Ahonen (Nokia – FI)

- Ali Ak (Interdigital – FR)

- Shintaro Akasaka (Mitsubishi Electric – JP)

- Anique Akhtar (Qualcomm – US)

- Bovsha Albert (Huawei – RU)

- Muhammad Asad (Qualcomm – US)

- Ed Asbun (Apple – US)

- Pekka Astola (Nokia – FI)

- Murali Babu (Ittiam – IN)

- Suyong Bahk (KHU – KR)

- Junho Bang (HNU – KR)

- Stefano Battista (BSOFT – IT)

- Nisha Bhaskar (Huawei – DE)

- Charles Bonnineau (InterDigital – CA)

- Han Boon (Panasonic – SG)

- Philippe Bordes (InterDigital – FR)

- Tomás Borges (HHI – DE)

- Angelo Bruccoleri (RAI – IT)

- A. Burakhan (Nokia – DE)

- Joohyung Byeon (Kwangwoon Univ. – KR)

- Renjie Chang (Tencent – CN)

- Yao-Jen Chang (Qualcomm – US)

- Fang-Yi Chao (Tencent – US)

- Ching-Yeh Chen (MediaTek – US)

- Chun-Chi Chen (Qualcomm – US)

- Fangdong Chen (Hikvision – CN)

- Hong-Hui Chen (MediaTek – US)

- Lien-Fei Chen (Tencent – US)

- Wei Chen (Kwai – US)

- Xinxin Chen (WHU – CN)

- Ya Chen (Interdigital – FR)

- Yi-Hsin Chen (MediaTek – US)

- Yufan Chen (ZJU – CN)

- Zhenzhong Chen (WHU – CN)

- Jingwei Chi (UESTC – CN)

- Man-Shu Chiang (MediaTek – US)

- Yi-Jen Chiu (Intel – US)

- Kiho Choi (KHU – KR)

- Hyejeong Choi (KWU – KR)

- Muhammed Coban (Qualcomm – US)

- Francesco Cricri (Nokia – FI)

- Kai Cui (Qualcomm – US)

- Chung Dean (Panasonic – SG)

- Zhipin Deng (Bytedance – CN)

- Keqin Ding (TCL – CN)

- Didier Doyen (InterDigital – FR)

- Alberto Duenas (Warner Bros. Discovery – US)

- Thierry Dumas (InterDigital – FR)

- Akhsar Dzugaev (Huawei – RU)

- Wei Feng (Xidian Univ. – CN)

- Jiaye Fu (PKU – CN)

- Benjamin Galmiche (Canon – FR)

- Shuo Gao (Xiaomi – CN)

- Ying Gao (ZTE – CN)

- Patrick Garus (Qualcomm – US)

- Simon Gauntlett (Dolby – UK)

- Yunying Ge (Huawei – CN)

- Ivan Gribushin (Huawei – RU)

- Tiansheng Guo (Huawei – CN)

- Jaemin Ha (Sejong Univ. – KR)

- Jari Hagqvist (Nokia – FI)

- Kao Hayashi (Sony – JP)

- Wenxuan He (HUST – CN)

- Cornelius Hellge (HHI – DE)

- Christian Helmrich (HHI – DE)

- Arianne Hinds (Tencent – US)

- Shih-Ta Hsiang (MediaTek – US)

- Yuling Hsiao (MediaTek – US)

- Chih-Wei Hsu (Mediatek – US)

- Nan Hu (Qualcomm – US)

- Cheng Huang (ZTE – CN)

- Han Huang (Tencent – US)

- Jingying Huo (Xidian Univ. – CN)

- Lee-Jun Hyun (KHU – KR)

- Jeongwon Hwang (KWU – KR)

- Jung Hyun (KHU – KR)

- Roberto Iacoviello (RAI – IT)

- Atsuro Ichigaya (NHK – JP)

- Byeungwoo Jeon (SKKU – KR)

- Sanghun Jeon (LGE – KR)

- Hyunki Jeong (SKKU – KR)

- Hong-Jheng Jhu (Kwai – US)

- Ke Jia (Alibaba – CN)

- Xiaochun Jiang (PKU – CN)

- Xiaoran Jiang (INSA – FR)

- Thong Jing (Panasonic – SG)

- Dae Yeon Ju (KWU – KR)

- John Juhyung (WILUS – KR)

- Cheolkon Jung (Xidian Univ. – CN)

- Huang Junnan (Huawei – CN)

- Kashyap Kammachi (Nokia – FI)

- Jewon Kang (Ewha W. University – KR)

- Sylvain Kervadec (Orange – FR)

- Aro Kim (KNU – KR)

- Chulkeun Kim (LGE – KR)

- Dong-hwi Kim (KNU – KR)

- Jae-Gon Kim (KAU)

- Jiyoung Kim (WILUS – KR)

- Kyung Yong Kim (Wilus – KR)

- Mintae Kim (KWU – KR)

- Sung Min Kim (LGU+ – KR)

- Taeyu Kim (KWU – KR)

- Yongheon Kim (HNU – KR)

- Yongseong Kim (SKKU – KR)

- Youngwook Kim (KWU – KR)

- Heiner Kirchhoffer (HHI – DE)

- Konstantinos Konstantinides (Dolby Labs – US)

- Moonmo Koo (LGE – KR)

- Esin Koyuncu (Qualcomm – US)

- Maja Krivokuća (InterDigital – FR)

- Bart Kroon (Philips – NL)

- Che-Wei Kuo (Kwai – US)

- Hyukmin Kwon (HYU – KR)

- Théo Ladune (Orange – FR)

- Chen-Yen Lai (MediaTek – US)

- Mikael Le ?? (InterDigital – FR)

- Nam Le (Nokia – FI)

- Pascal Le ?? (InterDigital – FR)

- Brian Lee (Dolby – US)

- Hyemi Lee (KWU – KR)

- Jangwon Lee (LGE – KR)

- Junghyun Lee (KHU – KR)

- Kaeun Lee (HNU – KR)

- Louis Lee (Acer – US)

- Minhun Lee (KWU – KR)

- Seongeun Lee (HNU – KR)

- Soonbin Lee (HHI – DE)

- Sunyoung Lee (Atins – KR)

- Yung-Lyul Lee (SJU – KR)

- Luo Lei (UESTC – CN)

- Bin Li (Hisense – CN)

- Hao Li (Huawei – CN)

- Wenhao Li (Transsion – CN)

- Xinwei Li (Alibaba – CN)

- Yifan Li (Xidian Univ. – CN)

- Yue Li (SJTU – CN)

- Yun Li (Qualcomm – US)

- Wang-Q Lim (HHI -DE)

- Chun-Lung Lin (ITRI – US)

- Jian-Liang Lin (Qualcomm – US)

- Jibing-Chieh Lin (ITRI – US)

- Kai Lin (Huawei – CN)

- Po-Han Lin (Qualcomm – US)

- Yu-Cheng Lin (MediaTek – US)

- Peter Liu (AMD – CA)

- Shan Liu (Tencent – US)

- Chih-Hsuan Lo (MediaTek – US)

- Fedor Loginov (Huawei – RU)

- Van Luong (Apple – US)

- Ajay Luthra (Picsel Labs – US)

- Changyue Ma (Kwai – US)

- Wenzhuo Ma (WHU – CN)

- Yanzhuo Ma (Xidian Univ. – CN)

- Kirill Malyshev (Huawei – RU)

- Ismail Marzuki (InterDigital – CA)

- Xuewei Meng (Qualcomm – US)

- Tae Meon (Sharp – US)

- Philipp Merkle (HHI – DE)

- Dawid Mieloch (PUT – PL)

- Marta Milovanovic (Philips – NL)

- Iole Moccagatta (Intel – US)

- Olivier Mocquard (InterDigital – FR)

- Didier Nicholson (EKTACOM – FR)

- Hyejin Oh (Ewha – KR)

- Sang-hyo Park (KNU – KR)

- Seori Park (LGE – KR)

- Seungwook Park (Hyundai – KR)

- Maciej Pedzisz (Nokia – UK)

- Khoa Pham-Dinh (Nokia – FI)

- Frank Plowman (Xiaomi — UK)

- Tangi Poirier (InterDigital – FR)

- Srivatsa Prativadibhayankaram (Fraunhofer IIS – DE)

- Saurabh Puri (InterDigital – CA)

- Luyi Qin (WHU – CN)

- Qipu Qin (Xidian Univ. – CN)

- Haodong Qu (WHU – CN)

- Miloš Radosavljević (InterDigital – FR)

- Adarsh Ramasubramonian (Qualcomm – US)

- Gagan Rath (InterDigital – FR)

- Bappaditya Ray (Qualcomm – US)

- Kevin Reuzé (InterDigital – FR)

- Marc Rivière (Ateme – FR)

- Antoine Robert (InterDigital – FR)

- Patrice Rondao (Nokia – BE)

- Chris Rosewarne (Canon – AU)

- Taguchi Ryoya (NEC – JP)

- Mehdi Salehifar (Bytedance – US)

- Gustavo Sandri (InterDigital – FR)

- Johannes Sauer (Huawei – DE)

- Tong Shao (Dolby – US)

- Yiting Shao (Qualcomm – US)

- Gongjia Sheng  (HUST – CN)

- Tian Shu (WHU – CN)

- Ahmed Sidiya (Sharp – US)

- Praneet Singh (Dolby – US)

- Rickard Sjöberg (Ericsson – SE)

- John Son (WILUS – KR)

- Nan Song (OPPO – CN)

- Jan Studeny (Ericsson – SE)

- Junying Su (WHU – CN)

- Maxim Sychev (Huawei – RU)

- Yasser Syed (Comcast – US)

- Calvin-Khang Ta (Dolby – US)

- Sylvain Thiebaud (InterDigital – FR)

- Herbert Thoma (Fraunhofer IIS – DE)

- Franck Thudor (InterDigital – FR)

- Jiaxin Tian (Xidian Univ. – CN)

- Alexandre Tissier (Xiaomi – FR)

- Renan Utida (InterDigital – CA)

- Aditya Vishwanath (Panasonic – SG)

- Biao Wang (Tencent – US)

- Guiqi Wang (vivo – CN)

- Hongtao Wang (Qualcomm – US)

- Jianfeng Wang (Qualcomm – US)

- Jiayi Wang (ZJU – CN)

- Limin Wang (Nokia – US)

- Liqiang Wang (Tencent – CN)

- Lu Wang (Xidian Univ. – CN)

- Qimeng Wang (HUST – CN)

- Xinru Wang (UESTC – CN)

- Yang Wang (Bytedance – CN)

- Ye-Kui Wang (Bytedance – US)

- Yifan Wang (Tencent – US)

- Yifei Wang (WHU – CN)

- Yingbin Wang (Tencent – CN)

- Kai Wen (Sharp – JP)

- Martin Winken (HHI – DE)

- Jung Won (ETRI – KR)

- Min Woo (Samsung – KR)

- Gang Wu (vivo – CN)

- Wenqiang Xiao (WHU-CN)

- Xiatian Xie (HUST – CN)

- Alisa Xing (Hisense – CN)

- Fang Xing (Hisense – CN)

- Jianfeng Xu (KDDI – JP)

- Jizheng Xu (Bytedance – US)

- Linchen Xu (SYSU – CN)

- Motong Xu (Tencent – US)

- Ruichen Xu (Xidian Univ. – CN)

- Zhuowei XU (TCL – CN)

- Toru Yamada (NEC – JP)

- Ning Yan (Kwai – US)

- Mingjia Yang (PKU – CN)

- Ruiying Yang (Nokia-FI)

- Yuqing Yang (Huawei – CN)

- Yayun Yao (Xidian Univ. – CN)

- Emma Ye (Qualcomm – US)

- Sehoon Yea (Intel – US)

- Shen Yi (Panasonic – SG)

- Shanzhi Yin (CityUHK – HK)

- Wenbin Yin (Bytedance – CN)

- Yujie Yin (NWPU – CN)

- Hui Yong (KHU – KR)

- Jeeye Yoon (LGE – KR)

- Yong-Uk Yoon (Tencent – US)

- Jin Young (SJU – KR)

- Hualong Yu (ZJU – CN)

- Lu Yu (ZJU – CN)

- Ruoyang Yu (Qualcomm – US)

- Lin Yu-Cheng (MediaTek – US)

- Kai Zhang (Bytedance – US)

- Li Zhang (Bytedance – US)

- Na Zhang (Bytedance – CN)

- Wenzhuo Zhang (WHU-CN)

- Yan Zhang (Qualcomm – US)

- Zhi Zhang (Qualcomm – US)

- Zixiang Zhang (Alibaba – CN)

- Jane Zhao (LGE – US)

- Lei Zhao (Bytedance – CN)

- Xiaozhen Zheng (DJI – CN)

- Yue Zheng (Huawei – CN)

- Chuan Zhou (vivo – CN)

- Minhua Zhou (Broadcom – US)

- Han Zhu (Tencent – CN)

- Weijia Zhu (Qualcomm – US)

- Zhiwei Zhu (NWPU – CN)

- Naima Zouidi (Ofinno – US)

- Ivan Zupancic (TCL – CN)

# Annex C to JVET report:
Recommendations of the 21st meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N 371

## Recommendations of the 21st WG 5 meeting

## 1. Reports

### 1.1 Meeting Reports

### 1.1.1 WG 5 approves

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
| 361 | Report of the 20th JTC 1/SC 29/WG 5 meeting | Jens-Rainer Ohm | N | 2025-08-01 | 25447 |

## 2. MPEG-C (ISO/IEC 23002 - MPEG video technologies)

### 2.1 Part 7 - Versatile supplemental enhancement information messages for coded video bitstreams

### 2.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23002-7 - Versatile supplemental enhancement information messages for coded video bitstreams |  |  |  |  |
| 373 | Disposition of comments received on ISO/IEC 23002-7:2024/DAM 1 | Jill Boyce | N | 2025-10-12 | 25737 |
| 374 | Text of FDIS ISO/IEC 23002-7:202x (4th ed.) Versatile supplemental enhancement information messages for coded video bitstreams | Jill Boyce | N | 2026-01-09 | 25738 |

| ### 2.1.2 |  | ### WG 5 requests integrating the in-progress development of ISO/IEC 23002-7:2024/Amd.1 into a 4th edition of ISO/IEC 23002-7. An FDIS ballot for the 4th edition is thus expected to be issued instead of FDAM 1 ballot. The requested editors for the 4th edition are Jill Boyce, Jie Chen, Sachin Deshpande, Miska M. Hannuksela, Sean McCarthy, Gary J. Sullivan, Hendry Tan, and Ye-Kui Wang. The development of the 4th edition will not expand the scope of the original project. This text is under development jointly in partnership with ITU-T SG21 and will be published as technically aligned twin text corresponding to a future edition of Rec. ITU-T H.274. |
| --- | --- | --- |

| ### 2.1.3 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23002-7:202x Versatile supplemental enhancement information messages for coded video bitstreams (4th ed.), as this is a collaborative project with ITU-T SG 21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

| ### 2.1.4 |  | ### WG 5 requests to make ISO/IEC 23002-7:202x (4th ed.) publicly available, anticipating public availability elsewhere of a corresponding twin text by ITU-T. |
| --- | --- | --- |

## 3. MPEG-H (ISO/IEC 23008 - High efficiency coding and media delivery in heterogeneous environments)

### 3.1 Part 2 - High efficiency video coding

### 3.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23008-2 - High efficiency video coding |  |  |  |  |
| 375 | Disposition of comments received on ISO/IEC 23008-2:2025/DAM 1 | Ye-Kui Wang | N | 2025-10-11 | 25739 |
| 376 | Text of FDIS ISO/IEC 23008-2:202x (7th ed.) High efficiency video coding | Ye-Kui Wang | N | 2026-01-09 | 25740 |

| ### 3.1.2 |  | ### WG 5 requests integrating the in-progress development of ISO/IEC 23008-2:2025/Amd.1 into a 7th edition of ISO/IEC 23008-2. An FDIS ballot for the 7th edition is thus expected to be issued instead of FDAM 1 ballot. The requested editors for the 7th edition are Ye-Kui Wang, Benjamin Bross, Sachin Deshpande, Gary J. Sullivan, and Alexis Tourapis. The development of the 7th edition will not expand the scope of the original project. This text is under development jointly in partnership with ITU-T SG21 and will be published as technically aligned twin text corresponding to a future edition of Rec. ITU-T H.265. |
| --- | --- | --- |

| ### 3.1.3 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23008-2:202x High efficiency video coding (7th ed.), as this is a collaborative project with ITU-T SG 21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

| ### 3.1.4 |  | ### WG 5 requests to make ISO/IEC 23008-2:202x (7th ed.) publicly available, anticipating public availability elsewhere of a corresponding twin text by ITU-T. |
| --- | --- | --- |

## 4. MPEG-I (ISO/IEC 23090 - Coded representation of immersive media)

### 4.1 Part 3 - Versatile video coding

### 4.1.1 WG 5 recommends approval of the following documents

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | ISO/IEC 23090-3 - Versatile video coding |  |  |  |  |
| 377 | Disposition of comments received on ISO/IEC 23090-3:2024/DAM 1 | Ye-Kui Wang | N | 2025-10-11 | 25741 |
| 378 | Text of FDIS ISO/IEC 23090-3:202x (4th ed.) Versatile video coding | Ye-Kui Wang | N | 2026-01-09 | 25742 |

| ### 4.1.2 |  | ### WG 5 requests integrating the in-progress development of ISO/IEC 23090-3:2024/Amd.1 into a 4th edition of ISO/IEC 23090-3. An FDIS ballot for the 4th edition is thus expected to be issued instead of FDAM 1 ballot. The requested editors for the 4th edition are Gary J. Sullivan, Benjamin Bross, Miska M. Hannuksela, and Ye-Kui Wang. The development of the 4th edition will not expand the scope of the original project. This text is under development jointly in partnership with ITU-T SG16 and will be published as technically aligned twin text corresponding to a future edition of Rec. ITU-T H.266. |
| --- | --- | --- |

| ### 4.1.3 |  | ### WG 5 requests not to use the OSD tool for preparation of the text of ISO/IEC 23090-3:202x Versatile video coding (4th ed.), as this is a collaborative project with ITU-T SG 21 that requires coordination between the organizations to ensure alignment of the content. |
| --- | --- | --- |

| ### 4.1.4 |  | ### WG 5 requests to make ISO/IEC 23090-3:202x (4th ed.) publicly available, anticipating public availability elsewhere of a corresponding twin text by ITU-T. |
| --- | --- | --- |

## 5. Explorations

### 5.1 Enhanced compression beyond VVC capability

| ### 5.1.1 |  | ### WG 5 thanks Bytedance, Ericsson, ETRI, Fraunhofer HHI, Hanyang University, Huawei, InterDigital, KDDI, Kwai, Nokia, OPPO, Qualcomm, TCL, Tencent, Wuhan University, and Xiaomi for submitting responses to the Joint Call for Evidence on video compression with capability beyond VVC. |
| --- | --- | --- |

| ### 5.1.2 |  | ### WG 5 thanks TCL for providing 4K displays used in the CfE viewing. Bossen Technologies, Fraunhofer HHI and RWTH Aachen are thanked for providing play-out and testing equipment. Mathias Wien is thanked for organizing and supervising the test, and Christian Lehmann is thanked for helping with the test setup. Christian Lehmann and Vittorio Baroncini are thanked for assistance in preparing and conducting the viewing. The numerous experts who volunteered to participate in the viewings of CfE submissions and for the dry-run of multi-layer verification tests are also thanked. |
| --- | --- | --- |

| ### 5.1.3 |  | ### WG 5 thanks Alibaba, Bossen Technologies, Bytedance, Dolby, Ericsson, Google, Huawei, InterDigital, KDDI, Nokia, OPPO, Panasonic, Qualcomm, Samsung, Sharp, Tencent, vivo, and Xiaomi for generating VTM and ECM bitstreams that were used in the context of the Call for Evidence. |
| --- | --- | --- |

| ### 5.1.4 |  | ### WG 5 thanks Frank Bossen and Mathias Wien for decoding the bitstreams, for cross-checking results in the preparation of the CfE evaluation, and for preparing the summary of results. |
| --- | --- | --- |
| ### 5.1.5 |  | ### WG 5 thanks Huawei and Zhejiang University for offering new video test sequences in the categories of gaming content and 4-UHD/HDR, respectively. |

## 6. Management

### 6.1 General

| ### 6.1.1 |  | ### WG 5 notes that action appears to not yet have been taken on some previous requests for public availability of ISO/IEC deliverables without cost, and kindly requests that ISO/IEC 23008-5:2017 (Ed. 2), ISO/IEC 23008-5:2017/Amd.1:2017, ISO/IEC 23008-8:2018 (Ed. 2), ISO/IEC 23008-8:2018/Amd1:2019, and ISO/IEC 23091-2:2025 (Ed. 3) be made freely available, considering that equivalent publications are freely available elsewhere from ITU-T as editions of ITU-T H.265.2, ITU-T H.265.1, and ITU-T H.273. |
| --- | --- | --- |

### 6.2 Collaboration with ITU-T

| ### 6.2.1 |  | ### The JVET chair proposes to hold the 41st JVET meeting during 14 – 23 January 2026 under ISO/IEC JTC 1/SC 29 auspices (with contribution deadline of 7 January 2026, and no plan to meet during the weekend 17 - 18 January), to be conducted as teleconference meeting. Subsequent meetings are planned to be held during 24 April – 1 May 2026 under ISO/IEC JTC 1/SC 29 auspices in Santa Eulària, ES; during 7 – 15 July 2026 under ITU-T SG21 auspices in Geneva, CH; during 17 – 23 October 2026 under ISO/IEC JTC 1/SC 29 auspices in Hangzhou, CN; during 13 – 22 January 2027 under ISO/IEC JTC 1/SC 29 auspices in Brisbane, AU; during 17 - 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/SC 29 auspices in Shenzhen, CN; during January 2028 under ISO/IEC JTC 1/SC 29 auspices, date and location t.b.d.; and during April 2028 under ITU-T SG21 auspices, date and location t.b.d. |
| --- | --- | --- |

### 6.3 Ad hoc groups

### 6.3.1 WG 5 approves the following document

| No. | Title | In Charge | TBP | Available | S/N |
| --- | --- | --- | --- | --- | --- |
|  | Ad hoc groups |  |  |  |  |
| 379 | List of AHGs established at the 21st WG 5 meeting | Jens-Rainer Ohm | N | 2025-10-11 | 25743 |

### 6.4 Expression of Thanks

| ### 6.4.1 |  | ### WG 5 thanks Marius Preda for the service of managing and maintaining the document site jvet-experts.org, and Institut Mines-Télécom for hosting the sites. |
| --- | --- | --- |

| ### 6.4.2 |  | ### WG 5 thanks ITU for the excellent hosting of the 40th JVET meeting in Geneva. Simão Campos and Stefano Polidori are thanked for the help in preparation and organisation, and Kelya-Axelle Beke is thanked for maintaining the registrations. The following members of ITU staff are thanked for their dedication and daily help in the technical setup of meeting facilities: Gent Bajrami, Paul Marko, Gvantsa Nikolishvili, and Marc Antoine Zanou. |
| --- | --- | --- |

## The meeting was closed at 1445 CET on 2025-10-12.