| Joint Video Experts Team (JVET) of ITU-T SG21 WP3/21 and ISO/IEC JTC 1/SC 29 41st Meeting, by teleconference, 14–23 January 2026 | Document: JVET-AO_notes_d6 |
| --- | --- |

| Title: | Meeting Report of the 41st Meeting of the Joint Video Experts Team (JVET), by teleconference, 14 – 23 January 2026 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chair of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contacts): | Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: | +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chair of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-first meeting during 14 – 23 January 2026 meeting as an online-only meeting.

For ISO/IEC purposes, JVET is alternatively designated ISO/IEC JTC 1/‌SC 29/‌WG 5, and this was the twenty-second meeting as WG 5. The JVET meeting was held under the chairmanship of Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.16 of this document. It is further noted that work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued in JVET as a single joint team, and explorations towards possible future need of standardization in the area of video coding are also conducted by JVET, as negotiated by the parent bodies.

The JVET meeting began at approximately 1300 UTC on Wednesday 14 January 2026. Meeting sessions were held on all days except the weekend days of Saturday and Sunday 17 and 18 January 2026, until the meeting was closed at approximately XXXX hours UTC on XXday 2X January 2026. Approximately XXX people attended the JVET meeting, and approximately XXX input documents (not counting crosschecks, reports, and summary documents), 19 AHG reports, 2 EE summary reports, X BoG report(s), and X incoming liaison document(s) were discussed. The meeting took place in coordination with a meeting of various SC29 Working Groups and Advisory Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the perspective of the SC 29 parent body, under whose auspices this JVET meeting was held. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, reference software and conformance testing packages. Further important goals were reviewing the results of the Exploration Experiment (EE) on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and to plan next steps for investigation of candidate technology towards further standard development. In the latter context, the evaluation of responses received on the Joint Call for Evidence on video compression with capability beyond VVC was a major activity at his meeting.

As a primary goal, the JVET meeting reviewed the work that had been performed in the interim period since the fortieth JVET meeting in producing the following documents:

a) JVET documents

JVET-AN1005 Future CICP extensions (Draft 2)

JVET-AN1016 AVC with extensions and corrections (Draft 4)

JVET-AN1017 Support for additional VSEI messages in AVC (Draft 4)

JVET-AN1018 HEVC with extensions and corrections (Draft 3), also forwarded for consent as SG21-TD210/PLEN ITU-T H.265 (V11) "High efficiency video coding" (Rev.)

JVET-AN1019 VSEI with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD212/PLEN ITU-T H.274 (V4) "Versatile supplemental enhancement information messages for coded video bitstreams" (Rev.)

JVET-AN2001 VVC with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD211/PLEN ITU-T H.266 (V4) "Versatile video coding" (Rev.)

JVET-AN2008 Conformance testing for versatile video coding (Draft 8), also forwarded for consent as SG21-TD214/PLEN ITU-T H.266.1 (V3) " Conformance specification for ITU-T H.266 versatile video coding" (Rev.)

JVET-AN2019 Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC)

JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4)

JVET-AN2021 Verification test plan for VVC multilayer coding (update 7)

JVET-AN2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AN2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19)

JVET-AN2026 Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC

JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC, also issued as AG 5 N 180

JVET-AN2032 Technologies under consideration for future extensions of VSEI (version 10)

JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience

JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools

JVET-AN2041 Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026

b) documents produced as WG 5 documents only:

WG 5 N 373 Disposition of comments received on ISO/IEC 23002-7:2024/DAM 1

WG 5 N 374 Text of FDIS ISO/IEC 23002-7:202x (4th ed.) Versatile supplemental enhancement information messages for coded video bitstreams

WG 5 N 375 Disposition of comments received on ISO/IEC 23008-2:2025/DAM 1

WG 5 N 376 Text of FDIS ISO/IEC 23008-2:202x (7th ed.) High efficiency video coding

WG 5 N 377 Disposition of comments received on ISO/IEC 23090-3:2024/DAM 1

WG 5 N 378 Text of FDIS ISO/IEC 23090-3:202x (4th ed.) Versatile video coding

As main results, the JVET produced XX output documents from the current meeting (update after meeting):

JVET-AN1005 Future CICP extensions (Draft 2)

JVET-AN1016 AVC with extensions and corrections (Draft 4)

JVET-AN1017 Support for additional VSEI messages in AVC (Draft 4)

JVET-AN1018 HEVC with extensions and corrections (Draft 3), also forwarded for consent as SG21-TD210/PLEN ITU-T H.265 (V11) "High efficiency video coding" (Rev.)

JVET-AN1019 VSEI with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD212/PLEN ITU-T H.274 (V4) "Versatile supplemental enhancement information messages for coded video bitstreams" (Rev.)

JVET-AN2001 VVC with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD211/PLEN ITU-T H.266 (V4) "Versatile video coding" (Rev.)

JVET-AN2008 Conformance testing for versatile video coding (Draft 8), also forwarded for consent as SG21-TD214/PLEN ITU-T H.266.1 (V3) " Conformance specification for ITU-T H.266 versatile video coding" (Rev.)

JVET-AN2019 Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC)

JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4)

JVET-AN2021 Verification test plan for VVC multilayer coding (update 7)

JVET-AN2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AN2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19)

JVET-AN2026 Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC

JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC, also issued as AG 5 N 180

JVET-AN2032 Technologies under consideration for future extensions of VSEI (version 10)

JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience

JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools

JVET-AN2041 Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026

- The following 6 documents were produced as WG 5 documents only, without a corresponding JVET output document or direct repetition of their content in this meeting report:

WG 5 N 373 Disposition of comments received on ISO/IEC 23002-7:2024/DAM 1

WG 5 N 374 Text of FDIS ISO/IEC 23002-7:202x (4th ed.) Versatile supplemental enhancement information messages for coded video bitstreams

WG 5 N 375 Disposition of comments received on ISO/IEC 23008-2:2025/DAM 1

WG 5 N 376 Text of FDIS ISO/IEC 23008-2:202x (7th ed.) High efficiency video coding

WG 5 N 377 Disposition of comments received on ISO/IEC 23090-3:2024/DAM 1

WG 5 N 378 Text of FDIS ISO/IEC 23090-3:202x (4th ed.) Versatile video coding

For the organization and planning of its future work, the JVET established XX “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, X Exploration Experiments (EE) were defined. The next ten JVET meetings were planned for 24 April – 1 May 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES; during 7 – 15 July 2026 under ITU-T SG21 auspices in Geneva, CH; during 17 – 23 October 2026 under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN; during 13 – 22 January 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU; during 17 – 28 April 2027 under ITU-T SG21 auspices in Geneva, CH; during 7 – 16 July 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI; during 20 – 29 October 2027 under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN; during January 2028 under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.; during April 2028 under ITU-T SG21 auspices, date and location t.b.d.; and during July 2028 under ITU-T SG21 auspices, date and location t.b.d.

The document distribution site https://jvet-experts.org/ was used for distribution of all documents. The most recent versions of JCT-VC and JCT-3V documents can now be accessed directly via the JVET site, whereas all uploaded versions are also available from http://wftp3.itu.int/av-arch/jctvc-site/ and http://wftp3.itu.int/av-arch/jct3v-site/, respectively. JVET documents are also provided via http://wftp3.itu.int/av-arch/jvet-site/, but as this is cannot currently be maintained as a mirror with the jvet-experts.org site, documents are typically only made available with a delay of two or three meeting cycles, to avoid that late uploads of output documents would be missing.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 21 Visual Coding Experts Group (VCEG) and ISO/IEC JTC 1/‌SC 29/‌WG 5. The parent bodies of the JVET are ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/21 and ISO/IEC JTC 1/‌SC 29 held its forty-first meeting during 14 – 23 January 2026 meeting as an online-only meeting.

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards, as well as associated conformance test sets, reference software, verification testing, and non-normative guidance information. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on:

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Furthermore, explorations towards possible future need of standardization in the area of video coding are also conducted by JVET. Currently, the following topics are under investigation:

- Exploration on Neural Network-based Video Coding

- Exploration on Enhanced Compression beyond VVC capability

JVET had been tasked by the parent bodies to develop a call for proposals on video compression with capability beyond existing standards, with plan to issue a first draft call from the current meeting.

This report contains three important annexes, as follows:

- Annex A contains a list of the documents of the JVET meeting

- Annex B contains a list of the meeting participants

- Annex C contains the meeting recommendations of ISO/IEC JTC 1/‌SC 29/‌WG 5 for purposes of results reporting to ISO/IEC.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2026_01_AO_Virtual/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the fortieth JVET meeting in producing the following output documents:

a) JVET documents

JVET-AN1005 Future CICP extensions (Draft 2)

JVET-AN1016 AVC with extensions and corrections (Draft 4)

JVET-AN1017 Support for additional VSEI messages in AVC (Draft 4)

JVET-AN1018 HEVC with extensions and corrections (Draft 3), also forwarded for consent as SG21-TD210/PLEN ITU-T H.265 (V11) "High efficiency video coding" (Rev.)

JVET-AN1019 VSEI with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD212/PLEN ITU-T H.274 (V4) "Versatile supplemental enhancement information messages for coded video bitstreams" (Rev.)

JVET-AN2001 VVC with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD211/PLEN ITU-T H.266 (V4) "Versatile video coding" (Rev.)

JVET-AN2008 Conformance testing for versatile video coding (Draft 8), also forwarded for consent as SG21-TD214/PLEN ITU-T H.266.1 (V3) " Conformance specification for ITU-T H.266 versatile video coding" (Rev.)

JVET-AN2019 Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC)

JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4)

JVET-AN2021 Verification test plan for VVC multilayer coding (update 7)

JVET-AN2023 Exploration experiment on neural network-based video coding (EE1)

JVET-AN2024 Exploration experiment on enhanced compression beyond VVC capability (EE2)

JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19)

JVET-AN2026 Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC

JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC, also issued as AG 5 N 180

JVET-AN2032 Technologies under consideration for future extensions of VSEI (version 10)

JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience

JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools

JVET-AN2041 Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026

Further important goals were reviewing the results of the EE on Neural Network-based Video Coding, of the EE on Enhanced Compression beyond VVC capability, of other technical input on novel aspects of video coding technology, and planning next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The document distribution site https://jvet-experts.org/ was used for distribution of all documents.

Document registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting follow the UTC time, except as otherwise noted.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect one of the various software packages but have no normative effect on text are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in one of the test model descriptions such as VTM, HM, etc. (an error, oversight, or messiness) or in the associated software package are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of a draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

- Other parenthetical comments may be used for describing the impact or motivation of a decision. Some decisions are recorded with the word “agreed” rather than “Decision:”, especially for work items under study, non-normative, editorial and planning matters.

This meeting report is based primarily on notes taken by the JVET chair, or other experts that were appointed to chair parallel sessions. It is indicated who took the notes in review of a given category (or individual documents în a category), and by which day and time the review was conducted. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report. Expressions such as “X.XX%” indicate that the desired results were not available at the time the information was recorded.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 7 January 2026. Any documents uploaded after 1159 hours Paris/Geneva time on Thursday 8 January 2026 were considered “officially late”, with a grace period of 12 hours (to accommodate those living in different time zones of the world). The deadline does not apply to AHG reports and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-AO0238 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meeting, or documents which were requested to be produced for the purpose of improving specification text, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions has been to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This has been agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-AO0XXX (a proposal on …), uploaded 01-XX,

- …

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline.

The following other documents not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-AO0XXX (an investigation on …), uploaded 01-XX,

- …

All cross-verification reports at this meeting (except for JVET-AO0134, JVET-AO0205, JVET-AO0224, …) were registered late, and/or uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following contribution registrations were noted that were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-AO0062, JVET-AO0065, JVET-AO0143, JVET-AO0196, JVET-AO0206, JVET-AO0225, JVET-AO0232, JVET-AO0228, JVET-AO0240, JVET-AO0250, … .

The following cross-verification report(s) were still missing by the end of the meeting, but were uploaded later: JVET-AO0XXX, … (sentence kept for future use). The following report(s) had not become available yet three weeks after the end of the meeting: JVET-AO0XXX, … . These were marked as withdrawn by the JVET chair, assuming the registration had become obsolete.

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then be counted as a late contribution if the update was after the document deadline). At the current meeting, this situation did not apply to any contributions. (did apply to document(s) JVET-AO0XXX, being initially empty of results and flagged as late in the list above, based on the time of the first reasonable document upload | sentence kept for future use).

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision, although participants tend to seldom notice what is recorded there.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there may have been some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors may have also noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-AN1000, the Future CICP extensions (draft 2) JVET-AN1005, the AVC with extensions and corrections (draft 4) JVET-AN1016, the Support for additional SEI messages in AVC (draft 4) JVET-AN1017, the HEVC with extensions and corrections (draft 3)  JVET-AN1018, the VSEI with extensions and corrections (draft 1)  JVET-AN1019, the VVC with extensions and corrections (draft 1)  JVET-AN2001, the Conformance testing for versatile video coding (Draft 8) JVET-AN2008, the Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC) JVET-AN2019, the Film grain synthesis technology for video applications ed. 2 (Draft 4) JVET-AN2020, the Verification test plan for VVC multilayer coding (update 7) JVET-AN2021, the Description of the EE on Neural Network-based Video Coding JVET-AN2023, the Description of the EE on Enhanced Compression beyond VVC capability JVET-AN2024, the Algorithm description of Enhanced Compression Model 19 (ECM 19) JVET-AN2025, the Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC JVET-AN2026, the Results of the Joint Call for Evidence on video compression with capability beyond VVC JVET-AN2029, the Technologies under consideration for future extensions of VSEI (version 10) JVET-AN2032, the Common test conditions for technology targeting ultra low latency and packet loss resilience JVET-AN2039, the Draft of complexity reporting template for coding algorithms and tools JVET-AN2040, and the Preliminary announcement of JVET AHG17 meeting in Aachen, DE, during 23-27 February 2026 JVET-AN2041, had been completed and those which were available were approved. In a few cases, the corresponding WG 5 N-numbered documents had not yet been uploaded, and this was requested to be done as soon as possible (kept for future use). The software implementations of VTM version 23.13, ECM version 19.0, and NNVC version 15.0 were also approved.

It was further noted that some editorial modifications were necessary on the WG 5 versions of standard deliverables, to be compliant with the ISO/IEC rules.

Only minor editorial issues were found in the meeting report JVET-AN1000; no need to produce an update was identified (see section 2.14 for details).

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/21 or ISO/IEC JTC 1/‌SC 29/‌WG 5 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting through the ISO Meetings website for ISO/IEC experts, and through the Q6/21 rapporteur for ITU-T experts. The password for meeting access had been sent to registered participants via these channels. Links to the Zoom sessions (without the necessary password) were available in the posted meeting logistics information and the calendar of meeting sessions in the JVET web site.

The following rules were established for those participating remotely via Zoom teleconference meeting:

- Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally. The online queue will be interleaved with the room queue, though it may not always be guaranteed that the sequence perfectly follows the sequence by which hand raising occurred.

- Stay muted unless you have something to say. People are muted by default when they join and need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

- Identify who you are and your affiliation when you begin speaking. The same applies for speakers in the room to let online participants know who is speaking.

- Use your full name and company/organization and country affiliation in your joining information, since the participation list of Zoom would also be used to compile the online part of attendance records.

- Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

- It was pointed out the according to ISO rules, any recording or transcription of zoom sessions is not permitted. Participants who are performing recording will be removed from the session unless they stop recording upon request.

- Generally, do not use video for the teleconferencing calls in order to avoid overloading internet connections; enable only voice and screen sharing.

- Extensive use of screen sharing is encouraged, to enable participants to view the presented material and the meeting notes. At times, multiple sources of screen sharing may be enabled, so it may be necessary for participants to understand that this is happening and to understand how to select which shared screen they want to see.

## Agenda

The agenda for the meeting, for the further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages, was as follows:

Opening remarks and review of meeting logistics and communication practices

Roll call of participants

Approval of the agenda

Code of conduct policy reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Review of target dates

Reports of ad hoc group (AHG) activities

Report of exploration experiment on neural-network-based video coding (EE1)

Report of exploration experiment on enhanced compression beyond VVC capability (EE2)

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of contributions on conformance and reference software development

Consideration of contributions on software development in exploration activities

Consideration of contributions on coding-independent code points for video signal type identification

Consideration of contributions on film grain synthesis technology

Consideration of contributions on gaming content compression

Consideration of contributions on generative face video compression

Consideration of contributions on optimization of encoders and receiving systems for machine analysis of coded video content

Consideration of contributions on assessment of implementation complexity of video coding technology

Consideration of contributions on aspects of ultra-low latency and packet loss resilience in the context of video compression

Consideration of contributions and planning of next steps towards a Call for Proposals on future video coding standardization, development of a Draft Call for Proposals, and definition of criteria for technology assessment

Consideration of contributions on errata relating to standards in the domain of JVET

Consideration of contributions on technical reports relating to standards and exploration study activities in the domain of JVET

Consideration of contributions providing non-normative guidance relating to standards and exploration study activities in the domain of JVET

Consideration of information contributions

Consideration of future work items

Coordination of visual quality testing

Liaisons, coordination activities with other organizations

Review of project editor and liaison assignments

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, future meeting planning, other planning issues

Other business as appropriate for consideration

Closing

- The agenda was approved as suggested.

- The times of meeting sessions followed the needs of a teleconference meeting (see under section 2.15), with highest priority given to the aim of achieving the goals of the meeting. Sessions were announced in the online JVET calendar in advance as far as possible, but it was agreed that some activities (such as breakout sessions) could be held at short notice.

## ISO, IEC and ITU-T Code of Conduct reminders

Participants were reminded of the ISO and IEC Codes of Conduct, found at

https://www.iso.org/publication/PUB100011.html.

https://www.iec.ch/basecamp/iec-code-conduct-technical-work

These include points relating to:

- Behave ethically

- Promote and enable all voices to be heard

- Engage constructively in ISO and IEC activities

- Respect others (in meetings, when writing, on social media)

- Declare actual and potential conflicts of interest and manage them appropriately

- Protect confidential information

- Protect ISO and IEC assets

- Avoid and prevent any form of bribery or corruption

- Uphold the consensus process

- When disputes arise, escalate, resolve and uphold agreed resolution

Participants were also reminded of the UN Codes of Conduct, applying to work under ITU-T, found at https://www.un.org/management/sites/www.un.org.management/files/un-system-model-code-conduct.pdf

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM and ECM software implementation packages use the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11 of June/July 2009. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software header is also used in the process of designing the VTM and ECM software, and for evaluating proposals for technology to be potentially included in these designs. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well. The SADL and NNVC packages for neural network-based video coding use the same licensing terms.

Software packages that had been developed in prior work of the JVT, the JCT-VC and the JCT-3V have similar considerations and are maintained according to the past practice in that work.

## Communication practices

The documents for the meeting can be found at https://jvet-experts.org/. The site contains links for creating a user account for document uploading and for contacting an administrator – e.g., to update the email address associated with an account, and experts were reminded to keep this information up to date. It was reminded to send a notice to the chairs in cases of changes to document titles, authors, etc.

It was noted that also the most recent versions of JCT-VC and JCT-3V documents can be accessed directly via the JVET site. A mirror for JCT-VC, JCT-3V and JVET document access (including all versions with original upload dates) has also been provided via the ITU ftp site, providing a “documents” subfolder in the directory of each respective meeting.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1384 (as of 13 Jan. 2026). All discussions (including those on AVC, HEVC, VVC, CICP, etc.) shall be conducted on the JVET reflector rather than any of the old reflectors (including JVT, JCT-VC, and JCT-3V) which are retained for archiving purposes.

It is further emphasized that the document JVET-AJ1012 gives valuable hints about communication practices as well as other IT resources used in JVET, such as software, conformance, and test materials.

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARMC: Adaptive re-ordering of merge candidates

- ARSS: Adaptive reference sample smoothing

- ATM: AVC-based multiview and 3D test model

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter

- AVC: Advanced video coding – video coding standard developed by JVT, formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10

- BA: Block adaptive

- BC: See CPR or IBC

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001)

- BDOF: Bi-directional optical flow (formerly known as BIO)

- BDPCM: Block-wise DPCM

- BIF: Bilateral filter

- BL: Base layer

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardization

- BoG: Break-out group

- BR: Bit rate

- BT: Binary tree

- BV: Block vector (used for intra BC prediction)

- CABAC: Context-adaptive binary arithmetic coding

- CBF: Coded block flag(s)

- CC: May refer to context-coded, common (test) conditions, or cross-component

- CCALF: Cross-component ALF

- CCLM: Cross-component linear model

- CCCM: Cross-component convolutional model

- CCP: Cross-component prediction

- CCSAO: Cross-component SAO

- CE: Core Experiment – a coordinated experiment conducted for study of coding technology

- CfE: Call for Evidence

- CfP: Call for Proposals

- CG: Coefficient group

- CGS: Colour gamut scalability (historically, coarse-grained scalability)

- CICP: Coding-independent code points (Rec. ITU-T H.273 | ISO/IEC 23091-2)

- CIIP: Combined inter/intra prediction

- CL-RAS: Cross-layer random-access skip

- CNN: Convolutional neural network

- CPB: Coded picture buffer

- CPMV: Control-point motion vector

- CPMVP: Control-point motion vector prediction (used in affine motion model)

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- CST: Chroma separate tree

- CTC: Common test conditions

- CVS: Coded video sequence

- DCI: Decoder capability information

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics)

- DCTIF: DCT-derived interpolation filter

- DF: Deblocking filter

- DIMD: Decoder intra mode derivation

- DMVR: Decoder motion vector refinement

- DoCR: Disposition of comments report

- DPB: Decoded picture buffer

- DPCM: Differential pulse-code modulation

- DPS: Decoding parameter sets

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR)

- DT: Decoding time

- DQ: Dependent quantization

- ECS: Entropy coding synchronization (typically synonymous with WPP)

- EE: Exploration experiment – a coordinated experiment conducted for study of coding technology prior during exploration work prior to establishment of a formal project (EEn for some number n is exploration experiment number n)

- EMT: Explicit multiple-core transform

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element)

- ECM: Enhanced compression model – a software codebase and test model algorithm for future video coding exploration

- ECV: Extended Colour Volume (up to WCG)

- EIP: Extrapolation based intra prediction

- EL: Enhancement layer

- EOS: End of (coded video) sequence

- ET: Encoding time

- FGRC: Film grain regions characteristics

- FRUC: Frame rate up conversion (pattern matched motion vector derivation)

- GCI: General constraints information

- GDR: Gradual decoding refresh

- GFV: Generative face video

- GLM: Gradient linear model

- GOP: Group of pictures (somewhat ambiguous)

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- GS: Gaussian splatting

- HBD: High bit depth

- HDR: High dynamic range

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2

- HLS: High-level syntax

- HM: HEVC Test Model – a software codebase and video coding design containing selected coding tools that conforms to the HEVC standard design (possibly with under-development extensions) – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM)

- HMVP: History based motion vector prediction

- HOP: High-complexity operating point for NNLF.

- HRD: Hypothetical reference decoder

- HTM: HEVC-based multiview and 3D test model (developed by JCT-3V)

- HyGT: Hyper-cube Givens transform (a type of NSST)

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample)

- IBF: Intra boundary filtering

- ILP: Inter-layer prediction (in scalable coding)

- ILRP: Inter-layer reference picture

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC)

- IRAP: Intra random access picture

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JCT-3V: Joint collaborative team on 3D video (for AVC and HEVC 3D video extensions)

- JCT-VC: Joint collaborative team on video coding (for HEVC)

- JEM: Joint exploration model – a software codebase previously used for video coding exploration

- JM: Joint model – the primary software codebase that has been developed for the AVC standard

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions

- JVET: Joint video experts team (originally joint video exploration team, initially for VVC, later expanded in scope)

- JVT: Joint video team (for AVC)

- KLT: Karhunen-Loève transform

- kMAC: thousands of multiply-accumulate operations – used as a complexity metric

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures

- LB-CCP: Local-boosting CCP

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation

- LM: Linear model

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LOP: Low-complexity operating point for NNLF.

- LP or LDP: Low delay P – the variant of the LD conditions that uses P frames

- LUT: Look-up table

- LTRP: Long-term reference picture

- MANE: Media-aware network element

- MC: Motion compensation

- MCP: Motion compensated prediction

- MCTF: Motion compensated temporal pre-filtering

- MDNSST: Mode dependent non-separable secondary transform

- MIP: Matrix-based intra prediction

- MM-CCCM: Multi-model CCCM

- MMLM: Multi-model (cross component) linear mode

- MMVD: Merge with MVD

- MOS: Mean opinion score (a metric for subjective quality evaluation)

- MPEG: Moving picture experts group (an alliance of working groups and advisory groups in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET)

- MPM: Most probable mode (in intra prediction)

- MRL: Multiple reference line intra prediction

- MSE: Mean-squared error, a distortion metric

- MV: Motion vector

- MVD: Motion vector difference

- NAL: Network abstraction layer

- NNLF: Neural network loop filter

- NNPF: Neural network post filter

- NNVC: Neural network-based video coding (experimental software package)

- NSQT: Non-square quadtree

- NSPT: Non-separable primary transform

- NSST: Non-separable secondary transform

- NUH: NAL unit header

- NUT: NAL unit type (as in AVC and HEVC)

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F)

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value

- OLS: Output layer set

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display)

- ONNX: Open Neural Network Exchange – a format used to convert code from common neural network software packages into SADL code

- operation point: A temporal subset of an OLS

- PDPC: Position-dependent (intra) prediction combination

- PERP: Padded equirectangular projection (a 360° projection format)

- PH: Picture header

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format)

- PMMVD: Pattern-matched motion vector derivation

- POC: Picture order count

- PoR: Plan of record

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set

- PSNR: Peak signal-to-noise ratio, a fidelity metric derived from MSE

- PTL: Profile/tier/level combination

- QM: Quantization matrix

- QP: Quantization parameter (sometimes conflated with quantization step size)

- QT: Quadtree

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD)

- RADL: Random-access decodable leading (type of picture)

- RASL: Random-access skipped leading (type of picture)

- R-D: Rate-distortion

- RDO: Rate-distortion optimization

- RDOQ: Rate-distortion optimized quantization

- RDPCM: Residual DPCM

- ROI: Region of interest

- ROT: Rotation operation for low-frequency transform coefficients

- RPL: Reference picture list

- RPLM: Reference picture list modification

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC

- RPS: Reference picture set

- RQT: Residual quadtree

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q)

- RVM: Rate variation measure

- SAD: Sum of abolute differences, a measurement of distortion

- SADL: Small adhoc deep learning librar

- SAO: Sample-adaptive offset

- SBT: Subblock transform

- SbTMVP: Subblock based temporal motion vector prediction

- SCIPU: Smallest chroma intra prediction unit

- SCM: Screen content model – a software codebase and test model algorithm for the HEVC screen content coding extensions

- SD: Slice data; alternatively, standard-definition

- SDH: Sign data hiding

- SDT: Signal-dependent transform

- SE: Syntax element

- SEI: Supplemental enhancement information

- SGPM: Spatial geometric partition mode

- SH: Slice header

- SHM: Scalable HM – a software codebase and test model algorithm for SHVC

- SHVC: Scalable high efficiency video coding

- SIF: Switchable (motion) interpolation filter

- SIMD: Single instruction, multiple data

- SMVD: Symmetric MVD

- SPO: SEI processing order

- SPS: Sequence parameter set

- SR: Super-resolution

- STMVP: Spatial-temporal motion vector prediction

- STRP: Short-term reference picture

- STSA: Step-wise temporal sublayer access

- TALF: Temporal adaptive loop filter

- TBA/TBD/TBP: To be announced/determined/presented/public

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content

- TIMD: Template-based intra mode derivation

- TM: Template matching

- TMRL: Template-based multiple reference line prediction

- TMVP: Temporal motion vector prediction

- TS: Transform skip

- TSRC: Transform skip residual coding

- TT: Ternary tree

- TuC: Technology under consideration

- UGC: User-generated content

- UWP: Unequal weight prediction

- VCEG: Visual coding experts group (ITU-T Q.6/21, the relevant rapporteur group in ITU-T WP3/21, which is one of the two parent bodies of the JVET)

- VCM: Video coding for machines

- VLOP: Very low-complexity operating point for NNLF

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy

- VQA: Visual quality assessment

- VT: Verification testing

- VTM: VVC Test Model – a software codebase and test model algorithm for the VVC standard

- VUI: Video usability information (part of an SPS, similar to an SEI message that applies to a video sequence)

- VVC: Versatile Video Coding, video coding standard developed by JVET, formalized by ITU-T as Rec. ITU-T H.266 and by ISO/IEC as ISO/IEC 29090-3

- WCG: Wide colour gamut

- WG: Working group, a group of technical experts (usually used to refer to the WGs of ISO/IEC JTC 1/SC 29)

- WPP: Wavefront parallel processing (usually synonymous with ECS)

- Block and unit names in MPEG-2 and AVC:

  - MB: Macroblock – a 16x16 array for the luma component and corresponding 8x8 arrays for the chroma components (basically the same as a CTU in later standards, but with a fixed size)

  - MB pair: A pair of vertically contiguous macroblocks in a frame that is coupled for use in macroblock-adaptive frame/field decoding

  - Sub-MB: One quarter of the samples of a macroblock, i.e., an 8x8 luma block and two corresponding chroma blocks of which one corner is located at a corner of the macroblock for a picture that has three sample arrays or an 8x8 luma block of which one corner is located at a corner of the macroblock for a monochrome picture or a picture that is coded using three separate colour planes.

  - Sub-MB partition: A block of luma samples and two corresponding blocks of chroma samples resulting from a partitioning of a sub-macroblock for inter prediction for a picture that has three sample arrays or a block of luma samples resulting from a partitioning of a sub-macroblock for inter prediction for a monochrome picture or a picture that is coded using three separate colour planes

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU)

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions)

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape

  - PB: Prediction block, a luma or chroma block of a PU

  - PU: Prediction unit, has the same size as a CU in the VVC context

  - TB: Transform block, a luma or chroma block of a TU

  - TU: Transform unit, has the same size as a CU in the VVC context

## Standards, TRs, supplements and technical papers approval and publication status

- MPEG-2 | H.262 (the video coding specification is common text and not publicly available without cost per ISO special request)

  - ITU-T H.262 V3 was approved in 2012-02; its Amd 1 was approved in 2013-03 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-09

  - ISO/IEC 13818-2:2013 (Ed. 3) FDIS ballot closed 2012-05-08; FDAM 1 ballot closed 2013-04-12 and was not published separately; it was instead incorporated directly into the V3 text and published 2013-10

  - Conformance testing (not joint with ITU-T)

    - ISO/IEC 13818-4:2004 (Ed. 2) FDIS closed 2004-08-22, published 2004-12-12; it specifies conformance testing for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), and Part 7 (AAC)

    - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing

    - Cor 1:2007, Cor 2:2009, Cor 3:2012 (publication availability unclear, as these do not appear to be directly available from the ISO website) may also have video relevance

  - Reference software (not joint with ITU-T)

    - ISO/IEC TR 13818-5:2005 (Ed. 2) final ballot closed 2005-07-24, published 2005-10; it specifies reference software for Part 1 (Systems), Part 2 (Video), Part 3 (Audio), Part 7 (AAC) and Part 11 (IPMP)

- AVC (twin text)

  - ITU-T H.264 V14 was Consented at 22nd meeting on 2021-04-30 (with annotated regions, shutter interval, and miscellaneous corrections), approved 2021-08-22, published 2021-10-13 (ITU work programme link)

  - ISO/IEC 14496-10:2020 (Ed. 9) FDIS ballot closed 2020-11-27, published 2020-12-15

  - ISO/IEC 14496-10:2022 (Ed. 10), had been forwarded from DIS directly for publication 2022-01-21 (with annotated regions, shutter interval, and miscellaneous corrections) with an editing period, submitted to ITTF in 2022-05 after consultation with ISO staff on format of graphics files, upgraded to “DIS approved for registration” in ISO Project system 2022-07-04, published 2022-11-07

  - Preliminary draft text for YCgCo-Re and YCgCo-Ro issued at 26th meeting, second draft including SMPTE ST 2128 IPT-PQ-C2 issued at 28th meeting 2022-10, third draft issued at 29th meeting 2023-01, fourth draft issued at 30th meeting 2023-04, formal project requested and CD of 11th edition issued at 31st meeting 2023-07, DIS issued at 32nd meeting 2023-10, DAM ballot closed 2024-04-15

    - ITU-T H.264 V15 Consented 2024-04-26, last call began 2024-07-16, approved 2024-08-13, published 2024-11-13

    - ISO/IEC 14496-10:2025 (Ed. 11) FDIS issued at 34th meeting 2024-04, DIS approved for registration as FDIS 2024-09-17, FDIS ballot issued 2025-01-14, FDIS ballot closed 2025-03-23; published 2025-07-10

  - Amendment to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, ready to issue FDAM or FDIS at current meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting, targeting consent in 2026-07 (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.264.1 V6 Approved 2016-02-13, published 2016-06-17

    - Various amendments of ISO/IEC 14496-4:2004 (Ed. 2), including:

      - ISO/IEC 14496-4:2004/AMD 6:2005 Advanced Video Coding conformance

      - ISO/IEC 14496-4:2004/AMD 9:2006 AVC fidelity range extensions conformance

      - ISO/IEC 14496-4:2004/AMD 30:2009 Conformance testing for new profiles for professional applications

      - ISO/IEC 14496-4:2004/AMD 31:2009 Conformance testing for SVC profiles

      - ISO/IEC 14496-4:2004/AMD 38:2010 Conformance testing for Multiview Video Coding

      - ISO/IEC 14496-4:2004/AMD 41:2014 Conformance testing of MVC plus depth extension of AVC

      - ISO/IEC 14496-4:2004/AMD 42:2014 Conformance testing of Multi-Resolution Frame Compatible Stereo Coding extension of AVC

      - ISO/IEC 14496-4:2004/AMD 43:2015 3D-AVC conformance testing

      - ISO/IEC 14496-4:2004/AMD 45:2016 Conformance Testing for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps Extension of AVC

  - Reference software (twin text)

    - ITU-T H.264.2 V6 Approved 2016-02-13, published 2016-05-30

    - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) have been published, including:

      - ISO/IEC 14496-5:2001/AMD 6:2005 Advanced Video Coding (AVC) and High Efficiency Advanced Audio Coding (HE AAC) reference software

      - ISO/IEC 14496-5:2001/AMD 8:2006 AVC fidelity range extensions reference software

      - ISO/IEC 14496-5:2001/AMD 15:2010 Reference software for Multiview Video Coding

      - ISO/IEC 14496-5:2001/AMD 18:2008 Reference software for new profiles for professional applications

      - ISO/IEC 14496-5:2001/AMD 19:2009 Reference software for Scalable Video Coding

      - ISO/IEC 14496-5:2001/AMD 33:2015 Reference software for MVC plus depth extension of AVC

      - ISO/IEC 14496-5:2001/AMD 34:2014 Reference software of the multi-resolution frame compatible stereo coding of AVC

      - ISO/IEC 14496-5:2001/AMD 35:2015 3D-AVC Reference software

      - ISO/IEC 14496-5:2001/AMD 39:2016 Reference software for the Multi-resolution Frame Compatible Stereo Coding with Depth Maps of AVC

      - ISO/IEC 14496-5:2001/AMD 42:2017 Reference software for the alternative depth information SEI message extension of AVC

- HEVC (twin text)

  - ITU-T H.265 V7 approved 2019-11-29, published 2020-01-10

  - ISO/IEC 23008-2:2020 (Ed. 4) FDIS ballot closed 2020-07-16, published 2020-08-27

  - ITU-T H.265 V8 Consented at the 22nd meeting (with shutter interval information SEI message and miscellaneous corrections), approved 2021-08-22, published 2021-10-13

  - ISO/IEC 23008-2:2020/AMD 1:2021 (shutter interval information SEI message) published 2021-07-12

  - ISO/IEC 23008-2:2023 (Ed. 5) began as CDAM 2 High-range levels output of 25th meeting of January 2022, CDAM ballot closed 2022-04-15, conversion to 5th edition with miscellaneous corrections planned at 26th meeting of 2022-04, text submitted for DIS ballot 2022-07-10, DIS ballot closed 2023-01-10, FDIS issued 29th meeting of 2023-01, FDIS ballot opened 2023-08-06, closed 2023-10-02, published 2023-10-30

  - ITU-T H.265 V9 Consented at 31st meeting 2023-07, approved 2023-09-13, and pre-published 2023-09, published 2023-11-24

  - Preliminary draft HEVC text for YCgCo-Re and YCgCo-Ro issued at 26th meeting 2022-04, second draft including SMPTE ST 2128 issued at 28th meeting 2022-10, third draft at 29th meeting 2023-01, fourth draft at 30th meeting 2023-04, formal work item requested and CDAM1 issued 31st meeting 2023-07, DAM issued with new (multiview) profiles and SEI messages at 32nd meeting 2023-10, DAM ballot closed 2024-04-08

    - ITU-T H.265 V10 Consented 2024-04-26, last call opened 2024-07-01, closed 2024-07-28, approved 2024-07-29, pre-published 2024-08-06, published 2024-10-07

    - ISO/IEC 23008-2:2025 (Ed. 6) FDIS issued from 35th meeting 2024-04, DIS approved for registration as FDIS 2024-10-03, FDIS registered for formal approval 2024-10-13, FDIS ballot issued 2024-12-30, FDIS ballot closed 2025-02-24, published 2025-03-20

  - Amendment to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.265 (V11) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending pre-publication (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.265.1 V3 approved 2018-10-14, published 2019-01-15

    - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06

    - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15

  - Reference software (twin text)

    - ITU-T H.265.2 V3 approved 2016-12-22, published 2017-04-10

    - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for high efficiency video coding, FDIS issued from 2016-02 meeting, published 2017-03-01

    - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for screen content coding extensions, FDAM issued from 2017-04 meeting, FDAM ballot opened 2017-08-10, closed 2017-10-07, published 2017-11-09

- VVC (twin text)

  - ITU-T H.266 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23090-3:2021 (Ed. 1) published 2021-02-16

  - ITU-T H.266 V2 with operation range extensions, Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-06-06, published 2022-07-12

  - ISO/IEC 23090-3:2022 (Ed. 2) with operation range extensions, approval at WG level to proceed to FDIS 2022-01-21, FDIS ballot opened 2022-06-29, closed 2022-08-24, published 2022-09-25

  - ISO/IEC 23090-3:2024 (Ed. 3), initated as (Ed. 2) / Amd.1 New level and systems-related supplemental enhancement information, CDAM 1 issued from 26th meeting, ballot closed 2022-07-14, DAM 1 issued from 27th meeting, ballot closed 2023-01-03, FDIS issued at WG level 2023-07, FDIS ballot opened 2024-05-11, closed 2024-06-26, published 2024-07-17

  - ITU-T H.266 V3 Consented 2023-07, approved 2023-09-29 and pre-published 2023-09, published 2023-11-29

    - H.266 (2023) Erratum 1 published 2025-05 to correct errors in a few figures

  - ISO/IEC 23090-3:2024/DAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.266 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending-prepublication (ITU work programme link)

  - Conformance testing (twin text)

    - ITU-T H.266.1 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-15:2022 (Ed. 1) approval at WG level to proceed to FDIS 2022-10-15, upgraded to “DIS approved for registration” in ISO Projects system 2021-10-24, upgraded to “FDIS registered for formal approval” 2022-07-11, FDIS ballot closed 2022-11-04, published 2022-11-24

    - ISO/IEC 23090-15:2024 (Ed. 2) began as Amd.1 Operation range extensions – CDAM 1 issued from 24th meeting 2021-10, DAM 1 issued from 25th meeting 2022-01-21, upgraded to “CD approved for registration as DIS” status in ISO Projects system 2022-05-31, upgraded to “DIS registered” 2022-06-22, DAM ballot closed 2022-11-15, consolidated into FDIS 2nd edition issued as an output of the 29th meeting in January 2023, ballot opened 2024-04-08, closed 2024-06-03, published 2024-07-04

    - ITU-T H.266.1 V2 Consented 2023-07, approved 2023-09-13 and pre-published 2023-09, published 2023-10-19

    - ISO/IEC 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, closed 2025-06-30, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in mXXXX, ready for FDIS at current meeting (ISO Projects link)

    - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending-prepublication (ITU work prgramme link)

  - Reference software (twin text)

    - ITU-T H.266.2 V1 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, published 2022-07-12

    - ISO/IEC 23090-16:2022 V1 approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-04-21, upgraded to “FDIS registered for formal approval” 2022-04-22, FDIS ballot initiated 2022-07-24, FDIS ballot closed 2022-09-19, published 2022-10-23

    - ITU-T H.266.2 V2 Consented 2024-04-26, last call opened 2024-05-16, closed 2024-06-12, approved 2024-06-13, pre-published 2024-08-06, published 2024-10-09

    - ISO/IEC 23090-16:2025 (Ed. 2) Request & CD issued 2024-04, consultation deferred due to meeting timing, consultation initiated 2024-09-06, closed 2024-11-01, DIS issued at 36th meeting 2024-11, approved for registration as DIS 2024-12-18, DIS registered 2025-01-7, DIS ballot opened 2025-03-09, closed 2025-06-01, approved to proceed to publication without an FDIS at 29th meeting 2025-07, public availability requested 2025-07, published 2025-11-19, not available without cost. (Roughly corresponding to H.266.2 V2 already approved and published in ITU-T)

- VSEI (twin text)

  - ITU-T H.274 V1 approved 2020-08-29, published 2020-11-10

  - ISO/IEC 23002-7:2021 (Ed. 1) published 2021-01-28

  - ITU-T H.274 V2 Consented 2022-01-28, Last Call began 2022-04-01, Approved 2022-05-22 (after 1 Last Call comment and Additional Review), pre-published 2022-06-17, published 2022-07-25

  - ISO/IEC 23002-7:2022 (Ed. 2) approval at WG level to proceed to FDIS 2022-01-21, upgraded to “DIS approved for registration” status in ISO Projects system 2022-05-05 and “FDIS registered for formal approval” 2022-05-08, FDIS ballot closed 2022-09-27, published 2022-10-30

  - ISO/IEC 23002-7:2024 (Ed. 3) began as (2nd Ed.) Amd.1 Request for new edition and CD for additional SEI messages issued at 27th meeting, ballot closed 2022-10-10, DAM registered 2022-11-13, DAM ballot closed 2022-04-06, FDIS 3rd edition issued 2023-07, FDIS ballot began 2024-07-25, closed 2024-09-20, published 2024-10-30

  - ITU-T H.274 V3 Consent 2023-07, approved 2023-09-29, pre-published 2023-10-11, published 2024-03-12

  - ISO/IEC 23002-7:202x (Ed. 4) Request & CDAM 1 issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.274 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending-prepublication (ITU work programme link)

- CICP (twin text)

  - ITU-T H.273 V2 (with 4:2:0 sampling alignment and corrections for range of values for sample aspect ratio, ICTCP equations for HLG, and transfer characteristics function for sYCC of IEC 61966-2-1) Consented on 2021-04-30, Last Call closed during the 23rd meeting with approval on 2021-07-14, published 2021-09-24

  - ISO/IEC 23091-2:2021 (Ed. 2) had been forwarded from DIS directly for publication in 2021-04 and published 2021-10-18

  - ITU-T H.273 V3 Consent 2023-07, approved 2023-09, not published due to waiting for publication of SMPTE ST 2128

  - ITU-T H.273 V4 Consented 2024-04-26, last call opened 2024-06-16, closed 2024-07-13, approved 2024-07-14, pre-published 2024-08-06, published 2024-10-07

  - ISO/IEC 23091-2:2025 (Ed. 3) Request for new edition and CD for new edition (including YCgCo-Re and YCoCg-Ro) issued at 27th meeting, ballot closed 2022-10-10, DIS registered 2022-11-13, DIS ballot closed 2023-04-06, preliminary draft text for including SMPTE ST 2128 issued at 28th meeting, incorporated into preliminary FDIS at 30th meeting 2023-04, FDIS issued 2024-04 (after waiting for publication of SMPTE ST 2128, then proceeding), FDIS registered for formal approval 2024-10-24, FDIS ballot issued 2024-12-03, FDIS ballot closed 2025-01-28, published 2025-02-25

- Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics (twin text)

  - ITU-T H-Series Supplement H.Sup15 V1, approved 2017-01-27, published 2017-04-12

  - ISO/IEC TR 23008-14:2018 (Ed. 1) published 2018-08-06

- Signalling, backward compatibility and display adaptation for HDR/WCG video coding (twin text)

  - ITU-T H-Series Supplement H.Sup18 V1, approved 2017-10-27, published 2018-01-18

  - ISO/IEC TR 23008-15:2018 (Ed. 1) published 2018-08

- Usage of video signal type code points (twin text)

  - ITU-T H-Series Supplement H.Sup19 V3 approved 2021-04-30, published 2021-06-04

  - ISO/IEC TR 23091-4:2021 (Ed. 3) published 2021-05-23

- Working practices using objective metrics for evaluation of video coding efficiency experiments (twin text)

  - ITU-T H-Series Supplement HSTP-VID-WPOM V1 approved 2020-07-03, published 2020-11

  - ISO/IEC TR 23002-8:2021 (Ed. 1) published 2021-05-20

- Film grain synthesis technologies for video applications (twin text)

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Request for subdivision and WD 1 issued at 25th meeting 2022-01-21, WD 2 issued at 27th meeting, WD 3 issued at 28th meeting, CDTR issued at 29th meeting 2023-01, consultation period ended 2023-07-09, DTR text was issued from the 31st meeting in July 2023, put on hold by ISO staff editors, DTR ballot opened 2024-03-05, closed 2024-04-30, NB comments handled by ISO staff editor in consultation with project editors, published 2024-07-24

  - ITU-T H-Series Supplement H.Sup21 V1 (ex H.Sup-FGST), Agreement 2025-01-24 by ITU-T SG21, pre-publication 2025-03-26, published 2025-05-21

  - ISO/IEC TR 23002-9 (Ed. 2) Edition 2 planned but not yet in formal work programme of ISO/IEC

- Optimization of encoders and receiving systems for machine analysis of coded video content (twin text)

  - ISO/IEC TR 23888-3 (Ed. 1) Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closed 2025-11-03, summary of voting in m75361, ready for action at current meeting (ISO Projects link)

  - ITU-T provisional name H.Sup.MACVC (ITU work programme link)

- Freely available ISO/IEC standards (without cost) in the ISO Webstore and IEC Webstore include

  - ISO/IEC 13818-4:2004 Conformance testing for MPEG-2 (confirmed available 2025-10-31)

  - ISO/IEC 13818-4:2004/Amd 3:2009 Level for 1080@50p/60p conformance testing (confirmed available 2025-10-31)

  - ISO/IEC TR 13818-5:2005 Software simulation for MPEG-2 (confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-4:2004 specify conformance for AVC (base text confirmed available 2025-10-31)

  - Various amendments of ISO/IEC 14496-5:2001 (Ed. 2) specify reference software for AVC (base text confirmed available 2025-10-31)

- Intended to be freely available (without cost) but unavailable (without cost) standards and technical reports include the following. (These should be checked for previously issued requests for free availability.)

  - ISO/IEC 14496-10:2025 (Ed. 11) AVC, published 2025-07-10 (public availability request to be confirmed)

  - ISO/IEC 23002-7:2024 (Ed. 3) VSEI, published 2024-10-30 (public availability requested 2023-07)

  - ISO/IEC 23008-2:2025 (Ed. 6) HEVC, published 2025-03 (public availability requested 2024-04)

  - ISO/IEC 23008-5:2017 (Ed. 2) Reference software for HEVC, published 2017-03-01 (public availability requested 2016-02, reiterated 2025-10)

  - ISO/IEC 23008-5:2017/AMD 1:2017 Reference software for HEVC screen content coding extensions, published 2017-11-09 (public availability not requested 2017-01, 2017-04, 2017-07, 2017-10, 2018-01, public availability requested 2025-10)

  - ISO/IEC 23008-8:2018 (Ed. 2) Conformance specification for HEVC, published 2018-08-06 (public availability requested 2018-10, reiterated 2025-10)

  - ISO/IEC 23008-8:2018/AMD 1:2019 Conformance testing for HEVC screen content coding (SCC) extensions and non-intra high throughput profiles, published 2019-10-15 (public availability requested 2025-10)

  - ISO/IEC 23090-3:2024 (Ed. 3) VVC, published 2024-07-17

  - ISO/IEC 23090-15:2024 (Ed. 2) Conformance specification for VVC, published 2024-07-04

  - ISO/IEC 23090-16:202x (Ed. 2) Reference software for VVC, not yet published, public availability requested 2025-07

  - ISO/IEC 23091-2:2025 (Ed. 3) CICP, published 2025-02-25 (public availability requested 2024-04, reiterated 2025-10)

  - ISO/IEC TR 23008-14:2018 (Ed. 1) Conversion and coding practices for HDR/WCG Y′CbCr 4:2:0 video with PQ transfer characteristics, published 2018-08-06, public availability requested 2025-07

  - ISO/IEC TR 23008-15:2018 (Ed. 1) Signalling, backward compatibility and display adaptation for HDR/WCG video coding, published 2018-08, public availability requested 2025-07

  - ISO/IEC 23090-16:2025 (Ed. 2) Reference software for VVC, public availability requested 2025-07

  - ISO/IEC TR 23091-4:2021 (Ed. 3) Usage of video signal type code points, published 2021-05-23, public availability requested 2025-07

  - ISO/IEC TR 23002-8:2021 (Ed. 1) Working practices using objective metrics for evaluation of video coding efficiency experiments, published 2021-05-20, public availability requested 2025-07

  - ISO/IEC TR 23002-9:2024 (Ed. 1) Film grain synthesis technologies for video applications, published 2024-07-24 (not requested 2024-04), public availability requested 2025-07

- It appears necessary to check if all older software and conformance packages are publicly available – it might be that public availability for some was never requested, e.g. for those that were produced by JCT-3V.

## Draft standards progression status for active work items

- AVC ISO/IEC 14496-10:2025/DAM 1 to support some SEI messages of VSEI v4 requested at 37th meeting 2025-01, registered to work programme 2025-01-25, CDAM issued at 38th meeting 2025-04, consultation initiated 2025-04-23, closed 2025-06-18, DAM issued at 39th meeting 2025-07, ballot opened 2025-10-08, closed 2025-12-31, summary of voting in m75350, ready to issue FDAM or FDIS at current meeting (ISO Projects link)

  - ITU-T H.264 (V16) no action at current meeting, targeting consent in 2026-07 (ITU work programme link)

- HEVC ISO/IEC 23008-2:2025/DAM 1 to support additional (multiview) profiles and some SEI messages of VSEI v4, project requested at 36th meeting 2024-11, project registered in ISO/IEC work programme 2025-01-25, CDAM issued at 37th meeting, consulation began 2025-02-01, closed 2025-03-29, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-07-03, closed 2025-09-15, FDIS issued from 40th meeting 2025-10, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.265 (V11) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending pre-publication (ITU work programme link)

- VVC ISO/IEC 23090-3:2024/DAM 1, Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-29, closed 2025-09-21, FDIS issued at 40th meeting 2025-10, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.266 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending-prepublication (ITU work programme link)

- VVC conformance ISO/IEC DIS 23090-15 (Ed. 3) project requested at 37th meeting, project registered in work programme 2025-01-25, CD issued at 38th meeting 2025-04, consultation initiated 2025-05-05, pending closure 2025-06-30 during the current meeting, DIS issued at 39th meeting 2025-07, closed 2025-12-22, summary of voting in mXXXX, ready for FDIS at current meeting (ISO Projects link)

  - ITU-T H.266.1 (V3) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending-prepublication (ITU work programme link)

- VSEI ISO/IEC 23002-7:2024 (Ed. 3)/DAM 1 Request & CDAM issued 2024-04, consultation deferred due to meeting timing, updated text issued 2024-07, CD consultation initiated 2024-09-05, closed 2024-10-31, DAM issued at 38th meeting 2025-04, DAM ballot opened 2025-06-30, closed 2025-09-22, FDIS issued at 40th meeting 2025-10, pending FDIS ballot, public availability requested 2025-10 (ISO Projects link)

  - ITU-T H.274 (V4) consented 2025-10-17, last call started 2025-12-01, ended 2026-01-12, pending-prepublication (ITU work programme link)

- ISO/IEC TR 23002-9 (Ed. 2) Film grain synthesis technology for video applications – Edition 2 planned but not yet in formal work programme of ISO/IEC

- H.Sup.MACVC | ISO/IEC TR 23888-3 (Ed. 1) Optimization of encoders and receiving systems for machine analysis of coded video content – ISO/IEC TR 23888-3 Request for subdivision issued from 33rd JVET meeting 2024-01, CDTR issued from 34th meeting 2024-04, consultation deferred due to meeting timing, updated text issued from 35th meeting 2024-07, consultation further deferred due to meeting timing, further updated text issued from 36th meeting 2024-11, consultation initiated 2025-01-14, consultation period ended 2025-03-11, DTR issued at 38th meeting 2025-04, DTR issued at 38th meeting 2025-04, DTR ballot opened 2025-09-08, closes 2025-11-03, summary of voting in m75361, ready for action at current meeting (ISO Projects link)

- A request for free availability in ISO/IEC has to be made for each edition, amendment and corrigendum, and the request needs to be approved in the WG 5 Recommendations. A request form also needs to be filled out (but the form does not need to be issued as a WG 5 document). A freely available URL for the ITU publication should be provided for the ongoing work items when they become finalized.

## Opening remarks

Remarks during the opening session of the meeting Wednesday 14 January at 1330-XXXX UTC were as follows.

- Timing and organization of the meeting and online access and calendar posting of session plans were reviewed

  - The initial number of documents was lower (approximately 185 vs. 200-230 for recent meetings by the time of opening)

  - Parallel sessions were considered to be necessary (SEI and core coding aspects), starting Wednesday evening

  - Aspects of common interest (draft CfP, non-SEI HLS, GS SEI) to be reviewed at plenary level

- Future non-virtual meetings will be held as hybrid meetings with best-effort remote access (when under ISO/IEC SC 29 auspices) or as physical meetings with remote participation (when under ITU-T SG21 auspices). In terms of practical arrangements, this is identical.

- Plans for subsequent non-virtual meetings were reviewed: April 2026 (Santa Eulària), July 2026 (Geneva), October 2026 (Hangzhou), January 2027 (Brisbane), April 2027 (likely Geneva), July 2027 (Tampere), October 2027 (Shenzhen), April 2028 (likely Geneva); meeting location for January 2028 is still open.

- Plans for an AHG17 interim meeting in Aachen, DE, 25-27 February 2026 for further preparation of CfP test conditions

- Depending on the status of preparing future standardization activities, it may be necessary to extend the duration of meetings, or plan for AHG meeting days prior to the regular meeting (e.g., for analysis of CfP submissions)

- The meeting logistics, agenda, working practices, policies, and document allocation considerations were reviewed.

  - Access to the meeting was provided using Zoom. The meeting notes by the session chair were to be continually shared via zoom screen sharing

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

- The results of the previous meeting and the meeting report JVET-AN1000 were reviewed (which was finalized shortly before the meeting, but sufficiently mature drafts had been available before in the ITU ftp site). Only minor issues in the meeting report were noted and were not considered sufficient to warrant issuing a revision.

- At the current meeting, attendance will be recorded via the zoom reports. It is therefore important to follow the conventions of naming as mentioned before. Participants who cannot be correctly identified will not appear in the attendance list of the report.

- There were no objections voiced in the opening plenary to the consideration of late contributions.

- There were again a significant number of documents registered where authors’ given names were not abbreviated according to the JVET custom (which helps produce shorter headings in the JVET meeting report), and/or company affiliation was missing in the authors’ list. Participants were reminded to stick to JVET’s conventions. As for now, the JVET chair took action of correcting those cases in the document registry.

- To say it again:

  - Experts were asked not to pick a specific JVET number for regular documents – this function is reserved for AHG reports, summary reports, and output docs. Reserving numbers without filling a precise title shall also be avoided – the chair may flag such documents as withdrawn, as they cannot be allocated to a certain category in the meeting notes.

  - Experts were asked to always register JVET documents via the “jvet-experts.org” site, not via the MPEG DMS site, as WG 5 docs (as that feature of the DMS site has not been working properly).

  - Experts were asked to inform the chair when the title of a document is changed, or if authors are added. Otherwise, that might not be correct in the meeting notes. Provisional titles such as “EEx contribution” shall be avoided.

- The following ballot results had become available through the SC 29 secretariat:

  - Summary of Voting on ISO/IEC 14496-10:2025/DAmd 1 (Ed 11) m75350

  - Summary of Voting on ISO/IEC DTR 23888-3 Optimization of encoders and receiving systems for machine analysis of coded video content m75361

  - Summary of Voting on ISO/IEC DIS 23090-15 Conformance testing for VVC m75347

- It is planned to produce the FDIS of a new edition for AVC at this meeting (instead of issuing FDAM), with long editing period, expecting ballot comments to return before the July meeting (where also submission for ITU-T consent on the next versions of H.264 is planned). Alternatively, it would be possible to issue the FDAM, and produce the FDIS in July. A DoCR needs to be produced in any case, plus a suitable recommendation.

- The ballot on the DTR 23888-3 contains comments for technical changes. If JVET wants to consider those, it would be necessary to issue another DTR. Ballot results could still become available before the July 2026 meeting, when it is planned to submit the ITU version. A DoCR needs to be produced in any case.

- The 3rd edition of VVC conformance could be published directly from DIS stage, if there are no technical comments. No DoCR may be necessary, but a suitable recommendation. This would be desirable as the twin text of ITU-T H.266.1 was already consented in the previous meeting.

- Plans for generating the next versions of reference software for AVC, HEVC and VVC (which would need to support new SEI messages from VSEIv4) need to be developed. Earliest target date for ITU-T consent would be July 2026 – should requests be made for ISO? For AVC, it might further be useful to generate a new part of ISO/IEC 14496 containing the entire newest JM software at least, and cancel the corresponding outdated amendments of 14496-5.

- The primary goals of the meeting were:

  - Proceed AVC amendment and prepare next version/edition

  - Next steps towards TR on machine analysis

  - VVC conformance 3rd ed. (ISO/IEC) finalization

  - Draft CfP preparation, including various aspects of test conditions and complexity analysis

  - TuC on VSEI and CICP

  - Non-SEI HLS aspects

  - Exploration Experiments

    - Neural network-based video coding

    - Enhanced compression beyond VVC

  - Liaison communication – JPEG (outgoing liaison was intended to be sent by last meeting, to be updated), SG21 sent one to SC 29 WGs on Gaussian splatting (requires joint meeting, may not need response)

  - VVC white paper update (remove VSEI, some update of the 2021 version)

- Joint meetings were expected with MPEG AG 5 (on matters involving visual quality assessment), and with MPEG WG 2 Requirements and ITU-T VCEG on future video standardization (see section 7.3). Another joint meeting is expected  be necessary on Gaussian splatting (SEI proposals and proposal for JVET work item) with VCEG, WGs 2, 4 and 7.

- As a follow-up to communication after previous meetings, parent bodies are making further considerations about future JVET management structures – no action at current meeting.

- Principles of standards development were discussed.

- Scheduling of sessions was discussed – see under sections 2.6 and 2.15.

## Scheduling of discussions

- The plans for the times of meeting sessions were established as follows, in UTC (which for this meeting was 1 hour behind the time in Geneva and Paris; 8 hours ahead of the time in Los Angeles, 8 hours behind the time in Beijing, 9 hours behind the time in Seoul and Tokyo, etc.). No session was scheduled to last longer than 2 hrs.

- 1300–1500 1st “afternoon” session [break after 2 hours]

- 1520–1720+1 2nd “afternoon” session

- [“evening” break – nearly 4 hours]

- 2100–2300 1st “night” session [break after 2 hours]

- 2320–0120+1 2nd “night” session

Particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 14 January, 1st day

  - Afternoon sessions:

    - 1300–1410 Opening remarks, review of practices, agenda, IPR policy reminder, scheduling

    - 1415–1505 Reports of AHGs 1-5

    - 1525–1720 Reports of AHGs 6-19 (except 13)

  - Night sessions:

    - 2100–2300 EE1 (summary report and related)

    - 2320–0120+1 EE2 summary report

    - 2100–0120+1 HLS track: 6.1, 6.2.1

- Thu. 15 January, 2nd day

  - Afternoon sessions:

    - 1300–1515 Plenary: AHG13 report, Draft CfP discussion (4.16.1)

    - 1535–1730 Draft CfP discussion (4.16.2)

    - 1530–1730 HLS track: 6.2.1, Remaining 6.1, 6.2.2, 6.2.3

  - Night sessions:

    - 2100–2300 EE1 5.1.1 summary report finalization and 5.1.3/5.1.4

    - 2320–0120+1 EE2 5.2.1 summary report finalization and 5.2.3/5.2.4

    - 2100–0120+1 HLS track: 6.2.1, Remaining 6.1, 6.2.2, 6.2.3, 6.2.4, 6.2.5, 6.2.6, 6.2.7

- Fri. 16 January, 3rd day

  - Afternoon sessions:

    - 1300–1500 Ultra low latency and packet loss resilience (4.17)

    - 1520–1730 Complexity analysis (4.8, 4.18)

    - 1530–1730 HLS track: 6.2.5, 6.2.6, 6.2.7

  - Night sessions:

    - 2100–2300 Draft CfP discussion (4.16)

    - 2320–0120+1 Remaining EE1 (5.1.3)

    - 2100–0120+1 HLS track: 6.2.5, 6.2.6, 6.2.7, 6.2.8, 6.2.9

- Mon. 19 January, 4th day

  - 0500–0800 MPEG information sharing session

  - Afternoon sessions:

    - 1300–1500 JVET plenary: Coordination, scheduling, planning, reporting, plenary-level doc review

    - 1530–1600 Joint meeting with VCEG and WG 4: Lenslet video coding

    - 1600–1730 Joint meeting with VCEG, WG 4 and WG 7: Gaussian splat coding

  - Night sessions:

    - 2100–2300 BoG on draft CfP (F. Bossen, M. Wien)

    - 2100–2300 HLS track: 4.19 CICP, 6.5 non-SEI HLS, 6.3.2 Other SEI

    - 2320–0120+1 BoG on complexity criteria (X. Li, Y. Zhao)

    - 2320–0120+1 HLS track: 6.2.8, 6.2.9, 6.2.13, 6.2.14, 6.2.15, 6.2.16, 6.2.17, 6.2.18

- Tue. 20 January, 5th day

  - Afternoon sessions:

    - 1300–1500 JVET session (chaired by E. Alshina): Remaining EE1/NNVC (5.1.3/5.1.4)

    - 1300–1500 Review/discussion of GS SEI (6.3.1) joint with WG 4 and WG 7

    - 1600–1720 JVET session (chaired by. Y. Ye): non-EE2 (5.2.4)

    - 1520–1720 HLS track: 6.2.8, 6.2.9, 6.2.13, 6.2.14, 6.2.15, 6.2.16, 6.2.17, 6.2.18

  - Night sessions:

    - 2100–2300 Report of BoGs, Follow-up discussion on ULL/ER in CfP

    - 2320–0120+1 BoG on CfP drafting

    - 2100–0120+1 HLS track: 6.2.8, 6.2.9, 6.2.13, 6.2.14, 6.2.15, 6.2.16, 6.2.17, 6.2.18

- Wed. 21 January, 6th day

  - 0500–0600 MPEG information sharing session

  - Afternoon sessions:

    - 1300–1500 Remaining doc review/revisits NNVC 5.1.x, ECM 5.2x

    - 1300–1500 HLS track: 6.2.13, 6.2.16, 6.2.17, 6.2.18, Revisits, 6.3.2

    - 1520–1720 Remaining doc review 4.2, 4.10, 4.12, 4.15

    - 1520–1720 HLS track: 6.2.16, 6.2.17, 6.2.18, Revisits, 6.3.2

  - Night sessions:

    - 2100–2340 JVET plenary: Scheduling, status and open issues from review in main & HLS tracks, draft liaison responses, review of draft CfP

    - 0000+1–0150+1  remaining doc review 4.x, 5.2.5, 5.2.4

    - 2320–0120+1 HLS track: 6.2.18, Revisits, 6.3.2, 6.4

- Thu. 22 January, 7th day

  - Afternoon sessions:

    - 1300–1420 HLS remaining business, discuss mandate proposal of joint AHG on GS.

    - 1430–1600 Joint meeting with VCEG, WG 2 and AG 5: CfP next generation video coding

    - 1620–1720 tbd

  - Night sessions:

    - 2100–2300 JVET Plenary: DoCR review, review of complexity reporting template, remaining doc review, revisits, output doc planning and EE/AHG planning

    - 2320–0120+1 tbd

- Fri. 23 January, 8th day

  - 1300–1720 (with break) JVET plenary:

    - Any remaining doc review & revisits

    - EE document review

    - CfP planning and draft CfP review

    - Establishment of AHGs

    - Approval of output docs

    - Review of WG 5 meeting recommendations

    - Software timeline

    - Future planning, a.o.b.

  - 2100–2300 MPEG information sharing session

  - 00XX–00XX (Saturday) WG 5 approval of meeting recommendations, closing of meeting

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks and summary reports, and may not be completely accurate; documents which are allocated to multiple sections are only counted in one of them):

- AHG reports (19) (section 3)

- Project development (section 4)

  - AHG1: Deployment and advertisement of standards (2)

  - AHG2: Text development and errata reporting (0)

  - AHG3: Software development (2)

  - AHG3: Test conditions and metrics (0)

  - AHG4: Subjective quality testing and verification testing (1) TBP

  - AHG4: Test and training material (0)

  - AHG5: Conformance test development (0)

  - AHG7: ECM tool assessment (3)

  - AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (1) TBP

  - AHG10: Encoding algorithm optimization (1)

  - AHG13: Film grain technology (0)

  - Implementation studies (1)

  - Profile/tier/level specification (0)

  - AHG15: Gaming content compression (2)

  - AHG16: Generative face video (0)

  - AHG17: CfP on video coding technology (12)

  - AHG18: Ultra-low latency and error resilience (4)

  - AHG19: Hardware implementation complexity (4) 2 TBP

  - CICP (1)

- Low-level tool technology proposals (section 5) with subtopics (number counts excluding BoG and summary reports)

  - AHG11/AHG14 and EE1: Neural network-based video coding (32) (section 5.1)

  - AHG6/AHG12 and EE2: Enhanced compression beyond VVC capability (39) (section 5.2)

- AHG9: High-level syntax (HLS) proposals (section 6) with subtopics

  - Aspects of SEI messages in VSEI, VVC, HEVC and AVC (8) (section 6.1)

  - SEI messages in TuC for VSEI (54) (section 6.2)

  - SEI messages on other topics (14) (section 6.3)

  - SEI software and showcases (2) (section 6.4)

  - Non-SEI HLS aspects (4) (section 6.5)

- Joint meetings, plenary discussions, BoG reports (2) liaison (4), summary of actions (section 7)

- Project planning (section 8)

- Establishment of AHGs (section 9)

- Output documents (section 10)

- Future meeting plans and concluding remarks (section 11)

The document counts above do not include cross-checks and summary reports.

# AHG reports (19)

These reports were discussed during 1415–1720 UTC (with break 1505-1525) on Wednesday 14 Jan. 2026 (chaired by JRO).

JVET-AO0001 JVET AHG report: Project Management (AHG1) [J.-R. Ohm (chair), G. J. Sullivan (vice chair)]

The reflector used for discussions by the JVET and all of its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

The number of subscribers (on the day before the beginning of the current meeting) was 1384 (compared to 1348 by the time of the previous meeting). Furthermore, the previous lists of joint teams (which were still kept open as archives) had the following number of subscribers:

- JCT-VC – 1158 subscribers

- JCT-3V – 680 subscribers

- JVT-experts – 2078 subscribers

It is likely that many subscriptions of these latter three reflectors would have become obsolete, as no emails are being sent over them any more, and therefore automatic removal of addresses causing permanent bounces by being invalid (as it happens with the main JVET reflector) cannot be executed by the reflector management system.

- Goals and activity

The work of the JVET overall had proceeded well in the interim period with increased number of input documents (as compared to the previous meeting) submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and output documents from the preceding meeting had been produced as planned.

Output documents from the preceding meeting had been made initially available at the JVET web site (https://jvet-experts.org/) or the ITU-based JVET ftp site (http://wftp3.itu.int/av-arch/jvet-site/2025_10_AO_Geneva/). JCT-VC and JCT-3V documents can also be accessed directly via the JVET site. Previous meeting documents of JVT, JCT-VC, JCT-3V and JVET are also available from the ITU-based ftp site, where sub-folders of the different groups can be found under https://www.itu.int/wftp3/av-arch/, and sub-folders ‘./documents/’ were created in the folders of the respective meetings (it is noted that, for practicality reasons, for JVET this is only done after all meeting documents including output documents are completely available in all versions, such that there may be some delay in availability). All those files come with the original time stamps by which the original versions of documents were uploaded.

The list of output documents produced since the last meeting included the following:

- JVET-AN1005 Future CICP extensions (Draft 2) [Posted 2025-11-03]

- JVET-AN1016 AVC with extensions and corrections (Draft 4) [Posted 2026-01-14]

- JVET-AN1017 Support for additional VSEI messages in AVC (Draft 4 [Posted 2025-10-26]

- JVET-AN1018 HEVC with extensions and corrections (Draft 3), also forwarded for consent as SG21-TD210/PLEN ITU-T H.265 (V11) "High efficiency video coding" (Rev.) [Posted 2025-10-17, last update 2025-12-13]

- JVET-AN1019 VSEI with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD212/PLEN ITU-T H.274 (V4) "Versatile supplemental enhancement information messages for coded video bitstreams" (Rev.) [Posted 2025-10-17]

- JVET-AN2001 VVC with extensions and corrections (Draft 1), also forwarded for consent as SG21-TD211/PLEN ITU-T H.266 (V4) "Versatile video coding" (Rev.) [Posted 2025-10-17, last update 2025-12-15]

- JVET-AN2008 Conformance testing for versatile video coding (Draft 8), also forwarded for consent as SG21-TD214/PLEN ITU-T H.266.1 (V3) " Conformance specification for ITU-T H.266 versatile video coding" (Rev.) [Posted 2026-01-01]

- JVET-AN2019 Description of algorithms version 13 and software version 15 in neural network-based video coding (NNVC) [Posted 2025-12-13]

- JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4) [Posted 2026-01-14]

- JVET-AN2021 Verification test plan for VVC multilayer coding (update 7) [Posted 2026-01-08]

- JVET-AN2023 Exploration experiment on neural network-based video coding (EE1) [Posted 2025-10-11, last update 2025-10-26]

- JVET-AN2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [Posted 2025-10-12]

- JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19) [Posted 2025-12-15]

- JVET-AN2026 Edits on top of the Joint Call for Evidence on video compression with capability beyond VVC [Posted 2025-10-13]

- JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [Posted 2025-12-20]

- JVET-AN2032 Technologies under consideration for future extensions of VSEI (version 10) [Posted 2025-11-19, last update 2026-01-06]

- JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [Posted 2025-11-01, last update 2025-12-24]

- JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools [Posted 2025-12-02]

- JVET-AN2041 Preliminary Announcement of JVET AHG17 Meeting in Aachen, DE, during 23-27 February 2026 [Posted 2025-10-23]

The nineteen ad hoc groups had made progress, and reports from those activities had been submitted. Numerous teleconferences had been held by AHGs. Furthermore, two exploration experiments (EE) on neural network-based video coding and on enhanced compression beyond VVC capability were conducted, and 5 submissions were received as responses to the Joint Call for Evidence on video compression with capability beyond VVC.

The arrangements for the 41st meeting had been announced in the JVET reflector, in the JVET logistics document (https://www.itu.int/wftp3/av-arch/jvet-site/2026_01_AO_Virtual/JVET-AO_Logistics.docx), and in the WG 5 calling notice (N 380) and agenda (N 381) for the 22nd WG 5 meeting.

Software integration was finalized approximately according to the plan.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

More than 180 input contributions (not counting the AHG reports, summary reports and crosschecks) had been registered for consideration at the current meeting.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, maintenance, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

To retain a consistent numbering scheme, the number range of output documents starting from 1001 was reserved for the previous JCT-VC and JCT-3V topic items listed above, whereas the number range starting from 2001 was retained for VVC, VSEI and exploration activities.

A preliminary basis for the document subject allocation and meeting notes for the 41st meeting had been made publicly available on the ITU-hosted ftp site as http://wftp3.itu.int/av-arch/jvet-site/2026_01_AO_Virtual/JVET-AO_notes_d0.docx.

Further discussion needs to be conducted with the parent bodies about future JVET management structures, considering that substantially higher workload could be expected in upcoming standardization activities.

- Recommendations

- The AHG recommends its continuation.

JVET-AO0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice chairs)]

- Ad hoc group activity

  - Output documents produced

    - JVET-AN1016 AVC with extensions and corrections (Draft 4) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang]

The 16th edition of Rec. ITU-T H.264, approved in 2024-08, specifies additional SEI messages for SEI processing order SEI message, the processing order nesting SEI message, the encoder optimization information SEI message, the source picture timing information SEI message, the modality information SEI message, the generative face video SEI message, the generative enhancement face video message, the digitally signed content initialization SEI message, the digitally signed content selection SEI message, digitally signed content verification SEI message and the AI usage restrictions request SEI message (through referencing to Rec. ITU-T H.274 | ISO/IEC 23002-7).

    - JVET-AN1017 Support for additional SEI messages in AVC (Draft 4) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wan]

This document contains the draft text for changes to the Advanced Video Coding (AVC) standard (Rec. ITU-T H.264 | ISO/IEC 14496-10). The changes include the support of five new SEI messages, namely the source picture timing information, modality information, digitally signed content initialization, digitally signed content selection, and digitally signed content verification SEI messages, through referencing the Versatile Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7).

Changes that have been integrated for the 40th JVET meeting in October 2025:

- JVET-AN0185 AHG9: Miscellaneous on VSEI codec interfaces

- JVET-AN0059 AHG9: On the SPO and PON SEI messages in VSEI version 4 and in the AVC, HEVC, and VVC interface text

- JVET-AN0184 AHG9: Miscellaneous on VSEI v4 draft : add the derivation of CroppedWidth and CroppedHeight for the AVC interface.

- JVET-AN0363 Interface text for SEI message extensions

- JVET-AN0229 AHG9: On the film grain characteristics SEI message in VSEI v4

    - JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis]

This document provides a draft text with all changes in JVET-AM1006 integrated to the full text of Rec. ITU-T H.265 (V10) (07/2024), plus some remaining HEVC errata items from JVET-AL1004 and all JVET decisions at the 40th JVET meeting for HEVC, including relevant suggestions from the DAM ballot DoCR in WG5 output document N0375.

The draft text is provided in an attachment to this document.

The attachment in -v1 of this document was used as submission for ITU-T consent. The text in -v1 of this document, which is the same as the ITU-T consented text, was used as the basis for generating the Last Call text for ITU-T H.265 v11 and as the alignment basis for generating the FDIS text for the seventh edition of ISO/IEC 23008-2.

Below is a summary of the changes per all JVET decisions at the 40th JVET meeting for HEVC, including relevant suggestions from the DAM ballot DoCR in WG5 output document N0375:

- JVET-AN0071: Update to Equation D-61 per the recommended solution in JVET-AN0071-v3

- JVET-AN0185: 1) Update to the constraint on NNPFC SEI message repetition, 2) addition of constraints related to cubemap projection SEI message in a processing chain, and 3) addition of a missing payloadType value to SeiProcessingOrderSeiList

- JVET-AN0186: Typo correction

- JVET-AN0229 on the FGC SEI message: 1) Update to the syntax extension, 2) Addition of an inference rule, and 3) some editorial changes

- JVET-AN0059: Update the constraint on cubemap projection SEI message in the interface text for the SEI processing order and processing order nesting SEI messages

- JVET-AN0184: Addition of a constraint that the value of spti_max_sublayers_minus1 shall be less than 7.

- Aligning NNPF interface text with VVC, by integrating adopted VVC NNPF interface text changes from the JVET-AK, JVET-AL, and JVET-AM meetings

- Change of "generative_face_enhancement" to "generative_enhancement_face_video"

- JVET-AN0363: Setting the variable SeiExtensionEnabledFlag set equal to 1

- JVET-AN0183: Addition of a constraint requiring all layers identified by pri_region_layer_id[ i ] for i in the range from 0 to pri_num_regions_minus1, inclusive, are output layers of an OLS.

- JVET-AN0072: Update of the reference of IEC 60559:1989 to ISO/IEC 60559 (in force): Information technology – Microprocessor systems – Floating-point arithmetic

- Change of "ai_usage_restrictions" to "ai_usage_restrictions_request"

- Addition of a summary of changes in this new version relative to the previous version

- DAM ballot comment 005: Avoiding use of the forbidden word “could”

- DAM ballot comment 010: Correction of inconsistent indentation in the FGC SEI message syntax table

- DAM ballot comment 012: Removal of the following unnecessary constraint: It is a requirement of bitstream conformance that when fg_spatial_resolution_present_flag is present in the SEI message, the return value of the more_data_in_payload( ) function in the sei_payload( ) syntax structure containing the SEI message shall be equal to 1.

- DAM ballot comment 013: Mention of the SPO SEI message in NOTE 10 in the FGC SEI message semantics

- DAM ballot comment 014: Change of "cut frequency" to "cutoff frequency".

- DAM ballot comment 017: Use consistent indentation for the bullet-listed paragraphs in D.4.4

Below is a summary of the additional changes in -v2 of this document, relative to the text in -v1 (note that the -v1 text was submitted for the ITU-T consent, these additional changes were for alignment with the ISO text and, in some cases, also for consistency or bug fixing within the ITU-T text itself, and the -v2 text was submitted for ITU-T Last Call):

- Slightly reworded the description of additions in this new version, including additionally mentioning the extension to the FGC SEI message.

- Replaced “tableMPSSyncTableMpsValWpp” with “TableMpsValWpp” (what a typo!)

- Removed the clause and year when referencing ISO/IEC 11578.

- Updated references of ISO 11664 to ISO/CIE 11664.

- Fixed the semantics of omni_viewport_tilt_centre[ i ] (missing “180 * ”, ).

- Replaced “Y = !X” with “Y = 1 − X”. (Item 1 in 4.3.1 of JVET-AL1004; Also aligning with VVC v4)

- Rewording of several NOTEs, including changing “it is possible that something will happen” to “something can happen”.

- Made the clause “3D reference displays information SEI message semantics” one level higher, from G.14.3.2.3 to G.14.3.3.

- Some minor changes.

    - JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang]

This document provides a draft text of changes to VSEI integrated to the full text of Rec. ITU-T H.274 (V3) (09/2023), including JVET-AN2006 and all JVET decisions at the 40th JVET meeting for VSEI, including relevant suggestions from the DAM ballot DoCR in WG5 output document N00349.

The draft text is provided in an attachment to this document. The attachment in -v1 of this document was used as submission for ITU-T consent. This text, which is the same as the ITU-T consented text, will be used as the basis for generating final ITU-T H.274 V4.

    - JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan]

This document provides a draft text with all changes in JVET-AM2005 integrated to the full text of Rec. ITU-T H.266 (V3) (09/2023), all changes in ITU-T H.266 v3 erratum 1 (replacing figures 4, 5, 6, and 16), plus some remaining VVC errata items from JVET-AL1004, and all JVET decisions at the 40th JVET meeting for VVC, including relevant suggestions from the DAM ballot DoCR in WG5 output document N0377.

The draft text is provided in an attachment to this document.

The attachment in -v1 of this document was used as submission for ITU-T consent. The text in -v1 of this document, which is the same as the ITU-T consented text, was used as the basis for generating the Last Call text for ITU-T H.266 v4 and as the alignment basis for generating the FDIS text for the fourth edition of ISO/IEC 23090-3.

Below is a summary of the changes per all JVET decisions at the 40th JVET meeting for VVC, including relevant suggestions from the DAM ballot DoCR in WG5 output document N0377:

- JVET-AN0059: Removal of a constraint in D.12.13

- JVET-AN0363: Setting of the variable SeiExtensionEnabledFlag equal to 1

- JVET-AN0161: Addition of ", inclusive" to a constraint in D.12.16

- JVET-AN0183: Addition of a constraint requiring all layers identified by pri_region_layer_id[ i ] for i in the range from 0 to pri_num_regions_minus1, inclusive, are output layers of an OLS.

- JVET-AN0184: Addition of a constraint that the value of spti_max_sublayers_minus1 shall be less than 7.

- JVET-AN0185: Update to the constraint on NNPFC SEI message repetition

- Change of "generative_face_enhancement" to "generative_enhancement_face_video"

- Change of "ai_usage_restrictions" to "ai_usage_restrictions_request"

- DAM ballot comment 008: For consistency, added square brackets in the assignment expressions for AlfFixFiltCoeff and AlfClassToFiltMap in 7.4.3.18

- DAM ballot comment 009: Reworded the NOTE in 8.5.1, including replacing “The array refMvLX is stored in MvDmvrLX” with “The array of luma motion vectors after decoder-side motion vector refinement refMvLX is stored in the array MvDmvrLX” and replacing “The array of non-refine luma motion vectors MvLX” with “The array of luma motion vectors before decoder-side motion vector refinement mvLX”.

- DAM ballot comment 011: Reworded the terms defined at the beginning D.12.11 to be in the same style as in the definitions in clause 3.

- DAM ballot comment 012: Reworded the first NOTE in D.12.11 as body text.

- DAM ballot comment 013: Reworded the NOTE 3 in D.12.11 as body text.

- DAM ballot comment 013: Replaced “OMI” in D.12.16 with “object mask information”.

Below is a summary of the additional changes in -v2 of this document, relative to the text in -v1 (note that the -v1 text was submitted for the ITU-T consent, these additional changes were for alignment with the ISO text and, in some cases, also for consistency or bug fixing within the ITU-T text itself, and the -v2 text, except correctio of the equation index “(1052-bis)” followed by update of all fields, was submitted for ITU-T Last Call):

- Removed redundant settings of SeiExtensionEnabledFlag equal to 1.

- Fixed bug ticket #1651 (References to various non-existent syntax elements relating to sps_partition_constraints_override_enabled_flag).

- Fixed bug ticket #1652 (Inconsistent heading style of B.2, B.2.1, B.2.2, B.3, B.4 headings; still need to be applied to the ISO text).

- Fixed bug ticket #1653 (Replaces seven instances of “nuh_unit_type” with “nal_unit_type”).

- Replaced one occurrence of sps_loop_filter_across_subpic_enabled_pic_flag[ i ] with sps_loop_filter_across_subpic_enabled_flag[ i ].

- Replaced one occurrence of “nubmer of layer” with “number of layers”.

- Removed the definitions of “AC transform coefficient” and “associated GDR subpicture”, which are not used anywhere.

- Splitting of the definitions of “byte-aligned”, “level”, and “transform unit” similarly as in the ISO text such that the definition subclause numbers are all aligned between the two texts.

- Moved the definition of “inter prediction” such that the definitions are consistently in alphabetical order.

- Added the definitions of the abbreviations CLVS, CLVSS, CVSS, and TU.

- Removed “Provides a look at the next n bits in the bitstream with n being its argument.” From the definition of next_bits( n ).

- Added a NOTE on the b(8) syntax descriptor.

- In 7.4.2.4.5, replaced “subpicture level values” with “subpicture index values”. (JVET-AL1004 VVC item 2)

- Fixed spaces missing or extra spaces per JVET-AL1004 VVC item 1.

- In D.4.2, “max_val_initial_removal_delay_for_splicing” should be “bp_max_initial_removal_delay_for_concatenation” (JVET-AL1004 VVC item 6).

- In Table A.2 added the row for level 15.5 and slightly updated the description of Table A.2.

- Added a clarification that expressions of “1/2”, “1/4”, “1/16” and “1/32” etc. don’t use ‘/’ as the integer division with truncation of the result toward zero; rather they are just ordinary fractions in prose (JVET-AL1004 VVC item 5).

- Avoided indexing equations with the suffixing of “-bis” after an equation number.

- Some minor changes.

- Related input contributions

No input contributions were noted as relevant to the work of this ad hoc group:

- Remaining VVC spec tickets

Closed since JVET-AN0002 was reported:

- #1609 NoBackwardPredFlag derivation ambiguity (Addressed in JVET-AI1004)

- #1617 Not initialized NumCtusInSlice[0] to 0. (Addressed in JVET-AI1004)

- #1624 Incorrect indexing in computation of motion vector offset. (Addressed in JVET-AI1004)

- #1627 "Decoding process for palette mode" does not say what to do with output samples. (Discussed in JVET-AI1004)

- #1628 Derivation of ModeTypeCondition should say "one or more". (Addressed in JVET-AI1004)

- #1630 Missing equations for applying AmvrShift. (Discussed in JVET-AI1004)

- #1631 Should "Motion vector storing process for geometric partitioning mode" store HpelIfIdx? (addressed in JVET-AI1004)

- #1632 Incorrect indexing used for choosing matrix intra sample prediction. (Addressed in JVET-AI1004)

- #1634 Matrices QStateTransTable, levelScale, AlfFixFiltCoeff, AlfClassToFiltMap are incorrectly transposed. (Addressed in JVET-AI1004)

- #1635 Incorrect inference for tu_y_coded_flag. (Discussed in JVET-AI1004)

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

- #1651 References to various non-existent syntax elements relating to sps_partition_constraints_override_enabled_flag.

- #1652 Incorrect heading style in JVET-S2001-v17.

Carried over (strikethough means addressed in the indicated errata report and can be closed):

- #1629 mtt_split_cu_vertical_flag context uses undefined variable chType. (Discussed in JVET-AI1004)

- #1650 Incorrect indexing for the h location component in ALF process. (Discussed in JVET-AJ1004)

New (since JVET-AN0002 was reported)

- #1654 Typo in derivation process for GPM motion vectors.

- Remaining HEVC spec tickets

Closed since JVET-AN0002 was reported:

- #1427 (8-155) and (8-157) do not seem to be used

- #1491 Duplicate invocation of 9.3.4.3 arithmetic decoding process

- #1498 Typos in the Table 9-43

- #1500 Typo in equation (8-69),(8-70)

- #1504 Small typos in profile_tier_level syntax in tabular form (7.3.3)

- #1505 Misleading bitstream requirement related to EOB NAL unit

- #1507 Duplicate row entries for CU QP delta syntax elements in Table 9-48

- #1520 Some smaller errors in the multiview spec

- #1522 Offset issue in clause 8.5.4.3

- #1644 Correction of wrong reference on VVC Annex C.4 regarding DpbOutputInterval constraint. Filed as VVC ticket but applicable to and corrected in HEVC as well. (Addressed in JVET-AI1004)

Carried over:

- (none)

New (since JVET-AN0002 was reported):

- (none)

- Recommendations

The AHG recommends to:

- Approve JVET-AN1016, JVET-AN1017, JVET-AN1018, JVET-AN1019, and JVET-AN2001 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review bug tickets, and other AHG2-related inputs and act on them if found to be necessary.

JVET-AO0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice chairs)]

- Introduction

The software model versions prior to the start of the meeting were:

- VTM 23.13 (October 2025)

- VTM-22.2-TuC4.0 (Feb. 2025)

- HM-18.0 (Apr. 2023)

- HM-16.21+SCM-8.8 (Mar. 2020)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- JM 19.1 (Apr. 2023)

- JSVM 9.19.15

- JMVC 8.5

- 3DV ATM 15.0 (no version history)

- HDRTools 0.26 (August 2025)

Software for MFC and MFCD is only available as published by ITU-T and ISO/IEC. It is planned to create repositories with the latest versions available in ITU-T H.264.2 (02/2016). All development history is lost.

- Software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow are documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

Although the development process is described in the context of the VTM software, it can be applied to all other software projects hosted on the GitLab server as well.

Between the 31st and 32nd meetings, the GitLab server was affected by a huge number of spam issues, which were generated using newly registered accounts. Administrator user account confirmation was enabled to protect the server from spam postings. Proponents should plan for some time for server admins to confirm new accounts.

Between the 37th and 38th JVET meeting extensive renewal work was performed on the build server infrastructure. To finish these works, changes to the build configurations are required for all software projects using build pipelines. The necessary information will be communicated to the software coordinators when available.

- VTM related activities

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

The software development continued using the GitLab server. VTM version 23.12 was tagged on October 10, 2025, VTM version 23.13 was tagged on October 11, 2025. VTM 23.14 is expected during the 41st JVET meeting. .

VTM 23.12 was tagged on October 10, 2025. Changes include:

- Fix memory leak when RPR is used

- Fix a link error of parcat when JVET_J0090_MEMORY_BANDWITH_MEASURE is on

- Initialize pointer to VPS of a pilot Slice object

- Correctly set input chroma format when reading a y4m file

- Avoid recomputing LMCS parameters when using debugBitstream by copying APS info (same as AFL APSs)

- Fix trace output of coded block flag (CBF)

- JVET-AM0227: Fix CABAC statistics collection

- JVET-AJ0258: Image Format Metadata SEI

- New enum for splits - removes magic numbers from CtxSplit

- Increase minimum required CMake version

- DSC: fix detection of last picture

- JVET-AK0206: Digitally signed content ID

- remove dead code in EncModeCtrl.h

- DSC: Align DSCI syntax with spec

- JVET-AL0222: Digitally signed content intended start and end flags

- JVET-AL0066: Add sample tolerance value to OMI SEI

- JVET-AM0164: move dsci_key_source_uri and increase length of dscv_signature_length_in_octets_minus1

- JVET-AL0058: Change aur_context to fixed length

- JVET-AM0334: Update to GFV SEI chroma key syntax

- JVET-AK0281: AUR SEI message in SPO SEI message

- JVET-AM0117: aur_sei_exclusion_flag

- JVET-AK0055: Add SPO SEI constraint of FPA, RWP, ERP, GCMP SEI messages

- JVET-AM0121: SPO SEI constraints

- JVET-AM0118: Digital signing of selected SEI messages

- Compute MS-SSIM only when required if RPR is enabled

- Update version to 23.12

VTM 23.13 was tagged on October 11, 2025. Changes include:

- remove macros from previous two meeting cycles

VTM 23.14 is expected to be tagged during the 41st JVET meeting. Changes are expected to include:

- JVET-AN0348: On reporting metrics for RPR

- Update CI pipeline settings

- Revert RPR side effects introduced by !2685

- Remove unnecessary load

- Fix and unify full-size allocation condition

- JVET-AN0058: On the NNPF SEI messages

- JVET-AN0060: On the OMI SEI message

- JVET-AN0237: film grain analysis

- JVET-AN0062 item 1: Generative AI usage restrictions

- PRI SEI: clean up code and fix picture output order

  - CTC Performance

In SDR CTC and high performance text conditions VTM 23.13 shows the same coding performance compared to VTM 23.11. No changes in coding time were observed.

For the HDR CTCs, coding performance of VTM 23.13 compared to VTM 23.11 are unchanged. A slight runtime reduction for both encoder and decoder of around 2-3% is observed.

Coding performance of VTM 23.13 compared to HM 18.0 are reported in table below.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -39.49% | -37.66% | -36.75% | -54.96% | -48.52% | -33.76% | -49.45% | -40.94% | 246% | 78% |
| Class H2 |  |  |  |  |  | -31.99% | -57.73% | -63.35% | 212% | 77% |
| Overall | -39.49% | -37.66% | -36.75% | -54.96% | -48.52% | -33.12% | -52.46% | -49.09% | 233% | 77% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.44% | -27.31% | -26.79% | -57.73% | -52.77% | -24.00% | -52.92% | -45.07% | 1473% | 99% |
| Class H2 |  |  |  |  |  | -21.76% | -47.23% | -50.60% | 1191% | 93% |
| Overall | -41.44% | -27.31% | -26.79% | -57.73% | -52.77% | -23.18% | -50.85% | -47.08% | 1363% | 97% |

  - Issues in VTM affecting conformance

The following issues in VTM master branch may affect conformance:

- Missing HLS features (see sections below)

  - Status of implementation of proposals of previous JVET meetings

The following list contains all adoptions of the Q and R meetings that were not marked as merged (or submitted) or specification only change in the software coordinator tracking sheet:

- JVET-Q0112

- JVET-Q0154: Disallow mixing of GDR and IRAP (Disallow mixing of GDR with any non-GDR).

- JVET-Q0164

- JVET-Q0402

- JVET-R0178: Require that when no_aps_constraint_flag is equal to 1, sps_lmcs_enabled_flag and sps_scaling_list_enabled_flag shall be equal to 0

- JVET-R0221

- JVET-R0046: Change the description of the bitstream extraction process per the value of max_tid_il_ref_pics_plus1[ ][ ] (aspect 1.2 per JVET-R0046-v4).

- JVET-R0065: Specify that GDR AUs shall be complete – i.e., all of the layers in the CVS shall have a picture in the AU (as with IRAP AUs).

- JVET-R0191: Update the range value for num_ols_hrd_params_minus1.

- JVET-R0222 aspect 1: Infer vps_max_sublayers_minus1 to be equal to 6 when sps_video_parameter_set_id is equal to 0 (i.e. VPS is not present). The exact editorial expression is at the discretion of the editor.

- JVET-S0196 (JVET-S0144 item 17)

- JVET-S0227 (JVET-S0144 item 22)

- JVET-S0077 (JVET-S0139 item 5)

- JVET-S0174 aspect 2 (JVET-S0139 item 18.b)

- JVET-S0156 aspect 3 (JVET-S0139 item 21)

- JVET-S0139 item 26 (no source listed, text only?)

- JVET-S0188 aspect 1 (JVET-S0139 item 28)

- JVET-S0139 item 40 (item does not exist)

- JVET-S0042 (JVET-S0142 item 1.b)

- JVET-S0174 aspect 1 (JVET S0143 item 19)

- JVET-S0096 aspect 3 (JVET-S0140 item 10)

- JVET-S0096 aspect 4 (JVET-S0140 item 13)

- JVET-S0159 aspect 3 (JVET-S0140 item 16)

- JVET-S0171 (JVET-S0256)

- JVET-S0118 (JVET-S0141 item 7)

- JVET-S0102 (JVET-S0141 item 9.a)

- JVET-S0157 item 2 (JVET-S0141 item 13)

- JVET-S0157 item 4 (JVET-S0141 item 14)

- JVET-S0175 aspect 3 (JVET-S0141 item 16)

- JVET-S0175 aspect 1, 2 (JVET-S0141 item 17)

- JVET-S0175 aspects 4 and 5 (JVET-S0141 item 18)

- JVET-S0175 aspect 6 (JVET-S0141 item 19)

- JVET-S0198/ JVET-S0223 (JVET-S0141 item 24)

- JVET-S0173 aspect 2 (JVET-S0141 item 40.b)

- JVET-S0173 item 1 (JVET-S0141 item 51)

- JVET-S0173 item 3 (JVET-S0141 item 52)

- JVET-S0173 item 5 (JVET-S0141 item 53)

- JVET-S0173 item 6 (JVET-S0141 item 54)

- JVET-S0173 item 4 (JVET-S0141 item 56)

- JVET-S0176 item 4 (JVET-S0141 item 60)

- JVET-S0154 aspect 5 (JVET-S0141 item 68)

- JVET-S0154 aspect 6 (JVET-S0141 item 69)

- JVET-S0154 aspect 8 (JVET-S0141 item 71)

- JVET-S0095 aspect 5 (JVET-S0145 item 5)

- JVET-S0095 aspect 6 (JVET-S0145 item 6)

- JVET-S0100 aspect 1, depends on JVET-R0193 (JVET-S0147 item 2)

- FINB ballot comments

- Make high tier support up to 960.

  - SEI TuC software

Per the decision during the 32nd JVET meeting, an SEI TuC repository was created based on VTM-22.2. The repository is located at:

https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

It should be noted that some of the technologies for which merge requests were submitted to TuC, got moved into the working draft. The merge requests were closed, when appropriate merge requests for VTM were available.

VTM-22.2-TuC-5.0 is expected to be tagged during the 41st JVET meeting.

The following merge requests were submitted and are pending:

- JVET-AM0086: PHOTOSENSITIVE CONTENT ADDITIONAL INFORMATION

- JVET-AL0219

- JVET-AK0142: "AHG9: Display Rectangles SEI"

- JVET-AI0340: Implementation of AI-restrictions usage SEI message

- HM related activities

There was no new HM version tagged during this meeting cycle.

The following MRs were merged:

- Fix build after merging !82

- Remove output picture marked as non reference from the DPB

- JVET-AK2006:Add support for SPTI SEI message

- Update build pipeline from VTM

- Modified CMake for ARM variations

- Avoid reading SEI content when payloadSize = 0

The following MRs are pending:

- JVET-AN0237: Film Grain Analysis

- JVET-AN0292-0293: Multiview HEVC encoding and decoding including 4:4:4 profiles

- Fix bumping process (add conditions on PicLatencyCnt, fixes for EOS before CRA cases)

- Mark all pictures as to not be output when encountering a CRA with noRaslOutputFlag = 1

- Fix condition to reset PicOrderCntMsb (could happen on CRA)

- Fix erroneously skipped pictures when decoding

- JVET-AL0339: Spatial resolution for film grain characteristics SEI message.JVET-AL0059: Packed regions information SEI (pending reviewing)

- JVET-AL0148: implementation of GFV and GFVE SEI messages (pending review)

- Avoid reading SEI content when payloadSize = 0 (one issue remains)

- JVET-AL0062: AI usage restrictions SEI message (pending review)

- JVET-AK2006: Add support for SPTI SEI message (pending review/rebasing)

- JVET-AL0061: Import encoder optimization information SEI Message form VTM (pending review)

- JVET-AL0148: implementation of GFV and GFVE SEI messages (pending review)

- Port the Y4M support [one issue remains]

- Mark the current picture as short-term ref (for SCM) [need SCC expert reviewer]

A new version of HM is expected to be tagged during or after the 41th JVET meeting.

The HM SCC (SCM) branch (HM-16.21+SCM-8.8) has not been updated for the recent HM versions. Updating SCM to, for example, HM-18.0+SCM-8.8 should be considered. It may though be helpful to move SCC related functionality into separate source files. Volunteer work towards merging the branches would be appreciated.

As reported in the previous reports, further information on lambda optimisation in HM would be appreciated, including comparison of allocation of bits within the GOP structures between HM and VTM.

Otherwise the HEVC bug tracker lists:

- 44 tickets for “HM”, (most of which were created before 2019; 1 created in last year),

- 1 ticket for “HM RExt” (created in 2020),

- 9 tickets for “HM SCC” (most of which were created before 2018),

- 1 ticket for “RExt Text” (created in 2015),

- 1 ticket for “SCC Text” (created in 2016),

- 8 tickets for text (6 created before 2021; 2 created in the last year),

- 2 tickets for encoder description (created in 2020 and 2014)

Help to address these tickets would be appreciated.

- 360Lib related activities

There had not been any further developments to 360Lib during this meeting cycle.

- SCM related activities

There had not been any further developments to SCC’s SCM during this meeting cycle.

- SHM related activities

An update to HM 18.0 and some smaller changes were merged. A new tag is planned.

- HTM related activities

There had not been any releases of HTM of MV-HEVC and 3D-HEVC.

One merge request is was merged:

- JVET-AE0295: MV Main 10 profile support (waiting for proponent response)

The next release will also include the following changes:

- JVET-Z0209: Early termination during calculating RDcost of depth

- HDRTools related activities

There had not been any further developments to HDRTools during this meeting cycle.

- JM, JSVM, JMVM related activities

There had not been any further developments to JSVM or JMVM during this meeting cycle.

There was no new JM version tagged during this meeting cycle, although merge requests were submitted for SEI messages in VSEI.

The following MRs are pending [with status indicated]:

- JVET-AK0107: Modality Information SEI (pending review)

- JVET-AK2006: NNPFC and NNPFA SEI message (pending review)

- JVET-AL0062: AI usage restrictions SEI message (pending review)

- JVET-AK2006: SPTI SEI Message (pending review)

- JVET-AL0061: Encoder Optimization Information SEI Message (pending review)

- JVET-AE0101: implement phase indication SEI message (pending review)

- JVET-AL0148: Add support for Generative Face Video (GFV) and Generative Face Video Enhancement (GFVE) SEI messages(pending review)

For the previous release of JM only basic testing was performed to confirm that the code still works, but performance testing is pending. Performance changes are unlikely but may occur due to bug fixes. A possible encoder/decoder mismatch was reported privately, but needs more investigation.

- Bug tracking

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain.

https://hevc.hhi.fraunhofer.de/trac/hevc

New account creation was disabled due to a large amount of spam registration. Extensive maintenance of the Trac bug tracker would be required to keep it secure. It should be considered to move bug tracking to GitLab, although no automatic conversion of existing bugs is available.

Bug tracking for HDRTools is located at:

https://gitlab.com/standards/HDRTools/-/issues

Please file all issues related to the VVC reference software and HDRTools into the appropriate bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

- CTC alignment and merging

There are currently 8 JVET CTC documents:

JVET-AL2010		VTM/HM 4:2:0 test conditions

JVET-Z2011		VTM/HM HDR test conditions

JVET-AA2018	VTM/HM high bit depth test conditions (without spreadsheet)

JVET-T2013		VTM non-4:2:0 test conditions

JVET-AA1100	HM non-4:2:0 test conditions

JVET-U2012		VTM 360 video test conditions

JVET-AC1009	SHVC test conditions

JVET-AC1015	SCM test conditions

JVET-AE1013		3DV test conditions

Further merging of HM RExt CTC into the appropriate VVC CTC was investigated (non 4:2:0 chroma formats), but proper comparable HM configuration files were not yet available by the beginning of this meeting.

The HM HDR test conditions require additional post-processing stages, due to WPSNR, deltaE100 and PSNRL100 values not being generated directly by the HM encoder. Merging of respective functions from VTM is recommended to streamline the process.

- Guidelines for reference software development

Current versions of the software guidelines are:

- JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

- JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

- Related proposals

JVET-AN0292 proposes to add multi-view functionality to HM. Mutli-view HEVC is usually supported by HTM. It is asserted that range extension support in HTM is very limited but required for the new multi-view profiles. It is asserted that it was easier to port multi-view functionality to HM instead if range extension to HTM.

- Recommendations

The AHG recommends to:

- Continue to develop reference software.

- Improve documentation, especially the software manual.

- Encourage people to test VTM and other reference software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in VTM and other reference software.

- Encourage people to submit non-normative changes that either reduce encoder run time without significantly sacrificing compression performance or improve compression performance without significantly increasing encoder run time.

- Design and add configuration files to the VTM software for testing of HLS features.

- Review VTM-related contributions and determine whether features should be added (or removed) from the software.

- Continue to investigate the merging of branches.

- Continue to investigate merging of CTC documents.

- Verify correctness of CTC documents and issue updates as appropriate

- Keep common test conditions aligned for the different standards.

It was noted that the support of new SEI messages is still lagging behind for HM (some merge requests pending) compared to VTM. For JM, no merge requests have been integrated yet (all pending). See also a note under JVET-AO0016 on software for additional processing of input and output of SEI messages.

It was reported that the newest version of H.264.2 is from 2016 (several code bases, JM, JSVM, JMVM, etc.). It would be useful to produce an updated version after finalization of the integration of the new SEI messages.

JVET-AO0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs)]

- Activities

- Preparations for the Joint Call for Evidence on video compression with capability beyond VVC

AHG4 supported the activities of AHG17 and prepared for the on-site testing activity.

- Test sequences

The set of JVET test sequences is hosted at https://vqa.lfb.rwth-aachen.de. A mirror of this site is available at https://datacloud.hhi.fraunhofer.de with the same login credentials. The directory structure of the previous ftp server has been maintained. The test sequences used for CfP/CTC are available in directory “/ctc”.

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e., members of ISO/IEC MPEG and ITU-T VCEG). Members of JVET may contact the JVET chair for login information. The password for the site is updated after each meeting with the Zoom password for that meeting. The tentative schedule for the changes is as follows:

- After the April meeting, password changes on 1st of June

- After the July meeting, password changes on 1st of September

- After the October meeting, password changes on 1st of December.

- After the January meeting, password changes on 1st of March.

- Related contributions

(listing document directly related to AHG4 as well as documents related to testing activities and test material)

| JVET number | Title | Source |
| --- | --- | --- |
| JVET-AO0004 | JVET AHG report: Test material and visual assessment (AHG4) | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) |
| JVET-AO0054 | [AHG17] Proposal of new sequences for the HD-SDR-RA category of the CfP | M. Abdoli, A. Tissier, R. G. Youvalari, P. Nikitin, F. Plowman, G. Shuo (Xiaomi) |
| JVET-AO0059 | Liverpool Test Sequences | J. Calic, A. Foster, D. Collado (BBC) |
| JVET-AO0061 | AHG4/AHG17: AIGC test sequences | J. Liu, X. Chen, Y. Gao, Z. Chen (Wuhan Univ.), Y. Pan, T. Yao, T. Mei (HiDream.ai) |
| JVET-AO0215 | AHG4: proposed updates for VVC multi-layer verification test plan | O. Chubach (MediaTek), P. de Lagrange (InterDigital), M. Wien (RWTH) |
| JVET-AO0248 | AHG4/AHG17: Response to the call for new HDR materials for future video coding development | J. Wang, J. Zhang, L. Yu |

- Recommendations

The AHG recommends:

- To take into account the proposed test sequences in the preparation of the CfP on video compression with capability beyond VVC.

- To continue to discuss and to update the non-finalized categories of the verification test plan for multilayer VVC based on JVET-AO0215.

- To collect volunteers to conduct further verification tests and subjective quality tests.

- To collect volunteers to actively contribute to the verification test development.

- To review the set of available test sequences for the verification tests as well as subjective quality tests and potentially collect more test sequences with a variety of content.

- To continue to collect new test sequences available for JVET with licensing statement.

JVET-AO0005 JVET AHG report: Conformance testing (AHG5) [I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice chairs)]

- Activities

The AHG communication is conducted through the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG5] in message headers. However, no correspondence marked as AHG5 was sent between the 40th and the 41st meetings.

- Timeline

The progress on the Conformance testing specification is proceeding per the timeline below:

- VVCv1 conformance:

  - ISO/IEC FDIS 23090-15 issued from 2021-10 meeting, FDIS registered for formal approval 2022-07-11, FDIS ballot closed 2022-11-04, standard published 2022-11-24

  - H.266.1 V1 Consent 2022-01-28, Last Call began 2022-04-01, Approved 2022-04-29, pre-published 2022-05-17, standard published 2022-07-12.

- VVCv2 conformance:

  - ISO/IEC 23090-15/Amd.1 CDAM: 2021-10

  - ISO/IEC 23090-15/Amd.1 DAM: 2022-01

  - DAM ballot closed 2022-11-15

  - ISO/IEC FDIS 23090-15:202x 2nd edition text output of 2023-01, preparation delayed to 2023-09, FDIS ballot opened 2024-04-08, FDIS ballot closed 2024-06-03, published 2024-07-04

  - H.266.1 V2 Consent 2023-07, Last Call began 2023-08-16, Approved 2023-09-13, pre-published 2023-10-06, published 2023-10-19

- VVCv3 conformance:

  - CD: 2025-04-15

  - DIS: 2025-07-25

  - FDIS: 2026-02-20

  - IS: 2026-06-30

  - H.266.1 V3 Consent 2025-10, Last Call began 2025-12

- Status on bitstream submission

The status at the time of preparation of this report is as follows:

- conformance bitstreams for VVC:

  - 104 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 283 total bitstreams have been provided, checked, and made available

  - No changes between the 40th and 41st meeting.

- conformance bitstreams for VVC operation range extensions:

  - 57 bitstream categories have been identified

  - 128 bitstreams of 57 identified categories have been provided, cross-checked and made available

  - No changes between the 40th and 41st meeting.

- additional conformance bitstreams for VVC Multilayer:

  - 3 bitstream categories have been identified

  - At least one bitstream has been submitted in each identified category

  - 7 total bitstreams have been provided, cross-checked, and made available

  - No changes between the 40th and 41st meeting.

- conformance bitstreams for new HEVC Multiview profiles

  - 2 HEVC Multiview Extended 10 bitstreams have been provided, cross-checked, and made available, no changes between the 40th and 41st meeting

  - 2 HEVC Multiview Extended bitstreams have been provided, cross-checked, and made available, no changes between the 40th and 41st meeting

  - 2 HEVC Multiview Main 10 bitstreams have been provided, cross-checked and made available, no changes between the 40th and 41st meeting.

- Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.

VVC activities:

The streams are available at ​ https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

VVC operation range extensions activities:

The streams are available at ​ https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

VVC Multilayer activities:

The streams are available at ​ https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC3rdEd/DIS/. No change in bitstreams and/or packages.

HEVC Multiview supporting extended bit depth activities:

The 6 packages are available at https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/HEVCMultiview/  No change in bitstreams and/or packages.

At the 40th JVET meeting it was reported that code was available to extend the HM encoder and decoder to support all the HEVC multiview profiles: support for the Multiview Main, Multiview Main 10, Multiview Extended, and Multiview Extended 10 profiles was ported from HTM to HM, and HM was extended to also support the Multiview Monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles in JVET-AH1006 and in WG5 N0281). These changes also enabled HM to also support multiview encoding with other chroma formats, such as 4:4:4. These HM code changes have been submitted in HM MR https://vcgit.hhi.fraunhofer.de/jvet/HM/-/merge_requests/98 and it is under review because of the amount of changes (23k+ lines)

JVET-AO0227 has been submitted to this meeting to provide updates on Multiview Reference Software.

No update on the generation of the Multiview Monochrome profiles conformance streams.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

- Contributions

JVET-AO0227 AHG5: Updates on Multiview Reference Software [E. Asbun, S. Choi, S. Paluri, A. M. Tourapis (Apple)].

- Ftp site information

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

- VVC:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

- VVC operation range extensions:

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVCv2

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVCv2

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, the following configuration is suggested:

In the Filezilla Edit  Settings  Connection menu, it was previously necessary to set the minimum TLS level to 1.0. Since around January 2024, this configuration issue went away, and ITU disabled the use of TLS 1.0 and 1.1 on its servers.

- Recommendations

The AHG recommends the following:

- Maintain and update the conformance bitstream database and contribute to report problems in JVET document 1004.

- Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and continue the generation, cross-checking, and documentation of the conformance streams for the HEVC Multiview profiles supporting extended bit depth, the HEVC Multiview Main 10 profile, and the HEVC Multiview monochrome profiles (Multiview Monochrome, Multiview Monochrome 10, Multiview Monochrome 12, and Multiview Monochrome 16 profiles, collectively referred to as the multiview format range extensions profiles).

- Study DIS of VVC conformance 3rd edition and suggest improvements to JVET-AL2028, as appropriate.

JVET-AO0006 JVET AHG report: ECM software development (AHG6) [V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs)]

- Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

VTM-11.0ecm anchor https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC is used for ECM performance evaluation.

The following changes were integrated into ECM-19.0:

JVET-AN0168: Regression CCP fusion (Test 2.8a) [MR 962]

JVET-AN0101: Remove single model for ALF-CCCM (Test-5.3a) [MR 964]

JVET-AN0095: Quantization center shift (Test-4.2c*) [MR 965]

JVET-AN0121: split_cu_flag context change (Test 1.1g) [MR 968]

JVET-AN0098: Regularization for ALF-CCCM (Test 5.1a) [MR 967]

JVET-AN0093: Joint reordering GPM with affine and intra prediction (Test 3.4a) [MR 969]

JVET-AN0236: Generated merge candidates based on template cost (Test 3.1) [MR 966]

JVET-AN0203: Move RGPM to group 2 and add TM off implementation [MR 970]

JVET-AN0090: Adaptive subsampling filter selection for CCLM/intra-CCCM/ALF-CCCM (Test 2.6a) [MR 971]

JVET-AN0214: CABAC contexts retraining using ECM-18 data [MR 973]

JVET-AN0158: Longer tap interpolation filtering (Test 2.4) [MR 974]

Fixes:

Fix number of non zeroed-out coefficients to be encoded by RDOQ in LFNST/NSPT case [MR 960]

Collection of bugfixes [MR 961]

Bugfixes related to IBC and similar technologies [MR 963]

Fix JVET-AM0138 for 12 bit [MR 959]

Bugfix TIMD Intra merge [MR 958]

Fix decoder mismatch/crash in JVET_AM0138_ENHANCED_TMP_MERGE_LIST_TIMD_BV [MR 957]

Fixed CABAC bits statistics in ECM decoder [MR 956]

The following changes were integrated into VTM-11.0ecm anchor:

Use enum for split type [MR 955]

ECM-19.0 and VTM-11.0ecm19.0 were tagged on November 11, 2025.

  - CTC Performance

In this section, ECM test results following ECM CTC configuration descried in JVET-AI2017 are summarized.

ECM-19.0 performance over ECM-18.0 anchor is summarized in the tables below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.23% | 0.23% | 100.2% | 104.1% | 100.4% | 100.3% |
| Class A2 | -0.09% | -0.23% | -0.20% | 98.5% | 103.2% | 100.6% | 100.4% |
| Class B | -0.06% | 0.19% | 0.37% | 95.8% | 96.5% | 98.5% | 100.2% |
| Class C | -0.13% | 0.04% | 0.02% | 98.5% | 102.9% | 100.6% | 100.1% |
| Class E | -0.13% | 0.04% | 0.17% | 100.2% | 103.1% | 101.5% | 100.0% |
| Overall | -0.09% | 0.07% | 0.14% | 98.3% | 101.4% | 100.1% | 100.2% |
| Class D | -0.13% | 0.19% | 0.16% | 99.0% | 101.2% | 100.7% | 100.0% |
| Class F | -0.29% | -1.20% | -1.49% | 110.4% | 117.8% | 100.1% | 100.1% |
| Class TGM | -0.01% | 0.07% | 0.06% | 114.8% | 104.4% | 100.4% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.26% | -0.46% | -0.80% | 102.5% | 109.9% | 100.2% | 100.0% |
| Class A2 | -0.24% | -0.90% | -0.68% | 107.9% | 112.3% | 100.3% | 100.2% |
| Class B | -0.19% | -0.57% | -0.87% | 101.4% | 102.1% | 100.2% | 100.3% |
| Class C | -0.27% | -0.84% | -0.68% | 106.7% | 110.3% | 100.5% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.24% | -0.68% | -0.77% | 104.3% | 107.8% | 100.3% | 100.2% |
| Class D | -0.24% | -0.33% | -0.62% | 105.6% | 108.0% | 100.5% | 100.0% |
| Class F | -0.11% | -0.91% | -1.49% | 112.7% | 117.7% | 99.9% | 99.9% |
| Class TGM | 0.07% | -0.15% | -0.25% | 119.6% | 124.5% | 100.1% | 99.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | -0.85% | -0.60% | 101.8% | 104.8% | 100.3% | 99.6% |
| Class C | -0.29% | -1.27% | -1.06% | 103.5% | 104.5% | 100.5% | 100.2% |
| Class E | -0.11% | 0.26% | 0.24% | 104.1% | 99.2% | 100.4% | 99.6% |
| Overall | -0.17% | -0.71% | -0.54% | 103.0% | 103.3% | 100.4% | 99.8% |
| Class D | -0.29% | -1.19% | -1.82% | 106.1% | 108.0% | 100.5% | 100.1% |
| Class F | -0.14% | -1.02% | -1.69% | 110.6% | 117.7% | 100.4% | 99.9% |
| Class TGM | 0.03% | -0.70% | -0.23% | 112.5% | 107.6% | 96.5% | 100.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-18.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -1.16% | -0.54% | 105.5% | 112.4% | 100.3% | 100.1% |
| Class C | -0.10% | -0.63% | -0.77% | 106.6% | 104.2% | 100.5% | 100.1% |
| Class E | -0.02% | 0.09% | 0.40% | 105.3% | 104.3% | 100.4% | 100.0% |
| Overall | -0.05% | -0.67% | -0.39% | 105.8% | 107.6% | 100.4% | 100.1% |
| Class D | -0.15% | -2.66% | -2.10% | 106.9% | 112.3% | 100.8% | 100.1% |
| Class F | 0.13% | -1.71% | -1.38% | 109.4% | 117.8% | 99.9% | 99.8% |
| Class TGM | 0.05% | -0.50% | -0.15% | 113.8% | 113.4% | 100.7% | 100.0% |

The below tables show ECM-19.0 performance comparing to VTM-11.0ecm19.0 anchor.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -14.87% | -16.35% | -27.31% | 1183.3% | 578.9% | 237.6% | 326.7% |
| Class A2 | -21.54% | -25.02% | -29.24% | 1117.5% | 618.4% | 238.9% | 326.5% |
| Class B | -15.11% | -23.02% | -20.90% | 1050.8% | 646.9% | 178.3% | 355.8% |
| Class C | -15.30% | -12.03% | -13.20% | 1030.4% | 649.6% | 122.4% | 394.7% |
| Class E | -19.69% | -23.36% | -21.40% | 1005.3% | 659.8% | 141.4% | 381.4% |
| Overall | -16.95% | -19.86% | -21.73% | 1070.2% | 633.0% | 173.8% | 358.0% |
| Class D | -13.04% | -8.61% | -9.49% | 993.9% | 692.2% | 110.1% | 414.3% |
| Class F | -31.03% | -35.19% | -35.37% | 616.5% | 656.3% | 125.9% | 374.4% |
| Class TGM | -43.73% | -49.29% | -48.58% | 505.2% | 645.6% | 150.3% | 351.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -27.70% | -25.43% | -37.71% | 1226.1% | 1275.1% | 153.4% | 247.6% |
| Class A2 | -30.98% | -35.54% | -40.94% | 1181.5% | 1462.9% | 153.2% | 247.5% |
| Class B | -25.57% | -34.66% | -31.70% | 1037.5% | 1254.2% | 144.8% | 268.3% |
| Class C | -27.32% | -24.94% | -25.55% | 1155.1% | 1555.8% | 117.4% | 321.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -27.55% | -30.40% | -33.11% | 1133.0% | 1374.4% | 140.1% | 272.5% |
| Class D | -28.00% | -24.79% | -26.22% | 1084.2% | 1651.9% | 105.4% | 360.4% |
| Class F | -33.69% | -37.31% | -38.21% | 807.9% | 865.5% | 121.6% | 296.0% |
| Class TGM | -43.03% | -48.96% | -48.99% | 721.6% | 745.4% | 125.8% | 280.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -22.69% | -41.03% | -35.94% | 1048.5% | 1184.3% | 167.3% | 266.0% |
| Class C | -25.22% | -29.20% | -31.15% | 1016.4% | 1236.8% | 125.8% | 316.7% |
| Class E | -22.82% | -29.50% | -27.98% | 1033.7% | 732.0% | 140.2% | 295.6% |
| Overall | -23.57% | -34.21% | -32.35% | 1034.0% | 1065.4% | 145.5% | 289.4% |
| Class D | -26.68% | -30.77% | -32.61% | 1040.5% | 1416.4% | 111.1% | 352.8% |
| Class F | -31.47% | -41.72% | -42.29% | 818.4% | 855.4% | 130.1% | 294.8% |
| Class TGM | -41.32% | -51.82% | -51.52% | 707.0% | 643.7% | 120.8% | 282.6% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over VTM-11.0ecm19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -20.61% | -49.22% | -44.58% | 922.2% | 1169.2% | 172.3% | 293.6% |
| Class C | -23.22% | -37.72% | -39.26% | 863.4% | 1172.2% | 127.1% | 344.9% |
| Class E | -20.96% | -38.14% | -36.76% | 935.2% | 737.6% | 144.1% | 318.4% |
| Overall | -21.56% | -42.61% | -40.85% | 905.3% | 1042.9% | 148.9% | 316.1% |
| Class D | -25.94% | -41.54% | -42.37% | 863.5% | 1382.6% | 111.6% | 374.7% |
| Class F | -29.50% | -47.20% | -47.58% | 765.2% | 793.5% | 130.5% | 318.5% |
| Class TGM | -39.51% | -54.10% | -53.63% | 739.6% | 668.2% | 126.8% | 294.8% |

- ECM memory consumption

ECM encoder memory consumption (VmPeak, GiB) is provided in ECM encoder log files and is summarized in the table below as maximum class-wise consumption rounded up to GiB.

|  | AI | RA | LB |
| --- | --- | --- | --- |
| class A (A1 & A2) | 9 | 16 |  |
| class B | 4 | 6 | 5 |
| class C | 2 | 3 | 3 |
| class D | 2 | 2 | 2 |
| class E | 3 |  | 3 |
| class F | 4 | 6 | 5 |
| class TGM | 4 | 8 | 7 |

It is encouraged to care about memory allocation when developing and integrating tools into ECM. In particular, it is strongly recommended to re-use already existed memory wherever possible, rather than systematically allocating new memory, and allocate only the required amount of memory if new memory is needed.

- Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.

- Encourage people to continue working on ECM memory consumption reduction.

- Encourage people to continue working on speeding up ECM encoder to reduce the simulation time.

JVET-AO0007 JVET AHG report: Tool assessment (AHG7) [X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice chairs)]

- Conference call on Complexity Analysis

The call was on November 21, 2025 Friday UTC 6:00am-6:57am. The report was uploaded as JVET-AO0042.

An input document on VVC complexity analysis was uploaded as JVET-AO0067.

- Group off tests

- Test settings and crosschecking

The following five groups of tools were defined.

- Group 1: Inter template matching tools

- Group 2: Coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process

- Group 3: Intra and IBC template matching (with search) related tools

- Group 4: Tools that require more processing on the neighboring reconstructed samples than VVC

- Group 5: Tools with large memory access

The testers and crosscheckers are planed in the table below.

| Tests | Testers / Crosscheckers | Testers / Crosscheckers |
| --- | --- | --- |
| Group 1 off | Johan Pardo (johan.esprit.pardo1@huawei.com) | Charles Salmon-Legagneur (charles.salmon-legagneur@interdigital.com) |
| Group 2 off | Jonathan Gan (v-jonathan.gan@oppo.com) | Xinwei Li (sid.lxw@alibaba-inc.com) |
| Group 3 off | Ryo Ishimoto (ishimoto.ryo@mail.sharp) | Zhipin Deng (zhipin.deng@bytedance.com) |
| Group 4 off | Hong-Jheng Jhu (jhuhong-jheng@kwai.com) | Xiang Li (xlxiangli@google.com) |
| Group 5 off | Lien-Fei Chen (lienfei.chen@global.tencent.com) | Jani Lainema (jani.lainema@nokia.com) |
| Group 1-5 off | Xiang Li (xlxiangli@google.com) | Hongtao Wang (hongtaow@qti.qualcomm.com) |

ECM-19.0 was used in the AHG7 tool off tests. The cfg files used are included in the ECM software package.

- Group 1 off

Group 1 includes inter template matching tools. The offgroup1.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  |  | Over VTM-11ecm19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | mHWMR | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101.0% | 100.8% | 100.3% | 103.9% | -14.87% | -16.35% | -27.31% | 1052.2% | 578.1% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101.4% | 101.4% | 100.3% | 103.8% | -21.54% | -25.02% | -29.24% | 989.9% | 637.1% |
| Class B | 0.00% | -0.01% | -0.01% | 101.0% | 100.6% | 100.0% | 103.2% | -15.12% | -23.03% | -20.90% | 996.9% | 657.2% |
| Class C | -0.02% | -0.01% | -0.01% | 101.6% | 102.9% | 101.3% | 103.1% | -15.31% | -12.04% | -13.21% | 971.9% | 634.5% |
| Class E | -0.01% | -0.01% | -0.01% | 101.2% | 101.5% | 101.0% | 103.3% | -19.70% | -23.37% | -21.41% | 941.2% | 708.8% |
| Overall | -0.01% | -0.01% | -0.01% | 101.2% | 101.4% | 100.5% | 103.4% | -16.95% | -19.86% | -21.74% | 989.5% | 643.1% |
| Class D | -0.03% | -0.02% | -0.03% | 100.6% | 102.5% | 99.6% | 103.2% | -13.07% | -8.63% | -9.51% | 930.7% | 668.3% |
| Class F | -0.01% | -0.01% | -0.01% | 101.1% | 102.2% | 105.1% | 103.2% | -31.04% | -35.20% | -35.38% | 638.2% | 683.0% |
| Class TGM | -0.01% | -0.01% | -0.01% | 101.5% | 99.4% | 103.5% | 103.2% | -43.74% | -49.29% | -48.58% | 520.5% | 701.4% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  |  | Over VTM-11ecm19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | mHWMR | Y | U | V | EncT | DecT |
| Class A1 | 4.89% | 4.75% | 5.26% | 84.4% | 70.3% | 99.9% | 99.4% | -24.14% | -22.12% | -34.25% | 991.3% | 1015.1% |
| Class A2 | 5.83% | 5.81% | 5.91% | 82.4% | 64.2% | 100% | 99% | -26.95% | -31.88% | -37.43% | 932.2% | 1081.7% |
| Class B | 4.49% | 4.74% | 4.65% | 79.1% | 65.6% | 99.9% | 99.5% | -22.19% | -31.52% | -28.47% | 793.7% | 992.2% |
| Class C | 5.04% | 5.03% | 5.36% | 73.7% | 60.4% | 100.0% | 99.9% | -23.62% | -21.15% | -21.53% | 783.3% | 1022.8% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.98% | 5.03% | 5.21% | 79.3% | 64.8% | 100% | 100% | -23.91% | -26.95% | -29.57% | 853.9% | 1022.3% |
| Class D | 4.19% | 4.22% | 4.10% | 72.8% | 57.7% | 100.0% | 100.0% | -24.92% | -21.56% | -23.11% | 715.8% | 1059.9% |
| Class F | 3.72% | 3.60% | 3.64% | 86.7% | 72.6% | 100.5% | 99.6% | -31.12% | -34.95% | -35.86% | 722.2% | 750.8% |
| Class TGM | 4.64% | 4.49% | 4.49% | 87.4% | 78.8% | 100.0% | 99.7% | -40.40% | -46.64% | -46.66% | 627.4% | 688.5% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  |  | Over VTM-11ecm19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | mHWMR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 5.63% | 5.85% | 5.97% | 70.8% | 57.6% | 100% | 102% | -18.31% | -37.59% | -32.06% | 717.2% | 801.9% |
| Class C | 5.72% | 5.71% | 6.20% | 69.4% | 52.1% | 100% | 103% | -20.94% | -25.17% | -26.91% | 671.6% | 756.8% |
| Class E | 6.08% | 6.91% | 6.67% | 76.8% | 65.7% | 102% | 102% | -18.12% | -24.72% | -23.21% | 724.6% | 674.6% |
| Overall (Ref) | 5.77% | 6.07% | 6.22% | 71.8% | 57.6% | 100% | 102% | -19.14% | -30.23% | -28.13% | 703.5% | 753.3% |
| Class D | 5.19% | 4.01% | 5.34% | 68.3% | 51.2% | 100% | 103% | -22.86% | -27.97% | -28.80% | 658.8% | 824.1% |
| Class F | 5.88% | 4.16% | 4.98% | 83.3% | 66.4% | 100% | 102% | -27.36% | -39.20% | -39.25% | 696.5% | 688.0% |
| Class TGM | 6.63% | 6.61% | 6.42% | 86.8% | 66.6% | 100% | 101% | -37.37% | -48.45% | -48.23% | 636.2% | 579.6% |

- Group 2 off

Group 2 includes coding tools that interleave the (merge/skip/AMVP/subblock/IBC/etc) list derivation with the intra prediction/reconstruction process. The offgroup2.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.49% | 0.48% | 94.7% | 96.7% | 100% | -14.74% | -15.97% | -26.94% | 1110.8% | 550.6% |
| Class A2 | 0.22% | 0.43% | 0.43% | 93.8% | 97.0% | 100% | -21.36% | -24.71% | -28.95% | 1047.5% | 605.3% |
| Class B | 0.18% | 0.47% | 0.58% | 93.2% | 97.0% | 97% | -14.96% | -22.66% | -20.44% | 998.1% | 578.9% |
| Class C | 0.14% | 0.27% | 0.41% | 92.5% | 95.0% | 99% | -15.17% | -11.80% | -12.86% | 944.0% | 479.1% |
| Class E | 0.28% | 0.54% | 0.61% | 93.8% | 98.3% | 99% | -19.46% | -22.95% | -20.91% | 955.3% | 584.9% |
| Overall | 0.19% | 0.43% | 0.51% | 93.5% | 96.7% | 99% | -16.79% | -19.52% | -21.33% | 1004.3% | 555.5% |
| Class D | 0.13% | 0.08% | 0.25% | 92.7% | 95.3% | 100% | -12.93% | -8.54% | -9.27% | 932.5% | 530.1% |
| Class F | 0.48% | 0.66% | 0.66% | 93.1% | 99.1% | 97% | -30.72% | -34.77% | -34.94% | 589.7% | 555.1% |
| Class TGM | 0.35% | 0.37% | 0.42% | 94.7% | 99.1% | 100% | -43.52% | -49.10% | -48.36% | 519.3% | 665.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 2.53% | 1.73% | 2.38% | 84.0% | 96.3% | 100% | -25.87% | -24.12% | -36.24% | 996.6% | 1374.8% |
| Class A2 | 1.48% | 1.49% | 1.86% | 83.4% | 97.9% | 100% | -29.94% | -34.59% | -39.81% | 949.6% | 1639.4% |
| Class B | 1.59% | 1.44% | 1.95% | 84.6% | 96.4% | 99% | -24.37% | -33.69% | -30.28% | 838.2% | 1485.2% |
| Class C | 1.24% | 1.58% | 1.54% | 83.8% | 96.6% | 99% | -26.43% | -23.76% | -24.40% | 889.7% | 1677.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.66% | 1.55% | 1.91% | 84.1% | 96.7% | 99% | -26.34% | -29.31% | -31.81% | 903.9% | 1540.8% |
| Class D | 0.64% | 0.14% | 0.80% | 83.5% | 96.5% | 98% | -27.53% | -24.63% | -25.55% | 814.3% | 1723.3% |
| Class F | 1.96% | 1.94% | 2.05% | 87.9% | 94.5% | 98% | -32.37% | -36.06% | -36.91% | 710.3% | 1011.4% |
| Class TGM | 3.41% | 5.08% | 5.31% | 90.5% | 99.7% | 100% | -41.21% | -46.68% | -46.58% | 642.9% | 920.3% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 2.03% | 3.49% | 3.45% | 79.5% | 95.5% | 99% | -21.12% | -39.04% | -33.75% | 830.9% | 967.1% |
| Class C | 1.93% | 2.96% | 3.20% | 80.6% | 93.9% | 99% | -23.84% | -27.19% | -28.98% | 847.6% | 1031.1% |
| Class E | 1.43% | 1.81% | 2.54% | 83.4% | 95.4% | 99% | -21.72% | -28.22% | -26.24% | 830.1% | 664.4% |
| Overall (Ref) | 1.84% | 2.89% | 3.14% | 80.8% | 95.0% | 99% | -22.18% | -32.39% | -30.28% | 836.2% | 899.5% |
| Class D | 0.90% | 1.58% | 3.09% | 78.3% | 93.8% | 98% | -25.99% | -29.71% | -30.45% | 816.9% | 1124.8% |
| Class F | 2.69% | 1.60% | 2.87% | 86.7% | 93.8% | 99% | -29.65% | -40.80% | -40.60% | 746.5% | 657.3% |
| Class TGM | 2.50% | 6.59% | 7.06% | 89.6% | 97.9% | 99% | -40.00% | -49.10% | -48.75% | 683.3% | 597.6% |

- Group 3 off

Group 3 includes intra and IBC template matching (with search) related tools. The offgroup3.cfg was used in addition to ECM CTC settings

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.19% | 4.02% | 6.36% | 71.6% | 88.9% | 100% | -13.85% | -13.26% | -23.10% | 819.1% | 494.4% |
| Class A2 | 2.68% | 6.97% | 6.59% | 71.6% | 79.2% | 100% | -19.37% | -20.24% | -25.08% | 773.3% | 479.1% |
| Class B | 2.70% | 5.43% | 5.56% | 69.0% | 78.2% | 100% | -12.86% | -19.27% | -16.74% | 736.7% | 511.0% |
| Class C | 2.55% | 3.08% | 3.43% | 69.6% | 77.3% | 100% | -13.19% | -9.45% | -10.32% | 717.6% | 502.6% |
| Class E | 3.81% | 4.84% | 4.41% | 69.9% | 77.8% | 101% | -16.62% | -19.80% | -17.96% | 705.2% | 534.8% |
| Overall | 2.60% | 4.83% | 5.20% | 70.1% | 79.8% | 100% | -14.81% | -16.34% | -17.97% | 746.1% | 504.7% |
| Class D | 1.57% | 2.04% | 2.43% | 69.8% | 78.9% | 100% | -11.68% | -6.79% | -7.31% | 692.7% | 598.7% |
| Class F | 7.38% | 8.65% | 8.38% | 72.7% | 67.1% | 101% | -26.19% | -30.06% | -30.35% | 455.0% | 435.9% |
| Class TGM | 12.55% | 14.48% | 14.93% | 77.4% | 54.3% | 99% | -36.81% | -42.13% | -41.12% | 398.6% | 355.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.04% | 2.41% | 5.77% | 93.1% | 98.0% | 100% | -26.96% | -23.70% | -34.88% | 1043.4% | 1137.8% |
| Class A2 | 1.65% | 3.86% | 3.67% | 93.8% | 98.6% | 100% | -29.84% | -33.18% | -38.81% | 1005.6% | 1363.8% |
| Class B | 1.38% | 4.65% | 4.92% | 91.6% | 98.0% | 100% | -24.55% | -31.65% | -28.38% | 904.8% | 1189.4% |
| Class C | 1.08% | 1.91% | 1.84% | 91.7% | 98.5% | 100% | -26.53% | -23.49% | -24.13% | 1050.8% | 1548.5% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.28% | 3.31% | 4.02% | 92.4% | 98.2% | 100% | -26.62% | -28.19% | -30.63% | 989.5% | 1299.9% |
| Class D | 0.85% | 1.46% | 1.72% | 91.9% | 96.5% | 100% | -27.39% | -23.69% | -24.89% | 935.9% | 1558.0% |
| Class F | 4.76% | 5.57% | 5.32% | 90.5% | 90.9% | 100% | -30.77% | -34.25% | -35.23% | 772.6% | 840.7% |
| Class TGM | 7.58% | 9.04% | 9.66% | 93.1% | 95.1% | 100% | -38.73% | -44.40% | -44.11% | 692.2% | 734.7% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.48% | 2.10% | 1.87% | 93.5% | 99.8% | 100% | -22.33% | -39.93% | -34.88% | 911.7% | 1154.9% |
| Class C | 0.55% | 1.18% | 1.09% | 93.7% | 99.0% | 100% | -24.82% | -28.35% | -30.39% | 906.3% | 1157.9% |
| Class E | 0.97% | 2.49% | 2.38% | 95.6% | 100.7% | 100% | -22.04% | -27.78% | -26.28% | 905.0% | 741.5% |
| Overall (Ref) | 0.62% | 1.89% | 1.73% | 94.1% | 99.8% | 100% | -23.09% | -33.03% | -31.23% | 908.3% | 1034.7% |
| Class D | 0.43% | 0.70% | 0.82% | 96.2% | 102.4% | 100% | -26.37% | -30.26% | -32.07% | 990.0% | 1443.8% |
| Class F | 3.91% | 4.48% | 4.85% | 91.6% | 93.9% | 100% | -28.93% | -39.36% | -39.63% | 783.5% | 829.7% |
| Class TGM | 5.08% | 6.56% | 6.45% | 93.3% | 97.0% | 100% | -38.38% | -48.85% | -48.67% | 665.8% | 652.5% |

.

- Group 4 off

Group 4 includes tools that require more processing on the neighboring reconstructed samples than VVC.

The offgroup4.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.51% | 5.22% | 5.73% | 84.6% | 89.2% | 100% | -13.59% | -12.21% | -23.24% | 1021.5% | 554.8% |
| Class A2 | 1.71% | 5.21% | 5.67% | 82.1% | 90.9% | 100% | -20.24% | -21.43% | -25.65% | 921.0% | 601.2% |
| Class B | 1.04% | 5.09% | 4.92% | 85.4% | 91.9% | 99% | -14.23% | -19.55% | -17.23% | 960.4% | 638.9% |
| Class C | 1.03% | 3.22% | 3.37% | 84.6% | 91.0% | 100% | -14.44% | -9.34% | -10.40% | 912.1% | 575.6% |
| Class E | 1.60% | 6.38% | 5.26% | 87.6% | 94.9% | 102% | -18.41% | -18.84% | -17.40% | 926.9% | 663.0% |
| Overall | 1.32% | 4.93% | 4.89% | 84.9% | 91.5% | 100% | -15.87% | -16.25% | -18.15% | 946.9% | 607.4% |
| Class D | 0.95% | 2.33% | 2.75% | 85.8% | 90.3% | 100% | -12.22% | -6.51% | -6.99% | 895.0% | 617.5% |
| Class F | 1.35% | 4.68% | 4.91% | 91.7% | 93.5% | 101% | -30.12% | -32.27% | -32.37% | 603.1% | 628.7% |
| Class TGM | 2.25% | 4.19% | 4.34% | 92.5% | 95.7% | 99% | -42.53% | -47.26% | -46.47% | 491.7% | 656.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 1.90% | 2.81% | 5.02% | 93.2% | 98.4% | 100% | -26.35% | -23.39% | -34.94% | 1163.0% | 1289.3% |
| Class A2 | 1.31% | 3.15% | 3.40% | 95.2% | 99.7% | 100% | -30.06% | -33.59% | -39.00% | 1139.6% | 1553.4% |
| Class B | 1.02% | 3.26% | 3.38% | 94.4% | 99.0% | 100% | -24.81% | -32.64% | -29.54% | 989.5% | 1360.9% |
| Class C | 1.16% | 1.98% | 1.94% | 95.2% | 98.4% | 100% | -26.48% | -23.45% | -24.11% | 1081.8% | 1496.3% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 1.29% | 2.81% | 3.33% | 94.5% | 98.9% | 100% | -26.61% | -28.53% | -31.06% | 1076.6% | 1417.8% |
| Class D | 1.00% | 1.51% | 1.81% | 95.3% | 97.5% | 100% | -27.26% | -23.68% | -24.83% | 1039.6% | 1571.6% |
| Class F | 1.31% | 2.74% | 3.22% | 96.4% | 97.6% | 101% | -32.89% | -35.75% | -36.33% | 830.8% | 897.5% |
| Class TGM | 1.00% | 1.71% | 1.77% | 96.3% | 98.2% | 99% | -42.50% | -48.14% | -48.12% | 703.1% | 749.8% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 1.12% | 1.16% | 1.16% | 96.6% | 98.1% | 100% | -21.83% | -40.49% | -35.28% | 1032.7% | 1241.1% |
| Class C | 1.48% | 1.49% | 0.77% | 97.8% | 98.9% | 99% | -24.13% | -28.15% | -30.62% | 1010.0% | 1277.7% |
| Class E | 1.22% | 3.20% | 1.54% | 98.8% | 99.0% | 100% | -21.87% | -27.34% | -26.81% | 957.3% | 790.4% |
| Overall (Ref) | 1.26% | 1.78% | 1.13% | 97.5% | 98.6% | 100% | -22.60% | -33.09% | -31.61% | 1005.8% | 1119.5% |
| Class D | 1.14% | -0.77% | 0.93% | 97.1% | 97.9% | 99% | -25.79% | -31.27% | -32.10% | 1003.4% | 1361.5% |
| Class F | 1.14% | 0.10% | 0.73% | 97.5% | 98.3% | 100% | -30.69% | -41.84% | -41.96% | 833.7% | 828.6% |
| Class TGM | 0.43% | 1.29% | 0.85% | 97.8% | 95.9% | 101% | -41.09% | -51.23% | -51.11% | 716.8% | 676.4% |

- Group 5 off

Group 5 includes tools which need large memory access.

The offgroup5.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  | Over VTM-11.0ecm19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 4.51% | 4.70% | 4.86% | 79.1% | 86.6% | 91% | -10.98% | -12.41% | -23.81% | 806.3% | 454.9% |
| Class A2 | 3.72% | 3.35% | 3.86% | 80.6% | 90.5% | 90% | -18.60% | -22.58% | -26.67% | 780.8% | 526.7% |
| Class B | 4.34% | 4.06% | 4.49% | 80.2% | 90.4% | 79% | -11.42% | -20.02% | -17.38% | 784.6% | 560.9% |
| Class C | 6.17% | 4.42% | 5.09% | 79.9% | 93.5% | 91% | -10.23% | -8.29% | -8.98% | 759.2% | 572.1% |
| Class E | 6.17% | 5.59% | 4.37% | 81.3% | 89.9% | 86% | -14.73% | -19.20% | -17.98% | 744.8% | 592.0% |
| Overall | 4.98% | 4.38% | 4.56% | 80.2% | 90.4% | 87% | -12.83% | -16.44% | -18.23% | 775.0% | 543.2% |
| Class D | 5.07% | 4.23% | 5.25% | 78.6% | 92.1% | 98% | -8.64% | -4.73% | -4.72% | 720.6% | 588.4% |
| Class F | 4.34% | 3.14% | 3.45% | 88.1% | 90.2% | 89% | -27.86% | -32.91% | -32.89% | 554.6% | 588.1% |
| Class TGM | 0.57% | 0.59% | 0.57% | 89.2% | 89.8% | 83% | -43.40% | -49.00% | -48.31% | 453.3% | 609.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  | Over VTM-11.0ecm19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 3.92% | 2.60% | 3.49% | 92.0% | 92.2% | 96% | -24.90% | -23.40% | -35.73% | 1064.6% | 1107.9% |
| Class A2 | 3.43% | 2.34% | 2.70% | 93.0% | 93.8% | 97% | -28.62% | -34.07% | -39.34% | 1034.7% | 1355.7% |
| Class B | 4.17% | 2.63% | 3.57% | 92.6% | 93.1% | 84% | -22.48% | -33.02% | -29.28% | 911.3% | 1233.9% |
| Class C | 5.33% | 3.76% | 3.95% | 95.9% | 95.2% | 91% | -23.43% | -22.13% | -22.63% | 1004.1% | 1498.2% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 4.28% | 2.87% | 3.48% | 93.4% | 93.6% | 90% | -24.44% | -28.40% | -30.81% | 989.6% | 1295.9% |
| Class D | 4.78% | 2.99% | 4.09% | 94.2% | 94.8% | 96% | -24.51% | -22.48% | -23.11% | 912.7% | 1605.2% |
| Class F | 3.17% | 2.27% | 2.34% | 87.6% | 91.6% | 89% | -31.43% | -35.78% | -36.59% | 715.1% | 820.6% |
| Class TGM | 0.86% | 0.73% | 0.81% | 86.0% | 83.6% | 88% | -42.52% | -48.60% | -48.58% | 602.9% | 614.0% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19.0 |  |  |  |  |  | Over VTM-11.0ecm19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 3.43% | 1.00% | 1.47% | 92.8% | 89.7% | 81% | -20.06% | -40.54% | -35.06% | 917.5% | 1080.2% |
| Class C | 5.07% | 1.98% | 2.26% | 95.4% | 91.1% | 90% | -21.46% | -27.84% | -29.64% | 910.4% | 1200.6% |
| Class E | 4.32% | 3.73% | 0.19% | 88.3% | 90.9% | 91% | -19.48% | -26.95% | -28.09% | 806.9% | 745.7% |
| Overall (Ref) | 4.20% | 2.01% | 1.41% | 92.5% | 90.5% | 86% | -20.38% | -32.91% | -31.51% | 886.2% | 1019.9% |
| Class D | 4.38% | 1.49% | 1.93% | 94.4% | 91.9% | 96% | -23.43% | -29.81% | -31.31% | 889.2% | 1358.3% |
| Class F | 3.10% | 0.29% | 1.10% | 86.8% | 90.4% | 87% | -29.26% | -41.52% | -41.56% | 707.1% | 786.4% |
| Class TGM | 1.05% | 0.99% | 0.74% | 84.9% | 87.6% | 85% | -40.69% | -51.33% | -51.12% | 604.8% | 627.4% |

- Group 1-5 off

In this test, all the tools in the group 1-5 are switched off. The offgroup1-5.cfg was used in addition to ECM CTC settings.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 9.74% | 19.31% | 25.10% | 38.0% | 40.3% | 89% | -6.52% | -1.08% | -10.40% | 361.0% | 164.4% |
| Class A2 | 11.38% | 22.31% | 23.11% | 38.2% | 37.8% | 90% | -12.57% | -9.48% | -14.42% | 344.0% | 176.2% |
| Class B | 10.45% | 20.97% | 20.10% | 36.7% | 33.7% | 84% | -6.24% | -8.38% | -5.63% | 376.2% | 213.1% |
| Class C | 11.58% | 13.65% | 14.98% | 36.8% | 36.8% | 97% | -5.73% | -0.41% | -0.55% | 374.2% | 215.4% |
| Class E | 15.75% | 23.02% | 19.48% | 37.9% | 36.1% | 96% | -7.05% | -6.61% | -6.25% | 375.2% | 228.7% |
| Overall | 11.62% | 19.63% | 20.19% | 37.4% | 36.5% | 91% | -7.36% | -5.28% | -6.86% | 367.5% | 200.5% |
| Class D | 9.13% | 11.57% | 13.09% | 36.5% | 34.5% | 97% | -5.11% | 1.93% | 2.37% | 365.1% | 220.9% |
| Class F | 19.32% | 24.48% | 25.84% | 50.5% | 38.0% | 94% | -18.02% | -20.46% | -19.72% | 279.3% | 221.8% |
| Class TGM | 20.23% | 24.93% | 26.01% | 55.6% | 34.3% | 88% | -32.64% | -36.97% | -35.63% | 258.3% | 229.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 | 16.28% | 16.30% | 25.09% | 54.0% | 58.1% | 94% | -16.00% | -13.53% | -23.44% | 500.4% | 479.0% |
| Class A2 | 16.04% | 19.94% | 20.78% | 54.2% | 55.9% | 94% | -19.95% | -23.10% | -28.79% | 481.9% | 532.2% |
| Class B | 14.54% | 20.28% | 21.09% | 48.7% | 55.1% | 82% | -14.71% | -21.58% | -17.35% | 459.4% | 620.0% |
| Class C | 15.20% | 15.89% | 17.14% | 47.6% | 50.9% | 90% | -16.21% | -13.04% | -12.75% | 492.9% | 639.6% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref) | 15.36% | 18.25% | 20.77% | 50.5% | 54.7% | 89% | -16.41% | -18.00% | -19.63% | 480.8% | 575.9% |
| Class D | 12.22% | 13.61% | 15.41% | 46.1% | 50.1% | 95% | -19.04% | -14.41% | -14.68% | 465.1% | 683.7% |
| Class F | 19.09% | 21.03% | 22.61% | 52.9% | 58.3% | 89% | -21.20% | -24.69% | -24.80% | 425.7% | 453.4% |
| Class TGM | 20.87% | 25.38% | 26.12% | 54.9% | 54.7% | 89% | -31.26% | -36.28% | -36.00% | 373.1% | 353.0% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-19 |  |  |  |  |  | Over VTM-11-ECM19 |  |  |  |  |
|  | Y | U | V | EncT | DecT | mPeakR | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 13.44% | 13.79% | 14.84% | 43.4% | 44.6% | 79% | -12.29% | -33.09% | -26.63% | 434.1% | 448.1% |
| Class C | 14.69% | 12.58% | 14.09% | 42.4% | 38.1% | 89% | -14.34% | -20.49% | -21.52% | 401.6% | 451.3% |
| Class E | 14.48% | 17.60% | 13.31% | 45.9% | 56.1% | 84% | -11.63% | -17.46% | -18.43% | 422.9% | 382.8% |
| Overall (Ref) | 14.12% | 14.34% | 14.21% | 43.7% | 44.8% | 83% | -12.81% | -24.98% | -22.87% | 420.2% | 431.8% |
| Class D | 12.32% | 10.90% | 13.30% | 41.5% | 41.8% | 94% | -17.58% | -23.17% | -23.67% | 404.0% | 497.1% |
| Class F | 18.85% | 17.26% | 18.36% | 50.1% | 51.1% | 87% | -18.62% | -31.90% | -31.90% | 384.5% | 354.4% |
| Class TGM | 19.59% | 25.76% | 26.97% | 56.1% | 45.8% | 85% | -29.90% | -39.95% | -39.37% | 412.3% | 292.3% |

JVET-AO0008 JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) [S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs)]

- Activities

The AHG 8 used the main JVET reflector, jvet@lists.rwth-aachen.de, for email discussion. Between the last and this meeting, there was no AHG8 email sent to the main reflector. There is no AHG8 input contribution to this meeting. The current focus is on finalization of TR (v1).

- Software and Common Test Conditions

AHG 8 related software and documents can be accessed at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm. This repository contains two projects: common test conditions, reporting templates with anchor results, evaluation scripts and task networks are available in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/ofm-ctc, and software implementation examples are hosted in https://vcgit.hhi.fraunhofer.de/jvet-ahg-ofm/vtm-ofm. For this meeting cycle, common test conditions remain unchanged as described in output document JVET-AI2031.

- Technical Report

Comments on the Draft Technical Report (DTR) will be reviewed, and the next step will be discussed during this meeting.

- Input contributions

There is no other input contributions related to AHG 8 mandates except this report.

- JVET-AO0008	JVET AHG report: Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8)

- Recommendations

The AHG recommends to:

- Review received comments.

- Proceed to finalization.

Editors were asked to conduct an analysis of the comments, and suggest which ones are useful, and what action should be taken in the TR. If some of the comments that are believed to be useful have been assessed to be too technical by the ISO staff, it could be useful to issue a second DTR rather than running into discussions that delays the process of publication.

JVET-AO0009 JVET AHG report: SEI message studies (AHG9) [J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs)]

- Related contributions

A total of 77 contributions are identified relating to the mandates of AHG9 (see section 6). Some contributions relate to more than one mandate and/or SEI message. Some contributions also relate to the work of AHG16.

The following is a list of contributions related to the mandates of AHG9.

- Study the SEI messages in VSEI, VVC, HEVC and AVC.

- Study JVET-AN1017, identify any issues and propose solutions as appropriate.

- Study JVET-AN2032, including study of SEI messages with different options, when those are present, and propose improvements.

- Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing.

- Identify potential needs for additional SEI messages.

- Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context.

- Study the alignment of the same SEI messages in different standards.

- Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM.

- Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de) with [AHG9] in message headers. No emails with [AHG9] were exchanged.

- Recommendations

The AHG recommends to:

- Review all related contributions; and

- Continue SEI messages studies.

JVET-AO0010 JVET AHG report: Encoding algorithm optimization (AHG10) [K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs)]

- Activity

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail was sent related to AHG10.

- Related contributions

Following contributions were identified relating to AHG10 and summarized in the following sections.

- JVET-AO0136: AHG3/AHG10: On the GOP-based-RPR decision source

This document reports a fix to the VTM reference software to use MCTF filtered samples for GOP based RPR decisions instead of original samples. The issue was introduced unintentionally as part of memory optimizations of VTM.

The fix has already been merged to VTM and the BDR impact when GOP based RPR is enabled for QP 32 to 47 are as follows:

RA:-0,51%(Y),-0,10%(U), -0,18%(V); 95,4%(EncT) 88,8% (DecT), 91,7%(EncVMP),92,6% (DecVMP)

LB:-0,03%(Y),-0,06%(U), -0,08%(V); 99,4%(EncT)100,2%(DecT),99,7%(EncVMP),100,0% (DecVMP)

For CTC (GOP based RPR not enabled in CTC):

RA:0,00%(Y),0,00%(U),0,00%(V);100,6%(EncT),100,0%(DecT),111,0%(EncVMP),101,2%(DecVMP)

LB:0,00%(Y),0,00%(U), 0,00%(V);100,0%(EncT),100,7%(DecT),106,3%(EncVMP),101,4%(DecVMP)

The contribution recommends checking if a similar unintended change in behavior exists in any other SW packages derived from VTM, e.g. NNVC or ECM, and possibly align the software’s.

Cross-checks in JVET-AO0219 and JVET-AO0266.

- JVET-AO0220:  AHG12: Optimizations for JVET-AN0271 software

This document reports some optimizations to the JVET-AN0271 (Software optimization and complexity reduction of ECM). The modification relates to encoder only improvements from the JVET-AN0267 (Ericsson, Fraunhofer HHI and Nokia responses to Joint CfE on Video Compression with Capability beyond VVC) namely 16-tap filter for MCTF, BIM using lambda modification and disabling LMCS for SDR. Also, some bugfixes related to RPR, IntraNN, IntraTMP, PDPC, TIMD, ALF, etc.

The BDR impact compared to JVET-AN0271 is as follows:

CTC RA: -1.27% (Y), -3.97% (U), -3.73% (V); 97.6% (EncT), 100.4% (DecT),​

CfE RA: -1.77% (Y), -5.58% (U), -5.46% (V); 97.5% (EncT), 100.5% (DecT).

The document also reports the results in comparison with VTM-23.13 for CfE (not rate matched):

CfE RA: -21.60%(Y), -36.07%(U),	-41.35%(V); 1282.3% (EncT), 764.4% (DecT)

CfE LDB: -16.63%%(Y), -33.51%(U),	-29.43%(V); 694.7% (EncT), 482.4% (DecT)

Additionally, also with GOP based RPR enabled on Test (not rate matched):

CfE RA: -23.18%(Y), -37.92%(U), -43.10%(V); 1027.9% (EncT), 515.9% (DecT)

CfE LDB: -17.91%(Y), -37.08%(U), -32.98%(V); 652.4% (EncT), 417.6% (DecT)

- Recommendations

The AHG recommends that the related input contributions are reviewed, and to further continue the study of encoding algorithm optimizations in JVET.

It was commented that in JVET-AN0267 and JVET-AN0271 some elements might also be beneficial for an VTM encoder. It was suggested that an appropriate way to proceed would be to make a related proposal for VTM optimization.

JVET-AO0011 JVET AHG report: Neural network-based video coding (AHG11) [E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs)]

- Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, for emails exchange.

  - Teleconferences

The AHG conducted two joint teleconferences with AHG14 and EE1 during the interim period. The teleconferences were held on November, 18 and December, 16. In those teleconferences, the following topics were discussed:

- NNVC15.0 software integration status and anchor performance.

- Hybrid plus E2E AI video codec framework development status report.

- MPEG AI survey.

- Training reproducibility

- Overflow aware model quantizing for bit-exact reproducibility

- EE1 tests final design.

Details are summarized in the AHG11 & AHG14 teleconference report JVET-AO0041.

  - Common Test Conditions

Anchor for the NN-based video coding activity made available though the Git repository used for the AHG activity:

https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC_anchor_performance_NNVC-15-VTM_vs_NNVC-15.xlsm

also distributed by AhG14 in JVET-AO0014 and announced during intermediate AhG meeting.

  - Effect of training

Training of NNVC algorithms cannot be precisely reproduced due to the float point arithmetic used in training. However, using the same training dataset and same training procedure typically leads to BD-rate difference in inference within 0.1%.  Higher differences indicate a problem with training reproducibility.

Some of EE1 sub-tests made comparison between different loss-functions in training (L1, Charbonnier Loss + Ternary Loss) for Deep Reference Picture (DRF) technology. A 0.3% gain was found if loss function is a combination of Charbonnier Loss and Ternary Loss. More details can be found in JVET-AO0108.

It was found that non-deterministic bilinear 2D up-sampling in pytorch leads to significant deviation in training results for NN-based super-resolution (even several runs of training on a same platform deviates a lot). After several fixes to make training more deterministic and alignment of CUDA versions learning curves became very close. This activity is reported in JVET-AO0127. All trainings are impacted by the non deterministic aspects and discussions on correcting the current training scripts are expected.

  - Bit-exact reproducibility

MPEG Requirements (WG2) issued a survey on bit-exact reproducibility of AI algorithms used for coding and neural network models storage. Information about this survey was presented and discussed during NNVC teleconference. At least two responses to this survey were submitted to WG2 with copy to JVET (JVET-AO0147 and  JVET-AO0249).

During NNVC teleconference it was demonstrated that quantizing to int16 is not enough to achieve bit-exact reproducibility of neural-network algorithms. Additionally, overflow must be avoided. A methodology for overflow aware quantization of neural-network models was presented during teleconference as a constructive way of achieving bit-exact reproducibility of AI-based algorithms. This information was also submitted as input contribution JVET-AO0148. Another approach for achieving bit-exact reproducibility, based on the use of stable float convolution,  is proposed in  JVET-AO0178. Difference between those two approaches seams to be insignificant.

  - Regarding complexity

The number of multiplications, reported in kMAC/pxl, is primarily used for complexity assessment. CPU run time is also reported. Tests in EE1 are designed to ensure BD-rate gains without exceeding the complexity of previously developed NNVC tools, or to provide substantial reduction in complexity.

  - EE Coordination

The AHG finalized, conducted, and discussed the EE on NN based video coding. A summary report for the EE is available in JVET-AO0023.

EE1 related contributions and cross-checks are summarized in EE1 summary report.

  - Performance Evaluation

The performance and complexity of NN-based tools available in NNVC SW is summarized in the table below. All test data provided by AhG14. Encoding and decoding run time is very dependent on cluster used for simulation. Run time data in this table are all from InterDigital, except content adaptive tools results that are from Nokia.

There were no changes in default configuration of NNVC SW. The main reason for increase of decoding run time relatively to NNVC configured as VTM (with all NNVC tools disabled) is the removal of redundant operations in NNVC-VTM (as was agree at the last meeting). Changes compared to the last meeting are should in blue font. More detailed historical records about NNVC tools performance can be found in attached to this report excel.

| Test vs NNVC (configured as VTM) | Random Access cfg |  |  |  |  | kMAC/pxl |  |  |  | Param (Mprm) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Total | Filter | Intra | SR or DRF | Total | Filter | Intra | SR or DRF |
| NN-Intra & LOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-15.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 39 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NNVC-14.0 (LOP6) | -8.0% | -14.9% | -13.5% | 1.2 | 27 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NN-Intra & HOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (HOP5) | -14.1% | -19.8% | -20.1% | 3.4 | 1180 | 471 | 466 | 4.8 | 0 | 2.7 | 1.41 | 1.3 | 0 |
| NN-Intra & VLOP filter (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 (VLOP4) | -6.1% | -7.5% | -6.0% | 1.1 | 14 | 9.9 | 5.1 | 4.8 | 0 | 1.3 | 0.07 | 1.3 | 0 |
| NN-Intra & LOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC 15.0 caLOP4 | -8.5% | -18.0% | -17.5% | 2.3 | 38 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NNVC 14.0 caLOP4-decp | -8.6% | -15.7% | -16.3% | 2.3 | 25 | 22 | 17.2 | 4.8 | 0 | 1.6 | 0.25 | 1.3 | 0 |
| NN-Intra & VLOP filter content adaptive (2 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-15.0 caVLOP4 | -6.9% | -8.5% | -7.2% | 2.3 | 18 | 10.2 | 5.4 | 4.8 | 0 | 1.3 | 0.02 | 1.3 | 0 |
| NN-Intra & LOP filter & adaptive resolution coding (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-14.0 NNSR | -8.6% | -13.7% | -12.5% | 1.1 | 24 | 26.1 | 16.8 | 4.8 | 4.7 | 1.6 | 0.05 | 1.3 | 0.2 |
| NNVC-14.0 RPR | -8.6% | -13.7% | -12.6% | 1.1 | 24 | 21.4 | 16.6 | 4.8 | 0 | 1.5 | 0.25 | 1.3 | 0 |
| NN-Intra & LOP filter & DRF (3 tools) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-15.0 DRF | -10.0% | -16.2% | -15.4% | 1.7 | 869 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |
| NNVC-14.0 DRF | -9.8% | -15.9% | -14.2% | 1.7 | 604 | 477 | 16.6 | 4.8 | 420 | 7.7 | 0.25 | 1.3 | 6.1 |

More details and analysis for tools and tools combination are provided in AhG14 report.

  - End-to-end AI coded reference picture

Multi-layer frame work was tested in EE1-6 category for three different end-to-end AI image codecs. One special test was run by forcing skip mode in the enhancement layer to emulate re-placement of VTM I-picture by end-to-end AI coded one, emulating a single layer encoding. Details will be discussed in EE1 report.

It is not trivial to incorporate a fundamentally different codec into VTM and have a good overlap of RD-curves and get a reliable BD-rate comparison. In addition, if the ‘base layer’ output by the AI codec was not optimized for PSNR but ‘enhancement’ layer encoder minimizes the MSE, then a large amount of residual data is generate. Alignment between base and enhancement layer and lost/cost functions is required to enable meaningful testing.

Nevertheless, more than 1% gain over VTM in random access configuration test and more than 5% gain in all-intra configuration were demonstrated in EE1. In non-EE1 contribution BD-rate gain over VTM reaches 1.7% (random access) and 8.4% (all intra).

If multi-layer framework is used to employ an externally coded end-to-end AI picture as a prediction, then redundant high-level syntax elements are coded. This issue is known and one AhG11 document (JVET-AO0173) proposes single-layer framework.

  - NNVC implementation aspects

One informational contribution (JVET-AO0247) reports power consumption measurement for AI coding system on pure CPU and combination of GPU or FPGA. Experiments include data transfer between CPU and GPU/FPGA.

  - New design elements proposal

There are several contributions targeting improvement of NNSR, LOP6 and end-to-end AI externally coded picture.

- List of related contributions

In total 38 NNVC related contributions submitted for this meeting. Among them 3 reports, 12 document describing EE1 tests, 10 cross-checks, 4 contributions of bit-exact reproducibility of AI methods (including 2 responses to MPEG WG2 survey), 7 new proposals improving or modifying NNVC tools under study (LOP filter, NNSR and external AI coded picture), one contribution on training and one on energy consumption.

| JVET number | Title | Source |
| --- | --- | --- |
| Reports (3) |  |  |
| JVET-AO0011 | JVET AHG report: Neural network-based video coding (AHG11) | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) |
| JVET-AO0041 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| JVET-AO0023 | EE1: Summary report of exploration experiment on neural network-based video coding | E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators) |
| EE1 contributions (12) |  |  |
| JVET-AO0066 | EE1-1.1: LOP with Overlapped Feature Integration | J. Chi, A. Li, Y. Du, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion) |
| JVET-AO0074 | EE1-1.2: Combination test of EE1-1.1 and EE1-1.2 | J. Han, C. Jung, Q. Qin (Xidian Univ.), J. Chi, A. Li, Y. Du, C. Zhu, L. Lei, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion) |
| JVET-AO0095 | EE1-3.2: Neural network in-loop filtering usage based on activation map | M. Santamaria, D. Buğdayci Sansli, F. Cricri (Nokia) |
| JVET-AO0108 | EE1-4.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization | X. Chen, W. Zhang, N. Fu, J. Zhang, Z. Chen (Wuhan Univ.) |
| JVET-AO0117 | EE1-3.1: Normative TDO method as an alternative option in NNVC | H. Kwon, H. Ko (Hanyang Univ.), D. Kim, S.-C. Lim (ETRI) |
| JVET-AO0128 | EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) | H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI) |
| JVET-AO0129 | EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM) | H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI) |
| JVET-AO0133 | EE1-3.3: Combination of EE1-3.1 and EE1-3.2 | D. Kim, S.-C. Lim (ETRI), H. Kwon, H. Ko (Hanyang Univ.), D. Buğdayci Sansli, F. Cricri, M. Santamaria (Nokia) |
| JVET-AO0176 | EE1-6.1: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AO0177 | EE1-6.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AO0186 | EE1-6.1.1 and EE1-6.2.1: DCVC-FM as a Learned End-to-End Intra Frame Codec | M. Aderdor, T. Solovyev, E. Alshina (Huawei) |
| JVET-AO0204 | EE1-6.1.3 and EE1-6.2.3: JPEG-AI as a Learned End-to-End Intra Frame Codec | A. Karabutov, P. Jia, E. Alshina (Huawei) |
| Cross-checks (10) |  |  |
| JVET-AO0236 | Crosscheck of JVET-AO0117 (EE1-3.1: Normative TDO method as an alternative option in NNVC) | J. Pardo (Huawei) |
| JVET-AO0237 | Crosscheck of JVET-AO0133 (EE1-3.3: Combination of EE1-3.1 and EE1-3.2) | J. Pardo (Huawei) |
| JVET-AO0257 | Crosscheck of JVET-AO0066 (EE1-1.1: LOP with Overlapped Feature Integration) |  |
| JVET-AO0258 | Crosscheck of JVET-AO0095 (EE1-3.2: Neural network in-loop filtering usage based on activation map) |  |
| JVET-AO0265 | Cross-check of JVET-AO0133 (EE1-3.3: Combination of EE1-3.1 and EE1-3.2) | T. Dumas (InterDigital) |
| JVET-AO0271 | Crosscheck of JVET-AO0128 (EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM)) | T. Yang, W.-X. He, X.-T. Xie, Q. Liu (HUST) |
| JVET-AO0272 | Crosscheck of JVET-AO0074 (EE1-1.2: Combination test of EE1-1.1 and EE1-1.2) | J. Sauer, Y. Ge (Huawei) |
| JVET-AO0274 | Crosscheck of JVET-AO0108 (EE1-4.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization) | N. Bhaskar (Huawei) |
| JVET-AO0275 | Crosscheck of JVET-AO0055 (AHG14: Dynamic convolution for LOP6 neural in-loop filtering) | N. J. Gadgil, S. Watsa, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung) |
| JVET-AO0276 | Crosscheck of JVET-AO0267 (AHG11: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) | N. Bhaskar (Huawei) |
| Training (1) and implementation aspects (1) |  |  |
| JVET-AO0127 | AHG11/AHG14: Improving Training Reproducibility in NNVC | H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI), T. Yang, W.-X. He, Q. Liu (HUST), J. Chi, Ce Zhu (UESTC), L. Luo (CQUPT) |
| JVET-AO0247 | [AHG11] JPEG-AI Profiling Report on Runtime and Energy Consumption | G. Paul, D. Bhowmik (Newcastle University) |
| Bit-exact reproducibility (4) |  |  |
| JVET-AO0147 | AhG11 Huawei and HiSilicon response to the MPEG Survey on AI reproducibility and NN model hosting | E. Alshina, Y. Zhao (Huawei), Q. Wu (HiSilicon) |
| JVET-AO0148 | AhG11 Overflow aware neural-network model quantization | E. Alshina, T. Solovyev (Huawei) |
| JVET-AO0178 | [AHG11] Stable float convolution for neural network inference | H. Zhang, F. Cricri, N. Le, N. Zou, B. Koyuncu (Nokia) |
| JVET-AO0249 | AhG19 AhG11 Interdigital response to the MPEG Survey on AI reproducibility and NN model hosting | F. Galpin, R. James, D. Tian (Interdigital) |
| New NNVC technical proposals (7) |  |  |
| JVET-AO0055 | AHG14: Dynamic convolution for LOP6 neural in-loop filtering | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0143 | AHG11: Cross-component enhanced NNSR | T. Yang, W.-X. He, X.-T. Xie, Q. Liu (HUST) |
| JVET-AO0144 | AHG11: Enhancing LOP6 with Re-Exploited Boundary Strength Guidance | X.-T. Xie, W.-X. He, T. Yang, Q. Liu (HUST) |
| JVET-AO0173 | [AHG11] A Hybrid Framework Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec | N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia) |
| JVET-AO0191 | [AHG11]: DCVC-RT as an End-to-End Learned Intra Frame Codec | M. Aderdor, T. Solovyev, E. Alshina (Huawei) |
| JVET-AO0194 | [AHG11]: Luma Chroma Balancing for DCVC End-to-End Learned Intra Frames | M. Aderdor, T. Solovyev, E. Alshina (Huawei) |
| JVET-AO0267 | AHG11: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement | T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.) |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Continue investigating neural network-based video coding tools, including coding performance and complexity.

- Continue study of the framework for hybrid codec combined with End-to-End AI coded pictures.

- Discuss device interoperability and bit-exact reconstruction of NNVC algorithms.

- Recommend promising technologies for next EE1 round.

JVET-AO0012 JVET AHG report: Enhanced compression beyond VVC capability (AHG12) [M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs)]

- Activities

The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-AN2024). The combined improvements of the ECM-19.0 over VTM-11.0ecm19.0 anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -14.87% | -16.35% | -27.31% | 1204.8% | 601.9% |
| Class A2 | -21.54% | -25.02% | -29.24% | 1102.5% | 644.2% |
| Class B | -15.11% | -23.02% | -20.90% | 1040.4% | 605.5% |
| Class C | -15.30% | -12.03% | -13.20% | 1010.8% | 600.4% |
| Class E | -19.69% | -23.36% | -21.40% | 994.8% | 634.1% |
| Overall | -16.95% | -19.86% | -21.73% | 1061.7% | 614.7% |
| Class D | -13.04% | -8.61% | -9.49% | 1002.9% | 671.9% |
| Class F | -31.03% | -35.19% | -35.37% | 759.6% | 776.4% |
| Class TGM | -43.73% | -49.29% | -48.58% | 588.9% | 687.7% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -27.70% | -25.43% | -37.71% | 1235.5% | 1313.9% |
| Class A2 | -30.98% | -35.54% | -40.94% | 1190.4% | 1511.6% |
| Class B | -25.57% | -34.66% | -31.70% | 1013.7% | 1221.9% |
| Class C | -27.32% | -24.94% | -25.55% | 1136.5% | 1427.0% |
| Class E |  |  |  |  |  |
| Overall | -27.55% | -30.40% | -33.11% | 1122.8% | 1348.3% |
| Class D | -28.00% | -24.79% | -26.22% | 1058.8% | 1531.7% |
| Class F | -33.69% | -37.31% | -38.21% | 959.7% | 1004.2% |
| Class TGM | -43.03% | -48.96% | -48.99% | 829.8% | 842.2% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -22.69% | -41.03% | -35.94% | 994.3% | 1080.4% |
| Class C | -25.22% | -29.20% | -31.15% | 970.5% | 1123.2% |
| Class E | -22.82% | -29.50% | -27.98% | 1004.6% | 706.9% |
| Overall | -23.57% | -34.21% | -32.35% | 988.8% | 984.3% |
| Class D | -26.68% | -30.77% | -32.61% | 996.2% | 1268.5% |
| Class F | -31.47% | -41.72% | -42.29% | 951.1% | 951.5% |
| Class TGM | -41.32% | -51.82% | -51.52% | 777.6% | 692.2% |

The rate reduction for natural sequences over VTM in RA configuration for {Y, U, V} increased from ECM-18.0’s {-27.37%, -29.94%, -32.58%} to ECM-19.0’s {-27.55%, -30.40%, -33.11%}. For SCC sequences (class TGM) the rate reduction for RA configuration changed from ECM-18.0’s -43.07%, -48.90%, -48.87} to ECM-19.0’s { -43.03%, -48.96%, -48.99%}.

- Contributions

In addition to 23 EE2 contributions, 13 (comparing to 42 last meeting) EE2-related and AHG12-related contributions were received. The EE2-related and AHG12-related contributions can be subdivided as follows:

- Intra (8)

JVET-AO0092, "Non-EE2: Gradient based Multi-Model EIP", P.-K. Liu, C.-Y. Liu, C.-C. Lin, S.-P. Wang, C.-H. Yau, C.-L. Lin (ITRI)

JVET-AO0153, "AHG 12/17: On EIP in JVET-AN0267 software", Z. Xie, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AO0154, "AHG 12/17: IntraTMP on JVET-AN0267 software", L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-AO0163, "Non-EE2: Improvement on chroma MPM", Z. Li, W. Niu, X. Zeng, M. Jia, Y. Wang, C. Huang (ZTE)

JVET-AO0174, "Non-EE2: Modification of EIP filter shapes", Y. Liu, Z. Zhang, Y. Huo, Z. Zhang, J. Cai (Transsion)

JVET-AO0175, "Non-EE2: Scharr Filter for Rectangular blocks", A. C. Sidiya, S. Deshpande (Sharp)

JVET-AO0200, "AHG12: Tensor-Based Intra Mode Derivation in VTM and JVET-AN0267 software", Y.-U. Yoon, H. Huang, L.-F. Chen, R. Chernyak, S. Liu (Tencent)

JVET-AO0218, "Intra lfnstIdx Prediction", Y. Wang, R. Chernyak, S. Liu (Tencent)

- Cross Component Prediction (1)

JVET-AO0146, "EE2-related: Clipping operation refinements of Test 1.10 for the CCCM modes", P. Onno, G. Laroche, B. Galmiche (Canon)

- In Loop Filters (1)

JVET-AO0071, "AHG12: Simplification on TALF", L. Xu, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)

- Coefficients Quantization (1)

JVET-AO0139, "AhG12: On the Dead-Zone in Quantization", M. Balcilar, M. Blestel, P. Andrivon (Ofinno)

- Entropy Coding (1)

JVET-AO0199, "Non-EE2: Counter-based Temporal Probability Initialization for Temporal CABAC", Z. Xiang, R. Chernyak, H. Huang, S. Liu (Tencent)

- Other (1)

JVET-AO0220, "AHG12: Optimizations for JVET-AN0271 software”,	Y.-J. Chang, C.-C. Chen, M. Coban, K. Cui, P. Garus, N. Hu, M. Karczewicz, P.-H. Lin, X. Meng, V. Menon, V. Seregin, Y. Shao, G. Verba, H. Wang, E. Ye, R. Yu, Y. Zhang, Z. Zhang, W. Zhu (Qualcomm)

- Recommendations

The AHG recommends:

- To review all the related contributions.

Regarding contributions on JVET-AN0267 and JVET-AN0271, see notes under AHG10. JVET should not work collectively on further optimization of those packages.

JVET-AO0013 JVET AHG report: Film grain technologies (AHG13) [W. Husak, P. de Lagrange (co-chairs), A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs)]

Presented Thursday 15 Jan. 1305-1335 UTC

- Discussion in AHG

This period was slow with no meetings or discussions held.

From the previous meetings, the following five items should be in the second edition:

- Updated Technical Report (TR) text to fix errors and clumsy wording

- Updated tools for the TR

- Video test sequences specific to film grain that were added to the draft report was removed after conferring with the other chairs due to the sequences not being located in a permanent location.

- Related contributions

Six contributions related to AHG13 were identified as of 06/26/2025.

- One contribution was the AHG report:

  - JVET-AO0013 JVET AHG report: Film grain technologies (AHG13)

- Four other contributions were uploaded at the time of the report drafting:

  - JVET-AO0064 AHG9/AHG13: On Film Grain Regions Characteristics SEI Message

  - JVET-AO0102 AHG9: On handling film grain characteristics SEI message and film grain regions characteristics SEI message

  - JVET-AO0103 AHG9: On alpha channel adaptation in film grain regions characteristics SEI message

  - JVET-AO0104 AHG9: On region-based adaptation in film grain regions characteristics SEI message

  - JVET-AO0105 AHG9: Miscellaneous aspects of film grain regions characteristics SEI message

  - Contributions

There were five contributions registered other than the AHG report.  All four were uploaded as of 10/02/25.

    - JVET-AO0064 AHG9/AHG13: On Film Grain Regions Characteristics SEI Message

This contribution relates to modifications to film grain regions characteristics (FGRC) SEI message, which is included in the Technologies under consideration for future extensions of VSEI. Following is proposed:

- Proposal 1: It is proposed to signal fgr_regions_model_id[ i ] for the i–th region in the picture, only when fgr_film_grain_enabled_flag[ i ] is equal to 1 and fgr_single_model_flag is equal to 0. The inference rule for fgr_regions_model_id[ i ] is also modified when it is not present.

- Proposal 2: It is proposed to modify the inference for fgr_comp_model_value[ m ][ c ][ i ][ 0 ] when not present. It is proposed to infer fgr_comp_model_value[ m ][ c ][ i ][ 0 ] to be equal to 0 when not present.

- Proposal 3: It is proposed to signal the syntax elements fgr_region_top[ i ], fgr_region_left[ i ], fgr_region_width[ i ], and fgr_region_height[ i ] with ue(v) coding instead of u(16) coding. Alternatively, it is proposed to signal a syntax element which specifies the length of these syntax elements and then to code them with u(v) coding.

    - JVET-AO0102 AHG9: On handling film grain characteristics SEI message and film grain regions characteristics SEI message

This contribution proposes to clarify how to handle film grain characteristics (FGC) SEI message and film grain regions characteristics (FGR) SEI message in a bitstream.

The summary of the proposed modifications is as follows:

- Option 1: When both an FGC SEI message and an FGR SEI message are present and have persistence over the same picture,

  - Option 1−1: Specify that decoders shall ignore the FGC SEI message.

  - Option 1−2: Specify that the order applying the FGC and FGR SEI messages is specified by an SEI processing order (SPO) SEI message.

  - Option 1−3: Add a syntax element that indicates how decoders work.

- Option 2: Specify that an FGC SEI message and an FGR SEI message shall not be present and have persistence over the same picture.

    - JVET-AO0103 AHG9: On alpha channel adaptation in film grain regions characteristics SEI message

This contribution proposes modifications that are asserted to provide improvement to alpha channel adaptation in the film grain regions characteristics SEI message in TuC for future extensions of VSEI. The proposed modifications are as follows:

1	Clarify that monochrome alpha plans are used for alpha channel adaptation.

2	On alpha auxiliary layer used for alpha channel adaptation

–	Option 1: clarify that the layer with the least layer ID value is used.

–	Option 2: clarify that the layer with the least layer ID value, which is greater than the current layer ID, is used.

–	Option 3: add syntax element indicating layer ID to be used.

3	Disallow from disabling both alpha channel adaptation and region-based adaptation.

    - JVET-AO0104 AHG9: On region-based adaptation in film grain regions characteristics SEI message

This contribution proposes modifications that are asserted to provide improvement to region-based adaptation in the film grain regions characteristics SEI message in TuC for future extensions of VSEI. The proposed modifications are as follows:

- Signal fgr_regions_model_id[ i ] only when fgr_film_grain_enabled_flag[ i ] is equal to 1.

- Disallow the values of fgr_region_width[ i ] and fgr_region_height[ i ] equal to 0.

- Modify the signaling of multiple models for future extensions.

- Relax the constraint on the unsupported value of fgr_regions_model_id[ i ].

- Introduce a syntax element that indicates applying order of film grain synthesis for regions.

- Introduce a syntax element that indicates subpicture ID corresponding to a region.

    - JVET-AO0105 AHG9: Miscellaneous aspects of film grain regions characteristics SEI message

This contribution proposes miscellaneous modifications that are asserted to provide improvement to the film grain regions characteristics SEI message in TuC for future extensions of VSEI. The proposed modifications are as follows:

- Specify the value range of fgr_pic_width_in_luma_samples and fgr_pic_height_in_luma_samples.

- Specify the value of fgrModelidValue[ x ][ y ] when fgr_region_based_adaptation_flag is equal to 0.

- Clarify the input which film grain synthesis applies to.

- Recommendations

The AHG recommends:

- Continue editing the second edition for the TR;

- the related input contributions (non-AHG9) to be reviewed;

- any liaisons to be reviewed;

- to continue conformance discussion;

- to consider any newly proposed SEI message extensions;

- to discuss the possibility of creating as appropriate documents providing Engineering Guideline (EG) or recommended practices (RP) relating to film grain technologies and their applications; and

- to continue the study of film grain technologies in JVET.

It was commented that the second version of the TR (based on JVET-AN2020) was also intended to attach software related to some of the elements described. The maturity of software elements should be studied in the AHG (add mandate).

The current status of tentatively defining conformance has many open questions. It is unclear how and whether exact reproduction can be defined, and if it is desirable.

It was commented that the success of film grain synthesis technology in the market highly depends on ubiquity of devices that implement it.

It was commented that it would be desirable to have more support for film grain technology in a future standard. However, the CfP should not include this, as it is mostly about advances in coding tools an algorithms, and known algorithms are mostly about pre and post processing.

JVET-AO0014 JVET AHG report: NNVC software development (AHG14) [F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs)]

- Software development

  - Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM in the NNVC branch.

NNVC software is based on latest VTM, VTM-23.13 at the time of software integration.

Legacy version (up to NNVC-13.0) can be found in branch VTM-11.0_nnvc and contains history of development of tools integrated up to version 13.0.

NNVC-15.0 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc is used for NNVC performance evaluation.

  - Software changes

    - Current NNVC

Several commits were merged in the NNVC repository. The following changes were integrated:

| Status | Repository | MR | JVET |  |
| --- | --- | --- | --- | --- |
| Done | NNVC | 405 |  | fix concat in hybrid mode |
| Done | SADL | 168 | AN0197 | Add new layer Sub |
| Done | SADL | 187 |  | SIMD for kernel 5x5 |
| Done | NNVC | 404 |  | fix error message in non hybrid mode |
| Done | NNVC | 403 |  | fix bug in DRF+RPR |
| Done | NNVC |  |  | avoid to overwrite of temporary file in hybrid mode |
| Done | NNVC |  | AN0153 | JVET-AN0153 test EE1-2.2. |
| Done | NNVC | 402 | AN0213 | sadl v15 |
| Done | NNVC | 395 |  | hybrid framework corner case fix |
| Done | NNVC | 399 |  | update CI to new environement |
| Done | NNVC |  |  | rebase with upstream VTM 23.13 |
| Done | SADL |  |  | more utest for max pool |
| Done | SADL |  | AN0128 | AhG14: Improvement of SIMD implementation with expanded SIMD operators in SADL for LOP with Overlapped Feature Integration |
| Done | CTC |  | AN0047 | Environmental information collection script |
| Done | NNVC | 393 | AN0225 | LMCS on base layer |
| Done | NNVC | 384 | AN0014 | [DO NOT SQUASH] cherry pick vtm master |
| Done | NNVC | 387 | AN0195 | DRF training code |
| Done | SADL | 182 |  | fix gridsample layer |
| Done | SADL |  |  | better overflow in accumulator |
| Done | NNVC | 383 | AN0213 | update to intermediate sadl with corrected float version |
| Done | NNVC | 380 | AN0209 | RPR_DRF_BugFix |
| Done | NNVC | 378 | AN0054 AN0215 | TDO lambda for high QP |
| Done | NNVC | 375 | AN0215 | allow to activate nnlf per hierarchy layer idx |
| Done | NNVC | 379 |  | Fix #139: NNSR verbose log |
| Done | NNVC | 382 |  | add details on hybrid coding in documentation |
| Done | NNVC | 381 | AN0054 AN0215 | JVET-AN0054 JVET-AN0215 NNVC CFE code |
| Done | SADL | 175 | AN0196 | Fix Flatten Bug: Align Flatten with ONNX (2D output) |
| Done | SADL | 166 |  | Fix slice bug for tensors batch dimension > 1 |
| Done | SADL | 174 | AN0196 | int32 SIMD implementation for layer_relu |
| Done | SADL | 172 | AN0196 | int32 SIMD implementation for layer_add |
| Done | SADL | 169 | AN0196 | Fix SIMD BiasAdd scaling: shift before add |
| Done | SADL | 167 |  | Add copyright info in Tile |
| Done | SADL | 165 | AN0196 | Fix SIMD Add scaling when q1 > q2 (pre-align scales before accumulation) |
| Done | SADL | 180 |  | fix bad dispatch of max pool simd |
| Done | SADL | 178 | AN0197 | Add new layer Floor |
| Done | SADL | 171 | AN0196 | Extend GridSample: add padding_mode=zeros |
| Done | SADL | 173 | AN0196 | Extend Concat: concatenation along any axis. |
| Done | SADL | 170 | AN0196 | Extend Resize: 4× up, 0.5×/0.25× |
| Done | SADL | 164 | AN0196 | Bugfix for ReduceMean |
| Done | NNVC | 386 | AN0195 | DRF models update |
| Done | NNVC | 388 | AN0195 | DRF inference code cleaning |
| Done | VTM |  |  | rebase with upstream VTM 23.12 |
| Cancelled | NNVC | 385 |  | add missing new parameter from VTM in NNVC cfg files |

The following MR were integrating/pending after the v15.1:

| Status | Repository | MR | JVET |  |
| --- | --- | --- | --- | --- |
| done | SADL | 191 | AN0222 | JVET-AN0222: abs operator |
| done | SADL | 190 | AN0222 | JVET-AN0222: Depth2Space operator |
| pending | SADL | 192 | AN0222 | JVET-AN0222: BitShift operator |
| pending | SADL | 189 | AN0222 | JVET-AN0222: clip operation |
| dispatched | SADL | 179 | AN0222 | JVET-AM0222: New NN elements for SADL |
| missing | NNVC |  |  | clean-up RPR policy code (VTM and NNSR compatible) |
| pending | SADL | 177 | AN0197 | Add new layer Gather |
| pending | SADL | 176 | AN0197 | Add new layer GatherElements |
| pending | NNVC | 408 |  | Fix for multi-layer in class F (VTM issue) |
| pending | NNVC | 409 |  | Fix concat with SEI and no embedded NN |

The RPR policy between VTM and NNVC is still to be aligned.

  - Software overview

The framework for Hybrid end-to-end coding of intra slice has been improved for flexibility of testing for EE1. Tests can now be done on clusters in parallel using external encoder/decoder.

A bug originating from VTM multi-layer in some conditions (HashME) was found and is being fixed.

The master branch is kept synchronized with the upstream repository (VTM).

  - NNVC-15

  - Software version

NNVC 15.1 was tagged 19th December 2025.

NNVC-15.0 was tagged 14th November 2025.

NNVC-14.1 was tagged 28th August 2025

NNVC-14.0 was tagged July 28th, 2025 (contains VLOP4, DRF, hybrid e2e)

NNVC-13 was tagged April 28th, 2025 (contains LOP6)

NNVC-12 was tagged February 6th, 2025 (containing final LOP5 model)

NNVC-12rc1 was tagged February 5th, 2025 (containing new intermediate LOP5 model)

NNVC-11.0 was tagged December 10th, 2024 (bit accurate with 11rc)

NNVC-11.0rc was tagged December 3rd, 2024

NNVC-10.0 was tagged August 9th, 2024

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

- CTC performance

CTC performance are not changed compared to NNVC-14. The changed configurations are DRF and ALOP. Some unnecessary processing have been removed from VTM mode, hence the decoding complexity of NNVC is increase with respect to anchor (even if the absolute complexity is not changed).

Note: see configurations section for naming convention.

  - Comparison to VTM

    - NNVC-14.0 VTM vs NNVC-15.0 VTM

Note: NNVC-15 is now based on VTM-23.13.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.01% | 0.00% | 0.00% | 0.01% | 0.01% | 0.01% | 104% | 104% |
| Class A2 | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 103% | 102% |
| Class B | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 104% | 108% |
| Class C | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 104% | 114% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 104% | 108% |
| Class D | 0.02% | 0.02% | 0.02% | 0.03% | 0.03% | 0.03% | 103% | 108% |
| Class F | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 105% | 117% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | 103% | 104% |
| Class C | 0.01% | 0.01% | 0.01% | 0.02% | 0.01% | 0.01% | 102% | 108% |
| Class E | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 105% | 109% |
| Overall | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 103% | 106% |
| Class D | 0.02% | 0.02% | 0.02% | 0.03% | 0.03% | 0.03% | 102% | 108% |
| Class F | 0.01% | 0.01% | 0.01% | 0.02% | 0.02% | 0.01% | 104% | 110% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 104% | 109% |
| Class C | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 102% | 110% |
| Class E | 0.03% | 0.03% | 0.03% | 0.04% | 0.03% | 0.03% | 105% | 104% |
| Overall | 0.01% | 0.01% | 0.01% | 0.02% | 0.02% | 0.02% | 104% | 108% |
| Class D | 0.04% | 0.03% | 0.04% | 0.06% | 0.05% | 0.06% | 102% | 105% |
| Class F | 0.01% | 0.01% | 0.01% | 0.02% | 0.01% | 0.02% | 105% | 115% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 110% | 106% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 108% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 108% | 110% |
| Class C | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 0.01% | 107% | 120% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 108% | 117% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 108% | 112% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 106% | 109% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 104% | 117% |

Notes: results by InterDigital, crosschecked by xxx.

Small changes are due to additional flags in Slice header for hybrid mode.

    - NNVC-15.0 VTM vs NNVC-15.0 anchor

Anchor: NNVC in VTM mode.

Test: NNLF LOP6 + NNIP.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.98% | -13.88% | -15.28% | -8.83% | -16.00% | -17.48% | 117% | 3833% |
| Class A2 | -8.31% | -14.98% | -9.57% | -8.43% | -15.43% | -9.57% | 116% | 3805% |
| Class B | -7.71% | -15.96% | -14.70% | -7.81% | -18.20% | -17.56% | 117% | 4023% |
| Class C | -7.43% | -14.34% | -13.72% | -7.62% | -14.96% | -15.20% | 114% | 3860% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.01% | -14.92% | -13.53% | -8.09% | -16.34% | -15.32% | 116% | 3897% |
| Class D | -7.94% | -13.67% | -12.50% | -6.81% | -15.31% | -13.82% | 108% | 3638% |
| Class F | -4.36% | -8.60% | -7.33% | -5.40% | -11.13% | -11.46% | 122% | 2124% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -5.90% | -10.85% | -10.00% | -6.14% | -10.75% | -12.09% | 109% | 4293% |
| Class C | -6.04% | -11.93% | -9.96% | -6.74% | -10.61% | -8.79% | 104% | 3963% |
| Class E | -6.44% | -4.83% | -8.39% | -7.63% | -4.53% | -6.20% | 116% | 4499% |
| Overall | -6.08% | -9.71% | -9.58% | -6.71% | -9.15% | -9.52% | 109% | 4229% |
| Class D | -6.80% | -10.02% | -6.71% | -6.34% | -6.01% | -3.24% | 101% | 3647% |
| Class F | -3.60% | -6.11% | -5.16% | -5.57% | -7.07% | -8.52% | 112% | 2485% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -6.48% | -13.30% | -12.04% | -6.52% | -14.04% | -14.98% | 114% | 4073% |
| Class C | -6.45% | -14.40% | -12.76% | -6.93% | -14.41% | -13.72% | 106% | 3821% |
| Class E | -7.37% | -6.53% | -8.71% | -8.49% | -7.24% | -8.27% | 121% | 4938% |
| Overall | -6.69% | -11.97% | -11.45% | -7.15% | -12.46% | -12.88% | 113% | 4184% |
| Class D | -7.33% | -11.60% | -9.55% | -6.41% | -8.52% | -8.45% | 101% | 3777% |
| Class F | -3.86% | -7.61% | -5.46% | -5.49% | -10.40% | -11.80% | 116% | 2473% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | All Intra Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.32% | -15.82% | -17.29% | -9.08% | -19.30% | -19.74% | 165% | 2963% |
| Class A2 | -7.74% | -15.36% | -11.33% | -8.26% | -16.09% | -11.47% | 163% | 2465% |
| Class B | -7.67% | -15.11% | -15.80% | -7.80% | -17.73% | -18.38% | 162% | 2400% |
| Class C | -7.79% | -13.37% | -13.30% | -7.99% | -16.94% | -17.03% | 152% | 1824% |
| Class E | -11.31% | -16.94% | -18.34% | -11.53% | -17.51% | -19.77% | 159% | 2681% |
| Overall | -8.59% | -15.19% | -15.17% | -8.75% | -17.51% | -17.39% | 160% | 2393% |
| Class D | -7.67% | -11.57% | -12.23% | -7.10% | -16.06% | -16.68% | 145% | 1641% |
| Class F | -5.54% | -9.46% | -8.35% | -5.99% | -12.65% | -12.64% | 134% | 1993% |

Note: Results from Interdigital, crosschecked by xxx.

    - NNVC-15.0 VTM mode vs NNVC-15.0 HOP.5

Same results as NNVC-14.0.

    - NNVC-15.0 VTM mode vs NNVC-15.0 VLOP

Same results as NNVC-14.0.

    - NNVC-15.0 VTM mode vs NNVC-15.0 CALOP

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-v15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -11.26% | -16.21% | -21.41% | -13.16% | -19.85% | -23.14% | 238% | 3999% |
| Class A2 | -8.48% | -17.96% | -15.21% | -8.01% | -19.34% | -15.74% | 229% | 3843% |
| Class B | -8.04% | -21.88% | -19.38% | -6.68% | -24.12% | -21.60% | 239% | 3851% |
| Class C | -6.92% | -14.54% | -14.11% | -7.08% | -16.74% | -16.42% | 235% | 3479% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -8.47% | -18.01% | -17.55% | -8.35% | -20.34% | -19.35% | 236% | 3775% |
| Class D | -6.92% | -13.18% | -12.39% | -6.64% | -14.74% | -14.09% | 227% | 2810% |
| Class F | -4.07% | -9.47% | -8.71% | -4.90% | -11.80% | -12.52% | 248% | 2080% |

Note: Results from Nokia, crosschecked by xxx.

    - NNVC-15.0 VTM mode vs NNVC-15.0 CAVLOP

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-15 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.30% | -7.24% | -8.63% | -11.19% | -9.02% | -10.52% | 232% | 1846% |
| Class A2 | -6.71% | -8.97% | -6.12% | -6.88% | -11.02% | -6.30% | 225% | 1779% |
| Class B | -6.72% | -10.17% | -7.65% | -6.05% | -13.16% | -9.50% | 234% | 1793% |
| Class C | -5.34% | -7.07% | -6.21% | -5.90% | -8.95% | -7.64% | 233% | 1716% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -6.87% | -8.52% | -7.16% | -7.20% | -10.78% | -8.57% | 232% | 1780% |
| Class D | -5.13% | -5.93% | -5.23% | -5.52% | -7.79% | -7.43% | 227% | 1442% |
| Class F | -3.18% | -3.92% | -3.98% | -4.16% | -5.16% | -5.73% | 241% | 1033% |

Note: Results from Nokia, crosschecked by xxx.

  - Comparison to NNVC-15 anchor

    - NNVC-14 anchor vs NNVC-15.0 anchor

Identical results.

    - NNVC-15 anchor vs NNVC-15.0 VLOP

Same as NNVC-14.0.

    - NNVC-15 anchor vs NNVC-15.0 HOP

Same as NNVC-14.0.

    - NNVC-15.0 vs NNVC-15.0 RPR

Same as NNVC-14.0.

    - NNVC-15 RPR anchor vs NNVC-15.0 NNSR

Same as NNVC-14.0.

    - NNVC-15 anchor vs NNVC-15.0 DRF

Anchor: NNLF LOP6 + NNIP

Test: NNLF LOP6 + NNIP + DRF

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-15 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.40% | -0.61% | -1.97% | -1.65% | -1.81% | -2.26% | 152% | 2261% |
| Class A2 | -2.48% | -1.14% | -2.24% | -3.05% | -2.63% | -2.99% | 142% | 2094% |
| Class B | -1.79% | -1.36% | -1.60% | -2.29% | -2.43% | -2.35% | 148% | 2374% |
| Class C | -3.05% | -2.46% | -3.06% | -4.20% | -4.19% | -4.59% | 131% | 2141% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.19% | -1.46% | -2.19% | -2.82% | -2.82% | -3.06% | 143% | 2231% |
| Class D | -3.18% | -0.56% | -1.59% | -3.74% | -3.36% | -3.79% | 128% | 2030% |
| Class F | -0.99% | -0.68% | -0.38% | -1.56% | -1.39% | -1.27% | 179% | 6365% |

Note: Results from InterDigital, crosschecked by xxx.

- Contributions

We have 9 contributions for AhG14 and 1 telco report.

| JVET-AO0041 | [AHG11] [AHG14] Teleconference on NNVC | E. Alshina, F. Galpin |
| --- | --- | --- |
| JVET-AO0055 | AHG14: Dynamic convolution for LOP6 neural in-loop filtering | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0056 | AHG14: GlobalAveragePool layer for SADL | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0057 | AHG14: SADL viewer module using Netron | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0058 | AHG14: Bug fix for Conv2D sampling grid misalignment in SADL | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0073 | AHG14: Decoding time reduction in SADL using SIMD acceleration | J. Han, C. Jung, Q. Qin (Xidian Univ.) |
| JVET-AO0100 | AhG14: SIMD Improvements of Operators in SADL Library | L. Qin, N. Fu, Z. Chen (Wuhan Univ.) |
| JVET-AO0127 | AHG11/AHG14: Improving Training Reproducibility in NNVC | H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI), T. Yang, W.-X. He, Q. Liu (HUST), J. Chi, Ce Zhu (UESTC), L. Luo (CQUPT) |
| JVET-AO0217 | AhG14: Improved SADL implementation | D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft) |
| JVET-AO0260 | AhG14: SADL update | F. Galpin (Interdigital) |

- Configurations

The following configurations are used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration |
| --- | --- | --- |
| NNVC VTM mode | None | encoder_xxx_vtm.cfg |
| NNVC Anchor/EE1 | Intra Pred + LOP.6 | encoder_xxx_nnvc.cfg |
| NNVC. HOP | Intra Pred + HOP.5 | encoder_xxx_nnvc.cfg + nn-based/HOP5.cfg |
| NNVC. VLOP | Intra Pred + VLOP.4 | encoder_xxx_vtm.cfg + nn-based/vlop4.cfg |
| NNSR | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| RPR | Anchor RPR | nn-based/rpr.cfg + nn-based/nnsr_classA_multiratio.cfg |
| NNPF | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |
| CALOP | Intra pred+adaptive LOP | encoder_randomaccess_nnvc.cfg+nn-based/intra.cfg+nn-based/calop.cfg |
| NNVC DRF | NNVC anchor + DRF | encoder_xxx_nnvc.cfg + drf.cfg |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Keep synchronized with VTM upstream

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.

JVET-AO0015 JVET AHG report: Gaming content compression (AHG15) [S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs)]

- Software repositories

Software developed by AHG15 consists of forks of VTM/ECM that have support for reading auxiliary data that is provided for sequences of classes G1/G3. A third repository contains scripts for the verification of the auxiliary information. The repositories can be found at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gcc.

- Very short telco recap

Detailed telco notes can be found in JVET-AO0043.

Recommendations from telco:

- Regarding CfP

  - It was recommended to have Wukong 1-3 in low-delay setting for CfP

  - It was recommended to have sequences with auxiliary in the CfP test set.

  - There was interest to have sequences that have interface elements (such as menues, bars, etc.) in the CfP.

- Regarding gaming CTC

  - Results according to gaming CTC should be brought to next meeting (for Wukong). Huawei is running tests, ECM still needs a few days

  - It was suggested to provide reduced runtime setting of VTM for gaming CTC sequences

- AhG15 should work on text for a call for 4K new gaming sequences (for improvement of gaming CTC). No work done yet.

- Work on software for estimating depth maps for other gaming sequences (without auxiliary data) was reported. Software should be added to AhG15 repository.

- Test results

Test results were obtained according to JVET-AJ2027. The list of sequences/classes is copied for convenience below:

| Class | Sequence name | Frame count | Low delay frame count | Frame rate | Bit depth | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1 (SDR) | Level1_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | Darktree_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | ARPG2_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | DesertTown3_SDR | 600 | 300 | 60 | 10 | M | M | M |
|  | SunTemple3_SDR | 600 | 300 | 60 | 10 | M | M | M |
| G2 | GTAV | 600 | 300 | 60 | 8 | M | M | M |
|  | Minecraft | 600 | 300 | 60 | 8 | M | M | M |
|  | JianlingTemple | 600 | 300 | 60 | 8 | M | M | M |
|  | BaoleiYard | 300 | - | 60 | 8 | M | M | - |
| G3 (HDR) | Level1_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | Darktree_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | ARPG2_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | DesertTown3_HDR | 600 | 300 | 60 | 10 | O | O | O |
|  | SunTemple3_HDR | 600 | 300 | 60 | 10 | O | O | O |

For class G3 the HDR versions of Level1 and Darktree are still missing.

  - VTM-11.0ecm19.0 vs ECM-19.0

    - SDR

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -12.0% | -24.7% | -19.5% | 1007% | 649% | #DIV/0! |
| Class G2 | -19.0% | -23.6% | -27.7% | 1022% | 736% | #DIV/0! |
| Overall | -15.1% | -24.2% | -23.1% | 1014% | 686% | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -28.1% | -46.4% | -37.6% | 918% | 1858% | #DIV/0! |
| Class G2 | -25.5% | -37.3% | -38.3% | 1002% | #NUM! | #DIV/0! |
| Overall | -26.9% | -42.3% | -37.9% | 954% | #NUM! | #DIV/0! |

* ECM decoder crashed for BaoleiYard in RA.

|  | Low delay B Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -21.3% | -55.4% | -44.3% | 958% | 1709% | #DIV/0! |
| Class G2 | -18.9% | -50.8% | -49.6% | 1049% | 1598% | #DIV/0! |
| Overall | -20.4% | -53.7% | -46.3% | 991% | 1667% | #DIV/0! |

|  | Low delay P Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class G1 | -19.7% | -60.7% | -51.5% | 854% | 1447% | #DIV/0! |
| Class G2 | -16.7% | -57.6% | -57.0% | 959% | 1475% | #DIV/0! |
| Overall | -18.6% | -59.6% | -53.6% | 892% | 1457% | #DIV/0! |

    - HDR

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-11ecm19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -36.7% | -17.3% | -15.7% | -57.7% | -55.9% |  |  |  | 1246% | 685% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -54.4% | -33.6% | -34.4% | -71.9% | -71.0% |  |  |  | 839% | 1424% |
|  | Low Delay B |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -50.7% | -27.9% | -25.9% | -75.2% | -73.1% |  |  |  | 840% | 1645% |
|  | Low Delay P |  |  |  |  |  |  |  |  |  |
|  | Over VTM-11ecm19.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class G3 | -50.8% | -27.0% | -25.4% | -75.4% | -72.0% |  |  |  | 867% | 1353% |

  - Different runtime configurations of VTM

Results were obtained using gaming CTC settings with the difference that VTM-23.13 has been used instead of VTM-11ecm19. Below only overall averages are shown. Full details are in the provided spreadsheets.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-23.13 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| reduced runtime 1 | 0.0% | 0.0% | 0.0% | 99% | 95% | #DIV/0! |
| reduced runtime 2 | 2.9% | 0.2% | 0.0% | 37% | 96% | #DIV/0! |
| reduced runtime 3 | 3.3% | 4.2% | 4.2% | 32% | 98% | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-23.13 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| reduced runtime 1 | 1.9% | 2.8% | 3.0% | 59% | 94% | #DIV/0! |
| reduced runtime 2 | 4.7% | 4.4% | 4.4% | 40% | 97% | #DIV/0! |
| reduced runtime 3 | 13.2% | 16.4% | 16.9% | 18% | 93% | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |
|  | Over VTM-23.13 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| reduced runtime 1 | 1.3% | 2.7% | 2.8% | 62% | 102% | #DIV/0! |
| reduced runtime 2 | 3.9% | 4.3% | 4.3% | 42% | 101% | #DIV/0! |
| reduced runtime 3 | 10.7% | 16.5% | 17.3% | 20% | 91% | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |  |
|  | Over VTM-23.13 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| reduced runtime 1 | 1.3% | 3.0% | 3.0% | 59% | 103% | #DIV/0! |
| reduced runtime 2 | 3.9% | 3.9% | 3.8% | 42% | 99% | #DIV/0! |
| reduced runtime 3 | 10.3% | 16.2% | 17.0% | 22% | 92% | #DIV/0! |

- Input contributions

Two contributions listed below have been identified relating to the mandates of AHG15.

| Report |  |  |
| --- | --- | --- |
| JVET-AO0151 | [AhG15] Generated Depth Map Pictures for test sequence in gaming category | V. Zakharchenko (Nokia) |
| JVET-AO0207 | AHG15: Using Wukong sequences in gaming CTC | J. Sauer, Y. Zhao, Y. Sun, E. Alshina (Huawei) |

- Recommendations

The AHG recommends to:

- Review input documents on gaming content compression

- Identify if/how auxiliary information can be used for coding of gaming content

JVET-AO0016 JVET AHG report: Generative face video compression (AHG16) [Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs)]

- Activities

Regarding the mandate on developing and maintaining the GFVC software, the AHG16 GFVC software tool and accompanying usage instructions and exemplar configurations for experimentation are maintained in the GIT repository at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc. During this AHG period, the color calibration post-processing module from JVET-AN0057 has been integrated into the AHG16 software. Merge requests to add support for GFV and GFVE SEI messages to HEVC and AVC (as in JVET-AL0148) were submitted to HM and JM software before the 39th meeting, and are currently pending integration.

Regarding the mandate on studying the GFVC performance on additional test content, one contribution (JVET-AO0051) was received at this meeting. JVET-AO0051 reports the performance of coding human video using the GFVC framework, demonstrating that GFVC can outperform VVC on human video sequences using the GFV SEI message and showcasing the broader applicability of the GFVC framework and the GFV SEI message.

- Related contributions

The following input contribution to this meeting is related to the activities of AHG16:

- JVET-AO0051, AHG9/AHG16: Feasibility Study and Showcase on Generative Human Video Coding based on GFV SEI Message [B. Chen, R.-L. Liao, J. Chen, Y. Ye (Alibaba), Z. Zhang, S. Yin, X. Ju, S. Wang (CityUHK)]

- Recommendations

The AHG recommends to:

- Review the related contribution,

- Discontinue this AHG.

Revisit: Is it useful to generate a permanent repository (instead of AHG branches that may be forgotten) plus documentation for software parts that do extraction of SEI message parameters directly from video, and perform corresponding synthesis. Examples would be NNPF, FGC, GFV, …

JVET-AO0017 JVET AHG report: Preparation of Call for Proposals (AHG17) [J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs)]

- Activities

Two online AHG calls were held on 2025-11-25 and 2025-12-16. The reports of these meetings are available in JVET-AO0041.

Based on the discussions at these meetings, the proposed draft CfP document JVET-AO0045 was generated as input to the 41st meeting, departing from the CfE document by reflecting proposed modifications as discussed during the 40th JVET meeting and in the AHG telcos.

- Related contributions

| JVET number | Title | Source |
| --- | --- | --- |
| JVET-AO0017 | JVET AHG report: Preparation of Call for Proposals (AHG17) | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs) |
| JVET-AO0044 | AHG17: AhG meeting notes | M. Wien, J. Ohm, F. Bossen |
| JVET-AO0045 | [AHG17] Draft Joint Call for Proposals on video compression with capability beyond VVC (initial text proposal from AHG) | J. Ohm, M. Wien, F. Bossen |
| JVET-AO0052 | AHG17/AHG18: Suggested CfP text for ultra-low latency and packet loss resilience category | S. Ikonin, X. Ma, E. Alshina (Huawei) |
| JVET-AO0054 | [AHG17] Proposal of new sequences for the HD-SDR-RA category of the CfP | M. Abdoli, A. Tissier, R. G. Youvalari, P. Nikitin, F. Plowman, G. Shuo (Xiaomi) |
| JVET-AO0061 | AHG4/AHG17: AIGC test sequences | J. Liu, X. Chen, Y. Gao, Z. Chen (Wuhan Univ.), Y. Pan, T. Yao, T. Mei (HiDream.ai) |
| JVET-AO0072 | AHG17/AHG18: On ULL evaluation metrics toward CfP | T. Chujoh, Y. Kidani, K. Kawamura (KDDI) |
| JVET-AO0130 | [AHG17] Suggestions on Draft CfP, CTCs and complexity reporting | F. Cricri, N. Le, L. Murn, M. Pedzisz, D. Buğdayci Sansli, N. Zou, C. Feldmann, H.R. Tavakoli, M. Santamaria, H. Zhang, J. Lainema, M. M. Hannuksela (Nokia) |
| JVET-AO0135 | [AHG17] Comments on draft CfP | E. François, F. Bossen, F. Le Leannec, F. Galpin, K. Naser, T. Poirier (InterDigital) |
| JVET-AO0184 | AHG17: Reporting on HDR coding metrics | W. Husak, T. Nguyen Canh, P. Yin (Dolby), D. Rusanovskyy (Nokia) |
| JVET-AO0226 | AHG17: Evaluation of CfE Sequences Considering Sequence Characteristics | R. Ishimoto, F. Zheming, T. Ikai (Sharp) |
| JVET-AO0248 | AHG4/AHG17: Response to the call for new HDR materials for future video coding development | J. Wang, J. Zhang, L. Yu |

It is noted that multiple documents have partial allocation to AHG17 and other AHGs. This includes JVET-AO0045, JVET-AO0052, JVET-AO0061, JVET-AO0072, and JVET-AO0248.

- Recommendations

The AHG recommends:

- To further progress the development of the draft CfP document.

- To study the proposed new test sequences as potential candidates for inclusion in the CfP.

- To review the remaining input contributions related to AHG17 during the meeting.

- To hold an AHG17 meeting in Aachen, DE, during 25-27 February 2026, for further refinement of the draft CfP, test conditions, and submission procedures.

JVET-AO0018 JVET AHG report: Ultra-low latency and packet loss resilience (AHG18) [S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs)]

- Activities

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). Few organizational e-mails were sent to reflector.

  - Teleconference

The teleconference was held on December 17th with 31 participants attending.

The following topics were discussed:

- ULL CTC software and excel template introduction according to JVET-AN2039 ULL CTC, JVET-AO0047

- ULL CfE response software introduction, JVET-AO0048

- CfP discussion

The meeting minutes are available in the document JVET-AO0049.

  - Simulation software

Adoption of last JVET meeting (JVET-AM0195, GDR fixes) has been merged to ‘ull-master’ branch: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM/-/tree/ull-master.

- Related contributions

A total of 8 contributions are identified relating to the mandates of AHG18. They are listed below. Some contributions also relate to the work of AHG17.

| Report |  |  |
| --- | --- | --- |
| JVET-AO0049 | AHG18: Teleconference on ultra-low latency and packet loss resilience | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) |
| Test condition and evaluation methodology proposals |  |  |
| JVET-AO0047 | AHG18: Simulation software for ULL CTC JVET-AN2039 | V. Khamidullin, K. Malyshev, I. Gribushin, M. Sychev, S. Ikonin, E. Alshina (Huawei) |
| JVET-AO0052 | AHG17/AHG18: Suggested CfP text for ultra-low latency and packet loss resilience category | S. Ikonin, X. Ma, E. Alshina (Huawei) |
| JVET-AO0072 | AHG17/AHG18: On ULL evaluation metrics toward CfP | T. Chujoh, Y. Kidani, K. Kawamura (KDDI) |
| Other proposals |  |  |
| JVET-AO0048 | AHG18: Description of the software related to ULL CfE response JVET-AN0080 | V. Khamidullin, K. Malyshev, A. Dzugaev, I. Gribushin, M. Sychev, X. Ma, S. Ikonin, E. Alshina (Huawei) |
| JVET-AO0123 | AHG18: Standalone network simulation application on top of ULL software | K. Malyshev, I. Gribushin, V. Khamidullin, M. Sychev, S. Ikonin, E. Alshina (Huawei) |
| JVET-AO0208 | AHG18: GDR functionality with LowDelay-B configuration | A. Dzugaev, M. Sychev, K. Malyshev, S. Ikonin, E. Alshina (Huawei) |
| Crosscheck |  |  |
| JVET-AO0205 | [AhG18] Crosscheck report for JVET-AO0047 | V. Zakharchenko (Nokia) |

- Recommendations

The AHG recommends:

- Review all input contributions.

- Discuss test conditions and evaluation methodology, including CfP related suggestions.

- Collect test cases and specific requirements.

- Continue development of simulation software.

- Continue the study of ultra-low latency and packet loss resilience technologies in JVET.

JVET-AO0019 JVET AHG report: Hardware implementation complexity (AHG19) [Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs)]

- Activities

One AHG19 teleconference was held on Dec. 17, 2025. A tutorial on VVC hardware implementation was given by VITEC, followed by some discussions on hardware complexity metrics. Around 100 participants attended this meeting. Report of this teleconference can be found in JVET-AO0046.

AHG19 also initialized a questionnaire on VVC hardware encoder implementation challenges. It was posted to the JVET reflector on Dec. 15, 2025, and an updated v2 version was re-posted on Dec. 19, 2025 (also attached in JVET-AO0046). Responses to this questionnaire are expected at 41th and 42th JVET meetings (deadline Apr. 10, 2026). So far, one response has been received.

- Input contributions

Contributions listed below have been identified relating to the mandates of AHG19.

| JVET-AO0046 | [AHG19] Teleconference on hardware implementation complexity | Y. Zhao |
| --- | --- | --- |
| JVET-AO0053 | [AHG19] Response to the questionnaire on VVC hardware encoder implementation challenges | M. Wu, J. Yu (HiSilicon), Y. Zhao, E. Alshina (Huawei) |
| JVET-AO0060 | AHG19: Introduction to hardware video encoder complexity | P. Wetzel (VITEC) |
| JVET-AO0075 | [AHG19] Per-CU max RDO number and weighted sum of RDO number | Y. Zhao, E. Alshina (Huawei), K. Naser, F. Le Léannec (InterDigital), Y. Tokumo, T. Ikai (Sharp) |
| JVET-AO0137 | [AHG19] Analysis of the HEVC encoder toolset in commercial hardware applications | M. Porto, D. Palomino, L. Agostini, G. Corrêa, B. Zatt, A. Schuch, V. Costa (ViTech - UFPel) |
| JVET-AO0249 | AhG19 AhG11 Interdigital response to the MPEG Survey on AI reproducibility and NN model hosting | F. Galpin, R. James, D. Tian (Interdigital) |

- Recommendations

The AHG recommends to:

- Review related input documents

# Project development (34)

## AHG1: Deployment and advertisement of standards (2)

Contributions in this area were discussed during 1455–1510 UTC on Monday 19 Jan. 2026 (chaired by JRO).

JVET-AO0020 Deployment status of the HEVC standard [G. J. Sullivan]

This information contribution contains a survey of deployed products and services using the HEVC standard and the formal specifications in which it is supported, along with a brief introduction to the standard written for broad readership. Revision marking is included to show changes relative to JVET-AL0020-v2 of July 2025.

As of January 7, 2026, the reported support in browsers had further risen to 94% for “all users” (97% for “all tracked”, 96% for “tracked desktop”, and 97% for “tracked mobile”), as further detailed below:§

  - 94% for “all users”: 15% “Supported”, 79% “Partially Supported”

  - 97% for “all tracked”: 16% “Supported”, 81% “Partially Supported”

  - 96% for “tracked desktop”: 4% “Supported”, 92% “Partially Supported”

  - 97% “tracked mobile”: 24% “Supported”, 74% “Partially Supported”

Bitmovin reported the following statistics in September 2025 from a video developer survey of July 2025 with an undisclosed number of responding “video developers and industry experts” from 59 countries:

- 41% of developers “currently using” HEVC in production

- 15% of developers “planning to implement” HEVC within 12 months

JVET-AO0021 Deployment status of the VVC standard [G. J. Sullivan]

This contribution contains a survey of deployed products and services, publicly available software source code, related tools, and formal specifications supporting the VVC standard (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Changes relative to JVET-AN0021-v1 of October 2025:

Bitmovin reported the following statistics in September 2025 from a video developer survey of July 2025 with an undisclosed number of responding “video developers and industry experts” from 59 countries [20]:

  - 6% of developers “currently using” VVC in production

  - 8% of developers “planning to implement” VVC within 12 months

FFmpeg version 8.0 “Huffman” in August 2025 included support for a video acceleration API and some other features that had not been fully supported previously, including intra block copy (previously reported above as included in version 7.1), palette mode (a feature only in 4:4:4 profiles), and the adaptive colour transform (another feature only in 4:4:4 profiles).

Felenasoft’s Xeomi video surveillance software added support for VVC in version 24.9.24 in September 2024 (unknown whether this is only decoding or also encoding capability).

Huawei released its Mate 70 Pro HarmonyOS phone in October 2025 with VVC hardware decoding capability [111].

Ali 266 encoder .. added in v3 of JVET-AO0021

## AHG2: Text development and errata reporting (0)

This section is kept as a template for future use.

## AHG3: Software development (2)

Contributions in this area were discussed during 1520–1535 UTC on Wednesday 21 Jan. 2026 and during 0000–0030 UTC on Thursday 22 Jan. 2026 (chaired by JRO).

JVET-AO0136 AHG3/AHG10: On the GOP-based-RPR decision source [A. Wieckowski (HHI)]

VTM contains an encoder algorithm allowing to signal-adaptively change the encoded picture resolution (GOPbasedRPR). The encoding resolution decision is performed once per GOP and is based on the first picture of each GOP. VTM software also contains a temporal pre-filter, which is applied in the default motion compensated configurations. In the original implementation of GOPbasedRPR the resolution decision was based on the temporally pre-filtered version of the first picture of a GOP. One of the changes in VTM-22.1 was a picture buffer refactoring ported from ECM. A side-effect of that refactoring was a change of the GOPbasedRPR decision source from the temporally pre-filtered to the true original picture. The side effect was deemed unintended and fixed after being noticed. This contribution reports the effects of that change.

Effect of the picture buffer refactoring from VTM-22.1 on GOPbasedRPR for the QP range 32-47, random access conditions.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over bfd56244 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 2,31% | -0,19% | 1,34% | 115,6% | 138,3% | 74,8% | 125,6% |
| Class A2 | 0,78% | 0,83% | -0,78% | 110,1% | 126,7% | 71,7% | 115,0% |
| Class B | -0,01% | 0,23% | 0,39% | 101,2% | 105,5% | 73,8% | 99,0% |
| Class C | 0,00% | 0,00% | 0,00% | 99,4% | 99,1% | 84,5% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0,61% | 0,21% | 0,24% | 105,2% | 113,6% | 76,3% | 107,3% |
| Class D | 0,00% | 0,02% | 0,01% | 99,4% | 98,2% | 93,3% | 100,0% |
| Class F | 0,00% | 0,00% | 0,00% | 99,5% | 98,4% | 91,4% | 100,0% |
| Class TGM | 0,00% | 0,00% | 0,00% | 100,1% | 99,5% | 90,5% | 100,0% |

Effect of reverting the GOPbasedRPR decision back to being based on the temporally pre-filtered picture for the QP range 32-47, random access conditions.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over 7a06fbc3 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -2,14% | 0,20% | -1,18% | 87,7% | 73,1% | 82,7% | 74,8% |
| Class A2 | -0,50% | -0,59% | 0,93% | 91,7% | 79,3% | 85,3% | 88,1% |
| Class B | 0,04% | -0,06% | -0,38% | 98,9% | 95,6% | 96,8% | 100,5% |
| Class C | 0,00% | 0,00% | 0,00% | 100,1% | 101,8% | 97,8% | 101,6% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,51% | -0,10% | -0,18% | 95,4% | 88,8% | 91,7% | 92,6% |
| Class D | 0,00% | -0,01% | 0,00% | 100,2% | 101,5% | 98,8% | 100,1% |
| Class F | 0,00% | 0,00% | 0,00% | 100,0% | 101,1% | 99,9% | 100,4% |
| Class TGM | 0,00% | 0,00% | 0,00% | 100,0% | 101,3% | 99,9% | 102,1% |

Effect of the picture buffer refactoring from VTM-22.1 on GOPbasedRPR for the QP range 32-47, low-delay B conditions.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over bfd56244 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0,08% | -0,03% | 0,43% | 101,0% | 100,3% | 118,0% | 99,4% |
| Class C | 0,00% | 0,00% | 0,00% | 100,0% | 97,7% | 131,0% | 97,7% |
| Class E | 0,00% | 0,00% | 0,00% | 99,9% | 95,4% | 85,2% | 100,0% |
| Overall | 0,03% | -0,01% | 0,18% | 100,4% | 98,2% | 112,6% | 99,0% |
| Class D | 0,00% | 0,00% | 0,00% | 100,1% | 98,5% | 96,8% | 100,0% |
| Class F | 0,00% | 0,00% | 0,00% | 99,1% | 95,9% | 91,5% | 100,0% |
| Class TGM | 0,00% | 0,00% | 0,00% | 98,5% | 97,5% | 70,0% | 100,0% |

Effect of reverting the GOPbasedRPR decision back to being based on the temporally pre-filtered picture for the QP range 32-47, low-delay B conditions.

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over 7a06fbc3 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,08% | -0,15% | -0,18% | 99,3% | 99,7% | 100,6% | 99,8% |
| Class C | 0,00% | 0,00% | 0,00% | 99,6% | 100,7% | 99,3% | 100,0% |
| Class E | 0,00% | 0,00% | 0,00% | 99,5% | 100,3% | 98,9% | 100,3% |
| Overall | -0,03% | -0,06% | -0,08% | 99,4% | 100,2% | 99,7% | 100,0% |
| Class D | 0,00% | 0,00% | 0,00% | 100,3% | 104,1% | 99,7% | 99,0% |
| Class F | 0,00% | 0,00% | 0,00% | 100,1% | 100,6% | 100,1% | 96,5% |
| Class TGM | 0,00% | 0,00% | 0,00% | 99,6% | 100,3% | 108,3% | 97,1% |

Agreed that the reversion to the original VTM is desirable, even more as it is planned to generate additional VTM encodings for CfP that are using the RPR tool.

As soon as the NNVC software would be aligned with the newest VTM, this should also be resolved. It might however need to be checked whether the model for NNSR was based on non-filtered pictures, such that it might be suboptimum in comparison against RPR.

JVET-AO0219 Crosscheck of JVET-AO0136 AHG3/AHG10: On the GOP-based-RPR decision source [X. Li (Google)] [late]

JVET-AO0266 Cross-check of JVET-AO0136 (AHG3/AHG10: On the GOP-based-RPR decision source) [K. Andersson (Ericsson)] [late]

JVET-AO0227 AHG5: Updates on Multiview Reference Software [E. Asbun, S. Choi, S. Paluri, A. M. Tourapis (Apple)] [late]

This contribution discusses the current status of the reference software implementation for the multiview profiles of the HEVC standard.

In an earlier contribution presented in the previous meeting, JVET-AN0292, it was reported that the HM reference software was extended to include support for all currently defined multiview profiles, including the multiview monochrome profiles, in both the encoder and the decoder barring limitations in terms of mixing, at the layer level, different chroma formats, bitdepths, and/or resolutions. While extending this software to generate conformance bitstreams required for 3GPP SA4 activities, it was observed that the encoder was not able to include VUI information in the VPS of the generated bitstream. This contribution provides a patch on top of the provided software to include such support. Additional minor fixes that were discovered during the generation of such bitstreams are also provided.

It was suggested to issue another merge request regarding the fixes.

It was suggested to investigate that CTC conditions of HM, in particular also for the range extensions, would still work OK after the merge. Needs to be clarified with K. Sühring which document describes the RExt – is JVET-T2013 already merged conditions for HM and VTM, or is it an old JCT-VC document not yet in the list of JVET documents. (this should have been JCTVC-xxx1006) Revisit.

The merge request should be issued immediately, such that the development of conformance streams could already be started before the software is officially merged into HM (which could take time).

It was suggested that this could also be first merged into a separate branch of HM.

It is also noted that the software also would give support for a hypothetical 444 multiview profile.

It is further expected that merging it into a separate branch would be temporary, as the software supporting the new multiview profiles should become part of the next editions of H.265.2 and ISO/IEC 23008-5.

## AHG3: Test conditions and metrics (0)

This section is kept as a template for future use.

JVET-AO0184 is related, requesting an update of HDR CTC in terms of metrics.

## AHG4: Subjective quality testing and verification testing (1)

Contributions in this area were discussed during XXXX–XXXX UTC on XXday X Jan. 2026 (chaired by XXX).

JVET-AO0215 AHG4: proposed updates for VVC multi-layer verification test plan [O. Chubach (MediaTek), P. de Lagrange (InterDigital), M. Wien (RWTH)]

TBP

## AHG4: Test and training material (0)

This section is kept as a template for future use.

JVET-AO0054, JVET-AO0059, JVET-AO0061, and JVET-AO0248 are related.

## AHG5: Conformance test development (0)

This section is kept as a template for future use.

JVET-AO0227 is related.

## AHG7: ECM tool assessment (3)

Contributions in this area were discussed during 1520–1650 UTC on XXday X Jan. 2026 (chaired by JRO).

JVET-AO0042 AHG7: Report of AHG7 conference call on the Refinement of Complexity Reporting Template [X. Li]

No need for presentation – report from AHG telco.

JVET-AO0067 Module-Level Complexity Analysis of VVC [X. Li (Google), E. Alshina (Huawei), I. Moccagatta (Intel)]

In this contribution, a module-level complexity analysis of VVC is provided. The analysis is included in the attached Excel sheet.

It was commented that entropy coding complexity may need some more detail (worst case number of cycles per bin, dependencies (including dependency from decoded samples), etc.

It was suggested to add question for dependency between inter and intra prediction.

It was agreed that this is a good starting point for the missing annex (analysis for VVC) to be attached to the complexity reporting template JVET-AO2040.

Further discussion in BoG

JVET-AO0114 AHG7: Study on CU size normalized RDO count [Y. Tokumo, S. Hong, T. Ikai (Sharp)]

This contribution proposes RDO count metrics based on comparing CU size normalized RDO count and no weighting based RDO count. The experiment confirms that no weighting based RDO count method as in JVET-AN0118 has more linearity between EncT and RDO count than the method w/ normalization by CU size. In terms of source code modification, JVET-AN0118 is more compact. It is recommended to adopt the JVET-AN0118 to the next version of VTM and ECM software.

|  | Method | Result | Correlation coefficient (ECM CTC) |
| --- | --- | --- | --- |
| AN0118 | No weighting based RDO count | Linearly correlated | 0.980 |
| AO0114 | CU size normalized RDO count | Not linearly correlated | 0.905 |

According to these results, number of RDO has a high influence on encoding time, but using the absolute number with weighting (as per JVET-AN0118) appears better than the normalization by CU size.

It was commented that JVET-AO0075 is related. See further notes there.

JVET-AO0131 AHG3/7: Reference software extension for instruction counting [J. Sauer, Y. Zhao, Y. Sun, E. Alshina (Huawei)]

This document reports findings on instruction counting using special CPU hardware registers. Similar results can be obtained as shown in JVET-AN0119 or JVET-AN0157, but without a runtime performance penalty. It is shown, that instruction count is more stable than measuring runtime. We propose that reporting instruction count should be allowed as an alternative to reporting runtimes or even to be used instead. Other hardware counters that may also be of interest to AHG19 are cache-misses and branch-misses.

It was claimed that one advantage compared to runtime measurement might be that the instruction count is independent of homogeneity of cluster, SIMD availability, etc.

When using virtual machines, it may not be possible to access the hardware registers.

It was also commented that admin privileges might be necessary.

Some concern was raised to completely replace runtime by instruction count, or make the choice optional. It might be useful as additional information.

It was asked if comparability would still be given when different compilers (even compiler updates) or different degree of compiler optimization was used. SIMD would also be counted as only one instruction.

It was commented that for completely different algorithms, this would not be a good basis for comparison.

Further study – using additional data might be useful in the context of future standard development.

Traditional tools for profiling can deliver the same (or even more detailed) information, but those would slow down the runtime.

## AHG8: Optimization of encoders and receiving systems for machine analysis of coded video content (1)

Contributions in this area were discussed during XXXX–XXXX UTC on XXday X Jan. 2026 (chaired by XXX).

JVET-AO0292 Suggested changes on Draft ISO/IEC TR 23888-3 [S. Liu, J. Chen, C. Hollmann, J. Ström (Editors)]

TBP

## AHG10: Encoding algorithm optimization (1)

Contributions in this area were discussed during 1535–1620 UTC on Wednesday 21 Jan. 2026 (chaired by JRO).

JVET-AO0136 is related.

JVET-AO0288 AHG10: Suggested update of VTM reference software based on JVET-AN0267  K. Andersson, P. Wennersten (Ericsson), X. Li (Google Inc) [late]

This document proposes to update the VVC reference software with modifications related to MCTF and block importance mapping (BIM) from JVET-AN0267. A draft MR related to the changes is available as MR2907.

The BDR results on CTC are as follows:

RA (Y/U/V): -2,17%/-2,08%/-2,14%	EncT: 99,9%, DecT: 99.8%

LDB (Y/U/V): -2,09%/1,05%/0,38%	EncT: 100,5%, DecT: 99.7%

The current VVC reference software is using a 6-tap filter for MCTF and includes block importance mapping (BIM = 1) that adjust the QP for CTUs based on how likely it is that they are used for reference. During the CfE some modifications were applied to MCTF and BIM in JVET-AN0267 [1] that are tested here and proposed to be included in the VVC reference software. To that end the MCTF is using the default motion compensation interpolation filter instead of a 6-tap filter, MCTF is derived on a block size of 16x16 instead of a block size of 8x8, and a new BIM mode (BIM = 2) is introduced that controls the lambda instead of modifying QP. BIM is further aligned with RPR to get a correspondence between BIM derived on a block in original resolution and the usage of BIM for a corresponding area when encoding in reduced resolution. BIM is furthermore continuous instead of step wise.

It was requested to update the contribution such that benefit of the different elements (MCTF filter size, MCTF block size, BIM) would become clear.

Decision(SW): Adopt JVET-AO0288 (MCTF and BIM changes), also include in JVET-AO2002.

CTC configuration would not be changing, BIM stays disabled

It was commented that enabling BIM is somewhat like hidden QP adaptation that might be undesirable in CTC.

It was suggested that also the effect on ECM and HM should be investigated.

NNVC will be aligned with the VTM change.

For CfP, a decision of anchor configuration is to be done at next meeting, and by how far the anchor would deviate from the approach of discouraging preprocessing.

JVET-AO0294 Crosscheck of JVET-AO0288 (AHG10: Suggested update of VTM reference software based on JVET-AN0267) [F. Bossen] [late] [miss]

## AHG13: Film grain technology (0)

This section is kept as a template for future use.

JVET-AO0064, JVET,AO0099, JVET-AO0102, JVET-AO0103, JVET-AO0104, and JVET-AO0105 are related.

## Implementation studies (1)

Contributions in this area were discussed during 1625–1655 UTC on Wednesday 21 Jan. 2026 (chaired by JRO).

JVET-AO0221 Recent optimizations of Ali266 for real-time VVC encoding on smartphones [L. Wang, J. Liang, K. Xie, S. Fang, X. Zhao, J. Li, J. Chen, Y. Ye (Alibaba)] [late]

This contribution presents an optimized Ali266 encoder that enables real-time Versatile Video Coding (VVC) encoding on smartphones. Through mobile-specific optimizations—including optimized selection of coding tools, ARM instruction-level acceleration, and a suite of fast algorithms—Ali266 achieves efficient VVC encoding directly on smartphone CPUs. We validated Ali266’s encoding speed and performance on the iPhone 16 Pro, a flagship smartphone released in 2024 that has since gained a large global user base. This advancement demonstrates that high-efficiency video compression can run locally on the modern smartphone platform, significantly expanding the accessibility of VVC technology especially for many user-generated content (UGC) scenarios. To our knowledge, this is the first announcement of real-time VVC software encoder on smartphones.

In v2 of this contribution, information about Ali266’s encoding speed on Xiaomi 17 (Android) was added. The results show that Ali266 also achieves real-time performance on this Android flagship smartphone, further confirming its effectiveness across diverse mobile platforms.

Realtime is possible for 1080p60fps, 10bit sequences are converted to 8bit, max bit rate was around 3 Mbps fo VVC.

BD rate gains versus the HEVC hardware encoder of Iphone are reported 44% on average.

It was commented that the HEVC hardware encoder might not have been optimized for PSNR. It might be good also to study different metrics.

Hardware accelerators were not used.

No detailed information was given on VVC tools that were used. Partitioning different from options of HEVC was used.

It was commented that energy consumption may be an issue. Was it possible to run the smartphone for longer, e.g. several minutes? This would be necessary for a real product.

It was emphasized that the usage of 8 bit was fundamental to allow this encoder implementation. It was commented that this can be done in the context of a 10 bit profile and might not require a separate profile.

## Profile/tier/level specification (0)

This section is kept as a template for future use.

## AHG15: Gaming content compression (2)

Contributions in this area were discussed during 1700–1745 UTC on Wednesday 21 Jan. 2026 (chaired by JRO).

JVET-AO0043 AHG15: Telco on gaming content compression [S. Puri. J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs)]

No need for presentation

JVET-AO0151 [AhG15] Generated Depth Map Pictures for test sequence in gaming category [V. Zakharchenko (Nokia)]

In scope of the activities of the AhG 15 in preparation for the Joint Call for Proposals on Video Compression with Capability beyond VVC some new sequences had been introduced for the gaming category evaluation. It has been reflected by several experts that additional information such as depth map pictures may improve functionality of the development of the future video coding technologies.

Some sequences introduced in JVET-AN0053 were mentioned to be useful for the test development, however it has been mentioned the depth map picture information not provided with the content as documented in JVET-AO0043.

This document provides description of the method as well as reference to the generated depth map information of the sequences identified in JVET-AO0043 that were identified for testing purposes of the gaming content coding.

It was reported that depth maps can also be made available for sequences where “true” depth maps exist, such that it could be compared how similar those are, and if the benefit on compression of the video sequence is similar, and also the impact on compressing the depth map might be interested to study.

It was commented that it would be desirable to also have camera parameters available.

It was asked if it would also be possible for camera captured sequences. Further study recommended on this.

It is not yet clear if the software could be made available. This should be added to the AHG15 software branch once permission would be granted by the owner.

It was agreed to include the generated depth maps in the repository of the content server, including description of interpretation, camera parameters per frame, also for those sequences where the “true” depth maps from rendering are available.

JVET-AO0207 AHG15: Using Wukong sequences in gaming CTC [J. Sauer, Y. Zhao, Y. Sun, E. Alshina (Huawei)]

In JVET-AN0053, several new 4K gaming sequences have been made available to JVET. After initial expert viewing they have been found to more interesting/challenging than the 4K sequence currently part of the CTC test set. In this contribution results for the new sequences according to the gaming CTC JVET-AJ2027 are provided. It is proposed to include the new sequences into the CTC.

It was commented that the Wukong sequences have relatively similar  characteristics, and in particular for CTC (also regarding future standardization) the set of sequences should be more diversified.

It was agreed (as an intermediate solution) to include two of the Wukong sequences into class G2, and keep Baolei yard. This would make that class consist out of 6 sequences (3x HD, 3x 4K) and gives class G2 a larger variety rather than adding a new class with lack of variety. When development of a new standard starts, CTC would need to be updated anyway, and it would be good if more variety of gaming material (in terms of content, rendering, motion  characteristics, etc.) would be available then.

Based on PSNR graphs in JVET-AO0207, Wukong3 (most challenging one), and Wukong 2 (somewhat medium challenging) were selected.

## AHG16: Generative face video (0)

This section is kept as a template for future use.

JVET-AO0051 is related.

## AHG17: CfP on video coding technology beyond VVC (12)

JVET-AO0044 AHG17: AhG meeting notes [M. Wien, J. Ohm, F. Bossen]

No need for presentation (summary of telcos)

### Proposed draft CfP text (5)

Contributions in this area were discussed during 1340–1705 UTC (with break) on Thursday 15 Jan. 2026 (chaired by JRO).

JVET-AO0045 [AHG17] Draft Joint Call for Proposals on video compression with capability beyond VVC (initial text proposal from AHG) [J. Ohm, M. Wien, F. Bossen]

Short presentation for overview was given.

JVET-AO0052 AHG17/AHG18: Suggested CfP text for ultra-low latency and packet loss resilience category [S. Ikonin, X. Ma, E. Alshina (Huawei)]

This contribution suggests a text for functionality test on ultra-low latency and packet loss resilience as specific category for future Call for Proposals. Based on contribution JVET-AN0204, JVET-AN2039 ULL CTC, CfE evaluation discussion,  and discussion of AHG18 teleconference summarised in JVET-AO0049 a marked-up version of JVET-AO0045 “Draft Joint Call for Proposals on video compression with capability beyond VVC (initial text)” covering ultra-low latency and packet loss resilience category is provided.

It was commented that it would be desirable to have more close rate matching, like in the other categories. It was argued that this test would be more about seeing differences in network-caused artifacts rather than compression artifacts.

It was commented that it may be questionable to base the test on specific network conditions (such as WiFi and 5G), as those are quickly changing in the real world.

It was asked if it would be possible to use the same anchors as in the other categories?

It was commented that it seems to be a high number of test cases, and UHD LD would be demanding in terms of encoding. However, not all of them would need to be tested visually.

It was commented that traditionally error resilience is handled on the network side (where some methods that are commonly applied are not used in the proposed test conditions.

It was commented that it might overburden the CfP to include extensive testing on this aspect. It might better be done during standard development.

Other comments were made that this could be a distinguishing feature of a new standard, and from this aspect would be important to be included.

A comment was made that under low latency condition, traditional methods such as external channel coding or retransmission fail, such that building error resilience into the source coding appears important.

Visual testing is important in this context, as no reasonable objective criteria are known.

Generally, diverging opinions about including it. It was suggested that an option might be to define it initially, and drop it when not sufficient interest is observed (e.g., will only be considered if at least X submissions are received).

It was asked to analyze the total number of test points that would need to be submitted per category with the proposed setup, and discuss in context of 4.17 review options to potentially reduce.

Follow-up discussion on Tuesday 20 2230-2310:

Questions raised:

- Testing only non-feedback case (would be simpler)? Would feedback require different tools?

- Mandatory to use the given network simulator? Is it representative, and if different ones would be allowed, how would proposals be comparable

- More than one QP point?

- What would be an appropriate anchor?

- How would different proposals be compared?

It was commented that it may be difficult to define conditions that would enable us to distinguish the benefit of tools in comparison to other aspects such as encoder optimization, rate control, error concealment, etc.

There was no objection that the topic is important, but various concerns were raised that the state is not mature enough to be included in the CfP. Would be a very interesting topic for investigation afterwards, in the collaborative phase, and potentially seeking collaboration with other bodies (network).

It was asked whether in the context of low latency error resilience is the most important aspect, or if it is also a matter of tool design.

No consensus to have this included in CfP.

JVET-AO0130 [AHG17] Suggestions on Draft CfP, CTCs and complexity reporting [F. Cricri, N. Le, L. Murn, M. Pedzisz, D. Buğdayci Sansli, N. Zou, C. Feldmann, H. R. Tavakoli, M. Santamaria, H. Zhang, J. Lainema, M. M. Hannuksela (Nokia)]

This document proposes several suggestions and modifications to the current draft Call for Proposals on Video Compression with Capability beyond VVC and other related documents. In particular, the following aspects are addressed: complexity evaluation, objective visual quality metrics, test sequences, training procedures and training data for learning-based coding tools, definition of a learning-based coding tool, and some other miscellaneous aspects.

Various suggestions for clarifications in the complexity reporting template JVET-AN2040, and on including additional objective perceptual metrics (e.g., VMAF, LPIPS) in the draft CfP, or in subsequent experimental investigation.

It was asked if a method was also proposed how to deal with multiple metrics, in particular if they are contradicting?

Further, ways to use additional “hidden” test sets are suggested, where “option C” would rather be relevant for the collaborative phase. It was commented that the concept of a “neutral entity” might not be practical.

Further the aspect of conducting training verification during CfP or thereafter is discussed in the contribution.

It was later clarified that the aspects of adding more perceptual metrics and extended training verification at this moment are not sufficiently clear to be included in the draft CfP.

JVET-AO0135 [AHG17] Comments on draft CfP [E. François, F. Bossen, F. Le Leannec, F. Galpin, K. Naser, T. Poirier (InterDigital)]

This contribution provides comments related to the draft CfP document (JVET-AO0045). Based on an analysis of the CfE results, sequences and bitrates changes are suggested. Additionally, the following points are discussed:

- Proposed refinement to section 4.1 of the draft CfP document related to quantization constraints.

- Request to keep 3 operation points (x0.2, x1 and x5 VTM encoding runtime) in the constrained category.

- Request to separately report in a summary table BD-rates with wPSNR for HDR PQ content and PSNR for HDR HLG content.

- Recommendation to jointly work on a commonly agreed objective YUV metric that should be reported in the CfP responses.

- Suggestion to consider a more comprehensive test set, made of a set of full-length sequences for visual evaluation, augmented by a larger set of shorter sequences (e.g., 1s or 2s long) to reduce possible overfitting.

It was commented that Beatriz might also be a candidate for replacement, with relatively low rate and good benefit of RPR. However, the quality at lowest rate point appears appropriate from the CfE.

It was commented that, if no cropped 8K replacement for Fashion Lady would be found, it would be OK to have only three in that category. Logistically, it might be more important to have balance between HLG and PQ in the HDR categories for testing.

Comments on Draft CfP:

Comment 1 – about Quantization settings

In Section 4.1 of the Draft CfP, we suggest being more specific in the following sentence:

- Quantization settings should be kept static. When a change of quantization is used it shall be described.

We suggest asking proponents to describe the Quantization settings within the GOP (e.g. QP deltas per TiD), and within the picture.

We also suggest asking proponents to describe the way lambda settings are made.

Comment 2 – about runtime-constrained category

It is important to consider more than 2 operation points, as used during the CfE process. This is to have a good understanding of a coding solution’s behaviour across different complexity points. Limiting to only 2 operation points would not give enough information. It is also important to keep the 5x VTM operation point as gains are not yet saturating at the 1x VTM operation point. The 5x VTM operation point provides additional information on higher complexity encoder trends.

For practical evaluation, we suggest keeping the 3 encoder run time targets defined in the CfP draft:

- About 5 times (500%) the encoder run time of the VTM anchor

- About same (100%) as the encoder run time of the VTM anchor

- About 0.2 times (20%) the encoder run time of the VTM anchor

We also suggest to not prevent proponents to provide additional / other operation points, even if these points may not be perceptually evaluated.

It was clarified that having three points is reasonable to recognize trends.

A comment was made that the 0.2xVTM runtime point appears too low as a requirement for proposals, which may at the point of CfP not optimized for fast runtime. Other experts had the opinion that this point is important. It was argued that this was also defined considering the outcome of the workshop.

Comment 3 – about reporting HDR metrics

We suggest that wPSNR shall be reported for HDR PQ content and PSNR shall be reported for HDR HLG content. The average gain for each of these HDR categories should be separately reported in a summary table.

Comment 4 – about reporting YUV metrics

We suggest to jointly work on a commonly agreed objective YUV metric. If we succeed in defining this metric, we suggest including it in CfP submissions.

YUV metrics are typically obtained by combining Y, U, and V distortion in one of several ways:

- Weighted average of BD-rate values

- Weighted average of PSNR values

- Weighted average of MSE values

Weighted average of BD rate appears inappropriate.

It was commented that one problem on weighted averages of PSNR is the sequence dependency. Weighted MSE may be more consistent.

Comment 5 – about avoiding over-fitting

The solution of considering a hidden test set made of a set of additional sequences is mentioned in JVET-AO0045 to limit the risk of overfitted solutions.

An alternative approach is to consider a more comprehensive test set, made of a set of full-length sequences to be used for visual evaluation (as currently considered in the draft CfP), augmented by a larger set of shorter sequences (e.g., 1s or 2s long) offering more content variety to reduce possible overfitting. As these shorter sequences may generally not undergo formal visual evaluation, a wider selection may be available as they may not need to be “well behaved” in the context of formal visual evaluation.

Such an approach may be more practical after the CfP phase when core experiments are established for the standard development phase. Indeed, it does not require a neutral entity to maintain a separate set of sequences and to conduct cross-checks.

Note that this approach would not preclude to test a draft standard on an even wider set of sequences from time of time (e.g., on an annual basis).

It was agreed to leave the range of run time targets in 5.3 of the CfP draft as it is, i.e. remove the first yellow highlighted paragraph in that section (right below the bullet list).

JVET-AO0238 Comments on the Draft CfP Document Regarding Potential Significant Variability in Decoding Time [K.-W. Liang, T. Ikai (Sharp)] [late]

This contribution proposes to address concerns regarding potential significant variability in decoding time among responses to the Call for Proposals (CfP). In particular, decoding time of certain neural-network-based tools, such as neural-network-based loop filters (e.g., VLOP14 and VLOP11), has been reported to be approximately 14× and 7× that of the VTM anchor, respectively, according to the AHG11 report (JVET-AN0011).  Such large differences in decoding complexity may make it difficult to perform fair and meaningful comparisons across different compression technologies.

It was commented that currently no constraint is imposed on decoder runtime. Requiring to submit additional bitstreams for cases of overly large decoding runtime might not be appropriate. For algorithms based on neural networks, additional GPU runtime is requested for NN based algorithms in the complexity reporting template. Furthermore, decoding runtime would probably not be a relevant criterion for selection.

It was agreed that in the draft CfP, in section 10 (requirements for submissions), a sentence should be added under C (“optional information”) that any additional information about properties and performance of the algorithm or specific tools is welcome.

BoG (F. Bossen, M. Wien) for developing draft CfP document from JVET-AO0045

- Considering the suggestions from JVET-AO0130, JVET-AO0135, JVET-AO0184, JVET-AO0072

- Define additional rate points at the high end

- General editorial improvements

### Test sequences and conditions (7)

Contributions in this area were discussed during 1705–1730 UTC on Thursday 15 Jan. 2026, and during 2100–2325 UTC on Friday 16 Jan. 2026  (chaired by JRO).

JVET-AO0072 is also related.

JVET-AO0054 [AHG17] Proposal of new sequences for the HD-SDR-RA category of the CfP [M. Abdoli (Xiaomi)] [late]

Four sequences, namely “DJ”, “ILandSound”, “Mathilde”, “TravellerSwim” are proposed for inclusion in the SDR-HD-RA category of AHG17 in preparation for the next generation codec CfP.

The proposed sequences share the following signal characteristics:

| - Resolution: | 1920x1080 (HD) |
| --- | --- |
| - Bit-depth: | 10-bit |
| - Color space: | SDR ITU Rec. BT709 |
| - Frame-rate: | 50Hz (except for Mathilde at 25Hz) |
| - Chroma sampling: | YUV 4:2:0 |
| - Duration | 10 seconds |

It was commented that the sequences have different characteristics than current ones (more dark, noisy). It was commented that they may be less challenging than the current ones.

It was further commented that the argument of not using “old content” is questionable. The intent of the contribution is rather to provide a mix in that category.

Sequences are available in the content server.

It was requested to also provide the VTM encoded bitstreams (range 17 … 45).

It was suggested to propose a range and potential rate points for visual testing.

JVET-AO0059 Liverpool Test Sequences [J. Calic, A. Foster, D. Collado (BBC)]

A set of new 4k HLG test sequences is proposed to be evaluated for potential use in standardization, e.g. for picture quality, verification and conformance testing of bitstreams. All sequences were shot during a music festival in and around Liverpool, UK in May 2025. The camera used to capture the sequences was a Sony FX9, while the video was recorded in 4K DCI resolution at 50fps via the 16-bit Linear RAW sensor output (XDCA-FX9 module) to an Atomos recorder, turning it into 12-bit Log ProRes RAW. The ProRes RAW video was graded and post-processed to obtain YUV sequences with HLG HDR BT.2020 transfer characteristics and color gamut using the DaVinci Resolve software. The YUV files were further processed using ffmpeg to convert them into YUV420 10bit video for encoding with VTM. The UHD 3840x2160 assets were produced by cropping the 4K DCI video. All sequences have been encoded using VTM-23.8.

It was suggested to also prepare variants of VTM using RPR.

It was commented that for some of the sequences the camera seems to be a bit shaky.

Several experts found the sequences interesting in terms of properties (some of them demanding in bit rate, high detail, dark, etc.).

It was asked if SDR versions could be made available (e.g., to replace SDR 4K sequences)? This should be possible. It was commented that it would be desirable to have those available soon, in order to include them in the viewing in the AHG meeting in February.

It was requested to also provide the VTM encoded bitstreams.

It was commented that the chroma position alignment may not be correct when ffmpeg was used. This should be checked, and potentially corrected by using HDRtools, for example.

JVET-AO0061 AHG4/AHG17: AIGC test sequences [J. Liu, X. Chen, Y. Gao, Z. Chen (Wuhan Univ.), Y. Pan, T. Yao, T. Mei (HiDream.ai)]

In this proposal, six 24-fps AIGC sequences, including four 1920×1080 and two 1080×1920 sequences, are described and contributed to JVET. We recommend them as candidate test sequences in developing new-generation video coding standard.

It was asked if the sequences were originally generated in HD resolution? It was confirmed that was the case, the origin of the generation are images of HD resolution.

It was commented that it would be desirable to have sequences with higher frame rate, and also more than 8 bit (which might have caused some banding artifacts). The latter (higher bit depth) appears more important. With higher bit depth, might it also be possible to generate HDR versions?

The model for generation would not be available.

It was asked if the generation was performed in RGB or YUV? They were generated in RGB and converted to YUV.

It was commented that the content seems to be less complex than camera captured content. in terms of SI/TI. Also from the PSNR-rate graphs, it appears that the sequences are not difficult to code.

Also, SI/TI is often not correlated with rate demand of sequences.

One comment was also made that some sequences also seemed to have some blurred areas.

It was commented that the sequences are not in the “photorealistic” type of AI generated content.

It was commented that it might be desirable to investigate how RPR would perform on this type of sequences.

It was concluded that in the future, AIGC content will become more important, and various experts found the sequences very interesting. However, for the CfP it may be too early to include those sequences forming a new category, or replacing some camera captured sequences. JVET should also get a better understanding about wider variety of AIGC, using different models, variety of characteristics, etc., before going into definition of test cases. It was suggested to organize an informal viewing session either in the AHG meeting in February or during the 42th meeting to get an understanding about the compression characteristics of those sequences.

JVET-AO0184 AHG17: Reporting on HDR coding metrics [W. Husak, T. Nguyen Canh, P. Yin (Dolby), D. Rusanovskyy (Nokia)]

This contribution proposes clarification that wPSNR shall be used for HDR PQ content and PSNR shall be used for HDR HLG content when reporting objective HDR metrics in responses to the Call for Proposals on video compression with capability beyond VVC and no average between these metrics to be reported. This proposal is aligned with JVET-AM0287 On HDR coding and metrics and JVET-AC2011 Common Test Conditions for HDR/WCG.

It is also proposed to update the text of the Common Test Conditions for HDR/WCG by removing the requirements for deltaE100 and PSNRL100 metrics reporting of HDR QP content.

Agreed that this should be made clearer in the CfP. The best solution would be to put all HLG sequences (regardless if they are 4K or cropped 8K) into one category, and all PQ content into another category. This means that also the naming conventions need an update.

JVET-AO0226 AHG17: Evaluation of CfE Sequences Considering Sequence Characteristics [R. Ishimoto, F. Zheming, T. Ikai (Sharp)]

This contribution reports an evaluation of tool performance using CTC and CfE sequences through a CfE software (JVET-AN0271), which supports most of ECM tools with the same or similar algorithm and reduced encoding time. The experiments are conducted using a 5x encoder configuration. The results indicate that the CfE sequences demonstrate relatively similar gains in terms of average coding efficiency across most tools, while the distributions have some variations that could be useful for potential discussions on the validity of sequences.

In v2, ALF is added.

It was commented that looking at distribution of gains rather than average appears highly useful.

JVET-AO0248 AHG4/AHG17: Response to the call for new HDR materials for future video coding development [J. Wang, J. Zhang, L. Yu (ZJU)] [late]

In JVET-AN0177, six 4K HDR sequences were provided as a response to the call for new HDR materials for future video coding development. A joint viewing was conducted for the sequences from JVET-AN0177 and the parallel document m74033 proposed in MPEG AG5. Based on the expert feedback, this contribution includes nine sequences, six of which have been updated through re-editing and color grading, while the remaining sequences remain unchanged.

It was commented that the sequences were found to be clipped at approx. 75%, and also do not go into the black level, so they are not fully using the HDR range. It was requested to conduct offline discussion with J. Calic, A.Wieckowski and M. Wien to clarify how the raw material had been process.

As the content otherwise has some interesting characteristics, it should be investigated if re-grading of the raw material would be possible until the AHG meeting in February.

Revisit: Clarify what steps that would need to be done by proponents of new sequences in preparation of the viewing during the AHG meeting (M. Wien to propose)

## AHG18 Ultra-low latency and packet loss resilience (4)

Contributions in this area were discussed during 1300–1500 UTC on Friday 16 Jan. 2026 (chaired by JRO).

JVET-AO0052 is also related.

JVET-AO0049 AHG18: Teleconference on ultra-low latency and packet loss resilience [S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs)]

No need for review – report from discussion in telco

JVET-AO0047 AHG18: Simulation software for ULL CTC JVET-AN2039 [V. Khamidullin, K. Malyshev, I. Gribushin, M. Sychev, S. Ikonin, E. Alshina (Huawei)]

This contribution describes the simulation software submitted for AGH18 on ultra-low latency and packet loss resilience based on contribution JVET-AN2039. The purpose of this document is to give an introduction and help to be familiar with software. The document describes main modifications of encoder and decoder applications of VTM, entry point for network transmission emulation, and basic logic of main modules. The software also includes the fixes of JVET-AN0081 approved by 40th JVET meeting. The software is available as a branch at https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM/-/tree/jvet-an2039.

The software consists of a modified VTM (encoder and decoder). It is using a trace file that is coming from an external network simulator. The trace file is used to drop NAL units from the bitstream (currently simulated by jitter buffer in the encoder application). The decoder receives a modified stream where NAL units ar missing, but has been modified in a way that it does not crash and conducts a concealment (frame freezing until a full set of reference pictures is available).

For the unicast application, feedback is generated from the jitter buffer such that the encoder is aware of lost frames, and can update the bitstream accordingly.

For the broadcast application (no feedback), one option is using a scalable stream, where base layer packets get priority in transmission.

It was asked if FEC has been used? This was not the case. In principle, this could be implemented, but for that purpose it would be better to decouple network packetization from the encoder.

It was also commented that some mechanisms that are currently under discussion in the network community for the benefit of low latency applications, are not yet readily developed.

Decision(SW): include JVET-AO0047 in AHG18 software

JVET-AO0205 [AhG18] Crosscheck report for JVET-AO0047 [V. Zakharchenko (Nokia)]

It was verbally reported that all tests have been completed, and that the software is working appropriately and matches with the description.

JVET-AO0048 AHG18: Description of the software related to ULL CfE response JVET-AN0080 [V. Khamidullin, K. Malyshev, A. Dzugaev, I. Gribushin, M. Sychev, X. Ma, S. Ikonin, E. Alshina (Huawei)]

This contribution describes the software related to the document JVET-AN0080 “AHG18: CfE response in additional functionality on ultra-low latency and packet loss resilience” presented and evaluated at 40th JVET meeting. The software contains further modifications on top of previously disclosed in contribution JVET-AN0045. The software is available as branch in AhG18 repository, named ‘jvet-an0080’: https://vcgit.hhi.fraunhofer.de/jvet-ahg-ull/VVCSoftware_VTM/-/tree/jvet-an008. The software implements the coding technology beyond VVC that supports packet loss resilient functionality in ultra-low latency video communication scenario. The technology re-uses concept of data partitioning of H.263/4, which separates prediction and residual information to different sub-streams. Additionally, entropy coding of residual coefficients is performed using multiple independent entropy coded chunks. The error mask is derived based on lost chunks indication, and further propagated following intra- and inter prediction from erroneous blocks. An error cancellation technique including NN-based loss aware filter (LAF), implemented using SADL library, is further applied to reduce subjective impact of reconstruction errors caused by missed residual information. The software was evaluated according to ULL CTC JVET-AN2039. In best preforming configuration with transmission priorities the simulation results demonstrate 2.6 dB of Y-PSNR improvement on top of multi-layer VTM in broadcast scenario, with only 2.3% of overhead, and average freeze ratio reduction from 23% down to 0% on top of single layer VTM in unicast scenario, with 2.6% overhead including effect of CCLM disabled.

No detailed presentation, as the contribution JVET-AN0080 had been reviewed in the last meeting.

It was agreed that making the software of JVET-AN0080 available in the AHG18 branch is useful for study by other experts.

JVET-AO0123 AHG18: Standalone network simulation application on top of ULL software [K. Malyshev, I. Gribushin, V. Khamidullin, M. Sychev, S. Ikonin, E. Alshina (Huawei)]

This document proposes the software modifications on top of AHG18 simulation software allowing to use network simulation as standalone application separated from encoding and decoding process. The standalone application, similarly to JVET-AK0057, parses Annex B format bitstream, and provides a flexibility for testing of various video coding solutions under desired network scenario using pre-encoded uncorrupted bitstreams, and is applicable for broadcasting scenario with no feedback. The proposed modifications keep the network model behavior same as described in CTC document JVET-AN2039 and implemented in JVET-AO0047, providing the capability of configuring network transmission priorities using configuration file additionally. Option to link network model with encoding application is also possible, which makes possible to test unicast scenario using VTM based software.

Feedback to encoder is implemented in a way that the jitter buffer could inform the encoder about the successful transmission (would require modification of current encoder app).

In current implementation, the network app interprets NAL units as packets, and also gets the priority information from the NAL unit headers (NAL unit type, layer ID to be prioritized can be specified by confif file).

It was commented that for the purpose of using FEC (or other types of redundancy), one option might be to employ unspecified NAL units.

The implementation of feedback mechanism such that those could easily be used by any encoder app seems to require further study. For this, a first step would be to implement the feedback mechanism in the encoder app of JVET-AO0047 such that it can be used with the separated network app.

JVET-AO0208 AHG18: GDR functionality with LowDelay-B configuration [A. Dzugaev, M. Sychev, K. Malyshev, S. Ikonin, E. Alshina (Huawei)] [late]

This contribution provides an extension of GDR functionality to support multi-reference configuration like LDB. Proposed solution is described and accompanied with simulation results obtained according ULL CTC (JVET-AN2039) and GDR configuration from (JVET-AN0195).

Current implementation of GDR in VTM enforce GOP=1 once GDR enabled which leads to IPPI configuration. There are two SW fixes proposed for fixing issues with GDR in combination with LDB configuration. The removal of this configuration restriction and further fixing functionality demonstrate BD-PSNR-Y 5.5 dB improvement on average under ULL CTC in broadcast scenario. The freezing ratio reduced from 68% down to 44% versus classic VTM anchor.

IDR pictures were replaced by GDR pictures with same intra refresh period, and GDR period of 8 pictures. GDR increases the bit rate by 7.8% in the lossless case.

Results are reported for a (non-scalable) broadcast scenario, where APS NAL units are transmitted with priority. It was noted that the results are almost identical for priority and no-priority cases, which may be due to the fact that APS packets often lost, or available ones are still appropriate to be used.

It was reported that for the unicast (with feedback) scenario, no benefit was observed.

V. Zakharchenko offered to conduct a cross-check. Revisit.

JVET-AO0072 AHG17/AHG18: On ULL evaluation metrics toward CfP [T. Chujoh, Y. Kidani, K. Kawamura (KDDI)]

This contribution reviews the framework for low-latency techniques in previous international video coding standards and discusses evaluation metrics about video coding delay. The purpose of this contribution is to help discussions regarding the testing conditions on ultra-low latency (ULL) in the call for proposals (CfP) on video compression with capability beyond VVC.

It was agreed that requesting such information in CfP submissions would be useful. It was however pointed out that HRD concepts might not apply to some algorithms, or might be wrongly interpreted.

When coded bits per picture are known, many other of the metrics can be determined.

It was asked how different proposals would be compared in terms of the reported metrics? The initial seems to unveil at least the worst case of delay, and if it is exceedingly high.

It was commented that, in the current view of AHG18, processing time for encoding and decoding is not considered.

## AHG19 Hardware implementation complexity (4)

Contributions in this area were discussed during 1650–1730 UTC on Friday 16 Jan. 2026 (chaired by JRO).

JVET-AO0147, JVET-AO0247, JVET-AO0249, and JVET-AO0286 are related on aspects of AI hardware complexity.

JVET-AO0046 [AHG19] Teleconference on hardware implementation complexity [Y. Zhao]

No need for presentation.

JVET-AO0053 [AHG19] Response to the questionnaire on VVC hardware encoder implementation challenges [M. Wu, J. Yu (HiSilicon), Y. Zhao, E. Alshina (Huawei)]

TBP (if time allows – was already presented in BoG JVET-AO0287)

JVET-AO0060 AHG19: Introduction to hardware video encoder complexity [P. Wetzel (VITEC)]

For information – was presented in telco.

JVET-AO0075 [AHG19] Per-CU max RDO number and weighted sum of RDO number [Y. Zhao, E. Alshina (Huawei), K. Naser, F. Le Léannec (InterDigital), Y. Tokumo, T. Ikai (Sharp)]

This contribution introduces an implementation on top of VTM23.11 that measures the maximum number of per-CU RD cost computations, called maximum RDO number (Max-RDO), as well as a weighted sum of all RDOs (Sum-RDO) by considering both the number of pixels inside a block and the RDO number of the block. It is observed that 1) Max-RDO is up to 168 under VTM default configuration and that 2) there is a high correlation between Sum-RDO and encoding runtime. Naturally, the RDO numbers are platform-independent.

Definition of RDO to be counted: Execution of a forward transform plus quantization in case of Max-RDO (no weighting), or only those that are going to bit-level encoding for Sum-RDO, and weighting by CU size.

These definitions are different from those used in JVET-AO0114. It is noted that the authors of JVET-AO0114 are also co-authors of JVET-AO0075.

It was commented that it might be useful to differentiate between single tree and dual tree, as in the first case only one RDO is counted for luma  and chroma.

For analysis of encoding algorithms, the number of RDOs to be executed appears important from the reports of the two documents. Further study appears necessary about the best definition of an RDO, or if it may even be necessary to make independent counts of certain types of RDO (luma/chroma, with/without entropy coding, etc.), because these may have different impact on the runtime or difficulty in hardware implementation.

JVET-AO0137 [AHG19] Analysis of the HEVC encoder toolset in commercial hardware applications [M. Porto, D. Palomino, L. Agostini, G. Corrêa, B. Zatt, A. Schuch, V. Costa (ViTech - UFPel)]

TBP (if time allows – was already presented in BoG JVET-AO0287)

At the end of the session on Jan. 16, it was requested that, as a next step from the initial review of documents in 4.8 and 4.18, a BoG (X. Li, Y. Zhao) should work on

- Further improvement of JVET-AN2040 – including discussion under JVET-AO0067, and JVET-AO0130

- Considering possibly missing aspects regarding hardware implementation, based on an initial review of JVET-AO0053, JVET-AO0060 and JVET-AO0137

## CICP (1)

Contributions in this area were discussed during 2100–2150 UTC on Monday 19 Jan. 2026 (chaired by JRO).

JVET-AO0171 CICP definitions for depth maps and alpha planes [E. Thomas, E. Potetsianakis, E. Alexiou, M.-L. Champel (Xiaomi)]

The current CICP future extensions (JVET-AN1005) introduces new code points for depth map and alpha plane signals. However, it is asserted that both concepts are not defined in the current CICP draft text. It is further asserted that without such definitions those code points would not constitute interoperable code points. As a result, it is proposed that the CICP specification define those concepts.

It was suggested to use definition from MIAF (23000-22) to define alpha maps.

It was commented that also AVC and HEVC (at least) have definition for alpha, which appear more general, and also SEI messages exist for definition of depth.

It was commented that it is desirable to have such clarification about the interpretation of the values. One option might also be to express that this is not a specification but an example for interpretation. For alpha, it might be good to say something like that transparency is for blending with a background image.

One problem about using SEI messages for depth interpretation could be that those are only foreseen for multi-layer streams.

One aspect of alpha might also be that it depends on the purpose it is intended for (blending, transparent screen, opaqueness, etc.).

Another option might be to allow certain SEI messages in single-layer streams.

It might need to be clarified that this is only used for single-layer streams and no chroma planes.

It was commented that CICP codepoints should not be wasted for multiple interpretations of auxiliary information.

It was pointed out that there are many definitions in the ISO terminology database for “alpha”-related terms. https://www.iso.org/obp , including in JPEG.

Further study on previous definitions before taking action.

# Low-level tool technology proposals (72)

## AHG11/AHG14: Neural network-based video coding (32)

### Summary and BoG reports

Contributions in this area were discussed during 2100–2300 UTC on Wednesday 14 Jan. 2026 and during 2100–2210 UTC on Thursday 15 Jan. 2026 (chaired by JRO).

JVET-AO0041 [AHG11] [AHG14] Teleconference on NNVC [E. Alshina, F. Galpin]

No need for presentation – became part of AHG11/14 reports

JVET-AO0023 EE1: Summary report of exploration experiment on neural network-based video coding [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)]

Abstract

This report summarizes the activities of the Exploration Experiment 1 (EE1) performed between the 40th and 41st JVET meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, evaluate their complexity aspects, and clarify training procedure. Two teleconferences have been conducted during this meeting circle. Six categories of experiments were included into this round of EE: LOP NN-based filter, VLOP NN-based filter, normative TDO (time distortion optimization), NN-Inter, NN-based super resolution and framework for end-to-end learnable picture codec for I-frames coding improvement.

Introduction

Code base for the EE1 tests was NNVC15.0, anchor is default configuration of NNVC-15.0 (NN-Intra and LOP6 filter enabled). NNVC common test conditions [1], results and complexity reporting template were used.

For proposals in all categories, proponents used AhG11 training set, which consists of DIV2K [2], BVI-DVC [3], TVD [4]. NN-based Inter tools were trained using Vimeo 90K triplet [5]. Comparison is done between tests which use the same sub-set of training data.

It was agreed for tests competing with technologies in NNVC to configure the proposed solution targeting the complexity close to existing NNVC tool, but not exceeding it:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- the number of channels (both input and output) in neural network modules which are modified must be keep multiple of 16,

- Number of Parameters EE1 test  Number of Parameters NNVC (if possible).

If it is not possible to respect both constrains 1) and 2) at the same time then two sub tests need to be performed: first test respecting the constrain 1) only – in order to prove the value of the proposed change, and the second test respecting constrain 2) with minor violation of 1)   - in order to provide software and hardware friendly design.

According to the EE description only tests with results provided to quantized (int 16) model are considered for adoption to NNVC, float point model results are provided just for information.

Inference cross-check is required for all EE1 tests. Candidates for adoption to NNVC are required to undergo training cross-check, implementation must be compatible with SADL.

Exact parameters settings were announced by proponents by 2nd AhG11/14 teleconference on December 16 [6].

List of tests in this round of exploration experiment includes:

EE1-1: LOP in-loop filter

  - EE1-1.1: LOP with Overlapped Feature Integration  JVET-AO0066 (UESTC, Transsion)

  - EE1-1.2: Combination test of EE1-1.1 and Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels  JVET-AO0074  (Xidian Univ., UESTC, Transsion)

EE1-2: VLOP in-loop filter

  - EE1-2.1: VLOP with new backbone block based on Spatial-Channel Mixing JVET-AO0129 (KHU, ETRI)

EE1-3: Normative TDO methods

  - EE1-3.1: Normative TDO method as an alternative option in NNVC JVET-AO0117 (Hanyang Univ., ETRI)

  - EE1-3.2: Neural network in-loop filter usage based on activation map JVET-AO0095 (Nokia)

  - EE1-3.3: Combination of EE1-3.1 and EE1-3.2 JVET-AO0133 (Hanyang Univ., ETRI, Nokia)

EE1-4: NN-Inter

  - EE1-4.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AO0108  (Wuhan Uni.)

  - EE1-4.2: FlowWarp Operator for DRF Integer Inference Optimization - withdrawn

EE1-5 NN-based super-resolution complexity reduction

  - EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) JVET-AO0128 (KHU, ETRI).

EE1-6 Framework for externally coded pictures

  - ‘EE1-6.1 multi-layer’

    - EE1-6.1.1:   DCVC-FM as a Learned End-to-End Intra Frame Codec JVET-AO0186 (Huawei)

    - EE1-6.1.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec JVET-AO0176  (Nokia)

    - EE1-6.1.3:   JPEG-AI as a Learned End-to-End Intra Frame Codec JVET-AO0204 (Huawei)

  - EE1-6.2 ‘single-layer-like framework’

    - EE1-6.1.1:   DCVC-FM as a Learned End-to-End Intra Frame Codec JVET-AO0186 (Huawei)

    - EE1-6.1.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec JVET-AO0177  (Nokia)

    - EE1-6.1.3:   JPEG-AI as a Learned End-to-End Intra Frame Codec JVET-AO0204 (Huawei)

EE1-1: LOP in-loop filter

Anchor is NNVC-15, LOP6 filter and NN-Intra enabled.

EE1-1.1: LOP with Overlapped Feature Integration  JVET-AO0066 (UESTC, Transsion)

EE1-1.1.1: only training modification, filter architecture LOP6

EE1-1.1.2: training modification and proposed LOP architecture

Key changes are in the in-head block of LOP6. LOP6 receives as input only DCT transformed Luma reconstruction (RecY) and prediction (PredY), EE1-1.1.2 filter receives additionally ‘Overlap DCT’ processed RecY and PredY.

The changes introduced by proposed solution lead to the complexity reduction in Chroma branch by removing BBB blocks and changing head block, reducing number of channels in head block and aligning number of channels to be multiple of 16.

    - EE1-1.2-. Combination test of EE1-1.1 and Backbone Block Enhancement of LOP In-Loop Filter with Over-Parameterized Training and Variable Channels  JVET-AO0074  (Xidian Univ.,Xidian Univ. UESTC, Transsion)

additional SIMD acceleration is used for EE1-1.2 (which was not used for LOP6).

    - EE1-1.2.1: Only over-parametrization aspect

    - EE1-1.2.2: over-parametrization as in EE1-1.2.1 and new learning rate from EE1-1.1.1

    - EE1-1.2.3: large convolution kernels (51 and 15) and variable number of channels

    - EE1-1.2.4: combination EE1-1.1 and EE1-1.123 (LOP architectural modification + new learning rate)

Changes of filter architecture associated with EE1-1.2.3: large convolution kernels (51 and 15) and variable number of channels.

No reliable encoder run-time provided (but should not different from the anchor), decoder run-time is reliable. In all tests kMAC/pxl doesn’t exceed complexity of LOP6.

|  | Random Access configuration |  |  |  |  | All Intra configuration |  |  |  |  | kMAC/pxl |  |  | Num Param, M |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Total | Filter | Intra | Total | Filter | Intra |
| NNVC(LOP6) | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% | 21.4 | 16.6 | 4.8 | 1.5 | 0.25 | 1.3 |
| EE1-1.1.1* | -0.1% | -0.3% | -1.1% | - | 100% | 0.0% | -0.1% | -1.0% | - | 100% | 21.4 | 16.6 | 4.8 | 1.5 | 0.25 | 1.3 |
| EE1-1.1.2 | -0.2% | 0.1% | -0.1% | - | 88% | -0.1% | 0.2% | -0.3% | - | 84% | 21.3 | 16.5 | 4.8 | 1.5 | 0.25 | 1.3 |
| EE1-1.2.1* | -0.1% | -0.4% | -0.7% | - | 97% | 0.0% | -0.1% | -0.7% | - | 98% | 21.4 | 16.6 | 4.8 | 1.5 | 0.25 | 1.3 |
| EE1-1.2.2* | -0.2% | -0.6% | -0.6% | - | - | -0.1% | -0.3% | -0.2% | - | - | 21.4 | 16.6 | 4.8 | 1.5 | 0.25 | 1.3 |
| EE1-1.2.3 | -0.2% | -0.4% | -0.3% | - | 96% | -0.2% | -0.5% | -0.5% | - | 99% | 21.4 | 16.6 | 4.8 | 1.5 | 0.25 | 1.3 |
| EE1-1.2.4 | -0.3% | -0.6% | -0.4% | - | 97% | -0.2% | 0.0% | -0.1% | - | 99% | 21.4 | 16.6 | 4.8 | 1.5 | 0.25 | 1.3 |

* no changes in LOP6 architecture, only training was modified.

Training and inference are cross-checked for EE1-1.1 tests. For EE1-1.2 tests training cross-check resulted larger than 0.1% BD-rate difference – unsuccessful.

It was commented that the deviation in training for 1.2 might have been caused by using different set of training data. Further study (EE) necessary for 1.2.x category.

For 1.1, the benefit in terms of compression is not large for the architecture change 1.1.2 (compared to 1.1.1 which only changes the learning rate in training), but it is understood to have some advantage such that the modification of number of channels being multiple of 16 reduces decoding time (some reduction also by removing some chroma backbone blocks). Training time is also not increased compared to LOP6.

Decision: Adopt JVET-AO0066 test 1.1.2 (both training modification and architecture change).

EE1-2: VLOP in-loop filter

  - EE1-2.1: VLOP with new backbone block based on Spatial-Channel Mixing JVET-AO0129 (KHU, ETRI)

Proposed modifications are depicted on Figure below. Computational complexity is 4.39kMAC/pxl (13% less than VLOP4) and the number or parameters in 62.8k (14% less than VLOP4).

|  | Random Access configuration |  |  |  |  | All Intra configuration |  |  |  |  | kMAC/pxl |  |  | Num Param, M |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Total | Filter | Intra | Total | Filter | Intra |
| NNVC(VLOP4) | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% | 9.9 | 5.1 | 4.8 | 1.4 | 0.073 | 1.3 |
| EE1-2.1 | 0.6% | 0.6% | 0.4% | - | - | 0.5% | 0.7% | 0.5% | 100% | 93% | 9.2 | 4.4 | 4.8 | 1.4 | 0.063 | 1.3 |

No cross-check for inference and training.

It was commented that it is not clear whether the introduction of the new type of backbone block (which gives reduction of complexity) is necessary. Evidence could be provided by showing a configuration with a complexity comparable to current VLOP. Introducing another complexity point still relative close to VLOP would not be justified. Further study in EE.

EE1-3: Normative TDO methods

In NNVC-15, three NNLF modes are available for B-/P-slices: OFF, ON with base QP, ON with base QP + offset. Tests in this category modify signalling of NNLF. In some situations, NNLF is disabled according to the certain rules (known to encoder and decoder).

  - EE1-3.1: Normative TDO method as an alternative option in NNVC JVET-AO0117 (Hanyang Univ., ETRI)

    - EE1-3.1.1, the proposed normative TDO for VLOP, LOP and HOP was tested.

    - EE1-3.1.2, the proposed normative TDO for VLOP, LOP and HOP was tested with sub-tests using the modified TDO lambda sets (targeting no-loss from TDO).

If Threshold># of BS partitions
→Skip NNLF filtering for the block e.g., 128×128 or 256×256.
Else 
→Enable NNLF filtering for the block.

Cost=D+λ*(# of NNLF filtered blocks)

  - EE1-3.2: Neural network in-loop filter usage based on activation map JVET-AO0095 (Nokia)

    - EE1-3.2.1, study the proposed normative activation map usage for NNLF

Additionally, to BS of current CTU, activation map (BS) of reference block is taken into account.  For a given block in an inter-coded slice, the NNLF is enabled or disabled based on a pixel-wise activation map and boundary strength values of the block itself. The activation map is a pixel-wise map that has the same shape as the target block and it is constructed using the NNLF mode from its reference block(s).

  - EE1-3.3: Combination of EE1-3.1 and EE1-3.2 JVET-AO0133 (Hanyang Univ., ETRI, Nokia)

    - EE1-3.3.1: “no interaction” between EE1-3.1 and EE1-3.2, i.e. both tools are evaluated by the encoder separately and the best one is chosen using as criteria TDO cost and RDO cost.

    - EE1-3.3.2: the signalling of EE1-3.1 is also used in EE1-3.2 to further deactivate the NNLF. For this purpose, the BS threshold is also derived when EE1-3.2 is being evaluated.

|  |  | Random Access configuration |  |  |  |  | Low Delay B configuration |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Decoder decision based on | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Over NNVC-15 with HOP |  |  |  |  |  |  |  |  |  |  |  |
| EE1-3.1.1 | BS cur., Th | 0.1% | 0.2% | 0.1% | - | - | -0.2% | -0.4% | -0.6% | - | - |
| EE1-3.1.2 | BS cur., Th | 0.0% | 0.0% | 0.0% | - | - | -0.1% | -0.2% | -0.3% | - | - |
| EE1-3.2 | BS cur., BS ref. | 0.0% | 0.0% | 0.0% | 99% | 98% | -0.2% | -0.1% | -0.5% | 100% | 97% |
| Over NNVC-15 with LOP |  |  |  |  |  |  |  |  |  |  |  |
| EE1-3.1.1 | BS cur., Th | 0.1% | 0.0% | 0.0% | 100% | 77% | -0.1% | -1.1% | -0.8% | 100% | 69% |
| EE1-3.1.1* | BS cur., Th (non-normative) | 0.2% | 0.0% | 0.0% | 100% | 76% | 0.0% | -0.9% | -0.6% | 100% | 66% |
| EE1-3.1.2 | BS cur., Th | 0.0% | 0.0% | 0.0% | 100% | 91% | -0.1% | -0.2% | -0.2% | 100% | 84% |
| EE1-3.2 | BS cur., BS ref., | 0.0% | -0.1% | -0.1% | 100% | 95% | -0.2% | -1.4% | -1.2% | 100% | 89% |
| EE1-3.3.1 | BS cur., BS ref.,Th | 0.0% | -0.1% | -0.1% | 100% | 90% | -0.3% | -1.2% | -0.9% | 100% | 84% |
| EE1-3.3.2 | BS cur., BS ref.,Th | 0.0% | -0.1% | -0.1% | 100% | 89% | -0.3% | -1.6% | -1.3% | 100% | 81% |
| Over NNVC-15 with VLOP |  |  |  |  |  |  |  |  |  |  |  |
| EE1-3.1.1 | BS cur., Th | 0.1% | 0.0% | -0.1% | 99% | 72% | 0.0% | 0.0% | -0.1% | 99% | 60% |
| EE1-3.1.1* | BS cur., Th (non-normative) | 0.2% | 0.2% | 0.1% | 100% | 70% | 0.2% | 0.1% | 0.1% | 100% | 57% |
| EE1-3.1.2 | BS cur., Th | 0.0% | 0.0% | 0.0% | 100% | 92% | -0.1% | 0.0% | 0.2% | 99% | 79% |
| EE1-3.2 | BS cur., BS ref. | 0.0% | 0.0% | 0.0% | 100% | 94% | -0.1% | -0.1% | -0.2% | 99% | 86% |
| EE1-3.3.1 | BS cur., BS ref., Th | 0.0% | 0.0% | -0.1% | 100% | 91% | -0.1% | 0.1% | -0.3% | 100% | 80% |
| EE1-3.3.2 | BS cur., BS ref., Th | 0.0% | 0.0% | -0.1% | 100% | 89% | -0.1% | -0.2% | -0.4% | 99% | 75% |

Cross-check of inference is successful for all tests, no training cross-check needed (filter architecture and parameters are not changed).

It was commented that the encoder-side RD decision criteria of 3.1.1 (with threshold signalled) and 3.1.1* (non-normative) are not exactly identical. 3.1.2 is a variant of 3.1.1 with different strategy of threshold determination.

3.2 does not introduce additional signalling, but requires to store an activation map to be derived for each reference picture and stored.

The combinations 3.3 are two different flavors of combining 3.1.2 and 3.2.

It was commented that overall, the additional benefit of normative changes compared to the non-normative solution 3.1.1* (which is already part of NNVC but disabled by default) is not large. It has comparably a clearly lower decoder run time than the normative changes that come with no loss, and could also be configured differently in terms of threshold used at encoder.

It was also commented that the worst case decoder complexity is not changed by any of the variants.

It was suggested that it might be interesting to study the usage of the decision logic of 3.3.2 in an encoder-only fashion (does not need to be in EE).

EE1-4: NN-Inter

  - EE1-4.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization JVET-AO0108  (Wuhan Uni.)

    - EE1-4.1.1: no DRF architectural changes, only new training (Charbonnier Loss + Ternary Loss)

    - EE1-4.1.2: DRF ‘small’ trained with Charbonnier Loss + Ternary Loss

    - EE1-4.1.3: DRF ‘small’ trained with L1 Loss

Figure: ‘small’ DRF

- The Charbonnier Loss serves as a smoothed variant of the L1 loss. By introducing a small constant ϵ, it effectively approximates the L1 loss while resolving the non-differentiability issue at the zero point:

Lchar=E I-I2+ϵ2

Where E is mathematical expectation,  I and I denote the predicted frame and the ground truth, respectively, with ϵ set to 10-6.

- The Ternary Loss captures local structural features by analysing relative pixel differences within a patch (e.g., 7 ×7) primarily on the Y channel. It is defined as:

Lternary=1Nx y∈Ω(x) T(Ix,Iy)-T(Ix,Iy)

where Ω(x) represents the local neighbourhood centred at pixel x, and T(·) is differentiable approximation of the Ternary/Hamming transformation applied to the the relative intensity differences to preserve structural details. Specifically, Ternary maps local pixel differences into soft ternary patterns (indicating whether neighbours are smaller, similar, or larger than the centre) to measure structural correlation in a differentiable way.

|  | Random Access |  |  |  |  |  |  |  | kMAC/pxl |  | Num Param, M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | psnrY | psnrU | psnrV | ssimY | ssimU | ssimV | EncT | DecT | Total | NN-Inter | Total | NN-Inter |
| NNVC-LOP6 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 100% | 100% | 21 | 0 | 1.5 | 0 |
| NNVC-DRF | -2.2% | -1.5% | -2.2% | -2.8% | -2.8% | -3.1% | 148% | 2497% | 498 | 477 | 7.6 | 6.1 |
| EE1-4.1.1 | -2.5% | -1.2% | 0.2% | -2.9% | -2.3% | -1.8% | 143% | 2407% | 498 | 477 | 7.6 | 6.1 |
| EE1-4.1.2 | -1.3% | 1.6% | -1.4% | -2.0% | -0.9% | -1.9% | 111% | 762% | 83 | 62 | 5.1 | 3.6 |
| EE1-4.1.3 | -1.4% | 0.6% | -1.4% | -2.1% | -1.3% | -1.9% | 109% | 743% | 83 | 62 | 5.1 | 3.6 |

Training and inference are successfully cross-checked for all tests.

It is found that the modification of loss function in training only has benefit for the current architecture of DRF. For the version with significantly reduced complexity, the L1 loss is better.

Decision: Adopt JVET-AO0108. Replace current DRF by 4.1.1, and add 4.1.3 as a “low operation point” of DRF.

It was discussed that for default configuration, the solution of 4.1.3 would still be too complex, at this moment to be disabled by default.

EE1-5: NN-based super-resolution complexity reduction

Anchor for for this test is NNVC with NNSR enabled.

  - EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) JVET-AO0128 (KHU, ETRI).

Proposed NNSR architecture:

Number of parameters 12.5k (36% less than NNSR-15), MACs/pixel: 3k (36% less than NNSR-15).

|  | Random Access |  |  |  |  | All Intra |  |  |  |  | kMAC/pxl |  | Num Param, M |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Total | NN-SR | Total | NN-SR |
| NNVC(NNSR) | 0.0% | 0.0% | 0.0% | 100% | 100% | 0.0% | 0.0% | 0.0% | 100% | 100% | 26 | 4.7 | 1.7 | 0.19 |
| EE1-5.1 | 0.0% | -0.1% | -0.2% | - | - | 0.0% | -0.3% | -0.2% | - | - | 24 | 3.0 | 1.5 | 0.01 |

Cross-check for inference and training was reported in JVET-AO0271. By a quick review of that document during the presentation of the summary report it was not obvious if the results of training cross-check were matching. Otherwise, 5.1 would be an interesting candidate for replacing current NNSR, as it reduces complexity, while it still provides a very small chroma gain.

Follow-up discussion on Thu. 15 Jan. 2100, after clarification was achieved (see JVET-AO2023v5)

As a conclusion, it was found that the performance deviation between the model re-trained by cross-checker and originally proposed method is negligible (0.0x%).

It is noted that the gain compared to using RPR is rather low, and that the decision mechanism for using reduced resolution coding in EE1 is different from VTM.

Due to complexity reduction compared to existing NNSR in NNVC, the inclusion is assessed to be beneficial.

Decision: Adopt JVET-AO0128 Test 5.1

EE1-6 was discussed Thursday 15 Jan. 2125.

- EE1-6: Framework for externally coded pictures

  - ‘EE1-6.1 multi-layer’

    - EE1-6.1.1:   DCVC-FM as a Learned End-to-End Intra Frame Codec JVET-AO0186 (Huawei, InterDigital)

    - EE1-6.1.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec JVET-AO0176  (Nokia)

    - EE1-6.1.3:   JPEG-AI as a Learned End-to-End Intra Frame Codec JVET-AO0204 (Huawei)

  - EE1-6.2 ‘single-layer-like framework’

    - EE1-6.1.1:   DCVC-FM as a Learned End-to-End Intra Frame Codec JVET-AO0186 (Huawei, InterDigital)

    - EE1-6.1.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec JVET-AO0177  (Nokia)

    - EE1-6.1.3:   JPEG-AI as a Learned End-to-End Intra Frame Codec JVET-AO0204 (Huawei)

Results in this section provided for information. Goal of the tests (at this stage) is to verify developed framework for external E2E AI coded reference picture. None of the test in this category has full SADL implementation. JPEG-AI has quantized entropy part and can be decoded on different device with minor deviation (which will cause a drift I video coding. DCVC-FM and NLIC use float point arithmetic in entropy and so cannot be properly decoded on different device. NLIC code was provided encrypted. So code analysis and complexity numbers cross-check is not possible. Should not-readable code be allowed in EE?

In the table below ‘R’ corresponds to EE1-6.1 tests -  ‘multi-layer’ framework with residual. E2E AI coded picture is used as a prediction, three variants for block reconstruction are possible based on encoder decision:

- Encoder selects Skip mode then reconstruction is identical to E2E AI coded picture.

- Encoder selects Inter mode then E2E AI coded picture serves as prediction and residual is added.

- Encoder selects Intra mode then regular VVC Intra coding is used.

‘0’ corresponds to EE1-6.2 tests - ‘multi-layer’ framework with zero residual. Original intention was to emulate ‘single-layer’ and . replaces VVC Intra coding (in I-picture). But it was implemented by setting maximum possible QP at enhancement layer and having essentially zero residual. In this situation encoder theoretically can choose VVC Intra (with zero residual) or skip mode (reconstruction is equal to E2E AI codec output),

Last index indicates different E2E AI codec engines:

- DCVC-FM		  370 kMAC/pxl 		   22.7 M parameters

- NLIC				1330 kMAC/pxl 		142.8 M parameters

- JPEG AI			    22 kMAC/pxl 		   5.0 M parameters

|  |  | Random Access |  |  |  |  |  |  |  | All Intra configuration |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | psnrY | psnrU | psnrV | ssimY | ssimU | ssimV | EncT | DecT | psnrY | psnrU | psnrV | ssimY | ssimU | ssimV | EncT | DecT |
| R | 1 | -1.1% | -6.0% | -3.4% | -1.9% | -9.6% | -6.7% | 97% | 8.7 | -5.3% | -11.6% | -10.8% | -8.1% | -18.9% | -17.9% | 47% | 207 |
|  | 2 | -0.2% | -4.2% | -2.4% | -1.4% | -6.2% | -3.9% | - | 13.9 | 7.5% | 22.9% | 23.5% | 1.8% | 100.0% | 100.0% | - | 355 |
|  | 3 | 9.6% | 17.1% | 18.7% | 3.1% | 19.4% | 22.2% | 94% |  | 37.1% | 62.5% | 56.0% |  |  |  | 31% |  |
| 0 | 1 | -0.7% | -0.1% | 3.4% | -1.7% | -7.1% | -3.5% | 96% | 8.6 | -6.1% | -1.9% | -4.2% | -8.6% | -16.1% | -16.2% | 32% | 204 |
|  | 2 | -0.1% | -1.4% | 0.5% | -1.2% | -4.6% | -2.2% | - | 13.8 | 7.5% | 22.9% | 23.5% | 7.5% | 22.9% | 23.5% | - | 355 |
|  | 3 | 13.3% | 37.0% | 38.6% | 3.8% | 37.1% | 40.2% | 94% |  | - | - | - | - | - | - | 12% |  |

Typically, variant with residual shows better performance (in RA test).

Performance is very dependent on encoder logic.

Since architectures of E2E AI codec are very different from NNVC RD-curves overlap is typically poor (for NLIC and JPEG AI, not for DCVC-FM) and so BD-rate measurement is not-very reliable. Situation can be improved with more efforts on rate matching between E2E AI codec and VTM I-picture rates (as it was achieved with DCVC-FM).

Multi-layer frame work allows to demonstrate coding benefits, but encoder needs to be more carefully designed for each individual E2E AI codec. For example, the combination of not trained for MSE JPEG AI at base layer and minimizing MSE encoder at enhancement layer is meaningless.

All tests typically show encoder run-time reduction compare to the anchor. But functionality for run-time counting in multi-layer framework with externally coded frame needs to be improved.

Gain looks promising and experiments can be continued. Non-EE contributions show even better performance.

It was discussed whether it is appropriate that a participant in the EE was not providing the code for the AI coded base layer (was the case for the NLIC algorithm).

One opinion was expressed that, as long as the EE is about testing the interface and not assessing algorithms, that may be OK. Encoder run time cannot be measured, and architecture parameters cannot be confirmed, but as long as the purpose is not the competition of AI-based base layer algorithms, that may not be too relevant

Other experts had the opinion that it would be necessary at least for cross-checks to know details of the AI algorithm.

In the current interface, the enhancement layer can determine from the residual in which areas the base layer encoding has failed.

As soon as a joint cross-layer optimization of base and enhancement layer would be targeted, a “black box” as base layer seems no longer to be appropriate.

Some of the BD number reported may be unreliable due to overlapping curves.

The EE confirms that the interface itself is stable and can be used with different AI E2E algorithms.

It was suggested to include analysis about usage of the different modes (skip, inter, intra) of the enhancement layer.

It was also reported that one problem that was solved in the EE was the different usage of MCTF and LMCS at base and enhancement layer, which has been originally aligned for DCVC-FM in the framework. NLIC did not use LMCS for base layer.

### EE1 contributions: Neural network-based video coding (12)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AO0023. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AO0066 EE1-1.1: LOP with Overlapped Feature Integration [J. Chi, A. Li, Y. Du, C. Zhu, L. Luo, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)]

JVET-AO0257 Crosscheck of JVET-AO0066 (EE1-1.1: LOP with Overlapped Feature Integration) [Z. Xu (TCL)] [late]

JVET-AO0074 EE1-1.2: Combination test of EE1-1.1 and EE1-1.2 [J. Han, C. Jung, Q. Qin (Xidian Univ.), J. Chi, A. Li, Y. Du, C. Zhu, L. Lei, H. Guo (UESTC), Y. Huo, Y. Liu, Z. Zhang (Transsion)]

JVET-AO0272 Crosscheck of JVET-AO0074 (EE1-1.2: Combination test of EE1-1.1 and EE1-1.2) J. Sauer, Y. Ge (Huawei) [late]

JVET-AO0095 EE1-3.2: Neural network in-loop filtering usage based on activation map [M. Santamaria, D. Buğdayci Sansli, F. Cricri (Nokia)]

JVET-AO0258 Crosscheck of JVET-AO0095 (EE1-3.2: Neural network in-loop filtering usage based on activation map) [Z. Xu (TCL)] [late]

JVET-AO0108 EE1-4.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization [X. Chen, W. Zhang, N. Fu, J. Zhang, Z. Chen (Wuhan Univ.)]

JVET-AO0274 Crosscheck of JVET-AO0108 (EE1-4.1: Deep Reference Frame Generation for Inter Prediction Enhancement with Structural Re-parameterization) N. Bhaskar (Huawei) [late]

JVET-AO0117 EE1-3.1: Normative TDO method as an alternative option in NNVC [H. Kwon, H. Ko (Hanyang Univ.), D. Kim, S.-C. Lim (ETRI)]

JVET-AO0236 Crosscheck of JVET-AO0117 (EE1-3.1: Normative TDO method as an alternative option in NNVC) [J. Pardo (Huawei)] [late]

JVET-AO0128 EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM) [H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)]

JVET-AO0271 Crosscheck of JVET-AO0128 (EE1-5.1: NNSR with new backbone block based on Spatial-Channel Mixing (SCM)) T. Yang, W.-X. He, X.-T. Xie, Q. Liu (HUST) [late]

JVET-AO0129 EE1-2.1: VLOP4 with new backbone block based on Spatial-Channel Mixing (SCM) [H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI)]

JVET-AO0133 EE1-3.3: Combination of EE1-3.1 and EE1-3.2 [D. Kim, S.-C. Lim (ETRI), H. Kwon, H. Ko (Hanyang Univ.), D. Buğdayci Sansli, F. Cricri, M. Santamaria (Nokia)]

JVET-AO0237 Crosscheck of JVET-AO0133 (EE1-3.3: Combination of EE1-3.1 and EE1-3.2) [J. Pardo (Huawei)] [late]

JVET-AO0265 Cross-check of JVET-AO0133 (EE1-3.3: Combination of EE1-3.1 and EE1-3.2) [T. Dumas (InterDigital)] [late]

JVET-AO0176 EE1-6.1: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AO0277 Crosscheck of JVET-AO0176 (EE1-6.1: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec) [M. Aderdor (Huawei)] [late]

JVET-AO0177 EE1-6.2: Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

JVET-AO0186 EE1-6.1.1 and EE1-6.2.1: DCVC-FM as a Learned End-to-End Intra Frame Codec [M. Aderdor, T. Solovyev, E. Alshina (Huawei), F. Galpin, F. Urban, E. François (InterDigital)]

JVET-AO0204 EE1-6.1.3 and EE1-6.2.3: JPEG-AI as a Learned End-to-End Intra Frame Codec [A. Karabutov, E. Alshina (Huawei)]

### EE1 related and beyond-EE contributions: Neural network-based video coding (13)

Contributions in this area were discussed during 2210–2310 UTC on Thursday 15 Jan. 2026 (chaired by JRO), during 2345–0125+1 UTC on Friday 16 Jan. 2026 (chaired by JRO), and during 1300–1510 UTC on Tuesday 20 Jan. 2026 (chaired by E. Alshina and F. Galpin).

JVET-AO0055 AHG11: Dynamic convolution for LOP6 neural in-loop filtering [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

This contribution describes the use of a dynamic convolution layer as a replacement for static convolution in a LOP6 neural-network-based in-loop filtering model. The proposed layer maintains the same convolution interface (e.g., kernel size, stride, padding, and grouping) while selecting a per-sample convolution kernel as a weighted combination of multiple expert kernels. The expert weights are derived from simple global statistics of the input feature map (i.e., per-channel mean and mean absolute value) followed by a linear router and hard-sigmoid gating. Although the proposed approach increases the number of parameters, the measured MAC count remains essentially unchanged compared to the anchor model, targeting improved coding performance without increasing MAC-based computational complexity. Using the SADL count_mac tool for SADL floating-point CPU inference with an input size of 144×144, the proposed dynamic convolution model has 1,183,156,516 MACs, 860k parameters, and 3.6592 MB parameter memory. Under common test conditions, the proposed method provides PSNR improvements of Y: 0.72%, U: 5.23%, and V: 5.48% in RA configuration.

It was asked which loss function was used? Same as for LOP, also no other specific training tricks were used.

Attention module was not changed. It was commented that this might be replaced. It has been tried, but so far not successful.

It was commented that the memory size and number of parameters is significantly increased relative to LOP6 (factor 3-4), and training time also increases.

So far, no integer model has been investigated due to lack of time.

Several experts expressed interest to study the proposal in EE. In this context, performance should be studied with a version that has equal or lower number of kMAC/pix than current LOP, integer implementation, and study reducing the number of parameters.

JVET-AO0275 Crosscheck of JVET-AO0055 (AHG14: Dynamic convolution for LOP6 neural in-loop filtering) N. J. Gadgil, S. Watsa, R. N. Gadde, W. I. Choi, K. P. Choi (Samsung) [late]

JVET-AO0127 AHG11/AHG14: Improving Training Reproducibility in NNVC [H. Cho, S. Bahk, H. Y. Kim (KHU), D. Kim, S.-C. Lim (ETRI), T. Yang, W.-X. He, Q. Liu (HUST), J. Chi, L. Luo, Ce Zhu (UESTC)]

This document reports on the training reproducibility issues observed in the current NNVC software and proposes a solution to address them. It identifies that non-deterministic behaviors in trainer.py and environmental mismatches prevent identical training results. The proposed solution involves code-level modifications for deterministic operations and environment-level alignment. Experimental results verify that these changes restore intra-machine reproducibility. Additionally, the results demonstrate slight improvements in inter-machine reproducibility.

It was commented that trying to enforce deterministic behaviour to training procedure may not be desirable, as deviation when using different initialization might actually be an indicator that a network might has converged to a kind of random behaviour in terms of its performance.

It was commented that deterministic training using different machine environment may also be difficult, exact match may be almost impossible.

Decision(SW): It was agreed to include the method proposed in JVET-AO0127 as an optional element of the NNVC software, which may be used in addition to the current training scripts in cases where the training crosscheck is not successful. It may also be helpful as tool to identify implementation problems in the context of training verification.

JVET-AO0144 AHG11: Enhancing LOP6 with Re-Exploited Boundary Strength Guidance [X.-T. Xie, W.-X. He, T. Yang, Q. Liu (HUST)] [late]

This contribution proposes a Re-Exploited Boundary Strength Guidance module for LOP6, which fuse the input boundary strength (BS) again after backbones. The performance of our float model compared to the NNVC-15.0 anchor (NNIntra+LOP6) is reported to be:

AI: { -0.05%, -0.22%, -0.20%}

RA: { -0.19%, -0.10%, -0.09%}

It was asked why the performance for class A1 is much worse in AI than in RA.

The model is currently a float model. How would integerization be implemented for the sigmoid function that is used in the model?

Study in EE. Investigate reduction of numbers of kMAC / parameters to those in current LOP, create an integer version of the network, visual assessment of cases with extreme behaviour (highest/lowest gain) to identify potential problems. Also clarification about usage of boundary strength should be provided

JVET-AO0147 AhG11 Huawei and HiSilicon response to the MPEG Survey on AI reproducibility and NN model hosting [E. Alshina, Y. Zhao (Huawei), Q. Wu (HiSilicon)]

This document provides a response to the survey on AI reproducibility and NN model. The response was designed considering the compression standards, including end-to-end learning-based compression systems and hybrid compression systems using AI-enabled modules. Identical contribution is submitted to MPEG Requirements (WG2).

Notes by F. Galpin

Answers in the context of JPEG AI, VVC/H.266, next generation video codec standard (beyond VVC/H.266).

AI accelerators claimed to be non-deterministic when using floating-point. Integer operations need some care. One example is JPEG-AI entropy decoder.

In JPEG-AI, bit exact behavior is required at least on the entropy decoder and guaranteed.

Some other parts can allow non bit exact reproducibility but conformance need to be defined.

In first version of JPEG-AI, bit exact reconstruction was not part of specification. Recently, a bit exact version has been proposed for use case and requirement, using same methodology as for the entropy decoder part.

JVET-AO0148 AhG11 Overflow aware neural-network model quantization [E. Alshina, T. Solovyev (Huawei)]

This contribution provides details of overflow aware neural network quantizer which guarantees bit-exact behaviour and so device interoperability. Additionally to quantizing to integer values neural network the sum of model parameters’ absolute values must not exceed ration between max values of accumulation register and temporal buffer.  It is recommended to conduct theoretical analysis for dynamic range of neural-network based algorithms and provide mathematical proof ensure bit-exact behaviour as condition for adoption to the standard draft (if any neural-network based tool is considered).

Notes by F. Galpin

Similar methodology to what is done in legacy video codec design is used: design the operations for the worst case inputs in term of accumulation/overflow. It is part of the training process.

Proponent was asked for testing the training integration on current NNVC tools. Currently, not tried but believed to be possible with NNLF tools.

Proponent recommended to conduct theoretical analysis for dynamic range of neural-network based algorithms and provide mathematical proof ensure bit-exact behaviour as condition for adoption to the standard draft.

JVET-AO0173 [AHG11] A Hybrid Framework Integrating End-to-End Learned Intra-Frame Codec with Conventional Codec [N. Zou, A. B. Koyuncu, A. Hallapuro, F. Cricri, H. Zhang, J. Ahonen, M. M. Hannuksela (Nokia)]

This contribution proposes a hybrid framework that integrates end-to-end learned intra-frame compression (NLIC) methods with conventional compression techniques. The framework involves using NLIC-coded intra frames and VTM-coded inter frames. Furthermore, for each intra frame, the encoder decides whether to code it with NLIC or with VTM. Two sets of results are provided, depending on whether the NLIC was optimized by means of perceptual finetuning (PFT). With this hybrid framework, under the Random-Access configuration, the resulting BD-rates over NNVC-15.0 VTM (with NN tools off) anchor are reported to be as follows:

The system optimized with MSE and rate losses (no perceptual finetuning):

Overall  	-0,66  % (Y), -2,90  % (Cb), -1,31  % (Cr)

Additionally, the Random-Access simulation results are evaluated with 8 perceptual metrics, the resulting perceptual BD-rates over NNVC-15.0 VTM (with NN tools off) anchor are reported to be as follows:

|  | AVG | msssim Torch | Vif | Fsim | nlpd | iw-ssim | vmaf | psnrHVS | lpips |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| W/O PFT | -1,57% | -1,87% | -2,17% | -1,15% | -0,90% | -1,57% | -0,64% | -0,66% | -2,69% |
| W/ PFT | -5,59% | -8,48% | -4,57% | -3,33% | -3,18% | -6,91% | -1,16% | -1,24% | -11,49% |

It was noted that there is no gain in AI – proponents explain that the model was not optimized for that case (models were optimized for different QP), such that the encoder always decides for VVC intra.

The algorithm is the same as in EE1-6.

It was asked what the benefit would be compared to the multi-layer framework (saving the HLS overhead)?

This cannot be directly compared, because the switching mechanism has not been used in the EE.

One option would be to investigate in the multi-layer framework what the benefit of the switching mechanism at picture level would be (which requires duplicate encoding AI/VVC), and analyze  from the bitstream how large the HLS overhead is.

It was commented that this could later be extended to local switching mechanisms.

Investigate in EE in the context of multi-layer interface.

JVET-AO0178 [AHG11] Stable float convolution for neural network inference [H. Zhang, F. Cricri, N. Le, N. Zou, B. Koyuncu, L. Murn (Nokia)]

This document proposes a mechanism for using a floating-point data format in convolution operations. It is reported that the proposed mechanism can achieve bit-exact reproducibility across different computation environments and that it completely avoids overflows. The proposed method uses the mantissa bits and sign bit of floating-point numbers in convolution calculations, while keeping bias addition and ReLU/PReLU operations in real floating-point numbers. Float32 is the only data format for hardware performing neural network inference. It is reported that the proposed method, applied to the NNLF in NNVC software, achieves similar performance as the floating-point and int16/int32 format using the SADL library, with about 20% of decoding time when the NNLF is run on a GPU and the rest on a CPU for class B sequences.

The authors would like to donate the SW to NNVC, as an optional NN inference mechanism, so JVET experts can study the mechanism further and can choose to report runtimes with neural network-based coding tools running on GPU in addition to CPU.

IEEE 754 specifies float point arithmetic, but summation of more than two summands and overflow are  not deterministic.

Inputs of quantized convolutions are quantized: weights are quantized off-line, but temporal buffer values are quantized during inference. During the inference it is needed to compute maximum abs value for input (whole tensor).

Conceptually the same as JVET-AO0148, except:

- de-scaling factors are float in JVET-AO0178 (power of 2 in JVET-AO0148),

- de-scaling factor are decided during inference (dynamic quantization), but de-scaling factors in JVET-AO0148 are off-line trained (static quantization).

For some operations (such as max pooling, convolution, multiplication with constant, …) proponent implemented proposed approach in addition to SADL. Some operations such as Sigmoid, Tanh, GeLu, power function, exponential are not supported by proposal (and there is no plan to support). With specific version of CUDA, forced deterministic behavior, TF32 disabled the results of LOP6 are claimed to be reproducible across devices, some minor losses are observed in test. For some sequence (class A1) abnormal behavior was observed.

Several experts gave multiple examples of not reproducible operations IEEE 754, because IEEE 754 was not targeting bit-exact reproducibility.

It was confirmed (by NNVC SW coordinators) that NNLF in NNVC is not bit-exact and might have miss match (drift) for video signals out of CTC.

It was commented that run time saving in proposed implementation for GPU tests compared to CPU is lower than expected and was demonstrate in earlier contributions. Proponent confirmed that on-line decision of de-scaling factors (dynamic quantization) is the major source of high increase time.

One expert commented that proper name for technology is operational bit-exact implementation (bit not theoretical bit-exact in the worst case).was commented that it would be beneficial to made SW available to JVET members.

Study in EE1 as alternative implementation of LOP6, check speed benefits, bit-exactness and performance implication was agreed to be the practical way to made this implementation available.

JVET-AO0191 [AHG11]: DCVC-RT as an End-to-End Learned Intra Frame Codec [M. Aderdor, T. Solovyev, E. Alshina (Huawei)]

This contribution reports the coding gains when using DCVC-RT as an I-frame coder on top of VTM 23.13. DCVC-RT is the next iteration in the DCVC series after DCVC-FM. The external decoder has a complexity of 365 kMACs/pixel and 36.4 M Parameters. The coding gains are as follows:

On top of NNVC 15.1 configured as VTM 23.13:

AI: 		-8.4% (Y) 		-16.1% (U)		-15.4% (V)

RA: 	-1.7% (Y)		-8.4%   (U)		-5.4%   (V)

On top of NNVC 15.1 configured as VTM 23.13 + DCVC-FM I-frames:

AI: 		-3.3% (Y) 		-5.3%   (U)		-5.2%   (V)

RA: 	-0.7% (Y)		-2.7%   (U)		-2.1%   (V)

Include in EE as replacement for DCVC-FM

JVET-AO0194 [AHG11]: Luma Chroma Balancing for DCVC End-to-End Learned Intra Frames [M. Aderdor, T. Solovyev, E. Alshina (Huawei)]

This contribution presents the effect of using chroma QP offset to shift coding gains from chroma to luma in the case of external I-frames. Learning based end-to-end codecs such as DCVC-FM and DCVC-RT are trained on RGB / YUV444 based loss functions that treat all channels equally. When applied to  YUV420, these codecs tend to show disbalanced results with much higher gain in the chroma components. Previous contributions with DCVC codecs all used a pre- and post-processing method to shift coding gain from chroma to luma. In this contribution, we aim to shift gains from chroma to luma more efficiently by using chroma QP offset. CTC results using DCVC-RT I-frames on top of NNVC 15 configured as VTM 23.13 are the following:

With DCVC-RT as an external I-frame coder, no chroma scaling:

RA: 	-1.0% (Y)		-21.8%   (U)		-18.8%   (V)

With DCVC-RT as an external I-frame coder with ChromaQPOffset=2:

RA: 	-3.5% (Y)		-6.00% (U)		-3.50% (V)

It was commented that be beneficial also to study for other algorithms if they might be improved in performance by such a modification.

It was agreed to use this configuration change along with DCVC-RT.

JVET-AO0247 [AHG11] JPEG-AI Profiling Report on Runtime and Energy Consumption [G. Paul, D. Bhowmik] [late]

JPEG AI was used as an example, it was run on CPU only, combination of CPU and GPU combination of CPU and FPGA. For this split entropy part was always kept on CPU. Data transfer between CPU and GPU/FPGA was taken into account. Both run time saving and energy consumption reduction was demonstrated by moving part of JPEG AI from CPU to GPU and FPGA (savings are more for FPGA).

Proponent plans to extend study and include HEVC to the test study.

Rate control was enabled for JPEG AI encoder (recommended to disable for apple-to-apple comparison with, let’s say, HM SW).

JVET-AO0249 AhG19 AhG11 InterDigital response to the MPEG Survey on AI reproducibility and NN model hosting [F. Galpin, R. James, D. Tian (InterDigital)] [late]

In this response, the analysis on the sources of reproducibility issues with AI accelerators is provided. Consequently, it is proposed to consider two use cases according to the levels of reproducibility for AI-based media compression standards.

- Hard reproducibility. As for legacy video coding conformance point, it imposes a bit exact match of the decoder. It imposes the highest level of constraints on hardware and follows the legacy standardization process for video codec.

Additionally to integer and overflow control all operations which are used must be standardized. For example, for sigmoid layer all platforms use different implementation.

- Soft reproducibility. It relaxes the requirements of bit exactness, while also relaxing the constraints on hardware, depending on the desired level of reproducibility.

For WG7 best efforts on elimination randomness are taken, but some deviation is allowed. Entropy is bit-exact, as in  JPEG AI. Normative tool tools which provides soft reproducibility proven on operational manner (not mathematical proof, but tests on some test set) is under study in the core experiment.

Training reproducibility is highly desirable (same position was expressed in JVET-AO0147).

It is suggested to conduct additional survey on availability and reliability of integer operations available  in AI accelerators.

For ‘2D video coding’ standard development (traditional scope of JVET) bit-exact reproducibility should be a goal. If it is achievable with available AI accelerators it is not clear at this point. But AI accelerators are evolving quickly. One expert confirmed that with available generic AI accelerators many operations are not available in integer and bit-exact form.

Integer implementation also helps to reduce power consumption which can be another bonus.

JVET-AO0267 AHG11: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement T. Shu, X. Chen, W. Zhang, N. Fu, Z. Chen (Wuhan Univ.) [late]

This contribution proposes a Very Small Deep Reference Frame (VS-DRF) network with an inference complexity of 8.8 kMAC per pixel and 243k parameters. Building upon the DRF network in NNVC, the proposed model reduces computational complexity by removing the dependency on external pre-trained optical flow estimation network IFRNet and adopting the depthwise separable convolution blocks. The model is implemented using SADL with int16 fixed-point inference. Experimental results on NNVC-15.0 under the RA configuration show overall BD-rate gains of −0.42%, −0.27% and −0.64% for the Y, U and V components, respectively. The corresponding encoding and decoding time ratios are 106% and 298% of the anchor, respectively.

It was commented that this would be an interesting complement with the other operation points of DRF.

Investigate in EE. Training crosscheck, number of channels should be multiple of 16 to speed up run time. It was commented that, if verified, this could be a good candidate as additional tool in NNVC default config.

It was further commented that it would be interesting to analyse the overlap in terms of compression gain with the other NNVC default tools. Therefore, also results on top of VTM (without other tools) should be reported.

JVET-AO0276 Crosscheck of JVET-AO0267 (AHG11: Very Small Deep Reference Frame Generation Network for Inter Prediction Enhancement) [N. Bhaskar (Huawei)] [late]

JVET-AO0286 Methodology to Estimate Energy Consumption of Neural Network Inference on Mobile Devices [S. Caglar, Z. Chen, E. Steinbach (TU Munich)] [late]

This contribution reports an analysis and a measurement methodology for evaluating the energy consumption of operation-level neural network inference on mobile devices. Runtime and power are measured for representative neural network operations, including convolution, fully connected, pooling, and skip connections across different devices and hardware platforms. By varying individual operation parameters in controlled experiments, it is observed that multiply-accumulate operation (MAC) count tends to correlate more strongly with runtime than with power, whereas power measurements exhibit greater variability and a stronger dependence on the underlying device and hardware. Energy consumption is considered as the joint effect of average power and runtime over a defined inference interval. Based on these observations, this contribution discusses energy as an additional complexity dimension beyond MAC count and runtime alone and outlines a measurement-oriented methodology that may support future device-aware energy evaluation of neural network inference.

Was presented Wed. 21 Jan. 1305 (chaired by JRO)

It was commented by another expert that indeed other aspects such as memory access are relevant for power consumption than kMAC/pix, and there is high dependency also on the overall complexity of the network.

It was further commented that in the context of JPEG-AI a table was developed which considers the contribution of various operations to power consumption.

It was commented that in the context of video also the realtime capability (time for decoding frame) is important. It was also commented that a video algorithm would most probably require dedicated hardware which in terms of power consumption might be much more efficient than either CPU or GPU. In that context, operations per sample might be the most important factor.

It was commented that from the graphs NPU seems to have most benefit for the more complex networks.

Further study is welcome.

### SADL and NNVC implementation, CTC (7)

Contributions in this area were discussed during 1325–1440 UTC on Wednesday 21 Jan. 2026 (chaired by JRO).

JVET-AO0260 AhG14: SADL update [F. Galpin (InterDigital)]

- New in version v15

  - Improvements

  - MR 174 SIMD Int32 ReLU

  - MR 173 Extend concatenation

  - MR 169 SIMD int32 add

  - MR 171 Extend gridsample

  - MR 170 Extend resize

  - MR 185: add overflow also on accumulator

  - MR 183 AhG14: Improvement of SIMD implementation with expanded SIMD operators in SADL for LOP with Overlapped Feature Integration

  - New

  - MR 178 New Floor layer

  - MR 168 New Sub layer

  - Fixes

  - MR 175 Flatten alignment with onnx

  - MR 166 Batch > 1

  - MR 164 ReduceMean

  - MR 180 fix bad dispatch of max pool simd

  - MR 182 fix bicubic/nearest zeros padding mode

  - MR 187 Fixes and SIMD support for Kernel Size = 5

  - MR 186 detect possible issues in maxpool

- New proposed in version v16

Already merged (from JVET-AN0222):

  - MR 190 JVET-AN0222 New Depth2Space layer

  - MR 191 JVET-AN0222 New Abs layer

Several MR in the queue:

- new layers:

  - MR 189 JVET-AN0222 New Clip layer

  - MR 192 JVET-AN0222 New BitShift layer

  - MR 177? JVET-AN0197 New Gather layer

  - MR 176? JVET-AN0197 New GatherElements layer

- Improvement:

  - MR 197 JVET-AO0100 SIMD for slice

  - MR 196 JVET-AO0100 SIMD biasadd

  - MR 195 JVET-AO0100 SIMD concat

  - MR 194 JVET-AO0100 better SIMD mul

  - MR 193 JVET-AO0100 better SIMD add

Several contributions improving SADL (no MR yet):

| JVET-AO0056 | AHG14: GlobalAveragePool layer for SADL | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| --- | --- | --- |
| JVET-AO0057 | AHG14: SADL viewer module using Netron | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0058 | AHG14: Bug fix for Conv2D sampling grid misalignment in SADL | W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS) |
| JVET-AO0073 | AHG14: Decoding time reduction in SADL using SIMD acceleration | J. Han, C. Jung, Q. Qin (Xidian Univ.) |
| JVET-AO0217 | AhG14: Improved SADL implementation | D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft) |

For information – no specific decision necessary on JVET-AO0260.

JVET-AO0056 AHG14: GlobalAveragePool layer for SADL [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

This contribution presents an extension to the Small AdHoc Deep Learning (SADL) library by adding a new layer: GlobalAveragePool. The extension enables an efficient and resolution-agnostic implementation of global spatial averaging, which is frequently used in modern neural-network blocks (e.g., routing/conditioning).

The proposal was supported to be beneficial. Regarding the implementation of division, care should be taken in the context of implementation/merge request.

It was asked if the network graph was static. This was confirmed. When the model size is changed, a new graph would be constructed in SADL.

Decision(SW): Adopt JVET-AO0056.

JVET-AO0057 AHG14: SADL viewer module using Netron [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

This document provides an informative description of a Netron model reader module that enables visualization of SADL models. The module is intended as an auxiliary tool to assist inspection and debugging of SADL graphs (e.g., validating converter results) and does not propose any normative change to SADL or NNVC. The module is available from the author upon request.

It was commented that it would be desirable to include this in SADL, but it needs to be clarified if it would be possible depending on usage conditions of Netron.

It was agreed that some mechanism should be found to make the module accessible to EE1 participants. Proponents of JVET-AO0057 and SADL coordinator are tasked to clarify.

JVET-AO0058 AHG14: Bug fix for Conv2D sampling grid misalignment in SADL [W. Gwun, K. Choi (KHU), B.-S. Kim, I. Cho, S. Hahm (KBS)]

This contribution proposes a bug fix in the SADL Conv2D implementation (i.e., ixj path) for stride-known configurations. The current core loop applies a half-stride start offset independent of output index offsets, which can misalign the sampling grid in no-padding configurations and lead to incorrect output values. The proposed change conditions the half-stride offset on the output index offsets and updates the corresponding AVX2 implementations.

It was commented that for bugs found in an existing module, it is better to make a report and merge request immediately. A meeting contribution could be useful if the fix is not straightforward.

It was further commented that most likely current tools of NNVC would not be affected by this bug.

Agreed that the bug fix appears beneficial.

JVET-AO0073 AHG14: Decoding time reduction in SADL using SIMD acceleration [J. Han, C. Jung, Q. Qin (Xidian Univ.)]

This contribution presents methods for accelerating several operators in the Small Adhoc Deep Learning Library (SADL) using SIMD acceleration. Experimental results demonstrate that the proposed operators significantly reduce decoding time in SADL.

Additionally, an alignment of negative clipping range (from -32767 to -32768) is suggested.

It was commented that separate merge requests would be necessary for each element. The clipping issue is also raised in JVET-AO0217.

It was commented that the proposed accelerations for convolutions 1x1 and 3x3 might only work if the number of channels is a power of two.

Agreed in principle, to be further clarified with SADL coordinator which of the elements could be harmonized with speedups proposed in another contribution. See further notes under JVET-AO0217.

JVET-AO0100 AhG14: SIMD Improvements of Operators in SADL Library [L. Qin, N. Fu, Z. Chen (Wuhan Univ.)]

This contribution presents the SIMD improvements of operators in the Small AdHoc Deep Learning (SADL) library from Wuhan University. These improvements include layers: Mul, Biasadd, Add, Slice and Concat. The implementation utilizes the AVX2 instruction set. The experimental results demonstrate an improvement of the decoding time for the extended implementation.

Decoding runtime of NNVC15 default is reduced by approx. 7% consistently over all classes.

Merge request was already issued. Code was inspected by SADL coordinator and found appropriate.

Decision(SW): Adopt JVET-AO0100.

JVET-AO0217 AhG14: Improved SADL implementation [D. Rusanovskyy, J. Lainema (Nokia), D. Tolmachov, D. Shevchenko, I. Zaporozhets (Aimprosoft)] [late]

This contribution proposes a modification to the SADL inference of NNVC NNLF. By extending SIMD AVX2 implementations to selected layers of SADL, NNLF inference run-time reduction of 2% for EncT and 46% for DecT is achieved for LOP and VLOP anchors.  It is proposed to adopt proposed SADL changes into the reference NNVC software.

Reported performance of the proposed changes on HOP, LOP and VLOP anchors (NNVC15 CTC, with nnIntra ON) in RA configuration is following:

NNLF			Y		Cb 		Cr 		EncT  	DecT:

HOP:			0.00%		0.00%		0.04%		82%	71%

LOP6:			-0.01%		0.00%		0.05%		97%	54%

VLOP4:		0.00%		-0.01%		-0.01%		99%	54%

VLOP3:		0.01%		-0.01%		0.02%		98%	54%

Performance of the proposed changes for VLOP4 with nnIntra OFF in RA:

vs. respective anchor, RA:

VLOP4 only:		0.0%		0.0%		0.0%		98%	50%

vs. VTM, RA

VLOP4+nnIntra	-6.1%		-7.5%		-6.0%		113%	880%

VLOP4		-4.5%		-6.1%		-4.4%		101%	795%

To improve run-time of the inference for LOP and VLOP NNLF models, SADL reference inference software was modified by more extensive use of SIMD AVX2 optimized code for several convolution layers, such as conv2d_1x1, conv2d_3x3 and conv2d_jxi, as well as bias_add.

In particular, the following changes were introduced:

-	Convolution 1x1, 3x3:  Vector implementations, including those with number of channels not multiple of 8.

Processing of 8 kernels in parallel in a single batch for convolution and output process (add and saturate), when possible. Processing of remaining kernels (not dividable by 8) is performed by vector implementation with mask. Optimization to reduce data loading and output.

-	Convolution jxi:

Source code optimization, removal of conditional execution within the loop by precomputing indexes, this allow more efficient vectorization which was implemented.

-	biasadd:

Optimized loading process, vectorization of the shifts with support of data extension to 32 bits to allow correct saturation process.

-	Alteration of the saturation range to [-32768, 32767] is also introduced to scalar implementation to achieve alignment with SIMD AVX2 code.

-	Reduced data bandwidth between CPU and memory.

Proposed changes to the reference SW have been submitted as MR199.

It was asked which of the changes contributes how much to the run time reduction. It was estimated to be approximately equal (except for saturation range).

Changes agreed in principle.

Merge request received for convolution elements. It was requested to issue merge requests for each element separately. In cases of overlap (two different implementations of same approach) the proponents of JVET-AO0073 and JVET-AO0217) should negotiate with each other, and with the SADL coordinator which is the better approach. In case of no consensus, SADL coordinator has discretion to decide.

It was commented to further study the impact on encoder runtime, in particular when VLOP and NN intra are combined.

It was commented by proponent that the small deviation in performance may be due to the different precision in biasadd. This should also be further studied in the context of enabling the different elements separately.

JVET-AO0289 Crosscheck of JVET-AO0217 (AhG14: Improved SADL implementation)  D. Appelgren, J. Ström (Ericsson) [late]

## AHG6/AHG12: Enhanced compression beyond VVC capability (39)

### Summary and BoG reports

Contributions in this area were discussed during 2320–0125+1 UTC on Wednesday 14 Jan. 2026 and during 2330–0050+1 UTC on Thurssday 15 Jan. 2026 (chaired by JRO).

JVET-AO0024 EE2: Summary report of exploration experiment on enhanced compression beyond VVC capability [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)]

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, transform and coefficient coding, and in-loop filtering.

The software basis for this EE is ECM-19.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-19.0. ECM-19.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-an-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-an-ee2/simulation-results if cross-check reports are not submitted as input contributions.

Issues

Test 1.1: initial release was 2 weeks after the deadline.

Test 1.3: initial release was 2 weeks after the deadline.

Test 1.10: no initial release, the software was committed on the final release deadline.

Test 2.1: no initial release, the software was committed on the final release deadline.

Test 2.2: no initial release, the software was committed on December 15th, near the final release deadline.

Test 2.3: no initial release, the software was committed on December 18th, near the final release deadline.

Test 2.5: no initial release, the software was committed on the final release deadline.

Test 4.2: no initial release, the software was committed on January 7th, which is 2 weeks after the final release deadline.

List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Reference sample generation for intra prediction | Thuong Nguyen Canh (Dolby) |  |
| 1.2 | Combined angular- and gradient-PDPC | G. Kulupana (Nokia) |  |
| 1.3 | Block prediction using the 8-tap cubic interpolation filter in SGPM | J. Lee (KAU) J. Lee (ETRI) | JVET-AO0134 N. Zouidi (Ofinno) JVET-AO0245 Y. Bai (ZTE) |
| 1.4 | Position-dependent weighted prediction extension | Y. Wang (ZTE) | S. Blasi (Nokia) |
| 1.5a | Test the threshold for block size larger than 32 | S.Wan (NWPU) S. Xie (ZTE) |  |
| 1.5b | No reference sample smoothing for all block sizes | S.Wan (NWPU) S. Xie (ZTE) |  |
| 1.6a | Adding DIMD and MDIP average mode to MPM | W. Niu (ZTE) | JVET-AO0243 J. Lee, S.-C. Lim (ETRI) |
| 1.6b | MDIP mode modification | W. Niu (ZTE) | JVET-AO0256 J. Cai (Transsion) |
| 1.6c | Test 1.6a + Test 1.6b | W. Niu (ZTE) | JVET-AO0255 Y. Liu (Transsion) |
| 1.7 | Multiple candidate selection for MDIP | L. Wang (Xidian Univ) | JVET-AO0230 Z. Lyu (vivo) |
| 1.11 | Test 1.6b + Test 1.7 | W. Niu (ZTE) L. Wang (Xidian Univ) |  |
| 1.8 | IntraTMP with DMVR | K. Naser (InterDigital) | JVET-AO0234 X. Zeng, W. Niu (ZTE) JVET-AO0239 M. Blestel (Ofinno) |
| 1.9 | TMRL angle offset refinement | G. Rath (InterDigital) | JVET-AO0242 D. Ruiz (Ofinno) |
| 1.10 | CCCM with clipping | L.-C. Xu (SYSU) | JVET-AO0269 Y. Wang (ZTE) |
| 2 Inter prediction |  |  |  |
| 2.1 | GPM without blending for screen content | J. Kim (WILUS) | JVET-AO0235 A. Tissier (Xiaomi) |
| 2.2 | On improving adaptive GPM blending | Z. Zhang (Transsion) |  |
| 2.3a | Sub-block merge mode availability extension with enlarged block set and without CU size limitation | Y. Liu (Transsion) | JVET-AO0229 F. Wang (OPPO) |
| 2.3b | Sub-block merge mode availability extension with enlarged block set and CU size limitation | Y. Liu (Transsion) | JVET-AO0229 F. Wang (OPPO) |
| 2.4a | Additional spatial adjacent candidates | C. Wang (OPPO) | JVET-AO0252 S.Hong (Nokia) |
| 2.4b | Modified temporal collocated candidates | C. Wang (OPPO) | JVET-AO0252 S.Hong (Nokia) |
| 2.4c | Number of pairwise-average candidates | C. Wang (OPPO) | JVET-AO0252 S.Hong (Nokia) |
| 2.4d | Test 2.4a + Test 2.4b + Test 2.4c | C. Wang (OPPO) | JVET-AO0252 S.Hong (Nokia) |
| 2.5 | Pairwise merge candidates with motion vector scaling | S. Hong (Nokia) | JVET-AO0223 C. Wang (OPPO) |
| 3 Transform and coefficients coding |  |  |  |
| 3.1 | Finding optimal simple quantizer borders and reconstruction points | M. Balcilar (Ofinno) M. Le Pendu (InterDigital) | JVET-AO0244 Y. Yu (OPPO) |
| 3.2a | Modified coding of a coefficient level in TSRC | R. Xu (Xidian University) Y. Yu (OPPO) | JVET-AO0259 M. Abdoli (Xiaomi) |
| 3.2b | Test 3.2a with low QPs | R. Xu (Xidian University) Y. Yu (OPPO) | JVET-AO0259 M. Abdoli (Xiaomi) |
| 3.2c | Test 3.2b with JVET-AN0086 encoding change enabled | R. Xu (Xidian University) Y. Yu (OPPO) | JVET-AO0259 M. Abdoli (Xiaomi) |
| 3.3a | Modified multiple transform set selection for inter LFNST/NSPT | J. Huo (Xidian University) |  |
| 3.3b | Modified Advanced SBT | J. Huo (Xidian University) |  |
| 3.3c | Test 3.3a + Test 3.3b | J. Huo (Xidian University) |  |
| 4 In-loop filtering |  |  |  |
| 4.1 | On improving the bi-filtering modes in TALF | P. Astola (Nokia) | JVET-AO0222 N. Song (OPPO) |
| 4.2 | ALF coefficient coding group extension | I. Jumakulyyev (Nokia) | JVET-AO0246 V. Shchukin (Ericsson) |
| 4.3 | Modified merge candidate list in ALF-CCCM | H. Zhang (OPPO) | JVET-AO0233 M. Jia (ZTE) |
| 4.4 | Improvement on frame-level inheritance mode in ALF-CCCM | M. Jia (ZTE) | JVET-AO0224 H. Zhang (OPPO) |
| 5 Other |  |  |  |
| 5.1 | Independent CTUs for TM-padding | N. Neumann (Nokia) |  |

Intra prediction

Test 1.1: Reference sample generation for intra prediction (JVET-AO0170)

In the test, unavailable reference samples for intra prediction are replaced with the corresponding same location samples in the collocated picture as shown in the next figure. If those samples are not available, then the currently existing padding method is used.

This method is applied to blocks larger than 8x8 and for luma only. The use of the collocated picture is constrained that the POC difference between the current and the collocated pictures should be smaller than 2.

Test 1.1a: Reference sample generation for intra prediction.

Test 1.1b: Test 1.1a with forward LMCS applied to the collocated samples.

Test 1.2: Combined angular- and gradient-PDPC (JVET-AO0142)

ECM includes two types of Position Dependant Prediction Combination (PDPC) applied to angular prediction modes. For positive angles closer to horizontal and vertical modes, gradient PDPC is applied. For angles further away from horizontal and vertical modes, angular PDPC is applied.

In the test, both angular and gradient PDPC is applied instead of just angular PDPC.

Test 1.2: Combined angular and gradient PDPC.

Test 1.3: On interpolation filter for SGPM (JVET-AO0110)

In ECM, the interpolation filter used in Spatial Geometric Partitioning Mode (SGPM) is selected based on the Mode-Dependent Intra Smoothing (MDIS) condition. When the MDIS condition is satisfied, a 6-tap or 4-tap Gaussian interpolation filter is selected. Otherwise, the cubic interpolation filter is used. The cubic interpolation filter employs 16-tap or 10-tap cubic filter based on the block width or height.

In the test, the switching between cubic and Gaussian interpolation filters in SGPM is removed, and a single 8-tap cubic interpolation filter is used for all cases.

Test 1.3: 8-tap cubic interpolation filter for SGPM.

Test 1.4: Position-dependent weighted prediction extension (JVET-AO0121)

In the test, the position dependent blending used in DIMD, TIMD, OBIC is modified as follows.

wix,y= wDimdi+∆i-2∆iy(H-1)  à wix,y= (wDimdi+∆i)(H-1-y)(H-1)

wix,y= wDimdi+∆i-2∆ix(W-1)  à wix,y= (wDimdi+∆i)(W-1-x)(W-1)

Where wDimdi is the weight of the DIMD mode used in the blending, ∆i is predefined and set to 10. The adjustment is applied when non-angular weight is greater than 0. It is asserted that the modification reduces the number of multiplications since it ensures that the weight is equal to 0 at block boundaries.

This modification is applied to the fusion modes of TIMD, OBIC, and IntraTMP.

For IntraTMP, the maximum weight deviation of the sample-based blending (range ∆i) is made CU-size dependant where for large CUs (greater or equal to 128 samples) the value is equal to 10 rather than 16.

Test 1.4: Position-dependent weighted prediction extension.

Test 1.5:  On filtering condition for angular modes (JVET-AO0140)

In Test 1.5a, the threshold for applying the {1,2,1}-reference samples smoothing filter and the Gaussian interpolation filter in MDIS is increased from block size 32 to 256.

In Test 1.5b, the use of both filters is disabled.

Test 1.5a: Threshold 256 for MDIS {1,2,1} reference filter.

Test 1.5b: No reference sample smoothing for all block sizes.

Test 1.6:  Improvement for candidates on MPM (JVET-AO0111)

In ECM, the MPM list is composed of the DIMD mode, dimdBlend modes, intra modes of the left (L), above (A), below-left (BL), above-right (AR), and above-left (AL) neighbouring blocks, and the intra modes of non-neighbouring blocks. If MPM sorting is enabled, up to 5 modes with the smallest SAD cost are added. The SAD cost is computed between the prediction and the reconstruction samples of the template. The directional modes with added offset are added into the general MPM list, and then the default modes, until the general MPM list with 22 entries is constructed. The MPM list is also used in TIMD candidate list generation process.

In the derivation process of the Most Dominant Intra Prediction (MDIP) mode, up to 8 candidates are derived using HoG and reordered based on the template cost. The candidate with the smallest cost is used as MDIP mode and it is signalled before MPM information at CU level. MDIP mode is excluded from MPMs. If there are no available candidates, then DC mode is used.

In Test 1.6a, the average of the DIMD and the MDIP modes is added to the MPM list between the DIMD and the dimdBlend mode.

In Test 1.6b, in the derivation process of the MDIP mode, angular modes are added ensuring that DC mode is included in the list.

Test 1.6a: Adding DIMD and MDIP average mode to MPM.

Test 1.6b: MDIP mode modification.

Test 1.6c: Test 1.6a + Test 1.6b.

Test 1.7: Multiple candidate selection for MDIP (JVET-AO0138)

In the test, intra prediction mode with the second lowest template cost is used as the second MDIP mode. One more bin is signalled to indicate whether the candidate choice.

Test 1.7: Multiple candidate selection for MDIP.

Test 1.11: Test 1.6b + Test 1.7 (JVET-AO0113)

Test 1.8: IntraTMP with DMVR (JVET-AO0183)

In the test, IntraTMP is supplemented with a blending using BVs considered during template matching.

Two BVs are selected from the IntraTMP list, the second BV is refined using the two reference blocks. To limit the number of computations, a block matching is done only if the distance between the two block vectors is larger than 3.

The final predictor is derived by blending two predictors corresponding to those BVs with weights {3,1}.

Test 1.8: IntraTMP with DMVR.

Test 1.9: TMRL angle offset refinement (JVET-AO0241)

In ECM, a TMRL list candidate is defined as a pair (multiRefIdx, intraDir), where multiRefIdx denotes the reference line index and intraDir denotes the index of the prediction mode.

In the test, a candidate pair is modified to be a tuple (multiRefIdx, intraDir, dirOffset), where dirOffset is a flag indicating if a direction offset is used in the prediction with zero indicating that no offset is applied.

The offset is computed from the mode angle parameter as follows.

If intraDir >= EXT_DIA_IDX, (that is, the mode is vertical)

gradientA(intraDir)  = ( A(intraDir)  >= 0 ? A(intraDir +1) : A(intraDir –1) ) – A(intraDir)

Else,

gradientA(intraDir)  = ( A(intraDir)  >= 0 ? A(intraDir –1) : A(intraDir +1) ) – A(intraDir).

The offset value for intraDir is defined as:

deltaAngle(intraDir) = gradientA(intraDir) / 2.

With the new definition of a TMRL list candidate, the list is generated in two passes. In the first pass, the dirOffset value is set to 0 for all candidates. In the second pass, all candidates with multiRefIdx >= minRefIdx (minRefIdx is set to 5 in the test) are checked with other values of dirOffset.

In each check, using the reference arrays (multiRefIdx) and the predictions mode (intraDir), the template is predicted again with offsets of intraPredAngle, and the list is updated with the new dirOffset value.

Test 1.9: TMRL angle offset refinement.

Test 1.10: CCCM with clipping (JVET-AO0159)

In the test, a new clipping constraint is introduced based on the chroma values within the reference area for CCCM and its variants. The clipping range is determined by the minimum and maximum chroma reference samples, rather than the entire valid pixel range [0, 2bitdepth - 1].

Additionally, in multi-model CCCM, the above-mentioned clipping range derived from the reference samples is applied only to model 1 (where co-located luma values are smaller than or equal to mean luma values), and not to model 2.

Test 1.10: CCCM with clipping.

Test 1.1: No benefit (small loss) on average, only some sequences in class F.

Test 1.2: Gain is very small in AI, and chroma loss in RA. Though it does not have impact on runtime, it requires some additional processing which does not justify to take action.

Test 1.3 avoids switching between interpolation filters, and even gives a very small gain. It was commented that at this moment, fine-granular optimization of ECM in isolated places is not of high priority.

Test 1.4 is a slight simplification with no gain, not of high priority for ECM development at this point.

Test 1.5 does not provide benefit, and the disabling of the smoothing filter in 1.5b does not show any significant change in runtime.

Test 1.6 has losses (particular in chroma) in RA, and more modes are added. No run time reported.

Test 1.7 has slight gain in AI (with run time increase, but small losses in RA (no run time reported).

Combination 1.11 (1.6b and 1.7) is similar as 1.7 standalone. No benefit overall for any of those.

Test 1.8 is not changing run time, as the newly introduced processing mode for refinement is not more complex than other existing modes. Some gain in AI, almost no gain in RA. Considering the amount of changes (>400 lines of code) the benefit in compression is not attractive.

Test 1.9 has small gain for AI, no gain for RA, but requires additional processing (two passes to generate the list). Not enough benefit to justify action.

Test 1.10 is providing some gain (mostly in chroma) in AI, but practically no gain in RA (on average, where losses occur in some sequences such as BQ Terrace). It is commented by cross-checkers that the approach is straightforward and simple to implement.

It was commented that clipping is not applied to all CCCM modes. There is a related contribution JVET-AO0146 that shows better gain when applied to all modes.

Further investigate in EE for further possible improvements, also to potentially avoid losses for some sequences in RA.

Inter prediction

Test 2.1: GPM without blending for screen content (JVET-AO0094)

In ECM, GPM employs five predefined blending widths determined based on the minimum of the block’s width and height. If the minimum of a block’s width and height is greater than 32, the GPM blending width is selected from {1/2τ, τ, 2τ, 4τ, 8τ}, otherwise, it is selected from {1/4τ, 1/2τ, τ, 2τ, 4τ}. A CU level index geoBldIdx corresponding to the selected blending width is explicitly signalled.

For SCC, the 1/4τ blending width is always used for SGPM-coded blocks, while GPM-coded blocks use the adaptive blending method.

In the test, for GPM blocks the blending width is restricted to 1/4τ and to 1/2τ for blocks with min(width, height) >= 32. This scheme is indicated by SPS flag.

Test 2.1: GPM without blending for screen content.

Test 2.2:  On improving adaptive GPM blending (JVET-AO0126)

In ECM, the selected blending width index, geoBldIdx, is encoded using a single syntax element. The encoding process first signals one bit indicating whether geoBldIdx equals to 2. If not, two additional bits are signaled to indicate the actual index value (0, 1, 3, or 4).

In the test, the signalling is modified for 8x8 blocks, to signal first whether the blending index geoBldIdx is equal to 0. It is asserted to be the most chosen options for those blocks.

The block size check whether is added for the index signalling whether it equals to 0 for 8x8 blocks.

Test 2.2: GPM blending index for 8x8 blocks.

Test 2.3: Extended availability conditions for sub-block merge mode (JVET-AN0092)

In ECM, sub-block merge mode is available when the CU is larger or equal than 8×8 with an area of at least 128 samples, or when it has an area of at least 32 samples and affine blocks are found among the 7 adjacent and 5 non-adjacent neighbouring blocks using sub-block merge, inter affine, GPM-affine or SGPM-affine modes.

In the test, the availability conditions include more adjacent and non-adjacent blocks in the checking process with four aspects.

- Edge scanning of finer-granularity adjacent neighbouring blocks

As shown in the below figure (a), along the left and top boundaries of the current CU, additional candidates are examined with a step size of 4 samples. If any of these adjacent neighbouring blocks are coded in affine mode, GPM-affine mode or SGPM-affine mode, the sub-block merge mode is enabled.

- Recursive extension of adjacent neighbouring blocks

As shown the below figure (b), beyond the direct left, top, and top-left CUs, additional "second-layer neighbors" are derived by applying additional offsets. These extended neighbours are also checked, and the presence of affine, GPM-affine, or SGPM-affine blocks activates the sub-block merge mode.

- Inclusion of non-adjacent neighbouring blocks

As shown in the next figure, a set of non-adjacent neighbouring blocks with more distant positions is obtained according to the width and height of the current CU (e.g., far left-down, far right-up, middle-top, middle-left). If one of these non-adjacent neighbouring blocks is an affine, GPM-affine, or SGPM-affine block, the sub-block merge mode is made available.

- Remove the limitation of CU Size

Test 2.3a:

The sub-block merge mode is available when the set of adjacent and non-adjacent blocks is found using sub-block merge, inter affine, GPM-affine, or SGPM-affine modes without limitation of CU size.

Test 2.3b:

The sub-block merge mode is available when the set of adjacent and non-adjacent blocks is found using sub-block merge, inter affine, GPM-affine, or SGPM-affine modes with the CU size larger than 32 samples.

Test 2.3a: Sub-block merge mode availability extension applied for all CUs.

Test 2.3b: Test 2.3a applied for CUs with area larger than 32.

Test 2.4: Additional candidates for regular inter AMVP candidate list (JVET-AO0068)

In ECM, regular inter AMVP candidate list is constructed using spatial adjacent candidates, temporal collocated candidates, spatial non-adjacent candidates, and HMVP candidates.

For TMVP, two collocated pictures are utilized to provide temporal motion information. For the first collocated picture, the motion shift for locating TMVP is adaptively determined from multiple locations according to template costs. While for the second collocated picture, the TMVP is derived by fetching the motion information only from the center or the bottom-right of the collocated block.

After refinement and reordering for the whole list, the candidate with the minimal template matching cost is selected as MVP.

In the test, 3 aspects are introduced for AMVP.

1.  MVs at additional spatial adjacent positions are used to construct the candidate list. As show in the next figure, a new above spatial candidate is fetched from neighbouring block B5 and a new left spatial candidate is fetched from neighbouring block A4. Two above-middle spatial candidates are fetched from neighbouring block B3 and B4, and two left-middle spatial candidates are fetched from neighbouring block A2 and A3. These additional spatial candidates are added into the list after the current existing adjacent spatial candidates and before the chained MV candidates.

2. The same adaptive method used for the first collocated picture is used to derive the TMVP candidates for the second collocated picture.

3. Depending on available space in the AMVP candidate list, up to two pairwise-average candidates may be added. The first pairwise-average candidate is derived from the first and second candidates in the AMVP candidate list after template matching reordering and refinement. The second pairwise-average candidate is derived from the first and third AMVP candidates. Each pairwise candidates’ MV will be refined as for the initial candidates. Finally, the AMVP candidate list is reordered again by template matching cost.

The whole process of adding new candidates to the regular inter AMVP candidate list is shown in the figure below.

Test 2.4a: Additional spatial adjacent candidates.

Test 2.4b: Modified temporal collocated candidates.

Test 2.4c: Number of pairwise-average candidates.

Test 2.4d: Test 2.4a + Test 2.4b + Test 2.4c.

Test 2.5: Pairwise merge candidates with motion vector scaling (JVET-AO0195)

In the current pairwise-average merge candidate derivation, some pairwise candidates are generated only when the two selected base merge candidates refer to the same reference index. When the reference indices are different, a pairwise candidate is not generated.

In the test, it is allowed to form pairwise merge candidates across different reference indices by using MV temporal scaling to project one MV to a reference picture with the other candidate’s reference index, while keeping the overall merge candidate list size unchanged. After projection, the pairwise-average candidate is derived in the same way as for the same reference index case.

Test 2.5a: Pairwise merge candidates with motion vector scaling.

Test 2.5b: Test 2.5a with inter TM disabled.

Results RA/LB

Test 2.1: Very small gain (considering that it is only beneficial for screen content). Cross-checker confirmed results and expressed support. No other experts expressed interest for adoption.

Test 2.2: No benefit (loss) in compression, but requires adding an additional check.

Test 2.3: Losses in compression, encoder/decoder runtime increased by around 1%.

Test 2.4: Three different elements tested separately in a-c, and combined in d (Combination was original proposal). No benefit shown for any of those, whereas more complexity is introduced in constructing the candidate list.

Test 2.5: Requires additional processing by generating more pairwise candidates, but resulting in slight compression loss.

Transform and coefficient coding

Test 3.1: Unbiased simple quantizer (JVET-AO0109)

In ECM, there are three different quantization methods: Dependent Quantization (DQ), Rate Distortion Optimized Quantization (RDOQ) and a simple uniform quantization named Uniform Reconstruction Quantizer (URQ).

In URQ, a coefficient is quantized by using uniformly distributed reconstruction points (integer center in the next figure). However, the quantization bounds are not equally distributed and are controlled on the encoder side by a bias parameter β. Given β<1/2 , the dead-zone (0th index quantization interval) is larger than the other quantization intervals whose size is equal to the quantization step size of the quantizer. Consequently, the reconstruction points are not in the middle of their quantization intervals, resulting in a bias of the URQ quantizer controlled by β. In current ECM, the value of β is inherited from HEVC as β=171/512≈1/3 for Independent Random-Access Point (IRAP) pictures, and β=85/512≈1/6 for the rest of the pictures.

If the given coefficient is t (and the absolute value is |t|), URQ finds its quantization index q by the following relationship:

q=signt.t∆+β.

Here, ∆ is the quantization step size, sign(.) is the sign operation that outputs the sign of the operand and . is the floor operation that outputs the integer part of floating points values.

In the dequantization process, the mapping of quantization indices q (dequantization process Q-1q) to reconstructed coefficients t' is given by the following relationship:

t'=Q-1q= q×∆.

In ECM, the quantization center shifting method further shifts the reconstruction points in the dequantization process, where the dequantized coefficient t’ is computed from the quantization level q and a lookup table T of QCS weights, as follows:

t'=1024-TqQ-1q+TqQ-1q'≫10    if  T>q>0

where t' is the dequantized coefficient,  T is the size of the lookup table, Q-1q is the dequantized value of the quantization level q, and q’ is the auxiliary quantization level

q'=q+q>0 ?1:-1.

Scalar quantization is suboptimal if the expected value of the coefficients that drop into the same quantization grid is not equal to the reconstruction point of that given quantization grid. If there is any mismatch between the expected coefficients value and the reconstruction points, it is called biased quantization.

It is claimed that dequantization centers are biased, so in the test a de-biasing of the URQ quantizer is performed by adding an offset to the reconstruction points shown in the figure below.

The de-quantization value (t’) is determined by the following equation where the quantization index of the corresponding coefficient is q and sign operation outputs the sign of the non-zero input and outputs zero for zero input.

t'=Qp-1q= q+signq(0.5-β)×∆

The offset of (0.5-β)×∆ is added to the absolute valued non-zero reconstruction points on to the reconstruction points of URQ. Since (0.5-β) is a floating-point value, the implementation is done in the form of the integer-based method of QCS framework with a modified lookup table. This table is derived from β, such that T0=0 and Tq=Wβ for q>0 with:

Wβ=round(0.5-β×1024×ρ),

where ρ is an encoder parameter, and round is the rounding to the nearest integer. Using ρ=1 corresponds to the reconstruction points in the middle of their quantization intervals. In the test, ρ=0.9.

Three SPS-level syntax elements are added, UBQ flag indicating the usage of the method followed by URQBeta0 and URQBeta1 to indicate the β parameter with fixed-length coded over 8 bits, corresponding to integer-based representation of β  as 512* β, β∈[0,0.5].

In Test 3.1a, the two coded values URQBeta0 and URQBeta1 are associated to IRAP and non-IRAP slices respectively. UBQBeta0 and UBQBeta1 are set to values inherited from HEVC:

UBQ                        :1

UBQBeta0			:171

UBQBeta1               :85

In Test 3.1b, the two coded values URQBeta0 and URQBeta1 are instead associated to the first Tid (Tid0) and the last Tid in RA configuration. The corresponding parameter values for intermediate Tids are linearly interpolated both by the encoder and decoder. The UBQ flag, UBQBeta0 and UBQBeta1 are set as in test 3.1a.

The tests are performed in non-CTC setting as follows:

DepQuant: 0

NumSignPred: 0

NumSignPredOverrideByQP: 0 0

RDOQ: 0

RDOQTS: 0

UBQ: 1

UBQBeta0: 171

UBQBeta1: 85

Test 3.1a: Dequantization offset based on IRAP slice type.

Test 3.1b: Dequantization offset based on temporalID.

Test 3.2: Coding of a coefficient level in TSRC (JVET-AO0069)

In ECM, some coefficient levels in a TSRC block may not be coded with a significant flag, so two bins are needed to code a coefficient level zero. When the pre-defined CABAC bin budget is exhausted, a truncated Rice-Golomb binarization method with Rice parameter 1 is used instead to binarize the coefficient levels directly. The binarized bins are coded in the bypass mode.

In the test, a context coded significant flag is always signalled to indicate whether the current coefficient level is zero.

To comply with the pre-defined CABAC bin budget constraint, the number of context bins equal to the total number of positions of all non-zero subblocks are reserved so that the significant flag for each position of all non-zero subblocks within a TSRC block is always coded with a context. The pre-defined CABAC bin budget is updated accordingly by subtracting the number of reserved context bins. The updated CABAC bin budget is then used to control the coding of other context coded syntax elements in the TSRC.

In addition, all significant flags are coded together within a subblock in a separate coding loop before coding any other context coded syntax element and sign flags may also be coded in a separate coding. As a result, there is no interleaving of context coded bins and bypass coded bins within a subblock, the same as the current ECM.

For low QP applications, there may be more large coefficient levels in a TSRC block. In the test, an adaptive Rice parameter is used to code such coefficient levels and signal the Rice parameter at slice level. An SPS flag is first signalled to indicate if an adaptive Rice parameter will be used or not. If the SPS flag is enabled, a Rice parameter is further coded for the current slice.

For CTC, the SPS flag is set to disable the usage of adaptive Rice parameter and the Rice parameter is set to 0. For low QP, the SPS flag is set to enable the usage of adaptive Rice parameter. Similar to VTM encoder is used to select the value of Rice parameter and further signal this parameter at slice header. Like the current ECM, the remaining coefficient levels are binarized with Rice-Golomb codewords and then coded in the bypass-mode.

Test 3.2a: Modified coding of a coefficient level in TSRC.

Test 3.2b: Test 3.2a with low QPs.

Test 3.2c: Test3.2b with JVET-AN0086 encoding change enabled

Test 3.3: On multiple transform set selection for inter LFNST/NSPT and advanced SBT (JVET-AO0251)

In ECM, multiple transform set selection (MTSS) for intra LFNST/NSPT is applied to IntraNN, DIMD, OBIC, TIMD, SGPM, MIP, EIP, and IntraTMP modes. For inter LFNST/NSPT, MTSS is applied to the GPM mode. The 1st and 2nd candidates should have different transform sets or different transpose types.

For inter MTSS, the 2nd IPM is directly derived from the mode with the 2nd highest HoG amplitude, without performing checks similar to those in intra MTSS. This may cause that the two candidates in inter MTSS to have the same or similar transform sets with identical transpose types.

In the test, two modifications are introduced.

1. A consistency check and adjustment for the 2nd candidate in inter MTSS when it becomes redundant with the 1st candidate.

For inter MTSS, if the 1st and 2nd candidates have the same or similar transform sets with identical transpose types, the 2nd candidate is modified. For NSPT, the 1st IPM is used to select kernels from set0 (regular intra mode kernels) instead of the 2nd IPM, aligning with the operation in intra MTSS. For LFNST, the 2nd IPM is replaced with its closest regular angular mode having different set (i.e., 65 or 3).

2. A fix in ASBT to ensure that the sliding-window search covers the entire TU region as shown in the figure below.

Test 3.3a: Modified multiple transform set selection for inter LFNST/NSPT.

Test 3.3b: Modified advanced SBT.

Test 3.3c: Test 3.3a + Test 3.3b.

Test 3.1b not relevant for AI and LB (no TID)

It was commented by cross-checkers that the size of the deadzone (not the offset) is signalled, and it would appear more straightforward to signal the offset rather than deriving it from the deadzone.

It was further commented that it might be more useful to make adaptation at picture level.

It was questioned what would be the benefit of introducing yet another type of quantization, which is not compatible with common encoder methods like RDOQ. It was also asked how much of the gain could be achieved by pure encoder optimization.

The anchor is non-CTC (no DQ, no RDOQ, no SDH, no sign pred.).

Discussion continued on Thursday 15 Jan. 2335

The difference between CTC the proposal is reported to be around 3%, giving 15% encoder runtime decrease in ECM compared to CTC.

The main intention is enabling lower complexity encoding. It was however commented that often real-world encoders are using some simplified version of RDOQ, and it would be desirable to see results with RDOQ and SDH enabled.

It was reported that approximately half of the gain can be retained by using the elements of encoder-only optimization.

Interest was expressed by other experts to investigate several aspects further in EE, such as signalling offset directly, usage of RDOQ/SDH, comparison against encoder-only solution. Also a comparison against CTC should be reported.

For test 3.2, It was asked which was the modification of Rice parameter in the low QP case? It was clarified that it is practically identical to method on VTM (SPS and slice-level signalling).

It was suggested that test 3.2b should better have been compared against an anchor with adaptive Golomb coding enabled.

It is guaranteed that the budget of context coded bins is never exceeded (unlike previous proposals)

It was commented that significant loss occurs in CTC sequences for LB. Therefore, proponents are only suggesting usage for screen content classes.

It would be undesirable to define a different method of budget constraint only for screen content classes. Furthermore, in an exploratory software as ECM this does not have any priority.

For test 3.3, results are largely incomplete, where the partial results available show non-uniform behaviour over different classes and sequences, and for LB where the results are more complete, loss is observed on average.

In-loop filtering

Test 4.1: On improving the bi-filtering modes in TALF (JVET-AO0093)

In ECM, Temporal adaptive loop filter (TALF) uses the reconstructed pixels in the reference pictures, and the motion-compensation padded samples located in the padded area of the reference. TALF has four uni-filtering modes and two bi-filtering modes. The bi-filtering process is given by the following equation,

o=c(i,j)×K((r0x'+i,y'+j+r1x'+i,y'+j)>>1)-saoLumax,y,b

In the above equation,

- o is the offset generated by TALF filter and added to the luma ALF output;

- c(i,j) is one of the positions within the filter window;

- r0 represents the reference samples from the reference picture 0

- r1 represents the reference samples from the reference picture 1

- (x,y) is the current filtering position in the current picture;

- (x',y')  is the collocated position or the MV guided position in the reference picture;

- K(d,b) is a clipping function which restricts the value d within the range from -b to b;

- saoLuma represents the output samples after the SAO filtering.

Two filter shapes are employed in TALF.

In the test, the computation of the average from the 3x3 centermost filter inputs is removed, and instead the samples from the reference pictures are treated independently. Additionally, one input is used as a bias term.

It is asserted that this results in a simplification due to the removal of 11 additions and right shifts per filtered sample. Also, one addition is removed due to the use of the bias term, as the bias term is just one value instead of sum of two values in the symmetric kernel. In total the number of additions required per filtered sample is reduced from 26 to 14 i.e., nearly by 50%.

In the test, the number of filter inputs remains 13 for both filter shapes as summarized in the next table.

| Input # | Source | Index in filter kernel |
| --- | --- | --- |
| 0 | ref pic 0 | 0 |
| 1 | ref pic 0 | 1 |
| 2 | ref pic 0 | 2 |
| 3 | ref pic 0 | 3 |
| 4 | ref pic 0 | 4 |
| 5 | average | 5 |
| 6 | average | 6 |
| 7 | bias | undefined |
| 8 | ref pic 1 | 4 |
| 9 | ref pic 1 | 3 |
| 10 | ref pic 1 | 2 |
| 11 | ref pic 1 | 1 |
| 12 | ref pic 1 | 0 |

Test 4.1: Modified bi-filtering modes in TALF.

Test 4.2: ALF coefficient coding group extension (JVET-AO0132)

In ECM, ALF coefficient filter taps for online training includes spatial taps (#0 - #9), fixed filter output-based taps (#10 - #29), reconstruction buffer before DBF (#30 - #32), residual based taps (#33 - #34), Gaussian smoothed before DBF reconstruction buffer (#35), and Laplacian taps (#36 - #40).

ALF coefficients are coded by Huffman coding in two separate coding groups. Group 1 is used for the center coefficients and Group 0 is used for the rest of the coefficients.

In the test, center, direct neighbouring and distant coefficients are signalled separately in different groups:

- Group 2: For direct neighbouring coefficients (shown in green).

- Group 1: For center coefficients (shown in grey).

- Group 0: For the rest of the coefficients (shown in white).

The existing frequency table extended and re-trained, i.e., for the new group, Group 2, frequency numbers are derived, and the existing group frequencies are updated by running ECM encoder on BVI-DVC dataset.

Test 4.2a: Retrained Huffman tables for ECM.

Test 4.2b: ALF coefficient coding group extension.

Test 4.3: Additional candidates for merge mode in ALF-CCCM (JVET-AO0070)

In the current ALF-CCCM, a CTU may inherit, through a merge list, both the block size and the model type from the above or left CTU. The usage of this merge mode is signalled using a single CABAC coded flag for each CTU when ALF-CCCM is present in the left and/or above CTU. If both left and above CTUs are coded with ALF-CCCM, another CABAC coded flag is signalled to indicate which CTU’s ALF-CCCM parameters are used in coding of the current CTU.

In the test, ALF-CCCM merge list is extended by including more spatial adjacent CTU candidates so that the current CTU may have more choices of block size and model type from the merge list to inherit.  Additional CTU candidates from top-left CTU are checked if the left CTU candidate is not available and the top-right CTU will be checked if the top CTU candidate is not available. To remove redundant candidate in the list, a pruning operation is applied.

The merge list size is kept unchanged, and it contains no more than 2 CTU candidates.

Test 4.3: Modified merge candidate list in ALF-CCCM.

Test 4.4: Improvement on picture-level inheritance mode in ALF-CCCM (JVET-AO0141)

In ALF-CCCM, a picture may inherit the block sizes and model types for all CTUs from a reference picture. The reference picture is derived from the L0 and L1 lists. Only reference pictures with ALF-CCCM present in at least one CTU are considered. The reference picture with the smallest POC distance to the current POC is selected. If enabled, the picture level inheritance skips the CTU-level signalling completely for the current picture. This choice is signalled using a single CABAC coded flag if a reference picture with at least one CTU utilizing ALF-CCCM is present.

In the test, the maximum number of inheritable reference pictures is increased from 1 to 2. Two reference pictures with the smallest POC distance to the current POC are considered, where the block sizes and model types of all CTUs in these two reference pictures are not identical.

If at least one reference picture is available, a flag will be signalled to indicate that the picture-level inheritance mode is used. If two reference pictures are available, an additional flag will be signalled to indicate a choice of the picture.

Test 4.4: Modified picture-level inheritance mode in ALF-CCCM.

Test 4.1 is complexity reduction by reducing the number of additions in TALF by approx. 50%. This leads to approx. unchanged performance in RA, and small loss in LB.

No support was expressed for taking action.

For test 4.2, test 4.2a (retraining Huffman tables) has losses in particular for LB. Also test 4.2b has losses in LB chroma, and gain in RA is almost zero. Not sufficient benefit for taking action.

For test 4.3, gain is close to zero, but slight increase of run times (both encoder and decoder) is reported. No reasonable tradeoff.

For test 4.4, small gain is observed for RA, whereas loss occurs in LB, Runtimes are also increased, which is understandable as more checks are necessary both at encoder and decoder. No reasonable tradeoff.

### EE2 contributions: Enhanced compression beyond VVC capability (22)

There was no presentation or discussion about specific proposals in this category – contributions were discussed in the context of the EE summary report JVET-AO0024. For actions decided to be taken, see section 5.2.1, unless otherwise noted.

JVET-AO0068 EE2-2.4: Additional candidates for regular inter AMVP candidate list [C. Wang, Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AO0252 Crosscheck of JVET-AO0068 (EE2-2.4: Additional candidates for regular inter AMVP candidate list) [K. Panusopone (Nokia)] [late]

JVET-AO0069 EE2-3.2: Coding of a coefficient level in TSRC [Y. Yu, L. Xu, J. Gan, H. Yu, D. Wang (OPPO), R. Xu, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

JVET-AO0259 Crosscheck of JVET-AO0069 (EE2-3.2: Coding of a coefficient level in TSRC) [M. Abdoli (Xiaomi)] [late]

JVET-AO0070 EE2-4.3: Additional candidates for merge mode in ALF-CCCM [H. Zhang, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)]

JVET-AO0233 Crosscheck of JVET-AO0070 (EE2-4.3: Additional candidates for merge mode in ALF-CCCM) [M. Jia (ZTE)] [late]

JVET-AO0093 EE2-4.1: On improving the bi-filtering modes in TALF [P. Astola, D. Buğdayci Sansli, J. Lainema (Nokia)]

JVET-AO0222 Crosscheck of JVET-AO0093 (EE2-4.1: On improving the bi-filtering modes in TALF) [N. Song (OPPO)] [late]

JVET-AO0094 EE2-2.1: GPM without blending for Screen Content [J.-Y. Kim, J.-H. Son, K. Kim, J.-S. Kwak (WILUS)]

JVET-AO0235 Crosscheck of JVET-AO0094 (EE2-2.1: GPM without blending for Screen Content) [A. Tissier (Xiaomi)] [late]

JVET-AO0109 EE2-3.1 a/b : Unbiased Simple Quantizer [M. Balcilar, M. Blestel, P. Andrivon (Ofinno), M. Le Pendu, F. Le Léannec (InterDigital)]

JVET-AO0244 Crosscheck of JVET-AO0109 (EE2-3.1 a/b : Unbiased Simple Quantizer) Y. Yu (OPPO) [late]

JVET-AO0263 Crosscheck of JVET-AO0109 (EE2-3.1 a/b: Unbiased Simple Quantizer) [P. Nikitin (Xiaomi)] [late]

JVET-AO0110 EE2-1.3: On interpolation filter for SGPM [J. Lee, G. Moon, K. Kim, J.-G. Kim (KAU), J. Lee, S.-C. Lim (ETRI)]

JVET-AO0245 Crosscheck of JVET-AO0110 (EE2-1.3: On interpolation filter for SGPM) Y. Bai (ZTE) [late]

JVET-AO0134 Crosscheck of JVET-AO0110 (EE2-1.3: On interpolation filter for SGPM) [N. Zouidi (Ofinno)]

JVET-AO0111 EE2-1.6: Improvement for candidates on MPM [W. Niu, Z. Li, S. Xie, M. Jia, Y. Bai, C. Huang (ZTE)]

JVET-AO0256 Crosscheck of JVET-AO0111 (EE2-1.6b: Improvement for candidates on MPM) [J. Cai (Transsion)] [late]

JVET-AO0243 Crosscheck of Test 1.6a in JVET-AO0111 (EE2-1.6: Improvement for candidates on MPM) J. Lee, S.-C. Lim (ETRI) [late]

JVET-AO0255 Crosscheck of JVET-AO0111 (EE2-1.6c: Improvement for candidates on MPM) [Y. Liu (Transsion)] [late]

JVET-AO0113 EE2-1.11: Combination of EE2-1.6b and EE2-1.7 [W. Niu, Z. Li, M. Jia, S. Xie, Y. Bai, C. Huang (ZTE), L. Wang, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

JVET-AO0270 crosscheck of JVET-AO0113: EE2-1.11: Combination of EE2-1.6b and EE2-1.7 [K. Naser (InterDigital)] [late]

JVET-AO0121 EE2-1.4: Position-dependent Weighted Prediction Extension [Y. Wang, X. Zeng, Z. Li, M. Jia, C. Huang (ZTE)]

JVET-AO0262 Crosscheck of JVET-AO0121 (EE2-1.4: Position-dependent Weighted Prediction Extension) [S. Blasi (Nokia)] [late]

JVET-AO0122 EE2-2.3: Extended Availability Conditions for Sub-Block Merge Mode [Y. Liu, Y. Huo, Z. Zhang (Transsion)]

JVET-AO0229 crosscheck of JVET-AO0122 (EE2-2.3: Extended Availability Conditions for Sub-Block Merge Mode) [F. Wang (OPPO)] [late]

JVET-AO0126 EE2-2.2: On improving adaptive GPM blending [Z. Zhang, Y. Huo, Y. Liu (Transsion)]

JVET-AO0132 EE2-4.2: ALF coefficient coding group extension [I. Jumakulyyev, D. Buğdayci Sansli, J. Lainema (Nokia)] [late]

JVET-AO0246 Crosscheck of JVET-AO0132 (EE2-4.2: ALF coefficient coding group extension) V. Shchukin (Ericsson) [late]

JVET-AO0138 EE2-1.7: Multiple candidate selection for MDIP [L. Wang, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)]

JVET-AO0230 Crosscheck of JVET-AO0138 (EE2-1.7: Multiple candidate selection for MDIP) [Z. Lyu (vivo)] [late]

JVET-AO0140 EE2-1.5: On filtering condition for angular modes [S. Wan, Z. Zhu, Y. Yin, H. Xu (NWPU), S. Xie, X. Zeng, C. Huang (ZTE)]

JVET-AO0253 Crosscheck of JVET-AO0140 (EE2-1.5: On filtering condition for angular modes) [C. Zhou (vivo)] [late]

JVET-AO0141 EE2-4.4: Improvement on Frame-level Inheritance Mode in ALF-CCCM [M. Jia, Y. Bai, X. Zeng, W. Niu, Z. Li, C. Huang (ZTE)]

JVET-AO0224 Crosscheck of JVET-AO0141 (EE2-4.4: Improvement on picture-level inheritance mode in ALF-CCCM) [H. Zhang (OPPO)]

JVET-AO0142 EE2-1.2: Combined Angular- and Gradient-PDPC [G. Kulupana, S. Blasi, N. Neumann, J. Lainema (Nokia)]

JVET-AO0268 Cross-check of JVET-AO0142 (EE2-1.2: Combined Angular- and Gradient-PDPC) [I. Zupancic, A. Filippov (TCL)] [late]

JVET-AO0159 EE2-1.10 CCCM with clipping [L.-C. Xu, C.-F. Liao, Y.-K. Lu, F. Liang (SYSU)]

JVET-AO0269 Crosscheck of JVET-AO0159 (EE2-1.10: CCCM with clipping) [Y. Wang (ZTE)] [late]

JVET-AO0170 EE2-1.1: Reference sample generation for intra prediction [T. N. Canh, P. Yin, S. McCarthy (Dolby)]

JVET-AO0183 EE2-1.8: IntraTMP with DMVR [T. Dumas, K. Naser, A. Robert, F. Le Léannec (InterDigital)]

JVET-AO0234 Crosscheck of JVET-AO0183 (EE2-1.8: IntraTMP with DMVR) [X. Zeng, W. Niu (ZTE)] [late]

JVET-AO0239 Crosscheck of JVET-AO0183 (EE2-1.8: IntraTMP with DMVR) M. Blestel (Ofinno) [late]

JVET-AO0195 EE2-2.5: Pairwise Merge Candidates with Motion Vector Scaling [S. Hong, L. Wang, K. Panusopone, D. Rusanovskyy (Nokia)]

JVET-AO0223 Crosscheck of JVET-AO0195 (EE2-2.5: Pairwise Merge Candidates with Motion Vector Scaling) [C. Wang (OPPO)] [late]

JVET-AO0241 EE2-1.9: TMRL angle offset refinement [G. Rath, K. Naser, F. Le Léannec, T. Dumas (InterDigital)] [late]

JVET-AO0242 Crosscheck of JVET-AO0241 (EE2-1.9: TMRL angle offset refinement) [D. Ruiz Coll (Ofinno)] [late]

JVET-AO0251 EE2-3.3: On Multiple transform set selection for inter LFNST/NSPT and Advanced SBT [J. Huo, W. Zhang, F. Yang (Xidian Univ.), B. Li, F. Xing, P. Han, Z. Wang, W. Song (Hisense)] [late]

### EE2 related contributions (1)

Contributions in this area were discussed during 0050–0105 UTC on Friday 16 Jan. 2026 (chaired by JRO).

JVET-AO0146 EE2-related: Clipping operation refinements of Test 1.10 for the CCCM modes [P. Onno, G. Laroche, B. Galmiche (Canon)]

This contribution presents the results of applying a clipping operation on the Chroma sample prediction of the different Cross-Component Convolutional Model variants present in ECM-19.0. In this proposal, the clipping range is computed from the minimum and maximum values extracted from the reference area containing the Chroma samples used for the model determination and they are then modified according to some characteristics of the sample values of this reference area.

The results obtained by applying this clipping operation on the CCCM modes are the following:

-0.03%	 -0.21%	 -0.27% – 100.3%, 100.4% for the AI configuration,

-0.01%	 -0.12%	 -0.06%	 – 100.2%, 100.2% for the RA configuration.

RA results not complete yet.

Runtime is larger than for EE2-1.10, which might be caused by the larger number of modes where clipping is performed.

It was commented that it might be interesting to consider more adaptation of thresholds, e.g. depending on block size.

Study in EE.

### ECM modifications and software improvements beyond EE2 (11)

#### Intra and CIIP (5)

Contributions in this area were discussed during 1600–1710 UTC on Tuesday 20 Jan. 2026 (chaired by Y. Ye).

JVET-AO0092 Non-EE2: Gradient based Multi-Model EIP [P.-K. Liu, C.-Y. Liu, C.-C. Lin, S.-P. Wang, C.-H. Yau, C.-L. Lin (ITRI)]

This contribution proposes to add the Gradient-based Multi-Model EIP (GMMEIP) filter candidates into the MMEIP candidate list. The additional filter candidates utilize the gradient magnitude as a sample classification metric to extend the applicability of MMEIP and further enhance the coding performance. The experimental results demonstrate that the proposed method achieves consistent bitrate reduction across all classes of sequences, with particularly pronounced improvements observed in Class A sequences. The overall simulation results of the proposed method on top of ECM-19.0 under All Intra configuration are  { -0.02% Y, 0.02% U, -0.04 % V, 101.9% encT, 100.3% decT}.

Use gradient of 3 neighboring samples to select the MMEIP’s filters

The proposed gradient-based MMEIP uses the MMEIP candidate list, no new syntax is introduced.

It was asked if new RDO is introduced, the proponent said no new RDO.

It was commented that the decoder would also need to calculate the gradients.

RA results are not available at the time of presentation, the proponent verbally reported that approx. half of the gain (0.01%) can be retained, and no encoding time increase. increase in encoding time is due to calculation of gradients, the proponent asserts that further code optimization can reduce the encoding time. Cross checker however reported that the increase in encoding time is more likely due to doubling the MMEIP candidate size.

Cross checker confirmed the coding performance and encoding time increase.

It was commented that current tradeoff of gain vs. encoding time increase is not favorable, and tests with similar tradeoff in the current round of EE were not adopted at this meeting.

Further study is recommended.

JVET-AO0254 Crosscheck of JVET-AO0092 (Non-EE2: Gradient based Multi-Model EIP) [C. Zhou (vivo)] [late]

JVET-AO0163 Non-EE2: Improvement on chroma MPM [Z. Li, W. Niu, X. Zeng, M. Jia, Y. Wang, C. Huang (ZTE)]

This proposal proposes to modify the chroma MPM list construction, specifically, to modify the MPM list construction for non-CCP mode. The simulation results on top of ECM-19.0 are summarized as follows:

AI : {-0.02%, 0.02%, -0.03%, 100.0%, 100.3%}

Two aspects:

- Add chroma DIMD modes to non-CCP mode candidate list

- Enhance presorting as follows:

cross

Checker confirmed the results.

It was asked if the two aspects were separately tested. The proponent suggests that most of the gain reported are likely from the first aspect.

RA configuration has not been tested.

Investigate in EE, separately test the two aspects, RA results to be provided.

JVET-AO0231 Crosscheck of JVET-AO0163 (Non-EE2: Improvement on chroma MPM) [Z. Lyu (vivo)] [late]

JVET-AO0174 Non-EE2: Modification of EIP filter shapes [Y. Liu, Z. Zhang, Y. Huo, Z. Zhang, J. Cai (Transsion)]

This document reports the modification of EIP filter shapes. The modified EIP filters include the nearest spatial input sample in the diagonal prediction order and other spatial input samples. The performance of the proposed method over ECM-19.0 is reported to be: AI {-0.02%, 0.01%, 0.00%, 99.9%, 100.2%, 99.2%, 100.0%}.

Proposed modified EIP filters:

The proposal does not add additional filters, existing filters are replaced by these.

It was asked whether number of taps in each of these filters is changed compared to the existing filters. The proponent reported no increase in filter taps compared to the existing filters.

It was commented that the proposed S-filter could incur more dependency, and hence could negatively impact the parallelization ability in the diagonal direction supported by the current EIP mode. The other two proposed filters do not incur further dependency.

Multiple experts (including the original EIP proponent) commented that the proposed design would somewhat increase the processing cycle of the EIP mode.

It was commented that the coding gain vs. encoding time seems to be reasonable, and the impact on parallelization could be studied during EE.

Investigate in EE, also study a variant of the proposal that does not impact the existing parallelization capability in the diagonal direction.

JVET-AO0175 Non-EE2: Scharr Filter for Rectangular blocks [A. C. Sidiya, S. Deshpande (Sharp)]

It is proposed to use the Scharr filter instead of the current Sobel filter for HOG-based gradient computation in ECM. It is asserted that Scharr filter reduces directional bias in discrete gradient estimation, offering improved rotational symmetry over Sobel filter.. A normalization factor is applied with the proposed Scharr filter to maintain a gradient dynamic range comparable to Sobel filter, ensuring compatibility with existing thresholds and histogram binning.

On top of ECM19.0, simulation results of the proposed method are reported as follows:

For AI configuration: -0.01%, 0.01%, -0.01%, EncT and DecT not reliable.

The (very small) gains are consistent across all classes, as shown below.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-190 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.02% | -0.01% | 97.8% | 99.0% | 100.1% | 100.0% |
| Class A2 | -0.02% | 0.10% | 0.01% | 97.7% | 98.5% | 100.1% | 99.9% |
| Class B | 0.00% | -0.01% | -0.06% | 97.5% | 99.1% | 100.0% | 100.0% |
| Class C | -0.02% | -0.06% | -0.03% | 97.5% | 99.0% | 100.4% | 100.0% |
| Class E | -0.01% | 0.02% | 0.11% | 97.6% | 99.8% | 99.0% | 99.9% |
| Overall | -0.01% | 0.01% | -0.01% | 97.6% | 99.1% | 100.0% | 100.0% |
| Class D | 0.01% | -0.24% | -0.31% | 97.2% | 99.6% | 100.1% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

The proposed method would introduce block shape dependent filtering scheme for HOG-based gradient computation.

It was commented that the introduced Sharr filter needs the multiplication operation whereas the current Sobel filter can be implemented without any multiplications. The proponent suggested that the coefficients of +/-10 and +/-3 can also be implemented as two additions rather than one multiplication.

It was reported by the proponent that the proposed filter can only bring gains for rectangular shaped blocks (i.e, does not work on square block shapes).

For small blocks (when 2x2 kernel is applied), filter is not changed in this proposal.

It was commented that the gains are quite small, with some small increase in implementation complexity (e.g. more filter coefficients and block shape dependency).

Further study is recommended.

JVET-AO0218 Intra lfnstIdx Prediction [Y. Wang, R. Chernyak, S. Liu (Tencent)] [late]

In this contribution, a new way to signal the intra LFNST index is proposed. The proposed prediction method tests the possible inverse transforms and utilizes the boundary continuity metric as the prediction criterion. The proposed method is tested on ECM-19.0, and the simulation results are summarized as follows:

AI: -0.01%(Y), -0.17%(U), -0.14%(V), 98.4% (EncT), 98.9% (DecT)

Presentation slides to be uploaded.

Invocation of the proposed also has the following conditions:

- If MTSS is allowed, the proposed method is disabled.

- If the proposed method is allowed, sign prediction is disabled.

The proposal is an additional signalling method because when the above conditions are met, we would need the existing signalling method in ECM 19.0.

Non-CTC results (where sign prediction is disabled) are not available at the time of presentation.

The proponent commented that the proposed method wouldn’t be effective for inter LFNST.

Multiple experts expressed interest in studying in EE given the tradeoff is favourable – that is, although gain is small, there is some reduction in encoding time.

Investigate in EE, the following aspects should be studied, RA results to be provided:

Bring non-CTC results with sign prediction off JVET-AO0279 Crosscheck of JVET-AO0218 (Non-EE2: Intra LfnstIdx Prediction) [M. Koo (LGE)] [late]

#### Other (6)

Contributions in this area were discussed during 1710–1750 UTC on Tuesday 20 Jan. 2026 (chaired by Y. Ye) and 0050–0150 UTC on Thursday 22 Jan. 2026 (chaired by JRO).

JVET-AO0071 AHG12: Simplification on TALF [L. Xu, N. Song, Y. Yu, H. Yu, D. Wang (OPPO)]

This contribution proposes to modify the filtering process of TALF to generate the offset without dependency on the output sample of SAO. The number of filter coefficients is reduced by 1. The proposed method is implemented on top of the ECM-19.0 software. The simulation results are summarized as below:

RA: {0.00%, -0.06%, -0.06%, 100%, 100%}

LDB: {-0.04%, 0.22%, 0.11%, 100.1%, 100.0%}

Class-by-class results are as follows:

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.11% | -0.07% | 99.9% | 99.5% | 100.0% | 100.4% |
| Class A2 | -0.01% | -0.07% | 0.00% | 100.2% | 100.9% | 100.1% | 100.5% |
| Class B | 0.01% | -0.02% | -0.06% | 100.0% | 99.9% | 100.0% | 100.0% |
| Class C | -0.01% | -0.06% | -0.08% | 99.9% | 99.7% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.06% | -0.06% | 100.0% | 100.0% | 100.0% | 100.2% |
| Class D | 0.01% | -0.07% | -0.16% | 99.9% | 100.0% | 100.0% | 100.0% |
| Class F | 0.11% | -0.20% | -0.19% | 99.6% | 99.1% | 99.8% | 100.1% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-19.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.08% | 0.02% | -0.34% | 100.3% | 99.9% | 100.1% | 100.0% |
| Class C | -0.01% | 0.25% | 0.00% | 99.8% | 100.3% | 100.0% | 100.0% |
| Class E | -0.03% | 0.51% | 1.00% | 100.2% | 99.9% | 100.2% | 100.0% |
| Overall | -0.04% | 0.22% | 0.11% | 100.1% | 100.0% | 100.1% | 100.0% |
| Class D | -0.01% | -0.32% | 0.04% | 101.0% | 99.6% | 99.9% | 100.0% |
| Class F | 0.00% | -0.57% | -0.06% | 100.2% | 99.8% | 100.1% | 99.8% |

Current ECM-19.0 flow is as follows:

Dependency on SAO output is removed using the following proposed flow, also as a consequence filter tap is reduced by 1:

The proposed method would allow TALF to be performed in parallel to deblocking and BIF/SAO/CCSAO, improving parallelization of the loop filters in ECM.

It was commented that although the gain is relatively small, class-by-class gains are consistent, and this seems to be a straightforward simplification.

Cross checker reported matched results.

Potential reasons for the luma loss in class F and chroma loss in LDB configuration were unknown.

Investigate in EE.

JVET-AO0273 Cross-check of JVET-AO0071 regarding the simplification on TALF P. Onno (Canon) [late]

JVET-AO0139 AhG12: On the Dead-Zone in Quantization [M. Balcilar, M. Blestel, P. Andrivon (Ofinno)]

This work proposes the use of an adaptive dead-zone in ECM’s quantizers, where the dead-zone size varies according to the quantization parameter (QP). Adjusting the dead-zone introduces an adaptive offset to the reconstruction points of the quantization indices. The parameters required to determine the dead-zone size are signaled as a SPS-level syntax element. The impact on coding efficiency and runtimes of the proposed method over ECM-19.0 under the CTC are reportedly:

AI     : -0.05% (Y)	 -0.15% (U)	   -0.19% (V)    99.0% (EncT)	99.5% (DecT)

RA   : -0.10% (Y)	 -0.08% (U)	   -0.20% (V)    98.2% (EncT)	99.8% (DecT)

LDB : -0.16% (Y)	  0.01% (U)	    0.21% (V)    99.9% (EncT)	100.4% (DecT).

Proposed adaptive dead zone for TCQ:

The following QP-adaptive equation is used:

Where a and b are signaled as sequence-level parameters with the following values:

| Mode/SliceType | Channel Type | a | b |
| --- | --- | --- | --- |
| AI/ I-Slice | Luma | 0.0125 | -0.3 |
|  | Chroma | 0.0125 | -0.3 |
| RA/ B-Slices | Luma | 0.0125 | -0.3 |
|  | Chroma | 0.0125 | -0.175 |
| LDB/ B-slices | Luma | 0.0125 | -0.2 |
|  | Chroma | 0.0125 | -0.175 |

It was commented that a corresponding encoder-only adaptive deadzone method (i.e. no change to dequantization) should also be studied.

The proponent suggested that the current method can also be applied to the RDOQ case. It was suggested to perform some merging of this contribution together with the EE test on JVET-AO0109, which uses a modified deadzone but not adaptive deadzone. Discussion about how to do merging or combination of elements in this contribution with those in JVET-AO0109 to be performed offline among interested experts and reviewed at plenary during review of EE2 description.

It was suggested that given block-level (rather than picture-level or slice-level) QP adaptation is commonly used in real applications (e.g. for rate control and/or ROI coding), how QP-dependent quantization would behave in these real scenarios should also be considered.

Investigate in EE, besides testing the proposed method in CTC, the following should also be tested:

- Encoder-only variant of the proposed adaptive deadzone method

- Non-CTC cases, including 1 additional high QP point (i.e. QP 42), RDOQ, sign data hiding on

- Combination with JVET-AO0109 (to be discussed offline and reviewed during plenary)

JVET-AO0199 Non-EE2: Counter-based Temporal Probability Initialization for Temporal CABAC [Z. Xiang, R. Chernyak, H. Huang, S. Liu (Tencent)]

(abstract of v2 – problem with word doc v2)

In this contribution, a new initialization probability derivation method known as Counter-based Temporal Probability Initialization (CTPI) is proposed for context models. CTPI uses statistical information collected from all CTUs in all qualified previously coded slices to derive initialization probability. The proposed CTPI method is tested on ECM-19.0, and the simulation results are summarized as below:

RA: -0.04%(Y), -0.06%(U), 0.03%(V), 100.1% (EncT), 100.1% (DecT)

LB: -0.04% (Y), 0.11% (U), 0.16% (V), 100.1% (EncT), 100.2% (DecT)

It was commented that the necessity of a counter for every context may requires a substantial amount of additional memory.

It was commented that this might not be practical for a standard (considering the temporal dependency in entropy coding), but in this regard is not worse than the current status in ECM.

Several experts supported investigation in EE. It was requested to also investigate a variant where only the counter based probability is used for initialization, and also cases of re-training only, and the proposal on top of re-training.

JVET-AO0261 Crosscheck of JVET-AO0199 (Non-EE2: Counter-based Temporal Probability Initialization for Temporal CABAC) [P. Nikitin (Xiaomi)] [late]

JVET-AO0278 Cross-check of JVET-AO0199 (Non-EE2: Counter-based Temporal Probability Initialization for Temporal CABAC) [Y. Sun, Y. Zhao, E. Alshina (Huawei)] [late]

JVET-AO0282 Non-EE2: On GPM mode [H. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)] [late]

This contribution proposes a modification to the derivation of the GPM offset direction. Specifically, the offset direction is decided by identifying whether the partitioning angle is in a proximity of the horizontal or vertical direction. On top of the ECM-19.0 software, the simulation results {Y, U, V, EncT, DecT} for all the test sequences in the CTC are reported below (2 QP points test results are copied from anchor in RA configuration and 1 QP point test result is copied from anchor in LDB configuration.):

RA*: -0.07%, -0.08%, -0.10%, EncT 100.0%, DecT 100.0%

LB*: -0.02%, 0.15%, -0.19%, EncT 99.9%, DecT 100.0%

Cross-checker supported investigation in EE, considering the gain for a relatively small change.

Investigate in EE

JVET-AO0291 Crosscheck of JVET-AO0282 (Non-EE2: On GPM mode) [H.-J. Jhu (Kwai)] [late]

JVET-AO0283 Non-EE2: Deblocking for ASBT [S. Hong, L. Wang, K. Panusopone, D. Rusanovskyy (Nokia)] [late]

In the current ECM-19.0, with Advanced Sub Block Transform (ASBT) enabled, it is asserted that mismatches in encoder and decoder reconstructed video can be triggered when deblocking filter length is being selected without considering all ASBT sub-partitions. In some cases, this could lead to filtering across the picture boundary.

Simulation results show that the mismatch problem is solved with the proposed solution.

It was commented that the case does not happen under CTC conditions. Further study is necessary to understand the problem, and understand whether the proposed solution is appropriate.

Investigate in EE.

JVET-AO0293 Cross-check of JVET-AO0283 (Non-EE2: Deblocking bug fix for ASBT) [K. Andersson (Ericsson)] [late]

JVET-AO0285 Non-EE2: GPM enhancements for non-square blocks [Y. Kidani, H. Kato, T. Chujoh, K. Kawamura (KDDI)] [late]

This contribution proposes modified derivation methods of the Geometric Partitioning Mode (GPM) partition offset direction and angle. The first modification is that the GPM offset direction is determined by comparing the block aspect ratio with the GPM partition angle's aspect ratio, as in JVET-AO282. The other one is enabling the derivation of the GPM partition angle based on a block-diagonal line, as proposed in JVET-AJ0107, under the LDB configuration. On top of the ECM-19.0, the proposed method reportedly shows the following BD-rate performances of Y, U, and V components, EncT, and DecT {Y, U, V, EncT, DecT} for all the test sequences in the JVET's common test condition:

-	RA: -0.08%, 0.02%, 0.05%, EncT 99.9%, DecT 99.9%,

-	LDB: -0.10%, 0.06%, 0.03%, EncT 99.6%, DecT 99.4%.

Approach similar to JVET-AO0282. Two different aspects, entitled as method 1 and method 2. It was asserted that method 1 might be combinable with JVET-AO0282.

Investigate in EE.

### CTC for EE2 and software aspects (including non-ECM SW) (5)

Contributions in this area were discussed during 1440–1510 UTC on Wednesday 21 Jan. 2026 (chaired by JRO).

JVET-AO0145 AHG6: Larger CTU size for Class B in ECM Common Test Conditions [G. Laroche, P. Onno, B. Galmiche (Canon)]

This contribution proposes to modify the current common test conditions for enhanced compression tool as defined in JVET-AI2017. The modification is related to Class B of Random-Access configuration. Two aspects are proposed:

- The CTU size is set equal to 256 instead to 128.

- The Maximum TT size is set equal to 64 instead of 128.

The results, compared to ECM-19.0 are as follows for Class B:

-0.02%	-0.63%	-0.65%	  94.6%	  102.2% for the RA configuration.

The results, compared to ECM-19.0 are as follows for CTC:

-0.01%	-0.21%	-0.22%	  98.2%	  100.7% for the RA configuration.

The cross-checker found similar enc time decrease, but no decoding time increase.

Agteed to modify CTC as suggested in JVET-AO0145.

JVET-AO0264 Crosscheck of JVET-AO0145 (AHG6: Larger CTU size for Class B in ECM Common Test Conditions) [L. Xu, N. Song (OPPO)] [late]

JVET-AO0153 AHG 12/17: On EIP in JVET-AN0267 software [Z. Xie, L. Xu, Y. Yu, H. Yu, D. Wang (OPPO)]

This contribution reports the tool-on results of EIP using the JVET-AN0267 software. In addition, more tests were carried out, including modification to the EIP regularization, removal of the EIP fast algorithm and addition of BV-EIP. The test conditions follow the CTC of ECM, where QPs {22, 27, 32, 37}, all-intra and random-access configurations were used. The anchor is JVET-AN0267 and results are summarized as below:

Test 1: EIP tool-on test

All-intra: -0.15% (Y), -0.03% (U), -0.10% (V), 108.0% (EncT), 103.4% (DecT)

Random-access: -0.08% (Y), -0.20% (U), -0.08% (V), 101.7% (EncT), 99.5% (DecT)

Test 2: EIP regularization modification

All-intra: -0.37% (Y), -0.30% (U), -0.38% (V), 107.9% (EncT), 104.6% (DecT)

Random-access: -0.15% (Y), -0.13% (U), -0.24% (V), 102.1% (EncT), 100.2% (DecT)

Test 3: Test 2 + removal of EIP fast algorithm

All-intra: -0.40% (Y), -0.31% (U), -0.39% (V), 109.5% (EncT), 105.0% (DecT)

Random-access: -0.12% (Y), -0.27% (U), -0.11% (V), 102.2% (EncT), 100.3% (DecT)

Test 4: Test 3 + BV-EIP

All-intra: -0.48% (Y), -0.38% (U), -0.45% (V), 110.0% (EncT), 105.4% (DecT)

Random-access: -0.14% (Y), -0.18% (U), -0.24% (V), 101.6% (EncT), 99.9% (DecT)

Contribution for information. No action by JVET.

JVET-AO0154 AHG 12/17: IntraTMP on JVET-AN0267 software [L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)]

(include abstract – something goes wrong with copy from word doc)

Contribution for information. No action by JVET.

JVET-AO0200 AHG12: Tensor-Based Intra Mode Derivation in VTM and JVET-AN0267 software [Y.-U. Yoon, H. Huang, L.-F. Chen, R. Chernyak, S. Liu (Tencent)]

This contribution introduces a Tensor-Based Intra Mode Derivation (TBIMD) method as an alternative to histogram-based decoder-side intra mode derivation (DIMD), leveraging a structure tensor formulation. TBIMD derives angular intra prediction modes by directly estimating the dominant local orientation through eigenvector analysis of the structure tensor. Furthermore, predictor blending weights are adaptively determined based on tensor coherency, enabling prediction with fewer blending candidates. TBIMD is tested on VTM-23.11 and NX2, and the simulation results are summarized below.

TBIMD over VTM-23.11

AI: -0.62% (Y), -0.47% (U), -0.54% (V), 113.2% (EncT), 101.6% (DecT)

RA: -0.33% (Y), -0.28% (U), -0.36% (V), 102.1% (EncT), 100.3% (DecT)

TBIMD over NX2

AI: 0.02% (Y), 0.01% (U), -0.11% (V), 99.2% (EncT), 97.4% (DecT)

RA: -0.03% (Y), -0.11% (U), -0.04% (V), 99.1% (EncT), 99.0% (DecT)

Contribution for information. No action by JVET.

JVET-AO0220 AHG12: Optimizations for JVET-AN0271 software [Y.-J. Chang, C.-C. Chen, M. Coban, K. Cui, P. Garus, N. Hu, M. Karczewicz, P.-H. Lin, X. Meng, V. Menon, V. Seregin, Y. Shao, G. Verba, H. Wang, E. Ye, R. Yu, Y. Zhang, Z. Zhang, W. Zhu (Qualcomm)]

In JVET-AN0271, a reimplemented software based on ECM-18.0 and VTM-23.8 was proposed, and the software was made available in the AHG12 GitLab repository. This contribution presents an updated version of the software with further improved compression efficiency.

Simulation results comparing to the anchor without the introduced changes for RA configuration under CTC and CFE test conditions are summarized as follows.

CTC: -1.27% (Y), -3.97% (U), -3.73% (V); 97.6% (EncT), 100.4% (DecT)

CFE: -1.77% (Y), -5.58% (U), -5.46% (V); 97.5% (EncT), 100.5% (DecT).

The included modifications are summarized as follows:

- In MCTF, the unit size is increased to 16 and a 12-tap interpolation filter is used for the MCTF motion compensation.

- Introduced an additional configuration option (option 2) to the existing tool ‘Block Importance Mapping (BIM)’, where instead of modifying the QPs used for each CU, the new option adjusts the λ values based on the importance of a block.

- Disable LMCS for SDR sequences.

- Several bug fixes for tools including RPR, IntraNN, IntraTMP, PDPC, TIMD, ALF, etc.

Contribution for information. No action by JVET.

It was commented that the LMCS optimization of VTM may not be suitable for other software packages with different tools.

# High-level syntax (HLS) and related proposals (81)

## AHG9: Aspects of SEI messages in VSEI, VVC, HEVC and AVC (8)

Contributions in this area were discussed during 2100 Wednesday 14 Jan – 0040, Thursday 15 Jan. 2026, and 0010 Thursday 21 Jan. 2026 (chaired by J. Boyce).

It is noted that VSEI v4 Last Call was completed on Jan 12. We may be able to squeeze some tiny editorial bugs into the pre-publication process. The ISO text for VSEI has not yet been submitted.

We can plan to make an errata document for VSEI at this meeting. We can also make a list of tiny editorial bugs and try to include them in the pre-publication process.

JVET-AO0081 AHG9: Proposed corrections for miscellaneous aspects on VSEI V4 [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE), M. M. Hannuksela (Nokia)]

This contribution proposes several modifications that are asserted to be corrections to resolve issues or ambiguities identified in certain aspects of the VSEI V4. The proposed corrections are as follows:

(Related to GFV and GFEV SEI messages)

1	Correction of reversed semantics of gfv_matrix_pred_flag and gefv_matrix_pred_flag

(Related to TDI SEI messages)

2	Modify the persistence scope of the TDI SEI message

(Related to SPO SEI messages)

3	Change the constraint on po_sei_payload_type and its relation to SeiProcessingOrderSeiList

4	On clarifying the handling of unsupported SEI payload types in SEI Processing Order (SPO) SEI messages

5	Clarification of AURR signalling in processing chains and sub-chains of SPO SEI messages

Regarding item 1, there is an identified bug.

Agreed to include item 1 in the list of tiny editorial bugs for possible inclusion in the pre-publication process.

Regarding item 2, it is proposed to replace “decoded picture” language with “picture unit and its decoded picture”.

Agreed to include item 2 semantics language change in errata.

Item 2 also includes a note, but the proposed wording was questioned. The Note 3 describes machine-learning that was applied as pre-processing prior to encoding. No action on the proposed note.

Regarding item 3, the current semantics language is problematic for new SEI messages defined in the future. The proposed solution would require changes to the referring coding standard. The standards may not be updated at the same time, and AVC is due to be updated in the near future.

It is noted that there is not a current problem, and a solution could wait until a new version of VSEI.

Agreed to add the  item 3 proposed change to the AVC draft to be output from this meeting, as well as to the errata for VSEI, HEVC, and VVC.

Regarding item 4, it was suggested that no normative change is needed. The understanding is that an SEI type that is not recognized should be treated like an SEI message that cannot be processed.

Item 5 option 2 is a technical change that extends the current design. A question was raised about backwards compatibility of making such a change.

Item 5 option 1 includes “should” language in a note. Discussed after offline discussion about wording of the note chaired by S. Deshpande on 21 January 2026 from 16:55-17:00.

In v2, additional co-author is added and text is provided for the NOTE.

In v2 The wording about sub chain is removed from normative part (as was proposed in v1). So the v2 proposal only adds a NOTE. Agreed to add the note as in v2.

Item 5 option 1 also includes semantics language changes outside of the note.

JVET-AO0083 AHG9: On SEI message semantics for human viewing and machine analysis [X. Xu, S. Liu (Tencent)]

In semantics descriptions of current VSEI v4 (JVET-AN1019) of various related syntax elements, value combination constraints are imposed on video content for human viewing and machine analysis. It is asserted in this contribution that a loosened constraint on this part may be more generic. Therefore, two alternative suggestions are made:

1: to remove such a bitstream conformance constraint; or

2: to add a syntax element to extend the usage of video content, beyond the existing human viewing and machine analysis coverage.

Both options are technical changes.

Option 2 proposes a new syntax element for the NNPFC, PO, and EOI SEI message, but are not in the extension, which is problematic.

It was noted that the value of 0 can be used for both po_for_human_viewing_idc and for po_for_machine_analysis_idc. It was suggested that additional information in an extension could be provided when those values were both 0.

Further study to provide more justification of the use case and to restructure to be backwards-compatible with VSEI v4.

JVET-AO0084 AHG9: Extensions of decoded picture hash (DPH) SEI message [X. Xu, S. Liu (Tencent)]

Extensions of DPH (decoded picture hash) SEI message are proposed in this contribution. Currently only 3 hash functions, MD5 (16 Bytes), CRC (2 Bytes) and checksum (4 Bytes) are supported in the DPH SEI message. On the other hand, various other types of hash functions, with variations in length and category, are widely used in applications. In order to support more hash functions with flexibility in the SEI message, extensions are made to provide a generic framework of describing the key information of supported hash functions, including the following:

1: type of hash function

2: calculation methodology

3: length of the hash key

4: hash key information

It was noted that the original motivation for the decoded picture hash SEI is to debug decoders and not to provide security. It is an old SEI message and it would perhaps be undesirable to extend it. If such functionality was needed, a new SEI message could potentially be created.

The DSC SEI messages are aimed at security.

The use case for this extension is unclear. No action.

JVET-AO0085 AHG9: Editorial improvements for VSEI v4 and TuC [X. Xu, S. Liu (Tencent)]

This contribution suggests a list of editorial improvements for the most recent VSEI v4 specification (JVET-AN1019) and TuC document (JVET-AN2032).

Only VSEI items to be reviewed in this section.

Item 1 identified a bug in semantics of rwp_reserved_zero_5bits. Item 2, item 3, and item 4 are small editorial bug fixes.  Item 5 updates semantic to account for added sdi_aux_id[ ] type. Items 6, 7, 8, and 10 use “0” instead of “zero” in semantics.  Item 11 removes unnecessary [ i ]. Agreed to add all of these to the list of tiny editorial bugs.

No action on item 9.

JVET-AO0087 AHG9: On layer identifier range and number of layers information in the SEI messages [X. Xu, S. Liu (Tencent)]

Inconsistent semantics descriptions of several SEI messages in VSEI v4 (JVET-AN1019) and TuC document (JVET-AN2032) for specifying the value range of layer identifier, are identified in this contribution. Solutions are suggested to better align all semantics descriptions w.r.t. layer identifier.

The range of layer ID values allowed for PRI SEI are larger than used in SDI SEI, or that are enabled by the current video coding standards. It was noted that this difference is intentional to be future proof for a new referring standard that may support a larger range of layer ID values.

It is proposed to add an interface variable, MaxLayerId. It was suggested that there is no need to make a change to the current design.

For the TuC, it would be possible to change the allowable range for any of the new SEI messages if considered to be desirable.

Adding the proposed interface variable would increase the length of the interface text.

No action.

JVET-AO0088 AHG9: On consistency of ViewId and NumViews in the SEI messages [X. Xu, S. Liu (Tencent)]

Inconsistent semantics descriptions in several SEI messages in current VSEI v4 (JVET-AN1019), for the view identifier and number of views are identified in this contribution. A solution is suggested to better align all related semantics descriptions.

TBP

JVET-AO0089 AHG9: On consistency of maximum number of temporal sub-layers in the SEI messages [X. Xu, S. Liu (Tencent)]

TBP

JVET-AO0212 Extension mechanism for new AVC SEI messages [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

In this input document, an extension mechanism is proposed to be added to the SEI payload syntax structure in AVC. The proposed extension mechanism is identical to the extension mechanism that exists in VVC. The extension mechanism is proposed to be present only for SEI messages added in the next version (and future versions) of the AVC standard. Existing SEI messages and legacy AVC decoders are not impacted by the proposed extension mechanism.

This contribution is proposed for the upcoming new version of AVC, which will reference VSEI v4. It would apply to SEI message in VSEI v3.

The NNPFA/NNPFC messages are in a published version of AVC, so it would not enable support for the extensions in the new version of AVC.

It was suggested to check that the proposed syntax structures are defined in AVC. If not, they would need to be added.

There may be other changes needed in VSEI.

It was suggested that an additional payload type could be assigned in AVC for extensible/extended versions vs.  non-extensible/un-extended versions of the same SEI message.

The current proposal has a condition payloadType <= 212. It is noted that specific payloadType values could be listed without restricting them to a range.

It was noted that we have attempted to match payload type values for the same SEI message across standards.

It was suggested that the proposed syntax table could be restructured to have common syntax elements for bit_equal_to_one and bit_equal_to_zero.

There was some support expressed for the proposal.

Further discussion requested.

A -v2 version adds text from JVET-AO0280.

It is suggested to be modified to include a list of payload types instead of a less than condition. It is also suggested that the additional payload types allocated to the FGC and NNPFA SEI message with values in the AVC range.

Revisit once a new version is provided that implements the above suggestions.

JVET-AO0280 Editorial study of the extension mechanism for new AVC SEI messages proposed in JVET-AO0212 [G. J. Sullivan (Dolby)] [late]

This contribution contains draft text for editorial refinement of the extension mechanism for new AVC SEI messages proposed in JVET-AO0212 (v1). The contribution is intended to be technically identical to that in JVET-AO0212, with editorial refinement only. This approach would result in having 4 SEI messages that use the prior approach and 11 with the new one. Until updated, the VSEI standard would contain an incorrect statement about the extensibility of AVC.

The suggested text is provided with highlighting relative to JVET-AN1016 (the AVC draft text produced from the previous meeting). Some aspects of this refinement, such as the handling of the semantics of SeiExtensionBitsPresentFlag, also apply to other standards.

It had been suggested that for the NNPFA and FGC SEI messages that were extended in VSEI v4, additional payload types can be assigned for the extended versions.

It was suggested to look up the history regarding extensibility of AVC SEI messages.

Some additional editorial suggestions and corrections of existing specs are also proposed. Those are delegated to the editors.

One of the editorial suggestions is to clarify that the value of SeiExtensionBitsPresentFlag can be modified by the VSEI spec.

There is a note in VSEI v4 saying that the extended FGC and NNPFA SEI messages are not supported in AVC. This note should be removed or reworded in a new version. This could be included in the errata document.

The above two editorial suggestions agreed to be added to the errata document.

A modified syntax is proposed that is an editorial alternative to the proposal in JVET-AO0212.

It was suggested the condition ( payloadType <= 212 ) could be replaced with a list of payload types, and could continue to be changed in the future.

For a duplicate payload type, it is suggested to use values that are in the AVC range.

AVC interface text would be needed for the FGC SEI message.

## AHG9: Aspects on new and extended SEI messages in TuC for VSEI (54)

### General (9)

Contributions in this area were discussed during 0040 – 0125 on Thursday 15 Jan. 2026 and 2100 - 2200 Thursday 15 Jan 2026 (chaired by J. Boyce).

JVET-AO0050 AHG9: Miscellaneous editorial changes for VSEI TuC [Y.-K. Wang, J. Xu (Bytedance), Y. Li, K. Yang, Y. Xu (SJTU)]

This contribution proposes a set of miscellaneous changes, assertedly editorial, for VSEI TuC. The proposed changes are included in an attachment to this contribution.

A -v2 to be uploaded with additional changes during review.

Agreed to use as the starting point for the TuC output of this meeting.

JVET-AO0085 AHG9: Editorial improvements for VSEI v4 and TuC [X. Xu, S. Liu (Tencent)]

Also in 6.1

This contribution suggests a list of editorial improvements for the most recent VSEI v4 specification (JVET-AN1019) and TuC document (JVET-AN2032).

The TuC changes are reviewed in this section.

Agreed on items in section 2.1, 2.2, 2.3, 2.4, 2.5, 2.6, 2.7, and 2.8.

For 2.6, check if the ecfi portioning is in a 2D grid with 2 array indices or a single index.

Further discussion requested on section 2.9.

Further discussed 2345 on Wednesday 21 Jan. 2026.

Agreed on item in section 2.9 of the -v3 version, on quality metrics.

JVET-AO0086 AHG9: On reserved zero bits semantics [X. Xu, S. Liu (Tencent)]

Inconsistent semantics descriptions in between current VSEI v4 (JVET-AN1019) and TuC document (JVET-AN2032) for reserved zero bits in various related syntax elements are identified in this contribution. Solutions are suggested to better align all zero bits semantics descriptions.

The rn_alignment_* syntax elements would be better named with “reserved” as there are fixed positions.

The contribution is referred to the editors.

JVET-AO0096 AHG9: On interface text for SEI messages in TuC [J. Nam, H. Tan, J. Lee, C. Kim, J. Lim, S. Kim (LGE)]

This contribution proposes modification to some aspects in the interface text of VVC for SEI messages in TuC. The proposed modifications are as follows:

1. On updating interface text for DOI SEI

2. On updating interface text for CR SEI

3. On specifying interface text for BRI SEI

4. On specifying interface text for CMI SEI

For item 1, suggests to use doi_nuh_layer_id[ i ] as a variable index rather than i

It was suggested to also indicate what the value of i should be described in the interface. It is currently described in the SEI message.

It is also noted there is also an array index of j for some of the interface variables.

Some modification is seen to be necessary.

A key question is what should be done in the codec interface vs. in the SEI message itself. Should the interface provide information about all layers present in the bitstream or just those layers used by the SEI message? Where should index vs layer ID be used?

If information of all layers present in the bitstream is included in the interface, it would allow identical text for multiple SEI messages and simpler interface text.

It was suggested that the PRI SEI message includes information only for the layers used. This might be considered editorial and possible to change in an errata.

Offline activity suggested.

Further discussion requested.

Further discussed 0000 on Thursday 22 Jan 2026.

Agreed with all items in the -v2 version.

JVET-AO0150 AHG9: On versatile SEI RBSP and versatile SEI message [J. Chen, Y. Ye (Alibaba), M. Pettersson (Ericsson)]

This contribution proposes the following changes to versatile_rbsp() and versatile_sei_message() in TuC

1)	Remove the constraint “When vsei_payload_size_flag is equal to 0, the SEI message shall be the only SEI message in the NAL unit”, as it is asserted to be redundant.

2)	Allow multiple bytes signaling for vsei_payload_type_byte to support more than 256 payload types.

3)	Extend vsei_payload_size_flag to vsei_payload_size_byte_num to indicate the number of bytes signaled for SEI payload size.

For the first item, it was suggested to move the constraint to an informative note.

NOTE: When vsei_payload_size_flag is equal to 0, the NAL unit contains only one SEI message.

Agreed to change the constraint to the above note.

Item 2 uses a similar mechanism as in VVC to allow a longer than one byte payload type.

Item 2 is agreed.

Regarding item 3, a question was raised about what is the largest possible SEI message?  With the contribution, the maximum payload size is 16 MB vs. previously is 16 KB.

A codec spec could impose a stricter constraint than the syntax allows.

Some concerns were raised about the precise text language, regarding the inference language.

It was suggested to use u(v) for vsei_payload_size_byte.

Further discussion requested on item 3.

Further discussed 1700 – 1705 Wednesday 21 Jan 2026.

Agreed to item 3 in the -v2 version of the document.

JVET-AO0167 AHG9: Proposed Localization and Mapping SEI message for version 5 of VSEI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm), G. Teniou (Tencent)]

This contribution proposes adding the localization and mapping (LAM) SEI message into the working draft of Version 5 of VSEI.  The LAM SEI message is meant to be available for HEVC and VVC.

Request discussion in plenary about timing of a WD for a new version of VSEI v5. This will also likely involve parent body discussion.

No software is currently available for the LAM SEI message.

JVET-AO0169 AHG9: On flf [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

This proposal clarifies the description of floating-point types.  Two options are given both achieving two goals.  Option A maintains reference to ISO/IEC 60559:2020 but limits and clarifies the text.  Option B is aligned with ISO/IEC 60559:2020 but defines the needed parsing and reconstruction independently.

1 Eliminate NaN and infinite values from being allowed.

2 Clarify use of exponent bias in construing floating point values

Changes for version 2 of this document:

- Used green highlighting or reconstruction the five formulas.

- Added description of motivation for each option.  Option A minimal text changes and option B including full description to remove normative reference to ISO/IEC 60559:2020.

- The description in option B is simplified and a note is added in option B noting that the parsing and reconstruction agree with that in ISO/IEC 60559:2020.

There was a question of if bias can be calculated based on w and t in ISO/IEC 60559:2020 or if anything more is needed to fully describe.

It was suggested that rather than restricting values in the syntax descriptor, constraints may be placed in individual SEI messages or syntax elements.

Maybe a note could be added that this allows values of +/- 0 and +/- infinity and NaN, and these may be further restricted for some usages.

Side activity to consider if additional information is needed, e.g. for bias, and to provide wording for a note.

Further discussion requested.

Further discussed 1710 on Wednesday 21 Jan 2026.

Changes for version 3 of this document

- Details on definitions of emin and bias values in ISO/IEC 60559:2020 are noted.

- Proposes text of note formulated to mention general result of float parsing and reconstruction may give NaN, infinite or signed zero values.

- Notes addition limits of floating-point values, such as excluding infinite values etc., should be added in uses.

The -v3 version of the document proposed to add this note:

NOTE – The values of realVal specified in ISO/IEC 60559:2020 include NaN, infinite and signed zero values.

Agreed to add this note, changing “infinite” to “infinity”.

Further study on the questions of the definitions of emin and bias values in ISO/IEC 60559:2020, especially considering potential for values that are not specifically defined in a table in that spec.

JVET-AO0185 AHG9: Editorial changes on VSEI TuC [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes following editorial changes on the SEI messages in the TuC.

- Alpha overlay component derivation bug fix in DOI SEI message

- Closed bracket missed in NNPFC SEI message

- Unified naming for layer identifier syntax elements

v2 is updated to align with AO-0198, as the SDI SEI message extension is proposed to be removed.

Item 1 is also proposed in JVET-AO0125.

Agreed to item 1, item 2, item 3.

JVET-AO0187 AHG9: Miscellaneous aspects of SEI messages in VSEI TuC [H. Tan, J. Lee, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes several modifications to quality metrics SEI message and constituent rectangles SEI message that are currently included in the TuC for future extensions of VSEI. (Related to quality metrics SEI message)

1	Specify inference value for syntax element qm_metric_description_present_flag[ i ]. The inference is as follows: when qm_metric_definitions_present_flag is equal to 1 and qm_metric_description_present_flag[ i ] is not present, its value is inferred to be equal to 0.

2	Condition the signalling of qm_bit_equal_to_zero and qm_metric_description[ i ] such that they are present only when there is at least one metric with qm_metric_description_present_flag[ i ] equal to 1.

(Related to constituent rectangles SEI message)

3	Update the current constraints related to support for 4:4:4 and 4:2:2 chroma format so that they can be used in multi layers.

Agreed to item 1, item 2.

For item 3, the current design doesn’t work to represent 4:4:4 if the Y, U, and V components are in separate layers.

Agreed to item 3.

During review, it was suggested that there is another bug in the message that has the wrong height calculation for 4:2:2, as it should not be divided in half.

Another bug was mentioned that an additional bullet point to generate the target picture for 4:2:2.

JVET-AO0156 may address that issue.

JVET-AO0190 AHG9: On the SEI message OLS constraint [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes to constrain the layers indicated by CR, DOI, BRI, ECFI and SDI SEI messages to output layers of at least one output layer set (OLS) in the TuC.

- OLS constraint on the layers of each CR group

- OLS constraint on DOI SEI message

- OLS constraint on BRI SEI message

- OLS constraint on ECFI SEI message

- OLS constraint on SDI SEI message

It was questioned if the CR SEI needs such a constraint for all usages. Some applications will not require that all CRs be available.

For an example use case with forming a 4:4:4 target picture, an application may choose to have 3 separate single layer decoders (e.g. Main 10 profile), but this constraint would require a unnecessary OLS that contains all 3 layers in order to be compliant.

The DOI VVC interface in the TuC already has language with a constraint on OLS.

“It is a requirement of bitstream conformance that there shall be at least one OLS with index olsIdx that has values of OutputLayerIdInOls[ olsIdx ][ j ] equal to doi_nuh_layer_id[ i ] and doi_alpha_layer_id[ i ], when present, for each value of i in the range of 0 to doi_num_display_overlays_minus1 , inclusive, and for any values of j in the range of 0 to NumOutputLayersInOls[ olsIdx ] − 1, inclusive. “

For the BRI SEI message, some applications may only need the high order bits. A lower capability decoder device might require less accuracy than a higher capability device.

The SDI SEI is an older SEI message, so there are backwards compatibility implications. It is also noted that the SDI SEI itself doesn’t indicate any suggested process that would require the presence of multiple layers. The SDI SEI doesn’t have interface text. No action for SDI SEI constraint.

Further discussion requested for CR, BRI, and ECFI.

Further discussed 1720 on Wednesday 21 Jan. 2026.

It was suggested that adding such constraints may cause a problem for sub-bitstream extraction.

For the ECFI message, an application may find it useful to have some information.

No action.

It was questioned if the current constraints on PRI and DOI are appropriate.

### Scalability dimension information SEI message (4)

Contributions in this area were discussed during 2200–2230 UTC on  Thursday 15 Jan. 2026 (chaired by J. Boyce).

JVET-AO0161 AHG9: on the SDI SEI message dependency for ASAI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm), ), M. M. Hannuksela, V. Zakharchenko, J. Boyce [Nokia]]

This proposal introduces SDI SEI message dependency constraints for the ASAI SEI messages per comments from AN meeting.

There is some support for imposing a constraint, but there were some concerns with the specific proposed wording. It was suggested to move the first bullet item into a separate constraint.

It was questioned why the constraint applies only to depth and not to alpha or other auxiliary types.

Discussed again on 21 January 2026 from 23:55-23:59 chaired by S. Deshpande.

Version v2 was discussed and adds co-authors.

It was commented that option 2 is a bit more flexible compared to option 1.

Agreed option 2.

.

JVET-AO0162 AHG9: on the SDI SEI message dependency for GRI [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm), M. M. Hannuksela, V. Zakharchenko, J. Boyce [Nokia]]

This proposal introduces SDI SEI message dependency constraints for the GRI SEI messages per comments from AN meeting.

It was suggested that the language could be made more precise.

Discussed again on 21 January 2026 from 23:50-23:55 chaired by S. Deshpande.

Version v2 was discussed and adds co-authors.

Agreed.

JVET-AO0193 AHG9: On the SDI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes the following constraints to the SDI SEI message in TuC.

- Constrain the associated layer of an AUX_SEGMENT auxiliary layer to be a primary layer.

- Constrain the auxiliary layer and its associated layer shall not be the same layer

- Constrain the confidence maps shall be identical when multiple confidence auxiliary layers associated with the same described layer.

v2 is updated to align with AO0198, as the SDI SEI message extension is proposed to be removed.

It is noted that the wording of the picture segmentation info SEI refers to “one or more associated primary pictures”. A constraint is proposed that limits to primary pictures.

It was suggested that it might be beneficial to allow segmentation of auxiliary pictures such as depth.

For item 1, there is a preference to modify the wording to refer to “associated pictures” rather than “associated primary pictures”.

Item 2 is agreed.

Item 2.3 chaired by S. Deshpande during 22:35-22:45 UTC on 15 Jan 2026.

Item 2.3 allows more than one confidence map layers and the constraint needs them to be identical.

It was commented if two different algorithms might generate two slightly different confidence maps. It was commented if in such a case separate SEIs should be used and one SEI should only allow one confidence map only.

JVET-AO0198 proposes allowing a confidence layer to be associated with only one layer (e.g. depth or alpha).

After further discussion there is no direct conflict between item 2.3 and JVET-A0198.

More support for not disallowing two confidence maps and keeping the flexibility of allowing them to be different.

No action.

JVET-AO0198 AHG9: SDI SEI clean-up related to interaction between JVET-AN0265-v2 and JVET-AM0090 [J. Boyce, M. M. Hannuksela (Nokia)]

Chaired by S. Deshpande during 22:15-22:25 UTC on 15 Jan 2026.

It is proposed to remove the SDI SEI message extension from JVET-AM0090 in the VSEI TuC for signalling a described layer for confidence layers, because it is asserted to be unnecessary given the changes added to the TuC from JVET-AN0265-v2.

Agreed as a cleanup.

It is also proposed to modify the semantics of sdi_num_associated_layers_minus1[ i ] when sdi_aux_id[ i ] is equal to 5 to impose a constraint that a single layer is associated with the confidence layer, in order to avoid uncertainty in which layer the confidence layer describes.

Modification are asserted to be editorial (only the change marks are newly proposed).

Agreed as a cleanup.

### NNPF SEI messages (2)

Contributions in this area were discussed during 2245 UTC–on Thursday 15 Jan. 2026 – 0030 UTC XXXX on Friday 16 Jan. 2026 (chaired by J. Boyce).

JVET-AO0063 AHG9: On Neural-network Post-filter Signaling [S. Deshpande (Sharp)]

Following is proposed related to Neural-network Post-filter (NNPF) Signaling in VSEI TuC:

- It is proposed to signal the syntax element nnpfc_ratio_luminance_sign_flag only when nnpfc_ratio_luminance_value is not equal to 0.

If the value of nnpfc_ratio_luminance_value is equal to 0, it means that the average luminance is unchanged, but changes can still be applied in regions.

It is noted that it is common to signal a sign flag only for non-zero values.

Agreed.

JVET-AO0118 AHG9: On a demosaicking purpose for the NNPFC SEI message [C.-H. Demarty, L. Blondé, A. Ak (InterDigital)]

This contribution proposes the introduction of a demosaicking purpose to the Neural Network Post-Filter Characteristics (NNPFC) SEI message, together with metadata allowing description of the demosaicking operation by the NNPF. It is reported that a recently introduced SEI message assumes that demosaicking occurs at the decoder side, but no normative text currently describes how demosaicking should be performed or which tools are intended to be used. The proposed signalling addresses part of this gap by providing the use of neural-network post-filters for demosaicking.

V2 includes some editorial changes.

A demosaicing purpose was proposed.

It was suggested to have demosaicing as a sub-purpose of colourization.

JVET-A0119 proposes changes to the Sample interleaving SEI message. It was suggested to use this proposal in combination with the sample interleaving SEI.

The proposed syntax elements describe the capabilities of the NNPF to process the picture. It was questioned why an encoder would signal metadata in an NNPF that describes a filter that is not suitable for demosaicing of the bitstream. It was unclear what stage would benefit from this information.

The offset information might be sent with the content.

JVET-AO0078 proposes a post-processing operation SEI.

This proposal specifically describes using an NN to do the demosaicing operation.

Agreed to add a demosaicing purpose, but not to add any syntax elements.

### Constituent rectangles SEI message (2)

Contributions in this area were discussed during 0030–0125 UTC on Friday 16 Jan. 2026 (chaired by J. Boyce).

JVET-AO0156 AHG9: On the constituent rectangles SEI message [J. Xu, Y.-K. Wang, K. Zhang (Bytedance)]

This proposal proposes the following items to the constituent rectangles SEI message specified in JVET-AN2032.

- Add the inference for cr_num_rects_in_layer_minus1.

- Modify the inference for cr_subpics_partitioning_flag.

- Modify the inference for cr_rect_type_idc.

- Modify the inference for cr_rect_id.

- Modify the inference for cr_associated_rect_id_present_flag.

- Modify the inference for cr_associated_rect_layer_idx_present_flag and cr_associated_rect_layer_idx.

- Modify the semantics for cr_associated_rect_id.

- Modify the inference for cr_rect_group_idx.

- Modify the inference for cr_rect_type_description_present_flag.

- Modify the inference for cr_subpic_idx.

- Modify the inference for cr_rect_type_description.

- Add a constraint of cr_group_enhanced_chroma_format_idc.

- Add target picture generation for 4:2:2 format and fix the crRectHeight constraint.

- Do not signal cr_rect_group_idx when cr_num_groups is equal to 1.

- Signal cr_rect_type_description according to cr_num_rects_in_layer_minus1.

The proposed changes address problems in the design when layer reuse was added.

Agreed items 1, 3, 4, 5, 6. 7, 8, 9, 10, 11, 13, 14, 15.

In item 2 agreed to fix the missing array index, but don’t add the additional words to the inference language because it is unnecessary.

It was questioned if “can” should be used instead of “may” in the semantics. The ISO staff does not like to see “may” in a note.

For item 12 agreed with rewording from “shall not be equal to 3”, to “shall be in the range of 0 to 2, inclusive”

Some additional typo fixes are also included.

JVET-AO0189 AHG9: On the layer ID of CR and ECFI SEI messages [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes to constrain the layer that CR and ECFI SEI message present in the TuC to ensure the consistency.

V2 proposes alternative options for the layer constraints.

Agreed to option 3 for CR and ECFI. Also modify the language for DOI and BRI to be consistent.

JVET-AI0079 item 1 is related.

### Display rectangles SEI message (2)

Contributions in this area were discussed during 1520–1700 UTC on Friday 16 Jan. 2026 (chaired by J. Boyce).

JVET-AO0124 AHG9: On display rectangles SEI message [K. Abe, T. Nishi (Panasonic)]

This contribution proposes improvements to the syntax and the semantics of the display rectangles SEI (DR SEI) in the VSEI TuC draft of JVET-AN2032. The proposed modifications are as follows:

Item 1: Fix and align wrong syntax element names

dr_fill_content_width[ i ], dr_fill_width[ i ]  >>>  dr_fill_content_region_width[ i ]

dr_fill_top_left_x[ i ]                    >>>  dr_fill_content_region_top_left_x[ i ]

dr_fill_top_left_y[ i ]                    >>>  dr_fill_content_region_top_left_y[ i ]

Item 2: Fix bugs of missing syntax elements

Add signaling of missing syntax elements used in processing when dr_fill_method_idc is equal to 2.

Agreed to item 1.

For item 2, restructure the syntax table conditions to avoid having the dr_fill_content_region_width[ i ] occur twice in the table.

Agreed to item 2, as modified as described above.

Editorial fixes also agreed.

JVET-AO0165 AHG9: On display rectangles gradient filling [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)]

Chaired initially by S. Deshpande and further chaired by H. Tan including the decisions, during 15:30-17:05 UTC on 16 Jan 2026.

Gradient based filling method has been introduced in JVET-AN0124 and adopted to the VSEI TuC. It provides mechanisms to fill unpopulated areas outside the display rectangle using a gradient method. This contribution proposes to

- Constraints that guarantee display rectangles to always overlap with cropped decoded pictures.

- Extend the gradient filling method by adding center

- Add automatic value determination for start and end value based on cropped decoded picture.

The proposed solution includes the following aspects:

- Usage of one reserved value of dr_fill_gradient_idc to support centered radial based filing methods.

- Addition of a presence flag for the gradient fill start and end values

- Addition of a syntax element indicating the log2 of the border sizes used for inference of start and end values.

- Addition of inference method of start and end values, when not present, based on the sample values on the edges of the decoded picture

- Addition of constraints to maintain overlap between display rectangles and cropped decoded pictures.

It was asked if it is necessary to define too many details via dr_fill_gradient_idc[ i ] about the gradient directions and how to fill the area etc. It was commented that this allows the content creator to specify an exact experience.

It was commented that if the content/ scene changes, then the fill may need to be updated. This is achieved by sending a new SEI (which may need frequent SEI messages). It was commented that may be decoder should make such choices. It was commented that the encoder may want to give precise experience as is done in the current design.

It was commented that may be the exact implementation of the gradient calculation can be left out and just an indication of gradient type can be signaled. It was commented that this may make the user experience/ artistic intent vary. It was commented that the area filled is not the primary area. It was commented that overall experience includes the fill area as well.

It was commented that Danmu SEI has similar idc, where it is asserted to be included due to what applications support. It was asked if existing applications in the case of this SEI also have such fill support.

It was asked if there is editorial change required for the last else part in the proposal.

It was requested to check if some use cases are available for the overall aspect of the filling.

It was commented that the addition of radial gradient (the new idc value 4) and the conditional signaling and inference aspects of the proposal (whether to allow starting and ending fill value to not be signaled and be inferred) could be separated for the discussion.

Regarding addition of new idc value 4: agreed (including adding equations and details as proposed).

Regarding conditional signaling and inference: the proposal applies to all the gradient values 0-4. It was asked if this part of the proposal requires the decoder to update the values per picture. Revisit this aspect after offline discussions.

It was commented that there could be a TR on how to use VSEIs and some aspects could be specified in there. The white paper on VSEI is an informational document and not the right place for such a specification. There are some existing TRs that reference some SEIs/ VSEIs. There is currently no general purpose TR for VSEIs.

It was suggested to decide about including detailed equations for each idc or not when a message is promoted from TuC to WD.

Further study encouraged on if the detailed equations/ descriptions of the fill method should be specified or not.

### SEI processing order and processing order nesting SEI messages (1)

Contributions in this area were discussed during 1700–1730 UTC on Friday 16 Jan. 2026 (chaired by J. Boyce).

JVET-AO0101 AHG9: Text description information SEI message in processing chain [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes to include a text description information (TDI) SEI message in a processing chain and add the semantics when a TDI SEI message is included in a processing chain.

The summary of the proposed modifications is as follows:

–	Item 1: In the VVC interface text, add the payloadType value of TDI SEI message type value in SeiProcessingOrderSeiList.

–	Item 2: In the HEVC interface text,

–	Include the TDI SEI message in HEVC (i.e., add the payloadType value of TDI SEI message).

–	Add the payloadType value of TDI SEI message type value in SeiProcessingOrderSeiList.

–	Item 3: In the AVC interface text,

–	Include the TDI SEI message in HEVC (i.e., add the payloadType value of TDI SEI message).

–	Add the payloadType value of TDI SEI message type value in SeiProcessingOrderSeiList.

–	Item 4: Add the semantics when a TDI SEI message is included in a processing chain.

Proposes to add TDI in HEVC and AVC, regardless of SPO. Previously we required SW in the reference software code bases for each standard an SEI from VSEI is included in. Request plenary discussion on the potential inclusion of TDI for HEVC and AVC.

Item 1, agreed to add to errata for VVC and VSEI.

Item 2, proposed text looks good, but depends on plenary decision about adding TDI to HEVC.

item 3, AVC does not have a CLVS defined. The TDI persistence scope is CLVS. Is this an issue for any other SEI messages in AVC? We still have the opportunity to modify the text of the AVC interfaces in the upcoming AVC version. It may be helpful to add some clarification to say that a CLVS is treated like a CVS for the SEI message.

It was suggested that in the SPO interface, CLVS is changed to CVS.

item 4, it is suggested that the proposed constraint is not appropriate for all TDI purposes.

Further study to consider if a constraint could be targeted for the AI marking purpose of the TDI SEI.

### Encoder optimization information SEI message (7)

Contributions in this area were discussed during 2105–2300 UTC on 16  Jan. 2026 (chaired by S. Deshpande).

JVET-AO0082 AHG9: On picture generation method indication in encoder optimization information SEI message [X. Xu, S. Liu (Tencent)]

The existing EOI SEI message as described in current VSEI v4 (JVET-AN1019) supports the functionality of temporal frame resampling. However, it is always assumed that the restoration method of missing pictures is always frame interpolation. However, it is argued that frame extrapolation is also used in practice, especially in low delay in applications. Therefore, it is suggested in this contribution to consider the following changes in the EOI SEI message:

1: an indication of temporal restoration method (interpolation or extrapolation)

2: an indication of number of generated pictures for temporal restoration.

It was commented by the proponent that document [2] referred from VCM was adopted in VCM.

JVET-AO0097 is related.

It was asked if the indicated extrapolation has been done by encoder or it is something that should be done by the decoder for recovery. Proponent: More from encoder side. It is up to decoder if it wants to take action for restoration or not.

It was commented that JVET-AO0097 is also written from encoder perspective, but had some assumption about how decoder system recovers those frame.

Some typos in the name were observed.

It was asked how the bitstream conformance works with persistence.

JVET-AO0097 AHG9: Temporal extrapolation support for the EOI SEI message [M. M. Hannuksela, J. Boyce, F. Cricri (Nokia)]

It is proposed to add support for indicating whether decoding systems should perform temporal extrapolation or interpolation to recover frames that existed before encoding with the following additions to the extension of the encoder optimization information (EOI) SEI message:

- eoi_temporal_extrapolation_flag indicating if temporal extrapolation or interpolation is suggested for recovery of the original picture rate.

- When temporal extrapolation is suggested for the picture rate recovery, eoi_temporal_extrapolation_dir_flag indicates the direction of temporal extrapolation (forward or backward in output order).

- eoi_temporal_clvs_flag indicating whether pictures are suggested to be extrapolated/interpolated to precede the first picture in the CLVS or extrapolated to follow the last picture in the CLVS.

A clarification was asked about the language of eoi_temporal_extrapolation_dir_flag.

It was commented that EOI SEI should not ask decoder to do something specific.

The specification language in this SEI is a hint to the decoder.

Discussion about JVET-AO0082 and JVET-AO0097:

There was support to add the “extrapolation flag” in an EOI SEI extension and condition the flag using the if conditions from the above proposals – which are technically equivalent, but the flag values are flipped. Agreed to have such a flag using the text from JVET-AO0097.

About the extrapolation direction flag and temporal clvs flag: Agreed using the text from JVET-AO0097

We already have a flag similar to the clvs boundary flag in NNPF.

It was asked if “unknown” value should be considered for some flags. This aspect is for further study.

Regarding the bitstream conformance constraint from JVET-AO0082: the text from JVET-AO0082 should be taken and modified with the flag values from JVET-AO0097. JVET-AO0097 v2 text will includes this additional change.

Agreed to the the above.

JVET-AO0098 AHG9: On depth-based optimization in the EOI SEI message [M. M. Hannuksela, J. Boyce (Nokia)]

See notes under JVET-AO0281.

JVET-AO0157 [AHG9] EOI SEI message: Depth ranges definition [G. Teniou, S. Wenger, A. Hinds (Tencent)]

See notes under JVET-AO0281.

JVET-AO0172 [AHG9] EOI SEI message: Essential corrections to TuC [G. Teniou, S. Wenger, A. Hinds (Tencent)]

This contribution identifies corrections to be applied to the existing text of the EOI SEI message in the TuC in JVET-AN2032.

Several aspects are identified where the text in TuC has not been updated to correspond to the text from VSEI V4 final spec text. This “rebases” the text in TuC to the VSEI V4 final text. Agreed and delegated to the editors

Additionally semantics changes are included for eoi_luma_adaptation_idc to correspond to it being a ue(v) instead of u(8). On that aspect the current semantics are deliberate in defining 255 as the maximum value. So no change on that aspect.

JVET-AO0188 AHG9: On the EOI SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes to constrain the temporal sublayer of the current picture shall be in the range of eoi_min_tid to eoiMaxSublayer, inclusive, for the EOI SEI messages in the TuC. Proposed text:

It is a requirement of bitstream conformance that the temporal sublayer identifier value of the current picture associated with the SEI message is in the range of eoi_min_tid to eoiMaxSublayer, inclusive.

A case of sublayer extraction was pointed out as possibly causing an issue – for the currently proposed constraint and also in general for EOI SEI.

The example of problematic case relates to the case where the EOI SEI may have been put in a sublayer with TID > eoiMaxSublayer. Also if eoi_min_tid > 0 and if encoder desired to put the EOI SEI in TID 0 that would be prevented by the proposed constraint.

Futher discussion requested if updated text resolving above identified issues is made available.

Further discussed 1700 Wed 21 Jan. No action.

JVET-AO0281 AHG9: On depth-based optimization in the EOI SEI message [G. Teniou, S. Wenger, A. Hinds (Tencent), M. Hannuksela, J. Boyce (Nokia)] [late]

This contribution is a merge of JVET-AO0098 from Nokia and JVET-AO0157 from Tencent. It proposes:

- Addition of eoi_object_determination_idc, gated by the indicated use of object-based optimization. Value 0 indicates an unspecified or unknown object determination method. Value 1 indicates a depth-based object determination method. Agreed

- When depth-based object determination method is indicated, eoi_depth_source_type_idc is present with semantics as specified in the VSEI TuC with the addition of a code point for computer-generated content. Consider to reword it as “Computer-generated content with depth”. Agreed

- Removal of the eoi_depth_guided_encoding_method_idc as it can be interpreted in the eoi_object_based_idc values. Agreed

- Introduction of the support for multiple depth ranges not limited to foreground/background configurations and identifies the depth range on which the focus is set, when known. Agreed

- Reuse of the eoi_quant_threshold_delta mechanism already in VSEI v4 to signal delta QP for each depth range. This also removes eoi_forground_average_qp and eoi_background_average_qp.  Agreed

- Removal of eoi_depth_map_presence_flag, since it is redundant with the SDI SEI message. Presence of AUX_DEPTH already supports this use case. Agreed

Additionally 0x20 value for eoi_object_based_idc should be removed.

TuC already contains depth based eoi optimization.

### Digitally signed content messages (4)

Contributions in this area were discussed during 2340 on 2026 –00135 on 20 Jan. 2026 UTC and 15:20-15:45 on 20 Jan 2026 on (chaired by S. Deshpande).

JVET-AO0106 AHG9: On bitstreams for subpicture-based signing in digitally signed content SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to clarify the bitstream conditions for subpicture-based signing in the digitally signed content SEI messages in TuC for future extensions of VSEI. The proposed modifications are as follows:

- Disable subpicture-based signing if all subpictures are not independently coded. Also clarify which SPS is referred in the current constraint.

It was commented that in VVC we don’t have the concept of activation of parameter sets.

Delegate to the editors: specify SPS as that referred by the pictures associated with the DSCI SEI message.

Following comments were made about the independent subpictures aspect: it was asked if we need to separate case of some subpictures being dependent and some being independent – e.g. collection of subpictures. It could be encoder’s decision about which subpicture to sign. It may not make sense to sign dependent subpictures. The constraint as proposed does not achieve this.

There was also a suggested case where dependent subpictures may mix with independent subpictures where a group of such mixture is to be extracted. It was commented that this may not be supported.

It was agreed that subpictures that are signed should be independently coded.

Revisit the text to capture the above agreement after offline discussion and considering agreement on item 2 below.

- On the dsci_picture_segment_id[ i ] values,

- Specify that the dsci_picture_segment_id[ i ] value shall be equal to one of the subpicture ID values.

- The subpictures that are associated with dsci_picture_segment_id[ i ] shall be independently coded.

It was asked if supictures and picture segments should be same or not. It was commented that picture segment is a VVC logical construct and Subpicture is a VVC concept. The proposal is already for VVC interface.

Agreed item 2.

3 Specify that a subpicture group associated with a verification substream shall have a rectangular shape without gaps. In the v2 of this contribution, the text implementation of item 3 was updated.

It was asked if the verification substreams are within the same picture. That was the understanding of proponents.

It was suggested to make verification substreams independent. JVET-AO0197 has a different way of handling groupings. It was commented that we should support signing independent subpictures together otherwise scalable nesting would be sufficient.

One possibility: Each verification substream shall be indepdent of other verification substreams when sgemtn flag is on

Another possibility: Verification substreams that are associated with group of subpictures cannot dependent on other verification substreams.

Suggestion: Don’t prevent some substream dependencies within set of some subpictures.

Revisit based on the outcome of JVET-AO0197.

JVET-AO0107 AHG9: Miscellaneous aspects of digitally signed content SEI messages [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes miscellaneous modifications that are asserted to provide improvement to the digitally signed content (DSC) SEI messages in TuC for future extensions of VSEI (JVET-AN2032). The proposed modifications are as follows:

1 On implicit association mode when subpicture-based signing is enabled:

- Option 1: Specify that when dsci_vss_implicit_association_mode_flag is equal to 1, dsci_picture_segment_mode_flag shall be equal to 0.

- Option 2: Modify the syntax table such that dsci_picture_segment_mode_flag is present only when when dsci_vss_implicit_association_mode_flag is equal to 0.

Current text does not specify implicit association mode operation with subpictures.

In option 2, SeiExtensionBitsPresentFlag is set to 1, but there are no bits signaled in extension under certain condition currently.

Option 1 is agreed.

2 Modify signaling of a picture segment ID and an associated verification substream ID to avoid redundancy.

The main benefit is possible bit-savings. Minor typos in example were fixed.

It was commented that DSCI typically occurs every few seconds. So bit-savings are not very important. Having two for loops as proposed may have benefit of harmonization with the approach of JVET-AO0197 (e.g. to later support a subpicture being in more than one substream).

It was commented that the constraint included at the end of the text may have some issues regarding extraction use cases.

Agreed but with the removal of the constraint.

3 Constrain the total number of picture segments.

No action needed on item 3 dues to the decision on item 2 above.

JVET-AO0197 AHG9: DSC subpicture support using segment sets [J. Boyce, M. M. Hannuksela (Nokia)]

A backwards compatible approach is proposed for digital signing of sets of subpictures with the DSC SEI messages, with entire pictures also signed. A DSC signature is provided for segment sets of a substream, where a subpicture is a segment. A default segment set is defined that includes all segments in the picture, i.e. the default 0-th segment set contains the entire picture, and is always signed. Other segment sets which contain a subset of the segments are referred to as additional segment sets. Receiving systems are able to verify that entire pictures in a substream have not been modified, and can also verify that any additional signed segment sets in a substream have not been modified.

This could be an alternative to the current TuC subpictures design.

It was commented that using multiple dsci_id could also support backward compatibility using current approach. In current TuC one subpicture can only be assigned to one substream.

Does the proposed approach allow knowing in DSCI which subpictures are in which substream.

Answer: DSCI not currently, but yes in DSCS. But it can be added redundantly in DSCI.

Is SPS and PPS included or excluded in the signing for substreams in case of extraction? Answer: Initial thought is Parameter sets will be included in each substreams.Parameter sets can’t be rewritten as otherwise re-signing will be required.

It was asked if implicit association is no longer supported with the proposed approach.

There was some discussion regarding comparing overheads of the proposed approach and the TuC approach.

It was also suggested to consider harmonizing the proposed approach and the one in TuC.

Following options were discussed:

Option 1: Further study encouraged to harmonize proposed approach with the current TuC approach

Option 2: Add to TuC as an alternative option

Option 3: Add to TuC and remove the current approach in TuC

Option 1 is chosen.

Revisit to consider unified solution’s desirable characteristics.

JVET-AO0214 AHG9: editorial updates for VSEI TuC Digitally Signed Content SEI [I. Sodagar (Dolby)]

This contribution proposes a minor editorial correction for VSEI TuC Digitally Signed Content. The proposed changes are included in an attachment to this contribution.

A limit is proposed on the number of dsci_verification_substream_id[i] to be in the range of 0 to dsci_num_verification_substream_minus1.

It was asked if this will have backward compatibility issue with VSEI V4. It was the understanding that this is already in VSEI v4 , thus making this change editorial.

Agreed.

### Film grain regions characteristics SEI message (6)

Contributions in this area were discussed during 1545–XXXX UTC on Tuesday 20 Jan. 2026 (chaired by J. Boyce).

JVET-AO0064 AHG9/AHG13: On Film Grain Regions Characteristics SEI Message [S. Deshpande, J. Samuelsson-Allendes (Sharp)]

This contribution relates to modifications to film grain regions characteristics (FGRC) SEI message, which is included in the Technologies under consideration for future extensions of VSEI. Following is proposed:

- Proposal 1: It is proposed to signal fgr_regions_model_id[ i ] for the i–th region in the picture, only when fgr_film_grain_enabled_flag[ i ] is equal to 1 and fgr_single_model_flag is equal to 0. The inference rule for fgr_regions_model_id[ i ] is also modified when it is not present.

- Proposal 2: It is proposed to modify the inference for fgr_comp_model_value[ m ][ c ][ i ][ 0 ] when not present. It is proposed to infer fgr_comp_model_value[ m ][ c ][ i ][ 0 ] to be equal to 0 when not present.

- Proposal 3: It is proposed to signal the syntax elements fgr_region_top[ i ], fgr_region_left[ i ], fgr_region_width[ i ], and fgr_region_height[ i ] with ue(v) coding instead of u(16) coding. Alternatively, it is proposed to signal a syntax element which specifies the length of these syntax elements and then to code them with u(v) coding.

JVET-AO104 item 1 is similar to proposal 1, but with different syntax. This contribution’s proposal 1 was preferred.

Agreed to proposal 1, proposal 2, proposal 3.

JVET-AO0099 AHG9: On the use of alpha planes with the FGRC SEI message [M. M. Hannuksela, J. Boyce (Nokia), J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Chaired by S. Deshpande on 16:20-16:45 on 20 Jan 2026.

The film grain regions characteristics (FGRC) SEI message in the VSEI TuC has support for using an alpha plane auxiliary picture to indicate sample-wise weights for applying the film grain. It is asserted that the VSEI TuC text for the alpha plane support in the FGRC SEI message has a few problems. To fix the asserted problems, the following changes are proposed in this contribution:

- It is proposed to indicate an associated layer among the alpha auxiliary layers associated with the same primary layer with an index fgr_alpha_layer_idx, the presence of which is gated by fgr_alpha_channel_adaptation_flag equal to 1. Specification text updates on the FGRC SEI message are proposed to refer specifically to the associated alpha auxiliary layer.

The contribution JVET-A0103 item 2 is related and has 3 options, two of which do not include syntax change. The third option is similar but signals layer ID instead layer idx, but that option does not need to be discussed after review of the item 1 in this proposal.

It was commented that additional signaling (instead of just constraints) provided more flexibility and thus is preferred.

Agreed.

- The following additional constraints are proposed:

  - When fgr_alpha_channel_adaptation_flag is equal to 1, the number of alpha auxiliary layers associated with the primary layer containing the FGRC SEI message shall be greater than 0.

  - The width and height of the cropped decoded pictures of the associated film grain auxiliary layer shall be respectively equal to the width and height of the picture for which the film grain parameters are indicated.

  - When fgr_alpha_channel_adaptation_flag is equal to 1, it is proposed that a scalability dimension information (SDI) SEI message with one or more alpha plane auxiliary layers associated with the current layer shall precede the FGRC SEI message in decoding order in the current CVS.

Item 2.c was added in version 2 of this contribution.

The proposed constraints replace existing constraints which were asserted to be a bit less precise.

It was asked if the current constraints are not sufficiently clear and more detailed text as proposed really needed. The proposed text makes the constraint more explicit and precise. It was commented that the proposals adds fair amount of new text.

There was a general discussion about having a precise constraint versus more generally worded constraint. And which one of those should be preferred. The comments mentioned deciding on case-by-case basis. Also some experts preferred more precise constraint.

Agreed to 2a and 2c.

Regarding item 2b: There was a comment keeping the current flexibility and adding a clarification statement/ note : that the auxiliary picture may need to be resampled (delegate such a note to the editors).

Also we have auxiliary sample alignment SEI. However that SEI relates to auxiliary and primary picture, where as this one related to alignment between auxiliary picture and film grain.noted that some change in the wording from sdi_num_associated_primary_layers_minus1[ i ] to sdi_num_associated_layers_minus1[ i ] should be made.

JVET-AO0102 AHG9: On handling film grain characteristics SEI message and film grain regions characteristics SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes to clarify how to handle film grain characteristics (FGC) SEI message and film grain regions characteristics (FGR) SEI message in a bitstream.

The summary of the proposed modifications is as follows:

- Option 1: When both an FGC SEI message and an FGR SEI message are present and have persistence over the same picture,

  - Option 1−1: Specify that decoders shall ignore the FGC SEI message.

  - Option 1−2: Specify that the order applying the FGC and FGR SEI messages is specified by an SEI processing order (SPO) SEI message.

  - Option 1−3: Add a syntax element that indicates how decoders work.

- Option 2: Specify that an FGC SEI message and an FGR SEI message shall not be present and have persistence over the same picture.

In the v2 of this contribution, an editorial error in option 2 was fixed.

An expected use case for having both FGR and FGC in the same bitstream is so that decoders that only support FGC may use it.

Use of “shall” for the decoder in this context seems problematic. Using “should” seems more appropriate.

It also seems desirable that the language say that at most one of the two SEI messages should be applied. A note could possibly be used.

A question was raised about interaction with SPO. There should not be any requirement that SPO be present. When SPO is not present, all SEI messages are in the default chain. Consider if any special language is needed if both FGC and FGR are in the same processing chain.

Revisit after new language based on option 1-1 is available as described above.

JVET-AO0103 AHG9: On alpha channel adaptation in film grain regions characteristics SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes modifications that are asserted to provide improvement to alpha channel adaptation in the film grain regions characteristics SEI message in TuC for future extensions of VSEI. The proposed modifications are as follows:

JVET-A0103 item 2 : please see notes under JVET-AO0099 item 1.1	Clarify that monochrome alpha plans are used for alpha channel adaptation.

2	On alpha auxiliary layer used for alpha channel adaptation

–	Option 1: clarify that the layer with the least layer ID value is used.

–	Option 2: clarify that the layer with the least layer ID value, which is greater than the current layer ID, is used.

–	Option 3: add syntax element indicating layer ID to be used.

3	Disallow from disabling both alpha channel adaptation and region-based adaptation.In the v2 of this contribution, the implementation text for item 2 were updated.

For item 1, it was suggested that the proposed text has incorrectly removed “c = 0 .. 2” text in the last paragraph.

Agreed to item 1, with the fix described above.

For item 2, see notes under JVET-AO0099 item 1.

Regarding item 3, concern was expressed that the proposed constraint would be overly restrictive because it may provide information such as the target resolution even for a single region which is not supported by the existing FGC SEI.

No action on item 3.

JVET-AO0104 AHG9: On region-based adaptation in film grain regions characteristics SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Chaired by S. Deshpande on 21:00-21:30 on 20 Jan 2026.

This contribution proposes modifications that are asserted to provide improvement to region-based adaptation in the film grain regions characteristics SEI message as follows:

1 Signal fgr_regions_model_id[ i ] only when fgr_film_grain_enabled_flag[ i ] is equal to 1. See notes under JVET-AO0064 proposal 1.

2 Disallow the values of fgr_region_width[ i ] and fgr_region_height[ i ] equal to 0. Agreed

3 Modify the signaling of multiple models for future extensions.

Some concern was expressed about the amount of proposed changes not being minor. The benefit is unclear. A comments was made that it is not clear than more than two models will be used. Even the current second model may not be currently very popular (Autoregressive). No action.

Alternative suggestion was to change fgr_model_id from u(2) to u(1). There was some objection to remove that ability.

4 Relax the constraint on the unsupported value of fgr_regions_model_id[ i ]. Includes a number of syntax and semantics change.

It was commented that this change may result in some regions adding the film-grain and some regions not adding them – even when encoder asked to add film-grain. This behavior may not be optimal – compared to dropping the entire SEI in that case. No action.

5 Introduce a syntax element that indicates applying order of film grain synthesis for regions.

Currently nothing is broken, the proposal adds a new mode of operation (in addition to current behavior) for processing regions in a different order. It was commented that the current approach works and there is no real additional benefit to the added syntax and alternative ordering. No action.

6 Introduce a syntax element that indicates subpicture ID corresponding to a region.

Motivation is possible bit-savings in SEI and making it not possible to signal incorrect values. It was commented that the proposed method idc could be signaled outside of the for loop.

A comment was made that the primary goal is bit-savings and the case would be VVC specific and so not very attractive. No additional functionality is added by the proposal.

It was commented that subpictures in VVC don’t support overlap. Also the information about subpictures needs to come from SPS (a different location). The subpictures and film grain regions are not the same concept. Also there are many text changes proposed.

VSEI does not have a concept of subpictures and it may not be easy to define that concept. It was commented that TuC are already using subpictures and there is a definition for it in TuC.

No action.

JVET-AO0105 AHG9: Miscellaneous aspects of film grain regions characteristics SEI message [J. Lee, H. Tan, C. Kim, J. Nam, J. Lim, S. Kim (LGE)]

Chaired by S. Deshpande on 21:30-22:00 on 20 Jan 2026.

This contribution proposes miscellaneous modifications that are asserted to provide improvement to the film grain regions characteristics SEI message in TuC for future extensions of VSEI. The proposed modifications are as follows:

1 Specify the value range of fgr_pic_width_in_luma_samples and fgr_pic_height_in_luma_samples.

Agreed

2 Specify the value of fgrModelidValue[ x ][ y ] when fgr_region_based_adaptation_flag is equal to 0. Agreed

3 Clarify the input which film grain synthesis applies to. The proposed text is mainly copied from FGC.

This is editorial correction and aligns FGRC to FGC SEI. Agreed

### Quality metrics SEI message (4)

Contributions in this area were discussed during 2320–UTC on Friday 16 Jan. 2026 – 0020 UTC on Saturday 17 Jan. 2026 (chaired by J. Boyce).

JVET-AO0091 AHG9: On quality metrics SEI message [X. Xu, S. Liu (Tencent)] [late]

Quality metrics SEI message aims to provide valuable quality information of the coded bitstream. On top of the existing design, additional syntax elements indicating the lowest quality value and the location of picture in the CLVS when such a case occurs, are suggested in the QM SEI message. It is asserted that this information is of importance to access the overall quality of the video content.

It is noted that the current QM SEI message enables signaling of per picture quality metrics.

A question was raised about temporal scalability and particular pictures being dropped.

The proposed syntax includes a pic index to indicate the lowest quality picture. It was suggested that it would be more appropriate to use the POC to uniquely identify the picture.

It may be more appropriate to use the name worst quality rather than lowest quality because some metrics decrease with improved quality. The semantics use the qm_metric_increasing_flag[ ][ ].

It was suggested that the lowest quality segment of pictures may be more important than the lowest quality individual picture. It is noted that having per picture quality metrics would allow the application to determine segment quality based on how it defined a segment.

If three colour components are signaled, the worst quality picture is signaled separated for each colour component.

No action.

JVET-AO0115 AHG9: On minimum value for the quality metrics SEI message [A. Ak, C.-H. Demarty (InterDigital)]

The quality metrics SEI message in the VSEI TuC specifies metric types as defined, unspecified or user-defined. For unspecified and user-defined metric types, descriptive information is signaled via additional syntax elements. During the last meeting, it was proposed to signal a max quality metric value for unspecified quality metric types to improve the receiver’s ability to interpret the quality metric values.

This contribution proposes to additionally signal a minimum quality metric value to further improve the interpretability of the signaled quality metric values.

The proposed syntax table is missing curly brackets.

For decreasing quality metrics, the minimum value is suggested to be particularly relevant.

It was suggested to add a constraint that the minimum value is less than the max value.

It was suggested to have separate presence flags for the min and max values rather than using a common presence flag.

A question was raised about handling of floating point values, especially for unspecified metrics. A question was also raised about signaling of infinite values. Further study on those questions is encouraged.

Agree to add the minimum value signaling as proposed.

JVET-AO0116 AHG9: On the quality metrics SEI message [A. Ak, C.-H. Demarty (InterDigital)]

Chaired by S. Deshpande during 00:05-00:45 UTC on 17 Jan 2026.

The quality metrics SEI message in the VSEI TuC specifies the video quality measured as a result of the last encoding/processing step. If the video content used for encoding is not pristine, it is reported that the signaled quality metric value may not represent the final quality after encoding since it does not take the source video quality into account. It is further reported that this leads to sub-optimal encoding and processing decisions after decoding the video.

This contribution proposes adding additional syntax elements to the quality metrics SEI message to specify the source video quality of the video. The presence of these syntax elements will enable to estimate the final quality.

It was commented that if you have multiple hops as in the example and if Q2 was higher (even when Q2 may have been low) and was reported as source after using Q3 that is still problematic.

It was asked if current QM SEI message can be repeated multiple times at each stage to achieve the intended use case.

A related contribution from previous meeting was JVET-AN0247 which was marked as a further study and assertedly had less new syntax. The use case has some support.

It was asked what is done if there is no knowledge of the very first source’s quality and how the intended use case is benefited by the proposed addition. It was commented that no-reference quality metric may be used. It was commented that in that case such a metric could also be applied at the last step.

It was commented that there are many syntax elements (send semantics) proposed making the overall SEI message complicated. It was asked if the syntax can be simplified.

Further study encouraged.

JVET-AO0192 AHG9: On the QM SEI message [Y. He, L. Kerofsky, S. Zhao, M. Karczewicz (Qualcomm)]

This contribution proposes the constraint update on the qm_three_component_flag[ i ] of the QM SEI messages in the TuC.

Agreed.

### Display overlays information SEI message (2)

Contributions in this area were discussed during 0040–0130 UTC on Saturday 17 Jan. 2026 (chaired by J. Boyce).

JVET-AO0079 AHG9: On DOI, BRI, and ECFI SEI messages for multi layer operation [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE), M. M. Hannuksela(Nokia), J. Xu (Bytedance)]

This contribution proposes the following items related to the DOI, BRI, and ECFI SEI messages:

1.	Clarify the presence layer of DOI, BRI, and ECFI SEI messages.

2.	Clarify the application of subsequent SEI messages based on the presence layer.

3.	Clarify the payload requirements for DOI SEI cancellation.

For item 1, see notes under JVET-AO0189.

For item 2, it was questioned was raised about the wording of the note and the usage it is trying to address.

The proposed note includes “shall” which is not supposed to be used in a note.

No action on item 2.

Item 3 identifies a problem with the current constraint. It is noted that the packed regions info SEI message has similar text.

Further discussion requested on item 3 after updating the proposed solution text.

Further discussed at 0105 UTC on Thursday 21 Jan. 2026, Chaired by S. Deshpande.

Further study on proposal 3 in section 4 of -v2. Proposal 3 touches on the issue of repetition of the DOI SEI message in higher layers.

In the -v2 version, proposal 4 was added as a result of the discussion on item 3.

It is noted that in the current design the DOI SEI message can be repeated in higher layers.

Agreed to proposal 4 in section 5 of -v2.

JVET-AO0125 AHG9: On display overlays information SEI message [K. Abe, T. Nishi (Panasonic)]

This contribution proposes improvements to the syntax and the semantics of the display overlays information SEI (DOI SEI) in the VSEI TuC draft of JVET-AN2032. The proposed modifications are as follows:

Proposal 1: Editorial bug fixes and improvements

Proposal 2: Resolve the issue of using the same index name with multiple meanings

Proposal 3: Resolve the issue of referencing the index in constituent rectangles SEI (CR SEI) from the index in DOI SEI

Proposal 4: Restrict the value of constituent rectangle type in CR SEI when referencing alpha component

Agreed for editorial changes in proposal 1.

Agreed for editorial name changes of the indices in proposal 2.

Agreed for bug fix in proposal 3.

For proposal 4, it is noted that cr_rect_type_idc[ ][ ] is not always present, and when not present is inferred to be texture. This constraint would require that it be present. Is it a problem if it is not properly labelled as alpha?

It was suggested that the CR SEI could infer cr_rect_type_idc[ ][ ] to an unspecified type rather than to texture.

It is not clear that such a constraint is necessary.

### Bitdepth range information SEI message (0)

Section kept as template for future use.

JVET-AO0079 is related.

### Colour mapping information SEI message (3)

Contributions in this area were discussed during 0050–0140 UTC and 01300-1330 UTC on Wednesday 20 Jan. 2026 (chaired by J. Boyce).

JVET-AO0168 AHG9: On CMI [L. Kerofsky, Y. He, S. Zhao, M. Karczewicz (Qualcomm)]

It is asserted the application of anchor point-based mapping in the CMI message has a loss of precision when the decoded image has greater bit depth than the bit-depth used to describe the color map

This proposal modifies the use of anchor points to preserve precision when the bitdepth of anchor points is less than bit depth of video i.e. nominalBitdepth < BitDepthY.

- First item proposes to limit range of anchor points based on the bit-depth of the LUT

- Second item proposes to modify the description of using the anchor points to generate the color mapped image.

The v2 version of this document fixes some variable name typos and modifies the use of scaled anchor points to be similar to that of JVET-AO0211.

Version v3 of this document fixes a value and aligns the limit of cmi_num_anchor_points_minus2 with JVET-AO0202 by replacing “-1” with “-2”.

Item 1 is related to JVET-AO0202.

Agreed on item 1.

Item 2 shifts the signaled syntax elements to have the effective bit depth of the target picture. In the case that the target picture has a lower bit depth, bit depth is reduced.

It is noted that the encoder may select the nominal bit depth to achieve the desired accuracy.

Agreed on item 2.

JVET-AO0202 AHG9: On the colour map information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

This proposal proposes the following items to the danmu information SEI message specified in JVET-AN2032.

- Apply the nominal bit depth and range to colour mapping function in polynomial form.

- Clip normDecSample according to the nominal range.

- Clip displaySample according to the display range.

- Scale normDecSample according to FullRangeFlag.

- Specify a range for cmi_function_nominal_bitdepth_minus8.

- Specify a range for cmi_num_bits_minus8.

- Modify the range for cmi_num_anchor_points_minus2.

- Specify a constraint for cmi_min_x, cmi_num_anchor_points_minus2 and cmi_delta_x_minus1.

- Inference cmi_uniform_spacing_flag when cmi_colour_map_id is equal to 1.

- Increase cmi_function_nominal_bitdepth_minus8 when cmi_colour_map_id is equal to 1 and modify the predefined values for cmi_anchor_point_x and cmi_anchor_point_y correspondingly.

- Increase cmi_function_nominal_bitdepth_minus8 when cmi_colour_map_id is equal to 2 or 3.

The contribution incorrectly labels Figure 1 as Jet while it is Turbo colour map.

Items 1, 2, and 3 relate to the polynomial equations.

It was questioned why for the polynomial functions it is desirable to map to the nominal bit depth.

Item 3 is also proposed in JVET-AO0211.

Item 2 was suggested to not be needed given the decision on JVET-AO0168.

No action on items 1 and 2.

Agreed to item 3.

Item 4 considers if the VUI indicates limited range or full range.

Revisit item 4 after offline study of the interaction with JVET-AO0168.

Regarding item 5, if a constraint is placed on the value of cmi_num_bits_minus8, its signalling should be changed from u(8).

Agreed to constrain cmi_num_bits_minus8 to be in the range of 0 to 8, inclusive, with the signalling changed to ue(v).

Agreed to item 6.

For item 7, see notes in JVET-AO168.

Agreed to item 8.

For item 9, the proposal should be updated to reflect the constraint on cmi_num_bits_minus8 to be in the range of 0 to 8.

Revisit item 9 after the proposal is updated as described above.

For item 10, it was questioned if there is any interaction with JVET-AO168.

Agreed to item 10.

For item 11, the cmi_function_nominal_bidepth_minus8 syntax element inference for Turbo and Viridis can be removed as they are not used.

Agreed as stated above.

JVET-AO0211 On colour mapping SEI message [J. Samuelsson-Allendes, S. Deshpande (Sharp)]

JVET-AO0166 AHG9: Showcase on Colour Mapping Information SEI [T. Biatek, J. Boyce, M. M. Hannuksela (Nokia)] [late]

Chaired on 21 January 2026 from 13:15-13:30 by S. Deshpande.

Colour Mapping Information (CMI) SEI has been adopted in the VSEI TuC during the 38th JVET meeting ( JVET-AL0219 ). The CMI SEI enable description of the use of colour maps that transform monochrome samples into RGB samples that have been selected for improved human visualization. This informative contribution provides a showcase and a TuC implementation of the CMI SEI, demonstrating its relevance for visualisation of monochromatic coded content. In addition, this contribution highlights some issues in the current design that need to be fixed for proper implementation.

Note: Version 2 is just adding the slide deck and video clip to the contribution.

A video clip was shown during the presentation.

The bitshifts on negative integers results in compiler warings. This was identified in the last meeting as not being an issue as long as the shift is positive. This was confirmed by several experts (including that this issue has been discussed during inverse transform design).

Also using integer 64 was required/ recommended along with handling of possible overflow. Further study was recommended on the possible overflow issue. It was commented if the number of bits of precision should be studied for the overflow issue. It was commented that the example of Viridis was dissussed as not to be done. Also with the agreed changes at this meeting a further study regarding overflow should be conducted to confirm that there are not issues.

Software is almost ready but will be updated to include agreed fixes from this meeting before submitting a MR.

This input contribution relates to the colour mapping SEI message in the TuC for VSEI. The following items are proposed:

- Add usage of displayBitDepth also for polynomial color maps (in addition to piecewise linear color maps).

- Add clipping operation to pseudo code for polynomial form.

- Rewrite of the function for piecewise linear color maps to operate with integers only, while at the same time preserving as high precision as possible.

In item 1, in addition to use of displayBitDepth adds a rounding offset for polynomials.

A question was raised if there should also be a rounding offset for the piecewise linear process.

Agreed on item 1.

For item 2, see notes on JVET-AO0202.

For item 3, the current design has a mix of fixed and floating point math for piecewise linear and this proposes to do only fixed point math.

Agreed on item 3.

### Localization and mapping information SEI message (1)

Contributions in this area were discussed during XXXX–XXXX UTC on XXday X Jan. 2026 (chaired by XXX).

JVET-AO0216 AHG9: Miscellaneous items for the LAM, PCI and RN SEI messages [J. Xu, Y.-K. Wang (Bytedance)]

Chaired by S. Deshpande on 22:00-22:25 on 20 Jan 2026.

This proposal proposes the following items to the localization and mapping (LAM), photosenistive content information (PCI) and resolution nesting (RN) SEI messages specified in JVET-AN2032.

For the LAM SEI message,

- Specify parameters according to lam_camera_id. This proposes making the cancel flag specific to camera id. The changed design will need multiple cancel flag SEIs to cancel all of the cameras in multiple camera case. This removes the functionality of global cancel for all cameras.

It was commented that in splicing case, the splicer may not know what id to cancel. It was answered that splicing case would be at the boundary of CLVS, but scope of this SEI is within a CLVS period. The aspect of a global cancel flag is for further study.

Agreed.

- Specify the behavior when lam_rotation_format_id is equal to 3. Agreed (editorial).

- Specify a constraint for lam_content_near_depth and lam_content_far_depth. Sensibility constraint. Agreed.

For the PCI SEI message,

- Specify a constraint for pci_content_std1_guideline1_idc and pci_content_std1_guideline2_idc. Sensibility constraint. Agreed.

- Specify a constraint for pci_content_std2_clause52_idc and pci_content_std2_clause53. Sensibility constraint. Agreed.

- Specify a constraint for pci_content_std3_clause231_idc and pci_content_std3_clause232. Sensibility constraint. Agreed.

the RN SEI message,

- Specify a constraint for rn_pic_width_in_luma_samples. Sensibility constraint. Agreed.

- Specify a constraint for rn_pic_height_in_luma_samples. Sensibility constraint. Agreed.

### Danmu information SEI message (3)

Contributions in this area were discussed during 2225UTC on Tuesday 20 Jan. – 0050 UTC Wednesday 21 Jan. 2026 (chaired by J. Boyce).

JVET-AO0077 AHG9: On danmu information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes the following items related to the danmu information SEI:

1.	Modify the language signalling so that it can be optional.

2.	Include language information in the default properties.

It was suggested to have a flag that indicates whether or not language is present, rather than modifying the num languages syntax element to have a minus1. The proposed semantics for di_num_languages do not describe that case when there are no languages.

There was support for having the option to not signal the language.

It was suggested that the inference for di_lang_idx[ i ] when di_num_languages is equal to 0 would be that it was unspecified or determined via external means.

This particular language occurs in many locations in the spec: “unknown or unspecified or specified by external means not specified in this Specification.” This language is also present: “or "unknown or unspecified or determined by other means not specified in this Specification”

Revisit item 1 to consider the additional language for inference and adding semantics for the zero languages case, as described above.

Item 2 is also proposed in JVET-AO0155 item 4 with the same syntax.

Support to have signaling of a default language.

Agreed for item 2.

JVET-AO0155 AHG9: On the danmu information SEI messsage [J. Xu, Y.-K. Wang, B. Xu, L. Zhang, K. Zhang (Bytedance)]

This proposal proposes the following items to the danmu information SEI message specified in JVET-AN2032.

- Modify the name and semantics of di_global_fixed_duration_flag.

- Specify inferred values for default danmu parameters.

- Unify the default parameter and fixed duration machnism.

- Add a syntax element to specify a default language index and infer the language index correspondingly when a danmu uses default parameters.

- Add a syntax element to specify presentation priority for danmu

- Modify the notes for startPoc[ i ] and endPoc[ i ].

- Add POC definition in the text.

For item 1, it was suggested to use the language for unspecified discussed in the review of JVET-AO0077. The appropriateness of the proposed new syntax element name depends on some of the other proposals in this contribution.

It is suggested that the SEI message doesn’t need to assume that a fixed duration will be used, and that a changing duration can be determined by the application, and hence the current name, di_global_fixed_duration_flag may not be appropriate.

Agreed to item 1 with modified language for unspecified, item 2, item 3.

For item 4, see notes for JVET-AO0077.

Regarding item 5, priority can be provided by the service provider.It was suggested that the Danmu SEI is primarily intended for archival purposes where priority information would be less important.

It is noted that the current proposal does not make the priority optional but always included. A presence flag could be added or a value could mean unspecified.

Agreed to item 5.

For item 6, it was suggested to remove “all the pictures” from the note, to just say “picture”

It was suggested that POC should be signaled as a delta rather than an absolute POC. The current syntax elements signal delta POCs.

Notes are not supposed to use “should”. If we use the should language, it can be written to not be a note.

It was suggested to use “visible” language instead of “presented”.

Agreed to item 6 as modified to not be a note and to say the “a danmu should be visible for pictures with POC values in the range of startPoc[ ] to endPoc[ ], inclusive.

Agreed to item 7.

JVET-AO0164 AHG9: on the Danmu SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

This contribution proposes refinements to the Danmu Information (DI) SEI message syntax and semantics based on feedback received during the JVET-AM and JVET-AN meetings regarding JVET-AM0182 and JVET-AN0232. The following enhancements are introduced:

- di_fixed_font_size_present_flag and di_fixed_font_size_idc, which indicate whether a single global relative Danmu font size (e.g., small, medium, or large) is signaled

- di_open_font_compliance_flag, which indicates whether the Danmu text is Open Font Format (OFF) compliant.

Proposal 1 signals a small/medium/large font size that applies to all Danmus.

It was suggested to use “rendered text size” rather than “font size” in the new syntax element.

Agreed to proposal 1, with syntax element naming deferred to the editors.

Regarding proposal 2, it is not clear how the proposed font compliance flag would be used because font is not signalled in the Danmu SEI message.

No action on proposal 2.

### Graphics rendering information SEI message (1)

Contributions in this area were discussed during 1330–1400 UTC on Wednesday 21 Jan. 2026 (chaired by J. Boyce).

JVET-AO0203 AHG9: On the graphics rendering information SEI message [J. Xu, Y.-K. Wang (Bytedance)]

This proposal proposes the following items to the graphics rendering infromation (GRI) SEI message specified in JVET-AN2032.

- Modify the derivation of PrevGriSeiPresentFlag.

- Modify the inference for various syntex elements when PrevGriSeiPresentFlag is equal to 1.

- Infer gri_far_plane when PrevGriSeiPresentFlag is equal to 0.

- Infer gri_focal_length when PrevGriSeiPresentFlag is equal to 0.

- Modify the constraint of gri_near_plane and gri_far_plane.

For item 1, it was suggested to use output order instead of decoding order for the derivation of the PrevGriSeiPresentFlag to be consistent with the persistence of the SEI message.

Agreed to item 1 with replacing decoding order with output order.

Agreed to item 2.

For item 3, it was suggested that if the near plane was undefined, the inference should also be undefined, as is the case in the current design.

The proponent questioned why the near plane has an inference value and the far plane does not, and proposes that they be treated in a similar way.

Revisit item 3 and item 4.

For item 5, gri_depth_inverse_flag should be taken into account if a constraint is going to be made on the relative values.

Agreed to item 5 with the modification to have separate conditions for the two values of gri_depth_inverse_flag.

### Photosensitive content information SEI message (1)

Contributions in this area were discussed during 1400–1430 UTC on Wednesday 21 Jan. 2026 (chaired by J. Boyce).

JVET-AO0216 is related.

JVET-AO0080 AHG9: On photosensitive content information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

This contribution proposes the following items for the photosensitive content information (PCI) SEI message:

- Item 1: Modify byte alignment condition for PCI description strings

- Item 2: Method for signalling application-defined photosensitivity classification

It was noted that the reserved bits were intended to allow for future values.

It was questioned if there are use cases where both the standard indicators and the application defined indicators are needed. It is noted that in the current design, the standard indicators have a value to indicate unknown.

It was noted that the current semantics described values in the order 1, 0, 2, 3, which is somewhat confusing and it is suggested to make an editorial change to put in 0, 1, 2, 3 order.

It seems strange to have an unknown value for the application defined indicator when there is a presence flag.

Two possible options to avoid that situation:

- a presence bit followed by a single bit indicator

- no presence bit and a two bit indicator which includes an unknown value. This could use two of the current 4 reserved bits

No action on item 1.

Agreed to add a 2 bit application defined indicator, without a presence flag, using 2 of the 4 current reserved bits. The semantics to be aligned the current std1, etc. indicators, which will be editorially change to be in order of 0, 1, 2, 3.

### Sample interleaving SEI message (2)

Contributions in this area were discussed during 1430–1520 UTC on Wednesday 21 Jan. 2026 (chaired by J. Boyce).

JVET-AO0119 AHG9: On the Sample Interleaving SEI message [C.-H. Demarty, L. Blondé, A. Ak (InterDigital)]

This contribution proposes the addition to the Sample Interleaving SEI message of information related to the mosaic picture, to ease both the re-interleaving process and subsequent demosaicing process.

It is reported that the Sample Interleaving (SI) SEI defined at the previous meeting enables signalling of de-interleaving and grouping operations and provides metadata for the single case of a Bayer colour filter array. It is asserted that other mosaic picture types are used that can benefit from the extension of the Sample Interleaving SEI message to other colour filter array types. It is also asserted that the SEI message can also drive a further demosaicing process at the decoder side, when other acquisition-related parameters are added to the message.

The proposed addition includes signalling of the image mosaicking type and of additional CFA motifs, with parameters describing the motif spatial arrangement, and colour components, enabling more reliable reconstruction of mosaic pictures and more effective use of decoder-side demosaicing tools.

Discussion on the values of some variables in the Sample Interleaving SEI message is also proposed.

In v2 version, the introduction is reworked to better emphasize the three points of the proposal:

- Point #1: add syntax elements to the SI SEI message to enlarge the scope to other types of filter arrays for the de-interleaving/interleaving process;

- Point #2: add syntax elements to the SI SEI message to enable carrying information of the mosaic picture useful for a subsequent demosaicing process.

- Point #3: remove all that is related to gamma correction in the SI SEI message

Proposes a table of colour filter types and defines dx, dy, low code value, and high code value for those entries, and supports an application defined value where dx, dy, and low and high code values are explicitly signalled.

It was suggested that the low and high code values should be signalled.

The table should include reserved values.

There is an unspecified value in the table with dx=0 and dy=0. Would the SEI message even be sent in this case? Perhaps dx and dy should also be undefined rather than assigned a value of 0. The proponent suggests that the values of 0 would avoid actually doing an interleaving process.

It was questioned if normative references would be needed for the table entries and if such references be found.

It is noted that the proposed entry 7 provides additional information that isn’t just a name.

Two table entries include a company name, which we avoid in our specifications.

It is preferred to explicitly signal the dx and dy values than to avoid them with the table.

The contribution proposes to support more colours, including hyperspectral. There is no current support in any of our specs for describing hyperspectral content.

It was noted that we tend to provide full descriptions or refer to a normative reference.

Proposal 3 suggests to remove the gamma correction aspect of the sample interleaving SEI message and suggests that the demosaicing process should be applied on non-gamma corrected data.

It was noted that the current process maps the samples back to the linear domain before applying the demosaicing process.

Further study encouraged.

Chaired further by S. Deshpande from 1600-1610 on 21 January 2026.

Items for further study include:

-WhetherProposes a table of colour filter types and defines dx, dy, low code value, and high code value for those entries, and supports an application defined value where dx, dy, and low and high code values are explicitly signalled.

It was suggested that the low and high code values should be signalled.

The table should include reserved values.

There is an unspecified value in the table with dx=0 and dy=0. Would the SEI message even be sent in this case? Perhaps dx and dy should also be undefined rather than assigned a value of 0. The proponent suggests that the values of 0 would avoid actually doing an interleaving process.

It was questioned if normative references would be needed for the table entries and if such references be found.

It is noted that the proposed entry 7 provides additional information that isn’t just a name.

Two table entries include a company name, which we wantavoid in our specifications.

It is preferred to explicitly signal the dx and dy values than to avoid them with the table.

The contribution proposes to support additional patterns other than Bayer more colours, including hyperspectral. There is no current support in any of our specs for describing hyperspectral content.

-Whether we want to support demosaicing information

-Whether we want to support hyperspectral

-Whether to signal the parameters versus using a table (or nothing)

-how and whether to handle bit depth of Bayer samples versus coded data (do we want to include non-linearity/ gamma in this conversion)

In general an incremental approach to design this message (from its current empty stat) is considered as a possible way forward.It was noted that we tend to provide full descriptions or refer to a normative reference.

Proposal 3 suggests to remove the gamma correction aspect of the sample interleaving SEI message and suggests that the demosaicing process should be applied on non-gamma corrected data.

It was noted that the current process maps the samples back to the linear domain before applying the demosaicing process.

Further study encouraged.

JVET-AO0149 AHG9: Text fixes for sample interleaving SEI message [J. Chen, Y. Ye (Alibaba)]

Chaired on 21 January 2026 from 15:45-16:00 by S. Deshpande.

This contribution is proposed to fix the asserted bugs in the current text of sample interleaving SEI message in TuC version 10. The proposed fixes include:

- Specify the persistence scope of the SI SEI message. Two options proposed. Option 1: semantics change only. Option 2: adds syntax.

It was commented that the sample interleaving is  not expected to change in a CLVS, thus the option 1 may be preferred. Additionally it was suggested that all SI SEI messages should have the same content.

It was commented that such changes are typically done when the SEI message is mature and at this point this SEI is empty. One option was to remove the second sentence.

Agreed option 1 with further addition of a sentence about similar content for all SEIs.

- Add the definition for the variable DecodedPicture[x][y] which is used in the text but undefined. Agreed (editorial) with changing the range to have height-1 and width-1 and the word inclusive. Also change the name from DecodedPicture to DecodedPictureY.

Some other editorial fixes: delegated to the editors.

### Resolution nesting SEI message (0)

Section kept as template for future use.

JVET-AO0216 is related.

### Enhanced colour format information SEI message (0)

Section kept as template for future use.

JVET-AO0079 and JVET-AO0189 are related.

## Identify potential needs for additional SEI messages, extensions of existing SEI messages, and study of AVC and HEVC SEI messages for use in VVC (14)

### SEI messages related to Gaussian splatting technology (7)

Contributions in this area were discussed in a joint meeting with WG 4 and WG 7 during 1300–1510 UTC on Tuesday 20 Jan. 2026 (chaired by JRO, M. Preda and L. Yu).

JVET-AO0152 [Plenary] JVET work on 3D Gaussian Splat SEI [S. Wenger, G. Teniou, J. Ricard (Tencent), Y. He, G. Van der Auwera (Qualcomm)]

Was discussed in joint meeting with VCEG. See notes in section XX.

JVET-AO0179 AHG9: Description of a Test Model workflow for 3DGS compression using GSI SEI [J. Ricard, G. Teniou, S. Wenger (Tencent)]

This contribution describes a workflow for the compression of 3D Gaussian Splatting (3DGS) data. The proposed software platform implements an encoding and decoding chain that organizes 3DGS attributes into video frames using the implicit packing mode defined in the Gaussian Splatting Information (GSI) SEI message. The workflow includes point sorting, splitting of position data into MSB/LSB blocks, quantization of the data, and the use of modified VTM/HM test model reference software to compress the video and embed the GSI SEI metadata.

To maximize spatial correlation and ensure that adjacent pixels in the video blocks represent close values, the encoder sorts the point cloud. It is also possible to pack only attributes of one type (from different Gaussians) into one picture, which increases the correlation compared to the case where different attributes are put side by side.

It was claimed by the proponent that the SEI message provides more flexible options of packing than V-PCC, and also has more efficient packing of LSB/MSB packing of the geometry information. Experts from WG 7 commented that for V-PCC the encoder might also exercise alternative ways of packing than the one used in the test model, and could in principle also pack attributes of one 3DGS picture into multiple video pictures.

JVET-AO0180 AHG9: Gaussian splatting information SEI message [Y. He, J. Jung, L. Kerofsky, G. van der Auwera, M. Karczewicz (Qualcomm), J. Ricard, G. Teniou, S. Wenger (Tencent), J. Xu, Y.-K. Wang, L. Zhang (Bytedance)]

This contribution proposes a Gaussian Splatting Information (GSI) SEI message to support video-based Gaussian Splat Coding (GSC). The proposed GSI SEI message harmonizes the previous work in JVET-AN0100 and JVET-AN0205 to achieve efficient and lightweight representation of 3DGS. It provides a high degree of flexibility in storing 3DGS components across video planes, making it suitable for diverse 3DGS data formats.

Attributes of splats are packed into samples of a video picture. In a pre-processing step, splats are sorted such that the samples can efficiently be represented by the subsequent encoding (see doc. JVET-AO0179).

One 3D picture described by splats can be packed into multiple video pictures (entitled as constituent frames). For dynamic splats, this would mean that the video framerate could be different from the number of 3D splats per second.

Syntax and semantics were not reviewed in detail.

It was commented that it is relevant that the upper limit of the GS size would depend on the picture size and number of samples per second defined in a certain level. This however not relevant as long as not dynamic splats are encoded, for a single 3DGS no true real-time decoding is needed.

It was commented that it is important that quantization is done individually per component, which is foreseen in the SEI message. It high precision is needed, the information of an attribute could be packed into more than one sample.

JVET-AO0181 AHG9: Experimental results of 3DGS compression using Gaussian Splatting Information SEI message [J. Ricard, G. Teniou, S. Wenger (Tencent)]

This contribution presents experimental results for Video-based Gaussian Splat Coding (GSC). The results are generated using the Test Model workflow described in JVET-AO0179,  using the Gaussian Splatting Information (GSI) SEI message described in  JVET-AO0180. The performance is compared to the standard GPCC anchors defined in the Gaussian Splat Coding Common Test Conditions.

Versions v2 and v3 of this document add additional comparison results presented in particular the results presented by [m75737].

The experimental evaluation compares the proposed TMCV solution (utilizing the GSI SEI) against the following references using the MPEG GSC Common Test Conditions:

- SPZ: Niantic SPZ

- SOG: Spatially Ordered Gaussians

- Draco: Google Draco

- vpcc_m75025: Video-based Point Cloud Compression amendment 1 [m75025].

- vpcc_m75050: Video-based Point Cloud Compression amendment 1 [m75050].

- gpcc_mpeg152: Geometry-based anchor provides following MPEG 152.

- gpcc_combined: GPCC with several methods proposed in this meeting:

  - Min-Max Quantization for improved coding efficiency [m75009].

  - Canonical Scale and Rotation for improved coding efficiency [m75490].

  - High-precision RAHT for improved coding efficiency [m75584 ].

  - 3-channel coding for complexity reduction [m75636].

It is noted that “GPCC_combined” is approximately the expected GPCC test model at the end of this meeting.  m75050 is approximately the previous test model.

The results reflect the latest results provided at this meeting in [m75737].

Table 1. BD-Rate results of Forward-facing sequences: TMCV versus GPCC_combined.

Table 2. BD-Rate results of Object-centric sequences TMCV versus GPCC_combined.

Table 3. BD-Rate results of Forward-facing sequences: TMCV versus VPCC_m75050.

Table 4. BD-Rate results of Object-centric sequences TMCV versus VPCC_m75050.

It was asked why not results on the full set of sequences from CTC was investigated (all are mandatory).

It was requested to provide full results in an update of the contribution when available. Revisit.

It was remarked that the approach is significantly slower for the encoder compared to G-PCC, but faster than V-PCC.

HM was used in both VPCC and this contribution. It was commented that it highly depends on the degree of encoder optimization for the property of point clouds how the performance would be.Further investigation is necessary to cross-check complete results, and identify why (potentially) in a number of cases a significant difference is observed.

JVET-AO0182 AHG9: Software availability for the Gaussian Splatting Information SEI Test Model [J. Ricard, G. Teniou, S. Wenger (Tencent)]

This contribution announces the availability of the software implementation supporting the Gaussian Splatting Information (GSI) SEI message workflow.

For information. No need for detailed presentation.

JVET-AO0210 AHG9: Splatting information SEI [J. Boyce, M. M. Hannuksela (Nokia)]

A splatting information SEI message is proposed, which provides support for representing a volumetric scene with a variety of splatting types, including 3D Gaussian splats. Additional splatting types supported in the proposal are 2D Gaussian splats and Beta splats. Because the technologies for volumetric scene representation are rapidly evolving, the proposed design is extensible to add support for additional splatting types and includes support for a custom type determined by the application.

Characteristics of the proposal can be summarized as follows:

- Support for multiple splatting types: 3D Gaussian splats, 2D Gaussians splats, Beta splats

  - Table listing allowable attributes and attribute components for each specified splatting type

  - Reserved splatting types for future extensibility

  - Support for unspecified (application determined) splatting type

- Support for packing of attribute components into multiple layers and/or multiple temporal pictures

- Each picture split into a grid of same size regions, optionally separated by guard bands, with an index assigned to each region

- Per layer indication if attributes with 3 components have all 3 components placed into a single region or into separate regions

- Flag to indicate if all allowable attribute components are present

- Flag to indicate if default region packing used

  - If not default packing, signal indication for each attribute if it is present, and when present, signal for each attribute or attribute component its layer index, POC, and region index

- Optional transform per attribute

- Optional scale and offset values for each attribute component

The general concept is similar as for JVET-AO0180, as both JVET-AO0180 and JVET-AO0210 were developed from JVET-AN0100. The basic approach of packing could be similar, but JVET-AO0210 tries to introduce additional flexibility for supporting more types of splats, and packing into multiple pictures.

It was commented that it might be undesirable to define too many different packing formats, as it might increase the complexity of decoder implementation.

JVET-AO0213 AHG9: On implicit representations with the Gaussian splatting information SEI message [S. Lee, S. Sasse, Y. Sanchez, R. Skupin, T. M. Borges, C. Hellge, T. Schierl (HHI)]

This contribution is a follow-up on JVET-AN0264 that introduced support for implicit representations for coding 3DGS data by using NNPF SEI messages. The proposal offered superior coding efficiency and level requirements compared to naively mapping 3DGS attribute data onto the coded video planes explicitly.

In this contribution, a scene-adaptive contraction function is introduced to allow improving the mapping of 3D positions to the feature plane space and, hence, further improved coding efficiency. It is proposed to extend the syntax of the earlier proposed Gaussian Splatting Information SEI message with signaling of a contraction index into a list of pre-defined contraction functions as well as respective scale and offset parameters.

Basic concept:

- A Gaussian Splatting Information SEI message for mapping feature planes of the implicit 3DGS attribute representations onto video plane regions.

- Using NNPF SEI messages to recover the 3DGS attributes from the mapped feature planes

The working principle of the approach before performing encoding that is taken in this proposal is illustrated in the subsequent figure.

The bitstream consists of the encoded feature plane and parameters of NNPF needed for reconstruction.

A primary distinction compared to JVET-AN0264 is the contraction which is expected to provide more compression. Results on this are not available yet.

It was commented that this approach is an alternative way compared to VPCC, where in principle also a point cloud could be mapped into a feature space that is optimized for encoding in video pictures.

Initial conclusions:

Work together in joint AHG (to report to JVET in same way as JVET AHGs) to study the suggested packing methods also in comparison to V-PCC, to identify whether the SEI proposals and related implementations are lacking some elements, optimization, etc.

For coordination of experiments to be conducted towards the next meeting, Yiyi Liao should be contacted.

It was suggested to consider a possible agreement such that the method of carrying the data is secondary and different ways of encapsulating might even co-exist, as long as the organization of data and usage and tools is somewhat aligned.

### Other (8)

Contributions in this area were discussed during 2245–2320 UTC on Monday 19 Jan. 2026 (chaired by J. Boyce).

JVET-AO0076 AHG9: Signalling content moderation information in an SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE)]

Chaired by S. Deshpande from 16:10-16:25 on 21 January 2026.

This contribution proposes to indicate, in an SEI message, content moderation information such as whether content moderation has been applied, and optionally, the method and reason for the moderation. Three options are proposed as follows:

- Option 1: New SEI message for indicating content moderation information

- Option 2: Indication of content moderation information using text description information SEI

- Option 3: Indication of content moderation information using encoder optimization information SEI

It was commented that referring to some external authoritative standard to refer to regarding content moderation should be considered to make it clear what exactly is meant by content moderation. E.g. Otherwise the privacy protection operations (e.g. blurring etc.) may also be considered content moderation.

It was also asked if video elementary stream is the right place to indicate this kind of information.

It was asked what is the intended decoder behavior if it gets this SEI message. Answer: user may be informed, e.g. in UI.

Generally if we decide to do something related to the content moderation option 2 (as a general label or a new purpose) may be sufficient.

Further study.

JVET-AO0078 AHG9: Decoder post-processing information SEI message [C. Kim, H. Tan, J. Lee, J. Nam, J. Lim, S. Kim (LGE), X. Xu, S. Liu (Tencent), L. Kerofsky, Y. He (Qualcomm)]

Chaired by S. Deshpande from 16:25-16:55 on 21 January 2026.

This contribution proposes a decoder post-processing information (DPI) SEI message that signals recommended post-decoding processes to obtain the intended picture representation for a designated by encoder. The proposed SEI is asserted provides a unified mechanism to indicate the type and purpose of the required processing, and, when necessary, the information associated with that processing.

It was commented that many current SEIs have post processing defined, so what is the role of this new SEI message (For example frame packing was done by encoder, film grain to be synthesized etc.).

The goal of this SEI is to reversing something that is done by encoder.

It was asked if this SEI is only to reverse or recover what EOI SEI has done. That was generally the current goal. An alternative is to add these to EOI extension – such as what was agreed in this meeting regarding temporal extrapolation related information signaling in EOI extension.

The current scope is for reversing the EOI process, however the proposal v1 also includes sample interleaving SEI.

Another comments about we should clarify what is the role of this SEI compared to other already defined SEIs (e.g. NNPF extrapolation as indicated by NNPF SEIs).

It was commented that if sample interleaving reverse operations are indicated in this SEI then sample interleaving SEI may not be needed.

One proponent company thought it will be good to limit the scope of this SEI to EOI reverse process only.

It was commented that this SEI may balloon in size if it is a central place to define general reversing operations.

It was commented that as EOI SEI is already standardized requiring a new SEI to reverse the operations defined in extensions of EOI may cause confusion/ fragmentation and may diminish the usage of EOI SEI.

It was commented by proponents that EOI design (without ID) makes it inefficient (but is not broken) when additional extensions are made to EOI.

A comment was made that there may not be a need for this SEI at this stage due to the above mentioned aspects.

A commented was made that the decoder may not need to reverse the operations of the encoder and it may be up to the system design to do that if needed. Additionally in some case the revers operation may not be possible/ easy. Proponents thought it is better to give information to reverse the operations to the decoder.

Further study (possibly with the limited scope and to address above comments/ concerns).

JVET-AO0090 AHG9: Additional auxiliary picture type to support V-PCC bitstreams [X. Xu, S. Liu (Tencent)]

JVET-AO0112 AHG9: Dynamic Range Conversion Characterization Metadata SEI [D. Touzé, N. Caramelli, F. Plissonneau, P. Morvan, B. Redmann (InterDigital)]

This contribution proposes to define an SEI message for carrying Dynamic Range Conversion Characterization metadata defined in the SMPTE ST 2094-60 standard. These dynamic metadata specify five anchor points that characterize HDR-to-SDR or SDR-to-HDR conversions in production workflows.

It is asserted that the main goal of the proposed SEI is to provide a signaling mechanism to help production equipment to automatically select or configure the proper conversion adapted to the content producer’s requirements, helping the interoperability and mixing of different equipment and technologies in automated workflows.

A liaison was sent from SMPTE to MPEG in document 75822, which includes the following:

SMPTE would like to make JVET and MPEG aware of the publication of SMPTE ST 2094-60 “Dynamic Range Conversion Characterization Metadata – Application #6.”

SMPTE requests that MPEG and JVET include a dedicated SEI message for such metadata in the next version of the Recommendation ITU-T H.274.

It was questioned if a T.35 SEI message using a SMPTE code would be able to be used instead of a dedicated SEI message. A participant indicated that a T.35 SEI message is being used for other SMPTE ST 2094-X standards, such as SMPTE ST 2094-50.

It was noted that SMPTE recently received its own T.35 manufacturer code.

A question was raised about how frequently the professional workflows are modified.

Support from additional companies would provide more justification as to why a dedicated message is needed.

We can respond in the liaison to SMPTE to ask why they do not want to use a T.35 code, given that it is suggested for use with the SMPTE ST 2094-50 standard.

W. Husak, A. Tourapis, and D. Touze were asked to prepare a draft liaison statement response to SMPTE.

JVET-AO0120 AHG9: Historical Information SEI message [J. Pardo, A. Karabutov, H. You, A. Boev, J. Sauer, T. Solovyev, E. Alshina (Huawei)]

JVET-AO0158 AHG9: AI Content Provenance (AICP) SEI message [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

JVET-AO0160 AHG9: on MI SEI extension for multispectral picture [S. Zhao, Y. He, L. Kerofsky, M. Karczewicz (Qualcomm)]

## SEI Software and showcases (2)

Contributions in this area were discussed in JVET plenary during XXXX–XXXX UTC on XXday X Jan. 2026 (chaired by XXX).

JVET-AO0051 AHG9/AHG16: Feasibility Study and Showcase on Generative Human Video Coding based on GFV SEI Message [B. Chen, R.-L. Liao, J. Chen, Y. Ye (Alibaba), Z. Zhang, S. Yin, X. Ju, S. Wang (CityUHK)]

JVET-AO0166

## Non-SEI HLS aspects (3)

Contributions in this area were discussed in JVET plenary during 2150–2245 UTC on Monday 19 Jan. 2026 (chaired by JRO).

JVET-AO0201 Dual layer scalable VVC coding with external HEVC base layer [F. Urban, F. Galpin, E. François, P. de Lagrange (InterDigital)]

Multilayer coding with an external base layer is a functionality that is available in SHVC but not yet in VVC. A multilayer extension for an external base layer was proposed in JVET-AN0178 to add this functionality to VVC.

Using a VVC enhancement layer on top of HEVC instead of using SHVC reportedly shows a compression advantage of over 55%.

It was asked what applications might be? An example was given that HEVC legacy devices might be able to decode the base layer.

Why would a new profile be needed? Because currently VVC is not supporting this functionality in the VPS, and the only way to change it is by defining a profile.

The base layer does not need to be HEVC, can be any external video codec.

JVET-AO0209 VVC multilayer extension for external layers [J. Boyce, M. M. Hannuksela (Nokia)]

Modifications are proposed to contribution JVET-AN0178 “VVC multilayer extension for an external base layer” to enable layers other than a base layer with nuh_layer_id equal to 0 to be provided by external means. Modifications to the VVC backwards-compatible vps_extension( ) syntax structure of JVET-AN0178 are proposed. Profile language is also proposed to the existing VVC profiles to disallow external layers, and a new External Multilayer Main profile is proposed that allows use of external layers. An example use case for this functionality is externally provided stereo multi-view video coded using HEVC with spatial scalable enhancement stereo layers coded using VVC.

It is noted that this contribution is not proposed for use with the hybrid end-to-end intra coder under study in NNVC, which is asserted to not require a VVC backwards-compatible design.

One expert commented that the design of JVET-AO0209 appears more flexible than JVET-AN0178/JVET-AO0201 that it was developed from.

It was asked if it is possible that all layers are external? As external layers are intended to be used as reference, this could not be the case.

The systems layer would take care of synchronization.

It was commented that due to the fact that the multi-layer design of VVC was intended to be simpler than HEVC, therefore an external base layer option was not included in first place. In HEVC, profile, tie, and level (PTL) are specified to be layer-specific, while in VVC PTLs are specified at the output layer set (OLS) level.

In VVC, the design of base and enhancement layer is more similar, such that a base layer decoder could be repurposed as EL decoder.

In terms of complexity, there may not be an advantage of having two different decoders in base and enhancement.

It was requested to bring more evidence about potential applications and their requirements, before going to define a new profile. Coding efficiency as reported in JVET-AO0201 may not be an argument, as it is probably still worse than single-layer VVC.

JVET-AO0284 A Bitstream Extension Mechanism for New Technologies Based on Video Coding [Y. Liao, L. Yu (Zhejiang Univ.), X. Jin (Tsinghua SIGS), L. Zhao (China Mobile)] [late]

This contribution proposes an extension method based on existing video coding standards, e.g. VVC/HEVC, to support emerging technologies such as Lenslet Video Coding (LVC) and Gaussian Splatting Coding (GSC). The core idea is to utilize existing extension mechanisms in the syntax structures of VVC/HEVC to embed LVC or GSC-related bitstreams as extended sub-bitstreams directly into standard VVC/HEVC bitstreams to form a unified carriage mechanism. The extended sub-bitstream shall start with a specific leading bit pattern, which is fully compliant with the specified syntax structure extension of VVC/HEVC and may avoid any potential conflict with any future extension of VVC and HEVC. Meanwhile, the syntax structures for LVC or GSC-related bitstreams and their decoding processes do not need to be additionally defined in the VVC/HEVC standards, but can be specified in the respective LVC or GSC standards.

Was presented in joint meeting Mon. 19 Jan. 1540. See notes there.

# Plenary meetings, joint meetings, BoG reports, and liaison communications

## General

The following topics were discussed in JVET plenary Monday 19 Jan. 1300–XXXX:

- Status of documents – 2 cross-checks (JVET-AO0261 and JVET-AO0263) still missing – owner are requested to withdraw in case they don’t plan to finalize.

- Liaison communication (sec. 7.5) – 4 liaison inputs, responses to be reviewed on Wednesday, as the JVET sessions on Thursday start by the same time as the AG 3 session for reviewing liaison letters.

- Scheduling for the remaining week was discussed (further detail on scheduling is recorded in section 2.12)

- Joint meetings (different zoom link, see calendar)

  - Joint meeting with VCEG WG 4 on LVC (Mon. 1530-1600)

  - Joint meeting with VCEG WG 4/7 on GSC (Mon. 1600-1730)

  - Joint meeting with VCEG WG 2 AG 5 on NGVC (Thu. 1430-1600)

- Review status from main track

  - EE1 …

  - EE2 …

  - Review of complexity aspects (AHG7/19) – BoG

  - Review of CfP preparation (AHG17) – BoG

  - Review of ULL/ER (AHG18) – follow- up discussion about potential inclusion in CfP.

- Review status from HLS track and discussion on potential open issues

  - VSEI V4 FDIS – not yet submitted, some editorial changes as result of discussions at current meeting, and will also be integrated to H.274 before pre-publishing.

  - Aspects related to AVC mostly finished

  - Open issue: Text description SEI was requested to be included in AVC, but no software exists yet. Interest was also expressed to include in HEVC, but that has already been submitted for publication. It was agreed in plenary to defer it for some future versions.

  - A request was made to start a WD on localization mapping SEI, but so far no software exists yet. It was agreed in the plenary that a minimum requirement is implementation in software (at minimum writing and parsing), and in cases where concern is raised about usefulness, a convincing showcase needs to be demonstrated.

  - Discussion about a potential timeline for VSEIv5 was deferred for a later meeting.

  - Roughly half of contributions has been reviewed (not yet in the category of new messages), various revisits, likely not contentious.

- Output document planning (sec. 10)

- Review of documents at plenary level: categories 4.1, 4.3/4.4, 4.5, 4.12, 4.14

The following topics were discussed in JVET plenary Wednesday 21 Jan. 2100–XXXX:

- Chairs of joint AHG: W. Husak, J. Jung. They should get in contact with the corresponding chairs from WG 4 and 7, to negotiate mandates. Proposal on mandates to be discussed in HLS track on Thursday 1300. JVET will install a “mirror” AHG which may reflect mandates of interest to its work, and may have a subset of the mandates of the joint AHG.

- Status and open issues from review

  - main track: …

  - HLS track: Extensibility of SEI messages in AVC: It was generally agreed that new SEI messages in AVC could have an extension mechanism, but SEI messages that are already existing should not be extended. There are two SEI messages that are currently extensible in the context of VVC and HEVC (NNPFA and FGC), but not extensible in AVC. For those a possible solution could be to define the intended extensions by new SEI messages which would be extensible in the future, but coexist with the previous ones having different codepoints. To allow more study of this issue, it appears appropriate to defer the FDIS until April or July.

- Draft liaison responses to JPEG and SMPTE were reviewed.

- The latest version of the draft CfP was reviewed.

## MPEG information sharing meetings

Information sharing sessions with other WGs and AGs of the MPEG community were held on Monday 19 Jan. 0500–0800, Wednesday 21 Jan. 0500–0600, and Friday 23 Jan. 2100–2300.

The status and plans for the work in the MPEG WGs and AGs was reviewed at these information sharing sessions.

## Joint meetings

Raw notes by G. J. Sullivan from joint meetings on Monday; needs update and add section headers

- Lenslet Video Coding: Joint VCEG, MPEG WG 4 (Video) and JVET Monday 15:30-16:00 UTC (https://iso.zoom.us/j/92536820588)

- Gaussian Splatting: Joint VCEG, MPEG WGs 4 (Video) and 7 (Graphics) and JVET Monday 16:00-17:30 UTC  (same link https://iso.zoom.us/j/92536820588)

Lenslet

Lu Yu, JRO, GJS, Joern, Yan

WG 4 wants to create a new standard for lenslet video coding.

Additional coding tools are considered possible, but are planned to be outside of the coding loop.

So the new standard would be a post-processor (from the perspective of the “inner decoder”), producing transformed output pictures from each output picture.

Some HLS would be added/inserted.

Is it necessary to touch parameter sets, or could this be SEI or new NAL unit types?

Compatibility with existing decoders was suggested to be a concern.

If a new profile of HEVC or VVC is defined, this could cause existing decoders to reject the bitstream.

A bitstream extension mechanism is proposed in JVET-AO0284

Contrasted with MIV, V-PCC, VCM approach, it is proposed to put data within an HEVC or VVC bitstream.

The proposal would identify a short bit pattern as an extension type code for parameter sets.

It was commented that a similar extension mechanism had previously been proposed.

Should the base video standard identify what the particular pattern means?

Do parameter sets really need to be changed?

Several said SEI might be a better place to specify a post processing.

It was suggested that we could just use one reserved bit as the extension mechanism if parameter set extension is needed.

It was asked what would be the expected behaviour of an existing VVC or HEVC decoder. Comment – similar to frame packing – no impact on low-level decoding process.

Likely not a new profile from the HEVC or VVC perspective.

Could need an extension of every standard (AVC, HEVC, VVC).

How much extension data is involved?

Commented that parameter sets are ordinarily for the inner decoding, not post-proc.

It was suggested that post-proc could be normative even if this is done with an SEI message (e.g., like Green metadata, would not necessarily need to be directly specified in VSEI).

WG 4 suggested not to mention this extension in the video specs. Others thought at least identifying the extension code and payload size would be needed, saying there is potential for so-called “collisions” in the future to occur.

It was suggested that the interaction with layered coding and with schemes such as rectangle extractions and compositing would need to be considered.

It was suggested that integrating a specified functionality, possibly with a large amount of extra data, should be considered, together with schemes such as film grain.

Further study seems needed. An SEI approach such as with Green metadata, should be further studied.

SEI with normatively specified post-processing was suggested to be appropriate for this and other similar schemes.

Note architecture diagrams to be updated in the JVET-AO284 doc.

16:20 UTC

- Gaussian Splatting: Joint VCEG, MPEG WGs 4 (Video) and 7 (Graphics) and JVET Monday 16:00-17:30 UTC  (same link https://iso.zoom.us/j/92536820588)

Lu Yu, JRO, GJS, Joern, Yan, Marius

- Incoming liaison statement from SG 21.

- Summary of status in MPEG WG 4 & 7.

- ~7 technical documents for SEI submitted.

- JVET-AO152 suggested for focus.

Liaison statement

This liaison statement expresses to ISO/IEC JTC 1/SC 29 and its working groups an interest in collaboration on 3D Gaussian Splat coding technologies that take advantage of software and hardware implementations of existing and future video coding standards jointly developed by ITU-T and ISO/IEC

ITU-T Study Group 21 (SG21) wishes to inform ISO/IEC JTC 1/SC 29 and its working groups of its recent discussions and developments regarding 3D Gaussian Splat coding technologies.

Significant industry interest has been voiced during our meetings in support of standardizing technologies for 3D Gaussian Splat coding, particularly taking advantage of software and hardware implementations of existing and future video coding standards jointly developed by ITU-T and ISO/IEC.

According to proponents, such industry interest can be effectively addressed using standardized supplemental enhancement information (SEI) messages, and several related contributions have been submitted to recent meetings of the ITU-T/ISO/IEC Joint Video Experts Team (JVET). This approach has been discussed within SG21, and no objections to this approach have been raised.

We suggest that an appropriate standard for such SEI messages is the Video Supplemental Enhancement Information (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), which provides a unified framework for SEI technologies applicable to the modern video coding standards jointly developed by ITU-T and ISO/IEC. We further note that VSEI is developed and maintained within JVET.

In light of the shared interests and potential for alignment, SG21 proposes a joint discussion between SG21 and SC 29 experts during the upcoming scheduled teleconference meetings of Q6/21 and MPEG in January 2026, to explore possible coordination and exchange of technical perspectives on this topic.

We appreciate the continued collaboration between our groups and look forward to further engagement.

Suggests VSEI as approach.

It was commented that SG21 might not be aware of the status in SC 29, but it was commented that the status in SC 29 was generally known to the SG21 participants.

Status in WG 4 & 7 – three

- Amendment of V-PCC [WG 7 targeted CDAM at this meeeting]

- Amendment of G-PCC [WG 7 targeted CDAM in April 2026]

- Exploration [targeting CfP, to include dynamic splatting]

PCC = point cloud compression. V = video-based (e.g. using VVC or HEVC). G = geometry-based.

Gaussian splats use attributes associated with points in a point cloud.

V3C is the framework used for V-PCC, MIV and V-DMC (mesh coding)

G-PCC has new profile with extended transform precision, modified quantization, and other enhancements.

V-PCC uses four video frames (or single frames with regions) that carry:

- Geometry

- Texture

- Occupancy

Atlas data syntax, describing patches.

Principal focus of V-PCC planned near-term amendment is static Gaussian splats, coded using I frames.

There is also study of dynamic splats, planned for future CfP.

Bit depth support was discussed; even 8-bit video approaches are considered.

JVET-AO152 was discussed.

This contribution argues that it is within JVET’s mandate to work on SEI messages for VSEI to interpret samples reconstructed from coded video pictures according to H.26[4/5/6] / AVC, HEVC, VVC and future video codec designs as Gaussian Splat parameters.  Technical, business, and venue-related considerations are included.  For this meeting, it is proposed to allow work, including proposal presentation and TuC-level decision making, in the JVET HLS breakout.

Use of a video approach was suggested by the proponent to leverage high-volume implementations of video standards.

VSEI was suggested by the proponent, due to its applicability to multiple video coding standards.

Basic architectural approaches as described for the lenslet handling could apply in principle, there could be different groups where the work could be done.

It was commented that having good ways to compare approaches (e.g. common test conditions) and avoiding duplications are desirable. It was pointed out that multiple approaches are already being pursued.

It was commented that MPEG WG 2 is also relevant and studying related matters.

Discarding of SEI messages has been practiced. It was commented that the “no frame packing” flag can help.

Conformance specification is also a matter of concern. A conformance specification for how to use the data may be needed.

We have already agreed that, in principle, there can be normative conformance requirements that involve an SEI message (or similar kinds of data, such as reserved ignored data or other NAL units).

The mapping between the 3D representation and the 2D representation was suggested to be critical. It was suggested that different encapsulation types could be specified in a harmonized way. A basic scheme could be the same aside from how the data is encapsulated.

Having joint investigation of the topic by MPEG WG 7 with JVET (as well as WG 4) was suggested, starting at the current meeting, including consideration of the 6 other noted contributions (which are not all technically distinct). Daily joint sessions of WGs 4 & 7 of 2 hours per day [1300 UTC Tuesday] had been planned already. (https://iso.zoom.us/my/mpeg3dg?pwd=eExPY1RWZVpXZys3emhobkp6QkdWUT09)

The encapsulation method was suggested to be lower immediate priority than the effectiveness of the basic scheme.

Dynamic splats were suggested to be very important.

Another participant emphasized a desire to develop something relatively quickly that can be implemented with relatively low resource requirements.

It was agreed to proceed with the joint meeting session as noted. Notes captured during that meeting can be found under 6.3.1.

### Joint session XXXX-XXXX XXday X Jan. on next generation video standardization CfP: MPEG WG 2 / Requirements, MPEG WG 5 / JVET, MPEG AG 5 Visual Quality Assessment, and VCEG (ITU-T Q6/21)

This joint session was chaired by Jens-Rainer Ohm (JVET Chair and WG 5 Convenor), Gary Sullivan (VCEG Rapporteur and SC 29 Chair), Mathias Wien (AG 5 Convenor), and Jörn Ostermann (AG 2 Convenor), on behalf of Igor Curcio (WG 2 Convenor).

Notes were taken by XXX.

## BoGs (2)

JVET-AO0287 BoG report on Complexity Reporting Template and Hardware Implementation [X. Li, Y. Zhao]

The BoG met from 11:30pm UTC 1/19 Mon. to 1:38am UTC 1/20 Tue.

- Mandates

- Further improvement of JVET-AN2040 – including discussion under JVET-AO0067, and JVET-AO0130

- Considering possibly missing aspects regarding hardware implementation, based on an initial review of JVET-AO0053, JVET-AO0060 and JVET-AO0137

- Recommendations

- Further study the development of hardware-encoding-mimicking platform on top of VTM based on the input from JVET-AO0053, e.g., using the 3-stage CTU pipeline as a starting point.

- Further study the hardware encoding configurations based on input from JVET-AO0137.

- Take attached Suggested-AO2040-d1 and Suggested-AO2040-annex-3.2 as the starting point for JVET-AO2040.

- Discussions

  - Agenda

- Review the three AHG19 documents and consider missing hardware aspects

- Refine JVET-AN2040

  - AHG19 Hardware implementation complexity

JVET-AO0053 [AHG19] Response to the questionnaire on VVC hardware encoder implementation challenges [M. Wu, J. Yu (HiSilicon), Y. Zhao, E. Alshina (Huawei)]

This contribution introduces an example of hardware encoder architecture and summarizes some major differences between hardware encoder and reference model. Besides, it provides answers to the AHG19 questionnaire on VVC hardware encoder implementation challenges.

The encoder architecture contains three CTU-based stages, one for pre-analysis, one for RDO loop, and one for loop filter and entropy coding.

In the RDO loop stage, there are parallel encoding decisions for the coding tree structure.

Inside the encoding processing of each CU, multiple RDO paths execute in parallel and each contains a series of sequential operations: Prediction, Transform, Quantization, Inverse TQ, Reconstruction, RDO cost calculation.

As to the response to AHG19 questionnaire:

The design is targeting at 4K@120fps VVC encoder for smartphones.

Difficulties for encoder implementation from some of VVC tools are due to:

- Coding gain issues

  - Some tools offer small coding gain, and hardware engineers must design fast algorithms from scratch

- RDO requirements

  - Tools with poor area vs. coding-gain trade-off may be excluded in encoder implementation

  - Examples: MRL, ISP, MMVD, GPM, CIIP, eMTS for inter blocks, SBT.

- Hardware complexity

  - Some tools significantly increase structural complexity and require large hardware area

  - Examples: MTT, 128×128 CTU

- Critical path impact

  - Examples: LFNST, DQ

- Data dependence

  - Cannot perform pre‑analysis outside the RDO loop

Parallel merge is useful for hardware encoder design, since it allows pre‑computing merge candidates for multiple CUs and provides engineers with greater flexibility to optimize RDO resource allocation.

Encoding algorithms in VTM that are difficult for hardware implementation: RDOQ and deblocking in RDO, as they increase RDO loop complexity.

Comments & Suggestions for new standard design:

- Develop coding tools that achieve good coding gain with less reliance on RDO

- Reduce and control dependencies between CU mode decisions

- Further optimize VVC coding tools for better trade-offs and hardware encoding friendliness

- Design in-loop filters that provide good coding gain without frame-level decisions

- Consider hardware design limitations when designing/proposing new coding tools

- Separately evaluate the complexity of RDO-loop vs. non-RDO-loop operations

- Test coding tools under realistic hardware-encoding configurations

  - e.g., two reference frames, GOP size ≤ 8, Constrained RDO complexity

Recommend JVET to consider the 3‑stage pipeline architecture as one of reference hardware encoder architectures during the development of the next-generation standard

Recommend AHG19 to consider developing hardware‑encoding-mimicking simulation framework similar to the 3-stage pipeline architecture on VTM

Some questions and comments during the Q/A:

- Why dual-tree for I slice is difficult for implementation? Due to using the same architecture for I and P/B slices.

- It is clarified by proponent that I and P/B slice RDO strategy may be different.

- It is clarified by proponent that around 25% coding gain over HEVC hardware encoder can be achieved with ~3x chip area.

- It is clarified by the proponent that DQ is more difficult than RDOQ for hardware encoder implementation.

- It is clarified by the proponent that the difficulty from luma-chroma dependency is architecture-dependent.

- It is clarified by the proponent that frame-level parallelism may be needed for 4K 120fps encoder implementation.

- It is commented by an expert that similar challenges to those elaborated in this proposal has also been observed.

- It is commented by several experts that developing hardware encoding mimicking simulation platform on top of test model is meaningful and they support this activity.

JVET-AO0060 AHG19: Introduction to hardware video encoder complexity [P. Wetzel (VITEC)]

For information – was presented in telco.

JVET-AO0137 [AHG19] Analysis of the HEVC encoder toolset in commercial hardware applications [M. Porto, D. Palomino, L. Agostini, G. Corrêa, B. Zatt, A. Schuch, V. Costa (ViTech - UFPel)]

This report summarizes the HEVC encoding tools available in some commercial hardware-based video codecs. It also presents a methodology for obtaining the toolset of a given HEVC hardware encoder.

Four smartphone and one GPU HEVC hardware encoders were tested, as follows:

- NVIDIA NVENC, embedded in NVIDIA RTX 4070 Ti GPU.

- Apple A15 Bionic, embedded in Apple iPhone 13 Smartphone.

- Apple A16 Bionic, embedded in Apple iPhone 14 Pro Smartphone.

- Qualcomm Snapdragon 8 Gen 2, embedded in Samsung Galaxy S23 Plus Smartphone.

- MediaTek Helio G95, embedded in Redmi Note 10S Smartphone.

Bitstream generated by the tested platforms are collected and analyzed to figure out detailed constraints of some tools, CTU size, GOP structure, etc..

| Tools and Parameters | NVENC (RTX 4070 Ti) | A15 Bionic (iPhone 13) | A16 Bionic (iPhone 14 Pro) | Snapdragon 8 Gen 2 (Galaxy S23 Plus) | Helio G95 (Redmi Note 10S) |
| --- | --- | --- | --- | --- | --- |
| CTU size | 32x32 | 32x32 | 32x32 | 32x32 | 16x16 (UHD Videos) 32x32 (FHD videos) |
| GOP size | 6 | 4 | 4 | 1 | 1 |
| GOP structure | P-B-B-B-B-B | P-B-B-B | P-B-B-B | P | P |
| Intra period | 250 | 60 (60fps videos) 32 (30 fps videos) | 60 (60fps videos) 32 (30 fps videos) | Same as video framerate | Same as video framerate |
| Reference frames | Up to 5 | Up to 2 | Up to 2 | 1 | 1 |
| Biprediction support | Yes | Yes | Yes | No | No |
| APM support (sizes) | Yes (8x32, 32x8, 24x32,32x24) | No | No | No | No |
| MV interval on X | [-1361, 1523] | [-316, 315] | [-316, 315] | [-189, 188] | [-77, 76] |
| MV interval on Y | [-512, 507] | [-188, 187] | [-188, 187] | [-77, 60] | [-45, 44] |
| FME support (quarter pixel) | Yes | Yes | Yes | Yes | Yes |
| Intra modes | All | Without odd directional modes for 4x4 chroma and 8x8 luma PBs | All | Without angular mode 34 on UHD4K@60fps | Without angular mode 34 |
| Intra PB limitations | No 4x4 PB support in UHD 4K@60fps | No 4x4 PB support in UHD 4K@60fps | No 4x4 PB support in UHD 4K@60fps | No 4x4 PB support in UHD 4K | Only 16x16 PBs |
| Inter PB limitations | No 4x8 and 8x4 PB support | No 4x8 and 8x4 PB support | No 4x8 and 8x4 PB support | No 4x8 and 8x4 PB support | Supports only 8x16, 16x8, 16x16 and 32x32 PBs |
| Luma TB sizes | 4x4 , 8x8, 16x16, 32x32 | 4x4 (absent for 4K@60fps), 8x8, 16x16, 32x32 | 4x4 (absent for 4K@60fps), 8x8, 16x16, 32x32 | 4x4, 8x8, 16x16, 32x32 | 8x8, 16x16 |
| Chroma TB sizes | 4x4 (3), 8x8 (2), 16x16 (1) | 4x4, 8x8, 16x16 | 4x4, 8x8, 16x16 | 4x4, 8x8, 16x16 | 4x4, 8x8, |
| DST available for 4x4 TUs | Yes | Yes (except for 4K@60fps videos) | Yes (except for 4K@60fps videos) | Available only for FHD videos | No |

Merge candidate number and enabling of parallel merge (e.g., by checking sps related syntax, e.g., sps_six_minus_max_num_merge_cand, sps_log2_parallel_merge_level_minus2) have not been tested. The bitstreams of this study are provided in Google Drive (link in the Annex A of this contribution), which allows other interested experts to check more details on tool configurations.

The proponents tried to use the best quality tool configuration of the encoders to their knowledge. One expert commented that the tested data generally match with his experience, but Qualcomm chip may support 2 reference frames according to his experience (maybe by using a different configuration).

Coding efficiency study on some of the encoders can be found in one VCIP2025 paper from the proponents.

  - Improvement of JVET-AN2040

JVET-AO0067 Module-Level Complexity Analysis of VVC [X. Li (Google), E. Alshina (Huawei), I. Moccagatta (Intel)]

In this contribution, a module-level complexity analysis of VVC is provided. The analysis is included in the attached Excel sheet.

Please refer to the attached Suggested-AO2040-annex-3.2 which is modified on top of the table of JVET-AO0067.

JVET-AN2040 Draft of complexity reporting template for coding algorithms and tools [X. Li, E. Alshina, I. Moccagatta]

Please refer to the attached Suggested-AO2040-d1 which is modified on top of JVET-AN2040.

Initial version presented Tuesday 20 Jan. 2100 UTC.

During presentation in JVET, it was commented that criteria that might be relevant for hardware complexity would be the necessity of complex RDO for effectiveness of certain tools, dependencies between mode decisions and necessity of frame-level decisions rather than local (CTU). All these also affect the degree of parallelism that can be used.

It was suggested that during standard development it might be useful to also investigate a complexity point that considers more hardware friendliness.

An update of the questionnaire JVET-AO2040 is included with the BoG report which should be reviewed later in the week. It does not to be in the final state at this meeting.

The BoG does not need another meeting.

JVET-AO0290 Report of BoG on CfP drafting [F. Bossen, M. Wien]

First review conducted in JVET Tuesday 20 Jan. 2145UTC.

Notes taken during that review are in italics font below.

The BoG on CfP drafting met on January 19, 2026 from 2100 until approximately 2310 UTC. This report includes notes from the meeting as well as recommendations. A draft CfP document based on JVET-AO0045 and that includes recommended changes is attached.

- Introduction

The BoG was task to develop a draft CfP document from JVET-AO0045

- Considering the suggestions from JVET-AO0130, JVET-AO0135, JVET-AO0184, JVET-AO0072

- Define additional rate points at the high end

- General editorial improvements

- Discussion

  - Related to JVET-AO0130

Issue: Date of final CfP publication. The meeting report from the Geneva meeting mentions a July 2026 date. The introduction of the draft CfP mentions April 2026. It is suggested to modify the introduction to mention a final draft CfP in April 2026 and a final CfP in July 2026. This is consistent with the decision from Geneva.

It should be clarified that the intent is to have the testing conditions frozen in April (final draft), and that the changes made in July (final CfP) are expected to be minor.

Issue: hidden dataset. Paragraph should be further refined. For example, approximate size of hidden dataset should be known at time of CfP issuance (along with constraints, e.g., only a subset of runtime constraints or bitrates). Need to clarify timeline and who will define the hidden dataset. Need to clarify what happens if decoder fails on hidden dataset.

It was agreed that clarification is needed. Possibilities are

- No visual testing to be conducted

- No precise rate matching

- Number of points for constrained complexity?

- Definition of number of sequences, duration of sequences (can be shorter), size of pictures (number HD, UHD, etc.), conditions RA/LD

- Timeline needs to be better defined

- Consequences of not delivering? Problems with only some sequences?

It would be sufficient for the draft to indicate that this will be better clarified in the final draft CfP

Issue: disclosure of training scripts for learning-based tools. A sentence was added in section 11 to bring attention to the fact that training scripts may have to be disclosed upon further evaluation after the CfP (similar to disclosure of source code). The sentence was made more general to include any tool. A definition of “learning-based tool” may be needed for the purpose of complexity reporting. Further discussion required, in particular with respect to a number of parameters threshold.

It was agreed that the formulation with “may” regarding the disclosure of training scripts is appropriate for indicating that this could happen during evaluation for standardization.

It was agreed not to require usage of certain training data. It is already written that it is mandatory to report which material was used for training. If a proposal is accepted as to be further investigated, part of that could be that it is re-trained with different material. Usage conditions can also be clarified later.

  - Related to JVET-AO0135

Issue: description of quantization parameter changes. BoG suggests adding clarification.

  - Related to JVET-AO0184

Reorganize HDR content into PQ and HLG classes instead of 4k and 8k cropped (as per meeting notes).

  - Related to JVET-AO0072

Added sentence is Annex D to require providing per frame bit counts for each test point.

  - Fifth target bitrate

It was previously agreed to add a fifth target bitrate above the highest target bitrate for the purpose of objective evaluation. It is suggested to have the fifth bitrate be equal to two times the fourth rate. Further refinements may be made if necessary (QP values at suggested rates should be checked).

A lower bound for rate matching may be needed for the fifth bitrate to guarantee sufficient overlap between the rate-distortion curves of the anchor and the test.

Further checks about appropriateness of the fifth bitrate to be done in the interim period.

  - Miscellaneous

Remove comments related to sequences (we have a copy in JVET-AO0045).

Clarified that not all test points may be evaluated subjectively.

Clarified desired usage of submitted decoder executable. It was asked whether decoder executable should be limited in size (because of potentially very large weight tables). Need to specify platform more precisely (e.g., CPU architecture).

It was agreed that the size limitation should not be imposed.

Agreed to add clarification that the reproduction of the sequences from the decoder output should be bit-exact.

Bits per frame to be reported, separate file, format to be specified

## Liaison communications (4)

m75762 Liaison Statement from ITU-T SG 21 to SC 29 on technologies for 3D Gaussian splat coding [SG21-LS126]

to be reviewed in joint meeting. No need to be responded by JVET.

m75822 Liaison statement from SMPTE to SC 29/WG 4 and 5 on Requesting SEI mechanism for new SMPTE ST 2094-60 standard

Relates to JVET-AO0112. W. Husak, D. Touzé, and A. Tourapis will work on a response, recommending to use T.35 approach that appears more flexible for them.

m75838 Liaison statement from SC 29/WG 1 to WG 5 on JPEG AI [SC 29/WG 1 N 101353]

from October meeting

m75840 Liaison statement from SC 29/WG 1 to WG 5 on JPEG AI [SC 29/WG 1 N 101424]

from January meeting

A liaison letter from JPEG (SC 29/WG 1) had been received as m74904 / WG 1 N 101272 / SC 29 N 22963 during the 40th meeting. It was originally planned to send a response, but that never happened. E. Alshina will draft a single liaison response for all three letters received by JPEG.

The liaison document WG 5 N XXX was reviewed in JVET on XXday XX July at XXXX – XXXX. The draft reply was also presented in the MPEG AG 3 Communication meeting Thursday 25 Jan. during XXXX – XXXX.

# Project planning

## Software timeline (update)

ECM 19.0 software (including all adoptions) was planned to be available 3 weeks after the meeting (3 Nov.).

The NNVC 15.0 codebase software was planned to be available 3 weeks after the meeting (3 Nov. 3), including all elements needed for CTC and EE1. Additional integration and harmonization with VTM may be deferred for version 15.1.

VTM23.13 software was recently released. Additional versions will be released as appropriate (e.g. for integration and updates of SEI messages included in JVET-AN1019 and JVET-AN2001).

Updates on top of HM18.0 and JM19.1 software will be released as appropriate (e.g., integration and updates of SEI messages included in JVET-AN1018 and JVET-AN1017).

As a general rule in software development, a person who is executing a merge shall not be from the same company as the person who submitted that merge request.

## Core experiment and exploration experiment planning

An EE on neural network-based video coding was established, as recorded in output document JVET-AO2023.

An EE on enhanced compression technology beyond VVC capability using techniques other than neural-network technology was also established, as recorded in output document JVET-AO2024.

Initial versions of these documents were presented and approved (see section 10).

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM/VTM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- Normative contributions (relating to changes in bitstream/decoder) shall include draft specification text

- Proposals shall contain all details relevant for understanding and be self-contained. In cases where the document is a follow-up of a previous contribution, the overall concept and the novelties should be highlighted at minimum

- Coding tool and encoder optimization proposals shall contain Excel sheets that allow assessment on a per-sequence basis

- Algorithm description text is strongly encouraged for non-normative contributions that are intended to be included in model description documents (VTM, ECM, etc.), and that is required for inclusion in TR drafts.

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions (not exercised currently)

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized that those rules which had been set up or refined during the 12th JVET meeting should be observed. In particular, for some CEs of some previous meetings, results were available late, and some changes in the experimental setup had not been sufficiently discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of a draft standard by the next meeting or in the near future.

- “Exploration experiments” (EEs) are also coordinated experiments. These are conducted on technology which is not foreseen to become part of a draft standard in the near future. The investigating methodology for assessment of such technology can also be an important part of an EE. (Further general rules for EEs, as far as deviating from the CE rules below, should be discussed in a future meeting. For the current meeting, procedures as described in the EE description document are deemed to be sufficient.)

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials as described below).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology. Withdrawing parts of experiments that were intended to show the individual benefits of a tool or parts of a tool is strongly discouraged. Combination tests may not be considered in such cases. Any changes made to individual tools in a combination shall be documented.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). In cases where combinations of different elements are planned to be tested, mutual cross-checking of the individual elements by other parties of the combination is discouraged. The combination must be cross-checked by an independent party. The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

- It is mandatory to report encoder optimizations made for the benefit of a tool, and if an equivalent optimization could be applied on the anchor, a comparison against the improved anchor shall be provided.

- A new proposal can be included in a CE based on group decision, regardless if an independent party has already performed a cross-check in the meeting when it was first proposed.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments for SDR video are described in the prior output document JVET-AL2010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g., not referring to methods as “improved”, “optimized”, “enhanced”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

The CE development workflow was previously described at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/Core-experiment-development-workflow

However, it was noted that the link doesn’t seem to exist anymore.

CE read access is available using shared accounts: One account exists for MPEG members, which uses the usual MPEG account data. A second account exists for VCEG members with account information available in the TIES informal ftp area (IFA) system at:

https://www.itu.int/ifa/t/2017/sg16/exchange/wp3/q06/vceg_account.txt

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation was requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to be marked as “withdrawn”.

T2 = Test model software release + 2 weeks: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3.

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline minus 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular contribution deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, JVET would like to see comprehensive cross-checking done, with analysis of whether the description matches the software, and a recommendation of the value of the tool and given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology into a standard or test model.

Availability of specification text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the specification will be. There must also be sufficient time to study this in detail. CE contributions without sufficiently mature draft specification text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

# Establishment of ad hoc groups (update)

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

Chairs of AHGs had been asked to send draft mandates to JRO before XXXX on X Jan., preferably copy from the table below and sending with changemarks or yellow highlight of changes.

Review of AHG plans was conducted during the plenary on XXday X Jan. 2026 at XXXX–XXXX.

| Title and Email Reflector | Chairs | Interim mtg. |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG and experiment studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. - Supervise processing and delivery of output documents. - Conduct consultation with parent bodies on future JVET management structures. | J.-R. Ohm (chair), G. J. Sullivan (vice-chair) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize draft text outputs of the meeting (JVET-AN1016, JVET-AN1017, JVET-AN1018, JVET-AN1019, and JVET-AN2001). - Collect reports of errata for VVC, VSEI, HEVC, AVC, CICP, and the published related technical reports. - Coordinate with AHG3 to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), F. Bossen, A. Browne, S. Kim, S. Liu, J.-R. Ohm, G. J. Sullivan, A. Tourapis, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test models (VTM, HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, 360Lib, and HDRTools) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Enable software support for recently standardized additional SEI messages (for both VTM and HM), and SEI messages in TuC (the latter in a separate branch of VTM). - Discuss and make recommendations on the software development process. - Perform comparative tests of test model behaviour using common test conditions, including HDR, high bit depth and high bit rate. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Prepare drafts of merged and updated CTC documents for HM and VTM, as applicable. | F. Bossen, X. Li, K. Sühring (co-chairs), E. François, Y. He, K. Sharman, V. Seregin, A. Tourapis (vice-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Consider plans for additional verification testing of VVC capability, particularly target conducting tests for VVC multi-layer features, and update the test plan accordingly. - Maintain the video sequence test material database for testing the VVC and HEVC standards and potential future extensions, as well as exploration activities. - Study coding performance and characteristics of available and proposed video test material. - Identify and recommend appropriate test material for testing the VVC standard and potential future extensions, as well as exploration activities. - Identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material, in coordination with other AHGs, as appropriate. - Maintain and update the directory structure for the test sequence repository, as necessary. - Collect information about test sequences that have been made available by other organizations. - Prepare and conduct expert viewing for purposes of subjective quality evaluation. - Coordinate with AG 5 in studying and developing further methods of subjective quality evaluation, e.g. based on crowd sourcing, as well as studying objective metrics in that context. - Coordinate with AHG17 on investigating sequences and making arrangements for viewing at the next meeting. - Coordinate with AHG18 on investigating visual impact of data losses. - Prepare availability of viewing equipment and facilities arrangements for future meetings. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), W. Husak, S. Iwamura, P. de Lagrange, S. Liu, X. Meng, S. Puri, A. Segall, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Study DIS of VVC conformance 3rd edition and suggest improvements to JVET-AL2028, as appropriate. - Study the draft conformance bitstreams for new HEVC multiview profiles in JVET-AM1008, and further develop related conformance bitstreams. - Coordinate with AHG3 on implementation of the new HEVC multiview profiles. - Study the requirements of VVC, HEVC, and AVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database, and contribute to report problems, and suggest actions to resolve these. - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | I. Moccagatta (chair), F. Bossen, T. Ikai, S. Iwamura, H.-J. Jhu, K. Kawamura, P. de Lagrange, S. Paluri, K. Sühring, Y. Yu (vice-chairs) | N |
| ECM software development (AHG6) (jvet@lists.rwth-aachen.de) - Coordinate development of the ECM software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Prepare and deliver ECM-19.0 software version (and potential updates), corresponding VTM anchor, and the reference configuration encodings according to the ECM common test conditions. - Investigate encoder speedup and other software optimization such as reduction of memory consumption. - Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate. | V. Seregin (chair), J. Chen, R. Chernyak, F. Le Léannec, K. Zhang (vice-chairs) | N |
| Tool assessment (AHG7) (jvet@lists.rwth-aachen.de) - Investigate methodology of tool assessment, such as the aspects of memory access and bandwidth, number of maximum processing cycles, block decoding dependencies, number of context coded bins, pipeline and parallelization. - Study JVET-AN2040 and suggest improvements. - Coordinate with AHG6 on resolving tool-off test related software issues (missing tool controls and software bugs). - Prepare configuration files and generate bitstreams and results of tool-on/tool-off testing. - Prepare reporting of tool assessment results. - Develop methodology of more reliable runtime measurement. - Collaborate with AHG19 on encoding complexity assessment. | X. Li (chair), L.-F. Chen, Z. Deng, J. Gan, E. François, R. Ishimoto, H.-J. Jhu, J. Lainema, X. Li, J. Pardo, A. Stein, H. Wang (vice-chairs) | Y (tel., 2 weeks notice) |
| Optimization of encoders and receiving systems for machine analysis of coded video content (AHG8) (jvet@lists.rwth-aachen.de) - Solicit and study non-normative encoder and receiving systems technologies that enhance performance of machine analysis tasks on coded video content. - Identify and collect test materials that are suitable to be used by JVET for machine analysis tasks. - Discuss improvements on the evaluation framework, including evaluation procedures and methodologies. - Coordinate software development and experiments on optimization of encoders and receiving systems for machine analysis of coded video content, including combinations of proposed technologies. - Maintain the software implementation examples and develop tool combination examples in the repository, including sufficient documentation in terms of operation and performance. - Evaluate proposed technologies and their suitability for machine analysis applications. - Propose potential improvements to JVET-AL2030 on optimization of encoders and receiving systems for machine analysis of coded video content. - Study the potential of using SEI messages for the purpose of machine analysis in coordination with AHG9. - Investigate the impact of using different machine task models in the evaluation of the compression performance of tools optimized for machine analysis tasks. | S. Liu, J. Ström, S. Wang, M. Zhou (AHG chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VSEI, VVC, HEVC and AVC. - Study JVET-AN1017, identify any issues and propose solutions as appropriate. - Study JVET-AN2032, including study of SEI messages with different options, when those are present, and propose improvements. - Collect software and showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages. - Study SEI messages specified in HEVC and AVC for potential use in the VVC context and SEI messages in VSEI for potential use in the HEVC or AVC context. - Study the alignment of the same SEI messages in different standards. - Coordinate with AHG3 for software support of SEI messages for JM, HM, and VTM. | J. Boyce, Y.-K. Wang (co-chairs), T. Chujoh, S. Deshpande, M. M. Hannuksela, Y. He, P. de Lagrange, G. J. Sullivan, H. Tan, A. Tourapis, S. Wenger, P. Wu (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as tool adaptation and configuration, and perceptually optimized adaptive quantization for encoder optimization. - Study the impact of non-normative techniques of preprocessing for the benefit of encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study optimized encoding for reference picture resampling and scalability modes in VTM, and coordinate with AHG4 on improving encoders and test settings for multi-layer verification testing. - Study optimized encoding and suitable test settings for noisy materials, such as sequences containing film grain. - Study optimized encoding and tool combinations for low latency and for low complexity. - Consider neural network-based encoding optimization technologies for video coding standards. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. - Study the potential of defining default or alternate software configuration settings and test conditions optimized for either subjective quality, higher objective quality, or encoding with improved complexity/performance tradeoff, and coordinate such efforts with AHG3, AHG6, and AHG17. - Study the effect of varying configuration parameters depending on temporal layer, such as those related to deblocking, partitioning, chroma QP. | K. Andersson, P. de Lagrange, A. Duenas (co-chairs), T. Ikai, T. Solovyev, A. Tourapis (vice chairs) | N |
| Neural network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Evaluate and quantify the performance improvement potential of NN-based video coding technologies compared to existing video coding standards such as VVC, including both individual coding tools, architectures and content adaptation with NN parameters overfitting. - Study potential improvements of the NNVC CTC document JVET-AJ2016. - Study the impact of training (including the impact of loss functions) on the performance of candidate technologies and identify suitable material for testing and training. Promote the call for training materials, distribute it, and actively communicate with content owners. - Analyse complexity characteristics for technologies under study, including transformers, perform complexity analysis, and develop complexity reductions of candidate technology. - Discuss and propose improved metrics to perform complexity analysis of NN architectures, in particular also investigate bit-exact reproducibility of NN-based methods on various platforms in coordination with AhG14. - Finalize and discuss the EE on neural network-based video coding. - Coordinate with other groups, including SC 29/AG 5 on the evaluation and assessment of visual quality, and AHG12 on the interaction with ECM coding tools. - Coordinate with AHG14 on items related to NNVC software development, in particular, evaluate the interface in the NNVC for end-to-end optimized AI coded reference pictures. | E. Alshina, F. Galpin, S. Liu (co-chairs), J. Li, Y. Li, R.-L. Liao, M. Santamaria, T. Shao, M. Wien, P. Wu (vice chairs) | Y (tel., 2 weeks notice), first on Nov. 18, second on December 16 |
| Enhanced compression beyond VVC capability (AHG12) (jvet@lists.rwth-aachen.de) - Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC. - Discuss and propose refinements to the ECM19 algorithm description JVET-AN2025. - Coordinate with AHG7 to study the performance and complexity tradeoff of these video coding tools. - Coordinate with AHG6 on ECM software development. - Establish a repository to make other codebases that implement enhanced compression available to interested JVET members. - Support AHG6 in generating anchors according to the test conditions in JVET-AI2017. - Analyse the results of exploration experiments described in JVET-AN2024 in coordination with the EE coordinators. - Coordinate with AHG11 to study the interaction with neural network-based coding tools. | M. Karczewicz, Y. Ye, L. Zhang (co-chairs), B. Bross, R. Chernyak, X. Li, K. Naser, Y. Yu (vice-chairs) | N |
| Film grain technologies (AHG13) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies. - Discuss and propose refinements to the draft of the TR 2nd ed. JVET-AN2020. (study inclusion of software?) - Study alternative film grain models and their associated documentation. - Discuss and enumerate updates, improvements, and additions for the second edition of the technical report. - In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies, and perform an assessment of newly available test materials in that regard. - Investigate metrics for measuring film grain fidelity in itself, or as present in a video. - Discuss the potential need for film grain conformance guidelines. - Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies. - Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements. - Identify potential need for additional film grain technology and signalling, if needed. - Coordinate development of film grain technology software and configuration files. - Coordinate with AG 5 on improving the plan for subjective quality testing of the FGC SEI message JVET-AJ2022, and conduct preparations for such testing. - Coordinate with AHG3 for software support of the FGC SEI message. | W. Husak, P. de Lagrange (co-chairs), A. Duenas, X. Meng, M. Radosavljević, A. Segall, G. Teniou, A. Tourapis (vice-chairs) | Y (tel., 2 weeks notice) |
| NNVC software development (AHG14) (jvet@lists.rwth-aachen.de) - Coordinate development of the NNVC software and associated configuration files. - Prepare and deliver NNVC-15.0 software version (and potential updates), based on updated VTM with adopted contributions and hybrid framework, and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AJ2016. Study the impact of the addition of new dataset on the already integrated models. - Continue to bridge the gap between NNVC and most recent VTM as necessary. - Continue to develop missing functionalities for hybrid end-to-end framework exploration. - Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) and Very Low Operation Point (VLOP) configurations. - Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed. - Coordinate with NNVC algorithm and software description (JVET-AN2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate. | F. Galpin (chair), R. Chang, A. Karabutov, Yue Li, Yun Li, M. Santamaria, J. N. Shingala, Z. Xie (vice chairs) | Y (tel., 2 weeks notice), first on Nov. 18, second on Dec. 16 |
| Gaming content compression (AHG15) (jvet@lists.rwth-aachen.de) - Identify gaming content application scenarios and their requirements for codec operation. - Identify and characterize required types of content; solicit contributions, collect, and make a variety of gaming content available, in coordination with AHG4 and AG 5. - Produce VTM and ECM anchor encodings according to CTC JVET-AJ2027, and provide test results at the next meeting. - Develop and maintain interfaces for supporting use cases of camera parameters and depth maps in gaming applications, including mechanisms for efficient transporting these elements in the coded video bitstream. - Develop and maintain software for estimation of depth maps for gaming sequences, and evaluate the effect of the estimated maps against the original depth maps on coding efficiency where they are available using established accuracy metrics - Evaluate JVET test models (such as ECM, VTM, NNVC, etc.) under the proposed test conditions. - Investigate possibilities to enhance compression capability for gaming content. - Investigate the possibility to estimate depth maps and camera parameters for those gaming sequences where they are not available. - Study conversion of depth maps using integer representation, and identifying efficient bit-depth resolution of depth maps to support identified use-cases that will be an input to compression. - Solicit contributions from industry on typical bitrate/quality/resolution used for gaming content compression. | S. Puri, J. Sauer (co-chairs), R. Chernyak, A. Duenas, L. Wang, V. Zakharchenko (vice chairs) | N |
| Generative face video compression (AHG16) (Discontinue) (jvet@lists.rwth-aachen.de) - Maintain GFVC software tools, associated configuration files, and software usage documentation. - Study the extension of GFVC software capability to handle video content with higher resolutions. - Identify and study additional test content, including content with higher resolutions, suitable for use in GFVC performance evaluation. - Study GFVC performance under test conditions defined in JVET-AJ2035, as well as performance on additional test content, and wider bitrate ranges. | Y. Ye (chair), H.-B. Teo, Z. Lyu, S. McCarthy, S. Wang (vice chairs) | N |
| Preparation of Call for Proposals (AHG17) (jvet@lists.rwth-aachen.de) - Review the set of categories, test sequences and rate points based on the results of the CfE, and propose updates for the test set to be used in the CfP. - Identify test sequences extending the represented range of applications and investigate the suitability for use in visual quality assessment. - Develop proposed text for a draft CfP document, considering JVET-H1002 and JVET-AN2026. - Coordinate the generation of VTM anchors and other bitstreams for the test cases defined in the updated test set, and make them available according to the timeline. - Prepare logistics and plans for tentatively conducting an AHG meeting after the 41st JVET meeting, and issue an announcement JVET-AN2041. | J.-R. Ohm, M. Wien, F. Bossen (co-chairs), E. Alshina, V. Baroncini, J. Chen, R. Chernyak, Z. Deng, P. de Lagrange, C. Lehmann, L. Li, P. Nikitin, D. Rusanovskyy, G. Verba (vice chairs) | Y (tel., 2 weeks notice) |
| Ultra-low latency and packet loss resilience (AHG18) (jvet@lists.rwth-aachen.de) - Study JVET-AN2039 common test conditions and propose improvements as appropriate. - Investigate creation of practical simulation software based on VTM, including network transmission aspects, including consideration of network functionality that can aid timely video-coding based error correction, and conduct performance evaluation. - Identify potential requirements and feasibility of standard based technologies to support ultra-low delay requirements, including packet loss resilient decoding. - Investigate packet loss resilient technologies beyond VVC supporting ultra-low delay coding for interactive and live broadcasting scenarios. - Coordinate with AHG4 on investigating visual impact of data losses. | S. Deshpande, S. Ikonin, V. Zakharchenko (co-chairs), S. Fößel, C. Kim, X. Ma, S. Puri, J. Ström, S. Wenger (vice-chairs) | Y (tel., 2 weeks notice) |
| Hardware implementation complexity (AHG19) (jvet@lists.rwth-aachen.de) - Investigate hardware encoding complexity constraints in typical applications (e.g., mobile devices and hardware transcoding), and identify challenges and evaluate coding tools from hardware encoding implementation perspectives. - Solicit and develop hardware encoding complexity measurements (e.g., maximum number of RDO decisions per CU / CTU), and use them to analyze the performance of test models (e.g., VTM) and coding tools. - Consider the possibility of designing, developing and maintaining a hardware-encoding-mimicking simulation framework on top of test models, e.g., constraining RDO number per CU, constraining coding tree decision complexity (e.g., maximum tree depth, per-node split modes), developing rate estimation function interface, low-complexity CU/TU-level bit estimation function, etc. - Collaborate with AHG7 on aspects of hardware encoding and decoding complexity perspectives, and in improving JVET-AN2040. | Y. Zhao, J. Park, I. Moccagatta (co-chairs), H. Huang, T. Ikai, X. Li, K. Naser, N. Song, G. Verba (vice chairs) | Y (tel., 2 weeks notice) |

It was suggested to discontinue AHG8 by the time of finalization of the TR (likely April 2026), and discontinue AHG16 by the next meeting. For both AHGs, it is necessary to find a persistent home for the software.

It was confirmed that the rules which can be found in document ISO/IEC JTC 1/‌SC 29/‌AG 2 N 046 “Ad hoc group rules for MPEG AGs and WGs” (available at https://www.mpegstandards.org/adhoc/), are consistent with the operation mode of JVET AHGs. It is pointed out that JVET does not maintain separate AHG reflectors, such that any JVET member is implicitly a member of any AHG. This shall be mentioned in the related WG Recommendations. The list above was also issued as a separate WG 5 document (ISO/IEC JTC 1/‌SC 29/‌WG 5 N XXX) in order to make it easy to reference.

# Output documents (update)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as a WG 5 output document, a separate version under the WG 5 document header should be generated. This version should be sent to GJS and JRO for upload.

The list of JVET ad hoc groups was also issued as a WG 5 output document WG 5 N XXX, as noted in section 9.

List of JVET-only outputs as separate WG 5 document, or as a list/table in the WG 5 recommendations.

JVET-AO1000 Meeting Report of the 41st JVET Meeting [J.-R. Ohm] [WG 5 N XXX] (2026-02-20)

Initial versions of the meeting notes (d0 … dX) were made available on a daily basis during the meeting.

Remains valid – not updated: JVET-AC1001 Guidelines for HM-based software development [K. Sühring, F. Bossen, X. Li (software coordinators)]

Remains valid – not updated: JVET-Y1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 16 [C. Rosewarne, K. Sharman, R. Sjöberg, G. J. Sullivan] [WG 5 N 103]

Primary editor: C. Rosewarne

Remains valid – not updated: JVET-AH1003 Coding-independent code points for video signal type identification (Draft 3) [G. J. Sullivan, A. Tourapis]

Primary editor: G. J. Sullivan.

JVET-AO1004 Errata report items for VVC, VSEI, HEVC, AVC, and Video CICP [Y.-K. Wang, B. Bross, I. Moccagatta, C. Rosewarne, G. J. Sullivan]

Primary editor: Y.-K. Wang.

Some issues with latest editions of VSEI/VVC/HEVC

JVET-AN1005 Future CICP extensions (Draft 2) [J. Boyce, E. Thomas, A. Tourapis] (2025-10-26)

Primary editor: J. Boyce.

Including update from …

Remains valid – not updated JVET-AM1006 HEVC additional profiles and SEI messages (Draft 4) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-08-01)

Primary editor: Y.-K. Wang.

A DoC WG 5 N 375 on the DAM was reviewed and approved on Saturday 11 Oct. at 1125-1140.

It was requested in the WG 5 recommendations to integrate the amendment into a new edition, and an integrated version was issued as WG 5 N 376 FDIS ISO/IEC 23008-2 HEVC 7th ed. (with editing period 2026-01-09). The latter is based on JVET-AM1006 plus the additional modifications documented under JVET-AN1018.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye] [WG 11 N 15778]

Remains valid – not updated: JVET-AM1008 Conformance testing for HEVC multiview extended and monochrome profiles [I. Moccagatta, T. Fu, S. Paluri, A. Tourapis]

New version depending on review of JVET-AO0227?

Remains valid – not updated: JVET-AC1009 Common test conditions for SHVC [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan]

Remains valid – not updated: JVET-AJ1011 White paper on HEVC [B. Bross, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang] [AG 3 N 174]

Remains valid – not updated: JVET-AJ1012 Overview of IT systems used in JVET [J.-R. Ohm, I. Moccagatta, K. Sühring, M. Wien]

Update of bug tracking system description expected in next meeting, and update on zoom meeting rules (no recording etc.).

Remains valid – not updated: JCT3V-G1003 3D-AVC Test Model 9 [D. Rusanovskyy, F. C. Chen, L. Zhang, T. Suzuki] [WG 11 N 14239]

Remains valid – not updated: JCT3V-K1003 Test Model 11 of 3D-HEVC and MV-HEVC [Y. Chen, G. Tech, K. Wegner, S. Yea] [WG 11 N 15141]

Remains valid – not updated: JVET-AE1013 Common test conditions of 3DV experiments [K. Sühring, M. Wien]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye] [WG 11 N 16049]

Remains valid – not updated: JVET-AC1015 Common test conditions for SCM-based screen content coding [K. Sühring]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AN1016 AVC with extensions and corrections (Draft 4) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-XX-XX)

Primary editor: K. Sühring.

To prepare the next integrated edition of H.264, including changes from JVET-AN1017.

JVET-AO1017 Support for additional SEI messages in AVC (Draft 5) [K. Sühring, J. Boyce, G. J. Sullivan, Y.-K. Wang] (2026-XX-XX)

Primary editor: K. Sühring

A preliminary DoC WG 5 N XXX on the DAM was reviewed and approved on XXday X Jan. at XXXX-XXXX.

Changes are editorial improvements, interface changes, constraints, and bug fixes agreed at this meeting, including elements from:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

Remains valid – not updated: JVET-AN1018 HEVC with extensions and corrections (Draft 3) [Y.-K. Wang, B. Bross, S. Deshpande, G. J. Sullivan, A. Tourapis] (2025-10-17)

Primary editor: Y.-K. Wang.

This was submitted to ITU consent on H.265 11th ed. It is noted that this may not be identical with the final version published by ITU-T.

Remains valid – not updated: JVET-AN1019 VSEI with extensions and corrections (Draft 1) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-10-17)

Primary editor: J. Boyce.

This was submitted to ITU consent on H.274 4th ed. It is noted that this may not be identical with the final version published by ITU-T.

No output: JVET-Axx1020 through JVET-Axx1099

Remains valid – not updated JVET-AA1100 Common Test Conditions for HM Video Coding Experiments [K. Sühring, K. Sharman]

This specifies only the CTC for non-4:2:0 colour formats. The corresponding document for VVC is JVET-T2013, with no unification yet.

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AN2001 VVC with extensions and corrections (Draft 1) [Y.-K. Wang, B. Bross, M. M. Hannuksela, G. J. Sullivan] (2025-10-17)

Primary editor: Y.-K. Wang.

This was submitted to ITU consent on H.266 4th ed. It is noted that this may not be identical with the final version published by ITU-T

JVET-AO2002 Algorithm description for Versatile Video Coding and Test Model 23 (VTM 23) [Y. Ye, A. Browne, S. Kim] [WG 5 N 284]

Primary editor: Y. Ye.

New elements from notes elsewhere in this report (kept for future use):

- RPR clarification (JVET-AO0136)

- MCTF (JVET-AO0136 and JVET-AO0288)

- BIM (JVET-AO0288)

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

It was suggested that editorial improvements submitted as input to the next meeting would be welcome.

Remains valid – not updated: JVET-AJ2003 Guidelines for VTM-based software development [F. Bossen, X. Li, K. Sühring]

Remains valid – not updated: JVET-T2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 12) [Y. Ye, J. Boyce]

Remains valid – not updated: JVET-AM2005 Additions and corrections for VVC version 4 (Draft 13) [G. J. Sullivan, B. Bross, M. M. Hannuksela, Y.-K. Wang] (2025-08-01)

Primary editor: G. J. Sullivan.

A DoC WG 5 N 377 on the DAM was reviewed and approved on Saturday 11 Oct. at 1140-1155.

It was requested in the WG 5 recommendations to integrate the amendment into a new edition, and an integrated version was issued as WG 5 N 378 FDIS ISO/IEC 23090-3 VVC 4th ed. (with editing period 2026-01-09). The latter is based on JVET-AM2005 plus the additional modifications documented under JVET-AN2001.

Remains valid – not updated: JVET-AM2006 Additional SEI messages for VSEI version 4 (Draft 7) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, S. McCarthy, G. J. Sullivan, H. Tan, Y.-K. Wang] (2025-08-01)

Primary editor: J. Boyce.

A DoC WG 5 N 373 on the DAM was reviewed and approved on Saturday 11 Oct. at 1210-1245 (with some finalization in HLS track thereafter, which included a name change of AI usage restrictions later agreed by JVET, and otherwise purely editorial issues).

It was commented that for referencing some external specs in DSC, it will be necessary to fill form A.5 for justification in the ITU-T submission.

It was requested in the WG 5 recommendations to integrate the amendment into a new edition, and an integrated version was issued as WG 5 N 374 FDIS ISO/IEC 23002-7 VSEI 4th ed. (with editing period 2025-09-01). The latter is based on JVET-AM2006 plus the additional modifications documented under JVET-AN1019.

Remains valid – not updated: JVET-AJ2007 Guidelines for NNVC software development [F. Galpin, S. Eadie, L. Wang, Z. Xie, Y. Li]

Might benefit from an update in the future, regarding some details of SADL implementation.

Remains valid – not updated: JVET-AN2008 Conformance testing for versatile video coding (Draft 8) [I. Moccagatta, F. Bossen, S. Iwamura, P. de Lagrange]

Primary editor: I. Moccagatta

This was submitted to ITU consent on H.266.1 3rd ed.

An equivalent version was under ballot as ISO/IEC DIS 23090-15 3rd ed. A DoC WG 5 N XXX on the DIS was reviewed and approved on XXday X Jan. at XXXX-XXXX. The DIS text will be sent for publication as final standard.

Remains valid – not updated: JVET-AJ2009 Reference software for versatile video coding 2nd edition (Draft 2) [F. Bossen, K. Sühring, X. Li] [WG 5 DIS N 322)]

Software relating to H.266.2 and ISO/IEC 23090-16 can be found at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/2nd-edition.

Primary editor: F. Bossen.

A request for next edition(s) of reference software(s) could be issued in the next meeting, and also CD(s) could be produced by the same meeting, as SC 29 is meeting beginning of February and could approve such projects without issuing a ballot. Issuing CD(s) by the January meeting (if mature enough) could avoid that an ISO version would lag behind an ITU version which might be submitted for consent in July 2026. Another consideration could be to start the process for ISO early enough in 2026, such that FDIS and ITU consent could be synchronized in April 2027.

Remains valid – not updated: JVET-AL2010 VTM and HM common test conditions and software reference configurations for SDR 4:2:0 10 bit video [F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring]

JVET-AO2011 VTM and HM common test conditions and evaluation procedures for HDR/WCG video [E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

Links to test sequences need to be updated due to the change of the content server.

New version including removal of metrics, and clarification for metrics to be used in HLG and PQ, as suggested in JVET-AO0184

Remains valid – not updated: JVET-U2012 JVET common test conditions and evaluation procedures for 360° video [Y. He, J. Boyce, K. Choi, J.-L. Lin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-T2013 VTM common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin]

Links to test sequences need to be updated due to the change of the content server.

Remains valid – not updated: JVET-AJ2016 Common test conditions and evaluation procedures for neural network-based video coding technology [E. Alshina, F. Galpin, R.-L. Liao, S. Liu, A. Segall]

Remains valid – not updated: JVET-AI2017 Common test conditions and evaluation procedures for enhanced compression tool testing [M. Karczewicz, Y. Ye]

It was commented that it might be good to add conditions relating to HDR, even though currently no HDR tests are conducted in CTC.

Remains valid – not updated: JVET-AA2018 Common test conditions for high bit depth and high bit rate video coding [A. Browne, T. Ikai, D. Rusanovskyy, X. Xiu, Y. Yu]

Links to test sequences need to be updated due to the change of the content server.

JVET-AO2019 Description of algorithms version 14 and software version 16 in neural network-based video coding (NNVC) [F. Galpin, Yue Li, Yun Li, D. Rusanovskyy, T. Shao, J. Ström, L. Wang] (2026-XX-XX)

New elements in text and software from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AN2020 Film grain synthesis technology for video applications ed. 2 (Draft 4) [W. Husak, P. de Lagrange, A. Norkin, M. Radosavljević, A. Tourapis] (2026-01-09)

Update of analysis algorithm from JVET-AN0237.

JVET-AO2021 Verification test plan for VVC multilayer coding (update 8) [O. Chubach, P. de Lagrange, M. Wien] (2026-XX-XX)

New version based on JVET-AO0215

Remains valid – not updated: JVET-AJ2022 Plan for subjective quality testing of the FGC SEI message (update 4) [P. de Lagrange, W. Husak, M. Radosavljević, M. Wien]

Number can be re-used (activity discontinued).

JVET-AO2023 Exploration experiment on neural network-based video coding (EE1) [E. Alshina, R. Chang, F. Galpin, Yue Li, Yun Li, M. Santamaria, T. Shao, J. Ström, Z. Xie (EE coordinators)] (2026-XX-XX)

An initial draft of this document was reviewed and approved at XXXX-XXXX on XXday X January.

This round of EE1 tests includes:

…

JVET-AO2024 Exploration experiment on enhanced compression beyond VVC capability (EE2) [V. Seregin, D. Buğdayci Sansli, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang (EE coordinators)] (2026-XX-XX)

An initial draft of this document was reviewed and approved at XXXX-XXXX on XXday X January.

This round of EE2 tests includes:

…

JVET-AN2025 Algorithm description of Enhanced Compression Model 19 (ECM 19) [M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang] (2026-01-09)

New version?

New elements from notes elsewhere in this report:

- …

It is noted that the list above may not be complete; if some adoption is missing that is recorded somewhere else in the meeting notes it shall also be considered included.

JVET-AO2026 Draft of Joint Call for Proposals on video compression with capability beyond VVC [J.-R. Ohm, M. Wien, F. Bossen] (2026-XX-XX)

JVET-AJ2027 Common test conditions for gaming applications [J. Sauer, R. Chernyak, S. Puri, S. Thiebaud]

Add Wukong2 and Wukong3 to class G2 (see discussion under JVET-AO0207)

Remains valid – not updated: JVET-AL2028 Additions and corrections for VVC conformance (Draft 1) [S. Iwamura, P. de Lagrange, I. Moccagatta]

Remains valid – not updated: JVET-AN2029 Results of the Joint Call for Evidence on video compression with capability beyond VVC [F. Bossen, J.-R. Ohm, M. Wien] [AG 5 N 180] (2025-12-19)

JVET-AO2030 Optimization of encoders and receiving systems for machine analysis of coded video content (Draft 10) [S. Liu, J. Chen, C. Hollmann, J. Ström] [WG 5 2nd DTR N XXX]

Primary editor: S. Liu.

A DoC WG 5 N XXX on the previous DTR was reviewed and approved on XXday X Jan. at XXXX-XXXX.

Revisit: Proposed text as input document?

Remains valid – not updated: JVET-AI2031 Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content [S. Liu, C. Hollmann]

JVET-AO2032 Technologies under consideration for future extensions of VSEI (version 10) [J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, H. Tan, J. Xu] (2026-XX-XX)

Primary editor?

New elements from notes elsewhere in this report (this includes new features, editorial improvements, additions of constraints, clarifications, and bug fixes of existing SEI messages in TuC; addition of floating point descriptor types in two cases; extensions to existing SEI messages in VSEI v4):

…

Remains valid – not updated: JVET-AF2033 Report of verification test on VVC multi-layer coding: Content layering [S. Iwamura, P. de Lagrange, M. Wien] [AG 5 N 105)]

Remains valid – not updated: JVET-AI2034 Call for new HDR materials for future video coding development [E. François, W. Husak, S. Iwamura, D. Rusanovskyy, A. Segall, M. Wien] [WG 5 N 312)]

Remains valid – not updated: JVET-AJ2035 Test conditions and evaluation procedures for generative face video coding [S. McCarthy, B. Chen]

Remains valid – not updated: JVET-AG2036 Call for training materials for neural network-based video coding tool development [E. Alshina, F. Galpin, S. Liu, M. Wien] [WG 5 N 266)]

Remains valid – not updated: JVET-AJ2037 Report on subjective quality testing of the FGC SEI message (AG 5 N 140) [P. de Lagrange, W. Husak, M. Wien] [AG 5 N 140)]

Remains valid – not updated: JVET-AM2038 White paper on VSEI [J. Boyce, S. McCarthy, S. Deshpande, G. J. Sullivan, Y. Sanchez, Y.-K. Wang] [AG 3 N209]

Update white paper on VVC?

JVET-AN2039 Common test conditions for technology targeting ultra low latency and packet loss resilience [S. Ikonin, S. Deshpande, V. Zakharchenko] (2025-10-31)

New version?

JVET-AO2040 Complexity reporting template for coding algorithms and tools [X. Li, E. Alshina, I. Moccagatta] (2026-XX-XX)

JVET-AO2041 Announcement of JVET AHG17 Meeting in Aachen, DE, 25-27 February 2026 [J.-R. Ohm, M. Wien] (2026-XX-XX)

Meeting will be hybrid.

Experts requesting for Schengen visa might want to contact J.-R. Ohm and M. Wien as soon as possible.

# Future meeting plans, expressions of thanks, a.o.b., and closing of the meeting

The draft of the WG 5 recommendations (see Annex C) was reviewed and approved in JVET at XXXX–XXXX on XXday X January.

Future meeting plans were established with the following general guidelines (assuming face-to-face meetings):

- Meeting under ITU-T SG21 auspices when it meets (ordinarily starting meetings on the Monday, Tuesday or Wednesday of the first week and closing it on the Wednesday of the second week of the SG21 meeting – a total of 8-10 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/‌SC 29 auspices when its MPEG WGs meet (ordinarily starting meetings on the Wednesday, Thursday or Friday prior to the main week of such meetings and closing it on the same day as other MPEG WGs – a total of 8–10 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined, or AHG meetings might be scheduled prior to the meeting. In cases of online meetings, no sessions should be held on weekend days, such that meetings would typically start two days earlier.

Some specific future meeting plans were established as follows:

- During 24 April – 1 May 2026, 42nd meeting under ISO/IEC JTC 1/‌SC 29 auspices in Santa Eulària, ES, to be conducted as hybrid meeting,

- During 7 – 15 July 2026, 43rd meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 17 – 23 October 2026, 44th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Hangzhou, CN, to be conducted as hybrid meeting,

- During 13 – 22 January 2027, 45th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Brisbane, AU, to be conducted as hybrid meeting,

- During 17 – 28 April 2027, 46th meeting under ITU-T SG21 auspices in Geneva, CH, to be conducted as physical meeting with remote participation,

- During 7 – 16 July 2027, 47th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Tampere, FI, to be conducted as hybrid meeting,

- During 20 – 29 October 2027, 48th meeting under ISO/IEC JTC 1/‌SC 29 auspices in Shenzhen, CN, to be conducted as hybrid meeting,

- During January 2028, 49th meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.,

- During April 2028, 50th meeting under ITU-T SG21 auspices, date and location t.b.d.,

- During July 2028, 51st meeting under ISO/IEC JTC 1/‌SC 29 auspices, date and location t.b.d.

The agreed document deadline for the 42nd JVET meeting was planned to be XXday X April 2026.

XXX were thanked for offering new video test sequences.

Marius Preda was thanked for the service of managing and maintaining the document site jvet-experts.org. Institut Mines-Télécom was thanked for hosting the sites.

The 41st JVET meeting was closed at approximately XXXX hours UTC on XXXday X January 2026.

# Annex A to JVET report:
List of documents

Dates and times in the table below are in Paris/Geneva time (1 hr. ahead of UTC). It is noted that, if title or authorship of a document deviates from the title or author list in the body of the report, the list of documents in this annex contains the correct title and authors.

# Annex B to JVET report: List of meeting participants

The participants of the forty-first meeting of the JVET, according to the participation records from the Zoom teleconferencing tool used for the meeting sessions (approximately XXX people in total), were as follows:

# Annex C to JVET report:
Recommendations of the 22nd meeting of
ISO/IEC JTC 1/SC 29/WG 5 MPEG Joint Video Experts Team with ITU-T SG21

ISO/IEC JTC 1/SC 29/WG 5 N XXX