| ITU – Telecommunications Standardization Sector STUDY GROUP 21 Question 6 Video Coding Experts Group (VCEG) 76th Meeting: 27 March – 4 April 2025, by teleconference | Document VCEG-BX23-v1 |
| --- | --- |

| Question: | 6/21 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Jonathan Pfaff (HHI), Christof Fersch (Dolby), Sam Jelfs (Philips), Paul Haase (HHI) | Email: | jonathan.pfaff@hhi.fraunhofer.de, christof.fersch@dolby.com, sam.jelfs@philips.com, paul.haase@hhi.fraunhofer.de |
| Title: | Common test conditions and evaluation procedures for H.BWC technical experiments |  |  |
| Purpose: | Common test conditions |  |  |

_____________________________

# Abstract

This document defines common test conditions (CTC) and software reference configurations for biomedical waveform coding (H.BWC) experiments. Test conditions and configurations are recommended for the use in applicable technical contributions to VCEG. In addition, this document specifies six datasets to be used as well as an evaluation procedure.

# Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and facilitate the comparison of the outcome of experiments.

This document defines one test configuration for each of the following three categories of biomedical waveform data:

- Electrocardiography (ECG) data

- Electroencephalography (EEG) data

- Electromyography (EMG) data

# Datasets

Six datasets are specified for testing according to the following table:

| Dataset name | Dataset category | Source |
| --- | --- | --- |
| MIT | ECG | Connect to server: ftp://ftp.hhi.fraunhofer.de/ Download file: MIT_ECG_Dataset.zip All files inside of MIT_ECG_Dataset.zip are to be used for this dataset. |
| CHBMIT | EEG | Connect to server1: ftp://ftp.hhi.fraunhofer.de/ Download file: EEG_Data_New/chbmit_edf_hyphenChannelsRemoved.zip A subset of files to be used for development of the standard are listed in Appendix A |
| NMR55 | EEG | Eduardo López-Larraz and María Sierra-Torralba and Sergio Clemente and Galit Fierro and David Oriol and Javier Minguez and Luis Montesano and Jens G. Klinzing (2024). Bitbrain Open Access Sleep Dataset. OpenNeuro. doi:10.18112/openneuro.ds005555.v1.0.0 A subset of files to be used for development of the standard are listed in appendix A |
| NMR57 | EEG | Haydn G. Herrema and Michael J. Kahana (2024). Free Recall with Closed-Loop Stimulation at Encoding (Encoding Classifier). doi:10.18112/openneuro.ds005557.v1.0.0 A subset of files to be used for development of the standard are listed in appendix A |
| Ozdemir | EMG | Connect to server1: ftp://ftp.hhi.fraunhofer.de/ Download file: MENDELEY_Dataset.zip The files in folder 'raw' inside of MENDELEY_Dataset.zip are to be used for this dataset. |
| INCART | ECG | Connect to server1: ftp://ftp.hhi.fraunhofer.de/ Download file: INCART_Dataset.zip Original data in WFDB format is available at: https://physionet.org/content/incartdb/1.0.0/ |

Additional demonstrations of the performance of proposed technologies on other types of data are also highly welcome.

# Reference Software

The reference software is available at: https://vcgit.hhi.fraunhofer.de/vceg-sw/bwc

# Encoding configurations

For each of the dataset categories, two configuration files for running the encoder executable are available in the software repository:

- ECG: cfg/combinedPresetECG.cfg, cfg/combinedPresetECG_IndepChannel.cfg

- EEG: cfg/combinedPresetEEG.cfg, cfg/combinedPresetEEG_IndepChannel.cfg

- EMG: cfg/combinedPresetEMG.cfg, cfg/combinedPresetEMG_IndepChannel.cfg

The first configuration file realizes a joint coding of the channels while the second configuration file realizes an independent coding of the channels.

Each input sequence is to be encoded several times in order to achieve different bit rates.

A further config parameter InputBitDepth needs to be specified for encoding. It is set to 12 for the MIT dataset and to 16 for all other datasets.

The config parameter StepSizeForQP controls the quantization step size and thus the resulting size of the bit stream.

The following three lists specify values for parameter StepSizeForQP to be used when encoding sequences from the respective dataset category:

ECG:	[1, 1.125, 1.5, 2, 3, 4, 5.5, 7, 9, 11, 13.5, 16, 19, 22]

EEG:	[1, 1.125, 1.5, 2, 3, 4, 5.5, 7, 9, 11, 13.5, 16, 22, 33, 44, 55,
	 66, 77, 88, 99, 110]

EMG:	[1, 1.5, 2, 3, 4, 5.5, 7, 9, 11, 13.5, 16, 19, 22, 25.5, 29, 33,
	 37, 41.5, 46, 51, 56, 60.5]

Each of the values produces one individual bitstream per input sequence.

# Error measures and bit rate

Two error measures are employed to objectively evaluate the compressed representations of the bitstreams. Let N be the number of channels and let M be the number of samples per channel of an input sequence. Furthermore, let ai,j be the j-th sample (with 0≤j<M) of channel i (with 0≤i<N) and let ai,j be the corresponding reconstructed sample after decoding a bitstream.

The first error measure is PSNR1 and it shall be calculated as follows:

MSEi=1Mj=0M-1ai,j-ai,j2,

MSE=1Ni=0N-1MSEi,

PSNR1=-10*log10MSE2B-12.

The second error measure is PSNR2 and it shall be calculated as follows for the channels that have non-zero MSE:

PSNRi,MSE≠0=-10*log10MSEi,MSE≠02B-12,

PSNR2=1NMSE≠0i=0NMSE≠0-1PSNRi,MSE≠0,

where NMSE≠0 is the number of channels that have non-zero MSE.

The value of B that shall be set to 16.

The bit rate is measured by the number of bits per sample (BPS), defined as

BPS=#number of bits in the bitstream fileN∙M.

# Experimental evaluation

The anchor to be used as reference corresponds to a simulation run according to the encoding specification of section 4 of the tag with the highest version number in the reference software git repository as specified in section 3. This tag shall be announced after each meeting by the AhG chairs as soon as it is available.

A Bjøntegaard Delta rate is calculated for each input sequence between the technology under test and the anchor. For this, an excel reporting template (bwc-ctc.xlsm) and a corresponding Python script for calculating the corresponding BD rate values can be found in the software repository. This script implements the BD rate calculation according to the document JVET-H0030. Also, curves which plot the PSNR against the rate for the technology under test and the anchor are to be provided for each input sequence. Moreover, for each test sequence, the percentage changes of the geometric means of the runtimes relative to the anchor, taken over the working points from Section 4, shall be reported both for the encoder and for the decoder.

Results shall be produced both in the joint channel coding mode and in the independent channel coding mode.

#### Appendix A

Due to the large size and complexity of processing the EEG datasets, only a subset of the data is being used for current experiments. The following lists shall be used:

CHBMIT – Development:

chb24_13.edf

chb18_27.edf

chb24_15.edf

chb08_29.edf

chb24_19.edf

chb14_17.edf

chb20_04.edf

chb04_17.edf

chb05_30.edf

chb15_33.edf

chb05_31.edf

chb05_35.edf

chb16_01.edf

chb01_20.edf

chb06_06.edf

chb01_27.edf

chb13_12.edf

chb13_13.edf

chb09_10.edf

chb19_15.edf

chb14_37.edf

chb14_39.edf

chb15_04.edf

chb15_09.edf

chb01_40.edf

chb22_10.edf

chb02_10.edf

chb11_92.edf

chb02_12.edf

chb22_18.edf

chb02_14.edf

chb02_15.edf

chb18_10.edf

chb18_11.edf

chb24_01.edf

chb18_17.edf

chb18_18.edf

chb14_04.edf

chb04_06.edf

chb05_20.edf

chb05_21.edf

chb21_12.edf

chb15_26.edf

chb15_28.edf

chb11_14.edf

chb01_14.edf

chb21_19.edf

chb12_33.edf

chb12_36.edf

chb23_07.edf

chb23_09.edf

chb24_20.edf

chb13_55.edf

chb09_01.edf

chb09_04.edf

chb19_09.edf

chb17a_03.edf

chb01_32.edf

chb22_51.edf

chb02_07.edf

chb08_02.edf

chb03_24.edf

chb10_31.edf

chb19_29.edf

chb11_02.edf

chb01_07.edf

chb11_56.edf

chb22_26.edf

chb22_27.edf

CHBMIT – Verification

chb18_29.edf

chb04_11.edf

chb24_17.edf

chb20_02.edf

chb14_19.edf

chb15_30.edf

chb15_31.edf

chb05_37.edf

chb01_22.edf

chb01_24.edf

chb06_07.edf

chb13_10.edf

chb13_14.edf

chb13_16.edf

chb13_18.edf

chb19_12.edf

chb20_21.edf

chb12_10.edf

chb17c_03.edf

chb17c_04.edf

chb18_12.edf

chb18_13.edf

chb04_02.edf

chb08_18.edf

chb24_07.edf

chb24_09.edf

chb11_12.edf

chb01_11.edf

chb05_27.edf

chb11_16.edf

chb11_19.edf

chb01_18.edf

chb17b_59.edf

chb02_33.edf

chb02_35.edf

chb07_15.edf

chb12_39.edf

chb13_03.edf

chb23_06.edf

chb13_06.edf

chb03_07.edf

chb19_01.edf

chb20_12.edf

chb19_08.edf

chb17a_05.edf

chb15_46.edf

chb16_12.edf

chb15_49.edf

chb06_12.edf

chb06_14.edf

chb22_04.edf

chb02_01.edf

chb06_18.edf

chb22_07.edf

chb22_08.edf

chb19_24.edf

chb15_10.edf

chb15_11.edf

chb21_01.edf

chb15_15.edf

chb21_03.edf

chb22_20.edf

chb22_23.edf

chb12_21.edf

chb02_20.edf

chb07_04.edf

chb02_26.edf

chb07_08.edf

NMR55 – Development

sub-101_task-Sleep_acq-headband_eeg.edf

sub-104_task-Sleep_acq-headband_eeg.edf

sub-107_task-Sleep_acq-headband_eeg.edf

sub-110_task-Sleep_acq-headband_eeg.edf

sub-112_task-Sleep_acq-headband_eeg.edf

sub-113_task-Sleep_acq-headband_eeg.edf

sub-115_task-Sleep_acq-headband_eeg.edf

sub-118_task-Sleep_acq-headband_eeg.edf

sub-119_task-Sleep_acq-headband_eeg.edf

sub-121_task-Sleep_acq-headband_eeg.edf

sub-123_task-Sleep_acq-headband_eeg.edf

sub-124_task-Sleep_acq-headband_eeg.edf

sub-125_task-Sleep_acq-headband_eeg.edf

sub-126_task-Sleep_acq-headband_eeg.edf

sub-128_task-Sleep_acq-headband_eeg.edf

sub-12_task-Sleep_acq-headband_eeg.edf

sub-13_task-Sleep_acq-headband_eeg.edf

sub-19_task-Sleep_acq-headband_eeg.edf

sub-20_task-Sleep_acq-headband_eeg.edf

sub-22_task-Sleep_acq-headband_eeg.edf

sub-24_task-Sleep_acq-headband_eeg.edf

sub-25_task-Sleep_acq-headband_eeg.edf

sub-26_task-Sleep_acq-headband_eeg.edf

sub-27_task-Sleep_acq-headband_eeg.edf

sub-28_task-Sleep_acq-headband_eeg.edf

sub-29_task-Sleep_acq-headband_eeg.edf

sub-30_task-Sleep_acq-headband_eeg.edf

sub-31_task-Sleep_acq-headband_eeg.edf

sub-33_task-Sleep_acq-headband_eeg.edf

sub-37_task-Sleep_acq-headband_eeg.edf

sub-42_task-Sleep_acq-headband_eeg.edf

sub-43_task-Sleep_acq-headband_eeg.edf

sub-44_task-Sleep_acq-headband_eeg.edf

sub-46_task-Sleep_acq-headband_eeg.edf

sub-50_task-Sleep_acq-headband_eeg.edf

sub-51_task-Sleep_acq-headband_eeg.edf

sub-53_task-Sleep_acq-headband_eeg.edf

sub-54_task-Sleep_acq-headband_eeg.edf

sub-55_task-Sleep_acq-headband_eeg.edf

sub-59_task-Sleep_acq-headband_eeg.edf

sub-61_task-Sleep_acq-headband_eeg.edf

sub-62_task-Sleep_acq-headband_eeg.edf

sub-64_task-Sleep_acq-headband_eeg.edf

sub-67_task-Sleep_acq-headband_eeg.edf

sub-68_task-Sleep_acq-headband_eeg.edf

sub-69_task-Sleep_acq-headband_eeg.edf

sub-72_task-Sleep_acq-headband_eeg.edf

sub-73_task-Sleep_acq-headband_eeg.edf

sub-76_task-Sleep_acq-headband_eeg.edf

sub-78_task-Sleep_acq-headband_eeg.edf

sub-79_task-Sleep_acq-headband_eeg.edf

sub-80_task-Sleep_acq-headband_eeg.edf

sub-81_task-Sleep_acq-headband_eeg.edf

sub-84_task-Sleep_acq-headband_eeg.edf

sub-85_task-Sleep_acq-headband_eeg.edf

sub-86_task-Sleep_acq-headband_eeg.edf

sub-87_task-Sleep_acq-headband_eeg.edf

sub-89_task-Sleep_acq-headband_eeg.edf

sub-92_task-Sleep_acq-headband_eeg.edf

sub-93_task-Sleep_acq-headband_eeg.edf

sub-94_task-Sleep_acq-headband_eeg.edf

sub-95_task-Sleep_acq-headband_eeg.edf

sub-99_task-Sleep_acq-headband_eeg.edf

sub-6_task-Sleep_acq-headband_eeg.edf

sub-101_task-Sleep_acq-psg_eeg.edf

sub-102_task-Sleep_acq-psg_eeg.edf

sub-103_task-Sleep_acq-psg_eeg.edf

sub-104_task-Sleep_acq-psg_eeg.edf

sub-105_task-Sleep_acq-psg_eeg.edf

sub-106_task-Sleep_acq-psg_eeg.edf

sub-107_task-Sleep_acq-psg_eeg.edf

sub-109_task-Sleep_acq-psg_eeg.edf

sub-111_task-Sleep_acq-psg_eeg.edf

sub-113_task-Sleep_acq-psg_eeg.edf

sub-116_task-Sleep_acq-psg_eeg.edf

sub-118_task-Sleep_acq-psg_eeg.edf

sub-119_task-Sleep_acq-psg_eeg.edf

sub-120_task-Sleep_acq-psg_eeg.edf

sub-122_task-Sleep_acq-psg_eeg.edf

sub-128_task-Sleep_acq-psg_eeg.edf

sub-13_task-Sleep_acq-psg_eeg.edf

sub-15_task-Sleep_acq-psg_eeg.edf

sub-16_task-Sleep_acq-psg_eeg.edf

sub-17_task-Sleep_acq-psg_eeg.edf

sub-18_task-Sleep_acq-psg_eeg.edf

sub-19_task-Sleep_acq-psg_eeg.edf

sub-21_task-Sleep_acq-psg_eeg.edf

sub-23_task-Sleep_acq-psg_eeg.edf

sub-27_task-Sleep_acq-psg_eeg.edf

sub-30_task-Sleep_acq-psg_eeg.edf

sub-32_task-Sleep_acq-psg_eeg.edf

sub-36_task-Sleep_acq-psg_eeg.edf

sub-38_task-Sleep_acq-psg_eeg.edf

sub-41_task-Sleep_acq-psg_eeg.edf

sub-45_task-Sleep_acq-psg_eeg.edf

sub-47_task-Sleep_acq-psg_eeg.edf

sub-50_task-Sleep_acq-psg_eeg.edf

sub-52_task-Sleep_acq-psg_eeg.edf

sub-53_task-Sleep_acq-psg_eeg.edf

sub-56_task-Sleep_acq-psg_eeg.edf

sub-57_task-Sleep_acq-psg_eeg.edf

sub-58_task-Sleep_acq-psg_eeg.edf

sub-59_task-Sleep_acq-psg_eeg.edf

sub-61_task-Sleep_acq-psg_eeg.edf

sub-67_task-Sleep_acq-psg_eeg.edf

sub-68_task-Sleep_acq-psg_eeg.edf

sub-72_task-Sleep_acq-psg_eeg.edf

sub-74_task-Sleep_acq-psg_eeg.edf

sub-75_task-Sleep_acq-psg_eeg.edf

sub-76_task-Sleep_acq-psg_eeg.edf

sub-77_task-Sleep_acq-psg_eeg.edf

sub-81_task-Sleep_acq-psg_eeg.edf

sub-83_task-Sleep_acq-psg_eeg.edf

sub-84_task-Sleep_acq-psg_eeg.edf

sub-86_task-Sleep_acq-psg_eeg.edf

sub-88_task-Sleep_acq-psg_eeg.edf

sub-89_task-Sleep_acq-psg_eeg.edf

sub-90_task-Sleep_acq-psg_eeg.edf

sub-93_task-Sleep_acq-psg_eeg.edf

sub-94_task-Sleep_acq-psg_eeg.edf

sub-96_task-Sleep_acq-psg_eeg.edf

sub-97_task-Sleep_acq-psg_eeg.edf

sub-98_task-Sleep_acq-psg_eeg.edf

sub-99_task-Sleep_acq-psg_eeg.edf

sub-1_task-Sleep_acq-psg_eeg.edf

sub-3_task-Sleep_acq-psg_eeg.edf

sub-4_task-Sleep_acq-psg_eeg.edf

sub-7_task-Sleep_acq-psg_eeg.edf

NMR55 – Verification

sub-100_task-Sleep_acq-headband_eeg.edf

sub-102_task-Sleep_acq-headband_eeg.edf

sub-103_task-Sleep_acq-headband_eeg.edf

sub-105_task-Sleep_acq-headband_eeg.edf

sub-106_task-Sleep_acq-headband_eeg.edf

sub-108_task-Sleep_acq-headband_eeg.edf

sub-109_task-Sleep_acq-headband_eeg.edf

sub-111_task-Sleep_acq-headband_eeg.edf

sub-114_task-Sleep_acq-headband_eeg.edf

sub-116_task-Sleep_acq-headband_eeg.edf

sub-117_task-Sleep_acq-headband_eeg.edf

sub-120_task-Sleep_acq-headband_eeg.edf

sub-122_task-Sleep_acq-headband_eeg.edf

sub-127_task-Sleep_acq-headband_eeg.edf

sub-10_task-Sleep_acq-headband_eeg.edf

sub-11_task-Sleep_acq-headband_eeg.edf

sub-14_task-Sleep_acq-headband_eeg.edf

sub-15_task-Sleep_acq-headband_eeg.edf

sub-16_task-Sleep_acq-headband_eeg.edf

sub-17_task-Sleep_acq-headband_eeg.edf

sub-18_task-Sleep_acq-headband_eeg.edf

sub-21_task-Sleep_acq-headband_eeg.edf

sub-23_task-Sleep_acq-headband_eeg.edf

sub-32_task-Sleep_acq-headband_eeg.edf

sub-34_task-Sleep_acq-headband_eeg.edf

sub-35_task-Sleep_acq-headband_eeg.edf

sub-36_task-Sleep_acq-headband_eeg.edf

sub-38_task-Sleep_acq-headband_eeg.edf

sub-39_task-Sleep_acq-headband_eeg.edf

sub-40_task-Sleep_acq-headband_eeg.edf

sub-41_task-Sleep_acq-headband_eeg.edf

sub-45_task-Sleep_acq-headband_eeg.edf

sub-47_task-Sleep_acq-headband_eeg.edf

sub-48_task-Sleep_acq-headband_eeg.edf

sub-49_task-Sleep_acq-headband_eeg.edf

sub-52_task-Sleep_acq-headband_eeg.edf

sub-56_task-Sleep_acq-headband_eeg.edf

sub-57_task-Sleep_acq-headband_eeg.edf

sub-58_task-Sleep_acq-headband_eeg.edf

sub-60_task-Sleep_acq-headband_eeg.edf

sub-63_task-Sleep_acq-headband_eeg.edf

sub-66_task-Sleep_acq-headband_eeg.edf

sub-70_task-Sleep_acq-headband_eeg.edf

sub-71_task-Sleep_acq-headband_eeg.edf

sub-74_task-Sleep_acq-headband_eeg.edf

sub-75_task-Sleep_acq-headband_eeg.edf

sub-77_task-Sleep_acq-headband_eeg.edf

sub-82_task-Sleep_acq-headband_eeg.edf

sub-83_task-Sleep_acq-headband_eeg.edf

sub-88_task-Sleep_acq-headband_eeg.edf

sub-90_task-Sleep_acq-headband_eeg.edf

sub-91_task-Sleep_acq-headband_eeg.edf

sub-96_task-Sleep_acq-headband_eeg.edf

sub-97_task-Sleep_acq-headband_eeg.edf

sub-98_task-Sleep_acq-headband_eeg.edf

sub-1_task-Sleep_acq-headband_eeg.edf

sub-2_task-Sleep_acq-headband_eeg.edf

sub-3_task-Sleep_acq-headband_eeg.edf

sub-4_task-Sleep_acq-headband_eeg.edf

sub-5_task-Sleep_acq-headband_eeg.edf

sub-7_task-Sleep_acq-headband_eeg.edf

sub-8_task-Sleep_acq-headband_eeg.edf

sub-9_task-Sleep_acq-headband_eeg.edf

sub-100_task-Sleep_acq-psg_eeg.edf

sub-108_task-Sleep_acq-psg_eeg.edf

sub-110_task-Sleep_acq-psg_eeg.edf

sub-112_task-Sleep_acq-psg_eeg.edf

sub-114_task-Sleep_acq-psg_eeg.edf

sub-115_task-Sleep_acq-psg_eeg.edf

sub-117_task-Sleep_acq-psg_eeg.edf

sub-121_task-Sleep_acq-psg_eeg.edf

sub-123_task-Sleep_acq-psg_eeg.edf

sub-124_task-Sleep_acq-psg_eeg.edf

sub-125_task-Sleep_acq-psg_eeg.edf

sub-126_task-Sleep_acq-psg_eeg.edf

sub-127_task-Sleep_acq-psg_eeg.edf

sub-10_task-Sleep_acq-psg_eeg.edf

sub-11_task-Sleep_acq-psg_eeg.edf

sub-12_task-Sleep_acq-psg_eeg.edf

sub-14_task-Sleep_acq-psg_eeg.edf

sub-20_task-Sleep_acq-psg_eeg.edf

sub-22_task-Sleep_acq-psg_eeg.edf

sub-24_task-Sleep_acq-psg_eeg.edf

sub-25_task-Sleep_acq-psg_eeg.edf

sub-26_task-Sleep_acq-psg_eeg.edf

sub-28_task-Sleep_acq-psg_eeg.edf

sub-29_task-Sleep_acq-psg_eeg.edf

sub-31_task-Sleep_acq-psg_eeg.edf

sub-33_task-Sleep_acq-psg_eeg.edf

sub-34_task-Sleep_acq-psg_eeg.edf

sub-35_task-Sleep_acq-psg_eeg.edf

sub-37_task-Sleep_acq-psg_eeg.edf

sub-39_task-Sleep_acq-psg_eeg.edf

sub-40_task-Sleep_acq-psg_eeg.edf

sub-42_task-Sleep_acq-psg_eeg.edf

sub-43_task-Sleep_acq-psg_eeg.edf

sub-44_task-Sleep_acq-psg_eeg.edf

sub-46_task-Sleep_acq-psg_eeg.edf

sub-48_task-Sleep_acq-psg_eeg.edf

sub-49_task-Sleep_acq-psg_eeg.edf

sub-51_task-Sleep_acq-psg_eeg.edf

sub-54_task-Sleep_acq-psg_eeg.edf

sub-55_task-Sleep_acq-psg_eeg.edf

sub-60_task-Sleep_acq-psg_eeg.edf

sub-62_task-Sleep_acq-psg_eeg.edf

sub-63_task-Sleep_acq-psg_eeg.edf

sub-64_task-Sleep_acq-psg_eeg.edf

sub-66_task-Sleep_acq-psg_eeg.edf

sub-69_task-Sleep_acq-psg_eeg.edf

sub-70_task-Sleep_acq-psg_eeg.edf

sub-71_task-Sleep_acq-psg_eeg.edf

sub-73_task-Sleep_acq-psg_eeg.edf

sub-78_task-Sleep_acq-psg_eeg.edf

sub-79_task-Sleep_acq-psg_eeg.edf

sub-80_task-Sleep_acq-psg_eeg.edf

sub-82_task-Sleep_acq-psg_eeg.edf

sub-85_task-Sleep_acq-psg_eeg.edf

sub-87_task-Sleep_acq-psg_eeg.edf

sub-91_task-Sleep_acq-psg_eeg.edf

sub-92_task-Sleep_acq-psg_eeg.edf

sub-95_task-Sleep_acq-psg_eeg.edf

sub-2_task-Sleep_acq-psg_eeg.edf

sub-5_task-Sleep_acq-psg_eeg.edf

sub-6_task-Sleep_acq-psg_eeg.edf

sub-8_task-Sleep_acq-psg_eeg.edf

sub-9_task-Sleep_acq-psg_eeg.ed

NMR57 – Development

sub-R1124J_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1154D_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1154D_ses-3_task-FR3_acq-bipolar_ieeg.edf

sub-R1154D_ses-4_task-FR3_acq-bipolar_ieeg.edf

sub-R1161E_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1166D_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1166D_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1170J_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1195E_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1195E_ses-2_task-FR3_acq-bipolar_ieeg.edf

sub-R1200T_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1202M_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1204T_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1223E_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1293P_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1124J_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1145J_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1154D_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1154D_ses-4_task-FR3_acq-monopolar_ieeg.edf

sub-R1161E_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1163T_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1166D_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1170J_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1195E_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1195E_ses-2_task-FR3_acq-monopolar_ieeg.edf

sub-R1200T_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1201P_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1204T_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1223E_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1223E_ses-1_task-FR3_acq-monopolar_ieeg.edf

NMR57 – Verification

sub-R1124J_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1145J_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1154D_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1154D_ses-2_task-FR3_acq-bipolar_ieeg.edf

sub-R1161E_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1163T_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1163T_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1170J_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1195E_ses-1_task-FR3_acq-bipolar_ieeg.edf

sub-R1201P_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1202M_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1217T_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1223E_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1235E_ses-0_task-FR3_acq-bipolar_ieeg.edf

sub-R1124J_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1154D_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1154D_ses-2_task-FR3_acq-monopolar_ieeg.edf

sub-R1154D_ses-3_task-FR3_acq-monopolar_ieeg.edf

sub-R1161E_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1163T_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1166D_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1170J_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1195E_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1202M_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1202M_ses-1_task-FR3_acq-monopolar_ieeg.edf

sub-R1217T_ses-0_task-FR3_acq-monopolar_ieeg.edf

sub-R1235E_ses-0_task-FR3_acq-monopolar_ieeg.edf
sub-R1293P_ses-1_task-FR3_acq-monopolar_ieeg.edf

______________________

Credentials for accessing the FTP server can be obtained upon request.