| ITU – Telecommunications Standardization Sector STUDY GROUP 21 Question 6 Video Coding Experts Group (VCEG) 78th Meeting: 13 January – 23 January 2026, by teleconference | Document VCEG-BZ11-v1 |
| --- | --- |

| Question: | 6/21 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Sophie Pientka, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Email: | firstname.lastname@hhi.fraunhofer.de |
| Title: | Report of CE-1 |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Abstract

This document reports on the simulation results of Core Experiment 1 (CE1), which investigate the removal of the DST and the removal of a combination of sample wise prediction modes with the DCT as proposed in the first aspect of SG21-C478.

- Software and configurations

The proposed changes are evaluated under the BWC Common Test Conditions (CTC). Note that the CTC specifies an independent channel coding configuration, which is referred to as ICC in this document.

The software used for the experiments is BWC-4.0 with the bugfixes as of Dec 9, 2025, with the proposed modifications implemented on top of this baseline. For the BD-rate calculation, the anchor is the unmodified BWC-4.0 software using the encoder configuration specified for each sub-category, while the test candidate is the modified BWC-4.0 software using the same encoder configurations.

- Experimental results

CTC (Non-ICC Configuration)

|  | Lossy Compression |  |  |  | Lossless Compression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  | Over BWC-3.0 |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT | BR-R | EncT | DecT |
| MIT (ECG) | -0,70% | -0,70% | 101% | 94% | 0,00% | 100% | 101% |
| INCART (ECG) | -0,69% | -0,70% | 101% | 96% | -0,11% | 100% | 101% |
| CHBMIT (EEG) | -0,13% | -0,13% | 101% | 97% | -0,01% | 99% | 99% |
| NMR55 (EEG) | -0,09% | -0,09% | 101% | 96% | 0,00% | 100% | 100% |
| NMR57 (EEG) | -0,07% | -0,07% | 100% | 98% | 0,00% | 100% | 100% |
| TILT ILLUSION (EEG) | -0,04% | -0,04% | 100% | 98% | 0,00% | 100% | 100% |
| Ozdemir (EMG) | -0,04% | -0,04% | 101% | 96% | -0,35% | 113% | 97% |
| PTT (PPG) | -0,46% | -0,42% | 101% | 96% | 0,00% | 100% | 100% |
| WristPPG (PPG) | -0,08% | -0,09% | 101% | 98% | 0,00% | 101% | 99% |
| Overall | -0,26% | -0,25% | 101% | 96% | -0,05% | 101% | 100% |

CTC – Independent Channel Coding (ICC)

|  | Lossy Compression |  |  |  | Lossless Compression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  | Over BWC-3.0 |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT | BR-R | EncT | DecT |
| MIT (ECG) | -0,69% | -0,69% | 101% | 94% | -0,03% | 100% | 100% |
| INCART (ECG) | -0,48% | -0,47% | 101% | 95% | -0,02% | 100% | 100% |
| CHBMIT (EEG) | -0,10% | -0,10% | 101% | 94% | -0,01% | 100% | 99% |
| NMR55 (EEG) | -0,09% | -0,09% | 101% | 95% | 0,00% | 100% | 100% |
| NMR57 (EEG) | -0,06% | -0,05% | 101% | 95% | -0,01% | 100% | 98% |
| TILT ILLUSION (EEG) | -0,04% | -0,04% | 100% | 97% | -0,01% | 99% | 99% |
| Ozdemir (EMG) | -0,04% | -0,04% | 100% | 95% | 0,00% | 101% | 100% |
| PTT (PPG) | -0,40% | -0,40% | 101% | 95% | -0,01% | 101% | 99% |
| WristPPG (PPG) | -0,09% | -0,09% | 100% | 98% | -0,01% | 101% | 98% |
| Overall | -0,22% | -0,22% | 101% | 95% | -0,01% | 100% | 99% |

- Observation and Summary

The experimental results show that the modifications to the transform signaling proposed in the first aspect of SG21-C478 provide a simplified codec and coding efficiency improvements for the standard CTC configurations. Based on these findings, the proposed modifications should be incorporated into the next version of the BWC software.

- Changes to the draft specification text

Prediction trafo block data syntax

| prediction_trafo_data_block ( ) { | Descriptor |
| --- | --- |
| for( ch = 0; ch < numChannels; ch++ ) { |  |
| if( cgps_allow_block_matching_pred_flag ¦ ¦ ( cgps_allow_cross_channel_pred_flag && ( ch & DepChMask ) > 0 ) ) |  |
| block_matching_or_cross_channel_pred_flag | ae(ch) |
| if( block_matching_or_cross_channel_pred_flag ) { |  |
| if( cgps_allow_block_matching_pred_flag && cgps_allow_cross_channel_pred_flag && ( ch & DepChMask ) > 0 ) |  |
| cross_channel_pred_flag | ae(ch) |
| if( cross_channel_pred_flag ) |  |
| cross_channel_prediction_data( ch ) |  |
| else |  |
| block_matching_prediction_data( ch ) |  |
| }else{ |  |
| BlockPredMode = BPM_OFF |  |
| if (cgps_allow_DC_pred_flag ¦ ¦ cgps_allow_line_fit_flag ){ |  |
| block_pred_off_flag | ae(ch) |
| if ( !block_pred_off_flag ){ |  |
| if (cgps_allow_DC_pred_flag && cgps_allow_line_fit_flag ){ |  |
| block_pred_LF_flag | ae(ch) |
| if( block_pred_LF_flag ) |  |
| BlockPredMode = BPM_LF |  |
| else |  |
| BlockPredMode = BPM_DC |  |
| }else{ |  |
| BlockPredMode = cgps_allow_DC_pred_flag ? BPM_DC : BPM_LF |  |
| } |  |
| } |  |
| } |  |
| } |  |
| TransformMode = ( cgps_allow_transform_skip = = 0 ) ? TM_DCT : TM_OFF |  |
| if ( cgps_ allow_transform_skip = = 1 ){ |  |
| dct_flag | ae(ch) |
| if(dct_flag) |  |
| TransformMode = TM_DCT |  |
| } |  |
| sample_pred_mode( ch ) |  |
| if( spred_lpf_flag ) |  |
| linear_predictive_filtering_data( ch ) |  |
| … |  |

Sample pred mode syntax

| sample_pred_mode( ch ) { | Descriptor |
| --- | --- |
| if( cgps_allow_lpf_flag) |  |
| spred_lpf_flag | ae(ch) |
| if( !spred_lpf_flag && TransformMode = = TM_OFF && cgps_allow_sample_pred_fixed_weights_flag) |  |
| spred_rem_mode_idx | ae(ch) |
| } |  |

- Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

________________________