| ITU – Telecommunications Standardization Sector STUDY GROUP 21 Question 6 Video Coding Experts Group (VCEG) 78th Meeting: 13 January – 23 January 2026, by teleconference | Document VCEG-BZ13-v1 |
| --- | --- |

| Question: | 6/21 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | Sophie Pientka, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Email: | firstname.lastname@hhi.fraunhofer.de |
| Title: | Report of CE-3 |  |  |
| Purpose: | Proposal |  |  |

_____________________________

# Abstract

This document reports on the simulation results of Core Experiment 3 (CE3), which investigate that the presence of the LMS flag is conditioned on the last scan position as proposed in the third aspect of SG21-C478.

- Software and configurations

The proposed changes are evaluated under the BWC Common Test Conditions (CTC). Note that the CTC specifies an independent channel coding configuration, which is referred to as ICC in this document.

The software used for the experiments is BWC-4.0 with the bugfixes as of Dec 9, 2025, with the proposed modifications implemented on top of this baseline. For the BD-rate calculation, the anchor is the unmodified BWC-4.0 software using the encoder configuration specified for each sub-category, while the test candidate is the modified BWC-4.0 software using the same encoder configurations.

- Experimental results

CTC (Non-ICC Configuration)

|  | Lossy Compression |  |  |  | Lossless Compression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  | Over BWC-3.0 |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT | BR-R | EncT | DecT |
| MIT (ECG) | -0,02% | -0,02% | 101% | 101% | 0,00% | 101% | 100% |
| INCART (ECG) | -0,01% | -0,02% | 101% | 95% | 0,00% | 100% | 98% |
| CHBMIT (EEG) | 0,00% | 0,00% | 101% | 95% | 0,00% | 100% | 94% |
| NMR55 (EEG) | -0,01% | 0,00% | 101% | 98% | 0,00% | 100% | 98% |
| NMR57 (EEG) | 0,00% | 0,00% | 100% | 100% | 0,00% | 100% | 100% |
| TILT ILLUSION (EEG) | 0,00% | 0,00% | 100% | 100% | 0,00% | 100% | 100% |
| Ozdemir (EMG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 101% | 101% |
| PTT (PPG) | 0,00% | 0,00% | 100% | 99% | 0,00% | 100% | 100% |
| WristPPG (PPG) | 0,00% | 0,00% | 100% | 100% | 0,00% | 100% | 99% |
| Overall | 0,00% | 0,00% | 100% | 99% | 0,00% | 100% | 99% |

CTC – Independent Channel Coding (ICC)

|  | Lossy Compression |  |  |  | Lossless Compression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  | Over BWC-3.0 |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT | BR-R | EncT | DecT |
| MIT (ECG) | -0,03% | -0,03% | 101% | 101% | 0,00% | 101% | 99% |
| INCART (ECG) | -0,01% | -0,01% | 101% | 100% | 0,00% | 101% | 100% |
| CHBMIT (EEG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 100% | 100% |
| NMR55 (EEG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 100% | 100% |
| NMR57 (EEG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 100% | 100% |
| TILT ILLUSION (EEG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 100% | 100% |
| Ozdemir (EMG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 101% | 100% |
| PTT (PPG) | 0,00% | 0,00% | 101% | 99% | 0,00% | 101% | 100% |
| WristPPG (PPG) | 0,00% | -0,01% | 100% | 100% | 0,00% | 100% | 98% |
| Overall | -0,01% | -0,01% | 101% | 100% | 0,00% | 100% | 100% |

- Observation and Summary

The experimental results show that the modifications to the LMS flag in the third aspect of SG21-C478 provide a coding efficiency improvements for the standard CTC configurations. Based on these findings, the proposed modifications should be incorporated into the next version of the BWC software.

- Changes to the draft specification text

Quant res sample data syntax

| quant_res_sample_data( ch ) { | Descriptor |
| --- | --- |
| … |  |
| last_pos_index_gt0_flag | ae(ch) |
| if( last_pos_index_gt0_flag ) { |  |
| last_pos_index_rem | ae(ch) |
| if( Log2BlockSize > 8 && Log2BlockSize = = Log2BlockSizeLast ) { |  |
| last_pos_index_sign | aep() |
| } |  |
| last_scan_pos = 1 + last_pos_index_rem |  |
| if( last_pos_index_sign ) { |  |
| last_scan_pos = −last_scan_pos |  |
| } |  |
| last_scan_pos = last_scan_pos + LastScanPosPrev |  |
| } else |  |
| last_scan_pos = 0 |  |
| if( Log2BlockSize > 8 ) { |  |
| LastScanPosPrev = last_scan_pos |  |
| Log2BlockSizeLast = Log2BlockSize |  |
| } |  |
| if ( cgps_allow_lms_flag &&( (cgps_allow_cc_lms_flag && ch > 0 && LastScanPos > 0 ) ¦¦ LastScanPos >1) ) |  |
| lms_data ( ) |  |
| … |  |

LMS data syntax

| lms_data ( ) { | Descriptor |
| --- | --- |
| lms_flag | ae(v) |
| if ( lms_flag ) { |  |
| if( cgps_allow_lms_split_flag && (cgps_allow_cc_lms_flag && ch > 0 && (LastScanPos > ( 1 << ( Log2BlockSize –1 ) ) ) ¦¦ LastScanPos > ( 1 << ( Log2BlockSize –1 ) )+1) |  |
| lms_split_flag | ae(v) |
| … |  |
| } |  |

- Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

________________________

Additional results

Since the modifications also include that the presence of the LMS split flag is conditioned on the last scan position, the changes were also evaluated with a modified anchor. Therefore, the proposed changes are evaluated under modified test conditions, similar to CE 2. The BWC Common Test Conditions (CTC) are used with the addition that the parameter cgps_allow_lms_split_flag is set to 1. Note that the CTC specifies an independent channel coding configuration, which is also modified by setting the parameter cgps_allow_lms_split_flag to 1. This configuration referred to as ICC with enforced LMS split in this document.

The software used for these experiments is BWC-4.0 with the bugfixes as of Dec 9, 2025 and the change that LMS split is always active in case LMS is used and the coding of the split flag is enabled by cgps_allow_lms_split_flag. The proposed modifications of the presence of the LMS flag and the LMS split flag are implemented on top of this baseline. For the BD-rate calculation, the anchor is the unmodified BWC-4.0 software with forced use of the LMS split using the encoder configuration specified for each sub-category, while the test candidate is the modified BWC-4.0 software with the forced use of the LMS split using the same encoder configurations.

Joint channel coding – Anchor: Enforce LMS split; Test: Enforce LMS split with proposed modification

|  | Lossy Compression |  |  |  | Lossless Compression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  | Over BWC-3.0 |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT | BR-R | EncT | DecT |
| MIT (ECG) | -0,09% | -0,09% | 101% | 102% | 0,00% | 101% | 100% |
| INCART (ECG) | -0,06% | -0,07% | 101% | 96% | -0,01% | 101% | 98% |
| CHBMIT (EEG) | -0,03% | -0,03% | 101% | 95% | 0,00% | 101% | 93% |
| NMR55 (EEG) | -0,03% | -0,02% | 101% | 99% | 0,00% | 100% | 98% |
| NMR57 (EEG) | -0,01% | -0,01% | 100% | 100% | 0,00% | 100% | 99% |
| TILT ILLUSION (EEG) | 0,00% | 0,00% | 100% | 100% | 0,00% | 100% | 99% |
| Ozdemir (EMG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 101% | 100% |
| PTT (PPG) | -0,04% | -0,04% | 100% | 101% | 0,00% | 101% | 100% |
| WristPPG (PPG) | 0,00% | 0,01% | 100% | 100% | 0,00% | 100% | 99% |
| Overall | -0,03% | -0,03% | 100% | 99% | 0,00% | 101% | 98% |

Independent channel coding – Anchor: Enforce LMS split; Test: Enforce LMS split with proposed modification

|  | Lossy Compression |  |  |  | Lossless Compression |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BWC-3.0 |  |  |  | Over BWC-3.0 |  |  |
|  | BD-PSNR1 | BD-PSNR2 | EncT | DecT | BR-R | EncT | DecT |
| MIT (ECG) | -0,07% | -0,07% | 101% | 103% | 0,00% | 101% | 100% |
| INCART (ECG) | -0,03% | -0,03% | 101% | 101% | 0,00% | 101% | 100% |
| CHBMIT (EEG) | -0,01% | 0,00% | 101% | 100% | 0,00% | 101% | 100% |
| NMR55 (EEG) | -0,01% | -0,01% | 101% | 100% | 0,00% | 100% | 100% |
| NMR57 (EEG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 101% | 100% |
| TILT ILLUSION (EEG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 101% | 100% |
| Ozdemir (EMG) | 0,00% | 0,00% | 101% | 100% | 0,00% | 102% | 100% |
| PTT (PPG) | -0,02% | -0,02% | 101% | 100% | 0,00% | 101% | 100% |
| WristPPG (PPG) | 0,00% | -0,01% | 101% | 100% | 0,00% | 100% | 99% |
| Overall | -0,01% | -0,02% | 101% | 101% | 0,00% | 101% | 100% |