| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2nd Meeting: Geneva, CH, 21-28 July, 2010 | Document: JCTVC-B090 |
| --- | --- |

| Title: | ALF memory compression and IBDI/ALF coding efficiency test results on TMuC-0.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA Mehmet Umut Demircin Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1861 madhukar@ti.com +1-214-567-4005 demircin@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

ALF estimation is a complex process that involves calculation of correlation coefficients for determining Wiener filter, selection of filter size (5x5, 7x7 or 9x9), and selection of frame blocks on which to apply the filter. The deblocked frame buffer is read multiple times in the ALF estimation process. For example, in existing TMuC-0.1, the deblocked frame buffer is read more than 10 times dramatically increasing the memory bandwidth required. Increased memory bandwidth leads to increased cost and increased power. This contribution proposes the use of memory compression for ALF bandwidth reduction. Note that ALF memory compression is different from Tool Experiment 2 (TE2) IBDI and memory compression in the sense that ALF memory compression operates on deblocked filter output (which is ALF input) whereas TE2 techniques operate on ALF output (which is reference frame). Simulation results on TMuC-0.1 show that ALF memory compression from 12bits8bits achieves 33% memory bandwidth and memory size reduction at the cost of average BD-Rate increase of 0.22%. ALF memory compression from 12bits6bits achieves 50% memory bandwidth and memory size reduction at the cost of average BD-Rate increase of 0.32%.

This contribution also presents ALF and IBDI coding efficiency simulation test results on TMuC-0.1 when one tool is turned off at a time. When ALF is turned off, there is an average BD-Rate increase of 5.82% for ClassA&B sequences and 3.97% over all sequences. A noticeable data point is that maximum BD-Rate increase when ALF is turned off is 13.26%. When IBDI is turned off, there is an average BD-Rate increase of 3.88% for Class A&B sequences and 2.61% over all sequences.

# Introduction

Adaptive loop filter (ALF) was first introduced in ITU-T Video Coding Experts Group (VCEG). It has since then been included in the Test Model under Consideration (TMuc) of the next generation HEVC video coding standard [1]. In ALF technique, an in-loop Wiener filter is applied on the deblocking filter outpu so as to get a match with the input video frame. The Wiener filter is designed to minimize error between the deblocking filter output and the input video frame and is transmitted in slice/frame header.

Figure 1 shows ALF estimation process. ALF estimation is a complex process that involves calculation of correlation coefficients for determining Wiener filter, selection of filter size (5x5, 7x7 or 9x9), and selection of frame blocks on which to apply the filter. The deblocked frame buffer is read multiple times in the ALF estimation process. For example, in existing TMuC the deblock frame buffer is read more than 10 times dramatically increasing the memory bandwidth required. Increased memory bandwidth leads to increased cost and increased power. So a technique for reducing the number of accesses to deblocked frame buffer is required.

# ALF memory bandwidth reduction

This contribution proposes the use of memory compression for ALF bandwidth reduction as shown in Figure 2. The deblocking filter output is compressed before being stored in external memory and it is uncompressed after reading it from frame memory. The data that is written to and read from external memory is in compressed form thereby leading to memory bandwidth savings. There is a savings in the amount of memory needed to store deblocked frame buffer too. Note that ALF memory compression is different from Tool Experiment 2 (TE2) IBDI and memory compression in the sense that ALF memory compression operates on deblocked filter output (which is ALF input) whereas TE2 techniques operate on ALF output (which is reference frame).

# ALF memory bandwidth reduction results

Memory compression technique used can be a block-based technique or a line-based technique. For a block based technique, one can allow for dependency between blocks because the data access pattern in ALF is in raster scan format.

Table 1 shows BD-Rate increase when the memory compression technique of JCTVC-B089, TI (8x8 block size, please see [2] for more details) is used in TMuC-0.1 [3]. A total of 121 frames were encoded using the scripts in cfg\cfp-fast directory of TMuC-0.1. Alpha.bat was used in simulations. Simulation results on TMuC-0.1 show that ALF memory compression from 12bits8bits achieves 33% memory bandwidth and memory size reduction at the cost of average BD-Rate increase of 0.22%. ALF memory compression from 12bits6bits achieves 50% memory bandwidth and memory size reduction at the cost of average BD-Rate increase of 0.32%.

Other memory compression techniques such as those presented by JCTVC-BXXX, TOSHIBA [4] and JCTVC-BXXX, NEC [5] can also be used.

Table 1: BD-Rate increase because of using ALF memory compression.

|  |  | ALF MemCompress (12bits -> 8bits) |  | ALF MemCompress (12bits -> 6bits) |
| --- | --- | --- | --- | --- |
| Memory size and bandwidth reduction |  | 33.33% | 50% |  |
|  |  | BD-Rate increase |  |  |
| S01 | Traffic | 0.31 | 0.65 |  |
| S02 | PeopleOnStreet | 0.03 | 0.06 |  |
| S03 | Kimono | 0.02 | -0.01 |  |
| S04 | ParkScene | 0.07 | 0.12 |  |
| S05 | Cactus | 0.22 | 0.29 |  |
| S06 | BasketballDrive | 0.08 | -0.01 |  |
| S07 | BQTerrace | 0.80 | 1.16 |  |
| S08 | BasketballDrill | 0.40 | 0.40 |  |
| S09 | BQMall | 0.06 | 0.26 |  |
| S10 | PartyScene | 0.09 | 0.36 |  |
| S11 | RaceHorses | 0.05 | -0.08 |  |
| S12 | BasketballPass | 0.27 | 0.11 |  |
| S13 | BQSquare | 0.60 | 1.10 |  |
| S14 | BlowingBubbles | 0.29 | 0.71 |  |
| S15 | RaceHorses | 0.06 | 0.30 |  |
|  | All Seq Avg | 0.22 | 0.36 |  |
|  | All Seq Min | 0.02 | -0.08 |  |
|  | All Seq Max | 0.80 | 1.16 |  |
|  | Class A&B avg | 0.22 | 0.32 |  |
|  | Class A&B min | 0.02 | -0.01 |  |
|  | Class A&B max | 0.80 | 1.16 |  |

# IBDI/ALF coding efficiency test results on TMuC-0.1

Table 1 shows BD-Rate increase when ALF and IBDI is turned off one at a time in TMuC-0.1 [3]. A total of 121 frames were encoded using the scripts in cfg\cfp-fast directory of TMuC-0.1. Alpha.bat was used in simulations. When ALF is turned off, there is an average BD-Rate increase of 5.82% for ClassA&B sequences and 3.97% over all sequences. A noticeable data point is that maximum BD-Rate increase when ALF is turned off is 13.26%. When IBDI is turned off, there is an average BD-Rate increase of 3.88% for Class A&B sequences and 2.61% over all sequences.

|  |  | BD-Rate increase |  |
| --- | --- | --- | --- |
|  |  | ALF off | IBDI off |
| S01 | Traffic | 5.34 | 6.11 |
| S02 | PeopleOnStreet | 7.33 | 0.40 |
| S03 | Kimono | 4.12 | 5.29 |
| S04 | ParkScene | 3.03 | 2.86 |
| S05 | Cactus | 3.92 | 3.02 |
| S06 | BasketballDrive | 3.75 | 4.15 |
| S07 | BQTerrace | 13.26 | 5.35 |
| S08 | BasketballDrill | 4.23 | 2.55 |
| S09 | BQMall | 1.76 | 1.04 |
| S10 | PartyScene | 2.12 | 0.57 |
| S11 | RaceHorses | 3.71 | 0.55 |
| S12 | BasketballPass | 1.28 | 1.73 |
| S13 | BQSquare | 2.76 | 2.36 |
| S14 | BlowingBubbles | 0.53 | 1.95 |
| S15 | RaceHorses | 2.44 | 1.18 |
|  | All Seq Avg | 3.97 | 2.61 |
|  | All Seq Min | 0.53 | 0.40 |
|  | All Seq Max | 13.26 | 6.11 |
|  | Class A&B avg | 5.82 | 3.88 |
|  | Class A&B min | 3.03 | 0.40 |
|  | Class A&B max | 13.26 | 6.11 |

# References

- JCT-VC, “Test Model under Consideration”, JCTVC-A205, Joint Collaborative Team on Video Coding meeting, 15-23 April 2010, Dresden, Germany.

- M. U. Demircin, M. Budagavi, M. Zhou, S. Dikbas, “TE2: Compressed Reference Frame Buffers (CRFB),” JCTVC-B089, Joint Collaborative Team on Video Coding meeting, 21-28 July 2010, Geneva, Switzerland.

- JCT-VC, TMuC-0.1 Reference Software, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.1/  and http://hevc.kw.bbc.co.uk/git/w/jctvc-tmuc.git.

- JCTVC-BXXX, TOSHIBA

- JCTVC-BXXX, NEC

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).