| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C076 WG11 Number: m18099 |
| --- | --- |

| Title: | Summary of Tool Experiment 2 on IBDI and memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TE coordinators |  |  |

_____________________________

# Abstract

This contribution is a summary of tool experiment 2, IBDI and memory compression.  Nine companies and one university have been registered in TE2 and four proposed tools have been evaluated on the common condition with cross-verification after all.  There are two test conditions.  One is the case of keeping the bit depth of reference memory to the input bit depth on IBDI and the other is the case of reduction of reference memory access bandwidth and size.  Detailed results will be reported by proponents.

# Introduction

In this tool experiment, IBDI (Internal Bit-Depth Increase) and memory compression have been discussed.  There are following two purposes;

- Improvement of coding efficiency by increasing internal process of video codec while minimizing reference frame memory access bandwidth.

- Reduction of reference frame memory access bandwidth and reference frame memory size.

Nine companies and two universities have been registered in TE2 and there are four proposals after all and they have been evaluated on the common condition.

# Document list

- Summary document

  - JCTVC-C076 [TE coordinator] Summary of TE2 on IBDI and memory compression

- Proponent's documents

  - JCTVC-C093 [NEC] TE2: 1-D DPCM-based memory compression

  - JCTVC-C073 [Panasonic] TE2: Reference frame compression using image coder

  - JCTVC-C060 [TI] TE2: TI reference frame compression proposal

  - JCTVC-C075 [TOSHIBA] TE2: Adaptive scaling for bit depth compression on IBDI

- Related documents

  - JCTVC-C072 [Zhejiang Unv.] TE2: A memory bandwidth measurement algorithm for reference frame compression

  - JCTVC-C074 [Panasonic] Cross-check of memory compression results from NEC (JCTVC-B057/JCTVC-Cxxx)

  - JCTVC-C094 [NEC] An extension of DPCM-based memory compression to 2-D structure with ADPCM

  - JCTVC-C095 [NEC] Performance improvement of DPCM-based memory compression by adaptive quantization

  - JCTVC-C150 [Yonsei Unv.] Cross-check of memory compression results from TOSHIBA (JCTVC-C075)

# Proposed Tools

Four tools have been proposed.  Three tools are compression methods and one is a tool regarding test condition.

Table 1  Proposed tool

| Tool | Proponent | Method |
| --- | --- | --- |
| 1 | NEC | DPCM by 8-pixel |
| 2 | Panasonic | Hadamard transform and bit-plane coding |
| 3 | TI | DWT and DC prediction by 8x8-block |
| 4 | TOSHIBA | Adaptive scaling by 4x4-block and de-scaling by a pixel |

# Cross-verification

Proponents conducted source code exchange for each other.  Cross-checkers run the software independently and compared their results with proponent’s results.

Table 2  Cross checker

| Tool | Proponent | Cross-checker |
| --- | --- | --- |
| 1 | NEC | Panasonic, JVC |
| 2 | Panasonic | TOSHIBA |
| 3 | TI | TOSHIBA |
| 4 | TOSHIBA | NEC, Yonsei Unv. |

# Experimental results

All experiments for all sequences based on the common conditions, JCTVC-B302r2 [1] have been conducted.  On this common condition, the following evaluation criteria are defined.

1.	Measure impact on bitrate/PSNR using provided data. Use 4-point BD-PSNR and BD-Rate.

2.	Memory compression ratio with stored format and controlling method

3.	Memory access measures (Measurement module will be provided by some proponents)

4.	Complexity (encoding and decoding times)

5.	Subjective quality (informal comments)

## Coding efficiency

Since Table 3 and Table 4 show loss results from anchor condition, the smaller values mean better results.

These values are calculated by using 4-point BD-rate.

Note that these values of compression ratios are rough analyses and correct values will reported by each proponent.

Table 3  Summary of loss BD-bitrate (%) on high efficiency anchor

| Proponent | NEC |  | Panasonic |  | TI |  | TOSHIBA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rate | 12-bit to 8-bit |  | 12-bit to 5.33-bit |  | 12-bit to 8-bit |  | 12-bit to 8-bit |  |
| Condition | Random access | Low delay | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Class A | 0.7 | N/A | 1.7 | N/A | 0.2 | N/A | 0.2 | N/A |
| Class B | 0.9 | 1.8 | 2.1 | 2.9 | 0.2 | 1.0 | 0.2 | 0.6 |
| Class C | 0.9 | 1.5 | 3.9 | 5.3 | 0.3 | 1.0 | 0.1 | 0.4 |
| Class D | 1.0 | 1.3 | 3.5 | 4.3 | 0.3 | 0.6 | 0.1 | 0.4 |
| Class E | N/A | 9.6 | N/A | 7.6 | N/A | 5.4 | N/A | 5.5 |
| Total | 0.9 | 3.1 | 2.9 | 4.7 | 0.3 | 1.7 | 0.1 | 1.4 |

Table 4  Summary of loss BD-bitrate (%) on low complexity anchor

| Proponent | NEC |  | Panasonic |  | TI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rate | 8-bit to 6-bit |  | 8-bit to 5.33-bit |  | 8-bit to 4-bit |  |
| Condition | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Class A | 7.1 | N/A | 2.0 | N/A | 0.0 | N/A |
| Class B | 8.7 | 10.4 | 2.3 | 3.1 | 0.4 | 0.3 |
| Class C | 8.6 | 10.3 | 3.5 | 4.3 | 2.9 | 4.2 |
| Class D | 9.3 | 9.7 | 3.3 | 3.7 | 5.9 | 9.1 |
| Class E | N/A | 24.2 | N/A | 5.4 | N/A | 0.0 |
| Total | 8.6 | 12.8 | 2.9 | 3.0 | 2.5 | 3.4 |

## Complexity

These results of encoding and decoding time on Table 5and Table 6 have been conducted by each proponent independently.  Since the platform of each proponent is different, these results are informative absolutely.

Table 5  Increase rate (%) of average encoding and decoding times on high efficiency anchor

| Proponent | NEC |  | Panasonic |  | TI |  | TOSHIBA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Condition | Random access | Low delay | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Encoding Time | -2.6 | -0.4 | 3.0 | 1.9 | 8 | 6 | 0.32 | 0.47 |
| Decoding Time | 3.0 | 3.0 | 96.6 | 100.1 | 47 | 36 | 1.32 | 1.51 |

Table 6  Increase rate (%) of average encoding and decoding times on low complexity anchor

| Proponent | NEC |  | Panasonic |  | TI |  |
| --- | --- | --- | --- | --- | --- | --- |
| Condition | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Encoding Time | -1.9 | 1.2 | 1.9 | 0.3 | 3 | 7 |
| Decoding Time | 3.5 | 2.4 | 114.2 | 111.2 | 47 | 55 |

## Memory access

The issue of memory access has been discussed by participants of TE2.  As a result, the module of MC read memory bandwidth on decoder side has been provided by NEC.

Table 7  Compression rate (%) of MC read memory bandwidth on high efficiency anchor

|  | NEC |  | Panasonic |  | TI |  | TOSHIBA |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| 32bit/ 64bit | -24.7 | -24.9 | -44.5 | -44.8 | -14.7 | -14.8 | -26.9 | -27.4 |
| 64bit/128bit | -25.6 | -25.7 | -50.5 | -51.0 | -23.8 | -24.4 | -34.7 | -35.5 |
| 128bit/128bit | -27.1 | -27.2 | -54.3 | -55.1 | -29.8 | -30.8 | -39.8 | -40.9 |
| 256bit/256bit | -31.8 | -31.8 | -61.3 | -62.2 | -44.5 | --45.7 | -48.3 | -49.5 |

Table 8  Compression rate (%) of MC read memory bandwidth on low complexity anchor

|  | NEC |  | Panasonic |  |
| --- | --- | --- | --- | --- |
|  | Random access | Low delay | Random access | Low delay |
| 32bit/ 64bit | -12.3 | -10.5 | -25.9 | -25.5 |
| 64bit/128bit | -17.3 | -16.7 | -39.5 | -40.3 |
| 128bit/128bit | -21.4 | -20.8 | -46.1 | -47.0 |
| 256bit/256bit | -28.3 | -27.8 | -56.0 | -56.9 |

## Subjective quality (informal comments)

There were the following comments:

# Conclusion

In this tool experiment, we have finished to exam four proposals and verified their performance.

# Reference

[1] T. Chujoh, “Tool Experiment 2 on IBDI and memory compression,” JCT-VC Document, JCTVC-B302r2, Geneva, Julyl 2010.