| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C110 WG11 Number: m18133 |
| --- | --- |

| Title: | TE12.4: Transform unit quadtree tested against 2-level by Fraunhofer HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | Tool experiment result report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 622 benjamin.bross@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of testing the transform unit quadtree (RQT) against the 2-level representation within the scope of tool experiment 12 evaluating TMuC Tools [1]. The 2-level TU representation was tested against the RQT default. BD-rates from -0.2% up to 0.7% are reported. The encoder runtimes of the 2-level representation vary from 31% of the RQT encoder runtimes for intra only coding constraint set to 102% of the RQT encoder runtimes for random access. Coding constraint sets

- Simulation environment

## Software

The software used to run the simulations is TMuC 0.7 [2]. The binaries were compiled on a 64 bit Ubuntu 9.04 Linux using the included make files and gcc 4.3.3. The coding conditions used for the simulations are specified in [3]. These default settings, both for configuration file and compile-time, are used in this test as anchor. Changes from this anchor configuration are listed in the next sections.

### Alternative configuration file settings

No changes of configuration file settings are needed.

### Alternative compile-time settings

The following macros have to be disabled in TypeDef.h:

#define HHI_RQT 				0

#define HHI_RQT_CHROMA_CBF_MOD 	0

#define HHI_RQT_INTRA 		0

## Hardware

Three different types of computers have been used to run the simulations  all running a 64 bit Linux operating system. Their hardware is listed in  Table 1-1.

Table 1-1 Computers used to run the simulations.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 24 | Intel(R) Xeon(R) CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 9.04, 64 bit |
| 2 | 8 | Intel(R) Xeon(R) CPU X5460 @ 3.16GHz | 32 GB | Ubuntu 9.04, 64 bit |
| 3 | 8 | Intel(R) Xeon(R) CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 9.04, 64 bit |

# Results

The coding efficiency results in this section are given in terms of BD-rate [4][5]. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-C110.xls.

Furthermore relative encoding and decoding times in percentage of the anchor times are given. The numbers are averaged over all sequences. Due to the heterogeneous structure of the computer cluster consisting of three different computer types and non-constant workloads, these numbers can only be taken as a rough indication of the complexity.

The results are presented in three tables: Table 2-2, Table 2-3, and Table 2-4. Each table represents a test for a constraint set using the high efficiency (HE) and low complexity (LC) settings.

Table 2-2 Average BD-rates and relative encoding/decoding runtimes for Intra constraint set.

|  | Intra HE |  |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 4.3 | 3.7 |  | -0.2 | -1.0 | -3.7 |
| Class B | 0.7 | 3.6 | 3.7 |  | 0.4 | -0.2 | -1.9 |
| Class C | 0.9 | 3.1 | 2.6 |  | 1.3 | 0.0 | -0.6 |
| Class D | 0.5 | 3.1 | 2.5 |  | 1.1 | -0.2 | -0.5 |
| Class E | 0.8 | 7.7 | 5.3 |  | 0.0 | -7.1 | -4.8 |
| All | 0.7 | 4.1 | 3.4 |  | 0.6 | -1.4 | -2.0 |
| Enc Time[%] | 39% |  |  |  | 31% |  |  |
| Dec Time[%] | 97% |  |  |  | 96% |  |  |

Table 2-3 Average BD-rates and relative encoding/decoding runtimes for random access constraint set.

|  | Random Access HE |  |  | Random access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 6.8 | 3.6 | -1.0 | -2.3 | -4.1 |
| Class B | 0.4 | 4.1 | 3.0 | -0.6 | -2.5 | -3.2 |
| Class C | 0.2 | 2.3 | 1.7 | 0.4 | -1.7 | -2.1 |
| Class D | 0.3 | 3.2 | 2.4 | 0.2 | -1.9 | -1.7 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 3.8 | 2.6 | -0.2 | -2.1 | -2.6 |
| Enc Time[%] | 102% |  |  | 98% |  |  |
| Dec Time[%] | 110% |  |  | 104% |  |  |

Table 2-4 Average BD-rates and relative encoding/decoding runtimes for low delay constraint set.

|  | Low delay HE |  |  | Low delay LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 5.0 | 3.7 | 0.9 | -4.6 | -5.1 |
| Class C | -0.2 | 1.6 | 1.0 | 1.5 | -3.1 | -3.2 |
| Class D | -0.1 | 2.9 | 1.7 | 1.7 | -2.8 | -3.1 |
| Class E | 0.6 | 4.3 | 3.9 | -1.7 | -9.1 | -8.1 |
| All | 0.0 | 3.5 | 2.6 | 0.8 | -4.6 | -4.7 |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |

# References

- JCT-VC, “Tool Experiment 12: Evaluation of TMuC Tools”, JCTVC-B312, JCT-VC Meeting, Geneva, July 2010.

- JCT-VC, “Test Model under Consideration Software 0.7”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.7

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-B300, JCT-VC Meeting, Geneva, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.