| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010 | Document: JCTVC-C159 WG11 Number: m18185 |
| --- | --- |

| Title: | TE12: Cross verification of RDOQ |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kenneth Andersson Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46 10 7141538 kenneth r andersson @ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a cross-verification of the impact of turning off RDOQ [1] in TMuC 0.7 software as part of TE 12 [2]. The PSNR, bitrate and BD-rate matched DoCoMo’s simulations of low complexity configurations and RIM’s simulations of high efficiency configurations. The result shows a loss of 4.7%, 4.5% and 2.5% for luma of high efficiency configurations all intra, random access and low-delay. For low complexity configurations the loss for luma is smaller than for high efficiency configurations (1.9%, 3.2% and 0.8%). The encoding time is reduced to 69%, 92% and 92% for high efficiency configurations all intra, random access and low-delay and for low complexity configurations 74%, 87% and 89%, compared to TMuC 0.7. The decoding time varies between 99% and 106% compared to TMuC 0.7.

- Results

The results were obtained using TMuC 0.7 both for the anchor and the test of turning off RDOQ (RDOQ:0). The encoder and decoder were compiled on Linux using gcc 4.3.3. The simulations were performed on a Linux cluster using Intel processors of 2.9GHz.

A summary of the results is shown below. BD-rate is according to [3]. The PSNR, bitrate and BD-rate matched DoCoMo’s simulations of low complexity configurations and RIM’s simulations of high efficiency configurations. Encoding time and decoding time are ratios of time measures of RDOQ turned off and TMuC 0.7. Time is measured as user time.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4,7 | -0,8 | 1,7 | 1,9 | -2,2 | -1,0 |
| Class B | 5,7 | -1,0 | 1,2 | 2,5 | -3,2 | -2,4 |
| Class C | 4,6 | -1,8 | -0,9 | 1,9 | -3,1 | -2,8 |
| Class D | 4,3 | -2,1 | -1,2 | 1,7 | -2,8 | -2,6 |
| Class E | 3,6 | 4,9 | 1,0 | 1,3 | 0,3 | -1,1 |
| All | 4,7 | -0,4 | 0,2 | 1,9 | -2,4 | -2,2 |
| Enc Time[%] | 69% |  |  | 74% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3,3 | 7,1 | 9,6 | 1,9 | 8,3 | 10,0 |
| Class B | 5,0 | 7,2 | 9,0 | 2,9 | 9,8 | 10,2 |
| Class C | 4,5 | 8,2 | 8,7 | 3,4 | 10,6 | 10,7 |
| Class D | 4,3 | 7,0 | 7,1 | 3,9 | 11,6 | 11,4 |
| Class E |  |  |  |  |  |  |
| All | 4,4 | 7,4 | 8,5 | 3,2 | 10,3 | 10,6 |
| Enc Time[%] | 92% |  |  | 87% |  |  |
| Dec Time[%] | 102% |  |  | 106% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3,5 | 3,6 | 3,6 | 1,1 | 2,7 | 3,4 |
| Class C | 2,2 | 3,6 | 4,1 | 0,7 | 1,9 | 2,1 |
| Class D | 2,4 | 3,0 | 3,4 | 1,1 | 2,7 | 2,8 |
| Class E | 1,3 | 3,2 | -0,7 | 0,0 | 1,0 | -0,1 |
| All | 2,5 | 3,4 | 2,9 | 0,8 | 2,2 | 2,2 |
| Enc Time[%] | 92% |  |  | 89% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

The BD-rate shows some significant loss for both luma and chroma for most configurations except for low-delay low complexity very the loss is 0.8% for luma and for intra where there even is some gain for chroma when RDOQ is turned off. The full results can be found in attached excel sheet.

- References

- Marta Karczewicz, Yan Ye, Insuk Chong, “Rate Distortion Optimized Quantization”, ITU-T SG16/Q6, VCEG-AH21, Antalya, Turkey, January 2008.

- Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools”, Geneva, Switzerland, July 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.