| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J_Notes_d3 |
| --- | --- |

| Title: | Meeting report of the 10th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Stockholm, SE, 11-20 July 2012 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its tenth meeting during 11–20 July 2012 at the City Conference Centre (CCC), a.k.a. Norra Latin, in Stockholm, SE. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

A meeting of AHG 9 (high level syntax) was held on Tuesday 10 July 2012. Discussions and recommendations of this AHG are in included in section 5.12, where decisions on these issues were made by the JCT plenary.

The JCT-VC meeting sessions began at approximately 900 hours on Wednesday 11 July 2012. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1300 hours on Friday 20 July. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC).

The primary goals of the meeting were to review the work that was performed in the interim period since the ninth JCT-VC meeting in producing the 7th HEVC Test Model (HM7) software and text and editing the 7th HEVC specification Draft (which was issued as an ISO/IEC Study of Committee Draft document), review the results from an interim Core Experiment (CE), review technical input documents, establish the 8th draft of the HEVC specification (to be issued as an ISO/IEC Draft International Standard – DIS) and the eighth version of the HEVC Test Model (HM8), and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

The JCT-VC produced three particularly important output documents from the meeting: the HEVC Test Model 8 (HM8), the HEVC specification draft 8 a.k.a. Draft International Standard (DIS), and a document specifying common conditions and software reference configurations for HEVC coding experiments. Moreover, plans were established to conduct X future CEs in the interim period until the next meeting.

For the organization and planning of its future work, the JCT-VC established XX "Ad Hoc Groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 10–19 Oct 2012 under WG 11 auspices in Shanghai, CN, 14–23 January 2013 under ITU-T auspices in Geneva, CH, 17-26 April 2013 under WG 11 auspices in Incheon, KR, and XX July – 02 Aug 2013 under WG 11 auspices in Vienna, AT.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its tenth meeting during 11-20 July 2012 at the City Conference Centre (CCC), a.k.a. Norra Latin in Stockholm, SE. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

A meeting of AHG 9 (high level syntax) was held on Tuesday 10 July 2012, the day before the JCT-VC meeting started, at the same meeting site.

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Wednesday 11 July 2012. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately 1300 hours on Friday 20 July. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a co-located fashion with a meeting of ISO/IEC WG 11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC).

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2012_04_I_Geneva/.

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the ninth JCT-VC meeting in producing the 7th HEVC Test Model (HM) software and text and editing the 7th HEVC specification Working Draft (WD7), review the results from interim Core Experiments (CEs), review technical input documents, establish the 8th draft of the HEVC specification and the 8th version of the HEVC Test Model (HM8), and plan a new set of Core Experiments (CEs) for further investigation of proposed technology.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

Document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (rather than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a bug in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding … simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of contexts, number of context-coded bins, etc.) … "Decision (Compl.):"

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible in the interest of aiding study, although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 2 July 2012.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 3 July 2012 were considered "officially late".

Most documents in this category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave us a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-J0371 to JCTVC-J0XXX were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "J0370+" range include break-out activity reports that were generated during the meeting and are therefore considered report documents rather than late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with this issue was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical proposal contributions were registered in time but were uploaded late:

- JCTVC-J0XXX (a technical proposal) [uploaded XX-XX]

- ...

The following other documents not proposing normative technical content were registered in time but uploaded late:

- JCTVC-J0XXX (a contribution ...)

- ...

The following cross-verification reports were uploaded late: JCTVC-J0XXX, ... .

The following document registrations were later cancelled or otherwise never provided or never discussed due to lack of availability or registration errors: JCTVC-J0XXX, ... .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-J0XXX (a contribution of ... , corrected ...)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by the cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly the meeting report JCTVC-I1000, the HEVC Test Model (HM) JCTVC-I1002, the Draft Specification JCTVC-I1003, and the Draft Disposition of Comments JCTVC-I1004 were approved. The HM reference software produced by the AHG on software development and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chair asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it has previously been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/ SC 29/ WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/ SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/ SC 29/ WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the new HEVC standard and for evaluating proposals for technology to be included in this design. Additionally, after development of the coding technology, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use their real names when subscribing and sending messages and must respond to inquiries regarding their type of interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- AIS: Adaptive intra smoothing.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- AMVR: Adaptive motion vector resolution.

- APS: Adaptation parameter set.

- ARC: Adaptive resolution coding.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ET: Encoding time.

- GPB: Generalized P/B – a not-particularly-well-chosen name for B pictures in which the two reference picture lists are identical.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/ SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- PIPE: Probability interval partitioning entropy coding (roughly synonymous with V2V for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SPS: Sequence parameter set (as in AVC).

- TE: Tool Experiment – a coordinated experiment conducted between the 1st and 2nd or 2nd and 3rd JCT-VC meeting.

- TM: Test Model – a video coding design containing selected coding tools; as contrasted with the TMuC, see HM.

- TMuC: Test Model under Consideration – a video coding design containing selected proposed coding tools that are under study by the JCT-VC for potential inclusion in the HEVC standard.

- Unit types:

  - CTB: code tree block (synonymous with LCU or TB).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB or TB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TB: tree block (synonymous with LCU – LCU seems preferred).

  - TU: transform unit.

- V2V: variable-length to variable-length prefix coding (roughly synonymous with PIPE for most discussion purposes, although the term PIPE tends to be more closely associated with proposals from Fraunhofer HHI while the term V2V tends to be more closely associated with proposals from RIM).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

DIS ballot timing issue.

## Scheduling of discussions

Scheduling: Generally 0800 – 2200

Wednesday: 0900 start; Plenary to 1pm; Split tracks 1430 onwards; Track A ended 2000.

Mon morning not meet for MPEG plenary.

Wed morning not meet for MPEG plenary.

Fri 20 end by lunchtime.

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, and discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm.

- AHG reports (14) Track P (section 2)

- Project development, status, and guidance (3) Track P (section 3 and section 5.1)

- CE1: Intra transform mode dependency simplifications (8) Track B (section 4.1) Thu 3.

- Clarifications and bug fix issues (4) Track P (section 5.1)

- HM settings and common test conditions (0) Track P (section 5.3)

- HM coding performance (2) Track P (section 5.4)

- Profile/level definitions (16) Track P (section 5.5)

- Source video test material (1) Track P (section 5.6)

- Functionalities (11  3) Track A (section 5.7)

- Deblocking filter (16) Track B (section 5.8) Thu. 1. (revisits)

- Non-deblocking loop filters (68) Track B (section 5.9) (BOG ALF: Yamakage; SAO: Elena): Review Thu 2. (revisits)

- Block structures and partitioning (8) Track B (section 5.10) Wed 3.

- Motion and mode coding (36) Track B (section 5.11) Thu 4.

- High-level syntax and tile/slice structures (108) Track A (section 5.12)

- Quantization (14) Track B (section 5.13) Wed 1.

- Entropy coding (4) Track B (section 5.14) Thu 5.

- Transform coefficient coding (47) Track B (section ) Thu 6.

- Intra prediction and mode coding (7) Track B (section 5.16) Wed 2.

- Transforms (3) Track B (section 5.17) With CE1

- Memory bandwidth reduction (6  4) Track A (section 5.18)

- Alternative coding modes (41) Track A (section 5.19)

- Non-normative: Encoder optimization, post filtering (5) Track A (section 5.20)

NOTE – The number of contributions noted in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: 40; Track A: 171; Track B: 211.

Regarding CE, decide about subjective viewing later.

# AHG reports

The activities of ad hoc groups that had been established at the prior meeting are discussed in this section.

AHG1 [miss]

given verbally (business as usual, no issues of concern)

JCTVC-J0002 JCT-VC AHG report: HEVC Draft and Test Model editing (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs)]

The seventh High Efficiency Video Coding (HEVC) test model (HM7) was developed from the sixth HEVC test model (HM6), following the decisions taken at the 9th JCT-VC meeting in Geneva (27 April to 7 May 2012).

Two editorial teams were formed to work on the two documents that were to be produced:

JCTVC-I1002 HEVC Test Model 7 (HM 7) Encoder Description

- Il-Koo Kim

- Ken McCann

- Kazuo Sugimoto

- Benjamin Bross

- Woo-Jin Han

JCTVC-I1003 High Efficiency Video Coding (HEVC) text specification draft 7 [2]

- Benjamin Bross

- Woo-Jin Han

- Jens-Rainer Ohm

- Gary J. Sullivan

- Thomas Wiegand

Editing JCTVC-I1003 was assigned a higher priority than editing JCTVC-I1002.

An issue tracker (http://hevc.kw.bbc.co.uk/trac) was used in order to facilitate the reporting of issues on the text of both documents.

One version of JCTVC-I1002 and eight successive versions of JCTVC-I1003 were published by the Editing AHG following the 9th JCT-VC meeting in Geneva.

The main changes in JCTVC-I1002 and JCTVC-I1003, relative to the previous JCTVC- H1002, were listed in the report.

The recommendations of the HEVC Draft and Test Model Editing AHG were to:

- Approve the edited JCTVC-I1002 and JCTVC-I1003 documents as JCT-VC outputs

- Continue to edit both documents to ensure that all agreed elements of HEVC are fully described

- Encourage the use of the issue tracker (http://hevc.kw.bbc.co.uk/trac) to facilitate the reporting of issues with the text of either document

- Compare the HEVC documents with the HEVC software and resolve any discrepancies that may exist, in collaboration with the Software AHG

- Continue to improve the overall editorial quality of the HEVC draft text specification, to allow it to proceed to DIS ballot

- Ensure that properly drafted candidate text for both the HEVC draft text specification and the HM Test Model (if appropriate) is available prior to making any decision to change the HEVC specification

The last item above was particularly noted, as a "ratcheting up" of the need for stability and focus on getting things complete, coherent and finalized.

The AHG recommends that text should be provided and approved before making a decision for inclusion – in the previous period some text was arriving quite late in some cases. Confirmed by group consensus.

JCTVC-J0003 JCT-VC AHG report: Software development and HM software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Sühring]

A brief summary of activities related to each mandate is given below.

1. Development of the software was coordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was made available through the SVN servers set up at HHI and the BBC, as announced on the JCT-VC email reflector.

2. Version 7.0 of the software was delivered to schedule and reference configuration encodings were provided according to the common test conditions through an ftp site at the BBC.

ftp://ftp.kw.bbc.co.uk/hevc/hm-7.0-anchors/

3. Version 7.1 of the software was delivered ahead of the 10th JCT-VC meeting.

4. Some high-level adoptions were still outstanding at the time of writing.

Multiple versions of the HM software had been produced and announced on the JCT-VC email reflector. The changes made for each version were summarized in the report. A detailed history of all changes made to the software can be viewed at http://hevc.kw.bbc.co.uk/trac/timeline.

Released versions of the software are available on the SVN server at the following URL:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,

where version_number corresponds to one of the versions described below (eg., HM-7.0). Intermediate code submissions can be found on a variety of branches available at:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,

where branch_name corresponds to a branch (eg., HM-7.0-dev).

Version 7.0 of the software was released on 23rd May 2012. It includes all the changes adopted at the 9th JCT-VC meeting that affect the common test conditions. This release was announced on the email reflector.

The performance change since HM-6.0 was tabulated in the report.

Some gain was achieved from adding AMP, but a number of small losses tended to offset that. Class F AI HE10 showed substantial gain, primarily due to adding the transform skip feature.

Version 7.1 of the software was released on 30th June 2012. It contains a number of bug fixes and the majority of adoptions form the 9th JCT-VC meeting that do not affect the common conditions. A number of integrations were still outstanding at the point of writing. There is virtually no performance change between HM-7.0 and HM-7.1 under the common conditions.

Version 7.2 was planned for release during the current meeting.

In addition to the regular HM development process, one branch was created to expose tools to a wider audience:

- HM-7.1-dev-ahg13, which contains contains modifications to the reference picture buffers and list construction.

Recommendations of the AHG were as follows:

- Continue to develop reference software based on HM version 7.1 and improve its quality.

- Remove macros introduced inHM previous HMversions before startingintegration towards HM8.0 such as to make the software more readable

- Continue to identify bugs and discrepancies with text, and address them

- Test reference software more extensively outside of common test conditions

Several simplifications (restriction of motion comp in small PUs, SAO, etc.) added up in losses of up to 1%; AMP compensates for that such that the performance of HM7 is only very slightly inferior than HM6. The approx. 1% loss appears in HE10, where AMP was already switched on before (except class F, where transform skip which was adopted provides additional gain (4% rate reduction of HM7 vs. HM6).

JCTVC-J0004 JCT-VC AHG report: High-level parallelism (AHG4) [M. Horowitz (eBrisk), M. Coban (Qualcomm), F. Henry (Orange Labs), K. Kazui (Fujitsu), A. Segall (Sharp Labs), W. Wan (Broadcom), S. Wenger (Vidyo), M. Zhou (TI)]

There was no significant email activity for this AHG.

The chairs of AHG4 were unaware of any high-level parallel processing related open issues at this time.

The relevant changes made to the HM were reviewed.

The 22 related input documents to the Stockholm meeting were listed and categorized into five categories:

- General (not specific to tiles or WPP)

- Tiles

- WPP (wavefront parallel processing)

- Entry points

- P&L (profile and level)

JCTVC-J0005 JCT-VC AHG report: Entropy Coding Improvements (AHG 5) [A. Segall (chair), C. Auyeung, K. Chono, G. Martin-Cocher, T. Nguyen, J. Sole, V. Sze, W. Wan]

There were approximately 28 e-mail messages exchanged on the reflector.  Messages were primarily related toward discussing and recommending (i) test conditions, (ii) software and (iii) reporting methods.  Five of these messages included both AHG5 and AHG6 activities.

Recommendations made by the AHG were reviewed in the report, and included

- Test conditions related to cu_qp_delta

- Reporting methods for contributions considering the case of context bin reduction

- The use of a software patch and Excel spreadsheet for collecting the necessary data for the above.

- Recommending that only the worst case number of context and bypass coded bins be mandatory for reporting in related contributions.

The 38 input contributions related to the AhG activities were listed in the report. They were categorized into the following subjects:

- Delta QP (4)

- Reference Index (3)

- SAO (9)

- Transform level coding (4)

- Other (4)

- Cross checks (14)

Note that JCTVC-J0194 is categorized in the “Other” sub-category but also related to Delta QP, Reference Index and SAO sub-categories.

Overview of characteristics of delta QP contributions:

|  | HM7.0 | JCTVC-J0060 | JCTVC-J0226 | JCTVC-J0089 |  | JCTVC-J0298 |
| --- | --- | --- | --- | --- | --- | --- |
| Binarization | TU | EG0 | TU+EG0 |  | TU+EG0 | TU+FLC |
| Max_bin | 27 | 11 | 15 |  | 15 | 16 |
| Max_ctx_bin | 26 | 1 | Method1: 5 Method2: 3 Method3: 2 Method4: 1 |  | 5 | 4 |
| Max_bypass_bin | 1 | 10 | Method1: 10 Method2: 12 Method3: 13 Method4: 14 |  | 10 | 12 |
| Interleaving of context bins and bypass bins | Yes | No | No |  | No | No |
| Max Average BD rate changes (%) (Y,U,V) Class F is excluded |  | 0.20/0.15/0.55 (Y/U/V) | Method 1 0.03/0.10/0.34 (Y/U/V) Method 2 0.06/0.11/0.09 (Y/U/V) Method 3 0.06/0.20/0.26 (Y/U/V) Method 4 0.09/0.20/0.33 (Y/U/V) |  | 0.03/0.09/0.14 (Y/U/V) | 0.02/0.11/0.24 (Y/U/V) |

Overview of characteristics of reference index related contributions:

|  | HM7.0 | JCTVC-J0098 |  | JCTVC-J0176 | JCTVC-J0297 |
| --- | --- | --- | --- | --- | --- |
| Binarization | TU | Method1 TU | Method2 TU+EG0 | TU | TU+FLC |
| Max_bin | 15 | 15 | 11 | 15 | 8 |
| Max_ctx_bin | 15 | 2 | 2 | 4 | 4 |
| Max_bypass_bin | 0 | 13 | 9 | 11 | 4 |
| Max Average BD rate changes (Y,U,V) Class F is excluded |  | 0.01%(Y) 0.13%(U) 0.13%(V) |  | 0.01%(Y) 0.13%(U) 0.13%(V) | 0.00%(Y) 0.00%(U) 0.00%(V) |

JCTVC-J0006 JCT-VC AHG report: In-loop filtering (AHG6) [T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke, A. Norkin, P. Onno (vice-chairs)]

There were about 40 email exchanges for ALF on the JCT-VC main reflector. Most of the emails were for decoding time discussion for ALF. About 10 emails for SAO were exchanged related to merge_up_flag and frame-based SAO parameter optimization.  In response to the discussion for merge_up_flag, there are several contributions for this topic.

In addition, HM/WD tickets are reported and most of the tickets have been solved.

The relevant ticket issues and input contributions were listed in the report.

The AHG recommended to study all input contributions and to create a BoG for DF, SAO and ALF.

For ALF, the AHG recommended to conduct an informal subjective picture quality viewing.

BoG to be run

JCTVC-J0007 JCTVC AHG Report: Memory bandwidth restrictions in motion compensation (AHG7) [T. Suzuki (chair), W. Wan, M. Zhou (vice-chairs)]

Six contributions were noted to be relevant. The relevant technical proposes were noted as follows:

- On bi-predictive motion vectors for inter PUs of 8x4 and 4x8, JCTVC-J0086 and JCTVC-J0312 propose to disable to encode bi-pred MV for 8x4 and 4x8 by changing CABAC. In the current spec, it is allowed to send such MV, but decoder discards it.

- On constraints on high resolution and high frame-rate application, JCTVC-J0175 proposes to constrain bi-pred 8x8 for large picture only (e.g. 4K). Since such constraint is not necessary for HDTV, this contribution proposes to change the constraint depending on the level (picture size).

- On bi-pred merge candidate derivation, JCTVC-J0218 proposes to restrict the merge MV during merge candidate derivation.

JCTVC-J0008 JCTVC AHG Report: Loss robustness (AHG8) [Arturo Rodriguez (Chair)]

No activity was reported

JCTVC-J0009 JCT-VC AHG report: High-level syntax (AHG9) [G. J. Sullivan (AHG meeting co-chair), Y.-K. Wang (AHG chair and AHG meeting co-chair), J. Boyce, Y. Chen, M. M. Hannuksela, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan, W. Wan, P. Wu (AHG vice chairs)]

This AHG report was reviewed in Track A after lunch on Wed.

There were some email discussions relating to this AHG on the following topics:

- Constraint on number of bits per coding tree block

- Usefulness of restricted_ref_pic_lists_flag and related

- Slice granularity and end_of_slice_flag

There are input documents to this meeting addressing the issues related to the first topics.

Related contributions were listed in the AHG report.

The AHG held a face-to-face meeting from 0900‒1800 on Tuesday 10 July 2012 at the Conference Center venue of 10th JCT-VC meeting that was to begin on the following day. The AHG meeting was chaired by Ye-Kui Wang and Gary Sullivan. Meeting minutes and AHG recommendations made during the face-to-face meeting are also included in the report.

In the time available in the AHG meeting, the AHG reviewed 12 contributions in the following three topic areas:

- NAL unit header (4 contributions)

- Picture order count (3 contributions)

- Slices (5 contributions)

The notes reported by the AHG on these contributions were used as the starting basis of the notes recorded in this report.

During the review of this AHG report, a participant remarked that we should consider imposing some restriction on changes of the APS within a picture.

JCTVC-J0010 JCT-VC AHG report: Hooks for scalable coding (AHG10) [J. Boyce, J. Kang, J. Samuelsson, W. Wan, Y. K. Wang]

No particular email reflector discussion was reported. There were 17 contributions noted as relevant, categorized into the following areas.

- NUH

- VPS

- SPS

- MV coding

- VUI

- SEI

- Layer switching

JCTVC-J0011 JCT-VC AHG report: Lossless Coding (AHG11) [Wen Gao (chair), Keiichi Chono, Felix Henry, Jizheng Xu, Minhua Zhou, Pankaj Topiwala (vice chairs)]

During the interim period between 9th and 10th JCT-VC meeting, the adopted lossless coding related contributions had been integrated into HM7.1 software and HEVC specification text (JCTVC-I1003). During the integration process, An HM Ticket, #580, reported an encoder/decoder mismatch for lossless coding under LB configurations. The bug was fixed in HM7.0-dev-r2461.

During the AHG11 discussions on JCT-VC reflector, it was suggested to use HM7.0-dev-r2461 as the reference software since it was not clear whether HM7.1 would be released in time for lossless coding related simulations. It was also noted that HM7.0-dev-r2461 only has code on frame-level lossless coding.

Furthermore, the following test conditions and test scenarios are discussed and agreed on JCT-VC reflector, listed as follows:

Test Conditions: AI-main, LB-main, and RA-main, with following setting on lossless coding

- LosslessCuEnabled set to 0

- TransquantBypassEnableFlag set to 1

- CUTransquantBypassFlagValue set to 1

Test Scenarios:

- Frame level Lossless coding: All frames are lossless coded.

  - QP setting: QP = 0

  - Class A-F sequences

- Region based lossless coding: In each frame, three regions with the following top-left and bottom right coordinates are lossless coded.

  - Region 1: (272, 32) , (1119, 143)

  - Region 2: (208, 352), (495, 623)

  - Region 3: (1152,  192+16*floor(POC/100)),  (1231, 223+16*floor(POC/100))

- Two F-class 720p sequences to be tested: SlideShow and SlideEditing.

- QP setting: QP=22, 27, 32, 37

There were 10 contributions identified as related to AHG11, together with two cross verification reports.

AHG12

Ten relevant documents were noted to have been contributed, covering the following areas:

- Non-8-bit coding (1 contribution)

- Non-4:2:0 coding by extension of current design (4 contributions)

- Non-4:2:0 coding using new methods (3 contributions)

- Deriving 4:2:0 from 4:4:4 (1 contribution)

- Test sequences (1 contribution)

It was recommended to present the identified documents and seek to define a timeline for range extensions.

In discussion of the AHG report, the primary timeline expectation seemed to be 1 year beyond version 1.

- Timeline for range extensions to be specified

- How many profiles would be useful?

JCTVC-J0013 JCT-VC AHG report: Reference picture buffering and list construction (AHG13) [R. Sjöberg, Y. Chen, Hendry, T.K. Tan, Y.-K. Wang]

The test recommendation for reference picture buffering and list construction proposals was discussed on the reflector. It was decided to include reference picture duplication into test case 2.8 and remove all references to the combined list as that was taken out from the draft standard at the previous JCT-VC meeting. The document was sent out for review on June 18 on the main reflector. No comments were received and that document was uploaded as JCTVC-I0608.

It was noted that JCTVC-I0608 was not uploaded until after the previous meeting had ended, although the document was uploaded as an input to that meeting because of the way the web site was functioning at the time it was uploaded. It was suggested for that to also be provided as a new input to the current meeting so that it will be more appropriately categorized as being input to the current meeting.

The RPS bit cost measurements were reported to show the average percentage of related syntax bits that are spent on RPS related syntax. This was in the range of 0.0–0.7%.

Twelve relevant proposal contributions were identified in the report.

JCTVC-J0014 JCTVC AHG Report: Study on HEVC conformance requirements (AHG14) [T. Suzuki, W. Wan]

There was no discussion on the reflector, and no relevant contributions were submitted.

The AHG chairs raise the following questions on the reflector to initiate discussions.

- Whether to define HEVC conformance similar to the past standards

- Test methodology: The followings are defined for AVC:

  - dynamic test: to confirm decoder can decode in real time

  - static test: to check the decoded picture is perfectly matched with HM output

- What kind of bitstreams should be generated.

- The need for a plan to develop the conformance spec.

It was noted that the development of bitstreams eventually used for the prior conformance test set used for AVC was begun with bitstream exchange activity.

Design and exchange of bitstreams should be started after this meeting.

# Project development, status, and guidance

## Conformance test set development

JCTVC-J0291 Instructive (and sometimes evil) conformance bitstreams [C. Fogg (Harmonic), A. Wells (Ambarella)]

## Draft text specification improvements [the right place for this?]

# Core experiments

## CE1: Intra transform mode dependency simplifications

### Summary

JCTVC-J0021 CE1: Summary report of Core Experiment on intra transform mode dependency simplifications [K. Ugur, A. Saxena (CE coordinators)]

Three non-CE contributions were also noted to be relevant. These are listed in section 5.17.

In this core experiment, two simplifications were tested. Simplification 1 uses 2D DST for all intra prediction modes of 4x4 luma TUs rather than using mixed transform types (4 difference cases). Simplification 2 uses DST for all intra prediction modes of 4x4 luma TUs except that the DC mode is coded with DCT. Both simplifications show coding efficiency loss ranging between 0.0–0.1% on average excluding class F. In class F there was some more loss – between 0.2‒0.8% on average.

Visual testing was planned to be done for simplification 1 only.

It is intended to do visual testing

The results of visual testing were as follows:

- Testing performed with 14 participants, mainly CE1 participants

- Tests indicate that there is no visual difference

(BoG report about the tests will be provided).

Decision: Adopt simplification 1

Due to this adoption, there was no need to discuss the remaining documents under CE1

### Contributions

JCTVC-J0030 CE1: Cross-verification of Intra transform mode dependency simplifications (JCTVC-J0021) [R. Cohen (MERL)]

JCTVC-J0034 CE1: Cross-check of Intra transform mode dependency simplifications  [A. Saxena, E. Alshina, F. Fernandes (Samsung)]

JCTVC-J0035 CE1: Nokia’s results on intra transform mode dependency simplifications [K. Ugur, O. Bici (Nokia)]

This contribution presents the CE1 results by Nokia on intra transform mode dependency simplifications. In this core experiment, two simplifications were tested. Simplification 1 uses 2D DST for all intra prediction modes of 4x4 luma TU’s. Simplification 2 uses 2D DST for all intra prediction modes of 4x4 luma TU’s except the DC mode is coded with 2D DCT. Both simplifications show coding efficiency loss ranging between 0.0%–0.1%.

BR increase is 0.5% in class F (0.8% for low QP) for simplification 1, 0.3% for simplification 2.

JCTVC-J0276 CE1: Crosscheck of Nokia’s results on intra transform mode dependency simplifications (JCTVC-J0035) for low QPs [R. Joshi (Qualcomm)]

JCTVC-J0129 CE1: Cross-check of mode-dependent transform simplifications [C. Yeo, Y. H. Tan (I2R)]

An additional variant is tested where DST is used for all 4x4 TUs for both luma and chroma and for both inter and intra. This was not tested visually, and was not advocated for adoption. It was remarked that the 4x4 transform is a subset of the larger transforms anyway.

JCTVC-J0388 Cross-check of simplification 3 of JCTVC-J0129 [K. Ugur (Nokia)] [late]

JCTVC-J0243 CE1: Cross-check of intra transform mode dependency simplifications [J. Xu (Microsoft)]

# Non-CE Technical Contributions

## HEVC Standard Development

### Technical suggestions

JCTVC-J0292 Suggested figures for HEVC specification [C. Fogg (Harmonic)]

JCTVC-J0293 Lumpy Intra frames in HEVC [C. Fogg (Harmonic)]

## Clarification and Bug Fix Issues

JCTVC-J0335 Explicit Specification of Bounds on Syntax Elements [B. Heng, T. Hellman, W. Wan (Broadcom)]

JCTVC-J0133 raises the same/similar issue

JCTVC-J0225 is also related

JCTVC-J0336 Clarification of the semantics of no_residual_data_flag [Z. Yang, P. Chen, W. Wan (Broadcom)]

## HM settings and common test conditions

No contributions were noted to be specific to this subject area.

## HM coding performance

JCTVC-J0128 On software complexity: decoding 720p content on a tablet [F. Bossen (Docomo Innovations)]

JCTVC-J0236 Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile [J. Xu (Microsoft)]

## Profile/level definitions (for version 1 of HEVC)

### NB comments

JCTVC-J0437 UKNB Comment on HEVC Profiles	[UK National Body] [late]

JCTVC-J0477 JNB comments on HEVC extensions to support non-4:2:0, n-bit video [Japan National Body]

JCTVC-J0478 JNB comments on UHDTV support in HEVC [Japan National Body]

### Main Profile

JCTVC-J0125 On profiling [Y.-K. Wang, H. Reddy (Qualcomm), A. Luthra (Motorola), L. Winger (Magnum)]

JCTVC-J0151 Study on HEVC profiles and levels [T. Suzuki, M. Haque, A. Tabatabai (Sony)]

JCTVC-J0264 Dependent slices support in HEVC main profile [T. Schierl, V. George, A. Henkel, D. Marpe (HHI)]

JCTVC-J0289 On parallel partitions in profiles [A. Wells (Ambarella), C. Fogg (Harmonic)]

JCTVC-J0307 Inclusion of adaptive loop filtering (ALF) in Main profile of the HEVC standard [T. Yamakage (Toshiba), Y.-W. Huang (MediaTek), I. S. Chong (Qualcomm)]

JCTVC-J0330 Inclusion of ALF in Main profile and additional test results [T. Ikai, Y. Yasugi, T. Yamamoto, T. Tsukuba, T. Aono (Sharp

JCTVC-J0338 Specification of Parallelization Tools in HEVC Version 1 [W. Wan, T. Hellman (Broadcom)]

JCTVC-J0282 AHG4: On mandatory parallelism [S. Worrall, A. Wise (Aspex), J. Viéron (Ateme), P. Andrivon, P. Bordes (Technicolor)]

JCTVC-J0284 Tiles and Wavefront Parallel Processing Restrictions For The Main Profile [S. Worrall, A. Wise]

JCTVC-J0147 Subjective evaluation on ALF [J. Takiue, T.K. Tan, A. Fujibayashi, Y. Suzuki (NTT Docomo)]

### Levels

Note: Potentially also consider J0225, J0335 here.

JCTVC-J0056 Additional level for coding high resolution pictures [K. Ugur, J. Lainema, M. Hannuksela (Nokia)]

JCTVC-J0154 Proposal for definition of levels in HEVC [Mukta Kar, Arianne Hinds, Yasser Syed, Arturo Rodriguez, Wade Wan, Ajay Luthra, Lowell Winger, Clint Sheridan, Paul Haskell]

JCTVC-J0334 Adding a Level Restriction on Coding Tree Block Size [W. Wan, T. Hellman (Broadcom)]

### Beyond Main Profile

JCTVC-J0037 On still picture profile [K. Ugur, J. Lainema, M. Hannuksela (Nokia)]

JCTVC-J0152 Proposals on HEVC professional extensions [T. Suzuki, K. Sharman, N. Saunders, A. Tabatabai (Sony)]

## Source video test material

JCTVC-J0197 4:4:4 test sequence [K. Sugimoto (Mitsubishi)] [late]

## Functionalities (11  3)

### Colour component sampling and higher bit-depth (9  1)

[After coffee break Fri.]

JCTVC-J0078 AHG12: Non-4:2:0 formats syntax modifications [P. Andrivon, P. Bordes (Technicolor)]

This contribution presents syntax modifications to Draft 7 (JCTVC-I1003_d2) of HEVC in order to prepare the support of non-4:2:0 formats in HEVC profiles. It is reported that proposed Draft 7 syntax modifications are twofold: adaptation of syntax to support non-4:2:0 chroma subsampling formats and simplifications by parsing only chroma syntax elements that are necessary in the decoding process.

This contribution discusses high-level syntax only. It describes adjustments to syntax – mostly as done for AVC.

It was remarked that the separate colour plane mode has not been so popular, although it is reportedly used in some applications – e.g. military. It was remarked that now that we have other parallelism tools that may make this less necessary.

Editorially, it is desirable to go ahead and put support for 4:2:2 and 4:4:4 into the HLS in the draft – in pathways that are not actually exercised by currently-allowed syntax element values – but not the separate colour plane flag. This was agreed.

JCTVC-J0079 AHG12: On beyond 8 bit-depth support in HEVC [P. Andrivon, P. Bordes (Technicolor)]

This contribution reports an analysis of HEVC text Draft 7 d1 as well as experiments results of Test Model 7 (HM7.0 and HM7.1rc1) with regards to beyond 8 bit-depth signals coding support, namely 10, 12 and 14-bit. It is reported that no major issues have been identified for 8, 10 and 12 bit-depths for both HEVC Draft 7 d4 and HM7.0 (RA-HE and AI-HE). It is claimed that several issues appeared for 14 bit-depth and fixes were proposed to increase 14 bit-depth support. 14-bit coding reportedly shows consistent results with 8, 10 and 12 bit-depth coding with proposed global patch. It is stated that all software patches were integrated in HM-7.1-dev and all text modifications are present in HEVC Draft 7 d6. Besides, it is claimed that HM7.0 picture-level lossless coding for 10, 12, 14 bits reconstruct perfectly pictures. Finally, it is suggested that JCT-VC should define beyond 8 bit-depth time-line in HEVC for professional extensions candidates and UHDTV.

No action needed – current text and software seem to support bit depth appropriately.

JCTVC-J0191 Extension of HM7 to Support Additional Chroma Formats [P. Silcock, K. Sharman, N. Saunders, J. Gamei (Sony)]

In this proposal, a model based upon HM7.0 that provides support for 4:2:2, 4:4:4 and 4:0:0 chroma formats is presented. In this model, 4:2:0 coding is also supported and output files for 4:2:0 are reported to match those provided by HM7.0 for the 8 standard test configurations and other non-standard configurations, with encoding/decoding times similar to HM7.0. The changes that have been made to extend HM7.0 to support 4:2:2/4:4:4 are described.

It was noted that there was a prior contribution I0521.

Like I0521, this contribution used non-square transforms.

Compression test results were provided in relation to JM 18.3.

The software was submitted with the contribution.

It was suggested to set up an AHG and have work done to develop an HM software branch to be merged e.g. by the next meeting.

The contribution reviewed the various aspects of the design that required adjustment, and explored multiple approaches to these. Some examples included:

- Motion compensation interpolation filtering

- Angle adjustment for intra prediction

- Transform gain scaling

The importance of test material availability was also noted.

JCTVC-J0357 AHG12: 4:2:2/4:4:4 chroma format extension for HEVC Version 2 [K. Kawamura, T. Yoshino, S. Naito (KDDI Corp.)]

This contribution proposes an extension scheme for supporting 4:2:2/4:4:4 chroma formats that is part of HEVC Version 2. The support is reportedly achieved by minimum changes from the current specification. The modified HM7.0 software, in which all video coding tools support extended chroma formats, is provided.

Similar to J0191 in spirit. Intra prediction was approached somewhat differently. RDO was adjusted in the intra handling design to account for chroma in addition to luma.

Compression test results were provided in relation to JM 18.3.

See additional notes in discussion of J0191.

JCTVC-J0358 Chroma intra prediction based on residual luma samples in 4:2:2 chroma format [K. Kawamura, T. Yoshino, H. Kato, S. Naito (KDDI Corp.)]

This contribution presents an additional chroma intra mode based on inter-channel correlation of residual samples for the 4:2:2 chroma format. Predicted Cb/Cr values are sum of regular prediction (same as DM) and linear equation using reconstructed luma-residual values with a parameter alpha. The parameter alpha is derived and coded on the encoder side. Anchor method is a modified HM7.0 that supports the 4:2:2 chroma format described in JCTVC-J0358. Compared to the modified HM7.0, the average BD-bitrate gains are 0.7%, 3.4%, 2.2%, and 1.4% for all intra HE configuration respectively for Y, U, V, and YUV components.

The design is somewhat different – using residual rather than reconstruction – than our current LM chroma prediction scheme.

Treated as an information document.

JCTVC-J0233 Syntax and semantics of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content [Tao Lin, Peijun Zhang, Shuhui Wang, Kailun Zhou, Xianyi Chen]

This contribution presents the syntax and semantics of dual-coder mixed chroma-sampling-rate (DMC) coding for full-chroma (YUV444) screen content. The proposed DMC coding adds a full-chroma dictionary-entropy coder to the existing chroma-subsampled (YUV420) HEVC coder. The existing YUV420 HEVC syntax and semantics can be used without alteration. Three new syntax elements (matching_string_distance, matching_string_length_minus2, unmatchable_sample_residual) are added to enable YUV444 dictionary-entropy coding.

Prior contributions included JCTVC-H0065, JCTVC-H0073, JCTVC-I0272.

The contributor indicated a plan to integrate the DMC scheme into the HM and provide experiment results in the future. Hard-edged graphics areas (such as text regions) seemed to use the dictionary coding technology.

JCTVC-J0352 BD-rate performance vs. dictionary size and hash-table memory size in Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content [Peijun Zhang, Tao Lin, Xianyi Chen, Shuhui Wang, Kailun Zhou] [late]

This contribution presents BD-rate performance comparison of dual-coder mixed chroma-sampling-rate (DMC) coding for full-chroma (YUV444) screen content using different dictionary (as part of DPB) sizes from 4 MB to 16 KB and hash-table memory size from 16 MB to 10.5 KB. Some comparisons of results for the scheme relative to 4:2:0 coding with HEVC.

JCTVC-J0353 R-D cost based effectiveness analysis of Dual-coder Mixed Chroma-sampling-rate (DMC) coding for 4:4:4 screen content [Xianyi Chen, Tao Lin, Peijun Zhang, Shuhui Wang, Kailun Zhou] [late]

This contribution presents R-D cost based effectiveness analysis of dual-coder mixed chroma-sampling-rate (DMC) coding for full-chroma (YUV444) screen content. The DMC coding technique codes a CU using a dictionary-entropy coder and a hybrid coder simultaneously and calculating two R-D costs Jdict and Jhybrid. The coder with smaller R-D cost is selected as the optimal coder to code the CU. For a given screen picture, to look at and understand the overall coder selection distribution across all CUs in the picture, a ratio Jhybrid/Jdict map can be plotted to visualize the coder selection distribution and to evaluate how effective the two coders are. The ratio distribution maps reportedly reveal that the two coders are complementary and play very different roles to compress effectively different contents.

Further study is encouraged, particularly in relation to 4:4:4 extension of HEVC.

JCTVC-J0127 Integer Color Transforms and Resampling Filters for HEVC Applications [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

TBP.

### Interlaced (2)

JCTVC-J0258 Interlaced coding performance and Chroma consideration [Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse (Ateme)]

TBP.

JCTVC-J0466 Performance of HEVC for Interlaced Video [A. Luthra, D. Baylon (Motorola Mobility)] [late]

TBP.

## Deblocking filter

JCTVC-J0066 On Cross-Slice Deblocking Edge Ordering [P. Kapsenberg (Intel)]

This contribution claims that the deblocking edge processing order, which is specified to be vertical edges first followed by horizontal edges for the whole picture, requires slice header data from potentially many previous slices to be saved in order to successfully deblock a slice. A change is proposed that mandates that the relevant slice header syntax elements be the same for all slices in a picture.

One expert points out that instead of storing the offset data and QP values at each slice boundary, an implementation might potentially store the final computed offset values.

Storage on LCU basis: would be 10 bit per LCU

The reduction in memory is not large – we would rather retain the flexibility that slice-wise adaptation provides.

No action.

JCTVC-J0090 AHG6: Transform Dependent Deblocking [G. Van der Auwera, R. Joshi, M. Karczewicz (Qualcomm)]

Visual quality evaluations were asserted to have demonstrated that certain video content suffers from severe blocking artifacts that are transform size dependent and are particularly visible for the maximum transform size, as specified in the SPS syntax. This maximum TU size is encoder implementation dependent and it is asserted that this makes it important for the HEVC deblocking filter process to be flexible and general enough to reduce this type of blocking artifacts. This contribution proposes to signal specific deblocking adjustment parameters to control the deblocking strength in cases where at least one of two adjacent video blocks P and Q is included in a TU of maximum size. The advantage of this method is asserted to be not only applying stronger deblocking filtering to the edges of the TUs of maximum size, but also avoiding oversmoothing in picture areas that are unaffected by these largest blocking artefacts. This contribution also proposes a stronger β threshold curve with increased β values outside of the QP range of the common test conditions (>41), but within the QP range of the current HM7 curve. The advantage is that the deblocking strength can be further increased beyond the HM7 strength without increasing the number of values to be stored in memory. The experimental results report on the maximum TU size deblocking method with the HM7 β threshold curve and the proposed curve. Visual quality examples illustrate the reduction of the blocking artifacts originating from the maximum TU sizes.

One expert points out that the problem of storing the adaptation parameters for each slice (J0066) would become more serious through this (this would also apply to other contributions suggesting more adaptation capabilities).

JCTVC-J0418 AHG6: Cross-check of JCTVC-J0090 Transform Dependent Deblocking Strength [Shuo Lu (Sony)] [late]

JCTVC-J0096 Suppression of blocking artifacts at 32x32 transform boundaries [D.-K. Kwon, M. Budagavi (TI)]

This contribution reports that blocking artifacts at 32x32 transform boundaries remain even after deblocking for some video sequences outside of common conditions test sequences. It is reported that these blocking artifacts at 32x32 transform boundaries can be suppressed by increasing beta_offset_div2 and tc_offset_div2 signaled in PPS or slice header, but at the cost of BD-rate degradation. When compared to HM-7.0 anchor (i.e. beta_offset_div2 = tc_offset_div2 = 0), setting beta_offset_div2 and tc_offset_div2 to 13 is reported to result in average BD-rate degradation in the range of 12.7% to 18.8% for Main and HE10 common conditions. (AI-Main: 12.7%, RA-Main: 13.3%, LB-Main: 17.7%, LP-Main: 15.3%, AI-HE10: 12.9%, RA-HE10: 13.8%, LB-HE10: 18.8%, and LP-HE10: 17.7%).

In this contribution, it is proposed to signal in PPS or slice header new syntax elements tu32_beta_offset_div2 and tu32_tc_offset_div2 that control beta and tc offsets for 32x32 transform boundaries. It is asserted that the proposed signaling of tu32_beta_offset_div2 and tu32_tc_offset_div2 smoothes 32x32 transform boundaries while reducing BD-rate degradation. When compared to HM-7.0 anchor, setting tu32_beta_offset_div2 and tu32_tc_offset_div2 to 13 results in BD-rate degradation in the range of 1.2% to 2.0% for Main and HE10 common conditions (AI-Main: 1.3%, RA-Main: 1.2%, LB-Main: 1.8%, LP-Main: 1.3%, AI-HE10: 1.5%, RA-HE10: 1.4%, LB-HE10: 2.0%, and LP-HE10: 1.7%).

An alternative approach, which is same as a previous proposal JCTVC-I0244, is also proposed. This method increases bS value for 32x32 transform boundaries so that strong loop filter could be applied more frequently. When compared to HM-7.0 anchor, this method results in BD-rate degradation in the range of 0.1% to 1.0% for Main and HE10 common conditions (AI-Main: 1.0%, RA-Main: 0.4%, LB-Main: 0.1%, LP-Main: 0.1%, AI-HE10: 1.0%, RA-HE10: 0.4%, LB-HE10: 0.1%, and LP-HE10: 0.1%).

The claim is made that by applying strong smoothing only for 32x32 TU boundaries the BD rate loss is less than applying it for all sizes.

JCTVC-J0403 AHG6: Cross-check of JCTVC-J0096 Suppression of blocking artifacts at 32x32 transform boundaries [S. Lu, O. Nakagami (Sony)] [late]

JCTVC-J0181 AHG6: On deblocking filter parameters [S. Lu, O. Nakagami, M. Ikeda, T. Suzuki (Sony)]

This contribution proposes to increase the adaptive capability of deblocking filter by expanding the effective value range of variable β. Proposed solutions aim at expanding the value range for high QP to enable stronger filtering. It is reported that better visual quality can be achieved. The proposed solution does not change the core process of the current deblocking filter and reportedly has no influence on common test conditions.

Proposal 1: Having a steeper increase of beta offset towards higher QP values (such that beta would similarly increase as tc offset does currently) – no change of syntax

Proposal 2: introduce new parameters for adjusting the strength of exponential increase of beta and tc offsets.

JCTVC-J0409 Crosscheck of JCTVC-J0181: AHG6: On deblocking filter parameters [D.-K. Kwon (TI)] [late]

JCTVC-J0445 Cross check of TI’s suppression of blocking artifacts at 32x32 transform boundaries [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0286 AHG6: Adaptive deblocking filtering [A. Norkin (Ericsson)] [late]

The document studies a problem with blocking artifacts in the Riverbed, WestWindEasy and China Speed sequences and proposes modifications to HM7.0 that fix these problems. The proposed modifications reportedly reduce the blocking artifacts on the Riverbed, China Speed and West Wind Easy sequences and also reportedly improve the visual quality on the sequences in the common test conditions. The proposed modifications result in the following changes in BD-rate: (0.1%, 0.0%, 0.1% and −0.1%) on Main profile and (0.3%, 0.2%, 0.6%, 0.2%) on HE10 configuration, the decoding time is similar to that of the anchor. The modifications do not include sending additional parameters to the deblocking filter.

Proposed solution 1: Reduce intra boundary smoothing for horizontal, vertical and DC modes (change to intra prediction) in case of smaller intra blocks

Proposed solution 2: Apply stronger filter to larger intra prediction blocks, modify tc offset for 32x32 boundary.

Proposed solution 3: Apply stronger filter by allowing larger variations close to the block boundaries, e.g. such that deblocking is also useful for inclined surfaces (criterion: keeping the 2nd derivative constant instead of 1st derivative).

Several experts expressed the opinion that the amount of changes in this proposal is fairly high and the proposal has not thoroughly been studied as it became available late.

Subjective tests with 4 sequences Riverbed etc. as suggested in AHG (coordinator: T Suzuki)

- 2 rates equivalent to QP 32, 37

- 4 proposals (286, 181, 96, 90)

- A vs. B comparison: Proposal against anchor with max offset, need to compare at same rate points

Revisit after the tests.

Several experts expressed the opinion that there is a problem which needs to be resolved. The existence of that problem needs to first be confirmed by the subjective tests, and action to be taken should be discussed afterwards. This could also mean to further study possible solutions.

JCTVC-J0494 Crosscheck of JCTVC-J0286: AHG6: Reduction of block artifacts in HEVC for large blocks [D.-K. Kwon, M. Budagavi (TI)] [late]

JCTVC-J0556 AHG6: Cross Check of JCTVC-J0286 Algorithm 2 [G. Van der Auwera (Qualcomm)] [late] [miss]

JCTVC-J0091 AHG6: Chroma QP Offset and Deblocking [G. Van der Auwera, M. Karczewicz (Qualcomm)]

The cb_qp_offset and cr_qp_offset syntax elements are signalled in the PPS and specify the offsets that are added to the luma QP before deriving the corresponding chroma QP values. The HM7 chroma deblocking filter determines the chroma filtering strength without considering the cb_qp_offset and cr_qp_offset values, which can significantly modify the chroma QP values for coding and, therefore, the filtering strength of chroma blocking artifact edges may be too weak or too strong. To resolve this issue, it is proposed to include the cb_qp_offset and cr_qp_offset values into the chroma deblocking filter process. The HM7.0 anchor is reproduced under common test conditions. The chroma deblocking strength correction is illustrated.

The solution is slightly more complex, as the tc table lookup becomes necessary for each of the chroma components separately.

It would also imply that the chroma qp offset values need to be stored for the purpose of deblocking.

The decoder would become more complex, an advantage might be that an encoder using QP offset for rate control would not need to consider the effect on the deblocking. However, rate control algorithms might be designed which take this into account.

No action.

JCTVC-J0372 Cross-check of JCTVC-J0091 on Chroma QP offset and deblocking filter [J. Xu (Sony)] [late]

JCTVC-J0343 Use of Chroma QP offsets in Deblocking [S. Kanumuri, G. J. Sullivan (Microsoft)]

This appears to propose the same change as J0091. No separate presentation therefore seemed necessary.

JCTVC-J0186 On Deblocking Filter and DC Component of Quantization Matrices [K Sato (Sony)]

When quantization matrices are applied, the QPs in a bitstream and the ones actually used for encoding / decoding differ. However, not the latters but the formers are used for deblocking filtering with the current HEVC specification. To correct this gap, it was proposed by JCTVC-I0280 that the value of all QM components be taken into account for deblocking at the 9th JCTVC meeting in Geneva.

The author partly supports this idea. However, taking all QM components into account requires increase in complexity. In addition, higher frequency components do not affect blocking artefacts so much.

This contribution proposes for only the DC component of quantization matrices to be used to adjust QP for deblocking filtering.

A rationale for this could be a scenario where through the quant matrices different QP values are used for different transform sizes. Otherwise, this could simply be implemented via tc offset.

The decoder operation becomes more complex as it is necessary to determine the transforms sizes on both sides of the boundary and adjust the QP value accordingly.

The original proponents of I0280 also verbally express that they support this proposal.

The general opinion is that the benefit is not obvious enough to justify the additional decoder complexity.

JCTVC-J0419 Crosscheck report of On Deblocking Filter and DC Component of Quantization Matrices (JCTVC-J0186) [M. Shima (Canon)] [late]

JCTVC-J0211 On the reference picture comparison for boundary strength [J. Kim, Hendry, B. Jeon (LG)]

No presenter was available Thu morning.

JCTVC-J0398 AHG6: Crosscheck of the reference picture comparison for boundary strength in JCTVC-J0211 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

## Non-deblocking loop filters

### Adaptive loop filter

The subsequent documents were discussed in the BoG on ALF (JCTVC-J0521).

JCTVC-J0036 ALF coefficient coding with a single k-table [K. Ugur (Nokia)]

Abstract:

This contribution proposes to remove position dependent coding of coefficients and instead use EG(0) for all the coefficients. Two versions of signed EG(0) VLC were tested – (1) Unsigned EG(0)+separate sign and (2) se(v). The results reportedly show that there is 0.0%–0.1% change in coding efficiency.

Benefit:

Luma & chroma coefficient coding consistent

Removes kes(v) parsing process entirely from the specification

Removes position dependency k-tables

Coding efficiency:

(1) 0.06% loss proposal 1 (Unsigned EG(0) + separate sign)

(2) 0.07% loss proposal 2 (se(v) – Signed Exp-Golobm coding)

Cross-check:

Originally, this was proposed by Nokia.  Later, TI jointly proposed.  TI and Nokia cross-checked the results individually.

Availability of text:

Available in the contribution.

Discussion:

kes(v) is used in other place, so no hardware reduction.

Current text has differences leading zero or leading one, but can be unified.

Same coding engine is used as coefficient level coding (but ALF coef. is signalled in APS).

There is no prediction in coding, therefore, EG(k) is preferable to compensate this.

Relation with JCTVC-J0346, see further discussion below.

JCTVC-J0346 Unifying ALF coefficient coding with coeff_abs_level_remaining coding [J. Lou, Y. Yu, L. Wang (Motorola Mobility)]

Abstract:

In the current HEVC, a fixed k-parameter Exp-Golomb code is used for ALF coefficient binarization. However, k-th Exp-Golomb code is only used for ALF coefficient coding which introduces extra complexity. It is proposed to unify the ALF coefficient coding with coeff_abs_level_remaining coding.

There are three options in this contribution, but discussion was focused on scheme 3; Both the Luma and Chroma ALF coefficients are binarized (CABAC binarization process) with a unary code and a variable length code with parameter 3.

Benefit:

Simplification and negligible loss (on average) of coding efficiency.

Coding efficiency:

QP=22–37, negligible loss in luma, gain in chroma (0.2%)

QP=32–47, less than 0.1% loss in luma, more gain in chroma (0.4%)

Discussion in BoG:

Better than current one, but new coding is not preferable (TU+fixed length).

Sharing CABAC engine for coefficients and header is not preferable.

Do we have any information about ALF header information size?

Position dependency is not necessary.

Unify luma/chroma syntax process is preferable.

An expert checked the size of ALF header using Kimono 1080p LB, QP22, 100 frames.

The increase of number of bits for the ALF header compared to current HM was around 15% increase in bits. (It was further mentioned 17% would be worst case). The usual number of bits used in coeff coding was around 1000–2000 bits.

Among those two proposals, one expert expressed that the cleaner text (J0036) is preferable. The coding loss is acceptable as it is only header information. The DIS editor also suggested J0036.

After these considerations, the BoG suggested the adoption of the JCTVC-J0036 (se(v) syntax).

Decision: Adopt J0036 se(v) syntax

JCTVC-J0493 Cross-check of ALF Coefficient Coding in JCTVC-J0346 [W.-S. Kim (TI)] [late]

JCTVC-J0337 Fix for ALF Padding Process [P. Chen, W. Wan (Broadcom)]

Abstract:

Virtual boundary processing has been adopted into the HEVC draft text to remove the line buffer requirement for ALF processing in LCU-based decoder implementations. The idea behind virtual boundary processing is to adjust the filter support depending on the filter location such that the last several LCU rows do not need to be stored in line buffers and wait to process these rows until the bottom neighboring LCUs become available. The current padding defined for chroma processing contradicts this purpose.

A modification of the padding process for chroma is proposed to remove the current dependency and resulting line buffer requirement. When chroma top edge pixels are extrapolated upwards, it is proposed to extrapolate by two rows, except when the top edge is also the picture boundary, where it is proposed to extrapolate by three rows.

Benefit:

Removal of the current dependency in chroma ALF padding process.

Coding efficiency:

No loss.

Cross-check:

Source code and proposed text changes were checked and found to match. BD-Rate results matched too. The cross-checker supported the proposal.

Availability of text:

Available in the contribution.  Only two characters need to be modified.

Discussion in BoG:

Fine slice granularity is not supported.

See further discussion below under  JCTVC-J0050.

JCTVC-J0427 Crosscheck of JCTVC-J0337: Fix for ALF Padding Process [M. Budagavi (TI)] [late]

JCTVC-J0050 AHG6: ALF with modified padding process [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

Abstract:

Another solution for chroma padding.  (1) A later block in processing order has a lower priority than a prior block to extend boundary samples when the two blocks have an overlapping to-be-padded area (i.e., only use LCU0).

Additionally, (2) the horizontal size and the vertical size of the ALF filter shape are nine samples and seven samples, respectively. Therefore, it is proposed to change the number of padded samples in the vertical direction from four to three, while keeping the number of padded samples in the horizontal unchanged as four. Only seven lines of modifications are required in the HM software.

Benefit:

Can work with FGS. (But we do not support FGS in Main profile at this moment.)

Coding efficiency:

No loss.

Discussion in BoG:

If filter shape is 7x5, there is no problem.

The purpose to fix chroma padding process is the same as JCTVC-J0337.

Follow-up discussion in track B

It is mentioned that another more radical option would be to completely remove the padding (which is only needed in case that ALF across slice boundary is disabled by the slice header flag).

This would also make it more consistent with SAO and deblocking.

The suggested intention is that it would be desirable to remove specific padding for ALF at slice boundaries and tile boundaries and picture boundaries. A BoG (coordinated by Y. Huang) was asked to study the implications and possible simplifications of the draft text. Revisit.

To be further discussed in plenary: Will there ever be FGS in HEVC? If not, it is unlikely that visual problem occur by removing the padding.

JCTVC-J0325 Crosscheck of J0050: AHG6: ALF with modified padding process [M. Budagavi (TI)] [late]

JCTVC-J0266 AHG6: Modification to loop filtering across slice boundaries [S. Esenlik, M. Narroschke, T. Wedi (Panasonic)]

Abstract:

The proposal advocates aligning all three loop filters by modifying the SAO and ALF slice boundary control operation. The problem is emphasized especially in the case of top-to-down gradual decoder refresh operation (the most common GDR implementation) where slices in a frame are refreshed starting from the top-left corner of a frame. It is proposed that all three loop filters are controlled jointly at the top/left slice boundaries by the slice_loop_filter_across_slices_enabled_flag in the slice header and not at the bottom/right slice boundaries. In other words If slice_loop_filter_across_slices_enabled_flag is equal to 0 in a slice, then the following are applied according to the proposal:

- Deblocking is disabled at both sides of the top/left slice boundary. (No change with respect to current HEVC [2])

- SAO is controlled at both sides of the top/left slice boundary.

- Padding is used for ALF at both sides of the top/left slice boundary. 
(in case that ALF padding would be entirely removed this must be modified such that ALF is disabled at top/left boundaries)

Discussion in BoG:

An editor suggested to cleanup text. Proponent will work for text revision with the editor.

The revised text will be circulated to the interested parties.

Recommendation of BoG: Adopt this proposal.

Text needs to be further aligned with the work of BoG to remove the ALF padding, in principle the proposal is seen to be valuable and likely to be adopted. Revisit.

JCTVC-J0448 AHG6: Cross check of Modification to loop filtering across slice boundaries (JCTVC-J0266) [T. Ikai (Sharp)] [late]

JCTVC-J0320 Multicore-friendly ALF luma region coding [M. Tikekar, M. Budagavi, V. Sze (TI)]

Abstract:

The region coding scheme for luma ALF in HM-7.0 does not allow regions 0 and 15 to share filters although they are positioned adjacent to each other. This contribution proposes the addition of an extra flag to allow that share for a more coherent design.  The proposed solution introduces an extra flag alf_filter_pattern_flag[0] which signifies regions 15 and 0 are merged if the flag is 0 (i.e., circular merging between filter#0 and filter#15).

Not relevant according to BoG

JCTVC-J0399 AHG6: Crosscheck of multicore-friendly ALF luma region coding in JCTVC-J0320 [C.-Y. Tsai, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0048 AHG6: ALF with non-normative encoder-only improvements [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), T. Yamakage, T. Itoh, T. Chujoh (Toshiba), I. S. Chong, M. Karczewicz (Qualcomm)]

Abstract:

Non-normative encoder-only ALF improvements are proposed. The major changes are reusing up to eight previous adaptation parameter sets and estimating rates more accurately in the rate-distortion optimization (RDO) process. The bug-fix of ticket #574 (i.e., swapping coef[2] and coef[4] in the bitstream) is also included. Compared with the ALF in HM-7.0, the proposed ALF can increase luma coding gains by 0.2–1.1% in terms of BD-rate.

- Consider the rate of LCU on/off control flags during picture-level on/off decision  Bug fix

- Use CABAC for rate estimation of LCU on/off control flags during LCU on/off decision

- Reuse up to eight previous adaptation parameter sets during picture-level filter selection

- Recheck picture-level ALF-off after LCU on/off decisions (only for high efficiency mode)

- Increase from one to three redesigns of filter coefficients (only for high efficiency mode)

Decision(SW): Adopt for HM8 and HE10 test conditions.  Adopt Bug-fix ticket #574.

JCTVC-J0253 AHG6: Cross-check for non-normative ALF improvements (JCTVC-J0048) [S. Esenlik, M. Narroschke (Panasonic)]

JCTVC-J0390 AHG6: Further cleanups and simplifications for the ALF in JCTVC-J0048 [C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), T. Yamakage, T. Itoh, T. Chujoh (Toshiba), I. S. Chong, M. Karczewicz (Qualcomm)] [late]

Abstract:

This proposal presents further cleanups and simplifications for ALF, which is mainly in response to some expert’s request.

- Software package 1:

On top of the JCTVC-J0048 software, four modifications are added as follows.

- Reduction of the filter coefficient precision from 9-bit to 7-bit

- Reduction of filter shape from cross9x7+square3x3 to cross7x7+square3x3

- Fix of RDO for considering a previous APS

- Code cleanups

- Software package 2

On top of the software package 1, when samples are equal to 8-bit (i.e., Main conditions), filter coefficients are normatively constrained on the encoder side as follows.

- Sum of positive non-center filter coefficients times 510 plus center filter coefficient times 255 shall be in the range of [0, 215−1−32).

- Sum of negative filter coefficients times 510 shall be in the range of [−215, 0).

In this way, 16-bit accumulation can be achieved for filtering 8-bit samples in Main conditions.

- Software package 3

All the cleanups, fixes, non-normative improvements, and simplifications in JCTVC-J0048 and the previous two software packages are integrated in the software package 3. In addition, the followings are also included.

- Cleanups and fixes for APS in JCTVC-J0047

- Unifying exponential golomb coding of ALF with other parts by using leading zeros

- Applying virtual boundary processing for the last luma LCU row and the first chroma LCU row (a missing adoption in software and text)

- What is more, the software package 3 is based on HM-7.1, where the ALF part can be easily reused for developing HM-8.0.

Benefit:

Simplification to enable 16-bits accumulation operation for highly parallelized processing.

Code cleanups.

Coding efficiency:

0.1% loss in luma compared with JCTVC-J0048 (Software 2), which mainly comes from reduction of one coefficient.

Cross-check:

It is confirmed that the cross-check results of two softwares match to ones by the proponents.

Availability of text:

Available in the contribution.

Discussion in BoG:

Whether 7x7+3x3 is desirable (cf. current: 9x7+3x3)?  Show the results to the person who requested.

If picture size is large, larger filter gives better coding gain.

Several experts expressed their opinion that filter shape should be unchanged at this stage.

About 4x speedup by SIMD.  It is similar to transform.

Concern about encoder complexity is expressed.  The coefficients 7-bit quantization scheme is similar to RDOQ.  One expert expressed his opinion that from the decoder perspective, this is nice to have.

Constraint to 16-bits accumulation is necessary?  Yes. Recommend to adopt this at this meeting, and provide the better/simpler encoder at the next meeting.

Recommendation of BoG:

(from software1) Reduction of the filter coefficient precision from 9-bit to 7-bit

(from software1) Fix of RDO for considering a previous APS

(from software1) Code cleanups

(from software2) Sum of positive non-center filter coefficients times 510 plus center filter coefficient times 255 shall be in the range of [0, 215−32)

(from software2) Sum of negative filter coefficients times 510 shall be in the range of [-215, 0).

(from software3) Cleanups and fixes for APS in JCTVC-J0047 except the part3 of JCTVC-J0047 has to be confirmed by HLS experts

(from software3) Applying virtual boundary processing for the last luma LCU row and the first chroma LCU row (a missing adoption)

Issue of filter shape needs to be further discussed in Track B.

Conduct subjective viewing by using the simplest one (i.e., Software package 2)

What will be tested?

- ALF off vs J0390 software 2 (most simplified 16 bits version) in total 40 test cases (random access and LD B, 2 rate points) – will be started Thu afternoon

- J0390 vs J0048 (16 bits simplification vs. non-simplified version – approx. 20 test cases – will be run Friday or later

Follow-up discussion in Track B:

- 16 bit processing highly desirable but should not produce visual artifacts.

- Some concern expressed about the current encoder complexity

- There may be other ways to achieve this at the encoder, e.g. discarding filters that would violate the constraint

Further revisit after subjective viewing.

JCTVC-J0440 Crosscheck of JCTVC-J0390: AHG6: Further cleanups and simplifications of the ALF in JCTVC-J0048 [D.-K. Kwon, M. Budagavi (TI)] [late]

JCTVC-J0049 AHG6: Comparison between ALF and bi-prediction MC [C.-Y. Chen, C.-Y. Tsai, Y.-C. Chang, C.-Y. Cheng, Y.-W. Huang, S. Lei (MediaTek)]

Abstract:

ALF and bi-prediction MC are compared using 15 1080p sequences, where five of the sequences are common test condition class B sequences and the rest were commonly used during KTA software study period.

Coding efficiency:

Main-LP without ALF is used as the anchor. Main-LP with ALF and Main-LB without ALF are tested against the anchor. The software is in the uploaded package of JCTVC-J0048, which contains non-normative and encoder-only improvements for ALF.

If integer only interpolation is used for bi-prediction, the gain by bi-prediction becomes lower.

Based on the above reasons (trade-off between coding efficiency and power consumption), for real-time low-delay encoding-decoding applications (e.g. video phones and video conferencing) with full HD resolution, ALF could be a better trade-off than bi-prediction MC.

Visual quality is different between bi-prediction and uni-prediction (with ALF).

Information contribution, no action.

JCTVC-J0144 AHG6: Hue/saturation-based chroma ALF design and LCU-based on/off control by encoder [T. Yamakage, T. Itoh, Takeshi Chujoh (Toshiba), C.-Y. Chen, C.-Y. Tsai, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek), I. S. Chong, M. Karczewicz (Qualcomm)]

Abstract:

This contribution provides hue/saturation-based chroma ALF design and on/off control scheme at encoder side. Coding error in Cb/Cr domain is converted to hue/saturation domain. Based on the coding error statistics in hue/saturation domain, Cb/Cr filter coefficients are designed by Wiener filter design method. In addition, LCU-based ALF on/off control for Cb/Cr is decided based on hue/saturation domain.

Benefit:

Reduce hue change that is easily recognized with a small loss of coding efficiency in chroma.

Coding efficiency:

0.2% loss in chroma.

Cross-check: None.

Discussion: It is interesting since the same idea can be applied to other RDO parts.

Information contribution, no action.

JCTVC-J0047 AHG6/AHG9: Syntax for APS ID [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

Abstract:

1.	Cleanups of APS (non-normative change)

2.	Fix of APS – Send an APS only when ALF is enabled for the picture (non-normative change)

3.	Fix of APS ID – Send an APS ID only when ALF is enabled for the slice (normative change)

Benefit:

Cleanups and fix of APS.

Coding efficiency improvement.

Coding efficiency:

No gain since aps_id is always 0 in HM7.  In practical application, since aps_id can be 0 to 63, some gain is expected.

Cross-check:

The software is reportedly carefully checked and verified. The test results exactly match with those provided by the proponent.

Availability of text:

Available in the contribution.

Recommendations of BoG:

- Adopt 1 and 2 to HM8.

- Adopt 3 to WD8/HM8, however, if other APS element(s) would be adopted, this proposal should be modified accordingly.

Discussion in track B:

- Current draft requires max 64 APS to be stored – is this too much? Revisit in HL syntax

- Decision: Adopt 1 and 2 as suggested by BoG,

- adopt 3 conditionally (revisit)

JCTVC-J0332 AHG6/AHG9: Cross check of Cleanups and fixes for APS (JCTVC-J0047) [T. Ikai (Sharp)] [late]

JCTVC-J0288 On Loop Filter Disabling [G. Van der Auwera, R. Joshi, Y.-K. Wang, M. Karczewicz (Qualcomm)]

(not reviewed in BoG)

no presenter available Thu afternoon in track B

to be reviewed

### Sample adaptive offset

The following documents were initially discussed in a BoG, and disposition later confirmed in Track B (See BoG report XXX):

#### SAO merge flags

Recommendation of BoG: to use only one context for merge syntax  with current initialization (J0041 and  J0054 and partially in J0178) if merge will be in a design.

Candidate to adopt: J0041 (conditional that merge will be in the design)

JCTVC-J0041 AHG5/AHG6: On reducing context models for SAO merge syntax [E. Alshina, A. Alshin, J.H. Park, (Samsung), C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

This contribution reduces 3 context models in HEVC design with small gain (−0.04%).  Revisited after another related contributions review.

JCTVC-J0323 AHG5/AHG6: Cross check report of reducing context models for SAO merge syntax (JCTVC-J0041) [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

Supportive comment from cross-checker.

JCTVC-J0178 AHG5: On SAO syntax elements coding [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

This contribution reduces 2 context models, since merge left flag is encoded using 1 ctx instead 3.  This part was supported by experts. No action as 0041 does more.

SAO type coding is modified as well: first bin (SAO on/off flag) is encoded with ctx, remainder (pure SAO type) is encoded using fixed length (3 bins) code and by-pass. This part should be re-discussed with SAO type contribution

It is additionally proposed to concatenate the remaining type index flags for all three components (which is claimed to be useful in case of using combined merge flags).

This is asserted not to be a significant benefit in terms of throughput but would make the standard text more complicated. No action.

JCTVC-J0406 Cross-check of SAO syntax elements coding (JCTVC-J0178) [J. Sole (Qualcomm)] [late]

Comment: The method of separation of cxt coded and bypassed bins need to be re-discussed together with other contribution

      - SAO left band

Recommendation of BoG: Move left band position after SAO magnitudes and signs (as suggested in J0046, J0054, J0148 and partially in J0268)

Candidate to adopt J0046 and J0054 (both are identical)

JCTVC-J0046 AHG6: On left band position coding in SAO [E. Alshina, A. Alshin, J.H. Park, (Samsung)]

JCTVC-J0054 AhG6: Bypass bins grouping in SAO [J. Sole, I.S. Chong, M. Karczewicz (Qualcomm)]

JCTVC-J0392 AHG6: Crosscheck of SAO bypass bins grouping in SAO in JCTVC-J0054 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

#### SAO performance

Recommendation of BoG: (1) Test combination J0044 with  J0139 (encode bug fixes) and measure performance improvement from another contributions in this category compare to this encoder only modifications. (2) set visual test for J0213.

JCTVC-J0179 On sao_merge_left_flag for effective Mx1 CTB coding [T. Ikai, T. Yamamoto, Y. Tasugi (Sharp)]

Groups several LCUs to use the same SAO parameters set. This is equivalent to forcing merge left flag to be true for several LCUs. The number of samples in a group is constant = 64x64x2=32x32x8.

Increases the latency and buffer in encoder side. This is contradictory with low latency LCU based SAO philosophy. It was requested to modify encoder to have 1 LCU latency and provide additional results.

The BoG requested test data for fast encoder version.

It is verbally reported that no gain can be realised in case of low-latency encoder.

no action

JCTVC-J0371 Cross-check of JCTVC-J0179 on sao_merge_left_flag for effective Mx1 CTB coding [J. Xu (Sony)] [late]

Comment: should be considered as coding efficiency contribution.

JCTVC-J0355 AHG5, AHG6: Coding of SAO merge left and merge up flags [Koohyar Minoo, David Baylon (Motorola Mobility)]

This contribution combines 3 merge flags (Y,U,V) to 1. As result the number of context models is reduced from 3 to 1 (w/o initialization change).

0.2% Y BD-rate gain. Should be discussed together with other coding efficiency contributions.

(reduction of context coded bins is marginal but it is definitely not more complex)

Candidate for adoption

(may depend on disposition of 0045)

JCTVC-J0379 Cross-check of JCTC-J0355 on coding of SAO merge left and merge up flags [C. Auyeung (Sony)] [late]

Comment from cross-checker: speed up encoder side since fewer variants should be tested

JCTVC-J0044 Encoder modification for SAO [E. Alshina, A. Alshin, J.H.Park, (Samsung)]

Recommendation of BoG: test other coding efficiency contributions on top of this (Recommendation of BoG: adopt s/w if combination results with 0139 are good). 0.2%

JCTVC-J0173 Cross-check of J0044 on SAO encoder modification from Samsung [G. Laroche, T. Poirier, P. Onno (Canon)] [late]

Comment: test with other coding efficiency contributions.

JCTVC-J0097 Evaluation of picture-based SAO optimization in HM-7.0 [D.-K. Kwon, W.-S. Kim (TI)]

Reports the gain of picture based SAO optimization (not ctc) vs LCU based optimization, but results are not that good (compare to Sharp’s J0179). Seems fix provided by TI is not optimal.

Recommendation of BoG: Adopt (s/w) bug fix from J0097 (off by default)

Recommendation: keep picture based optimization in s/w (<=3 months) since we have volunteers to improve this.

JCTVC-J0192 AHG6: Crosscheck of evaluation of picture-based SAO optimization in HM-7.0 in JCTVC-J0097 [C.-M. Fu, Y.-W. Huang (MediaTek)]

Comment: just bug fix, but doesn’t utilize all possibilities of picture based optimization

JCTVC-J0139 AhG6: SAO Parameter Estimation Using Non-deblocked Pixels [W.-S. Kim (TI)]

Modifies encoder only. Additionally to current design non-de-blocked pixels are taken into account during SAO parameters estimation.

Provides a gain (0.1%), more gain for small LCU (32x32 0,3% Y-BD-rate gain).

Should be tested with other coding efficiency contribution. Recommendation of BoG: Adopt s/w if combination results are good.

Analyze the code, combine with J0044 and run test for LCU 64x64 and 32x32 (Elena & Woo-Shik).

Revisit:

Discussion in Track B: Concentrate first on normative changes that make syntax more efficient.

JCTVC-J0391 AHG6: Crosscheck of SAO parameter estimation using non-deblocked pixels in JCTVC-J0139 [C.-M. Fu, Y.-W. Huang (MediaTek)] [late]

Requested to analyse the code.

JCTVC-J0213 AHG6: A threshold for SAO edge offset [T. Sugio, T. Matsunobu, T. Nishi (Panasonic)]

Changes pixel processing in SAO. Targets to visual quality.

−0.04% Y-BD-rate gain; 0,06% Chroma BD-rate (drop)

Adds complexity (condition check).

If possible set visual test in order to check whether problem exists or not. No action for current solution. Very low priority

JCTVC-J0365 Cross-check of Threshold for SAO Edge Offset in JCTVC-J0213 [W.-S. Kim (TI)] [late]

#### SAO magnitudes

An order of SAO syntax elements (with already agreed changes):

SAO merge left (ctx)

SAO merge up (ctx)

SAO on/off flag (ctx)

---------------------------------

SAO type (ctx or by-pass)

SAO magnitudes (by-pass)

SAO signs (by-pass)

SAO left band position (by-pass)

JCTVC-J0043 AhG5: On bypass coding for SAO syntax elements [E. Alshina, A. Alshin, J.H. Park (Samsung)]

SAO magnitude is encoded with by-pass, 0,0% (Y), 0,2% (U) 0,1%(V). 2 ctxs removed from HEVC design. The number of ctx coded bins in worst case reduced 94%. Statistically 0,3% reduction of ctx coded bins in HM7.0. No reduction for total bins number in the worst case.

Recommendation of BoG: adopt J0043 and use by-pass coding for all bins of SAO magnitude.

Candidate for adoption

JCTVC-J0193 AHG5: Crosscheck of bypass coding for SAO syntax elements in JCTVC-J0043 [C.-M. Fu, Y.-W. Huang (MediaTek)]

Comment: by-pass should be used for SAO magnitude coding.

JCTVC-J0106 AHG6/AHG5: SAO offset coding [I. S. Chong, J. Sole, M. Karczewicz (Qualcomm)]

Additionally to JCTVC-J0043 modifies SAO magnitude binarization. Before TU was used for SAO magnitudes coding. Combination of TU and fixed length code is proposed in this contribution. As results 94% of ctx coded SAO bins reduction in the worst case (the same with J0043) and 62% reduction for total amount of SAO bins in the worst case.

Specification change is additional 11 lines paragraph describing TU and fixed length binarization for SAO magnitude.

No need to combine truncated unary and fixed length. Several experts confirm that in bypass mode, unary code with maximum length 31 bins can also be done in one cycle (but requires more buffer potentially).

It is not obvious that J0106 would give an advantage – more evidence and study needed. Count of maximum number of coded bins does not give the whole figure, as fixed length bins, context-coded bins and unary bins cannot be weighted equally.

In software the plain unary code would be less complex.

Combination of fixed + unary is already used in last position coding (but there it is context coded bins). Remaining level coding is similar (combination of exp Golomb and Rice-Golomb with transition parameter)

Would be desirable to define minimum number of binarization schemes (from the perspective of the spec). Not define a specific binarization scheme just for SAO offset. It is reported that EG0 was tried, but a loss of 0.15% was observed.

What is the current maximum length of unary code in bypass mode for any syntax element? 23. With the solution of J0043 this would be extended to 31 (in HE10 settings), for main profile, the maximum length of unary code for SAO offset would be 7 anyway. No need to define a specific binarization scheme in main profile.

In principle, the standard text does not limit the maximum number of bins in unary code, but the limitation is implicit by the maximum length allowed by any syntax element.

Further study is suggested, particularly w.r.t. extension of SAO to higher bit depth and the related binarization of the syntax elements.

(Note: J0178 suggests something similar for type coding).

JCTVC-J0387 AHG6/AHG5: Cross-check of JCTVC-J0106 (SAO offset coding) [T. Yamakage, T. Itoh (TOSHIBA)] [late]

The following documents were discussed in track B, after grouping had been performed in the BoG

JCTVC-J0141 AhG6: SAO Offset Bypass Coding [W.-S. Kim, M. Budagavi, V. Sze (TI)]

Two modifications are suggested. In both cases 2 bins of SAO magnitudes are encoded with ctx. For remaining bins by-pass is used. No reduction for the number of context models. As results 88% reduction of ctx coded SAO bins is achieved for the worst case. Additionally in second solution the combination of TU and fixed length coding is proposed. As result total amount of SAO bins is reduced 53%.In both solutions the order of syntax elements coding was changed in order to combine ctx coded and by-passed bins. Number of changed in s/w and specification is higher compare to J0106.

Performance

Solution 1: 0.0%(Y); 0.0%(U), 0.0%(V)

Solution 2: 0.1%(Y); 0.4%(U), 0.4%(V) (BD-rate drop)

no action – other proposals e.g. JCTVC-J0043 provides better simplification.

JCTVC-J0317 Cross-Check of proposal J0141 of TI (AhG6: SAO Offset Bypass Coding) [C. Kim (Samsung)] [late]

The amount of specification and s/w changes doesn’t justify benefits J0141 provides.

#### SAO sign

JCTVC-J0031 Unification of band and edge offsets with respect to sign for SAO [K. Andersson, P. Wennersten, R. Sjöberg (Ericsson)]

This proposal unifies band and edge offset syntax with respect to sign while having similar coding efficiency compared to HM-7.0 (0.0 % average BDR for common conditions). The modification consist of removing all 4 sign syntax element for band offset and instead having implicit derivation of sign similar as for edge offset.

Presentation not uploaded.

Some concerns are expressed that the constraints on the sign of the band offset potentially could introduce artifacts.

No action

JCTVC-J0172 Cross-check of J0031 on SAO signs from Ericsson [P. Onno, G. Laroche, T. Poirier (Canon)] [late]

#### SAO type

JCTVC-J0045 AhG6: On SAO type sharing between U and V components [E. Alshina, A. Alshin, J.H. Park, (Samsung), G. Laroche, C. Gisquet, P. Onno, (Canon)]

This contribution proposes to share SAO type, merging left and merging up flags between U and V color components. The number of context coded bins for SAO syntax is reportedly reduced by 36% with suggested modifications (which is 1.2% total amount of context coded bins reduction). In addition, this simplification respectively provides 0.2%, 0.2% and 0.3% Luma BD-rate gain with LCU sizes of 64x64, 32x32 and 16x16. It is asserted that proposed change simplifies memory access during simultaneous processing of U and V components and helps for efficient SAO implementation.

Shares type, merge left and merge up for the two color components.

Difference compared to J0355: In 335 merge is also shared for luma, but it allows different types for the two chroma components

Various things would be interesting to test in combination with 355:

- share merge only between chroma components

- share merge between luma and chroma, and additionally share type between chroma comp.

Potential candidate for adoption

JCTVC-J0467 AhG6: Verification of J0045 on SAO chroma processing [F. Bossen (DOCOMO Innovations)]

JCTVC-J0065 AhG5/AhG6: On SAO type index coding [Y. H. Tan, C. Yeo (I2R)]

This contribution proposes two modifications to the coding of SAO type index. In the first modification, the SAO type index is coded with a truncated unary code instead of a unary code. In the second modification, only first 2 bins (signaling SAO on/off and band/edge offset) of the syntax element are coded with contexts while the edge offset type is coded with a fixed length code in bypass mode. Both modifications reportedly reduce the maximum number of context coded bins for each syntax element coded while improving chroma coding performance by ~0.3%.

2nd version uses context coding for type and for one more bin for EO/BO, fixed length code for type.

Several experts raise the opinion that it might be better to use context coding only for on/off, and use bypass coding for type

Related contributions: J0178, 0104, 0148, 0268

None of the five contributions (except 148) modifies on/off coding

Except for J0065, the four methods are different in binarization of type:

- J0104 : trunc. unary, 4 bins max.

- J0148 : same as 104

- J0178 : fixed length 3 bins

- J0268 : one flag EO/BO, fixed length 2 bins in case of EO (max total 3 bins)

J0065 has two versions, one is identical to 104, the other is similar to J0268 but using context coding for EO/BO.

One argument is brought that a scheme which give same chances at least to the four edge directions appears to have an additional benefit

Candidate for adoption: J0268: Version with one context-coded bin on/off, remaining bins bypass: One flag EO/BO, fixed length 2 bins for edge orientation in case of EO

J0207 is using the same binarization as J0104, but applies context coding to the type part as well. This is undesirable.

JCTVC-J0349 AHG5/AHG6: Cross-verification of JCTVC-J0065 on SAO type index coding [K. Chono, K. Tokumitsu (NEC)] [late]

JCTVC-J0104 AHG6/AHG5: Fix and simplification for SAO type index [I. S. Chong, J. Sole, M. Karczewicz (Qualcomm), C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

In HM-7.0, SAO type index is coded using unary binarization. The first bin is coded with one context model, while all non-first bins are coded with another one. In this contribution, since the maximum value of SAO type index is 5, it is proposed to replace unary binarization with truncated unary binarization for SAO type index. It is also proposed to use bypass coding for all non-first bins of the truncated unary binarization for SAO type index. BD-rates are reportedly 0.0%/0.0%/0.0%/0.0% for Main-AI/RA/LB/LP respectively.

Uses truncated unary, on/off is context coded (no modification), other bins are bypass coded

No change in performance.

JCTVC-J0449 AhG5/6: Cross-check for SAO type coding using truncated unary and by-pass CABAC mode (JCTVC-J0104) [E.Alshina (Samsung)] [late]

Cross-checker mentions that by changing unary to truncated unary, it can be assumed that band type will potentially selected more frequently.

Change is simple.

JCTVC-J0268 AHG6: On SAO signalling [J. Xu, A. Tabatabai (Sony)]

This proposal is a follow-up study of JCTVC-I0246 under HM7.0. The coding of SAO type is reconfigured to have separate signalling for SAO On/Off, SAO type BO and EO and EO/BO side information (classes or band positions). Grouping both context coded bins and by-pass coded bins is also proposed to improve throughput of CABAC. Using AHG5 template, it is reported that percentages of context coded bins for SAO are reduced from 3.12% to 3.11% for Main and no change for HE10 on average, percentages of by-pass coded bins for SAO are increased from 0.23% to 0.64% for Main and from 0.06% to 0.18% for HE10. Theoretical worse case analysis in AHG5 also reports that the max number of context coded bins for SAO types is reduced from 6 to 2. Experimental results report BD-rate performance as 0.0%/−0.2%/−0.3% for Y/U/V under All Intra Main, 0.0%/−0.3%/−0.4% for Y/U/V under Random Access Main, −0.1%/−0.2%/−0.3% for Y/U/V under Low Delay Main, 0.0%/−0.1%/−0.2% for Y/U/V under All intra HE10, 0.0%/−0.2%/−0.3% for Y/U/V under Random Access HE10, and 0.0%/−0.2%/−0.1% for Y/U/V under Low Delay HE10.

Interleaving/grouping is not needed any more

General remark by F Bossen: SAO gives highest gain in LD P – was any of the simplifications tested with that? A: Usually similar performance, some tested it (e.g. 104)

JCTVC-J0374 AHG6: Cross-check of JCTVC-J0268: on SAO signalling [Koohyar Minoo (Motorola)] [late]

JCTVC-J0207 Improved Type Coding for SAO [Sangoh Jeong, Seungwook Park, Byeongmoon Jeon (LGE)]

This contribution was initially accidentally uploaded as an "associated resource" instead of as the main contribution. This was corrected on 07-04.

This contribution proposes a modified sao_type_index for SAO. This scheme replaces unary binarization with truncated unary binarization for the index of a SAO type to remove coding redundancy in CABAC.

See discussion under J0065 above

JCTVC-J0394 Crosscheck of J0207: Improved type coding for SAO [D.-K. Kwon (TI)] [late]

JCTVC-J0103 AHG6: Decoupling SAO LCU on/off from SAO type  [I. S. Chong, M. Karczewicz (Qualcomm), C.-W. Hsu, C.-M. Fu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

In HM-7.0, each LCU has one SAO type for each color component, and the SAO type can represent SAO-off, EO, or BO. In this proposal, SAO on/off signaling is decoupled from the SAO type. One SAO LCU on/off flag for each color component is sent before the other syntax elements of the color component. In addition to the new SAO LCU on/off flag coding, constrained merge syntax is also proposed where a merge-left flag is not coded and inferred as zero when the left LCU is SAO-off. Results reportedly show 0.0%/0.3%/0.3%/0.5% luma coding gains for Main-AI/RA/LB/LP, respectively.

SAO on/off is now becoming dependent from SAO on/off of other LCUs (which is not the case currently)

Dependency/interference with other candidate adoptions (e.g. common usage of merge)

Two contexts (instead of one) are used for SAO on/off

Merge left flag is becoming dependent from SAO on/off (whereas currently SAO on/off is dependent on merge left)

The amount of text changes is not trivial

Several concerns raised – no action.

JCTVC-J0450 AhG6: Cross-check for decoupling SAO LCU on/off from SAO type (JCTVC-J0103) [E. Alshina (Samsung)] [late]

JCTVC-J0269 SAO on/off flag coding [J. Xu, A. Tabatabai (Sony)]

This proposal decoupled the SAO on/off from SAO type coding and encoded SAO on/off flags jointly for all color components. Experimental results state BD performance of Y/U/V is 0.1%/0.0%/−0.1% for All Intra Main, 0.1%/0.0%/−0.1% for All Intra HE10, −0.3%/−0.4%/−0.3% for Random Access Main, −0.2%/−0.2%/−0.3% for Random Access HE10, −0.3%/−0.8%/−0.9% for Lowdelay B Main, and −0.3%/−0.4%/−0.4% for Lowdelay B HE10. There is also a combined solution along with JCTVC-J0xxx. The combined solution reports BD performance of Y/U/V is 0.1%/−0.4%/−0.5% for All Intra Main, 0.1%/−0.3%/−0.4% for All Intra HE10, −0.3%/−0.7%/−0.7% for Random Access Main, −0.2%/−0.5%/−0.7% for Random Access HE10, −0.3%/−1.3%/−1.3% for Lowdelay B Main, and −0.3%/−0.7%/−0.6% for Lowdelay B HE10.

Too large amount of changes compared to benefit – no action.

JCTVC-J0296 Cross-check of SAO on/off flags coding in JCTVC-J0269 [W.-S. Kim, D.-K. Kwon (TI)] [late]

JCTVC-J0140 AhG6: SAO Complexity Reduction with SAO LCU Flag Coding [W.-S. Kim, D.-K. Kwon (TI)]

As the current SAO syntax in HM-7.0 was originally designed considering quad-tree partitioning, it is asserted that it is not well structured for LCU based signalling. In this contribution, a new syntax structure for SAO is proposed, where SAO off is decoupled from SAO type, and SAO merge flags are removed. It is asserted that the complexity is decreased by reducing the number of context coded bins. At the same time, the coding gain is reportedly achieved by 0.6/0.2/0.1% for LCU size of 16/32/64 for Y (Main) when SAO LCU flag coding is used with removal of merge flags. When only merge up flag is removed with SAO LCU flag coding, the coding gain is reported as 0.7/0.4/0.2% for LCU size of 16/32/64 for Y (Main).

Different versions are presented with total removing of merge, only removing merge up, etc. Even in the simplest method which entirely removes merge, to use the method with SAO LCU flag, three contexts and 4 context coded bins are necessary (which may be more than in some of the other simplified solutions that are discussed).

Draft text is not provided for the version that entirely removes merge.

Some discussion whether the removal of merge would be a substantial benefit in terms of implementation and buffer saving; in particular merge left is regarded to be less critical. It is seen as an advantage that it would reduce the dependencies across tile boundaries.

Supported by one company other than proponents, several concerns expressed – no action.

JCTVC-J0324 AhG6: Cross check report of SAO Complexity Reduction with SAO LCU Flag Coding (JCTVC-J0140) [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

Cross checker’s opinion is that the concept and amount of changes are similar to J0269, but it is more implementation friendly due to the removal of merge.

#### CABAC init

According to proponents, there is no need to discuss the J0062 and J0316 contributions when 0041 is considered.

JCTVC-J0062 SAO CABAC Context Initialisation to Reduce Parallel Encoding Losses [G. Clare, F. Henry (Orange Labs)]

JCTVC-J0455 Crosscheck of SAO CABAC Context Initialisation to Reduce Parallel Encoding Losses (JCTVC-J0062) [M. Coban (Qualcomm)] [late]

JCTVC-J0401 Crosscheck of SAO CABAC Context Initialisation in JCTVC-J0062 [Hendry, B. Jeon (LG)] [late]

JCTVC-J0316 New initialization value for SAO merge signals [I. S. Chong, L. Guo, M. Karczewicz (Qualcomm)]

      - High level signalling

JCTVC-J0087 AHG6: Independent luma and chroma SAO on/off control at slice level [M. Zhou (TI)]

In the current HM7.0 design it is normatively restricted at the slice level that chroma SAO has to be turned off when luma SAO is off. Such a normative encoder restriction is undesirable because it restricts encoder flexibility. This contribution advocates removing such a restriction and making SAO on/off control flag fully independent at slice level. The proposed change does not affect coding efficiency.

Presentation not uploaded

Only one slice per picture is used, therefore not surprising that there is no loss

HL signaling should be as simple as possible

Without the condition, extension to 4:4:4 is more straightforward

One expert said that the original intention of conditional parsing had been the support for fast encoding (but this is not completely obvious)

Several experts support the change. Decision: Adopt.

JCTVC-J0051 AHG6: Crosscheck of independent luma and chroma SAO on/off control at slice level in JCTVC-J0087 [C.-M. Fu, Y.-W. Huang (MediaTek)]

JCTVC-J0132 AHG6: SAO One Unit Parameter Signalling [Y. Chiu, W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai (Intel)]

This contribution proposes a one unit SAO parameter signalling scheme to increase the flexibility of SAO syntax design. Flags are added into slice header to specify if all of LCUs in the slice are processed using the same SAO parameters.

In case of flag set to on, all merge flags will be inferred to be on.

No results were presented, though it is a proposal for better coding efficiency.

In the last meeting, a similar method was proposed for APS (I0130), but this did not show coding efficiency benefit.

The slice header would not be a suitable place for coding efficiency tools (slices are mainly for resync purposes)

No action.

#### Combinations

JCTVC-J0148 AHG5: Bypass coding for SAO syntax elements [T. Matsunobu, K. Terada, H. Sasai, T. Nishi (Panasonic)]

In this contribution, a modified design of CABAC is proposed to reduce the number of context models and bins for SAO syntax elements. In this proposal, sao_lcu_enable_flag is added for decoupling the SAO LCU on/off state from sao_type_idx. sao_offset_abs and the sao_type_idx are binarized using bypass models instead of using context models. sao_lcu_enable_flag is signalled at the top of bins for SAO. Average BD-rate loss by this proposal relative to HM7.0 anchors was reportedly 0.03% for Y, 0.10% for U and 0.15% for V component respectively. This proposal can reduce the worst case of the number of bins per sample for SAO, which are coded using context models, from 1.50 to 0.01.

Similar to 268, 104, 065, 207, 178.

- Context coding applied to sao_lcu_enable which is encoded first.

- Two solutions in the contribution, solution2 uses one context.

- Merge flag is also bypass coded.

- It is verbally reported that context initialization is not changed

- Q: How would it perform when merge is completely omitted? Some results are still missing.

no support by companies other than proponents – no action

JCTVC-J0350 AHG5/AHG6: Cross-verification of JCTVC-J0148 on bypass coding for SAO syntax elements [K. Chono, K. Tokumitsu (NEC)] [late]

JCTVC-J0347 AHG6/AHG5: Simplified SAO coding [I. S. Chong, J. Sole, M. Karczewicz (Qualcomm)]

In HM-7.0, each LCU has one set of SAO signaling for each color component. Each set of SAO signalling includes merge signals (i.e., merge_up and merge_left) and if merge signals are not used, new set of SAO syntax (i.e., sao_type_idx, sao_offset_abs, sao_offset_sign, and sao_band_position) is sent to the decoder. In this proposal, we propose to simplify above SAO signaling, especially for merge  flags, sao_type_idx. Results reportedly show 0.0%/0.3%/0.3%/0.4% luma coding gains for Main-AI/RA/LB/LP, respectively. We further propose to simplify sao_offset_abs coding. Results reportedly show 0.0%/0.3%/0.3%/0.3% luma coding gains for Main-AI/RA/LB/LP, respectively.

According to proponents, this has already been discussed in context of other proposals (it is a combination of J0103, J0104, J0106/J0043, J0041) –  no need for presentation.

JCTVC-J0422 Cross-check of AHG6/AHG5: Simplified SAO coding (J0347) by Qualcomm and Mediatek [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

Candidates for adoption:

J0041

J0043

J0046=J0054

J0268

J0355

J0045 (pending on benefit in combination with J0355)

- Test combination of J0355 and J0045 as said elsewhere

- J0045 would have benefit implementation-wise

- Provide combined text

For the first 4 items (can be done in parallel with previous):

- integrate text and software

Afterwards: Provide a combined text and result for all candidates combined.

Revisit

Encoder only: Decision(SW): Adopt J0044, J0139 (J0139 with enc conf flag)

Encoder bug fix: Decision(SW): J0097

### Other

JCTVC-J0165 LCU-based framework with zero pixel line buffers for non-local means filter [M. Matsumura, S. Takamura, A. Shimizu (NTT)]

In this contribution, non-local means (NLM) filter is applied to HM7.0 after SAO and before ALF; and LCU-based framework that allows reconstructing the decoded picture in LCU order at encoder and decoder, which offers low-delay capability, is proposed. With picture-based RDO, the average BD-rate for luma component and chroma component improves 0.38–1.79% and 0.05–1.46% respectively. With LCU-based RDO, those are 0.21–1.46% and 0.62-2.08% respectively. Subjective quality improvements were also observed.

Operated as fourth filter in the loop.

Q: Could this also be operated as post filter?

Encoder/decoder runtime increase approx. 10%

No action.

JCTVC-J0190 Cross-check of LCU-based framework for non-local means filter (JCTVC-J0165) [T. Yoshino, K. Kawamura, S. Naito (KDDI)]

## Block structures and partitioning

### General

JCTVC-J0133 TU Depth Clean-up [Y. Chiu, P. Kapsenberg, W. Zhang, L. Xu, Y. Han, Z. Deng, X. Cai (Intel)]

It has been observed that there are two TU depth related issues in current HEVC text and software. Firstly, there’s no limitation to the values of maximum TU depths in SPS. Secondly, the derivation scheme of context index for split_transform_flag may result in unwanted context, which was reported in ticket #533. To clarify the usage of TU depth and ensure its correctness, this contribution proposes clean-up solutions for these issues.

First issue: Revisit together with J0335

Second issue (ticket #533): There is an issue with CNU. In principle, the ticket could be solved as editorial issue (avoiding use of undefined context by replacing CNU through CNT), but what is suggested here is to combine the solution of the ticket with a reduction by one context.

Decision: Adopt the suggested solution on the second issue.

JCTVC-J0359 Cross-check for JCTVC-J0133 TU Depth Clean-up [X. Zhang, O.C. Au (HKUST)] [late]

JCTVC-J0360 Cross-check of TU Depth Clean-up (JCTVC-J0133) [J. Xu (Microsoft)] [late]

### NSQT

JCTVC-J0138 NSQT Simplification [X. Zheng, Y. Yuan, H. Yu, Y. He (??)]

This document provides a simplification of NSQT. Both software implementation and text modification are provided in this contribution. The proposed solution simplifies the non-square transform quadtree split process, and merges the luma non-square transform quadtree and chroma non-square transform quadtree. As a result, NSQT has become a lot easier to describe and implement. Experimental results show that the proposed solution does not have negative impact on coding performance. Both software encoding and decoding time are slightly reduced compared to HM7.0 anchor.

It is suggested to inhibit splitting of non-square transform blocks into square transform blocks at the last splitting stage.

Compared to the old method, class F shows small loss (0.1+%), otherwise approx. equal.

Gain of the new NSQT version versus RA main is about 0.4%, versus LDB main about 1% (not much different for HE 10).

Comments:

- Text still has some problems (and would need careful checking)

- From view point hardware implementation, it is a little bit better but does not solve the biggest problem (irregularity by branching from the square-shaped quadtree, computation of memory addresses etc.).

Still too complex to be considered in the main profile.

In general, this new version of NSQT is seen as a step in the right direction. Some discussion about whether the current NSQT should be removed entirely or be replaced by the new scheme. After this it was suggested to leave everything as it is in the DIS (considering that more urgency is on other issues)

Further study.

JCTVC-J0370 Cross-verification of JCTVC-J0138 on NSQT simplification [M. Zhou (TI)] [late]

JCTVC-J0415 Cross-verification of JCTVC-J0138 NSQT simplification [L. Guo (Qualcomm)] [late]

JCTVC-J0514 Cross-check of JCTVC-J0138 on NSQT simplification [X. Fang, K. Panusopone, L. Wang (Motorola Mobility)] [late]

JCTVC-J0364 Implicit transform block split process for asymmetric partitions [X. Zheng (HiSilicon)]

This contribution provides an implicit TU split solution for asymmetric partitions when “QuadtreeTUMaxDepthInter” is set to 1. Experimental results show that the proposed solutions can contribute average coding gain from 0.3% to 1.3% at Main profile configurations. Both encoder and decoder complexity are same as HM7.0.

Two solutions are suggested:

- non-square transforms for asymmetric partitions (gain compared to RA main is about 0.6%, versus LDB main about 1.2%)

- additional implicit square transform split for asymmetric partition

Solution 1 appears more complex than the simplified NSQT of J0138

Solution 2 could be practical without too much implementation burden, but results are not available and it is unclear how many changes would be necessary to the text.

Revisit on solution 2 (if time allows and if reasonable gains can be shown)

JCTVC-J0473 Cross-check of J0364 on Implicit transform block split process for asymmetric partitions [E.François (Canon)] [late]

## Motion and mode coding

JCTVC-J0098 AHG5: Bypass bins for reference index coding [V. Seregin, J. Sole, X. Wang, M. Karczewicz (Qualcomm), V. Sze, M. Budagavi (TI)]

Context-coded bins reduction in reference index coding is proposed.  All the bins after the second bin are coded with CABAC bypass mode. The worst-case of context-coded bins is reduced from 15 to 2. Experimental results show no performance change and 8% of bypass bins in average under four common test configurations. Additionally, the proposal also results in one context removal.

Comments:

- Several experts express support for this obvious simplification

- Straightforward change of text and software (also confirmed by cross-checker)

Decision: Adopt J0098

JCTVC-J0382 Cross-check report for bypass bins for reference index coding (JCTVC-J0098) [H. Sasai, K. Terada (Panasonic)] [late]

JCTVC-J0176 AHG5: Reference index coding [C. Rosewarne, M. Maeda (Canon)]

This contribution presents a method for encoding reference indices that introduces bypass coding into the binarisation of the reference index and removes two contexts from the context model.  Under common conditions the simulation results indicate 0.0%, 0.0%, 0.0% in AI Main, AI HE10, RA Main, RA HE10 configurations, 0.0%, 0.1%, 0.0% in LDB Main, 0.0%, 0.0%, 0.1% in LDB HE10 configuration.

Comments:

- J0098 appears simpler, and (at least in common test conditions) no advantage in terms of compression

JCTVC-J0489 Cross-check report for reference index coding (JCTVC-J0176) [S.H. Kim, A. Segall (Sharp)] [late]

JCTVC-J0297 AHG5:ref_idx coding [S.H. Kim, L. Kerofsky, A. Segall (Sharp)]

In HM-7.0, ref_idx requires 15 context coded bins in worst-case scenario.  In order to improve the throughput efficiency, this contribution proposes a new binarization method combining truncated unary and fixed length coding (TUFLC). In the proposal, the first four bins are context coded and the remaining bins are bypass coded.  It is asserted that the proposed method 1 and method 2 reduce the worst case number of context coded bins from 15 bins to 4 bins and 2 bins, respectively. It is also asserted that the proposed method reduces the worst case number of bins (both context coded and bypassed coded) from 15 bins to 8 bins.

Both methods use a new binarization

Method 2 similar (in terms of context coded bins and performance) to J0098. J0098 potentially uses more bypass bins (not under common test condition), but it is not evident that this would be critical

JCTVC-J0511 Cross-check of high throughput binarization for reference index coding (JCTVC-J0297) [J. Chen (Qualcomm)] [late]

JCTVC-J0101 Splitting contexts for MVD coding [V. Seregin, J. Chen, X. Wang, M. Karczewicz (Qualcomm)]

In HM7.0, the first two binarized bins of motion vector difference (MVD) are coded with adaptive context and the remaining bins are coded with bypass mode; and there is no MVD signaling for list L1 if mvd_l1_zero_flag is signaled as true. Contexts for the first two bins are shared for all blocks regardless of their associated inter prediction direction or reference lists. However, MVD from different directions may have very different statistical properties, in which case context sharing may not be suitable. That is true especially if motion estimation is unbalanced at encoder side, for example some MVDs are set to zero for a particular list. This contribution proposes assigning a separate context to the first MVD bin with respect to uni-predicted, bi-predicted list L0 and bi-predicted list L1 MVD.

Three methods are suggested, method 2 is preferred by the proponents which adds 1 additional context.

Several experts mentioned that no obvious benefit is shown, and a very similar approach was already presented in previous meetings

When the contribution was discussed, one company supported this (cross-checker)

No action.

JCTVC-J0167 Cross-check report of JCTVC-J0101 on Splitting contexts for MVD coding [T.Chujoh (Toshiba)] [late]

JCTVC-J0240 Consistent coding of motion information for B slices with identical reference picture lists [J. Xu (Microsoft)]

B slices with two identical reference picture lists, or GPB pictures, are widely used in HEVC. Those slices are specially handled in the current design. When uni-directional prediction is applied, only prediction from list 0 can be used; and when bi-directional prediction is applied, the mvd for list 1 are both zero. Both methods are helpful to improve the coding efficiency. However, in the current draft, the former one is performed implicitly by an encoder and the latter one is signaled explicitly. This document discusses two different ways to unify the design.

Solution 1 reinvokes use_l0_only_flag which was removed by the last meeting.

Solution 2 is identical to J101

When this contribution was discussed, one more company other than proposing companies and cross-checker supported this, but no consensus could be achieved on taking any action.

JCTVC-J0397 Crosscheck of consistent coding of motion information for B slices with identical reference picture lists in JCTVC-J0240 [J.-L. Lin, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0315 AHG5: Context reduction for MVD coding [C. Kim, J. Kim, J.H. Park (Samsung)]

In the current HM, the first two bins of motion vector difference (MVD) are coded with context. The remaining bits are coded with bypass mode. The two flags, abs_mvd_greater0_flag, and abs_mvd_greater1_flag, are used for context at both flags. From the statistical analysis, the distribution between ‘0’ and ‘1’ is similar, so, the use of context is not much benefit for separating ‘0’ and ‘1’. Therefore, we perform abs_mvd_greater1_flag rather than abs_mvd_greater0_flag  where abs_mvd_greater0_flag is coded with bypass mode. The proposed method reduces context from 2 to 1. No loss and complexity is observed both normal QP and lowQP (1,5,9,13). Moreover, total bins (context + bypass) is reduced by 0.1% and contexts is also reduced 1.5% average for RA and LD

The proposal changes the sequence of syntax elements (and deviates logically from what is done elsewhere e.g. in transform coding) and introduces a new binarization (cannot be interpreted as truncated unary anymore)

Several experts express opinion that benefit is not clear.

No support by other companies

No action.

JCTVC-J0385 Cross-check of Context Reduction for MVD Coding in JCTVC-J0315 [W.-S. Kim (TI)] [late]

JCTVC-J0177 AHG5: Merge index coding [C. Rosewarne, M. Maeda (Canon)]

This contribution presents a method for binarising the merge_idx syntax element.  The proposed binarisation removes a transition from arithmetic to bypass coding, resulting in the merge_idx syntax element making exclusive use of arithmetic coding. This change resulted in 0.0%, 0.0%, 0.0% in AI Main, AI HE10, RA Main and RA HE10 configs. Results were 0.0%, 0.1%, 0.0% in LDB Main and 0.0%, 0.0%, 0.3% in LDB HE10 and -0−0.1%, -0−0.1% and 0.0% in LDP Main, -0−0.1%, 0.0%, -0−0.1% in LDP HE10.

Proposal is to replace 3 bypass coded bins in merge index by arithmetic coding bin. Gives small benefit in LD P.

No benefit in (coding efficiency vs. complexity)

No action.

JCTVC-J0433 AHG5: Cross check of Canon’s Merge index coding (JCTVC-J0177) by Qualcomm [I. S. Chong, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0180 Zero merge candidate simplification [B. Li, H. Li (USTC), H. Yang (Huawei)]

This contribution presents a modification on the construction of the zero merge candidates for B slices. Specifically, bi-directional zero merge candidates are replaced by uni-directional zero merge candidates in B slices. It is asserted that the complexity of motion compensation can be reduced with the proposed modification, while the BD-Rate change is reported to be 0.0% under common test condition.

Comments:

In terms of implementation (hardware or software), this does not give a big deal. It touches the most simple part of the merge process.

Several experts express the opinion that there would be no good reason to change a part that has been stable for several meetings.

No action.

JCTVC-J0188 Cross-check of J0180 on uni-directional zero merge candidate [Y. H. Tan, C. Yeo (I2R)]

JCTVC-J0203 Simplification on zero merge candidate derivation [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

This contribution presents a simplification on zero merge candidate derivation. In order to unify the zero merge candidate derivation process in HM7.0 and reduce average memory bandwidth of motion compensation, it is proposed to derive L0 uni-predictive zero merge candidate instead of bi-predictive zero merge candidate regardless of slice_type in the zero merge candidate derivation process. The experimental result reportedly shows that the proposed method introduces no coding loss on average in all test conditions.

Same as J0180, no need for presentation

JCTVC-J0443 Cross-check report of simplification on zero merge candidate derivation (JCTVC-J0203) [T. Sugio (Panasonic)] [late]

JCTVC-J0204 Dependency removal of temporal merge candidate and combined bi-predictive merge candidate derivation [S.-C. Lim, H. Y. Kim, J. Lee, J. S. Choi (ETRI)]

This contribution presents a dependency removal between temporal merge candidate and combined bi-predictive merge candidate derivation. In order to improve the throughput of merge candidate list derivation, it is proposed to use only spatial merge candidates for combined bi-predictive merge candidate derivation process. The experimental result reportedly shows that the proposed method introduces maximum 0.2% of coding loss in HE10-LB test condition and 0.1% of coding loss on average in the other test conditions.

Comments:

- The problem does not exist any more due to a change made by the last meeting (fixed reference list in spatial candidate derivation, I0116)

No action

JCTVC-J0381 Crosscheck of JCTVC-J0204: Dependency removal of temporal merge candidate and combined bi-predictive merge candidate derivation [K Sato (Sony)] [late]

JCTVC-J0170 On Temporal and Combined Merge motion vector predictors derivation for Merge/Skip mode [G. Laroche, T. Poirier, P. Onno (Canon)]

This contribution presents a simplification of the motion vector derivation process for the Merge/Skip modes. The aim is to reduce the number of cycles for the worst case for hardware implementation that are needed to perform the whole motion vector prediction derivation. The proposed simplification consists in scaling the temporal candidates in parallel to the derivation of the combined predictors. The proposed modification reports only 0.1% BDR loss.

Same as J0204, no need for presentation

JCTVC-J0410 Cross-check of J0170 on temporal and combined motion vector predictors derivation for merge/skip mode [T. Lee (Samsung)] [late]

In the previous meeting, bi-pred restriction for small PUs is adoptedError: Reference source not found where bi-predictive motion information for 8x4 and 4x8 is converted to uni-predictive List0 motion after motion information construction. Assuming that encoder restricts motion search on bi-prediction for such small PUs, the only place where bi-predictive motion is given is the merge mode. In this proposal, bi-predictive merge candidates are restricted during merge candidate derivation process. By removing or converting bi-predictive merge candidates, merge candidate derivation process is reduced in complexity and converting bi-predictive motion on motion compensation process is unnecessary assuming that encoder restricts motion search on bi-prediction for small PUs.

“Proposal 1” is suggested solution of proponent.

Something similar was discussed in the last meeting in the context of memory BW reduction, but by that time proposals that suggested change of the merge process for small PUs were not adopted to keep the design common over all PU sizes; it does not have a significant impact on complexity anyway.

No action.

JCTVC-J0145 Simplification on spatial AMVP candidate derivation [Y. Lin, J. Zheng (HiSilicon)]

This document proposes to simplify the derivation process of spatial AMVP candidates. Two spatial candidates are derived by checking non-scaled and scaled MV candidate based on left and above neighboring positions in HM7. It is proposed that two steps involved in the scaled MV candidate derivation are combined and only 3 neighboring positions are checked for the scaled MV candidate. It is asserted that the non-scaled MV candidate is always checked before the scaled one. The specification text is largely simplified. The test results show no loss of coding efficiency of RA-main: 0.0%, -0−0.1%, 0.0%, RA-HE10: 0.0%, 0.0%, 0.0%, LB-main: -0−0.1%, 0.0%, 0.0%, LB-HE10: 0.0%, -0−0.2%, -0−0.1% under common test conditions.

One comment:

- Benefit not obvious, as the processing steps can anyway be done in parallel, and only one candidate is scaled.

- Draft editor mentions that at least one of the conditions may be confusing.

- The proposal again introduces dependency between left and top candidates which was removed before

No support by other experts. No action.

JCTVC-J0158 Cross check of HiSilicon Technologies’ proposal JCTVC-J0145 [J. Kim (LGE)] [late]

JCTVC-J0155 On MV prediction [J. Dong, Y. Ye (InterDigital)]

This proposal simplifies the scaling process for MV prediction. The simplification includes two parts: 1) reducing the range of possible POC differences, and 2) combining two integer approximations into one. Under the common test conditions specified in JCTVC-I1100, the proposed method provides bit-exact results of HM-7.0.

Simplification not obvious – no action.

Reduction of range of POC differences may not be reasonable when leaving common test conditions.

JCTVC-J0400 Crosscheck of simplification of the scaling process for MV prediction in JCTVC-J0155 [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late]

Cross-checker made a hardware implementation analysis and found an increase in number of gates.

JCTVC-J0219 On signalling the syntax of MVP flag [X. Zhang, Y. Shi, O.C. Au, F. Zou, C. Pang (HKUST)]

In current HM7.0, the motion information inside a reference list for a certain PU is signalled in the order: reference index, MVD information, and MVP flag. There are several syntaxes for MVD information, which are transmitted or parsed conditionally. At the decoder side, only after all MVD related syntaxes are parsed, the MVP flag can be parsed and then the selected MVP will be found. This contribution proposes to signal the MVP flag in front of the MVD related syntaxes. With such a change, decoding process of MV will be parallel-friendly, while no performance change is observed.

Some support is expressed

Other experts express that the benefit of this is not obvious and this “micro-level” parallelism is not really needed.

no action.

JCTVC-J0348 Cross-check for JCTVC-J0219 on signalling the syntax of MVP flag [J. Dong, Y. Ye (InterDigital)] [late]

Cross checker expresses support

JCTVC-J0225 Restrictions to the maximum motion vector range [Alistair Goudie (Imagination Technologies)]

This contribution proposes restrictions to be applied to the maximum motion vector range and motion vector difference. The main aim of the proposal is to define the maximum resolution of motion vector components when being stored as spatial/temporal candidates. Three alternatives on the severity of the restriction are presented, along with two methods of implementing the restriction.

AVC has a restriction of MV at profile/level

HEVC has restriction only in VUI

Preferred solution of proponent: Encoder/bitstream restriction; alternative: clipping at decoder

Related: J0335, level definitions – Revisit there

JCTVC-J0278 AMP mode support for minimum CUs of size greater than 8x8 [M. Coban, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes AMP support for minimum CU of size greater than 8x8. In the current WD all partition modes except AMP is supported for minimum CUs of size greater than 8x8. For minimum CU’s of size 16x16 addition of AMP mode support results in average BD-rate reductions of  0.8%, 0.9%, 1.2% and 1.3% for RA-Main, RA-HE10, LD-Main and LD-HE10 configurations, respectively.

Relates to ticket #327 which was closed

As the SCU size is an encoder choice, why would an encoder not use SCU 8x8 right away instead? A decoder has to support it anyway.

It was also mentioned that the log2_minblocksize_minus3 flag  may be useless, as any decoder has to support. Perhaps for future profiles?

No action on J0278.

JCTVC-J0413 Crosscheck of AMP mode support for minimum CUs of size greater than 8x8 in JCTVC-J0278 [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late]

JCTVC-J0312 Redundancy removal on InterDir syntax [C. Kim, T. Lee, E. Alshina (Samsung)]

Reportedly identical with J0086 which was already adopted in Track A. No need to be presented in Track B.

JCTVC-J0302 Restricted usage of motion vectors for long-term reference picture in motion vector prediction process [I.-K. Kim, Y. Park, J. H. Park (Samsung)]

In this contribution, two solutions are proposed to improve the current design by marking motion vectors from LTRPs as unavailable instead of using it without scaling. The first solution is not to insert motion vectors into the candidate list by marking it unavailable if the motion vectors are from LTRPs. The motion vector predictor is marked as unavailable before the scaling process is performed. The benefit of this approach is that by removing the inefficient motion vectors which are scaled or non-scaled motion vectors from LTRPs, more efficient motion vectors can be included in the list. The second solution is that motion vectors are not inserted into the candidate list when the scaled motion vectors are likely to be inefficient. In this solution, the differentiation between LTRP and short-term reference picture (STRP) is not required. When POC difference (Tr) between reference picture of current PU and reference picture of candidate PU (co-located PU or neighbor PU) are larger than pre-determined threshold (THpoc_diff), motion vector scaling is not used. The motion vector predictor is marked as unavailable before the scaling process is performed. Both solutions are applied to both spatial and temporal motion vector prediction. Although average coding efficiency gains from the second solution for each configuration are negligible, coding efficiency gains for Class F under Low delay B main and Low delay B HE10 are 0.1% and 0.1%, respectively.

Discussed in track B but seems to rather relate to HL syntax / LTRP.

Solution 1: Mark MV as unavailable whenever the refidx refers a LTRP

Solution 2: Mark as unavailable if POC difference larger than a threshold, otherwise use it as usual (including scaling).

Q: How large is the threshold (currently 8/16 depending on frame structure). Would it be possible to make it switchable by encoder?

Some concern is raised about solution 2, as it modifies the AMVP process and has implication on hardware complexity. Solution 1 may be OK

What is purpose? Coding efficiency? How large is the benefit? May be better to leave it as it is, i.e. use LTRP MV without scaling.

JCTVC-J0341 Cross check of JCTVC-J0302 on Restricted usage of motion vectors for long-term reference picture in motion vector prediction process [Y. Takahashi, O. Nakagami, T. Suzuki (Sony)] [late]

### Hooks in scalability: Motion related

JCTVC-J0121 AHG10: Motion related hooks for HEVC multiview/3DV extension based on long-term reference pictures [Y. Chen, Y.-K. Wang, L. Zhang, V. Seregin (Qualcomm)]

JCTVC-J0510 AHG10: Mental cross check of JCTV-J0121 on motion related hooks for HEVC multiview/3DV extension [O. Bici, M. Hannuksela (Nokia)] [late]

JCTVC-J0519 Mental cross-check of JCTVC-J0121 (Motion related hooks for HEVC multiview/3DV extension based on long-term reference pictures) [Chong Soon Lim (Panasonic)] [late]

JCTVC-J0122 AHG10: Hooks related to motion for the 3DV extension of HEVC [Y. Chen, Y.-K. Wang, L. Zhang (Qualcomm)]

JCTVC-J0523 Mental cross-check of JCTVC-J0122 solution 5 [Y. Takahashi, O. Nakagami, T. Suzuki (Sony)] [late]

JCTVC-J0524 Mental cross-check of JCTVC-J0122: AHG10: Hooks related to motion for the 3DV extension of HEVC [J. Boyce (Vidyo)] [late]

JCTVC-J0224 AHG10: Hook for scalable extensions: Signalling TMVP reference index in slice header [O. Bici, M. Hannuksela, K. Ugur (Nokia)]

JCTVC-J0505 AHG10: Mental cross-check of JCTVC-J0224 [Y.Chen(Qualcomm)] [late]

## High-level syntax and slice structure (108)

### NAL unit header (9  7)

J0063, J0231, J0250, J0112, J0113, and J0174 were all suggested to be related. Only the first four of these were discussed in the preceding AHG9 meeting.

JCTVC-J0063 AHG9: Syntax for NAL Packet Priority [Eun-Seok Ryu, Yan Ye, Yuwen He, Yong He (InterDigital)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

Pictures in the same temporal level in a hierarchical-B structure can have different influence on error propagation and on decoded video quality. Currently in HEVC draft 7, the NAL unit header does not indicate packet priority within the same temporal layer. This contribution proposes two syntax options to indicate such priority of a NAL unit.

R. Sjoberg expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was remarked that the prioritization may depend on the loss concealment method.

It was noted that the nal_ref_flag is basically always currently equal to 1 except at the highest temporal layer.

"Method 1" would change nal_ref_flag to be a nal_priority_flag indicating relative priority within a temporal level.

"Method 2" would provide a priority_id in the AUD, which could carry more bits than the nal_ref_flag.

It was noted that there would be no normative purpose for the proposed priority_id – that it is just metadata – and could be an SEI message.

Some participants expressed some skepticism about the value of the proposed priority_id.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0522 Mental crosscheck of JCTVC-J0063: Syntax for NAL Packet Priority [R. Sjöberg (Ericsson)] [late]

JCTVC-J0231 On nal_ref_flag [T. K. Tan, Junya Takiue (NTT Docomo)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution seeks to clarify the purpose of the nal_ref_flag.  The nal_ref_flag does not seem to have any use in the decoding process apart from the final marking of the picture as reference or non-reference picture.

This contribution proposes to remove the nal_ref_flag syntax element from the NAL unit header and create 3 new NAL unit types for coded slices that can be either used for reference or not.

J0463 is reportedly a (late) cross-check.

It was suggested that an alternative would be to use the maximum temporal later value as a non-reference picture indication, rather than using the NUT or nal_ref_flag.

This (or something like it) seemed promising.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0250 Indication of non-reference pictures [R. Sjöberg, J. Samuelsson (Ericsson)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution proposes to change the semantics of nal_ref_flag so that in addition to indicating non-reference pictures of the highest layer, it is also capable of indicating whether a picture in any layer is a non-reference picture of its own layer. This contribution claims that this makes it possible to indicate whether a picture is a reference picture or not in a sub-stream where the highest layer has been removed and thus indicate whether that picture can safely be removed from the sub-stream without affecting the decoding of the remaining pictures in the sub-bitstream.

It was commented that this would make it more difficult for a "middle box" to identify pictures that can be dropped. Unless the highest temporal ID in the bitstream is known, the non-reference pictures cannot be dropped.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0520 Mental cross-check of JCTVC-J0250 [Yan Ye] [late]

JCTVC-J0112 AHG9: Various comments on HEVC draft 7 [Y.-K. Wang (Qualcomm)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan.)

This document proposes the following:

- Removal of nal_ref_flag, and push the saved bit to reserved_one_5bits to make it to reserved_one_6bits; this proposal is suggested to be ignored if the proposal in JCTVC-J0113 is adopted

- Change of temporal_id to temporal_id_plus1 (The DPB size in level definitions only support hierarchical coding structures with GOP size up to 16 (see document JCTVC-J0111 for analyses of DPB size requirements for different GOP sizes), i.e., typically up to 5 temporal layers are supported.) However, it was commented that there may be a desire in the future to use deeper temporal nesting.

- Change of the value 0 of nal_unit_type from "Unspecified" to "Reserved".

- An alleged editorial fix to the semantics of the extension syntax elements in VPS, SPS, PPS, APS and slice data. This was not agreed. However, it was agreed that some further clarification of the intended tolerance of decoders for reserved values would be desirable.

- Extending of the extension mechanism to all types of NAL units. The participants did not support this part of the proposal.

- To support SEI NAL units that may follow the first VCL NAL unit in the same access unit

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0469 AHG9: Mental cross-check of JCTVC-J0112 [T. C. Thang (UoA), Hendry (LG)] [late]

JCTVC-J0113 AHG10: High-level syntax hook for HEVC multi-standard extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

This is a follow-up proposal of JCTVC-I0355. A prior document G149 was also suggested as relevant background information.

In JCTVC-I0355, a high-level syntax hook for HEVC multi-standard scalable or 3DV extensions wherein the base layer or view is AVC compatible is proposed. It proposed to use the following principles for the syntax design:

- For the NAL unit header length in the multi-standard scalable or 3DV extensions to be the same as in the existing HEVC design;

- That there should be sufficient NAL unit types to be used by HEVC and its potential future extensions, ideally the same as in the existing HEVC design;

- For the AVC NAL units to be distinguishable from the NAL unit header itself.

A comment was given in response to JCTVC-I0355 that an AVC decoder would not be able to distinguish between an AVC NAL unit and an HEVC NAL unit.

In this proposal, an improved design is proposed to solve the above issue while the above three design principles are still followed.

The proposed change to the NAL unit header is to remove nal_ref_flag and re-arrange the syntax elements in the HEVC NAL unit header.

The proposal uses the NUT range from 16 to 31 from the AVC perspective.

A participant questioned the need to multiplex AVC within HEVC within the bitstream level – suggesting to depend on the system level to provide that capability.

Cross-checked in J0492.

Revisit.

JCTVC-J0492 AHG10: Mental cross-check of JCTVC-J0113 [M. M. Hannuksela (Nokia)] [late]

JCTVC-J0174 AHG 9 / AHG 10: On NAL unit header [T. Thang (UoA), J. Kang, H. Lee, J. Lee (ETRI), Hendry, B. Jeon (LG)]

This contribution discusses two items. First, it is suggested that the functionality of nal_ref_flag might be redundant as described in JCTVC-I0251 and JCTVC-I0355 so that the removal of the flag should be considered. To cover the flag functionality to differentiate reference and non-reference pictures, it is suggested to add a constraint to the semantics of temporal_id such that temporal_id of NAL units that contain slices of a non-reference picture must not be equal to 0.

It was commented that we could have a non-reference slice NUT.

Revisit that.

Second, it is asserted that if future extensions of HEVC also use the current fixed 2 bytes NAL unit header size, there is only 5 bits, which is the reserved_one_5bits, available to be used to describe layer identification. The contributor suggested that this might be too small when considering that the extension might cover not only scalability extension but also multiview extension. Furthermore, it might not be necessary to treat temporal identification from identification of other scalability / view types in the extensions of HEVC. Therefore, it is proposed to combine reserved_one_5bits and temporal_id and change its name to layer_id.

Revisit.

JCTVC-J0464 AHG9: Mental cross-check of JCTVC-J0174 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0463 AHG9: Mental cross-check of JCTVC-J0231 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0432 On NAL Unit Header and Video Parameter Set Design [B. Choi, J. Kim, J. Park (Samsung)] [late]

The NAL unit header and VPS design are proposed for HEVC 3D/scalable extension with consideration of hybrid 3D/scalable coding design with non-HEVC video coding standards. This proposal is originally prepared for JCT2 (HEVC 3D extension). However, it is also proposed in JCT because it contains some syntax change in the HEVC draft.

This proposal did not actually propose a change of the NUH of the base spec.

The VPS aspect of the contribution proposes to have an indicator of the type of standard associated with each layer. It was commented that this is a more elaborate scheme than what is expressed in our requirements and that it is something that could be done later if appropriate. Further study of this is encouraged.

JCTVC-J0239 AHG10 - Selective inter-layer prediction signaling for scalable extension [J. Xu (Microsoft)]

Proposes a flag to be included in the NAL unit header to indicate whether the current picture (layer representation) is used by layer representations from higher layers, similar as the discard_flag in the AVC based SVC, and change the 5-bit reserved_one_5bits to be 4-bit reserved_one_4bits.

Karsten: If a base layer is encoded first, and then add enhancement layers later.

Stephan: Instead of using one flag, we could use NAL unit types, equivalent to use of a fraction of bit. We should not reduce the 5 bits for layer_id for future extensions.

Further study was encouraged.

[YK to add more detailed notes.]

JCTVC-J0428 AHG10: Mental cross-check of Selective inter-layer prediction signalling (JCTVC-J0239) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

### Random access and adaptation (12 – done)

#### Random access point (RAP) pictures (7 – done)

JCTVC-J0107 AHG9: On RAP pictures [Y.-K. Wang, Y. Chen, R. J. Joshi, A. K. Ramasubramonian (Qualcomm)]

This document includes the following proposals related to RAP pictures (i.e., IDR, CRA and BLA pictures):

Topic 1: To include the support for handling a CRA picture as a BLA picture based on an indication through external means. Decision: Adopted.

Topic 2: To enable prediction from decodable leading pictures (non-TFD leading pictures) associated with a RAP picture by normal pictures associated with the same RAP picture, and by leading pictures associated with the next RAP picture (wherein leading pictures associated with a RAP picture are those pictures following the RAP picture in decoding order but preceding the RAP picture in output order, and normal pictures associated with a RAP picture are those pictures following a RAP picture in both decoding order and output order and preceding, in decoding order, the next RAP picture).

It was noted that contribution J0251 would eliminate non-TFD leading pictures.

Decision: The pictures that follow a RAP picture (including an IDR picture) in both decoding and output order cannot reference any leading picture.

Topic 3: To change the definition of RAP picture. No action.

Topic 4: To mandate the activation of VPS, SPS, PPS and APS at each BLA picture. No action needed.

Topic 5: To include a constraint to disallow output-order interleaving of non-TFD leading pictures with TFD pictures or pictures earlier than the same associated CRA or BLA picture in decoding order, and a constraint to disallow decoding-order interleaving of TFD pictures and following pictures associated with a RAP picture.

The spirit is that output order is as follows:

- pictures that precede the RAP in decoding order, then non-TFD leading pictures, then RAP, then following pictures

- TFD pictures must precede non-TFD leading pictures in output order

- but there is no relative output order constraint in regard to the order of TFD and pictures that precede the RAP in decoding order.

Decision: Agreed.

Regarding decoding order, all leading pictures associated with a RAP picture shall precede, in decoding order, all pictures that follow the RAP picture in output order. Decision: Agreed (consensus assessed by T. K. Tan).

Topic 6: To change  related to the inference of no_output_of_prior_pics_flag equal to 1.

It was remarked that a difference in language between Annex C and clause 7 is intentional, not an error, but it was agreed that some clarification might be beneficial if the current decoding conformance language is not sufficiently clear.

The second aspect proposed was to change "first IDR or BLA picture in the bitstream" to "first picture in the bitstream" in a few places relating to no_output_of_prior_pics_flag inference. Decision: Agreed.

Topic 7: To use one more NAL unit type to differentiate TFD & TLA pictures and non-TLA TFD pictures

No action taken on this aspect.

The use of the recovery point SEI message was discussed in this context, and it was remarked that the position of the recovery point is signalled as an unsigned POC difference to be added to the POC of the current picture. This does not provide the equivalent functionality of the AVC recovery point SEI message and therefore seemed to be a bug. It was suggested to change the ue(v) encoding to se(v) with a range of –MaxPicOrderCntLsb/2 to MaxPicOrderCntLsb / 2 – 1. Decision (BF): Agreed.

JCTVC-J0499 AHG9: A mental cross-check of JCTVC-J0107 (On RAP pictures) [M. M. Hannuksela (Nokia)] [late]

- JCTVC-J0345 Editorial modifications to HEVC text specification relating to reference picture sets and random access points [G. J. Sullivan, S. Kanumuri (Microsoft)]

Delegated to editors for consideration. (Any aspects that conflict with recorded decisions are not to be used.)

JCTVC-J0215 AHG 9: On NAL unit type [Hendry, B. Jeon (LG)]

(Discussion chaired by M. Hannuksela.)

Proposes to remove two out of the 4 current non-IDR RAP types:

- CRA without TFD (similar proposal in J0344)

- BLA with TFD

It was noted that this related to J0344, so this was discussed together with J0344 – see notes in the section on that document.

J0482 provides a cross-check

JCTVC-J0482 Mental cross-check of JCTVC-J0215 On NAL unit type [T. C. Thang (UoA)] [late]

- JCTVC-J0344 Refinement of random access point support [S. Kanumuri, G. J. Sullivan (Microsoft)]

(Discussion chaired by M. M. Hannuksela.)

(Chaired by M. M. Hannuksela)

This contribution proposed three modifications relating to RAP pictures:

1) A constraint on IDR pictures to provide a simplified form of random access.

2) A constraint that leading pictures of RAP pictures must precede non-leading pictures in decoding order, in order to simplify the scanning of a bitstream for leading pictures.

3) Modify the NAL unit type definitions for RAP pictures to avoid duplicate functionality and convey more RAP type information in the NAL unit type.

Item 2 had been already resolved by notes taken elsewhere.

A comment was expressed that a NAL unit type for IDR picture with leading pictures allowed is desirable.

A comment was expressed that if decodable leading pictures for a CRA picture are originally present but are removed during splicing (including a conversion of the CRA picture to a BLA picture), no HRD parameters for the coded video sequence starting from the BLA picture are readily present in the bitstream.

Decision: Modification 3 was adopted with the addition of a NAL unit type for IDR picture with leading pictures allowed, i.e. the CRA/BLA/IDR NAL unit types are:

| Description | SAP types possible |
| --- | --- |
| CRA picture | 1, 2, 3 |
| BLA picture | 1, 2, 3 |
| BLA picture with no associated TFD pictures | 1, 2 |
| BLA picture with no leading pictures | 1 |
| IDR picture with no leading pictures | 1 |
| IDR picture (which may have leading pictures) | 1, 2 |

A cross-check was promised to be provided by M. M. Hannuksela (not yet available).

To convert a CRA to BLA, the converter would need to consider: 1) no_output_of_prior_pics_flag, 2) rap_pic_id, 3) nal_unit_type.

It was suggested to provide a note in the spec about how the proposed type 7 would be envisioned to be used.

It was suggested that, relative to the proposal, we should have a NUT for an IDR that may have leading pictures.

Decision: Adopt as modified to have a NUT for IDR with leading pictures.

JCTVC-J0551 Mental cross-check of JCTVC-J0344 (Refinement of random access point support) [M. M. Hannuksela (Nokia)] [late]

- JCTVC-J0251 Restrictions on leading pictures of CRA and BLA [J. Samuelsson, R. Sjöberg (Ericsson)]

(Discussion chaired by M. M. Hannuksela.)

It was suggested that document J0310 is related.

It was commented that encoders typically intend to have decodable leading pictures displayed.

No action taken.

Cross-check was promised to be provided by T. K. Tan.

JCTVC-J0547 Mental cross-check of JCTVC-J0251: Restrictions on leading pictures of CRA and BLA [TK Tan (NTT Docomo)] [late]

JCTVC-J0229 AHG9: Comments and clarification on CRA, BLA and TFD pictures [T. K.Tan (NTT Docomo)]

(Discussion chaired by M. M.Hannuksela)

Editorial improvement suggestions were made in section 1.1 of the contribution on the use definitions of sequence start point (SSP) access unit and sequence start point (SSP) picture. Delegated to editors for consideration.

Editorial improvement suggestions were made in section 1.2 of the contribution. Delegated to editors for consideration.

Renaming of TFD picture as random access skip (RAS) picture was delegated to editors for consideration.

A comment was expressed that it would be nice if the reference decoder checked whether the bitstream conforms to all constraints of the standard.

Cross-check provided in JCTVC-J0462.

JCTVC-J0462 AHG9: Mental cross-check of JCTVC-J0229 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0310 Revival of decodable backward predicted pictures that are output preceding a RAP picture [Arturo Rodriguez (Cisco), A. K Katti, H-Y Hwang]

(Discussion chaired by M. M. Hannuksela.)

[Add more summary info.]

Decision: Adopt a new NAL unit type value for non-TFD (i.e. decodable) leading pictures of any RAP picture. All leading pictures of any RAP picture shall either be marked with a NAL unit type of TFD or non-TFD leading picture.

A cross-check will reportedly be provided by L. Winger.

JCTVC-J0543 Mental cross check of concepts in JCTVC-J0310 [Yasser Syed (Comcast)] [late]

JCTVC-J0552 Mental cross-check of Revival of decodable backward predicted pictures that are output preceding a RAP picture (JCTVC-J0310) [Lowell Winger (??)] [late]

#### Splicing and editing (2 – done)

JCTVC-J0108 AHG9: Splicing-friendly coding of some parameters [Y.-K. Wang, Y. Chen (Qualcomm)]

During splicing, two bitstreams may refer to few parameter sets with the same ID for each type of parameter sets but with different content. This document proposes that all parameter set IDs are fixed-length coded, and placed before any entropy-coded syntax elements in each parameter set or coded slice NAL unit. Furthermore, it is proposed that the syntax element no_output_of_prior_pics_flag and the syntax element rap_pic_id are placed before any entropy-coded syntax elements in the slice header, and the syntax element rap_pic_id is fixed-length coded. It is asserted that the changes enable lightweight splicing of bitstreams.

A cross-check is in J0501.

It was remarked that this has implications for extensibility, as fixed-length coding restricts the number of possible values that can be supported. It also affects coding efficiency, as it may sometimes use more bits than would be required for VLC coding.

An aspect relating to mandating a value for no_output_of_prior_pics_flag needs further discussion. No action on that aspect.

It was noted that this proposal interacts with the proposal to create a slice header parameter set.

Regarding moving the rap_pic_id and no_output_of_prior_pics_flag before VLC data

It was suggested not to allow the value 0 for the rap_pic_id.

It was asked whether we actually still need rap_pic_id. Decision: Drop rap_pic_id.

Regarding the no_output_of_prior_pics_flag before VLC data – Decision: Move it.

Various potential alternative approaches were discussed for the parameter set ID aspects, especially in the slice header. Further study was encouraged about that.

It was remarked that the draft is missing the condition that first_slice_in_pic_flag = 1 for testing for the first VCL NAL unit. Decision (Ed.): It was agreed that this should be fixed.

JCTVC-J0501 AHG9: Mental cross-check of JCTVC-J0108 (Splicing-friendly coding of some parameters) [M. M. Hannuksela (Nokia)] [late]

#### Temporal layer access (TLA) pictures (3 – done – new one!)

JCTVC-J0156 AHG 10: Generalized definition of the TLA for scalable extension [C. K. Kim, Hendry, B. Jeon (LGE)]

This contribution suggests that the layer switching feature enabled by current TLA NAL unit for temporal scalability may be extended for other scalability aspects such as spatial and quality scalabilities. It was proposed to generalize the semantics of current TLA NAL unit to provide a hook for similar concept for scalable extensions. This was asserted to extend the temporal layer switching to any scalability layer switching and does not need to add new NAL types for that purpose.

It is assessed that the proposed generalization does not change the concept of TLA for HEVC specification.

S. Deshpande indicated a plan to submit a cross-check.

It was remarked that in the current context, this seems to be just an editorial change proposal, and that it could be possible to modify the semantics and syntax element names later, when the extended functionality is needed.

It was remarked that an example shown corresponded to what is considered an IDR picture in a higher spatial layer in the SVC design, and that this could also be the case in a future scalable HEVC design.

No action seemed needed for version 1.

JCTVC-J0526 AHG9: Mental Cross-check of JCTVC-J0156 - Generalized definition of the TLA for scalable extension [S. Deshpande (Sharp)] [late]

JCTVC-J0246 On temporal layer access pictures [B. Choi, Y. Park, I. Kim, J. Kim, J. Park (Samsung)] [late]

Similarly to the BLA picture, an additional picture type called a “broken link TLA (BLT)” is proposed for identifying the TLA pictures with a broken link or a temporal layer switching. The leading pictures associated with the TLA or BLT is also marked as TFD pictures for easy discarding in systems.

It was remarked that one example shown corresponds somewhat more to a CRA case than a BLA case.

It was questioned whether the coding efficiency improvement likely to be provided using the example scheme would be worth the complication of adding more NUTs to support this.

It was noted that the example case only applies to high-delay encoding.

No simulation results were provided to establish the coding efficiency advantage.

It was remarked that there was a temporal layer switching point SEI message in SVC that can provide such functionality (not using a NUT).

No cross-check was provided.

For further study.

JCTVC-J0305 AHG10: On Gradual Temporal Layer Access [S. Deshpande (Sharp)]

This document proposes gradual temporal layer access (GTLA) pictures. It is asserted that the GTLA pictures provide more flexibility in selection of reference pictures while providing temporal layer switching functionality. It is asserted that gradual temporal layer access functionality is useful in allowing selection of desired frame rate in a step-by-step manner.

No simulation results were provided to establish the coding efficiency advantage.

Decision: Adopted.

JCTVC-J0500 AHG10: Mental cross-check of JCTVC-J0305 (On Gradual Temporal Layer Access) [M. M. Hannuksela (Nokia)] [late]

JCTVC-J0549 NAL unit types for non-reference pictures within the same temporal sub-layer [J. Samuelsson, R. Sjöberg (Ericsson), T. K. Tan, J. Takiue (NTT Docomo)] [late] [miss]

TBP.

### Slices and slice header parameters (16  4)

#### Picture order count (POC) (3  1)

JCTVC-J0084 AHG9: Restrict Picture Order Count to 40-bit [M. Zhou (TI)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

At the 9th JCTVC meeting in Geneva, the dynamic range of picture order count (PicOrderCntVal) was increased from 32-bit to 64-bit. 64-bit PicOrderCntVal can support continuous 120 fps video recording for up to 4874.5 million years. However, on the decoder side there is certain complexity associated with carriage of 64-bit PicOrderCntVal. It is therefore recommended to restrict PicOrderCntVal to 40 bits, which can already support continuous 120 fps video recording for up to 290.5 years.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

At the previous meeting, we thought there was no real impact to the increase of the specified range.

Additional discussion of this is needed in the JCT-VC meeting.

JCTVC-J0110 AHG9: On POC [R. L. Joshi, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan and J. Boyce.)

This document proposes the following:

- An editorial change to the definition of picture order count to discuss specific cases

- Removal of the function DifPicOrderCnt( ) or consistently using it in the specification, particularly for derivation processes for motion vectors and reference indices

- Removal of one of the POC-related constraints in Annex C.4

- An SEI message conveying additional POC information for intra pictures to enable derivation of output order for intra-picture-only trick mode playback

(Cross check not yet provided.)

Discussion of each item:

Topic 1. Editorial only. Something needs to be changed. Editors can consider this (and where this should go).

Topic 2. Notes that the POC difference computation function is not always used, and thus the constraint on POC difference is not applied everywhere that POC differences are computed. It may be desirable to impose the constraint when differences are computed for motion vector derivation, but perhaps not for RPS difference values (e.g. for an LTRP in the RPS). For further study.

Topic 3. About prevRefPic, it was commented that there seems to be an editing error, as our intent was to include prevRefPic in the set of pictures for which max and min POC counts are computed and the difference range constraint is imposed, but it was not put into the text that way. J. Boyce assessed the consensus that this was agreed and should be corrected.

The contribution suggested to include only prevRefPic and the current picture, and not to also include pictures in the DPB that are marked as used for short-term reference or waiting for output. J. Boyce assessed that there was no consensus to make this change.

A general comment was made that bitstream constraints are useful for early detection of bitstream errors.

Topic 4. Proposed new SEI message for intra pictures.

It was commented that the encoder could just send more POC LSBs to ensure coherence. It was responded that the presence of this SEI would provide an assurance that the issue was taken care of.

It was asked whether there could be a splicing problem with this – e.g., when discarding some GOPs so that a new bitstream starts at a CRA. It was suggested that sending the POC MSB difference rather than some POC MSB's value might be better.

It was asked how to detect an "intra picture".

It was commented that all RAP pictures are already in decoding order in the bitstream, so this may not be needed.

It was commented that non-RAP intra pictures would not be expected to ordinarily be used.

The AHG recommended this to be for further study with no action to be taken at this meeting.

JCTVC-J0248 On prevRefPic definition [J. Samuelsson, R. Sjöberg (Ericsson)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution proposes to revert the definition of prevRefPic to always be a picture in temporal layer 0, for the sake of simplicity and to correct a problem with TLA pictures having POC values that depend on the presence of prior inappropriate pictures.

Instead of using the previous reference picture that has a temporal_id equal to or less than the temporal_id of the current picture for prevRefPic, this document proposes to use the previous reference picture that has a temporal_id equal to 0.

(Cross check not yet provided.)

It was agreed that the TLA problem exists and needs to be corrected.

The AHG recommended adoption.

#### Slices (5  3)

JCTVC-J0083 AHG9: On slice header parsing overhead reduction [M. Zhou, M. Mody (TI)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

In the current HM7.0 design there is reportedly a large difference (over 100x) in slice header parsing overhead between the typical and worst case. It was asserted that the worst case slice header parsing can be beyond capability of real-time decoding in which slice header parsing is implemented in software. Weighted prediction tables and reference picture parameter sets (RPS) are parsing intensive parts of slice header. To reduce the worst case slice header parsing overhead, this contribution proposes the following two changes: 1) move RPS from slice header to APS, 2) reduce weighted prediction table cycle overhead by either constraining the size of the reference picture lists (option 1), or enabling signalling of default weighted prediction tables in APS and limiting the number of slice headers per picture which can override weighted prediction tables signalled in APS (option 2). The proposed methods can reportedly reduce the worst case slice parsing overhead by roughly 2x to 3x based on cycle estimate on ARM9.

It was remarked that slice header sharing (e.g. J0109) is related.

(Cross check not yet provided.)

It was commented that since the RPS is the same in all SHs, the decoder can just skip over those bits after parsing them from the first SH.

The weighted prediction syntax aspect needs additional discussion (e.g. "option 1").

JCTVC-J0109 AHG9: Header parameter set (HPS) [Y. Chen, Y.-K. Wang (Qualcomm), M. M. Hannuksela (Nokia)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by G. Sullivan and J. Boyce.)

At the previous JCT-VC meeting, the proposal on slice header prediction in JCTVC-I0070 was discussed. It was noted in the meeting notes that using some kind of parameter set to enable slice header prediction seemed promising. Following such a direction, this document proposes a slice header prediction mechanism based on a so-called header parameter set (HPS), with two slightly different alternative approaches. In the first approach (single-AU HPS), an HPS would be required to be used only within one access unit. In the second approach (multi-AU HPS), an HPS may be used by multiple access units.

It is asserted that the proposed mechanism avoids the drawbacks in the JCTVC-I0070 design, the single-AU HPS approach is asserted to provide similar coding gains as reported in JCTVC-I0070, and the multi-AU HPS approach is asserted to provide opportunities for coding gains with increased complexity.

S. Wenger expressed his plan to submit a cross-check.

This was designed in a similar manner in concept to the previously-proposed APS partial update – multiple HPS IDs are proposed to be carried – one for each of several categories of syntax elements such as RPS selection, prediction weight selection, etc. A flag is proposed for single-slice encoding without using the scheme.

It was commented that the carrying of the multiple IDs seemed a bit complicated.

It was commented that the APS could perhaps be used for this.

The motivation is to provide coding efficiency improvement for multi-slice sharing of header data. There was discussion of loss resilience, but this did not seem to be a significant motivation.

With 6x6 CTB tiles and one slice per tile, the coding efficiency benefit was reportedly approximately 1.5% when reported in a prior contribution (I0070).

Aside from coding efficiency improvement, extensibility for SVC & 3D usage was suggested as a motivation.

It was commented that getting a better understanding of the coding efficiency impact would be needed.

Additional discussion needed in JCT-VC meeting.

JCTVC-J0216 AHG 9: Signalling slice index to detect lost slice earlier [Hendry, B. Jeon (LG)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

This contribution discusses the possibility of detecting lost slices earlier when multiple slices are used per picture. The following changes are proposed:

- Replacing the current syntax element first_slice_in_pic_flag (coded with u(1)) with slice_idx_in_pic (coded with ue(v)).

- Adding a flag last_slice_in_pic_flag (coded with u(1)).

Coding efficiency results without loss reportedly show that the proposed modification affects BD-rate on average 0.1%Y, 0.1%U, 0.1%V for all intra and random access cases (AI-HE10, AI-Main, RAHE10, RA-Main), and 0%Y, 0%U, 0%V for low delay B cases.

It was asked how it would be useful to know, e.g., that the 2nd and 3rd slices were lost (without knowing which CTBs were in those slices until parsing the 1st slice).

It was commented that if a back-channel was available, the slice ID number might be useful to send.

It was commented that the system layer would ordinarily provide a packet counter functionality.

No action on this recommended by the AHG.

(Cross check not yet provided.)

JCTVC-J0416 AHG 9: Cross-check of J0216 - Signalling slice index to detect lost slice earlier [A. K. Ramasubramonian (Qualcomm)] [late] [miss]

JCTVC-J0217 On dependent slices [T. Lee, J. Park (Samsung)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

In the previous meeting, dependent slice support was adopted. According to the specification, a dependent slice cannot be used with entropy slice while it can be used with either WPP or tile within a set of pictures. Since entropy slice is regarded as similar tools to WPP or tile to enable parallel processing, it is proposed to enable dependent slice with entropy slice within a set of pictures for consistency. Or alternatively, it is proposed to restrict the usage of dependent slices to be used only if WPP is used.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

It was commented that entropy slices and dependent slices are somewhat overlapping in functionality, which is why their use is mutually exclusive.

Sub-picture ultra-low-delay was mentioned as a potential motivation for dependent slices (e.g. in combination with tiles or in non-WPP usage). Document J0264 was mentioned as providing further information.

As a syntax cleanup only, conditional coding of dependent_slice_enabled_flag was proposed in the PPS did seem to make sense, and was recommended for adoption by the AHG. No other action on this was recommended by the AHG. Decision: Adopted (assuming cross-check availability).

JCTVC-J0255 AHG9: Slice Prefix for sub-picture and slice level HLS signalling [T. Schierl, V. George, R. Skupin, D. Marpe (HHI)]

(This was initially reviewed in the AHG9 meeting, where discussion of this was chaired by Y.-K. Wang and G. Sullivan.)

In this document it was proposed to apply a functionality like SEI messages as well as additional high level syntax items, beyond the ones included in the NAL unit header, on a per slice level basis. Such messages were proposed as a "slice prefix NAL unit". Syntax and semantics of the slice prefix and slice-level/sub-picture SEI messages, use cases for low delay/sub-picture CPB operations, tile signalling and region of interest (ROI) signalling were described.

Y.-K. Wang expressed a basic understanding of the proposal prior to the availability of a more formal indication of a cross check.

Several potential sub-picture-level SEI messages were described.

It was commented that during the design of AVC the possibility was discussed to have slice-specific SEI messages or otherwise not require SEI messages to be at the beginning of the picture, but this was not done (for simplicity) because there was no clear need for it at the time.

It was commented that even today we still have reserved NUTs in AVC that could be used for such a purpose.

It was commented that we could just allow SEI NUT to follow the first VCL NALU of the picture, rather than using a different NUT for the slice level SEI than for the picture-level SEI.

Some participants commented that ultra-low-delay decode could benefit from definition of a timing SEI specification.

A participant suggested to also enable some SEI type of NALU that could follow after the relevant VCL NALU or the AU rather than precede it – e.g., for checksum data.

The general concept was supported by most participants in the AHG, and there was further discussion in the JCT-VC meeting.

Possible concepts:

- Allow ordinary SEI NUT to follow the first VCL NAL unit and precede the last VCL NAL unit.

- Define a prefix SEI NUT (or more than one of these, or combined with above concept).

- Define a suffix SEI NUT.

For items 2 & 3 above, we already have NUTs defined that have these properties – all that remains would be to "unreserve" some – which does not necessarily need to be done in the first edition of the standard.

For approach 1, we would need to affect edition 1.

This proposal had some specific proposed slice-level SEI messages.

- Sub-picture timing

- Sub-picture buffering period

- Tile information

- Tile dimensions

- Region of interest

The first of these seemed to have the most interest expressed by participants. Some related proposals are discussed in section 5.12.9.1. Further study of the others was encouraged.

Proper and complete text was needed for the sub-picture timing topic. Open for follow-up.

#### Reference picture list (RPL) and weighted prediction (WP) parameters (8 – done)

JCTVC-J0055 AHG9: Weighted Prediction Parameter Signalling [Yong He, Yan Ye (InterDigital)]

In HEVC draft 7, the explicit Weighted Prediction parameters in B slices are always signalled for both reference picture list 0 and reference picture list 1 after list combination was removed at the 9th JCT-VC meeting. In this contribution, two revised WP parameter signalling methods are proposed. Up to two 1-bit flags are added to pred_weight_table(), with the first 1-bit flag indicating if any WP parameters are signalled for the entire list 1, and the second 1-bit flag indicating if any WP parameters are signalled for a particular entry on list 1. Simulation results reportedly show that, for fade-white sequences, when compared to HM7.0 with weighted prediction being used, the proposed signalling method 1 achieves BD rate of −0.1% and −1.6% for random access main and for low-delay B main, respectively. The proposed signalling method 2 reportedly achieves BD rate of −0.2% for random access main and −1.6% for low delay B main with weighted prediction enabled. Similar RD performance gains are observed for HE10 configurations and for fade-black sequences.

For this proposal, the extra syntax is sent only when weighted prediction is used and the length of the two lists is the same. The only syntax elements gated by the flag are the prediction weight syntax elements.

No action taken.

JCTVC-J0223 Cross-check of Weighted Prediction Parameter Signalling (JCTVC-J0055) [A. Tanizawa, T. Chujoh (Toshiba)] [late]

JCTVC-J0120 AHG9 & AHG13: Identical reference picture lists [Y. Chen, M. Coban, Y.-K. Wang, M. Karczewicz (Qualcomm)]

In HEVC, for B slices, there are chances when RefPicList0 and RefPicList1 are identical, and when explicit weighted prediction is in use, the prediction weights are also identical. It is proposed to indicate this and thus reduces the bits for signalling RefPicList1 and its prediction weights. It is reported that the proposed algorithm saves about 48% for the number of bits used to signal weighted prediction parameters for the LB case and about 10% for the RA case.

It proposes to send a flag in each slice header of B slices.

This increases overhead whenever the flag is 0 in order to save overhead when the flag is 1.

In discussion, it was noted that a condition could be put at the SPS or PPS level to determine whether the flag is sent.

The overall effect in the CTC is unknown – probably roughly negligible. When WP is used and the modified lists are identical, it would save probably roughly 1-3%.

No action.

JCTVC-J0504 AHG9 & AHG13: Cross-check of JCTVC-J0120: Identical reference picture lists [Philippe Bordes, Pierre Andrivon (Technicolor)] [late]

JCTVC-J0182 AHG 9 / AHG 13: On identical reference picture lists [Hendry, Y. Jeon, S. Park, B. Jeon (LG)]

Roughly similar to J0120 – no action taken.

- JCTVC-J0222 AHG9: Improved weighted prediction parameter signaling [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)]

Somewhat similar in spirit to J0120, J0055, J0182 – no action taken.

- JCTVC-J0295 Cross-check of improved weighted prediction parameter signalling (JCTVC-J0222) [Yong He (InterDigital)] [late]

JCTVC-J0221 AHG9: Clean-up of semantics and decoding process on weighted prediction [A. Tanizawa, T. Chujoh, T. Yamakage (Toshiba)]

First aspect suggests an editorial improvement, which is delegated to the editors.

Second aspect proposes to limit the offset syntax element for chroma to the range from −512 to +511.

Decision: Adopt this range limit.

JCTVC-J0252 AHG9: Cross-check of JCTVC-J0221: Clean-up of semantics and decoding process on weighted prediction [P. Bordes, P. Andrivon (Technicolor)]

JCTVC-J0503 AHG9: Simplification of weighted prediction signaling in PPS [P. Bordes, P. Andrivon (Technicolor)] [late]

In HEVC, the signaling of Weighted Prediction in the Picture Parameters Set uses two flags: one for slices P and one for slices B. It is proposed to merge these two flags into one single flag.

This change seems unnecessary – no action taken.

JCTVC-J0525 Mental cross-check of JCTVC-J0503 (AHG9: Simplification of weighted prediction signaling in PPS) [Yan Ye (??)] [late]

- JCTVC-J0119 AHG13: On reference picture list modification [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)]

In this contribution, a change of the reference picture list modification (RPLM) design is proposed. It is reported by the proponents that, for test cases 2.8 and 3.5 in the common test conditions for reference picture marking and list construction proposals in JCTVC-H0725, 24% bit reduction of RPLM bits was achieved for the low-delay configuration compared to the RPLM method in HEVC WD7, and the performance is the same for the random access configuration. It is further that the proposed RPLM method, when applied to HEVC-based 3DV, outperforms the RPLM method in HEVC draft 7, when applied to 3DV, with 34% bit reduction on average for non-base views under the 3DV common test conditions.

The AVC-like method of RPLM was suggested to be better when the lists are long.

The current proposal is simpler than what was in WD 5. The biggest difference between the proposal and the current scheme is the ability to terminate the syntax loop before exhaustively listing every picture in the list.

However, we also have a desire for stability, and currently don't seem to have a strong need for long lists (with our current profile plan).

No action taken on this.

- JCTVC-J0446 Cross-check report of AHG13: On reference picture list modification (JCTVC-J0119) [Sue M. T. Naing, Chong Soon Lim (Panasonic)] [late]

### Reference picture set (8  4)

JCTVC-J0513 Common conditions for reference picture marking and list construction proposals [R. Sjöberg, Y.-K. Wang, M. Hannuksela, T.K. Tan, Y. Ye] [late]

TBP.

JCTVC-J0185 AHG13: On short_term_ref_pic_set [S Lu, K Sato (Sony)]

Short_term_ref_pic_set was asserted to contain the following redundancies:

- inter_ref_pic_set_prediction_flag is transmitted even if idx = 0, where inter_ref_pic_set_prediction cannot be applied

- in common test condition the value of delta_idx_minus1 remains 0 for all idx values but is transmitted for all idx values

In this contribution it is proposed to modify syntax related to short_term_ref_pic_set to remove above redundancies.

This was closely related to J0234. See the notes in the section on that document.

Cross-check in J0517.

JCTVC-J0517 Mental cross-check of JCTVC-J0185 (On short_term_ref_pic_set) [Chong Soon Lim (Panasonic)] [late]

JCTVC-J0234 Inter-RPS complexity reduction [J. Ström, R. Sjöberg, J. Samuelsson (Ericsson)]

This contribution proposes two changes to how inter-RPS parameters are encoded, and one change that removes an alleged underspecification. The first proposed change is to not transmit the variable delta_idx_minus1 when encoding inter-RPS parameters in the SPS, and instead infer that it has the value 0. The variable is still transmitted when encoding inter-RPS parameters from the slice header. The second proposal is to signal a flag called used_by_curr_present_in_inter_rps_flag in the SPS. If this flag is false, all inter-RPS coded reference pictures are assumed to have used_by_curr_pic_flag[ j ] = 1 and this means that the flag does not need to be transmitted. Both proposals have been tested on the entire AHG13 set of configuration files as specified in JCTVC-I0608. The proposal claims to lower the average complexity slightly and reduce the number of bits spent on sending the RPS data in the SPS with −11.8%. Separately, the first and second proposals are reported to reduce the number of bits by −7.0% and −4.8% respectively. The third proposal is to add semantics for what happens if the RPS row to predict from does not exist; then the empty set will be used for prediction.

Cross-check to be provided by Docomo.

Several participants expressed a preference for elements #1 and #3 of this proposal relative to the current draft and relative to J0185.

The element #2 of the contribution was less well supported.

Tentative plan is to adopt #1 and #3 (and not #2) of this proposal. Revisit to confirm.

JCTVC-J0545 Mental cross-check of JCTVC-J0234: Inter-RPS complexity reduction [TK Tan (NTT Docomo)] [late]

JCTVC-J0115 AHG13: Signalling of long-term reference pictures in the slice header [A. K. Ramasubramonian, Y.-K. Wang, R. L. Joshi, Y. Chen (Qualcomm)]

It was reported that there is a problem with the signalling of long-term reference pictures (LTRPs).

An error-resilience problem is reported for the current signalling and derivation of long-term reference pictures (LTRPs). It is stated that the root of problem was that the reference picture set (RPS) derivation depends on the status of the decoded picture buffer (DPB), which was the main reason for replacing the sliding-window and memory management control operation (MMCO) based reference picture buffer management mechanism in AVC with the RPS based mechanism. This document proposes to send the delta POC values, either between the current picture and the LTRP or between the LTRP and the previous random access point (RAP) picture in decoding order to solve the reported error resiliency issue, and to satisfy the RPS design principle that the RPS derivation is self-contained (i.e., not depending on the DPB status). The simulation results are in the attachment of the proposal document.

Two cross-checks planned (J0402 and J0530), but not yet submitted when reviewed.

The current syntax can (only) have some case of non-robust behaviour if the encoder chooses to put it in that mode (not sending MSB cycle difference) and re-uses the same POC LSBs for multiple LTRPs.

The proposal would require always sending the MSB cycle difference rather than allowing the encoder to make this choice, and offers a way to code that difference more efficiently if the LTRP is close in position to the preceding RAP. These two aspects can, in principle, be considered as two separate suggestions.

There was no consensus that the proposed changes were appropriate to do. No action taken.

JCTVC-J0402 Crosscheck of Signalling of long-term reference pictures in the slice header in JCTVC-J0115 [Hendry, B. Jeon (LG)] [late] [miss]

JCTVC-J0530 AHG13: Crosscheck for Signalling of long-term reference pictures in the slice header (JCTVC-J0115) [??] [late]

JCTVC-J0116 AHG13: Signalling of long-term reference pictures in the SPS [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), C. S. Lim (Panasonic), S. Deshpande (Sharp) , Hendry (LG)]

This document proposes to enable the inclusion of candidate long-term reference pictures, as part of the reference picture set signalling in the sequence parameter set. Two different options are provided. The document reports that, for test condition 2.6 in JCTVC-H0725, the proposed method uses 36% and 42% fewer bits, respectively for the two proposed options, to signal the syntax elements related to long-term reference pictures in the sequence parameter set and the slice header, when compared to the signalling in HEVC text specification draft 7.

The first presented option was a design considered at the preceding meeting in I0340, for which further study had been suggested. The second was a design that had also been in a version of I0340.

(A four-company proposal – also supported by additional non-proponents.)

Decision: Adopt "option 2" with u(v) coding of lt_idx_sps.

JCTVC-J0480 Crosscheck of long term reference pictures in SPS (JCTVC-J0116) [TK Tan (??)] [late]

JCTVC-J0118 AHG13: On signalling of MSB cycle for long-term reference pictures [A. K. Ramasubramonian, R. L. Joshi, Y.-K. Wang (Qualcomm)]

This document proposes changes to the semantics of the syntax elements poc_lsb_lt[ i ], delta_poc_msb_present_flag[ i ], and delta_poc_msb_cycle_lt[ i ] to improve the efficiency of signalling the MSB cycle for long-term reference pictures in the slice header. Changes to the RPS derivation process are also proposed, including swapping of the order of STRP and LTRP subset derivations, such that the STRP subset is derived first.

If the proposal in JCTVC-J0115 is adopted, then this proposal is claimed to become irrelevant and should be ignored.

No experiment results were provided.

Revisit semantics of delta_poc_msb_present_flag.

JCTVC-J0538 Mental Cross-check for JCTVC-J0118 [S. Deshpande (Sharp)] [late]

JCTVC-J0164 AHG 13: Simplified signalling of MSB cycle [Hendry, B. Jeon (LG)]

TBP

JCTVC-J0539 Mental Cross-check for JCTVC-J0164 [S. Deshpande (Sharp)] [late]

### Video parameter set (VPS) and sequence parameter set (SPS) (9)

A BoG (coordinated by J. Boyce) was asked to initially review the contributions in this area along with the remaining issues in the NUH category (section 5.21.1).

JCTVC-J0074 AHG10 Hooks for Scalable Coding: Sequence Parameter Set Design [M. M. Hannuksela (Nokia)]

It is asserted that the SVC and MVC extensions of H.264/AVC have at least the following shortcomings when it comes to high-level syntax design.

- Extending H.264/AVC, SVC, and MVC with new scalability types, such as depth views, has been and is complicated due to different assignments of NAL unit types to VCL and non-VCL NAL units, the HRD being dependent on the assignment of NAL units to VCL and non-VCL NAL units, and the sub-bitstream extraction process(es) ignoring any future scalable extensions.

- A different sequence parameter set is needed even if very few syntax element values (e.g. only profile and level indications) change between layers (in SVC) or views (in MVC).

The following kinds of modifications are proposed to reportedly avoid problems similar to those faced with SVC and MVC:

- It is proposed to reserve a few NAL unit type values specifically for VCL NAL units, e.g. NAL unit types 9 to 12.

- It is proposed to specify a sub-bitstream extraction process for HEVC version 1 with temporal_id and a set of reserved_one_5bits values as inputs.

- Sequence parameter set RBSP may use temporal_id greater than 0 to convey proper profile and level information and HRD parameters for temporal_id-based bitstream subsets.

- Sequence parameter set syntax and semantics are modified to allow copying syntax elements other than profile and level indications from another sequence parameter set of the same seq_parameter_set_id.

- The HRD parameters for conformance are taken from the sequence parameter set of the highest layer (even if were not decoded).

Regarding aspect number 5, there was some discussion of how it would be possible to deactivate a layer after a new CVS begins that has the same SPS ID as used in some prior CVS.

It was remarked that operation point definitions in the VPS may be a way to address some of these aspects. Another participant remarked that this may mean that the base layer decoder needs to pay attention to the VPS.

Some participants remarked that aspect number 3 may not be necessary, as there could be other ways to deal with this.

It was remarked that aspect number 4 may not be completely necessary, and seems dependent on aspect number 3.

A participant suggested that it would be desirable to examine which parts of the proposed text are associated with which aspects of the proposal.

Aspect number 2 seemed the most generally supported by the group. Aspect number 1 was also suggested as potentially ready for action.

Revisit.

JCTVC-J0459 AHG10: Mental cross-check of JCTVC-J0074 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0075 AHG10 Hooks for Scalable Coding: Video Parameter Set Design 	[M. M. Hannuksela (Nokia)]

The video parameter set (VPS) is proposed to be extended in scalable extensions of HEVC to contain:

- The dependencies between layers (also referred to as component sequences in this contribution).

- The mapping of reserved_one_5bits, i.e. layer or component sequence/picture identifier, to specific scalability properties (e.g. dependency_id, quality_id, view order index).

Two alternative approaches are proposed for the indicating the dependencies between the layers and the mapping of scalability properties to layer identifiers:

- Cross-layer VPS describing the dependencies of between layers of the entire coded video sequence and the properties of all layers. A single VPS is active for all layers. If layers are extracted from the bitstream, the cross-layer VPS may describe layers that are no longer present in the bitstream.

- Layered VPS describing the dependencies and properties of a single layer. The layered VPS NAL unit uses reserved_one_5bits and hence VPS NAL units are extracted along with other layer-specific NAL units in sub-bitstream extraction. A different VPS is active for each layer, although the same vps_id is used in all active VPSes.

The cross-layer VPS design does not require changes in HEVC version 1, while it is asserted that the vps_max_layers_minus1 syntax element becomes redundant in the layered VPS design and can be removed.

A participant expressed some skepticism about the need for the layered VPS mechanism.

No action was requested for version 1.

JCTVC-J0460 AHG10: Mental cross-check of JCTVC-J0075 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0114 AHG10: On video parameter set [Y. Chen, Y.-K. Wang (Qualcomm)]

The video parameter set (VPS) was adopted into HEVC, and includes mainly sequence-level temporal scalability related information. This document proposes a changed VPS syntax as well as the corresponding changes in SPS (including VUI) and slice header syntaxes, to enable the use of VPS in session negotiation as well as to reduce the number of bits needed for the representation of SPSs. It is asserted that since SPSs in many application scenarios are transmitted out-of-band, which means the smaller the overall size of all the SPSs the shorter the initial delay, as out-of-band transmission is reliable at the cost of increased initial delay in error-prone environment. An example design of the VPS for future extensions based on the VPS proposed in this document is included in JCTVC-J0124.

The contribution suggests that it is reasonable to minimize the size of VPS data (and SPS data).

The proposal includes making VPS fundamental to the base layer.

The design intent issue of making the VPS be a system-level characteristics description versus being decoding configuration data was discussed.

A concept predicting data between VPS and SPS was discussed.

Revisit.

JCTVC-J0502 AHG10: Mental cross-check of JCTVC-J0114 (Video parameter set HEVC base specification) [M. M. Hannuksela (Nokia)]

JCTVC-J0124 AHG10: On video parameter set for HEVC extensions [Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-J0196 AHG10: Proposed modification to video parameter set [K. Sugimoto, S. Sekiguchi (Mitsubishi)]

JCTVC-J0438 AHG10: Cross-check of JCTVC-J0196 [J. Xu (Microsoft)] [late]

JCTVC-J0257 AHG9/AHG10: Design of the Video Parameter Set [R. Skupin, V. George, T. Schierl]

JCTVC-J0484 AHG9/AHG10: Mental cross-check of JCTVC-J0257: Design of the Video Parameter Set [J. Boyce (Vidyo)] [late]

JCTVC-J0261 AHG9: Signalling of VPS Activation [T. C. Thang (UoA), J. W. Kang, H. Lee, J. Lee, J. S. Choi (ETRI)]

JCTVC-J0481 Mental crosscheck for JCTVC-J0261 Signaling of VPS Activation [Hendry, B. Jeon (LG)] [late]

JCTVC-J0270 HEVC VUI Parameters with Extension Hooks  [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

JCTVC-J0528 Mental cross-check of JCTVC-J0270: HEVC VUI Parameters with Extension Hooks [J. Boyce (Vidyo)] [late]

JCTVC-J0487 Scalable Video Coding Signalling in VPS [A. Luthra (Motorola Mobility)] [late]

JCTVC-J0546 AHG9/10: vps_id in slice header (partial re-proposal of JCTVC-I0524) [M. M. Hannuksela (Nokia)] [late]

### Miscellaneous high-level syntax topics and syntax cleanups (12)

#### APS loss detection (1)

JCTVC-J0072 AHG9 High-Level Syntax: APS loss detection [M. M. Hannuksela (Nokia)]

JCTVC-J0457 AHG9: Mental cross-check of JCTVC-J0072 [Y.-K. Wang (Qualcomm)] [late]

#### Motion vector prediction related high-level syntax design (4)

JCTVC-J0071 High-level Syntax: Motion vector prediction issue for long-term reference picture [Y. Takahashi, O. Nakagami, T. Suzuki (Sony)]

JCTVC-J0356 Cross-verification on motion vector prediction issue for long-term reference picture (JCTVC-J0071) [I.-K. Kim (Samsung)] [late]

JCTVC-J0527 AHG10: Mental cross-check of JCTVC-J0071 (High-level Syntax: Motion vector prediction issue for long-term reference picture) [Y. Chen (??)] [late]

JCTVC-J0187 On temporal_mvp_enable_flag [K Sato (Sony)]

JCTVC-J0361 Cross-check report of JCTVC-J0187 on temporal_mvp_enable_flag [S.-C. Lim, H. Y. Kim, J. Lee (ETRI)] [late]

#### Multi-topic high-level syntax documents (1)

JCTVC-J0290 High layer syntax issues [C. Fogg (Harmonic), A. Wells (Ambarella)]

#### Syntax cleanups (6)

JCTVC-J0183 Syntax Issues [K. Sato (Sony)]

JCTVC-J0518 Mental cross-check of JCTVC-J0183 [H. Aoki (NEC)] [late]

JCTVC-J0220 cu qp delta enabling syntax [T. Lee, J. Park (Samsung)]

JCTVC-J0273 A simple ordering issue for VUI parameters syntax [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

JCTVC-J0531 Mental cross-check of JCTVC-J0273: A simple ordering issue for VUI parameters syntax [J. Boyce (Vidyo)] [late]

JCTVC-J0300 Slice Header Syntax Cleanup [Yue Yu, Jian Lou, Limin Wang(Motorola Mobility)]

JCTVC-J0495 Mental cross-check of JCTVC-J0300 [Y.-K. Wang (Qualcomm)] [late]

### High-level parallelism (18)

A BoG coordinated by M. Horowitz was asked to review the contributions in this category.

#### General (2)

JCTVC-J0123 AHG4: On tiles and wavefronts [M. Coban, Y.-K. Wang (Qualcomm)]

JCTVC-J0486 Mental cross-check of On tiles and wavefront parallel processing (JCTVC-J0123) [M. Horowitz (eBrisk)] [late]

JCTVC-J0249 Decoder parallelism indication [J. Samuelsson, R. Sjöberg]

JCTVC-J0479 Mental cross-check of JCTVC-J0249: Decoder parallelism indication [J. Boyce (Vidyo)] [late]

#### Tiles (8  6)

JCTVC-J0039 AHG4/AHG9: Syntax for restricting slices and tiles [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

JCTVC-J0471 Crosscheck of Syntax for restricting slices and tiles (JCTVC-J0039) [M. Coban, W.-J. Chien (Qualcomm)] [late]

JCTVC-J0042 AHG4/AHG9: Syntax for restricting tile numbers and tile sizes [C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

JCTVC-J0412 Cross-check of “AHG4/AHG9: Syntax modifications for tile width constraint” (JCTVC-J0042) [M. Horowitz, S. Xu (eBrisk)] [late]

JCTVC-J0085 AHG9: On number of tile rows limit [M. Zhou(TI)]

JCTVC-J0088 AHG4: Enable parallel decoding with tiles [M. Zhou (TI)]

JCTVC-J0206 AHG 4: Asynchronous Tile Output [Hendry, B. Jeon (LG)]

JCTVC-J0423 Crosscheck of Asynchronous Tile Output (JCTVC-J0206) [G. Clare, F. Henry (Orange Labs] [late]

JCTVC-J0209 AHG 4: Constraint for slice and tile [Hendry, J. Kim, B. Jeon (LG)]

JCTVC-J0485 Mental cross-check of AHG 4: Constraint for slice and tile (JCTVC-J0209) [M. Horowitz (eBrisk)] [late]

JCTVC-J0235 On tile size restrictions in HEVC [A. Fuldseth (Cisco)]

JCTVC-J0383 A cross-check report for JCTVC-J0235 on tile size restriction in HEVC [K. Misra, K. Seung-Hwan, A. Segall (Sharp)] [late]

#### Wavefront parallel process (WPP) (4)

JCTVC-J0032 An HEVC transcoder converting non-parallel bitstreams to/from WPP [G. Clare, F. Henry (Orange Labs)]

JCTVC-J0040 AHG4/AHG9: Syntax for restricting slices and WPP [C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei (MediaTek)]

JCTVC-J0472 Crosscheck of Syntax for restricting slices and WPP (JCTVC-J0040) [M. Coban, W.-J. Chien (Qualcomm)] [late]

JCTVC-J0279 CU skip and split CABAC context modification for flexible wavefront parallel parsing [M. Coban, W.-J. Chien, M. Karczewicz (Qualcomm)]

JCTVC-J0425 AHG4: Improving parallelization efficiency of WPP using Overlapped Wavefront [M. Alvarez-Mesa, V. George, T. Schierl (HHI), C. C. Chi, B. Juurlink (TU Berlin)] [late]

#### Entry point signalling (5)

JCTVC-J0033 WPP entry points simplification [G. Clare, F. Henry (Orange Labs)]

JCTVC-J0162 Crosscheck for JCTVC-J0033 [Hendry, B. Jeon (LG)]

JCTVC-J0081 AHG4: Signaling sub-stream entries in APS for parallel decoding [M. Zhou (TI)]

JCTVC-J0299 AHG4: On entry point coding [S.H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-J0475 Cross-check of JCTVC-J0299 - On entry point coding [A. Fuldseth (Cisco)] [late] [miss]

JCTVC-J0322 AHG4: Entry point signaling for wavefront substreams within tiles [K. Misra, K. Seung-Hwan, A. Segall (Sharp)]

### DPB size and other level related limits (9)

#### DPB size (6)

JCTVC-J0111 AHG9: On DPB size [R. L. Joshi, A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-J0163 On DPBsize reduction [S. Lu, T. Suzuki (Sony)]

JCTVC-J0417 AHG9: Cross-check report for On DPBsize reduction (JCTVC-J0163) [H. Sasai, T. Nishi (Panasonic)] [late]

JCTVC-J0199 AHG9: Comments on DPB Size [Thomas Davies, Arild Fuldseth (Cisco)]

JCTVC-J0210 MaxDpbSize setting based on the picture size [A.Fujibayashi, T.K.Tan (NTT Docomo)]

JCTVC-J0283 Field Coding and MaxDpbSize constraints [D. Flynn, A. Gabriellini, M. Mrak, M. Naccari (BBC)]

JCTVC-J0367 On level considerations for interlaced content coding [Jérôme Viéron, Pierre Larbier, Jean-Marc Thiesse (Ateme)]

JCTVC-J0496 Variable MaxDpbSize based on maximum picture size and maximum number of reorder pictures [Arturo Rodriguez, A. K. Katti, H-Y Hwang (??)] [late]

#### Other limits (4)

JCTVC-J0059 Suggested constraint on number of bits/bins per LCU [K. Chono (NEC)]

JCTVC-J0080 AHG9: On number of slices per picture limit [M. Zhou (TI)]

JCTVC-J0082 AHG9: On number of bits per LCU limit [M.Zhou, V. Sze (TI)]

JCTVC-J0442 Summary of discussion and input contributions on constraint on number of bits/bins per LCU [??] [late]

### Hypothetical reference decoder (HRD) (5)

#### Ultra-low-delay HRD

JCTVC-J0136 AHG9: Improvement of HRD for sub-picture based operation [K Kazui, J Koyama, S Shimada, A Nakagawa (Fujitsu)]

JCTVC-J0541 Mental Cross-check for JCTVC-J0136 - Improvement of HRD for sub-picture based operation [S. Deshpande (Sharp)] [late]

JCTVC-J0272 Simplifications of HRD parameters for Temporal Scalability [Munsi Haque, Kazushi Sato, Ali Tabatabai, Teruhiko Suzuki (Sony)]

JCTVC-J0534 Mental Cross-check for JCTVC-J0272 - Simplifications of HRD parameters for Temporal Scalability [S. Deshpande (Sharp)] [late]

JCTVC-J0306 AHG10: On Sub-picture Based CPB [S. Deshpande (Sharp)] [late]

JCTVC-J0465 AHG9: Mental cross-check of JCTVC-J0306 [Y.-K. Wang (Qualcomm)] [late]

#### Other HRD aspects

- JCTVC-J0137 AHG9: New high-level syntax for simple HEVC stream editing [K Kazui, J Koyama, S Shimada, A Nakagawa ((Fujitsu)]

This contribution proposes changes of high-level syntax relating to simple HEVC streaming editing.

The proposed changes are 1) adding alternative parameters of cpb_removal_delay and dpb_output_delay for TFD picture removal, 2) adding specification of an SEI message for indicating editing point, and 3) defining a parameter indicating continuity of bitstream at BLA picture.

Cross-checked in J0476 (the author of which was not available for the discussion).

It was commented that this has a relationship to J0444.

In discussing the proposed SEI message, it was questioned whether we need to support the envisioned operation where a bitstream is chopped at an arbitrary position in decoding order without regard to the consequences of this action.

It was suggested that the proposed functionality could be addressed by metadata definition after finalization of version 1.

For the first two aspects, further study was recommended.

Revisit the third aspect.

- JCTVC-J0476 AHG9: Cross-check of JCTVC-J0137 [H. Aoki (NEC)] [late]

JCTVC-J0444 Repeat picture output and HRD management [A. K. Katti, H.-Y. Hwang (Cisco)] [late]

This contribution proposes signalling that conveys repeated picture output and the corresponding HRD operation when fixed_pic_rate_flag is equal to one. The signalling is asserted to be simpler or necessary for proper HRD operation of a coded video sequence (CVS) corresponding to a source with a picture rate lower than the intended fixed picture output rate, such as when the source corresponds to 24 Hz film content. The proposed technique was asserted to also solve potential issues with DPB management when a bitstream transitions from one CVS to another CVS, the no_output_of_prior_pics_flag is not equal to one, and the fixed picture output rate does not change.

The contribution proposes syntax to indicate a number of repetitions of picture output for each picture that is output.

The contribution had an assumed coupling of POC to time that is not found in the current draft. This did not seem to be fundamental to the concept of the proposal.

The syntax was proposed to be added to the slice header. It was suggested that such information might be more appropriate to put such information into an SEI message.

It was commented that similar information was previously expressed in pic_struct in the picture timing SEI message of AVC.

It was remarked that coding a whole-picture skip picture could be a way to produce repeated output. However, it was noted that this involves actually decoding pictures at a higher frame rate.

The marking of pictures as "needed for output" is proposed to be changed so that a picture remains held in the DPB until its final repeated output time.

It was remarked that a splicer could insert "dummy pictures" at splice points.

A significant part of the subject is a question of drawing the appropriate boundary between the decoding process and a post-decoding adaptation for a display process.

It was remarked that there may be some conflict with the way the clock tick is used here and how it is defined to be used in the HRD.

Further study was encouraged.

JCTVC-J0553 Mental cross-check of Repeat picture output and HRD management (JCTVC-J0444) [Lowell Winger (??)] [late]

JCTVC-J0548 AHG10: On HRD parameters for (temporal) sub-layers [M. Haque (Sony), Y.-K. Wang (Qualcomm), M. M. Hannuksela (Nokia)] [late] [miss]

### VUI and SEI messages (9)

#### Video usability information (VUI) (1)

JCTVC-J0287 Additional sample_ratio_idc values for square samples [Arturo Rodriguez (Cisco)]

JCTVC-J0554 Mental cross-check of Additional sample_ratio_idc values for square samples (JCTVC-J0287) [Lowell Winger (??)] [late]

JCTVC-J0555 Mental cross-check of Additional sample_ratio_idc values for square samples (JCTVC-J0287) [Lowell Winger (??)] [late]

(duplicate?)

#### Frame packing arrangement (FPA) SEI message and related (4)

JCTVC-J0070 2D compatible frame packing arrangement SEI [O. Nakagami, T. Suzuki (Sony)]

JCTVC-J0536 AHG9: Mental cross-check of 2D compatible frame packing arrangement SEI (JCTVC-J0070) [M. Arena (??)] [late]

JCTVC-J0073 AHG9 High-Level Syntax: Graceful Handling of Frame-Packed Pictures by Ignorant Decoders [M. M. Hannuksela (Nokia)]

JCTVC-J0491 AHG9: Mental Cross-check for JTCVC-J0073 - Graceful Handling of Frame-Packed Pictures by Ignorant Decoders [S. Deshpande (Sharp)] [late]

JCTVC-J0458 AHG9: Mental cross-check of JCTVC-J0073 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0198 AHG9: Frame packing arrangement SEI message modification [M. Arena, P. Sunna (RAI)]

JCTVC-J0540 Mental cross-check of JCTVC-J0198 [O. Nakagami, T. Suzuki (Sony)] [late] [miss]

JCTVC-J0247 On frame packing arrangement SEI [B. Choi, C. Kim, J. Kim, J. Park (Samsung)]

JCTVC-J0542 Mental Cross-check for JCTVC-J0247 - On frame packing arrangement SEI [S. Deshpande (Sharp)] [late]

#### Other SEI messages (4)

JCTVC-J0038 Exif data SEI message [K. Ugur, J. Lainema, M. Hannuksela (Nokia)]

JCTVC-J0456 AHG9: Mental cross-check of JCTVC-J0038 [Y.-K. Wang (Qualcomm)] [late]

JCTVC-J0064 SEI messages for gradual decoding refresh [H. Aoki, K. Chono (NEC)]

JCTVC-J0483 AHG9: Mental cross-check of JCTVC-J0064 [K.Kazui (Fujitsu)] [late]

JCTVC-J0149 Signalling of Luminance Dynamic Range in Tone mapping information SEI [S. Hattori, T. Suzuki (Sony)]

JCTVC-J0245 SEI Message for profile and level signaling for temporal scalability and extensions [J. Boyce, D. Hong, W. Jang]

JCTVC-J0507 Crosscheck of JCTVC-J0245: SEI Message for profile and level signaling for temporal scalability and extensions [R. Sjöberg (Ericsson)] [late]

## Quantization

### QP prediction / delta QP coding

JCTVC-J0060 AHG5: Simplified cu_qp_delta coding [K. Chono (NEC)]

This contribution presents a simplified cu_qp_delta coding based on EG0 binarization. The simplified scheme maps a given cu_qp_delta into the unsigned value and represents it with the EG0 binary string. Only the first bin of the EG0 binary sequence is context-coded; the rest are bypass-coded. Thus it reduces the number of context-coded bins of cu_qp_delta in worst-case scenario from 11% to 2%. It is reportedly shown that the simplified scheme changes BD-rate results by 0.20% on average in AHG5 common test conditions.

The relatively large loss (compared to other proposals) is asserted to be due to change in mode decision.

JCTVC-J0429 AHG5: Cross-check of simplified cu_qp_delta coding with EG0 (JCTVC-J0060) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-J0089 AHG5: Bin reduction for delta QP coding [V. Sze, M. Budagavi (TI), V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

To improve the throughput of the CABAC, the number of context coded and bypass bins should be reduced.  Hardware is designed for the worst case, thus it is also important to reduce the worst case number of context coded bins.  In HM-7.0, cu_qp_delta accounts for 18% of the context coded bins in the worst case.  This is due to the fact that it is binarized with truncated unary and all the bins are context coded.  This contribution proposes using a combination of truncated unary and Exp-Golomb coding (TUEGk), similar to what is already used for motion vector difference.  Only the truncated unary portion is context coded and the Exp-Golomb portion is bypass coded. Furthermore, the bypass bins are grouped for throughput improvement; specifically, the sign bin is signaled after the magnitude bins. These modifications were implemented in HM-7.0 for TU with cMax=5 and Exp-Golomb, with k=0. Asserted benefits of this proposal include: 5x reduction of worst case context bins (from 26 to 5), ~2x reduction of total bin (from 27 to 15), grouping of bypass bins, simplified binarization process, and reduction in number of contexts by 1. The worst case number of bins for 8-bits and 10-bits are the same (0.06 context coded bins/pixel and 0.22 total bins/pixel).  Under the test conditions set by the AHG5 on entropy coding improvements, where cu_qp_delta is signaled for every 8x8 CU, and this proposal has overall BD-rate between −0.01 to 0.03.

Presentation not uploaded.

Main purpose of this change would be to prevent throughput problems in cases where QP is changed frequently (which is not usual but could hypothetically occur)

The test conditions with TM5 like rate control (as in previous quantization CE) were used

Several experts ask whether the results in terms of BD rate would still be similar when delta QP changes would be different from the current?

JCTVC-J0331 AHG5: Cross check of Bin reduction for delta QP coding (JCTVC-J0089) [T. Tsukuba (Sharp)] [late]

JCTVC-J0226 AhG5: Reduction of context bins and bypass bins grouping for cu_qp_delta [K. Terada, H. Sasai, T. Nishi (Panasonic)]

In the HEVC Draft 7, most of bins for the cu_qp_delta are arithmetic coded using the context models. This contribution proposes to use bypass coding instead of context based one for higher throughput and also proposes to concatenate bypass coded bins for parallel processing. The proposed modification was evaluated on top of HM7.0. Experimental result reportedly showed that the proposed modification can reduce context coded bins by 64% for cu_qp_delta and can reduce isolated bypass coded bins by 29% with less than 0.1% BD-rate loss.

Presentation not uploaded.

Different methods are investigated; proponent suggests the method “step2c” where everything is coded in bypass mode. Question: How does it compare versus direct usage of EG0 coding (method of JCTVC-J0060)? A: slightly better.

It is also commented that one more decision stage may be necessary.

JCTVC-J0328 AHG5: Cross-check of reduction of context bins and bypass bins grouping for cu_qp_delta (JCTVC-J0226) [V. Sze (TI)] [late]

Confirms that results match. Different versions are trading throughput versus efficiency.

JCTVC-J0298 AHG5:cu_qp_delta coding [S. H. Kim, L. Kerofsky, A. Segall (Sharp)]

In HM-7.0, cu_qp_delta requires up to 26 and 32 context coded bins in main profile and HE_10 configuration, respectively.  In order to improve the throughput efficiency, this contribution proposes a new efficient binarization method by combining truncated unary and fixed length coding (TUFLC).  Only a predefined number (cMax) of bins are context coded and the rest of the bins are bypass coded.  It is reported that the proposed method reduces the worst case number of context coded bins from 26 (or 32 in HE_10)bins to 4 bins. Moreover, it is reported that the total number of bins are reduced from 17 (or 33 in HE10) to 16 bins for all condition. The complexity reduction results in a BD-rate change from −0.02% to 0.02% on average in AHG5 common test conditions that is asserted to be negligible.

Comparably low loss (compared to other methods).

One benefit is that the length of the code is limited (unlike EG code).

JCTVC-J0506 Cross-check of high throughput binarization for cu_qp_delta coding (JCTVC-J0298) [J. Chen(Qualcomm)] [late] [miss]

Also related: One of the methods proposed in JCTVC-J0194 (see below)

Action on the proposals for modification of delta QP coding:

- It is generally desirable to decrease the possible maximum throughput at minimum loss in compression performance (J0089 and J0298 would give that)

- Structure of binarization should be consistent with schemes used for other syntax elements, e.g. MV differences

Decision: Adopt J0089 (Cmax=5), as it better fits the second criterion, and was already (similarly) proposed in the last meeting.

### Quantization matrices

JCTVC-J0130 HVS Default Quantization matrices for 4x4 DCT/DST [C. Yeo, Y. H. Tan (I2R)]

This contribution proposes a set of default quantization matrices for 4x4 DCT/DST combinations derived from the same HVS based procedure used to obtain default quantization matrices for HEVC. It is claimed that the obtained default quantization matrices for the different combinations of DCT and DST share at least 9 out of 16 entries. It is advocated that this suggests that a single quantization weighting matrix may be sufficient for all combinations of 4x4 DCT/DST for intra luma coding.

JCTVC-J0275 Non-CE1: DST quantization matrix syntax for CE1 Simplification 2 [R. Cohen, A. Vetro (MERL)]

Core Experiment 1 (CE1) is testing two simplifications: Simplification 1 uses the 2-D DST for 4x4 luma Intra transform block, and Simplification 2 is the same except that the 2-D DCT is used when the Intra prediction mode is DC. The purpose of this contribution is to re-propose the syntax in JCTVC-I0419, which supports an additional scaling list for 4x4 luma Intra transform blocks, in case Simplification 2 of CE1 is adopted. This syntax supports two default scaling lists for 4x4 luma Intra transform blocks: one associated with the DCT and the other associated with the DST. It is reported that because Simplification 2 of CE1 does not apply the DCT and DST together in the same 4x4 transform block, it is not necessary to switch between the two scaling lists in a transform block.

Proposes the syntax in JCTVC-I0419, which supports a separate scaling list for 4x4 luma Intra transform blocks – one for the DCT residual case and one for the DST residual case – in the context of the case of "simplification 2" of CE1.

It was asked whether any visual benefit had been shown for this.

The statement made at the last meeting, that a switching of quant matrices dependent on intra mode at 4x4 block level is undesirable, is still valid.

Some doubt is raised whether usage of a separate quant matrix for 4x4 DST does provide a visually distinguishable benefit (the proposed matrices are quite similar between DCT and DST anyway)

The question is also raised how suitable the current DCT default matrices are, as in practice most implementers would use own matrices with adaptation to the video material characteristics.

If CE1 simplification 1 would be adopted,  matrices do not need to be switched, but even in that case the current default matrix should be retained, as no evidence was shown (in terms of visual testing) that it is not appropriate for the 2D DST.

### Other

JCTVC-J0076 Improving HEVC compression efficiency by intensity dependant spatial quantisation [M. Naccari, M. Mrak, D. Flynn, A. Gabriellini (BBC)]

The properties of the human visual system can be exploited to improve the compression efficiency of the HEVC standard. In this proposal the pixel intensity masking of the human eye is used to apply coarser quantisation in darker and brighter image area. The proposed perceptual quantisation tool performs an Intensity Dependant Spatial Quantisation (IDSQ) similarly to the quantisation performed by the Intensity Dependant Quantisation (IDQ) tool presented at the 9th JCT-VC meeting in contribution I0257. However, the IDSQ tool proposed here is designed to make the inverse quantisation step at the decoder independent from the average pixel data which may introduce latency and pipelining refactoring during decoding. The proposed IDSQ has been implemented in the HM-7.0 and its performance is assessed by measuring the bitrate reduction with respect to the HM-7.0 codec. For the same perceptual quality level bitrate reductions of up to 25% are achieved and on average 3.4% across all tested points.

Most gain observed at low QP

In contrast to previous proposal, the adaptive scaling operation is done in the spatial domain, such that it is not necessary to wait with the transform until the prediction is available.

In the discussion, it is mentioned that this is likely to still have more latency than the current design and also requires additional buffers.

Further study, potentially interesting in the context of range extensions.

JCTVC-J0189 Cross check report of IDSQ (J0076) proposed by BBC [A Ichigaya (NHK)]

JCTVC-J0509 Cross-check: Subjective viewing on intensity dependant spatial quantisation (JCTVC-J0076) [K. Sugimoto, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-J0318 Consideration on Chroma QP Range Extension for HEVC version 1 and 2 [J. Xu, A. Tabatabai, K. Sato (Sony)]

No need to present, is covered by the decision on J0342.

JCTVC-J0342 Chroma QP range extension [G. J. Sullivan, S. Kanumuri, J. Xu, Y. Wu (Microsoft)]

This contribution proposes a way to extend the range of QP values used for chroma from [0, 39] to [0, 51]. The table used in the derivation of chroma QP values from luma QP is proposed to be modified to enable the extended range. In addition, the use of slice-level chroma QP offsets is discussed. The proposed scheme is asserted to have no significant impact on BD bit rate for common test conditions, as the modification mostly applies outside of the range of values that is tested in the common conditions. The results reportedly show a very small improvement for luma and a very small loss for chroma. Results are also reported for higher QP values than are used in the common conditions.

This contribution also proposes that offset range limits similar to those specified in AVC (ranging from −12 to +12) should be placed on the chroma QP offset values.

For the default case (offset 0), the ratio of luma versus chroma quant stepsize is suggested to be bouded to 2 (unlike 4 as it is currently)

No effect on current common conditions (except one small change at QPluma=34 which is done for the purpose of simpler implementation of the table).

Current HEVC spec does not have a limit on the offset values (whereas the software has the limit +/- 12 as suggested here)

Decision: Adopt all parts of the proposal.

It was later mentioned by one expert (in the session about deblocking filtering) that the inclusion of chroma QP offset values at the slice layer may make it necessary to store these for the operation of the deblocking filter (provided they would be used as suggested in J0091).

JCTVC-J0373 Cross-check of JCTVC-J0342 on chroma QP range extension [J. Xu, A. Tabatabai (Sony)] [late]

The proposal is supported by cross-checkers

## Entropy coding

JCTVC-J0194 AHG5: Max exponential golomb code for reducing number of bins [K. Sugimoto, S. Sekiguchi, T. Murakami (Mitsubishi)]

This contribution proposes modifications to the CABAC binarization for ref_idx, cu_qp_delta and sao_offset. In HM-7.0, truncated unary binarization is used for these syntax, which introduces 15, 32 and 32 context coded bins in the worst case. To reduce the throughput of the CABAC, this contribution proposes using kth order Max Exp-Golomb coding. Max Exp-Golomb coding is similar to Exp-Golomb coding, but the longest code is one bit shorter than Exp-Golomb code by truncating the code using the maximum possible index.

The proposed method was implemented in HM-7.0, and it is reported that there is negligible coding efficiency impact with reducing the number of context coded bins per syntax element in the worst case down to 4, 5 and 5.

It is reported that bit rate increase in terms of BD-rate is 0.0% for ref_idx and 0.0–0.1% for sao_offset in CTC. The reported bit rate increase for cu_qp_delta is 0.2% in AHG5 test condition.

It is also proposed to apply 1st order Max Exp-Golomb code for mvd binarization. By signaling motion search range, maximum bins for mvd is also reduced.

Adopting this would mean that another binarization scheme would need to be defined, as EG0 cannot be replaced (is also used for transform coefficients)

Unlike EG0, this method might not have an implicit prefix/suffix error detection capability.

No action.

JCTVC-J0351 AHG5: Cross-verification of JCTVC-J0194 on max exponential golomb code for reducing number of bins [K. Chono (NEC)] [late]

JCTVC-J0195 AHG5: on CABAC initialization [K. Sugimoto, A. Minezawa, S. Sekiguchi (Mitsubishi)]

This contribution proposes a modification on initialization table selection for CABAC context. The proposed method uses only two initialization tables instead of three tables in HM7. In P-slices and B-slices, one of intra-oriented or inter-oriented tables can be used by signaling cabac_init_flag, while intra-oriented table is always used for I-slices same as in HM7. To verify the performance, not only common test sequences but also a sequence including several scene changes is also used. In the simulations, intra-oriented initialization table was used in each picture just after scene change. Simulation results show that performance gain is 0.0% for all settings in CTC and CTC in multiple 1500 byte slice mode. It is also reported that the performance gain 0.1 for LDB main setting and 0.0% for all other settings for the sequence with frequent scene changes.

Two changes are proposed on the cabac_init_flag syntax element used for CABAC context initialization:

- Reduce the number of tables for CABAC initialization to two instead of three in the current design.

- Modify the definition of cabac_init_flag to be a flag to allow for P and B slice types to be initialized with either the “Intra slice type” table or “Inter slice type” table.

Was already presented in I0065

One expectation mentioned in context of I0065 was potential gain after scene change – this does not seem to be the case.

No support by other companies – no action.

JCTVC-J0208 AHG5: Bypass bins grouping on Intra mode coding [H. Sasai, K. Terada, T. Nishi (Panasonic)]

This contribution presents a modification on intra mode coding parameters. It is proposed that the signalling order changes to improve the throughput of entropy coder by grouping bypass-coded bins. The proposed solution was implemented in HM7.0 and their coding efficiencies were evaluated. The results show no difference on coding performance.

Q: How large is the benefit in terms of hardware throughput? Is this syntax element critical? Most likely not.

Software implementation would become slightly more complicated due to the additional grouping.

No action.

JCTVC-J0453 Cross-verification of JCTVC-J0208 : Bypass bins grouping on Intra mode coding [W.-J. Chien (Qualcomm)] [late]

Cross-checkers support the idea, as it is consistent with things that are done elsewhere

JCTVC-J0279 CU skip and split CABAC context modification for flexible wavefront parallel parsing [M. Coban, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution proposes removal of CU skip and split CABAC dependency on the upper CTB row for enabling parallel parsing of wavefront substreams with CTB row level synchronization instead of more restrictive CTB level synchronization. It is reported that the proposed context modeling CU skip and split flag results in 0.02%, 0.05%, 0.09% BD-rate loss for AI-Main, RA-Main, LD-Main configurations, respectively, and 0.02%, 0.06%, 0.14% BD-rate loss for AI-HE10, RA-HE10, LD-HE10 configurations, respectively.

Presentation not uploaded.

Loss is higher (0.3–0.4%)  for smaller CTB such as 16x16, but negligible as compared to the overall loss that small CTB has.

Several experts support the idea.

Current text does not seem to be complete (missing precise definition of “availability”)

Revisit.

Cross-check J0414 (include header)

Confirms that the results are correct and the dependency is removed.

When asked for confirmation about the matching of software and text, the cross-checker is no longer in the room.

## Transform coefficient coding

### Significance map coding

JCTVC-J0068 Refined significant map context derivation for large TU [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

In the last meeting, the position based context derivation technique (JCTVC-I0296) was adopted.This contribution presents a new context derivation table for significant_coeff_flag for large transform. The refined context derivation table reportedly provides average 0.1% coding gain on common condition.

No or little change in complexity, gain is around 0.03%

No additional contexts.

Three zones (which requires one more comparison).

It is mentioned that by inclusion of I0296 in the last meeting, a slight loss of 0.1% was observed.

Not sufficient benefit to justify inclusion. No action

JCTVC-J0200 Cross verification of Refined significant map context derivation for large TU (JCTVC-J0068) [H. Nakamura, T. Kumakura (JVC Kenwood)]

JCTVC-J0354 Modification of context assignment for significance flag coding for large TUs [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

A modification of context assignment for significance flag coding for large TUs is proposed. It is claimed that the proposed modification simplifies context derivation logic. For common test conditions and AI-main, RA-main, LB-main, AI-HE10, RA-HE10, and LB-HE10 configurations the average BD-rate difference is −0.10%, −0.06%, −0.01%, −0.09%, −0.05%, and 0.04%, respectively (average: −0.045%). Two additional contexts for luma and one for chroma are reportedly used.

three additional contexts

also 3 zones

slightly more complex than J0068

Not sufficient benefit to justify inclusion. No action.

JCTVC-J0375 Cross-verification of Modification of context assignment for significance flag coding for large TUs (JCTVC-J0354) [R. Cohen (Mitsubishi)] [late]

JCTVC-J0102 AHG5: Unification of context derivation for coefficient group flag [V. Seregin, J. Sole, M. Karczewicz (Qualcomm)]

Coefficient group flag is coded using bottom and right flags of already processed sub-blocks. However in one case, when transform unit size is 8x8 with horizontal or vertical scanning, only previous coded coefficient group flag is used to derive the context for current group flag. In this contribution, it is proposed to unify those cases by applying the same context derivation method for horizontal and vertical scans as it is used for diagonal scan.

Questionable whether this is a simplification. Actually, in an actual implementation, the conditional checks that are removed from the spec may not be necessary (e.g. they are not implemented this way in the current software).

The scans themselves are not changed. But coefficient groups are slightly changed.

No action

JCTVC-J0451 Cross check of Qualcomm’s proposal JCTVC-J0102 [J. Kim (LGE)] [late]

JCTVC-J0308 Context index derivation method for level 0 significance map coding [K. Panusopone, W.-Y. Kung, X. Fang, L. Wang (Motorola Mobility)]

This contribution proposes to share contexts for level 0 significance map coding between 4x4 and 8x8 TUs. To reduce complexity, the proposal uses common logic to derive context index instead of using mapping table. The simulation results show that overall performance loss is less than 0.1%.

Only some implementations might draw a benefit (and even then questionable how large the benefit would be) – not justified against the loss that occurs.

No action.

JCTVC-J0439 Cross-verification of JCTVC-J0308 on Context index derivation method for level 0 significance map coding [X. Zheng (HiSilicon), H. Yu (Huawei)] [late]

    - Last position coding

JCTVC-J0319 Further Unification of Luma/Chroma Context Derivation for Last Position Coding [L. Guo, M. Karczewicz, W.-J. Chien (Qualcomm)]

In HM7, equation-based context derivation is used for last significant position coding where Luma and Chroma components share the same equation but with different parameters (offset and shift). This contribution proposes to further unify last coding context derivation by using the same shift parameter for Luma and Chroma components. Experimental results reportedly show there is 0.0% Y-BD rate change under common test conditions.

In case of chroma coding, 2 additional contexts

Still, luma and chroma are not yet exactly the same (only identical for value of n)

Benefit not obvious

No action.

JCTVC-J0395 Cross check of Qualcomm’s proposal JCTVC-J0319 [J. Kim (LG)] [late]

### Coefficient level coding

JCTVC-J0095 AHG5: Pairwise Coding of Greater-than-1 Flags [N. Nguyen, D. He, G. Martin-Cocher (RIM)]

This document applies the concept of pairwise coding to the coding of greater-than-1 flags to improve the worst case throughput.  It is reported that the pairwise coding method improves the worst case throughput from 1.875 to 1.625 context-coded bins per transform coefficient.  Moreover, it allows two greater-than-1 flags to be coded in parallel and reduces the average number of context-coded bins per transform coefficient by between 4.5% and 8.5%.  These benefits come at a cost of 6 additional contexts.  The pairwise coding method presents opportunities for further study with respect to its interaction with intra transform skipping and RDOQ.

For information only – work not complete yet.

Some interference with transform skip – loss observed for class F.

One expert mentions that in general it may be desirable to also look at RDOQ off cases.

JCTVC-J0142 On coeff_abs_level_remaining coding [M. Budagavi, V. Sze (TI)]

HEVC HM-7.0 codes coeff_abs_level_remaining using a VLC that consists of a unary coded prefix and a fixed length suffix whose size depends on the prefix and a parameter cParam that is adaptively updated after each coeff_abs_level_remaining is coded. This VLC is basically a variant of exp-golomb (k) code. The maximum codeword length for this VLC is 37 bits. It is asserted that VLC with maximum codeword length of 32 bits or less is desirable from throughput and complexity reduction perspective especially for architectures that use 32-bit processors and/or 32-bit memories. This contribution studies several different VLCs obtained by modifying HM-7.0 coeff_abs_level_remaining VLC (denoted as U-FLC123-L37 in this contribution) so that the maximum codeword length is less than or equal to 32 bits. The VLCs are reported to result in average BD-Rate of 0.0% to 0.1% in normal and low QP range for Class A-E. For Class F sequences, the VLCs are reported to result in average BD-Rate of −0.1% to 0.2% for normal QP range and −1.7% to 0.2% for low QP range. The VLC with the smallest maximum codeword length in this contribution is TU-FLC135-L29 with maximum codeword length of 29 bits. This VLC is asserted to be a good choice from complexity reduction and BD-Rate degradation point of view. Among the VLCs with maximum codeword length equal to 32 bits, TU-FLC123-L32 is asserted to be a good choice in term of complexity reduction, closeness to HM-7.0 VLC and BD-Rate degradation. The average BD-Rate degradation of both of these VLCs is 0.0% for normal and low QP Class A-E. For Class F sequences, there is BD-Rate savings of −0.6% and −0.4% respectively for low QP value and 0.0% for normal QP range.

Suggested change in one solution: Make transition at level 3 of code instead of level 9.

Maximum suffix length is 14

Maximum number of bins is 18 (instead of 23)

Various variants (“proposals”) are in the contribution. Draft text is only provided for proposal 2, whereas proposal 1 would be the more preferable solution.

In general, the reduction to 32 bits is seen as beneficial

Revisit: Provide draft text for proposal 1, confirm by crosscheck (including the confirmation of matching text and software).

JCTVC-J0378 Cross-check report for coeff_abs_level_remaining maximum codeword length reduction (JCTVC-J0142) [H. Sasai, K. Terada (Panasonic)] [late]

Crosscheck was only done for Proposal 2

JCTVC-J0557 Cross-check JCTVC-J0142 - Cutoff-Value Modification for Remaining Absolute Transform Coefficient Level [T. Nguyen (Fraunhofer HHI)] [late] [miss]

JCTVC-J0227 AhG5: Truncated unary binarization for coeff_abs_level_remaining [K. Terada, H. Sasai, T. Nishi (Panasonic)]

This contribution proposes to use truncated unary binarization for coeff_abs_level_remaining which maximum value is defined in the HEVC draft 7.

No need for presentation, as the general intention is to keep the unary code.

JCTVC-J0454 Cross-verification of JCTVC-J0227:Truncated unary binarization for coeff_abs_level_remaining [W.-J. Chien (Qualcomm)] [late]

JCTVC-J0228 AhG5: Unification of transformed coefficient counter [K. Terada, H. Sasai, T. Nishi (Panasonic)]

This contribution proposes to unify two counters used in the transformed coefficients coding for simplification. The proposed modification was evaluated on top of HM7.0. Experimental result reportedly showed that proposed modification can be introduced into the HM7.0 without major impact on coding efficiency.

The removal of one counter appears beneficial in terms of clarity of the design.

Revisit: Proponents to suggest a solution after investigating the potential benefit of the additional method in J0408.

JCTVC-J0362 Cross-verification on unification of transformed coefficient counter (JCTVC-J0228) [I.-K. Kim (Samsung)] [late]

JCTVC-J0408 AhG5: Crosscheck of JCTVC-J0228 [Y. Piao, J. H. Park (Samsung)] [late]

This cross-check suggests an alternative method on top of J0228, but comes without IPR statement. New version with IPR statement will be provided.

JCTVC-J0254 Position-based Context Derivation of coeff_abs_level_greater1_flag Coding [T. Ji, X. Yu, D. He, G. Martin-Cocher (RIM)]

This contribution proposes a position-based context derivation scheme for the syntax element coeff_abs_level_greater1_flag coding. In the current HM7.0 design, the context indices for coeff_abs_level_greater1_flag can only be derived in a sequential manner, since the context for one coeff_abs_level_greater1_flag is dependent on its immediately previous coeff_abs_level_greater1_flag . It is proposed here to derive contexts for coeff_abs_level_greater1_flag in a CG based on the position of the coeff_abs_level_greater1_flag in the scanning order. Thus, it completely removes the context dependency for coeff_abs_level_greater1_flag within a CG, and allows all context indices for coeff_abs_level_greater1_flag in one CG to be derived in parallel. The BD rate penalty is on average 0.1%.

Loss is largest in all-intra settings and particularly in class F.

Q: Is it really a benefit? The proposal does not remove the dependency between GT1 and GT2 flags

Even though this is one syntax element which could be frequently occurring (15% ), there is no consensus how critical this really is (may be depending on implementation platform)

Not evident that the advantage that is claimed justifies the loss in performance.

No action.

JCTVC-J0488 Cross-verification of JCTVC-J0254: Position-based Context Derivation of coeff_abs_level_greater1_flag Coding [W.-J. Chien (Qualcomm)] [late]

JCTVC-J0424 Crosscheck of JCTVC-J0254 [Y. Piao, J. H. Park (Samsung)] [late]

JCTVC-J0285 Early by-pass mode switch of greater1_flag [J. Chen, W.-J. Chien, J. Sole, M. Karczewicz(Qualcomm)]

This proposal targets improving CABAC throughput by reducing context-coded bins for coefficients level coding, specifically greater1_flag. In HM7.0, maximum eight greater1_flags are context-coded in one coefficient group before switching to by-pass mode. This contribution proposes stopping context-coded greater1_flag after the occurrence of one or two greater1_flags equal to 1 in a coefficient group. The maximum context-coded greater1_flag is kept as eight in the proposal.

Simulation results of method 1 (stopping context-coded greater1_flag after the occurrence of two greater1_flags equal to 1) shows average 0.0%, 0.0%, 0.0%, 0.0%, −0.1%, 0.0% BD rate impact, respectively for AI-Main, AI-HE10, RA-Main, RA-HE10, and LD-Main, LD-HE10 of common test condition. The average BD rate impact is 0.0%, −0.1%, −0.1%, 0.0%, 0.0%, 0.0% in low QP test condition (QP = 1, 5, 9, 13).

Simulation results of method 2 (stopping context-coded greater1_flag after the occurrence of one greater1_flag equals to 1) shows average 0.0%, 0.0%, 0.1%, 0.1%, 0.1%, 0.1% BD rate impact, respectively for AI-Main, AI-HE10, RA-Main, RA-HE10, and LD-Main, LD-HE10 of common test condition. The average BD rate impact is 0.0%, 0.0%, 0.2%, 0.3%, 0.2%, 0.4 % in low QP test condition.

Reduces 6 contexts for GT1 flag.

Worst case of number of context-coded bins may not be reduced (depending on where the transition is made). However, it is claimed that the throughput in Rice code may also be critical, and therefore the worst-case throughput is reduced.

Not obvious how large the benefit is, and whether there might be better solutions

Further study (AHG)

JCTVC-J0490 Cross-check report for early bypass mode switch of greater1_flag (JCTVC-J0285) [S. H. Kim, A. Segall (Sharp)] [late]

JCTVC-J0304 Context assignment for parallel coefficient level coding [W.-J. Chien, J. Sole, M. Karczewicz (Qualcomm)]

In current HM, the context modeling of the syntax element coeff_abs_level_greater1_flag is dependent on the value of previous coded coeff_abs_level_greater1_flag. Up to 8 coeff_abs_level_greater1_flag might be coded consecutively. This dependency hinders the throughput of the entropy coding and allegedly, it is one bottleneck in current CABAC design. This contribution removes the dependency of previous coded coeff_abs_level_greater1_flag for context modeling.

15-20% of the bitstream could use this flag

Similar to J0254, slightly simpler and slightly less loss

Further study (AHG)

JCTVC-J0498 Cross-check report for context assignment for parallel coefficient level coding (JCTVC-J0304) [H. Sasai, K. Terada (Panasonic)] [late]

### Scans

JCTVC-J0105 Simplification of coefficient scanning order [R. Cohen (Mitsubishi)]

The current HEVC text specification maps 9 Intra prediction modes to the horizontal coefficient scan order and 9 other Intra prediction modes to the vertical coefficient scan order. This contribution proposes reducing the range of modes from 9 to 8. It is reported that by reducing the range in this way to a power of 2, a table-lookup or set of range comparisons can be replaced by a Boolean check on a few bits of the Intra prediction mode value. Three variations of this method are presented. Average BD-Rate changes range from 0.1% - 0.2% for Simplification 1 and 0.0% - 0.1% for Simplifications 2 and 3. Changes to the HEVC text specification and additional results from combining this proposal with JCTVC-J0093 are provided as well.

Several experts express opinion that the advantage is not obvious and the penalty is non-negligible.

No action.

JCTVC-J0135 Cross-verification of Simplification of coefficient scanning order (JCTVC-J0105) by BBC [M. Mrak, M. Naccari (BBC)]

JCTVC-J0150 Removal of zigzag scan from scaling list coding [M. Shima (Canon)]

This contribution reports the effects of replacing zigzag scan used for scaling list coding with existing up-right diagonal scan.  In regards of scaling list coding performance, using the test matrices used in past CE4, the simulation results show that replacing zigzag scan with up-right diagonal scan will lead to about 0.6% scaling list coding bit reduction on average.  For other aspects, it is reported that the proposed changes will remove one section of HEVC draft text defining zigzag scan (about half a page) and about 50 lines of code from HM 7.0 software.

Several experts support this

HEVC would now be different from other standards by not having zigzag scan.

Decision: Adopt

JCTVC-J0205 Cross-check report on removal of zigzag scan from scaling list coding (JCTVC-J0150) [S.-C. Lim, H. Y. Kim, J. Lee (ETRI)]

JCTVC-J0256 Removal of the 8x2/2x8 coefficient groups [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

Coefficient coding in HEVC is processed in 4×4 sub-blocks (coefficient groups). The exceptions are the 8×2 and 2×8 sub-blocks for the 8×8 TU when horizontal and vertical scans are used. It is proposed to remove these non-square coefficient groups by using 4×4 based horizontal and vertical scans. The significance map coding is unified for all the scans and for TU sizes larger than 4×4. The performance of the method is 0.06% / −0.03% / −0.02% for AI / RA / LD Main, and 0.09% / 0.00% / 0.02% for AI  / RA / LD HE. For the RDOQ off case, results are 0.00% / −0.07% / 0.00% for AI / RA / LD Main and 0.01% / −0.05% / 0.06% for AI / RA / LD HE. The method is combined with the significance map patterns in JCTVC-J0354 giving a performance of 0.02% / −0.06% / −0.02% for AI / RA / LD Main, and 0.06% / −0.04% / 0.07% for AI  / RA / LD HE.

Several experts express opinion that this is a desirable unification (using at least same 4x4 sub-block structure) as for other cases

Would also resolve a current bug in the spec for 2x8/8x2

Still uses different sequence of sub-blocks for horizontal and vertical (latter identical to current diagonal) – Q: would it cause a loss when same sequence is used everywhere?

Decision: Adopt J0256 method 1

JCTVC-J0421 Cross-check of Removal of the 8x2/2x8 coefficient groups (J0256) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-J0281 Additional horizontal and vertical scan for transform coefficients [C. Auyeung (Sony)]

This contribution proposes two proposals to add horizontal and vertical scans to HM7 to improve coding efficiency. In the proposal A, horizontal and vertical scan is added to the coding of 8x8 chroma transform coefficients. This required one character change in the HM7 source code, and it resulted in −0.2% to intra main chroma BDR and −0.1% to intra HE10 chroma BDR. In proposal B, the horizontal and vertical scan is added to all transform sizes. It resulted in BDR of −0.1% for luma intra main and luma intra HE10. It also resulted in BDR or −0.3% to −0.4% for chroma intra main and chroma intra HE10.

Method 1 applies current 2x8 / 8x2 subblock schemes also for chroma (0% gain in luma, 0.2%  in chroma)

Method 2 applies 1x16 / 16x1 also for 16x16 and 32x32 and everything also for chroma (0.1% gain in luma, 0.4% gain in chroma)

Q: Would it be desirable to combine method 1 with J0256 (i.e. use the new hor/ver 4x4 sub-blocks also for chroma in 8x8 case)

- no results available on such combination (but CE gain can be expected to be irrelevant)

- would the current contexts work?

- what would be the benefit in terms of implementation?

could be further studied.

JCTVC-J0368 Cross-check of additional horizontal and vertical scans (JCTVC-J0281) [J. Sole (Qualcomm)] [late]

### Sign data hiding

JCTVC-J0067 Simplified decision for sign hiding [T. Ikai (Sharp)]

This contribution presents the simplified decision method for sign hiding (JCTVC-I0291). The proposed method uses the number of coefficients in the sub-block to decide whether a sign flag is hidden or not. It removes the derivation of lastNZPosInCG, which is the last coefficient position in the sub-block. It is reported that average BD-rate loss is 0.1 %, 0.0 %, 0.0% at IO, RA, LB case respectively on common condition. And on RDOQ off condition, the average BD-rate loss is reported as 0.1 %, 0.1 %, 0.2 % at IO, RA, LB case.

Benefit is not obvious, but loss in compression performance

No action.

JCTVC-J0329 Cross-check of Simplified decision for sign hiding (JCTVC-J0067) [V. Sze (TI)] [late]

JCTVC-J0094 Simplification of multiple sign bit hiding criterion [J. Wang, D. He, X. Yu, G. Martin-Cocher (RIM)]

This contribution proposes to simplify the Multiple Sign Bit Hiding (SBH) by applying the number-of-non-zeros based criterion to coefficient groups that do not contain the last significant coefficient. The proposed method removes the need to find and store the position of the last non-zero coefficient for every CG which translates in removing up to 15 checks in the worst-case scenario. Under HM7.0 common test conditions, on average −0.01%, −0.02%, −0.05% BD-rates are observed for AI-Main, RA-Main, LB-Main settings, and 0.00%, 0.01%, 0.02% BD-rates are observed for AI-HE10, RA-HE10, LB-HE10 settings. When RDOQ is turned off, on average −0.02%, −0.08%, −0.07% BD-rates are observed for AI-Main, RA-Main, LB-Main settings, and −0.03%, −0.07%, −0.01% BD-rates are observed for AI-HE10, RA-HE10, LB-HE10 settings. Further encoder-only changes to related part in RDOQ result in average BD-rates (−0.11%, −0.07%, −0.09%, −0.26%, −0.37%, −0.61%) in common test conditions.

Benefit is not obvious, as the way it is written in the spec may not be the only way how to implement.

No support by other experts

No action.

JCTVC-J0333 Cross-verification of Simplification of multiple sign bit hiding criterion (JCTVC-J0094) [Y. Yu, W. Wan (Broadcom Corp)] [late]

JCTVC-J0447 Crosscheck of Simplification of multiple sign bit hiding criterion (JCTVC-J0094) [G. Clare, F. Henry (Orange Labs)] [late]

JCTVC-J0099 Sign data hiding simplification [J. Sole, M. Karczewicz (Qualcomm)]

Two methods that allegedly simplify the sign data hiding algorithm are proposed. A first method removes the first non-zero position dependency from the sign hiding criterion. There is no average BD-rate difference on common conditions and −0.1% for RDOQ off case. A second method applies the hiding criterion only once per transform unit (instead of once every 16 coefficients) and doesn’t need to keep track of the position of the first and last non-zero coefficients in the coefficient group. The average BD-rate for all configurations is −0.1% for common conditions and −0.2% for RDOQ off case. Further encoder-only changes related to RDOQ result in average BD-rates in common test conditions of −0.1% for Main profile and  −0.3%, −0.4%, −0.6% for AI, RA, and LB-HE, respectively.

Questions raised:

- This method could be applied to single coefficients – would this have visual impact?

- Might there be interference with other elements e.g. transform skip?

No consensus to take action.

JCTVC-J0431 Cross-check of: "Sign data hiding simplification" (J0099) [A. Gabriellini, M. Mrak, M. Naccari (BBC)] [late]

Cross-checker supports method 2

JCTVC-J0377 Cross-check of JCTVC-J0099 on sign data hiding simplification [C. Auyeung (Sony)] [late]

JCTVC-J0274 Parity Inference [J. Wang, D. He, X. Yu, G. Martin-Cocher (RIM)]

This proposal describes a technique to infer the parity of the first quantized coefficient in a coefficient group. In common test conditions, experimental results based on HM7.0 reference software show that the average Luma BD-rates are (−0.28%, −0.31%, −0.30%, −0.58%, −0.41%, −0.31%) in (AI-Main, RA-Main, LD-Main, AI-HE10, RA-HE10, LD-HE10) settings with RDOQ off, and (−0.22%, −0.10%, −0.10%, −0.62%, −0.50%, −0.65%) with RDOQ on. Note that the results include rotation of residual block in transform skipping as detailed in JCTVC-J0093, which affects HE10 test conditions. The method also reduces the maximum number of context-coded bins in a coefficient group from 25 to 24.

Comments:

- Depending on the parity inference, the parsing of GT1 flag may be skipped. Does this mean that parsing can no longer be performed independent?

- Is it really a simplification?

- Comment by text editor: Seems to be difficult to integrate

- Encoding and decoding time increased (confirmed by cross-checker)

No action.

JCTVC-J0405 Cross check of Parity Inference (JCTVC-J0274) [T. Ikai, T. Tsukuba (Sharp)] [late]

### CBF

JCTVC-J0157 Cross-check of luma CBF inference proposed by Microsoft [J. Kim (LGE)]

JCTVC-J0161 Removal of the inference of luma CBF at the transform depth level zero in inter CU [J. Kim, B. Jeon (LGE)]

This contribution proposes a method to remove the inference of luma CBF when both chroma CBFs are zero and the transform depth level is zero in inter predicted CUs. It simplifies the CBF coding and decoding with BD-rate change of 0.0%, 0.1%, 0.1% and 0.1%, 0.1%, 0.1% for RAMAIN and RAHE10 respectively. It also shows 0.1%, 0.3%, 0.3% and 0.1%, 0.4%, 0.4% BD-rate change for LBMAIN and LBHE10.

Follow-up of contribution I0152 which was adopted in the 9th meeting. Even though it was claimed by that time that all inference dependency on luma CBF was removed, this was in fact not the case.

The previous contribution gave 0.1% BD rate increase as well.

JCTVC-J0321 Cross-verification of JCTVC-J0161 Removal of the inference of luma CBF at the transform depth level zero in inter CU [L. Guo (Qualcomm)] [late]

JCTVC-J0241 Simplification on CBF inference and coding [J. Xu (Microsoft)]

No presenter available Fri afternoon.

JCTVC-J0157 Cross-check of luma CBF inference proposed by Microsoft [J. Kim (LGE)]

JCTVC-J0303 Simplification on context derivation of cbf_luma syntax element [I.-K. Kim, J. H. Park (Samsung)]

For simplification of context selection of cbf_luma syntax element, maximum transform size checking for cbf_luma is removed in this proposal. Average loss from this simplification is negligible (0.0%). It is also observed that 0.1% and 0.2% losses in random access HE10 and low delay B HE10 configurations, respectively.

Several expects express support for this obvious simplification

Decision: Adopt.

JCTVC-J0376 Cross-check report for Simplification on context derivation of cbf_luma syntax element (JCTVC-J0303) [H. Sasai (Panasonic)] [late]

JCTVC-J0533 Chroma CBF cleanup [P. Kapsenberg (Intel)] [late]

This contribution claims that the context increment for cbf_cb and cbf_cr can exceed the maximum value intended. A solution is proposed that clamps the context increment to 2.

Relates to bug with ticket #576

Impact on 4:4:4? Would in principle be possible, but would restrict the number of contexts.

Would it support larger CTB sizes than 64 which might require tree depth >3?

Another solution is suggested that adds and specifies the additional context (but that would currently not be used)  - Benjamin Bross will take action

## Intra prediction and intra mode coding

### General

JCTVC-J0131 Motion estimation-based fast intra prediction(ME-FIP) [Xiaohai He, Guoyun Zhong, Yuan Li, Di Wu, Linbo Qing (Sichuan Univ.)]

No presenter available Wed. 18:25

JCTVC-J0271 Luma intra planar prediction filter [C. Auyeung (Sony)]

This contribution proposes to apply the luma intra DC prediction filter to filter the luma intra planar prediction. When the DC prediction filter is applied to both luma intra DC prediction and the luma intra planar prediction in HM7, both Intra Main and Intra HE10 resulted in BDR of −0.1%. When the DC prediction filter is disabled for DC prediction in HM7, both Intra Main and Intra HE10 resulted in BDR of 0.1% and BDR of −0.2% for Class F test sequences.

Concern is raised that this causes additional complexity in software implementation. Code optimized for DC cannot be re-used for planar.

Planar mode is already the most complex mode

JCTVC-J0301 Cross-verification of Luma intra planar prediction filter (JCTVC-J0271) [R. Cohen (Mitsubishi)]

### Chroma modes

JCTVC-J0052 Bug-fix of intra chroma DM mode for Intra_NxN with SCU size larger than 8x8 [T.-D. Chuang, Y.-W. Huang, Y.-H. Huang, T.-A. Lin, K.-B. Lee, C.-C. Ju, S. Lei (MediaTek)]

A mismatch between HM-7.0 software and HEVC text specification draft 7 is reported as follows. In the HM-7.0 software, an intra CU sends only one intra chroma prediction mode regardless of the PU partition mode. However, in the text specification (JCTVC-I1003_d4.doc), an intra CU sends one intra chroma prediction mode for each chroma PU, which is inconsistent with HM-7.0 software when NxN PU partition mode is selected. The inconsistency needs to be resolved.

Two possible solutions are reported here. Method 1 follows more or less what is in the software but includes a bug fix to let the four chroma PUs refer to the four different luma PUs when the intra CU is an SCU larger than 8x8 in luma and DM mode and NxN PU partition mode are selected. It is reported to achieve 0.0–0.6% luma coding gains and −0.1–1.0% chroma coding gains when the SCU size is set to 16x16 in luma. Method 2 follows what is specified in text but includes a bug fix to send only one intra chroma prediction mode when the intra CU is an SCU equal to 8x8 in luma and DM mode and NxN PU partition mode are selected. It is reported to achieve 0.0–0.4% luma coding gains and 0.6-2.4% chroma coding gains for when the SCU size is set to 16x16 in luma.

Matched software and text are provided for both methods. It is proposed to adopt one of the proposed methods as a bug-fix to the current HEVC.

Results are presented with SCU 16x16 as for 8x8 (common test conditions) no effect would be visible

What is suggested here is less a “bug fix” than a new proposal which suggests to derive different prediction modes for the case where several chroma PBs are contained in a CU. The decision of last meeting was to use only one chroma prediction mode (i.e. only one prediction direction etc. for all chroma PBs) within a CU. This was extensively discussed and it was confirmed that currently software and text are aligned.

No action.

JCTVC-J0159 Cross check of MediaTek’s proposal JCTVC-J0052 [J. Kim (LGE)]

JCTVC-J0160 Parameter derivation for intra LM chroma prediction [J. Kim, B. Jeon (LGE)]

This contribution proposes a method to employ the samples around CUs (i.e. Coding blocks) instead of TUs(i.e. Transform blocks) in order to get parameters for intra LM chroma mode. It aims to reduce the number of calculation of parameters and corresponding calculation burden. It shows 0.0%, −0.7%, −0.2% BD-rate under the AIHE10 condition. It also shows 0.0%, −0.7%, −0.6% and 0.0%, 0.1%, −0.3% BD rate −0.3%, −0.2% BD-rates respectively under the RAHE10 and LBHE10 condition. It alters two words in the Text.

The initial computation becomes more complex (and may be subject to overflow in addition as the area is 2x as large at maximum); also introduces potential latency in first chroma block; on the other hand, the overall complexity is reduced as the process is only invoked once. For CU of size 8x8 (worst case) it does not help.

One expert mentions that the method may even have a slight complexity disadvantage due to the larger extension to CU size.

No action.

JCTVC-J0146 Cross-check report of JCTVC-J0160 on LM parameter estimation [Y. Lin (HiSilicon)]

## Transforms

JCTVC-J0267 On DST/DCT adaptation [J. Xu, C. Auyeung, A. Tabatabai (Sony)]

This proposal extended DST-DCT selection based on Intra mode in CE1 to Chroma of 4x4 TU. Experimental results report that BD-rate performances for Solution A are under CE1 Test 1 is −0.1% for Y, 1.2% for U and 1.1% for V, under CE1 Test 2 is −0.1% for Y, −0.3% for U and −0.3% for V, and under CE1 Test 3 is −0.1% for Y, 1.1% for U and 0.9% for V. BD-rate performances for Solution B are under CE1 Test 1 is −0.1% for Y, 0.2% for U and 0.0% for V, under CE1 Test 2 is −0.1% for Y, −0.5% for U and −0.6% for V, and under CE1 Test 3 is −0.1% for Y, 0.2% for U and −0.2% for V.

Extends the DCT/DST selection to chroma. Currently the chroma uses the DCT style transform always.

No action.

JCTVC-J0369 Non-CE 1: Cross-Check of JCTVC-J0267 [A Saxena, E Alshina, F Fernandes (Samsung)] [late]

JCTVC-J0452 Cross Check of JCTVC-J0267 Test 2 [G. Van der Auwera (Qualcomm)] [late]

## Memory bandwidth reduction (4 – done)

JCTVC-J0086 AHG7: Disallow bi-predictive mode for 8x4 and 4x8 inter PUs [M. Zhou (TI)]

In the current HM7.0 design an encoder is allowed to send bi-predictive motion vectors for inter PUs of 8x4 and 4x8 and a decoder discards list 1 motion vectors for those PUs in order to reduce the worst case memory bandwidth requirements of motion compensation. Such a design may cause design confusion on the encoder side and unnecessarily complicate the decoder design validation. This contribution advocates to prohibit bi-predictive motion vectors for inter 8x4 and 4x8 PUs by modifying the CABAC binarization of inter_pred_idc. In the proposed algorithm, the CABAC binarization of inter_pred_idc for inter 8x4 and 4x8 PUs is modified to (FL, cMax = 1), i.e. “1” for Pred_L1 and “0” for Pred_L0, and for inter PUs of size 8x8 and above the same CABAC binarization process defined in HM7.0 is used, i.e. “1” for Pred_BI, “01” for Pred_L1 and “00” for Pred_L0. Compared to HM7.0, the proposed algorithms leads to an average gain of 0.0/0.0/0.1/0.1% (in RA-Main/RA-HE10/LB-Main/LB-HE10). Prohibiting bi-pred mode for 8x4 and 4x8 inter-PUs at syntax level is helpful for avoiding potential confusions from both encoder and decoder side.

Decision: Adopt.

JCTVC-J0214 AHG7: Cross-check report of disallow bi-predictive mode for 8x4 and 4x8 inter PUs (JCTVC-J0086) [T. Sugio (Panasonic)]

JCTVC-J0218 Bi-pred merge restriction for small PUs [T. Lee, J. Park (Samsung)]

In the previous meeting, bi-pred restriction for small PUs is adopted where bi-predictive motion information for 8x4 and 4x8 is converted to uni-predictive List0 motion after motion information construction. Assuming that encoder restricts motion search on bi-prediction for such small PUs, the only place where bi-predictive motion is given is the merge mode. In this proposal, bi-predictive merge candidates are restricted during merge candidate derivation process. By removing or converting bi-predictive merge candidates, merge candidate derivation process is reduced in complexity and converting bi-predictive motion on motion compensation process is unnecessary assuming that encoder restricts motion search on bi-prediction for small PUs.“Proposal 1” is suggested solution of proponent.

Something similar was discussed in the last meeting in the context of memory BW reduction, but by that time proposals that suggested change of the merge process for small PUs were not adopted to keep the design common over all PU sizes; it does not have a significant impact on complexity anyway.

No action.

JCTVC-J0366 Cross-check of J0218 on Bi-pred merge restriction for small PUs [P. Onno (Canon)] [late]

JCTVC-J0166 An experimental comparison of memory bandwidth between P and B-slice coding [T. Chujoh (Toshiba)]

An experimental comparison of memory bandwidth between P and B-Slice coding for low delay configuration is reported. By using the measurement module of memory bandwidth that has been developed by the previous AHG of reference memory compression and has been distributed to previous CE of motion compensation, the numerical results of memory bandwidth analysis is shown.  As experimental results, the memory bandwidth increase of B-slice compared to P-slice is reportedly an average of 41% and a maximum of 70%. The contributor suggested that these results should be considered in profile/level discussions.

LB had about 4–5% improvement in compression relative to LP, but with about twice the memory bandwidth.

This was an information contribution – no action was requested.

JCTVC-J0314 Cross-verification of "An experimental comparison of memory bandwidth between P and B-slice coding" (JCTVC-J0166) [S. Fukushima, H. Nakamura (JVC Kenwood)]

JCTVC-J0175 AHG7: Memory bandwidth limitation for higher level [K. Kondo, T. Suzuki (Sony), U. Motoharu, S. Fukushima (JVC Kenwood Corporation)]

This contribution proposes to restrict the bi-prediction 8x8 with the control flag in SPS. In current HM 7.0 design, bi-prediction 8x4 and 4x8 is replaced to uni-prediction for all levels. The limitation for memory bandwidth is no problem for HD sequences, but it may not be enough for higher level in the future. So this contribution presents that the encoder send a flag whether bi-prediction 8x8 is replaced or not, and the decoder replaces the inter direction for also 8x8 as according to the flag. The flag is sent in sequence parameter sets (SPS). In this experiment, it is shown that with this technique the coding efficiency loss is decreased.

A flag is proposed to prohibit 8x8 bipred by inferring list 0 prediction, conditioned on an additional SPS flag, as done for 4x8/8x4. Having the flag improves coding efficiency relative to encoder-only avoidance of bi-pred at the 8x8 level. It was suggested that the use of this constraint be considered for large picture formats (e.g. in profile/level conformance requirements).

Decision: Adopt (using the approach described in J0086).

JCTVC-J0311 Cross check of memory bandwidth reduction for higher level (JCTVC-J0175) [T. Yamamoto (Sharp)]

## Alternative coding modes (29 – done)

A BoG (coordinated by K. Chono) was requested to review the contributions in this category.)

JCTVC-J0512 BoG report on alternative coding modes [K. Chono (NEC)]

This BoG was established in order to study contributions on alternative coding modes (I_PCM, Transform skipping mode, and lossless coding). Proposals of each alternative coding mode were categorized and summarized in the attached XLS sheet.

It is recommended that JCT-VC experts review the summary XLS sheet and discuss the necessity of presentations of categorized schemes in the main session.

### I_PCM (4 – done)

For I_PCM related proposals, the schemes are categorized as follows:

- I_PCM signalling

  - Burst IPCM (JCTVC-J0153 – also proposed at previous meeting)

  - Conditional signalling of pcm_flag (JCTVC-J0277)

- Harmonization of filtering control of I_PCM and lossless coding modes

  - Syntax harmonization of the I_PCM and TransQuantBypass modes (JCTVC-J0168)

  - Semantics harmonization of loop filtering control for I_PCM and TransQuantBypass modes (JCTVC-J0169)

No action taken in this area.

JCTVC-J0153 Burst I_PCM Signalling [M. Coban, W.-J. Chien, J. Chon, M. Karczewicz (Qualcomm)]

JCTVC-J0411 Crosscheck of Burst IPCM Coding Signalling (JCTVC-J0153) [G. Clare, F. Henry (Orange Labs)] [late]

JCTVC-J0277 Interaction between pcm_flag and cu_transquant_bypass_flag [R. Joshi, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-J0168 AHG11: Syntax harmonisation of the I_PCM and TransQuantBypass modes [E. François, P. Onno, G. Laroche, T. Poirier, M.Shima (Canon)]

JCTVC-J0426 Crosscheck of Syntax harmonisation of the I_PCM and TransQuantBypass modes (JCTVC-J0168) [G. Clare, F. Henry (Orange Labs)] [late]

JCTVC-J0169 AHG11: Loop filtering control for I_PCM and TransQuantBypass modes [E. François, P. Onno, G. Laroche, T. Poirier, M.Shima (Canon)]

### Transform skipping (16 – done)

For transform skipping related proposals, the schemes are categorized as follows:

- Scanning

  - Rotate scan (JCTVC-J0053 and JCTVC-J0093)

  - V/H scan type swapping scan (JCTVC-J0202 Part2, JCTVC-J0213, and JCTVC-J0313 Part1)

- Additional context for TS mode coefficients

  - Context modeling for significance (JCTVC-J0069 Part1)

  - Context modeling for abs_one_greater_than_one  (JCTVC-J0069 Part2)

- Enable TS mode in inter-coded TU

  - TS mode flag sharing among Y/U/V components (JCTVC-J0077)

  - No sharing (JCTVC-J0237  Part1)

- QM for TS mode coefficients

  - QM using 16 for TS (JCTVC-J0237 Part2, JCTVC-J0184 part1, and JCTVC-J0201)

  - QM using DC for TS (JCTVC-J0092 and JCTVC-J0184 part3)

  - QM using explicit weight for TS (JCTVC-J0184 part2)

- TS high-level syntax

  - Move TS enabling flag from SPS to PPS (JCTVC-J0184 part4)

- Conditional signalling of TS flag in chroma

  - Luma part. dependent TS for chroma (JCTVC-J0171 and JCTVC-J0389)

  - Luma dependent TS for chroma (JCTVC-J0265 Part2)

  - CU-size dependent TS for chroma (JCTVC-J0265 Part1)

- Coding of TS mode coefficients

  - Skip lastX/lastY pos signalling in TS mode (JCTVC-J0309)

During the BoG discussion, it was generally agreed the following three items are discussed in the main session.

- Modified scanning for TS mode (two schemes had been proposed)

  - J0202/J0212/J0313 – If ordinary scan is vertical, scan horizontal, and vice versa (not changing if diagonal) – Gain in Class F only: 0.9% AI, 0.7% RA, 0.3% LB (about the same for both Main and HE10). It was not tested for lossless.

  - J0053/J0093 – Rotates the data 180 degrees, then applies ordinary scan – Gain in Class F only: 1.6% AI, 1.1% RA, 0.7% LB for HE10. Some gain in other classes: 0.4% in Class C and 0.5% in Class D in AI HE10. When applied to lossless case, about 2.2% overall in AI Main.

There was discussion of whether these proposals should be considered a "bug fix" versus coding efficiency improvement.

Note: TS is in the Main profile.

No consensus was reached to make either proposed change.

- TS mode for inter TU;

  - TS (JCTVC-J0237 Part1 MS): Remove one condition from the current ones for TS mode flag signaling. Parsing is simplified and TS mode is enabled for inter TUs.

  - TS (JCTVC-J0077): Remove one condition from the current condition and add a new condition for TS mode flag signalling. The proposal can omit signaling of TS mode flag in chroma inter TUs and save some bits.

Comment: TS for inter TUs is nice to have if the number of parsing conditions do not become complex.

Comment: TS for inter TSs of BBC can save encoder running time. However, TS of MS can do the same thing by a non-normative change at encoders.

Comment: Simpler solution should be considered.

Decision: Adopt JCTVC-J0237 Part1 MS

- Non-normative change to TS mode decision for chroma TS (in J0171 and J0389 – revisit after offline discussion and software checking to see which to use)

Remaining items that the BoG suggested to be discussed (although did not necessarily recommend):

- J0069 context modelling in TS mode for significance_flag and possibly abs_greater_than_one – there was an objection to this as adding customization and complexity specific to TS – no action

- J0309 to omit signalling for last coefficient for TS mode block – similarly, no action.

- In first section of J0184: TransformSkipEnableFlag is in SPS – it was proposed to move this to the PPS. It was noted that TransQuantSkipEnableFlag is in PPS, and PCM was another mentioned example – it seems desirable for similar flags to be in the same place. Decision: Agreed to move TransformSkipEnableFlag to PPS.

It was mentioned that J0468 described a combination document for combining two of the other reviewed proposals J0069 part 1 (signficance map) and J0093 (rotation) – Class F 2.2% AI, 1.6% RA, 1.0% for LD, some gain in other classes. It also describes combination of J0069 part 1 (signficance map) and J0202/J0212/J0313.

Regarding lossless coding, as reviewed in BoG

- J0093 had rotation results for that as well (see above).

- J0230 described a lossless coding scheme by sending a lossless residual in addition to a lossy coded result. Further study was encouraged.

- J0238 suggested to remove CU level flag for lossless and associate the value QP = 4 with TS with lossless and remove QP < 4 from the standard. It was remarked that a revision of the document described a way to deal with the wrapping of QP values due to deltaQP.

- J0435 was somewhat related to J0238, and include adjustment of the deblocking filter and deltaQP signalling.

- J0339 and J0340 proposes a scheme intended to make regional lossless coding more efficient – e.g. 1% for LB, 0.6% for RA, 0.2% for AI (requiring a line buffer).

JCTVC-J0053 Residue scan for intra transform skip mode [J. An, L. Zhao, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-J0069 Constant coefficient context for intra transform skipping [T. Tsukuba, T. Yamamoto (Sharp)]

JCTVC-J0259 Cross-verification of Constant coefficient context for intra transform skipping (JCTVC-J0069) by BBC [M. Mrak, M. Naccari (BBC)] [late]

JCTVC-J0077 Transform Skipping for Inter Predicted Coding Units [A. Gabriellini, M. Mrak, D. Flynn, M. Naccari (BBC)]

JCTVC-J0092 Transform Skipping in the presence of Scaling Lists [D. Flynn, M. Mrak, M. Naccari, A. Gabriellini (BBC)]

JCTVC-J0093 Rotation of Residual Block for Transform Skipping [D. He, J. Wang, G. Martin-Cocher (RIM)]

JCTVC-J0100 Cross-check of transform skipping for inter prediction (JCTVC-J0077) [J. Sole, R. Joshi (Qualcomm)] [late]

JCTVC-J0171 On transform skip mode for chroma TUs [E. François, P. Onno, C. Gisquet, G. Laroche (Canon)]

JCTVC-J0380 Cross-check of JCTVC-J0171 on transform skip mode for chroma TUs	[C. Auyeung (Sony)] [late]

JCTVC-J0184 On Transform Skip [Y. Morigami, K Sato (Sony), T Sugio, H Sasai (Panasonic)]

JCTVC-J0532 Cross-check of JCTVC-J0184 [J. Xu (Microsoft)] [late]

JCTVC-J0535 Mental cross-check of JCTVC-J0184 on transform skip [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim (KHU)] [late] [miss]

JCTVC-J0201 QM bypass for transform skip mode [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI)]

JCTVC-J0420 Crosscheck report of QM bypass for transform skip mode (JCTVC-J0201) [M.Shima (Canon)] [late]

JCTVC-J0202 Coefficient scan for transform skip mode [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI)]

JCTVC-J0386 Cross check of Coefficient scan for transform skip mode (JCTVC-J0202) [T. Yamamoto (Sharp)] [late]

JCTVC-J0212 Modified scan method for intra transform skipping [T. Sugio, T. Nishi (Panasonic)]

JCTVC-J0327 Crosscheck of JCTVC-J0212: Modified scan method for intra transform skipping [M. Budagavi (TI)] [late]

JCTVC-J0237 Inter transform skipping [J. Xu (Microsoft)]

JCTVC-J0326 Crosscheck of JCTVC-J0237: Inter transform skipping [M. Budagavi (TI)] [late]

JCTVC-J0537 Mental cross-check of JCTVC-J0237 (the part on quantization for transform skipping TUs) [Y. Morigami, K. Sato (Sony), T. Sugio, H. Sasai (Panasonic)] [late]

JCTVC-J0244 Cross-check of JCTVC-J0053 and JCTVC-J0093 [J. Xu (Microsoft)] [late]

JCTVC-J0265 Intra Transform Skipping: Smallest CU and Implicit Chroma [G. Van der Auwera, M. Karczewicz (Qualcomm)]

JCTVC-J0430 Cross-check of: "Intra Transform Skipping: Smallest CU and Implicit Chroma" (J0265) [A. Gabriellini, M. Mrak, M. Naccari (BBC)] [late]

JCTVC-J0309 Removal of last coefficient coding in transform skipping mode [K. Panusopone, Y. Yu, X. Fang, L. Wang (Motorola Mobility)]

JCTVC-J0508 Cross-check of JCTVC-J0309 on removal of last significant coefficient position coding in transform skip [C. Yeo, Y. H. Tan (I2R)] [late]

JCTVC-J0313 Modified coefficient scan order mapping for transform skip mode [R. Cohen, A. Vetro (MERL)]

JCTVC-J0407 Cross-check of JCTVC-J0313 (Modified coefficient scan order mapping for transform skip mode) [C. Yeo, Y. H. Tan (I2R)] [late]

JCTVC-J0389 Simplification for intra transform skip mode [L. Zhao, J. An, Y.-W. Huang, S. Lei (MediaTek), D. Zhao (Harbin Institute of Technology)] [late]

JCTVC-J0393 Cross-check of Simplification for Intra Transform Skip Mode in JCTVC-J0389 [W.-S. Kim (TI)] [late]

JCTVC-J0468 Combination of JCTVC-J0069 and JCTVC-J0093 [T.Tsukuba, T.Yamamoto, T.Ikai (Sharp), D.He, J.Wang, G.Matin-Cocher (RIM), L.Guo, R.Joshi, M.Karczewicz (Qualcomm)] [late]

JCTVC-J0515 Cross-verification of Combination of JCTVC-J0069 and JCTVC-J0093 (JCTVC-J0468) by BBC [A. Gabriellini, M. Naccari, M. Mrak (BBC)] [late]

### Lossless compression (9 – done)

JCTVC-J0061 AHG11: Study report of HEVC lossless coding [K. Chono (NEC)]

JCTVC-J0436 AHG11: Study report of JCTVC-J0169 on loop filtering control for I_PCM and TransQuantBypass modes [K. Chono (NEC)] [late]

JCTVC-J0230 Improvement of HEVC lossless coding using transform coefficient coding [Minsu Cheon, Yumi Sohn, Sunil Lee, Ilkoo Kim, Jeonghoon Park (Samsung)]

JCTVC-J0516 Cross-check of JCTVC-J0230 Improvement of HEVC Lossless coding using transform coefficient coding [X. Yu, D. He (RIM)] [late]

JCTVC-J0232 HEVC lossless coding for medical image compression [Minsu Cheon, Yumi Sohn, Sunil Lee, Ilkoo Kim, Jeonghoon Park (Samsung)]

JCTVC-J0238 Lossless coding via transform skipping [J. Xu (Microsoft)]

JCTVC-J0363 AHG11: Cross-check of JCTVC-J0238 [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)] [late]

JCTVC-J0339 AHG11: Methods to Encode Lossless Coding Indication Flag [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)]

JCTVC-J0340 AHG11: New signalling mechanism for lossless coding [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)]

JCTVC-J0435 Lossless Coding and Loop Filter Control for Transform Skip [G. Van der Auwera, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)] [late]

JCTVC-J0474 AHG11: evaluation of lambda settings for lossless coding [E.François, M.Shima, P.Onno, G.Laroche, T.Poirier (Canon)] [late]

### Non-normative: Encoder optimization, decoder speed improvement, post filtering, loss concealment, rate control

JCTVC-J0057 Adaptive rate control for HEVC [J. Si, S. Ma, W. Gao (Peking Univ.), M. Yang (Huawei)]

JCTVC-J0126 Code for verifying the coding tree block raster, tile and z-scan processes [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-J0242 Encoding with fixed Lagrangian multipliers [J. Xu (Microsoft)] miss]

JCTVC-J0134 Cross-check of Encoding with Fixed Lagrangian Multipliers (JCTVC-J0242) [Y. Chiu, W. Zhang, ]J. Zhang (Intel) [late]

JCTVC-J0260 Source code bug fix on the RQ model equation [H. M. Choi, J. H. Nam, D. G. Sim (KWU)]

JCTVC-J0262 Initial QP set for rate control [H. M. Choi, J. H. Nam, D. G. Sim (KWU)]

JCTVC-J0263 Reports on improved RD performance of rate control [H. M. Choi, J. H. Nam, S. M. Yoo, D. G. Sim (KWU), Y.-H. Kim, J. H. Park (KETI)] [late]

## To be allocated | category not clear

JCTVC-J0143 AHG5: Crosscheck of redundancy removal on syntax in JCTVC-Jxxxx [J. Lee, S. Kim, S. Lee (Yonsei Univ.)] [late]

## Withdrawn and missing contributions

JCTVC-J0058 [Withdrawn]

JCTVC-J0117 [Withdrawn]

JCTVC-J0294 [Withdrawn]

JCTVC-J0384 [Withdrawn]

JCTVC-J0396 [Withdrawn]

JCTVC-J0404 [Withdrawn]

JCTVC-J0434 [Withdrawn]

JCTVC-J0441 [Withdrawn]

JCTVC-J0461 [Withdrawn]

JCTVC-J0497 [Withdrawn]

# Plenary Discussions and BoG Reports

## Project development

JCTVC-J0470 HEVC Reference Software Manual [F. Bossen, D. Flynn, K. Suehring (SW Coordinators)] [late] [miss]

## BoGs

JCTVC-J0512 BoG report on alternative coding modes [K. Chono (NEC)]

See section 5.19.

JCTVC-J0521 BoG report on Adaptive Loop Filter (ALF) [T. Yamakage]

Discussion: See under ALF.

JCTVC-J0529 BoG: CE1 visual test report [K. Ugur (Nokia), A. Saxena (Samsung)] [late]

Discussion: See under CE1.

JCTVC-J0544 BoG report on ALF boundary processing [Y.-W. Huang] [miss]

JCTVC-J0550 BoG report on VPS and NAL unit header [J. Boyce] [miss]

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XX Oct. 2012.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs

Because a draft design and HEVC test model (referred to as the HM) have now been established, group coordinated experiments are now referred to as "core experiments" rather than "tool experiments".

A preliminary CE description is to be approved at the meeting at which the CE plan is established.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for experiments were described in the output document JCTVC-F900.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the HM 5.0 software basis. Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

Non-final CE plan documents were reviewed and given tentative approval during the meeting (in some cases with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-F9xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obliged to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at the preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments

Preferred Common Conditions for experiment testing that are intended to be appropriate for both CEs and other experiments were selected by the group and described in output document JCTVC-F900.

## Software development (to be updated)

The software coordinator had already started integrating bug fixes on top of HM 3.3 software, and had strongly recommended for proponents of adopted proposals to re-implement them in HM 3.3 and test in this environment before integrating them into HM 4.x. All tools were planned to again be thoroughly tested after integration in HM 4.x.

HM 4.0 should be available within 3 weeks after the meeting, and will be used for CEs. HM 4.1 is planned to be available 3 weeks later.

## Subjective test plan for design

Plan a subjective test enabling results at the Feb meeting.

Plans for subjective testing: Preparation of visual comparison for the February meeting will need to be discussed in the next meeting. Initial idea: Use a set of rates e.g. as from CfP, and run HM and JM on same conditions. It may not be necessary to run all different constraint cases, RA may be sufficient.

Need to prepare JM & HM encodings, roughly as was done for the CfP.

# Establishment of ad hoc groups

JCTVC-J0280 Request to establish an AHG to reduce computation in residual coding [C. Auyeung, T. Suzuki (Sony)]

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC Draft and Test Model editing (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-I1002 HEVC Test Model 7 (HM 7) Encoder Description. - Produce and finalize JCTVC-I1003 HEVC text specification Draft 7. - Gather and address comments for refinement of these documents. - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, T. Wiegand (vice-chairs) | N |
| Software development and HM software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution to JCT-VC members - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 7.0 software version and the reference configuration encodings according to JCTVC-I1100 based on common conditions suitable for use in most core experiments (expected within 2 weeks after the meeting). - Prepare and deliver HM 7.1 software (and additional "dot" version software releases as appropriate) and appropriate software branches that include additional items not integrated into the 7.0 version (expected within three weeks after the 7.0 software release). - Perform analysis and reconfirmation checks of the behaviour of technical changes adopted into the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with HEVC Draft and Test Model editing AhG to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| High-level parallelism (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the implication of requiring the use of specific high-level parallelism tool(s) for very high resolution video to guarantee decoders can utilize parallel decoding. - Study the implication of having both tiles and WPP in one profile when the use of specific high-level parallelism tool(s) are required for very high resolution video. - Identify and work to resolve issues relating to the draft text description of existing high-level parallelism related tools and the associated HM implementation. - Identify and discuss additional issues relating to high-level parallelism. | M. Horowitz (chair), M. Coban, F. Henry, K. Kazui, A. Segall, W. Wan, S. Wenger, M. Zhou (vice-chairs) | N |
| Entropy Coding Improvements (AHG5) (jct-vc@lists.rwth-aachen.de) - Study the number of context coded bins of the current HM design across various syntax elements, especially in the worst-case scenario. - Investigate possible reductions to context coded bins; in particular, the reduction for reference index, delta QP, SAO offsets, and transform coefficient level coding. - Discuss and arrange test conditions and software. - Investigate the trade-off between coding efficiency and the context-coded bin reduction. - Study the CABAC initialization tables in the current HM design and their impact on test sequences both inside and outside the common conditions. - Investigate other possible improvements of the entropy coding in the current HM design. - Report the results of these studies to the JCT-VC and recommend solutions where feasible. | A. Segall (chair), C. Auyeung, K. Chono, G. Martin-Cocher, T. Nguyen, J. Sole, V. Sze, W. Wan (vice-chairs) | N |
| In-loop filtering (AHG6) (jct-vc@lists.rwth-aachen.de) - Clean up and stabilize the HM software, the draft text and the HM encoder description on non-deblocking in-loop filtering. - Study extending the adaptability range of the deblocking filter - Study harmonized control scheme of SAO and ALF. - Study SAO simplification helping efficient implementation. | T. Yamakage (chair), K. Chono, Y. J. Chiu, I. S. Chong, M. Narroschke, A. Norkin, P. Onno (vice-chairs) | N |
| Memory bandwidth restrictions in motion compensation (AHG7) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth in motion compensation. - Study memory bandwidth reduction method in the HM software and the draft text. - Study the impact of the memory bandwidth reduction techniques on complexity and coding efficiency. | T. Suzuki (chair), W. Wan, M. Zhou (vice-chairs) | N |
| Loss robustness (AHG8) (jct-vc@lists.rwth-aachen.de) - Create and/or maintain tools to test loss robustness including error patterns and a loss simulator. - Identify techniques and conditions for testing the loss robustness of the design. - Study the degree of loss robustness of the HM design and identify deficiencies. - Identify and study the interdependencies in the HM design in relation to loss robustness, and the potential consequences of these interdependencies. - Investigate solutions to improve loss robustness. - Investigate the trade-off between coding efficiency and loss robustness. | A. Rodriguez, P. Onno (co-chairs) | N |
| High-level syntax (AHG9) (jct-vc@lists.rwth-aachen.de) - Study NAL unit header, sequence parameter set, picture parameter set, adaptation parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs, including checking and fixing texts for SEI messages currently included by referring to the AVC specification. - Study the hypothetical reference decoder (HRD) syntax and operations, including making sure that the correct use of max_dec_pic_buffering[ i ] in the HDR text, as well as the text for bitstream conformance and decoder conformance. - Work towards simplification and general minor cleanup of the high-level syntax. - Assist in software development and text drafting for the high-level syntax in the HEVC design. | Y. K. Wang (chair), J. Boyce, Y. Chen, S. Deshpande, M. M. Hannuksela, M. Haque, K. Kazui, T. Schierl, R. Sjöberg, T. K. Tan, W. Wan, P. Wu (vice-chairs) | Y (one day before the July meeting) |
| Hooks for scalable coding (AHG10) (jct-vc@lists.rwth-aachen.de) - Investigate hooks that would be needed for support of bitstream scalability in HEVC syntax. | J. Boyce (chair), J. Kang, J. Samuelsson, W. Wan, Y.-K. Wang (vice-chairs) | N |
| Lossless coding (AHG11) (jct-vc@lists.rwth-aachen.de) - Complete the software using the adopted lossless coding signaling mechanism and integrate it into HM7.0. - Review the draft text on lossless coding, and suggest cleanup as appropriate. - Study and create test conditions for evaluating the efficiency of lossless signaling method. - Study and investigate possible improvements. | W. Gao (chair), K. Chono, F. Henry, J. Xu, M. Zhou, P. Topiwala (vice-chairs) | N |
| Support for range extensions (AHG12) (jct-vc@lists.rwth-aachen.de) - Examine modifications to the technical design that provide an increase in picture fidelity and support for non-4:2:0 or non-8-bit picture formats - Study aspects of the technical design and software that need modification to support non-4:2:0 chroma formats. - Study aspects of the current technical design and software that need modification to support bit-depths beyond 8 bit. - Assist and advise in the work of removing any implicit assumptions of 8-bit depth and 4:2:0 formatting from the current draft and software (where feasible, without introducing technical design changes). | D. Flynn (chair), D. Hoang, K. McCann, E. Francois, K. Sugimoto, P. Topiwala (vice-chairs) | N |
| Reference picture buffering and list construction (AHG13) (jct-vc@lists.rwth-aachen.de) - Provide source code that enables HM encoding of all test cases described in JCTVC-H0725 and produce anchor data. - Identify and work to resolve issues relating to the draft text description of reference picture handling and list construction, and the associated HM software functionality. - Study possible improvements related to reference picture buffering and list construction. - Study the loss resilience properties of reference picture handling and its support in the HM software (in coordination with the loss resilience AHG). | R. Sjöberg (chair), Y. Chen, Hendry, T. K. Tan, Y.-K. Wang (vice-chairs) | N |
| Study on HEVC conformance requirements (AHG14) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Study to develop a potential set of HEVC conformance bitstreams | T. Suzuki, W. Wan (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-I1000 Meeting Report of 9th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Suehring, D. Flynn, F. Bossen, (software coordinators)]

Remains valid

JCTVC-I1002 High Efficiency Video Coding (HEVC) Test Model 7 (HM 7) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N XXXXX)

JCTVC-I1003 High Efficiency Video Coding (HEVC) text specification draft 7 [B. Bross (primary), W.-J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand] (WG 11 N XXXXX)

JCTVC-I1004 Draft Disposition of Comments on.. [B. Bross, G. J. Sullivan, J.-R. Ohm, XXXX] (WG 11 N XXXXX)

JCTVC-I1100 Common HM test conditions and software reference configurations [F. Bossen]

Modifications include:

- SAO settings as from CE1.

- AMP on

- ALF conf. (RA mode as of I0603) in HE10, with current-picture optimization

- Transform skip off in main, on in HE10

Agreed.

Software HM7.0 availability (suitable for CTC coding efficiency experiments) two weeks after the end of the meeting. 7.1 three weeks later

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description, otherwise it cannot be claimed to be part of the CE

Document deadline of July 2012 meeting isJuly 2nd.

JCTVC-I110 Core Experiment 1: Intra transform mode dependency simplifications [K. Ugur, A. Saxena]

# Future meeting plans, expressions of thanks, and closing of the meeting

tbd

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 11–20 July 2012 under WG 11 auspices in Stockholm, SE.

- 10–19 October 2012 under WG 11 auspices in Shanghai, CN.

- 14–23 January 2013 under ITU-T auspices in Geneva, CH.

- 17-26 April 2013 under WG 11 auspices in Incheon, KR.

ITU was thanked for its excellent hosting of the 9th meeting of the JCT-VC and for providing the viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately 1753 hours on Monday 7 May 2012.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the sixth meeting of the JCT-VC, according to a sign-in sheet passed around during the meeting (approximately 255 in total), were as follows: