| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 14th Meeting: Vienna, AT, 25 July–02 Aug. 2013 | Document: JCTVC-N_Notes_d1 |
| --- | --- |

| Title: | Meeting report of the 14th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Vienna, AT, 25 July – 2 Aug. 2013 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fourteenth meeting during 25 July – 2 Aug. 2013 at the Event Hotel Pyramide, Vösendorf, Vienna, AT. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 25 July 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 02 Aug. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the eleventh JCT-VC meeting in producing the 11th HEVC Test Model (HM11) software and text, review the results from three interim Core Experiments on range extensions (RCEx) and three Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were also considered.

In addition to experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting: Draft 4 for HEVC conformance testing, draft 3 for HEVC range extensions, draft 3 of SHVC extensions and SHVC Test Model 3, and two documents specifying common test conditions and software reference configurations for experiments – one for HEVC range extension experiments, and one for scalable coding experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next five JCT-VC meetings are planned for 23 Oct. – 1 Nov. 2013 under ITU-T auspices in Geneva, CH, 9–17 Jan. 2014 under WG 11 auspices in San José, US, 27 March – 4 Apr. 2014 in Valencia, ES, 30 June – 9 July 2014 in Sapporo, JP, and XXX.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its fourteenth meeting during 25 July–02 Aug. 2013 Event Hotel Pyramide, Vösendorf, Vienna, AT. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Thursday 25 July 2013. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 2 Aug. 2013. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided at 
http://wftp3.itu.int/av-arch/jctvc-site/2013_07_N_Vienna/

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the eleventh JCT-VC meeting in producing the 11th HEVC Test Model (HM11) software and text, review the results from three interim Core Experiments on range extensions (RCEx) and three Core Experiments on scalable extensions (SCEx), and review technical input documents. Important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Monday, 15 July 2013.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Tuesday 16 July 2013 were considered "officially late".

Most documents in the “late” category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-N0295 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "M0295+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late: (update)

- JCTVC-N0XXX (a proposal relating to ...) [uploaded XX-XX]

- ...

The following other profile/level proposal contributions were registered on time but were uploaded late:

- JCTVC-N0XXX (a profile/level proposal on ...) [uploaded XX-XX]

- ...

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-N0XXX (an information contribution on ...) [uploaded XX-XX]

- ...

The following cross-verification reports were registered on time but were uploaded late: JCTVC-N0XXX [uploaded XX-XX],....

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-N0XXX, ....

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-N0XXX (a ..., corrected by a late upload on XX-XX)

- ...

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely more extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The report documents of the previous meeting, particularly including the meeting report JCTVC-M1000, the HEVC Test Model (HM) JCTVC-M1002, the Conformance Draft JCTVC-M1004, the Reference Software Draft JCTVC-M1010, the Draft Specification of Range Extensions JCTVC-M1005, the SHVC draft specification JCTVC-M1008 and SHVC test model 2 (SHM2) JCTVC-M1007 were approved. The HM reference software produced by the AHG on software development, the reference software versions for range extensions and SHVC, and HM software technical evaluation was also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- GRP: Generalized residual prediction.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SPS: Sequence parameter set (as in AVC).

- TB: transform block.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- TFD: Tagged for discard.

- Unit types:

  - CTB: code tree block (synonymous with LCU).

  - CU: coding unit.

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB).

  - PU: prediction unit, with four shape possibilities.

    - 2Nx2N: having the full width and height of the CU.

    - 2NxN: having two areas that each have the full width and half the height of the CU.

    - Nx2N: having two areas that each have half the width and the full height of the CU.

    - NxN: having four areas that each have half the width and half the height of the CU.

  - TU: transform unit.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – the draft HEVC standard corresponding to the HM.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

The status of HEVC version 1 in ISO/IEC and ITU-T was noted. The FDIS 23008-2 had been submitted for ballot in ISO/IEC (published?). In ITU-T, the text was published as Rec. H.265 on 2013-XX-XX.

The HEVC conformance testing and reference software specification had been submitted as ISO/IEC CD 23008-5, but had been planned to be split into two separate parts for software (23008-5) and conformance (23008-8) at DIS stage. The ballot closing date for XXX was 2013-07-14 and for YYY was .

23008-8 conformance ballot results m29645.

23008-5 software ballot results m29941.

The range extensions draft 2 had been submitted as ISO/IEC 23008-2/PDAM1. The ballot closing date was 2013-07-07, and ballot results were reported in m29644.

It was noted that in the most-recently-established voting process in ISO/IEC, a "No" vote has a different status than it previously did for the DIS / DAM ballot stage. WG 11 NBs should make sure to be aware of the implications of their votes, and may wish to consider voting "Yes with comments" in some circumstances in which they would previously have been inclined to vote "No with comments".

Goals: Progress of work on extensions, conformance & reference software (Study of), verification testing?

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0900 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with topics of SHVC HLS on the first day, SHVC CE work and related contributions starting on the second day, and RExt and other topics starting on the third day. Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- First day (Thu. 25 July): 0900–2000 (approximately), ....

- Second day (Fri. 26 July): 0900–2000 (approximately), Tracks A and B (and BoG on HLS)

- Third day (Sat. 27 July):

- Fourth day (Sun 28 July):

- Fifth day (Mon 29 July): Morning MPEG plenary,

  - Morning MPEG plenary

  - 1400:

- Sixth day (Tue 30 July):

- Seventh day (Wed 31 July):

  - Morning MPEG plenary

  - 1130:

  - Stopping time 1800 (for social event)

- Eighth day (Thu 1 Aug):

- Ninth day (Fri 2 Aug):

  - 0800 Morning wrap-up

  - 1300 End

## Contribution topic overview (update)

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, and discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. (Note: allocation to tracks were subject to changes)

- AHG reports (XX) Track P (section 2)

- Communication to and by parent bodies (XX) Track P (section 3.1)

- Conformance testing development (XX) Track P (section 3.2)

- Version 1 bug reports and cleanup (XX) Track P (section 3.3)

- Coding performance, implementation, and design analysis (XX) Track P (section 3.4)

- Profile an level definitions (XX) Track P (section 3.5)

- HEVC and RExt use cases (XX) Track P (section 3.6)

- Source video test material (XX) Track P (section 3.7)

- SHVC CE1: Support for additional resampling phase shifts (XX) Track X (section 4.1)

- SHVC CE2: Combined inter-layer syntax prediction and motion data compression (XX) Track X (section 4.2)

- SHVC CE3: Inter-layer filtering (XX) Track X (section 4.3)

- RExt CE1: Inter-component decorrelation (XX) Track X (section 5.1)

- RExt CE2: Prediction and coding techniques for transform skip blocks (XX) Track X (section 5.2)

- RExt CE3: Intra coding methods for screen content (XX) Track X (section 5.3)

- Non-CE RExt (XX CE related, XX other) Track X (section 6.1)

- Non-CE SHVC (XX CE related, XX other) Track X (section 6.2)

- High-level syntax in RExt & single layer (XX) Track X (section 6.3)

- High-level syntax in SHVC and 3D extensions (XX) BoG | joint with JCT-3V (section 6.4)

- High-level syntax in SHVC (XX) BoG | Track X (section 6.5)

- SEI messages (XX) BoG | Track X (section 6.6)

- Non-normative (XX) Track X (section 6.7)

- Withdrawn and unclear category (XX) (section 6.8)

- Plenary discussions and BoG reports (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC. (sections 8, 9, and 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: XX; Track A: XXX; Track B: XXX.

Version 1 verification testing (need to consult with V. Baroncini):

- AVC vs. HEVC

- Emphasis on subjective quality

- Scenarios (random access, low-delay, all-intra?, photographs?)

- Test sequence selection

- Bit rates & resolutions & bit depths

- Reference software vs. proprietary encoders – TBD

  - Note that perceptual optimization tricks certainly apply to both AVC and HEVC

  - Perhaps do both a fixed-QP similarly configured reference software capability comparison and a comparison to some non-RS encoding (accompanied by suitable caveats)

Review prior test plans (e.g. for AVC verification tests and CfPs).

# AHG reports

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

JCTVC-N0001 Ad hoc group report: JCT-VC project management [G. J. Sullivan, J.-R. Ohm]

M0001

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

The primary goals of the meeting were to review the work that was performed in the interim period since the 13th JCT-VC meeting in producing the 11th HEVC Test Model (HM11) software and text, and further consider on possible corrective actions. Other important topics of the meeting were the review of progress made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. In this context, three interim Core Experiments on range extensions (RCEx) and three Core Experiments on scalable extensions (SCEx) had been conducted. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal.

The work of the JCT-VC overall had proceeded well and actively in the interim period. Active discussion had been carried out on the group email reflector (which had 1750 subscribers as of 2013-07-24), and the output documents from the preceding meeting had been produced.

Except as noted below, all report documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2013_04_M_Incheon/), particularly including the following:

- The meeting report (JCTVC-M1000) [Posted 2013-07-25]

- The HEVC software guidelines (JCTVC-H1001) [Not updated for several meeting cycles]

Note: The "H" document, in this case, remains valid as the latest approved software guidelines.

- The HM 11 encoder description (JCTVC-M1002) [Posted 2013-07-23]

- HEVC conformance specification Working Draft 3, submitted as ISO/IEC CD (JCTVC-M1004) [Posted 2013-05-10]

- HEVC range extensions Working Draft 3 (JCTVC-M1005) [First posted 2013-06-15, last updated 2013-06-21]

- HEVC range extensions common test conditions and software reference configurations (JCTVC-L1006) [The “L” document remains valid, but a small update was posted 2013-06-04]

- SHVC Test Model 2 (JCTVC-M1007) [Posted 2013-06-06]

- SHVC Working Draft 2 (JCTVC-M1008) [First posted 2013-05-21, last updated 2013-06-14]

- Common SHM test conditions and software reference configurations (JCTVC-M1009) [Posted 2013-05-07, last updated 2012-05-23]

- HEVC HM11 Reference Software, submitted as ISO/IEC Study of CD (JCTVC-M1010) [Posted 2013-06-14]

- Description of HEVC Scalable Extensions Core Experiment 1 (SCE1): Support for additional resampling phase shifts (JCTVC-M1101) [Posted 2013-05-18]

- Description of HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression (JCTVC-M1102) [Posted 2013-05-20 with final update 2013-06-24]

- Description of HEVC Scalable Extensions Core Experiment 3 (SCE3): Inter-layer filtering (JCTVC-M1103) [First posted 2013-05-18 with final update 2013-06-24]

- Description of HEVC Range Extensions Core Experiment 1 (RCE1): Inter component decorrelation methods (JCTVC-M1121) [First posted 2013-05-17 with final update 2013-06-25]

- Description of HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks (JCTVC-M1122) [First posted 2013-04-26 with final update 2013-06-04]

- Description of HEVC Range Extensions Core Experiment 2 (RCE2): HEVC Range Extensions Core Experiment 3 (RCE3): Intra Coding Methods for Screen Content (JCTVC-M1123) [First posted 2013-05-01 with final update 2013-05-29]

The current HM reference software manual has been continually updated along with the software itself, and a version-controlled copy is now included in the "doc" directory of the HM software repository. A PDF version has been produced and is included in the same location prior to each HM release.

The various ad hoc groups and the six core experiments had made progress, and various reports from those activities had been submitted.

The software for HM version 11.0 had been prepared and released approximately as scheduled. Progress was made to build the software for SHVC and Range Extensions on top of HM11 as well.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-11.0.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-11.0-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 330 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted. However, the relatively early upload deadline established for the current meeting (2013-07-15, seven days in advance of the meeting) has helped to enable advance study of most technical input material.

A preliminary basis for the document subject allocation and meeting notes for the 14th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted web site.

JCTVC-N0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs)]

JCTVC-N0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [F. Bossen, D. Flynn, K. Sühring] [miss]

JCTVC-N0004 JCT-VC AHG Report: HEVC conformance test development (AHG4) [T. Suzuki, W. Wan]

- JCTVC-N0005 JCT-VC AHG report: HEVC range extensions development (AHG5) [M. Naccari, C. Rosewarne]

JCTVC-N0006 JCT-VC AHG report: Range extensions draft text (AHG6) [J. Sole, D. Flynn, C. Rosewarne, T. Suzuki]

N0007 TBA

- JCTVC-N0008 JCT-VC AHG report: Screen content coding (AHG8) [H. Yu, R. Cohen, A. Duenas, T. Lin, J. Xu]

- JCTVC-N0009 JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9)[M. M. Hannuksela, J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang]

N0009

This report summarized the activities of the ad-hoc group of high-level syntax for HEVC extensions (AHG9) between the 13th JCT-VC meeting and the 14th JCT-VC meeting and proposed a categorization of the related input contributions.

There have not been many discussions related to the mandates of this ad-hoc group over the reflector between the 13th and 14th JCT-VC meetings.

An accompanying change-marked version of JCTVC-N_Notes_d0 suggested a refined categorization of AHG9-related contributions.

JCTVC-N0010 JCT-VC AHG report: SHVC core experiments (AHG10) [X. Li, J. Boyce, P. Onno, Y. Ye]

JCTVC-N0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela] [miss]

JCTVC-N0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He, T.-D. Chuang, D.-K. Kwon] [miss]

N0013 TBA

N0014 TBA

JCTVC-N0015 JCT-VC AHG report: SHVC hybrid codec scalability (AHG15) [J. Boyce, K. Kawamura]

Here, "hybrid scalability" refers to the use of non-HEVC base layers.

The SHM 2.0 software release of May 15 supports an AVC base layer.

Reporting template and anchors for experiments using an AVC base layer were distributed on May 23 as attachments to JCTVC-M1009.

Two relevant contributions were noted: JCTVC-N0050 and JCTVC-N0211.

The VPS extension design in the SHVC WD includes an avc_base_layer_flag syntax element to indicate an AVC base layer.  An editorial comment in the document reads:

[Ed. (YK): For possible support of base layer of other codecs, e.g. MPEG-2, a flag is not sufficient.]

Ajay Luthra raised the question of whether some reserved fields should be added now to the VPS to support an MPEG-2 base layer in the future. It was noted that a vps_extension2_flag syntax element is already present in the design.

It was agreed that there does not seem to be a clear need to identify what non-HEVC encoding was used to generate a base layer unless the base layer is encapsulated within the same video elementary stream.

It was noted that access unit specification along with definition of base layer decoding order and output order (as related to enhancement layer decoding order and output order) is needed. And the HRD flow relationship between the base and enhancement layers. Our plan has been that the access unit structure, decoding order and output order are locked together for the base and enhancement layers.

If we do decide to specify encapsulation of the base layer within the video elementary stream, we should have a type code syntax capable of indicating multiple base layer coding types (e.g. AVC, MPEG-2, and other values reserved for future use).

It was noted that an encapsulation method is not currently in our draft text.

Revisit to determine whether to change the current HEVC-vs-AVC base layer indication or add the type indication.

JCTVC-N0016 JCT-VC AHG report: Single-Loop Scalability (AHG16) [M. Wien, J. Boyce, M. Budagavi, K. Mishra, K. Ugur]

JCTVC-N0017 JCT-VC AHG report: SHVC complexity assessment (AHG17) [M. Budagavi, E. Alshina, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai]

N0018 TBA

JCTVC-N0019 JCT-VC AHG report: Verification test preparation (AHG19) [T.K. Tan (NTT Docomo), V. Baroncini (FUB), W. Wan (Broadcom), M. Karczewicz (Qualcomm), J. Wen (Tsinghua Uni)]

N0020 TBA

N0021 TBA

# Project development, status, and guidance

## Communication to and by parent bodies

## Conformance test set development

JCTVC-N0284 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, G. Sullivan, W. Wan] [miss]

## Version 1 bug reports and cleanup

JCTVC-N0041 Editors' proposed corrections to HEVC version 1 [B. Bross, G. J. Sullivan, Y.-K. Wang]

JCTVC-N0094 HEVC version 1 (corrigendum): Derivation of CPB removal time [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-N0230 SAO software cleanup and non-normative encoder-only bug-fixes [C.-Y. Tsai, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-N0306 Cross-check for HM11.0 SAO code clean-up and encoder bug-fix [E. Alshina, A. Alshin (Samsung)] [late]

JCTVC-N0324 Clarifications on HEVC NALU order [Jean Le Feuvre, Cyril Concolato, Mickael Raulet] [late] [miss]

## HEVC coding performance, implementation demonstrations and design analysis

### Coding performance

### Implementation demonstrations

JCTVC-N0313 4EVER HEVC demonstrations during Roland Garros tournament [S. Kervadec, M. Raulet, J. Le Feuvre, J. Vieron, M. Clare (??)] [late]

JCTVC-N0276 A hardware oriented implementation of HEVC encoding [Ryoji Hashimoto, Seiji Mochizuki, Kenichi Iwata (Renesas)]

### Design analysis

## Profile and level definitions (requirements related)

JCTVC-N0050 also has aspects related to profile/level.

JCTVC-N0178 HEVC profiles for medical imaging applications [P. Amon, A. Hutter, U.-E. Martin, N. Wirsz (Siemens)]

JCTVC-N0191 AHG 5 and 18: Profiles for Range Extensions [K. Sharman, N. Saunders, J. Gamei, T. Suzuki (Sony)]

JCTVC-N0312 A proposal on HEVC 4:2:2 profile [S. Sekiguchi, A. Minezawa, H. Sakate (Mitsubishi)] [late] [miss]

JCTVC-N0237 HEVC version 1: Level 2.2 for support of WVGA@30fps [Y.-K. Wang (Qualcomm)]

JCTVC-N0346 Proposed editorial changes for maximum number of slices and tiles per picture limit [M. Zhou, B. Heng, W. Wan (Broadcom)] [late]

## HEVC and RExt use cases (requirements related)

JCTVC-N0291  [AHG21] Best-effort decoding of 10-bit sequences: use cases, requirements and specification methods [D. Flynn, G. Martin-Cocher, D. He (RIM)]

## Source video test material

JCTVC-N0163 AHG14: Wide Color Gamut Test Material Creation [Pierre Andrivon, Philippe Bordes (Technicolor)]

JCTVC-N0179 Selected medical imaging sequences for HEVC development [P. Amon, A. Hutter, U.-E. Martin, B. Heigl (Siemens)] [miss]

JCTVC-N0294 AHG8: 4:4:4 game content sequences for HEVC Range Extensions development [R. Cohen (MERL), F. Liu, C. S. Ping, N.-M. Cheung, Y. Chau, S.-K. Yeung (Singapore Univ. of Technology and Design)]

# Core experiments in SHVC

## SCE1: Support for additional resampling phase shifts

### SCE1 summary and general discussion

JCTVC-N0031 SCE1: Summary Report of SHVC Core Experiment on support for additional re-sampling phase shifts [E. Alshina, X. Li, J. Dong]

### SCE1 primary contributions

JCTVC-N0045 SCE1: Results of Test 1.1.3 on accurate chroma position alignment [X. Li, J. Chen, L. Guo, M. Karczewicz (Qualcomm), J. Dong, Y. Ye, Y. He (InterDigital), K. Ugur, J. Lainema (Nokia)]

JCTVC-N0046 SCE1: Results of Test 1.2.1 on Adaptive Re-Sampling Filter [X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0078 SCE1: Test 2.2 report [K. Minoo, D. Baylon]

JCTVC-N0182 SCE1: Results of test 1.1 and 1.2 on sampling offset signaling [J. Dong, Y. Ye, Y. He (InterDigital), X. Li, J. Chen, L. Guo, M. Karczewicz (Qualcomm), K. Ugur, J. Lainema (Nokia)]

JCTVC-N0308 SCE1: Verification of test 2.1 on picture and sequence level adaptive re-sampling filter [E. Alshina, A. Alshin (Samsung)] [late]

### SCE1 cross checks

JCTVC-N0193 SCE1: Cross-check for Test 2.2 on phase compensation by signaling filter coefficients at PPS with sample shift [J. Dong, Y. Ye (InterDigital)]

JCTVC-N0225 SCE1: Cross-check for Test1.2, The accuracy of signaled up-sampling phase offset [K. Minoo, D. Baylon (??)]  [late]

JCTVC-N0226 SCE1: Cross-check for Test1.3, accurate Chroma position alignment [K. Minoo, D. Baylon (??)] [late]

## SCE2: Combination of inter-layer syntax prediction and motion data compression

### SCE2 summary and general discussion

JCTVC-N0032 SCE2: Summary report of SHVC core experiment on combination of inter-layer syntax prediction and motion data compression [Christophe Gisquet, Kazushi Sato]

### SCE2 primary contributions

JCTVC-N0139 SCE2: Results on test 2.1 [C. Gisquet, P. Onno, E. François, G. Laroche (Canon)]

JCTVC-N0239 SCE2: Result of Test 1.1 [C. Gisquet (Canon), K. Sato, J. Xu (Sony)]

JCTVC-N0241 SCE2: Result of Test 1.4 [C. Gisquet (Canon), K. Sato, J. Xu (Sony), K. Misra (Sharp)]

JCTVC-N0245 SCE2: Result of Test 2.2 [K. Sato, J. Xu (Sony)]

JCTVC-N0252 SCE2 test 2.3 motion buffer modification results [K. Misra, A. Segall, J. Zhao (Sharp)]

JCTVC-N0255 SCE2 test 1.3 results [K. Misra, A. Segall (Sharp), K. Sato, J. Xu (Sony)]

JCTVC-N0302 SCE2: Results on combination test 1.2 [C. Gisquet, K. Misra, A.Segall (??)] [late]

### SCE2 cross checks

JCTVC-N0122 SCE2: Cross-check of test2.3 on motion field buffer update [J. Lee, H. Lee, J. W. Kang (ETRI)]

JCTVC-N0240 SCE2: Crosscheck Result of Test 1.2 [K. Sato (Sony)]

JCTVC-N0243 SCE2: Crosscheck Result of Test 2.1 [K. Sato (Sony)]

JCTVC-N0257 SCE2 Cross check report of test 1.1 [K. Misra, A. Segall (Sharp)] [late]

JCTVC-N0259 SCE2 Cross check report of test 2.2 [K. Misra, A. Segall (Sharp)] [late]

JCTVC-N0300 Crosscheck of SCE 2 test 1.4 [C. Kim, B. Jeon (LGE)] [late]

JCTVC-N0301 Crosscheck of SCE 2 test 1.3 [C. Kim, B. Jeon (LGE)] [late]

## SCE3: Inter-layer filtering

### SCE3 summary and general discussion

JCTVC-N0033 SCE3: Summary Report of SHVC Core Experiment on Inter-layer Filtering [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong]

### SCE3 primary contributions

JCTVC-N0047 SCE3: Results of Test 3.2 on Adaptive Re-Sampling Filter [W. Pu, X. Li, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0140 SCE3: Results of test 3.3 on Interlayer SAO filtering for SHVC [G. Laroche, J. Taquet, P. Onno (Canon)]

JCTVC-N0151 SCE3: Results of test 3.1 on switchable alternative inter-layer filter [P.Wei, V.Seregin, J.Chen, X.Li, M.Karczewicz (Qualcomm), E.Alshina, A.Alshin (Samsung)]

JCTVC-N0152 SCE3: Results of test 3.4.2 on simplified cross-color inter-layer inter layer prediction [E.Alshina, A.Alshin, Y.Cho (Samsung), J.Dong, Y.Ye, Y.He (InterDigital)]

JCTVC-N0184 SCE3: Results of test 3.4.1 on chroma enhancement for inter layer prediction [J. Dong, Y. Ye, Y. He (InterDigital)]

JCTVC-N0220 SCE3: Results of test.3.5.1 on adaptive up-sampling of base layer picture using simplified separable bilateral filters [J. Zhao, K. Misra, A. Segall (Sharp)]

JCTVC-N0303 SCE3: Verification of test 3.2 on adaptive re-sampling filter [E. Alshina, A. Alshin (Samsung)] [late]

JCTVC-N0304 SCE3: Verification of test 3.3 on high frequency pass inter-layer SAO [E. Alshina, A. Alshin (Samsung)] [late]

### SCE3 cross checks

JCTVC-N0194 SCE3: Cross-check for Test 3.1 on switchable alternative inter-layer filter [J. Dong, Y. Ye (InterDigital)] [late]

JCTVC-N0208 SCE3: Crosscheck of test 3.4.2 on simplified cross-color inter-layer inter layer prediction [X. Li] [late]

JCTVC-N0221 SCE3: Cross-check of test 3.1 on switchable alternative filters (all phases) [J. Zhao, A. Segall (Sharp)] [late]

JCTVC-N0251 SCE3 : cross-check of Test 3.4.1 on chroma enhancement for inter layer prediction [J. Xu (Sony)] [late]

JCTVC-N0305 SCE3: Cross-check for test 3.5 (bi-lateral inter-layer filter) [A. Alshin, E. Alshina (Samsung)] [late]

JCTVC-N0343 SCE3: Cross-check of SCE3 Test3.1 non-switchable inter-layer filter [K. Ugur (Nokia)] [late] [miss]

# Core experiments in Range Extensions

## RCE1: Inter-component decorrelation methods

### RCE1 summary and general discussion

### RCE1 primary contributions

JCTVC-N0170 RCE1: Description and Results for Experiment 3, 5, and 6 [T. Nguyen (Fraunhofer HHI)] [late]

JCTVC-N0227 RCE1: EXP1/4 The performance of extended chroma mode for non 4:2:0 format [J. Kim, B.Jeon (LGE)]

### RCE1 cross checks

JCTVC-N0114 RCE1: Cross-check on Experiment 5 [A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-N0171 RCE1: Cross Check Results for Experiment 1 and 4 [T. Nguyen (Fraunhofer HHI)] [late]

JCTVC-N0298 RCE1: Cross-check on Experiment 6 [Wei Pu, Woo-Shik Kim (Qualcomm)] [late] [miss]

## RCE2: Prediction and coding techniques for transform-skip blocks

### RCE2 summary and general discussion

JCTVC-N0035 RCE2: Summary report on HEVC Range Extensions Core Experiment on prediction and coding techniques for transform-skip and transform-bypass blocks [R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari] [miss]

### RCE2 primary contributions

JCTVC-N0044 RCE2 Test B.1: Residue rotation and significance map context [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0052 RCE2 subtest C.2: Extension of residual DPCM to lossy coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0053 RCE2 subtest C.4: Replacement of DC mode for transform-skip and transform-bypass blocks [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0071 RCE2: Simplified sample-based weighted prediction (Test C.3) [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)]

JCTVC-N0073 RCE2: Description and report for Test D.3 [E. Wige, A. Kaup (Universität Erlangen-Nürnberg), P. Amon, A. Hutter (Siemens), R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0074 RCE2: Experimental results for Test C.1 [M. Naccari, M. Mrak (BBC)]

JCTVC-N0083 RCE2 subtest D.1: Extension of intra and inter residual DPCM to lossy coding [R. Joshi, J. Sole, M. Karczewicz (Qualcomm), M. Naccari, M. Mrak (BBC)]

JCTVC-N0096 RCE2: Test B.2: Swapping of scan order for transform skip/bypass [R. Cohen, A. Vetro (MERL)]

JCTVC-N0101 RCE2 Test A.3: Simplified update of the coefficient level Rice parameter [J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0175 RCE2 Test B.3: Mirroring of Coefficients and significance map context [R.Weerakkody, M.Mrak (BBC)]

JCTVC-N0207 RCE2: Combination of Mirroring of Coefficients (Test B.3), Significance map context (Test B.1) and RDPCM (Test D.1) [R.Weerakkody, M.Mrak, M.Naccari (BBC)]

JCTVC-N0228 RCE2: Results for Test A.2 Rice parameter update [M. Budagavi (TI)]

JCTVC-N0268 RCE2 Test A.1: Rice parameter signaling for transform-skip blocks [S.H. Kim, A. Segall (Sharp)]

### RCE2 cross checks

JCTVC-N0076 RCE2: Cross-check for Test B.2 [M. Naccari, M. Mrak (BBC)]

JCTVC-N0097 RCE2: Cross-verification of JCTVC-N0044, Test B.1: Residue rotation and significance map context [R. Cohen (MERL)]

JCTVC-N0098 RCE2: Cross-verification of JCTVC-N0073, Description and report for Test D.3 [R. Cohen (MERL)]

JCTVC-N0099 RCE2: Cross-verification report for subtest B.3 (N0175) on mirroring of coefficients [R. Joshi (Qualcomm)]

JCTVC-N0103 RCE2: Cross-verification of Test C.1 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-N0105 RCE2: Cross-check of Test A.2 on coefficient level Rice parameter update (JCTVC-N0228) [J. Sole (Qualcomm)]

JCTVC-N0123 RCE2: Cross-check of Test A.2 [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0136 RCE2: Cross-check of Sub-test D.1 (JCTVC-N0083) by the BBC and Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-N0156 RCE2: Crosscheck for Test C.3 [W. Zhang, Y. Chiu (Intel)] [miss]

JCTVC-N0164 RCE2: Cross-check results for Test C.4 (JCTVC-N0053) [E. Wige, A. Kaup (Universität Erlangen-Nürnberg), P. Amon, A. Hutter (Siemens)]

JCTVC-N0328 Cross-check report for RCE2 C2 [J. Ye, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0329 Cross-check report for RCE2 A1 [J. Ye, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0330 Cross-check report for RCE2 B3 [J. Ye, Z.Ma, M. Xu (Huawei)] [late]

JCTVC-N0337 RCE2: Cross-check report for Test A3 [Seung-Hwan Kim, Andrew Segall (Sharp)] [late]

JCTVC-N0354 Non-RCE2: Results for combination of Mirroring of non-transformed residual blocks (Test B.3) with other methods [R. Weerakkody, M. Mrak, M. Naccari (BBC)] [late] [miss]

## RCE3: Intra coding methods for screen content

JCTVC-N0036 RCE3: Summary Report of HEVC Range Extensions Core Experiment 3 on Intra Coding Methods for Screen Content [L. Guo]

### RCE3 summary and general discussion

### RCE3 primary contributions

JCTVC-N0205 RCE3: Results of test 3.3 on Intra motion compensation [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-N0247 RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding [L. Guo, M. Karczewicz, J. Sole (Qualcomm)]

JCTVC-N0287 RCE3 Test 2: Multi-stage Base Color and Index Map [W. Zhu (BJUT), J. Xu (Microsoft), W. Ding (BJUT)]

### RCE3 cross checks

JCTVC-N0104 RCE3: Cross-verification of Test 3.1 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-N0125 RCE3: Cross-check of Test 3.2 [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0126 RCE3: Cross-check of Test 3.3 [X. Wei, J. Zan (Huawei)] [late]

JCTVC-N0326 Cross-check report for RCE 3.1 [X. Wang, Z. Ma, M. Xu (Huawei)] [late]

JCTVC-N0327 Cross-check report for RCE 3.3 [X. Wang, Z. Ma, M. Xu (Huawei)] [late]

# Non-CE Technical Contributions

## Range extensions

### General

### RCE1 related (inter-component decorrelation)

JCTVC-N0266 Non RCE1: Inter Color Component Residual Prediction [W. Pu, W.-S. Kim, C. Chen, L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0223 In-loop Chroma Enhancement for HEVC Range Extensions [J. Dong, Y. Ye, Y. He (InterDigital)]

JCTVC-N0359 Non-RCE1: Cross-check of JCTVC-N0223 In-loop Chroma Enhancement for HEVC Range Extensions [L. Guo (Qualcomm)] [late]

### RCE2 related (prediction and coding for transform skip)

JCTVC-N0042 Non-RCE2: Restriction on the Residual DPCM block size [J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0279 Cross-check of JCTVC-N0042 on Restriction on the Residual DPCM block size [E. François (Canon)] [late]

JCTVC-N0072 RCE2-related: Variants of simplified sample-based weighted prediction [P. Amon, A. Hutter (Siemens), E. Wige, A. Kaup (Universität Erlangen-Nürnberg)]

JCTVC-N0075 Non-RCE2: Complexity reduction for inter residual DPCM in lossless coding [M. Naccari, M. Mrak (BBC)]

JCTVC-N0293 Cross-check of complexity reduction for inter residual DPCM in lossless coding (JCTVC-N0075) [J. Xu (Microsoft)] [late]

JCTVC-N0079 Non-RCE2: Simplified sample based intra prediction for lossless coding [J.Zhu, W. Zheng, K.Kazui (Fujitsu)]

JCTVC-N0080 Non-RCE2: Skip of neighbouring samples filtering in intra prediction for lossless coding [J. Zhu, K. Kazui (Fujitsu)]

JCTVC-N0100 Non-RCE2: Unified lossless residual coding [Y. H. Tan, C. Yeo (I2R)]

JCTVC-N0321 Cross-check for JCTVC-N0100: Non-RCE2: Unified lossless residual coding [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-N0137 Non-RCE2: Golomb-Rice parameter initialization for transform-skip and transquant-bypass modes [V. Kolesnikov, C. Rosewarne, M. Maeda (Canon)]

JCTVC-N0350 Non-RCE2: Cross-verification of JCTVC-N0137, Golomb-rice parameter initialization for transform-skip and transquant-bypass modes [R. Cohen (MERL)] [late]

JCTVC-N0176 Non-RCE 2: On sample adaptive intra prediction for oblique modes in lossless coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

JCTVC-N0319 Crosscheck of sample adaptive intra prediction for oblique modes in lossless coding (JCTVC-N0176) [D.-K. Kwon (TI)] [late]

JCTVC-N0177 Non-RCE 2: On sample adaptive intra prediction for oblique modes in lossy coding [A. Saxena, H. Chen, F. Fernandes (Samsung)]

JCTVC-N0181 Non-RCE2: Rice Parameter Initialization [M. Karczewicz, J. Sole, R. Joshi (Qualcomm)]

JCTVC-N0222 Non-RCE2: Results for combination of methods [J. Sole, R. Joshi, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-N0353 Cross-check for JCTVC-N0222, Non-RCE2: Results for combination of methods [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-N0232 Non-RCE2: Rice parameter update method [J. Min, S. Lee, C. Kim (Samsung)]

JCTVC-N0325 Non-RCE2: Cross-verification of JCTVC-N0232 Rice parameter update method [L. Guo (Qualcomm)] [late]

JCTVC-N0258 Non-RCE2: Extension of TU-Based Inter RDPCM [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0342 Cross-check of JCTVC-N0258 on TU-Based inter RDPCM extension [H. Yang (Huawei)] [late] [miss]

JCTVC-N0281 Non-RCE2 Rice parameter extension for transform-skip blocks [S.H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-N0333 Non-RCE2 Cross-check of N0281 (Rice parameter extension for transform-skip blocks) [J. Min, S. Lee (Samsung)] [late]

JCTVC-N0288 Non-RCE2: Transform skip on large TUs [X. Peng, J. Xu (Microsoft), L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0335 Cross-check for JCTVC-N0288: Non-RCE2: Transform skip on large TU [M. Naccari, M. Mrak (BBC)] [late]

JCTVC-N0167 Transform skip based on minimum TU size [Kwanghyun Won, Seungha Yang, Byeungwoo Jeon (SKKU)] [late]

JCTVC-N0289 Cross-check of transform skip based on minimum TU size (JCTVC-N0167) [J. Xu (Microsoft)] [late]

### RCE3 related (intra coding for screen content)

JCTVC-N0169 Non-RCE3:Template-based palette prediction [Wenjing Zhu, Haitao Yang (Huawei)]

JCTVC-N0351 Cross-check of template-based palette prediction (JCTVC-N0169) [J. Xu (Microsoft)] [late] [miss]

JCTVC-N0206 Non-RCE3: Intra motion compensation with variable length intra MV coding [D.-K. Kwon, M. Budagavi (TI)]

JCTVC-N0348 Non-RCE3: Cross-check of Intra Motion Compensation in JCTVC-N0206 [W.-S. Kim (Qualcomm)] [late] [miss]

JCTVC-N0235 Non-RCE3: base color merging for MBCIM [J. Xu, A. Tabatabai (Sony)]

JCTVC-N0323 Non-RCE3: Cross-check of JCTVC-N0235 base color merging for MBCIM [L.Guo (Qualcomm)] [late]

JCTVC-N0249 Non-RCE3: Modified Palette Mode for Screen Content Coding [L. Guo, M. Karczewicz, J. Sole, R. Joshi (Qualcomm)]

JCTVC-N0332 Non-RCE3: Cross-check of N0249 (Modified Palette Mode for Screen Content Coding) [J. Min, S. Lee (Samsung)] [late]

JCTVC-N0254 Non-RCE3: Pipeline Friendly Intra Motion Compensation [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0256 Non-RCE3: 2-D MV Supported Intra Motion Compensation [C. Pang, J. Sole, L. Guo, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-N0340 Non-RCE3: Cross-check of N0256 (Intra Motion Compensation with 2-D MVs) [J. Min, E. Alshina (Samsung)] [late] [miss]

JCTVC-N0285 Non-RCE3: Intra motion compensation for screen contents [J. Min, M.W Park, S. Lee, C. Kim (Samsung)]

JCTVC-N0341 Non-RCE3: Crosscheck for JCTVC-N0285 Intra motion compensation for screen contents [C. Pang (Qualcomm)	] [late]

JCTVC-N0231 AHG 8: Intra mode coding for screen contents [J.Min, S.Lee, C.Kim (Samsung)]

JCTVC-N0322 AHG 8 Cross-check for JCTVC-N0231: Intra mode coding for screen contents [M. Naccari, M. Mrak (BBC)] [late]

### Transforms and transform coefficient coding

JCTVC-N0138 AHG5: Square transform deblocking for 4:2:2 [C. Rosewarne, V. Kolesnikov, M. Maeda (Canon)]

JCTVC-N0192 AHG 5: 32x32 Scaling List Derivation for Chroma [K. Sharman, N. Saunders, J. Gamei (Sony)]

### Intra prediction

JCTVC-N0143 On Mode Dependent Intra Smoothing for Range Extension [G. Laroche, C. Gisquet, T. Poirier (Canon)]

JCTVC-N0349 Cross-check of Mode Dependent Intra Smoothing in JCTVC-N0143 [W.-S. Kim (Qualcomm)] [late] [miss]

JCTVC-N0183 Non-RCE 2: Enhanced angular intra prediction for screen content coding [H. Chen, A. Saxena, F. Fernandes (Samsung)]

JCTVC-N0358 Cross-check for JCTVC-N0183 Non-RCE2: Enhanced angular intra prediction for screen content coding	[M. Naccari, M. Mrak (BBC)] [late]

### High bit depth

JCTVC-N0142 AHG18: On 16-bits support for Range Extensions [E. François, J. Taquet (Canon)]

JCTVC-N0188 AHG 5 and 18: Internal Precision for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-N0189 AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-N0338 Cross-check report for JCTVC-N0189 : AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths [Seung-Hwan Kim, Andrew Segall (Sharp)] [late] [miss]

JCTVC-N0190 AHG 5 and 18: Entropy Coding Throughput for High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-N0201 SAO extension for higher bit-depth coding [Alexis Tourapis (??)]

JCTVC-N0253 Cross-check of JCTVC-N0201 on SAO extension for higher bit-depth coding [J. Xu, A. Tabatabai (Sony)] [miss]

JCTVC-N0275 AHG18: Modified scaling factor for transform-skip blocks to support higher bit depths greater than equal to 14 [S.H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-N0336 Cross check AHG 5 and 18: Entropy Coding Throughput for High Bit Depths [Wei Pu, Woo-Shik Kim] [late] [miss]

### Lossless and screen content coding related contributions

JCTVC-N0113 Cross Residual DPCM for HEVC lossless coding [Yung-Lyul Lee, Sung-Wook Hong]

JCTVC-N0115 On RGB to YCbCr conversion for screen contents [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)] [late]

### Other

JCTVC-N0141 AHG5: on chroma QP for HEVC Rext [E. François, C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-N0295 AhG5: Cross-check of Chroma QP for HEVC RExt in JCTVC-N0141 [W.-S. Kim (Qualcomm)] [late]

JCTVC-N0116 AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)] [late]

JCTVC-N0148 AhG8: Guided Image Filtering for Screen Content Coding [T. Vermeir (Barco), J. De Cock, G. Van Wallendael, S. Van Leuven (Ghent University - iMinds)]

JCTVC-N0246 AHG5: Modified SAO for range extensions [S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-N0331 AHG 5: Cross-check of N0246 (Modified SAO for range extensions) [J. Min, E. Alshina (Samsung)] [late]

JCTVC-N0261 AhG5: Memory Bandwidth Reduction for HEVC Rext [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0310 Cross check of Memory Bandwidth Reduction for HEVC RExt (JCTVC-N0261) [G. Laroche (??)] [late] [miss]

JCTVC-N0263 AhG5: Deblocking Filter in 4:4:4 Chroma Format [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-N0320 Crosscheck of JCTVC-N0263 on deblocking filter in 4:4:4 chroma format [D.-K. Kwon (TI)] [late]

JCTVC-N0292 RExt: Fidelity adaptive coding mode [D. Flynn, N. Nguyen, D. He (RIM)]

JCTVC-N0309 Signalling of chroma sampling filter [K.Kazui (Fujitsu), T.Chujoh(Toshiba)]  [late] [miss]

## SHVC

### General

JCTVC-N0150 AhG17: complexity analysis of SHM2.0 [E.Alshina, A.Alshin (Samsung)]

JCTVC-N0242 Editorial improvements on SHVC Draft Text 2 [J. Chen, J. Boyce, Y. Ye, M. Hannuksela]

### SCE1 related (resampling phase)

JCTVC-N0149 Non-SCE1: Results of test 1.2 on sampling offset signalling with accurate interpolation filter [E.Alshina, A.Alshin (Samsung)]

JCTVC-N0214 Non-SCE1: Dynamic range control of intermediate data in re-sampling process [J. Chen, X. Li, M. Karczewicz (Qualcommm)]

JCTVC-N0219 Non-SCE1: On arbitrary spatial ratio scalability in SHVC [J.Chen, X.Li, M.Karczewicz (Qualcomm), E.Alshina, A.Alshin (Samsung), K.Ugur (Nokia)]

JCTVC-N0272 Non SCE1: On handling resampling phase offsets with fixed filters [K. Minoo, D. Baylon, A. Luthra (??)]

JCTVC-N0315 Cross-check of JCTVC-N0219: Non-SCE1 On arbitrary spatial ratio scalability in SHVC [P. Lai, S. Liu, S. Lei (Mediatek)][late]

JCTVC-N0317 Non-SCE1: Crosscheck of JCTVC-N0149 on sampling offset signalling with accurate interpolation filter [X. Li (Qualcomm)] [late] [miss]

JCTVC-N0345 A downsampler supporting arbitrary spatial ratios [J. Dong, Y. Ye (InterDigital)] [late] [miss]

### SCE2 related (inter-layer syntax prediction and motion compression)

JCTVC-N0233 Non-SCE2: scaled MV in motion field buffer update [J. Xu, A. Tabatabai, K. Sato (Sony)]

JCTVC-N0262 Cross check report of JCTVC-N0233 [K. Misra, A. Segall (Sharp)] [late]

### SCE3 related (inter-layer filtering)

JCTVC-N0061 Non-SCE3.3: Inter-layer interpolation-based SAO filtering for SHVC [Alexey Filippov, Vasily Rufitskiy (Huawei)]

JCTVC-N0344 Non-SCE3.3: Crosscheck of JCTVC-N0061 on Inter-layer interpolation-based SAO filtering for SHVC [X. Li (Qualcomm)] [late] [miss]

JCTVC-N0070 Non-SCE3: Inter-layer prediction modes based on base layer sharpness filter [M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei)]

JCTVC-N0229 Non-SCE3: Region based Inter-layer Cross-Color Filtering [X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin, Y. Cho (Samsung)]

JCTVC-N0311 Cross check of Region based Inter-layer Cross-Color Filtering (JCTVC-N0229) [E. François (??)] [late]

JCTVC-N0250 Non-SCE3.3: Modified Interlayer SAO with highpass processing [S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-N0307 non-SCE3: Cross-check for inter-layer SAO design from MediaTek [E. Alshina, A. Alshin (Samsung)] [late]

### Sampling position

JCTVC-N0111 Sample position mapping with already defined scaling factor [V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0318 Cross check JCTVC-N0111 Sample position mapping with already defined scaling factor [Y. He (InterDigital)] [late] [miss]

JCTVC-N0248 Support of Field Coding for Signalling of Chroma Phase for Upsampling [K. Sato (Sony)]

JCTVC-N0283 Aspect ratio scalability based on SHM-2.0 [Y. Liu, J. Ostermann (Leibniz Uni Hannover)] [miss]

JCTVC-N0334 Derivation of picture and slice level information for resampled interlayer reference picture [J. Chen, V. Seregin, X. Li, K. Rapaka, M. Karczewicz (Qualcomm)] [late] [miss]

### Up-/downsampling filters

JCTVC-N0055 On resampling process for outside-bounds samples [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCTVC-N0086 Independent tile upsampling for SHVC [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

JCTVC-N0144 AHG13: Slice based upsampling filter for improved error resiliency [K. Ugur, M. M. Hannuksela (Nokia)]

JCTVC-N0153 On adaptive re-sampling filter coefficients coding [E.Alshina, A.Alshin (Samsung)]

JCTVC-N0265 AHG13: Complexity reduction of the up-sampling process [K. Andersson, R. Sjöberg (Ericsson)]

JCTVC-N0280 AHG13: cross-check of JCTVC-N0265 on Complexity reduction of the up-sampling process [E. François (Canon)] [late]

JCTVC-N0273 On the selection of fixed filters for upsampling [K. Minoo, D. Baylon (??)]

### Residual prediction

JCTVC-N0106 Generalized residual prediction with motion vector clipping for SHVC [K. Kim, H. Jo, J. Ryu, D. Sim, S.-J. Oh (KWU)]

JCTVC-N0204 ILR enhancement with differential coding for SHVC reference index framework [Y. He, Y. Ye, X. Xiu (InterDigital)]

JCTVC-N0277 Cross-check of JCTVC-N0204 on ILR enhancement with differential coding for SHVC reference index framework [E. François (Canon)] [miss]

JCTVC-N0234 Low-Complexity Generated Inter-layer Reference for SHVC [X. Li, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0314 Cross-check of JCTVC-N0234 on Low-Complexity Generated Inter-layer Reference [?? (??)][late]

### Key picture concept and single-loop decoding

JCTVC-N0161 Using decoded pictures from higher layer as references in SHVC [K. Rapaka, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0352 Cross-check of JCTVC-N0161 from Qualcomm on using decoded pictures from higher layer as references in SHVC [J. Xu, C. Auyeung (Sony)] [late] [miss]

JCTVC-N0186 AHG16: Single-loop decoding of SNR scalability for refIdx based SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital)]

JCTVC-N0187 Investigation on using H-ILP picture for refIdx based SHVC [X. Xiu, Y. Yan, Y. He (InterDigital)]

JCTVC-N0202 AHG16: Key picture concept and single loop decoding [C. Feldmann, F. Jäger, M. Wien (RWTH Aachen University)]

JCTVC-N0297 AHG16: cross-verification of JCTVC-N0202 on Key picture concept and single loop decoding [X. Xiu (InterDigital)] [late]

JCTVC-N0203 AHG16: Cross-Check of JCTVC-N0186 Single-loop decoding of SNR scalability for refIdx based SHVC [C. Feldmann, M. Wien (RWTH Aachen University)] [miss]

### Color and bit-depth scalability

JCTVC-N0145 AHG20: Backwards compatible enhancement of chroma format [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

JCTVC-N0347 Crosscheck report of JCTVC-N0145 backwards compatible enhancement of chroma format [K. Misra, A. Segall (Sharp)] [late] [miss]

JCTVC-N0146 AHG14: On resampling & color gamut scalability [K. Ugur, A. Aminlou (Nokia)]

JCTVC-N0168 AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results [Philippe Bordes, Pierre Andrivon, Franck Hiron (Technicolor)]

JCTVC-N0274 AHG14: Cross-checking of JCTVC-N0168 from Technicolor on color gamut scalable video coding using 3D LUT [C. Auyeung (Sony)] [miss]

JCTVC-N0278 AHG14: cross-check of JCTVC-N0168 on Color Gamut Scalable Video Coding using 3D LUT: New Results [P. Onno (Canon)] [late]

JCTVC-N0218 AhG14: On bit-depth scalability support [E. Alshina, A. Alshin (Samsung)]

JCTVC-N0271 AHG14: Color gamut scalable video coding with piecewise linear predictions [C. Auyeung (Sony)]

JCTVC-N0339 Cross-check report for JCTVC-N0271 : AHG14: Color gamut scalable video coding with piecewise linear predictions [Seung-Hwan Kim, Andrew Segall (Sharp)] [late] [miss]

## HL syntax for range extensions and single-layer HEVC coding (4)

JCTVC-N0155 HLS: Thumbnail Support in HEVC [C. Wang, C. Wang, W. Zhang, Y. Chiu (Intel)]

JCTVC-N0269 HLS: Non-significant slice segments with tiles for single layer HEVC extensions [C. Auyeung (Sony)]

JCTVC-N0063 REXT/MV-HEVC/SHVC HLS: auxiliary picture layers [M. M. Hannuksela (Nokia)]

JCTVC-N0077 AHG 5: On support for alpha channel in HEVC [M. Naccari, M. Mrak (BBC)]

## HL syntax in SHVC and 3D extensions (67)

### Generic HLS issues (2)

JCTVC-N0135 MV-HEVC/SHVC HLS: Extended maximum number of layers [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)] [late]

This is a follow-up proposal of JCTVC-M0164. To support having more than 64 layers, proposes to use three of the reserved bits in slice headers for extra bits of nuh_layer_id. Approximately 500 layers can be represented with extra 3 bits of nuh_layer_id.

Primarily motivated by "super multiview" application for displays with many views.

Question: Would such an application use one layer per view?

Suggestion to put additional bits into slice header, further investigation necessary whether this is a good place. The current proposal suggests 15 additional bits for layer_id, which according to some experts' opinion might be excessive.

It was remarked that the proposed syntax requires parsing the PPS before being able to access the bits of the extended layer ID, which may be undesirable.

It was noted that we have substantial syntax freedom for non-base layers, although we are constrained by compatibility for layer ID zero.

Near-term profiles likely will not need to support many layers, but it is important to provide extensibility in terms of the number of layers. Some way of allowing extension is highly desirable. Compatibility with existing decoders would be desirable, e.g. for decoding a subset of views.

Another possibility would be to assign one value of layer_id as “reserved for extension”.

Further offline discussion about best way to achieve this

Not clear whether there is any need for immediate action w.r.t. the MV-HEVC draft – likely not.

It was remarked that JCT3V-E0092, JCT3V-E0223 and JCT3V-E0224 are related (not submitted as JCT-VC contributions), where it is suggested to put additional bits into parameter sets. These proposals will also be registered as JCT-VC docs, and were discussed in the context of generic HLS issues.

Presentation not uploaded.

Revisit after considering related contributions.

JCT3V-E0092

This contribution proposes an extension mechanism for layer identifiers to increase the number of supported layers in MV-HEVC and 3D-HEVC. The range of nuh_layer_id is extended by an additional syntax element within the NAL units. The concept of so-called layer clusters allows using the existing extraction processes to select groups of related layers as proposed during the 4th meeting. The syntax has been modified slightly to ensure a backward compatible base layer and to be aligned with MV-HEVC Draft 4.

Again focused on "super multiview" with many views.

Proposes to signal, in VPS, a number of extra bits of layer ID.

It was commented that just having reserved fields might be adequate from a syntax perspective.

It was commented that some form of syntax involving "if( layer_id != 0)" branch in the syntax could be used.

It was commented that using layer ID equal to 63 as an escape code indication could be an alternative way to deal with the layer ID range.

Having an extended NUH for use with some particular profile was also discussed.

It was remarked that having a view-subset decoding capability for a lower-capability decoder is desirable. The proponent suggested that having a "clustering" of views to indicate which subset to decode is also desirable.

It was generaly agreed that support for some extensibility to more views with subset capabilities would be desirable, but we don't want to burden "mainstream" decoders with significant extra work to accomplish that, and we don't want the standard to contain purely "hypothetical syntax" that is normatively forbidden to be used.

Revisit after off-line thinking.

JCTVC-N0244 / JCT3V-E0075 MV-HEVC/SHVC HLS: Cross-layer POC alignment [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

In this proposal, a mechanism is proposed in order to ensure that the POC values all pictures of each access unit are the same even when it is allowed that access units for some pictures are IRAP pictures with NoRaslOutputFlag equal to 1 while others are not. Draft text was provided.

The contribution proposes a "poc_reset_flag" syntax flag.

When set to 1, the flag changes the POC of the previously-decoded pictures of the same layer in the DPB, by subtracting an offset from their POC values. It was remarked that this has loss resilience implications (when the picture which causes the POC reset is lost), and further study of that aspect was encouraged (e.g. to add some SEI message to improve the detection and handling of lost pictures).

Base layer decoder would work different from a single-layer (version 1) decoder (which is likely not critical).

Handling of long term pictures? Is claimed to be solved.

Revisit after further offline investigation.

JCTVC-N0355 3D/MV-HEVC HLS: Extending the supported number of layers [K. Suehring, G. Tech, R. Skupin, T. Schierl (FhG HHI)] [late]

JCTVC-N0356 3D/MV-HEVC HLS: Dependency signaling for extending the supported number of layers [G. Tech, K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)] [late]

JCTVC-N0357 3D/MV-HEVC HLS: Flexible layer clustering for extending the supported number of layers [G. Tech, K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (HHI)] [late]

JCTVC-N0267 / JCT3V-E0087 MV-HEVC/SHVC HLS: On changing of the highest layer ID across AUs and multi-mode bitstream extraction [Y.-K. Wang, Y. Chen (Qualcomm)]

This document discusses allowing changing of the highest value of nuh_layer_id across AUs within a CVS (which is currently allowed, due to an adoption intended to allow ARC), and proposes a multi-mode bitstream extraction process. The contribution raises a number of issues relating to this topic.

Related to N0110.

Question: Does “higher layer” usually mean equal or higher resolution? Currently, yes. However, in the case of using the “single_layer_for_non_irap_flag” it might also make sense to allow prediction of lower resolution from higher resolution, which would require defining decimation filters for prediction.

Revisit.

    - Random access, layer switching structures and cross-layer alignment of pictures types (8)

JCTVC-N0065 MV-HEVC/SHVC HLS: On IDR picture constraints [M. M. Hannuksela (Nokia)]

JCTVC-N0066 MV-HEVC/SHVC HLS: Layer-wise startup of the decoding process [M. M. Hannuksela (Nokia)]

JCTVC-N0084 MV-HEVC/SHVC HLS: On various cross-layer alignments [Y.-K. Wang, A. K. Ramasubramonian, J. Chen, Hendry (Qualcomm)]

JCTVC-N0090 MV-HEVC/SHVC HLS: Cross-layer non-alignment of IRAP pictures [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen, K. Rapaka (Qualcomm)]

JCTVC-N0121 MV-HEVC/SHVC HLS: Random access of multiple layers [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-N0124 MV-HEVC/SHVC HLS: Random layer access [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-N0130 MV-HEVC/SHVC HLS: On temporal sub-layer management [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Items 1 to 4 of this contribution seem relevant to this agenda category.

JCTVC-N0147 MV-HEVC/SHVC HLS: On restriction and indication of cross-layer IRAP picture distribution [J. Chen, Y.-K. Wang, K. Rapaka, A.-K. Ramasubramonian, Hendry (Qualcomm)]

Item 4 of JCVC-N0195 is relates to this agenda category: A restriction on the alignment of IDR and BLA pictures within the same access unit is proposed to be relaxed.

### Parameter sets (13)

#### General (4)

JCTVC-N0085 MV-HEVC/SHVC HLS: On parameter sets [Y.-K. Wang, Y. Chen, K. Rapaka (Qualcomm)]

JCTVC-N0129 MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

Items 1 and 2 are related to this agenda category:

1.	The semantics modification of single_layer_for_non_irap_flag to generalize the functionality. In the proposed text, more than two layers are allowed in IRAP access units.

2.	The syntax elements max_tid_il_ref_pics_plus1[ i ] and default_one_target_output_layer_flag are optionally signalled according to the value of single_layer_for_non_irap_flag.

Items 5 to 6 of JCTVC-N0130 seem relevant to this agenda category.

JCTVC-N0157 MV-HEVC/SHVC HLS: On signalling of sps_max_sub_layers_minus1 [J.W. Kang, H. Lee, J. Lee, J.S. Choi (ETRI)]

JCTVC-N0165 On VPS extension [Y. Cho, B. Choi, M. Park, J. Y. Lee, H. Wey, C. Kim (Samsung)]

Items 3 and 5 of JCTVC-N0195 are related to this agenda category:

3. A bitstream constraint related to values for syntax elements direct_dependency_flag[i][j] and max_one_active_ref_layer_flag is proposed.

5. A bitstream constraint related to the values of syntax elements splitting_flag and dimension_id_len_minus1[i] is proposed.

#### Signalling of representation format (3)

JCTVC-N0092 / JCT3V-E0060 MV-HEVC/SHVC HLS: Representation format information in VPS [A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), J. Boyce (Vidyo)]

JCTVC-N0238 On Source Representation Information Signaling in VPS [S. Deshpande (Sharp)]

JCTVC-N0264 HLS: MV-HEVC/SHVC HLS: VPS extension for multi-format [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung) (Samsung)] [late]

#### Efficient parameter set parameters signalling (3)

JCTVC-N0162 MV-HEVC/SHVC HLS: Inter-layer scaling list inheritance for HEVC extensions [Martin Pettersson, Thomas Rusert]

JCTVC-N0200 On Scaling List Data Signaling [S. Deshpande (Sharp), S. Liu (MediaTek), S. Lei (MediaTek), K. Sato (Sony)]

JCTVC-N0212 SHVC HLS: On Inter Layer Parameter Set [Y. He, Y. Ye, Y. He (InterDigital)]

#### ViewId signalling (3)

JCTVC-N0051 MV-HEVC/SHVC HLS: ViewId and view position index [J. Boyce (Vidyo)]

JCTVC-N0067 MV-HEVC/SHVC HLS: on associating ViewId with nuh_layer_id and camera position [M. M. Hannuksela (Nokia), L. Chen (USTC)]

JCTVC-N0299 MV-HEVC/SHVC HLS: On use of splitting_flag with flexible coding order [Andrey Norkin, Thomas Rusert (Ericsson)] [late]

Item 3 of JCTVC-N0085 is related to this agenda category.

### Signalling for inter-layer dependency and inter-layer prediction reference (27)

#### Sequence-level inter-layer dependency signalling (2)

JCTVC-N0058 MV-HEVC/SHVC HLS: On dependency type [T. Ikai, T. Uchiumi (Sharp)]

The contribution presents a direct_dependency_type representation which is assertively more effective for many cases. In the proposed representation, the case in which direct_dependency_type is equal to 0, represent sample and motion dependency.  With the proposed change, direct_dependency_type can be exempted by setting direct_dep_type_len equal to 0 for the case sample and motion dependency is used for all layers.

The proposal would add another type “no dependency” with dependency_id=3, and shift the “motion+sample” from id=3 to the new id=0.

The benefit in terms of bit rate saving would be minor in current test conditions, but it is claimed that the benefit might be higher with more layers.

One expert mentions that “no dependency” can already be signalled differently in the current spec.

No action.

JCTVC-N0132 MV-HEVC/SHVC HLS: On interlayer prediction type [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

In order to enable an independent configuration of each inter-layer dependency type and a bit-efficient signalling, a bit-mask prediction_type_mask[ i ] and direct_prediction_type_flag[ i ][ j ][ k ] are signalled in VPS extension similar to scalability_mask[ i ]. In addition, a proposed flag motion_only_decoding_flag indicates that only the motion vector related data are needed for inter-layer prediction without the full decoding of pixel data. It removes unnecessary decoding process of unused decoded pixel data.

The proposal would save bit rate in case of sequences where inter-layer motion prediction is not used at all (not in current CTC)

Further, a flag is proposed at slice level to indicate that only motion dependency is used. This is intended for a case that only motion information from the base layer is used for the inter-layer prediction. No inter-picture prediction would be performed for this case (i.e. also no TMVP, which is not fully clear from the semantics description in the contribution). One expert suggests that this flag is rather a kind of indication metadata that could be put into an SEI message (does not have impact on normative decoding process).

Adds more flexibility, but benefit not obvious, and the suggested mask makes the parsing slightly more complicated.

No action on prediction_type_mask; further study on the motion_only_decoding_flag as SEI message. Further study whether this could have possible impact on saving DPB memory.

#### Sub-layer related inter-layer prediction signalling (4)

JCTVC-N0060 MV-HEVC/SHVC HLS: TemporalID alignment and inter-layer prediction restriction [T. Ikai, Y. Yamamoto (Sharp)]

The contribution proposes to introduce a flag inter_layer_tid_alignment_flag in VPS to indicate if TemporalID is aligned across layers.  It is also proposed that if the inter_layer_tid_alignment_flag is 1 and max_tid_il_ref_pics_plus1[0] is less than 7, max_tid_il_ref_pics_plus1[0] is used for common value of max_tid_il_ref_pics_plus1 across layers and inter-layer related signaling in slice segmentation header is not sent if the TemporalID of a layer is larger than the common max value.

Additionally in case the above proposal (Option1) is not agreed, it is proposed as Option2 to include a syntax general max_tid_il_ref_pics_plus1 for common value of max_tid_il_ref_pics_plus1

Option 1: In case of base layer restriction on tid, inherit same restriction over all layers (saves the sending of the max_tid flag, and some other syntax in slice header for dependent layers)

Question: What is the intention of this?

It would restrict the encoder flexibility; bit rate saving likely not large.

The semantics for deriving the parameters in the contribution seems to be incomplete.

Option 2: Max_tid is optionally inherited to the enhancement layers, syntax in slice header is inherited similar as in option 1.

No action.

JCTVC-N0120 MV-HEVC/SHVC HLS: Signaling for Inter-layer prediction indication [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

In the previous meeting, the method that controlling the use of inter-layer prediction based on the temporal sub-layer at sequence level was adopted in SHVC / MV-HEVC draft text. This contribution propose a present flag indicating whether the use of inter-layer prediction is controlled based on temporal sub-layer or not.

Two alternatives: Common signalling of max_tid_..._present_flag for all layers, or individually for each layer

Main intent is bit rate saving (3 bits per layer at sequence level).

The decoder operation is becoming slightly more complex, as it needs to be checked whether the flag is present or not, and whether the information is inferred or to be parsed (this also applies to N0060 option 2)

Several experts expressed support for JCTVC-N0120 “alternative 1”, common signalling of the “present flag” for all layers, and max_tid=7 is assigned for all layers. However, the semantics part of the text in the contribution seems to require more investigation – new version to be uploaded, revisit.

JCTVC-N0109 MV-HEVC/SHVC HLS: Signalling for sub-layer dependency [V. Seregin, Y.-K. Wang (Qualcomm)]

This contribution proposes to further classify the sequence-level inter-layer dependency, which is currently derived from the syntax element direct_dependency_flag[ i ][ j ], to be sub-layer specific, by utilizing information carried by the syntax element max_sublayer_for_ilp_plus1[ i ]. Specifically, the value of NumDirectRefLayers that is currently defined for each layer is defined for each sub-layer of each layer, thus becoming a two-dimension array. The sub-layer classified variable is then applied in reference picture marking and picture-level inter-layer reference picture signalling, in order to mark certain pictures as "unused for reference" earlier and release the picture buffer for storing other decoded pictures or to avoid sending of unnecessary bits in slice headers for signaling of pictures used for inter-layer prediction by each picture.

Main intentions: Earlier identification of pictures as “unused for reference” (by changing the derivation process, also increasing the storage for NumDirectRefLayers); saving of bit rate.

Max additional memory would be 64x6 bytes at sequence level.

No clear evidence how large the benefits would be (e.g. a case of dependency conditions where the earlier identification of “unused for reference” would save DPB memory, reduction of bits). Perform offline analysis, update input contribution, revisit.

JCTVC-N0196 On Sub-layer Non-reference Pictures Indication for Inter-layer Prediction [S. Deshpande (Sharp)]

This document proposes syntax and semantics in VPS for indicating that sub-layer non-reference pictures belonging to a layer are not used for inter-layer prediction. A bitstream conformance constraint is proposed based on this indication which gets used in the decoding process for inter-layer reference picture set. Additionally a change to the marking process for sub-layer non-reference pictures not needed for inter-layer prediction is proposed.

In r1 revision some changes to the proposed specification text are made with no change to the proposed design.

Goal similar with N0109 – DPB memory saving. N0196 further allows earlier identification of sub-layer non-reference pictures not used for inter-layer reference. Explicit signalling is used. As a possible intention, this could also be used as a bit-stream restriction to limit worst-case decoder complexity.

Variant 1 only performs signalling for highest tid, variant 2 performs signalling for each tid up to maximum.

One expert mentions that a similar benefit could already be achieved by encoders appropriately using temporal scalability (unless the number of 7 sub-layers would not be sufficient).

Powerpoint presentation slide with an example is shown which shall be uploaded. This example however only shows a case where the benefit can be drawn from temporal scalability. Provide more examples where it becomes evident that the additional syntax is necessary. Revisit.

#### General inter-layer RPS signalling and derivation (7) (TBP)

The following item of the JCTVC-N0057 is relevant for the agenda item: RPS includes only pictures whose direct_dependency_type shows sample prediction dependency.

JCTVC-N0059 MV-HEVC/SHVC HLS: On slice segment header extension [T. Ikai, T. Uchiumi (Sharp)]

The aspect for slice-based inter-layer prediction signalling is relevant for this agenda item.

JCTVC-N0081 MV-HEVC/SHVC HLS: On inter-layer prediction related syntax [J. Xu, A. Tabatabai, O. Nakagami, T. Suzuki (Sony)]

The following aspect of JCTVC-N0107 is related to this agenda item: This contribution proposes removal of the slice header syntax element inter_layer_sample_pred_only_flag.

JCTVC-N0118 MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling [J. Chen, Y. Chen, Hendry, Y.-K. Wang, K. Rapaka (Qualcomm)]

The following aspect (item 3) of JCTVC-N0129 is related to this agenda item: Signalling of inter-RPS syntax can be skipped when single_layer_for_non_irap_flag is equal to 1 and the current picture is not an IRAP picture.

JCTVC-N0154 MV-HEVC/SHVC HLS: On signalling of inter-layer RPS in slice segment header [J.W. Kang, H. Lee, J. Lee, J.S. Choi (ETRI)]

JCTVC-N0195 Comments On SHVC and MV-HEVC [S. Deshpande (Sharp)]

Items 1 and 2 are related:

- In slice segment header signalling if NumActiveRefLayerPics is equal to NumDirectRefLayers[nuh_layer_id] then the inter_layer_pred_idc[i] syntax elements are not signalled as they can be inferred.

- A gating flag is proposed for signalling syntax elements related to inter-layer prediction in slice segment header.

JCTVC-N0217 MV-HEVC/SHVC HLS: On SHVC High Level Syntax [Y. He, X. Xiu, Y. Ye, Y. He (Interdigital)]

JCTVC-N0131 MV-HEVC/SHVC HLS: On interlayer reference picture set [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

#### Signalling of TMVP and collocated pictures (8)

JCTVC-N0057 MV-HEVC/SHVC HLS: On inter-layer picture selection in RPS and colPic [T. Ikai, T. Uchiumi (Sharp)]

In the current spec, any inter-view prediction pictures are included in RPS irrelevant to dependency type and the RPS includes motion only dependent pictures. Because motion only dependent pictures are not used as the reference samples, the ref_idx which used to indicate the picture of the reference sample is not efficient due to the non-used picture existence in the reference picture list.  Alternative colPic indication syntax for temporal motion vector derivation by layer_id has been introduced to support the motion only dependent picture which might be not included in the reference picture list, so there is inefficiency due to inclusion of motion only dependent picture in RPS. However this issue has not yet been addressed.

Similarly, any inter-view prediction pictures with motion dependency are included as candidates for the temporal picture indicated by the alternative colPic indication. That means the colPic can be selected by either the conventional syntax based on ref_idx or the alternative syntax based on layer_id.  Because of this redundancy, the layer_id base indication is inefficient. This proposal proposes that 1) RPS doesn’t include motion only dependent picture and 2) alternative colPic indication is only used in the case that the colPic is motion only dependent picture (i.e. not included in RPS).

The intention is to save memory for the samples of pictures where only motion-related inter-layer dependency is active. However, the current version of the contribution does not fully specify the handling of RPS. Also not clearly specified that TMVP would not be used.

It is also mentioned that the current design of putting all pictures into RPS has been made out of several reasons, including error resilience in case of losses.

General impact on consistency of the spec not fully clear.

Further study on first part (RPS not to include reference pictures that are only used for motion prediction)

TBP: Second part to be presented in the context of JCTVC-N0059.

Some aspects of JCTVC-N0059 are relevant for this agenda item (on collocated pictures).

JCTVC-N0064 MV-HEVC/SHVC HLS: on storage of motion fields [M. M. Hannuksela (Nokia)]

The proposal consists of two parts:

- collocated_picture_constraint_flag in the SPS extension indicating, when equal to 1, that no TMVP is used for pictures within the same layer.

- An informative note describing when the storage of a motion field is required and when it becomes no longer needed.

Basic idea is using a new flags to indicate that TMVP is not used for subsequent pictures within the same layer. This allows a decoder to infer whether storage of MV is necessary or not (current version 1 TMVP disable flag does not care about inter-frame or inter-layer usage).

No impact on normative decoding process. Could also be defined as an SEI message.

(Note: The current SPS flag disabling TMVP in version 1 has impact on decoding, e.g. MV prediction).

The current spec does not normatively specify the usage of memory for the MV data. Therefore, a better place for this would be an SEI message – revisit.

JCTVC-N0112 MV-HEVC/SHVC HLS: High-level syntax for temporal motion vector prediction [Hiroya Nakamura, Motoharu Ueda, Hideki Takehara, Shigeru Fukushima (JVC Kenwood)]

This contribution proposes to add a syntax element(s) to distinguish the following conditions for temporal motion vector prediction in sequence level along with alt_collocated_indication_flag in slice level.

- no temporal motion vector prediction

- temporal motion vector prediction from reference layer or the same layer, and signalling in slice level

- temporal motion vector prediction from only reference layer

- temporal motion vector prediction from only the same layer

Powerpoint presentation to be uploaded.

The suggested change at sequence level is similar to N0064, but can additionally differentiate the cases whether TMVP is used only over the temporal sequence of the same layer, or both (which can however anyway be inferred from NumActiveMotionPredRefLayers syntax element in case of N0064); generally N0064 as SEI message would be preferred.

Another suggestion in N0112 is a change at slice level, signalling the alt_collocated_xx elements only for case b), whereas currently it is for cases b) and c) (in cases a and c, it is suggested to be inferred). This may only have minor impact on bit rate savings. It is also mentioned that during the last meeting a discussion (on JCTVC-M0457) was performed whether those syntax elements in slice header might imply a low-level change, since the elements would be used at PU level. See also below on N0107 etc.

JCTVC-N0102 MV-HEVC/SHVC HLS: On alternative collocated picture [Y. Lin (HiSilicon), J. Zan (Huawei)]

This contribution discusses issue on using alternative collocated picture which was adopted in last meeting. TMVP derivation process in HEVC specification exploits not only a collocated picture but also a reference picture list indicator (i.e. collocated_from_l0_flag) which is used in collocated MV derivation. However the adopted method of using alternative collocated picture only signals the collocated picture, but not the reference picture list indicator. Therefore it seems not work well. Two solutions on the issue are proposed. The first solution proposes to avoid usage of the reference picture list indicator in collocated MV derivation by changing the corresponding condition checking. In this way the collocated MV can be derived regardless of the reference picture list indicator when alt_collocated_indication_flag is enabled. The second solution is to additionally signal the reference picture list indictor for the alternative collocated picture.

It is agreed that the current solution around alt_collocated_indication_flag has a problem that when the low-delay condition is not true there is no means to choose between list 0 and list 1, whereas the collocated_ref_layer_idx is always put into list 0.

The first suggested solution is a potential low-level change. Beyond the current WD, which changes 8.5.3.2.7, it is further suggested to change 8.5.3.2.8 (adding a condition about alt_collocated_indication_flag in the derivation process of collocated MV at PU level).

The second suggested solution (adding collocated_from_l0_flag for B slices in case of inter-layer) is asserted to solve the problem.

JCTVC-N0107 MV-HEVC/SHVC HLS: On collocated picture indication and inter_layer_sample_pred_only_flag [V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm)]

This contribution proposes removal of the slice header syntax elements alt_collocated_indication_flag, collocated_ref_layer_idx, and inter_layer_sample_pred_only_flag, and the corresponding decoding processes. The first two syntax elements involve low-level decoding process changes, including the temporal motion vector prediction, which are not allowed for MV-HEVC and SHVC. The last syntax element is used to enable avoiding inclusion of intra-layer pictures into reference picture lists, which is already enabled in RPS signalling by setting those pictures to be not used for reference by the current picture.

About alt_collocated_indication_flag, collocated_ref_layer_idx:

The dispute on whether the inclusion of the syntax elements in slice header would imply a low-level change or not is still open.

One expert mentions that using the existing collocated_ref_idx (as suggested in N0107) instead of collocated_ref_layer_idx would not have an implication on the length of the ref pic list, unless any change would be made to exclude pictures which are used only for motion prediction from the ref pic list.

About inter_layer_sample_pred_only_flag: It is suggested to achieve the same functionality by setting used_by_curr_pic_flag, used_by_curr_pic_s0_flag, and used_by_curr_pic_s1_flag equal to 0 for each entry of a reference picture set in reference picture set signalling. This is asserted to be correct, and suggested to be embraced by several experts. Revisit in the context of N0081, which suggests something similar.

JCTVC-N0119 MV-HEVC/SHVC HLS: On collocated picture indication [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

In SHVC/MV-HEVC draft text, reference layer picture can be used as a collocated picture for temporal motion vector prediction. For this, there are two syntax elements ‘alt_collocated_indication_flag’ and ‘collocated_ref_layer_idx’ in the slice segment header. However, when ‘alt_collocated_indication_flag’ is equal to 1 and current slice is B slice, since current draft text does not specify the syntax element ‘collocated_from_l0_flag’, it is not possbile to know whether a collocated picture is derived from reference picture list 0 or reference picture list 1. Also the motion vector of collocated prediction block cannot be derived without low-level changes. This contribution proposes two alternatives to solve these problems.

Powerpoint presentation to be uploaded.

First solution similar to N0102 (adding collocated_from_l0_flag for B slices in case of inter-layer references), but puts this syntax element prior to the alt_collocated_indication_flag; second solution infers the collocated_from_l0_flag.Proponents themselves indicate that first solution would be more consistent.

JCTVC-N0185 On low-delay flag checking process of SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital), Y. Lin, X. Zheng, X. Chen (HiSilicon)]

In this contribution, the low-delay checking process in SHVC Test Model (SHM2.0) is modified to reportedly improve the efficiency of temporal motion vector prediction (TMVP) for enhancement layer (EL) coding. More specifically, the low-delay flag is set to true if the inter-layer prediction (ILP) picture is used as the co-located picture for EL TMVP derivation, such that the motion vector (MV) of the co-located prediction unit (PU) always comes from the same reference picture list of the target MV of the current PU for better TMVP prediction. The proposed modification is a slice-level change as the low-delay flag is determined per slice and referred to by all the PUs of a coded picture. Experimental results show that the proposed change reportedly achieves 0.4%, 0.4% and 0.5% BD-rate savings on average for 2x, 1.5x and SNR scalability in RA configuration, compared to the anchors of SHM2.0.

Powerpoint presentation to be uploaded.

Suggestion to use the LD flag to differentiate between l0 and l1 candidate in case of inter-layer reference. A similar approach was suggested in JCTVC-M0065, which had been commented as follows:

“The low-delay flag is determined at the block level, therefore it would not be applicable to the refidx approach. However, whereas HEVC spec defines it this way, the conditions would not change for all blocks of the slice. It may be implementation specific, whether this can be asserted as a low-level change or not. Further evidence should be provided.”

Revisit: Further discussion necessary on meaning of “low-level change” in the overall context of the alt_collocated_indication_flag, affecting potential decisions on N0185, N0119, N0112, N0102. Alternatively, the solution suggested in N0107 could solve the problem in a reasonable way.

JCTVC-N0260 Cross check report of JCTVC-N0185 on low-delay flag checking process of SHVC [K. Misra, A. Segall (Sharp)] [late]

#### Reference picture list construction (4) (TBP)

JCTVC-N0082 MV-HEVC/SHVC HLS: On initialization process of reference picture lists for HEVC extensions [O. Nakagami, T. Suzuki (Sony)]

JCTVC-N0095 MV-HEVC/SHVC HLS: Inter-layer reference pictures in reference picture list initialization [A. K. Ramasubramonian, Y. Chen, L. Zhang (Qualcomm)]

JCTVC-N0216 MV-HEVC/SHVC HLS: On Reference Picture List Modification [Y He, X. Xiu, Y Ye (InterDigital)]

JCTVC-N0316 MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction [Andrey Norkin, Usman Hakeem (Ericsson)] [late]

#### Management of resampled or filtered inter-layer reference pictures (2) (TBP)

JCTVC-N0128 MV-HEVC/SHVC HLS: Reference picture marking and picture removal [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-N0282 MV-HEVC/SHVC HLS: On handling of filtered inter-layer reference [P. Lai, S. Liu, S. Lei (MediaTek)]

### Tiles and parallel processing (4)

JCTVC-N0158 MV-HEVC/SHVC HLS: Bitstream restrictions on tiles and wavefronts across layers [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian, J. Chen (Qualcomm)]

JCTVC-N0159 MV-HEVC/SHVC HLS: Parallel Processing Indications for Tiles in HEVC Extensions [K. Rapaka, X. Li, J. Chen, W. Pu, Y.-K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-N0160 MV-HEVC/SHVC HLS: On signaling of offset delay parameters and tile alignment [K. Rapaka, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

JCTVC-N0199 On Tile Alignment [S. Deshpande (Sharp), K. Misra (Sharp)]

JCTVC-N0069 and JCTVC-N0087 (in SEI category) are also related.

### Hypothetical reference decoder (HRD) and DPB management (13)

#### General principles of DPB operation (5)

JCTVC-N0048 Extensions to support layer addition and removal, access unit structure and changes to HRD model in scalable HEVC [S. Narasimhan, A.Luthra (Arris)]

JCTVC-N0049 Consideration of buffer management issues and layer management in HEVC scalability [S. Narasimhan, A. Luthra (Arris), K.Sato (Sony Corp), A. Tabatabai (Sony Electronics Inc)]

JCTVC-N0093 MV-HEVC/SHVC HLS On DPB operations [A. K. Ramasubramonian, Y. Chen, Y.-K. Wang (Qualcomm)]

JCTVC-N0172 MV-HEVC/SHVC HLS: Layer-wise DPB operation and size indications [M. M. Hannuksela (Nokia)]

JCTVC-N0198 On DPB Operation [S. Deshpande (Sharp)]

#### DPB parameter signalling (4)

JCTVC-N0056 MV-HEVC/SHVC HLS: On inter-layer reference picture output marking [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp)]

JCTVC-N0062 MV-HEVC/SHVC HLS: access unit boundary detection [M. M. Hannuksela (Nokia)]

JCTVC-N0091 MV-HEVC/SHVC HLS: DPB-related parameters in SPS and VPS [A. K. Ramasubramonian, Y.-K. Wang (Qualcomm)]

JCTVC-N0127 MV-HEVC/SHVC HLS: On decoded picture buffer [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-N0157 MV-HEVC/SHVC HLS: On signalling of sps_max_sub_layers_minus1 [J.W. Kang, H. Lee, J. Lee, J.S. Choi (ETRI)]

Some aspects of JCTVC-N0172 relate to this agenda category.

JCTVC-N0197 On Signaling DPB Parameters in VPS [S. Deshpande (Sharp)]

      - Other HRD related aspects (4)

JCTVC-N0062 MV-HEVC/SHVC HLS: access unit boundary detection [M. M. Hannuksela (Nokia)]

JCTVC-N0110 SHVC HLS: Earlier DPB clearing for adaptive resolution change [V. Seregin, Y. Chen, Y.-K. Wang (Qualcomm)]

The following aspect of JCTVC-N0128 is related to this agenda item: A picture removal process from a decoded picture buffer (DPB) is proposed. When the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied. The purpose of the second item is to avoid the removal of inter-layer reference pictures before inter-layer prediction.

JCTVC-N0290 Ultra-low delay with SHVC, MV-HEVC and 3D-HEVC [R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (HHI)]

## HL syntax in SHVC (10)

### Generic HLS issues (6)

(Thu 25th p.m.)

JCTVC-N0108 SHVC HLS: Avoid resampling process for pictures used only for inter-layer motion prediction [V. Seregin, Y.-K. Wang, Y. Chen (Qualcomm)]

In SHVC, the picture and the motion field of an inter-layer reference pictures may both need to be resampled for the prediction of a current picture. However, it is not always necessary to invoke them jointly since there might be cases when that it is not required to add the resampled picture from the reference layer as a reference picture of the current picture. This contribution proposes skipping the invocation of the resampling process for the inter-layer reference pictures used only for inter-layer motion prediction.

The change is basically editorial, except for the fact that we limit complexity with a constraint on how much processing is allowed to be invoked.

The idea of having motion resampling separated from texture resampling was adopted at the previous meeting. It was suggested that the idea of this was to allow sample prediction without motion prediction or vice-versa, but perhaps not to allow sample prediction from a different picture than motion prediction. It was questioned whether there is actual value in using motion prediction without sample prediction.

Revisit to determine the rational for the underlying intent.

JCTVC-N0213 SHVC HLS: On Resampling Buffer Indication [Y. He, Y. Ye (InterDigital)]

[Add abstract.]

The proponent noted that for SNR scalability it is not necessary for the decoder to allocate a resampling buffer, and proposed to add a flag to indicate such a case.

It was remarked that the decoder can directly observe the picture size and offset values to determine what to do without needing a flag. The flag would enable determining the resampling requirement at the VPS level rather than the SPS level; however, buffer allocation requires using information at the SPS level already.

It was remarked that some of the relevant parameters are proposed to be moved up from the SPS to the VPS in N0092. N0215 is also noted to be related.

Plan no action, pending whether a need is identified for this indication after consideration of other contributions.

JCTVC-N0215 SHVC HLS: On SNR Scalability Indication [Y. He, Y. Ye (InterDigital)]

This contribution proposes to signal SNR scalability for the needs of re-sampling process, inter-layer filters and certain applications. It is asserted that it would be beneficial to differentiate SNR scalability from spatial scalability in high level syntax so that the encoder and decoder operations can be configured and/or initialized according to the relevant coding tools to be supported.

It was remarked that the decoder can directly observe the picture size and offset values to determine whether the scalability is SNR scalability without needing a flag.

It has not yet been determined whether there are different coding tools for SNR and spatial scalability.

It was remarked that we basically need to first determine of these more fundamental questions before worrying about this indication.

Plan no action, pending whether a need is identified for this indication after consideration of other contributions.

JCTVC-N0209 SHVC HLS: SHVC Skipped Picture Indication [J. Boyce (Vidyo), X. Xiu, Y He, Y Ye (InterDigital)]

It is proposed to add a skip_picture_flag syntax element in the SHVC slice segment header to indicate if a picture is skipped, and to define a normative skipped picture decoding process.  The proposals are asserted to be similar to SVC's slice_skip_flag and associated decoding process, but apply to an entire picture rather than a single slice.

The contribution proposed not performing deblocking for skipped pictures. It was remarked that the motion compensation process will generate blocking artefacts, so deblocking should be performed (or explicitly indicated whether it is to be performed or not).

It was remarked that it may be desirable to establish slice boundaries even in the case of skipping. In the prior SVC design, skipping was a slice-level operation rather than a picture-level operation, and slice boundaries are established.

It was suggested to check whether temporal motion prediction from the enhancement layer might perform better than using the base layer motion for a skipping operation.

It seemed agreed that having some kind of skipping capability is desirable. However, further study is needed to determine whether that should operate at the slice level or picture level (or perhaps tile level), and exactly how it should operate.

JCTVC-N0210 SHVC HLS: Adaptation Picture Set for SHVC [Y. He, J. Dong, Y. Ye (InterDigital)]

This contribution is a follow-up of JCTVC-M0179. It is proposed to use Adaptation Parameter Set (APS) to carry picture level parameter information to support scalable extension of HEVC. The syntax elements of inter-layer processing, such as inter-layer filtering coefficients, sample prediction and motion prediction syntax elements are proposed to be signaled in APS instead of slice header to save bits and keep the existing slice header syntax intact. The APS syntax, semantic and simulation results are provided in this contribution.

An example use is the inter_layer_information() syntax.

It was remarked that

- We should first determine that there is a need for the syntax that would benefit from being sent using an APS, rather than trying to decide to have an APS first than the figure out what to put in it.

- Some coding efficiency impact should be measured or calculated to determine the extent of the potential need.

Plan no action, pending whether a need is identified after consideration of other contributions.

JCTVC-N0296 Cross-check of JCTVC-N0210 on APS for SHVC [H. Yang (Huawei)] [late]

Cross-check based on SCE 3.4.1 chroma enhancement filter parameters.

### Signalling of cropped inter-layer reference (2)

JCTVC-N0054 Signalling and restriction for scaled reference layer offsets [T. Tsukuba, T. Yamamoto (Sharp)]

This contribution proposes a modified signalling and restriction regarding scaled reference layer offsets. On the modified signalling, a presence flag indicating whether to signal scaled reference layer offsets parameters is added, and a present flag for scaled reference layer offsets is added for each direct reference layer. On the restriction, a semantic restriction is proposed to restrict inter layer sample prediction outside or across the bounds specified by scaled reference layer offsets.

It is asserted that the signalling modification is beneficial to improve the coding efficiency and to simplify decoder implementation by removing additional boundary operation for scaled reference layer offsets.

The proposal suggests to remove num_scaled_ref_layer_offsets, as it is asserted that this syntax element is redundant with VPS information.

The contribution proposed the following changes to the syntax:

- Removing num_scaled_ref_layer_offsets.

- Adding scaled_ref_layer_offset_params_present_flag.

- Adding scaled_ref_layer_offset_present_flag for each direct reference layer.

Measurement data was not provided regarding the loss from disallowing prediction off the edge of the reference layer. The group was rather skeptical about the desirability of disallowing the extrapolation case.

It was remarked that removing num_scaled_ref_layer_offsets introduces an undesirable dependency of the parsing of SPS on VPS.

It was remarked that there is already a way to not send offsets for all layers.

No action.

JCTVC-N0089 MV-HEVC/SHVC HLS: On signalling of scaled reference offset [J. Chen, A.-K. Ramasubramonian, Y.-K. Wang, Y. Chen, X. Li (Qualcomm)]

This contribution proposes moving the scaled reference offsets syntax from SPS extension to VPS extension. In addition, similar to SVC, it is proposed for the scaled reference offsets to be further signalled in the slice header to have picture-level adaptation.

The motivation for this proposal depends somewhat on N0092 / E0060.

It was agreed that the scaled reference offsets should be at the same level of syntax as the picture size unless we want to allow picture-level adaptation of the offset values.

Regarding allowing picture-level adaptation of the offset values, this might add decoder complexity and it did not seem very likely that encoders would want to use that feature.

### Hybrid scalability (2)

JCTVC-N0050 / JCT3V-E0037 Specification text to support AVC base layer in HEVC layered extensions [J. Boyce (Vidyo), K. Kawamura (KDDI)]

This contribution proposes specification text to support an AVC base layer for SHVC, and proposes a new Hybrid Scalable Main profile. The proposed draft text changes provided are based upon the SHVC editor’s draft input contribution JCTVC-N0242, and fit within one additional page of specification text.  While specifically proposed for SHVC, the changes could also apply to MV-HEVC and other HEVC layered extensions.

v2 adds an author, and specifies that avc_base_layer_flag shall be equal to 1 in the proposed Hybrid Scalable Main profile.

It was noted that the N12956 MPEG requirements document has a 'should' regarding AVC base layer support.

Draft text was provided.

The proposal included syntax, encapsulation format, decoding process, and a draft profile.

For the prior drafted profiles, it was proposed to disallow using a non-HEVC base layer.

Do we want encapsulation?

Do we want MPEG-2? (This would need some working out of what is equivalent to a NAL unit and a POC.)

It was noted that the proposed encapsulation would be opaque to existing AVC decoders, such that an extraction process would be needed to pull out the AVC base layer for consumption by such a decoder.

If instead of specifying encapsulation within an HEVC elementary stream, we depend on the system to provide the base layer by external means, the base layer could be compatible with an existing non-HEVC decoder.

If we don't define the encapsulation, the scope of our profile would be only the enhancement layer capability.

The proponent indicated that since AVC is so structurally similar, it would be easier to specify how HEVC works with AVC than for other base layers.

It was remarked that it seems cleaner not to have cross-specification base layer normative referencing within the HEVC standard. That should be done in some other specification(s).

Revisit after further offline study.

JCTVC-N0211 SHVC HLS: On AVC Base Layer GOP Structure Indication [Y. He, Y. Ye, Y. He (InterDigital)]

It is proposed to add an identical GOP structure indicator for the AVC base scalability to indicate if the AVC base layer GOP structure is the same as the GOP structure of the enhancement layers. The benefit of such indicator is to inform the decoder if all the pictures associated with the same output time can be decoded consecutively or not, and further facilitate the decoding initialization and memory allocation.

It was agreed that our current plan is that the GOP structure should always be aligned, at least in the HEVC base layer case. This is implied by our current AU definition. This should be editorially clarified if necessary.

If we have an external specification the provides base layer pictures, perhaps that specification could determine its own rules. But this is our current plan within the scope of HEVC.

See also notes for N0050 and N0015.

## SEI messages (13)

### Motion and prediction constrained SEI messages (5)

JCTVC-N0069 Motion and inter-layer prediction constrained SEI message [K. Ugur, M. M. Hannuksela (Nokia)]

One potential application of SHVC is to enhance the quality of a region of interest (ROI) of the base layer picture with the enhancement layer. In such use, as only a certain region of the picture is enhanced, it would be useful to indicate the details of this operation to avoid redundant decoding of certain regions and minimize complexity. It is asserted that tiles, and more specifically motion-constrained tiles, can be used to support the ROI quality enhancement use case. This contribution proposes some modifications to the motion-constrained tile sets SEI message with the aim of supporting ROI enhancement.

Presentation to be uploaded.

Two scenarios:

- Enhancing an ROI, leaving some non-ROI area the same as the base layer (except perhaps for resampling).

- Enhancing an ROI without using BL texture, leaving some non-ROI area the same as the base layer. (A further constraint relative to the first scenario.)

The second scenario is related to N0117 and N0236.

Proposes to:

- Indicate "skipped tile sets" where the EL area is identical to the (possibly upsamplesd) BL.

- Indicate no-inter-layer-prediction tile sets.

by adding additional syntax elements for each tile set rectangle.

In discussion, it was asked what is the purpose of the layer ID syntax element and its relationship with scalable nesting and the layer ID in the NUH.

It was remarked that N0159 is also related.

Revisit after consideration of all potentially-related contributions.

JCTVC-N0087 Inter-layer constrained tile sets SEI message [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

JCTVC-N0088 Extension of (temporal) motion constrained tile sets SEI message [K. Suehring, R. Skupin, Y. Sanchez, T. Schierl (FhG HHI)]

JCTVC-N0117 HLS: Extensions to Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami, T. Suzuki (Sony)]

JCTVC-N0236 HLS: Extensions of motion-constrained tile sets SEI message for interactivity [C. Auyeung, J. Xu (Sony)]

### Layer presence and dependency change SEI messages (2)

JCTVC-N0173 MV-HEVC/SHVC HLS: On Layers Present SEI message [J. W. Kang (ETRI), J. Lee, H. Lee, J. S. Choi, T. C. Thang (UoA)]

JCTVC-N0174 MV-HEVC/SHVC HLS: On Layer Dependency Change SEI message [T. C. Thang (UoA), J. W. Kang, J. Lee, H. Lee, J. S. Choi (ETRI)]

### Frame packing SEI messages (3)

JCTVC-N0133 HLS: Frame packing arragement SEI message for bit depth extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)] [late]

JCTVC-N0134 HLS: Frame packing arragement SEI message for full resolution 3D [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)] [late]

JCTVC-N0270 Updated proposal with software for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [Y. Zhang, Y. Wu, S. Kanumuri, S. Sadhwani, G. J. Sullivan, H. S. Malvar (Microsoft)]

### Color enhancement SEI messages (2)

JCTVC-N0180 Color Mapping SEI message [Philippe Bordes, Pierre Andrivon, Patrick Lopez, Franck Hiron (Technicolor)]

JCTVC-N0224 SEI message: post filters to enhance the chroma planes [J. Dong, Y. Ye, Y. He (InterDigital)]

### Other SEI messages (1)

JCTVC-N0286 Time code SEI message [C. Fogg, B. Lewis, J. Lassahn, T. Prins (Elmental)]

## Non-normative: Encoder optimization, decoder speed improvement, post filtering, loss concealment, rate control

### Rate control

### Encoder optimization

### Software development

JCTVC-N0043 On software complexity: real time and parallel SHVC video decoder [W. Hamidouche, M. Raulet, O. Déforges (??)]

## Withdrawn and unclear allocation

JCTVC-N0068 Withdrawn

JCTVC-N0166 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the decision of the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XX Oct 2013.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-M1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CEs shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC.

## Software development (to be updated)

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 11.0 and SHM 2.0 should be available within 2 weeks after the meeting.

- HM 11.0+RExt should be available within 1 week after HM 11.0 availability.

- Availability of the range extensions software is expected 1 week after HM 9.0.

## Subjective verification test plan

[To be discussed as follow-up from AHG report.]

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-M1002 HEVC Test Model 11 (HM 11) Encoder Description. - Collect reports of errata for HEVC version 1 text specification. - Gather and address comments for refinement of these documents. - Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, T. Wiegand (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 11.0 software version and the reference configuration encodings according to JCTVC-L1100 common conditions (expected within 2 weeks after the meeting). - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Suggest configuration files for additional testing of tools. - Coordinate with HEVC Test Model editing and Errata Reporting AhG to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Study to develop a potential set of HEVC conformance bitstreams. | T. Suzuki, W. Wan (co-chairs) | N |
| HEVC range extensions development (AHG5) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats, bit depths beyond 8 bits, and auxiliary/alpha channel coding, in coordination with AHG7 and AHG18. - Perform memory bandwidth analysis of the range extensions technical design its proposed modifications. - Discuss and propose test conditions and test material for the development of the range extensions. - Study techniques for colour conversion and resampling and their relationship to non-4:2:0 chroma coding. | M. Naccari, C. Rosewarne (co-chairs) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-M1005 HEVC RExt draft text (HM 10-RExt-3). - Gather and address comments for refinement of the text. - Coordinate with AHG7. | J. Sole (primary), D. Flynn, C. Rosewarne, T. Suzuki | N |
| Range extensions software development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 11.0-RExt-3.0 software version and the reference configuration encodings according to JCTVC-L1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6. | D. Flynn, K. Sharman (co-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for screen content coding. - Make recommendations for test conditions for screen content coding. - Coordinate with finalization of the test conditions for CEs on lossless and screen content coding. | H. Yu (chair), M. Budagavi, R. Cohen, A. Duenas, T. Lin, J. Xu (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Study the hypothetical reference decoder (HRD) syntax and operations and the related text for bitstream conformance and decoder conformance. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. | M. M. Hannuksela (chair), J. Boyce, Y. Chen, A. Norkin, Y.-K. Wang, (vice-chairs) | N |
| SHVC core experiments (AHG10) (jct-vc@lists.rwth-aachen.de) - Establish configurations for SHVC core experiments - Generate anchors used for SHVC core experiments - Create reporting sheets for core experiments - Provide configuration data to be used in SHVC CEs (within one week after SHM 2.0 software is available) - Discuss and identify additional issues related to SHVC core experiments | X. Li (chair), J. Boyce, P. Onno, Y. Ye (vice-chairs) | N |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-M1007 SHVC Test Model 2 (SHM 2) text. - Produce and finalize JCTVC-M1008 SHVC text specification Draft 2. - Gather and address comments for corrections and editorial improvements of these documents. - Coordinate with the SHVC Software (SHM) development AHG to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 2.0 software (based on HM 10.1) for experimentation. - Provide software to CEs within two weeks after the meeting. - Bring software into alignment with HM 11 by the next meeting. - Discuss and identify additional issues related to SHVC software. | V. Seregin (chair), T. Chuang, Y. He, D. Kwon (vice-chairs) | N |
| SHVC inter-layer filtering (AHG13) (jct-vc@lists.rwth-aachen.de) - Study alternative upsampling and downsampling filters for spatial scalability. - Study candidate filters for SNR scalability. - Study phase relationships for inter-layer filters. - Identify alternative candidate resampling filters and distribute associated downsampled test material within 3 weeks after the meeting. - Study resampling filters with ratios other than 1.5 and 2.0 (and 1.0). - Discuss and identify additional issues related to inter-layer filtering. | A. Segall (chair), E. Alshina. J. Chen, J. Dong, P. Topiwala, M. Zhou (vice-chairs) | N |
| Colour gamut scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut scalable coding. - Study the interaction of colour gamut scalability with bit-depth or spatial scalability. - Identify test sequences and test conditions. - Discuss and identify additional issues related to colour gamut scalability. | A. Segall (chair), P. Bordes, J. Dong, A. Duenas, L. Guo, and D.-K. Kwon (vice-chairs) | N |
| Hybrid codec scalability (AHG15) (jct-vc@lists.rwth-aachen.de) - Coordinate with AHG12 on software and anchor support for AVC base layer - Study the rate-distortion benefits, complexity impact, and practicality of using inter-layer syntax prediction from AVC base layer - Study signalling methods of AVC base layer properties in SHVC high-level syntax. | J. Boyce, K. Kawamura (co-chairs) | N |
| Single-loop scalability (AHG16) (jct-vc@lists.rwth-aachen.de) - Study and compare single-loop and multi-loop scalability (focus on SNR as the more demanding case). - Analyze and compare the memory bandwidth of single-loop and multi-loop scalability. - Study the rate-distortion performance and complexity impact of single-loop scalability using residual refinement techniques and by signalling inter-layer prediction restrictions on a multi-loop design. - Study and analyze the performance and complexity impact of inter-layer inter prediction tools for single-loop and multi-loop scalability. | M. Wien (chair), J. Boyce, M. Budagavi, K. Misra, K. Ugur (co-chairs) | N |
| SHVC complexity assessment (AHG17) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them - Study restrictions to reduce memory bandwidth and complexity in scalable tools (e.g. restrictions on PU size, fractional-sample accuracy, inter-layer filters, use of bi-predictive PUs, reduction in the number of EL reference pictures etc.) - Evaluate the impact of such restrictions on coding efficiency. - Prepare a report analyzing performance and complexity of single-layer, simulcast, and scalable coding configurations for 1.0, 1.5, and 2.0 scalability cases. | M. Budagavi (chair), E. Alshina, J. Dong, E. François, J. Kang, X. Li, A. Tabatabai (vice-chairs) | N |
| High bit-rate & bit-depth operating points (AHG18) (jct-vc@lists.rwth-aachen.de) - Study the accuracy needed for internal processing elements to support bit depths up to 16-bits per sample - Study relationship to lossless coding capability - Verify rate-distortion optimization behaviour for high bit rates and high bit depths - Study entropy coding operation and throughput at high bit rates and high bit depths and potential needs for associated design modification - Identify test sequences and test conditions for testing high bit rate and high bit depth coding behaviour - Prepare software implementation for technical investigation of new features intended for high bit rates and high bit depths - Study coding performance at high bit-rate and high bit depth operating points and investigate the benefit over existing standards. | K. Sharman (chair), E. François, H.-Y. Kim (vice-chairs) | N |
| Verification test preparation (AHG19) (jct-vc@lists.rwth-aachen.de) - Identify test conditions and test sequences for verification of HEVC compression capability, including consideration of use case scenarios for random access and low delay operation - Identify bit rates, picture resolutions and bit depths appropriate for HEVC verification - Identify appropriate encoder usage for comparison of HEVC and AVC compression capability - Communicate with subjective test coordinator to ensure test design validity - Generate and collect candidate HEVC and AVC encoded bitstreams for HEVC verification testing | T. K. Tan, V. Baroncini (co-chairs), M. Karczewicz, W. Wan, J. Wen (vice-chairs) | N |
| Chroma format scalability (AHG20) (jct-vc@lists.rwth-aachen.de) - Identify candidate designs and associated syntax representations for achieving chroma format scalability - Develop software for testing of chroma format scalability designs - Analyze compression and complexity characteristics of chroma format scalability - Consider interactions and harmonization of chroma format scalability with regard to other forms of scalability, range extensions, and 3D extensions of HEVC | A. Segall (chair), A. Duenas, K. Ugur (vice-chairs) | N |
| Best-effort decoding with reduced decoding complexity (AHG21) (jct-vc@lists.rwth-aachen.de) - Identify use cases and key functional elements for application of reduced-complexity decoding processes for decoding of bitstreams encoded for HEVC and its extensions - Analyze tradeoffs for optimization of video quality in the application of reduced-complexity decoding processes - Particularly study characteristics of design and optimization for decoding processes with reduced bit depth - Consider and analyze the impact of inter-picture dependencies and cross-region spatial prediction dependencies in the application of reduced-complexity decoding processes - Study potential approaches to describe/specify the provision of best-effort decoding capabilities in the HEVC text specification | D. Flynn, J. Sole (co-chairs) | N |

# Output documents (to be updated)

Planning of HM encoder description – plan to improve this in AHG work toward publishing as part of the RS standard. TBD whether this would be in v.1 of the RS standard or added later.

Issues in conformance: The situation has been improving. profile_idc and level_idc need to be correct, some HM 9 bitstreams not updated, some planned bitstreams missing, many bitstreams need updates for corrections of these issues and others. Coverage still needs improvement. Various combinations of tiles & slices & loop filtering control parameters, SEI messages, "corner cases".

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-M1000 Meeting Report of 13th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm]

JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

Disposition of comments report

JCTVC-M1002 High Efficiency Video Coding (HEVC) Test Model 11 (HM 11) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N 13574) [2013-07-15]

Remove JCTVC-L1003 High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent) [B. Bross (primary), W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand] (WG 11 N 13333) [2013-03-01]

JCTVC-M1004 HEVC Conformance Draft 3 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 13xxx ISO/IEC CD) [2013-05-10]

JCTVC-M1005 HEVC Range Extensions Draft 3 [D. Flynn, J. Sole, T. Suzuki] (WG 11 N 13569, Study of ISO/IEC PDAM) [2013-06-15]

Remains valid – not reissued JCTVC-L1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, K. Sharman] [2013-02-08]

JCTVC-M1007 SHVC Test Model 2 (SHM 2) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13571) [2013-06-15]

Will contain encoder descript and will retain TextureRL.

JCTVC-M1008 SHVC Working Draft 2 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13570) [2013-06-15]

Will contain RefIdx and upsampling.

JCTVC-M1009 Common SHM test conditions and software reference configurations [X. Li, J. Boyce, P. Onno, Y. Ye] [2013-04-26]

JCTVC-M1010 HEVC HM 11 Reference Software [F. Bossen, D. Flynn, K. Sühring] (WG 11 N 13575 Study of reference software) [2013-05-10]

Remains valid – not re-issued: JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting were not considered binding on final CE plans as reviewed in closing plenary.

- JCTVC-M1101 HEVC Scalable Extensions Core Experiment SCE1: Support for additional resampling phase shifts [E. Alshina, J. Dong, L. Guo (CE Coordinators)]

- JCTVC-M1102 HEVC Scalable Extensions Core Experiment SCE2: Combination of inter-layer syntax prediction and motion data compression [K. Sato, C. Gisquet (CE Coordinators)]

- JCTVC-M1103 HEVC Scalable Extensions Core Experiment SCE3: Combined inter- and interlayer prediction [X. Li, P. Lai, D. Kwon, A. Saxena, X. Xiu (CE coordinators)]

- It was suggested to test M0155 and M0189. Although roughly consistent with "refIdx plus inter-layer resampling plus ordinary syntax and decoding process" concept, these were relatively high complexity proposed modification. The estimated potential gain for 1.5 and 2.0 LDB cases was 2%.

- No CE established.

- JCTVC-M1103 HEVC Scalable Extensions Core Experiment SCE3: Inter-layer filtering [J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, J. Park (CE coordinators)]

JCTVC-M1101 through JCTVC-M1104 were reviewed in non-final form in closing plenary. A three week finalization period was authorized.

JCTVC-M1121 HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods [T. Nguyen (primary), J. Sole, J. Kim (CE coordinators)]

JCTVC-M1122 HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip blocks [R. Joshi (primary), P. Amon, R. Cohen, S. Lee, M. Naccari (CE coordinators)]

Some YUV and some RGB. Some 10 bit and some 8 bits. Some 4:4:4 and some 4:2:2 and some 4:2:0.

Results for YUV and RGB to be reported separately (not averaged together).

JCTVC-M1123 HEVC Range Extensions Core Experiment 3 (RCE3) Intra coding method for screen content [L. Guo (CE coordinator)]

Both lossless and lossy, RGB and YUV, some non-screen content at 8 and 10 bits.

JCTVC-M1121 through JCTVC-M1123 were reviewed in non-final form in closing plenary. A three week finalization period was authorized.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 25 July – 2 Aug. 2013 under WG 11 auspices in Vienna, AT.

- 23 Oct. – 1 Nov. 2013 under ITU-T auspices in Geneva, CH.

- 9–17 Jan. 2014 under WG 11 auspices in San Jose, US.

- 27 Mar. – 4 Apr. 2014 under WG 11 auspices in Valencia, ES.

- 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP.

The agreed document deadline for the July 2013 meeting is Monday 15 July. Moreover, it was agreed to start the meeting with SHVC HLS starting on the first day, SHVC CE work and related contributions starting on the second day, and RExt and other topics starting on the third day.

XXX was thanked for the excellent hosting of the 14th meeting of the JCT-VC. XXX was thanked for providing viewing equipment used at the meeting.

The JCT-VC meeting was closed at approximately 1345 hours on Fri. 02 Aug. 2013.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the thirteenth meeting of the JCT-VC, according to a sign-in sheet circuated during the meeting (approximately XXX in total), were as follows: