| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 16th Meeting: San José, US, 9–18 Jan. 2014 | Document: JCTVC-P_Notes_d1 |
| --- | --- |

| Title: | Meeting report of the 16th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), San José, US, 9–18 Jan. 2014 |  |  |
| --- | --- | --- | --- |
| Status: | Report Document from Chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communications Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its sixteenth meeting during 9 – 18 Jan. 2014 at the Double Tree Inn in San José, US. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 900 hours on Thursday 9 Jan. 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 18 Jan. 2014. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

The primary goals of the meeting were to review the work that was performed in the interim period since the fifteenth JCT-VC meeting in producing the 13th HEVC Test Model (HM13) software and text, the range extensions (RExt) Draft 5 with associated test model and software, the Scalable HEVC (SHVC) extensions Draft 4 with associated test model and software, review the results from four interim Core Experiments on RExt (RCEx) and one Core Experiments on SHVC (SCEx), and review technical input documents. Important topics of the meeting included the review of progress made towards definitions of SHVC and RExt. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were considered, and a verification test plan was set up for HEVC version 1 performance testing.

In addition to experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting (update): HEVC test model (HM) 13, HEVC conformance testing draft 5, a set of guidelines for conformance test bitstream preparation, HEVC Defect Report draft 2 (for Version 1), HEVC range extensions draft 5 and RExt test model 5, SHVC extensions draft 4 and SHVC Test Model 4, a description of designs under study for SHVC hybrid scalability, verification test plan draft 2, and two documents specifying common test conditions and software reference configurations for experiments – one for HEVC range extension experiments, and one for scalable coding experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 27 March – 4 Apr. 2014 under WG 11 auspices in Valencia, ES, 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP, 16–24 October 2014 under WG 11 auspices in Strasbourg, FR, and XX – XX Feb. 2015 under ITU-T auspices in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its sixteenth meeting during 9 – 18 Jan. 2014 at the Double Tree Inn in San José, US. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 900 hours on Thursday 9 Jan. 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 18 Jan. 2014. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2014_01_P_SanJose/

## Primary goals

The primary goals of the meeting were to review the work that was performed in the interim period since the fifteenth JCT-VC meeting in producing the 13th HEVC Test Model (HM13) software and text, the range extensions (RExt) Draft 5 with associated test model and software, the Scalable HEVC (SHVC) extensions Draft 4 with associated test model and software, review the results from four interim Core Experiments on RExt (RCEx) and one Core Experiments on SHVC (SCEx), and review technical input documents. Important topics of the meeting included the review of progress made towards definitions of SHVC and RExt. Advancing the work on development of conformance and reference software for HEVC and its extensions is also a significant goal. Needs for corrections to version 1 were considered, and a verification test plan was set up for HEVC version 1 performance testing.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Friday, 3 Jan. 2014.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Saturday 4 Jan. 2014 were considered "officially late".

Most documents in the “late” category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-P0219 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "P0219+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-P0XXX (a proposal for ...) [uploaded 01-XX]

- ...

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-P0XXX (a contribution ...) [uploaded 01-XX]

- ...

The following cross-verification reports were registered on time but were uploaded late: JCTVC-P0XXX [uploaded 01-XX], ... .

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-P0XXX, ... .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution document was rejected as a "placeholder" without any significant content and was not corrected until after the upload deadline:

- JCTVC-P0XXX (an .. , corrected by a late upload on 01-XX)

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting (update)

The report documents of the previous meeting, particularly including the meeting report JCTVC-N1000, the HEVC Test Model (HM) JCTVC-N1002 (which still needs further attention to reach the quality we would like it to have), the Defect Report JCTVC-N1003, the Conformance Draft JCTVC-N1004, the Reference Software Draft JCTVC-N1010 (uploaded during the first day of the meeting), the Draft Specification of Range Extensions JCTVC-N1005, the SHVC draft specification JCTVC-N1008, the SHVC test model 2 (SHM2) JCTVC-N1007, the common test conditions for RExt (JCTVC-N1006) and SHVC (JCTVC-N1009), and the HEVC verification test plan JCTVC-N1011 were approved. The HM reference software and the reference software versions for range extensions and SHVC, were also approved.

The group was asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on HEVC project guidance

- Consideration of HEVC technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning.

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-acess skip.

- Consent: A step taken in ITU-T to formally consider a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC).

- DT: Decoding time.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- GRP: Generalized residual prediction.

- HE: High efficiency – a set of coding capabilities designed for enhanced compression performance (contrast with LC). Often loosely associated with RA.

- HEVC: High Efficiency Video Coding – the video coding standardization initiative under way in the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LC: Low complexity – a set of coding capabilities designed for reduced implementation complexity (contrast with HE). Often loosely associated with LD.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Often loosely associated with LC. Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- NAL: Network abstraction layer (as in AVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PPS: Picture parameter set (as in AVC).

- QM: Quantization matrix (as in AVC).

- QP: Quantization parameter (as in AVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD). Often loosely associated with HE.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SDIP: Short-distance intra prediction.

- SEI: Supplemental enhancement information (as in AVC).

- SD: Slice data; alternatively, standard-definition.

- SH: Slice header.

- SHVC: Scalable high efficiency video coding.

- SPS: Sequence parameter set (as in AVC).

- TB: transform block.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- TFD: Tagged for discard.

- Unit types:

  - CTB: Coding tree block (luma or chroma).

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU)

  - CB: Coding block (luma or chroma).

  - CU: Coding unit (containing both luma and chroma).

  - LCU: (formerly LCTU) largest coding unit (synonymous with CTB).

  - PB: Prediction block (luma or chroma)

  - PU: Prediction unit (containing both luma and chroma), with four shape possibilities.

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN: Having two areas that each have the full width and half the height of the CU.

    - Nx2N: Having two areas that each have half the width and the full height of the CU.

    - NxN: Having four areas that each have half the width and half the height of the CU.

  - TB: Transform block (luma or chroma).

  - TU: Transform unit (containing both luma and chroma).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a draft standard.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks (update)

The status of HEVC version 1 in ISO/IEC and ITU-T was noted. The FDIS 23008-2 had been submitted for ballot in ISO/IEC, which closed by July 2014. After that, ISO/IEC, publication remained under preparation (it was subsequently published on 2013-11-25). In ITU-T, the text had been published as Rec. H.265 on 2013-06-07.

The HEVC reference software and conformance testing specifications had been submitted as ISO/IEC DIS 23008-5 and DIS 23008-8, respectively. The ballot closing dates for both texts were scheduled for 2014-01-16, which is likely to be too late to enable issuing FDIS documents at the meeting that ends on 2014-01-17.

The range extensions draft 4 had been submitted as ISO/IEC 23008-2/DAM1. The ballot closing date was scheduled for 2014-01-16, which is likely to be too late to enable issuing the FDAM document at the meeting that ends on 2014-01-17.

The scalable extensions draft 3 had been submitted as ISO/IEC 23008-2/PDAM3. The ballot closing date was scheduled for 2013-12-31, which would be in time for issuing a DAM ballot document at the meeting that ends on 2014-01-17.

It was noted that in the most-recently-established voting process in ISO/IEC, a "No" vote has a different status than it previously did for the DIS / DAM ballot stage. WG 11 NBs should make sure to be aware of the implications of their votes, and may wish to consider voting "Yes with comments" in some circumstances in which they would previously have been inclined to vote "No with comments".

Goals included progress of work on extensions, conformance & reference software, and verification testing.

Ballots with a DIS timing problem (closing date Jan 16, 2014) were noted by topic as follows.

- RExt

- Software

- Conformance

- MV-HEVC

- PDAM ballot for SHVC closed XX-XX (WG 11 M32217). This should proceed to DIS as an output of this meeting.

- The next meeting begins 30 June, so any editing period for that needs to be relatively short (to check on that).

- Remarks on Lateness of documents.

- Remarks on Number of documents.

- Primary topic areas:

- HLS

- Non-HLS SHVC

- RExt

  - Intra block

  - Palette mode

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on SHVC HLS, SHVC and RExt CE work and related contributions during the first few days. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- First day 0900-1300: Opening and AHG reports (Donner)

- First day 1430-?

  - HLS (Donner - GJS)

  - SHVC (Carmel - JRO): SCE, then related, then other

  - RExt BoG (Monterey – Sharman & Rosewarne): RCEs, then related, then other

- Second day 0800-1300:

  - HLS (Donner - )

  - RExt (Siskiyou - )

  - SHVC (Carmel)?

  - Monterey?

## Contribution topic overview (update)

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. (Note: allocation to tracks were subject to changes)

- AHG reports (22) Track P (section 2)

- Communication to and by parent bodies (0) Track P (section 3.1)

- Conformance testing development (2) Track P (section 3.2)

- Version 1 bug reports and cleanup (1) Track P (section 3.3)

- RExt text improvements (3) Track P (section 3.4)

- Coding performance, implementation, and design analysis (13) Track P (section 3.4)

- Profile and level definitions (8) Track P (section 3.5)

- HEVC and RExt use cases (0) Track P (section 3.6)

- Source video test material (10) Track P (section 3.7)

- SHVC CE1: Arbitrary scalability ratio support (5) Track X (section 4.1)

- SHVC CE2: Key pictures and single loop decoding (8) Track X (section Error: Reference source not found)

- SHVC CE3: Inter-layer filtering (5) Track X (section Error: Reference source not found)

- SHVC CE4: Color gamut and bit-depth scalability (12) Track X (section Error: Reference source not found)

- RExt CE1: Inter-component decorrelation (5) Track X (section 5.1)

- RExt CE2: Rice parameter initialization and update (15) Track X (section 5.2)

- RExt CE3: Intra prediction techniques (13) Track X (section 5.3)

- Non-CE RExt (16 CE related, 48 other) Track X (section 6.1)

- Non-CE SHVC (13 CE related, 7 other) Track X (section 6.2)

- High-level syntax common issues in RExt, 3D, SHVC, single layer (XX) Track X (section 6.3)

- High-level syntax in SHVC and 3D extensions (4) BoG | joint with JCT-3V (section )
	BoG alignment, etc. (J. Boyce)

- High-level syntax in SHVC (56) BoG | Track X (section Error: Reference source not found)

- VUI and SEI messages (13) BoG | Track X (section 6.5)

- Non-normative (0) Track X (section 6.6)

- Plenary discussions and BoG reports (3) (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC. (sections 8, 9, and 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Overall approximate contribution allocations: Track P: XX; Track A: XXX; Track B: XXX.

# AHG reports (20)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Reviewed Thu 9th except as noted otherwise.)

JCTVC-P0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm (co-chairs)]

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

In the interim period since the 15th JCT-VC meeting, the 13th HEVC Test Model (HM13) software and text had been produced, Four interim Core Experiments on range extensions (RCEx) and one Core Experiment on scalable extensions (SCEx) were run. In preparation of the 16th meeting, progress was made towards definitions of Scalable HEVC (SHVC) extensions and range extensions into higher bit depths and non-4:2:0 colour sampling. Advancing the work on development of conformance and reference software for HEVC and its extensions was also a significant goal. Needs for corrections to version 1 were considered, and a verification test plan was further refined for HEVC version 1 performance testing.

The work of the JCT-VC overall had proceeded well and actively in the interim period. Active discussion had been carried out on the group email reflector (which had 1745 subscribers as of 2014-01-08), and the output documents from the preceding meeting had been produced.

Due to a decision made by MPEG after the closing of the JCT-VC meeting, several modifications relating to 4:4:4 profiles were not included in the 4:4:4 profile definitions of the ISO/IEC Study Text corresponding to HEVC range extensions Draft 5 (JCTVC-O1005). This is further discussed in the range extensions draft and the meeting report JCTVC-O1000. Other developments generally proceeded as expected in the interim period since the last meeting.

Except as noted below, all report documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2013_10_Geneva), particularly including the following:

- The meeting report (JCTVC-O1000) [Posted 2014-01-09]

- The HEVC software guidelines (JCTVC-H1001) [Not updated for several meeting cycles]

Note: The "H" document, in this case, remains valid as the latest approved software guidelines.

- The HM 13 encoder description (JCTVC-O1002) [Posted 2014-01-07]

- The HEVC (version 1) defect report draft 2 (JCTVC-O1003) [First posted 2013-11-19, last updated 2013-11-24]

- HEVC conformance specification Draft 5, submitted as ISO/IEC Study of DIS (JCTVC-O1004) [First posted 2013-11-25, last updated 2013-11-27]

- HEVC range extensions Draft 5, a subset of which was submitted as ISO/IEC Study of DAM (JCTVC-O1005) [First posted 2013-11-19, last updated 2013-12-25]

NOTE: Please see remarks above regarding the 4:4:4 profile content of this text.

- HEVC range extensions common test conditions and software reference configurations (JCTVC-O1006) posted 2013-11-16, last updated 2013-12-05]

- SHVC Test Model 4 (JCTVC-O1007) [Posted 2013-12-02]

- SHVC Draft 4, submitted as ISO/IEC Study of PDAM (JCTVC-O1008) [posted 2013-11-14, last updated 2013-12-07]

- Common SHM test conditions and software reference configurations (JCTVC-O1009) [Posted 2013-11-23]

- Guidelines for conformance test bitstream preparation (JCTVC-O1010) [Posted 2013-11-01]

- HEVC verification test plan draft 2 (JCTVC-O1011) [Posted 2013-11-02]

- Designs under study for SHVC hybrid scalability (JCTVC-O1012) [Posted 2013-11-08]

- Range Extensions Test Model 5 encoder description (JCTVC-O1013) [Posted 2013-12-14]

- Description of HEVC Scalable Extensions Core Experiment 1 (SCE1): Color gamut and bit depth scalability (JCTVC-O1101) [First posted 2013-11-04 with final update 2013-11-07]

- Description of HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding and high bit depth (JCTVC-O1121) [First posted 2013-11-07 with final update 2014-01-07]

- Description of HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization methods (JCTVC-O1122) [posted 2013-11-16]

- Description of HEVC Range Extensions Core Experiment 3 (RCE3): Intra block copy refinement (JCTVC-O1123) [posted 2013-11-20]

- Description of HEVC Range Extensions Core Experiment 4 (RCE4): Palette coding for screen content (JCTVC-O1124) [First posted 2013-11-19 with final update 2013-12-21]

The various ad hoc groups and the five core experiments had made progress, and various reports from those activities had been submitted.

The software for HM version 13.0 had been prepared and released approximately as scheduled. Software for SHVC and Range Extensions were built on top of HM13 as well.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-13.0.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-13.0-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 260 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted.

A preliminary basis for the document subject allocation and meeting notes for the 16th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

JCTVC-P0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, K. McCann (co-chairs), W.-J. Han, I.-K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang (vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between 15th meeting in Geneva (23 October – 1 November 2013) and the 16th meeting in San José (9 – 17 January 2014).

At the 15th JCT-VC meeting, a document detailing the Editors' proposed corrections to HEVC version 1 was endorsed as representing the current state of development for errata corrections to the HEVC version 1 specification.  In addition, the 13th HEVC test model JCTVC-O1002 was developed from the 12th HEVC test model JCTVC-N1002, with minimal changes.

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.  A total of 2 issues with the HEVC version 1 specification (JCTVC-L1003 and JCTVC-O0054) were reported on the tracker following the 15th JCT-VC meeting. No issues were reported on the tracker with the HM12 Encoder Description in JCTVC-N1002 during this period.

The HM13 Encoder Description was published as JCTVC-O1002.

The HEVC Defect Report 2 was published as JCTVC-O1003.

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Approve the HM13 Encoder Description in document JCTVC-O1002 as JCT-VC output.

- Determine whether or not an HM14 Encoder Description should be created, in the absence of any input documents or issues raised via the tracker. [To be further discussed during the meeting – plan to identify volunteers for particular topics – should check the software and try to identify who has the best knowledge of the algorithms for each aspect..]

- Encourage the use of the issue tracker to report issues with the text of both the HEVC v1 specification and the Encoder Description.

- Coordinate with the Software development and HM software technical evaluation AhG to address issues relating to any mismatches between software and text.

- Consider further proposals for editorial improvements to HEVC v1.

JCTVC-P0003 software development and software technical evaluation [miss]

Presentation deferred.

JCTVC-P0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, W. Wan]

Presentation deferred.

JCTVC-P0005 JCT-VC AHG report: HEVC range extensions development (AHG5) [C. Rosewarne, M. Naccari]

This report summarises the HEVC range extensions development Ad-Hoc Group (AHG5) between the 15th JCT-VC meeting in Geneva, CH (October-November 2013) and the 15th JCT-VC meeting in San José, CA, USA (January 2014).

After kick-off message issued on the 18th of November no further emails were sent to the reflector regarding AHG5.

The ‘HEVC Range extensions test model 1 encoder description’ (JCTVC-O1013) was prepared and uploaded to the document repository.

Contribution JCTVC-O1006 was uploaded, defining the test conditions as agreed at the 15th JCT-VC meeting for Range extensions development. As discussed at the 15th JCT-VC meeting, the number of encoded frames was reduced for several of the test sequences. This contribution also includes the spread sheet templates to be used in RExt Core Experiments (RCEs) at this meeting.

Relevant contributions were listed according to subject areas:

- RCE contributions (see RCE summary reports)

- Profiles & levels (6)

- High-level syntax and auxiliary pictures (8)

- High dynamic range (3)

- Cross-component decorrelation (4)

- Miscellaneous (5)

JCTVC-P0006 JCT-VC AHG report: Range extensions draft text (AHG6) [J. Sole, D. Flynn, C. Rosewarne, G. Sullivan, T. Suzuki]

This document reports on the work of the JCT-VC ad hoc group on Range extensions draft text (AHG6) between the 15th JCT-VC meeting in Geneva, CH (October-November 2013) and the 16th JCT-VC meeting in San Jose, US (January 2014).

The High Efficiency Video Coding (HEVC) Range Extensions test model was developed following the decisions taken at the 15h JCT-VC meeting in Geneva, CH (October-November 2013).

Four versions of JCTVC-O1005 were published by the Editing AhG following the 15th JCT-VC meeting in Geneva. Versions were based upon JCTVC-L1003_v34. The text of JCTVC-O1005 (revision 4) was submitted as WG11 N13933 Study text of ISO/IEC.

Changes in JCTVC-O1005 relative to the previous version were listed in the report.

- JCTVC-P0007 JCT-VC AHG report: Range extensions software (AHG7) [K. Sharman, D. Flynn]

This report summarizes the activities of Ad Hoc Group 7 on support for range extensions between the 15th and 16th JCT-VC meetings.

The HM12.0_RExt4.1 software was upgraded to HM12.0_RExt4.2 (macro removal and code tidy) and then to HM12.1_RExt4.2 (reflecting the changes made to the main HM branch), with both revisions released on Tuesday November 19th. All were tested according to the HM and RExt4.0 test conditions; the latter was also tested using the latest RExt5 test conditions.

On Tuesday November 19th, the development code for RExt5.0 (RExt5.0_rc1) with changes for all tools was announced, and a request was made for proponents to verify their tools.

During the integration process, it was noticed that one of the tools changed the encoder search algorithm. This would mean that the RExt model would no longer be backwards compatible with the general HM model. The coordinators requested a solution from the proponents, with the provided solution being a duplication of one of the longer inter-encoder-search functions. To try to avoid code replication, the coordinators quickly made changes so that only one search function was used and that it was compatible with HM. The version in rc1 was backwards compatible with HM, however, there was a mistake when cross-component-decorrelation was used in the inter search.

To expedite the process of release of RExt5.0, the coordinators fixed the problem, but also used the duplicated search algorithm provided by the proponents (controlled by a macro). It is this second search algorithm that is being used in standard RExt test conditions; the original search is used in HM test conditions.

RExt5.0 was released on Tuesday December 3rd, and version RExt5.1 (with many of the new tool macros removed and a code tidy) was released on Thursday December 5th.

It was noted that there is an input contribution P0059 to address the search situation.

The changes adopted to this RExt5 were due to proposals listed in the report.

The naming of inter-component decorrelation / cross-component prediction was discussed, and it was agreed to suggest the term "cross-component prediction" to the editors.

Some items were identified as still to be integrated:

- O0044 – CU-adaptive chroma QP offsets (highest priority – this item is normative)

- O0043 – Best-effort decoding

- N0383 – Motion constrained tile sets SEI

- O0079 – Chroma sampling filter hint SEI (some HM patch software was in the contribution )

- O0099 – Time code SEI (patch against HM5.1 provided by proponents)

Encoding performance differences were reported

HM12.1_RExt4.1 vs HM12.1_RExt5.1 under RExt5 AHG5 conditions

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 25.4% | 22.5% | 24.3% | 19.4% | 17.6% | 18.9% | 13.8% | 12.9% | 13.5% |
| YCbCr 4:4:4 | 1.5% | 7.8% | 8.2% | 1.9% | 5.1% | 7.4% | 2.0% | 3.6% | 5.2% |
| YCbCr 4:2:2 | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  | 105% |  |  | 103% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 18.2% | 15.4% | 17.8% | 14.1% | 10.7% | 13.7% |  |  |  |
| YCbCr 4:4:4 | 0.5% | 10.0% | 8.6% | 0.9% | 7.3% | 9.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 15.4% | 11.0% | 12.7% | 11.9% | 7.9% | 9.9% |  |  |  |
| YCbCr 4:4:4 | 0.1% | 7.3% | 6.4% | 0.6% | 5.0% | 7.2% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% |  |  |  |
| Enc Time[%] | 98% |  |  | 95% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |  |  |  |

HM12.1_RExt4.1 vs HM12.1_RExt5.1 under RExt5 AHG8 lossless conditions

|  | Average bit-rate increase |  |  |
| --- | --- | --- | --- |
|  | AI | RA | LB |
| Class F | 0.2% | -0.5% | -1.0% |
| Class B | 0.1% | 0.0% | 0.0% |
| RGB 4:4:4 SC | 27.1% | 20.0% | 17.0% |
| RGB 4:4:4 Animation | 14.8% | 2.5% | 1.3% |
| YCbCr 4:4:4 SC | 4.6% | 3.8% | 2.9% |
| YCbCr 4:4:4 Animation | 1.7% | 0.3% | 0.2% |
| RangeExt | 2.7% | 2.3% | 2.3% |
| RGB 4:4:4 SC (Optional) | 47.2% | 44.1% | 44.5% |
| YCbCr 4:4:4 SC (Optional) | 11.8% | 11.2% | 11.2% |
| Enc Time[%] | 100% | 92% | 93% |
| Dec Time[%] | 102% | 102% | 102% |

HM12.1_RExt4.1 vs HM12.1_RExt5.1 under RExt5 AHG8 lossy conditions

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.4% | 0.5% | 0.4% | 0.3% | 0.4% | 0.4% | 0.2% | 0.4% | 0.3% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| RGB 4:4:4 SC | 38.4% | 37.7% | 38.5% | 34.8% | 34.4% | 34.9% | 31.8% | 31.1% | 31.5% |
| RGB 4:4:4 Animation | 37.3% | 36.8% | 35.0% | 36.5% | 34.6% | 33.5% | 32.0% | 29.0% | 28.5% |
| YCbCr 4:4:4 SC | 5.4% | 9.7% | 9.4% | 5.3% | 7.6% | 8.3% | 4.7% | 6.1% | 6.8% |
| YCbCr 4:4:4 Animation | 2.6% | 13.1% | 7.9% | 3.1% | 10.3% | 7.2% | 3.4% | 7.5% | 6.0% |
| RangeExt | 0.9% | 3.1% | 6.8% | 0.9% | 2.4% | 5.6% | 1.0% | 2.0% | 4.2% |
| RGB 4:4:4 SC (Optional) | 45.2% | 46.4% | 46.6% | 47.9% | 49.0% | 50.0% | 49.3% | 51.4% | 52.3% |
| YCbCr 4:4:4 SC (Optional) | 9.8% | 11.7% | 10.4% | 9.1% | 8.9% | 9.4% | 7.6% | 7.1% | 8.0% |
| Enc Time[%] | 110% |  |  | 107% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.3% | 0.2% | 0.2% | 0.2% | 0.1% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |  |  |  |
| RGB 4:4:4 SC | 33.4% | 33.3% | 34.3% | 31.9% | 31.8% | 32.4% |  |  |  |
| RGB 4:4:4 Animation | 25.5% | 25.5% | 23.4% | 26.7% | 25.4% | 23.9% |  |  |  |
| YCbCr 4:4:4 SC | 4.3% | 9.6% | 9.4% | 4.3% | 7.6% | 8.4% |  |  |  |
| YCbCr 4:4:4 Animation | 1.0% | 12.5% | 6.6% | 1.4% | 11.5% | 7.0% |  |  |  |
| RangeExt | 0.4% | 4.9% | 7.0% | 0.5% | 3.4% | 6.8% |  |  |  |
| RGB 4:4:4 SC (Optional) | 43.9% | 44.9% | 45.1% | 48.2% | 49.7% | 50.3% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 8.0% | 11.2% | 9.5% | 7.9% | 9.1% | 8.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.3% | 0.2% | 0.5% | 0.2% | 0.2% | 0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RGB 4:4:4 SC | 30.3% | 29.1% | 30.2% | 29.6% | 29.0% | 29.6% |  |  |  |
| RGB 4:4:4 Animation | 23.3% | 20.6% | 18.8% | 24.3% | 20.7% | 19.5% |  |  |  |
| YCbCr 4:4:4 SC | 3.4% | 8.1% | 8.0% | 3.9% | 6.8% | 7.3% |  |  |  |
| YCbCr 4:4:4 Animation | 0.3% | 10.3% | 4.4% | 1.0% | 9.9% | 5.1% |  |  |  |
| RangeExt | 0.2% | 2.9% | 4.4% | 0.3% | 1.6% | 4.7% |  |  |  |
| RGB 4:4:4 SC (Optional) | 41.1% | 42.2% | 42.2% | 48.5% | 49.9% | 50.4% |  |  |  |
| YCbCr 4:4:4 SC (Optional) | 11.3% | 14.5% | 12.7% | 11.7% | 13.0% | 12.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

AHG18 test conditions have been improved by the change in Rice parameter update, reset and maximum value. However, the AHG18 test conditions cannot be run without modifying RExt4 and so these results were not provided in the report.

As can be seen, RExt4.1 is different for 4:4:4 test conditions, with those differences being mainly due to cross-component decorrelation (which is never used under 4:2:2 test conditions).

Recommendations included the following:

- Continue to develop reference software based on HM and improve its quality.

- Update encoder and documentation to be consistent with the current usage.

- Remove macros introduced in previous HM versions before starting integration towards RExt6.0 such as to make the software more readable.

- Continue to identify bugs and discrepancies with text, and address them.

- Test reference software more extensively outside of common test conditions.

- JCTVC-P0008 JCT-VC AHG report: Screen content coding (AHG8) [H. Yu, R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu]

This report summarizes the activities of the JCT-VC ad hoc group on screen content coding (AHG8) between the JCT-VC 15th meeting in Geneva, Switzerland, and the 16th meeting in San José, USA.

The sequences submitted in O0222 were revised and uploaded to the FTP site. The YUV version of these revised sequences were also uploaded to the FTP site.

The “sc_videoTesting_1920x1080_60_8bit” sequence submitted in O0256 was removed from the FTP site. In this sequence, the Traffic sequence was used as the screen background, which might not comply with its usage agreement.

The “sc_video_conferencing_doc_sharing” sequence was removed from the test material in the current RCEs. As reported in JCTVC-O0337, some anomalous rate-distortion results were observed while encoding some of the screen content sequences by the HM-RExt software under RCE test conditions. This was considered as an encoder issue, and further investigation and study were encouraged. The group recommended that such activities should be separated from the current RCE work. Furthermore, the majority of the group supported to take this particular sequence out from the RCE test material set, because it brought the most confusing results.

Below is the list of all the screen content sequences available to JCT-VC on the Hannover ftp site. Those marked in italics are used in the current RCEs.

Regarding test conditions for RCE2 and RCE3, due to the relatively short interim period before this meeting, the group suggested that the RCE tests related to AHG8 might consider using the same test conditions as the last time and all the old test sequences except for “sc_video_conferencing_doc_sharing”.

| JCT-VC number | sequence name | frames |
| --- | --- | --- |
| L0301 | sc_map_1280x720_60_8bit | 600 |
|  | sc_programming_1280x720_60_8bit | 600 |
|  | sc_wordEditing_1280x720_60_8bit | 600 |
| L0317 | sc_cad_waveform_1920x1080_20_8bit_200_r1 | 200 |
|  | sc_cg_twist_tunnel_1280x720_30_8bit_300_r1 | 300 |
|  | sc_pcb_layout_1920x1080_20_8bit_200_r1 | 200 |
|  | sc_ppt_doc_xls_1920x1080_20_8bit_200_r1 | 200 |
|  | sc_video_conferencing_doc_sharing_1280x720_30_8bit_300_r1 | 300 |
|  | sc_web_browsing_1280x720_30_8bit_300_r1 | 300 |
| M0431 | sc_doc_1280x720_10_8bit_500 | 500 |
|  | sc_SlideShow_1280x720_20_8bit_500 | 500 |
|  | sc_Web_1280x720_10_8bit_500 | 500 |
| N0294 | sc_robot_1280x720_30_8bit_300 | 300 |
|  | sc_viking_1280x720_30_8bit_300 | 300 |
| O0222 | Basketball_Screen_2560x1440_60p_8b | 623 |
|  | BigBuck_1920x1080_60p_8b | 404 |
|  | KimonoError1_2560x1440_60p_8b | 1006 |
|  | KimonoError2_2560x1440_60p_8b | 516 |
|  | KristenAndSaraScreen_1920x1080_60p_8b | 604 |
|  | MissionControlClip1_2560x1440_60p_8b | 601 |
|  | MissionControlClip2_2560x1440_60p_8b | 600 |
|  | MissionControlClip3_1920x1080_60p_8b | 603 |
| O0256 | sc_console_1920x1080_60_8bit | 600 |
|  | sc_desktop_1920x1080_60_8bit | 600 |
|  | sc_flyingGraphics_1920x1080_60_8bit | 600 |
|  | sc_realtimeData_1920x1080_60_8bit | 600 |
|  | sc_socialNetworkMap_1920x1080_60_8bit | 601 |
|  | sc_videoTesting_1920x1080_60_8bit | 601 |
| O0268 | sc_cg2twist_tunnel_1280x720_30_8bit | 300 |
| Others | sc_VenueVu_1920x1080_30_8bit | 300 |

Relevant contributions were listed in the report. The AHG recommended to schedule viewing sessions for the SCC related test results.

RCE3 and RCE4 were noted to be especially relevant for potential viewing needs.

- JCTVC-P0009 JCT-VC AHG report: High-level syntax for HEVC extensions (AHG9) [M. M. Hannuksela, J. Boyce, Y. Chen, S. Deshpande, A. Norkin, Y.-K. Wang, P. Wu]

This report summarizes the activities of the ad-hoc group of high-level syntax for HEVC extensions (AHG9) between the 15th JCT-VC meeting and the 16th JCT-VC meeting.

No coordinated AHG activity took place. No emails were exchanged over the JCT-VC reflector.

However, it was noted that there was a substantial number of related input contributions into this meeting (categorized in Sections 6.3.1, 6.3.2, 6.4.1, 6.4.4, 6.5.1, 6.5.2 and 6.5.3).

JCTVC-P0010 JCT-VC AHG report: Multi-layer picture order count derivation (AHG10) [G. J. Sullivan (chair)]

This document reports on the work of the JCT-VC ad hoc group on multi-layer picture order count derivation, including a report of the teleconferences held and the progress made during the interim period since the preceding meeting. (The reader of this report is assumed to be familiar with various jargon and abbreviations used in JCT-VC; recent meeting reports and the draft specification text for HEVC and its SHVC and MV-HEVC extensions can be consulted for definitions of terms.)

The AHG held six two-hour teleconferences with substantial discussions of the issues. 10–20 people participated in each call – each of which was announced with at least 7 days notice. The calls were held at different times of day to ensure reasonable opportunities for participation from different time zones. It was generally agreed that the current draft design seemed basically "broken".The discussions primarily focused on two proposals: a scheme proposed by Nokia (now revised as JCTVC-P0056 / JCT3V-G0042) and a different scheme proposed by Qualcomm and ZTE (now revised as JCTVC-P0041 / JCT3V-G0031). The proposals for each of these schemes were refined during the interim period, with multiple uploaded revisions.

Pending further input, the AHG concluded that the Qualcomm/ZTE scheme of JCTVC-P0041 / JCT3V-G0031 seems the most mature as the basis for moving forward. It has more mature text, has no significant identified problems, and is asserted to have a resilience advantage. It would be desirable to have that resilience assertion more confidently assessed; however, no significant problem seems to exist in that scheme, so its selection is recommended by the AHG unless additional input indicates otherwise. There was really no serious problem found in the other scheme as well – it was agreed that either scheme seems to be a substantial improvement over the current draft text.

The AHG noted that software verification has not been conducted to really show the operation of the scheme in practice with important test cases. Follow-up on this issue is needed, as much as possible, to increase our confidence that the scheme has all details worked out and would not have any problems in important use cases.

Further study is also needed to determine what is needed for cross-codec multi-layer operation.

Additional new contributions are noted to be relevant, including the following:

- JCTVC-P0067 / JCT3V-G0045 MV-HEVC/SHVC HLS: Comments on POC alignment [M. Li, P. Wu, G. Shang, Y. Xie (ZTE)] – a new proposal asserted to have some characteristics of each of the other two proposals

- JCTVC-P0260 / JCT3V-G0224 MV-HEVC/SHVC HLS: Additional information on the POC design in JCTVC-P0041/JCT3V-G0031 [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] – a late-registered information document; not yet available as of the beginning of the meeting

JCTVC-P0011 JCT-VC AHG report: SHVC text editing (AHG11) [J. Chen, J. Boyce, Y. Ye, M. Hannuksela, Y.-K. Wang]

This document reports the work of the JCT-VC ad hoc group on SHVC text editing (AHG11) between the 15th JCT-VC meeting in Geneva (23 October – 1 November 2013) and the 16th JCT-VC meeting in San Jose (9–17 January 2014).

The editorial team worked on both two documents: JCTVC-O1007 (SHVC Test Model 4 text) and JCTVC-O1008 (SHVC draft 4). Editing JCTVC-O1008 was assigned a higher priority than editing JCTVC-O1007.

One version of JCTVC-O1007 and three versions of JCTVC-O1008 were published by the editing AHG following the 15th JCT-VC meeting in Geneva.

The main changes in JCTVC-O1008, relative to the previous JCTVC-N1008 (SHVC Draft 3) were listed in the AHG report.

JCTVC-O1007 Test Model 4 document mainly contains the general descriptions of SHVC framework, texture data resampling process and motion field mapping process. The main change to the previous JCTVC-N1007 (SHM3) is the inclusion of up-sampling and down-sampling process for arbitrary spatial ratios.

Use SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to SHVC Draft and Test Model text.

It was noted that some input contributions concerned scalable enhancement of a field-coded base layer.

JCTVC-P0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He, D.-K. Kwon]

This report summarises activities of the AHG12 on SHVC software development between 15th and 16th JCT-VC meetings.

The current latest software version is SHM-4.1 contains almost all the items adopted last meeting, however some work is still needed on signaling POC LSB in enhancement layer IRAP pictures related to JCTVC-N0065.

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

Despite several emails sent to the reflector and private messages sent to the proponents, the following items had not been integrated:

- JCTVC-O0109 (Samsung) – planned to be completed during the meeting

- JCTVC-O0135 (Samsung) – planned to be completed during the meeting

- JCTVC-O0164 (Nokia/Sony/Arris) – HRD modification – this was noted to be a difficult area due to the general lack of HRD capability in our software

- JCTVC-O0137, JCTVC-O0200, JCTVC-O0223 (Samsung/HHI/Qualcomm) – it was remarked that this is a minor change and HHI had volunteered to fix it

Two software versions had been released by AHG12, integration details and performance summary are given in the next subsections. In the document, only HEVC base layer results are provided and AVC base layer data can be found in accompanying excel tables. Performance results were reportedly consistent with the adopted techniques.

Software version SHM4.0 based on HM12 was released according to the schedule, including a list of modifications as described in the AHG report.

Description of the common test conditions and anchor data were released as an output document JCTVC-O1009 “Common SHM test conditions and software reference configurations”.

Additional adopted items (listed in the AHG report) had been integrated on top of SHM4.0 version, software base was updated to HM12.1 and was released as SHM4.1.

According to the decision made last meeting, the informal downsampling tool was updated with JSVM based SHVC downsampling from JCTVC-O0071 which supports arbitrary spatial ratio and can be found as a standalone project in the SHM software package.

The downsampler reportedly generates reasonable output only in the scalability ratio range of approx. 1.3x ... 2.2x.

The supported command line and parameters can be found in the software itself and are also summarized in the report.

Notable changes included:

- More that two layer support

- Bit depth scalability

- Arbitrary scalability ratios

- Downsampling tool

JCTVC-P0013 JCT-VC AHG report: SHVC inter-layer filtering (AHG13) [E. Alshina, J. Chen, T. Yamamoto, Y. Ye, P. Topiwala]

This document reports on the work of the JCT-VC AHG on SHVC inter-layer filtering (AHG13) between the 15th JCT-VC meeting in Geneva, Switzerland, (22 Oct – 1 Nov. 2013) and 16th JCT-VC meeting in San Jose, US, (9 – 17 Jan. 2014), and lists the related input documents.

There have not been discussions related to the mandates of this ad-hoc group over the reflector between the 15th JCT-VC meeting and the 16th JCT-VC meeting. However, there were significant activities in the area of inter-layer filtering within the SCE1 core experiment (2 contributions) and in 4 non-SCE1 contributions. In addition to re-sampling filter different variants of color-gamut and bit-depth conversion are studied.

Particular topics noted included:

- There was one contribution about extracting a region of interest. A modified scale and reference position derivation process as well as the associated new parameters are proposed.

- The performance effect of accurately taking into account chroma position alignment during re-sampling processing was studied. There was reportedly one contribution on this issue.

- Field to frame scalability requires re-sampling process modification and phase off-set signaling proposed in thre contributions.

Related contributions were identified in the report.

JCTVC-P0014 JCT-VC AHG report: Colour gamut scalability (AHG14) [A. Duenas, P. Bordes, E. Alshina, Y. He, K. Kwon, X. Li]

This report summarizes the activities of the ad-hoc group of color gamut scalability (AHG14) between the 15th JCT-VC meeting and the 16th JCT-VC meeting.

Potential test sequences were provided by Technicolor and announced on the reflector on July 11, 2013.  The sequences were made available on the Hannover FTP site under /scalable/sequences/CGS and on a new FTP site (FTPserver: ftp-renn.thmulti.com, login: imx-wp3, password: HEVC2011wp).

Seven contributions related to the topic of wide color and bit depth gamut scalability were identified in the report.

JCTVC-P0015 JCT-VC AHG report: SHVC hybrid codec scalability (AHG15) [J. Boyce, A. Duenas, K. Kawamura, J. Samuelsson]

This document reports on the work of the JCT-VC ad hoc group on SHVC hybrid codec scalability (AHG15) between the 15th JCT-VC meeting in Geneva, Switzerland, (22 Oct – 1 Nov. 2013) and the 16th JCT-VC meeting in San Jose (9 – 17 Jan. 2014).

The JCTVC-O1012 output document, “Designs under study for SHVC hybrid scalability” was released on 8 Nov. 2013.  This document contained all three methods for NAL unit codec identification that are under consideration;

- by external means (systems only approach),

- by a prefix byte (HEVC inside AVC NAL units approach),

- by an HEVC encapsulation NAL unit type (AVC inside HEVC NAL units approach).

There was some reflector discussion regarding the intent of the JCTVC-O1012 document, and a clarification provided that the intent was to document various possible approaches for purposes of study, not to provide a single candidate text for adoption as a complete whole.

The reporting template and anchors using an AVC base layer were not updated for the SHM 4.0 software.  At the two previous meetings, there were no technical contributions using the anchors provided for the SHM 2.0 software.

Related contributions were identified. It was noted that having a field-coded base layer was one of the topics of contributions.

The AHG recommended that the following key questions be considered during the meeting:

- Which NAL unit codec identification method(s) should be supported, e.g. should either of the two proposed encapsulation methods be supported in addition to external means?

- Base layer AVC profile and tool restrictions, and in particular, if the base layer may use interlace?  And what coding tool(s) are required to support interlace-to-progressive scalability?

It was remarked that it was also necessary to determine the appropriate approach to the text specification in regard to in which documents the technical approach(es) are specified.

JCTVC-P0016 JCT-VC AHG report: Test sequence material (AHG16) [T. Suzuki, R. Chen, T. K. Tan, S. Wenger]

Presentation deferred.

JCTVC-P0017 JCT-VC AHG report: SHVC complexity assessment (AHG17) [E. Alshina, M. Budagavi, E. François, J. Kang, X. Li, A. Tabatabai, X. Xiu]

This document reports on the work of the JCT-VC AHG on SHVC complexity assessment (AHG17) between the 15th JCT-VC meeting in Geneva, Switzerland, (22 Oct – 1 Nov. 2013) and 16th JCT-VC meeting in San Jose, US, (9 – 17 Jan. 2014), and lists the related input documents.

Since no tool-level adoption for SHVC common test conditions were done at 15th JCTVC meeting, the performance and complexity numbers were reportedly un-changed. A summary of BD-rate performance for a two-layer scalable system was provided in the report, compared to HEVC single layer decoder.

The only core experiment that was performed during inter-meeting period was on color gamut scalability. Complexity assessment module was imported to SCE1 reference s/w, but not used in SCE activity.

A complexity analysis was suggested to be applied to the SCE1 tests in order to better understand complexity and performance benefits provided by color gamut scalability tools.

JCTVC-P0018 JCT-VC AHG report: high bit-rate and bit-depth operating points (AHG 18) [K. Sharman, R. Joshi, H-Y. Kim]

This report summarizes the activities of the Ad Hoc Group on high bit-rate and bit-depth operating points between the 15th JCT-VC meeting held in Geneva, Switzerland and the 16th JCT-VC meeting held in San José, USA.

The AhG intended to use the JCT-VC reflector for discussion, although there was no activity for this AhG. There have been some private discussions.

An updated set of test conditions was specified and used in RCE1 and RCE2, and announced to the community at the kick-off. The benchmarks (which were also emailed via the reflector) are attached to this contribution. The changes included additional sequences and synthesised high dynamic range video.

RCE1 is examining throughput and RCE2 is examining coding efficiency, which may impact AHG18.

Three other contributions were identified as relating to the topic (P0061, P0173, P0222). A fourth was also noted as somewhat related (P0162).

JCTVC-P0019 JCT-VC AHG report: Verification Test Preparation (AHG19) [TK Tan, V Baroncini, M Mrak, M Karczewicz, W Wan, J Wen]

Presentation deferred.

JCTVC-P0020 JCT-VC AHG Report: Multi-layer hypothetical reference decoder (AHG20) [K. Suehring (chair), S. Deshpande, M. M. Hannuksela, J. Kang, A. K. Ramasubramonian, A. Tabatabai (vice-chairs)]

This is summary AHG20 report on the work of the JCT-VC AHG on Multi-layer hypothetical reference decoder (AHG20) between the 15th JCT-VC meeting in Geneva and 16th JCT-VC meeting in San Jose, with the list of related input documents.

Four contribution was noted as related (P0138, P0069, P0156, P0192). Three of these were on DPB, and the fourth (P0138) was regarding CL-RAS pictures.

The AHG recommended to not continue to have such an AHG following the current meeting. However, it was remarked that further work in this area was probably still needed.

# Project development, status, and guidance (29)

## Communication to and by parent bodies (0)

## Conformance test set development (2)

JCTVC-P0085 Conformance testing [C. Fogg (Harmonic), A. Wells, D. Le Gall (Ambarella)]

JCTVC-P0276 Editor's proposed draft text of HEVC conformance testing [T. Suzuki, W. Wan, G. Sullivan] [late] [miss]

## Version 1 bug reports and cleanup (2)

JCTVC-P0064 Defect correction for HEVC version 1 (esp. modulo CPB removal delay wrapping) [G. J. Sullivan, L. Zhu (Microsoft), Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

JCTVC-P0223 JPNB comment on HEVC defect report [Japan National Body] [late]

## HEVC coding performance, implementation demonstrations and design analysis (10)

### Version 1 verification test (2)

JCTVC-P0042 Source content for HEVC verification testing [J. Boyce (Vidyo)]

JCTVC-P0054 Updates to HEVC verification test plan draft 2 [TK Tan, Vittorio Baroncini, Marta Mrak]

### RExt performance and design aspects (3)

JCTVC-P0200AhG8: Performance and complexity analysis of Range Extensions tools [J. Sole, T. Hsieh, C. Pang, M. Karczewicz (Qualcomm)]

JCTVC-P0201 AhG8: Performance of encoder and parameter only changes for Screen Content Coding [J. Sole, C. Pang, L. Zhang, M. Karczewicz (Qualcomm)]

JCTVC-P0213 Comparison of Compression Performance of HEVC Range Extensions Draft 5 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

### SHVC performance and design aspects (1)

JCTVC-P0169 Update of JCTVC-O0115 on a pipeline and parallel architecture of an SHVC decoder [W. Hamidouche, M. Raulet (IETR/INSA)]

### Interlace (3)

JCTVC-P0082 How to use pic_struct [C. Fogg, O. Bar-Nir, P. Haskell (Harmonic), A. Wells, D. Le Gall (Ambarella)]

JCTVC-P0083 pic_struct coding results [C. Fogg (Harmonic)]

JCTVC-P0261 Extension of the pic_struct element in HEVC [Alexandros Tourapis, David Singer, Krasimir Kolarov, Steve Saunders] [late]

### Implementation demonstrations (1)

JCTVC-P0158 Informal Subjective Video Quality Comparison Between the eBrisk-UHD HEVC and x264 AVC Encoders [M. Horowitz, F. Kossentini, H. Tmar (eBrisk)

## Profile and level definitions (requirements related) (11)

### RExt profiles and levels (7)

JCTVC-P0057 AHG 5: Profiles for Range Extensions [K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony)]

JCTVC-P0058 AHG 5 and 18: Request for 16 bit Profiles for Range Extensions [K. Sharman, N. Saunders, J. Gamei, T. Suzuki, A. Tabatabai (Sony)]

JCTVC-P0086 A proposal on level specification of Main 4:2:2 profiles [A. Minezawa, K. Miyazawa, S. Sekiguchi, H. Sakate (Mitsubishi)]

JCTVC-P0099 AHG5: Objective and subjective evaluations of cross-component decorrelation for range extensions profile [K. Kawamura, S. Naito (KDDI)]

JCTVC-P0106 AHG5: Recommended profiling of range extension coding tools [S. Lee, E. Alshina, C. Kim (Samsung), K. McCann (Zetacast)] [late]

JCTVC-P0107 AHG5: Tools for lossless coding of medical image/video content in RExt profiles [P. Amon, P. Wojaczek, A. Hutter, U.-E. Martin, N. Wirsz (Siemens)]

JCTVC-P0224 Request for an HEVC 4:4:4 8 bit profile [Gaelle Martin-Cocher, Marta Mrak, Patrice Onno, Chris Rosewarne, Arild Fuldseth, Rickard Sjöberg, Alberto Duenas, Marta Karczewicz, Andrew Segall] [late]

### SHVC profiles and levels (4)

JCTVC-P0127 AHG14: On a CGS profile for SHVC [P. Andrivon, P. Bordes, E. François (Technicolor), X. Ducloux (Thomson Video Networks), Y. Ye (Interdigital), A. Duenas (NGCodec), A. Segall (Sharp Labs)]

JCTVC-P0134 Strawman SHVC level constraints [J. Boyce (Vidyo)]

JCTVC-P0142 On SHVC level limits [M. M. Hannuksela (Nokia)]

JCTVC-P0203 Hybrid codec scalability profile in SHVC [J. Samuelsson, J. Enhorn, R. Sjöberg (Ericsson)]

See also section 6.4.3.

## HEVC, SHVC and RExt use cases (requirements related) (4)

JCTVC-P0159 High Dynamic Range video coding [S. Lasserre, F. Le Leannec, E. François (Technicolor)] [late]

JCTVC-P0080 HDR coding results for FruitStall and Oblivion [B. Mandel (Universal), C. Fogg (Harmonic), J.Helman (Movielabs)]

JCTVC-P0162 On 16-bit coding [A. Aminlou, K. Ugur (Nokia)] [late]

JCTVC-P0167 On Hybrid use cases for SHVC [Alberto Duenas (NGCodec)] [miss]

## Source video test material (2)

JCTVC-P0228 Description of HDR sequences proposed by Technicolor [S. Lasserre, F. Le Leannec, E. François (Technicolor)] [late]

JCTVC-P0229 Quantitative quality evaluation of images for HDR and WCG coding [S. Lasserre, F. Le Leannec, E. François (Technicolor)] [late]

# Core experiment in SHVC (6)

## SCE1: Colour gamut and bit depth scalability (7)

### SCE1 summary and general discussion

JCTVC-P0031 SCE1: Summary Report of Colour Gamut and Bit Depth Scalability [P.Bordes, Y.Ye, E.Alshina, X.Li, S.H.Kim, A.Duenas, K.Ugur, K.Sato]

Two test cases:

| Test A | AI, RA with SHM4.0 | AI, RA with SHM4.0_irap |
| --- | --- | --- |
| Test B | AI, RA with SHM4.0 | AI, RA with SHM4.0_irap |

Two use cases (UC1 and UC2) have been defined:

- Use case 1: LUT derived from the first picture of the sequence. In this case, we will use the regular SHM4.0 software with one single SPS, PPS at the beginning.

- Use case 2: LUT derived using one or several pictures of the previous RAP period. In this case, the modified software SHM4.0_irap will be used, with regular SPS, PPS insertion.

Two methods:

JCTVC-P0128 = “Test 1”

JCTVC-P0186 = “Test 2”

Results:

Use case 1, Test 1

The detailed experiment results are described in JCTVC-P0128. The cross-checking has been provided by Qualcomm (JCTVC-P0143) and Sony (JCTVC-P0234).

Results of use case 1, tests 1.A (second column) and 1.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0.9% | -4.1% | -3.3% | -2.2% | -5.1% | -4.1% |
| Overall (Test vs single layer) | 17.7% | 17.8% | 14.6% | 18.8% | 18.4% | 14.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -2.3% | -5.6% | -4.2% | -4.7% | -7.5% | -6.1% |
| Overall (Test EL+BL vs single EL+BL) | -21.5% | -21.8% | -24.0% | -21.0% | -21.6% | -24.1% |
| Enc Time[%] | 107.9% |  |  | 108.4% |  |  |
| Dec Time[%] | 101.7% |  |  | 102.5% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -2.6% | -4.2% | -5.8% | -3.3% | -4.6% | -5.8% |
| Overall (Test vs single layer) | 24.1% | 28.2% | 19.3% | 24.9% | 29.0% | 20.3% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -4.8% | -6.2% | -7.7% | -6.0% | -7.1% | -8.2% |
| Overall (Test EL+BL vs single EL+BL) | -16.6% | -13.4% | -19.3% | -16.4% | -13.2% | -19.0% |
| Enc Time[%] | 70.7% |  |  | 70.3% |  |  |
| Dec Time[%] | 105.4% |  |  | 102.9% |  |  |

Use case 1, Test 2

The detailed experiment results are described in JCTVC-P0186. The cross-checking has been provided by Qualcomm (JCTVC-P0144).

Results of use case 1, tests 2.A (second column) and 2.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -2.9% | -5.5% | -5.0% | -3.4% | -5.8% | -5.1% |
| Overall (Test vs single layer) | 15.3% | 16.1% | 12.7% | 17.3% | 17.5% | 13.9% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -6.0% | -8.5% | -7.4% | -6.9% | -9.2% | -7.9% |
| Overall (Test EL+BL vs single EL+BL) | -23.2% | -22.9% | -25.3% | -22.0% | -22.1% | -24.8% |
| Enc Time[%] | 99.9% |  |  | 79.2% |  |  |
| Dec Time[%] | 100.7% |  |  | 101.1% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -3.8% | -5.1% | -7.3% | -4.0% | -5.2% | -6.9% |
| Overall (Test vs single layer) | 22.6% | 27.1% | 17.4% | 24.0% | 28.4% | 18.9% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.8% |
| EL only (Test vs Ref) | -6.9% | -7.9% | -10.0% | -7.2% | -8.1% | -9.7% |
| Overall (Test EL+BL vs single EL+BL) | -17.7% | -14.0% | -20.7% | -17.0% | -13.5% | -20.0% |
| Enc Time[%] | 69.7% |  |  | 57.7% |  |  |
| Dec Time[%] | 107.1% |  |  | 107.4% |  |  |

Use case 2, Test 1

The detailed experiment results are described in JCTVC-P0128. The cross-checking has been provided by Qualcomm (JCTVC-P0143).

Results of use case 2, tests 1.A (second column) and 1.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -5.6% | -7.8% | -9.9% | -5.9% | -8.0% | -10.0% |
| Overall (Test vs single layer) | 11.9% | 13.1% | 6.7% | 14.0% | 14.7% | 7.8% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -11.5% | -13.3% | -15.4% | -12.0% | -13.7% | -15.7% |
| Overall (Test EL+BL vs single EL+BL) | -26.3% | -25.7% | -30.4% | -24.9% | -24.7% | -29.9% |
| Enc Time[%] | 96.3% |  |  | 96.7% |  |  |
| Dec Time[%] | 96.2% |  |  | 97.8% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -5.1% | -6.1% | -9.7% | -5.1% | -6.1% | -9.3% |
| Overall (Test vs single layer) | 20.9% | 25.7% | 14.4% | 22.5% | 27.1% | 15.9% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.9% |
| EL only (Test vs Ref) | -9.6% | -10.2% | -13.9% | -9.7% | -10.2% | -13.5% |
| Overall (Test EL+BL vs single EL+BL) | -19.3% | -15.5% | -23.4% | -18.4% | -14.8% | -22.6% |
| Enc Time[%] | 67.4% |  |  | 67.0% |  |  |
| Dec Time[%] | 104.7% |  |  | 107.7% |  |  |

Use case 2, Test 2

The detailed experiment results are described in JCTVC-P0186. The cross-checking has been provided by Qualcomm (JCTVC-P0144) and Samsung (JCTVC-P0248).

Results of use case 2, tests 2.A (second column) and 2.B (first column).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -7.8% | -8.9% | -11.6% | -7.8% | -8.9% | -11.5% |
| Overall (Test vs single layer) | 9.2% | 11.8% | 4.7% | 11.8% | 13.6% | 6.1% |
| Overall (Ref vs single layer) | 18.5% | 22.8% | 18.1% | 21.2% | 24.7% | 19.4% |
| EL only (Test vs Ref) | -15.2% | -15.9% | -18.6% | -15.2% | -15.9% | -18.4% |
| Overall (Test EL+BL vs single EL+BL) | -28.3% | -26.5% | -31.9% | -26.6% | -25.4% | -31.1% |
| Enc Time[%] | 90.9% |  |  | 93.0% |  |  |
| Dec Time[%] | 98.3% |  |  | 98.6% |  |  |

|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ |  |  |  |  |  |  |
| Overall (Test vs Ref) | -6.3% | -6.5% | -11.0% | -6.2% | -6.5% | -10.6% |
| Overall (Test vs single layer) | 19.3% | 25.1% | 12.7% | 21.2% | 26.6% | 14.3% |
| Overall (Ref vs single layer) | 27.5% | 33.7% | 26.9% | 29.2% | 35.2% | 27.9% |
| EL only (Test vs Ref) | -11.7% | -11.4% | -16.0% | -11.5% | -11.4% | -15.4% |
| Overall (Test EL+BL vs single EL+BL) | -20.5% | -15.7% | -24.6% | -19.4% | -15.0% | -23.8% |
| Enc Time[%] | 64.0% |  |  | 65.1% |  |  |
| Dec Time[%] | 106.1% |  |  | 106.3% |  |  |

Discussion:

Was the WP using same approach of optimization in use case 1 and 2? No, optimized per picture, “as is” in reference software by LMS.

If inter-layer texture prediction is used, WP is always used. Hypothetically, with two inter-layer references, it could be switched on or off.

It was pointed out during the discussion that in use case 1 (offline coding) optimization could also be done using several pictures.

It was also pointed out by one experts that the current software used in SCE1 crashes in LD configuration under Windows, not Linux (needs further investigation).

The LUT table methods use 9x9x9x3 table entries

The amount of side information is 6000 bits on average, but it is sequence dependent. If no octree split occurs (as may be the case for less colorful sequences), the amount is much lower (it is mentioned that from the results of the last meeting, the number of bits varied between approx. 2000 and 9000).

The LUT methods require 4 multiplications per sample per component, whereas WP requires 1 mult per sample per component. Furthermore, some more logic is required at the pixel level to determine the table entry to be used.

However, LUT operations are applied before upsampling.

Overall Conclusion:

- Several experts expressed concerns that the additional complexity (table size, inter component dependency) is undesirable

- Continue CE: More thorough complexty analysis; analysis of the impact of encoder optimization; impact of table size; more test material (?)

(revisit in context of non-CE, establish BoG to discuss the items of the continuing CE, e.g. complexity analysis)

### SCE1 primary contributions (2)

JCTVC-P0128 SCE1: Results on Core Experiment on Color Gamut and Bit-Depth Scalability, tests 1A & 1B [P.Bordes, P.Andrivon, E.Francois (Technicolor)]

This contribution reports the performance analysis of SCE1 on Color Gamut and Bit-Depth Scalability, based on the use of 3D color Look-Up Tables (CLUT) to perform inter-layer prediction. Results of tests 1.A and 1.B for use cases 1 and 2, described in SCE1 description (JCTVC-O1101) are provided.

Considering use case 1 (one single SPS, PPS inserted at the sequence start), it is reported that compared with the SCE1 anchor (SHM4.0 with Weighted-Prediction (WP) enabled on inter-layer prediction), the CLUT method achieves an average BD rate gain of {-2.2%, -5.1%, -4.1%} for Y, U, V in AI, and {-3.3%, -4.6%, -5.8%} for Y, U, V in RA using 8-bit base layer and 10-bit enhancement layer, and an average BD rate gain of  {-0.9%, -4.1%, -3.3%} for Y, U, V in AI, and {-2.6%, -4.2%, -5.8%} for Y, U, V in RA using 10-bit base layer and 10-bit enhancement layer.

Considering use case 2 (one SPS, PPS inserted at RAP periodicity of one second), it is reported that compared with the SCE1 anchor (SHM4.0 with Weighted-Prediction (WP) enabled on inter-layer prediction), the CLUT method achieves an average BD rate gain of {-5.9%, -8.0%, -10.0%} for Y, U, V in AI, and {-5.1%, -6.1%, -9.3%} for Y, U, V in RA using 8-bit base layer and 10-bit enhancement layer, and an average BD rate gain of {-5.6%, -7.8%, -9.9%} for Y, U, V in AI, and {-5.1%, -6.1%, -9.7%} for Y, U, V in RA using 10-bit base layer and 10-bit enhancement layer.

It is reported the complexity of the proposed method compared to anchors is slightly lower.

Complexity analysis is presented, assuming that LUT and upsampling are applied on the fly such that no additional memory is required. It is also reported that the number of multiplications and additions is reduced compared to WP anchors. However, some doubt is raised that more systematic analysis would be required, in particular considering the irregularity of LUT operations. Furthermore, due to the need to access all three color components, it is likely more complex in terms of memory access than WP.

Further, the assessment that worst case complexity is equivalent to WP is not fully correct, as WP could be used in the enhancement layer anyway (unless explicitly disabled in a scalable profile), and the LUT operations are additionally necessary in the inter-layer processing stage.

JCTVC-P0186 SCE1: Combined bit-depth and color gamut conversion with 3D LUT for SHVC color gamut scalability [Y. He, Y. Ye, J. Dong (InterDigital)]

This proposal tested the combined bit-depth and color gamut conversion method with online 3D LUT derivation for SHVC color gamut scalability (CGS) proposed in JCTVC-O0161 with SCE1 test conditions. Two usecases with two tests are considered. For usecase 1 test, compared to SCE1 anchors, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-3.1%, -5.6%, -5.0%} and {-3.9%, -5.1%, -7.1%} for AI and RA-2x, respectively. For usecase 2 test, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-7.8%, -8.9%, -11.5%}, and {-6.2%, -6.5%, -10.8%} for AI and RA-2x, respectively.

Differences between P0128 and P0186 (P0186 performs better in both use cases):

- Main reason for performance difference is the parameter estimation, which is more complex in P0186.

Two normative differences

- P0186 does 8-10 bit conversion before upsampling

- P0186 uses additional filtering for alignment of luma with chroma samples

P0186 achieves better performance by higher complexity (both encoder and decoder)

### SCE1 cross checks (4)

JCTVC-P0143 SCE1: Crosscheck report of SCE1.1 on Color Gamut and Bit-Depth Scalability (JCTVC-P0128) [X. Li (Qualcomm)] [late]

JCTVC-P0144 SCE1: Crosscheck report of SCE1.2 [X. Li (Qualcomm)] [late]

JCTVC-P0234 SCE1: Crosscheck Result of Use Case 1: Test 1.A & Test 1.B [K Sato (Sony)] [late]

JCTVC-P0248 SCE1: Crosscheck report of SCE1 test 2 (JCTVC-P0186) [A. Alshin, E. Alshina (Samsung)] [late]

# Core experiments in Range Extensions (44)

## RCE1: High bit rate coding at high bit depths (11)

### RCE1 summary and general discussion

JCTVC-P0032 RCE1: Summary report on HEVC Range Extensions Core Experiment 1 (RCE1) on high bit rate coding at high bit depths [R. Joshi, K. Sharman (CE coordinators)] [miss]

### RCE1 primary contributions (7)

JCTVC-P0060 RCE1: Results for tests B1, B2 and B3a [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-P0073 RCE1: Results for subtest A [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

JCTVC-P0074 RCE1: Results for subtests B5, B6, B7 [R. Joshi, J. Sole, M. Karczewicz (Qualcomm)]

### RCE1 cross checks (6)

JCTVC-P0077 RCE1: A crosscheck report on subtest B.1 [R. Joshi (Qualcomm)] [miss]

JCTVC-P0194 RCE1: Cross-verification of subtest A.2 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-P0195 RCE1: Cross-verification of subtest B.6 [S. Lee, C. Kim (Samsung)] [miss]

JCTVC-P0220 Cross-check report of 'RCE1: Results for subtest A' test A4 (JCTVC-P0073) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-P0221 Cross-check report of 'RCE1: Results for subtests B5, B6, B7' test B5 (JCTVC-P0074) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-P0245 RCE1: Cross-verification of subtest B.7 [S.-H. Kim, A. Segall (Sharp)] [late] [miss]

JCTVC-P0247 RCE1: Cross-check of P0073 RCE1 Subtest A.1 [J. Ye, S. Liu (MediaTek)] [late] [miss]

## RCE2: Rice parameter initialization (6)

### RCE2 summary and general discussion

JCTVC-P0033 RCE2: Summary report on HEVC Range Extensions Core Experiment 2 (RCE2) on Rice parameter initialization [C. Rosewarne, M. Karczewicz, K. Sharman, S.-H. Kim]

### RCE2 primary contributions (2)

JCTVC-P0168 RCE2: Test A2. Rice parameter initialization based on quantization parameter and bit-depth [S.-H. Kim, K. Misra, A. Segall (Sharp)]

JCTVC-P0199 RCE2: Results of Test 1 on Rice Parameter Initialization [M. Karczewicz, L. Guo, J. Sole (Qualcomm), K. Sharman, N. Saunders, J. Gamei (Sony)]

### RCE2 cross checks (3)

JCTVC-P0210 RCE2: Cross-check of test A2 (JCTVC-P0168) [L. Guo (Qualcomm)]

JCTVC-P0219 Cross-check report of ' RCE2: Results of Test 1 on Rice Parameter Initialization' (JCTVC-P0199) by Qualcomm and Sony [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-P0236 RCE2: Verification for Test 1 on Rice Parameter Initialization [E. Alshina (Samsung)] [late]

## RCE3: Intra block copy refinement (20)

### RCE3 summary and general discussion

JCTVC-P0034 RCE3: Summary report on HEVC Range Extensions Core Experiment 3 (RCE3) on Intra block copy refinement [J. Sole, E. Alshina, D.-K. Kwon, W.-H. Peng] [miss]

### RCE3 primary contributions (9)

JCTVC-P0053 RCE3: Subtest C.4 - Padding-based generation of unavailable samples in intra block copy [S. Lee, E. Alshina, C. Kim (Samsung)]

JCTVC-P0055 RCE3: Subtest B.4 - Sample masking for intra block copy [J. Lainema, K. Ugur (Nokia)]

JCTVC-P0145 RCE3: Subtest B.3 - Intra block copy with NxN PU [C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-P0146 RCE3: Subtest C.2 - Intra block copy with CU-based padding [C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-P0147 RCE3: Subtest C.3 - Intra block copy with CTU-based padding [C. Pang, J. Sole, L. Guo, M. Karczewicz (Qualcomm)]

JCTVC-P0176 RCE3: Results of Subtest B.1 on Nx2N/2NxN Intra Block Copy [T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)]

JCTVC-P0180 RCE3: Results of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy [T.-S. Chang, R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)]

JCTVC-P0189 RCE3: Results of Subtest D.1 on Combining Nx2N/2NxN Intra Block Copy with TU process [C.-C. Chen, R.-L. Liao, T.-S. Chang, W.-H. Peng, H.-M. Hang, C.-L. Lin, F.-D. Jou (NCTU/ITRI)] [late]

JCTVC-P0211 RCE3: intra block copy search range (tests A) [E.Alshina, A.Alshin (Samsung), C. Pang, J. Sole, M. Karczewicz (Qualcomm)]

### RCE3 cross checks (10)

JCTVC-P0088 RCE3: Cross-check on Subtest C.4 - Padding horizontally or vertically [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-P0105 RCE3: Cross-verification of subtest C.2 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-P0148 RCE3: Crosscheck report of RCE3 Subtest C.1 [C. Pang (Qualcomm)] [late]

JCTVC-P0185 RCE3: Crosscheck report of B.4 on sample masking [J. Xu (Sony)] [late]

JCTVC-P0188 RCE3: Cross-check report of Subtest B.3 - Intra Block Copy with NxN PU (JCTVC-P0145) [R.-L. Liao, T.-S. Chang, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU), C.-L. Lin, F.-D. Jou (ITRI)]

JCTVC-P0191.RCE3: Cross-check report of Subtest B.1 on Nx2N/2NxN Intra Block Copy (JCTVC-P0176) [X. Xu, S. Liu (MediaTek)] [miss]

JCTVC-P0193 RCE3: Cross-check report of Subtest D.2 on Nx2N/2NxN/NxN Intra Block Copy [X. Xu, S. Liu (MediaTek)] [miss]

JCTVC-P0196 RCE3: Cross-verification of subtest D.1 [S. Lee, C. Kim (Samsung)] [late]

JCTVC-P0232 Cross-check report of 'RCE3: Subtest C.3 - Intra block copy with CTU-based padding' (JCTVC-P0147) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late] [miss]

JCTVC-P0233 Cross-check report of 'RCE3: intra block copy search range (tests A)' test A5 (JCTVC-P0211) by Qualcomm and Samsung [C. Rosewarne, M. Maeda (Canon)] [late]

## RCE4: Palette mode coding (6)

### RCE4 summary and general discussion

JCTVC-P0035 RCE4: Summary report of HEVC Range Extensions Core Experiments 4 (RCE4) on palette coding for screen content [X. Guo, A. Saxena (CE coordinators)]

### RCE4 primary contributions (7)

JCTVC-P0108 RCE4: Test1. Major-color-based screen content coding [X. Guo, Y. Lu, S. Li (Microsoft)]

JCTVC-P0198 RCE4: Results of Test 2 on Palette Mode for Screen Content Coding [L. Guo, W. Pu, M. Karczewicz, J. Sole, R. Joshi, F. Zou (Qualcomm)]

### RCE4 cross checks (4)

JCTVC-P0104 RCE4: Cross-check on Test 2 - Palette Mode Coding [C. Park, C. Kim (Samsung)] [late]

JCTVC-P0225 Crosscheck of RCE4: Results of Test 2 on Palette Mode for Screen Content Coding [Z. Ma, H. Yu (Huawei)] [late]

JCTVC-P0226 Crosscheck of RCE4 test 1 [Z. Ma, H. Yu (Huawei)] [late]

JCTVC-P0271 RCE4: Cross-check on Test 1 - Major-color-based screen content coding [W. Pu, J. Sole (Qualcomm)] [late] [miss]

# Non-CE Technical Contributions (159)

## Range extensions (83)

### General (1)

JCTVC-P0166 Some Syntax Modifications for HEVC Range Extension [Y. Yu, L. Wang Arris)]

### RCE1 related (high bit rate coding and high bit depth) (1)

JCTVC-P0075 non-[RCE1, RCE2]: Combination of RCE1 subtests B5 and B6 with RCE2 subtest A1 [R. Joshi, L. Guo, J. Sole, M. Karczewicz (Qualcomm)]

### Implementation aspects of high bit rate and high bit depth (6)

JCTVC-P0044 On MinCR [T. Suzuki (Sony)]

JCTVC-P0061 AHG18: Worst-case Escape Code Length Mitigation [K. Sharman, N. Saunders, J. Gamei (Sony)]

JCTVC-P0066 AHG5: Luma-chroma prediction for different bitdepth [O. Nakagami, T. Suzuki (Sony)]

Relates to cross-component prediction.

JCTVC-P0100 AHG5: cross-check results of luma-chroma prediction for different bitdepth (JCTVC-P0066) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-P0173 AHG 5 and 18: Coding of high bit-depth source with lower bit-depth encoders and a continuity mapping [C. Auyeung, J. Xu (Sony)]

JCTVC-P0222 AhG 18: On SAO performance at high bit-depth and high bit-rates [E. Alshina, A. Tourapis, W.-S. Kim, Y.-W.Huang] [late]

### RCE2 related (Rice parameter initialization) (0)

### RCE3 related (intra block copy refinement) (26)

JCTVC-P0087 Non-RCE3: On padding/unavailable samples pre-set for intra block copy [A. Minezawa, K. Miyazawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

JCTVC-P0089 Non-RCE3 : On unavailable sample preset for intra block copy [C. Park, S. Lee, C. Kim (Samsung)]

JCTVC-P0102 Non-RCE3: On displacement vector coding for intra block copy [C. Park, S. Lee, C. Kim (Samsung)]

JCTVC-P0111 Non-RCE3: Motion predictor initialization for Intra Block Copy [G. Laroche, P. Onno, C. Gisquet, T. Poirier (Canon)]

JCTVC-P0149 Non-RCE3: Block vector signaling for intra block copy [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-P0250 Crosscheck of JCTVC-P0149 [J. Xu (Sony)] [late] [miss]

JCTVC-P0150 Non-RCE3: Block vector predictor initialization for intra block copy [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-P0151 Non-RCE3: Fast encoder search for RCE3 Subtest B.3 [C. Pang, J. Sole, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-P0279 Non-RCE3: Cross-check of fast encoder search for RCE3 Subtest B.3 (JCTVC-P0151) [Christophe Gisquet (Canon)] [late] [miss]

JCTVC-P0170 Non-RCE3: simplified difference vector coding for Intra block copy [S.-H. Kim, K. Misra, A. Segall (Sharp)] [miss]

JCTVC-P0178 Non-RCE3: Bv coding with default predictor [J. Xu, A. Tabatabai, O. Nakagami (Sony)]

JCTVC-P0273 Crosscheck report of JCTVC-P0178 on Bv coding with default predictor [C. Pang (Qualcomm)] [late] [miss]

JCTVC-P0190 Non-RCE3: On padding of overlapped area for IntraBC [K. Zhang, X. Xu, J. An, X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-P0202 Non-RCE3: On Intra block copy [C. Rosewarne, M. Maeda (Canon)]

JCTVC-P0212 RCE3: preset of un-available for intra block copy samples [E.Alshina, A.Alshin (Samsung)]

JCTVC-P0205 Intra block copy syntax clean up [X. Xu, K. Zhang, X. Zhang, S. Liu, S. Lei (MediaTek)]

JCTVC-P0230 Crosscheck of JCTVC-P0205: Intra block copy syntax clean up [Z. Ma (Huawei)] [late]

JCTVC-P0217 Initialization of block vector predictor for intra block copy [L. Zhu, J. Xu, G. J. Sullivan, S. Sankuratri, B. A. Kumar, F. Wu (Microsoft)]

JCTVC-P0275 Crosscheck report of JCTVC-P0217 (method 3) on initialization of block vector predictor for intra block copy [C. Pang (Qualcomm)] [late] [miss]

JCTVC-P0218 Non-RCE3 subtest B.2: Search Methods for Intra block copying for CU-level block vectors with TU-level prediction processing [L. Zhu, J. Xu, G. J. Sullivan (Microsoft)]

JCTVC-P0237 Non-RCE3: Major Color Padding Algorithm for Intra Block Copy [C.-H. Hung, E.-C. Ke, H.-T. Chiao (ITRI), W.-H. Peng (NCTU)] [late]

JCTVC-P0254 Crosscheck of JCTVC-P0237 on major color padding algorithm for intra block copy [??(??)] [late] [miss]

JCTVC-P0238 Non-RCE3: Padding schemes for intra block copy [Y.-J. Chang, H.-L. Tsai, C.-L. Lin, F.-D. Jou (ITRI), C.-C. Chen, R.-L. Liao, W.-H. Peng (NCTU)] [late]

JCTVC-P0255 Non-RCE3: Horizontal/vertical padding scheme for intra block copy [Yao-Jen Chang, Hua-Lung Tsai, Chun-Lung Lin, Fan-Di Jou (ITRI), Chun-Chi Chen, Ru-Ling Liao, Wen-Hsiao Peng, Hsueh-Ming Hang (NCTU)] [late]

JCTVC-P0272 Non-RCE3: A cross-check report for JCTVC-P0255 [A. Saxena, F. Fernandes (Samsung)] [late]

JCTVC-P0256 Non-RCE3: Vertical padding schemes for intra block copy [Yao-Jen Chang, Hua-Lung Tsai, Chun-Lung Lin, Fan-Di Jou (ITRI), Chun-Chi Chen, Ru-Ling Liao, Wen-Hsiao Peng, Hsueh-Ming Hang (NCTU)] [late]

### RCE4 related (palette mode) (42)

JCTVC-P0090 Non-RCE4: Advanced copy mode for palette coding [Erh-Chung Ke, Jih-Sheng Tu, Chao-Hsiung Hung (ITRI)]

JCTVC-P0251 Crosscheck of JCTVC-P0090 on advanced copy mode for palette coding [Y.-C. Sun, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-P0091 Non-RCE4 : On palette update for palette coding [C. Park, S. Lee, C. Kim (Samsung)]

JCTVC-P0093 Non-RCE4: Cross-CU major color index prediction [Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0265 Non-RCE4: Cross-check report for JCTVC-P0093 [X. Guo (Microsoft)] [late] [miss]

JCTVC-P0094 Non-RCE4: Inter-component major color table sharing [Y.-C. Sun, Y.-W. Chen, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0266 Non-RCE4: Cross-check report for JCTVC-P0094 [X. Guo (Microsoft)] [late] [miss]

JCTVC-P0095 Non-RCE4: Removal of syntax redundancy in RCE4 Test2 [Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0096 Non-RCE4: Major color table propagation through non-palette CUs [T.-D. Chuang, P. Lai, Y.-C. Sun, Y.-W. Chen, Y.-W. Huang, S. Liu, S. Lei (MediaTek)]

JCTVC-P0243 Crosscheck of JCTVC-P0096: Non-RCE4: Major color table propagation through non-palette CUs [F. Zou(Qualcomm)] [late] [miss]

JCTVC-P0267 Non-RCE4: Cross-check report for JCTVC-P0096 [X. Guo (Microsoft)] [late] [miss]

JCTVC-P0098 Non-RCE4: Four-neighbor major color index prediction [T.-D. Chuang, Y.-C. Sun, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0268 Non-RCE4: Cross-check report for JCTVC-P0098 [X. Guo (Microsoft)] [late] [miss]

JCTVC-P0101 Non-RCE4: Removal of syntax redundancy in RCE4 Test1 [T.-D. Chuang, Y.-C. Sun, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0269 Non-RCE4: Cross-check report for JCTVC-P0101 [X. Guo (Microsoft)] [late] [miss]

JCTVC-P0103 Non-RCE4: On run-length coding for palette coding [C. Park, S. Lee, C. Kim (Samsung)]

JCTVC-P0113 Non-RCE4: Run coding for palette mode [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-P0244 Non-RCE4: A cross-check report for JCTVC-P0113 [A. Saxena, F. Fernandes (Samsung)] [late] [miss]

JCTVC-P0114 Non-RCE4: Palette prediction for palette mode [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-P0239 Crosscheck of JCTVC-P0114: Non-RCE4: Palette Prediction for Palette mode [F. Zou (Qualcomm)] [late] [miss]

JCTVC-P0246 Non-RCE4: Cross-check of P0114 on Palette Prediction for Palette mode [P. Lai, S. Liu (MediaTek)] [late] [miss]

JCTVC-P0115 Non-RCE4: Transition copy mode for palette mode [C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-P0240 Crosscheck of JCTVC-P0115: Non-RCE4: Transition copy mode for Palette mode [F. Zou (Qualcomm)] [late] [miss]

JCTVC-P0257 Crosscheck of JCTVC-P0115: Non-RCE4: Transition copy mode for Palette mode [J. Xu (Sony)] [late] [miss]

JCTVC-P0116 Non-RCE4: On palette prediction modes coding [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-P0252 Crosscheck of JCTVC-P0116 on palette prediction modes coding [T.-D. Chuang, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-P0117 Non-RCE4: combined coding of run and index for RCE4 Test2 [C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-P0241 Crosscheck of JCTVC-P0117: Non-RCE4: combined coding of run and index for RCE4 Test2 [F. Zou (Qualcomm)] [late] [miss]

JCTVC-P0119 Non-RCE4: combination of improvements for Palette mode [G. Laroche, C. Gisquet, T. Poirier, P. Onno (Canon)]

JCTVC-P0242 Crosscheck of JCTVC-P0119: Non-RCE4: combination of improvements for Palette mode [F. Zou (Qualcomm)] [late] [miss]

JCTVC-P0152 Non-RCE4: Major color table (palette) merge from above and left CU [P. Lai, S. Liu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0153 Non-RCE4: Major color table (palette) sharing [P. Lai, S. Liu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-P0280 Non-RCE4: Cross-check of Major color table (palette) sharing (JCTVC-P0153) [Christophe Gisquet (Canon)] [late] [miss]

JCTVC-P0160 Non-RCE4: Palette prediction for palette coding [G. Jin, A. Saxena, F. Fernandes (Samsung)]

JCTVC-P0281 Non-RCE4: Cross-check of Palette prediction for palette coding (JCTVC-P0160) [Christophe Gisquet (Canon)] [late] [miss]

JCTVC-P0161 Non-RCE4: Simplification of major color based palette prediction [G. Jin, A. Saxena, C. Park, F. Fernandes (Samsung)]

JCTVC-P0270 Non-RCE4: Cross-check report for JCTVC-P0161 [X. Guo (Microsoft)] [late] [miss]

JCTVC-P0179 Non-RCE4: on palette coding mode in RExt [J. Xu, A. Tabatabai (Sony)]

JCTVC-P0274 Crosscheck report of JCTVC-P0179 on palette coding mode in RExt [C. Pang (Qualcomm)] [late] [miss]

JCTVC-P0231 Non-RCE4: Refinement of the palette in RCE4 Test 2 [W. Pu, F. Zou, J. Sole, M. Karczewicz, R. Joshi (Qualcomm)] [late]

JCTVC-P0249 Non-RCE4: A combination of the four-neighbor major color index prediction in JCTVC-P0098 and a simplified transition copy mode from JCTVC-P0115 on top of RCE4 Test1 [Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)] [late] [miss]

JCTVC-P0253 Non-RCE4: PU based Color Palette Coding [J. Ye, S. Liu, P. Lai, X. Xu, S. Lei (MediaTek)] [late]

### Lossless and screen content coding related contributions (3)

JCTVC-P0097 Extended Cross-Component Decorrelation for Animated Screen Content [A. Khairat, T. Nguyen, D. Marpe (Fraunhofer HHI)]

JCTVC-P0112 AHG8: Chroma interpolation filters for Lossless compression [G. Laroche, T. Poirier, P. Onno, C. Gisquet (Canon)]

JCTVC-P0214 Screen content coding using dictionary based mode [B. Li, J. Xu, F. Wu (Microsoft)]

JCTVC-P0277 Motion Vector Resolution Control for Screen Content Coding [Y. Zhou, B. Li, J. Xu, G. J. Sullivan, B. Lin (Microsoft)] [late]

### Other (4)

JCTVC-P0109 AHG8: Sample adaptive offset with multiple parameters [S.-T. Hsiang, S. Lei (MediaTek)]

JCTVC-P0278 AHG8: Cross-check report of Sample adaptive offset with multiple parameters (JCTVC-P0109) [Christophe Gisquet (Canon)] [late] [miss]

JCTVC-P0154 AHG5 and AHG8: Alpha parameter coding methods for inter-component residual prediction in HEVC range extension [X. Zhang, K. Zhang, J. An, S. Lei (MediaTek)]

- JCTVC-P0282 AHG5 and AHG8: Cross-check of parameter coding methods for inter-component residual prediction in JCTVC-P0154 [W.-S. Kim, W. Pu (Qualcomm)] [late]

## SHVC (16)

### General (2)

JCTVC-P0208 SHVC upsampling ratio constraint [K. Misra, A. Segall (Sharp)]

This contribution proposes a bitstream constraint on the upsampling ratio for SHVC. It is asserted that the current SHVC draft allows the ratio of dimensions of the reference layer picture and its scaled representation to be greater than 1.In such an event the SHVC decoder operation is not clear. The proposed bitstream constraint bounds this ratio to be less than or equal to 1

Revision1 of the document includes the proposed bitstream constraint language.

Discussion: There is no technical problem in current spec and software about supporting enhancement layer resolution that is lower than base layer. Therefore, there is no harm of allowing it.

No action.

JCTVC-P0209 On chroma format scalability using spatial scaling [K. Misra, S. Deshpande, A. Segall (Sharp)]

This contribution proposes enabling chroma format scalability within the existing SHVC design through the use of spatial scalability. It is asserted that the desired functionality can be enabled with the proposed text.

This is not relevant for Scalable Main Profile. It could become relevant for a later combination (e.g. with Main Profile as base layer and some RExt based decoder in the enhancement layer). However, it should be a simple exercise to re-write the re-sampling process in a way that it supports different ratios for luma and chroma.

Further consideration would only be reasonable once a concrete request for an application case is made, and after RExt is finalized.

### SCE1 related (colour gamut and bit depth scalability) (7)

JCTVC-P0063 Non-SCE1: Asymmetric 3D LUT for Color Gamut Scalability [X. Li, J. Chen, M. Karczewicz (Qualcomm)]

In this proposal, a method based on asymmetric 3D lookup table (up to 384 entries) is proposed for color gamut scalability. It is reported that on average 8.2% (AI-10bit), 8.2% (AI-8bit), 6.3% (RA-10bit) and 6.2% (RA-8bit) luma BD rate reduction was achieved over SCE-1 use case 1 anchor , and 8.4% (AI-10bit), 8.4% (AI-8bit), 6.6% (RA-10bit) and 6.4% (RA-8bit) luma BD rate reduction over SCE-1 use case 2 anchor. Note that the SCE-1 anchors employ weighted prediction to compensate color gamut difference between layers.

Lookup table with 8x2x2 partitions (instead of 9x9x9) – more partitions along Y direction.

Signalling in PPS, updating in slice header when necessary. (Note: table is only used in current slice)

Results in abstract are with picture level update; the contribution also provides results with use cases 1 and 2 of SCE1.

Applied after upsampling, therefore with 2x scalability decoding is more complex than SCE1 methods (P0197 is another proposal which applies this method before upsampling).

JCTVC-P0129 Non-SCE1: Cross-check report of Asymmetric 3D LUT for Color Gamut Scalability (JCTVC-P0063) [P.Bordes (Technicolor)] [late]

JCTVC-P0124 Non-SCE1: Color gamut scalability using modified weighted prediction [A. Aminlou, K. Ugur, M. M. Hannuksela (Nokia)]

SCE1 tests two tools utilizing look-up tables for increasing the coding efficiency of SHVC for color gamut scalability. This contribution proposes an alternative method that is based on modified weighted prediction process for improving the coding efficiency of color gamut scalability. The proposal makes three changes to HEVC weighted prediction so that it is more suitable for inter-layer color gamut mapping: Firstly, the YUV space is divided into an NYxNCbxNCr region and for each region different parameters are signaled. Secondly, WP utilizes a matrix based mapping to derive the prediction pixel values (the luminance value of the prediction pixel is calculated using luminance and chrominance values of the reference pixel). As a third modification, second order polynomial equations are used for matrix based mapping, instead of linear equations. Experimental results show that the proposed method improves the coding efficiency by 8.6% and 6.4% on average for AI and RA cases respectively. In addition, results for several variations and simplifications are also included in the contribution.

As an update to the contribution, full results for polynomial based matrix mapping are provided. In addition more details on encoder algorithm and the syntax are provided.

The title “modified weighted prediction” is misleading, as this is additional inter-layer processing (such as the LUT methods in SCE1) rather than modification of the WP in enhancement layer. Applied after upsampling.

Three elements: Divide YUV color space into NxNxN regions, use matrix mapping (introducing inter-component dependency), use second order polynomial to reduce the number of regions

Configurations (with results for AI):

N=8 with linear matrix (i.e. similar to 9x9x9 of SCE1), approx. 8% gain

N=1 with linear matrix (equivalent to WP, but inter-component dependency), approx. 3% gain

N=8 without matrix (i.e. piecewise linear), approx. 5% gain

N=1 with polynomial mapping), approx. 7% gain

Zero point of polynomial mapping is currently center value (e.g. 128); one expert points out that making this adaptive might further improve the performance (but also increase complexity).

Adaptation per RAP period

Signalling at slice header (this might be problematic in error prone environment).

JCTVC-P0227 Crosscheck report of JCTVC-P0124 on color gamut scalability using modified weighted prediction [K. Misra, A. Segall (Sharp)] [late] [miss]

JCTVC-P0197 Non-SCE1: improved color gamut scalability [Y.W. He, Y. Ye, J. Dong (InterDigital), X. Li, J. Chen, M. Karczewicz (Qualcomm)] [late]

This proposal tested two improvements based on asymmetric 3D LUT for SHVC color gamut scalability (CGS) proposed in JCTVC-P0063 under SCE1 core experiment test conditions. It can reduce the computation complexities. For usecase 2 test, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-8.3%, -10.0%, -12.9%}, and {6.0%, -6.9%, -10.5%} for AI and RA-2x, respectively.

Combines P0063 with elements of P0186 (8-10 bit conversion before upsampling, additional filtering, LUT before upsampling).

Moving LUT before upsampling increases bit rate 0.1%-0.2% for AI, 0.5% for RA.

Conclusion supported by proponents of SCE1 contributions: Usage of smaller lookup table is highly preferable.

Overall summary on SCE1 & P0063, P0124, P0197:

- Continue SCE1

- Only investigate 8x2x2 LUT configuration (P0063/P0197) in combination with entropy coding elements from P0128 and P0186

- Investigate P0124 configurations 2, 3, 4. To be discussed in BoG whether investigation of configuration 1 is also of benefit.

BoG (A. Duenas) to further discuss the setup of the CE (items to be investigated, test conditions) and the methodology for assessment of complexity. Revisit.

JCTVC-P0171 AHG14: Extension of SNR scalability with bit-depth scalability [C. Auyeung, O. Nakagami, K. Sato (Sony)]

In SHM WD4 JCTVC-O1008_v3, when the base layer and the enhancement layer have the same picture size and the scaled reference layer offsets are zero, video bit-depth scalability is not supported. This contribution proposes to enable SHVC to support bit-depth scalability when both the base layer and enhancement layer have the same picture size. One use case is the encoding of high dynamic range (HDR) video with color gamut scalability tools. In this use case of SHVC, the high dynamic range video with higher bit-depth is encoded in the enhancement layer and the corresponding low dynamic range video with lower bit-depth is encoded in the base layer, and both layers have the same picture size.

It is clarified during the discussion that the current spec does not prohibit different bit depth of base and enhancement layer in case of 1X (SNR) scalability, as formally the upsampling of the zero phase position is still expressed as multiplication which is rounded to the bit depth of the enhancement layer.

Further study in AHG on colour gamut and bit depth scalability.

JCTVC-P0235 Non-SCE1: Trade-off between coding efficiency and buffer size with the 3D-LUT-based method for Color Gamut Scalability [K Sato (Sony)] [late]

### Up-/downsampling process (3)

JCTVC-P0164 AHG13: chroma phase offset for SHVC resampling process [K. Rapaka, J. Chen, M. Karczewicz (Qualcomm)]

In this contribution, the coding performance impact of chroma sample position in SHVC resampling process are investigated. The test results shows that consideration of the actual chroma sample position in resampling process provides –0.1% to –0.4% Luma BD rate saving, compared to the current SHVC, which assumes position “b” of chroma sample in resampling process.

Additional test results are provided that show –0.1% to –1.0% Luma BD rate saving, compared to when using position “a” for chroma sample in resampling process

With the typical configurations (phases b and d), the loss is only 0.2-0.4% when different phases are used for chroma down and upsampling

It is unlikely that subjective differences would be visible (proponents report they did not find any difference subjectively)

Results only for AI, for RA bit rate difference would be almost unnoticeable

No action – retain the current “b” position for upsampling.

Remove mandate from AHG.

JCTVC-P0177 On handling re-sampling phase offsets with fixed filters [K. Minoo, D. Baylon, A. Luthra (ARRIS)]

This contribution discusses an approach to signaling phase offsets to improve inter-layer prediction precision and hence the compression performance of SHEVC. The proposed method uses phase offset per phase index per direction and per color type to correct the upsampling behaviour. This information is signalled at PPS so alternative phase correction can be applied per slice or picture, which benefits the use case of upsampling from field to frame.

Contribution version as presented was not uploaded yet.

Problem of phase misalgnment is claimed to occur with some formats > 2048.

Results shown with People on Street 1.5X showing 0.3% bit rate reduction for all cases (AI, RA, LD, LDP).

Some doubts are raised whether the problem of rounding error for picture sizes >2048 exists

More evidence needed (more sequences e.g. from the RExt and CGS test sets) that the potential misalignment of phase is a problem in terms of compression.

JCTVC-P0215 Tile Based Resampling for SHVC [R. Skupin, K. Suehring, Y. Sanchez, T. Schierl (Fraunhofer HHI)]

When performing tile- and layer-parallel processing of an SHVC encoded video sequence, the resampling process can affect parallelization as it does not regard tile boundaries. Constraining the encoder by Inter-Layer Constrained Tile Sets impacts compression efficiency. An alternative tile based resampling is proposed that enables the same degree of parallelism with lesser impact on compression efficiency.

A first revision of the document adds additional results of the Inter-Layer Constrained Tile Sets and the proposed scheme against anchor using the same tile configuration.

It is reported that for 2x scalability AI and a 4x4 tile configuration the ILCTS results in a loss of 15.1%, and for 1.5X 27.4%. Several experts expressed that this loss is unreasonably high and may be due to a bug (or not optimum encoder implementation) in the reference software implementation of ILCTS, e.g. that inter-layer prediction is disabled for CTU at the tile boundary, not for CU/PU as it should be.

The proposal would require normative change in the upsampler, whereas ILCTS is just an encoder restriction. Further clarification should be made with the implementers of the ILCTS RS, and if possible further results should be provided what the actual gap is. Revisit.

Investigation with different numbers of tiles (e.g. 2x2 for HD) would also be recommended.

### Inter-layer information derivation (1)

JCTVC-P0049 AHG 13: Scale and reference position derivation for sub-region extraction [T. Yamamoto, T. Tsukuba, T. Ikai (Sharp), E. Alshina (Samsung)]

When bitstream is generated by extracting the sub region of the original picture, the reconstructed pixel values could be changed if scale and/or phase are not preserved. This contribution proposes 1) new syntax in SPS extension and 2) modified scale and reference pixel location derivation. The proposed changes help keeping the same scale and phase, and thus useful for the applications using sub-region extraction.

The method had been presented before as O0056, and further study had been performed in AHG13 (inter-layer filtering). During this, some more improvement of the method was achieved.

Only relevant for case where an ROI is extracted from both base and enhancement layer (where the enhancement layer ROI cannot cover a larger area than the base layer ROI).

In principle, the same could be achieved with the current scaled ref layer offset, but not guaranteed at any combination of sample position of base and enhancement layers (where the allowed positions have some restriction due to CTU boundaries anyway).

It is also mentioned that something similar can be achieved by the ILCTS SEI message, and it is unclear what the additional benefit of the proposed method is.

Questions are raised about the relevant use cases that require the high accuracy of region extraction. More information about this is requested.

### Field to frame scalability (3)

JCTVC-P0163 AHG15: Interlaced to progressive scalability for SHVC hybrid codec use case [Y. Ye, Y. He, Y.W. He (InterDigital)]

JCTVC-P0165 Interlaced to progressive scalability in SHVC [J. Chen, K. Rapaka, Y.-K. Wang, M. Karczewicz (Qualcomm)]

JCTVC-P0175 On field to frame scalability [K. Minoo, D. Baylon, A. Luthra (ARRIS)]

## HL syntax common issues for range extensions, 3D, SHVC, and single-layer HEVC coding (11)

### Auxiliary picture (6)

Discussed 01-09 pm (GJS).

JCTVC-P0065 Guided transcoding using auxiliary pictures [K. Andersson, Y. Rai, T. Rusert, R. Sjöberg, J. Samuelsson (Ericsson)]

At the Geneva meeting it was proposed in JCTVC-O0127 to enable guided transcoding using SHVC. This time it is proposed to enable guided transcoding for both spatial and SNR scalability using auxiliary pictures without normative low-level changes. With this approach the higher fidelity is represented in the base layer and the side information for generation of another resolution/fidelity is represented as an auxiliary picture. The results reportedly show gains compared to simulcast coding of 21.5%/26.2% for all intra, 15.1%/18.9%/13.2% for random access, and 13.9%/17.6%/11.7% for low delay 2x/1.5x/SNR scalability for the common conditions. In comparison to traditional transcoding (re-encoding to another resolution/fidelity), the proposed guided transcoding reportedly significantly reduces the transcoding time to be comparable with the decoding time and to have less loss in BDR performance compared to single-layer coding. The proposed modification to the specification is to add a new auxiliary picture type AUX_TRANSCODING and an informative note on how to use a primary and auxiliary picture to enable guided transcoding.

The auxiliary picture would be coded at the target resolution of the transcoding process. The syntax would be ordinary syntax but the transform coefficients are meant to be discarded.

It was asked how much gain could be obtained if the syntax were modified so that no coefficients are sent instead of sending "junk" coefficients. This was suggested to be perhaps 5-10% of the size of the auxiliary picture data (and perhaps 2-3% of the total data), but this was said to be only a rough guess.

The downsampling process would probably need to be known – and using relatively short filters were suggested to be used for this.

The transcoder could instead just do a full re-encode with R-D search instead of using the guidance provided in the auxiliary picture. But if the encoder generates a reference picture using some method different from what was anticipated in the semantics, there would be drift which could make subsequent data substantially less relevant.

It was remarked that potentially just having a proprietary user-defined type of auxiliary picture (for which signalling is already anticipated) might be a way to enable this.

Some generalization of filter description, such as a filter description SEI message, was a suggested way to handle the downsampling filter. However, it was asked whether transcoders would really want to support a general family of filtering or would only want to implement a specific method – and perhaps just discard all data accompanied by an indication different than their favourite filter.

No specific filter description method was proposed.

Some experiment results relative to simulcast and relative to SHVC were provided. It was asked whether there was a really benefit relative to SHVC. The proponent said it was a matter of prioritization of whether the desire is having good fidelity for the enhanced resolution or good fidelity for the base layer. The prioritization of the layers with SHVC was also noted to be a relevant issue.

The relevance of the data in terms of potential drift relative to the original encoder would also depend on the requantization process (including QP and other aspects). If that process is fully specified to avoid this issue, it would involve output of only one specific bit rate as the output.

Feedback to the original encoder was suggested as a way to perform bit rate control; however, it was remarked that this again ends up having the usefulness of the feature depend on some (potentially undocumented and proprietary) external technology – so it was suggested that simply having the auxiliary picture indicated as a proprietary type of auxiliary picture might suffice.

It was agreed that, pending further input, no action seems (currently) justified to enable this other than defining an auxiliary picture type indicated as "unspecified" (already currently drafted as type codes between 128 and 143, inclusive).

Further study was encouraged. If further study results in new information at some point in the future to resolve the "gaps" in the concept, this could be reconsidered for action.

JCTVC-P0071 MV-HEVC/SHVC HLS: On auxiliary pictures [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)] [late]

Signalling the auxiliary picture types used for a CVS in VPS extension is proposed for session negotiation and sub-bitstream extraction.

It was commented that in the current VPS syntax, we have an "aux ID" that corresponds to an auxiliary picture type, and it was suggested that this may be sufficient to provide the intended functionality.

No action was taken on that aspect.

Additionally, for efficient layer-dependency signalling, layer dependency information, direct_dependency_flag[ i ][ j ] and direct_dependency_type[ i ][ j ], are proposed to be signalled by grouping a layer with primary pictures and layers with their associated auxiliary pictures, when they have the same layer dependencies.

It was remarked that the potential bit rate savings for this is minimal (no estimate was provided, but this seemed basically true). It was also commented that it may not be clear that the auxiliary pictures would have the same type of inter-layer dependency characteristics as the non-auxiliary pictures. The proponent suggested to consider the multiview case with depth map auxiliary pictures, wherein each view may be accompanied by a depth map and inter-layer referencing may have the same characteristics – and asserted that this is a common test case used in experiments.

It was noted that the proposal would retain the current type of dependency indication – this would add an additional type of dependency indication as an alternative rather than simplifying the syntax.

In the absence of an understanding that the number of bits saved would be significant, there seemed to be no interest from non-proponents, so no action was taken on this aspect either.

Furthermore, some constraints for slice header parameters of auxiliary pictures were proposed.

Proposed constraint 1:

- When present, the value of the slice segment header syntax elements pic_output_flag, no_output_of_prior_pics_flag, slice_pic_order_cnt_lsb, discardable_flag, cross_layer_bla_flag, poc_reset_flag, shall be the same in all slice segment headers of a primary coded picture and the associated auxiliary coded pictures.

It was remarked that one-way constraints may be more reasonable for some of these – establishing a constraint on the auxiliary as a function of what is happening in the primary.

It was also asked to consider each specific constraint – for each specific syntax element, and think about its reason for being constrained. Each seems to have its own characteristics, and detailed reasoning was not provided in the contribution.

Revisit after offline study for this.

Proposed constraint 2:

- For all slices of auxiliary pictures, slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0; or

- When AuxId[ lId ] is equal to AUX_DEPTH, slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0

It was remarked that if such a constraint is needed, it should be part of a profile constraint and specific to particular auxiliary picture types.

As applied to depth maps, this should be considered a 3V issue.

Proposed constraint 3:

- For all slices of auxiliary pictures, slice_deblocking_filter_disabled_flag shall be equal to 0

- When AuxId[ lId ] is equal to AUX_DEPTH, slice_deblocking_filter_disabled_flag  shall be equal to 0

It was remarked that if such a constraint is needed, it should be part of a profile constraint and specific to particular auxiliary picture types.

As applied to depth maps, this should be considered a 3V issue.

JCTVC-P0092 MV-HEVC/SHVC HLS: Proposal for supporting optional overlays with help of auxiliary pictures [N. Stefanoski, O. Wang, A. Smolic (DRZ), T. Szypulski (ESPN)]

Presenter not present 01-09 pm.

Presentation deferred.

JCTVC-P0135 MV-HEVC/SHVC HLS: Auxiliary pictures for multiple overlays [J. Boyce, S. Wenger (Vidyo)]

Presentation deferred for coordination with P0092 review.

JCTVC-P0207 RExt/MV-HEVC/SHVC: On Auxiliary Alpha Plane Pictures [K. Misra, S. Deshpande, A. Segall (Sharp)]

This contribution proposes a bitstream constraint that requires alpha plane auxiliary pictures associated with IDR primary pictures to be IDR. The proposed constraint guarantees that if random access is performed at the primary picture IDR then the corresponding alpha plane auxiliary picture is also a random access point.

In revision 1 the proposed bitstream constraint language is modified with change marks.

It was remarked that if we add such a constraint, additional details may be desirable – e.g., also saying that if the primary picture is IRAP, the auxiliary must be IRAP.

It was remarked that some constraints that may be applied in all cases would resolve particular cases involving alpha pictures.

It was remarked that we have other cases where we envision having an IDR picture in a base layer and no IRAP in an enhancement layer and therefore do not apply such a constraint (e.g., for layer-wise start-up).

The question is whether the spirit of the intent is to require the alpha to always be decodable along with the primary picture whenever the alpha is present, including for purposes of random access.

It was suggested that this seems like the sort of constraint we might specify if alpha decoding is part of a profile capability, but may not be necessary if alpha decoding capability is not profiled. No action taken for this reason.

JCTVC-P0122 On chroma auxiliary pictures [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

Presentation deferred for coordination with section 6.5.2.

### Other (5)

JCTVC-P0062 MV-HEVC/SHVC HLS: Redundant frames for SHVC/MV-HEVC/HEVC [M. Sychev, V. Stepin, V. Anisimovskiy, S. Ikonin (Huawei)]

This contribution presents the support for redundant frames in SHVC/MV-HEVC (/HEVC if possible). The proposed solution describes the syntax and semantics of how to support redundant frames (in the enhancement layers) by enabling more than one frame in the same layer to have the same POC, and proposes HRD behaviour when processing the frame with a duplicated POC. This contribution has two proposed solutions for usage of redundant frames for loss resilience and one for performing inter-layer prediction for redundant frames.

It was proposed for the redundant pictures to be coded in a different order than the primary coded pictures. It was asked whether a decoder would be expected to wait several frame periods for a redundant picture to arrive and then decode that picture for use as a reference picture for the prediction of other dependent pictures that have arrived in the meantime.

It was noted that the proposal is entirely new as a concept for HEVC, and has arrived at a late stage of the development of the current phase of extensions development.

It was asked whether, assuming we like the proposed functionality, it could be added in a later extension rather than being done within the current phase of work. This seemed possible in principle, so it was suggested that it may be appropriate to prioritize this lower than current in-progress work.

Several variants of the concept were described in the proposal, and some modifications were discussed in its discussion.

It was noted that the lack of redundant pictures in non-Baseline AVC profiles has not previously been identified as a serious problem.

No significant interest was expressed by non-proponents for short-term action on this.

Further study was encouraged, although it seemed unlikely that such a concept could be incorporated within our current phase of active extension developments.

JCTVC-P0084 Indication of SMPTE 2084, 2085 and carriage of 2086 metadata in HEVC [C. Fogg (Harmonic), J. Helman (Movielabs)]

The proponent indicated that this is related to P0050. See notes in that section.

JCTVC-P0118 RExt HLS: Picture referencing across CRA pictures [R. Sjöberg, J. Samuelsson, Y. Wang (Ericsson)]

TBP.

JCTVC-P0137 REXT/MV-HEVC/SHVC/3D-HEVC HLS: On indication of decoding process and profile-level-tier combinations [M. M. Hannuksela (Nokia)]

The contribution proposes the following three aspects. Aspect 3 is proposed only if aspect 1 is adopted.

- decoding_process_idc is included in the VPS for each layer. It specifies the decoding process (version 1, REXT, MV-HEVC, SHVC, 3D-HEVC) to be used for the layer and the constraints on sps_extension_type_flag[ i ] values for the layer.

- The use of the profile_tier_level( ) syntax structure for layers sets excluding the base layer is clarified as follows:

  - The independent layer with the smallest nuh_layer_id among independent layers in the layer set is considered to be the base layer in the decoding process except for the slice segment header decoding.

  - When the layer set does not contain layers with AuxId equal to 0, the profile_tier_level( ) syntax structure applies to a CVS in which AuxId for all the layers is considered to be equal to 0.

- The depth auxiliary picture type (AUX_DEPTH value of AuxId) is removed from MV-HEVC and the DepthFlag scalability dimension is used instead (scalability mask index equal to 0) with decoding_process_idc indicating either the version 1 or MV-HEVC decoding process for depth views.

It is asserted that aspects 1 and 2 provide the following functionality:

- A capability to indicate, e.g. in session negotiation, which profile-level-tier combination and decoding process are used for independent layers and auxiliary picture layers, particularly differentiating between version 1 and REXT decoding processes.

- A capability to indicate which decoding process is used for layers that are not included in any output layer sets, e.g. when the total number of views in the bitstream exceeds profile limits.

Regarding item 3, it was remarked that the coupling of the scalability dimension with coding tools in the current 3D HEVC design seems questionable (e.g., instead there indicators of coding features, subject to profile constraints).

It was suggested that item 3 should be acted upon even if the rest is not.

Potential decision: Use DepthFlag rather than AuxId for depth.

In our current design, auxiliary pictures may not have a profile that establishes constraints on their content. It was agreed that this is a problem.

It is asserted that it would be essential to be able to provide the functionality of indicating which decoding process (v1, REXT, MV-HEVC, SHVC, 3D-HEVC) is used for each layer for the following functionality:

- It should be known in session negotiation

  - which profile-tier-level combination applies to a set of auxiliary picture layers; and/or

  - which decoding process is used for particular auxiliary picture layers

- This information lets the receiver to choose whether to receive a particular auxiliary picture layer or which one of the auxiliary picture layers offered as alternatives to choose. For example, a decoder may be able to process auxiliary picture layers decodable with version 1 decoding process only and hence desires not to receive e.g. REXT-coded auxiliary picture layers.

- In case of simulcast layers, it should be known in session negotiation which decoding process is used for each independent layer (to let the decoder to decide whether to receive or choose from layers provided as alternatives similarly to above for auxiliary picture layers).

- If the entire bitstream conforms to no profile and some layers fall outside of any specified output layer sets, decoders may still want to know which decoding process is used for those layers. For example a bitstream may include a greater number of views than allowed in any profile.

The contribution seems to raise some important issues for consideration.

Revisit after offline study.

JCTVC-P0187 HEVCv1/MV-HEVC/SHVC HLS: On inference of NoOutputOfPriorPicsFlag [Y.-K. Wang, Y. Chen (Qualcomm)]

## HL syntax in SHVC and 3D extensions (35)

### Generic HLS issues (2)

JCTVC-P0043 Version 1/MV-HEVC/SHVC HLS: Access unit boundary detection [M. M. Hannuksela (Nokia)]

Planned to be chaired by GJS.

JCTVC-P0139 MV-HEVC/SHVC HLS: Header parameter set (HPS) [M. M. Hannuksela, H. Roodaki (Nokia)]

### POC alignment and derivation (5)

JCTVC-P0041 MV-HEVC/SHVC HLS: On picture order count [Hendry, A. K. Ramasubramonian, Y.-K. Wang, Y. Chen (Qualcomm), M. Li, P. Wu (ZTE)]

JCTVC-P0056 MV-HEVC/SHVC HLS: Layer-tree POC [M. M. Hannuksela (Nokia)]

JCTVC-P0067 MV-HEVC/SHEVC HLS: Comments on POC alignment [M. Li, P. Wu, G. Shang, Y. Xie (ZTE)]

JCTVC-P0260 MV-HEVC/SHVC HLS: Additional information on the POC design in JCTVC-P0041/JCT3V-G0031 [A. K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)] [late] [miss]

### HLS for hybrid scalability (3)

Discussed 01-09 evening (GJS).

JCTVC-P0140 MV-HEVC/SHVC HLS: On non-HEVC base layer [M. M. Hannuksela (Nokia)]

The contribution discusses two aligned designs for enabling non-HEVC-coded base layer:

- The decoded non-HEVC base layer pictures are provided by external means and their DPB related properties (NoOutputOfPriorPicsFlag, PicOutputFlag, PicOrderCntVal, and RPS) are either provided by external means or included in the HEVC bitstream using a specific NAL unit. This design is the same as in JCTVC-O0166/JCT3V-F0060.

- The decoded non-HEVC base layer pictures are provided by externals means or by including non-HEVC NAL units within specific HEVC NAL units. Similarly to the first option the DPB related properties (NoOutputOfPriorPicsFlag, PicOutputFlag, PicOrderCntVal, and RPS) of non-HEVC pictures are either provided by external means (when the pictures themselves are provided by external) or included in the HEVC NAL units together with the nested non-HEVC NAL units.

As the changes are asserted to be substantial and may require verification by both expert review and software implementation, the contribution was submitted for discussion rather than as a proposal. The contribution follows up on JCTVC-O0166/JCT3V-F0060.

It was asked why we would need RPS information. It was remarked that this is to provide a synchronized output for the base layer pictures as if they were HEVC pictures, and that it may not be needed if a substantial amount of the operation is controlled by external means.

It was asked whether the decoded pictures provided by external means really need to be arriving in the same decoding order as if they were HEVC pictures within the same bitstream.

It was remarked that if decoded pictures are provided by external means, a conformance test bitstream would need to include copies of these decoded pictures (or a way to generate/obtain them).

It was remarked that perhaps we don't need to have anything from the base layer except the availability of the decoded pictures and awareness of their representation format (e.g., width, height, bit depth and colour format, and perhaps field parity information).

No immediate action was requested.

JCTVC-P0184 Support of AVC base layer in SHVC [Y.-K. Wang, J. Chen, Y. Chen, Hendry (Qualcomm)]

This document propose a way for the support of AVC base layer in SHVC that is asserted to be the simplest in terms of the changes needed to the SHVC specification. The two key aspects of the proposed design are: 1) no encapsulation, meaning decoded base layer pictures are provided by external means; and 2) output of base layer pictures, including the synchronization with output of enhancement layer pictures, is controlled by external means. The spec text changes for the design are provided in the attachment of this document, with changes marked in relative to the latest SHVC spec text in JCTVC-O1008v3.

See also notes above on P0140.

Revisit with P0203.

JCTVC-P0203 Hybrid codec scalability profile in SHVC [J. Samuelsson, J. Enhorn, R. Sjöberg (Ericsson)]

See also section 3.5.2.

This contribution proposes to include the hybrid codec scalability profile as described in JCTVC-O1012 into the SHVC draft with the following modifications:

- To remove the option of encapsulating AVC NAL units in HEVC NAL units (and just keep the two options of no encapsulation and encapsulating HEVC NAL units in AVC NAL units).

- To specify that the base layer must obey all constraints specified for the High profile in the AVC specification.

The contribution asserts that only one encapsulation format is needed and that it is important that the AVC NAL units are unmodified (i.e. no additional header is put in front of the AVC NAL unit header).

See also notes above on P0140.

The contributor indicates that if the AVC is wrapped within HEVC headers, that wrapping would need to be removed in order to feed the base layer to the legacy decoder, and it was asserted that this could especially be a problem if the bitstream is encrypted.

Revisit with P0184 (and consider the alternative encapsulation approach in O1012 – the other approach provides temporal ID and layer ID, enabling bitstream extraction by a middle box without paying attention to the contents within the NALUs – if the encapsulation was the other way around, prefix NALUs may be needed for the base layer and some way to convey VPS and parsing the embedded HEVC NUHs would be needed for the enhancement layer).

JCTVC-P0183 AHG9: On AVC independent non-base layer indicator [Y. He, Y. Ye (InterDigital)]

This contribution proposes to expand the current avc_base_layer_flag to allow independent non-base layers to be coded in AVC. The proposal is to put a flag in the VPS extension to indicate, for each non-base layer, whether the layer is AVC or HEVC.

It was remarked that although the concept seems to make sense and provide a potentially useful capability if we assume that such a within-the-bitstream muxing is otherwise supported within HEVC syntax. However, it seems premature to conclude that this will be the case. This should be further considered when that higher-level question is answered. The concept was reportedly developed based on just examining potential syntax expression capability rather than a specific use case – further understanding of such use cases would be needed.

### High-level syntax and semantics cleanups (25)

#### Video parameter set (13)

JCTVC-P0045 MV-HEVC/SHVC HLS: On layer set definition [T. Ikai, T. Tsukuba, T. Yamamoto (Sharp)]

JCTVC-P0046 MV-HEVC/SHVC HLS: Additional layer set [T. Ikai, T. Tsukuba, T. Yamamoto (Sharp)]

JCTVC-P0048 MV-HEVC/SHVC HLS: Syntax clean-up of profile, tier and level information [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCTVC-P0052 MV-HEVC/SHVC HLS: VPS extension clean-up [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, H.-C. Wey, C. Kim (Samsung)]

JCTVC-P0070 MV-HEVC/SHVC HLS: On video parameter set extension [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-P0076 MV-HEVC/SHVC HLS: On VPS extension and VPS VUI [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCTVC-P0110 MV-HEVC/SHVC HLS: On default output layer sets [K. Ugur, M. M. Hannuksela (Nokia)]

JCTVC-P0125 MV-HEVC/SHVC HLS: On VPS extension offset and VPS VUI offset [A.K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

JCTVC-P0132 MV-HEVC/SHVC HLS: On alt_output_layer_flag [A.K. Ramasubramonian, Y.-K. Wang, Hendry, Y. Chen (Qualcomm)]

JCTVC-P0136 MV-HEVC/SHVC HLS: Improvements of Video and Picture Parameter Sets [Truong Cong Thang (UoA), Jung Won Kang, Jinho Lee, Hahyun Lee, Jin Soo Choi (ETRI)]

JCTVC-P0157 MV-HEVC/SHVC HLS: On Indications for Inter-layer Prediction [S. Deshpande (Sharp)]

JCTVC-P0078 MV-HEVC/SHVC HLS: On output_layer_flag [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCTVC-P0262 Support for out-of-band signaling in VPS to enable future layer additions [Ajay Luthra, Sam Narasimhan]

#### Sequence and picture parameter sets (2)

JCTVC-P0155 MV-HEVC/SHVC HLS: On Sequence Parameter Set [S. Deshpande (Sharp)]

JCTVC-P0181 MV-HEVC/SHVC HLS: On Picture Parameter Set [Y. He, Y. Ye (InterDigital)]

#### Hypothetical reference decoder (HRD) (4)

Planned to be chaired by GJS.

JCTVC-P0138 MV-HEVC/SHVC HLS: HRD parameters for bitstreams excluding CL-RAS pictures [M. M. Hannuksela (Nokia)]

Discussed 01-09 (GJS).

This contribution concerns CPB, and is the only contribution on that subject.

Cross-layer random access skip (CL-RAS) pictures need not be decoded and hence it is asserted that HRD parameters without CL-RAS pictures would be beneficial. It is proposed to indicate that HRD parameters for a bitstream without CL-RAS pictures and without RASL pictures associated with the first IRAP picture of each layer using a buffering period SEI message included in a scalable nesting SEI message that applies to a layer set. No new syntax is proposed in the contribution.

Cross-layer random access skip (CL-RAS) pictures are pictures that cannot be correctly decoded when the decoding process starts from an IRAP access unit that does not contain IRAP pictures in all layers. CL-RAS pictures are not indicated in the bitstream but they are concluded during the decoding process: a CL-RAS picture is a picture with nuh_layer_id equal to layerId such that LayerInitializedFlag[ layerId ] is equal to 0.

The figure below shows an example how CL-RAS pictures are concluded when the decoding starts from AU x (in which case the CL-RAS pictures are the green pictures marked with "associated with AU x") or from AU z (in which case the CL-RAS pictures are the green pictures marked with "associated with "AU z"). When the decoding process starts from AU x or AU z, the respective CL-RAS pictures (the green pictures marked with "associated with AU x" or "AU z", respectively) can be removed from the bitstream, while the bitstream remains conforming.

Association of CL-RAS pictures with IRAP access units. AU y does not have any association CL-RAS pictures in this bitstream (copied from JCTVC-O0212/JCT3V-F0072).

The contribution said that the indication of HRD parameters for bitstreams without CL-RAS pictures is the multi-layer equivalent of the indication of HRD parameters for bitstreams without RASL pictures for single-layer bitstreams. The HRD parameters for bitstreams without RASL pictures are indicated in the buffering period SEI message with the cpb_delay_offset, dpb_delay_offset, nal_initial_alt_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_offset[ i ], vcl_initial_alt_cpb_removal_delay[ i ], and vcl_initial_alt_cpb_removal_offset[ i ] syntax elements.

In this contribution, it is proposed to indicate that HRD parameters for a bitstream without CL-RAS pictures and without RASL pictures associated with the first IRAP picture of each layer using a buffering period SEI message included in a scalable nesting SEI message that applies to a layer set. The syntax elements cpb_delay_offset, dpb_delay_offset, nal_initial_alt_cpb_removal_delay[ i ], nal_initial_alt_cpb_removal_offset[ i ], vcl_initial_alt_cpb_removal_delay[ i ], and vcl_initial_alt_cpb_removal_offset[ i ] are interpreted for a bitstream that excludes both the CL-RAS picture and the RASL pictures (associated with the initial IRAP pictures of each layer). No new syntax is proposed in the contribution.

The detailed proposal is included as change marks in an accompanying specification text document.

It was remarked that in the v1 syntax, we have a NUT that tells the decoder whether there may be RASL pictures or not, and if not, the "alternative" HRD parameters are used by the decoder. We do not have this indication for CL-RAS pictures. (It was previously proposed for CL-RAS pictures to have a distinct NUT value, but this is not the approach that was adopted.)

A prior proposal suggested to have a (possibly externally-supplied) flag associated with the base layer IRAP picture with NoClrasOutputFlag equal to 1 to indicate whether RASL or CL-RAS pictures can be present or not. This approach would presumably work with the proposal.

The proposal avoided needing extra syntax as had been proposed previously in a similar-concept proposal O0212.

It was remarked that the flag could also be sent as an extension bit in the BP SEI message.

It was also remarked that a similar previously-specified flag called UseAltCpbParamsFlag could perhaps benefit from putting such a flag in the SEI message.

It was remarked that one flag may be sufficient for both purposes.

These can be considered, from the v1 perspective, as a form of "external means".

Potential decision (pending offline double-check): Adopt with the specification of the SEI extension flag.

JCTVC-P0069 MV-HEVC/SHVC HLS: Decoded picture buffer signalling [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-P0156 MV-HEVC/SHVC HLS: On DPB Parameters in VPS [S. Deshpande (Sharp)]

JCTVC-P0192 MV-HEVC/SHVC HLS: On decoded picture buffer management [Y.-K. Wang, A.K. Ramasubramonian, Y. Chen (Qualcomm)]

#### Miscellaneous HLS topics (6)

JCTVC-P0047 MV-HEVC/SHVC HLS: On sub-bitstream extraction [T. Tsukuba, T. Yamamoto, T. Ikai (Sharp)]

JCTVC-P0068 MV-HEVC/SHVC HLS: On parameter improvements [B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-P0079 MV-HEVC/SHVC HLS: comments on MV-HEVC WD 6 and SHVC WD 4 [H. Lee, J. W. Kang, J. Lee, J. S. Choi (ETRI)]

JCTVC-P0130 MV-HEVC/SHVC HLS: Miscellaneous HLS topics [A.K. Ramasubramonian, Hendry, Y.-K. Wang, Y. Chen, V. Seregin (Qualcomm)]

JCTVC-P0141 MV-HEVC/SHVC HLS: On temporal enhancement layers [M. M. Hannuksela (Nokia)] [late]

JCTVC-P0182 MV-HEVC/SHVC HLS: On Sub-bitstream extraction and re-writing process [Y. He, Y. Ye (InterDigital)]

## SEI messages (13)

### Motion and prediction constrained SEI messages (2)

JCTVC-P0051 HLS: Extensions to Temporal Motion-constrained tile sets SEI message [S. Hattori, O. Nakagami (Sony)]

JCTVC-P0172 HLS: Extension to temporal motion constrained tile sets SEI message for no display [C. Auyeung, A. Tabatabai (Sony)]

### Frame packing SEI messages (3)

JCTVC-P0121 On frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [K. Ugur, D. Bugdayci, M. M. Hannuksela (Nokia)]

JCTVC-P0174 On bit allocation of 4:2:0 compatible coding of 4:4:4 video via frame packing arrangement SEI message [K. Minoo, D. Baylon (ARRIS)]

JCTVC-P0216 Additional content interpretation type and experiments for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams [S. Reddy, S. Kanumuri, Y. Wu, S. Sadhwani, G. J. Sullivan, H. S. Malvar (Microsoft)]

### Other SEI messages (8)

JCTVC-P0050 HLS: SEI message for Knee Function Information [S. Hattori, O. Nakagami, T. Suzuki (Sony)]

JCTVC-P0084 Indication of SMPTE 2084, 2085 and carriage of 2086 metadata in HEVC [C. Fogg (Harmonic), J. Helman (Movielabs)]

TBP. Partly a VUI proposal and partly an SEI message proposal. The proponent indicated that this is related to P0050.

JCTVC-P0081 Comments on chroma_resampling_filter_hint SEI [C. Fogg (Harmonic)]

JCTVC-P0120 AHG9: Display hint SEI message for ROI layers [K. Ugur, M. M. Hannuksela (Nokia)] [late]

JCTVC-P0123 RExt HLS (AHG5 and 9): SEI message for alpha channel information [M. Naccari, M. Mrak (BBC)]

JCTVC-P0126 SEI message for Colour Mapping Information [P. Andrivon, P. Bordes, E. François (Technicolor)]

JCTVC-P0131 MV-HEVC/SHVC HLS: Layer up-switching information SEI message [A.K. Ramasubramonian, Hendry, Y.-K. Wang (Qualcomm)]

JCTVC-P0133 MV-HEVC/SHVC HLS: On recovery point and region refresh SEI messages [Hendry, Y.-K. Wang, A. K. Ramasubramonian (Qualcomm)]

JCTVC-P0204 MV-HEVC/SHVC HLS: Sub-bitstream property SEI message [Y. Chen, Y.-K. Wang, A. K. Ramasubramonian, Hendry (Qualcomm)]

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control (1)

### Rate control

### Encoder optimization

JCTVC-P0059 Unifying HM and RExt Inter-Prediction Search [K. Sharman, N. Saunders, J. Gamei (Sony)]

### Software development

## Allocation unclear, withdrawn (5)

JCTVC-P0072 Withdrawn

JCTVC-P0206 Withdrawn

JCTVC-P0258 Withdrawn

JCTVC-P0259 Withdrawn

JCTVC-P0263 Withdrawn

JCTVC-P0264 Withdrawn

# Plenary Discussions and BoG Reports

## Project development

Prior-identified open issues needing parent-body attention:

- [add]

New issues highlighted for parent-body attention:

- Field/frame in regard to scalability (for either AVC or HEVC base layers)

## BoGs

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the Friday of the week preceding the meeting (3 Jan).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs (to be updated)

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-M1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

CE plan final at same time as corresponding software except for SCE1 & 4 due to test sequence issues.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 12.0 and SHM 2.0 should be available within 2 weeks after the meeting. [To be fixed]

- HM 12.0+RExt should be available within 1 week after HM 12.0 availability.

## Subjective verification test plan

JCTVC-N0385 Draft HEVC verification test plan [T.K. Tan, V. Baroncini]

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-O1002 HEVC Test Model 13 (HM 13) Encoder Description. - Collect reports of errata for HEVC version 1 text specification. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, K. McCann (co-chairs), W.-J. Han, I. K. Kim, J.-R. Ohm, K. Sugimoto, G. J. Sullivan, Y.-K. Wang, (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 13.0 software version (by 2013-12-13) and the reference configuration encodings according to JCTVC-L1100 common conditions. [Check/fix any conflicting delivery date info. 12.1 to be released approx 11-01 is adequate] - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | F. Bossen (chair), D. Flynn, K. Sühring (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Discuss the work plan needed to develop HEVC conformance testing. - Study potential testing methodology to fulfil the requirements of HEVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki, W. Wan (co-chairs) | N |
| HEVC range extensions development (AHG5) (jct-vc@lists.rwth-aachen.de) - Study aspects of the technical design and develop software relating to the support of non-4:2:0 chroma formats, bit depths beyond 8 bits, and auxiliary/alpha channel coding, in coordination with AHG7 and AHG18. - Contact proponents to solicit input and produce HEVC Range extensions test model 1 encoder description (O01013). - Perform memory bandwidth analysis of the range extensions technical design and its proposed modifications. - Evaluate blocking artifacts in 4:4:4 chroma format coding and set up a plan for subjective testing of associated techniques at the next meeting. - Discuss and propose test conditions and test material in coordination with AHG16 for the development of the range extensions. | M. Naccari, C. Rosewarne (co-chairs) | N |
| Range extensions draft text (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-O1005 HEVC RExt draft text. - Gather and address comments for refinement of the text. - Coordinate with AHG7 on range extensions software development to address any identified issues regarding text and software relationship. | J. Sole (primary), D. Flynn, C. Rosewarne, G. Sullivan, T. Suzuki | N |
| Range extensions software development (AHG7) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM RExt software and its distribution - Produce documentation of software usage for distribution with the software - Prepare and deliver HM 12.1-RExt-5.0 software version and the reference configuration encodings according to JCTVC-O1006. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG6 on range extensions draft text to address any identified issues regarding text and software relationship. | D. Flynn, K. Sharman (co-chairs) | N |
| Screen content coding (AHG8) (jct-vc@lists.rwth-aachen.de) - Study (lossy and lossless) coding tools and performance of HEVC and its range extensions on screen content. - Evaluate and identify test material appropriate for testing screen content coding performance, in coordination with AHG16. - Make recommendations for test conditions for screen content coding. - Coordinate for finalization of the test conditions for CEs relating to lossless and screen content coding. | H. Yu (chair), R. Cohen, A. Duenas, D.-K. Kwon, T. Lin, J. Xu (vice-chairs) | N |
| High-level syntax for HEVC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Identify opportunities for common approaches for multi-view and scalable high-level extensions of HEVC. - Study NAL unit header, video parameter set, sequence parameter set, picture parameter set, and slice header syntax designs. - Study SEI messages and VUI syntax designs needed for HEVC extensions. - Assist in software development and text drafting for the high-level syntax in the HEVC extensions designs. - Coordinate efforts with JCT-3V AHG7 on high-level syntax issues in relation to 3D extensions. | M. M. Hannuksela (chair), J. Boyce, Y. Chen, S. Deshpande, A. Norkin, Y.-K. Wang, P. Wu (vice-chairs) | N |
| Multi-layer picture order count derivation (AHG10) (jct-vc@lists.rwth-aachen.de) - Study picture order count (POC) operation for HEVC layered coding, in regard to loss resilience, random access, step-wise layer transitions. - Identify problems associated with POC derivation. - Solicit and study candidate solutions for identified problems with POC derivation. - Hold teleconference discussions (pre-announced on the JCT-VC reflector with at least 7 days notice) to facilitate the work of the AHG. | G. Sullivan (chair) | Tel. |
| SHVC text editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-O1007 SHVC Test Model 4 (SHM 4) text. - Produce and finalize JCTVC-O1008 SHVC text specification Draft 4. - Gather and address comments for corrections and editorial improvements of these documents. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 4.0 software (based on HM 12) for experimentation. - Provide software to CEs within two weeks after the meeting. - Generate anchors and templates based on common test conditions. - Bring software into alignment with HM 13 by the next meeting. - Discuss and identify additional issues related to SHVC software. | V. Seregin (chair), Y. He, D. Kwon (vice-chairs) | N |
| SHVC inter-layer filtering (AHG13) (jct-vc@lists.rwth-aachen.de) - Study alternative upsampling and downsampling filters for spatial scalability. - Study the effect of taking into account chroma position alignment during resampling processing. - Study re-sampling process modification related to extracting regions of interest. - Discuss and identify additional issues related to inter-layer filtering. | E. Alshina (chair), J. Chen, P. Topiwala, T. Yamamoto, Y. Ye (vice-chairs) | N |
| Colour gamut and bit depth scalability (AHG14) (jct-vc@lists.rwth-aachen.de) - Study methods for colour gamut scalable coding. - Study the interaction of colour gamut scalability with bit-depth and spatial scalability. - Identify test sequences and test conditions, in coordination with AHG16. - Discuss and identify additional issues related to colour gamut scalability. | P. Bordes, A. Duenas, (co-chairs), E. Alshina, S. Deshpande, Y. He, D.-K. Kwon, X. Li (vice-chairs) | N |
| Hybrid codec scalability (AHG15) (jct-vc@lists.rwth-aachen.de) - Coordinate with AHG12 on software development and anchor support for AVC base layer. - Study the design under consideration for hybrid codec scalability text in JCTVC-O1012. - Study methods for base layer codec identification and picture alignment, including use of encapsulation, and identification of external means. - Study profile and level specification methods for hybrid codec scalability. | J. Boyce (chair), A. Duenas, K. Kawamura, J. Samuelsson (vice-chairs) | N |
| Test sequence material (AHG16) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for HEVC development. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG5 on range extensions development, AHG8 on screen content coding, AHG14 on colour gamut scalability, AHG18 on high bit rate and high bit depth operation, and AHG19 for verification test development. | T. Suzuki, R. Cohen (co-chairs), T. K. Tan, S. Wenger (vice-chairs) | N |
| SHVC complexity assessment (AHG17) (jct-vc@lists.rwth-aachen.de) - Study memory bandwidth, memory usage and computational complexity of scalable tools and methodologies to evaluate them. - Prepare a report analyzing performance and complexity of single-layer, simulcast, and scalable coding configurations. - Coordinate with SCE on colour gamut scalability and provide a template to be used for complexity analysis of SHM4.0, single-layer coding, and SCE techniques. | E. Alshina (chair), M. Budagavi, E. François, J. Kang, X. Li, A. Tabatabai, X. Xiu (vice-chairs) | N |
| High bit-rate & bit-depth operating points (AHG18) (jct-vc@lists.rwth-aachen.de) - Study the accuracy needed for internal processing elements to support bit depths up to 16-bits per sample. - Study relationship to lossless coding capability. - Verify rate-distortion optimization behaviour for high bit rates and high bit depths. - Study entropy coding operation and throughput at high bit rates and high bit depths and potential needs for associated design modification. - Identify test sequences (in coordination with AHG16) and test conditions for testing high bit rate and high bit depth coding behaviour. - Prepare software implementation for technical investigation of new features intended for high bit rates and high bit depths. - Study coding performance at high bit-rate and high bit depth operating points and investigate the benefit over existing standards. | K. Sharman (chair), R. Joshi, H.-Y. Kim (vice-chairs) | N |
| Verification test preparation (AHG19) (jct-vc@lists.rwth-aachen.de) - Identify test conditions and test sequences (in coordination with AHG16) for verification of HEVC compression capability, including consideration of use case scenarios for random access and low delay operation. - Identify bit rates, picture resolutions and bit depths appropriate for HEVC verification. - Identify appropriate encoder usage for comparison of HEVC and AVC compression capability. - Generate and collect candidate HEVC and AVC encoded bitstreams for HEVC verification testing. - Identify and coordinate arrangements toward the preparation of test sites for subjective testing. - Draft potential updates to the HEVC verification test plan JCTVC-O1011. | T. K. Tan, V. Baroncini (co-chairs), M. Karczewicz, M. Mrak, W. Wan, J. Wen (vice-chairs) | N |
| Multi-layer hypothetical reference decoder (AHG20) (jct-vc@lists.rwth-aachen.de) - Study the issues related to layer-specific and combined multi-layer HRD operation (including DPB and CPB). - Identify layer-specific or combined multi-layer HRD operations that need to be addressed at the system and/or video coding layer. - Study issues and implications of either or both HRD mechanisms when adding or removing layers at re-distribution points. - Study issues and implications of layer-specific buffer operation in relevant application contexts. - Study high-level syntax needs for HRD information, e.g. in SEI, SPS VUI, or VPS VUI. | K. Suehring (chair), S. Deshpande, M. M. Hannuksela, J. Kang, A. K. Ramasubramonian, A. Tabatabai (vice-chairs) | N |

# Output documents (to be updated)

Check delivery dates, doc numbers, links to doc files.

Planning of HM encoder description – plan to improve this in AHG work toward publishing as part of the RS standard. TBD whether this would be in v.1 of the RS standard or added later.

Issues in conformance: The situation has been improving. profile_idc and level_idc need to be correct, some HM 9 bitstreams not updated, some planned bitstreams missing, many bitstreams need updates for corrections of these issues and others. Coverage still needs improvement. Various combinations of tiles & slices & loop filtering control parameters, SEI messages, "corner cases".

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate those responsible for document production.

JCTVC-O1000 Meeting Report of 15th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm] (near next meeting)

Remains valid – not re-issued: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen, (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-O1002 High Efficiency Video Coding (HEVC) Test Model 13 (HM 13) Encoder Description [K. McCann (primary), B. Bross, W.-J. Han, I. K. Kim, K. Sugimoto, G. J. Sullivan] (WG 11 N 13937) [2014-01-06] (near next meeting)

JCTVC-O1003 High Efficiency Video Coding (HEVC) Defect Report Draft 2 [Y.-K. Wang, G. J. Sullivan, B. Bross] (WG 11 N 13931) [2013-11-18] (2 weeks)

JCTVC-O1004 HEVC Conformance Draft 5 [T. Suzuki, W. Wan, G. J. Sullivan] (WG 11 N 13942 ISO/IEC Study of DIS) [2013-11-18] (2 weeks)

JCTVC-O1005 HEVC Range Extensions Draft 5 [D. Flynn, J. Sole, G. Sullivan, T. Suzuki] (WG 11 N 13933, Study of ISO/IEC DAM) [2013-12-02] (2 Dec.)

JCTVC-O1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman] [2013-11-18] (2 weeks)

AHG 7 to confirm expected coding results and refine selections accordingly during preparation of the document.

JCTVC-O1007 SHVC Test Model 4 (SHM 4) [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela] (WG 11 N 13939) [2013-12-02] (2 Dec.)

JCTVC-O1008 SHVC Draft 4 [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, Y.-K. Wang] (WG 11 N 13935 Study of PDAM) [2013-12-07] (1 week after MV-HEVC draft)

JCTVC-O1009 Common SHM test conditions and software reference configurations [V. Seregin, Y. He] [2013-11-22] (1 week after software)

update to include updated anchor results and config files.

JCTVC-O1010 Guidelines for conformance test bitstream preparation [T. Suzuki, W. Wan] [2013-11-01] (immediate)

Update to include updated anchor results and config files.

JCTVC-O1011 HEVC verification test plan draft 2 (WG 11 N 13932) [T.K. Tan, V. Baroncini] [2013-11-01] (immediate)

JCTVC-O1012 Designs under study for SHVC hybrid scalability [J. Boyce, J. Samuelsson] [2013-11-08] (1 week)

JCTVC-O1013 HEVC Range extensions test model 5 encoder description [M. Naccari, C. Rosewarne, G. Sullivan] (WG 11 N 13938) [2013-12-13]

Remains valid – not re-issued: JCTVC-L1100 Common HM test conditions and software reference configurations [F. Bossen]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting were not considered binding on final CE plans as reviewed in closing plenary.

- JCTVC-O1101 HEVC Scalable Extensions Core Experiment SCE1: Colour Gamut and Bit Depth Scalability [P. Bordes, E. Alshina, A. Duenas, Y. He, X. Li (CE Coordinators)]

Not using high precision weighted prediction, due to desire for base layer to use Main or Main 10 profile.

T0 expected in 2 weeks.

JCTVC-O1101 was reviewed in non-final form in closing plenary. A three week finalization period was authorized.

- JCTVC-O1121 HEVC Range Extensions Core Experiment 1 (RCE1): High bit rate coding at high bit depths [J. Joshi, K. Sharman (CE coordinators)]

- JCTVC-O1122 HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization [C. Rosewarne, M. Karczewicz, K. Sharman, S.-H. Kim (CE coordinators)]

- JCTVC-O1123 HEVC Range Extensions Core Experiment 3 (RCE3) Intra block copy refinement [J. Sole (CE coordinators)]

- JCTVC-O1124 HEVC Range Extensions Core Experiment 4 (RCE4) Palette coding [L. Guo, X. Guo (CE coordinators)]

Review chaired by K.Chono.

1. No test on the combination. A revision of the document will be uploaded to remove the discussion of a combination test.

2. Subjective test will be removed if clear procedure is not given by the CE plan finalization date.

JCTVC-O1121 through JCTVC-O1124 were reviewed in non-final form in closing plenary. A three week finalization period was authorized.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Monday or Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Wednesday or Thursday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 27 Mar. – 4 Apr. 2014 under WG 11 auspices in Valencia, ES.

- 30 June – 9 July 2014 under ITU-T auspices in Sapporo, JP.

- 16–24 Oct. 2014 under WG 11 auspices in Strasbourg, FR.

- XX–XX Feb. 2015 under ITU-T auspices in Geneva, CH.

The agreed document deadline for the March/April 2014 meeting is XX March). No restrictions were planned to be imposed on the scheduling of agenda items within that meeting.

XXX was thanked for the excellent hosting of the 16th meeting of the JCT-VC. Huawei and Sony were thanked for providing viewing equipment used at the meeting. [Ed. more?]

The JCT-VC meeting was closed at approximately XXXX hours on Fri. 18 Jan. 2014.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the sixteenth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting (approximately XXX people in total), were as follows: