| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 19th Meeting: Strasbourg, FR, 17–24 Oct. 2014 | Document: JCTVC-S_Notes_d7 |
| --- | --- |

| Title: | Meeting report of the 19th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Strasbourg, FR, 17–24 Oct. 2014 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its nineteenth meeting during 17 – 24 Oct. 2014 at the Palais de la Musique et des Congres, Strasbourg, FR. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 17 Oct. 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 24 Oct. 2014. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

One primary goal of the meeting was to review the work that was performed in the interim period since the eighteenth JCT-VC meeting in producing

- For HEVC version 1, the HEVC test model (HM) 16, HEVC Defect Report for single layer coding (for Version 1), HEVC conformance testing draft 8, and the HEVC reference software for version 1 profiles;

- The text of HEVC versions 2, which subsumes the RExt, MV-HEVC and SHVC extensions;

- For HEVC range extensions (RExt), the RExt conformance draft 2;

- For HEVC scalable extensions (SHVC), the SHVC extensions draft 7 and SHVC Test Model 7;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 2, SCC draft text 1, and a document specifying common test conditions and software reference configurations for SCC experiments;

The other most important goals were to review the results from ten Core Experiments on Screen Content Coding (CE1-10), and review other technical input documents. Reviewing the progress made towards definition of screen content coding tools was one important topic of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Possible needs for corrections to version 2 were also considered.

In addition to XX new experiment plan descriptions, the JCT-VC produced XX other particularly important output documents from the meeting (update):

- For HEVC version 1, the HEVC test model (HM) 16, HEVC Defect Report for single layer coding (for Version 1), HEVC conformance testing draft 8, and the HEVC reference software for version 1 profiles;

- The text of HEVC edition 2, which subsumes the RExt, MV-HEVC and SHVC extensions and various corrigenda items;

- For HEVC range extensions (RExt), the RExt conformance draft 2;

- For HEVC scalable extensions (SHVC), the SHVC extensions draft 7 and SHVC Test Model 7;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 2, SCC draft text 1, and a document specifying common test conditions and software reference configurations for SCC experiments;

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for 10–18 (Tue.-Wed.) Feb. 2015 under ITU-T auspices in Geneva, CH, during 19–26 June 2015 under WG 11 auspices in Warsaw, PL, during 13–21 Oct. 2015 under ITU-T auspices in Lucca, IT, and ….

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its nineteenth meeting during 17 – 24 Oct. 2014 at the Palais de la Musique et des Congres, Strasbourg, FR. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 17 Oct. 2014. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 24 Oct. 2014. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the new next-generation video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2014_10_S_Strasbourg/.

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the eighteenth JCT-VC meeting in producing

- For HEVC version 1, the HEVC test model (HM) 16, HEVC Defect Report for single layer coding (for Version 1), HEVC conformance testing draft 8, and the HEVC reference software for version 1 profiles;

- The text of HEVC versions 2, which subsumes the RExt, MV-HEVC and SHVC extensions;

- For HEVC range extensions (RExt), the RExt conformance draft 2;

- For HEVC scalable extensions (SHVC), the SHVC extensions draft 7 and SHVC Test Model 7;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 2, SCC draft text 1, and a document specifying common test conditions and software reference configurations for SCC experiments;

The other most important goals were to review the results from ten Core Experiments on Screen Content Coding (CE1-10), and review other technical input documents. Reviewing the progress made towards definition of screen content coding tools was one important topic of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Possible needs for corrections to version 2 were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 07 Oct. 2014.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 08 Oct. 2014 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-S0200 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "S0200+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-S0XXX (a proposal on …) [uploaded 10-XX],

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-S0XXX (an information document about …) [uploaded 10-XX],

- …

The following cross-verification reports were registered on time but were uploaded late: JCTVC-S0XXX [uploaded 10-XX], … .

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-S0XXX, … .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The initial uploads of the following contribution documents were rejected as a "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-S0141 (a proposal on wavefront design in combination with palette mode, corrected by a late upload on 10-09)

- JCTVC-S0188 (a proposal on combining two technologies from CE6, corrected by a late upload on 10-16)

- …

Furthermore, the initial version of JCTVC-S0056 was by mistake referring to a different technology. A corrected version was uploaded late on 10-09.

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases except for JCTVC-S0187 (where a corrected version only became available by 10-17).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-R1000, the HEVC Test Model 16 (HM16) JCTVC-R1002, the Edition 1 Defect Report JCTVC-R1003, the Version 1 Conformance Draft JCTVC-R1004, the Version 1 Reference Software JCTVC-R1011, the RExt Conformance Testing Draft 2 JCTVC-R1012, the SHVC draft specification 7 JCTVC-R1008, the SHVC test model 7 (SHM7) JCTVC-R1007, the Screen Content Coding (SCC) Draft Text 1 JCTVC-R1005, the SCC test model 2 JCTVC-R1014, and the common test conditions for SCC (JCTVC-R1015) were approved. The HM reference software and its extensions for RExt, SHVC and SCC were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site http://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – the first formal ballot stage of the approval process in ISO/IEC.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE).

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- OLS: Output layer set

- NAL: Network abstraction layer (as in AVC and HEVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Rancom-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment.

- SCCE: Screen content core experiment.

- SCM: Screen coding model.

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a term for a draft standard that may sometimes be used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma).

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU)

  - CB: Coding block (luma or chroma).

  - CU: Coding unit (containing both luma and chroma).

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma)

  - PU: Prediction unit (containing both luma and chroma), with eight shape possibilities.

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN: Having two areas that each have the full width and half the height of the CU.

    - Nx2N: Having two areas that each have half the width and the full height of the CU.

    - NxN: Having four areas that each have half the width and half the height of the CU.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP.

  - TB: Transform block (luma or chroma).

  - TU: Transform unit (containing both luma and chroma).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

Opening remarks included:

- Remarks on lateness of documents.

- Remarks on the number of documents.

- Meeting scheduling 0800-2000 for first few days

- Upcoming publication of HEVC version 2 – including RExt, SHVC and MV-HEVC (Last Call in ITU-T closing 2014-10-28, FDIS: SC 29 N 14494 submitted to ITTF 2014-10-07)

- Attendance

- Review and approval of agenda

- IPR policy reminder and review of communication practices

Primary topic areas were noted as follows:

- Screen content coding

- Corrigenda items for version 2? None identified.

Other

- Verification testing for interlaced video, RExt, & SHVC – No input

- Reference software and conformance, RExt & SHVC

- Test model texts and software manuals

Unfinished (or less-than-optimally finished) deliverables

- RExt conformance – check on that.

Key deliverables from this meeting

- PDAM on SCC? (finishing in 2015 would require issuing this at the current meeting)

- Conformance & reference software for v2?

Two main tracks were followed for most meeting discussions (to be defined):

- HLS

- SCC

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on SHVC HLS, SHVC and RExt CE work and related contributions during the first few days. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Fri. 17 Oct., 1st day

  - 0900-1230 JCT-VC opening and review of AHG reports [JRO & GJS]

  - 1400-2000 Tracks A (Arp 1) & B (Arp 3) CE review (and related contributions)

- Sat. 18 Oct., 2nd day 0800-2000

  - 0900-1130 JCT-3V opening plenary

  - 0800-1800 Track A BoG on CE6-related non-CE (Bob & Yu-Wen) in Arp 1

  - 0900-1300 Track B BoG on complexity assessment of IBC vector coding (Chao Pang and Shan) in Arp 3

  - 1400-1600 Track A (5.1.13 “Deblocking”, 5.1.14 “SCC Complexity assessment” (aspects not covered in the morning BoG), 5.1.15 “SCC parallel processing”, 5.1.16 “Other”)

  - 1400-1800 Track B (CEs, IBC improvements, review of IBC complexity BoG)

- Sun (ended 6p)

  - 0800-1800 Tracks A (5.3 SEI & VUI, 5.4 Non-normative, 3.4.3 SCC performance comparison with RExt)

  - 0830-1400 Track B (7.5 Merge list construction, 7.6 MV/DV derivation / coding, 7.7 Other (VSP, SDC, DBBP, DLT, ARP etc.))

  - 0800-1800 CE 6 BoG

- Mon

  - 0900-1400 WG 11 plenary

  - 1430-1545 VCEG

  - 1600-1800 Joint meeting with VCEG & MPEG parents

  - 1800-2000 Tracks A review of CE6-related BoG

  - 1800-2000 Track B CE4/CE9/CE10-related, IBC, ACT

- Tue

  - 0800-1000 Track A review of CE6-related BoG

  - 1000-1100 Joint JCT-VC & JCT-3V to discuss conformance & reference software

  - 1100-1300 Track A CE6-related BoG further discussions

  - 1400-1800 Brainstorm/panel event jointly sponsored by VCEG & MPEG

- Wed

  - 0900-1100 WG 11 plenary

  - 1130-1300 Wrap ups

  - 1400-1700 Wrap ups

  - 1700-1800 VCEG & MPEG future video coding

  - 1900-2200 Social event

- Thu

  - 1300-1400 VCEG

  - 0800-1300, 1415-1900 Wrap ups

- Fri

  - 0800-1300 Finalization

## Contribution topic overview (update)

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Chairing of other discussions is noted for particular topics. (Note: Allocation to tracks was subject to changes)

- AHG reports (15) Track P (section 2)

- Project development status (2) Track P (section 3)

- SCC CE1: Vector entropy copy (14) Track B (section 4.1)

- SCC CE2: IBC signalling and partitioning (12) Track B (section 4.2)

- SCC CE3: Sub-block partitioning and flipping for IBC (11) Track B (section 4.3)

- SCC CE4: Intra line copy (5) Track B (section 4.4)

- SCC CE5: Maximum palette size and predictor (6) Track A (section 4.5)

- SCC CE6: Palette mode improvements (27) Track A (section 4.6)

- SCC CE7: String matching for palette index coding (7) Track A (section 4.7)

- SCC CE8: Single-colour and two-colour modes (7) Track A (section 4.8)

- SCC CE9: Intra-boundary filtering and cross-component prediction (8) Track B (section 4.9)

- SCC CE10: Intra string copy (8) Track B (section 4.10)

- Non-CE SCC (127) (section 5.1) with subtopics

  - CE1 related 5.1.1 (2, Track B) [BoG Sat.]

  - CE2 related 5.1.2 (12, Track B)

  - CE3 related 5.1.3 (0, Track B)

  - CE4 related 5.1.4 (4, Track B)

  - CE5 related 5.1.5 (3, Track A)

  - CE6 related 5.1.6 (54, Track A, BoG)

  - CE7 related 5.1.7 (0)

  - CE8 related 5.1.8 (4, Track A)

  - CE9 related 5.1.9 (2, Track B)

  - CE10 related 5.1.10 (1, Track B)

  - IBC related 5.1.11 (5, Track B)

  - Adaptive colour transform 5.1.12 (14, Track B)

  - Deblocking 5.1.13 (8, Track A)

  - Complexity 5.1.14 (3, Track A)

  - Parallel processing 5.1.15 (8, Track A)

  - Other 5.1.16 (9, Track A)

- High-level syntax (0) Track A (section 5.2)

- VUI and SEI messages (4) Track A (section 5.3)

- Non-normative (7) Track A (section 5.4)

- Plenary discussions and BoG reports (3) Track P (section 7)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 8, 9, and 10)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

What's left:

- Missing documents (S0281)

- CE plan review

- Future meetings, etc.

# AHG reports (15)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Reviewed Fri 17th a.m., jointly chaired by GJS & JRO, except as noted otherwise.)

JCTVC-S0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

In the interim period since the 18th JCT-VC meeting, the following documents had been produced:

For HEVC version 1, the HEVC test model (HM) 16, HEVC Defect Report for single layer coding (for Version 1), HEVC conformance testing draft 8, and the HEVC reference software for version 1 profiles;

The text of HEVC edition 2, which subsumes the RExt, MV-HEVC and SHVC extensions and various corrigenda items;

For HEVC range extensions (RExt), the RExt conformance draft 2;

For HEVC scalable extensions (SHVC), the SHVC extensions draft 7 and SHVC Test Model 7;

For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 2, SCC draft text 1, and a document specifying common test conditions and software reference configurations for SCC experiments;

Furthermore, ten Core Experiments on screen content coding tools (CE1…10) were run. Advancing the work on development of conformance and reference software for HEVC extensions was also a significant goal.

The work of the JCT-VC overall had proceeded well and actively in the interim period with a considerable number of input documents to the current meeting. Active discussion had been carried out on the group email reflector (which had 1744 subscribers as of 2014-10-16), and the output documents from the preceding meeting had been produced.

Except as noted below, output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2014_06_R_Sapporo/), particularly including the following:

The meeting report (JCTVC-R1000) [Posted 2014-10-17]

The HM 16 encoder description (JCTVC-R1002) [Posted 2014-10-14] (which now contains major new parts of the RExt encoder description)

The HEVC (edition 1) defect report on single-layer coding (JCTVC-R1003) [First posted 2014-07-14, last updated 2014-07-15]

The HEVC conformance specification Draft 8, submitted as ISO/IEC FDIS 23008-8 and for ITU-T Last Call (JCTVC-R1004) [Posted 2014-08-25]

HEVC screen content coding draft 1 (JCTVC-R1005) [First posted 2014-08-09, last updated 2014-09-25]

SHVC Test Model 7 (JCTVC-R1007) [Posted 2014-10-17]

SHVC Draft 7 (separate text JCTVC-R1008), also integrated into HEVC 2nd edition [First posted 2014-07-10, last updated 2014-10-01]

HEVC reference software for version 1 profiles, submitted as ISO/IEC FDIS 23008-8 and for ITU-T Last Call (JCTVC-R1011) [Posted 2014-08-05]

Range Extensions conformance draft 2 (JCTVC-R1012) [Posted 2014-XX-XX]

Edition 2 Draft Text of High Efficiency Video Coding (HEVC), Including Format Range (RExt), Scalability (SHVC), and Multi-View (MV-HEVC) Extensions, submitted as ISO/IEC FDIS 23008-8 and for ITU-T Last Call (JCTVC-R1013) [First posted 2014-07-11, last updated 2014-10-01]

Screen Content Coding Test Model 2 (JCTVC-R1014) [Posted 2014-10-17]

Common SCC test conditions and software reference configurations (JCTVC-R1015) [First posted 2014-08-08, last updated 2014-08-16]

Description of Core Experiment 1 (CE1): Vector entropy coding (JCTVC-R1101) [First posted 2014-07-08, last updated 2014-08-09]

Description of Core Experiment 2 (CE2): Intra block copy signalling and partitioning (JCTVC-R1102) [First posted 2014-07-09, last updated 2014-08-09]

Description of Core Experiment 3 (CE3): Sub-block partitioning and flipping for Intra block copy (JCTVC-R1103) [First posted 2014-07-08, last updated 2014-08-01]

Description of Core Experiment 4 (CE4): Intra Line Copy (JCTVC-R1104) [First posted 2014-07-08, last updated 2014-08-09]

Description of Core Experiment 5 (CE5): Maximum Palette Size and Maximum Palette Predictor Size (JCTVC-R1105) [First posted 2014-07-09, last updated 2014-08-23]

Description of Core Experiment 6 (CE6): Palette Mode Improvement (JCTVC-R1106) [First posted 2014-07-09, last updated 2014-08-09]

Description of Core Experiment 7 (CE7): String Matching for Palette Index Coding (JCTVC-R1107) [First posted 2014-07-09, last updated 2014-08-17]

Description of Core Experiment 8 (CE8): Single-Colour and Two-Colour Modes (JCTVC-R1108) [First posted 2014-07-08, last updated 2014-08-15]

Description of Core Experiment 9 (CE9): Intra Boundary Filtering and Cross-Component Prediction Interdependency (JCTVC-R1109) [First posted 2014-07-08, last updated 2014-08-15]

Description of Core Experiment 10 (CE10): Intra String Copy (JCTVC-R1110) [First posted 2014-07-09, last updated 2014-07-12]

The fifteen ad hoc groups and the ten core experiments had made progress, and various reports from those activities had been submitted.

The different software modules (HM16 including RExt; SHVC/SHM and Screen Content/SCM) had been prepared and released with appropriate updates approximately as scheduled.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-16.1.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-16.0-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful. It was noted that contributions had generally been submitted that were relevant to resolving the more difficult cases that might require further review.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

Approximately 270 input contributions to the current meeting had been registered. A significant number of late-registered and late-uploaded contributions were noted, even though most were cross-check documents.

A preliminary basis for the document subject allocation and meeting notes for the 19th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site.

JCTVC-S0002 HEVC test model editing and errata reporting (AHG2) B. Bross, K. McCann C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang, (vice-chairs)

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 18th meeting in Sapporo (30 June – 9 July 2014) and the 19th meeting in Strasbourg (17–24 October 2014).

The HM15 Encoder Description in document JCTVC-Q1002 and HEVC Edition 1 Defect Report Draft 4 in document JCTVC-Q1003 were approved as JCT-VC output documents at the 18th JCT-VC meeting.

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The HM16 Encoder Description was published as JCTVC-R1002.  This document represented a merging and enhancement of the previous HM Encoder Description and RExt Encoder Description documents.  The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

The HEVC Edition 1 Defect Report for Single-Layer Coding was published as JCTVC-R1003.

The HEVC Edition 2 Draft Text of High Efficiency Video Coding (HEVC), including Format Range (RExt), Scalability (SHVC), and Multi-View (MV-HEVC) Extensions, was published as JCTVC-R1013.

The recommendations of the HEVC test model editing and errata reporting AHG were for JCT-VC to:

Approve the HM16 Encoder Description in document JCTVC-R1002 as JCT-VC output

Approve the HEVC Edition 1 Defect Report for Single-Layer Coding in document JCTVC-R1003 as JCT-VC output

Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the Encoder Description

Identify and resolve any residual issues relating to mismatches between software and text

JCTVC-S0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [K. Suehring (Chair), K. Sharman, D. Flynn]

(Chaired Wed 10-22 p.m. by GJS)

This report summarizes the activities of the AhG on HEVC HM software development and software technical evaluation that have taken place between the 18th and 19th JCT-VC meetings. Activities focused on merging the Range Extension software into the main branch, code tidying and fixing bugs.

A brief summary of activities related to each mandate is given below.

Development of the software was co-ordinated with the parties needing to integrate changes. A single track of development was pursued. The distribution of the software was made available through the SVN server set up at HHI, as announced on the JCT-VC email reflector.

The HM user manual has been updated and a version controlled copy is included in the doc directory of the repository. A PDF version has been produced and is included in the same location prior to each HM release.

Version 16.2 is was released during the 19th JCT-VC meeting. A number of bugs had been identified and fixed.

There are a number of additional reported software bugs that should be fixed.

A detailed history of all changes made to the software can be viewed at https://hevc.hhi.fraunhofer.de/trac/hevc/timeline.

Released versions of the software are available on the SVN server at the following URL: 
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,

where version_number corresponds to one of the versions described below (eg., HM-16.0). Intermediate code submissions can be found on a variety of branches available at:

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,

where branch_name corresponds to a branch (eg., HM-11.0-dev).

HM 15.0_RExt8.0 was released on 18th July and incorporated the most of the agreed changes decided at the meeting. The coding performance slightly changed compared to HM15.0_RExt7.3 for due to the adopted contributions JCTVC-R0104 (removal of Hadamard in lossless coding) and JCTVC-R0105 (motion estimation starting point).

HM 15.0_RExt8.1 was released on 13th August. It included macro-removal and minor bug fixes. The coding performance did not change compared to HM15.0_RExt8.1 for common test conditions.

This was the last release of a software model dedicated for the development of the Range Extension (RExt) work area.

HM 16.0 was released on 19th August. It was essentially the same as HM15.0_RExt8.1, although the documentation was updated. There were no coding performance differences between HM15.0_RExt8.1 and HM16.0.

Following this version, the branch for continued HM development is HM-dev, rather than creating a branch specifically for the development of each HM release.

HM 16.1 was released on 19th September. It included the updating of documentation and some minor bug-fixes. Run-time information on the use of scaling list files was also added. It also included the agreed Knee function SEI message changes.

There were also two large changes made to the software: remnants of code relating to fine granularity slices were completely removed, and CTU-level variables and functions were renamed to correspond more closely with the specification and to make their use more transparent.

There were no coding differences between HM16.0 and HM16.1.

HM 16.1.1 was released on 24th September. This was a minor release, where the trace-function output was reverted to that used in HM, rather than some being output to a file and some being output to ‘stdout’.

HM 16.2 was released during the 19th JCT-VC meeting. It includes many minor field-coding bug-fixes. It also includes a significant slice-level code tidy, which has removed many Sbac encoders and decoders and has unified wavefront-parallel-processing and tile implementations, addressing many bugs en-route. In addition effort has been expended in removing unused code. The RExt backwards-compatibility macros have been removed, and most of the RExt macros have been renamed, and RExt comments (including NOTEs and TODOs) have been updated to highlight the integration of the two models.

Unless the release has been tagged, the development branch can be found under

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/HM-dev

Minor performance changes are expected between HM16.2 and HM16.1, due to the removal of the backwards-compatibility macros (which causes JCTVC-R0105 (see above) to be used), and due to the resetting of frac-bits at the start of slices.

Performance changes between HM16.2 and HM15.0 were shown in the report. There has been an increase in run-time due to the merger with the RExt code-base although the encoder has been aided by incorporating the faster inter decision process (JCTVC-R0105). The changes between HM16.2 and HM15.0_RExt7.3 are similar.

The AHG recommended to:

- Continue to develop reference software based on HM version 16.2 and improve its quality.

- Test reference software more extensively outside of common test conditions

- Add more conformance checks to the decoder to more easily identify non-conforming bitstreams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bitstreams that trigger bugs in the HM. Such bitstreams may also be useful for the conformance specification.

A key reported bug was concerning cross-component prediction. To the best of our knowledge, the degree of bit depth expansion is unintentional and should be fixed by a normative bitstream constraint to be specified in the next edition of the standard. However, our state of understanding of exactly how to introduce this constraint is not yet mature.

Decision: Add to the SCC draft text, an expression of a normative constraint associated with cross-component prediction to constrain the input to cross-component prediction to a 16 bit range when the extended precision inverse transform is disabled (constraining MinCoeff to MaxCoeff [check variable name]).

JCTVC-S0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, J. Boyce, K. Kazui, A. K. Ramasubramonian, Y. Ye]

(Chaired Wed 10-22 p.m. by GJS)

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

The guideline to generate the conformance bitstreams is summarized in JCTVC-O1010.

After the Sapporo meeting, some problems with the available bitstreams were reported. The revised bitstreams were uploaded at the following site, separating the bitstreams under ballot.

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/under_test/

Descriptions of some reported problems with the previously specified conformance tests bitstreams were provided in the report. These included:

- general_profile_idc having the reserved value 0.

- After end_of_slice_segment_flag with value 1, having non-zero data in a slice segment byte stream NAL unit.

- Having extension flags set in parameter sets and/or slice headers (this makes streams non-conforming to v1 but was intentional to test v1 decoders). In the conformance spec, the following clarification was added: "This bitstream does not conform with Main profile and Main10 profile since sps_extension_flag is equal to 1. However, Main profile and Main 10 profile decoders shall be able to decode this bitstream and shall ignore the SPS extension."

In discussion, it was asked whether the extension data conforms to version 2. This should be checked.

- Having read_bits( 1 ) called more times than maximum of 5 * RawCtuBits / 3 = 5120 times.

- Having a PPS that precedes the SPS it refers to in the bitstream (although both are present prior to their activation), which seems conforming was asked to be confirmed).

In discussion, it was agreed that this is not an error, and in fact this is a desirable characteristic to have in some bitstreams – so that we can make sure decoders will work with this NAL unit order.

- Having general_profile_compatibility_flag[ 0 ] not equal to 0.

Corrected bitstreams were made available on the ftp site.

Other conformance development issues were noted, including:

- A new bitstream was made available to test SAO corner cases.

In discussion, it was agreed that adding such a bitstream is desirable.

- There are some originally planned bitstream testing features that were still not covered.

Plans for RExt and SHVC conformance testing were described. See notes on joint meeting in section 7.2.

It was agreed that the available new bitstream and corrected bitstreams should be included in the next amendment / edition.

The AHG recommended to:

- approve the corrections of HEVC v.1 conformance bitstreams

- discuss and clarify the open questions to HEVC v.1 conformance bitstreams

- discuss how new bitstreams for HEVC v.1 be added

- discuss the further plan of HEVC extensions conformance test

- update the guideline of the bitstream generation for RExt and SHVC conformance

These recommendations were agreed.

JCTVC-S0005 Verification test preparation (AHG5) V. Baroncini (chair), M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, K. Sharman, T. K. Tan, J.-M. Thiesse, W. Wan (vice-chairs)

This report summarizes the Verification test preparation Ad-Hoc Group (AHG5) between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg (October 2014).

| Class | Test sequence |
| --- | --- |
| YCbCr 4:4:41 | Traffic_2560x1600_30_10bit_444_crop.yuv |
|  | EBULupoCandlelight_1920x1080_50_10bit_444.yuv2 |
|  | EBURainFruits_1920x1080_50_10bit_444.yuv2 |
|  | VenueVu_1920x1080_30_10bit_444.yuv |
|  | BirdsInCage_1920x1080_60_10bit_444.yuv |
|  | CrowdRun_1920x1080_50_10bit_444.yuv |
| YCbCr 4:2:0i | CattleDogs_1920x1080_420_60i_8bit.yuv |
|  | WaveRocks_1920x1080_420_60i_8bit.yuv |
|  | NewMobCal_1920x1080_420_50i_10bits.yuv3 |
|  | ParkRun_1920x1080_420_50i_10bits.yuv3 |
|  | SthlmPan_1920x1080_420_50i_10bits.yuv3 |

Notes:

- 4K 10-bit YCbCr in BT.709 test material cropped to HD is desired for the 4:4:4 test, to match display device limitations.

- These sequences are believed to be encoded in BT.2020.

- Sourced from SVT archive and with interlace-aware 4:2:2 -> 4:2:0 downsampling applied (via ChromaConvert utility).

Two sequences ‘Basketball’ and ‘Tennis’ may also be available.

Software

- HM-16.0 software with the following patches to be used:

  - Interlace PSNR patch1 (see ticket 1282).

  - Delta-QP for POC #1 setting via configuration setting2.

  - “SecondFieldBias” patch to increment GOP ‘depth’ based on POC and GOP size2.

- JM-18.6 software with no further patching required:

Notes:

1. Merged into future HM release.

2. Patches obtained from modified HM-13 software as prepared in the MPEG interlace AHG.

Encoding conditions notes:

- A "minimal" testset could be:

  - 4 (bit-rates) * 1 (RA) * 5 (4:2:0 8-bit sequences) = 20 streams

  - 4 (bit-rates) * 1 (RA) * 3 ( 4:2:2 10-bit sequences) = 12 streams

  - 4 (bit-rates) * 1 (RA) * 3 (4:4:4 10-bit sequences) = 12 streams

  - Total: 54 streams

- Resolution to test: Only HD (1080i50 or 60)?

  - Not sure if any current consumer display can handle 4K in 10-bits. Cropping 4K material to HD may be easier.

- Suggested bit-rates:

  - RA 4:2:0i 8-bit in HEVC: 1.5Mbps, 3Mbps, 5Mbps, 8Mbps (in AVC, double these bit-rates)

  - RA 4:2:2 10-bit in HEVC: 5Mbps, 8Mbps, 12Mbps, 16Mbps1

  - RA 4:4:4 10-bit in HEVC: TBD

- For RA, 1 second GOP selected.

- Addition of LD for 4:2:0 sequences could be added depending on resources of Vittorio and others contributors. During discussion, it was questioned whether this case is useful, since low-delay encoding does not seem widely used for interlaced content.

- The SAFF tool has a risk of artefacts at switchover points and current test material has no scene changes, so there is less likelihood of switching between field and frame coding.

- For JM, MBAFF and PicAff were enabled.

Notes:

- It was remarked that for higher bit rates, e.g. as used in contribution, MOS scores for the multiple QP points may all be high and subjective quality may be indistinguishable. In such cases, a comparison using objective metric may be appropriate, i.e. PSNR for the overall comparison and another metric to check for the absence of larger (visually significant) local variations.

Bitstream exchange

- Some SAFF-related materials used in earlier interlace study were identified and collected.

- QP selection for the 4:2:0i test sequences is underway.

Post-processing notes:

For 4:2:0 de-interlacing, use "off-line" de-interlacer so that a progressive YUV sequence can be played back for the subjective test. This avoids any temporal interaction resulting from difficulty in synchronising playback with the built-in hardware de-interlacer in the display.

The ffmpeg software has several de-interlacers and two candidates are suggested to be ‘w3fdif’ and "mcdeint".

Recommendations:

- Acquire more test sequences for the subjective test.

- Finalize test conditions and prepare bitstreams.

- Conduct the subjective test.

It was suggested to produce a draft test plan.

Draft test plan review (Wed a.m. GJS)

(Move notes to appropriate place in the report.)

A draft test plan was drafted and reviewed.

One aspect discussed was low delay interlace – what it means and how relevant it is. This will be considered in future development of the test plan.

JCTVC-S0006 SCC coding performance analysis (AHG6) H. Yu (chair), R. Cohen, A. Duenas, S. Liu, K. Rapaka, J. Xu (vice-chairs)

This report summarizes the activities of the JCT-VC ad hoc group on SCC coding performance analysis (AHG6) between the JCT-VC 18th meeting in Sapporo, Japan, and the 19th meeting in Strasbourg, France.

JCTVC-R1015 “Common conditions for screen content coding tests” was produced based on the agreements captured in the meeting notes. The AHG revised the common test condition described in JCTVC-Q1015 with minimum changes necessary to reflect SCM 2.0 related information. In particular, the following new SCM2.0 encoder settings were used for generating the anchors:

- IntraBlockCopyEnabled = 1

- HashBasedIntraBlockCopySearchEnabled = 1

- IntraBlockCopySearchWidthInCTUs = -1 (full frame)

- IntraBlockCopyNonHashSearchWidthInCTUs = 1

- ColorTransform = 1

- PaletteMode = 1

Results reporting templates were updated with the SCM2.0 anchor data.

For the alternative IBC operating point with constrained search range defined and specified in the JCTVC-R-Notes_dB document, the related SCM 2.0 macro settings are not included in R1015. However, was a recommendation in email on the JCTVC reflector to use the following configuration for this:

- IntraBlockCopyEnabled = 1

- HashBasedIntraBlockCopySearchEnabled = 1

- IntraBlockCopySearchWidthInCTUs = 3 (current CTU and 3 more to its left)

- IntraBlockCopyNonHashSearchWidthInCTUs = 3

- ColorTransform = 1

- PaletteMode = 1

No change was made in JCTVC-R1015 towards analysis methods for studying SCC coding performance. However, comments were made during the last meeting that suggested changes and improvements to the Excel templates for making comparisons among multiple proposals easier, e.g. using bar charts only showing average across text and graphics and mixed content. In fact, bar charts continue being used in a number of proposals this time. It was reported to be desirable to create a unified solution and include such a solution in the test results reporting template.

JCTVC-S0042 “AHG6 Inclusion of 420 screen content in the test conditions” proposes that 4:2:0 screen content, such as sequences in Class F sequences should be added to common test condition.

- In discussion, it was noted that in the WD, we have the following

  - IBC can operate in a non-4:4:4 mode (inherited from prior RExt design)

  - CCP, ACT, and palette can only operate in 4:4:4 mode

- Prior palette proposals have included a 4:2:0 operation method, but this is not in the WD

- CCP also does not operate in non-4:4:4 formats

- Requirements have not been established for non-4:4:4. Parent-level discussion would be needed to consider this.

JCTVC S0100 “AHG6: On Adaptive Color Transform (ACT) in SCM2.0” recommends that ACT should be disabled in common test conditions for YUV sequences. Changes were also made to the encoder to only evaluate ACT on for RGB content, such that the decoded CU-level ACT flags are always 1. It is reported that for coding RGB content, forcing CU-level ACT flags always to be on, preserved most of the gains achieved by CU-adaptive ACT. The main arguments are that from the provided test results, it is asserted that “ACT itself does not provide much benefit for YUV content”, and on RGB content, testing both CU-level on and off provides only 1% gain over only testing CU-level on. The proposal states that both suggestions would significantly reduce simulation runtime, while preserving coding efficiency.

Various tests have been designed in the CEs to analyze the coding efficiency as well as the associated complexity of the respective tools under test. In addition, a large number of non-CE contributions to his meeting report coding efficiency gain and provide complexity analysis data. In general, it is recommended that the coding performance of a particular coding tool or mode should be measured and described by both coding efficiency and associated complexity. Several experts have also suggested that the interaction of the newly adopted tools with the existing HEVC tools should be tested.

JCTVC-S0042 reports the test results that came from running SCM-2.0 with Class F sequences. It states that coding gain from IBC has been observed. Note however, that the Class F sequences were captured using an analog cable, and therefore the screen-content tools that were designed to work well on perfectly flat or limited-color blocks may be affected by noise in the analog-captured signal.

JCTVC-S0100 reports the results from two experiments designed for testing ACT. In the first test, it is asserted that the effects of ACT on YUV content mostly came from encoder-side bypassing IBC early termination, rather than form ACT itself. In the second test, it is asserted that for RGB content, the CU-level adaptive color transform only provides about 1% gain over the case where the color transform is executed for every CU encoded in the bitstreams.

JCTVC-S0069 proposes to enable the colour transform for Inter modes at sequence parameters set instead of enabling it at CU level. A BDR average of 0% and -0.1% compared to SCM2.0 for respectively RA and LDB configurations is reported with an encoding run time of 95% and 96%.

The AHG recommended to

- Study the proposed changes to the common test condition in S0042 and S0100, and create CEs if needed to further study these proposals.

- Discuss additional methods/formats for reporting test results.

- Continue to evaluate the coding performance of the newly adopted tools and their interaction with the existing HEVC tools in the Main profile and range extensions.

In further discussion on Wed 10-22 (GJS), the following simplified format was suggested for summary tables:

RGB, text & graphics with motion, 1080p & 720p

RGB, mixed content, 1440p & 1080p

RGB, Animation, 720p

RGB, camera captured, 1080p

YUV, text & graphics with motion, 1080p & 720p

YUV, mixed content, 1440p & 1080p

YUV, Animation, 720p

YUV, camera captured, 1080p

Enc Time[%]

Dec Time[%]

In further discussion 10-23 (JRO), it was agreed that this shall be done.

It was remarked that two of the 720p sequences have extremely low bit rates.

JCTVC-S0007 JCT-VC AHG report: SCC extensions text editing (AHG7) [R. Joshi, J. Xu (AHG co-chairs), Y. Ye, S. Liu, R. Cohen, Z. Ma (AHG vice-chairs)]

This document reports on the work of the JCT-VC ad hoc group on SCC extensions text editing (AHG7) between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014).and the 19th JCT-VC meeting in Strasbourg, FR (October 2014).

The first working draft for the High Efficiency Video Coding Screen Content Coding (HEVC SCC) extension was created as an output document following the decisions taken at the 18th JCT-VC meeting in Sapporo, JP (June-July 2014).

Three versions of JCTVC-R1005 were published by the editing ad hoc group (AHG) following the 18th JCT-VC meeting in Sapporo, JP, which were based on the seventh draft of HEVC range extension, i.e., JCTVC-Q1005_v9.

List of changes with respect to JCTVC-Q1005_v9:

- Integrated changes related to intra block copying as in JCTVC-P1005-v4

- Integrated IBC with changes in block vector predictor and block vector difference coding (JCTVC-R0309 and JCTVC-R0186)

- Integrated palette mode (JCTVC-R0348 + scan simplification + CU level escape flag)

- Integrated adaptive colour transform (JCTVC-R0147)

The screen content coding test model 2 (SCM 2) (document JCTVC-R1014) was released on 16th October, 2014. General tutorial descriptions for palette mode, residual adaptive colour transform and intra block copying were added. For the palette mode and the residual adaptive colour transform, the working of the encoder was also described in greater detail.

Recommendations:

- Approve the documents JCTVC-R1005 and JCTVC-R1014

- Resolve editorial comments in the SCC extensions text specification as appropriate (not necessarily as in-meeting activity, as these are only editorial)

- Compare the HEVC SCC extensions document with the HEVC SCC extensions software and resolve any discrepancies that may exist, in collaboration with the SCC extension software development (AHG8)

JCTVC-S0008 SCC extensions software development (AHG8) B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu, M. Xu (vice-chairs)

This report summarizes the activities of Ad Hoc Group 8 on screen content extensions software (SCM) development that have taken place between the 18th and 19th JCT-VC meetings.

Multiple versions of the HM SCM software were produced and SCM2.0 was announced on the JCT-VC email reflector. The integration details and performance summary of these revisions are provided in the next subsections. The performance results of software revisions were observed to be consistent with the adopted techniques.

HM15.0_RExt8.0_SCM2.0 has been tagged on the SVN HHI repository on August 1st. This release includes following adoptions:

- JCTVC-R0102 - Fix and improvement for hash based inter search

- JCTVC-R0104 - Removal of Hadamard transform in lossless coding.

- JCTVC-R0105 - Motion estimation starting point

- JCTVC-R0147 - Adaptive in-loop color transform

- JCTVC-R0162 - Intra block copy hash search chroma refinement

- JCTVC-R0186 - Block vector coding improvements for Intra block copy

- JCTVC-R0309 - Block vector prediction improvements for Intra block copy +

- JCTVC-R0081 - Encoder bug fixes

- JCTVC-R0348 - Palette coding tool

- Migration to HM-15.0+RExt-8.0

In addition as per the meeting discussion, the support for flexible intra block copy search range has been provided. This enables CE specific Intra BC search configurations that are different from to CTC.

The release was announced on the email reflector. The software can be downloaded at

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.0+SCM-2.0/

The performance of this version against HM-14.0+RExt-7.3+SCM-1.1 was described according to the common test conditions in JCTVC-R1015. For the lossy configuration, it is reported that this version provides BD-rate reduction of 33.0%, 23.4% and 18.1% for RGB 1080p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 31.3%, 17.6% and 11.9% for YUV 1080p text and graphics category in AI/RA/LB configuration, respectively. For the lossless configuration, it is reported that this version provides BD-rate reduction of 19.6%, 11.1% and 10.4% for RGB 1080p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 22.6%, 10.8% and 10.0% for YUV 1080p text and graphics category in AI/RA/LB configuration, respectively.

BD-Rate change in Lossy configuration relative to prior SCC test model

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -33.0% | -29.0% | -28.7% |
| RGB, text & graphics with motion,720p | -30.9% | -20.5% | -23.5% |
| RGB, mixed content, 1440p | -26.2% | -15.2% | -14.9% |
| RGB, mixed content, 1080p | -23.5% | -13.6% | -13.8% |
| RGB, Animation, 720p | -25.3% | -18.1% | -15.4% |
| RGB, camera captured, 1080p | -25.3% | -5.1% | -9.9% |
| YUV, text & graphics with motion, 1080p | -31.3% | -33.7% | -32.8% |
| YUV, text & graphics with motion,720p | -17.8% | -22.8% | -26.5% |
| YUV, mixed content, 1440p | -9.3% | -15.8% | -16.0% |
| YUV, mixed content, 1080p | -11.9% | -17.2% | -17.4% |
| YUV, Animation, 720p | 0.0% | -8.5% | -5.5% |
| YUV, camera captured, 1080p | 0.1% | -0.1% | -0.1% |
| Enc Time[%] | 148% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -23.4% | -18.6% | -18.3% |
| RGB, text & graphics with motion,720p | -30.5% | -17.5% | -22.2% |
| RGB, mixed content, 1440p | -31.4% | -16.4% | -15.8% |
| RGB, mixed content, 1080p | -25.3% | -13.1% | -12.9% |
| RGB, Animation, 720p | -25.6% | -16.2% | -11.8% |
| RGB, camera captured, 1080p | -27.9% | -5.4% | -14.1% |
| YUV, text & graphics with motion, 1080p | -17.6% | -21.2% | -20.7% |
| YUV, text & graphics with motion,720p | -14.8% | -19.1% | -24.0% |
| YUV, mixed content, 1440p | -7.1% | -14.9% | -15.0% |
| YUV, mixed content, 1080p | -8.6% | -15.3% | -15.0% |
| YUV, Animation, 720p | 0.0% | -8.3% | -4.7% |
| YUV, camera captured, 1080p | 0.4% | -0.1% | 0.1% |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -18.1% | -13.3% | -12.9% |
| RGB, text & graphics with motion,720p | -26.4% | -11.6% | -16.4% |
| RGB, mixed content, 1440p | -30.5% | -12.3% | -11.0% |
| RGB, mixed content, 1080p | -23.0% | -6.8% | -6.6% |
| RGB, Animation, 720p | -24.3% | -11.5% | -5.1% |
| RGB, camera captured, 1080p | -26.2% | -1.7% | -12.1% |
| YUV, text & graphics with motion, 1080p | -11.9% | -15.9% | -15.4% |
| YUV, text & graphics with motion,720p | -9.5% | -14.0% | -17.9% |
| YUV, mixed content, 1440p | -4.2% | -12.5% | -12.7% |
| YUV, mixed content, 1080p | -4.2% | -13.0% | -11.9% |
| YUV, Animation, 720p | -0.1% | -6.4% | -4.7% |
| YUV, camera captured, 1080p | 0.3% | 0.0% | -0.2% |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 106% |  |  |

BD-Rate change in Lossless configuration relative to prior SCC test model

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 19.6% | 19.7% | 13.6% | 28.2% |
| RGB, text & graphics with motion,720p | 13.5% | 11.9% | 2.4% | 26.1% |
| RGB, mixed content, 1440p | 4.4% | 4.4% | 4.3% | 4.5% |
| RGB, mixed content, 1080p | 4.2% | 4.2% | 4.2% | 4.2% |
| RGB, Animation, 720p | 4.3% | 4.3% | 4.3% | 4.3% |
| RGB, camera captured, 1080p | 0.4% | 0.2% | -0.6% | 1.0% |
| YUV, text & graphics with motion, 1080p | 22.6% | 22.6% | 12.5% | 32.9% |
| YUV, text & graphics with motion,720p | 12.4% | 11.9% | 1.1% | 23.2% |
| YUV, mixed content, 1440p | 2.4% | 2.5% | 2.0% | 3.1% |
| YUV, mixed content, 1080p | 2.8% | 2.8% | 2.8% | 2.8% |
| YUV, Animation, 720p | 1.7% | 1.7% | 1.7% | 1.7% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 177% |  |  |  |
| Dec Time[%] | 94% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 11.1% | 13.5% | 8.5% | 20.9% |
| RGB, text & graphics with motion,720p | 3.4% | 7.1% | 1.7% | 12.7% |
| RGB, mixed content, 1440p | 2.3% | 2.3% | 2.1% | 2.5% |
| RGB, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.9% |
| RGB, Animation, 720p | 1.8% | 1.8% | 1.8% | 1.8% |
| RGB, camera captured, 1080p | -0.2% | -0.2% | -0.4% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.8% | 15.1% | 10.3% | 24.2% |
| YUV, text & graphics with motion,720p | 3.0% | 7.3% | 0.9% | 16.2% |
| YUV, mixed content, 1440p | 1.3% | 1.3% | 1.3% | 1.3% |
| YUV, mixed content, 1080p | 0.5% | 0.5% | 0.5% | 0.5% |
| YUV, Animation, 720p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |  |
| Dec Time[%] | 106% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, text & graphics with motion, 1080p | 10.4% | 10.9% | 7.1% | 14.7% |
| RGB, text & graphics with motion,720p | 2.5% | 5.2% | 1.5% | 9.0% |
| RGB, mixed content, 1440p | 1.6% | 1.6% | 1.5% | 1.7% |
| RGB, mixed content, 1080p | 0.3% | 0.3% | 0.3% | 0.3% |
| RGB, Animation, 720p | 1.8% | 1.8% | 1.8% | 1.8% |
| RGB, camera captured, 1080p | -0.2% | -0.2% | -0.4% | 0.0% |
| YUV, text & graphics with motion, 1080p | 10.0% | 11.9% | 9.0% | 16.8% |
| YUV, text & graphics with motion,720p | 2.2% | 4.8% | 0.6% | 10.0% |
| YUV, mixed content, 1440p | 0.6% | 0.6% | 0.2% | 1.0% |
| YUV, mixed content, 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.6% | 0.6% | 0.6% | 0.6% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |  |
| Dec Time[%] | 106% |  |  |  |

HM15.0_RExt8.1_SCM2.1rc1 has been tagged on the SVN HHI repository on October 15th. This release includes:

- SCM source base updated to HM15.0_RExt8.1.

- Removal of unused macros.

- Fix for Ticket 1308

- Fix related to compilation issues when DEBUG_STRING is enabled.

The software can be downloaded at

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-15.0+RExt-8.1+SCM-2.0rc1/

HM16.0_SCM2.1 and HM16.1_SCM2.1 have been committed on the SVN HHI repository on October 17th that updates SCM source to HM16.0 and HM16.1.

The coding performance of HM15.0_RExt8.1_SCM2.1rc1, HM16.0_SCM2.1rc1 and HM16.1_SCM2.1rc1 was observed to similar compared to HM15.0_RExt8.1_SCM2.1.

It is planned to tag and announce these version on the email reflector during this meeting cycle after removal of SCM2.0 macros.

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ has been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-3.0, version SCC-2.0.

Recommendations

- Continue to develop reference software based on HM16.1_ SCM2.1 and improve its quality.

- Remove macros introduced in previous versions before starting integration towards SCM-3.0 such as to make the software more readable.

- Continue merging with later HM versions.

JCTVC-S0009 Complexity of palette mode coding (AHG9) A. Duenas (chair), R. Joshi, S.-H. Kim, X. Xiu (vice-chairs)

This document reports on the work of the JCT-VC ad hoc group on “Complexity of palette mode coding (AHG9)” between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg, FR (October 2014).

The AhG used the JCT-VC reflector for all discussions. A kick-off message was sent on July 16th, 2014. No other coordinated AhG activity took place, and no other emails were exchanged over the JCT-VC reflector between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg, FR.

The following four contributions were noted to be relevant:

- JCTVC-S0110 Non-CE6: Syntax Redundancy Fixes for Palette Mode [W. Pu, M. Karczewicz,V. Seregin, F. Zou, R. Joshi, J. Sole (Qualcomm)]

- JCTVC-S0134 Non-CE6: Simplified palette size coding [J. Zhao, S.-H. Kim, K. Misra, A. Segall (Sharp)]

- JCTVC-S0173 Non-CE6: Redundancy removal and simplification for Palette coding [Y. He, X. Xiu, Y. Ye (InterDigital)]

- JCTVC-S0201 Non-CE5: CU dependent color palette maximum size [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)]

JCTVC-S0010 JCT-VC AHG report: Complexity of IBC, intra line & intra string copy coding (AHG10) [J. Sole (chair), S. Liu, J. Xu (vice-chairs)]

This document reports on the work of the JCT-VC ad hoc group on “Complexity of IBC, intra line & intra string copy coding (AHG10)” between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg, FR (October 2014).

Analysis related to memory bandwidth and CABAC throughput have been carried out during this meeting cycle.

A software patch and excel sheets were released by Chun-Chi Chen for intra BC and intra line copy based on the memory bandwidth tool that had been used in SCCE1 (JCTVC-Q1121) for memory bandwidth calculations. The software includes in the SCM2.0 decoder the functionality to calculate the read/write memory access bandwidth per frame for "motion compensation" and "intra block copy" under various memory configurations.

In the context of CE4, the average results for different configurations and memory access parameters are provided.

The worst-case memory bandwidth has been computed using the formula

where m × n is the memory tile size (memory pattern) and M×N the accessed block size. Results are summarized in the table below:

|  | Configurations |  |  |  |  |  |  | Per-pixel Memory Access Bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Block Size |  | Num. of Samples | IF Length | Bi Pred? | Memory Pattern | m= | 4 | 8 | 4 | 8 | 4 |
| Prediction Mode | M | N | MxN | L | Y or N |  | n= | 1 | 1 | 2 | 2 | 4 |
| 8x8, Bi-prediction | 8 | 8 | 64 | 8 | Y |  |  | 9.375 | 11.25 | 10 | 12 | 12.5 |
| 4x4, IntraBC | 4 | 4 | 16 | 1 | N |  |  | 2 | 4 | 3 | 6 | 4 |
| 1x4, IntraLC (R0132) | 1 | 4 | 4 | 1 | N |  |  | 4 | 8 | 6 | 12 | 8 |

The CABAC throughput has also been analyzed for intra BC and intra line copying. The table below provides the worst-case scenario of context coded bins for intra BC and intra LC. The main difference between both methods were highlighted in the report.

| Prediction Mode | 4x4, IntraBC (SCM-2.0) |  | 4x1, IntraLC (R0132) |  |
| --- | --- | --- | --- | --- |
| (Note: CU Size = 8x8) | Syntax Elements | Number | Syntax Elements | Number |
| Mode Flag | cu_transquant_bypass_flag | 1 | cu_transquant_bypass_flag | 1 |
|  | cu_skip_flag | 1 | cu_skip_flag | 1 |
|  | intra_bc_flag | 1 | intra_bc_flag | 1 |
|  |  |  | intra_lc_flag | 1 |
|  | pred_mode_flag | 1 | pred_mode_flag | 1 |
|  | palette_mode_flag | 1 | palette_mode_flag | 1 |
| Partition | part_mode | 3 | part_mode | 3 |
| MV or BV | abs_bvd_greater0_flag | 8 | abs_mvd_greater0_flag | 32 |
|  | mvp_l0_flag | 4 | abs_mvd_greater1_flag | 32 |
| Residual (4 4x4 TBs, 3 Color Components) | rqt_root_cbf | 1 | rqt_root_cbf | 1 |
|  | cu_residual_act_flag | 1 | cu_residual_act_flag | 1 |
|  | cu_qp_delta_abs | 5 | cu_qp_delta_abs | 5 |
|  | cu_chroma_qp_offset_flag | 1 | cu_chroma_qp_offset_flag | 1 |
|  | cu_chroma_qp_offset_idx | 5 | cu_chroma_qp_offset_idx | 5 |
|  | log2_res_scale_abs_plus1 | 32 | log2_res_scale_abs_plus1 | 32 |
|  | res_scale_sign_flag | 8 | res_scale_sign_flag | 8 |
|  | cbf_cb | 5 | cbf_cb | 5 |
|  | cbf_cr | 5 | cbf_cr | 5 |
|  | cbf_luma | 4 | cbf_luma | 4 |
|  | split_transform_flag | 1 | split_transform_flag | 1 |
|  | transform_skip_flag | 12 | transform_skip_flag | 12 |
|  | explicit_rdpcm_flag | 12 | explicit_rdpcm_flag | 12 |
|  | explicit_rdpcm_dir_flag | 12 | explicit_rdpcm_dir_flag | 12 |
|  | last_sig_coeff_x_prefix | 36 | last_sig_coeff_x_prefix | 36 |
|  | last_sig_coeff_y_prefix | 36 | last_sig_coeff_y_prefix | 36 |
|  | sig_coeff_flag | 180 | sig_coeff_flag | 180 |
|  | coeff_abs_level_greater1_flag | 96 | coeff_abs_level_greater1_flag | 96 |
|  | coeff_abs_level_greater2_flag | 12 | coeff_abs_level_greater2_flag | 12 |
| Number of Ctx-coded Bins |  | 484 |  | 537 |
| Bins per pixel |  | 7.56 |  | 8.39 |

The recommendations of the AhG were to

- Consider the data provided in the context of this AhG

- Review the contributions related to complexity of intra block, line and string copy.

  - JCTVC-S0136, Non-CE4: On CABAC Throughput of Intra Line Copy

  - JCTVC-S0145, On IntraBC bandwidth

JCTVC-S0011 SHVC text editing (AHG11) J. Chen (chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs)

This document reports the work of the JCT-VC ad hoc group on SHVC text editing (AHG11) between the 18th JCT-VC meeting in Sapporo, Japan (30 June – 9 July 2014) and the 19th JCT-VC meeting in Strasbourg, France (17 – 24 October 2014).

The editorial team worked on the following three documents:

- JCTVC-R1007: SHVC Test Model 7 text

- JCTVC-R1008: SHVC text specification Draft 7

- JCTVC-R1013: Edition 2 Draft Text of High Efficiency Video Coding (HEVC), Including Format Range (RExt), Scalability (SHVC), and Multi-View (MV-HEVC) Extensions

Editing JCTVC-R1008 and JCTVC-R1013 was assigned a higher priority than editing JCTVC-R1007.

Six versions of JCTVC-R1013 were published by the editing AHG following the 18th JCT-VC meeting. The text of the final draft of JCTVC- R1013 (version 6) was submitted to ISO/IEC JTC1/SC29 for Final Draft International Standard ballot and to ITU-T SG16 for Consent. In JCTVC-R1013, the relative changes to Edition 1 of High Efficiency Video Coding (HEVC) are all marked with user name “HEVCv2”.

Seven versions of JCTVC-R1008 were published by the editing AHG following the 18th JCT-VC meeting. The main changes in JCTVC-R1008, relative to the previous JCTVC-Q1008 (SHVC Draft 6), are:

- Incorporation of all common SHVC and MV-HEVC HLS normative adoptions at the 18th JCTVC meeting.

- Integration of all SHVC-specific normative adoptions at the 18th JCTVC meeting.

- Emigration of all the text related to single layer coding features, including five single layer SEI messages, to JCTVC-R1013.

- Fixes of bug tracker tickets.

- Editorial improvements and fixes.

JCTVC-R1007 Test Model 7 document contains the general descriptions of normative processes including the SHVC framework, texture data resampling process, motion field mapping process and colour mapping process, as well as the non-normative downsampling process. The main changes to the previous JCTVC-Q1007 (SHM6) is the text refinement according to the adoptions at Sapporo meeting related to the upsampling and the colour mapping process. Further improvements to the Test Model are planned.

The AHG recommended to:

- Use the SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to the SHVC Draft and Test Model text.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the SHVC Software AHG.

- Continue to improve the quality of the SHVC test model document.

It was asked whether integration of the SHVC test model into the common HM would be feasible. It seemed this could be reasonable once the SHM software would be fully integrated with HM16 (same for multiview).

JCTVC-S0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He (AHG chairs)]

This report summarizes activities of the AHG12 on SHVC software development between 18th and 19h JCT-VC meetings.

The current latest software version is SHM-7.0 contains almost all the items adopted last meeting, however the following items are to be integrated:

- JCTVC-R0062: Pseudo-monochrome

- JCTVC-R0071: Cross-layer impacts of IRAP and EOS

- JCTVC-R0235: AHG10: Processing of bitstreams without an available base layer

- JCTVC-R0124: Constraint related to vps_base_layer_internal_flag

- JCTVC-R0157: When pps_pic_parameter_set_id greater than or equal to 8, colour_mapping_enabled_flag shall be equal to 0

Assignment of correct profile, level and tier indicated in encoder configuration file for output layer set

SHM software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/tags/

The software issues can be reported using bug tracker https://hevc.hhi.fraunhofer.de/trac/shvc

Only one software version has been released by AHG12, integration details and performance summary are given in the next subsections. In the document, only HEVC base layer results are provided, AVC base layer data and CGS results can be found in the accompanying Excel tables. Performance results are consistent with the adopted techniques.

Anchor data and templates have been generated based on common test conditions JCTVC-Q1009 and attached to the report.

Changes included into SHM-7.0 were listed.

Recommendations were as follows:

- Continue to develop reference software based on SHM-7.0 and improve its quality.

- Work on the migration to HM-16.x base which includes RExt

- Resolve identified software and working draft mismatches.

JCTVC-S0013 Loop filtering for SCC (AHG13) C. Rosewarne and L. Zhang (co-chairs), X. Xu (vice-chair)

This report summarizes the Loop filtering for SCC Ad-Hoc Group (AHG13) between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg (October 2014).

The following contributions were noted to be related: JCTVC-S0202, JCTVC-S0224, and JCTVC-S0273 (with associated cross-checks JCTVC-S0202, JCTVC-S0224, JCTVC-S0273).

JCTVC-S0014 Parallel processing for SCC (AHG14) K. Rapaka (chair), A. Duenas, S. Liu, S.-H. Kim (vice-chairs)

This document reports on the work of the JCT-VC ad hoc group on “Parallel processing for SCC (AHG14)” between the 18th JCT-VC meeting in Sapporo, JP (June-July 2014) and the 19th JCT-VC meeting in Strasbourg, FR (October 2014).

Related to mandates, the following seven contribution were noted:

- JCTVC-S0068 On IBC memory reduction [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

- JCTVC-S0070 On IBC constraint for Wavefront Parallel Processing [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

- JCTVC-S0088 On WPP with palette mode and intra BC mode [B. Li, J. Xu (Microsoft)]

- JCTVC-S0101 Intra Block Copy reference area for Wavefront Parallel Procsssing [P. Lai, X. Xu, S. Liu, T.-D. Chuang, S. Lei (MediaTek)]

- JCTVC-S0141 Using the wavefront store-and-sync design for palette table prediction variables [K. Misra, S. H. Kim, A. Segall (Sharp)]

- JCTVC-S0145 Bandwidth reduction method for intra block copy [K. Rapaka, T. Hsieh, C. Pang, V. Seregin, M. Karczewicz(Qualcomm)]

- JCTVC-S0220 On parallel processing capability of intra block copy [K. Rapaka, V. Seregin, C. Pang, M. Karczewicz(Qualcomm)]

JCTVC-S0015 JCT-VC AHG report: Test sequence material (AHG15) [T. Suzuki, V. Baroncini, R. Cohen, T. K. Tan, S. Wenger]

Available test sequences were listed in the report for HEVC v.1 CTC, RExt CTC, and SCC CTC and non-CTC sequences.

In discussion, it was suggested to also list other available test sequences that may be useful – e.g., the test sequences used for HEVC v1 verification tests (if available).

(Further discussion chaired by GJS on 10-22 p.m.)

The test sequences that were used in the verification test were collected and were additionally listed in a revision of the AHG report. They will be uploaded to the ftp site.

# Project development, status, and guidance (0)

## Communication to and by parent bodies (0)

See section 7.2.

## Conformance test set development (0)

It was noted that many features are similar for SHVC and MV-HEVC, and that it would be desirable to test the same features in both cases. This could help ease the effort and increase the coverage of testing for conformance test set development for the two sets of extensions. See further notes in section 7.2.

## Version 2 bug reports and cleanup (0)

See notes for S0003.

## HEVC coding performance, implementation demonstrations and design analysis (2)

### Version 1 performance

JCTVC-S0301 Some Results on Image Quality of HEVC and WebP [D. He, J. Wang] [late]

Presented Thu 10-23 p.m. GJS.

(This was a late information document.)

This document reports some results on HEVC RExt as a still-image coding tool versus the "WebP" proprietary coding design, where the distortion is measured in both PSNR (peak signal-noise-ratio) and SSIM (structural similarity). On a set of grayscale images with resolutions from 512x512 to 2048x2560, it was reported that HM 16.1.1 consistently outperforms WebP in terms of rate-distortion performance, regardless whether the distortion measure is PSNR or SSIM. On average, at mid-quality range, the average PSNR BD-rate of HM 16.1.1 versus WebP (Method 6 = high-quality setting) was reported as −25.0%, and the SSIM BD-rate was reported as −16.6%; at high-quality range, the average PSNR BD-rate of HM 16.1.1 versus WebP (Method 6) was reported as −20.9%, and the SSIM BD-rate was reported as −13.7%.

It was remarked that it is tricky to consider a comparison of the performance of our design relative to third-party. It was also noted that the contribution was a very late submission. It was suggested that we should make sure to be clear that we are not endorsing a comparative assessment of a third-party technology under these circumstances.

The contribution reported closer correspondence between PSNR and SSIM behaviour at higher bit rates than at lower bit rates.

### RExt performance/verification test (0)

### SHVC performance (0)

### SCC performance, design aspects and test conditions (2)

See also the AHG report 6, and contributions S0069 (advocating enable/disable at SPS level rather than CU level under some circumstances), S0100 (advocating disabling ACT for YUV CTC operation).

It was suggested that we should stop using a different sign convention when measuring BD BR impacts for lossy and lossless tests, since this seems confusing.

It was agreed that negative values should indicate reduced bit rate.

(S0100 is also suggesting a change of CTC wrt disabling ACT in YCbCr sequences.)

JCTVC-S0042 AhG6: Inclusion of 4:2:0 screen content in the HEVC SCC common test conditions [J. Sole, M. Karczewicz, R. Joshi, V. Seregin, K. Rapaka (Qualcomm)]

Presenter unavailable Sunday 10-19 p.m.

[Add abstract]

See additional notes in section 7.2.

In further discussion Thursday GJS a.m., it was agreed to add the four class F test sequences to the CTC.

We may want to make this row of the table distinct from the others.

Test results for Class F should not be necessary for tests that only affect 4:4:4 operation, and test results for 4:4:4 sequences should not be necessary for tests that only affect 4:2:0 operation. This was agreed. (But we need to make sure there are no bugs introduced for the irrelevant test cases.)

It was commented that at least some current Class F test sequences were captured with some analog process that seems questionable. This should be further studied.

Our draft text seems to have 4:2:0 capability for IBC. We're keeping it that way (for now).

Palette and ACT only work in 4:4:4 in the draft text, and we're keeping it that way (for now).

JCTVC-S0084 Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 8 and HEVC Screen Content Coding Extensions Test Model 2 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. Sullivan (Microsoft)]

(Chaired by J. Boyce on Sunday 10-19 p.m.)

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Main 4:4:4 12 Test Model 8 (using 8, 10, and 12 bit configurations), HEVC SCC Test Model 1 (SCM 2) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, and JCTVC-R0101 – updating the results by using the latest available reference software (JM-18.6, HM-15.0+RExt-8.0, HM-15.0+RExt-8.0+SCM-2.0), profile and test model designs, and the RExt and SCC test sequences. The overall results indicate that for RExt CTC sequences, RExt Main 4:4:4 12 Profile saves 25%, 33%, and 38% bit rate for AI-MT, RA-MT and LB-MT relative to AVC High 4:4:4 Predictive Profile. For SCC CTC sequences, RExt Main 4:4:4 12 Profile provides an even larger benefit. The overall results also indicate that for screen content sequences, the HEVC SCC Test Model 2 also improves quite substantially over HEVC SCC Test Model 1. For example, for RGB text and graphics with motion (TGM) 1080p sequences, the HEVC SCC Test Model 2 saves 33%, 23%, and 18% bits for AI, RA and LB lossy coding over HEVC SCC Test Model 1. Relative to what was reported most recently in JCTVC-R0101, the bit rate savings for SCC sequences mainly comes from the adoption of new coding tools, especially residual adaptive color transform and palette mode.

The contribution was for information only. The proponents were thanked for providing this useful data.

## Source video test material (0)

# Core experiments in SCC (98)

## CE1: Vector entropy coding (14)

### CE1 summary and general discussion (1)

(Chaired by JRO on Friday 10-17 p.m.)

JCTVC-S0021 CE1: Summary report for Core Experiment 1 on vector entropy coding [C. Pang, X. Xu (CE Coordinators)]

Test 1: BVD entropy coding for IBC mode

- JCTVC-S0142, CE1 Test 1.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

  - JCTVC-S0166, Crosscheck of CE1 Test 1.1 (JCTVC-S0142) [S.-T. Hsiang (MediaTek)]

- JCTVC-S0057, CE1 Test 1.3: CE1: Results of Test 1.3, Test 4.1 and Test 4.3, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0210, Crosscheck of CE1 Test 1.3 (JCTVC-S0057) [C.-H. Hung, C. -C Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)]

- JCTVC-S0162, CE1 Test 1.4: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-S0058, Crosscheck of CE1 Test 1.4 (JCTVC-S0162) [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

Test 2: MVD entropy coding for Inter mode

- JCTVC-S0142, CE1 Test 2.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

  - JCTVC-S0166, Crosscheck of CE1 Test 2.1 (JCTVC-S0142) [S.-T. Hsiang (MediaTek)]

- JCTVC-S0162, CE1 Test 2.2: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-S0226, Crosscheck of CE1 Test 2.2 (JCTVC-S0162) [C. Rosewarne, M. Maeda (Canon)]

Test 3: MVD and BVD entropy coding for Inter and IBC modes

- JCTVC-S0142, CE1 Test 3.1: CE1: Results of Test 1.1, Test 2.1 and Test 3.1, K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)

  - JCTVC-S0059, Crosscheck of CE1 Test 3.1 (JCTVC-S0142) [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

- JCTVC-S0061, CE1 Test 3.2: CE1: Results of Test 3.2 on MVD and BVD coding, G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)

  - JCTVC-S0146, Crosscheck of CE1 Test 3.2 (JCTVC-S0061) [K. Rapaka (Qualcomm)]

- JCTVC-S0162, CE1 Test 3.3: CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding, S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)

  - JCTVC-S0076, Crosscheck of CE1 Test 3.3 (JCTVC-S0162) [G. Laroche (Canon)]

Test 4: Combination tests on BVD entropy coding for IBC mode

- JCTVC-S0057, CE1 Test 4.1: combination of Test 1.1 + Test 1.3, CE1: Results of Test 1.3, Test 4.1 and Test 4.3, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0167, Crosscheck of CE1 Test 4.1 (JCTVC-S0057) [S.-T. Hsiang (MediaTek)]

- JCTVC-S0057, CE1 Test 4.3: combination of Test 1.3 + Test 1.4, CE1: Results of Test 1.3, Test 4.1 and Test 4.3, A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)

  - JCTVC-S0147, Crosscheck of CE1 Test 4.3 (JCTVC-S0057) [K. Rapaka (Qualcomm)]

The proposed methods were implemented on top of SCM2.0 and simulated under the SCC common test conditions.

Test 1 & Test 4: BVD entropy coding for IBC mode

- Test 1.1: BVD coding for IBC (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate BVD is zero. When BVD is not zero, another flag is coded to indicate if absolute level of BVD is greater than 4. If BVD is greater than 4, the remaining absolute level is coded using bypass 4th-order Exp-Golomb codes else is coded using a fixed length code. One flag is coded to indicate sign of the BVD.

- Test 1.3: Adaptive switching between differential and direct coding for intra block copy vectors (JCTVC-S0057)

In this test, one flag is added to specify whether the BV is predicted or not. If not predicted, BV is coded without prediction. The flag is bypass coded and signalled only when the BV component is non-zero. An additional modification of the coding process of sign flags of vectors to reduce redundancy is also tested.

- Test 1.4: BVD coding for IBC using a universal entropy coding scheme (JCTVC-S0162)

In this test, the absolute value of each BVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

- Test 4.1: Combination of Test 1.1 + Test 1.3 (JCTVC-S0057)

In this test, the combination of Test 1.1 and Test 1.3 is investigated.

- Test 4.3: Combination of Test 1.3 + Test 1.4 (JCTVC-S0057)

In this test, the combination of Test 1.3 and Test 1.4 is investigated.

Results for Test 1 & Test 4 on BVD coding for IBC mode

|  | All Intra |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) |  | Non-CE1 (1.1 + 4.1) |  |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.6% | -0.4% | -0.5% | -0.9% | -0.6% |  | -0.9% |  | -1.0% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | -0.3% | -0.4% | -0.3% | -0.4% | -0.4% |  | -0.6% |  | -0.5% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% |  | -0.3% |  | -0.4% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.3% | -0.2% |  | -0.4% |  | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.7% | -0.5% | -0.6% | -1.0% | -0.7% |  | -1.1% |  | -1.2% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.3% | -0.4% | -0.2% | -0.5% | -0.4% |  | -0.6% |  | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.3% | -0.1% | -0.1% | -0.4% | -0.2% |  | -0.4% |  | -0.5% |
| YUV, mixed content, 1080p | 0.0% | -0.2% | -0.4% | -0.1% | -0.2% | -0.5% | -0.3% |  | -0.5% |  | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% |
| Enc Time[%] | 96% | 100% | 100% | 100% | 102% | 102% | 100% | 100% |  | 106% |  |
| Dec Time[%] | 100% | 99% | 99% | 100% | 100% | 100% | 100% | 99% |  | 108% |  |

|  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) | Non-CE1 (1.1 + 4.1) |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.4% | -0.1% | -0.3% | -0.5% | -0.2% | -0.4% | -0.6% |
| RGB, text & graphics with motion,720p | -0.1% | 0.0% | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | -0.1% | -0.3% | 0.0% | -0.2% | -0.2% | -0.2% | -0.3% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.4% | -0.1% | -0.3% | -0.5% | -0.2% | -0.5% | -0.6% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% | -0.3% | -0.2% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.2% | 0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | -0.2% | 0.1% | 0.0% | -0.3% | 0.0% | -0.2% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% | 100% | 100% | 99% | 101% | 101% | 101% | 100% | 92% |
| Dec Time[%] | 100% | 100% | 99% | 101% | 100% | 99% | 100% | 100% | 96% |

|  | Low Delay |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) | Non-CE1 (1.1 + 4.1) |
| RGB, text & graphics with motion, 1080p | -0.1% | -0.2% | -0.4% | 0.0% | -0.3% | -0.5% | -0.2% | -0.4% | -0.5% |
| RGB, text & graphics with motion,720p | -0.2% | 0.0% | 0.0% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.2% | 0.2% | 0.1% | -0.1% | -0.1% | 0.0% | -0.2% |
| RGB, mixed content, 1080p | -0.3% | -0.3% | -0.5% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.1% | -0.1% | -0.3% | 0.0% | -0.2% | -0.4% | -0.1% | -0.3% | -0.5% |
| YUV, text & graphics with motion,720p | -0.2% | -0.1% | -0.1% | -0.1% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.3% | -0.2% | -0.2% | -0.3% | 0.0% | -0.2% | -0.4% |
| YUV, mixed content, 1080p | 0.1% | -0.5% | -0.4% | 0.3% | -0.2% | -0.1% | 0.0% | 0.0% | -0.7% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 84% | 100% | 100% | 100% | 101% | 100% | 101% | 101% | 89% |
| Dec Time[%] | 100% | 99% | 97% | 100% | 100% | 98% | 99% | 99% | 95% |

Complexity analysis

|  | Complexity |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.1 | 1.3(1) | 1.3(2) | 1.4 | 4.1(1) | 4.1(2) | 4.3 (1) | 4.3 (2) |
| # additional regular bins per BV | 2 | 0 | 0 | 12 | same as 1.1 | same as 1.1 | same as 1.4 | same as 1.4 |
| # additional contexts for BV | 2 | 0 | 0 | 4 for AI 2 for RA/LD | same as 1.1 | same as 1.1 | same as 1.4 | same as 1.4 |

From discussion:

-Method 1.3 requires additional decision to determine the setting of the flag. 1.3(2) changes the binarization of sign when it is not predicted. This could be interpreted as an additional predictor candidate. It adds complexity both for encoder and decoder, whereas the gain is small. No action.

- All three methods are adding complexity as compared to the current BV coding of the SCC draft, whereas the compression benefit is relatively small

- Methods 1.1.and 1.4 would likely increase the worst case number of context coded bins (by the last meeting, the new BV entropy coding using 2 context coded bins per 4x4 IBC block instead of 4 had brought the throughput to the same level as MV coding which uses 4 context coded bins per 4x8 block)

BoG (C. Pang) analyzing the worst case number of context coded bins in IBC for the current SCM and the methods 1.1 and 1.4; compare against current 4x8 MV coding, including considerations about reference index and mode coding.

See further notes under S0294 – method 1.1. to be further investigated in CE.

Test 2: MVD entropy coding for Inter mode

- Test 2.1: MVD coding for Inter (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate MVD is zero. When MVD is not zero, another flag is coded to indicate if absolute level of MVD is greater than 4. If MVD is greater than 4, the remaining absolute level is coded using bypass 4-order Exp-Golomb codes else is coded using a fixed length code. One flag is coded to indicate sign of the MVD.

Same binarization as 1.1

- Test 2.2: MVD coding using a universal entropy coding scheme (JCTVC-S0162)

In this test, the absolute value of each MVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

Same binarization as 1.4

Results for Test 2 on MVD entropy coding for Inter mode

|  | Random Access |  |
| --- | --- | --- |
|  | 2.1 | 2.2 |
| RGB, text & graphics with motion, 1080p | -2.3% | -2.8% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% |
| RGB, mixed content, 1440p | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.0% | -0.3% |
| YUV, text & graphics with motion, 1080p | -2.6% | -3.1% |
| YUV, text & graphics with motion,720p | -0.6% | -0.7% |
| YUV, mixed content, 1440p | -0.1% | -0.1% |
| YUV, mixed content, 1080p | -0.2% | -0.2% |
| YUV, Animation, 720p | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | -0.2% |
| Enc Time[%] | 95% | 95% |
| Dec Time[%] | 100% | 100% |

|  | Low Delay |  |
| --- | --- | --- |
|  | 2.1 | 2.2 |
| RGB, text & graphics with motion, 1080p | -3.0% | -3.6% |
| RGB, text & graphics with motion,720p | -1.1% | -1.3% |
| RGB, mixed content, 1440p | -0.2% | -0.3% |
| RGB, mixed content, 1080p | -0.4% | -0.5% |
| RGB, Animation, 720p | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p | -3.3% | -4.0% |
| YUV, text & graphics with motion,720p | -1.3% | -1.7% |
| YUV, mixed content, 1440p | 0.0% | -0.3% |
| YUV, mixed content, 1080p | -0.3% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | -0.2% |
| Enc Time[%] | 93% | 94% |
| Dec Time[%] | 100% | 100% |

Complexity analysis

|  | Complexity |  |
| --- | --- | --- |
|  | 2.1 | 2.2 |
| # additional regular bins per MV | 0 | 10 |
| # additional contexts for MV | 2 | 4 |
| modification to binarization | yes | no |

From discussion: The approaches are using the same binarization for IBC BV and MV coding, however, it is different from the binarization used in previous HEVC versions.

Method 1.3 was not investigated here, because it would give a loss in case of MV coding. This would mean that method 1.3 would only be used for BV, i.e. no harmonization.

Both methods show some interesting gain (around 2-3%) for the 1080p TGwM class, and no loss in any other class. However, the question was raised how much of this gain would be preserved when a conventional bit estimation for inter mode decision woud be used.

Method 2.2 (1.4) would increase the worst case number of context coded bins significantly (10 per 4x8 block).

Method 2.1 (1.1) would not increase the worst case number of context coded bins, but add more contexts.

Modifying the binarization specifically for screen content could likely enforce implementation of two different binarization methods (at least for those decoders that support the old profiles as well).

It is also mentioned by one expert that it would be desirable to re-use existing designs for the inter coding.

Unification of BV coding and MV coding would not be achieved, if the finding of the BoG would be that none of the methods is beneficial for BV coding due to the increase of worst case number of context coded bins.

It is mentioned by several experts that defining a different method of binarization only for MV coding in an SCC environment would be undesirable (eventually enforcing decoders to implement three different methods).

Test 3: MVD and BVD entropy coding for Inter and IBC modes

- Test 3.1: BVD coding for IBC and MVD coding for Inter (JCTVC-S0142)

In this test, first a context coded flag is coded to indicate BVD/MVD is zero. When BVD/MVD is not zero, another flag is coded to indicate if absolute level of BVD/MVD is greater than 4. If BVD/MVD is greater than 4, the remaining absolute level is coded using bypass 4-order Exp-Golomb codes else is coded using a fixed length code. One flag is coded to indicate sign of the BVD/MVD.

- Test 3.2: MVD and BVD entropy coding for Inter and IBC modes (JCTVC-S0061)

In this test, a flag at slice level is used to determine the residual coding method for both Inter and IBC modes. The current SCM2.0 BVD and MVD coding should be considered with this method or with one of the proposed BVD methods in section .

- Test 3.3: Unified BVD & MVD coding using a universal entropy coding scheme (JCTVC-S0162)

In this test, the absolute value of each BVD and MVD component is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy code in the bypass mode.

Results for Test 3 on MVD and BVD entropy coding for Inter and IBC modes

- Test 3.3* further provides the results with the constraint on the max number of regular bins per CU compared with those of the proposed method in test 3.3 without the constraint.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 & 3.3* |
| RGB, text & graphics with motion, 1080p | -0.3% | 0.0% | -0.4% |
| RGB, text & graphics with motion,720p | -0.2% | 0.0% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | 0.0% | -0.5% |
| YUV, text & graphics with motion,720p | -0.2% | 0.0% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% | 98% | 99% |
| Dec Time[%] | 100% | 100% | 101% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 & 3.3* |
| RGB, text & graphics with motion, 1080p | -2.4% | -1.5% | -2.7% |
| RGB, text & graphics with motion,720p | -0.6% | 0.1% | -0.9% |
| RGB, mixed content, 1440p | -0.1% | 0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | -0.1% | 0.0% | -0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p | -2.7% | -1.8% | -3.0% |
| YUV, text & graphics with motion,720p | -0.7% | 0.0% | -0.9% |
| YUV, mixed content, 1440p | 0.0% | 0.2% | -0.1% |
| YUV, mixed content, 1080p | -0.1% | 0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% | 99% | 95% |
| Dec Time[%] | 100% | 99% | 100% |

|  | Low Delay |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3 & 3.3* |
| RGB, text & graphics with motion, 1080p | -3.1% | -1.8% | -3.4% |
| RGB, text & graphics with motion,720p | -1.2% | -0.2% | -1.5% |
| RGB, mixed content, 1440p | -0.2% | 0.2% | -0.2% |
| RGB, mixed content, 1080p | -0.6% | 0.1% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p | -3.4% | -2.1% | -3.9% |
| YUV, text & graphics with motion,720p | -1.4% | -0.4% | -1.8% |
| YUV, mixed content, 1440p | -0.3% | 0.3% | -0.4% |
| YUV, mixed content, 1080p | -0.4% | 0.2% | -0.1% |
| YUV, Animation, 720p | -0.1% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 87% | 98% | 94% |
| Dec Time[%] | 100% | 98% | 99% |

Complexity analysis

|  | Complexity |  |  |
| --- | --- | --- | --- |
|  | 3.1 | 3.2 | 3.3* |
| # additional regular bins per BV | 2 | 0 | 12 |
| # additional regular bins per MV | 0 | 0 | 10 |
| # additional contexts for MV & BV | 4 | 0 | 2 |
| unification of MVD & BVD coding |  |  | same binarization & same contexts |

From discussion: Method 3.2 is not included in test 1 and 2, It indicates at slice level whether (for both MV coding and BV coding) either the conventional HEVC MV coding or the BV coding of SCM is used.

Behaviour not consistent - for some of the non screen content classes, losses occur.

Question is raised how the decision is made? During the presentation of the CE summary, contributors are not available.

3.1 is combining 1.1 and 2.1; 3.3 is combining 1.4 and 2.2.

3.3* is included in version 2 of JCTVC-S0162 (delivered 13-10), giving additional results that indicate no loss when the number of worst case additional context coded bins is further restricted. Formally, this had not been planned in the CE and should be considered as a new contribution. There is no cross-check, and during the discussion some concern is raised that this might imply additional checks by the encoder whether the constraint is met.

See further disposition under BoG report S0298.

### CE1 primary contributions (4)

JCTVC-S0057 CE1: Results of Test 1.3, Test 4.1 and Test 4.3 [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-S0061 CE1: Results of Test 3.2 on MVD and BVD coding [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-S0142 CE1: Results of Test 1.1, Test 2.1 and Test 3.1 [K. Rapaka, M. Karczewicz, C. Pang (Qualcomm)]

JCTVC-S0162 CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

### CE1 cross checks (9)

JCTVC-S0058 CE1: Cross-check on Test 1.4 [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-S0059 CE1: Cross-check on Test 3.1 [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-S0076 CE1: Cross-check of Test 3.2 [G. Laroche (Canon)] [late]

JCTVC-S0146 CE1: Cross-check on Test 3.2 on MVD and BVD coding (JCTVC-S0061) [K. Rapaka (Qualcomm)] [late]

JCTVC-S0147 CE1: Cross-check on Test 4.3 method1 and method 2 (JCTVC-S0057) [K. Rapaka (Qualcomm)] [late]

JCTVC-S0166 CE1: Cross check of Tests 1.1 & 2.1 [S.-T. Hsiang (MediaTek)] [late]

JCTVC-S0167 CE1: Cross check of Test 4.1 on combination of Test 1.1 + Test 1.3 [S.-T. Hsiang (MediaTek)] [late]

JCTVC-S0210 CE1: Cross-Check Results of Test 1.3 [C.-H. Hung, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)] [late]

JCTVC-S0226 Cross-check of CE1 Test 2.2 from ‘CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding’ (JCTVC-S0162) by Mediatek [C. Rosewarne, M. Maeda (Canon)] [late]

## CE2: Intra block copy signalling and partitioning (12)

### CE2 summary and general discussion (1)

(Chaired by chaired by JO, Friday 10-17 p.m.)

Test 1 Unification of IBC to inter (JCTVC-S0080)

In this test, IBC block is treated as an inter block with reference to the current picture so that IBC can reuse the design for the inter mode. The modifications are

- When Intra BC mode is enabled, adding the current picture (before deblocking and SAO) to the last position of list 0, marking as long-term reference picture.

- If the current slice is I slice and Intra BC mode is enabled, the syntax parsing process and decoding process of P slice is followed.

- When Intra BC mode is enabled, after decoding the current picture, mark the current picture as short-term reference picture.

The summarized results are showed below –

From discussion:

Signaling and coding of vectors is used as in inter coding (including merge), but the IBC operation itself differs from motion comp.

Current picture is put as long term reference into ref pic list.

IBC is operated in 4x4 block size, but only uses integer vectors, no filtering etc.

Test 2 - PU-based IBC using intra_bc_flag signalling (JCTVC-S0121)

In this test, IBC flag is signalled at PU level instead of CU level. Two sub-tests were conducted. In test 2a, IBC mode is coded as an intra mode and in test 2b, IBC mode is coded as an inter mode.

The summarized results of test 2a are shown below-

The summarized results of test 2b are shown below-

From discussion: Difference in AI is small, since the main effect is due to the combined usage of inter and IBC at the PU level.

Test 3 - IBC with block vector derivation (JCTVC-S0131)

In this test, five spatially neighboring BVs and their possible BV derivations are used as merge candidates for the current IBC block. The neighboring BVs and BV derivation are illustrated below –

Two sub tests were conducted. In test 3a, BV derivation is used as to construct more merge candidates for an IBC block. In test 3b, BV derivation is further applied to derive a BV/MV for an IBC block.

The summarized results of test 3a are shown below –

And the summarized results of test 3b are shown below –

From discussion:

- Current IBC does not have merge, only prediction from two candidates

- Merge appears to give considerable gain.

Test 5 – Combination of test 1 and test 2 (JCTVC-S0122)

The test is based on test 1. The modification is to signal IBC using intra_bc_flag at PU level. Two sub-tests were conducted. In test 5a, coding tools designed for Inter mode, such as skip mode, merge mode, MVD coding and AMVP are applied to IBC blocks. In test 5b, skip mode and merge mode in HEVC Inter coding are applied to IBC blocks, i.e., BVD and BVP coding are the same as the current IBC design.

The summarized results of test 5a are shown below –

And the summarized results of test 5b are shown below –

Test 5b with an encoder improvement as used in Test 3 is also provided. The summarized results are –

From discussion:

Test 5a combines test 1 and 2, using explicit PU level signalling from test 2 instead of refidx; the results are slightly better, probably because test 1 uses the last ref pic list position for the current picture, and a specially designed context is used for the CU level flag. Skip and merge and conventional MV coding are otherwise used

Test 5b does the same, but replaces the MV pred and MV difference coding for the IBC blocks by the BV pred and BV difference coding from current SCM. Skip and merge are still applied as for MV coding.

The tables for test 5b show encoder optimized results that were provided after the CE deadline, where the mode decision for merge in IBC is based on all three components (not only luma, similar as had been applied in test 3).

Test 6 – Combination of test 1 and the current IBC design (JCTVC-S0081)

In this test, a CU-level flag is sent to indicate IBC flag. Two sub-tests were conducted. In test 6a, both IBC merge mode and IBC AMVP mode are unified with inter mode. In test 6b, only IBC merge mode is unified with inter mode, i.e., BVD and BVP coding are the same as the current IBC design.

From discussion: Basically, test 6a/b are equivalent with 5a/b except that the IBC flag is signalled at the CU level, whereas test 5x uses PU level signalling. Further, 6a used the BV difference coding from SCM, whereas 5a uses the conventional MV difference coding.

The summarized results of test 6a are shown below –

The summarized results of test 6b are shown below –

From the discussion:

Generally, a significant compression benefit is observed by adding skip/merge to IBC. The method from test 3 does this in a IBC specific way (to some extend modifying the current merge list construction of MV coding), whereas the methods of test 1, 5, and 6 are using the same skip/merge process as used in MV coding.

- Some of the methods (test1, test 5a) would have the same worst-case-context-coded-bin problem as discussed in context of CE1, since they are using the MVD coding for IBC vectors.

- It is also questioned whether invoking merge at 4x4 blocks could cause some complexity problems.

1. and 2. should be in detail investigated in the BoG on complexity assessment of IBC vector coding (C. Pang, S. Liu)

See further notes under S0294 - to be further investigated in ongoing CE.

The PU-level switching gives some gain (both from test 2 and comparing test 5b against 6b, which should otherwise be identical), in the range of 1.5% for TGwM class, around 0.3% for some other classes. It is however reported in the cross-check that part of this gain may be due to the fact that the RD check for 2NxN and Nx2N partitions is disabled in the default SCM settings at 16x16 CU, whereas it is enabled for the PU level method (note: those partitions are already supported by current IBC).

One expert reports that according to his experience (in the context of test 3) the gain by allowing 8x16 and 16x8 RD check could give around 0.8%, which is doubted by yet another expert.

Additional results were asked to be provided reporting the gain of PU level switching against a modified SCM reference that enables 16x8 and 8x16 RD checks.

It was later reported that the gain drops by rougly 0.5% - to be further investigated in ongoing CE, but then with additional enabling the RD checks. (does not have highest, but could be interesting to give more insight when comparing CU-level IBC signalling versus the refidx based approach)

See notes on BoG S0294.

JCTVC-S0022 CE2: Summary report for Core Experiment 2 on intra block copy signalling and partitioning [J. Xu, S. Liu, K. Rapaka, X. Xiu (CE coordinators)]

### CE2 primary contributions (5)

JCTVC-S0080 CE2: Result of Test 1 [B. Li, J. Xu (Microsoft), X. Xu, S. Liu, S. Lei (MediaTek)]

JCTVC-S0081 CE2: Result of Test 6 [B. Li, J. Xu (Microsoft)]

JCTVC-S0121 CE2: Test 2 – Intra BC signalled at PU level [X. Xu, S. Liu, S. Lei (MediaTek)]

JCTVC-S0122 CE2: Test 5 – Intra BC unified with Inter using intra_bc_flag [X. Xu, S. Liu, S. Lei (MediaTek), B. Li, J. Xu (Microsoft)]

JCTVC-S0131 CE2: Test3 – IBC with block vector derivation [Y. He, Y. Ye, X. Xiu (InterDigital)]

### CE2 cross checks (6)

JCTVC-S0124 Cross check of CE2 Test 3 (JCTVC-S0131) [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-S0125 Cross check of CE2 Test 6 (JCTVC-S0081) [X. Xu, S. Liu (MediaTek)] [late]

JCTVC-S0127 CE2: Crosscheck of Test1 – Unification of IBC to inter [Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-S0171 Cross-check of ‘CE2: Test 5 – Intra BC unified with Inter using intra_bc_flag’ (JCTVC-S0122) by Mediatek and Microsoft [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-S0183 CE2: Cross-check of Test 2 [C. Pang (Qualcomm)] [late]

JCTVC-S0234 CE2 Test2: Crosscheck for Intra BC signalled at PU level (JCTVC-S0121) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

## CE3: Sub-block partitioning and flipping for Intra block copy (11)

### CE3 summary and general discussion (1)

(Chaired by chaired by JRO, Friday 10-17 p.m.)

JCTVC-S0023 CE3: Summary report for Core Experiment 3 on sub-block partitioning and flipping for Intra block copy [S. Liu (CE Coordinator)]

Test 1.1: Intra block copy masking

The mask is covering a square area extending from one of the borders of the PU and having a width varying from 1 to 3 sample rows or columns. All the sample values in the masked area are substituted with a single sample value obtained from the middle sample of the inner boundary of the mask. The black samples in figure below illustrate the location of the sample which value is used in the substitution process.

Figure 1: Example of masks on different borders of the prediction block

Test 1.2: Segmental prediction for intra block copy

For an IBC CU, a flag is signaled to indicate whether segIBC is applied. If segIBC is on, another flag is signaled to indicate the number of segments, 2 or 3, and then followed by the coded offsets of the segments. segIBC is only applied for lossy coding.

When segIBC is applied, two steps are applied to modify the IBC prediction block before it is used to predict the current block. Figure 2 illustrates an example of segIBC with 2 segments for an 8x8 block.

In the first step, samples in an IBC prediction block are classified into two or three segments. When the prediction block is divided into two segments, a sample is classified to segment 0 if its value is smaller than the average value of all the samples in the IBC prediction block. Otherwise, it is classified into segment 1. When the prediction block is divided into three segments, two thresholds T1 and T2 are calculated as T1= (T+Vmin)/2 and T2= (Vmax+T)/2, where T is the average value of all the samples in the IBC prediction block. Vmax and Vmin are the minimum sample value and the maximum sample value in the IBC prediction block respectively. A sample is classified into segment 0 if its value is smaller than T1; it is classified into segment 2 if its value is larger than T2; otherwise it is classified into segment 1.

In the second step, a single value is assigned to all samples in each segment of the IBC prediction block. The single value denoted as V is calculated as V = E + O, where E is an estimated value for the segment of the IBC prediction block and O is an offset signaled from the encoder to the decoder. E = (Vsegmax+Vsegmin)/2, where Vsegmax and Vsegmin are the minimum sample value and the maximum sample value in the segment of the IBC prediction block respectively.

At encoder, the offset for the segment of the IBC prediction block is calculated by subtracting the average value of all the original sample values in the corresponding segment of the current block and E.

For an IBC CU, a flag is signaled to indicate whether segIBC is applied. If segIBC is on, another flag is signaled to indicate the number of segments, 2 or 3, and then followed by the coded offsets of the segments. segIBC is only applied for lossy coding.

There are four types of configurations. The results below have 2 segmentations for each CU and with all residues set to be 0 when segIBC is applied.

Figure 2:  An example of segIBC with 2 segments for an 8x8 block.

Test 1.3: Intra block copy with flipping

In was proposed in and JCTVC-R0204 [4], JCTVC-R0097 [5] and JCTVC-R0116 [6] that the reference block can be flipped for Intra block copy prediction for higher coding gain. When Intra block copy with flipping is enabled, the reference block is flipped either vertically or horizontally specified by a flipping direction flag. Figure 3 illustrates the examples for vertical flipping and horizontal flipping in Intra block copy.

In this test, Intra block copy flipping may be applied to 2Nx2N, 2NxN, Nx2N, and NxN blocks. An ibc_flipping_flag is signalled for each Intra block copy prediction unit to indicate whether its prediction is flipped or not. Two results are provided for ibc_flag signalled at CU and PU level, respectively.

Figure 3: Vertical flipping and horizontal flipping in Intra block copy

Test 2.1: Combination of test 1.1 and test 1.3

Test 2.2: Combinations of test 1.2 and test 1.3

1.1-1.3 Lossy results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | -0.6% | -0.9% | -1.3% | -1.3% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.7% | -0.6% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -1.3% | -1.3% |
| RGB, mixed content, 1080p | -0.4% | -0.2% | -1.1% | -1.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.6% | -1.1% | -1.4% | -1.4% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.6% | -0.7% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -1.3% | -1.3% |
| YUV, mixed content, 1080p | -0.3% | -0.3% | -1.0% | -1.0% |
| YUV, Animation, 720p | 0.2% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 104% | 115% | 115% |
| Dec Time[%] | 100% | 101% | 101% | 101% |
|  | Random Access |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.8% | -2.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.4% | -0.8% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.7% | -1.0% |
| RGB, mixed content, 1080p | -0.4% | -0.1% | -0.6% | -0.7% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.8% | -2.1% |
| YUV, text & graphics with motion,720p | -0.1% | -0.4% | -0.4% | -0.7% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.6% | -0.9% |
| YUV, mixed content, 1080p | -0.2% | -0.2% | -0.5% | -0.8% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 100% | 102% | 105% |
| Dec Time[%] | 97% | 100% | 101% | 102% |
|  | Low delay B |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | -0.4% | -0.3% | -0.6% | -2.2% |
| RGB, text & graphics with motion,720p | -0.2% | -0.1% | -0.4% | -0.8% |
| RGB, mixed content, 1440p | -0.1% | 0.0% | -0.2% | -0.7% |
| RGB, mixed content, 1080p | -0.2% | -0.1% | 0.0% | -0.7% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.3% | -0.6% | -2.2% |
| YUV, text & graphics with motion,720p | -0.3% | -0.3% | -0.5% | -1.0% |
| YUV, mixed content, 1440p | -0.2% | -0.3% | -0.3% | -0.7% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | 0.0% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 99% | 102% | 104% |
| Dec Time[%] | 100% | 99% | 100% | 99% |

1.1-1.3 Lossless results

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.8% | 0.8% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.3% | 0.3% |
| RGB, mixed content, 1440p | -0.2% | 0.0% | 0.2% | 0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.2% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.0% | 1.0% | 1.0% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.4% | 0.4% |
| YUV, mixed content, 1440p | -0.2% | 0.0% | 0.2% | 0.2% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.3% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% | 101% | 112% | 114% |
| Dec Time[%] | 100% | 99% | 98% | 100% |
|  | Random Access |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.2% | 1.1% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.3% | 0.0% | 0.4% | 1.2% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 100% | 103% | 106% |
| Dec Time[%] | 100% | 101% | 101% | 102% |
|  | Low delay B |  |  |  |
|  | 1.1 | 1.2 | 1.3 CU | 1.3 PU |
| RGB, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.2% | 1.2% |
| RGB, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.2% | 0.0% | 0.4% | 1.3% |
| YUV, text & graphics with motion,720p | 0.0% | 0.0% | 0.2% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% | 101% | 103% | 106% |
| Dec Time[%] | 100% | 99% | 100% | 99% |

  - and 1.2 are targeting smaller regions, 1.3 adds flipping modes

all gains are somewhat lower than expected from last meeting (probably due to adoption of palette mode)

1.2 requires significant additional computation at encoder and decoder, not justified by the compression advantage

1.1 is performing padding of a certain part by one of the values (i.e. modifying the block used for prediction, depending on where the boundary is). The usage of the mode is using one context coded bins, the position and orientation are bypass coded.

1.3 requires memory shuffling. The information about flipping mode is context coded, the hor/vert mode is bypass coded.

Gains of 1.1 and 1.3 seem to be additive, as per results 2.1.

Both 1.1. and 1.3 require additional encoder complexity.

Both 1.1 and 1.3 increase the number of context coded bins by 1 per 4x4 block in worst case.

No support expressed for adoption for any of these.

Further study encouraged for the method 1.3, particularly w.r.t. possibility of fast encoding decisions, gain by only allowing vertical flipping (which should be easier to implement in common memory arrangements) and avoidance of the context coded flag.

### CE3 primary contributions (5)

JCTVC-S0032 CE3: Test 1.1 – Intra block copy masking [J. Lainema, M. M. Hannuksela (Nokia)]

JCTVC-S0107 CE3 Test 1.2: Segmental prediction for intra block copy [K. Zhang, J. An, X. Zhang, H. Huang, S. Lei (MediaTek)]

JCTVC-S0117 CE3: Test 1.3 PU Intra block copy with flipping [J. Ye, X. Xu, S. Liu, S. Lei (MediaTek)]

JCTVC-S0118 CE3: Test 2.1 Combined test of test 1.1 and test 1.3 [J. Ye, S. Liu, X. Xu, S. Lei (MediaTek), J. Lainema, K. Ugur, M. Hannuksela (Nokia)]

JCTVC-S0119 CE3: Test 2.2 Combined test of test 1.2 and test 1.3 [K. Zhang, J. An, X. Zhang, H. Huang, J. Ye, X. Xu, S. Liu, S. Lei (MediaTek)]

### CE3 cross checks (5)

JCTVC-S0077 CE3: Cross-check of test 1.1 [M. Pettersson, K. Andersson (Ericsson)]

JCTVC-S0235 CE3 Test2.2: Crosscheck for Combination of Test 1.2 and Test 1.3 (JCTVC-S0119) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-S0251 CE3: cross-check of Test 1.3: Intra block copy with flipping (JCTVC-S0117) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0252 CE3: cross-check of Test 2.1 Combined test of test 1.1 and test 1.3 (JCTVC-S0118) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0263 CE3 Test 1.2: Crosscheck for Segmental Prediction for Intra Block Copy (JCTVC-S0107) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

## CE4: Intra Line Copy (5)

### CE4 summary and general discussion (1)

(Chaired by chaired by JRO, Friday 10-17 p.m.)

JCTVC-S0024 CE4: Summary report for Core Experiment 4 on Intra Line Copy [C.-C. Chen, X. Xu, L. Zhang, T. Lin (CE Coordinators)]

All the proposed methods were implemented on top of the HM-15.0+RExt-8.0+SCM-2.0 software. Two test conditions formed based on the common test conditions (JCTVC-R1015) for screen content coding are used to evaluate the coding results for both the lossy and lossless operating points.

- Test Condition 1: Full-frame search range for IBC.

- Test Condition 2: 4-CTU (1 current + 3 left) search range for IBC.

Techniques to be tested under this test condition should follow the same restriction on the search range as that of IBC.

Test A: Search Range of Intra Line Copy

- Proponent: NCTU/ITRI (JCTVC-S0135)

- X-checker: Huawei USA R&D (JCTVC-S0248)

- Description: It is proposed to performing intra-copying operation for lines at PU level with 2-D BV for each line. In addition, a flag is present at PU level to indicate whether the row-wise splitting or the column-wise splitting is in use. Each line has its own BV shared across its three colour components. Pixels within the current CU cannot be used as reference.

- BD-rate and Processing Time (4-CTU configuration):

  - Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -6.4% | -6.1% | -6.2% | -3.7% | -3.3% | -3.4% | -2.4% | -2.1% | -2.2% |
| RGB, text & graphics w/ motion,720p | -3.3% | -3.0% | -3.0% | -2.6% | -2.3% | -2.3% | -0.9% | -0.8% | -0.9% |
| RGB, mixed content, 1440p | -2.5% | -2.3% | -2.4% | -1.7% | -1.5% | -1.6% | -1.0% | -0.8% | -0.9% |
| RGB, mixed content, 1080p | -3.7% | -3.4% | -3.4% | -3.0% | -2.4% | -2.5% | -1.5% | -1.2% | -1.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -6.2% | -5.7% | -5.8% | -3.3% | -2.9% | -2.9% | -2.0% | -1.7% | -1.8% |
| YUV, text & graphics w/ motion,720p | -3.0% | -2.7% | -2.9% | -2.5% | -2.1% | -2.2% | -0.9% | -0.7% | -0.5% |
| YUV, mixed content, 1440p | -2.4% | -2.4% | -2.4% | -1.8% | -1.4% | -1.7% | -0.9% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | -3.7% | -3.1% | -3.4% | -2.9% | -2.4% | -2.7% | -1.6% | -1.1% | -1.5% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.4% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 119% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 106% |  |  | 102% |  |  | 102% |  |  |

  - Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 7.1% | 4.8% | 11.1% | 5.3% | 2.7% | 8.3% | 4.4% | 2.5% | 6.0% |
| RGB, text & graphics w/ motion,720p | 2.1% | 0.5% | 4.6% | 1.2% | 0.3% | 3.6% | 0.6% | 0.2% | 1.7% |
| RGB, mixed content, 1440p | 1.1% | 0.4% | 1.7% | 0.2% | 0.1% | 0.3% | 0.1% | 0.0% | 0.2% |
| RGB, mixed content, 1080p | 1.5% | 1.5% | 1.5% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 7.4% | 5.2% | 11.2% | 5.6% | 3.3% | 8.5% | 4.9% | 3.1% | 6.4% |
| YUV, text & graphics w/ motion,720p | 2.5% | 0.6% | 5.8% | 1.5% | 0.3% | 4.6% | 0.8% | 0.2% | 2.2% |
| YUV, mixed content, 1440p | 1.2% | 0.5% | 1.9% | 0.2% | 0.2% | 0.3% | 0.2% | 0.0% | 0.3% |
| YUV, mixed content, 1080p | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 100% |  |  |

- BD-rate and Processing Time (Full-frame Configuration):

  - Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -3.8% | -3.6% | -3.6% | -2.2% | -1.9% | -2.0% | -1.5% | -1.3% | -1.3% |
| RGB, text & graphics w/ motion,720p | -1.3% | -1.1% | -1.2% | -1.1% | -0.9% | -0.9% | -0.7% | -0.6% | -0.4% |
| RGB, mixed content, 1440p | -1.5% | -1.3% | -1.4% | -1.1% | -0.9% | -1.0% | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1080p | -2.2% | -1.9% | -2.0% | -1.6% | -1.3% | -1.4% | -0.6% | 0.1% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -3.7% | -3.4% | -3.4% | -2.0% | -1.7% | -1.8% | -1.3% | -1.0% | -1.1% |
| YUV, text & graphics w/ motion,720p | -1.2% | -1.1% | -1.2% | -0.9% | -1.1% | -1.0% | -0.5% | -0.3% | -0.3% |
| YUV, mixed content, 1440p | -1.4% | -1.3% | -1.5% | -1.0% | -1.2% | -0.9% | -0.7% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | -2.1% | -1.7% | -2.0% | -1.6% | -1.4% | -1.6% | -0.7% | -0.4% | -1.1% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.4% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 117% |  |  | 103% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 103% |  |  |

  - Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 4.6% | 3.0% | 7.6% | 3.2% | 1.9% | 5.3% | 2.7% | 1.8% | 3.7% |
| RGB, text & graphics w/ motion,720p | 1.0% | 0.3% | 1.7% | 0.5% | 0.2% | 1.0% | 0.3% | 0.2% | 0.7% |
| RGB, mixed content, 1440p | 0.7% | 0.3% | 1.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.8% | 0.8% | 0.8% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 4.6% | 3.3% | 7.1% | 3.4% | 2.1% | 5.2% | 2.8% | 2.0% | 3.8% |
| YUV, text & graphics w/ motion,720p | 1.1% | 0.4% | 1.8% | 0.6% | 0.2% | 1.3% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.7% | 0.3% | 1.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 115% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |

- Worst-case Memory Access Bandwidth

|  | Per-pixel Memory Access Bandwidth (P) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Memory Patterns Prediction Mode | 4x1 | 8x1 | 4x2 | 8x2 | 4x4 |
| 8x8, Bi-prediction | 9.375 | 11.25 | 10 | 12 | 12.5 |
| 4x4, IBC | 2 | 4 | 3 | 6 | 4 |
| 1x4, ILC | 4 | 8 | 6 | 12 | 8 |

Test B: Intra Line Copy with Constrained BVs (Withdrawn)

Test C: Self-matching Intra Line Copy

- Proponent: Tongji University (JCTVC-S0194)

- X-checker: NCTU/ITRI (JCTVC-S0233)

- Description: In contrast to the intra line copy proposed of CE4 Test A (JCTVC-S0135), this test additionally relaxes the constraint that the reference lines overlapped with the current CU cannot be used as reference. Two prediction modes are proposed. First, when the reference line is overlapped entirely with the current CU, pixels within the overlapped area is predicted by using the predictor of the reference line. Second, when partial overlapping occurs, pixels of the non-overlapped part will be replicated to predict the pixels in the overlapped part.

- BD-rate and Processing Time (Full-frame Configuration):

  - Lossy Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics w/ motion, 1080p | -3.8% | -3.5% | -3.6% | -2.2% | -1.9% | -2.0% | -1.4% | -1.3% | -1.3% |
| RGB, text & graphics w/ motion,720p | -1.4% | -1.1% | -1.2% | -1.0% | -0.8% | -0.8% | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.5% | -1.4% | -1.4% | -1.1% | -1.0% | -1.0% | -0.7% | -0.6% | -0.7% |
| RGB, mixed content, 1080p | -2.2% | -1.9% | -2.0% | -1.6% | -1.3% | -1.5% | -0.8% | 0.3% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | -3.7% | -3.4% | -3.4% | -2.0% | -1.7% | -1.7% | -1.3% | -1.0% | -1.1% |
| YUV, text & graphics w/ motion,720p | -1.2% | -1.1% | -1.3% | -1.0% | -1.2% | -1.1% | -0.5% | -0.3% | -0.4% |
| YUV, mixed content, 1440p | -1.4% | -1.4% | -1.4% | -1.1% | -1.0% | -0.9% | -0.7% | -0.6% | -0.3% |
| YUV, mixed content, 1080p | -2.1% | -1.6% | -1.9% | -1.7% | -1.7% | -1.6% | -0.8% | 0.1% | -0.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.5% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 117% |  |  | 104% |  |  | 148% |  |  |
| Dec Time[%] | 166% |  |  | 157% |  |  | 171% |  |  |

  - Lossless Results:

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Min | Max | Avg. | Min | Max | Avg. | Min | Max |
| RGB, text & graphics w/ motion, 1080p | 4.5% | 2.9% | 7.5% | 3.2% | 1.8% | 5.4% | 2.6% | 1.7% | 3.8% |
| RGB, text & graphics w/ motion,720p | 0.9% | 0.3% | 1.7% | 0.4% | 0.2% | 1.0% | 0.3% | 0.1% | 0.7% |
| RGB, mixed content, 1440p | 0.7% | 0.3% | 1.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |
| RGB, mixed content, 1080p | 0.7% | 0.7% | 0.7% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics w/ motion, 1080p | 4.6% | 3.2% | 7.3% | 3.4% | 2.1% | 5.4% | 2.9% | 2.0% | 4.0% |
| YUV, text & graphics w/ motion,720p | 1.1% | 0.3% | 1.9% | 0.5% | 0.2% | 1.3% | 0.4% | 0.2% | 0.9% |
| YUV, mixed content, 1440p | 0.8% | 0.3% | 1.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.0% | 0.2% |
| YUV, mixed content, 1080p | 0.9% | 0.9% | 0.9% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 92% |  |  | 129% |  |  | 130% |  |  |
| Dec Time[%] | 136% |  |  | 177% |  |  | 153% |  |  |

From the discussion:

The CE report (particularly the presentation deck includes a very detailed analysis of the complexity)

Worst case number of context coded bins per pixel increased from 7.62 to 8.25

Worst case memory bandwidth is approximately doubled in the intra case.Worst case memory bandwidth might even be higher than in inter coding with memory patterns 8x4 which were not analysed.

One expert gives the hint that the memory bandwidth with full frame access might be unsolvable, but with restricted range it might be reasonable.

Conclusion:

- The increase in complexity from the methods investigated in CEis not justified by the compression advantage

- Consider continuation of CE based on review of non-CE proposals.

### CE4 primary contributions (2)

JCTVC-S0135 CE4: Results of Test A on Search Range of Intra Line Copy [R.-L. Liao, C.-C. Chen, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

JCTVC-S0194 CE4: Results of test C [Liping Zhao, Xianyi Chen, Tao Lin (Tongji)]

### CE4 cross checks (2)

JCTVC-S0233 Crosscheck of JCTVC-S0194: CE4: Results of test C [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)] [late]

JCTVC-S0248 CE4: Cross-check of S0135 (Results of Test A on Search Range of Intra Line Copy) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late]

## CE5: Maximum Palette Size and Maximum Palette Predictor Size (6)

### CE5 summary and general discussion (1)

(Chaired by GJS on Friday 10-17 pm.)

JCTVC-S0025 CE5: Summary report of core experiment 5 on investigation of maximum palette size and maximum palette predictor size [R. Joshi, X. Xiu (CE coordinators)]

This document summarizes the Core Experiment 5 (CE5) on investigation of maximum palette size and maximum palette predictor size. The BD-rate impact of changing the maximum palette size and maximum palette predictor size was studied.

In SCM-2.0, the maximum palette size is 31 and maximum palette predictor size is 64. In the 18th JCT-VC meeting in Sapporo, core Experiment 5 (CE5) was formed to investigate the impact of varying the maximum palette size and the maximum palette predictor size on BD-rate. Both increase and decrease in palette size and palette predictor size was considered. This contribution describes test conditions, test results, and comparisons.

All tests were conducted under SCM-2.0 common test conditions, which enabled full frame intra block copy (FF-IBC) as anchor.

Since palette is an intra tool, BD-results are presented and compared only for All-Intra (AI) lossy configuration. The trend for RA and LD-B configurations is similar to what is seen for AI configuration.

Subtest 1: Maximum palette size

Tester: Qualcomm, JCTVC-S0037

Crosschecker: InterDigital (JCTVC-S0271)

Maximum palette sizes of 15, 47, and 63 were tested (by setting the macro MAX_PLT_SIZE to 15, 47, and 63, respectively in SCM-2.0), relative to the current maximum of 32.

The results were tabulated in the CE summaryreport. Loss was observed when reducing the palette size, but little or no gain was observed by increasing it.

The spec uses truncated unary coding of CU-level palette size, which depends on the decoder knowing the maximum possible palette size. It was remarked that the truncation seems unnecessary. Hitting the maximum size also truncates the palette prediction syntax, which may save some bits and parsing effort.

Subtest 2: Maximum palette predictor size

Tester: InterDigital, JCTVC-S0189

Crosschecker: Qualcomm (JCTVC-S0222)

Maximum palette predictor sizes of 96 and 128 were tested for the default palette size in SCM 2.0, relative to the current predictor size of 64. The palette predictor size was signalled in the PPS.

It was asked whether it is actually beneficial for the maximum palette size to be smaller than the maximum palette predictor size. That question was left open.

For lossless coding, there was some gain by increasing the palette predictor size – e.g., 1% for 96 entries, 2% for 128 entries for 1080p YUV text & graphics with motion (not so much for other classes of content). There is also a related contribution outside of this CE that relates to this.

Subtest 3: Combinations

Subtest 3.a: Max size 15 with predictor size 32

Subtest 3.b: Max size 15 with predictor size 48

Losses were observed in both cases, although somewhat less so with the latter combination.

Decision (Cleanup): When palette mode is enabled, send maximum palette size and maximum palette predictor size at SPS level as ue(v), and establish a profile constraint to disallow values greater than 31 and 64, respectively.

### CE5 primary contributions (3)

JCTVC-S0037 CE5 subtest 5.1: Performance impact of varying the maximum palette size [R. Joshi (Qualcomm)]

JCTVC-S0097 CE5: Informational tests on reducing both maximum palette size and maximum palette predictor size [P. Lai, S. Liu, S. Lei (MediaTek)]

Part of CE?

JCTVC-S0189 CE5: Investigation of palette-based coding with maximum palette predictor size being equal to 96 and 128 [M. AzimiHashemi, X. Xiu, Y. He, Y. Ye (InterDigital)]

### CE5 cross checks (2)

JCTVC-S0222 CE5: Crosscheck of investigation of maximum palette predictor size (S0189) [R. Joshi (Qualcomm)] [late]

JCTVC-S0271 CE5: Cross-verification of JCTVC-S0037 on investigation of maximum palette size [X. Xiu, Y. He, Y. Ye] [late]

## CE6: Palette Mode Improvement (27)

### CE6 summary and general discussion (1)

(Chaired by GJS on Friday 10-17 p.m.)

JCTVC-S0026 CE6: Summary report of CE on improvements of palette mode [Y.-W. Huang, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma]

This document summarizes the Core Experiment 6 (CE6) on proposed modifications of palette mode. Out of the 15 planned tests, two tests were withdrawn, and the rest of the tests with some additional tests were conducted. Evidence was reportedly provided that palette mode can be further improved with affordable complexity.

Category A – Colour index run coding

Test A.1 – Restricted run coding – withdrawn

Test A.2 – Run coding for two-colour palette – withdrawn

Test A.3 – Restricted run coding

Proponent: Canon, JCTVC-R0085, JCTVC-S0062

Crosschecker: Qualcomm, JCTVC-S0034

This test limits the amount of palette_run syntax element decoded for the index mode of the current palette implementation. When the colour index is larger than a threshold, the palette_run is not signalled. The issue of the parsing dependency noted for this contribution will be investigated.

Some gain was shown – 0.0 to 0.5% per source content category (all-intra).

Note: Parsing depends on reconstructed palette index values.

Test A.4 – Binarization and context modeling for run coding

Proponent: MediaTek, JCTVC-R0136, JCTVC-S0163

Crosschecker: Qualcomm, JCTVC-S0035

In this test, the syntax element, palette_run, is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the truncated unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The context selection is conditioned on the palette_type_flag, bin index, and palette index. Both decoded palette index and reconstructed palette index values will be tested for context selection. The refinement value is binarized by the fixed-length code and the resulting bin string is entropy coded in the bypass mode.

Test A.4.1, in which parsing depends on reconstructed palette index values.

Test A.4.2, in which parsing is independent of reconstructed palette index values.

Some gain was shown – 0 to 2% per source content category (all-intra).

The results for A.4.1 are not significantly better than for A.4.2.

The average number of context coded bins is increased, but not the worst case. Two additional contexts are used.

Test A.5 – Contexts for run coding

Proponent: Qualcomm, JCTVC-R0174 part 1, JCTVC-S0038

Crosschecker: Canon, JCTVC-S0071

In this test, use of additional contexts depending on the index for coding index runs is tested. Additional contexts for greater than 0, greater than 1 and greater than 2 flags will be tested. The context is dependent on the index value (decoded) or the coded index (parsed). Additionally choice of context based on index value and the palette size was tested.

Test A.5.1, in which parsing depends on reconstructed palette index values.

Test A.5.2, in which parsing is independent of reconstructed palette index values.

Note: Run time percentages are inaccurate.

The results for A.5.1 are not significantly better than for A.5.2.

Test A.6 – Binarization for run coding

Proponent: Qualcomm, JCTVC-R0174 part 2, JCTVC-S0039

Crosschecker: Canon, JCTVC-S0072

In this test, exponential Golomb and Golomb Rice binarizations and their truncated versions for the palette run values were tested. Combination of tests A.5 and A.6 were also tested.

There were three discussed combinations with similar performance, all with the same number of contexts and the same binarization:

- A.4.2 (with 5 context coded bins)

- A combination of A.5.2 + A.6 (having two sets of results, either with 3 or 5 context coded bins, for which the presented CE results use 3, which is similar to the SCM complexity, but 5 seems better).

- A related non-CE late proposal (S0269, which was not yet cross-checked), which contains a combination of A.4, A.5, and A.6 and has very slightly better performance, using 5 context coded bins.

(Further discussion chaired by GJS Monday 10-20 p.m.)

Cross-check S0289 now available for S0269.

S0173 was a related contribution that was suggested by the BoG to be reviewed in this further discussion.

Some data was shown that appeared to indicate that S0173 would not perform as well as S0269. This data was asked to be uploaded so that we could have a record of what it showed.

Decision: Adopt S0269.

Category B – Colour index and escape colour coding

Test B.1 – Context coded CU-level escape colour flag

Proponent: Qualcomm, JCTVC-R0066, JCTVC-S0154

Crosschecker: InterDigital, JCTVC-S0195

In this test, use of regular CABAC bin to code the CU level escape flag will be tested. Context determination methods dependent on palette size and block size will be tested.

Test B.1.1, where two additional contexts are used.

Test B.1.2, where one additional context is used.

The measured gain is 0.0 to 0.2%.

Test B.2 – Encoder modification related to CU-based escape colour flag

Proponent: ITRI, JCTVC-R0075, JCTVC-S0048

Crosschecker: InterDigital, JCTVC-S0128

If the number of the pixels quantized to a major colour is smaller than or equal to a pre-assigned threshold, the pixels are changed to escape colour pixels. Then the corresponding major colour is removed. An adaptive threshold method will also be tested.

Note: The proposed method is non-normative and encoder-only.

Note: In addition to the the original size of major colour table (M), the proposed method searches M-1, M-2, and M-3 using rate-distortion optimization (RDO).

The measured gain is 0.0 to 0.3%, with a minor encoding time increase (a few percent for AI), no impact for lossless coding.

For further study in CE (suggested to consider some simplification or method with better gain).

Test B.3 – Colour index coding with contextualization

Proponent: University of Hanover, JCTVC-R0113, JCTVC-S0074

Crosschecker: MediaTek, JCTVC-S0168

In this test, a modified colour index signalling is evaluated. In particular, the most significant bin of the palette_index syntax element is coded with a context instead of bypass.

Gain is 0.0 to 0.1%.

Test B.4 – Binarization and context modeling for index coding

Proponent: MediaTek, JCTVC-R0135, JCTVC-S0164

Crosschecker: Fujitsu, JCTVC-S0215

In this test, the syntax element, palette_index, is represented by sending its most significant bit (MSB) index followed by its refinement value. The MSB index is binarized by the truncated unary code. The bin from the resulting bin string is entropy coded in the CABAC mode if the bin index is no greater than a bypass threshold and in the bypass mode, otherwise. The refinement value is binarized by the truncated binary code and the resulting bin string is entropy coded in the bypass mode.

Test B.5 – Escape colour prediction

Proponent: InterDigital, JCTVC-R0170, JCTVC-S0132

Crosschecker: ITRI, JCTVC-S0211

In this test, an escape colour prediction method was tested. A flag is signalled to indicate if all colour components of the current escape colour pixel are the same as that of the previous escape colour pixel. If not, the current escape colour value is predicted from an entry in the palette table. The index value of the table entry used for prediction and the prediction difference are coded.

Note: In addition to entries in the palette table, previous coded escape colours can also be used for predicting the current escape colour.

It was commented that this seems essentially similar to just using a larger palette or palette predictor.

Gain is very minor except for lossless case. Gain is significant for lossless case. However, the encoding time is substantially increased.

Further study in CE suggested. Having a faster encoding method would be desirable.

Category C – Additional colour index representation modes

Test C.1 – Transition copy mode

Proponent: Canon, JCTVC-R0084, JCTVC-S0063

Crosschecker: MediaTek, JCTVC-S0205

In this test, an index prediction method based on transitions will be evaluated. The following index value for the last occurrence of a given index value is stored and used as the inferred index value in the new prediction mode.

Note: Parsing depends on reconstructed palette index values.

Note: An encoder-only parameter change is applied (without changing the RDO algorithm flow) to select index, copy-above, or transition copy.

Gain is 0.0 to 0.6%, depending on the category of test material.

Test C.2 – Transition copy mode

Proponent: MediaTek, JCTVC-R0120, JCTVC-S0078

Crosschecker: Canon, JCTVC-S0073

In this test, a new index coding mode, a transition copy (TC) run mode was tested. In the TC run mode, the coded adjacent colour index patterns are recorded in the TC table. The TC table can be inherited across CUs. The recoded colour indices are used as the colour index predictors in the TC run mode.

Note: An encoder-only parameter change is applied (without changing the RDO algorithm flow) to select index, copy-above, or transition copy.

Gain is 0.0 to 1.2%, depending on the category of test material.

Test C.3 – Copy-from-previous-row mode

Proponent: Qualcomm, JCTVC-R0202, JCTVC-S0174

Crosschecker: MediaTek, JCTVC-S0203

In this test, a mode of “copy from previous rows” for the palette coding was tested. This method enables copying pixels from previously coded rows beyond the row above in the current CU. This is defined as an additional palette mode besides the existing “copy from left”, “copy above” and “escape” modes. The reference row index is coded in the bitstream to indicate which row is copied from. Restrictions on the number of rows available for ‘copy from previous rows” will be tested.

Note: An encoder-only RDO algorithm change is applied to estimate the run bits more precisely.

Gain is 0.0 to 1.4%, depending on the category of test material.

The gain of C.2 and C.3 are reported to be approximately additive (not cross-checked).

(Further discussion chaired by GJS Monday 10-20 p.m.)

S0178 was a related contribution that was suggested by the BoG to be reviewed in this further discussion.

It was remarked that the gain would be larger if IBC was tested with a more constrained search region, which is somewhat anticipated.

Results of the combination of C.2 and C.3 were reported in the late contribution S0188 and were since cross-checked (S0288 (NCTU) and S0295 (Nokia)).

Further discussion was chaired by GJS Thurs 10-23 a.m.

It was commented that we should not complicate and destabilize the design for gains this small. This also applies to our CEs – we should not be studying things that appear to have only a very minor potential for improvement (at least if they are complicating the design).

It was asked how important it is to consider gains that only apply in the lossless case, and commented that lossless coding is not higher priority than the lossy case.

These principles were agreed (CE plans suggested earlier in the meeting should take this into account).

A BoG (Robert Cohen & Yu-Wen Huang) considered CE6-related non-CE contribs.

Category D – Palette table coding

Test D.1 – Palette table coding

Proponent: Qualcomm, JCTVC-R0228, JCTVC-S0153

Crosschecker: MediaTek, JCTVC-S0204

In this test, a method that uses run-length coding to signal the binary vector predictor for palette was tested, where the run value, indicating the number of zero elements between the ones, is signalled as described in JCTVC-R0228. Additionally, sending the number of non-zero elements explicitly in the beginning will be tested. Run values are coded using Exponential Golomb coded or Golomb Rice codes or their truncated versions. Additionally, increased palette predictor size was with the run-length coding.

This was asserted to be a syntax (and text) simplification, with a small gain 0.0 to 0.3%, depending on the category of test material.

Decision: Adopted.

Additional results were provided for a palette predictor size increased to 128, with some additional gain.

### CE6 primary contributions (13)

JCTVC-S0038 CE6 subtest A.5: Contexts for run coding in palette mode [R. Joshi, M. Karczewicz, W. Pu, V. Seregin, J. Sole (Qualcomm)]

JCTVC-S0039 CE6 subtest A.6: Binarization for run coding in palette mode [R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin, J. Sole (Qualcomm)]

JCTVC-S0303 CE6: Cross-check of supplemental results from JCTVC-S0039, CE6 subtest A.6: Binarization for run coding in palette mode [R. Cohen (MERL)] [late]

JCTVC-S0048 CE6 Test B.2: Encoder modification of palette coding for escape pixels [Y.-J. Chang, C.-H. Hung, C.-L. Lin, C.-C. Lin, J.-S. Tu (ITRI)]

JCTVC-S0062 CE6: Results of Test A.3 on restricted run coding [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

JCTVC-S0063 CE6: Results of Test C.1 on transition copy mode [C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-S0074 CE6: Results for Test B3 on Improved Palette Index Coding with Contextualization [T. Laude (Leibniz Universität Hannover)]

JCTVC-S0078 CE6 Test C.2: Transition copy mode [Y.-C. Sun, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-S0132 CE6: Test B.5 – Escape colour prediction [Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-S0153 CE6: Test D.1 Run-length coding for reuse flags [M. Karczewicz, W. Pu, V. Seregin, R. Joshi, J. Sole (Qualcomm)]

JCTVC-S0154 CE6: Test B.1 Context coded CU-level escape colour flag [M. Karczewicz, W. Pu, V. Seregin, R. Joshi, J. Sole (Qualcomm)]

JCTVC-S0163 CE6: Results of Test A.4 on palette run coding [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-S0164 CE6: Results of Test B.4 on palette index coding [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-S0174 CE6: Test C.3 Copy previous row mode for palette coding [F. Zou, W. Pu, M. Karczewicz, R. Joshi, V. Seregin (Qualcomm)]

### CE6 cross checks (13)

JCTVC-S0034 CE6: Cross check results for Test A.3 [W. Pu (Qualcomm)]

JCTVC-S0035 CE6: Cross check results for Test A.4 [W. Pu (Qualcomm)]

JCTVC-S0071 CE6: Cross-check of Test A.5 [G. Laroche (Canon)] [late]

JCTVC-S0072 CE6: Cross-check of Test A.6 [G. Laroche (Canon)] [late]

JCTVC-S0073 CE6: Cross-check of Test C.2 [C. Gisquet (Canon)] [late]

JCTVC-S0128 CE6: Crosscheck of Test B.2 – Encoder modification of palette coding for escape pixels [Y. He, X. Xiu, Y. Ye (InterDigital)]

JCTVC-S0168 CE6: Cross check of Test B.3 on color index coding with contextualization [S.-T. Hsiang (MediaTek)] [late]

JCTVC-S0195 CE6: Cross-verification of Test B1 [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-S0203 Crosscheck for CE6 Test C.3 on copy from previous row [T.-D. Chuang (MediaTek)] [late]

JCTVC-S0204 Crosscheck for CE6 Test D.1 on palette table signalling [T.-D. Chuang (MediaTek)] [late]

JCTVC-S0205 Crosscheck for CE6 Test C.1 on transition copy mode [Y.-C. Sun (MediaTek)] [late]

JCTVC-S0211 CE6: Cross-Check Results of Test B.5 [C.-C. Lin, C.-H. Hung, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)] [late]

JCTVC-S0215 CE6: crosscheck report of CE6 test B.4 [Z. Xu, J. Zhu (Fujitsu)] [late]

## CE7: String Matching for Palette Index Coding (7)

### CE7 summary and general discussion (1)

(Chaired by GJS on Friday 10-17 p.m.)

JCTVC-S0027 CE7: Summary Report for Core Experiment 7 on String Matching for Palette Index Coding [Z. Ma, Y.-W. Huang (CE Coordinators)]

This document provides the summary of Core Experiment 7 (CE7) on String Matching for Palette Index Coding. CE7 includes three tests:

- Test 1 on 1-D string-based index coding;

- Test 2 on constrained 1-D string-based index coding;

- Test 3 on hybrid 1-D/2-D string-based index coding

Test 1 – 1-D string-based index coding

This test evaluates the color index map coding proposed in JCTVC-R0268 using 1D string match based index compression. 1-D string match is performed within current CU with the representation of matched pairs.

Test 2 - Constrained 1-D string-based index coding

This test evaluates the color index map coding proposed in JCTVC-R0304 using constrained 1D string match based index compression. This constrained 1-D string match is performed within current CU with constrained matched distance and length is inferred as the CU width. More details could be found JCTVC-R0304

Test 3 - Hybrid 1-D/2-D string-based index coding

This test evaluates the color index map coding proposed in JCTVC-R0268/R304 using hybrid 1D/2D string match based index compression. 1-D string match is performed within current CU, while hybrid 1D/2D search can be extended to the left 3 CTUs. More details could be found JCTVC-R0268/R0304

The best results were for test 3.

Lossy coding gain is 0.0 to 3.4% with current CU plus 3 to its left for IBC, depending on the category of test material.

Lossy coding gain is 0.0 to 1.9% relative to full-frame IBC, depending on the category of test material.

| Test | Mode | Current Proposal | Crosscheck(s) | Tested Proposal(s) |
| --- | --- | --- | --- | --- |
| 1 | 1-D | JCTVC-S0158 | JCTVC-S0216 | JCTVC-R0268 |
| 2 | Constrained 1-D | JCTVC-S0159 | JCTVC-S0091 | JCTVC-R0304 |
| 3 | Hybrid 1-D/2-D | JCTVC-S0160 | JCTVC-S0130 | JCTVC-R0304/R0268 |

A participant suggested that the extra memory requirement (36 kbyte on-chip memory increase) and worst-case context coded bins (13 per pixel) are problems.

It was remarked that the non-CE contribution S0151 (which has a smaller number of context coded bins) should be considered in relation to this, for further study. The proponent suggested to consider that contribution, or something like it.

### CE7 primary contributions (3)

JCTVC-S0158 CE 7 Test 1: 1-D String based Index Coding for Palette Index Coding [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D)]

JCTVC-S0159 CE 7 Test 2: Constrained 1-D String based Index Coding for Palette Index Coding [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D)]

JCTVC-S0160 CE 7 Test 3: Hybrid 1-D/2-D String-based Index Coding for Palette Index Coding [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D)]

### CE7 cross checks (3)

JCTVC-S0091 CE7: Cross-check of test 2 (JCTVC-S0159) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0130 CE7: Cross check results for Test 3 (JCTVC-S0160) [J. Ye, S. Liu (MediaTek)] [late]

JCTVC-S0216 CE7: crosscheck report of CE7 test 1 [Z. Xu, J. Zhu (Fujitsu)] [late]

## CE8: Single-Colour and Two-Colour Modes (7)

### CE8 summary and general discussion (1)

(Chaired by GJS on Friday 10-17 p.m.)

JCTVC-S0028 CE8: Summary report for Core Experiment 8 on Uni- and bi-colour mode [R. Cohen, T.-D. Chuang, C.-C. Lin, K. Rapaka (CE Coordinators)]

This document provides the summary of Core Experiment 8 (CE8) on uni- and bi-colour modes. CE8 includes three tests: Test A on bi-colour intra mode, Test B on single colour intra mode, and Test C on independent uniform prediction intra mode. A list of proposals related to CE8 that were not part of the CE8 description is also included.

Test A – Bi-colour intra mode

Bi-colour intra mode first transmits an index to specify which two colours are selected out of available spatial neighboring pixels of the current CU. Then an index map for the CU is established by the selected two colours. An index map coding is used to transmit the index map. Two methods are tested: Method 1 uses two diverse colors selected from five neighboring samples, and Method 2 uses two diverse colors selected from the above two and left two lines.

Gain 0.0 to 0.4%.

Test B – Single colour intra mode

A CU-level flag is signalled to indicate whether a CU is coded as single colour mode. When a CU is coded as single colour mode, it is reconstructed by filling this CU with one single value. Additionally, an index is transmitted to select the colour from the colour sample candidates derived from the spatial neighboring and / or previous CU samples to fill in the CU. Two methods are tested: Method 1 evaluates two spatial sample candidates from immediate neighboring samples, and Method 2 evaluates four spatial sample candidates from both four spatial neighboring samples plus colour candidates from the palette predictor table.

Gain 0.0 to 0.8%.

Test C – Independent uniform prediction intra mode

Independent uniform prediction uses a uniform colour value (triplet) as prediction samples for a block. The number of candidate uniform colours and their values are signalled on a slice level. A CU-level flag indicates whether the CU uses this mode. If there is more than one colour, an index indicates which colour to use.

Gain -0.5 to 0.4%.

| Test | Mode | Current Proposal | Crosscheck(s) | Tested Proposal(s) |
| --- | --- | --- | --- | --- |
| A | Bi-colour | JCTVC-S0049 | JCTVC-S0103 | JCTVC-R0112/Q0094 |
| B | Single colour | JCTVC-S0098 | JCTVC-S0190 | JCTVC-R0198/R0058 |
| C | Independent Uniform Prediction | JCTVC-S0176 | JCTVC-S0212 | JCTVC-R0200 |

Of the CE tested methods, test B method 1 seems the most significant candidate, but some of its gain may be just R-D search gain, and other aspects are changing.

S0050 and S0051 were identified as related.

A non-CE proposal S0108 was also suggested to be potentially related.

However, the available gains seem quite small, so no action taken.

### CE8 primary contributions (3)

JCTVC-S0049 CE8 Test A: Bi-colour intra mode for screen content coding [Y.-J. Chang, C.-C. Lin, J.-S. Tu, C.-L. Lin, C.-H. Hung (ITRI)]

JCTVC-S0098 CE8 Test B: Single colour intra mode, with supplementary results [P. Lai, S. Liu, Y.-W. Chen, T.-D. Chuang, Y.-C. Sun, S. Lei (MediaTek)]

JCTVC-S0176 CE8 Test C: Independent Uniform Prediction Intra Mode (IUP) [R. Cohen, X. Zhang (MERL)]

### CE8 cross checks (3)

JCTVC-S0103 CE8: Cross-check of Test A (JCTVC-S0049) Bi-colour intra mode for screen content coding [P. Lai (MediaTek)] [late]

JCTVC-S0190 CE8: Crosscheck of Test B Single colour intra mode [R. Cohen (MERL)]

JCTVC-S0212 CE8: Cross-Check Results of Test C [J.-S. Tu, C.-H. Hung, C. –C. Lin, Y.-J. Chang, C.-L. Lin (ITRI)] [late]

## CE9: Intra Boundary Filtering and Cross-Component Prediction Interdependency (8)

### CE9 summary and general discussion (1)

(Chaired by chaired by JRO, on Friday 10-17 p.m.)

JCTVC-S0029 CE9: Summary report for Core Experiment 9 on IBF/CCP interdependency [R. Cohen, S. Liu, J. Xu, L. Zhang (CE Coordinators)]

Test A.1 – Optionally disabling the usage of the intra boundary filters

It is proposed to conditionally disable the boundary filters for intra horizontal, vertical, and DC modes with flag controls in SPS and slice header. Two methods are tested: Method 1 disables all the intra boundary filters. For Method 2, if the proportion of 2nd and 3rd component 16x16 blocks having sharp edges exceeds a threshold, then the intra boundary filters are disabled.

Test A.2 – Optionally enabling the usage of the intra boundary filters on all components

It is proposed to enable the intra boundary filter for the second and third components in the 444 chroma sampling format. An enable flag is added at the SPS or PPS level.

Test A.3 – Modifying cross-component prediction to compensate for intra boundary filtering

It is proposed to modify the CCP process by adding an offset block to the reconstructed luma (or first component) residual block. This offset block is the difference between the luma prediction block after intra boundary filtering is applied and the same block before intra boundary filtering is applied. For blocks in which the adaptive color transform has been applied, the luma offset is also subtracted from the CCP computations for the Cg component.

Supplemental test: Combination of tests A.1 and A.2

Results from a supplemental test, which was not defined in the original CE9 description, are included here for completeness and to facilitate discussion. In this test, if the number of 16x16 sharp-edge blocks exceeds that of smooth-edge blocks, IBF is disabled for all components; Otherwise, IBF is enabled for all components.

A.1 method 1 always disables IBF, resulting in gains (0.1%...1.2%) for RGB classes, typically small losses for YUV

A.1 method 2 makes a frame-level decision (analysing the difference between pixels and based on this classifies the picture as sharp edge type), disabling the IBF for sharp edge types. This avoids the losses for YUV, whereas the gain in RGB is practically unchanged

A.2 enables IBF for all three components (0.1…1.5% gain), also small gain for YUV

A.3 modifies CCP, resulting in small changes (likely due to inclusion of ACT, gains had been larger at the last meeting).

Combination A.1/A.2 test disables for all three components when above the A.2 threshold, enabling for all three components when below. This gives more or less the same result as A.2

The differences w.r.t. compression seem to be small (in average, A.2 is 0.2-0.3% lower than A,1)

From the discussion, A.1 (adding an option to disable IBF for the first component) is asserted to be the simplest solution. However, some doubt is raised whether this would have impact on the visual quality.

Prepare viewing to compare A.1 method 1 versus A.2. If no differences are found, A.1 method 1 (sequence level disabling) should be used.

Further discussion was chaired by J. Boyce 10-23 p.m.

Informal expert viewing done, including still frames.

Decision: Adopt sequence level disabling of IBF, as described in Method A.1.

### CE9 primary contributions (3)

JCTVC-S0082 CE9: Result of Test A.2 [B. Li, J. Xu (Microsoft), X. Zhang, R. Cohen (MERL)]

JCTVC-S0102 CE9 Test A.1: Optionally disabling the usage of the intra boundary filters [X. Zhang, K. Zhang, J. An, H. Huang, S. Lei (MediaTek)]

JCTVC-S0177 CE9 Test A.3 Modifying cross-component prediction to compensate for intra boundary filtering [R. Cohen, X. Zhang (MERL)]

### CE9 cross checks (4)

JCTVC-S0191 CE9: Crosscheck of Test A.1 Optionally disabling the usage of the intra boundary filters [R. Cohen (MERL)]

JCTVC-S0221 CE9: crosscheck report of CE9 Test A.3 (JCTVC-S0177) [L. Zhang (Qualcomm)] [late]

JCTVC-S0264 CE9: cross-check of test A.1.3: Optionally disabling the usage of the intra boundary filters (JCTVC-S0102) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0268 CE9: Cross-check of JCTVC-S0082 Test A.2 [X. Zhang (MediaTek)] [late]

## CE10: Intra String Copy (8)

### CE10 summary and general discussion (1)

(Chaired by JRO on Sunday 10-19 p.m.)

JCTVC-S0030 CE10: Summary report for Core Experiment 10 on Intra String Copy [Y. Chen, J. Xu (CE Coordinators)]

Scan directions (Test 1)

Though most proposals use horizontal scan, the benefit of adaptive vertical and horizontal scans is to be tested.

Input documents: JCTVC-S0083.

Matching modes (Test 2)

2D matching is enabled in the common software and the benefit of having 1D matching is to be tested. The proponents are required to provide the complexity analysis of 1D matching in terms of memory bandwidth access as well as decoder buffer.

Input documents: JCTVC-S0083, JCTVC-S0192, and JCTVC-S0161.

Residue coding (Test 3)

The benefit of utilizing residue coding will be tested in CE and can be further tested together with different constraints as in Test 7.

Input documents: JCTVC-S0161.

Mapping table for offset/length coding (Test 4)

It is found that the matching string length has unusual characteristic that many special and large values are more frequently used than many small values. Based on the characteristics, a mapping of the string length values to two matching parameters (pos and len) is introduced to improve the coding efficiency for length values.

Input documents: JCTVC-S0192.

Frequent colour table (Test 5)

A frequent colour table is introduced to code escape pixels. There are 128 entries in the table, each contains three sample values. The table can be updated once an escape pixel is explicitly coded.

Input documents: JCTVC-S0161.

CABAC throughput (Test 6)

More efficient coding of the syntax elements in Intra String Copy.

Input documents: JCTVC-S0165.

Constraints for run length and number of runs (Test 7)

It is required that for each proposal, the worst case memory bandwidth analysis shall be provided. In addition, it is suggested to provide statistics for the average memory bandwidth.

Input documents: JCTVC-S0174 and JCTVC-S0192.

A common CE10 software was used that supports 2D matching for Intra String Copy with horizontal scanning order within a CU.

The memory bandwidth analysis methodology for the horizontal scan order in 2D matching mode hasn’t been well established, as concluded in the previous meeting.

Some of the new tests, including e.g., Test 1 (vertical scan), Test 2 (1D matching) may have implications for different patterns of memory bandwidth access, while how to evaluate the complexity (including memory access) for those methods needs to be discussed.

In the following tables, 4-CTU restricts both IBC and string copy to the same search range.

Performance of the CE 10 common software against SCM2.0 in CTC configuration

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.1% | -2.3% | -2.4% | -1.5% | -1.7% | -1.7% | -1.1% | -1.3% | -1.2% |
| RGB, text & graphics with motion,720p | -0.6% | -0.8% | -0.6% | -0.4% | -0.6% | -0.5% | -0.4% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% |
| RGB, mixed content, 1080p | -0.5% | -0.6% | -0.5% | -0.4% | -0.4% | -0.6% | -0.3% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.7% | -1.9% | -1.8% | -1.3% | -1.5% | -1.5% | -1.0% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.4% | -0.5% | -0.5% | -0.2% | -0.4% | -0.5% | -0.2% | -0.2% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% | 0.0% | 0.1% | -0.1% | -0.1% | -0.2% | -0.3% |
| YUV, mixed content, 1080p | -0.3% | -0.4% | -0.3% | -0.1% | -0.5% | -0.3% | -0.1% | 0.5% | -0.4% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | -0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 150% |  |  | 111% |  |  | 111% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 103% |  |  |

Performance of the CE 10 common software against SCM2.0 4-CTU configuration

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.5% | -1.8% | -1.7% | -1.1% | -1.4% | -1.3% | -0.8% | -0.9% | -0.9% |
| RGB, text & graphics with motion,720p | -0.3% | -0.4% | -0.3% | -0.4% | -0.4% | -0.3% | 0.0% | -0.1% | 0.0% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.3% | -1.3% | -1.3% | -1.1% | -1.2% | -1.3% | -0.9% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.3% | 0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% | 0.0% | 0.1% | 0.0% | 0.1% | 0.7% | 0.4% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | -0.5% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.3% | 0.1% | 0.0% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 134% |  |  | 112% |  |  | 109% |  |  |
| Dec Time[%] | 96% |  |  | 101% |  |  | 99% |  |  |

Performance of Test 1 against SCM2.0 in CTC configuration (JCTVC-S0083)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -5.5% | -5.9% | -6.0% | -3.4% | -3.8% | -3.8% | -2.4% | -2.6% | -2.5% |
| RGB, text & graphics with motion,720p | -2.9% | -3.4% | -2.9% | -2.5% | -2.9% | -2.6% | -1.3% | -2.0% | -1.7% |
| RGB, mixed content, 1440p | -0.7% | -0.9% | -0.9% | -0.4% | -0.6% | -0.6% | -0.3% | -0.5% | -0.4% |
| RGB, mixed content, 1080p | -2.5% | -2.8% | -2.7% | -1.7% | -2.0% | -2.1% | -0.7% | -0.3% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.3% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -4.7% | -5.0% | -5.0% | -2.9% | -3.3% | -3.2% | -2.0% | -2.3% | -2.3% |
| YUV, text & graphics with motion,720p | -2.2% | -2.5% | -2.6% | -2.0% | -2.5% | -2.7% | -1.2% | -1.6% | -1.5% |
| YUV, mixed content, 1440p | -0.6% | -0.8% | -0.9% | -0.3% | -0.5% | -0.5% | -0.2% | -0.4% | -0.7% |
| YUV, mixed content, 1080p | -2.2% | -2.5% | -2.6% | -1.7% | -2.3% | -2.4% | -0.4% | -0.4% | -0.7% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 190% |  |  | 132% |  |  | 124% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  | 101% |  |  |

Performance of Test 1 against SCM2.0 in 4-CTU configuration (JCTVC-S0083)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.4% | -3.8% | -3.7% | -2.3% | -2.6% | -2.6% | -1.5% | -1.7% | -1.7% |
| RGB, text & graphics with motion,720p | -1.8% | -2.2% | -1.9% | -1.7% | -2.0% | -1.9% | -0.6% | -1.0% | -0.9% |
| RGB, mixed content, 1440p | -0.4% | -0.6% | -0.5% | -0.3% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% |
| RGB, mixed content, 1080p | -0.8% | -0.8% | -0.9% | -0.5% | -0.7% | -0.6% | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -2.8% | -2.9% | -3.0% | -2.0% | -2.3% | -2.3% | -1.5% | -1.7% | -1.7% |
| YUV, text & graphics with motion,720p | -1.2% | -1.6% | -1.7% | -1.1% | -1.6% | -1.7% | -0.4% | -0.8% | -0.7% |
| YUV, mixed content, 1440p | -0.3% | -0.5% | -0.4% | -0.2% | -0.2% | -0.4% | -0.2% | -0.1% | 0.1% |
| YUV, mixed content, 1080p | -0.7% | -0.8% | -0.7% | -0.4% | -0.7% | -0.7% | -0.5% | -0.9% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.2% | 0.3% | 0.2% | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 176% |  |  | 128% |  |  | 120% |  |  |
| Dec Time[%] | 95% |  |  | 104% |  |  | 97% |  |  |

Performance of Test 2 against SCM2.0 in CTC configuration (JCTVC-S0083)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -3.2% | -3.5% | -3.5% | -2.1% | -2.4% | -2.4% | -1.4% | -1.6% | -1.4% |
| RGB, text & graphics with motion,720p | -1.1% | -1.3% | -1.2% | -1.2% | -1.5% | -1.3% | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% | -0.3% | -0.4% | -0.4% | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.7% | -0.7% | -0.7% | -0.5% | -0.5% | -0.6% | -0.1% | 0.4% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -2.5% | -2.7% | -2.6% | -1.7% | -2.1% | -2.0% | -1.2% | -1.3% | -1.2% |
| YUV, text & graphics with motion,720p | -0.7% | -0.9% | -1.0% | -1.1% | -1.6% | -1.9% | -0.3% | -0.1% | -0.3% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% | -0.1% | -0.3% | -0.1% | -0.2% | -0.2% | 0.3% |
| YUV, mixed content, 1080p | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% | 0.2% | 0.7% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | -0.1% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% |
| Enc Time[%] | 218% |  |  | 147% |  |  | 129% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  | 101% |  |  |

Performance of Test 2 against SCM2.0 in 4-CTU configuration (JCTVC-S0083)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -6.5% | -6.9% | -6.8% | -3.8% | -4.2% | -4.1% | -1.7% | -1.9% | -1.8% |
| RGB, text & graphics with motion,720p | -3.5% | -3.7% | -3.5% | -3.8% | -3.9% | -3.9% | -0.6% | -0.7% | -0.6% |
| RGB, mixed content, 1440p | -0.6% | -0.6% | -0.6% | -1.0% | -0.9% | -1.1% | -0.5% | -0.5% | -0.7% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.4% | -0.5% | -0.5% | -0.5% | -0.3% | -0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p | -4.7% | -5.1% | -4.8% | -3.1% | -3.7% | -3.5% | -1.6% | -1.8% | -1.8% |
| YUV, text & graphics with motion,720p | -2.0% | -2.1% | -2.3% | -2.9% | -3.3% | -3.8% | -0.1% | -0.3% | 0.0% |
| YUV, mixed content, 1440p | -0.3% | -0.4% | -0.3% | -0.4% | -0.6% | -0.6% | -0.2% | 0.3% | 0.0% |
| YUV, mixed content, 1080p | -0.5% | -0.4% | -0.3% | -0.2% | -0.3% | -0.1% | -0.5% | -0.8% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% | 0.1% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 197% |  |  | 143% |  |  | 129% |  |  |
| Dec Time[%] | 94% |  |  | 104% |  |  | 98% |  |  |

Note: 2D matching from CE10 software id disabled in the following proposal

Performance of Test 2 against SCM2.0 in CTC configuration (JCTVC-S0192, config1)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.3% | -1.5% | -1.4% | -0.9% | -1.0% | -0.9% | -0.4% | -0.6% | -0.5% |
| RGB, text & graphics with motion,720p | -1.1% | -1.0% | -1.2% | -1.4% | -1.4% | -1.5% | -0.8% | -0.7% | -1.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.6% | -0.6% | -0.6% | -0.3% | -0.4% | -0.5% |
| RGB, mixed content, 1080p | -1.1% | -1.1% | -1.0% | -1.1% | -1.2% | -1.0% | -0.8% | -0.8% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.9% | -1.2% | -1.1% | -0.7% | -1.3% | -1.2% | -0.4% | -0.7% | -0.7% |
| YUV, text & graphics with motion,720p | -0.4% | -0.9% | -1.2% | -1.0% | -2.2% | -2.5% | -0.5% | -0.5% | -1.4% |
| YUV, mixed content, 1440p | -0.1% | -0.4% | -0.2% | -0.3% | -0.9% | -0.7% | -0.4% | -0.7% | -0.1% |
| YUV, mixed content, 1080p | -1.0% | -2.7% | -2.1% | -0.8% | -3.1% | -2.2% | -0.8% | -3.2% | -2.8% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | -0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | -0.1% | 0.3% |
| Enc Time[%] | 123% |  |  | 107% |  |  | 105% |  |  |
| Dec Time[%] | 225% |  |  | 306% |  |  | 273% |  |  |

Performance of Test 2 against SCM2.0 in CTC configuration (JCTVC-S0192, config2)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.9% | -3.1% | -3.0% | -1.8% | -2.1% | -2.0% | -1.0% | -1.4% | -1.3% |
| RGB, text & graphics with motion,720p | -1.6% | -1.7% | -1.8% | -1.9% | -1.9% | -2.1% | -1.0% | -1.0% | -1.2% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.7% | -0.6% | -0.6% | -0.4% | -0.5% | -0.4% |
| RGB, mixed content, 1080p | -1.1% | -1.2% | -1.1% | -1.0% | -1.2% | -1.2% | -1.3% | -1.4% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -2.0% | -2.3% | -2.1% | -1.5% | -2.1% | -2.0% | -0.9% | -1.2% | -1.2% |
| YUV, text & graphics with motion,720p | -0.8% | -1.4% | -1.8% | -1.3% | -2.5% | -2.9% | -1.0% | -1.3% | -2.2% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.2% | -0.3% | -0.8% | -0.5% | -0.2% | -0.9% | -0.2% |
| YUV, mixed content, 1080p | -1.0% | -2.7% | -2.2% | -0.8% | -3.2% | -2.5% | -1.0% | -2.4% | -2.6% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% | 0.2% |
| Enc Time[%] | 146% |  |  | 114% |  |  | 109% |  |  |
| Dec Time[%] | 233% |  |  | 302% |  |  | 274% |  |  |

Performance of Test 2 against SCM2.0 in CTC configuration (JCTVC-S0161)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, text & graphics with motion,720p | 0.3% | 0.3% | 0.3% | 0.3% | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | -0.1% | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% | -0.1% | -0.3% | -0.3% | -0.2% | 0.3% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 1.1% | 1.1% | 1.1% | 0.3% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.6% | 0.6% | 0.6% | 0.2% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | -0.1% | 0.1% | -0.5% | -0.2% | -0.1% | -0.6% | -0.2% |
| YUV, mixed content, 1080p | 0.2% | -0.2% | 0.0% | 0.0% | -0.6% | -0.4% | 0.4% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.0% | 0.2% |
| Enc Time[%] | 108% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 126% |  |  | 189% |  |  | 194% |  |  |

Performance of Test 2 against SCM2.0 in 4-CTU configuration (JCTVC-S0161)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.6% | -0.6% | -0.2% | -0.3% | -0.3% | 0.0% | -0.1% | 0.0% |
| RGB, text & graphics with motion,720p | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | -0.3% | -0.6% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.5% | 0.5% | 0.6% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion,720p | 0.4% | 0.3% | 0.3% | 0.1% | 0.0% | -0.1% | 0.2% | -0.2% | 0.3% |
| YUV, mixed content, 1440p | 0.0% | -0.3% | -0.3% | -0.1% | -0.8% | -0.6% | -0.4% | -0.8% | -0.6% |
| YUV, mixed content, 1080p | 0.2% | 0.0% | 0.0% | 0.1% | -0.5% | -0.6% | -0.1% | -0.4% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 107% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 129% |  |  | 189% |  |  | 194% |  |  |

Performance of Test 3 against SCM2.0 in CTC configuration (JCTVC-S0161)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.0% | -1.3% | -1.2% | -0.7% | -0.8% | -0.7% | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.5% | -0.4% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% | -0.2% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.3% | -0.4% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.4% | -0.3% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.3% | 0.4% | -0.4% | -0.6% | -0.5% | -0.5% | -0.6% | -0.5% |
| YUV, text & graphics with motion,720p | 0.4% | 0.4% | 0.3% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p | 0.2% | 0.0% | 0.0% | 0.1% | -0.4% | -0.3% | -0.3% | -0.7% | -0.2% |
| YUV, mixed content, 1080p | 0.2% | 0.1% | 0.1% | 0.1% | -0.3% | -0.3% | 0.0% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.0% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 106% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 126% |  |  | 188% |  |  | 193% |  |  |

Performance of Test 3 against SCM2.0 in 4-CTU configuration (JCTVC-S0161)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.9% | -2.3% | -2.2% | -1.2% | -1.5% | -1.3% | -0.7% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -1.2% | -1.6% | -1.4% | -1.5% | -1.7% | -1.7% | -0.4% | -0.6% | -0.6% |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | -0.5% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.3% | -0.4% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -1.0% | -0.8% | -0.9% | -1.3% | -1.2% | -0.8% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | 0.1% | -0.2% | -0.3% | -0.3% | -0.6% | -0.9% | 0.0% | -0.1% | 0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.4% | -0.3% | -0.1% | -0.6% | -0.6% | -0.2% | -0.6% | -0.5% |
| YUV, mixed content, 1080p | 0.1% | -0.1% | -0.1% | 0.0% | -0.5% | -0.5% | -0.5% | -1.1% | -0.9% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 106% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 128% |  |  | 189% |  |  | 194% |  |  |

For test 4, The additional mapping table was not implemented based on the common CE software. More clarification is needed for the evaluation of this proposal. Results were provided only on Oct. 16th. (not available in input document S0192, and not conclusive according to proponent).

Performance of Test 5 against SCM2.0 in CTC configuration (JCTVC-S0161)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.9% | -1.2% | -1.1% | -0.7% | -0.7% | -0.7% | -0.5% | -0.5% | -0.5% |
| RGB, text & graphics with motion,720p | -0.4% | -0.3% | -0.4% | -0.3% | -0.5% | -0.3% | -0.1% | -0.3% | -0.4% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% | -0.1% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | 0.3% | 0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | 0.6% | 0.5% | 0.6% | -0.4% | -0.5% | -0.4% | -0.5% | -0.5% | -0.4% |
| YUV, text & graphics with motion,720p | 0.6% | 0.5% | 0.4% | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.1% | 0.0% | -0.1% | 0.1% | -0.3% | -0.2% | -0.4% | -0.2% | -0.1% |
| YUV, mixed content, 1080p | 0.2% | 0.0% | 0.2% | 0.1% | -0.4% | 0.0% | 0.0% | -0.1% | -0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | -0.1% | 0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% | 0.2% |
| Enc Time[%] | 110% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 127% |  |  | 188% |  |  | 195% |  |  |

Performance of Test 5 against SCM2.0 in 4-CTU configuration (JCTVC-S0161)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -2.1% | -2.0% | -1.2% | -1.4% | -1.3% | -0.8% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -1.1% | -1.4% | -1.2% | -1.3% | -1.5% | -1.4% | -0.1% | -0.4% | -0.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% |
| RGB, mixed content, 1080p | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% | -0.9% | -1.1% | -1.0% | -0.8% | -0.8% | -0.8% |
| YUV, text & graphics with motion,720p | 0.1% | -0.1% | 0.0% | -0.2% | -0.4% | -0.6% | 0.1% | 0.0% | -0.1% |
| YUV, mixed content, 1440p | 0.0% | -0.2% | -0.2% | -0.1% | -0.5% | -0.4% | -0.1% | -0.1% | -0.1% |
| YUV, mixed content, 1080p | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | -0.3% | -0.4% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | -0.1% | 0.2% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 108% |  |  | 105% |  |  | 103% |  |  |
| Dec Time[%] | 123% |  |  | 189% |  |  | 195% |  |  |

Performance of Test 6.1 against CE10 common software in CTC configuration (JCTVC-S0165)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.7% | -0.7% | -0.7% | -0.4% | -0.5% | -0.5% | -0.3% | -0.3% | -0.4% |
| RGB, text & graphics with motion,720p | -0.3% | -0.5% | -0.3% | -0.4% | -0.5% | -0.4% | 0.1% | -0.1% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1080p | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.5% | -0.5% | -0.5% | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% | -0.3% |
| YUV, text & graphics with motion,720p | -0.2% | -0.3% | -0.2% | -0.4% | -0.5% | -0.5% | -0.2% | -0.1% | -0.4% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% | 0.4% |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% | -0.2% | 0.3% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  | 98% |  |  |

Performance of Test 6.1 against CE10 common software in 4-CTU configuration (JCTVC-S0165)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -0.5% | -0.6% | -0.6% | -0.4% | -0.4% | -0.4% | -0.3% | -0.2% | -0.3% |
| RGB, text & graphics with motion,720p | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% | -0.3% |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | -0.2% | 0.0% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | -0.4% | -0.4% | -0.4% | -0.3% | -0.4% | -0.3% | -0.1% | -0.2% | -0.2% |
| YUV, text & graphics with motion,720p | -0.1% | -0.3% | -0.2% | -0.2% | -0.2% | -0.3% | 0.1% | -0.1% | 0.0% |
| YUV, mixed content, 1440p | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.3% | -0.1% | -0.4% | -0.3% |
| YUV, mixed content, 1080p | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | 0.0% | 0.3% | -0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.2% | 0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | -0.5% |  |  | -0.6% |  |  | -0.6% |  |  |
| Dec Time[%] | -0.2% |  |  | -0.3% |  |  | -0.3% |  |  |

Test 6.2 (entropy coding for 1D matching) provided a different method for the coding of offsets based in 1D matching.

It is claimed to be implemented based on JCTVC-S0083 with 1D matching enabled.

It achieves average Luma BD-rate savings of 0.4%, 0.3%, and 0.1% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings, respectively.

Test 6.3 (entropy coding the run length) is compared with the common software of this CE. It achieves average Luma BD-rate savings of 0.3%, 0.2%, and 0.2% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings, respectively.

Performance of Test 7.1 against SCM2.0 in CTC configuration (JCTVC-S0174)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.7% | -2.0% | -1.9% | -1.2% | -1.4% | -1.4% | -1.0% | -1.2% | -1.1% |
| RGB, text & graphics with motion,720p | -0.5% | -0.6% | -0.5% | -0.4% | -0.6% | -0.5% | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.4% | -0.4% | -0.3% | -0.3% | -0.4% | -0.3% | -0.3% | 0.1% | 0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.5% | -1.6% | -1.5% | -1.1% | -1.3% | -1.3% | -0.9% | -1.0% | -1.0% |
| YUV, text & graphics with motion,720p | -0.3% | -0.3% | -0.5% | -0.2% | -0.4% | -0.4% | -0.2% | -0.1% | -0.4% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.5% |
| YUV, mixed content, 1080p | -0.2% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | 0.1% | 0.1% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 145% |  |  | 113% |  |  | 109% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  | 102% |  |  |

Performance of Test 7.1 against SCM2.0 in 4-CTU configuration (JCTVC- S0174)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -1.1% | -1.3% | -1.3% | -0.9% | -1.1% | -1.0% | -0.7% | -0.8% | -0.8% |
| RGB, text & graphics with motion,720p | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.0% | 0.1% | 0.0% | -0.1% | 0.1% | -0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -1.0% | -1.0% | -0.9% | -0.9% | -1.0% | -1.0% | -0.8% | -0.9% | -0.9% |
| YUV, text & graphics with motion,720p | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% | -0.2% | 0.2% | 0.1% | 0.2% |
| YUV, mixed content, 1440p | -0.1% | -0.2% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% | 0.2% | 0.3% |
| YUV, mixed content, 1080p | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% | 0.1% | -0.2% | -0.5% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | -0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 142% |  |  | 115% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 101% |  |  |

For Test 7, JCTVC-S0192, the implementation is not based on the common CE software. It restricts the maximum number of strings to ¼ of pixels (i.e. encoder imposed restriction). However, coding performance gain was observed as -2.0%, -1.5%, -0.9% for YUV, text & graphics with motion, 1080p sequences for the AI, RA, LB settings when compared to CTC.

It was agreed to establish a BoG (chair Y. Chen) to investigate the complexity impact, particularly

- the memory bandwidth requirements for the CE10 common software and the test 1 method (which shows interesting coding gain, but could have impact on the memory access due to switching hor/vert scan).

- the worst case number of context coded bins, for the CE10 common software and the test 6 method(s)

See notes on that BoG report S0308.

The question was also raised, since the target appears similar as with intra line copy (CE4), the gains are usually lower. Due to some commonality in the target, it was suggested to combine the two CEs.

### CE10 primary contributions (5)

JCTVC-S0083 CE10: Result of Test 1 and 2 [B. Li, J. Xu (Microsoft)]

JCTVC-S0161 CE10: Hybrid 1-D/2-D Intra string search for HEVC screen content coding [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)]

JCTVC-S0165 CE10 Results of Test 6 on unified method for entropy coding intra string copy syntax elements [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek)]

JCTVC-S0175 CE10: Test 7.1 Constrained run for Intra String Copy [F. Zou, Y. Chen, V. Seregin, M. Karczewicz (Qualcomm)]

JCTVC-S0305 CE10: Cross-check of test 7.1 Constrained run for Intra String Copy (JCTVC-S0175) [B. Li, J. Xu (Microsoft)] [late]

For the tests checked, the RD results were verified.

JCTVC-S0192 CE10: Results of test 2, 4, 7 [Xianyi Chen, Shuhui Wang, Tao Lin (Tongji), Jing Ye, Shan Liu, Shawmin Lei (MediaTek)]

### CE10 cross checks (2)

JCTVC-S0092 CE10: Cross-check of Hybrid 1-D/2-D Intra string search for HEVC screen content coding (JCTVC-S0161) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0245 CE10: Cross-check of S0083 (Result of Test 1 and 2) [W. Wang, M. Xu, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0281 CE10: Crosscheck of JCTVC-S0192 Test 2 [F. Zou (Qualcomm)] [late] [miss]

# Non-CE Technical Contributions (141)

## SCC (127)

### CE1 related (vector entropy coding) (2)

(Chaired by JRO on Sunday 10-19 p.m.)

JCTVC-S0143 Non-CE1: Block vector coding for Intra block copy mode [K. Rapaka, M. Karczewicz, C. Pang (Qualcomm), K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)]

This contribution proposes a method to code intra block vector for intra block copy (IBC) mode. The proposed method combines aspects from CE1 Test 4.1 and JCTVC-R0182. The CE1 Test 4.1 combines binarization method from CE1 Test 1.1 with aspects in Test 1.3. It is argued that in some scenarios there exists redundancy on the IBC block vectors and removing these redundancies provides coding efficiency improvements. It is asserted that the proposed method effectively removes redundancies in block vector by inferring sign and absolute values of block vector components. It is reported that the proposed approach provides objective bit rate reductions of -1.0 %, -1.2% for All Intra 1080p text and graphics RGB and YUV categories, respectively over SCM-2.0 anchor.

By the time of the presentation of the contribution, a decision about the methods from CE1 has not been taken yet (BoG work not finished). The additional aspect of redundancy reduction is asserted to be interesting and should be further considered (CE).

One expert points out that the syntax should not become too complicated.

JCTVC-S0225 Cross-check of ‘Non-CE1: Block vector coding for Intra block copy’ (JCTVC-S0143) by Mitsubishi and Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

### CE2 related (intra block copy signalling and partitioning) (12)

(Chaired by JRO on Sunday 10-19 p.m.)

JCTVC-S0033 Non-CE2: Intra block vector coding for small PUs [J. Lainema, M. M. Hannuksela (Nokia)]

(include abstract)

Only first PU in CU is fully encoded, for remaining PUs the horizontal component is signalled as conventional, and the vertical component is either zero or inherited. At the same time, fast search s performed. Gain is approx. 0.5% for TGwM class.

Questions: Would it still provide gain in combination with merge?

How would it work when only applied for certain CU sizes? Likely, the method enforces smaller PUs when the model of constant vertical displacement fails

As the gains seem to be sequence dependent, it might also be desirable to turn this off.

Could some of the encoder complexity reduction also be achieved in a non-normative way?

Further study (CE).

JCTVC-S0036 Non-CE2: Transform skip signalling for intra block copy [S. Yang, H. J. Shim, D. Lee, B. Jeon (SKKU)]

Presented Monday 10-20 evening (JRO).

In this contribution, a collective signalling scheme is proposed for all transform skipped TUs at the maximum allowed RQT depth in intra block copy coded CU. In SCM 2.0, transform_skip_flag is coded per each TB not larger than the maximum allowable size for transform skip. This contribution proposes a modified transform skip signalling method which can representatively signal the transform skip in intra block copy coded CU. Under the AI condition, experimental results show an average gain of -0.19% in BD-rate. It is also reported that -0.49% and -0.31% BD-rate gain is achieved respectively for “text and graphics with motion, 1080p” RGB and YUV sequences.

The presentation deck was requested to be made available.

The approach is introducing another syntax element at the CU level to signal invoking transform skip for the whole RQT associated with the CU.

Relative small gain, no complexity reduction. Also not desirable that the parsing of the RQT is changed. Furthermore, IBC is still under further development, and the signalling at CU level may change.

No action.

JCTVC-S0056 Non-CE2 : Slice-level Intra block copy enabling [W. Lim, J. Ma, Y. Ahn, D. Sim (KWU)] [late]

This contribution proposes to signal Intra block copy enabling flag in slice header. From the experimental results of the current SCM2.0, Intra block copy achieves high coding gain compared to the current HM14.0. However, the selection ratio of Intra block copy is quite low in case of inter coding pictures because the temporal correlation is high especially when the temporal distance between the current picture and the reference picture is close. In addition, for coding the mixed contents, Intra block copy selection ratio in natural texture region is also low but the CU-level Intra block copy enabling flag is signaled for every CU. Therefore, in this contribution, we propose slice-level Intra block copy enabling.

Discussion:

- Does not reduce worst case complexity

- Is this meant as compression efficiency i.e. avoiding sending the IBC flag? No results given on that.

- Not obvious that slice header is the best place, currently it is anyway enabled in SPS

- Another option could be CTU or CU level, where CABAC coding would effect a lower rate.

At the current time where IBC is further developed it may be premature to decide about mechanisms of enabling or disbling it at a relatively low level.

JCTVC-S0065 Non-CE2: IBC encoder improvements for SCM2.0 [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

Was presented in track A / non-normative.

See also S0067.

JCTVC-S0261 Crosscheck of JCTVC-S0065 on IBC encoder improvements for SCM2.0 [C. Pang (Qualcomm)]

It was asked for a corrected version of this to be uploaded to fix the abstract.

JCTVC-S0112 Non-CE2: On Intra block copy [C. Pang, V. Seregin, M. Karczewicz (Qualcomm)]

In this contribution, several changes are proposed for Intra block copy, including enabling line buffer for block vector predictor derivation, using the deblocking process like Inter, and using DCT for 4x4 luma block. With these changes, the performance is reported to be an average BD-rate of -0.8% and -0.9% respectively for RGB and YUV 1080p text & graphics with motions sequences under AI configuration.

Some encoder improvements were additionally applied, as per CE2.

Encoder method 1 provides improvements by primarily checking candidates from virtual merge list in the IBC vector derivation method 2 also checks additional partitions

Re-using the line buffer of motion vectors may not be sufficient, as it may require one additional signaling flag for distinguishing and disallowing usage of MV as BV candidates. After some further consideration, it is confirmed that the line buffer can be re-used without additional memory at the decoder side (for an all-intra configuration, additional memory would be required).

Generally, the idea of using the line buffer appears to be beneficial for better compression of the BV. Further investigation of this aspect in CE.

Regarding the DCT, it is pointed out that very often transform skip is used with 4x4, which could be an explanation for the low benefit. It seems to be random whether DST or DCT is better. Which transform to use logically, may depend on whether IBC is classified as inter or intra. Currently, the RQT for IBC follows the inter RQT approach. Another DCT advantage that is pointed out during the discussion is the possibility to use the DC coefficient for constant residual. This requires some further consideration.

For deblocking, this part is related to contribution S0045 in section 5.1.13.

Revisit.

JCTVC-S0227 Cross-check of ‘Non-CE2: On Intra block copy’ (JCTVC-S0112) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-S0113 Non-CE2: Intra block copy with Inter signaling [C. Pang, K. Rapaka, Y.-K. Wang, V. Seregin, M. Karczewicz (Qualcomm), B. Li, J. Xu (Microsoft)]

In this contribution, the Intra block copy mode is signalled as Inter by adding the current picture to the reference picture list(s), similarly as in JCTVC-R0100, with some asserted clean-ups in concepts and signalling. In addition, extra constraints are added for completeness of the signalling. With these changes, the performance is reported to be an average BD-rate of -3.4% and -2.2% respectively for RGB and YUV 1080p text & graphics with motions sequences under AI configuration. The spec text changes, with change marks, are provided in an attachment of this contribution.

The contribution still uses 4x4 blocks

Comparable to CE2 test 1, but using BV coding from SCM instead of MV coding.

Some encoder restrictons, e.g. not using TMVP

Same encoder used as in test 1; compared to that, around 2% bit rate reduction due to usage of BVD.

16x8/8x16 RD check is also enabled. There are probably aspects where part of the gain is achieved by better encoder decisions.

Some more results given using the BV/MV coding method of CE1 1.1/2.1, which gives slightly better performance (around 0.3% for AI, around 2% for inter).

The concept would not fully allow re-using existing inter coding, since it uses 4x4 IBC and dedicated BV coding.

The provided text would allow bi-prediction with IBC (allowing the current picture in L0 and L1), but the encoder does not currently use such an option.

See notes for S0302.

JCTVC-S0302 Non-CE2: Intra block copy and Inter signaling unification [C. Pang, K. Rapaka, Y.-K. Wang, V. Seregin, M. Karczewicz (Qualcomm), X. Xu, S. Liu, S. Lei (MediaTek), B. Li, J. Xu (Microsoft)] [late]

(Chaired by M. Budagavi Wed. a.m.)

[add abstract]

Comment: Syntax table for BVD coding is not the same as RExt.

Comments: Why has the encoder run-time has increased?

Comments: Has the encoder only optimization been included (from Canon and Microsoft) in this proposal? Proponent commented that Canon and Microsoft contribution is orthogonal to this contribution and not included in this contribution. Does IBC merge still gives gain? If so, then this contribution is useful. Otherwise, there is not benefit for this contribution.

Comment: CABAC throughput could be an issue since bi-pred is allowed in spec. But software simulation uses uni-predicition.

Comment: What are the implications if there is an all Intra profile? Wireless display applications will need All Intra profile.

Comments: Apply MVD coding from HEVC V1 to this proposal. Proponent showed results for this. [Copy results from presentation].

Comment: Updated encoder only optimization results claimed to have improved performance. Proponent claims encoder only performance is 4-5% less than what is proposed in this contribution.

Comments: The gain in this contribution come from PU-level IBC.

Three consideration to make decision:

- PU-level IBC vs CU-level IBC

- Encoder only optimization

- Deblocking

Comment: In bi-pred, prediction can be done from both Intra and Inter.

Comment: Low level details are important to understand first. 1. Does IBC merge acutally help, 2. BVD coding: current design claimed to be possibly not optimal, other proposals on table.

Study further in CE and focus on low level details first to resolve questions posed.

JCTVC-S0307 Crosscheck of JCTVC-S0302 on Non-CE2: Intra block copy and Inter signalling unification [A. Minezawa, K. Miyazawa, S. Sekiguchi (Mitsubishi)]

JCTVC-S0284 Crosscheck of JCTVC-S0113 on Non-CE2: Intra block copy with Inter signaling [K. Miyazawa, A. Minezawa, S. Sekiguchi (Mitsubishi)] [late]

JCTVC-S0123 Non-CE2: Intra BC merge mode with default candidates [X. Xu, T.-D. Chuang, S. Liu, S. Lei (MediaTek)]

This document proposes to add default BVs to the merge candidate list if some of the entries in the list are empty. In this proposal, the syntax structure is the same as in CE2 test 5b, the changes made is to add some preset values as additional Intra BC merge candidates to fill up the merge candidate list. The experimental results show that the proposed method brings on average 5.3%, 6.7% and 7.0% bitrate saving as against SCM2.0 anchor for RGB TGM 1080p AI, RA and LB lossy coding, respectively; the proposed method brings on average 3.4%, 6.3% and 6.9% bitrate saving as against SCM2.0 anchor for YUV TGM 1080p AI, RA and LB lossy coding, respectively. Performance gain is also observed for the other cases.

The proposal suggests to add some additional default candidates (instead 0 position) such as left, above, 2xleft etc.

There are also some encoder changes (2 stage RDO decision), which were adopted from CE2 test 3 (but not used in test 5)

Benefit over test 5b around 1% for AI, 1.5% for inter for TGwM classes.

Further study (CE). Would also be interesting to have some analysis how often the additional candidates are used.

JCTVC-S0237 Non-CE2: Crosscheck for Intra BC merge mode with default candidates (JCTVC-S0123) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-S0087 Non-CE2: On block vector predictor [B. Li, J. Xu (Microsoft)]

This document proposes improvements on block vector predictor. First, this document proposes to test BVPs under full RDO for Intra BC mode (non-normative modification). The experimental results show that it brings 2.6% bit saving for RGB TGM 1080p AI lossy coding. Second, this document proposes to modify the BVP construction process. The experimental results shows about 3.8% bit saving for RGB TGM AI lossy coding.

The idea is to “simulate” the merge mode by testing RD cost for BV prediction with BV difference being zero. This already gives 2.6% gain in TGwM class by encoder modification (less for lossless, but still >1%).

Encoder runtime increases by approx. 20% for AI.

A similar idea has been proposed in S0065, however with some more modifications to reduce the runtime. However, the latter proposal does not use the RDO criterion.

The second part determines the two candidates like in AMVP.

Further study (CE).

JCTVC-S0207 Cross check of block vector predictor (JCTVC-S0087) [X. Xu (MediaTek)] [late]

JCTVC-S0172 Non-CE2: Unification of IntraBC mode with inter mode [Y. He, Y. Ye, X. Xiu (InterDigital), X. Xu, S. Liu, S. Lei (MediaTek), B. Li, J. Xu (Microsoft)]

This proposal is a combination of two CE2 tests: block vector derivation in CE2 Test 3, and unification of IntraBC with inter mode in CE2 Test 5b. Compared to Test 5b, the IntraBC merge process is separated from the inter merge process based on intra_bc_flag. The derived block vectors from Test 3 are added as IntraBC merge candidates. Compared to CE2 anchors, for lossy coding, the proposed scheme reportedly achieves average {Y, U, V} BD rate gain of {-4.5%, -6.0%, -5.8%}, {-5.6%, -7.5%, -7.4%} and {-5.6%, -7.2%, -7.0%} for the category (RGB/YUV, text & graphics with motion, 1080p) for AI, RA and LDB, respectively. And the lossless coding reportedly achieves total bit-rate saving of 2.7%, 4.5% and 4.8% for the category (RGB/YUV, text & graphics with motion, 1080p) for AI, RA and LDB, respectively.

The approach also uses a temporal candidate (from another reference picture when an IBC vector is available)

The approach requires storage of block vectors for another row of CTUs.

Benefit over 5b for TGwM is approx. 1% in AI, less than 1% in inter for lossy coding

For lossless coding, it is approx. 0.5% for AI, approx. 2% for inter cases.

Some interesting gain, but the impact on complexity (memory) requires further investigation. There may also be some influence of encoder optimization.

An analysis was requested about the current memory usage, for investigating which aspects should be further studied.

Such analysis was prepared offline and reviewed in further discussion Thursday 10-23 chaired by J. Boyce.

Discussion about what should be included within the CE on IBC unification. S0131 in particular is questioned whether to be included, and the consensus was to not include it.

JCTVC-S0262 Crosscheck of JCTVC-S0172 on unification of IntraBC mode with inter mode [C. Pang (Qualcomm)] [late]

### CE3 related (sub-block partitioning and flipping for Intra block copy) (0)

### CE4 related (intra line copy) (4)

(Chaired by JRO on Monday 10-20 evening)

JCTVC-S0136 Non-CE4: On CABAC Throughput of Intra Line Copy [R.-L. Liao, C.-C. Chen, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

In CE4 Test A (JCTVC-S0135), the Intra Line Copy (IntraLC) mode is studied to investigate the benefit in comparison to restricted and full-frame Intra Block Copy (IntraBC). As JCTVC-S0135 reported, IntraLC requires 528 context-coded bins in its worst-case scenario, with 40 bins and 44 bins more than that for Inter modes (488 bins) and IntraBC (484 bins), respectively. In this contribution, three modifications are applied to IntraLC for reducing the number of context-coded bins as follows: (a) bypass-coding the row_splitting_flag; (b) taking the most-recently decoded BV for BV prediction except the first line of each PU; (c) removing RDPCM for IntraLC. The experimental results show that the number of context-coded bins for IntraLC is reduced to 488 bins. When using the 4-CTU configuration, the average BD-rate reductions of (AI)0.3%/(RA)0.1%/(LB)0.0% and (AI)0.5%/(RA)0.1%/(LB)0.1% relative to CE4 Test A are achieved respectively for RGB and YUV TGM 1080p sequences in lossy coding. When using the full-frame configuration, the average BD-rate reductions of (AI)0.6%/(RA)0.2%/(LB)0.0% and (AI)0.7%/(RA)0.2%/(LB)0.0% relative to CE4 Test A are achieved respectively for RGB and YUV TGM 1080p sequences in lossy coding. The BD-rate reductions relative to the SCM-2.0 anchor in lossy coding are summarized as follows:

- 4-CTU configuration:

  - RGB, text & graphics with motion, 1080p	: (AI) -6.7,	(RA) -3.8, 	(LB) -2.4;

  - RGB, text & graphics with motion, 720p		: (AI) -3.3, 	(RA) -2.6, 	(LB) -1.1;

  - RGB, mixed content, 1440p			: (AI) -2.6, 	(RA) -1.8, 	(LB) -1.0;

  - RGB, mixed content, 1080p			: (AI) -3.9, 	(RA) -3.0, 	(LB) -1.4;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.7,	(RA) -3.4, 	(LB) -2.1;

  - YUV, text & graphics with motion, 720p 	: (AI) -3.2, 	(RA) -2.5, 	(LB) -0.9;

  - YUV, mixed content, 1440p			: (AI) -2.7, 	(RA) -1.9, 	(LB) -0.9;

  - YUV, mixed content, 1080p			: (AI) -4.0, 	(RA) -2.9, 	(LB) -1.6;

  - Encoding time		: (AI) 120%, 	(RA) 104%, 	(LB) 102%;

  - Decoding time		: (AI) 103%, 	(RA) 102%, 	(LB) 103%;

- full-frame configuration:

  - RGB, text & graphics with motion, 1080p	: (AI) -4.4;	(RA) -2.4, 	(LB) -1.5;

  - RGB, text & graphics with motion, 720p		: (AI) -1.5; 	(RA) -1.2, 	(LB) -0.5;

  - RGB, mixed content, 1440p			: (AI) -1.7; 	(RA) -1.1, 	(LB) -0.6;

  - RGB, mixed content, 1080p			: (AI) -2.5; 	(RA) -1.9, 	(LB) -0.7;

  - YUV, text & graphics with motion, 1080p	: (AI) -4.4;	(RA) -2.2, 	(LB) -1.3;

  - YUV, text & graphics with motion, 720p 	: (AI) -1.5; 	(RA) -1.1, 	(LB) -0.7;

  - YUV, mixed content, 1440p			: (AI) -1.6; 	(RA) -1.1, 	(LB) -0.6;

  - YUV, mixed content, 1080p			: (AI) -2.5; 	(RA) -1.8, 	(LB) -0.7;

  - Encoding time		: (AI) 128%, 	(RA) 107%, 	(LB) 108%;

  - Decoding time		: (AI) 102%, 	(RA) 104%, 	(LB) 104%;

Part of the reduction of context coded bins is achieved by enforcing RDPCM off when intra line copy is used. This does not introduce parsing dependency.

Intra line copy search range and copy range is 2 CTU in the full-frame configuration of IBC, and 4 CTU in the 4 CTU configuration of IBC. Therefore, ILC would not have a memory bandwidth problem under the assumption that an appropriate area can be accessed from cache.

The assumption about availability of sufficient cache size may need further consideration.

Further study in CE. Also study the combination of 4-CTU ILC and full-frame IBC (CTC, i.e. full frame only for 8x8 and larger).

JCTVC-S0249 Non-CE4: Cross-check of S0136 (On CABAC Throughput of Intra Line Copy) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0137 Non-CE4: Intra Line Copy with Extended Full-frame Search for Test A [R.-L. Liao, C.-C. Chen, C.-W. Kuo, W.-H. Peng, H.-M. Hang (NCTU/ITRI)]

In CE4 Test A (JCTVC-S0135), the intra line copy (IntraLC) mode was tested under full-frame configuration for search range. The search range of IntraLC and 4x4/4x8/8x4 IntraBC are both restricted to 2 CTUs (the current plus 1 left CTU). In this contribution, the full-frame search ranges were evaluated for IntraLC and 4x4/4x8/8x4 IntraBC. Three tests are conducted as follows: (Subset 1) 2-CTU search for 8x4/4x8/4x4 IntraBC and full-frame search for IntraLC; (Subset 2) full-frame search for 8x4/4x8/4x4 IntraBC and no IntraLC; (Subset 3) full-frame search for 8x4/4x8/4x4 IntraBC and full-frame search for IntraLC . Experimental results are reported as follows:

- (Subtest 1) full-frame search of IntraLC vs. SCM-2.0:

  - RGB, text & graphics with motion, 1080p	: (AI) -10.1,	(RA) -5.4, 	(LB) -3.6;

  - RGB, text & graphics with motion, 720p		: (AI) -4.6, 	(RA) -3.2, 	(LB) -1.6;

  - RGB, mixed content, 1440p			: (AI) -4.0, 	(RA) -2.5, 	(LB) -1.2;

  - RGB, mixed content, 1080p			: (AI) -7.0, 	(RA) -4.9, 	(LB) -2.1;

  - YUV, text & graphics with motion, 1080p	: (AI) -9.9,	(RA) -4.7, 	(LB) -2.9;

  - YUV, text & graphics with motion, 720p 	: (AI) -4.5, 	(RA) -3.2, 	(LB) -1.5;

  - YUV, mixed content, 1440p			: (AI) -4.1, 	(RA) -2.6, 	(LB) -1.4;

  - YUV, mixed content, 1080p			: (AI) -7.1, 	(RA) -5.3, 	(LB) -2.0;

  - Encoding time	: (AI) 128%, 	(RA) 109%, 	(LB) 108%;

  - Decoding time	: (AI) 102%, 	(RA) 102%, 	(LB) 103%;

- (Subtest 2) full-frame search of 8x4/4x8/4x4 IntraBC vs. SCM-2.0:

  - RGB, text & graphics with motion, 1080p	: (AI) -6.0,	(RA) -3.1, 	(LB) -2.3;

  - RGB, text & graphics with motion, 720p		: (AI) -3.5, 	(RA) -2.4, 	(LB) -1.2;

  - RGB, mixed content, 1440p			: (AI) -2.6, 	(RA) -1.7, 	(LB) -1.0;

  - RGB, mixed content, 1080p			: (AI) -4.7, 	(RA) -3.3, 	(LB) -1.6;

  - YUV, text & graphics with motion, 1080p	: (AI) -6.1,	(RA) -2.8, 	(LB) -1.8;

  - YUV, text & graphics with motion, 720p 	: (AI) -3.6, 	(RA) -2.5, 	(LB) -1.3;

  - YUV, mixed content, 1440p			: (AI) -2.8, 	(RA) -1.5, 	(LB) -0.8;

  - YUV, mixed content, 1080p			: (AI) -4.9, 	(RA) -3.4, 	(LB) -1.2;

  - Encoding time	: (AI) 114%, 	(RA) 108%, 	(LB) 107%;

  - Decoding time	: (AI) 100%, 	(RA) 101%, 	(LB) 101%;

- (Subtest 3) full-frame search of IntraLC on top of Subset 2 vs. SCM-2.0:

  - RGB, text & graphics with motion, 1080p	: (AI) -12.9,	(RA) -7.0, 	(LB) -4.8;

  - RGB, text & graphics with motion, 720p		: (AI) -6.2, 	(RA) -4.3, 	(LB) -2.3;

  - RGB, mixed content, 1440p			: (AI) -5.0, 	(RA) -3.2, 	(LB) -1.7;

  - RGB, mixed content, 1080p			: (AI) -8.2, 	(RA) -5.7, 	(LB) -2.5;

  - YUV, text & graphics with motion, 1080p	: (AI) -13.0,	(RA) -6.3, 	(LB) -4.0;

  - YUV, text & graphics with motion, 720p 	: (AI) -6.3, 	(RA) -4.5, 	(LB) -2.2;

  - YUV, mixed content, 1440p			: (AI) -5.2, 	(RA) -3.3, 	(LB) -1.5;

  - YUV, mixed content, 1080p			: (AI) -8.5, 	(RA) -6.2, 	(LB) -2.6;

  - Encoding time	: (AI) 143%, 	(RA) 116%, 	(LB) 115%;

  - Decoding time	: (AI) 102%, 	(RA) 102%, 	(LB) 103%;

- (Subtest 3) full-frame search of IntraLC on top of Subset 2 vs. Subset 2:

  - RGB, text & graphics with motion, 1080p	: (AI) -7.3,	(RA) -4.0, 	(LB) -2.6;

  - RGB, text & graphics with motion, 720p		: (AI) -2.9, 	(RA) -2.0, 	(LB) -1.1;

  - RGB, mixed content, 1440p			: (AI) -2.5, 	(RA) -1.6, 	(LB) -0.7;

  - RGB, mixed content, 1080p			: (AI) -3.6, 	(RA) -2.5, 	(LB) -0.9;

  - YUV, text & graphics with motion, 1080p	: (AI) -7.3,	(RA) -3.6, 	(LB) -2.2;

  - YUV, text & graphics with motion, 720p 	: (AI) -2.9, 	(RA) -2.1, 	(LB) -0.9;

  - YUV, mixed content, 1440p			: (AI) -2.5, 	(RA) -1.8, 	(LB) -0.8;

  - YUV, mixed content, 1080p			: (AI) -3.8, 	(RA) -2.9, 	(LB) -1.4;

  - Encoding time	: (AI) 125%, 	(RA) 107%, 	(LB) 108%;

  - Decoding time	: (AI) 103%, 	(RA) 102%, 	(LB) 102%;

Interesting study, but ILC with full frame would break the memory bandwidth limits and should therefore not be further investigated.

JCTVC-S0126 Cross check of Non-CE4 Intra Line Copy with Extended Full-frame Search for Test A (JCTVC-S0137) [X. Xu (MediaTek)] [late]

JCTVC-S0290 Non-CE4: Cross-verfication of JCTVC-S0137 on Intra Line Copy with Extended Full-frame Search for Test A [X. Xiu] [late]

### CE5 related (maximum palette size and maximum palette predictor size) (3)

See notes on S0201 in section on CE related contributions (section 5.1.6).

### CE6 related (palette mode improvement) (54)

BoG S0292

See also S0201.

JCTVC-S0292 BoG report on CE6 improvements of palette mode [R. Cohen, Y.-W. Huang]

(Chaired by GJS on Monday 10-20 p.m.)

This document contains the notes from the BoG meeting on CE6-related topics (palette mode improvements).

The mandate of this BoG was to review remaining (approximately 35) CE6-related proposals that were not discussed during the JCT-VC session on Friday October 17.

The BoG met on Saturday October 18, 2014 from 0820–1930 and Sunday October 19, 2014 from 0830–2015. Notes from the BoG are integrated in this report as noted. Recommendations from the BoG were agreed except as noted.

(Chaired by GJS on Tuesday 10-21 a.m. for continuation of review.)

JCTVC-S0043 Non-CE6: Delta QP signalling for palette [J. Sole, W. Pu, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)]

Notes from BoG:

The palette adopted at the Sapporo meeting quantizes escape values with the coefficient quantization parameter (QP). However, there is no mechanism to vary this QP for the CUs coded with palette, so an encoder has to work at constant QP as long as CUs are coded in palette mode.

Palette specifies a CU-level escape flag that indicates whether escape pixels exist in the current CU. This document, as in JCTVC-R0237, proposes to use the CU escape flag as entry point for the syntax elements that adjust the QP, i.e., like the cbf flags in residue coding. When CU-level escape flag is on, then delta QP can be signalled. In this way, encoders have the flexibility to use palette as well as adjust the QP. Furthermore, this change allows adjusting the QP for palette deblocking and permits to use palette as a PCM mode with a tunable quantization step.

Source code was provided, but no simulation results were provided because the common test conditions do not use the QP step size adjustment.

There was a question on whether a sufficient number of palette-coded blocks would use escape mode, and if there are only 1 or 2 escape pixels in a CU, this proposal would introduce additional overhead.

There was a comment on propagating the earlier-used QP values in the slice.

There was a suggestion on enabling rate control to show the performance of this proposal.

It was noted that the proposed syntax needs to be modified to check to see whether cu_qp_delta_abs has already been signalled for the CU, and if so, use that cu_qp_delta_abs instead of signalling another one. Therefore, the current proposal also needs to add a check for !IsCuQpDeltaCoded for luma and a check for !IsCuChromaQpOffsetCoded for chroma.

A participant commented that they would support this proposal if the syntax and software were corrected.

The proponents indicated that they would update their proposal and provide simulation results with a delta QP of zero to determine the impact on overhead. They will also look at the rate control to see if they can generate results with nonzero delta QP.

BoG Recommendation: Discuss further when updated syntax and simulation results are available.

Further discussion was held by JCT-VC Wed (GJS) p.m. The syntax fix had been included in a revision of the contribution and test results were provided with zero deltas – showing basically no measurable overhead.

Decision: Adopt (with the syntax fix).

JCTVC-S0047 Non-CE6: Modifications of copy-left and copy-above modes in index coding [J.-S. Tu, C.-L. Lin, C.-H. Hung, C.-C. Lin, Y.-J. Chang (ITRI)]

Notes from BoG:

In past meetings, several palette redundancy removal methods are provided. This proposal presents redundancy removal methods for the palette coding; in certain cases, the index value of current pixel can be deduced impossible to have the same index value of its neighbouring pixels. Therefore the index coding bits can be saved. This proposal also presents a copy-above mode modification method. The reported gain is 0.3% for 1080p text and graphics test sequences for full frame intra BC test condition at All Intra lossy case.

Three methods are proposed to reduce the number of runs or palette indices that need to be signalled.

A fourth supplemental result, not yet cross-checked, was also proposed (0.4% gain).

For the three methods, they do not have complete simulation results on their individual performance.

It was asked whether the gains are sufficient to propose adopting at this meeting, or whether further study in a CE should be done. The proponent said that study in a CE would be appropriate, and results for the individual proposed methods could be provided.

This method currently checks if RC ≥ 3. It was commented that JCTVC-S0156 proposes (non-normative) a parameter to select run mode, and that could affect whether the RC ≥ 3 would need to be changed accordingly.

It was asked whether parsing dependencies are introduced, as the neighboring reconstructed pixel values need to be checked. The proponent clarified that the dependency is on parsing the index, not on reconstructing any pixel values. Therefore it is asserted that parsing dependencies are not an issue.

BoG Recommendation: Further study in a CE with Method 3 of JCTVC-S0110, and to study the gains of the individual methods.

JCTVC-S0260 Cross-check of Non-CE6: Modifications of copy-left and copy-above modes in index coding (JCTVC-S0047) [S. H. Kim, A. Segall (Sharp)] [late]

JCTVC-S0051 Non-CE8: Multi-colour intra mode for screen content coding [Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

Notes from BoG:

JCTVC-S0050 proposed a combined single color and bi-color mode, which was a combination of two CE8 tests. This contribution proposes a multi-color intra mode, which is an extension of JCTVC-S0050. The multi-color intra mode includes single color mode, bi-color mode and the color modes with more colors. The colors of the multi-color intra mode are selected from the neighboring reference sources. For the modes with the number of colors over one, an index map is established to represent each pixel in the CU by an index. Compared to the anchor SCM2.0, the proposed mode reportedly shows that 1.0%, 1.0%, 0.8% and 0.8% BD-rate savings under All-Intra Lossy-coding conditions are achieved for the classes of “RGB, text & graphics with motion,1080p”, “RGB, text & graphics with motion,720p”, “YUV, text & graphics with motion,1080p” and “YUV, text & graphics with motion,720p”, respectively.

This was also briefly visited yesterday in the CE8 discussions.

It was clarified that the main part of this proposal is a new mode, as discussed yesterday. The proponent said that this could be viewed as the same as palette mode, but using a different method for generating the major colors. However, yesterday, the decision was that adding a new mode for these amount of gains was not desirable.

The proponents said they are currently working on incorporating this proposed method into the existing palette mode, thus not as an additional mode, and that would be of interest for study in a CE. It was commented that the signalling would be in palette mode, but the encoder would still need to perform an additional R-D check to decide whether to use this mode.

It was mentioned JCTVC-S0098 included a single-color mode incorporated into palette, up to 0.8% gain.

It was commented that because results aren’t available yet for their supplemental results, there isn’t sufficient information to recommend study in a CE. Later in the day, the proponents said that their supplemental results, not yet cross-checked, are available and will be uploaded in a revision of this document shortly.

It was commented that similar gains could be achieved using encoder-only methods, such as including a check for, e.g. one color or two colors in the existing palette mode.

BoG Recommendation: Further study, but not necessarily in a CE.

JCTVC-S0209 CE8-related: Crosscheck for Multi-colour intra mode for screen content coding (JCTVC-S0051) [Y.-W. Chen (MediaTek)] [late]

Grouped notes for S0052, S0053, and S0054

Notes from BoG.

Proposals JCTVC-S0052, JCTVC-S0053, and JCTVC-S0054 were combined into one presentation.

In JCTVC-S0052, escape coded pixels are predicted using escape coded pixels those occurred in previous palette-coded CUs. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, there is a flag to indicate current escape coded pixel is predicted or not. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 3.4% and 5.1% on TGM RGB and YUV (AI), 2.6% and 3.1% on TGM RGB and YUV (RA), 2.5% and 2.8% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 128;

The other is use max index value to indicate escape mode. Reported test results show gain of 2.0% and 3.4% on TGM RGB and YUV (AI), 1.6% and 2.1% on TGM RGB and YUV (RA), 1.6% and 1.9% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 128.

The size of the predictor array was tested using values of 64 and 128.

In JCTVC-S0053, escape coded pixels are predicted using palette and escape coded pixels those occurred in previous palette-coded CUs. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, use two flags to indicate current escape coded pixel is predicted from palette predictor, escape predictor or not predicted. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 2.6% and 3.8% on TGM RGB and YUV (AI), 1.8% and 2.1% on TGM RGB and YUV (RA), 1.7% and 1.9% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 64 and maximum palette predictor size is 64;

The other is use max index value to indicate escape mode. Reported test results show gain of 1.3 % and 2.3% on TGM RGB and YUV (AI), 0.9% and 1.3% on TGM RGB and YUV (RA), 0.9% and 1.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum escape predictor size is 64 and maximum palette predictor size is 64.

This proposal doesn’t code the differences, it copies the differences, in contrast to CE6 Test B.5, which codes the residuals.

In JCTVC-S0054, escape coded pixels are predicted using previous palette. There is a CU-level flag to indicate existence of escape coded pixel prediction mode in current CU. If the prediction exists, for each escape pixel, there is a flag to indicate current escape coded pixel is predicted or not. For those escape coded pixels which are predicted, there is an index to indicate which element of predictor is its prediction.

There are 2 tests. One is using escape flag to indicate escape mode. Reported test results show gain of 2.1% and 3.4% on TGM RGB and YUV (AI), 1.2% and 1.5% on TGM RGB and YUV (RA), 1.0% and 1.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum palette predictor size is 128. And, gain of 1.3% and 1.4.% on TGM RGB and YUV (AI), 0.8% and 0.7% on TGM RGB and YUV (RA), 0.8% and 0.6% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor is reported when maximum palette predictor size is 64.

The other is use max index value to indicate escape mode. Reported test results show gain of 0.8% and 2.1% on TGM RGB and YUV (AI), 0.3% and 0.7% on TGM RGB and YUV (RA), 0.2% and 0.5% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor when maximum palette predictor size is 128. And, small gain is reported when maximum palette predictor size is 64.

See notes for each proposal below.

Summary of lossless gains for MAX_PLT_PRED_SIZE=64:

JCTVC-S0052: 0.1% to 2.4%

JCTVC-S0053: 0.1% to 2.3%

JCTVC-S0054: 0.0% to 0.2%

There was a question on why combined palette/escape prediction performs lower than just prediction via escape. Answer is that there are two flags used for the combined palette/escape prediction.

It was commented that JCTVC-S0053 is similar to an escape coding related proposal in CE6 B.5, with some differences, e.g. CE6 B.5 used a predictor size of 32, whereas this proposal uses 64 or 128. CE6 Test B.5 had slightly lower gains. Because the current plans are to include CE6 Test B.5 in a CE, this proposal would be suitable for inclusion in that CE as well.

BoG Recommendation: Include this proposal in the CE that’s related to CE6 Test B.5.

JCTVC-S0052 Non-CE6: Escape coded pixel prediction using previous escape coded pixels for palette based coding [J. Ye, J. Zhu (Fujitsu)]

Notes from BoG.

Gains for all-intra RGB for method in which the escape mode uses a flag: Lossy AI gains range from 0.1% to 0.4%; lossless range from 0.4% to 5.1%.

Gains when escape mode uses an index: lossy from 0.0% to 0.2%; lossless from 0.2% to 3.4%.

Additional results using a smaller predictor size (64 as opposed to 128) were presented, lossless gains were up to 3.7%.

See grouped notes above for S0052, S0053, and S0054.

JCTVC-S0169 Non-CE6: Cross check of JCTVC-S0052 on escape coded pixel prediction using previous escape coded pixels for palette based coding [S.-T. Hsiang (MediaTek)] [late]

JCTVC-S0053 Non-CE6: Escape pixel prediction using previous escape coded pixels and palette for palette based coding [J. Ye, J. Zhu (Fujitsu)]

Notes from BoG.

Corresponding gains (MAX_PLT_PRED_SIZE = 128) are:

Using a flag: lossy: 0.0% to 0.4%; lossless: 0.3% to 3.8%

Using index: lossy: 0.0% to 0.1%; lossless: 0.1% to 2.3%

See grouped notes above for S0052, S0053, and S0054.

JCTVC-S0170 Non-CE6: Cross check of JCTVC-S0053 on escape pixel prediction using previous escape coded pixels and palette for palette based coding [S.-T. Hsiang (MediaTek)] [late]

Has "the" problem.

JCTVC-S0054 Non-CE6: Escape coded pixel prediction using previous palette for palette based coding [J. Ye, J. Zhu (Fujitsu)]

Notes from BoG.

Using a flag: lossy: Not tested; lossless: 0.2% to 3.4%

Using index: lossy: Not tested; lossless: 0.0% to 2.1%

Additional results for MAX_PLT_PRED_SIZE =64 were included in an updated document (lossless gains up to 2.4%).

See grouped notes above for S0052, S0053, and S0054.

JCTVC-S0246 Non-CE6: Cross-check of S0054 (Escape coded pixel prediction using previous palette for palette based coding) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0055 Non-CE6: Escape colour signalling [C.-C. Lin, C.-L. Lin, Y.-J. Chang, J.-S. Tu, C.-H. Hung (ITRI)]

Notes from BoG:

This contribution proposes escape colour signalling methods for the palette based coding. Escape colours are signalled by the copy above run mode. A CU-level flag is also proposed to enable/disable the proposed signalling method. Compared with the SCM 2.0 full frame IBC anchor, the proposed method can achieve total bit-rate savings for lossless coding conditions up to 1.0%, 0.2%, 0.2%, 0.1% for lossless AI for R component excluding categories of animation and camera captured.

Method 1 moves the escape colour signalling from copy left mode to copy above mode (only when escape present flag is on). Pixel-level flag indicates whether it is escape mode or copy above mode.

Method 2 Copy above run value starts from 1, so the pixel-level flag isn’t needed.

Method 3 also uses a CU level flag to indicate escape signalling change (copy left mode/copy above mode)

Summary of results:

Test 1: Method 1 + Method 3: Lossless RGB all-intra gains: 0.1 to 1.0%, for YCbCr: 0.0% to 0.8%

Test 2: Method 2 + Method 3: Lossless RGB 0.1% to 1.0%; YCbCr: 0.1% to 0.8%

The proponent asks for adoption of Test 2 (Method 2 + Method 3). (Note that Method 2 is a modification of Method 1).

It was commented that this is similar to using a flag for escape in JCTVC-S0052/0053/0054. This proposal, however, signals in copy-above mode. The proponent clarified that 52/53/54 is a pixel-level flag, but it signals before the palette mode flag (to choose copy-above or index mode). In this proposal, the escape signalling is only signalling copy above mode, and there’s a CU-level flag to switch to the conventional mode.

It was commented that this proposal deviates from the current design, with respect to indication of the escape color.

BoG Recommendation: A signalling method was recommended under the “Combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258” (under JCTVC-S0258), therefore deferred for further study.

JCTVC-S0214 Cross check non-CE6: Escape colour signalling [W. Pu (Qualcomm)] [late]

(Has "the" problem.)

JCTVC-S0060 Non-CE6: Modified unpredicted palette entries coding in palette based coding [Z. Wang, J. Zhu (Fujitsu)]

Notes from BoG:

Modification of unpredicted palette entry coding is presented. A flag is used to indicate whether the difference between the current palette entry and previous one or original value is coded. The process of encoding and decoding is proposed. Reported test results show gain of 0.5% and 0.7% on TGM RGB and YUV (AI), 0.1% and 0.2% on TGM RGB and YUV (RA), 0.1% and 0.2% on TGM RGB and YUV (LB) for lossless coding on top of SCM2 anchor.

This proposal comprises 3 steps:

- sort (encoder-side only) the palette depend on the value of the first color component

- reorder the palette (encoder side only)

- code the non-predicted palette entries, using one method to code the first component and another method for coding the remaining two components.

Lossy all-intra gains: 0.0% to 0.1%

Lossless all-intra gains: 0.1% to 0.7%

It was asked if this proposal replaces the current CABAC coding table for reuse flags. Answer: This proposal doesn’t change that.

The encoder/decoder run-time ratios were not reliable in the reported results and in the cross-check results. It was asked whether the encoder-side sorting introduces much complexity. Another person commented that the complexity increase is non-trivial. The proponent stated that the encoder implementation was not optimized. The proponent asserts that the complexity of the sorting part decreases.

BoG Recommendation: No action.

JCTVC-S0247 Non-CE6: Cross-check of S0060 (Modified unpredicted palette entries coding in palette based coding) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0064 Non-CE6: Last run flag for Palette mode [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

Notes from BoG:

This contribution is about a modification of the run coding in the current design of the palette coding mode. The SCM2.0 design of the run coding is modified in order to insert a flag after the palette_run > 2 flag for signalling if the current run is the last run for the processed CU. It is reported that the results of this modification brings an average BDR gain of -0.3% for AI, -0.2 % for RA and -0.2% for LDB for the lossy case on Screen Content sequences.

When the last run flag is 1, then the remainder of the CU uses the same mode (copy above or copy left) (in contrast to signalling the actual number of elements included in the run, as in the current syntax).

RGB all-intra lossy gains: 0.2% to 0.6%. RA: 0.0% to 0.3%; LB: 0.1% to 0.4%

The RA and LB results in the presentation differ from those in the document. The proponent will correct and upload the presentation.

The proponent also tested this method in CE6 A.6 configuration 1. The truncation part of the truncated Golomb code is replaced with this proposed last run flag. Gains averaged to about 0.0% for all-intra.

It was commented that JCTVC-S0111 is a related competing method. It was asserted that the truncated code and the method in JCTVC-S0111 could be combined, and the gains would be additive.

See JCTVC-S0111 for continued discussion of this proposal.

JCTVC-S0267 Cross-verification of JCTVC-S0064 on Last run flag for Palette mode [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-S0066 Non-CE6: Palette encoder improvements for SCM2.0 [C. Gisquet, G. Laroche, P. Onno (Canon)]

See also notes for S0067.

Notes from BoG:

This contribution is proposing three encoder improvements related to Palette coding mode in the SCM2.0 without introducing any normative change. It is reported that the proposed encoder changes enable decreasing the encoding runtime ratio to 97% compared to SCM2 while keeping the same BDR performance. It is furthermore asserted that the improvement is bigger in encoders where IBC has reduced runtime.

The three changes are:

The process determining palette elements is terminated if the proportion of escape-coded pixels exceeds a threshold.

If the current original pixel is identical to the previous original pixel, then the index associated with the previous pixel is reused.

Palette sharing is evaluated only if the current best mode for coding the CU is palette mode and if a large proportion of the palette elements (in non-shared mode) are predicted.

The cross-check is still running. It was noted that the cross-check run-time ratios will not be reliable.

This is an encoder-only change.

It was commented that the comparisons for method (2) on average may reduce complexity, but worst case may increase complexity.

Results are not available for the individual methods.

BoG Recommendation: No action.

JCTVC-S0278 Cross check Non-CE6: Palette encoder improvements for SCM2.0 [W. Pu (Qualcomm)] [late]

JCTVC-S0079 Non-CE6: Cross-CU palette colour index prediction [Y.-C. Sun, J. Kim, T.-D. Chuang, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

Notes from BoG:

JCTVC-S0114 is included in this discussion.

For JCTVC-S0079: In JCTVC-P0093, it was proposed for palette mode to use the nearest row of the above CU or the nearest column of the left CU to predict colour indexes of the current CU. In this proposal, the method is ported to SCM-2.0 and evaluated again. Experiments show that, compared with SCM-2.0,2.5%, 1.3%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p sequences” under AI, RA, and LB, respectively.

For JCTVC-S0114: This contribution proposes enabling ‘Copy Above’ prediction mode for the first line of the CU when it is coded as Palette mode. The index of neighbouring pixels of the CU is used as index predictor and the indices are derived with the palette of the current CU. It shows maximum 2.5%, 2.5%, 2.5% gain and average 0.6%, 0.7%, 0.7% gain for All Intra configuration. Maximum 1.3%, 1.5%, 1.4% and average 0.4%, 0.5%, 0.5% gain for Random Access, Maximum 0.9%, 1.2%, 1.2% and average 0.3%, 0.4%, 0.3% gain for Low Delay cases respectively. This contribution presents 4 variations of the method to decide the indices of neighbouring pixels.

A revised version of JCTVC-S0079 document with complete simulation results should be uploaded shortly.

Neighboring palette indices are used to predict the palette indices of the current CU.

JCTVC-S0079: Predicts the colour index from row from CU above or column from CU to the left, and applies redundancy index removal on the first row or column of the current CU. All-intra lossy gains: 0.3% to 2.5%; lossless gains: 0.0% to 1.1%.

JCTVC-S0114: Same as JTVC-S0079, except that redundancy index removal is not applied on the first row or first column. All intra lossy gains: 0.3% to 2.5%; lossless gains: 0.0% to 1.1%

Some simplifications for implementing the index conversion (from pixel to index) are also proposed:

- Infer NCP indices as 0 (similar to JCTVC-S0155). Gains of [this method + S0079]: 0.0% to 0.3%

- Reduce number of colors that are searched (from the full 32 to the first 8). Gains of [this method + S0079]: 0.2% to 2.3%

- Search only for the first color component: Gains of [this method + S0079]: 0.3% to 2.4%

- Directly copy the pixel value of NCP for the current pixel, instead of converting the NCP into palette indices. For example, if copy-above mode is used, instead of converting the pixel to index, the pixel from above is copied. Gains of [this method + S0079]: 0.1% to 1.9%

Simulation results (run-time ratios not accurate) for each of the above simplifications on top of JCTVC-S0079 were provided (see above for lossy all-intra gains).

A participant said that it would not be desirable to incur the parsing dependency from the JCTVC-S0079 and JCTVC-S0114 main parts of the proposal, especially if the decoder has corrupted reconstructed pixel values (thus mapping to the wrong index value), e.g. due to loss. Other participants said that this is not a parsing issue, it's related to error propagation as would be the case for the existing modes. The proponent asserted that there’s no parsing dependency. A participant said that introducing these copy left and copy above methods may limit the ways that the decoder can be implemented, e.g. can’t be parallelized on a block level (e.g., if one has to wait for reconstructed pixels to be available). That participant said more study of this may be needed in a CE.

Another question was what would the gains of simplification (1) be on top of the anchor (without JCTVC-S0079). Participants said that this would likely be similar to JCTVC-S0155).

The proponent of JCTVC-S0120 implemented something similar to JCTVC-S0079 (two lines instead of one line) on top of CE6 Test C.3 (proposes copy previous row).

BoG Recommendation: Further study in a CE (together with S0114 and S0155), to answer the questions on parallelization and the performance of simplification method (1). Also see the discussion on JCTVC-S0155.

Further information discussion: At the CU boundary, requires a search to match the pixel value above to an index in the current palette, using the closest value match. If the closet match exceeds a threshold SAD, uses index 0. It was remaked that currently the pixel values above are not needed in palette mode. One mentioned variant wsa to treat the above pixels as having been escape coded.

- JCTVC-S0114 Enabling copy above mode prediction at the boundary of CU [J. Kim, Y.-C. Sun, S. Liu, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

See notes for S0079.

- JCTVC-S0255 Non-CE6: Cross-check of enabling copy above mode prediction at the boundary of CU (JCTVC-S0114) [V. Seregin (Qualcomm)] [late]

JCTVC-S0155 Non-CE6: Palette copy above mode for the first row [V. Seregin, M. Karczewicz, W. Pu, R. Joshi, J. Sole (Qualcomm)]

Notes from BoG:

This contribution proposes to extend the use of the palette copy above mode for the very first row in the block. It is achieved by assigning the default palette index for the samples outside of the block. Reportedly, it provides 0.6% and 0.8% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration, respectively.

This proposes to enable copy above mode for the first row, as follows:

Signal palette mode for the first row with additional context

Assign a palette index 0 for the samples coded with copy above mode in the first row

Lossy all-intra gains: 0.0% to 0.5%

This method is related to parts of JCTVC-S0079 and S0114.

If, additionally, If left and above indices are equal, then assign palette index 0 for copy above mode; then the gains are 0.0% to 0.6%. This method is related to Section 2.2 of JCTVC-S0047.

Recommendation: As with the related parts of JCTVC-S0079, S0114, and S0047/S0110, further study in a CE (or split across two CE’s if S0047/S0110 is studied under a different CE).

See notes above for S0079.

JCTVC-S0219 Cross-verification of JCTVC-S0155 [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-S0291 Cross-check of JCTVC-S0079 [X. Guo (Microsoft)] [late]

JCTVC-S0099 Non-CE6: Syntax redundancy removal for palette mode with one index value [P. Lai, J. Kim, S. Liu, S. Lei (MediaTek)]

Notes from BoG:

For palette mode, it is possible that for all samples, their palette indices are all the same (e.g. all zeros), such as there is only one colour in the current CU’s palette. This contribution proposes that, for palette mode with one index value (e.g. zero), omit signalling the syntax elements related to the palette index map, and derive the unique palette index value (e.g., zero).

All Intra lossy gains: 0.0% to 0.1%; -0.1 to 0.1 for RA; -0.1% to 0.3% for LB

JCTVC-S0173 Part 1 is a superset of what’s proposed in JCTVC-S0099 (no text was available yet for JCTVC-S0099).

JCTVC-S0110 proposes a similar method.

See the discussion on JCTVC-S0173 for further discussion of this proposal.

JCTVC-S0105 CE6-related: Syntax fixes for zero palette in palette coding [K. Zhang, T.-D. Chuang, S. Liu, J. An, X. Zhang, S. Lei (MediaTek)]

There are two problems in the current syntax design for palette coding. First, the palette of the current CU may be coded in a sharing mode even when the palette size of the previous CU is zero. Second, there is no valid palette index for coding pixels in a CU when the palette size of the current CU is zero and the CU-level escape flag is 0. For solving the first problem of syntax redundancy, it is proposed to infer the palette_sharing_flag as 0 if the palette size of last coded palette is 0. For solving the second problem that can lead to decoder crash, it is proposed to infer palette_escape_val_present_flag as 1 if the palette size of current palette CU is 0. It also includes signalling value of pixels as PCM. The results reportedly show negligible coding performance change.

Fix 1 is more of a clean-up (syntax redundancy removal), whereas Fix 2 is a bug fix, without which the decoder could crash.

It was commented that if there’s no coding performance improvement for Fix 1, then don’t adopt it.

Fix 2 is the same as Modification 1 in JCTVC-S0173 and Method 1.2 of JCTVC-S0110.

Alternative method to Fix 2: If the palette size is zero, then use pcm mode to code the block. Negligible change on coding performance reported. It was commented that the encoder could choose to use pcm when the palette size is zero, instead of modifying the decoder syntax in this proposed alternative method.

BoG Recommendation: Adopt Fix 2 of JCTVC-S0105/Modification 1 of JCTVC-S0173/the part of Method 1.2 of JCTVC-S0110 that is identical to these methods (infer palette_escape_val_present_flag as 1 if the palette size of current palette CU is 0).

Offline discussion was requested on whether to include the additional syntax changes of JCTVC-S0110 Method 1.2 was requested. For Fix 1, no action.

Decision: Adopt per BoG recommendation

Continued discussion of S0099, S0110 and S0173:

Notes from BoG:

S0099 is the same as second part of modification 1 in S0173 and is a subset of Method 1.2 of S0110: palette_mode and palette_run will not be signalled when there is only one color in the CU due to one of two possibilities: 1) there is only one color in the palette table and there is no escape color, or 2) the palette table is empty.

palette_coding( x0, y0, nCbS ) {

…

if( indexMax > 0 && scanPos > = nCbS && palette_mode[xC_prev][yC_prev] ! = COPY_ABOVE )

palette_mode[ xC ][ yC ]

…

if( indexMax > 0 && palette_mode[xC][yC] ! = ESCAPE ) {

palette_run

runPos = 0

runMode = palette_mode[ xC ][ yC ]

…

BoG Recommendation: Adopt the above syntax changes shown above in italics (From S0099/S0110/subset of S0173).

Decision: Adopt per BoG recommendation

In-meeting offline discussion (expected to be completed by Sunday) was requested on whether to recommend for adoption the additional parts of S0110 (affecting palette_transpose_flag). See additional notes below in section on S0110.

JCTVC-S0173 Non-CE6: Redundancy removal and simplification for Palette coding [Y. He, X. Xiu, Y. Ye (InterDigital)]

Notes from BoG:

This proposal proposes to remove some syntax signalling redundancy and simplify the binarization method of palette_run value for palette coding. Compared to CE6 anchors, the lossy coding with proposed technologies achieves {Y, U, V} BD rate gain of {-0.2%, -0.2%, -0.2%}, {-0.2%, -0.2%, -0.2%} and {-0.2%, -0.2%, -0.3%} for the category (RGB/YUV, text & graphics with motion, 1080p and 720p) for AI, RA and LDB, respectively. For lossless coding, it achieves bit rate saving of 0.1%, 0.1% and 0.1% for the category (RGB, text & graphics with motion, 1080p) for AI, RA and LDB, respectively.

Two modifications are proposed:

Modification1: If palette size is 0, i.e. one color, then the palette_escape_val_present_flag is not signalled and is inferred to be true. A second part of this modification proposes that palette_mode and palette_run not be signalled when there is only one color in the CU due to one of two possibilities: 1) there is only one color in the palette table and there is no escape color, or 2) the palette table is empty.

Note that the above modification is a superset of JCTVC-S0099 (the two checks for indexMax >0 are both used in this proposal and JCTVC-S0099).

Modification 2: Simplifies the binarization method for palette_run: Proposes combining the binarization of Segments 2 and 3 to both use a 2nd-order Exp-Golomb instead of SCM 2.0’s Golomb-Rice for Segment 2 and 0th-order Exp-Golomb for Segment 3. This method thus reduces the length of the binarization for the smaller run values.

JCTVC-R0174 Section 1.1 (description) and Section 2.1(simulation results) propose the exact same modification as Modification 2. The proponents of JCTVC-R0174 decided to propose a different one of their coding methods in CE6 for the current meeting, so this method was not yet formally tested in a CE. The method that the proponents of JCTVC-R0174 decided to use is currently marked as a revisit in the meeting notes.

Gains of modification 1alone are 0.0% to 0.1% for all-intra lossy conditions.

Gains of modification 2 alone are 0.0% to 0.3%

Combined gains: 0.1% to 0.3%

BoG Recommendation for Modification 2: Include in the further discussion of CE6 Category A (under JCTVC-S0026). For the first part of Modification 1 (adopt), see the recommendation for JCTVC-S0105. For the recommendation on the second part of Modification 1 (adopt) of this proposal and of JCTVC S0099, see the “Continued discussion of S0099, S0110 and S0173” section in the JCTVC-S0105 notes.

JCTVC-S0280 Crosscheck of JCTVC-S0105 [F. Zou (Qualcomm)] [late]

JCTVC-S0110 Non-CE6: Syntax Redundancy Fixes for Palette Mode [W. Pu, M. Karczewicz, V. Seregin, F. Zou, R. Joshi, J. Sole (Qualcomm)]

Notes from BoG:

Note: Continued discussion of this proposal and JCTVC-S0047, S0099, S0105, and S0173 are under JCTVC-S0105.

This document proposes three syntax redundancy fixes related to palette mode.

First, the palette mode adopted at the Sapporo meeting contains a CU level syntax element palette_escape_val_present_flag which specifies whether escape pixel may exist in the current CU or not. If escape pixel does not exist and the palette size equals to 1, it can be implicitly inferred that all of the pixels in the current CU share the same color as the one in the palette while in the current design, redundant syntax elements, such as palette_mode, palette_run, are still required to be coded into the bitstream. This document proposes to simplify the codec by removing the two syntax elements when such case happens.

Second, in the scenario when palette size is equal to zero, i.e. all of the pixels in the current CU are escape pixels, the pallet syntax elements except palette_escape_val can be bypassed. Alternatively, a normative restriction can be imposed to ensure that when palette mode is used, the palette size is always greater than zero.

Third, the palette mode contains an implicit restriction to the bitstream to improve coding efficiency. Specifically, an INDEX run mode will always select the longest possible run. In another words, an INDEX run mode cannot be followed by a pixel which has the same index. This document reports a redundancy associated with this restriction and proposes a method to fix it. It is reported that (0.3%, 0.3%, 0.3%) and (0.4%, 0.3%, 0.3%) BD-rate reduction can be achieved for AI RGB text & graphics with motion1080p and AI YUV text & graphics with motion 1080p.

- Method 1.1 (note: not section number): Don't allow palette_size == 0

- Method 1.2: When the palette_size == 0, don’t signal the remaining palette-related syntax such as palette_transpose_flag, etc.

- Method 2: When palette_size == 1 and palette_escape_val_present_flag == 0, then the whole CU is composed of one color, so some additional palette-related syntax need not be signalled.

- Method 3: an INDEX run mode will always select the longest possible run. Because a run mode cannot be followed by a pixel having the same index, some improvement in coding efficiency is possible.

The all-intra lossy gains for Methods 1.1, 1.2, and 2 have essentially no effect on BD-Rate.

For Method 3, all-intra lossy gains are 0.0% to 0.4%.

It was commented that if the palette isn’t allowed to be empty (Method 1), then a coding penalty may be incurred. The proponent said that by disallowing an empty palette, the types of checks that other proposals make to handle palette_size == 0 do not have to be performed. It was commented that if the coding efficiency of the escape color is improved, then having an all-escape block may not incur a significant coding penalty. Another proponent prefers allowing an all-escape block.

It was commented that Section 2.2 in JCTVC-S0047 also addresses the same aspects as Method 3.

BoG Recommendations:

- For Method 1.1, no action.

- For Method 1.2 (part of this was adopted as discussed elsewhere, the rest was requested for offline in-meeting discussion), see discussions “Continued discussion of S0099, S0110 and S0173” after JCTVC-S0105.

- For Method 2, similar recommendation as Method 1.2 (partial adoption would occur if the syntax shown in the continued discussions is adopted, and the rest would be discussed offline in-meeting). Discussion of the text was requested.

- For Method 3, further study in a CE with JCTVC-S0047.

In later review of the BoG results, the concerned parties were satistified with the state of the text for method 2.

Decision: Adopt change (as has been clearly identified in an uploaded revision of S0110).

JCTVC-S0217 Non-CE6: cross-check of JCTVC-S0110 on Syntax Redundancy Fixes for Palette Mode [Y. He, X. Xiu, Y. Ye (InterDigital)] [late]

JCTVC-S0108 Non-CE6: Improvement On Palette Sharing Mode [W. Pu, M. Karczewicz, R. Joshi, F. Zou, V. Seregin, J. Sole (Qualcomm)]

Notes from BoG:

The palette mode adopted at the Sapporo meeting contains a CU-level syntax element palette_share_flag which specifies the palette entry signaling methods. If this flag equals to 1, the current CU’s palette is derived from previousPaletteSize and previousPaletteEntries without additional signaling. In this contribution, it is proposed to extend the palette_share_flag to exploit the additional coding gain when all reconstructed pixels within the current CU share the same color. It is reported that the proposed method achieves (0.3%, 0.3%, 0.3%) and (0.3%, 0.2%, 0.2%) BD bit rate reduction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, under SCC common test conditions.

The semantics of the palette_share_flag is modified, and the signalling of the palette_transpose_flag is moved to after the palette_share_flag is signalled, so that a check on the value of the palette_share_flag can be done to determine whether to signal the palette_transpose_flag and subsequent syntax.

All-intra lossy gains: 0.0% to 0.3%; lossless: 0.0% to 0.2%.

A participant questioned the benefit of doing this.

It was commented by more than one participant that instead of modifying the palette_share_flag, one could introduce a new flag to indicate that all of the pixels have the same color as previousPaletteEntries[ cIdx ][ 0 ], thus keeping palette_share_flag unmodified. A participant said it would be of interest to study these modifications separately. It was commented that the CE8 test of using single color using the palette entry as a candidate would perform similarly as this method.

It was noted that this proposal introduces an extra R-D check for determining whether the modified palette_share_flag is 0 or not.

BoG Recommendation: Further study, not necessarily in a CE.

JCTVC-S0266 Non-CE6: cross-check of improvement on palette sharing mode (JCTVC-S0108) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0111 Non-CE6: Improvement On Palette Run Coding [W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin, J. Sole (Qualcomm)]

Notes from BoG:

The palette mode adopted at the Sapporo meeting uses coefficient coding method to code the palette_run syntax element, as in SCC draft 1. CE6 Test A.4, A.5, and A.6 were established to study alternative CABAC context design and binarization methods. In this contribution, an improvement on palette_run binarization is proposed, which reportedly achieves 1.7% and 2.1% BD-rate redunction for AI RGB text & graphics with motion 1080p and AI YUV text & graphics with motion 1080p, respectively, compared with SCM2.0 anchor.

Proposal: Allocate a shorter codeword for the ‘run-to-the-end’ case.

- Method A: Treats palette_run = = 3 as a special case, to represent ‘run-to-the-end’ (change in semantics (on top of SCM 2.0 and on top of CE6 Test A.6 configurations)

- Method B: If the prefix is truncated, then insert one flag to indicate ‘run-to-the-end’ (change in binarization). Note that this method doesn’t apply to SCM 2.0, it’s on top of CE6 Test A.6 configurations.

Gains for Method A over SCM 2.0: lossy all-intra: 0.1% to 0.6%

Gains for Method B over CE6 Test A.6 configurations is similar.

The method in JCTVC-S0064 inserts a flag after the palette_run > 2 bin for signalling if the current run is the last run for the processed CU.

The proponent of JCTVC-S0064 supports Method A as well.

BoG Recommendation: Adopt Method A, on the condition that in-meeting simulation runs performed with Method A on top of JCTVC-S0269 yield results on the order of those shown for Method B here on top of CE6 Test A.6 configuration 2, compared to an anchor of SCM 2.0 (lossy all-intra 0.3% to 2.1%). For completeness, the proponents were also asked to implement Methods A and B on top of S0269 and run simulations to compare them to an anchor of S0269 as well. A cross-check was also asked to be performed and text was asked to be provided.

Later, in JCT-VC, results for methods A and B relative to S0269 were reported to be available in a new revision of S0111.

Some gain was shown: 0.0-0.3% for AI. The proponent suggested to focus on method B. The text impact was requested to be reviewed.

Given the small gain relative to S0269 and the apparent complication of the text that would be introduced by it, no action was taken on this.

JCTVC-S0228 Crosscheck of Non-CE6 on Improvement On Palette Run Coding (JCTVC-S0111) [C.-C. Lin (ITRI)] [late]

JCTVC-S0115 Clarifying decoder’s ambiguous behaviour for escape index in palette mode [J. Kim, S. Liu, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

Notes from BoG:

This contribution proposes two approaches to clarify decoder’s behaviour for escape index handling in palette mode. The horizontal traverse scan is assumed for describing the approaches as follows. One approach is preventing each escape index from being a reference pixel of ‘copy above’ mode by limiting the number of runs of ‘copy above’ mode at decoder side as it works at encoder side. It also derives prediction mode of a pixel below escape index as ‘index mode’ without signalling the mode. The other approach is allowing each escape index being a reference pixel of ‘copy above’ mode by copying the escape values of the above escape pixel. The two approaches for vertical traverse scan can be easily understood by analogy, and both the approaches can solve the ambiguity issue successfully with very little impacts on coding efficiency.

Method 1: Prevent ESCAPE from being included as a reference index

A) Pixel below ESCAPE is inferred to be INDEX mode; and

B) The run of COPY_ABOVE prediction mode will not include the pixel below the ESCAPE index. (decoder syntax)

Lossy all-intra gains: 0.0% to 0.1%; RA -0.1% to 0.1%; LB -0.4% to 0.2%; Lossless gains 0.0% to 0.2% for all cases.

Method 2: Allow an escape index to be a reference index

Copies the escape value of the three components for the current pixel

Lossy all-intra gains: 0.0%; RA 0.0% to 0.1%; LB -0.1% to 0.25; lossless 0.0% for all cases.

See additional notes under JCTVC-S0258.

JCTVC-S0270 CE6-related: Cross check of JCTVC-S0115 Test 2.1 [J. Zhao, S. H. Kim (Sharp)] [late]

JCTVC-S0279 Crosscheck of JCTVC-S0115 method 2 [F. Zou (Qualcomm)] [late]

Has "the" problem.

JCTVC-S0150 Non-CE6: Simplification on Escape Coding of Palette Mode in HEVC SCC [M. Xu, Z. Ma, W. Wang, H. Yu (Huawei USA R&D)]

Notes from BoG:

This proposal is identical to [JCTVC-S0181 without encoder improvements]. See the discussion under JCTVC-S0181 and the combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258 under JCTVC-S0258.

See additional notes under JCTVC-S0258.

JCTVC-S0232 Crosscheck of JCTVC-S0150: Non-CE6: Simplification on Escape Coding of Palette Mode in HEVC SCC [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)] [late]

JCTVC-S0156 Non-CE6: Palette parsing dependency and palette encoder improvement [W. Pu, F. Zou, V. Seregin, R. Joshi, M. Karczewicz, J. Sole (Qualcomm)]

Notes from BoG:

This contribution discusses a parsing dependency issue for the palette prediction mode, and suggests encoder improvements which reportedly provide 0.1%, 0.2% and 0.3%, 0.5% luma BD-rate reduction for All Intra 1080p text and graphics RGB and YUV categories in the lossy coding configuration for two encoder improvements, respectively.

- Method 1: Currently, if (copyRun + 2 > indexRun), choose COPY_ABOVE run mode. This document proposes: if (copyRun > indexRun), choose COPY_AbOVE run mode. All-intra lossy gains for this method are 0.0% to 0.2%.

- Method 2: The encoder is modified to compare the cost of copy left and copy above modes, and it chooses the mode with the lower cost. Lossy all-intra gains: 0.0% to 0.5%

It was also noted that in the Sapporo meeting, it was asserted that during parsing, the palette index has to be fully reconstructed to determine ESCAPE/INDEX copy modes. Now, it is asserted that the escape pixel’s index is always binarized to all ones '111…1', therefore there is no parsing dependency issue, i.e. the parsing is required, but the reconstruction of the palette index is not required to determine ESCAPE/INDEX copy modes.

Methods 1 and 2 are mutually exclusive.

CE6 Category C, there’s a revisit of Tests C.2+C.3, and the related JCTVC-S0188 includes Method 2 shown here. JCTVC-S0188 is being considered for adoption pending a cross-check.

BoG Recommendation (SW): Include the changes of Method 2 in the encoder software.

Decision (SW): Adopt encoder-only change as recommended.

Note that elsewhere is recorded a discussion of JCTVC-S0188 under CE6 Test C.2+C.3, and the encoder improvement in S0188 is identical to Method 2 here.

See additional notes under JCTVC-S0258.

JCTVC-S0297 Non-CE6: cross-check of palette parsing dependency and palette encoder improvement (JCTVC-S0156) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0157 Non-CE6: Copy mode and escape coded sample [V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole (Qualcomm)]

Notes from BoG:

This contribution addresses the problem of having escape coded samples in the above row and samples coded with the 'copy from above' mode. In the current software, such cases are not chosen by the encoder.  But from the decoder side, the run value signaled for the 'copy from above' mode may include escape coded samples. However the 'copy from above' mode is not defined for the escape samples.

- Method 1: Restrict encoder from generating escape reference sample in copy above mode

- Method 2: Restrict the copy above signaling if the above sample is escape:

  - Pixel below ESCAPE is inferred to be INDEX mode; and (Same as in S0115)

  - The encoder is constrained so that the ESCAPE pixel is not allowed to be included into the COPY_ABOVE mode.

- Method 3: Include escape into the copy above mode (This is mutually exclusive to Method 2)

- Method 4: Replace the escape sample with palette index 0 for copy above mode

Lossy all-intra gains for Methods 2-3 are within 0.0% to 0.1%.

See the combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258 under JCTVC-S0258.

JCTVC-S0259 CE6-related: Crosscheck report of JCTVC-S0157 [J. Kim, S. Liu (??)] [late]

JCTVC-S0181 Non-CE6: Removal of parsing dependency in palette-based coding [X. Xiu, Y. He, Y. Ye (InterDigital)]

Notes from BoG:

This contribution proposes to modify the design of palette-based coding in order to remove the parsing dependency of various syntax elements for CUs coded with palette mode. Two modifications are introduced to achieve this goal. Firstly, it is proposed to treat the palette index used to indicate escape colors as normal palette index and them using either run mode or copy-above mode. Secondly, it is proposed to separate the parsing of escape color values from the parsing of palette indices.

This document proposes:

- Code the palette index indicating escape colors using run mode and copy above mode;

- Separate the parsing of escape color triplets from the parsing of palette indices.

- The encoder used to generate results in this document also includes modifications to the decision between index and run modes, similar as in JCTVC-S0156.

Gains, including the encoder improvements mentioned above: Lossy All-intra gains: 0.1% to 0.5%; RA: 0.1% to 0.2%; 0.1% to 0.4%; Lossless: 0.1% to 0.95; RA 0.0% to 0.5%; 0.0% to 0.5%.

The gains without encoder improvements are identical to the gains reported in JCTVC-S0258. These gains are: Lossy all-intra and RA: -0.1% to 0.1%; LB: 0.0% to 0.3%. Lossless: 0.1% to 0.7%; RA 0.05 to 0.5%; LB 0.0% to 0.4%.

It was noted that JCTVC-S0150 is identical to [JCTVC-S0181 without encoder improvements]

It was noted that to implement the second part of this proposal (separate the parsing of escape color triplets from the parsing of palette indices) in the decoder, there can be several ways to accomplish this: Storing an array up to size 64x64 of a binary indication of whether the position is ESCAPE, or, re-scan all pixel positions in the CU to parse and fill in the escape color values, or, if the architecture chooses to scan twice, then one could merge the escape value decoding with the index-to-pixel value conversion process.

See additional notes under JCTVC-S0258.

JCTVC-S0256 Non-CE6: Cross-check of removal of parsing dependency in palette-based coding (JCTVC-S0181) [V. Seregin (Qualcomm)] [late]

JCTVC-S0258 Non-CE6: Unification of coding of escape indices and other palette indices [X. Xiu, Y. He, Y. Ye (InterDigital), V. Seregin, R. Joshi, M. Karczewicz, W. Pu, J. Sole (Qualcomm)] [late]

Notes from BoG:

In SCM2.0, escape coded samples are assigned the highest palette index (after expanding the palette index by 1). But in coding of palette indices, the escape index is treated differently from other palette indices. This contribution proposes a method that is asserted to unify the coding of escape index and other palette indices. The proposed method uses elements from JCTVC-S0157 (solution 3) and JCTVC-S0181. BD-rate results for the proposed method with and without encoder modification are presented. Without encoder modification, for the category of text and graphics with motion 1080p, the BD-rate saving for the Y/G component is 0.1% for the All-Intra lossy configuration, and the bit-rate savings for the Y/G component 0.6% for the All-intra lossless configuration. With encoder modification, the corresponding BD-rate savings and bit-rate savings are 0.4% and 0.8% for lossy and lossless configurations, respectively.

This proposal comprises the common parts of JCTVC-S0157 and JCTVC-S0181:

- Escape index is enabled for INDEX mode with a run value to specify the number of subsequent indices that are also escape colors

- Escape index allowed to be part of Copy mode run.

Without encoder improvements, gains for all-intra lossy: -0.1 to 0.1%; lossless: 0.1% to 0.7%.

The difference between JCTVC-S0258 and JCTVC-S0181 is that in JCTVC-S0258, escape color values are coded in the same interleaved way as in SCM 2.0, whereas in JCTVC-S0181 the coding of escape color values is moved outside of the coding loop for escape indices.

Combined discussion of JCTVC-S0115/S0150/S0156/S0157/S0181/S0258

Notes from BoG:

Condensed summary of these proposals:

JCTVC-S0115:

- Method 1: Prevent ESCAPE from being included as a reference index:

A) Pixel below ESCAPE is inferred to be INDEX mode; and

B) The run of COPY_ABOVE prediction mode will not include the pixel below the ESCAPE index. (decoder syntax)

- Method 2: Allow an escape index to be a reference index

BoG Recommendations for both Methods 1 and 2: Given the other recommendations, this is not needed.

JCTVC-S0150 and JCTVC-S0181 and JCTVC-S0258:

- All three include this: Code the palette index indicating escape colors using run mode and copy above mode;

BoG Recommendation for this method for coding the palette index: adopt JCTVC-S0258/S0150 subset/S0181 subset/S0157 Method 3.

Decision: Adopt this aspect as recommended by BoG.

- S0150 and S0181 additionally include: Separate the parsing of escape color triplets from the parsing of palette indices. One non-proponent said that they don’t see any benefit to doing this and therefore this change to the spec should not be made; they see added cost, asserting that one would have perform an additional scan of all pixel positions in the CU to parse and fill in the escape color values. Several proponents and a non-proponent disagreed, saying that there are other ways to implement it that doesn’t require an additional scan (e.g. see notes near end of JCTVC-S0181 discussion). A proponent said that the current spec’s inverleaved way of decoding requires (if done in one loop) that the dequantization occurs inside the loop for decoding escape colors (in contrast to the decoding process of the regular color indices). A non-proponent and proponent said that with the current specs, another implementation is possible: one can first parse indices and escape colors, and then invoke the mapping of indices to colors and perform inverse dequantization of escape colors. The non-proponent asserts that adoption of this method would always require a separate pass for parsing the escape values. The proponents said that this proposed solution is consistent with the HEVC v.1 for residual coding where the signaling of the significance map is separated from that of the individual residual values. A non-proponent disagreed, saying that in palette mode decoding, for the palette index map there are more syntax elements which have to be parsed and decoded first, in contrast to the significance map coding in HEVC v.1 residual coding. The proponent asserts that with this proposal, the current encoding process can be made to be more efficient; a non-proponent disagrees.

Two proponents proposed this, one non-proponent opposed, and one non-proponent supported this.

BoG Recommendation: Make final decision in JCT-VC session, and offline discussion is encouraged before this final decision.

In JCT-VC further discussion, some participants asserted that sending the index map information before sending the escape-coded values was consistent with the existing HEVC transform coefficient coding design. Another participant said that especially with large blocks, having the decoder need to wait until the entire map is sent prior to getting the values could increase latency in the decoding process, and noted that HEVC transform coefficient coding has a 4x4 grouping and asserted that this mitigates this latency, which is not used here.

It was agreed to study the question of interleaved-versus-non-interleaved escape value coding in an AHG study.

JCTVC-S0156: No further discussion needed, recommendation reached.

JCTVC-S0157:

- Method 1: Restrict encoder from generating escape reference sample in copy above mode:

BoG Recommendation: Given the other recommendations, this is not needed.

- Method 2: Restrict the copy above signaling if the above sample is escape:

- Pixel below ESCAPE is inferred to be INDEX mode; and (Same as in S0115)

- The encoder is constrained so that the ESCAPE pixel is not allowed to be included into the COPY_ABOVE mode.

BoG Recommendation for Method 2: Given the other recommendations, this is not needed.

- Method 3: Include escape into the copy above mode (this is also a subset of S0150/S0181/S0258) (see earlier recommendation for its adoption)

- Method 4: Replace the escape sample with palette index 0 for copy above mode. Recommendation: Given the other recommendations, this is not needed.

Inconsistency: It was noted that in the SCC text specifications JCTVC-R1005-v3, the definition of palette_mode[xC][yC] in Section 7.4.9.6 uses the term “sample value”, whereas the syntax in Section 7.3.8.8 (which will eventually be moved to the decoder process description as an editorial change) uses the indices paletteMap[xC][yC], not values, in COPY_ABOVE mode. The earlier implementations of palette mode (e.g. earlier versions of SCM, JCTVC-R0348, all CE’s conducted on palette, etc.) copy indices, not pixel values.

BoG Recommendation: Edit the semantics for palette_mode[xC][yC] to clarify that COPY_ABOVE mode copies the indices paletteMap[xC][yC], not sample values.

Decision (Ed.): Agreed per BoG recommendation.

Method 2 of S0157 is the same as part of Method 1 of S0115 (restrict the copy above signaling if the above sample is escape). S0157 uses an encoder constraint, whereas S0115 uses decoder syntax so that the pixel below the ESCAPE index is not included in the COPY_ABOVE run mode.

The method of coding the palette index indicating escape colors using run mode and copy above mode is common to JCTVC-S0150/S0181 and the joint S0258 proposals.

JCTVC-S0277 Non-CE6: Crosscheck of JCTVC-S0258 on unification of coding of escape indices and other palette indices [P. Onno (Canon)] [late]

JCTVC-S0120 Non-CE6: Copy previous mode [J. Ye, J. Kim, S. Liu, P. Lai, S. Lei (MediaTek)]

Notes from BoG:

This proposal presents a “copy previous” mode with the use of neighbour indices for palette coding. The proposed “copy previous” mode is signalled by the “copy above” mode flag in current SCM, followed by an offset which specifies the distance between the previous row and the current row. It is proposed that the “copy previous” mode starts from the first row in the CU. The simulation results report that the proposed method achieves 3.9%, 1.9% and 1.5% BR-rate savings for “YUV 1080p text and graphics” AI, RA and LB configurations, respectively against the SCM 2.0 anchor.

This is similar in concept to JCTVC-S0079 (same proponents) plus CE6 Test C.3 (copy previous row), except that in JCTVC-S0120 two rows are available for copying instead of one.

All-intra lossy gains: 0.7% to 3.9%; lossless: 0.1% to 2.0%

(Gains for JCTVC-S0079 were 0.3% to 2.5% lossy)

There was a question on if another LCU is visited when going two rows above, whether an additional line buffer would be needed. Answer: Currently for deblocking, a 2-line chroma buffer (4-line luma) is already available, so no additional buffers would be needed for this method.

The concerns addressed for JCTVC-S0079 apply to here as well.

BoG Recommendation: Further study in a CE. When defining the CE, keep in mind the outcome of the CE6 Test C.3 decision from this meeting.

JCTVC-S0253 Non-CE6: Crosscheck for Copy Previous Mode (JCTVC-S0120) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-S0134 Non-CE6: Simplified palette size coding [J. Zhao, S. H. Kim, K. Misra, A. Segall (Sharp)]

Notes from BoG:

This contribution proposes to change the binarization of “palette_num_signalled_entries”.  When the palette_num_signalled_entries is less than 16, there is no change to the binarization; it still uses the existing TR code. When palette_num_signalled_entries is equal to or greater than 16, the proposed binarization uses a concatenated TR binarization of a number 16 and a fixed length code. The benefit of the proposed method is that it reduces the worst case coded bins from 31 to 20 bins, which is 35.5% reduction in the number of bins. It has no effect on coding loss for almost all lossy and lossless coding tests.

| palette_coding( ) | previous_palette_entry_flag[] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | palette_num_signalled_entries | Section a.b.c.d. |  |

a.b.c.d	Binarization process for palette_num_signalled_entries

Inputs to this process are a request for a binarization for the syntax element palette_num_signalled_entries.

Output of this process is the binarization of the syntax element.

Depending on the value of palette_num_signalled_entries, the following applies:

- If palette_num_signalled_entries <16

  - Binarize the palette_num_signalled_entries using using TR binarization with cMax=16 and cRiceParam=0

- else

  - Binarize the value 16 using using TR binarization with cMax=16 and cRiceParam=0

  - Binarize the value of palette_num_signalled_entries-16 using 4-bit FLC

There was almost no effect on coding performance, so the proponent’s stated purpose for doing this is simplification.

Two non-proponents said that this method may reduce the number of bins, but may make the binarization more complex (the existing method is truncated unary coding).

BoG Recommendation: No action

JCTVC-S0213 CE6-related: Crosscheck report of JCTVC-S0134 [J. Kim, S. Liu (MediaTek)] [late]

JCTVC-S0138 Non-CE6: Exponential Golomb binarization for Palette Run [S. H. Kim, K. Misra, J. Zhao, A. Segall (Sharp)]

Notes from BoG:

This contribution proposes a concatenated binarization based on fixed length code and exponential golomb code for the syntax element ‘palette_run.’ Specifically, greater_than_2_flag was replaced by greater_than_5_flag. Here, if greater_than_5_flag is equal to 0 then fixed length code with length of 2 is used to signal the palette_run information between 2 and 5. If greater_than_5_flag is equal to 1, then exponential-golomb binarization is employed to code ‘palette_run-6’. It is reported that the proposed approach provides bit rate reductions of 0.3 %, 0.4% for All Intra 1080p text and graphics RGB and YUV categories, respectively.

Lossy all-intra gains: 0.0% to 0.4%

This proposes to replace the "greater than two" condition with a "greater than five" condition, which means that if palette_run is equal to 2, 3, 4, or 5, then fixed-length coding is used to signal it.

Code significant flag (context coded)

if( significant flag = =1 )

Code GR_1 flag (context coded)

if ( GR_1 flag ==1 )

Code GR_5 flag (context coded)

if( GR_5 flag ==0)

Code Fixed Length Code (‘Run-2’)

else Code Exp-Golomb (‘Run-6’)

The proponent recommends further study in a CE related to what may be adopted after the revisit of CE6 Tests A4, A5, A6 (JCTVC-S0269).

BoG Recommendation: Further study, perhaps in a CE.

The relationship to the action from JCTVC-S0269 would need to be considered.

JCTVC-S0229 Crosscheck of Non-CE6 on Exponential Golomb binarization for Palette Run (JCTVC-S0138) [C.-C. Lin (ITRI)] [late]

JCTVC-S0139 Using flat scaling lists for escape coded palette pixels [K. Misra, S. H. Kim, J. Zhao, A. Segall (Sharp)]

Notes from BoG:

This contribution proposes that the existing HEVC dequantizer along with flat scaling lists be used for palette pixels coded using escape mode. This approach was first proposed in JCTVC R0234. The contribution demonstrates that the use of the default scaling list can lead to visual artefacts for palette coded blocks.

Artefacts may arise when non-flat scaling lists are used. An experiment demonstrates that when SCM 2.0 is used with the default (non-flat) scaling lists, artefacts are visible.

This proposes that the dequantizer is restricted to using flat scaling lists when processing palette pixels.

There was a question on whether the artefact pixels are in a different CU from the object pixels.

There was a question on whether we’re sure the blocks with artifacts were coded using palette mode or a different mode.

Examination of the JCTVC-R1005 text shows that the dequantization of the palette-coded blocks do not use any scaling list.

We do not know at the moment what QP value was used.

BoG Recommendation: The artefacts are most likely caused by something else, given that palette mode doesn’t use a scaling list.

The proponent later indicated that the problem was at least partly an implementation bug rather than a design issue. Editorial refinement of the dequantization parts of the text seems desirable regarding the structure of the text to avoid excess duplication and ensure clarity and proper structure.

JCTVC-S0151 Non-CE6: 2-D Index Map Coding of Palette Mode in HEVC SCC [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)]

Notes from BoG:

This contribution presents a 2-D index map coding method designed for improving the performance of the palette mode in HEVC SCC WD and SCM 2.0. The proposed method is available as an additional method for coding palette indices. A flag associated with each run indicates which method to use (1-D or 2-D). The 1-D method is a modified version of the current method for coding palette indices. Its performance reported in this contribution was evaluated under common test conditions with various search range configurations. Experimental results demonstrate BD-Rate gain of 3.3% (IBC-1x4/2D-1x4), 20.2% (IBC-FF/2D-1x4), 14.8% (IBC-2x4/2D-2x4) and 18.4% (IBC-3x5/2D-3x5), over Anchor SCM-2.0+IBC-1x4, for lossy 1080p text graphics content encoded using All intra configuration, respectively.

Proposes using a 2-D string copy method to code the index map, in addition to the RUN-based 1D string match. This proposal also includes the JCTVC-S0150 method for coding the escape pixels.

If 2D_flag = 1, COPY_ABOVE flag is skipped, otherwise, COPY_ABOVE flag is coded using only one context (2 contexts are used in the default RUN-based 1 search)

Gains for 2D strings search range of 1x4: 1080p Text & Graphics all-intra about 2.1% for RGB, 1.7% YUV; 1.4% RGB 1.2% YUV for low delay. Encoder run-time ratio for all-intra: 129%, decoder 106%; LB: encoder 103%, decoder 103%. It was commented in that JCTVC-S0114, the gains of extending copy-above to one line above, higher gains were achieved.

For the case when the IBC search range is 2x4 and the 2D palette coding search range is 2x4, the time ratios are encoder 147% decoder 104% for all intra; encoder 106% decoder 103% for LB, and in this case, the BD-Rate gain as compared to an anchor using full-frame IBC is 0.6% for T&G 1080p RGB, and 0.8% for YUV.

The proponent said that their run times may not be accurate.

A crosscheck had not yet been registered when this was discussed in the BoG (later, one was provided, as noted below, in S0296).

The proponent clarified that the purpose of this proposal is that one can undo the losses incurred from decreasing the IBC search range by using this 2-D index map coding on palette mode.

A non-proponent recommended that this be further studied in a CE, as there are many questions about what the method is doing and what the relevant tested and anchor conditions are. Another non-proponent was concerned about the encoder run-time ratio increase given the gains. After the cross-check is uploaded, perhaps more accurate run-time ratios will be made available.

BoG Recommendation: Further testing in a CE, with sufficient attention given to defining the appropriate test conditions in the CE description (e.g. IBC search ranges and 2-D search range for palette coding, etc.).

JCTVC-S0296 Non-CE6: Cross-check of 2-D Index Map Coding of Palette Mode in HEVC SCC (JCTVC-S0151) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0152 Non-CE6: Index Coding Group (ICG) for 8x8 CU of Palette Mode in HEVC SCC [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)]

Notes from BoG:

This contribution presents a parallel processing method for index map coding using an index coding group. This method suggests that to improve the overall system performance, coding of color index map may be done on smaller subgroups, instead of on the original color index map. It is suggested that this method might be used for 8x8 CUs, given that 8x8 CUs are a bottleneck for the overall throughput. Experimental results demonstrate performance losses of approximately 0.2% AI/0.1% RA/0.0% LB lossy BD-Rate loss over all content), but with potential 4x throughput improvement for 8x8 index map processing. Small gains are observed for lossless coding scenarios (i.e., 0.5% bit rate reduction for 1080p text/graphics content in RGB format).

This proposes to split an 8x8 index map into four 2x8 groups suitable for parallel processing. An additional flag is proposed for each group, which indicates whether the RUN equals the full size of the current group. COPY_ABOVE is maintained across groups.

Some participants suggested that it may be of interest to see if one could perform this kind of parallelization in the encoder, without any normative changes.

BoG Recommendation: Further testing.

JCTVC-S0104 CE6-related: Cross-check of JCTVC-R0152 on Index Coding Group (ICG) for 8x8 CU of Palette Mode [P. Lai, J. Kim (MediaTek)] [late]

JCTVC-S0287 Crosscheck report of JCTVC-S0152 [K. Chono (NEC)] [late]

Note: Only a "software study" was reported here – not experiment results. It was asked whether we have other "cross-checks" like this. None were identified.

JCTVC-S0178 Non-CE6: Improved binarization and signalling of index coding for transition copy mode [M. Karczewicz, F. Zou, R. Joshi, V. Seregin (Qualcomm)]

Notes from BoG:

A CU level flag use_trans_flag is proposed to indicate the use of transition copy mode for palette coding. When use_trans_flag is 1, for index coding, an equal_to_transition flag is coded in bypass mode to indicate whether the index is equal to the transition index. The simulation results reportedly show that the proposed method achieves 0.2% and 0.3% BD-rate savings for 720p text and graphics RGB and YUV, respectively, when compared against CE6 Test C.2 for All-Intra lossy configuration. The worst-case context coded bins complexity is asserted to be better than CE6 test C.2.

Two flags on top of CE6 Test C.2 are proposed: use_trans_flag and equal_to_transition flag.

Lossy all-intra gains 0.0% to 1.2%. For a few classes of test sequences, these gains were 0.0% to 0.3% better than the gains achieved by CE6 Test C.2.

It was asked if the gains achieved by these two flags were tested separately (no).

It was commented that in the CE6 Test C.2+C.3 (JCTVC-S0188) test, a similar binarization as here was used.

The proponent asserts that using the equal_to_transition flag would be a complexity reduction (one less context-coded bin) than Test C.2+C.3.

BoG Recommendation: Make final decision during further discussion of JCTVC-S0188 in the JCT-VC session.

No action due to lack of adoption of transition copy mode.

JCTVC-S0275 Non-CE6: Cross-check of JCTVC-S0178 on improved binarization and signaling of index coding for transition copy mode [P. Onno (Canon)] [late]

JCTVC-S0200 Cross-check Non-CE6: Redundancy removal and simplification for palette coding [W. Pu (Qualcomm)] [late]

JCTVC-S0186 Non-CE6: on context modeling of palette_transpose_flag [X. Xiu, Y. He, Y. Ye (InterDigital)]

Notes from BoG:

This contribution proposes to simplify the context modeling of the syntax element palette_transpose_flag for palette-based coding by only using one context model for the CABAC entropy coding. Experimental results reportedly show that the proposed change does not introduce any coding performance loss for both lossy coding and lossless coding (0.4% BD-rate saving is observed for the category of RGB mixed content 1080p), while reducing the total number of context models used in palette mode by one.

The cross-checker said using two contexts should have given more gain over one context, so we don't have enough information to know why this is not the case.

BoG Recommendation: Further study.

Decision (cleanup): Adopt.

JCTVC-S0276 Non-CE6: Cross-check of JCVC-S0186 on context modeling of palette_transpose_flag [P. Onno (Canon)] [late]

JCTVC-S0188 Non-CE6: A combination of CE6 Test C.2 – transition-copy mode and CE6 Test C.3 (configuration 1) – copy-from-previous-row mode [Y.-C. Sun, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

Only rudimentary results were provided before the deadline, so the initial upload was rejected as a placeholder.

See notes for CE6 (section 4.6.1).

JCTVC-S0288 Crosscheck of JCTVC-S0188: Non-CE6: A combination of CE6 Test C.2 – transition-copy mode and CE6 Test C.3 (configuration 1) – copy-from-previous-row mode [R.-L. Liao, C.-C. Chen, W.-H. Peng, H.-M. Hang (NCTU/ITRI)] [late]

JCTVC-S0295 Cross-check of JCTVC-S0188 [D. B. Sansli, J. Lainema (Nokia)] [late]

JCTVC-S0269 CE6-related: Harmonization of CE6 Tests A4, A5, and A6 [S.-T. Hsiang, T.-D. Chuang, S. Lei (MediaTek), R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin, J. Sole (Qualcomm)] [late]

See notes for CE6 (section 4.6.1).

JCTVC-S0289 Cross check of Harmonization of CE6 Tests A4, A5, and A6 (JCTVC-S0269) [O. Nakagami (Sony)] [late]

JCTVC-S0201 Non-CE5: CU dependent color palette maximum size [W. Wang, Z. Ma, M. Xu, H. Yu (Huawei USA R&D)] [late]

Notes from BoG:

Considered in CE6 BoG.

Presenters were not available; however, the contribution was reviewed by the BoG.

This contribution recommends a CU dependent color palette maximum size to balance the trade-off between coding efficiency and complexity throughput. Simulation results demonstrate averaged 0.5%/0.3%/0.1% lossy BD-Rate loss, and averaged -0.8%/-0.2%/-0.1% lossless bit rate increase for AI/RA/LB encoder configurations, respectively. It provides gain compared with the scenarios that max. palette size and palette predictor size fixed at respectively 15 and 32, but it keeps the same throughput with small CU sizes.

Experimental results were shown for the proposed method using a max palette predictor size of 32 for all block sizes, and a max palette size = 15 for 8x8 and 16x16, and 31 for 32x32 and 64x64. Results were compared to both the SCM 2.0 anchor (max palette size = 31 and max predictor size = 64), and to an anchor using max palette size = 15 and max predictor size = 32. Compared to SCM 2.0, losses of up to 1.2% were observed. Compared to the other anchor tested, gains were up to 0.5%.

It was asked whether this is an encoder or decoder complexity reduction. The cross-checker said that he thinks they are targeting the encoder. It was commented that during the CE5 review in JCT-VC Track A discussions, one of the decisions under CE5 (JCTVC-S0025) was to send the maximum palette size and maximum palette predictor size at the SPS level. It was also commented that the encoder can choose not to construct a palette up to max palette size.

BoG Recommendation: Given the other decision and some questions about test conditions, no action was recommended.

JCTVC-S0106 CE5-related: Cross-check of JCTVC-S0201 on CU dependent color palette maximum size [P. Lai, J. Kim (MediaTek)] [late]

### CE7 related (string matching for palette index coding) (0)

(Chaired by XXX on YYYday 10-ZZ x.m.)

### CE8 related (single-colour and two-colour modes) (4)

See notes for CE8. No action.

JCTVC-S0050 Non-CE8: Combination of CE8 Test A and CE8 Test B for screen content coding [Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu, C.-H. Hung (ITRI)]

JCTVC-S0208 CE8-related: Crosscheck for combination of CE8 Test A and CE8 Test B for screen content coding (JCTVC-S0050) [Y.-W. Chen (MediaTek)] [late]

### CE9 related (intra boundary filtering and cross-component prediction Interdependency) (2)

(Chaired by JRO on Monday 10-20 p.m.)

JCTVC-S0046 Non-CE9: Intra-boundary filter control for non-camera captured content [C. Rosewarne, M. Maeda (Canon)]

This document proposes a method for controlling the intra-boundary filter. The method makes use of the prediction modes of neighbouring blocks to control the filter. It is reported that the proposed methods achieve average -1.6%, -1.0%, -1.0% bit-rate saving and -1.3%, -0.9%, -0.9%  bit-rate saving for RGB, Mixed Content, AI lossy coding for 1440p and 1080p classes respectively. For RA case, the corresponding result in 1440p class is -1.3%, -0.8%, -0.8% and for LB case, the corresponding result in 1440p class is -1.0%, -0.6% and -0.4%.

IBF is enabled dependent on prediction modes of neighboring blocks. IBF is operated over all three prediction modes. This goes beyond the proposals of the CE, as it is not only about global control, but introduces some new mode dependent (of neighboring blocks) control mechanism.

The gain is slightly higher (0.2-0.3%), not consistently over classes than for CE9 test A.2 (always enabling IBF “as is”). This does not justify the change and additional dependency.

No action.

JCTVC-S0298 Cross-check report of JCTVC-S0046 Intra-boundary filter control for non-camera captured content [K. Rapaka (Qualcomm)] [late]

JCTVC-S0199 Non-CE9: On chroma boundary filtering [F. Zou, R. Joshi, W. Pu, V. Seregin, M. Karczewicz (Qualcomm)] [late]

This proposal presents a chroma boundary filter design based on DM mode for 4:4:4 chroma format. The simulation results report that the proposed method achieves 1.5% and 1.3% BD-rate savings for mixed content RGB 1440p and 1080P respectively against the SCM2.0 anchor in the full frame intra BC test condition.

Gain only slightly higher than CE methods, but would imply changes to IBF at the block level. This does not justify the change and additional dependency.

No action.

JCTVC-S0293 Non-CE9: cross-check of chroma boundary filtering (JCTVC-S0199) [B. Li, J. Xu (Microsoft)] [late]

### CE10 related (intra string copy) (1)

(Chaired by JRO on Monday 10-20 p.m.)

JCTVC-S0250 Non-CE10: Improvement on coding of ISC parameters and comparison to Palette Coding [K. Zhou, L. Zhao, X. Chen, T. Lin (Tongji)] [late]

This contribution proposes an ISC scheme which applies and extends a few schemes used in Palette coding to code three ISC parameters, i.e. pos, len, no_reference_pixel (NRP, previously called unmatched pixel or unmatchable pixel). Using SCM20FF as anchor, SCM20 integrated with the proposed ISC (1D version) is tested and the lossless and lossy results are summarized in the following table:

| SCM20IBC_FF vs | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB, lossless case | Total | Avg | Min | Max | Total | Avg | Min | Max | Total | Avg | Min | Max |
| RGB, TGM, 1080p | 12.3% | 12.8% | 9.4% | 14.8% | 7.0% | 8.0% | 6.7% | 8.9% | 6.8% | 6.5% | 4.8% | 8.2% |
| RGB,TGM, 720p | 4.3% | 3.1% | 0.7% | 8.5% | 1.1% | 0.9% | 0.3% | 1.3% | 0.8% | 0.7% | 0.2% | 1.0% |
| RGB, MC, 1440p | 1.3% | 1.5% | 0.6% | 2.4% | 0.2% | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, MC, 1080p | 1.8% | 1.8% | 1.8% | 1.8% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, TGM, 1080p | 12.8% | 13.3% | 8.1% | 17.8% | 6.5% | 8.7% | 5.5% | 11.9% | 6.2% | 7.2% | 5.2% | 11.2% |
| YUV, TGM, 720p | 4.0% | 3.0% | 0.6% | 7.5% | 0.6% | 0.8% | 0.2% | 1.6% | 0.4% | 0.6% | 0.2% | 1.1% |
| YUV, MC, 1440p | 1.3% | 1.4% | 0.4% | 2.5% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, MC, 1080p | 1.7% | 1.7% | 1.7% | 1.7% | 0.3% | 0.3% | 0.3% | 0.3% | 0.1% | 0.1% | 0.1% | 0.1% |

| SCM20IBC_FF vs | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ISC 1D 1MB, lossy case | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, TGM, 1080p | -1.5% | -1.7% | -1.7% | -0.9% | -1.2% | -1.1% | -0.6% | -1.0% | -0.9% |
| RGB,TGM, 720p | -1.1% | -1.1% | -1.2% | -1.1% | -1.2% | -1.3% | -0.7% | -0.5% | -0.7% |
| RGB, MC, 1440p | -0.1% | -0.2% | -0.2% | -0.5% | -0.6% | -0.7% | -0.4% | -0.5% | -0.4% |
| RGB, MC, 1080p | -1.1% | -1.5% | -1.3% | -0.8% | -1.1% | -1.0% | -0.9% | -1.0% | -0.4% |
| YUV, TGM, 1080p | -1.2% | -1.7% | -1.5% | -0.8% | -1.7% | -1.5% | -0.5% | -1.2% | -1.0% |
| YUV, TGM, 720p | -0.4% | -0.9% | -1.3% | -0.9% | -2.0% | -2.4% | -0.5% | -1.1% | -1.9% |
| YUV, MC, 1440p | -0.2% | -0.9% | -0.7% | -0.2% | -1.2% | -1.0% | -0.5% | -1.2% | -0.9% |
| YUV, MC, 1080p | -1.0% | -3.1% | -2.6% | -1.0% | -3.3% | -2.8% | -1.1% | -3.3% | -2.5% |

Presentation deck not uploaded. The presentation has substantial additional content e.g. about the concept and memory bandwidth.

The contribution claims that string copy targets similar redundancy and applies similar concepts as palette, but is more universal.

It is claimed that worst case memory bandwidth can be restricted by encoder constraint (limiting the number of strings).

Significant gain in lossless case, but gain in lossy case is low

2D strings not yet investigated in this contribution.

General remarks about intra string copy:

- Solutions for memory bandwidth problem occurring with full-frame access need to be investigated. Encoder side restriction does not seem to be a good option, other options could be allowing small strings in closer environments.

- Generally, the gain in lossy coding is rather low compared to the additional implementation complexity.

Wait for the report of the BoG on CE10 which should analyse the complexity of the string copy methods and suggest concrete ways what to further investigate in case that the allowable limits are exceeded.

It was agreed to include the proposal in a CE.

### IBC improvements (other than CE1) (2)

(Chaired by JRO, Monday 10-20 p.m.)

See also contribution S0088 in section 5.1.15.

JCTVC-S0129 On indication of IBC [K. Andersson, M. Pettersson, J. Samuelsson, A. Norkin (Ericsson)]

It is proposed to signal a maximum IBC CU size to enable the possibility to avoid signaling local indications of Intra block copy usage for CUs that are larger than the maximum IBC CU size. When setting the maximum IBC CU size to 16x16 it is reported that the bitrate reduction is 0.1% for the common test conditions.

No significant benefit – no action.

JCTVC-S0238 Cross-check of JCTVC-S0129: On indication of IBC [J. Lainema (Nokia)] [late]

### Adaptive colour transform (14)

(Chaired by JRO, Monday 10-20 p.m.)

JCTVC-S0040 Enhanced chroma QP signalling for adaptive cross-component transform in SCC extensions [K. Chono (NEC)]

This contribution presents an enhanced chroma QP signalling method to enable the switching of chroma QP offset values between RGB color space and YCoCr color space within a picture when adaptive cross-component transform is used. It is recommended that the following items are considered for SCC test model adoption or studied in AHG toward the next meeting.

- Additional signaling of PPS/Slice-level chroma QP offset values when adaptive cross-component transform is activated at SPS.

- Switching of chroma QP offset values for reconstructing the second and third color component residuals according to the current CU color space.

Note: In version 2 of the doc, the extension of table size of chroma QP adjustment was removed by considering the current PPS extension design; syntax names related to adaptive cross-component transform were corrected according to JCTVC-R1005-v3.

The main motivation is for allowing higher flexibility in quantizer adaptation, beyond the alignment of QP values for ACT blocks that was introduced in the previous meeting (Note: According to several proposals, this alignment could lead to negative QP values, however, it is not part of S0040 to fix that)

The proponent requests to study these aspects in an AHG.

Related proposals: S0086, S0094, S0140, S0144, see further notes there.

JCTVC-S0086 On residual adaptive colour transform [B. Li, J. Xu (Microsoft)]

This document proposes several aspects to fix and modify the current design of residual adaptive color transform, including: 1) condition of encoding ACT flag; (2) the interaction on bit depth of ACT and CCP; (3) QP modification process for ACT; (4) unification of lossy conversion and lossless conversion; (5) residual_adaptive_colour_transform_enabled_flag signaling; (6) ACT for different bit depth.

Presentation deck not uploaded.

Item 1: It was discussed whether the additional check would impose unacceptable complexity, but the general opinion is that this is not the case. Imposing at the decoder side that ACT can never be used when DM is off would also allow to remove the encoder constraint that is currently in the spec.

Decision: Adopt item 1 (syntax, and additional removing the encoder constraint).

Item 2: This would imply a change to the CCP (different in SCC and RExt), and therefore is undesirable. Even though the prediction could be improved by extending the bit depth by 1, the impact on compression performance is minor (around 0.1% gain in lossless). No action.

Item 3: The correction of QP applied to ACT can lead to negative QP values. (other contributions on this). One solution is clipping, another is item 4.

Item 4: Increase the bit depth for all three components in lossy case. The motivation is unification of lossy and lossless (latter has increased bit depth for Co and Cg), and at the same time the QP problem is solved. The input bit depth for the subsequent DCT is also increased, which may have the implication that e.g. for 8 bit version 1 DCT designs cannot be used. No action.

Item 5: Propose moving ACT enabling flag to PPS, or make it dependent on 4:4:4.

Decision: Move flag to PPS.

Item 6: ACT for different bit depth of component is not defined. Two solutions proposed: disable ACT for that case, or normalize to same bit depth. S0180 also proposes the latter solution. This seems to be the preferred solution – see further discussion in context of S0180.

JCTVC-S0218 Cross-verification of JCTVC-S0086 [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-S0094 QP offset for Adaptive Colour Transform [R. Sjöberg, J. Samuelsson (Ericsson)]

Notes from review in BoG:

This contribution proposes to add an additional set of QP offset parameters that are applied for coding units that uses the Adaptive Colour Transform.

The contribution claims that additional flexibility and improved visual quality can be achieved through the introduction of three new QP offset syntax elements in the PPS and three new QP offset syntax elements in the slice header. The new syntax elements are proposed to be applied when Adaptive Colour Transform is used, i.e. when the cu_residual_act_flag is equal to 1. The contribution claims that it would be beneficial to be able to separately select which QP to use for Green, Blue, Red, Luma, Chroma Orange and Chroma Green depending on the content of the video and the desired compressed result.

The contribution further proposes to move the fixed QP offset that is applied to blocks that are color transformed (to compensate for that the transform is not normalized) so that it is included in the normal QP derivation. The contribution claims that this change ensures that the QP value stays in the allowed range.

Not fully presented, because proposed concepts included in JCTVC-S0300.

JCTVC-S0140 On transform coefficient scaling for adaptive colour transform [K. Misra, S. H. Kim, A. Segall (Sharp)]

Notes from review in BoG:

This contribution identifies a problem for the adaptive colour transform where the quantization parameter (qP) may take on negative values. This is a result of absorbing the scaling associated with adaptive colour transform within the transform coefficient scaling process. Note, that the transform coefficient scaling is undefined for negative qP values since the modulus operation is undefined for negative arguments. To avoid this issue it is proposed that for blocks where the effective qP may take on negative values, adaptive colour transform is disabled. Alternatively, one can clip qP to a valid range.

Some experimental results provided.  It was suggested that the bitrates in the experiments exceed the lossless bitrates for the sequences.

With option a, the QP derivation has to be derived before parsing, so there is a parsing dependency,  which is undesirable.

Option b seems to be a very simple and minimal solution to the bug of the negative QP values.

BoG Recommended to Adopt JCTVC-S0140 option b) clipping of negative QPs to zero to address the QP underflow issue. Also proposed in S0144, S0086.

See also notes on BoG report.

JCTVC-S0285 Cross-verification of JCTVC-S0140 on On transform coefficient scaling for adaptive colour transform [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-S0144 On Qp for adaptive color transform [K. Rapaka, L. Zhang, R. Joshi, M. Karczewicz (Qualcomm)]

Notes from review in BoG:

In the 18th JCTVC meeting, the adaptive color transform was adopted into the test model of SCC extension. The tool adaptively transforms prediction residuals on one color space into another and signals a flag to indicate its color space at CU level. This contribution identifies various issues in the current test model related to quantization parameter derivation when adaptive color transform is used and proposes solutions to fix them.

Very similar to S0300.  Same signaling as S0300 but difference in the decoding process in the usage of Cb offsets.   (8.6.1 vs 8.6.2.)  Proposed to only modify 8.6.2.

JCTVC-S0179 On inter-component de-correlation for screen content coding [X. Xiu, Y. He, Y. Ye (InterDigital)]

Notes from review in BoG:

This contribution proposes to improve the inter-component de-correlation methods in HEVC screen content coding draft 1 in order to simplify pipeline design and improve parallelization. Firstly, it is proposed to move inverse adaptive color transform from CU/PU-level to TU-level to unify with inverse cross-component prediction, which is performed at TU-level. Secondly, it is proposed to combine inverse cross-component prediction and inverse adaptive color transform into one single decoding process.

Compared to the SCM-2.0 anchor, experimental results show that the proposed solution has the same coding performance and similar encoding and decoding time, while offering the benefit of reduced latency.

First proposed change is editorial and software change.  Improved decoder times, because ACT operation can be avoided for all zero blocks.

Second proposed change is normative. Not much coding efficiency impact.  Small increase in encoding times. It was remarked that CCP is already present in RExt, and making a change might be undesirable.

BoG recommended SW adoption of the first option.  The associated editorial change is delegated to the editors.

See notes on BoG report review.

JCTVC-S0069 On adaptive colour transform and Inter modes [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

Notes from review in BoG:

The adaptive colour transform was adopted during the previous meeting. This contribution proposes to enable the colour transform for Inter modes at sequence parameters set instead of enabling it at CU level. A BDR average of 0% and -0.1% compared to SCM2.0 for respectively RA and LDB configurations is reported with an encoding run time of 95% and 96%.

Proposes to not send CU level ACT flag for inter, but adds picture level signaling.

Some losses and some gains on different sequences.  It was suggested that much of the encoding speed benefits might be accomplished with encoder-only modifications.

Related to S0100.

JCTVC-S0230 Cross-check of JCTVC-S0069 on adaptive colour transform and Inter modes [P. Lai, S. Liu (MediaTek)] [late]

JCTVC-S0100 AHG6: On Adaptive Color Transform (ACT) in SCM2.0 [P. Lai, S. Liu, S. Lei (MediaTek))]

Notes from review in BoG:

This contribution proposes to enable and disable adaptive colour transform (ACT) solely based on the content format (e.g., RGB or YUV). It presents two studies of the ACT in SCM2.0.

In part I, the encoding algorithm of ACT in SCM2.0 is studied. It is asserted that for coding YUV contents, the ACT encoding algorithm bypasses some early skips for checking Intra Block Copy (IBC) mode. Results of only removing this part of the encoding algorithm from ACT are provided, and compared against completely disabling ACT on SCM2.0. It is reported that, for YUV format, this encoder-side IBC checking modification used by the ACT encoder, contributes most of the coding efficiency differences provided by the entire ACT in SCM2.0. For example, average 2.6% out of 3.1% for YUV, test and graphics with motion, 1080p.

In part II, a study was conducted in which, when applicable, always turning on ACT for coding RGB content. Changes were made for encoder to only evaluate ACT on for RGB content, such that the decoded CU-level ACT flags are always 1. It is reported that for coding RGB content, forcing CU-level ACT flags always to be on, preserved most of the gains achieved by CU-adaptive ACT (less than 1% BD-rate impact out of 6% to 38% BD-rate impact). Meanwhile, encoding time is decreased by average of 9% / 7% / 7% in lossy AI / RA / LB for RGB content.

Asserts that most of the ACT gain for YUV comes from change to early termination of IBC. Proposes turning ACT off in the common conditions for YUV, which would reduce the encoding time (to 69%), with some loss associated.

It would be interesting to see experimental data for ACT off for YUV with the change to early termination of IBC or other encoder changes to see if the losses would be reduced.

BoG recommended discussion in the track about the IBC encoder algorithms related to early termination, and interaction with ACT.

The coding loss for the lossless case with the second proposal was high, up to 11%.

See also notes on BoG report review.

This was further discussed in JCT-VC, chaired by JRO on 10-23.

The contributor showed test results (to be uploaded in a revision of the contribution) that appear to show, for AI, that ~20% speed-up for YUV cases can be achieved by

1) Changing the CTC to not perform ACT for YUV cases and

2) Making an encoding early-termination check be performed regardless of whether ACT is performed or not.

with a modest penalty in coding performance in All-Intra coding YUV coding (-0.2% to +0.6%, depending on YUV category).

The contributor said that item 2 above is expected to improve RGB cases. This was questioned by a participant.

It was commented that the modest penalty in coding performance would be larger for RA and LD (e.g., 1.5% luma, 2.5% chroma for one class of LD).

Incomplete data was available to analyze this.

Decision (SW): Make the item 2 change only, without changing CTC.

Further study was encouraged to better understand the issues involved.

In a follow-up discussion in JCT Thu AM, additional results were presented showing for the AI lossless case that by disabling ACT for ICTcases and performing early skip IBC that is currently only applied with ACT, the gain by ACT becomes minor (disabling ACT, a loss of up to 0.3% is observed). It is however commented in the discussion that from the results of the original contribution the loss would be larger in RA and LDB.

An update of the contribution with the new results presented was to be provided.

It was remarked that the structure of the encoding for ACT and non-ACT cases in the software is undesirably inconsistent and has some excess copying of functions and undesirably different quadtree traversing orders. Further study to try to understand and clean up the software was encouraged.

JCTVC-S0244 AHG6: Cross-check of S0100 (On Adaptive Color Transform (ACT) in SCM2.0) [M. Xu, W. Wang, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0133 Adaptive Colour Transforms for Screen Content Coding [W. Dai, M. Krishnan, P. Topiwala (FastVDO)]

Notes from review in BoG:

Techniques for adaptive integer color transforms are usually developed for RGB 4:4:4 input data, converting the RGB data to one of several (YUV-like) integer color spaces prior to encoding/decoding. The decoded data is converted back to RGB 4:4:4, where performance quality is measured. In this proposal, the color-space transform converts prediction error in 4:4:4 chroma format into several YUV-like color spaces. Experiments shows that redundancy among three color components is further reduced.

Proposes 3 additional color transforms. The proposed new transforms fit into the framework used for YCoCg, but with different coefficients.

Reported gains of up to 3% for RGB, and 0.5% loss for Y in YUV (with 1.6 and 1.7% gains from U and V).

No cross-check, but is willing to provide software. No encoder/decoder times provided, but encoding time is significantly higher.

It was questioned if all 3 additional transforms are needed.

Further study can explore impact on encoder and decoder complexity, justification of the need for 3 additional transforms rather than 1.

JCTVC-S0265 Cross-check of inter-component de-correlation for screen content coding (JCTVC-S0179) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-S0180 Adaptive color transform for different luma and chroma bit-depth [X. Xiu, Y. He, Y. Ye (InterDigital)]

Notes from review in BoG:

This contribution proposes to modify the adaptive color transform in HEVC screen content coding draft 1 in order to handle the internal bit-depth difference between luma and chroma components. Specifically, it is proposed to align the bit-depths of luma and chroma components before adaptive color transform by left-shifting the component with lower bit-depth to match the bit-depth of the other component; then, the adjusted component is converted to its original bit-depth by right-shift after adaptive color transform. Additionally, two encoder-only bug fixes are also provided to enable the HEVC screen content reference software (SCM-2.0) to properly work with different internal luma and chroma bit-depths. Both the bug fixes and the proposed solution are implemented and tested on the SCM-2.0 software for two settings with unequal internal luma and chroma bit-depths, namely, 10-bit luma and 8-bit chroma (Setting one), and 12-bit luma and 8-bit chroma (Setting two).

Compared to the SCM-2.0 anchor, for Setting one, the proposed bug fixes could provide average {G, B, R} BD-rate savings of {1.6%, 2.6%, 2.6%}, {0.1%, 1.3%, 1.1%} and {0.8%, 0.7%, 0.8%} for AI, RA and LB configurations in RGB coding, and provide average {Y, Cb, Cr} BD-rate reductions of {1.6%, 5.2%, 5.1%}, {0.4%, 3.9%, 3.7%} and {0.4%, 3.5%, 3.4%} for AI, RA and LB configurations in YCbCr coding. For Setting two, the corresponding coding gains of the proposed bug fixes are {5.6%, 6.5%, 6.6%}, {4.8%, 8.8%, 7.5%} and {2.5%, 11.5%, 9.4%} for the {G, B, R} BD-rate savings in RGB coding, and are {3.4%, 6.5%, 7.0%}, {2.4%, 5.1%, 5.7%} and {1.9%, 4.5%, 5.0%} for {Y, Cb, Cr} BD-rate savings in YCbCr coding.

Compared to the SCM-2.0 anchor with the bug fixes, for Setting one, the proposed bit depth alignment solution could provide average {G, B, R} BD-rate savings of {15.4%, 10.1%, 10.7%}, {20.1%, 13.2%, 14.2%} and {23.6%, 18.1%, 19.2%} for AI, RA and LB configurations in RGB coding, and provide average {Y, Cb, Cr} BD-rate reductions of {0.1%, 1.3%, 0.9%}, {0.3%, 1.5%, 1.1%} and {0.3%, 1.3%, 1.0%} for AI, RA and LB configurations in YCbCr coding. For Setting two, the corresponding coding gains of the proposed bit depth alignment solution are {22.1%, 17.1%, 17.7%}, {40.0%, 33.7%, 34.6%} and {47.3%, 40.4%, 41.1%} for the {G, B, R} BD-rate savings in RGB coding, and are {0.1%, 1.3%, 0.9%}, {0.4%, 1.6%, 1.2%} and {0.3%, 1.6%, 1.2%} for {Y, Cb, Cr} BD-rate savings in YCbCr coding.

Related to S0086.

Proposes an encoder only change and a normative change.

The normative change requires an additional shift per sample if bit depths differ.

Different behavior required for lossy and lossless case, since the shift causes a loss. It was suggested to disable the combination of lossless, ACT and different bit depths.  Could perhaps be implemented as a semantic constraint on the value of the ACT flag when lossless and different bit depths. A constraint was seen as preferable to a syntax change.

It was questioned if any real applications would use different bit depths for color components in RGB.  It was suggested that an encoder could shift to align each color component prior to encoding.

It was mentioned that CCP does a bit depth alignment.

BoG recommended to adopt both the encoder only change and the normative change (to add a shift for bit depth), and to add a constraint for lossless as noted above.  A revised version of the contribution to reflect these decisions will be uploaded.

See notes on BoG report.

JCTVC-S0240 Cross-check report of JCTVC-S0180 on Adaptive color transform for different luma and chroma bit-depth [K. Rapaka (Qualcomm)] [late]

JCTVC-S0254 Unification of colour transforms in ACT [L. Zhang, J. Chen, M. Karczewicz (Qualcomm), B. Li, J. Xu (Microsoft)] [late]

Notes from review in BoG:

The current design of residual-domain adaptive color transform uses two sets of colour transforms (i.e., YCoCg, and YCoCg-R) for lossy and lossless coding, respectively. In this contribution, it is proposed to use YCoCg-R for both lossy and lossless coding. To keep the bit-depth unchanged for lossy coding mode, the values of Co and Cg components after forward YCoCg-R transform are scaled by a factor of 1/2. Simulation results show that there is almost no coding performance difference compared to current design.

BoG recommended to adopt. Test model will also need to be updated.

See notes on BoG report review.

JCTVC-S0286 Cross-verification of JCTVC-S0254 on unification of colour transforms in ACT [X. Xiu, Y. He, Y. Ye] [late]

JCTVC-S0300 Qp derivation and offsets signaling for adaptive color transform [K. Rapaka, L. Zhang, R. Joshi, M. Karczewicz(Qualcomm), K. Chono (NEC), J. Xu(Microsoft), R. Sjöberg(Ericsson), K. Misra, S. H. Kim, A. Segall (Sharp)] [late]

Notes from review in BoG:

In the 18th JCTVC meeting, adaptive color transform was adopted into the test model of SCC extension. The tool adaptively transforms prediction residuals of one color space into another and signals a flag to indicate its color space at CU level. Several contribution in the 19th JCTVC meeting have identified various issues in the current test model related to quantization parameter derivation when adaptive color transform is used and proposed solutions to fix them. This contribution combines aspects from proposal JCTVC-S0144, JCTVC-S0040, JCTVC-S0094 and JCTVC-S0086.

New syntax proposed to signal offsets for each of 3 components.

Some editorial issues found in contribution, variable vs syntax element.

Problem was identified with underflow of QP at extreme values, and solution proposed.  Proposes changes to 8.6.1 (for Cb, Cr) and 8.6.2 (for Y). The proposed solution could also provide coding efficiency benefits by allowing more flexibility in the offset selection, with different values depending on whether or not ACT is used on a CU basis.

With the current design, when ACT is used, chroma QP encoder algorithms deployed in AVC and HEVCv1 may not be optimal when applied for RGB.  Signaling of additional offsets provides that functionality.

The exact decoding process to be applied with the additional offsets are signaled is not well justified in the contribution.  Should the offsets apply to deblocking and QP prediction or not?

No experimental results provided.

It was suggested to make a CE.  Proponents questioned if a CE was necessary because this could considered to be a functionality, with 6 bits per PPS.

Encourage further study of signaling of ACT-specific chroma QP offsets, probably in the PPS, and how the decoding process would use the offsets, and provide evidence of benefits.

### Deblocking in SCC (AHG13) (6)

See also S0112.

JCTVC-S0044 AHG13: Chroma deblocking filter control for SCC [O. Nakagami, T. Suzuki (Sony)]

(Chaired by A. Norkin on Saturday 10-18 p.m.)

This contribution proposes chroma deblocking filter control for SCC.

First, disabling deblocking filter was tested under SCC CTC. The existing pps_deblocking_‌filter_‌disabled_‌flag was set to 1. It is reported that the BD-rate difference {G/Y, B/U, R/V} is {+1.1, −0.6, −0.2}/‌{+1.2, −1.6, −1.5}/‌{−0.8, −1.5, −1.4} for AI/RA/LB, respectively.

Then, disabling chroma deblocking filter was examined. The decoder was changed to skip the chroma process while the luma process was applied as the anchor. It is reported the result is {0.0, −0.6, −0.3}/‌{0.0, −1.7, −1.6}/‌{0.0, −1.6, −1.5}.

Visual evaluation was also conducted. It is reportedly shown that the deblocking filter affects the character recognition result of the coded picture.

Finally, text is proposed to control the chroma deblocking process. Syntax and semantics are presented.

The focus is on RGB cases.

For deblocking filter control, we have the following:

- Disabling

- Disable on tile boundaries

- Disable slice boundaries

- Beta offset

- Tc offset

- QP for luma, Cb, and Cr are somewhat distinct

- Deblocking for chroma is based on the slice-level QP, not the block-level QP

- Quant matrices for luma, Cb, and Cr are distinct

- Palette mode escape values use slice-level QP. (We should think about this, as a cleanup issue).

- Possibly, CU-level control of chroma QP offsets.

- Disabling of deblocking with PCM/TQB (controlled by an SPS-level flag)

For example, chroma filtering can be weakened by increasing the chroma QP while decreasing the chroma quant matrix entry values. It can also be weakened by sending a large chroma QP at the slice header level but using a smaller QP at the block level.

Basically, the only component-specific control we currently have for the deblocking filter is the component-specific quantization control.

Substantially different results were measured depending on the type of content. It was remarked that the overall average across different types of content is not necessary appropriate for this proposal.

It was remarked that objective measurements are not the best way to measure the impact of deblocking.

It was suggested that with the various controls that we already have, something resembling what is suggested can be approximately accomplished without adding a new control.

It was suggested that if we want to add another control, we could control each channel separately rather than grouping the two chroma channels together in the control mechanism.

It was remarked that we should try to avoid adding differences relative to the base spec as much as possible.

It was suggested to further study the issue since we already have chroma deblocking control in the spec. and we are not sure whether the current approach is optimal.

Also since we are discussing changes of other aspects of the deblocking, the motivation for this proposal might also change.

Further study was recommended.

JCTVC-S0224 Cross-check of ‘AHG13: Chroma deblocking filter control for SCC’ (JCTVC-S0044) by Sony [C. Rosewarne, M. Maeda (Canon)] [late]

The contributor was asked to submit a revision to correct the abstract.

JCTVC-S0045 AHG13: On deblocking for screen content coding [C. Rosewarne, M. Maeda (Canon)]

(Chaired by A. Norkin on Saturday 10-18 p.m.)

This contribution proposes a modification of the deblocking filter to edges between blocks where intra-block copy is used. In this method, if the block on either side of the edge uses intra-block copy, then the boundary strength is set to 1 (even if the other block uses intra-prediction). Visual testing was performed and did not uncover any reduction in subjective quality.

IBC currently uses strong deblocking, because it is treated as an intra mode. Intra uses Bs = 2.

The proposal is to always use Bs = 1 for IBC (both sides of the block boundary), and Bs = 2 for other intra edges.

The gains for chroma are up to 3.1% on RA and 4.8% on LD. Gains on luma are up to 0.2% on RA and 0.3% on LD.

It was remarked that the gains shown are mostly for chroma, and the gain could alternatively be achieved by reducing or disabling deblocking of chroma.

It was remarked that it would be desirable to consider harmonization of the proposals with the precious design and try to resuse the existing blocks as much as possible.

It was remarked that we should not filter the block boundary if the two adjacent BV are identical.

Further discussion was requested to be held after reviewing contributions on harmonization of IBC mode.

See also S0112.

Further discussion was chaired by A. Norkin Wed. 10-22 p.m.

It was remarked that this proposal would need an additional flag in the line buffer (1 bit per 8 samples) and additional check in the deblocking filter.

It was decided to study this proposal further in the CE together with the other proposal S0112.

JCTVC-S0202 Cross check of On deblocking for screen content coding (JCTVC-S0045) [O. Nakagami (Sony)] [late]

JCTVC-S0096 AhG13: Palette and deblocking [J. Sole, W. Pu, C. Pang, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)]

(Chaired by G. Sullivan on Saturday 10-18 p.m.)

The deblocking filter is applied to palette coded blocks. It was suggested, however, that generally, filtering and smoothing processes are avoided for screen content coding. The contribution proposes (as in JCTVC-R0213) to not deblock CUs coded with palette in the same way that PCM and lossless CUs are not deblocked. This is asserted to reduce complexity, avoid unwanted filtering and achieve BD-rate reductions (up to 0.2%). Additionally, it is suggested that a flag at SPS level could be included to enable/disable deblocking of palette coded CUs.

We currently treat palette coded regions as intra, thus applying strong deblocking to them. This is suggested to be undesirable.

Some gain is shown for disabling deblocking for palette coded regions.

Decision: Disable deblocking within palette coded regions. (Do not introduce a flag.)

What about the neighbour region? Should we disable filtering on both sides of the edge? Perhaps not. (The proposal does not disable it, which seems OK.)

JCTVC-S0273 Cross-check of ‘AhG13: Palette and deblocking’ (JCTVC-S0096) by Qualcomm [C. Rosewarne, M. Maeda (Canon)] [late]

### SCC complexity reduction (AHG9&10) (3)

JCTVC-S0068 AHG14: On IBC memory reduction [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

(Chaired by G. Sullivan on Saturday 10-18 p.m.)

An IBC PU predictor comes from reconstructed non-filtered blocks. This increases the memory needed for some implementations, an example being the case where loop filtered blocks need to be stored in addition to the reconstructed blocks because of IBC. To avoid this additional storage, this contribution proposes to signal which CTUs are available for IBC prediction. For these CTUs, the DBF and SAO are disabled. The proposed method reportedly gives an average of 0.7% loss for the AI configuration compared to the current SCM2.0.

This proposes to code a CTU-level flag, available_for_ibc_flag, to identify whether a block may be referenced by IBC. When equal to 1, DBF and SAO are disabled within that CTU. (On other side of an edge, there may be filtering.)

AI average loss is average 0.7%. About 2% average loss for RA & LD. This seems excessive.

The peak category of loss was YUV 1080p mixed content, which had 5.1% average loss.

It was remarked that some kind of modified encoding search could probably reduce that loss.

Further study of this was encouraged.

JCTVC-S0241 Cross-check report of JCTVC-S0068 On IBC memory reduction [K. Rapaka (Qualcomm)] [late]

JCTVC-S0145 On IntraBC bandwidth [K. Rapaka, T. Hsieh, C. Pang, V. Seregin, M. Karczewicz (Qualcomm)]

(Chaired by G. Sullivan on Saturday 10-18 p.m.)

This contribution proposes a method to reduce average bandwidth when intra block copy (IBC) mode is used for prediction. IBC mode uses previously decoded unfiltered samples within the same picture for its prediction. In the current test model, for the IBC mode the search range is unrestricted and can use any unfiltered decoded samples of the current picture that lie in previously decoded regions. This introduces additional bandwidth required to store unfiltered decoded samples of the current picture (in addition to filtered decoded samples required for temporal prediction). It is observed that on average not all previously decoded unfiltered samples of the current picture are used for prediction in IBC mode and the usage depends on the characteristics of content, for example, the usage peaks for text and graphic sequences and is minimal for camera-captured content. Hence, it is argued that always storing the previously decoded unfiltered samples for the current picture is inefficient from the bandwidth perspective. In this contribution it is proposed to indicate which of the previously decoded CTUs are used for IBC prediction so that only those CTBs could be stored thereby reducing the average bandwidth.

Similar to S0068, the contribution proposes to send one flag per CTU to identify which parts of the picture need to be additionally stored for non-filtered referencing. However, it does not disable the ILFs and does not reduce the worst case.

It was remarked that the write function is not as much of a problem as the readback.

The implications of needing to know the how many CTBs are in the slice and needing to send the flags in advance were questioned.

The proponent indicated that it would be possible to send this information as an SEI message.

It was asked whether IBC is constrained to reference only within a slice and/or within a tile. An editor said he thought such constraints exist, but would doublecheck.

Further study of this was encouraged.

### SCC parallel processing (AHG14) (8)

JCTVC-S0070 AHG14: On IBC constraint for Wavefront Parallel Processing [G. Laroche, T. Poirier, C. Gisquet, P. Onno (Canon)]

(Chaired by G. Sullivan on Saturday 10-18 p.m.)

This contribution is related to the IBC and WPP. It is proposed to constrain the IBC referencing range for WPP to avoid having the IBC predictor refer to a non-reconstructed block. The constraint rule consists in limiting the IBC search range to the potential reconstructed blocks when WPP is enabled. The proposed rule results in average BDR losses of 0.6%, 0.3%, 0.1% for AI, RA and LDB configurations, respectively, compared to SCM2.0 when WPP is enabled.

The proposal is to limit IBC references to preceding CTUs in wavefront order.

There is some coding efficiency penalty. The peak per-category loss is 1.4%.

It was discussed whether to impose the constraint always or only when ECS is enabled.

It was noted that even if the bitstream is not wavefront-structured, the post-parsing decoding process can be wavefront structured if this constraint is imposed.

The area constraint proposed in this contribution is somewhat less restricted than in some related contributions.

Decision (BF): Adopt the constraint, regardless of whether wavefront is being used. A CTU can reference one CTU to its right in the previous row, but not further to the right than this.

Further discussion was chaired by J. Boyce: Decision: In subsequent discussion, a further restriction proposed in S0088 was adopted.

JCTVC-S0231 AHG14: Cross-check of JCTVC-S0070 on IBC constraint for Wavefront Parallel Processing [P. Lai, X. Xu (MediaTek)] [late]

The cross-check had not yet been completed when the initial discussion occurred. It was later uploaded, and confirmed the results.

JCTVC-S0088 On WPP with palette mode and intra BC mode [B. Li, J. Xu (Microsoft)]

(Chaired by J. Boyce on Saturday 10-18 p.m.)

This document discusses the interaction between WPP and palette mode and Intra BC mode.

- This document proposes to synchronize previous palette information similar as CABAC status when WPP is used together with palette mode. The experimental results reportedly show that when synchronizing the previous palette information with the above CTU row, 0.5% and 0.7% bit rate saving is achieved for lossy AI text & graphics with motion (TGM) 1080p RGB and YUV, respectively.

- This document proposes to add bitstream conformance to limit the pixels used for Intra BC prediction when Intra BC is used together with WPP.

The contribution proposes to align palette prediction the existing CABAC design when WPP is used

The current design requires storage of the palette for a single LCU, and the proposal would require storage of the entire LCU row's worth of palette. Storage requirement is 287 bytes per LCU row if raster scan decoding is used. If WPP decoding is used, the storage requirement is N*194, with N threads.

A participant asked whether the previous palette or the palette predictor is proposed to be stored and used for the proposal. It is the palette predictor.

JCTVC-S0141 is related.

The coding gain when palette and WPP are used is in the range of 0.0–0.7% for lossy and 0.0–0.5% for lossless.

A second aspect of this contribution proposes to restrict which above CTUs are available for use for IBC, and is similar to JCTVC-S0070, but is more restrictive as to which CTUs can be used. For the less restrictive proposal in S0070, additional checks are required by the decoder to determine whether decoding can be done than with this proposal. No experimental data was provided in the contribution, but may be available in JCTVC-S0200.

Decision: Adopt the IBC constraint region proposed here, where one addition CTU to the right of the current CTU may be used in each row above, at a 45 degree angle. The constraint will apply regardless of whether or not WPP is used in the bitstream. This aspect of applying regardless of whether WPP used was proposed in S0220.

The palette aspect was further discussed 10-23 p.m., chaired by J. Boyce.

JCTVC-S0141 also proposed to align palette prediction with CABAC when WPP is used.

Decision: Adopt the palette prediction alignment with CABAC when WPP is used, also proposed in S0141.

JCTVC-S0242 Cross-check of S0088 (On WPP with palette mode and intra BC mode) [W. Wang, M. Xu, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0101 AHG14: Intra Block Copy reference area for Wavefront Parallel Processing (WPP) [P. Lai, X. Xu, S. Liu, T.-D. Chuang, S. Lei (MediaTek)]

(Chaired by J. Boyce on Saturday 10-18 p.m.)

This contribution presents modifications to the IBC reference area, such that when WPP is utilized in the bitstream, IBC can only reference blocks within the already reconstructed CTUs by the WPP threads. With WPP turned on (WaveFrontSynchro > 0 in configuration files), two reference area patterns are tested:

- A near-rectangular reference area

- A "ladder-shaped" reference area.

Related to JCTVC-S0070 and S0088. This proposal is even more restrictive, and has more loss, up to 2.9%. See notes on S0070, S0088, and S0220.

JCTVC-S0274 AHG14: Cross check of JCTVC-S0101 on Intra Block Copy reference area for Wavefront Parallel Processing [P. Onno (Canon)] [late]

JCTVC-S0141 Using the wavefront store-and-sync design for palette table prediction variables [K. Misra, S. H. Kim, A. Segall (Sharp)] [late]

This is the same as part of S0088.

See S0088.

JCTVC-S0257 Cross-check of using the wavefront store-and-sync design for palette table prediction variables (JCTVC-S0141) [V. Seregin (Qualcomm)] [late]

JCTVC-S0220 On parallel processing capability of intra block copy [K. Rapaka, V. Seregin, C. Pang, M. Karczewicz(Qualcomm)] [late]

Chaired by J. Boyce, Sat.

This contribution proposes methods to enhance parallel processing capability when intra block copy (IBC) mode is enabled. IBC mode uses previously decoded unfiltered samples within the same picture for its prediction. This introduces a dependency that prediction samples of the current block have to be reconstructed before processing the current block. In this contribution some restrictions and indications are proposed on IBC block vectors (BV) to allow decoder to be able to parallel process multiple CTUs in non-raster scan order.

In the revision 1 of the document, simulation results were provided for different delay restrictions proposed for intra BC.

Related to S0070, S0088, and S0101. This proposal proposes applying the IBC constraint regardless of whether WPP is used in the bitstream.

The contribution proposes, in "Option b", signalling in the slice header of what type of restriction is imposed. This could also be signaled in the PPS, SPS or VUI. It was questioned whether signaling is more appropriate than a normative restriction.

Three different constraint patterns were proposed, with experimental data provided for each. The experimental data does not use WPP in the bitstream encoding.

Decision: Adopt the figure 4.1 pattern of IBC constraint, as is also noted under JCTVC-S0088.

JCTVC-S0306 Cross-verification of JCTVC-S0220 On parallel processing capability of intra block copy [X. Xiu (InterDigital)] [late]

Has "the" problem.

### SCC Other (6)

JCTVC-S0075 Copy Mode for Static Screen Content [T. Laude (Leibniz Universität Hannover)]

(Chaired by G. Sullivan on Saturday 10-18 p.m.)

This contribution presents a copy mode which reportedly aims at the coding of static screen content. In particular, it is reported that the sample values of a block coded with the copy mode are reconstructed by copying the sample values from the corresponding block at the same position in the closest reference picture. Furthermore, it is asserted that the copy mode is only applied on CTU level. The contribution states that Y/G BD-rate changes of {-0.2%, 0.0%, -0.5%, -0.6%, 0.0%, 0.0%, -0.2%, -0.3%, -0.6%, -1.0%, 0.1%, 0.0%} and {0.0%, 0.0%, -0.2%, -0.1%, 0.0%, 0.0%, 0.0%, -0.1%, -0.1%, -0.1%, 0.1%, 0.0%} are achieved for LD and RA compared to SCM-2.0 under SCC common test conditions.

The contribution proposes a context-coded flag (with one context) prior to the skip flag. When the flag is equal to 1, the CTU is copied from the picture with the closest POC. (This conceptually differs slightly from the usual collocated picture which we normally signal explicitly.)

The reported gain ranges from −0.1 to 1.0% per sequence category.

It was commented that there may be an R-D optimization effect in the measured results.

It wsa suggested to try setting the max number of merge candidates to 1.

It was asked whether the percentage gains are in cases where the compression ratio is already very high.

It was asked whether there may be PSNR clipping effects.

Further study was encouraged to check the result and determine whether there is more than an RDO effect or a measurement phenomenon involved in the measured results.

JCTVC-S0085 Adaptive motion vector resolution for screen content [B. Li, J. Xu, G. Sullivan, Y. Zhou, B. Lin (Microsoft)]

(Chaired by JRO on Thursday 10-23 a.m.)

This document proposes and reports performance measurements for disabling fractional pixel motion compensation in high-level syntax, as previously proposed in JCTVC-P0277, JCTVC-Q0155 and JCTVC-R0106. For many screen contents, the spatial displacements are of full pixel and the encoder can be notified accordingly by the application system. Not to consider fractional-pel displacement not only save bits on motion vectors, it also reduces the complexity of both the encoder and decoder. The proposed modification does not change the syntax or parsing process. A method to determine the MV resolution in SCM-2.0 encoder is also developed. The modification is shown to improve coding efficiency as much as 7.9% for some screen content. For RGB text and graphics with motion (TGM) 1080p sequences, 2.7% and 4.0% bit saving on average is obtained for RA and LB lossy coding. The experimental results show that most coding gain could be obtained without increasing the encoding running time.

Proposed to include a slice-level flag.

Encoder decision in the presented results is multi-pass. It is shown that for the TGM class an encoder only solution does not give the same benefit (0.4% vs. 3.0% for RA, 0.5% vs. 4.1% for LDB). With a single-pass coding, the gain is still 2.7% and 4.0% for RA and LD (lossy).

The decoding of MV is unchanged. The suggested change is to re-interpret the MV (<<2) before the MC stage.

TMVP is not changed.

It is pointed out that in real applications a fast algorithm may not be as simple to build as in the reference software. The proponents suggest that there could also be situations where the encoder has knowledge about the presence of only integer motion.

The additional decoder complexity would be an additional condition check and two shift operations at the PU level. This additional complexity appears to be minor, and a good tradeoff with the achieved compression gain. In the context of SCC, new designs are likely to be anyway necessary at the PU level (e.g. when using same circuits for MV and BV dependent compensation).

Two options are proposed: Only switching quarter/full, or switching quarter/half/full pel precision.

According to several experts’ (and the proponents) opinion, the first option (QP/FP) is the preferred solution, no advantage is evident to also enable the half-pel option.

Decision: Adopt (first option – integer/quarter pel adaptation with enabling flag at slice level, and feature enabling flag at SPS).

The method (with fast encoder decision) shall be used in CTC. It was recommended to perform further study whether this is meeting the requirement of latency in LD case.

JCTVC-S0239 Cross-check report of JCTVC-S0085 on Adaptive motion vector resolution for screen content [K. Rapaka (Qualcomm)] [late]

JCTVC-S0187 Non-SCCE: SCC with extended LCU size [D. Jiang, X. Zhang, Y. Wu, Z. Wang (Lenovo)]

(Chaired by GJS on Wednesday 10-22 a.m.)

[add abstract]

Proposes 128x128 max CTU size and PU size. Max TU still 32x32.

Gain for AI is from 0.1 to 3.6% depending on category – a bit less in RA and LD.

Presentation table columns had a labelling mixup.

40-50% coding time increase.

It was remarked that the implications on implementation would be substantial – both in terms of the amount of memory that must be accessed together at high speed and in terms of the ability to adapt a design that was previously used for v1 implementation.

It was remarked that there is an interaction with other issues such as non-normative encoder decisions.

It was remarked that the method probably has the most benefit on material that is otherwise relatively easy to encode to a high compression ratio.

The information was appreciated, and it may be the case that a larger CTU size could improve coding efficiency; however, the implementation burden of this may be too much.

Further study was encouraged to try to identify the sources of the gain and determine whether some adjustments to the design (e.g., to palette mode coding or IBC or non-normative decisions or merge operations) might be able to yield similar results.

JCTVC-S0283 Crosscheck of JCTVC-S0187 on SCC with extended LCU size [X. Li, Y. Chen (Qualcomm)] [late]

JCTVC-S0272 Intra Reference Prediction by Cross-Component Prediction [K. Kawamura, S. Naito (KDDI)] [late]

(Chaired by G. Sullivan on Saturday 10-18 p.m.)

Since HEVC version 1 standardization, linear-model chroma prediction (LM chroma) was studied. However, a cross-component dependency by the LM chroma was suggested to be unacceptable for a hardware implementation. In this contribution, chroma intra reference samples are predicted from luma samples by using cross component prediction. The dependency in this contribution is reportedly the same as that by the cross-component prediction in RExt. BD-rate impacts for RGB/YUV videos under lossy coding with G/Y, B/U, R/V of the all intra condition were reported.

The contribution proposes a new intra prediction mode in which an intra prediction block for chroma is predicted using a linear model applied to luma boundary sample values, where the parameters of the linear model are derived at the decoder side.

The reported test results showed some loss in luma and gain in chroma – e.g., 0.0-0.6% loss in luma and 0.0-2.9% gain in chroma, depending on the sequence category. The biggest gains are in YUV cases, where not so much bit rate may be expended on chroma. Measurements would need to be refined to avoid the question of whether this is just bit rate allocation shifting from luma to chroma.

No action taken on this.

## HL syntax (0)

No contributions noted.

## SEI and VUI (5)

JCTVC-S0031 Additional Definitions of FPA SEI Message for Inclusion of Centralized Color-Depth Packing (CCDP) Formats [J.-F. Yang, K.-Y. Liao, H.-M. Wang, Y.-H. Hu (NCKU)]

(Chaired by G. Sullivan on Sunday 10-19 a.m.)

The most popular frame compatible formats for transmitting stereoscopic views are side-by-side (SbS) and top-and-bottom (TaB). Currently, the defined frame packing arrangement (FPA) SEI message supports these two formats and some others ones. However, there are no related standard packing formats for 3D video represented by color and depth information. This contribution proposes an FPA SEI message to cover a series of centralized color-depth packing (CCDP) formats for representation of 3D video with color and depth information. It is reported that 3D video encoded with the CCDP formats could be directly viewed on 2DTV displays without any pre-processing. The CCDP formats are also asserted to have better performance in both color-depth coding and multiview rendering results compared to the frame-compatible color-and-depth SbS packing method based on HM 13.0. The proposed additional FPA SEI Message includes the CCDP formats for delivery of 3D video services in the existing HEVC and AVC video coding standards.

The contribution proposes an x = { 3/4, 5/6, 7/8, 11/12, 15/16 } downscaling of the texture data and 1 − x downscaling of the depth data, with the depth data split, flipped and attached as top and bottom sidebars above and below the texture data.

There was a prior proposal JCT3V-F0087 as a late contribution in November 2013. No action was taken on it at that time.

It was asked what is the relationship between this proposal and the texture and depth view packing SEI message, which is reported in an AVC draft amendment (but not an AVC draft amendment). (Y. Chen, T. Senoh).

It was asked why this is proposed to JCT-VC rather than JCT-3V, and was suggested for it to be reviewed in JCT-3V rather than in JCT-VC.

JCTVC-S0095 HLS: Dependent RAP indication SEI message [R. Sjöberg, M. Pettersson, J. Samuelsson (Ericsson)]

(Chaired by G. Sullivan on Sunday 10-19 a.m.)

This is a follow-up contribution of JCTVC-R0059, for which further study was encouraged at the July meeting in Sapporo. The contribution proposes a new dependent RAP indication SEI message for the HEVC base specification and its extensions. The SEI message is used to indicate the presence of a dependent random access point (DRAP) picture in the bitstream. The following constraints are proposed when a DRAP picture is present:

- The DRAP picture may not include any picture in RefPicSetStCurrBefore, RefPicSetStCurrAfter, or RefPicSetLtCurr except its associated IRAP picture.

- The proposed DRAP picture would be required to be be a TRAIL_R picture with temporal id equal to 0 and layer id equal to 0

- Any picture that follow the DRAP picture in output order and decoding order shall not include, in its RPS, any picture that precedes the DRAP picture in output order or decoding order with the exception of the IRAP picture associated with the DRAP picture.

When performing random access at a DRAP picture, the associated IRAP picture must first be decoded and the value of pic_output_flag should be inferred to be equal to 0 for all pictures that precede the DRAP picture in output order.

No parameters are signalled in the SEI message.

It is asserted that DRAP pictures improve the compression efficiency for random access coded video, especially for video services that often have very static content including screen sharing and surveillance video.

The presentation was requested to be uploaded.

Some use cases and associated measured benefits were presented.

As proposed, the SEI message would contain no syntax elements. The prior proposal had included three syntax elements. The usefulness of one of those syntax elements had been questioned in the previous meeting's discussion.

It was asked whether there are multilayer issues that should be considered. As proposed, the layer ID is required to be zero.

It was remarked that higher-level signalling would be needed in the presented DASH scenario, to the extent that additional in-band signalling might not also be necessary. Other participants suggested that since the concepts are closely tied to the random access properties of the HEVC design, it may be beneficial to have a specification of those properties within the HEVC specification and that the in-band indication may be useful for other environments as well.

There was further discussion on 10-23 a.m. GJS & JRO.

Additional use cases were presented, avoiding the use of IRAP pictures in two scenarios:

- Server-controlled streaming

- Massive-multipoint video conference with late-participant joining

It was remarked that S0196 is related.

Another participant said that this approach seems simpler for decoders than needing to handle the redundant picture functionality. Here the decoder is not required to do anything out of the ordinary to decode the video.

Decision: Adopt.

JCTVC-S0148 Indication of the end of coded data for pictures and partial-picture regions [Y. Wu, L. Zhu, S. Sadhwani, G. J. Sullivan (Microsoft)]

(Chaired by J. Boyce on Sunday 10-19 a.m.)

This contribution proposes the specification of an SEI message to indicate the slice segment address of the next slice header (when present) for decoding latency minimization. It is asserted that it would be beneficial, especially in ultra-low latency use cases, to have an earlier indication of when the coded picture is complete, and particularly to have such an indication within the same access unit as that coded picture itself. Some decoders may be designed to wait for reception of the complete decoded picture before performing the parsing and decoding processes for the picture's VCL NAL units. Requiring such decoders to wait until the next access unit begins is asserted to sometimes add significant delay.

Moreover, for some decoding architectures that are based on region segmentation, it is asserted that it would be similarly beneficial for a decoder to have an indication of how much of the decoded picture has been sent in the preceding VCL NAL units (without waiting for the next slice header). To provide such an indication, an SEI message is proposed for HEVC to identify the slice segment address of the next slice header (when present). The SEI message is primarily proposed as a suffix SEI message.

The same problem is reported to be present in the AVC context, for which the contribution proposes using a previously-reserved NAL unit type (type code 22) for the same basic purpose, since suffix SEI messages are not (at least not yet) defined in AVC. More generally, it would be possible to add the general concept of suffix SEI messages to AVC, for which this message could be the first to be specified.

It was asserted that some decoders may want to buffer up received slices and wait until the entire picture (or region) is available to begin decoding. In the current specification, parsing would be required for the decoder to know that the entire picture has arrived.

The SEI message is also proposed for AVC, but only the HEVC proposal will be considered within the scope of JCT-VC.

Both prefix and suffix SEI message options are proposed. The contents always apply to the next coded slice.

During discussion, it was suggested to add a syntax element (flag) to indicate whether or not a dependent slice segment follows.

It was questioned what the implications would be for scalability, or other multi-layer context. The proposal could apply to a single layer.  It would allow detection of the end of picture, but not the end of the access unit.  While we have an access unit delimeter, that is carried in the next access unit, not the current access unit.

Some systems environments have an indication for end of picture, such as the marker bit in RTP.

There was support for the proposal. Interest was expressed in fully considering the multi-layer context. There is an expectation to adopt at the next meeting (Feb, Geneva) to allow more time for study.

JCTVC-S0196 HLS: On Redundant Pictures SEI message for HEVC [M.Sychev, S.Ikonin (Huawei)] [late]

(Chaired by G. Sullivan on Sunday 10-19 a.m.)

(The second version upload was corrupted. The first and third were not. The third version was the intended second version.)

This contribution follows up on the previous proposals P0062, Q0090 and R00159 proposing a redundant pictures indication for HEVC. The contributor asserted that the proposal would provide the ability for significantly reduce traffic overhead (up to few times) while joining a new participant to a multi-point video conference while maintaining visual quality. Other use cases, such as packet loss recovery, were also presented.

The contributor said that it was well-established that packet loss protection of video streams loss using existing HEVC and MMT tools is often not sufficient for current networks, and that current networks sometimes have packet loss probabilities up to 20%, as reflected in in IP network model standards ANSI TIA-921 and in Rec. ITU-T G.1050 in 2011. The contributor said that existing error protection tools are able to cover only up to 3% of packet loss. The contributor said that the proposal would provide a source-level packet loss protection scheme that would be effective for filling an asserted gap between current HEVC abilities and real network requirements.

The proposal would send a redundant picture in an out-of-order manner – in a different access unit – not in the same order as found in AVC redundant pictures.

Remarks from prior meetings included the following:

Meeting P: "No significant interest was expressed by non-proponents for short-term action on this. Further study was encouraged, although it seemed unlikely that such a concept could be incorporated within our current phase of active extension developments."

Meeting Q: "Further study was encouraged on this topic, but no immediate action was planned. This would be for consideration beyond the scope of the current phase of work."

Meeting R: "Some interest was expressed in further study of the idea, although it is not clear there is a desire to move toward adoption at this time. Further study would be needed to determine whether there is adequate need for this."

Two SEI messages were proposed. The first one would be sent with a redundant picture that indicates that it is a redundant picture, and indicating properties of the primary picture (POC and POC reset information). Such pictures would have PicOutputFlag equal to 0 and would not be referred to by any subsequent pictures (for temporal MV prediction or inter prediction referencing). The second could be sent with a primary picture, and indicate properties of the redundant pictures. The first one could be used with or without the second, and vice versa.

The pictures used as references by the redundant picture (if any) would need to be retained in the RPS of the intervening pictures.

A participant remarked on need for system-level support for the usage and delivery of such pictures.

A participant remarked that it did not seem clear that this is the appropriate approach for the multipoint acquisition use case. The differing timestamp of the redundant picture was mentioned as an aspect that seemed a bit complicated. Some systems might have alternative handling approaches.

It was remarked that the DRAP proposal S0095 is related.

No action was initially planned, pending further discussion to determine the level of interest.

This was further discussed 10-23 a.m., chaired by JRO & GJS.

Some form of redundant pictures was previously standardized, but has not proven popular in actual use.

As proposed, a bitstream would contain these extra pictures, out of order, and these would be indicated for decoding. This implies some loss of coding efficiency relative to not containing this data. If the decoder decodes these pictures when it does not need them, this would also add decoding complexity (relative to not having this data). The benefit would need to outweigh these considerations, and it was not clear to participants that this would often be the case. As previously remarked, other approaches may be more feasible and simpler (e.g., just DRAP pictures or server-side caching of original pictures – perhaps with some system-level support). No interest was expressed by non-proponents.

JCTVC-S0197 VUI codepoint for SMPTE ST 2085 (YDzDx) [C. Fogg, J. Helman (MovieLabs)]

(Chaired by G. Sullivan on Sunday 10-19 a.m.)

Changes to accommodate a code point to indicate YDzDx (SMPTE CD 2085) are proposed for VUI (Annex E) matrix_coeffs (Table E-5) elements in the HEVC and AVC specifications. The Society of Motion Pictures and Television Engineers development of ST 2085 is reportedly expected to be completed by February 2015. A liaison statement to MPEG (m34700) from SMPTE includes the latest draft of CD 2085.

It was commented that the video standards do not accommodate the modified code value range described in Annex A of the CD 2085 draft.

Some prior information about this was discussed at the San Jose meeting, but at that time the draft of ST 2085 was at an earlier stage (although essentially the same in technical content).

In the discussion, there was a generally favorable reaction. It was suggested that since this is something being standardized by SMPTE, we should presume that we would want to support it.

Decision: Adopt (into SCC draft). The proposal for support in AVC should be considered by the parent bodies.

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control (10)

JCTVC-S0067 Combination of several encoder improvements for SCM2.0 [G. Laroche, C. Gisquet, T. Poirier, P. Onno (Canon)]

(Chaired by G. Sullivan on Sunday 10-19 a.m.)

This contribution gathers several improvements related to IBC, palette coding mode and the adaptive residual colour transform. It is reported that these proposed encoder changes provide a BDR gain of 2% over SCM2.0 while decreasing the encoding runtime to 76% for Intra configuration. It is also asserted that the resulting combination offers a better reference for evaluating the complexity/gain trade-off of normative proposals over SCM2.0.

There were three elements of the contribution:

- JCTVC-S0065, Non-CE2: Intra Block Copy encoder improvements for SCM2.0, which adds inter-like fast termination algorithms and checks some other possibilities (improving coding efficiency per category in the range of 0.0-5.8% and speeding up the encoder by 19%).

- JCTVC-S0066, Non-CE6: Palette encoder improvements for SCM2.0, which deals with palette encoder algorithm – for speed-up purposes. This aspect was reviewed in a BoG, which recommended no action.

- Moreover, the adaptive colour transform selection for IBC, inter and merge modes was modified. The basic idea is to skip some residual evaluation when the current tested mode is not the best – for speed-up purposes.

  - If RGB, RCT is tested first, and disabling RCT is only tested for a mode if the mode being tested is he best mode so far.

  - If YUV, a similar approach where disabling RCT is the default presumption.

The palette-related improvements for this proposal are identical to JCTVC-S0066. The other improvements are not related to palette (e.g. IBC-related).

It was asked how "clean" are the code changes for items 1 & 3.

It was remarked that knowing the separate impact of the changes would be desirable.

However, the benefit seems substantial.

Partial cross-check was available as S0261.

Decision (S/W): Adopt, items 1 & 3. (The proponent also said they would help improve the text description of the test model algorithms.)

See also notes on S0149.

JCTVC-S0282 Crosscheck of JCTVC-S0067 [F. Zou (Qualcomm)] [late]

Has "the" problem.

JCTVC-S0149 Fast intra coding mode decision for screen content coding [Y. Ahn, X. Wu, W. Lim, J. Ma, D. Sim (KWU)]

(Chaired by G. Sullivan on Sunday 10-19 p.m.)

In this contribution, an early termination of intra coding mode decision is proposed to reduce an encoding complexity of the screen content coding (SCC) extensions. Comparing to HEVC version 1, the computational complexity of SCC extensions has been increased due to newly added intra coding mode as intra block copy (IBC). To reduce complexity from additional mode decision in encoder, adjustment of decision order for intra coding modes and early termination based on coding information of IBC 2Nx2N mode are proposed in this contribution. The proposed method reportedly achieved about 10% reduction in encoding time with BD-bitrate benefit of 0.3% compared with that of SCC extensions test model 2.0 encoder in all intra (AI) case.

The decision order is modified to check IBC 2Nx2N before checking ordinary-intra 2Nx2N, and early termination checks are used.

S0065 and S0067 are related. The technique discussed in this contribution is included in S0067. See notes on that contribution.

JCTVC-S0089 Improvement for hash based inter search [B. Li, J. Xu (Microsoft)]

(Chaired by J. Boyce on Sunday 10-19 p.m.)

This document proposes non-normative improvement for hash based ME. With the proposed method, the encoding time is reduced by 7% for lossy RA coding and lossy LB coding, without significant impact on the coding efficiency.

The coding impact was minimal, but there was loss of up to 0.3%, and gains up to 0.1%. The encoder speed gains were sequence dependent.

In the current design, homogenous blocks are not added to the hash table. In the proposal, CU-aligned homogenous blocks are added. A participant questioned why to use CU-alignment position as a criterion. The proponent asserted that using CU-alignment was a way to minimize the additional memory required for the proposal.

Decision (SW): Adopt into the test model SCM. Will also need to update test model document.

JCTVC-S0236 Crosscheck of Improvement for Hash Based Inter Search (JCTVC-S0089) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-S0090 On referencing structure supporting temporal scalability [B. Li, J. Xu (Microsoft)]

(Chaired by J. Boyce on Sunday 10-19 p.m.)

This document proposes a modified configuration to support temporal scalability when the random access coding structure is used. Compared with RA anchor, which does not support temporal scalability, enabling temporal scalability with the modified configuration brings 0.1% performance loss on average. When compared with the current configuration supporting temporal scalability in SCM-2.0, i.e., applying the referencing structure provided in encoder_randomaccess_main_4tids.cfg to encoder_randomaccess_main_scc.cfg, about 0.3% performance gain is achieved by the modified configuration.

The contribution was proposing a change to common test conditions.

The proposal reduces the penalty to use temporal scalability for RA. The proposed referencing hierarchy does not support temporal nesting.

Decision (SW): Adopt the change to the configuration file for random access for the HM.  Also fix the bug in the SCM that was identified.

Further discussion on 10-23 (GJS) in the JCT plenary discussed whether the SCM RA common conditions should be changed to support temporal scalability.

It was commented that SCM ordinarily tracks HM behaviour. There was no objection to that.

JCTVC-S0243 Cross-check of S0090 (On referencing structure supporting temporal scalability) [W. Wang, M. Xu, Z. Ma, H. Yu (Huawei USA R&D)] [late]

JCTVC-S0198 Conversion tools update [B. Mandel (Universal), C. Fogg (MovieLabs)] [late]

Discussed 10-23 p.m. (GJS).

This information document provides notice of recent updates to conversion tools that feature scripts written in ctlrender (Color Transform Language) by the Academy of Motion Pictures Arts and Sciences (AMPAS) that performs video conversion tasks, in particular High Dynamic Range (HDR) & Wide Color Gamut (WCG) signals. New scripts added since the Sapporo meeting in July 2014 utilize ctlrender’s internal tone mapping algorithm to render Standard Dynamic Range /Standard Color Gamut (SDR/SCG) signals, conforming to Bt.709, from larger color volumes in higher bitdepth HDR/WCG signal containers such as PQ (SMPTE ST 2084) / BT.2020.

This was an information document to inform participants of the availability of these software tools. It was presented only briefly, but is available for study.

JCTVC-S0206 On lambda-domain Rate Control [J. Wen, M. Fang, M. Tang (Tsinghua Univ.)] [late]

Discussed 10-23 p.m. (GJS).

This contribution reports several problems in the R-lambda model based rate control which was proposed in JCTVC-K0103. It is noted that R-lambda model based rate control fails to achieve the expected performance in some cases and the model may provide an inaccurate lambda without the clipping is applied, although it performs well on the test sequences.

The presentation was requested to be uploaded.

The contribution asserts that the model does not seem to be working well for some test clips. It suggests that further study be performed to improve upon the current behaviour – which actually reported a negative correlation between the lambda predicted by the model and that which should have been applied. Further experiments would be needed to confirm/determine what needs to be done. Further study was encouraged.

# Withdrawn (8)

JCTVC-S0041 Withdrawn

JCTVC-S0093 Withdrawn

JCTVC-S0109 Withdrawn

JCTVC-S0182 Withdrawn

JCTVC-S0116 Withdrawn

JCTVC-S0184 Withdrawn

JCTVC-S0185 Withdrawn

JCTVC-S0193 Withdrawn

JCTVC-S0223 Withdrawn

JCTVC-S0299 Withdrawn

# Plenary Discussions and BoG Reports

## General

Discussion JCT-VC plenary Sat. 16:00

Establish WD on SCC including

- IBC full-frame with new vector prediction (R0309) and binarization (R0186)

- Adaptive color transform (R0147)

Discussion JCT-VC plenary 07-08 12:00

WD on SCC to include R0348 Palette mode.

## Project development

Joint meetings are discussed in this section of this report.

Joint session with VCEG & MPEG parent-level and JCT-VC and JCT-3V, chaired by JRO, GJS, & JO Monday 10-20 1600-1800:

Non-4:4:4 for SCC? (e.g., JCTVC-S0042)

- The contribution does not request a 4:2:0 profile, but requests the capability to be present in the 4:4:4-capable profile so that encoders can choose to use that.

- Some participants indicated that if we think 4:2:0 is important, we should go ahead and define a profile for it.

- Agreed: It was suggested to consider 4:4:4 the primary requirement, but try to also consider the potential application of the same coding tools for 4:2:0.

- Decision: Adding new 4:2:0 coding capabilities would be adding some complexity to 4:4:4 decoders, but could be considered if the added complexity is not excessive and the benefit is significant – consider the complexity/benefit tradeoff. Profiling implications TBD.

(Outside of JCT-VC: JCT3V-J0106 Profiles, tiers, levels for 3D-HEVC – proposing one profile, very similar to MV-HEVC, with nested MV-HEVC decoding capability. Decision: Adopt (possibly with refinement of low-level aspects), with nested capability.)

SEI & VUI & CICP

- JCTVC-S0148 End-of-picture indication for AVC & HEVC – OK.

- JCTVC-S0031 / JCT3V-J0108 Frame packing for video with depth (and relationship with depth and texture view packing SEI message in AVC) – to be considered in JCT-3V – only interest if there's a significant benefit. JCT3V-J0109 is new proposed draft text, to be reviewed in JCT-3V to determine adequacy of editorial quality for proceeding with the ongoing work together with MFC+D.

- JCTVC-S0197 VUI codepoint for SMPTE ST 2085 (YDzDx) for AVC & HEVC – proceed.

- ST 2084 and 2086 for AVC – proceed.

- m35153 P3 colour primaries in CICP ST 428-2 or maybe -1 – OK to support SMPTE specs.
[Note also JEDEC P22 – TBD]

- Green metadata – SEI message referring to another standard for the details.

Unlimited level / unconstrained profile for AVC & HEVC – still plan to do, but no draft yet

(Future video coding exploration (WCG/HDR, compression improvement (workshop), other))

Joint session of JCT-VC and JCT-3V, chaired by JRO Tueday 10-21 1000-1100:

(See additional notes in JCT-3V report.)

- Reference software

  - It is desirable, and seems practical, for SHM to be able to decode MV-HEVC.

  - It is more essential for 3D-HEVC to be based on MV-HEVC RS codebase.

- Conformance

  - Status of conformance testing development

  - Feasibility of joint development of MV-HEVC and SHVC conformance testing was discussed

  - Coordination of the formatting of data and additional information output as a log file to be produced by the decoder to determine how the decoder is handling the pictures in the bitstream and the output layer set information

## BoGs

JCTVC-S0292 BoG report on CE6 improvements of palette mode [R. Cohen, Y.-W. Huang]

JCTVC-S0294 BoG report on complexity assessment of IBC block vector coding [C. Pang, S. Liu]

(Reviewed 10-23 a.m., JRO & GJS)

A BoG was held on Saturday October 18, 2014, 9:00 am to 11:15 am to provide more detailed complexity assessment of IBC vector coding. The agreed mandates of the BoG were:

- Analyze the worst case number of context coded bins in IBC for the current SCM and the methods CE1 Test 1.1 (JCTVC-S0142) and Test 1.4 (JCTVC-S0162). Compare against current 4x8 MV coding, including considerations about reference index and mode coding.

- Discuss whether invoking merge at 4x4 blocks could cause some complexity problems.

Item 1, intra case:

|  | Complexity |  |  |
| --- | --- | --- | --- |
|  | CE1 Test 1.1 | CE1 Test 1.4 | SCM 2.0 |
| # regular bins per 4x4 | 4 | 14 | 2 |
| # contexts per 4x4 | 4 | 6 | 2 |

|  | Complexity |  |  |  |
| --- | --- | --- | --- | --- |
|  | CE1 Test 1.1 | CE1 Test 1.4 | SCM 2.0 | Inter |
| # regular bins per 8x8 CU | 490 | 530 | 482 | 488 |
| % compared with SCM 2.0 | +1.6% | +10.0% | x | x |
| % compared with Inter | +0.4% | +8.6% | x | x |

Item 1, inter case

|  | Complexity |  |  |
| --- | --- | --- | --- |
|  | CE1 Test 2.1 | CE1 Test 2.2 | Inter |
| # regular bins per 8x8 CU | 488 | 508 | 488 |
| % compared with Inter | The same. | +4.0% | x |

An expert commented that the performance might be dependent on the ME method used for IBC. A couple of experts also ask for additional results with the bits estimation as Inter.

In the following table, the results of Test 3.1 (Test 1.1 + Test 2.1) and Test 3.3 (Test 1.4 + Test 2.2) with local search (no hash-based full frame search) and the bit estimation method as Inter in HM.

*: Use local search instead of full frame search.

**: Use bit estimation method as Inter in HM.

The following results were initially reported:

Coding performance for block vector coding of CE1 Test 3.1 and Test 3.3 reported in CE 1

|  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 | 3.1* | 3.3* | 3.1** | 3.3** |
| RGB, text & graphics with motion, 1080p | -0.3% | -0.4% | -0.1% | x | -0.3% | x |
| RGB, text & graphics with motion,720p | -0.2% | -0.4% | -0.1% | x | -0.2% | x |
| RGB, mixed content, 1440p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| RGB, mixed content, 1080p | -0.1% | -0.1% | 0.0% | x | -0.1% | x |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| YUV, text & graphics with motion, 1080p | -0.3% | -0.5% | -0.1% | x | -0.3% | x |
| YUV, text & graphics with motion,720p | -0.2% | -0.4% | -0.1% | x | -0.2% | x |
| YUV, mixed content, 1440p | -0.1% | -0.1% | -0.1% | x | -0.1% | x |
| YUV, mixed content, 1080p | 0.0% | -0.1% | 0.0% | x | 0.0% | x |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | x | 0.0% | x |
| Enc Time[%] | 97% | 99% | 103% | x | 87% | x |
| Dec Time[%] | 100% | 101% | 100% | x | 100% | x |

|  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 | 3.1* | 3.3* | 3.1** | 3.3** |
| RGB, text & graphics with motion, 1080p | -2.4% | -2.7% | -1.8% | x | -2.2% | x |
| RGB, text & graphics with motion,720p | -0.6% | -0.9% | -0.5% | x | -0.4% | x |
| RGB, mixed content, 1440p | -0.1% | -0.2% | -0.1% | x | 0.0% | x |
| RGB, mixed content, 1080p | -0.1% | -0.1% | -0.2% | x | 0.0% | x |
| RGB, Animation, 720p | -0.1% | -0.3% | -0.1% | x | 0.2% | x |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.0% | x | 0.3% | x |
| YUV, text & graphics with motion, 1080p | -2.7% | -3.0% | -2.2% | x | -2.5% | x |
| YUV, text & graphics with motion,720p | -0.7% | -0.9% | -0.5% | x | -0.6% | x |
| YUV, mixed content, 1440p | 0.0% | -0.1% | -0.2% | x | 0.0% | x |
| YUV, mixed content, 1080p | -0.1% | -0.1% | -0.1% | x | 0.0% | x |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% | x | 0.3% | x |
| YUV, camera captured, 1080p | 0.0% | -0.1% | 0.0% | x | 0.3% | x |
| Enc Time[%] | 100% | 95% | 95% | x | 80% | x |
| Dec Time[%] | 100% | 100% | 100% | x | 100% | x |

|  | LD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 3.1 | 3.3 | 3.1* | 3.3* | 3.1** | 3.3** |
| RGB, text & graphics with motion, 1080p | -3.1% | -3.4% | -2.4% | x | -2.7% | x |
| RGB, text & graphics with motion,720p | -1.2% | -1.5% | -1.1% | x | -0.8% | x |
| RGB, mixed content, 1440p | -0.2% | -0.2% | -0.2% | x | 0.1% | x |
| RGB, mixed content, 1080p | -0.6% | -0.3% | -0.7% | x | -0.3% | x |
| RGB, Animation, 720p | 0.0% | -0.1% | -0.1% | x | 0.3% | x |
| RGB, camera captured, 1080p | 0.0% | -0.2% | 0.0% | x | 0.3% | x |
| YUV, text & graphics with motion, 1080p | -3.4% | -3.9% | -2.8% | x | -3.1% | x |
| YUV, text & graphics with motion,720p | -1.4% | -1.8% | -1.6% | x | -1.1% | x |
| YUV, mixed content, 1440p | -0.3% | -0.4% | -0.2% | x | -0.2% | x |
| YUV, mixed content, 1080p | -0.4% | -0.1% | -0.6% | x | -0.2% | x |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.1% | x | 0.4% | x |
| YUV, camera captured, 1080p | 0.1% | -0.1% | 0.1% | x | 0.3% | x |
| Enc Time[%] | 87% | 94% | 90% | x | 83% | x |
| Dec Time[%] | 100% | 99% | 100% | x | 100% | x |

About item 2:

An expert commented that when using 4x4 merge, parallel processing cannot be performed at encoder and decoder. Need additional merge candidate list construction.

An expert commented that what is the coding performance improvement from encoder only changes instead of using merge. Additional results are requested. This is a general question for whether to bring in merge mode for IBC. In addition, what is the gain from IBC 4x4. Another expert also requests the performance improvement from IBC 4x4 merge.

Coding performance of enabling full RD check at the current BVP candidate positions (JCTVC-S0087)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | -2.6% | -2.6% | -2.6% | -2.4% | -2.2% | -2.3% | -2.4% | -2.1% | -2.1% |
| RGB, text & graphics with motion,720p | -0.9% | -1.0% | -0.9% | -0.6% | -0.8% | -0.7% | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p | -1.3% | -1.2% | -1.4% | -0.8% | -0.9% | -1.1% | -0.7% | -0.7% | -0.8% |
| RGB, mixed content, 1080p | -1.1% | -1.1% | -1.1% | -0.8% | -1.0% | -1.2% | -0.6% | -0.1% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p | -0.6% | -0.7% | -0.7% | -2.1% | -2.1% | -2.1% | -2.3% | -2.2% | -2.1% |
| YUV, text & graphics with motion,720p | -0.5% | -0.5% | -0.5% | -0.3% | -0.4% | -0.3% | -0.5% | -0.2% | -0.3% |
| YUV, mixed content, 1440p | -0.7% | -0.8% | -0.9% | -0.7% | -0.7% | -0.8% | -0.7% | -0.3% | -0.3% |
| YUV, mixed content, 1080p | -0.6% | -0.6% | -0.7% | -0.6% | -0.8% | -0.9% | -0.7% | 0.0% | -0.7% |
| YUV, Animation, 720p | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | 0.0% | 0.0% | -0.2% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 122% |  |  | 109% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  | 103% |  |  |

Coding performance of disabling 4x4

|  | AI (SCM2.0) |  |  | AI (JCTVC-S0065) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p | 1.0% | 1.0% | 0.9% | 1.8% | 1.8% | 1.7% |
| RGB, text & graphics with motion,720p | 0.6% | 0.4% | 0.4% | 0.9% | 0.8% | 0.8% |
| RGB, mixed content, 1440p | 0.5% | 0.4% | 0.3% | 0.8% | 0.7% | 0.6% |
| RGB, mixed content, 1080p | 0.9% | 0.6% | 0.6% | 1.3% | 1.0% | 1.0% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p | 0.9% | 0.8% | 0.8% | 1.8% | 1.6% | 1.6% |
| YUV, text & graphics with motion,720p | 0.6% | 0.5% | 0.5% | 0.8% | 0.7% | 0.6% |
| YUV, mixed content, 1440p | 0.4% | 0.2% | 0.3% | 0.7% | 0.6% | 0.6% |
| YUV, mixed content, 1080p | 0.8% | 0.6% | 0.5% | 1.0% | 0.7% | 0.9% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

An expert has concern regarding the memory bandwidth for 4x4. Another experts commented that as there is no interpolation for IBC, there should be no issue for the memory bandwidth for 4x4.

Recommendation of the BoG: If the Inter MV coding method is changed, it is better to have a unified solution for IBC and Inter.

Follow-up discussion in JCT-VC, Thu 10-23 am:

- It is generally desirable to seek more harmonization between inter and IBC (as e.g. proposed in various CE2 and related methods, and in JCTVC-S0302)

- One single method of binarization for BV/MV coding would be desirable

- In no case, three binarizations should be enforced (old MV coding, modified MV for SC, different BV coding)

- The worst case number of context coded bins should not increase

- Some of the gain reported by the new methods of MV binarization may be related to the preference for integer motion vectors, and may no longer apply after adoption of JCTVC-S0085.

- Method 1.4 would not comply with at least one of the criteria above

Discontinue CE1, further investigate method 1.1 in the context of CE2 (as also suggested as one option in S0302)

More understanding necessary about “encoder only” merge mode versus explicit merge mode (as already expressed somewhere under CE2 related docs)

JCTVC-S0304 BoG report on Adaptive Color Transform (ACT) J. Boyce

(Reviewed 10-23 a.m., JRO)

The BoG met 22 Oct 2014 to review previously unreviewed contributions related to Adaptive Color Transform.

The recommendations of the BoG were agreed except as otherwise noted.

The BoG recommended adoption of the following:

- JCTVC-S0140 option b), clipping of negative QPs to zero to address the QP underflow issue. Also proposed in JCTVC-S0144 and JCTVC-S0086.

- JCTVC-S0180, addition of shift in the ACT to align color components if they have different bit depths. Disallow the use of ACT for trans-quant-bypass CUs when bit depths differ for colour components.

- JCTVC-S0254, unification of the ACT between lossy and lossless (trans-quant-bypass), to use only the lossless transform.

Decision: Confirmed by JCT-VC.

The BoG recommended SW adoption of the following:

- JCTVC-S0179, first proposal to perform ACT operation on TU blocks. (No change to CU-based signaling.) It is also delegated to the SCC editors to consider aligning the draft text accordingly.

Decision (SW/Ed.): Confirmed by JCT-VC.

The BoG suggested track discussion about the IBC encoder algorithms related to early termination, and interaction with ACT, as raised in JCTVC-S0100.

JCTVC-S0308 BoG report on Intra String Copy (CE10) [Y. Chen]

Chaired by J. Boyce, Thursday

This document provides a report of the BoG on CE 10 proposals, focusing on the memory bandwidth analysis and CABAC throughput, from 6 pm to 6:45 pm, Oct. 22.

Some worst-case results were provided. Intra string copy requires twice the memory bandwidth as Intra line copy and 3-4 times the memory bandwidth as compared to Intra BC (4x4), even when large memory access pattern is used.

Worst case CABAC bins per pixel for 4x8 in SCM 2.0 is 7.6, for Intra string copy is 3.06, and for Intra String copy Test 6.1 is 15.06, roughly twice the 4x8 in SCM 2.0 case.

It was suggested that the memory bandwidth could be limited by restricting the sizes of the strings.  String size constraints could be based upon the relative prediction position.

It was agreed to have a CE on ISC.

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the Tuesday of the week preceding the meeting (7 Oct.).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs (to be updated)

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-M1100.

A deadline of three weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. SHM, HM, or HM+RExt). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

CE plan final at same time as corresponding software except for SCE1 & 4 due to test sequence issues.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-K11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

  - List of CEs in SCC

- CE1: IBC vector entropy coding and block vector derivation (R0074, R0088, R0123, R0181, R0133, R0134, R0165) coordinator: C. Pang [Note from review in plenary 07-08: Move R0165 to CE2]

- CE2: IBC signalling and partitioning (R0100 also tested against SCM with PU-level ibc_flag from 204, and if possible with AMP (note: R0241 implemented AMP but with CU-level signalling), to be further refined also with merge R0097, R0356 also relates to pure PU level signalling) coordinator: J. Xu

- CE3: IBC sub-block partitioning and flipping (R0204, R0050, R0117) coordinator: S. Liu

- CE4: Intra line copy (R0132, R0281) coordinator: C. C. Chen

- CE5: Global variation of palette size and palette predictor size (R0166, R0176, R0215) coordinator: R. Joshi

- CE6: Improvement of palette signalling with sub-categories run coding for palette (R0174, R0136, R0085, R0045, R0076), index/escape coding (R0113, R0135, R0075, R0170, R0066), transition copy (R0120, R0084) and predictor signalling (R0228) coordinator: Y.-W. Huang

- CE7: String matching for palette index coding (R0268, R0304) coordinator: Z. Ma

- CE8: Uni- and bi-color mode (R0112, R0198, R0200) coordinator: R. Cohen

- CE9: IBF/CCP interdependency (R0114, R0219, R0314) coordinator: R. Cohen

- CE10: Intra string copy (R0060, R0098, R0137, R0140, R0145, R0255) coordinator: Y. Chen, J. Xu

Note: In the context of CE10 presentation from the BoG, it was remarked that a restriction of number of runs per sequence is not appropriate. To enable real-time decoding, memory accesses and number of context coded bins have to be restricted per picture at least, and even at finer granular level (CTU, row of CTU) for applications requiring sub-picture delay (e.g. instantaneous display).

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC.

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 12.0 and SHM 2.0 should be available within 2 weeks after the meeting. [To be fixed]

- HM 12.0+RExt should be available within 1 week after HM 12.0 availability.

In joint discussion on 07-05, it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and strongly suggested that this should be done.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

[Ed. Add/fix coordination aspects as relevant.]

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) Coordinate overall JCT-VC interim efforts. Report on project status to JCT-VC reflector. Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) Produce and finalize JCTVC-S1002 HEVC Test Model 16 (HM 16) Improved Encoder Description, including merging of the RExt and prior HM test model descriptions Collect reports of errata for HEVC Gather and address comments for refinement of these documents. Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, K. McCann C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) Coordinate development of the HM software and its distribution. Produce documentation of software usage for distribution with the software. Prepare and deliver HM 16.x software versions and the reference configuration encodings according to JCTVC-L1100 and JCTVC-P1006 common conditions. Suggest configuration files for additional testing of tools. Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), D. Flynn, K. Sharman (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) Study the requirements of HEVC conformance testing to ensure interoperability. Prepare and deliver the [relevant output docs] Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt and SHVC conformance testing. Establish and coordinate bitstream exchange activities for HEVC. Identify needs for HEVC conformance bitstreams with particular characteristics. Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, K. Kazui, A. K. Ramasubramonian, W. Wan, Y. Ye (vice-chairs) | N |
| Verification test preparation (AHG5) (jct-vc@lists.rwth-aachen.de) Make preparations for verification testing of HEVC for interlaced video content. Make preparations for verification testing of HEVC range extensions. | V. Baroncini (chair), M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, T. K. Tan, J.-M. Thiesse, W. Wan (vice-chairs) | N |
| SCC coding performance analysis (AHG6) (jct-vc@lists.rwth-aachen.de) Study test conditions and coding performance analysis methods for SCC coding performance Analyze coding performance of draft and proposed SCC coding features | H. Yu (chair), R. Cohen, A. Duenas, S. Liu, K. Rapaka, J. Xu (vice-chairs) | N |
| SCC extensions text editing (AHG7) (jct-vc@lists.rwth-aachen.de) Produce and finalize HEVC screen content coding extensions working draft 1 and test model 2 text. Gather and address comments for refinement of the test model. Coordinate with AHG8 to address issues relating to mismatches between software and text. | J. Xu, R. Joshi (co-chairs), R. Cohen, S. Liu, Z. Ma, G. Sullivan, Y. Ye (vice-chairs) | N |
| SCC extensions software development (AHG8) (jct-vc@lists.rwth-aachen.de) Coordinate development of the HM SCM software and its distribution. Prepare and deliver HM 16.x-SCM-3.0 software version and the reference configuration encodings according to JCTVC-S1015. Prepare and deliver additional "dot" version software releases and software branches as appropriate. Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. Suggest configuration files for additional testing of tools. Coordinate with AHG7 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu, M. Xu (vice-chairs) | N |
| Complexity of palette mode coding (AHG9) (jct-vc@lists.rwth-aachen.de) Analyze complexity characteristics of proposed palette mode methods with regards to throughput, amount of memory, memory bandwidth, parsing dependencies, parallelism, pixel processing, and other aspects of complexity as appropriate. Quantify the average and worse case throughput (context-coded as well as bypass bins) for palette mode operation and compare it with the average and worse case throughput for other coding modes. Analyze and identify the complexity implications of interleaved and non-interleaved escape colour value signalling. Study latency implications of palette mode coding. Identify criteria to determine the hardware implementability of the key hardware modules. Identify bottlenecks in the current design with regard to implementation complexity. | A. Duenas (chair), R. Joshi, S.-H. Kim, W. Wang, X. Xiu (vice-chairs) | N |
| Complexity of IBC, intra line & intra string copy coding (AHG10) (jct-vc@lists.rwth-aachen.de) Analyze complexity characteristics of f IBC, intra line & intra string copy methods with regards to throughput, amount of memory, memory bandwidth, parsing dependencies, pixel processing. Analyze the complexity impact of the search area size on the design. Quantify the average and worst-case throughput (e.g., in context-coded bins) for these methods and compare them with the average and worst-case throughput for other coding elements. Identify criteria to determine the hardware implementability of the key elements. Identify bottlenecks in the current design with regard to implementation complexity. | J. Sole (chair), S. Liu, J. Xu (vice-chairs) | N |
| SHVC test model editing (AHG11) (jct-vc@lists.rwth-aachen.de) Produce and finalize JCTVC-S1007 SHVC Test Model 8 (SHM 8) text. Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) Prepare SHM 8.0 software (based on HM 16 if feasible) for experimentation. Generate anchors and templates based on common test conditions. Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, (co-chairs) | N |
| Loop filtering for SCC (AHG13) (jct-vc@lists.rwth-aachen.de) Identify how screen content coding quality is affected by existing in-loop filtering processes. Study the interaction of deblocking and SAO filtering with coding tools for screen content coding. Collect and study proposals on modified in-loop filtering, and evaluate the subjective and objective impact on coding performance. Analyze the complexity impact of in-loop filtering processes. | C. Rosewarne and L. Zhang (co-chairs), X. Xu (vice-chair) | N |
| Parallel processing for SCC (AHG14) (jct-vc@lists.rwth-aachen.de) Study the implications of SCC tools on parallelism, considering both single-core and multi-core architectures. Study implication of dependencies from previously decoded samples of the same picture on parallel processing tools such as tiles, wavefronts, etc. Identify and discuss additional issues relating to parallel processing capabilities of SCC tools. | K. Rapaka (chair), A. Duenas, S. Liu, S.-H. Kim (vice-chairs) | N |
| Test sequence material (AHG15) (jct-vc@lists.rwth-aachen.de) Maintain the video sequence test material database for HEVC development. Identify, collect, and make available a variety of video sequence test material. Study coding performance and characteristics in relation to video test materials. Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. Coordinate with the activities in AHG5 regarding interlaced video and range extensions development, and AHG6 regarding screen content coding. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), T. K. Tan, S. Wenger (vice-chairs) | N |

# Output documents (needs some updating)

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

+Verif test plan for RExt and interlaced HEVC

JCTVC-S1000 Meeting Report of 19th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2014-10-03] (near next meeting)

Remains valid – not re-issued: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-S1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Improved Encoder Description [K. McCann and C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 14703) [2014-10-10] (near next meeting)

JCTVC-S1003 Draft verification test plan for interlaced video and format range extensions [C. Rosewarne, A. Tourapis, G. Barroux, M. Naccari]

JCTVC-S1004 HEVC Version 1 Conformance Testing Defect Report [T. Suzuki, W. Wan, G. J. Sullivan (editors)] (WG 11 Nxxxx) [2014-08-15] (5 weeks)

JCTVC-S1005 HEVC Screen Content Coding Draft Text 2 [R. Joshi, J. Xu (editors)] (WG 11 N 1xxxx) [2014-08-15] (5 weeks)

SCC WD output:

- IBC

- Adaptive colour transform

- Palette mode

- Adaptive MV resolution

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)]

JCTVC-S1007 SHVC Test Model 8 (SHM 8) Introduction and Encoder Description [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela (editors)] (WG 11 N 14705) [2014-10-10] (near next meeting)

JCTVC-S1008 SHVC Conformance Testing Draft 1 [J. Boyce, A. K. Ramasubramonian]

Remains valid – not updated JCTVC-Q1009 Common SHM Test Conditions and Software Reference Configurations [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

JCTVC-R1011 HEVC Reference Software for Format Range Extensions Profiles  [F. Bossen, D. Flynn, K. Sühring, T. Suzuki (editors)] (WG 11 N 14709 ISO/IEC FDIS & ITU-T Last Call) [2014-07-25] (2 weeks)

JCTVC-R1012 HEVC Range Extensions Conformance Testing Draft 3 (WG 11 N 14713) [T. Suzuki, K. Kazui (editors)] [2014-09-12] (2 months)

JCTVC-R1013 No document

JCTVC-R1014 Screen Content Coding Test Model 3 Encoder Description (SCM 3) [R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye (editors)] (WG 11 N 14706) [2014-09-12] (2 months)

JCTVC-R1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2014-08-01] (3 weeks)

Agreed (GJS) Thu 10-23:

CTC changes: Adding 4:2:0, changing the reporting template categorization, changing sign for lossless reporting, separate template for 4:2:0.

SCM 3 software release time = 3 weeks.

CE description finalization time = basis software availability + 1 week.

CE software availability = basis software availability + 2 or 3 weeks, specific to the CE.

Generally, CE configurations should be harmonized with each other to the extent feasible.

Remains valid – not re-issued: JCTVC-L1100 Common Test Conditions and Software Reference Configurations for HM [F. Bossen (editor)]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting, such plans were not considered binding on final CE plans as reviewed in the closing plenary.

JCTVC-R1101 Description of Core Experiment 1 (CE1): Palette Mode Improvement [P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma (CE coordinators)] [2014-11-21] (4 weeks)

JCTVC-R1102 Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding [J. Xu, S. Liu, C. Pang, X. Xiu (CE coordinators)] [2014-11-21] (4 weeks)

6 parts (number of sub-parts each: 1, 1, 2, 2, 3, 1)

JCTVC-R1103 Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy [C.-C. Chen, Y. Chen, J. Xu, T. Lin, W. Wang (CE coordinators)] [2014-11-21] (4 weeks)

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting).

Some specific future meeting plans were established as follows:

- 10–18 Feb. 2015 20th meeting under ITU-T auspices in Geneva, CH.

- 19–26 June 2015 21st meeting under WG 11 auspices in Warsaw, PL.

- 13–21 Oct. 2015 22nd meeting under ITU-T auspices in Lucca, IT.

- …

The agreed document deadline for the 19th JCT-VC meeting is Tues. 7 Oct. Plans for scheduling of agenda items within that meeting remain TBA.

ITU-T SG 16 and XXX were thanked for the excellent hosting of the 18th meeting of the JCT-VC.

The JCT-VC meeting was closed at approximately 1300 hours on Wed. 9 July 2014.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the eighteenth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting (approximately XXX people in total), were as follows: