| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U_Notes_d2 |
| --- | --- |

| Title: | Meeting report of the 21st meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Warsaw, PL, 19–26 June 2015 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twentyfirst meeting during 19–26 June 2015 at the Mariott Hotel Warsaw in Warszawa, PL. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 19 June 2015. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 June 2015. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11– one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

One primary goal of the meeting was to review the work that was performed in the interim period since the twentieth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 2;

- Verification test plan for format range extensions and interlaced video;

- The RExt reference software draft 2 and conformance testing draft 4, the latter combined with improved version 1 conformance testing;

- The SHVC reference software draft 1, conformance testing draft 2 and SHM test model 9;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 4, SCC draft text 3, and a document specifying common test conditions and software reference configurations for SCC experiments.

The other most important goals were to review the results from two Core Experiments on Screen Content Coding (CE1–2), and review other technical input documents. Reviewing the progress made towards definition of screen content coding tools was the most important topic of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Further preparation of verification tests was conducted. Possible needs for corrections to version 2 were also considered.

In addition to X new experiment plan descriptions in screen content coding, the JCT-VC produced X other particularly important output documents from the meeting (update):

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 2;

- Verification test plan for format range extensions and interlaced video;

- The RExt reference software draft 2 and conformance testing draft 4, the latter combined with improved version 1 conformance testing;

- The SHVC reference software draft 1, conformance testing draft 2 and SHM test model 9;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 4, SCC draft text 3, and a document specifying common test conditions and software reference configurations for SCC experiments.

For the organization and planning of its future work, the JCT-VC established XX "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings are planned for Wed. 13 – Wed. 21 Oct. 2015 under ITU-T auspices in Lucca, IT, Fri. 19 – Fri. 26 Feb. 2016 under WG 11 auspices in San Diego, US, Wed. 25 May – Wed. 1 June 2016 under ITU-T auspices in Geneva, CH, and … .

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://mailman.rwth-aachen.de/mailman/listinfo/jct-vc.

- Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29/ WG 11 held its twentyfirst meeting during 19–26 June 2015 at the Marriott Hotel Warsaw in Warszawa, PL. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

## Meeting logistics

The JCT-VC meeting sessions began at approximately 0900 hours on Friday 19 June 2015. Meeting sessions were held on all days (including weekend days) until the meeting was closed at approximately XXXX hours on Friday 26 June 2015. Approximately XXX people attended the JCT-VC meeting, and approximately XXX input documents were discussed. The meeting took place in a collocated fashion with a meeting of WG11– one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Torino, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warszawa, 2015-06)		XXX people, XXX input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2015_06_U_Warsaw/ .

## Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the nineteenth JCT-VC meeting in producing:

- The HEVC test model (HM) 16 improved encoder description (including RExt modifications) update 2;

- Verification test plan for format range extensions and interlaced video;

- The RExt reference software draft 2 and conformance testing draft 4, the latter combined with improved version 1 conformance testing;

- The SHVC reference software draft 1, conformance testing draft 2 and SHM test model 9;

- For HEVC screen content coding (SCC) extensions, the HEVC screen content coding test model 4, SCC draft text 3, and a document specifying common test conditions and software reference configurations for SCC experiments.

The other most important goals were to review the results from two Core Experiments on Screen Content Coding (CE1–2), and review other technical input documents. Reviewing the progress made towards definition of screen content coding tools was the most important topic of the meeting. Advancing the work on development of conformance and reference software for recently finalized HEVC extensions (RExt, SHVC) is also a significant goal. Further preparation of verification tests was conducted. Possible needs for corrections to version 2 were also considered.

## Documents and document handling considerations

### General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report:

- Decisions made by the group that affect the normative content of the draft standard are identified in this report by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also circulated publicly by ftp during the meeting on a daily basis. Considering the high workload of this meeting and the large number of contributions, it should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much discussion of the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 9 June 2015.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 10 June 2015 were considered "officially late".

Most documents in the "late" category were CE reports or cross-verification reports, which are somewhat less problematic than late proposals for new action (and especially for new normative standardization action).

At this meeting, we again had a substantial amount of late document activity, but in general the early document deadline gave a significantly better chance for thorough study of documents that were delivered in a timely fashion. The group strived to be conservative when discussing and considering the content of late documents, although no objections were raised regarding allowing some discussion in such cases.

All contribution documents with registration numbers JCTVC-U0121 and higher were registered after the "officially late" deadline (and therefore were also uploaded late). However, some documents in the "U0121+" range include break-out activity reports that were generated during the meeting, and are therefore better considered as report documents rather than as late contributions.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following other technical design proposal contributions were registered on time but were uploaded late:

- JCTVC-U0xxx (a proposal on XXX) [uploaded 06-XX]

- …

The following other documents not proposing normative technical content were registered on time but were uploaded late:

- JCTVC-U0xxx (a document … ) [uploaded 06-XX]

- …

The following cross-verification reports were registered on time but were uploaded late: JCTVC-U0xxx [uploaded 06-xx], … .

The following contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JCTVC-U0xxx, … .

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting are not included in the above list, as these are considered administrative report documents to which the uploading deadline is not applied.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision.

"Placeholder" contribution documents that were basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, were considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting.

The following case did not occur at the 21st meeting: The initial uploads of the following contribution documents were rejected as a "placeholders" without any significant content and were not corrected until after the upload deadline:

- JCTVC-U0XXX (a proposal on … , corrected by a late upload on 06-XX)

- …

A few contributions had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors were noticed in other initial document uploads (wrong document numbers in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Measures to facilitate the consideration of contributions

It was agreed that, due to the continuingly high workload for this meeting, the group would try to rely extensively on summary CE reports. For other contributions, it was agreed that generally presentations should not exceed 5 minutes to achieve a basic understanding of a proposal – with further review only if requested by the group. For cross-verification contributions, it was agreed that the group would ordinarily only review cross-checks for proposals that appear promising.

When considering cross-check contributions, it was agreed that, to the extent feasible, the following data should be collected:

- Subject (including document number).

- Whether common conditions were followed.

- Whether the results are complete.

- Whether the results match those reported by the contributor (within reasonable limits, such as minor compiler/platform differences).

- Whether the contributor studied the algorithm and software closely and has demonstrated adequate knowledge of the technology.

- Whether the contributor independently implemented the proposed technology feature, or at least compiled the software themselves.

- Any special comments and observations made by a cross-check contributor.

### Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-T1000, the improved HEVC Test Model 16 (HM16) JCTVC-T1002, the Verification Test Plan for RExt and interlaced coding with version 1, JCTVC-T1003, the RExt Reference Software Draft 2 JCTVC-T1011, the RExt Conformance Testing Draft 4 (including improved Version 1 Conformance Testing) JCTVC-T1012, the SHVC test model 9 (SHM9) JCTVC-T1007, the SHVC Conformance Testing Draft 2 JCTVC-T1008, the SHVC Reference Software Draft 1 JCTVC-T1013, the Screen Content Coding (SCC) Draft Text 3 JCTVC-T1005, the SCC test model 4 JCTVC-T1014, and the common test conditions for SCC (JCTVC-T1015) were approved. The HM reference software and its extensions for RExt, SHVC and SCC were also approved.

The group had initially been asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, in regard to software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Also, there should be time to study combinations of different adopted tools with more detail prior to adoption.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology.

Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work.

## Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

## Agenda

The agenda for the meeting was as follows:

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Reports of Core Experiment activities

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of technology proposal contributions

- Consideration of information contributions

- Coordination activities

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments, establishment of AHGs, meeting planning, refinement of expected standardization timeline, other planning issues

- Other business as appropriate for consideration

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with preceding sentence declaring that contributor or third party rights are not granted, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft (current version JCTVC-M1010), the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

## Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

JCT-VC email lists are managed through the site https://mailman.rwth-aachen.de/mailman/options/jct-vc, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the reflector. CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by the original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the case of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

## Terminology

Some terminology used in this report is explained below:

- ACT: Adaptive colour transform

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See IBC.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE).

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- EL: Enhancement layer.

- ET: Encoding time.

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- IBC (also Intra BC): Intra block copy – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit depth (8 bits per sample) source video is encoded using higher bit depth signal processing, ordinarily including higher bit depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures

- MANE: Media-aware network elements.

- MC: Motion compensation.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector.

- OLS: Output layer set

- NAL: Network abstraction layer (as in AVC and HEVC).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation.

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- ROT: Rotation operation for low-frequency transform coefficients.

- RPS: Reference picture set

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment.

- SCCE: Screen content core experiment.

- SCM: Screen coding model.

- SD: Slice data; alternatively, standard-definition.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design between the 1st and 2nd or 2nd and 3rd JCT-VC meeting, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meeting.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion mixed with a relatively small amount of camera-captured content.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WD: Working draft – a term for a draft standard that may sometimes be used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group (usually used in reference to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU)

  - CB: Coding block (luma or chroma).

  - CU: Coding unit (containing both luma and chroma).

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma)

  - PU: Prediction unit (containing both luma and chroma), with eight shape possibilities.

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP.

  - TB: Transform block (luma or chroma).

  - TU: Transform unit (containing both luma and chroma).

## Liaison activity

The JCT-VC did not send or receive formal liaison communications at this meeting.

## Opening remarks

Opening remarks included:

- Meeting logistics, review of communication practices, and attendance recording badge pick-up reminder

Primary topic areas were noted as follows:

- Screen content coding

- Corrigenda items for version 3 (see, e.g., the AHG2 report).

- Verification testing for interlaced video, RExt, & SHVC

- Reference software and conformance, RExt & SHVC

- Test model texts and software manuals

- Common test conditions for coding efficiency experiments

Unfinished (or less-than-optimally finished) deliverables

- SHM 9 was missing at the opening of the meeting, some WG 11 deliverables needed

Key deliverables from this meeting

- DAM on SCC

- DAM for RExt conformance

- DAMs for SHVC conformance & reference software

- Verification test report for RExt

- Verification test plan for SHVC

Two main tracks were followed for most meeting discussions:

- Track A (GJS): CE1 (palette), etc.

- Track B (JRO): CE2 (intra block copy), etc.

## Scheduling of discussions

Scheduling: Generally meeting time was scheduled during 0800 – 2000, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and continue with parallel review on Screen Content Coding CE work and related contributions during the first few days. Ongoing refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate:

- Fri. 19 June, 1st day

  - 0900–1300 JCT-VC opening and review of AHG reports and CE 1 summary [JRO & GJS]

  - 1430–2000 Tracks A (Palette) & B (IBC)

- Sat. 20 June, 2nd day

  - 0900–1230

    - Track A Palette U0097, U0109, U0116, U0149, U0036), possibly high-level syntax, VUI&SEI messages, and o inputs

    - Track B (IBC)

  - 1530–2000 Tracks A & B

    - Track A: U0084, U0114, VUI&SEI messages, and non-normative inputs

      - Remaining open included U0052,arevisit of U0169 and U0090, U0112 (ambient viewing environment SEI message), U0115 (video bitstream transition SEI message), U0095 (encoder speed-up), U0152 (R-lambda rate control), new U0171 (palette with pixel copy)

- Sun. 21 June, 3rd day

## Contribution topic overview (update counts and track identifiers)

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized into "tracks" (A, B, or P) for "parallel session A", "parallel session B", or "Plenary" review, as follows. Discussions on topics categorized as "Track A" were primarily chaired by Gary Sullivan, whereas discussions on topic categorized as "Track B" were primarily chaired by Jens-Rainer Ohm. Some plenary sessions were chaired by both co-chairmen, and others were chaired by Gary Sullivan. Chairing of other discussions is noted for particular topics. (Note: Allocation to tracks was subject to changes)

- AHG reports (13) Track P (section 2)

- Project development status (11+11HDR) Track X (section 3)

- SCC CE1: Palette mode improvements (14) Track A (section 4.1)

- SCC CE2: IBC memory access (7) Track B (section 4.2)

- Non-CE SCC (XX) (section 5.1) with subtopics

  - CE1 Palette mode related (44) Track A (section 5.1.1)

  - CE2 Intra block copy related (1), Track B (section 5.1.2)

  - IBC/MC unification (16), Track B (section 5.1.3)

  - Other IBC related (5), Track B (section 5.1.4)

  - Tool complexity (0), Track X (section 5.1.5)

  - Other (4), Track X (section 5.1.6)

- High-level syntax (0) Track X (section 5.2)

- VUI and SEI messages (3+4HDR) Track A (section 5.3)

- Non-normative (6) Track A (section 5.4)

- Plenary discussions and BoG reports (XX) Track X (section 6)

- Outputs & planning: AHG & CE plans, Conformance, Reference software, Verification testing, Chroma format, CTC (sections 7, 8, and 9)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

# AHG reports (13)

The activities of ad hoc groups (AHGs) that had been established at the prior meeting are discussed in this section.

(Consideration of these reports was chaired by GJS & JRO on Friday 06-19, 0900-1100, except as noted.)

JCTVC-U0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm (AHG co-chairs)]

[Add notes]

JCTVC-U0002 JCT-VC AHG report: HEVC test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne (AHG co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on HEVC test model editing and errata reporting (AHG2) between the 20th meeting in Geneva, CH (February 2015) and the 21st meeting in Warsaw, PL (June 2015).

An issue tracker (https://hevc.hhi.fraunhofer.de/trac/hevc) was used in order to facilitate the reporting of errata with the HEVC documents.

The High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 2 of Encoder Description was produced as JCTVC-T1002. This document represented a refinement of the previous HM16 Improved Encoder Description document (JCTVC-S1002). In particular, numerous tool descriptions were improved and description of high level syntax and picture types was expanded. The resultant document provides a source of general tutorial information on HEVC Edition 1 and Range Extensions, together with an encoder-side description of the HM-16 software.

A discrepancy between the software and the HEVC Version 2 text has been identified that affects the CABAC alignment tool present in the high throughput profile. This error was first reported on the bug-tracker (ticket #1391), where it has been indicated that the software implements the intended function. A related contribution (JCTVC-U0031) has been submitted in relation to this discrepancy.

See notes on JCTVC-U0031.

JCTVC-U0003 JCT-VC AHG report: HEVC HM software development and software technical evaluation (AHG3) [K. Suehring, K. Sharman, D. Flynn (AHG co-chairs)]

(Consideration of this report was chaired by XXX on XXday 06-XX, XXXX-XXXX.)

JCTVC-U0004 JCT-VC AHG report: HEVC conformance test development (AHG4) [T. Suzuki, J. Boyce, K. Kazui, A. K. Ramasubramonian, Y. Ye (AHG co-chairs)]

(Consideration of this report was chaired by XXX on XXday 06-XX, XXXX-XXXX.)

JCTVC-U0005 JCT-VC AHG report: HEVC and range extensions verification test preparation (AHG5) [V. Baroncini, M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, T. K. Tan, J.-M. Thiesse, W. Wan (AHG co-chairs)] [miss]

(Consideration of this report was chaired by XXX on XXday 06-XX, XXXX-XXXX.)

JCTVC-U0006 JCT-VC AHG report: SCC coding performance analysis (AHG6) [H. Yu (AHG chair), R. Cohen, A. Duenas, P. Lai, K. Rapaka, J. Xu (AHG vice-chairs)]

This report summarizes the activities of the JCT-VC ad hoc group on SCC coding performance analysis (AHG6) between the JCT-VC 20th meeting in Geneva, Switzerland, and the 21st meeting in Warsaw, Poland.

Per the decisions captured in the meeting notes JCTVC-T_Notes_d8, the following changes have been made in JCTVC-T1015 “Common Test conditions for screen content coding”:

- Added 10 new 4:2:0 sequences; they were converted by sub-sampling the original 4:4:4 test sequences with simple filters described in the meeting notes.

  - Uploaded these sequences to the ftp site for all test material

  - provided updated version of results reporting templates (Lossy and Lossless)

- Added 4x1 CTU IBC test conditions for 4:4:4 sequences; provided associated encoder settings as HashBasedIntraBlockCopySearchEnabled = 1; IntraBlockCopySearchWidthInCTUs = 3; IntraBlockCopyNonHashSearchWidthInCTUs = 1.

Three related contributions were noted:

- JCTVC-U0150: New SCC test sequence for consideration [W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei)]

- JCTVC-U0051: Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

- JCTVC-U0058: Compression Performance of HEVC Screen Content Coding Extensions Test Model 4.x with slices [C. Gisquet, G. Laroche, P. Onno (Canon)]

It was recommended to:

- Review the proposed new test material.

- Discuss further improvements to CTC

- Continue to evaluate the coding performance of the newly adopted tools and their interaction with the existing HEVC tools in the Main profile and range extensions.

JCTVC-U0007 JCT-VC AHG report: SCC extensions text editing (AHG7) [R. Joshi, J. Xu (AHG co-chairs), R. Cohen, S. Liu, Z. Ma, G. Sullivan, Y. Ye (AHG vice-chairs)]

This document reports on the work of the JCT-VC ad hoc group on SCC extensions text editing (AHG7) between the 20th JCT-VC meeting in Geneva, Switzerland (February 2015).and the 21st JCT-VC meeting in Warsaw, Poland (June 2015).

The third working draft for the High Efficiency Video Coding Screen Content Coding (HEVC SCC) extension was created as an output document following the decisions taken at the 20th JCT-VC meeting in Geneva, Switzerland (February 2015).

JCTVC-T1005 was produced by the editing ad hoc group (AHG) following the 20th JCT-VC meeting in Geneva, Switzerland. One significant change was that JCTVC-T1005 is based on HEVC v2 (without the 3D-HEVC extension yet integrated).Two versions JCTVC-T1005 were produced. The two are identical except that the editorial notes have been removed from the second version of the document. The text of JCTVC-T1005 (version 2) was submitted to ISO/IEC JTC1/SC29 for a ISO/IEC PDAM ballot (23008-2/PDAM 5).

List of changes with respect to JCTVC-S1005:

- Converted the document to use the complete HEVC version 2 text as its basis (without the 3D-HEVC extension yet integrated).

- Integrated palette escape colour coding (JCTVC-T0112/T0118)

- Integrated IBC unified with inter (JCTVC-T0227)

- Integrated Palette extension to non-444 format (JCTVC-T0072/T0109/T0120)

- Integrated intra MPM fix for SCC (JCTVC-T0049)

- Integrated disabling 64x64 palette (JCTVC-T0058)

- Integrated context reduction for coding run_type in palette (JCTVC-T0078)

- Integrated signalling delta maximum palette predictor size in SPS (JCTVC-T0134)

- Integrated HLS refinement for adaptive motion vector resolution (JCTVC-T0069)

- Integrated Inference for palette_escape_val_present_flag (JCTVC-T0183)

- Integrated palette predictor initialization entries in PPS (JCTVC-T0048)

- Integrated removal of palette sharing flag (JCTVC-T0064)

- Integrated EG0 for new palette entries (JCTVC-T0064)

- Integrated content light level information SEI message (JCTVC-T0101)

- Integrated clipping MV to 16-bit for AMVR (Notes of JCTVC-T0059)

- Integrated QP offset for different colour spaces (JCTVC-T0140)

- Integrated clipping before ACT (JCTVC-T0132)

The screen content coding test model 4 (SCM 4) (document JCTVC-T1014) was released on 1st, June 2015. The main changes were the description of unified signalling for intra block copy and inter modes.

Related contributions were noted as follows:

- JCTVC-U0038 proposes editorial improvements to address feedback and comments related to the SCC draft text 3. It also summarizes omissions (e.g., two forgotten SEI messages – the dependent RAP picture SEI message and the coded region completion SEI message) and known open issues (inclusion of errata and confusion between intra/inter and IBC).

- JCTVC-U0031, JCTVC-U0091 and JCTVC-U0099 point to mismatch between draft text 3 and SCM 4.0 software.

- JCTVC-U0036, JCTVC-U0055, and JCTVC-U0089, although containing normative aspects, propose constraints on syntax elements.

- JCTVC-U0092, JCTVC-U0110 are proposals for general editorial improvements/cleanups of the draft text.

The recommendations of the HEVC SCC extension draft text AHG are to:

- Approve the documents JCTVC-T1005 and JCTVC-T1014 as JCT-VC outputs

- Address the comments and feedback on SCC extensions text specification as appropriate

- Compare the HEVC SCC extensions document with the HEVC SCC extensions software and resolve any discrepancies that may exist, in collaboration with the SCC extension software development (AHG8)

- Review the related contributions

JCTVC-U0008 JCT-VC AHG report: SCC extensions software development (AHG8) [B. Li, K. Rapaka (AHG co-chairs), R. Cohen, P. Chuang, X. Xiu, M. Xu (AHG vice-chairs)]

This report summarizes the activities of Ad Hoc Group 8 on screen content extensions software (SCM) development that have taken place between the JCT-VC 20th meeting in Geneva, Switzerland, and the 21st meeting in Warsaw, Poland.

Multiple versions of the HM SCM software were produced and SCM4.0 was announced on the JCT-VC email reflector. The integration details and performance summary of these revisions are provided in the next subsections. The performance results of software revisions were observed to be consistent with the adopted techniques.

HM-16.4_SCM-4.0 was tagged on the SVN HHI repository on March 18th 2014. This release includes following adoptions:

- JCTVC-T0048: Palette_predictor_initializer at PPS-level

- JCTVC-T0058: Disallow palette mode for 64x64 CUs

- JCTVC-T0063: EG0 signalling for palette_num_signalled_entries

- JCTVC-T0064: Remove the palette sharing flag and its context

- JCTVC-T0065: Grouping Palette Indices

- JCTVC-T0069: AMVR high-level syntax cleanup

- JCTVC-T0072/T0109/T0120: Palette Coding for non-4:4:4 format content

- JCTVC-T0078: Remove context in palette run mode

- JCTVC-T0087: Palette Table Generation

- JCTVC-T0116: Encoder improvements on IBC search

- JCTVC-T0118/T0112: On Escape color coding

- JCTVC-T0121: Encoder operation to infer split_transform_flag

- JCTVC-T0132: ACT dynamic range control

- JCTVC-T0134: Use delta to signal palette predictor size

- JCTVC-T0140: QP offset for different color spaces

- JCTVC-S0180/S0150/Meeting notes: Grouping escape at the end

- JCTVC-T0183: Add inference rule when  palette_escape_val_present_flag  is not present

- JCTVC-T0227: Intra block copy and inter signaling unification

- SW Bug fix:  For high bit-Depth test configuration

- Meeting notes:  Double the palette and palette predictor size

- Ticket#1376: Incorrect search range for Nx2N

- Migration to HM-16.3, HM-16.4.

The main changes to the configuration files are:

- Addition of 420 configuration files as in JCTVC-T1015

- Temporal scalability is supported in the RA test conditions.

The following adoptions had not been integrated to SCM 4.0 (These do not impact CTC):

- JCTVC-T0048/T0055/T0056:  IntraBC constraint for multiple slices/tiles SW fix. These aspects are integrated to SCC branch targeting SCM 4.1

- JCTVC-S0043 - palette delta QP coding: During integration, it was observed that original software palette design (from SCM 2.0) does not properly handle changing QP’s across CU’s (a functionality that is available in HM using –d options). This issue was brought to attention of palette experts and investigation is ongoing.

The release was announced on the email reflector. The software can be downloaded at

https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

The performance of this version against HM-16.2+SCM-3.0 was described according to the common test conditions in JCTVC-S1015. For the lossy 444 configuration, it is reported that this version provides BD-rate reduction of 1.7%, 3.4% and 2.9% for RGB 1080p & 720p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 1.0%, 3.2% and 2.8% for YUV 1080p & 720p text and graphics category in AI/RA/LB configuration, respectively. For the lossless 444configuration, it is reported that this version provides BD-rate reduction of 1.0%, -0.4% and -0.2% for RGB 1080p & 720p text and graphics category in AI/RA/LB configurations respectively and BD-rate reduction of 0.7%, -0.6% and -0.3% for YUV 1080p & 720p text and graphics category in AI/RA/LB configuration, respectively.

The tables below summarize BD-rate change for lossy and lossless 444 configurations respectively.

BD-Rate change in Lossy 444 configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.7% | -3.0% | -2.9% |
| RGB, mixed content, 1440p & 1080p | -1.1% | -2.4% | -2.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.0% | -2.4% | -2.4% |
| YUV, mixed content, 1440p & 1080p | -1.2% | -2.5% | -2.2% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 116% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.4% | -5.0% | -4.9% |
| RGB, mixed content, 1440p & 1080p | -1.2% | -3.3% | -3.3% |
| RGB, Animation, 720p | 0.2% | 0.0% | -0.2% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.3% |
| YUV, text & graphics with motion, 1080p & 720p | -3.2% | -5.6% | -5.9% |
| YUV, mixed content, 1440p & 1080p | -1.7% | -4.3% | -3.9% |
| YUV, Animation, 720p | 0.3% | -0.3% | -0.2% |
| YUV, camera captured, 1080p | -0.2% | 0.1% | -0.1% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 87% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.9% | -4.4% | -4.2% |
| RGB, mixed content, 1440p & 1080p | -1.7% | -3.7% | -3.8% |
| RGB, Animation, 720p | 0.1% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -2.8% | -4.6% | -5.1% |
| YUV, mixed content, 1440p & 1080p | -2.6% | -5.9% | -5.2% |
| YUV, Animation, 720p | 0.1% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 85% |  |  |
| Dec Time[%] | 86% |  |  |

BD-Rate change in Lossless 444 configuration

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -0.9% | -2.9% | 0.2% |
| RGB, mixed content, 1440p & 1080p | -1.1% | -0.9% | -2.3% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | -0.6% | -3.2% | 1.0% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.4% | -1.6% | 0.3% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 97% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | -0.2% | -1.5% | 1.2% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | -0.1% | 0.1% |
| RGB, Animation, 720p | 0.7% | 0.7% | 0.7% | 0.7% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.6% | -0.4% | -4.2% | 1.8% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | -0.1% | 0.2% |
| YUV, Animation, 720p | 0.5% | 0.5% | 0.5% | 0.5% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 118% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | -0.5% | -2.4% | 0.8% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.2% | 0.2% | 0.2% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | -0.7% | -3.7% | 1.3% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 126% |  |  |  |
| Dec Time[%] | 117% |  |  |  |

HM-16.4_SCM-4.0rc1 was tagged on the SVN HHI repository on March 13th 2015 before the release of HM-16.4_SCM-4.0 to allow proponents to cross-check integrations and interactions with other adoptions. Further minor cleanup and bug fix were done before tagging HM-16.4_SCM-4.0.

The release was announced on the email reflector. The software can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0rc1/

HM-16.3+SCM-3.2 has been tagged on HHI Server on February 25th 2015 and can be downloaded at https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.3+SCM-3.2/

The changes included in this release were:

- Updated SCM bit-depth support is enabled.

- Removal of macro’s related to previous integrations.

- Bug fix when high bit-depth support is enabled.

The JCT-VC issue tracker at https://hevc.hhi.fraunhofer.de/trac/hevc/ has been updated to allow bug reports to be entered for SCM, currently under milestone HM+SCC-5.0, version SCC-4.0.

Several changes were made after the release of SCM-4.0. Among them, the following changes have impact on CTC:

- r4450, fix ticket 1311, which has minor performance impact on 444 lossless coding (when ACT is enabled).

- r4451, Intra BC clean up to align with spec, which requires to signal RPLM for RA coding under CTC (when Intra BC is enabled). Additional bits are signalled at slice header.

Recommendations of the AHG were:

- Continue to develop reference software based on HM16.4_SCM4.0 and improve its quality and release HM16.4_SCM4.1.

- Remove macros introduced in previous versions before starting integration towards SCM-3.0 such as to make the software more readable.

- Continue merging with later HM versions.

Release of SCM 4.1 was expected to occur on 06-XX during the current meeting.

JCTVC-U0009 JCT-VC AHG report: Complexity of SCC extensions (AHG9) [A. Duenas (AHG chair), M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, X. Xiu (AHG vice-chairs)] [miss]

(Consideration of this report was chaired by XXX on XXday 06-XX, XXXX-XXXX.)

JCTVC-U0010 JCT-VC AHG report: Test sequence material (AHG10) [T. Suzuki, V. Baroncini, R. Cohen, T. K. Tan, S. Wenger, H. Yu (AHG co-chairs)]

(Consideration of this report was chaired by XXX on XXday 06-XX, XXXX-XXXX.)

JCTVC-U0011 JCT-VC AHG report: SHVC test model editing (AHG11) [J. Chen (AHG chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (AHG vice-chairs)]

This document reports the work of the JCT-VC ad hoc group on SHVC test model editing (AHG11) between the 20th JCT-VC meeting in Geneva, Switzerland (10–18 Feb. 2015) and the 21th JCT-VC meeting in Warsaw, Poland (19–26 June 2015).

This AHG also collected text problem reports.

During this period, the editorial team worked to improve the ISO/IEC 23008-2 document (HEVC version 2 specification text) prior to its publication. The following refinements and bug fixes have been included and forwarded to the SC 29 secretary for incorporation into the publication process:

- Fix: remove one inference rule of sub_layer_tier_flag, there are two inference rules for of sub_layer_tier_flag in current spec (this fix was already integrated into the published ITU-T HEVC version 2)

- Editorial fix: adding a missing "the" and removing an unnecessary and confusing “less than or” in the semantics of syntax element part_mode in 7.4.9.5 Coding unit semantics

- Fix: adding missing inference rule for the syntax element collocated_ref_idx

- Clarification fix: replacing "chroma residual samples" by "chroma prediction samples" in 8.5.3.3 Decoding process for inter prediction samples

- Fix: wrong the inputs (passing samples) of deblocking process invocation in 8.7.2.5.3 Decision process for luma block edges

- Fix: replacement of "buffering period" with "active parameter sets" and inserting a missing “access unit with" in an informative NOTE in D.3.3 (Picture timing SEI message semantics)

- Typo fix: fixing titles of clauses F.6, G.6, H.6 to correspond to titles in subclauses of clause 6

- Typo fix: missing changes from RExt specification in F.7.3.2.2.1 General sequence parameter set RBSP syntax

- Editorial fix: missing changes from RExt specification in F.3.7.6.1 General slice segment header syntax

- Minor typo fix in F.7.3.2.3.3 Picture parameter set multilayer extension syntax

- Fix: removal of "for each access unit" in the note on BaseLayerOutputFlag which is not access unit specific, in F.8.1.2 CVSG decoding process

- Fix: wrong index for the derivation of LayerInitializedFlag in F.8.1.3 Common decoding process for a coded picture

- Fix: changing " discardable_flag equal to 1" to " discardable_flag equal to 0" in one place in F.14.3.2.2 Buffering period SEI message semantics for multi-layer extensions

- Minor fix: wrong reference (F.8.5 -> G.8.4) in G.8.1.2 Decoding process for a coded picture with nuh_layer_id greater than 0

- Editorial fix: replacing “dependent layer” by "reference layer" in the Multiview Main and Scalable Main profile definition, since “dependent layer” is not a defined term

- Fix: Removed misleading redundant constraint expressions for SPS and PPS extension flag values in G.11.1.1 and H.11.1.1.

- Typo fix: index typo H.8.1.4.2 Resampling process of picture motion and mode parameters

- Typo fix: remove wrong brackets in equation H-85 and equation H-86

The editorial team did not work on the Scalable HEVC (SHVC) Test Model text during this period.

The AHG recommended to:

- Use SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to SHVC Draft and Test Model text.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the SHVC Software AHG.

- Continue to improve the quality of the SHVC test model document.

JCTVC-U0012 JCT-VC AHG report: SHVC software development (AHG12) [V. Seregin, Y. He]

(Reviewed verbally prior to upload.)

Add notes.

JCTVC-U0013 JCT-VC AHG report: SHVC verification testing (AHG13) [V. Baroncini, Y.-K. Wang, Y. Ye]

This AHG was established to prepare the plan for the verification testing of the SHVC specification.

The input contribution JCTVC-U0037 provides a draft SHVC verification test plan, including the following:

- Suggested timeline

- Suggested test cases covering 2-layer spatial 2x, 1.5x, SNR, and CGS test cases

- Suggested test sequences for each test case

- Provided preliminary coding results for SHVC and HEVC simulcast at the suggested rate points

Some open issues were identified and needed further discussion:

- It was suggested to compare SHVC with HEVC simulcast, and not compare with SVC. This was partly because

  - the number of test points would increase, making the test more difficult to perform

  - there may be some difficulty with the SVC software (e.g., low-delay B), and

  - there is an apparent higher interest in HEVC simulcast as the primary basis for comparison.

This was agreed.

- Issues with CGS test case, in particular lack of test sequences and lack of test equipment (e.g., display with WCG)

- Test point selection (BL and EL bit rates)

The AHG recommended

- To discuss JCTVC-U0037, and agree on a proposed draft SHVC verification test plan with possible refinements after discussion.

- To encourage participants to contribute test sequences suitable as subjective test material, especially material in the CGS test case.

- To start bitstream preparation following the agreed SHVC verification test plan.

# Project development, status, and guidance (XX)

## Conformance test set development (0)

## Version 1/2 bug reports and cleanup (2)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0031 Discrepancy between Version 2 text and software for High Throughput Profile [K. Sharman, N. Saunders (Sony)]

- JCTVC-U0156 On 4:2:0 chroma positioning for Rec. ITU-R BT.2020 [C. Fogg (MovieLabs), G. J. Sullivan (Microsoft), A. M. Tourapis (Apple)] [late]

## SCC text development (5)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0038 Proposed editorial improvements to HEVC Screen Context Coding Draft Text 3 [R. Joshi (Qualcomm), J. Xu (Microsoft), S Liu (MediaTek), Y. Ye (InterDigital)] [late]

JCTVC-U0055 High-level syntax refinement in monochrome signalling [C. Gisquet, G. Laroche, P. Onno (Canon)]

JCTVC-U0092 Draft Text modifications on palette [P. Lai, J. Ye, J. Kim, X. Xu, S. Liu, S. Lei (MediaTek)]

JCTVC-U0110 Bugfixes and cleanup for the text of palette mode in HEVC SCC draft [Y.-J. Chang, J.-S. Tu, C.-C. Lin, C.-L. Lin (ITRI)]

## HEVC coding performance, implementation demonstrations and design analysis (3)

### HM performance (0)

### RExt performance/verification test (0)

### SHVC performance/verification test (1)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0037 SHVC Verification Test Draft Plan [Y. He, Y. Ye (InterDigital), Hendry, Y.K. Wang (Qualcomm)]

### SCC performance, design aspects and test conditions (2)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0051 Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile [B. Li, J. Xu, G. J. Sullivan (Microsoft)]

JCTVC-U0058 Compression Performance of HEVC Screen Content Coding Extensions Test Model 4.x with slices [C. Gisquet, G. Laroche, P. Onno (Canon)] [late]

## Systems interfacing (1)

JCTVC-U0137 On the use of AVC and HEVC "unspecified" NAL unit types in file formats and systems [Y.-K. Wang (Qualcomm), D. Singer (Apple), G. J. Sullivan (Microsoft)] [late]

## Source video test material (1)

JCTVC-U0150 New SCC test sequence for consideration [W. Wang, M. Xu, F. Duanmu, H. Yu (Huawei)] [late]

## HDR (11)

(discuss with parent bodies to understand relation with CfE results)

(inputs on SEI JCTVC-T0032, JCTVC-T0033, JCTVC-T0048, JCTVC-T0098 also relate to this topic)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0040 HM reference software bug fixes and enhancements to address the HDR/WCG CfE [A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, S. Lee, D. Bugdayci, A. Ramasubramonian, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)]

JCTVC-U0041 On 4:4:4 to 4:2:0 conversion. Performance of downsampling and upsampling filters and MinMax and Closed Loop Filtering [A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, S. Lee, D. Rusanovskyy, A. Ramasubramonian, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)] [late]

JCTVC-U0042 Clipping during 4:2:0 to 4:4:4 conversions [A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)]

JCTVC-U0043 Deblocking in HEVC: Some observations from the HDR/WCG CfE [A.M. Tourapis, Y. Su, D. Singer (Apple), J. Sole, D. Rusanovskyy, M. Karczewicz (Qualcomm), C. Fogg (Movielabs), A. Duenas, F. Bossen (NGCodec)] [late]

JCTVC-U0044 Coding results of noise-reduced 4:2:0 and 4:4:4 HDR signals [C. Fogg (MovieLabs), K.Slavin (isovideo)]

JCTVC-U0045 High Dynamic Range with HEVC Main10 [D. Le Gall (Ambarella), A. Tourapis (Apple), W. Wan (Broadcom), C. Fogg (MovieLabs), A. Duenas (NG Codec), D. Rusanovskyy (Qualcomm)]

JCTVC-U0046 Studio requirements for next-generation video codecs [C. Fogg, J. Helman (MovieLabs)]

JCTVC-U0047 HDR Workflow precision and steps [C. Fogg (MovieLabs)] [late] [miss]

JCTVC-U0049 Wiener filter for HDR noise reduction pre-processing [A. Tourapis (Apple), C. Fogg (MovieLabs)]

JCTVC-U0085 Modulation-based HDR video coding with SDR backward compatibility [S. Lasserre, F. Le Leannec, E. François, T. Poirier (Technicolor)]

JCTVC-U0134 Upsampling and downsampling filters for HDR content [A.K. Ramasubramonian, D. Rusanovskyy, J. Sole, M. Karczewicz, S. Lee, D. Bugdayci Sansli, J. Chen (Qualcomm)] [late]

# Core experiments in SCC (XX)

## CE1: Palette mode improvements (XX)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

### CE1 summary and general discussion (1)

JCTVC-U0021 CE1: Summary report of CE on palette mode improvement [P. Lai, V. Seregin, X. Xiu, C. Gisquet (CE coordinators)]

(Consideration of this topic was chaired by GJS and JRO on Friday 06-19, 11:00-13:00 and by GJS 14:30-15:30.)

CE1 Test B proposed to remove condition checks for index adjustment. In this contribution, 3 different options were tested. In Test 1, index adjustment was removed when previous pixel has COPY_ABOVE mode. In Test 2, index adjustment was removed when previous pixel has COPY_INDEX mode. Test 3 removes index adjustment for both cases. Since Test 1 and Test 2 limit index adjustment based on previously decoded run_type, run_type should be signalled before grouped indices are signalled. Test 1 shows average 0.1%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively. Test 2 shows average 0.3%, 0.2%, 0.2% loss under AI, RA and LB configuration respectively. Test 3.1 which doesnot signal run_type before grouped indices shows average 0.3%, 0.1%, 0.1% loss under AI, RA and LB configuration respectively and Test 3.2 which signals run_type before grouped indices shows maximum 0.3%, 0.2%, 0.1% loss under AI, RA and LB configuration respectively.

Test A – Simplification for index map coding in palette mode

Test A.1 – Extended copy above mode

Proponent: MediaTek and Qualcomm, JCTVC-U0061

Crosschecker: ITRI International, JCTVC-U0121

Description:

This test evaluates palette mode using the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. With palette indices grouped at front, index adjustment bits is introduced to signal whether it is an actual max index value when the parsed index is Max max index value - 1.

Test A.2 –Extended copy above mode to the first line with grouping the palette_run_type_flag in front

Proponent: MediaTek, JCTVC-U0062

Crosschecker: Qualcomm, JCTVC-U0147

Description:

This test evaluates palette mode using the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. Syntax elements palette_run_type_flag are grouped and moved in front of the grouped palette indices. (The syntax of the palette run types, the palette indices, the palette run lengths, and the escape pixels are grouped respectively.) Palette index range adjustment does not depend on above position.

Runtime numbers for A.2 are suggested not to be accurate.

Lossy, 444, full-frame IBC

| 444, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -1.5% | -1.6% | -1.6% | -1.4% | -1.5% | -1.5% |
| RGB, mixed, 1440p & 1080p | -0.5% | -0.7% | -0.7% | -0.5% | -0.6% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -1.9% | -1.9% | -2.0% | -1.7% | -1.9% | -1.9% |
| YUV, mixed, 1440p & 1080p | -0.7% | -1.2% | -1.3% | -0.6% | -1.1% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% (cross-checker) |  |  | 134% |  |  |
| Dec Time[%] | 101% (cross-checker) |  |  | 130% |  |  |

| 444, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -0.9% | -1.0% | -1.0% | -0.9% | -1.0% | -1.0% |
| RGB, mixed, 1440p & 1080p | -0.4% | -0.5% | -0.5% | -0.4% | -0.5% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, TGM, 1080p & 720p | -1.1% | -1.2% | -1.4% | -1.0% | -1.2% | -1.4% |
| YUV, mixed, 1440p & 1080p | -0.6% | -1.1% | -1.4% | -0.5% | -0.9% | -1.1% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 99% (cross-checker) |  |  | 126% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 122% |  |  |

| 444, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -0.9% | -0.8% | -0.8% | -0.8% | -0.8% | -0.9% |
| RGB, mixed, 1440p & 1080p | -0.2% | -0.4% | -0.3% | -0.2% | -0.5% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| YUV, TGM, 1080p & 720p | -1.0% | -1.1% | -1.1% | -0.9% | -1.0% | -1.1% |
| YUV, mixed, 1440p & 1080p | -0.2% | -0.7% | -1.1% | -0.1% | -0.6% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% | 0.0% | -0.4% | 0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% (cross-checker) |  |  | 128% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 118% |  |  |

- Lossy, 444, 4x1-CTU IBC

| 444, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -1.8% | -1.8% | -1.9% | -1.7% | -1.8% | -1.8% |
| RGB, mixed, 1440p & 1080p | -0.6% | -0.8% | -0.9% | -0.6% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -2.2% | -2.2% | -2.4% | -2.1% | -2.0% | -2.2% |
| YUV, mixed, 1440p & 1080p | -0.9% | -1.4% | -1.4% | -0.8% | -1.3% | -1.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% (cross-checker) |  |  | 130% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 127% |  |  |

| 444, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -1.1% | -1.2% | -1.2% | -1.0% | -1.1% | -1.1% |
| RGB, mixed, 1440p & 1080p | -0.5% | -0.6% | -0.6% | -0.4% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -1.3% | -1.5% | -1.7% | -1.2% | -1.4% | -1.5% |
| YUV, mixed, 1440p & 1080p | -0.6% | -1.2% | -1.4% | -0.5% | -0.9% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, cam-captured, 1080p | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 106% (cross-checker) |  |  | 123% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 119% |  |  |

| 444, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -0.9% | -1.0% | -0.9% | -0.8% | -1.0% | -0.9% |
| RGB, mixed, 1440p & 1080p | -0.2% | -0.3% | -0.4% | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -1.0% | -1.0% | -1.3% | -0.9% | -1.0% | -1.0% |
| YUV, mixed, 1440p & 1080p | -0.3% | -1.2% | -0.9% | -0.1% | -1.0% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.2% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 103% (cross-checker) |  |  | 130% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 121% |  |  |

- Lossy, 420, full-frame IBC

| 420, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | -0.5% | -0.3% | -0.4% | -0.5% | -0.4% | -0.5% |
| YUV, mixed, 1440p & 1080p | -0.4% | -0.6% | -0.5% | -0.3% | -0.6% | -0.5% |
| YUV, Animation, 720p & 768p | -0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 104% (cross-checker) |  |  | 140% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 129% |  |  |

| 420, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% |
| YUV, mixed, 1440p & 1080p | -0.2% | -0.5% | -0.6% | -0.1% | -0.4% | -0.3% |
| YUV, Animation, 720p & 768p | 0.0% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 102% (cross-checker) |  |  | 133% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 123% |  |  |

| 420, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | -0.3% | -0.5% | -0.7% | -0.2% | 0.1% | -0.4% |
| YUV, mixed, 1440p & 1080p | -0.2% | -0.8% | -0.3% | -0.3% | -1.1% | -0.3% |
| YUV, Animation, 720p & 768p | -0.1% | -0.5% | 0.3% | 0.0% | -0.2% | 0.5% |
| Enc Time[%] | 101% (cross-checker) |  |  | 140% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 130% |  |  |

Discussion: A.2 characterized to be "cleaner" than A.1, with somewhat less gain (0.1%-0.2%), with overall gain in TGM AI full-frame IBC RGB 1.4%, AI full-frame IBC YUV 1.7%.

It was estimated that the runtime of A.1 or A.2 is about 6-8% slower than the anchor, partly due to having another pass for encoding decisions.

It was remarked that this adds the ability to predict from outside the current CU, which adds some complexity, as it changes the architecture. It was asked whether the 1.5% gain is worth the complication.

It was remarked that there is a related contribution U0096 with encoder-only modifications showing gains higher than this with similar runtime impact, and that encoding speed-up could be achived as well (e.g., by using early-termination detection).

It was remarked that other aspects of the design may have issues – e.g., B slice RPL and motion estimation quality. It is not clear, however, whether that has a real relationship to this.

No action was taken on this.

Test B – Simplification for index map coding in palette mode

Proponent: MediaTek, JCTVC-U0050

Crosschecker: Sharp, JCTVC-U0070

Description:

This test evaluates removing dependency introduced by index range adjustment. In order to apply the methods to the current palette index coding in SCM4.0, syntax reordering with “grouping palette run_type flag at front” is utilized in the tests. The following are presented:

Result 1:	Group run_type in front, no dependency from the above index

(remove index range adjustment when previous pixel is COPY_ABOVE mode).

Result 2:	Group run_type in front, no dependency from the left index

(remove index range adjustment when previous pixel is COPY_INDEX mode).

Results 3-1:	Without grouping run_type in front, completely remove index range adjustment.

Results 3-2:	Group run_type in front, completely remove index range adjustment.

Supplementary: Group run_type in front.

- Lossy, 444, full-frame IBC

- Lossy, 444, 4x1-CTU IBC

- Lossy, 420, full-frame IBC

It was said that the complexity reduction available in this test is rather small, and some significant loss occurs in the TGM category by doing so.

No action was taken on this.

Test C – Palette run-length coding scan order modifications

Test C.1 –Palette run-length coding with palette reverse scan order

- Proponent: InterDigital and Qualcomm, JCTVC-U0101

- Crosschecker: Canon, JCTVC-U0059

- Description:

This test evaluates the palette reverse scan as proposed in JCTVC-T0119. The proposed method signals one additional flag at CU level. If the flag is equal to 1, the palette indices of the current CU are scanned in reverse scan order; otherwise, the palette indices of the current CU are scanned in original scan order. The following illustrate the reverse scan order when horizontal scan is applied to one palette-coded CU.

Test C.2 – Palette run-length coding with rotated index scans (scan from bottom-right)

- Proponent: MediaTek, JCTVC-U0088

- Crosschecker: InterDigital, JCTVC-U0108

- Description:

This test evaluates rotated palette index scanning order as reported in JCTVC-T0174. A CU-level flag is used to indicate whether to apply the rotation. Constraint the rotation on CU size is also tested.

(a) Horizontal scan	  (b) Reverse horizontal scan (C.1)    (c) Rotated horizontal scan (C.2)

(a) Vertical scan	    (b) Reverse vertical scan (C.1)	(c) Rotated vertical scan (C.2)

RD2: Using the RDO method of CE.1 Test C.1 released on Apr. 8, 2015

RD3: Using the RDO method of CE.1 Test C.1 released on May 30, 2015

- Lossy, 444, full-frame IBC

- Lossy, 444, 4x1-CTU IBC

- Lossy, 420, full-frame IBC

This increases encoding complexity by checking more scan patterns. A gain of about 1% is shown for TGM AI was shown. There is also some decoder complexity increase, for being capable of more patterns. For software decoders, the impact seems small – adding some logic or tables up to 32x32 in size (but perhaps not a small impact for hardware).

Reversing and rotating scan patterns don't have a big difference in coding efficiency.

It was remarked that a related contribution U0146 was submitted shows more gain by combining U0096 (encoder-only changes with some increase in complexity) with the C.1 variation of test C to obtain 2.1% gain. It was also remarked that in U0096, an encoder-only change without any test C change was reported to obtain a 1.8% gain. So in that context the difference in coding efficiency for the additional scan patterns was only 0.3%.

It was suggested to also review U0127 before making a decision on this.

See notes on U0127 and U0096.

Test D – Modification of palette run coding

Proponent: Qualcomm, JCTVC-U0122

Crosschecker: MediaTek, JCTVC-U0117

Description:

If the current pixel is the first pixel in the line, a flag is signalled into the bitstream, indicating whether it ends at the last pixel in certain line. If so, the number of whole lines (L) the current run spans is coded into the bitstream using coefficient coding function (xWriteCoefRemainExGolomb) with order 0. Instead of directly signalling L, a mapping process is used to map maximal feasible value to zero and shift L up by 1 if it is less than the maxima. Otherwise, it falls back to normal palette run length coding with an independent set of context models.

If the current pixel is not the first pixel in the line, run length L is coded into the bitstream using xWriteCoefRemainExGolomb with order 0. Instead of signalling L directly, if the current run ends at one line below the current line, a mapping procedure is used to reorder the candidate positions in the line according to their horizontal distance between themselves and the current pixel.

- Lossy, 444, full-frame IBC

- Lossy, 444, 4x1-CTU IBC

- Lossy, 420, full-frame IBC

The improvement of coding efficiency is smaller than with categories A or C.

It was said that a substantial amount of text would be needed for this change. The mapping process, in particular, involves a significant block of logic, and additional logic would be needed in the syntax as well.

No action was taken on this.

### CE1 primary contributions (7)

JCTVC-U0050 CE1 Test B: Simplification for index map coding in palette mode [J. Kim, J. Ye, P. Lai, S. Liu, S. Lei (MediaTek)]

JCTVC-U0061 CE1: Test A.1: Extended copy above mode to the first line with index adjustment bits [Y.-C. Sun, J. Kim, T.-D. Chuang, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz (Qualcomm)]

JCTVC-U0062 CE1: Test A.2: Extended copy above mode to the first line with grouping the palette_run_type_flag in front [Y.-C. Sun, J. Kim, T.-D. Chuang, S. Liu, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek)]

JCTVC-U0088 CE1 Test C.2: Palette run-length coding with rotated index scans (scan from bottom-right) [P. Lai, J. Kim, S. Liu, S. Lei (MediaTek)]

JCTVC-U0101 CE1 Test C.1: Improved palette run-length coding with palette reverse scan order [X. Xiu, Y. Ye, Y. He (InterDigital), W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin (Qualcomm)]

JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding [M. Karczewicz, W. Pu, R. Joshi, F. Zou, V. Seregin (Qualcomm)] [late]

JCTVC-U0127 CE1: Additional results for CE1 subtest C [R. Joshi, M. Karczewicz, V. Seregin, W. Pu, F. Zou (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 15:00-15:30.)

Not originally planned in CE, classified as information document.

Additional results for CE1 subtest C were provided to understand how much do the different aspects (encoder, scans) contribute to the overall gain. The scans without any encoder modifications to the way in which runs are calculated reportedly show BD-rate improvements in the range of 0.0% to 0.5% for both full frame IBC and 1×4 CTU IBC, All-Intra configurations. The BD-rate gains from encoder modifications to the way in which runs are calculated are in the range of 0.0% to XXX% for full frame IBC and 0.0% to 0.6% for 1×4 CTU IBC, All-Intra configurations.

Thus, the gains shown in CE1 subtest C were shown to be a combination of two things:

- Additional scan patterns (normative)

- Improved run-type flag decisions and run value decisions (non-normative)

The gain from each of these sources is roughly half of the total gain each.

See notes on U0096, which includes a simplified form of the technique described in this contribution, and also includes some other techniques.

### CE1 cross checks (6)

JCTVC-U0059 CE1: Cross-check of Test C.1 from Interdigital/Qualcomm [C. Gisquet (Canon)] [late]

JCTVC-U0070 CE1: Cross check of Test B on Simplification for index map coding in palette mode [J. Zhao, S.H. Kim (Sharp)]

JCTVC-U0108 Cross-verification of CE1 Test C.2: Palette run-length coding with rotated index scans [X. Xiu, Y. He, Y. Ye (InterDigital)] [late]

JCTVC-U0117 Cross-check of JCTVC-U0122 CE1 Test D: Modification of Palette Run Coding [P. Lai, J. Ye, J. Kim (MediaTek)] [late]

JCTVC-U0121 CE1: Crosscheck of CE1 Test A.1 (JCTVC-U0061) [W. Digonnet, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)]

JCTVC-U0147 Cross check CE1 Test A.2: Extended copy above mode to the first line with grouping the palette_run_type_flag in front (U0062) [W. Pu (Qualcomm)] [late] [miss]

## CE2: Intra block copy memory access (XX)

(Consideration of this topic was chaired by JRO on Friday 06-19, 14:30-16:00.)

### CE2 summary and general discussion (1)

JCTVC-U0022 CE2: Summary report for Core Experiment 2 on Intra block copy memory access [K. Rapaka, C. Gisquet, J. Lainema, X. Xu (CE coordinators)]

JCTVC-U0053 CE2 Test 2: Intra block copy constraints on filtering

- Don’t allow IBC when filtering (SAO/DBF) is enabled. This requires a flag enabling/disabling the filters at the CTU level (otherwise, when disabling it for the whole slice, performance would be too bad). Requires 2-pass encoding.

JCTVC-U0034 CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions)

- Same as test 2, but inferring decision from previous coded blocks based on statistics (by tendency filtering is rather turned of for screen content, not for natural content).

During the discussion it is also mentioned by experts that, whereas DBF is less beneficial for screen content, SAO could be beneficial.

JCTVC-U0078 CE2 : Test 5 on intra block copy constraints on prediction.

- Disabling IBC from CTUs where 8x8 bi prediction is used. 5.1 intends to trade the additional memory accesses for IBC (once filtered, once unfiltered) against the current worst case. This constraint does not apply to current and previous CTU which are assumed to be available in cache. 5.2 uses IBC only when bi-prediction is not used in current slice. 5.3 is similar, but allows joint usage of IBC and bi pred, if the latter uses integer MC. 5.4 is similar to 5.1, allowing joint usage of IBC and bi pred, if the latter uses integer MC.

All methods come with losses for screen content classes

AI: Test 2 around 1%, Test 3 around 2%, test 5.x 0% (does not affect AI)

RA: Test 2 around 0.9%, Test 3 >2%, test 5.1 around 3%, 5.2/5.3 around 0.5%, test 5.4 around 2%

LDB: Test 2 around 1%, Test 3 around 3%, test 5.1 4-5%, test 5.2/5.3 0.7%, test 5.4 around 2.5%

It is pointed out that the results have larger variation over different sequences.

Average memory access reduction is reported.

Worst case memory bandwidth saving by the different methods shall be reported. It should also be reported what the absolute memory consumption of the different test sequence classes are: The average numbers indicate no saving for some classes where IBC is apparently not used, but in fact these may use bi prediction more frequently and be worse in memory access than some of the screen content classes.

Revisit.

It is generally agreed that 5.2/5.3 seem to come with a relative moderate loss, whereas only require a change at slice level to enable/disable certain combinations of IBC/bipred/integer MC. Before making a decision, a precise number about the saved memory bandwidth should be available.

### CE2 primary contributions (3)

JCTVC-U0034 CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions) [J. Lainema, M. M. Hannuksela (Nokia)]

JCTVC-U0053 CE2 Test 2: Intra block copy constraints on filtering [G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon)]

JCTVC-U0078 CE2: Test 5 on intra block copy constraints on prediction [K. Rapaka (Qualcomm)]

### CE2 cross checks (3)

JCTVC-U0035 CE2: Crosscheck of Test 2 on intra block copy constraints on filtering (frame level encoding decisions) [J. Lainema (Nokia)] [late]

JCTVC-U0060 CE2: Cross check of Test 5 on Intra block copy constraints on bi-prediction samples with use of local cache [P. Onno, G. Malard (Canon)] [late]

JCTVC-U0082 CE2: Cross check of CE2 Test 2: Intra block copy constraints on filtering [K. Rapaka (Qualcomm)] [late] [miss]

# Non-CE Technical Contributions (XX)

## SCC coding tools (XX)

### CE1 related (Palette mode improvements) (44)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0063 CE1-related: Colour-plane-based escape pixel coding [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 15:30-16:00.)

In HEVC residual decoding, the decoder decodes the transform block (TB) one by one among three colour components. Therefore, in HEVC decoder architecture, only a coefficient buffer for single colour component is required in entropy decoder. In SCC palette mode coding, there is no residual to be decoded, so it is suggested that the residual coefficient buffer can be reused to store the palette index map information, including palette index and escape values. However, in SCM-4.0 escape pixel coding, the escape values of three components of one sample are coded together. It is thus reported that three coefficient buffers for three colour components are required in the worst case, which increases the implementation cost and complexity of palette mode. This contribution proposes a colour-plane-based escape pixel coding to reduce the buffer requirement of escape pixels coding. The escape values of the same colour component are grouped together first and then signalled group by group. The entropy decoder can decode the escape values of one colour component at a time, so that only one coefficient buffer for single colour component is required. The experimental results reportedly show that there is no BD-rate change.

The only syntax change is a change of loop nesting – swapping the nesting of the loop for color components and the loop for the number of escape-coded pixels.

The proponent pointed to IPCM as another example where spatial grouping of component groups is ordered in the same manner.

It was noted that in the 4:2:0 and 4:2:2 cases, the number of colour components varies from pixel to pixel, and remarked that this is part of the justification for sending the colour planes separately for the IPCM case. It was discussed whether it seems easier to perform the parsing if the data is grouped as proposed, but the answer did not seem entirely clear.

It was remarked that a smaller buffer could be used (at least theoretically) with the proposal since only a buffer large enough to hold a single component would be needed rather than a buffer big enough to hold all three colour components.

U0087 approach #2 proposes the same change. See notes on U0087.

JCTVC-U0154 Cross check CE1-related: Colour-plane-based escape pixel coding (U0063) [W. Pu (Qualcomm)] [late] [miss]

JCTVC-U0087 Syntax Cleanup for Palette Mode [W. Pu, R. Joshi, T. Hsieh, M. Karczewicz, F. Zou, V. Seregin (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 16:00-16:30.)

In the current HEVC SCC draft specification, new palette entries are signalled by grouping each color component while the escape pixels are signalled by interleaving the three color components. In this contribution, two unification methods are proposed to clean up the palette mode syntax.

Approach #1 is to swap the loops for sending the palette entries so that they are sent in component-interleaved form as is done with the escape-coded entries.

Approach #2 is to swap the loops for sending the escape-coded entries so that they are sent in component-grouped order.

Approach #2 of this is the same as proposed in U0063.

It was suggested that approach #1 is more consistent with the current PPS palette predictor syntax.

It was suggested that approach #2 could use less memory if the decoder fills in the decoded values into the residual buffer(s) as it performs the decoding process (versus filling in an indexed array of buffered content that is scanned later to reconstruct the CU residual).

It was suggested that approach #2 is more consistent with how we do most of the design (e.g., transform blocks and DPCM residuals and scanned in component-priority order).

It was not entirely clear whether it really matters, but one needed to be chosen.

Decision: Adopt approach #2 (also swap the loop order to reflect this in the PPS).

JCTVC-U0064 CE1-related: Palette coding with inter-prediction [W. Zhu, K. Zhang, X. Zhang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:15-18:30.)

This contribution presents a new palette mode named inter-palette, which combines the palette mode and the inter mode directly. When a CU is coded as the inter-palette mode, the motion information is signalled in the same say as the inter mode with SIZE2Nx2N. Thus a prediction block can be generated by motion compensation. And samples in the CU are coded in the same way as the palette mode, except that an additional copy method ‘COPY_INTER’ is appended besides existing INDEX and COPY_ABOVE. If a sample chooses COPY_INTER, it will copy the value of the corresponding sample in the prediction block directly. Experimental results reportedly show that the segmental prediction method can achieve 2.6%, 1.8% and 0.9% BD-rate savings under AI, RA, and LB conditions, respectively with constrained intra block copy searching; and 2.0%, 1.3% and 0.7% BD-rate savings under AI, RA, and LB conditions, respectively with full-frame intra block copy searching for “RGB, text & graphics with motion, 1080p & 720p sequences”.

It was commented that it seems late in the process to try to propose a substantially different additional coding mode type.

It was commented that the gain is also not so large, and encoder optimization was mentioned as a potential factor that could be affecting our current performance.

It was commented that U0116 may have some similarities, although it is referencing the current picture rather than a different reference picture.

No action was take on this.

JCTVC-U0162 Cross-check of CE1-related: Palette coding with inter-prediction (JCTVC-U0064) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0066 CE1-related: Row-based copy pixel from neighbouring CU [T.-D. Chuang, Y.-C. Sun, J. Kim, Y.-W. Chen, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:30-18:45.)

In this contribution, a row-based copy pixel from neighbouring CU is proposed to reduce the complexity of CE1 Test A1 and A2 while maintaining the coding gains. A NumCopyPixelRow is first signalled to indicate the number of rows copied from the neighbouring CU. The rest of rows in the CU are coded by the original palette index map coding in SCM-4.0. Experiment results show that, compared with SCM-4.0 with full-frame IntraBC search, 1.4%, 0.9%, and 1.0% BD-rate savings are shown for “YUV, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively; 1.8%, 1.3%, and 1.0 % BD-rate savings are shown for “RGB, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively.

Similar comments were made for this proposal as for U0064.

No action was take on this.

- JCTVC-U0168 Crosscheck of JCTVC-U0066: CE1-related: Row-based copy pixel from neighbouring CU [F. Zou (Qualcomm)] [late]

JCTVC-U0086 CE1-related: Simplification on coding NumPaletteIndices [J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:00-18:15.)

This contribution proposes to modify the derivation process of NumPaletteIndices, i.e. the number of palette indices signalled for the current block. Two methods are provided to reduce the condition checks in the derivation process of NumPaletteIndices. Experimental results reportedly average -0.1% to 0.1% Luma B-D rate changes across all testing configurations and classes by using the proposed methods, compared with SCM4.0 anchor.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

It was commented that U0086 method 1 seems very simple and straightforward and understandable (NumPaletteIndices = (num_palette_indices_minus1 + 1), although it does have a (tiny) loss in some cases.

It was commented that method 2 could have some penalty for encoders that don't check whether all palette entries are used.

Decision: Adopt U0086 method 1.

JCTVC-U0093 Non-CE1: On Number of Palette Indices coding [S.-H. Kim, K. Misra, J. Zhao, A. Segall (Sharp)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:10-18:15.)

In current SCC draft, num_palette_indices_idc is partitioned into 3 non-contiguous sets and the variable NumPaletteIndices is derived using an equation selected based on the set membership of num_palette_indices_idc. The existing derivation process is asserted to be complex. This contribution proposes the use of a fixed predictor (2* MaxPaletteIndex) and a signaled residual to derive NumPaletteIndices. This approach is asserted to be much simpler. It is reported that the proposed simplification provides a maximum luma bit rate difference of 0.1% for all configurations including AI, RA, LD(B) for lossy and lossless, and for 444 and 420 format.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

The proposal replaces one syntax element with two, for which one of them is sometimes not sent. It was commented that this is a bit more complicated than other approaches, and thus perhaps not the best.

JCTVC-U0125 Cross-verfication of JCTVC-U0093: Non-CE1: On Number of Palette Indices Coding [X. Xiu (InterDigital)] [late]

JCTVC-U0099 Modified method for sending number of palette indices [R. Joshi, W. Pu, M. Karczewicz, F. Zou, V. Seregin (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:15-18:15.)

It is asserted that there is a mismatch between HEVC SCC text specification version 3 and SCM 4.0 software in the derivation of NumPaletteIndices. Furthermore a modification of the derivation of NumPaletteIndices is proposed. This modification removes one condition check and associated operations. Simulation results are provided to show that there is virtually no BD-rate change due to this modification.

The contribution notes that the current text has an error, which would need to be corrected if some other approach is not chosen.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

It was commented that this is a bit more complicated than other approaches, and thus perhaps not the best.

JCTVC-U0144 Cross-verification of JCTVC-U0099: Modified method for sending number of palette indices [X. Xiu (InterDigital)] [late] [miss]

JCTVC-U0105 Non-CE1: Simplification on deriving NumPaletteIndices for palette mode [X. Xiu, Y. He, Y. Ye (InterDigital)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 17:25-18:15.)

This contribution proposes to modify the derivation of NumPaletteIndices, which specifies the number of palette indices signalled for the current block. Compared to SCM-4.0 anchor, the proposed method reportedly has no impact on the average coding performance. It is asserted that the proposed modification simplifies the decoding of NumPaletteIndices.

Contributions U0086 (methods 1 & 2), U0093, U0099, U0105 are closely related to each other.

This proposal would add a constraint that disallows sending palette entries that are not use in the CU. It was commented that the current reference software sometimes does this, and that disallowing it might sometimes require an encoder to perform an extra pass to avoid letting this happen. It was therefore suggested not to choose this particular approach.

JCTVC-U0155 Crosscheck of JCTVC-U0105: Non-CE1: Simplification on deriving NumPaletteIndices for palette mode [W. Digonnet, C.-C. Lin, J.-S. Tu, Y.-J. Chang, C.-L. Lin (ITRI)] [late]

JCTVC-U0131 Crosscheck for simplification on coding numPaletteIndices (JCTVC-U0086) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-U0090 CE1-related: Palette Mode Context and Codeword Simplification [J. Ye, J. Kim, S. Liu, P. Lai, W. Zhu, K. Zhang, T.-D. Chuang, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:45-19:00.)

This contribution proposes to modify the palette coding process from three aspects.

- First, the context for last_palette_run_type_flag is removed, and last_palette_run_type_flag shares the context with palette_run_type_flag. Experimental results of item1 report average -0.13% to 0.17% Luma B-D rate changes across all testing configurations and classes, compared with SCM4.0 anchor. This was also proposed in U0148. It reduces the number of contexts by 1 and eliminates an apparently-useless need to treat the last palette run type differently. Decision: Adopt.

- Second, palette_transpose_flag is signalled after the last_palette_run_type_flag to group the CABAC context coded bin. Experimental results of item2 report no B-D rate changes, compared with SCM4.0 anchor. This is part of what is contained in U0133. See notes for U0133.

- Third, a modified binarization for num_palette_indices_idc is proposed based on a simplification of a derivation of Rice parameter, to avoid a division by 6 – changing it from c=2+Idx/6 to c=2+((Idx+4)>>3). Experimental results of item3 report average −0.1% to 0.1% Luma B-D rate changes across all testing configurations and classes, compared with SCM4.0 anchor. It was remarked that this is closely related to U0169. See notes for U0169.

JCTVC-U0126 Crosscheck report of JCTVC-U0090 [S.-H. Kim (Sharp)] [late]

JCTVC-U0148 Non-CE1: Context model unification for palette run type flags in HEVC SCC [M. Xu, W. Wang, F. Duanmu, H. Yu (Huawei)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 18:45-19:00.)

This proposal is the same as item 1 of U0090. See notes for that contribution.

JCTVC-U0166 Cross-check of Non-CE1: Context model unification for palette run type flags in HEVC SCC (JCTVC-U0148) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0133 Comment on signalling the palette_transpose_flag after last_palette_run_type_flag (JCTVC-U0090) [R. Joshi, V. Seregin, M. Karczewicz, W. Pu, F. Zou (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:00-19:10.)

In JCTVC-U0090, it is proposed that the palette_transpose_flag be signalled after last_palette_run_type_flag to group bypass bins together. It is asserted that when syntax elements related to palette delta QP or palette chroma QP offset are present, this does not result in grouping of more than one additional bypass bin. It is proposed that in addition to placing the palette_transpose_flag after last_palette_run_type_flag, the syntax elements related to palette delta QP and palette chroma QP offset be signalled after the palette_transpose_flag. It is asserted that due to this reordering, all the bins up to last_palette_run_type_flag are bypass-coded and grouped together.

Part of this proposal is also proposed in item 2 of U0090.

The basic motivation is to group together bypass-coded data in the palette mode syntax.

Decision: Adopt.

JCTVC-U0169 CE1 Related: Simplification of num_palette_indices_idc coding parameter derivation [F. Zou, V. Seregin, M. Karczewicz, W. Pu, R. Joshi (Qualcomm)] [late]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:10-19:20.)

This document presents a simplification method of Rice parameter derivation of numIndices in current palette coding. The results were tested on top of SCM4.0 as well as on top of CE1 A.1 and A.2. Simulation results reportedly show that the proposed simplification does not impact the performance of SCM4.0 for AI full frame IBC.

This is closely related to item 3 of U0090.

Like U0090, a modified binarization for num_palette_indices_idc is proposed based on a simplification of a derivation of Rice parameter, to avoid a division by 6 – changing it from c=2+Idx/6 to c=2+(Idx>>3).

Basically no effect on coding efficiency was reported.

It was remarked that since U0086 method 1 changed the values that will be coded by this, a new test should be run to identify whether the modified Rice parameter derivation would have a coding efficiency effect under the new circumstances. Using the modified non-normative encoding method U0096 in further testing would also be desirable, just to make sure it does not affect anything.

Revisit this and U0090 after obtaining additional test results for each.

JCTVC-U0091 CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft [J. Ye, S. Liu, P. Lai, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:20-19:25.)

This contribution reports a mismatch between the SCM 4.0 software and HEVC SCC working draft (WD). In specific, this mismatch is about how “maxPaletteRun” is derived when palette mode is used. Fixes on WD to match SCM 4.0 is provided, as well as fixes on SCM4.0 to match the WD. Simulation results report average up to −0.13% to 0.17% luma B-D rate changes across all testing conditions and classes when the software is modified to align with the WD.

Decision (BF): Change the text to match the software.

JCTVC-U0124 Crosscheck of JCTVC-U0091, CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft [R. Cohen (MERL)] [late]

JCTVC-U0089 Constraints on palette syntax elements [P. Lai, J. Kim, S. Liu, J. Ye, T.-D. Chuang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:25-19:55.)

This contribution presents constraints on palette syntax elements.

- A constraint on the SPS parameter palette_max_size was proposed to require the palette_max_size to be in the range of 0 to 1<<(MaxTbLog2SizeY*2). However, this does not seem fundamentally necessary. The maximum palette size would have some constraint such as a profile/level-specified constraint, and this seems sufficient.

- Decision (Ed.): Regarding constraining the sensibility of NumPaletteIndices, instead constrain the derived position to lie within the CU.

- Regarding constraining num_signalled_palette_entries, no action seemed needed because of other constraints that are already specified.

- Decision (Ed.): Regarding palette_predictor_run, constrain the derived position within the palette predictor to not exceed the size of the palette predictor.

JCTVC-U0163 Cross-check of Non-CE1: Palette Run Hiding (JCTVC-U0094) [B. Li, J. Xu (Microsoft)] [late] [miss]

JCTVC-U0094 Non-CE1: Palette Run Hiding [M. Karczewicz, W. Pu, R. Joshi, F. Zou, V. Seregin (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 19:55-20:05.)

It is observed that in palette mode, the first or the last run in the block is generally longer than the average run length for the block. In this document, it is proposed to use a flag to indicate whether the first or the last run length is not coded in the bitstream. It is reported that for RGB and YUV text & graphics with motion, 0.2% and 0.3% BD-rate reduction are achieved, respectively, for All-Intra configuration.

It was commented that although the idea seems interesting, it complicates the syntax and design while providing very little gain. No action was taken on this.

JCTVC-U0096 CE1 Related: Improved palette encoder [M. Karczewicz, F. Zou, V. Seregin, W. Pu, R. Joshi (Qualcomm)]

(Consideration of this topic was chaired by GJS on Friday 06-19, 15:10-15:25.)

This document proposes encoder-only changes for palette mode in the current SCM-4.0 software. The changes include: 1) using squared error to derive the palette and for index assignment; 2) eliminating some of the palette entries if the related pixels can be represented by other palette indices; 3) merging of two consecutive runs and jointly optimizing the run values of index run and copy above run pair. Experimental results reportedly show that compared to SCM-4.0 anchor on 4:4:4 sequences, the proposed combination reportedly provides the average 1.3% and 1.8% BD-rate savings for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively in AI with IBC full frame. And 1.5% and 2.2% BD-rate savings for the category text & graphics with motion, 1080p & 720p in both RGB and YCbCr color formats respectively in AI with IBC 4CTU. For 4:2:0 sequences, the corresponding BD-rate savings of the proposed method are 1.2% for AI.

Part of this is similar to U0127, but in a somewhat simplified/sped-up form.

When tested in combination with normative changes from subtest C.1, additional gain was shown, but with most of the gain from the non-normative aspects.

When compared to U0127, this contribution includes additional non-normative improvements, which provide additional gain.

The run-time is somewhat slower than the anchor, by about 6-8% slower.

Decision (N-N): Adopt.

JCTVC-U0145 Cross-verification of JCTVC-U0096: Improved palette encoder [X. Xiu (InterDigital)] [late] [miss]

JCTVC-U0097 Non-CE1: On maximum palette predictor size [Y.-J. Chang, C.-C. Lin, C.-L. Lin, J.-S. Tu (ITRI)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 09:00-10:00.)

Although the SCM4.0 sets the maximum palette predictor size as 128, there is no upper limitation for the maximum palette predictor size in the SCC draft text. This contribution presents four optional constraints and a cleanup for the palette predictor size. The contribution also reports the rate-distortion evaluation on different maximum palette predictor sizes, whose numbers are smaller than 128. It is reported that smaller values of the maximum palette predictor size such as 80, 96 and 112 only change 0.0-0.2% coding performance on All-Intra lossy coding and improves the coding time up to 1%.

It was remarked that the plan was to express limits in profile/level specifications; the lack of a currently-expressed limit is merely because we don't yet have a profile/level specification for SCC. The CTC establishes limits of 63 and 128 for the max palette size and max palette predictor size, respectively.

The contributor reported that the software currently requires recompilation to change these, as they are set by a macro rather than, or in addition to, a config parameter. Decision (SW): Fix this (if necessary and feasible) by managing the sizes in the config file so that experiments with different sizes (within reason) can be run without recompiling.

The contributor suggested having the support max palette predictor size be up to 128 or 96 or twice the max palette size or 1.5 times the max palette size. It was commented that a fixed number seems like a simple and adequate approach, and there is no apparent need to make the constraint different for different max palette sizes.

An editorial error in the spec was also reported, with naming confusion between max_predictor_palette_size and PaletteMaxPredictorSize. Decsion (Ed.): Fix this.

Test results were reported with max palette predictor sizes of 112, 96, 80 (relative to the current 128 as anchor). For lossy coding, the penalties were small (average penalty in each class at most 0.3%). For lossless coding, the penalties were somewhat larger (average penalty in each class at most 0.7%).

It was remarked that we should just make the limit be 128 unless it seems like the complexity savings for reducing it would be significant.

JCTVC-U0143 Crosscheck report of JCTVC-U0097 [S.-H. Kim (Sharp)] [late]

JCTVC-U0109 Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization [Y.-J. Chang, C.-L. Lin, C.-C. Lin, J.-S. Tu, W. Digonnet (ITRI)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 10:00-10:20.)

In the current palette design, the pixel-level palette run value is derived from the combination of the index of the most significant bit (MSB) and the refinement bits, i.e., the syntax elements palette_run_msb_id_plus1 and palette_run_refinement_bits. The first five bins of palette_run_msb_id_plus1 are context-coded in the current design, and the other bins are coded in bypass mode. It is noted that larger CUs usually have more palette runs , a 32x32 CU coded by palette mode will result in slower CABAC throughput on the parsing of the whole palette run values than for the other CU sizes, due to dependency of context-coded bins. It is observed that the fourth and fifth bin have approximately equal occurring probability on binary numbers (which is either 0 or 1). Therefore, this contribution proposes to simplify the derivation process of palette_run_msb_id_plus1 by using the bypass mode to code the fourth bin and the fifth bin. It is reported the BD-rate increases are 0.0-0.3% per class with an average across all classes smaller than 0.1%.

Results were provided for moving the bit plane switch-over by two bit positions, but not by moving it just one position.

It was commented that this does not affect the worst case, which is the case where the runs are all small. Larger runs are easier to handle than smaller runs, so making larger runs simpler doesn't help much. It was remarked that as a percentage of the overall total complexity of the decoding process, there may really be no measurable benefit, since it would only help when the mode is palette and the blocks are large and the runs are large, which are not what is likely to be what is consuming much of the total processing requirements of the decoding process.

No action taken on this.

JCTVC-U0136 Cross-checking of JCTVC-U0109: Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization [Y. He, X. Xiu, Y. Ye (InterDigital)] [late]

JCTVC-U0116 Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding [Tao Lin, Kailun Zhou, Liping Zhao, Xianyi Chen (Tongji)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 10:20-11:15.)

A "palette with pPxel cCoy" (PPC) tool is proposed to be added under the existing palette mode and coexists with all of the traditional palette coding tools. A CU is either coded by PPC or traditional palette mode. The palette predictor is shared. The palette in PPC is used only for LEFT copy identical to traditional palette mode. The search range for PPC is either 2CTU (current and left one) or 4CTU (current and left three). The coding performance is evaluated under two configurations of search/reference range: (TC1) FF IBC vs 2CTU PPC and (TC2) constrained 4CTU IBC vs 4CTU PPC. Using SCM40 as anchor, their coding results for Y component of YUV TGM AI lossy coding are reported as −4.2% and −7.7%, respectively for TC1 (full-frame IBC) and TC2 (4x1 IBC). The reported encoding time ratio is 5-6%.

The concept of the new mode/submode is similar to intra string copy. The proposed submode has alternating runs of strings coded by a simplified palette method and sample-copied strings.

There was a discussion of the application of this to 4:2:0 coding and how this would work. The contributor said that it was designed to function for 4:2:0. It was remarked that 4:2:0 test results may be missing from the contribution.

The proponent indicated that, relative to prior studies of ISC, the new proposal has more coding efficiency improvement and much lower encoding complexity. The additional improvement of coding efficiency was reportedly from making an effective combination of palette and ISC within a CU and from fixing some bugs.

The contributor reported that CUs that don't use the palette mode interrupt the palette prediction process, and said that allowing references to samples is a way of making values in nearby regions coded by such other modes accessible to a palette coded region.

It was asked why the encoder complexity is not higher, due to the need to search for strings of samples to copy.

A participant remarked that the worst-case memory access behavior of the proposed mode may be difficult to support.

The amount of text to be added is substantial.

There is an interaction between constrained intra prediction and the proposed mode.

Although the reported gain is interesting, given the late stage of the project, it seems very difficult to contemplate adding a new mode. Adopting this would put our schedule at serious risk.

No action taken. However, the JCT plenary reporting from Track A should highlight this topic.

JCTVC-U0151 Cross-check report of U0116: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding [W. Wang, M. Xu (Huawei)] [late] [miss]

JCTVC-U0146 CE1 Related: Combination of JCTVC-U0101 reverse scan and JCTVC-U0096 palette encoder improvement [V. Seregin, F. Zou, M. Karczewicz, W. Pu, R. Joshi (Qualcomm), X. Xiu, Y. Ye, Y. He (InterDigital)] [late]

See notes on U0021.

r

JCTVC-U0149 Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC [M. Xu, F. Duanmu, W. Wang, S. Minaee, H. Yu (Huawei)] [late]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 11:30-12:00.)

This contribution presents an index map coding scheme by splitting the index map into four equal-sized square sub index maps (SIMs). These SIMs share the same palette table derived from the current CU and coded using existing index map coding approach (i.e., index mode and copy-above mode, respectively) with adaptive scanning order. This scheme suggests improving the system performance by simultaneously processing the SIMs in parallel. Currently this index map splitting mode is applied on palette coded CUs with dimension ranging from 32x32 down to 8x8. Experimental results reportedly demonstrate a negligible BD-Rate performance loss (for lossy BD-Rate 1080p text & graphics content encoded using All intra configuration) but potential 4x processing efficiency improvement for index map encoding and decoding.

In the tested scheme, the splitting was not forced to be used – the encoder performed RDO decisions to determine whether to split the CU or not, and an extra flag was sent. The flag becomes a sort of palette sharing flag to signal palette sharing among 4 pseudo-CUs.

It was asked why not to split the CU into smaller CUs (perhaps using the same palette) rather than introducing a new level of conceptual block hierarchy.

The parsing is sequential – that part is not parallelized, but subsequent copying is proposed to be parallelized. It was commented that the ability to parallelize the copying is not so beneficial, such that the parsing is the main bottleneck issue, which needs to be sequential.

It was remarked that palettes and palette predictors are established during the parsing process, and within each CU the processing for palette mode is separate and thus the copying process for multiple CUs that are coded in palette mode can already be parallelized.

Our current maximum CU size for palette mode coding is 32x32, and the contributor said they had done some testing of the coding efficiency impact of disabling some block sizes, but only by disabling the smallest block sizes rather than keeping the smaller block sizes and disabling the larger ones.

A participant reported that our prior decision to disable the 64x64 size had a coding efficiency penalty of about 0.3-0.4%. Disabling 32x32 also would presumably have a larger coding efficiency impact, but the amount was uknown.

No action taken.

JCTVC-U0167 Cross-check of Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC (JCTVC-U0149) [B. Li, J. Xu (Microsoft)] [late] [miss]

JCTVC-U0036 On Zero Maximum Palette Size [K. Misra, S.-H. Kim (Sharp)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 12:00-12:15.)

Current SCC draft design allows maximum palette size to be set to 0. Zero palette sizes can be used to include blocks of quantized sample values within the bitstream.

Note, when maximum palette size is set to 0, it constraints the valid value for a subset of syntax elements. However the current SCC draft allows these syntax elements to take on invalid values. It is asserted that in such an event it is desirable to omit the inclusion of these syntax elements since the existing inference rules lead to valid values for the corresponding syntax elements. An alternative approach is to explicitly state conditional bitstream conformance requirements on the syntax element values.

The proposal concerns the high-level syntax.

When the palette_max_size is 0, it is proposed to not send the palette_max_predictor_size or to constrain its value. (Both of these are currently SPS-level information.) It was remarked that this change is not really necessary, as the current syntax will still function properly, although it may not seem very "clean" to send a non-zero value. Decision (BF/cleanup): Require palette_max_predictor_size to be equal to 0 when palette_max_size is 0.

It was proposed to condition the presence of palette_predictor_initializer_present_flag or to constrain its value based on the value of palette_max_size and palette_mode_enabled_flag. It was remarked that the presence conditioning would create a parsing dependency on PPS parsing using SPS syntax. Decision (BF/cleanup): Require palette_predictor_initializer_present_flag to be equal to 0 when palette_max_size or palette_mode_enabled_flag is 0.

JCTVC-U0052 On the palette escape pixel coding [B. Li, J. Xu (Microsoft)]

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

Escape pixels are supported in the palette mode. However, the binarization process of escape pixels depends on the CU QP values. ItThis introdus a parsing dependency between the palette escape pixel and QP value. This document presents several methods to remove the parsing dependency between the palette escape pixel and the CU QP value.

TBP.

JCTVC-U0153 Cross check: On the palette escape pixel coding (U0052) [W. Pu (Qualcomm)] [late] [miss]

JCTVC-U0084 On palette predictor initialization for Screen Content Coding [J. Ye, S. Liu, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 15:00-XX1600.Xo)

This contribution proposes to allow signalling a set of palette predictor initializers in SPS to reduce the overhead of signalling them in each PPS under some circumstances. When sps_palette_predictor_initializer_present_flag is equal to 1, a set of palette predictor initializers are signalled in SPS for the current CVS to use; otherwise, the entries in the palette predictor are initialized to 0. When pps_palette_predictor_initializer_present_flag is equal to 1, a set of palette predictor initializers are signalled in PPS; otherwise, the entries in the palette predictor are derived from SPS palette predictor initializers. Experimental results report similar B-D rate performance by using the proposed method and the PPS palette predictor initializer scheme in SCM4.0 for AI configuration. B-D rate savings are observed when the proposed method is used for inter configurations.

The amount of data for these syntax elements is reported to be about 400 bytes.

Scaling list data was suggested as a similar example of SPS data with PPS override.

It was asked whether we have anything similar in extensions.

It was noted that the reason for sending a new PPS may have nothing to do with wanting to send different palette initialization data, but as things currently stand, it would be necessary to send the palette initialization data whenever changing the PPS in any way.

Decision: Adopt.

JCTVC-U0130 Crosscheck for palette predictor initialization for Screen Content Coding (JCTVC-U0084) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-U0114 Generate the palette predictor initializers with low delay [J. Ye, T.-D. Chuang, S. Liu, Y.-W. Huang, S. Lei (MediaTek)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 15:30-15:45.)

This contribution proposes to generate the palette predictor initializers with low delay. In the current method, multipass operation on the whole current picture is performed for each picture.

In the proposed method, the first picture generates the palette predictor initializers from itself (as is done now), and for the rest of the pictures, the previous picture is used for generating the palette predictor initializers. Experimental results report similar B-D rate performance by using the proposed method and the current scheme in SCM4.0 for AI, RA and LB configurations. Basically no difference in coding efficiency (perhaps a tiny improvement) was reported.

Decision (N-N SW): Adopt.

JCTVC-U0141 Crosscheck of Generate the palette predictor initializers with low delay (JCTVC-U0114) [C. Gisquet (Canon)] [late]

JCTVC-U0171 Non-CE1: Palette with Pixel Copy (PPC) decoder IC reference design [Kailun Zhou, Tao Lin (Tongji)] [late]

TBP.

JCTVC-U0173 Crosscheck of Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding (JCTVC-U0116) [R.-L. Liao, C.-C. Chen, W.H. Peng, H.-M. Hang (NCTU/ITRI)] [late] [miss]

### CE2 related (intra block copy memory access) (1)

(Consideration of this topic was chaired by JRO on Friday 06-19, 16:15-16:30.)

JCTVC-U0065 CE2-related: Intra block copy searching constraints for reducing worst case bandwidth [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek)]

In HEVC-SCC, the IntraBC predictor comes from the unfiltered reconstructed pixels of the current picture. The unfiltered reconstructed pixels are required to be outputted which increases the worst case memory bandwidth for HEVC-SCC. In this contribution, the worst case bandwidth of HEVC and HEVC-SCC is analyzed. Two IntraBC encoder searching constraints are proposed to reduce the worst case bandwidth. In Method-1, a current picture is divided into 8×8 blocks. An 8×8 block cannot be referenced by IntraBC blocks if this 8×8 block is coded as an 8×8 bi-predicted block, two 4×8/8×4 uni-predicted blocks, or it is belongs to a 16×16 CU which is coded by 4×16+12×16 bi-predicted blocks or 16×4+16×12 bi-predicted blocks, and the 8×8 block is not IntraBC coded block and the use_integer_mv_flag is 0. In method-2, an additional constraint is added that if one MV in this 8×8 block is integer MV, this 8×8 block is allowed to be referenced by IntraBC blocks. With the proposed methods, the worst case bandwidth of the HEVC-SCC will be the same as the worst case bandwidth of the HEVC. Experimental results reportedly show that for method-1, no BD-rate losses are shown under AI configuration, 0.5% to 0.6% BD-rate losses are shown under RA configuration, and 0.7% to 1.0% BD-rate losses are shown under LB configuration. For method-2, no BD-rate losses are shown under AI configuration, 0.3% to 0.4% BD-rate losses are shown under RA configuration, and 0.2% to 0.6% BD-rate losses are shown under LB configuration.

Similar idea as in CE2 test 5, but as encoder/bitstream constraint

The contribution does not contain draft text, but it is asserted that it would not be simple to specify the method as a bitstream constraint, and may not be easy for an encoder to implement it.

No action.

### IBC/MC unification aspects (24)

(Consideration of this topic was chaired by JRO on Friday 06-19, 16:30-20:20, continued Saturday 06-20, 9:20-12:00.)

JCTVC-U0056 Unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes [G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon)]

This contribution is related to the Intra block copy and Inter unification. In the current SCM4.0, the Intra block copy method is signaled as Inter modes by setting the current frame as a reference frame. But the block vectors have systematically the full-pel resolution in opposition to the motion vectors which should be in full-pel or sub-pel resolution according to the value of use_integer_mv_flag in the slice header. In this contribution, it is proposed to unify the use of the adaptive motion vector resolution for IBC in order to unify the usage of full-pel or sub-pel vector for Inter modes for all reference frames. There is no impact on coding efficiency over SCM4.0.

Presentation deck to be uploaded.

The approach is at the slice level to signal the MV resolution separately for each reference picture in each list. The benefit is not obvious, does not simplify the text, and introduces parsing of additional flags at slice header.

JCTVC-U0159 Cross-check of JCTVC-U0056 on unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes [K. Rapaka (Qualcomm)] [late] [miss]

JCTVC-U0057 On Inter modes constraint for slices [G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon)]

This contribution relates to Inter prediction constraint for Inter prediction. In SCM4.1, an Inter block predictor from the current frame can’t come from another slice than the current one. This new constraint wasn’t present in HEVC v1. The losses of coding efficiency of a slice configuration compared to CTC is larger for screen content than for natural content and this may partly come from this constraint. This contribution proposes to enable or disable this constraint at sequence or slice level. The BDR results are in average, for SCC sequences and mixed content sequences, -13.6%, -8.3%, -3.4% for respectively AI, RA and LB configurations when using slices of 1500B.

Presentation deck to be uploaded.

The contribution refers to a configuration with fixed-size slices where the penalty is typically large. It is demonstrated that some of this penalty can be reduced (e.g. 30% vs. 40% BR increase for all-intra case).

During the discussion, it is pointed out that allowing IBC across slices might introduce some additional problems with tiles. Furthermore, this restriction was introduced by purpose to enable more stability in intra coded pictures under lossy transmission, and such cases are not considered in the contribution.

It is also pointed out that by using dependent slices, such a functionality could already be implemented.

No action

JCTVC-U0160 Cross-check of JCTVC-U0057 on Inter modes constraint for slices [K. Rapaka (Qualcomm)] [late] [miss]

JCTVC-U0076 Unification of motion vector resolution for screen content coding [X. Xu, S. Liu, S. Lei (MediaTek)]

In SCM-4.0, it is intended to unify the intra block copy operation with inter coding. However, a discrepancy exists in which the motion vectors pointing to the current reference picture are always integer-pel resolution while motion vectors pointing to temporal reference pictures can switch between integer-pel resolution and quarter-pel resolution at slice level. This contribution proposes to unify the resolutions of all motion vectors in the same slice. When the slice level use_integer_mv_flag is equal to 1, all motion vectors in this slice are integer-pel resolution; when this flag is equal to 0, all motion vectors in this slice are quarter-pel resolution. In particular, if the current picture is the only one reference picture available for the current slice, the use_integer_mv_flag for this slice is signalled or inferred to be 1. Experimental results report average 0.0 to 1.0% B-D rate increase for RA and LB 444 lossy configurations by applying the proposed unification method. The B-D rate change for AI is negligible.

The proposal saves approximately 6 lines in the specification text, but would need additional bitstream constraint in case of only using integer motion for IBC. Enforcing integer*4 motion vectors causes BR increase (RA 1%) according to the results.

For AI case, integer-pel MV is signalled in the proposal (therefore no loss).

Other methods exist which achieve similar unification without losses (see under U0081).

JCTVC-U0129 Crosscheck for unification of motion vector resolution for screen content coding (JCTVC-U0076) [W. Zhang, L. Xu, Y. Chiu (Intel)] [late]

JCTVC-U0077 Chroma motion vector derivation for intra block copy [X. Xu, S. Liu, S. Lei (MediaTek)]

In SCM-4.0, it is intended to unify intra block copy (IntraBC) with HEVC inter coding. However, the resolution of block vectors (i.e. motion vectors pointing to current picture) is always integer-pel, for both luma and chroma components, while the resolution of motion vectors pointing to temporal reference pictures can switch between integer-pel and quarter-pel at slice level. With this difference, the derivation process of chroma motion vectors for IntraBC blocks is different from that for blocks coded with HEVC inter mode. This contribution proposes several methods to resolve this difference and unify the chroma motion vector derivation procedure. In method A, an extra few rows and columns of pixels are restricted from being used as reference when a motion vector points to current reference to make sure the surrounding pixels which may be used for interpolation are all available. In method B, the allowed range of block vectors remain the same as SCM-4.0, instead extra rows and columns beyond the right and bottom boundaries of the reference area are padded by boundary or prefixed pixel values. Experimental results show that:

- The proposed method A.1 reports no change to 444 coding; it reports average 2.3%, 1.2% and 0.8% BD-rate increase for RGB TGM 1080p & 720p in AI, RA and LB 420 lossy coding, respectively;

- The proposed method A.2 reports no change to 444 coding; it reports average 2.8%, 1.6% and 0.8% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 420 lossy coding, respectively;

- The proposed method B.1 reports no change to 444 coding; it reports average 3.2%, 1.9% and 1.0% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 420 lossy coding, respectively;

- The proposed method B.2 reports no change to 444 coding; it reports average 3.2%, 1.9% and 0.9% BD-rate saving for RGB TGM 1080p & 720p in AI, RA and LB 444 lossy coding, respectively;

Chroma interpolation is used for cases of odd integer motion vectors. The approach A uses padding from version 1 at the picture boundary, and constraint by a 2-row/colum safety margin at CTU boundary (to avoid that samplefrom curent CTU are involved in IBC reference). A1 is using the constraint for all even and odd cases (which gives loss), whereas A2 constrains only the odd vector cases which provides interesting gains. The cases B also implement padding at CTU boundaries (which appears undesirable because it is more complexity and IBC specific operations which are not in the spirit of unification).

Most of the gains likely are more influenced from the CTU boundaries rather than the picture boundaries.

See further conclusion under U0103.

JCTVC-U0080 Intra block copy chroma interpolation for Non-444 [K. Rapaka (Qualcomm)]

During the 20th JCTVC JCT-VC meeting held in Geneva, the alignment of Intra block copy (IBC) with inter signaling, as proposed by JCTVC-T0227, was adopted. In the unification framework, luma and chroma block vectors are restricted to be in integer precision thus the prediction samples are generated without interpolation. However, for non-444 chroma formats, chroma block vectors may point to a fractional samples. For these case, in the current working draft chroma block vectors are rounded to integer pel accuracy. In this contribution it is proposed to enable fractional chroma interpolation for IBC block. In the process the IBC search region is reduced by interpolation samples when interpolation is applied. Compared to SCM 4.0 anchor on 420 sequences the proposed method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI of {2.1%, 2.7%, 27%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. Additional results with encoder optimization to compute SAD based on interpolated chroma reference samples are included. With this optimization. Compared to SCM 4.0 anchor on 420 sequences the proposed method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI of {2.5%, 3.1%, 3.0%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. The spec text changes are highlighted in red and yellow text.

Same as JCTVC-U0077 A2, but instead of padding at picture boundary, it also uses the 2-row/column margin. Therefore, slightly less gain.

See further conclusion under U0103.

JCTVC-U0139 Cross check report of JCTVC-U0080: Intra block copy chroma interpolation for Non-444 [X. Xu, S. Liu (MediaTek)] [late] [miss]

JCTVC-U0103 On chroma derivation of intra block copy for non-444 video [X. Xiu, Y. Ye, Y. He (InerDigital)]

In this contribution, two methods are proposed to modify the chroma block vector derivation process in the unification framework of intra block copy in order to improve the coding performance of intra block copy for non-444 videos. In the first method, the current chroma block vector clipping process is modified. In the second method, it is proposed to enable fractional chroma interpolation for intra block copy.

Compared to SCM-4.0 anchor on 420 sequences, the first method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {0.3%, 0.6%, 0.7%}, {0.1%, 0.3%, 0.5%} and {0.0%, 0.2%, 0.6%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. For lossless coding, the corresponding bit-rate savings are 0.1%, 0.1% and 0.1% for AI, RA and LB, respectively.

Compared to SCM-4.0 anchor on 420 sequences, the second method reportedly provides average {Y, Cb, Cr} BD-rate savings for AI, RA and LB of {2.5%, 3.1%, 3.0%}, {1.5%, 1.9%, 2.0%} and {0.8%, 1.4%, 1.6%} for the category YUV text & graphics with motion, 1080p & 720p, respectively. For lossless coding, the corresponding bit-rate savings are 0.6%, 0.9% and 1.0% for AI, RA and LB, respectively.

The first method uses a different rounding for fractional MVs, rounding towards zero.

The second method is identical with U0080 (spec text slightly different).

One experts points out that instead of the first method another option could be adding an offset and conventional rounding (instead of absolute value).

Conclusion about U0077 A2, U0080, and U0103:

In terms of impact on memory access: It cannot be argued that 4:4:4 is the worst case, because with same amount of reference picture memory 4:2:0 could use double picture size. However, the memory access should not be worse than in 8x8 bi-pred motion comp with quarter pel, as the luminance is still integer.

Clarify whether SCC would have a 4:2:0 profile, or whether it would support 4:2:0 in a generic profile, and how important gains in 4:2:0 are.

Following the discussion on one aspect of U0118 (releasing the BV vector constraint, and use normal padding at picture boundaries), this is unification is not necessarily desirable, and only gives small additional gain according to U0077 A2. Solution with the restriction of BVs by 2 luma samples margin in each direction (regardless if it is a picture boundary or CTU boundary) is asserted to be the most beneficial solution. Side activity (of proponents of U0077, U0080, U0103) to provide a unified text specification of this solution).

Include in side activity (K. Rapaka) on CE2 for worst case memory assessment, to confirm that it does not increase the worst case.

Discuss in JCTVC plenary whether we would desire a higher processing complexity of IBC (as per chroma interpolation) in case of 4:2:0 compared to 4:4:4. Note: The complexity would still be lower than in inter MC, but this might have consequences on worst-case complexity of intra-only profiles.

Revisit.

JCTVC-U0158 Cross-check of JCTVC-U0103 on chroma derivation of intra block copy for non-444 video [K. Rapaka (Qualcomm)] [late] [miss]

JCTVC-U0081 On unification of adaptive motion vector resolution [K. Rapaka (Qualcomm)]

During the 20th JCTVC JCT-VC meeting held in Geneva, the alignment of Intra block copy (IBC) with inter signaling, as proposed by JCTVC-T0227, was adopted. Also, in the current working draft (WD) adaptive motion vector resolution can be enabled using the syntax element use_integer_mv_flag. This contribution identifies several issues related motion vector (MV) derivation and storage when IBC and use_integer_mv_flag are enabled. Several solutions are proposed to unification design that resolve the identified problems. The performance impact of proposed solutions seems non-significant ranging from 0.0% to 0.1%.

Presentation deck to be provided.

Three methods are proposed

- Method 1 stores quarter-pel vectors in all cases. It unifies integer-pel motion comp and IBC, but moves the shift operation of integer pel motion comp. to the derivation stage (which was not originally intended when integer MC was adopted, since it was desired to keep the MV derivation stage untouched). Method 1also removes the clipping introduced in last meeting, and the inconsistency in deblocking and TMVP.

- Method 2 uses storage of quarter-pel vectors for both BV and MV in case of quarter-pel MC, and integer vectors in case of integer MC.

- Method 3 unifies integer-pel motion comp and IBC in the style of current integer motion comp. It still requires the clipping operation and does not solve the deblocking problem

Method 1 is identically proposed in 0107, method 3 in 0111.

Decision: Adopt Method 1

JCTVC-U0140 Cross check report of JCTVC-U0081: On unification of adapative motion vector resolution [X. Xu, S. Liu (MediaTek)] [late] [miss]

JCTVC-U0107 On stored decoded motion vector resolution [X. Xu, S. Liu, S. Lei (MediaTek)]

In SCM-4.0, the decoded motion vectors that point to a temporal reference picture are stored in either quarter-pel resolution or integer resolution, depending on the slice level flag use_integer_mv_flag. These stored motion vectors are used later as predictors for motion vector prediction and for boundary strength (BS) decision in deblocking filter stage. Two issues are discussed as follows: 1) HEVC inter deblocking filter process always requires motion vectors as input with quarter-pel resolution. 2) when temporal motion vector prediction is used, and the vector resolution of the current picture is different from the predictor in the reference picture, the prediction is not accurate. This contribution proposes to always store the decoded vectors using quarter-pel resolution to align with HEVC. Consequently, the motion vectors that are inputs to deblocking filter are consistently to be fractional-pel resolution such that more precise boundary strength decisions may be achieved. Furthermore, consistently stored motion vector resolutions also result in more precise motion vector prediction. Experimental results report up to average 0.2% B-D rate savings from the proposed method across all testing configurations and classes.

See under 0081

JCTVC-U0111 Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search [J.-S. Tu, C.-C. Lin, C.-L. Lin, Y.-J. Chang (ITRI)]

In the current unification scheme of Intra-Block Copy (IBC) and Inter prediction, additional operations for motion vector (MV) derivation process are required due to the inconsistency of MV resolution of IBC. This contribution proposes a MV resolution unification method, in which, the MV resolution of IBC would be set in integer-pixel precision without considering “use_integer_mv_flag”. The proposed scheme removes the redundant operations and further unifies IBC and Inter mode. The experimental results show that the BD-rate changes of the proposed method is -0.1% to 0.1% under lossy condition for 444-format.

See under 0081

JCTVC-U0161 Cross-check of JCTVC-U0111 on Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search [K. Rapaka (Qualcomm)] [late] [miss]

JCTVC-U0083 On indication of the use of current reference picture [C. Liu, S. Liu, X. Xu, S. Lei (MediaTek)]

In working draft of High Efficiency Video Coding (HEVC) extension on Screen Content Coding (SCC), the use of current picture as a reference picture is signalled in sequence parameter set (SPS). This contribution presents methods of signalling the use of current reference picture in picture parameter set (PPS) or slice header.

During the discussion, the following aspects were raised:

- Flag was put into SPS by purpose, allowing the decoder knowing at a very high level that IBC will be used

- No evidence is brought that enabling/disabling IBC at picture level gives benefit. If the characteristics of content radically changes (e.g. change between natural and screen content), an IDR picture will be used anyway, such that sending a new SPS is not an issue.

No action was taken on this. Basically, it is even possible with the current spec to exclude the current picture from the list of active reference pictures at the slice level.

(Note: U0079 is also related.)

JCTVC-U0100 On operation of DPB in Screen Content Coding [C. Liu, S. Liu, X. Xu, S. Lei (MediaTek)]

This contribution presents a few modifications of operation of decoded picture buffer (DPB) on High Efficiency Video Coding (HEVC) Screen Content Coding (SCC): Draft 3 so that the DPB can be operated appropriately for the scenario that the current picture may be used as a reference picture. .

IBC requires one more picture memory storage, because the current picture has to be retained once in a non-deblocked version (for IBC) and once deblocked (for subsequent predictions). The proposal is to put the non-deblocked into the decoded picture buffer, and replace it by the deblocked version (which is stored in an intermediate memory) after the current picture is fully decoded.

It is generally agreed that the approach is pointing into the right direction, in terms of handling the current non-deblocked picture as a reference picture, as well as considering the additional memory necessary to store it as part of the DPB, as well as removing it / replacing it by the deblocked version after the curent picture is fully processed. Basically this means that one less temporal reference is available when IBC is used, but the amount of emory is not increased.

The specification does not seem to be complete, e.g. some changes also appear necessary in section 8.1.3. Clarify offline with K. Rapaka, Y.-K. Wang, revisit.

JCTVC-U0102 On constrained intra prediction for the unification framework of intra block copy [X. Xiu, Y. Ye, Y. He (InterDigital)]

The unification framework of intra block copy (IBC) and inter signalling was adopted. However, it is asserted that, when constrained intra prediction (CIP) is enabled, the specification text in HEVC screen content coding draft 3 is ambiguous, and reference software SCM-4.0 is broken. In this contribution, it is proposed to disallow the samples of inter CUs, either predicted from temporal reference pictures or the current picture itself, to be used as references for intra prediction, when constrained intra prediction is enabled.

The issue was previously presented in JCTVC-O0155 in context of RExt IBC.

Now, the situation is different in the unified framework, where in principle bi-prediction inter/IBC is also possible, and more than two CTBs are used, and interpretation as inter/inter mode is different.

It is agreed that an issue exists, but more consideration is necessary to decide whether any of the suggested solutions is appropriate. Side activity (X. Xiu) to further study the proposal and discuss whether an immediate solution can be found, report back. Revisit.

Eventually, further study may be necessary also in the context of error resilience, because there could be a tradeoff between compression efficiency and error propagation.

JCTVC-U0123 Crosscheck report of JCTVC-U0102 [K. Misra, S.-H. Kim (Sharp)] [late]

JCTVC-U0104 On unification framework of intra block copy [X. Xiu, Y. Ye, Y. He (InterDigital)]

The unification framework of intra block copy (IBC) and inter signalling was adopted. This contribution proposes several design modifications to the current IBC unification framework. Firstly, it is proposed to add the reference picture currPic that contains already coded samples of the current picture prior to in-loop filtering to both initial reference picture lists L0 and L1 when IBC is enabled. Secondly, it is proposed to signal IBC search range in SPS. Thirdly, it is proposed to disable weighted prediction signalling for the currPic.

Aspect1: It is generally agreed that enabling current picture in both L0 and L1 is desirable, it was however diabled because there was a bug in the software implementation by the last meeting. The contribution is bringing a software patch which is claimed to fix that bug (to be confirmed by software coordinator).

Decision: Adopt aspect 1 (current picture in L0/L1). 0079 also proposes this aspect.

Aspect2 (signalling of search range). This does not have an impact on the decoding process and therefore should not be placed in SPS. The benefit is not seen to be obvious by other experts. In VUI, a restriction of inter MV already exists, such that it would somehow contradict the unification idea to express it differently for IBC.

Aspect3 (weighted prediction). The suggested approach is to set the weight parameters to default (offset 0, weight 1) when the current picture is the reference (modification in weight table at slice level). Currently, we don’t have any evidence that weighted prediction gives benefit in case of IBC. This aspect could save some complexity in implementations that use dedicated building blocks for current picture, but it does still allow re-using existing building blocks of MC.

Decision: Adopt aspect 3 (disabing WP with IBC).

It was also mentioned during the discussion that, if evidence about benefit is brought in the future, this decision could be reverted.

JCTVC-U0113 On reference picture list construction for intra block copy [X. Xu, S. Liu, S. Lei (MediaTek)]

In current HEVC SCC draft text, the current picture is put in the last position of reference picture list 0 during the initialization of this list (RefPicListTemp0) and before the possible reference picture list modification (RPLM) operations. As a consequence, when the number of reference pictures of list 0 for the current slice (num_ref_idx_l0_active_minus1 + 1) is smaller than the number of total reference pictures (NumRpsCurrTempList0) in the list, the current picture will not be included in the list (RefPicList0) as an active reference picture. This is different to the intended behaviour that the current picture is always used as a reference picture for intra block copy when the SPS flag “curr_pic_as_ref_enabled_flag” is enabled. In this contribution, it is proposed to slightly modify the reference picture list construction procedure such that the current picture is by default put in the last position of RefPicList0, which is indicated by num_ref_idx_l0_active_minus1. Simulation results show that there is no change in coding performance as compared to SCM-4.0 under CTC condition.

Two methods proposed which both put the current picture in last position, where the first method allows modifying the position via RPLM afterwards, the second method does not allow this.

In method 1, if the number of active pictures is equal to 1 and the picture is not an I slice, the current picture is not included unless using RPLM.

No difference in compression, since RPLM mechanisms are not used in current implementation.

It is pointed out that the software bug that is mentioned in the proposal was fixed in SCM4.0.

With the current spec, cases may happen where it is necessary to use RPLM in order to include the current picture into the list of active reference pictures. The proposal method 1 would revert this situation such that it is always included in the list of active pictures, but RPLM can be used to move the current picture out of the active range.

There is no evidence that the current spec (or the software) has a problem, therefore no urgent need to change it. See further notes under U0118.

JCTVC-U0120 Cross-check of JCTVC-U0113, On reference picture list construction for intra block copy [R. Cohen (MERL)]

JCTVC-U0142 Inter/Intra Block Copy Unification: Comments and Observations [A.M. Tourapis, Y. Su, D. Singer (Apple)] [late]

Not presented yet in track B (no presenter available Sat. morning), should be presented in JCT-VC plenary.

### Other IBC related aspects (5)

(Consideration of this topic was chaired by JRO on Saturday 06-20, 12:00-13:30.)

JCTVC-U0054 Non-CE2: On interpretation of the IBC vectors [C. Gisquet, G. Laroche, P. Onno (Canon)]

At the 20th meeting held in Geneva (CH) in February 2015, the alignment of Intra Block Copy on Inter signaling, as proposed by JCTVC-T0227, was adopted. Various normative changes were thus introduced, however, it was decided that the motion prediction mechanisms of HEVC v1 would better be kept unmodified, in spite of various proposals reporting gains by adapting these mechanisms. It is asserted that the normative changes from JCTVC-T0227 can be tweaked to recoup some of those gains. It is reported that the proposed changes provide a BDR gain of 0.4% to 0.7% over SCM4.0 for Screen Content classes in the lossy AI scenario, and 0.2% to 0.7% for the lossy RA scenario.

The normative changes suggested in the contribution are partially touching the unification of integer MV and BV coding/compensation as per adoption of 0081. Furthermore, some constraints are released on usage of zero merge candidate (but still not allowing references outside of picture boundary). This requires additional clipping at the decoder.

Some of the gain is likely due to further encoder optimization.

One problem with the proposal could be that the encoder may only be able to check the validity of a BV after performing the compensation.

U0118 has related aspect 4.

JCTVC-U0135 Crosscheck report of JCTVC-U0054 [J. Ye, S. Liu (MediaTek)] [late] [miss]

JCTVC-U0079 HLS: On Intra block copy signaling control [K. Rapaka, … (Qualcomm), A. Tourapis, … (Apple)]

During the 20th JCT-VC meeting held in Geneva, the alignment of intra block copy (IBC) with inter signaling, as proposed by JCTVC-T0227, was adopted. In the current working draft (WD), I-slices are disallowed when IBC is enabled (i.e., when curr_pic_as_ref_enabled_flag is equal to 1). On the other hand, support of the IBC mode in B-slices seems to be compromised, and may potentially be problematic, since the current picture is only added as a reference in List0. This contribution proposes to allow I-slices when the IBC mode is enabled, improve support of IBC in B slices, and to provide finer control of IBC at the picture and/or slice level.

Aspect 1: Enable I slices when IBC is enabled at SPS. (This does not mean that IBC is used in I slices.)

Decision: Adopt aspect 1.

Aspect 2: PPS flag to enable/disable IBC

Aspect 3: slice flag to enable/disable IBC

For aspects 2/3, see comments under U0083.

Aspect also recommends enabling IBC in both L0/L1 for B slices (already adopted per U0104, see notes there).

JCTVC-U0118 On intra block copy bitstream constraints [V. Seregin, K. Rapaka, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm)]

At the previous meeting, the intra block copy (IBC) design was changed to be aligned with inter mode, with several bitstream constraints added to control the intended intra block behaviour. It is asserted that, however, some of the constraints are not aligned with inter mode. In this contribution, IBC bitstream constraints are proposed to be converted into signalling and decoder side restrictions for robustness. It is proposed to modify the collocate_ref_idx signalling such that it cannot refer to the current picture, allow IBC motion vector to point outside of the picture as in inter mode, infer current picture as a reference when IBC mode is enabled and the number of active pictures in a slice is equal to 1, and convert a not-valid motion vector into a valid reference.

Aspect 3 (release constraint of BV pointing within picture) was discussed in context of the 4:2:0 chroma interpolation proposals (U0077 etc.). This proposal is going further in also allowing outside-picture vectors for luma.

Generally, the benefit of unification is not seen to be too obvious here, because the memory for the current picture is handled different from the other reference pictures, and the bitstream constraint is not a critical issue. No coding benefits are shown with the contribution, though it is verbally claimed that some benefits could eventually be achieved.

Further study.

Other aspects of this contribution were reviewed Saturday.

Aspect 4 (search range constraints), similar approach in spirit as in 0054. The idea is releasing the bitstream constraint and perform clipping of the vector at the decoder, such that it does not point to an area outside the current slice, follows WPP boundaries etc. If still no valid prediction area is available, a constant value is used for prediction.

It is claimed that this would simplify encoder implementation, since encoders would not need to check bitstream constraints.

This modifies the motion vector derivation process, and therefore deviates from the unification.

No action on aspect 4

Aspect1: Collocated picture constraint (current picture shall not be used as collocated picture)

The proposal is to add an additional check at the decoder (not sending the collocated picture ref idx when it is current picture) instead of formulating this as a bitstream constraint. It is claimed that this simplifies encoder implementation.

This is not a complicated check to be performed at encoder. No action.

Aspect 2: This provides a mechanism that the current picture is in the reference picture list by default (without using RPLM). Related to JCTVC-U0113.

It is agreed that there is a certain benefit to have a default where the current picture is in the reference picture list when IBC is enabled, and no need to use RPLM in certain cases.

Include further consideration in side activity on DPB (Y.-K. Wang et al.) to come up with a proposal for modification. The solution should still allow to remove the current picture from the list of active reference pictures (as per U0113 method 1 and U0118 aspect 2). Revisit.

### SCC tool complexity (AHG9) (0)

### SCC Other (5)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0106 On signaling adaptive color transform at TU level [X. Xiu, Y. He, Y. Ye (InterDigital)]

JCTVC-U0165 Cross-check of on signaling adaptive color transform at TU level (JCTVC-U0106) [B. Li, J. Xu (Microsoft)] [late]

JCTVC-U0119 On slice segment freeze signalling for screen content coding [T. Laude (Leibniz Universität Hannover)]

JCTVC-U0138 Chroma Deblocking for Screen Content Coding [A. Tourapis, Y. Su, D. Singer, W.S Kim, W. Pu, J. Sole, M. Karczewicz] [late]

JCTVC-U0172 Performance gain of IBC FF and loss of IBC FF vs 4CTU [X. Chen, T. Lin (Tongji)] [late]

## HL syntax (0)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

## SEI and VUI (7)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0032 Proposed addition of transfer characteristics in VUI [Y. Nishida, T. Yamashita, A. Ichigaya (NHK), T. Shimizu (ARIB)]

HDR related

JCTVC-U0033 High dynamic range compatibility information SEI message [M. Naccari, A. Cotton, S. Schwarz, M. Pindoria, M. Mrak, T. Borer (BBC)]

HDR related

JCTVC-U0048 Output code map SEI for fractional bit-depth increase [C. Fogg (MovieLabs), B. Mandel (Universal), A. Tourapis, Y. Su, D. Singer (Apple)]

HDR related

JCTVC-U0098 Dynamic Range Adjustment SEI [D. Bugdayci Sansli, A. Ramasubramonian, D. Rusanovskyy, S. Lee, J. Sole, M. Karczewicz (Qualcomm)]

HDR related

JCTVC-U0112 Ambient viewing environment SEI message [G. J. Sullivan (Microsoft)]

(Consideration of this topic was chaired by Y.K. Wang on Saturday 06-20, 16:10-17:00.)

This contribution proposes an SEI message to describe the ambient viewing environment assumed when mastering the associated video content. The contribution asserts that the ambient viewing environment in which video content is experienced by the viewer can have a substantial effect on perceptual quality and that this is widely recognized (e.g., in ITU-R BT.2035 and ITU-R BT.500). For example, viewing an image in the varying environments of a darkened theater, a typical home (at night or during the daytime), or outdoors can have a major effect on the viewing experience. It is reported that adaptive brightness and light sensor technology are becoming common in devices such as mobile phones, tablets, and laptop computers. It is asserted that conveying a representation of the conditions of the nominal viewing environment that was assumed when mastering the video content can enable receiving systems to adapt their local display (e.g., based on a user pre-setting or light sensor input) for optimal display. The proposed message is capable of conveying either of the types of viewing conditions specified in ITU-R BT.2035, and other conditions that can be specified using similar parameters.

It was asked whether VUI or SEI is more appropriate for this. It could be VUI, but it is generally easier to add SEI messages than extending VUI.

It was asked whether the scope of the message should be whole-CVS or more adaptive. In theory it could be more adaptive than a CVS scope.

How would receivers use this information? The nominal ambient viewing environment is the expected environment for viewing the content. For the best user experience, the viewing environment should have been built towards the nominal ambient viewing environment.

If there is a difference between the actual viewing environment and the nominal viewing environment, what to do? It'd be nice if there is some guidance for this. However, that seems not straightforward. If the environment itself is not easy to adjust, at the minimum, the display should be adjusted to compensate. But how?

It should be good to have some clarification on what is "the nominal ambient viewing environment".

It was generally agreed that the information is useful.

Two options to proceed: 1) adopt the proposal and suggest further study on whether some guidelines can be made when the actual viewing environment differs from the nominal viewing environment; 2) further study on whether some guidelines can be made when the actual viewing environment differs from the nominal viewing environment, and make a decision after that.

Revisit when more experts on display and related are available. 16:10-17:00

JCTVC-U0115 Video Bitstream Transition SEI mesage [W. Wan (Broadcom)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 17:50-18:30.)

This contribution proposes that SEI messages be defined to facilitate the signalling of an upcoming transition from a bitstream encoded with one compression standard to another bitstream encoded with a different compression standard. It is suggested that this type of signalling is needed in the video layer to facilitate effective support of on-the-fly codec switching. It is proposed that this functionality be defined to support switching between the ITU-T Rec. H.262 | ISO/IEC 13818-2, ITU-T Rec. H.264 | ISO/IEC 14496-10 and ITU-T Rec. H.265 | ISO/IEC 23008-2 compression standards.

The desirability of having this within the video layer vs. systems layer was discussed.

The desirability of the message would depend on the system environment:

- In some environments, no switching would occur

- In some environments, the service would operate by just switching between fixed sets of bits, and inserting the SEI message may not be so feasible.

Proposed to have the SEI message on the last picture in decoding order before a transition (or two pictures before, or some other amount of decoding order or period of time).

With a pre-encoded bitstream, it might be necessary to remove and/or replace the SEI message when adapting or splicing content.

The scope is somewhat crossing the CVS boundary.

Properties of upcoming bitstream defined as profile/level info.

It was commented that a NAL unit type could be used instead of an SEI message.

The applicability of the syntax to the codec used for the next segment needs to be considered – how to refer to the proprerties of the next bitstream.

Exactly how the data would be used was discussed.

An asserted advantage is that the position of the transition could be more precisely identified.

Broader discussion of the idea seems desirable.

Input to determine whether this would be desired by various orgs (DVB, ATSC, SCTE, etc.) may be desirable to solicit.

Discuss at higher level and further study.

JCTVC-U0128 An HEVC SEI Message for Green Metadata [S. Cheng, J. Wen (??)] [late]

An HEVC SEI message is proposed for the carriage of Green Metadata (ISO/IEC 23001-11 PDAM 1). The metadata enables cross-segment decoding for quality recovery after low-power encoding and extension for other future metadata.

TBP.

## Non-normative: Encoder optimization, decoder speed improvement and cleanup, post filtering, loss concealment, rate control, other information (6)

(Consideration of this topic was chaired by XXX on XXday 06-XX, XX:00-XX:00.)

JCTVC-U0039 Overview of the High Efficiency Image File Format [M. M. Hannuksela, V. K. Malamal Vadakital, J. Lainema (Nokia)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 19:00-19:30.)

This document presents an overview of the High Efficiency Image File Format (HEIF, ISO/IEC 23008-12). HEIF specifies the storage of individual images as well as image sequences into a container file. HEIF includes the storage specification of HEVC intra images and HEVC image sequences in which inter prediction is applied in a constrained manner. HEIF files are compatible with the ISO Base Media File Format (ISO/IEC 14496-12) and can also include other media streams, such as timed text and audio.

The tutorial overview information was appreciated.

JCTVC-U0095 Non-CE: Encoder modification for intra prediction mode using adaptive colour transform [Y.-J. Chang, C.-L. Lin, J.-S. Tu, C.-C. Lin (ITRI)]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 17:00-17:40.)

In SCM4.0, the intra prediction mode can enable the adaptive colour transform (ACT) to perform in-loop colour space conversion in the prediction residual domain. Because the ACT is adaptively enabled/disabled by a CU-level flag, it increases the encoder complexity to evaluate whether to enable the ACT. This contribution proposes to reduce the encoder complexity by disallowing the ACT-based intra prediction mode in 64x64 CU when the video format is YUV colour space. Besides, it further proposes to disallow the TU splitting for the ACT-based intra prediction in large CUs. Compared to the SCM4.0 anchor, it is reported that the proposed modification can save 5~8% encoding time in AI condition with minor gain change.

Two speed-up methods were proposed:

- Skipping evaluation of ACT when video is YUV and CU size is 64x64

- Skipping checking of small TU sizes with ACT enabled when CU size is large

5-8% savings in encding time is reported, with basically no coding efficiency impact.

A prior related contribution was T0080, which proposed to change the CTC to disable ACT for YUV video. However, that proposal sometimes had a significant coding efficiency loss.

It was noted that to run the encoder using the first modification, it would become necessary to know whether the video is YUV or not.

Suggestion: Adopt method 2 only (with macro to control, but using it in the CTC).

It was asked whether, for method 2, it really makes a difference whether ACT is off or on. The contributor said they had only tested it as reported.

Revisit after testing whether it really matters whether ACT is off or on.

JCTVC-U0164 Cross-check of encoder modification for intra prediction mode using adaptive colour transform (JCTVC-U0095) [B. Li, J. Xu (Microsoft)] [late] [miss]

JCTVC-U0132 Target bits saturation to avoid CPB overflow and underflow under the constraint of HRD [Y.-J. Ahn, X. Wu, W. Lim (KWU), D. Sim (Digital Insights)] [late]

(Consideration of this topic was chaired by GJS on Saturday 06-20, 18:30-17:00.)

In this contribution, an implementation to saturate a target bit cost is presented which is asserted to avoid overflow and underflow of the coded picture buffer (CPB) in rate control of HEVC reference model (HM). Rate control based on R-lambda model is employed for HM. Even though a hypothetical reference decoder (HRD) has been defined in HEVC for prevention of CPB overflow and underflow, it was reported that the reference software does not have any considerations fo the HRD constraint of Hs contribution proposes a target-bit saturation method to avoid CPB overflow and underflow under the constraint of HRD.

The focus of the contribution was CBR mode.

It was reported that our current rate control does not consider overflow/underflow.

HRD is a tough subject, and not considered substantially in the HM.

The direction of work is encouraged.

The amount of code was estimated at 100 lines.

It was asked how some of the values of the controlling constants were derived.

Decision (N-N SW): Adopt, with a controlling macro, disabled by default (pending further study to confirm behavior).

JCTVC-U0152 R-lambda model based rate control with pre-encoding process [M. Fang, M. Tang, J. Wen, Z. Wen (??)] [late]

TBP.

JCTVC-U0157 Cross check: R-lambda model based rate control with pre-encoding process (U0152) [W. Pu (Qualcomm)] [late] [miss]

## Withdrawn, unclear (8)

JCTVC-U0067 Withdrawn

JCTVC-U0068 Withdrawn

JCTVC-U0069 Withdrawn

JCTVC-U0071 Withdrawn

JCTVC-U0072 Withdrawn

JCTVC-U0073 Withdrawn

JCTVC-U0074 Withdrawn

JCTVC-U0075 Withdrawn

JCTVC-U0170 Withdrawn

# Plenary Discussions, Joint meetings, BoG Reports, and Summary of Actions Taken

## General

Topics for general discussion:

- Profiles/levels and limits (coding tools, IBC referencing range, palette size <= 64, palette predictor size <= 128, …)

- Palette with pixel copy prediction mode U0116 and U0171.

- SEI messages

  - HDR related

  - Ambient viewing environment SEI message (U0112)

  - Video bitstream transition SEI message (U0115)

## Project development

Joint meetings are discussed in this section of this report.

## BoGs

## Summary of normative decisions

The following is a summary of the normative decisions made at the meeting for the draft screen content coding extensions:

- …

  - …

For further detail, see notes in other sections. The above list is only provided as a summary.

# Project planning

## WD drafting and software

The following agreement was established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without WD text

- HM text strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be the XXXday of the week preceding the meeting (XX June 2015).

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

## General issues for CEs and TEs (to be updated)

Group coordinated experiments were planned. These fell into two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments were as described in the prior output document JCTVC-M1100.

A deadline of four weeks after the meeting was established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Timeline: 3 weeks to basis, 1 more week to description and participation, 2 more weeks to software. [Update other sections if they conflict.]

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document was reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE is described in an associated output document JCTVC-S11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans are recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities were established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

## Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

## Common Conditions for HEVC Coding Experiments (to be updated)

No particular changes were noted w.r.t. prior CTC. [update]

## Software development

The software coordinator had already started integrating changes on top of the prior HM software, and proponents of adopted proposals are required to integrate their changes into the latest version, in coordination with the software coordinator, and test in this environment. All tools were planned to again be thoroughly tested after integration.

Any adopted proposals where software is not delivered by the scheduled date will be rejected.

The planned timeline for software releases was established as follows:

- HM 16.4 should be available within 2 weeks after the meeting, 8 weeks to 16.x T1011 (DAM) delivery.

- SCM 4.0 (based on HM 16.3 or 16.4) should be available within 3 weeks after the meeting.

- SHM 9.x T1013 (PDAM, based on HM 15.0 or HM 16.x) should be available within 6 weeks after the meeting.

At the previous (Sapporo) meeting, it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and strongly suggested that this should be done. This remains desirable.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups will be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

[Ed. Add/fix coordination aspects as relevant.]

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| HEVC test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-T1002 HEVC Test Model 16 (HM 16) Update 2 of Encoder Description - Collect reports of errata for HEVC - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and HM software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), M. Naccari, J.-R. Ohm, K. Sharman, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| HEVC HM software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM software and its distribution. - Produce documentation of software usage for distribution with the software. - Prepare and deliver HM 16.x software versions and the reference configuration encodings according to JCTVC-L1100 and JCTVC-P1006 common conditions. - Suggest configuration files for additional testing of tools. - Coordinate with AHG2 on HEVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), D. Flynn, K. Sharman (vice-chairs) | N |
| HEVC conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Prepare and deliver the JCTVC-T1008 SHVC conformance draft 2, and JCTVC-T1012 Version 1 and RExt conformance draft 4 specifications. - Discuss work plans and testing methodology to develop and improve HEVC v.1, RExt and SHVC conformance testing. - Establish and coordinate bitstream exchange activities for HEVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain bitstream exchange database and draft HEVC conformance bitstream test set. | T. Suzuki (chair), J. Boyce, K. Kazui, A. K. Ramasubramonian, W. Wan, Y. Ye (vice-chairs) | N |
| HEVC and range extensions verification test preparation (AHG5) (jct-vc@lists.rwth-aachen.de) - Make preparations for verification testing of HEVC for interlaced video content. - Make preparations for verification testing of HEVC format range extensions. | V. Baroncini (chair), M. Karczewicz, M. Naccari, N. Ramzan, C. Rosewarne, T. K. Tan, J.-M. Thiesse, W. Wan (vice-chairs) | N |
| SCC coding performance analysis (AHG6) (jct-vc@lists.rwth-aachen.de) - Study test conditions and coding performance analysis methods for SCC coding performance. - Analyze coding performance of draft and proposed SCC coding features | H. Yu (chair), R. Cohen, A. Duenas, P. Lai, K. Rapaka, J. Xu (vice-chairs) | N |
| SCC extensions text editing (AHG7) (jct-vc@lists.rwth-aachen.de) - Produce and finalize HEVC screen content coding extensions draft 3 and test model 4 text. - Gather and address comments for refinement of the test model. - Coordinate with AHG8 to address issues relating to mismatches between software and text. | J. Xu, R. Joshi (co-chairs), R. Cohen, S. Liu, Z. Ma, G. Sullivan, Y. Ye (vice-chairs) | N |
| SCC extensions software development (AHG8) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the SCM software and its distribution. - Prepare and deliver HM 16.x-SCM-4.0 software version and the reference configuration encodings according to JCTVC-T1015. - Prepare and deliver additional "dot" version software releases and software branches as appropriate. - Perform analysis and reconfirmation checks of the behaviour of the draft design, and report the results of such analysis. - Suggest configuration files for additional testing of tools. - Coordinate with AHG7 to address any identified issues regarding text and software relationship. | B. Li, K. Rapaka (chairs), R. Cohen, P. Chuang, X. Xiu, M. Xu (vice-chairs) | N |
| Complexity of SCC extensions (AHG9) (jct-vc@lists.rwth-aachen.de) - Analyze complexity characteristics of current and proposed SCC coding methods with regards to throughput, amount of memory, memory bandwidth, parsing dependencies, parallelism, pixel processing, chroma position interpolation, and other aspects of complexity as appropriate. - Quantify and compare the average and worse case throughput (context-coded as well as bypass bins) for SCC coding methods. - Study latency and parallelism implications of SCC coding techniques, considering multicore and single-core architectures. - Identify criteria to determine the hardware implementability of the key hardware modules. - Identify bottlenecks in the current design with regard to implementation complexity. | A. Duenas (chair), M. Budagavi, R. Joshi, S.-H. Kim, P. Lai, W. Wang, X. Xiu (vice-chairs) | N |
| Test sequence material (AHG10) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG5 regarding interlaced video and range extensions development, and AHG6 regarding screen content coding. | T. Suzuki, V. Baroncini, R. Cohen (co-chairs), T. K. Tan, S. Wenger, H. Yu (vice-chairs) | N |
| SHVC test model editing (AHG11) (jct-vc@lists.rwth-aachen.de) - Produce and finalize JCTVC-T1007 SHVC Test Model 9 (SHM 9) text. - Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text. | J. Chen (chair), J. Boyce, M. M. Hannuksela, G. J. Sullivan, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| SHVC software development (AHG12) (jct-vc@lists.rwth-aachen.de) - Prepare SHM 9.0 software (based on HM 16.x) for experimentation. - Generate anchors and templates based on common test conditions. - Discuss and identify additional issues related to SHVC software. | V. Seregin, Y. He, (co-chairs) | N |
| SHVC verification testing (AHG13) (jct-vc@lists.rwth-aachen.de) - Identify test sequences and test conditions for SHVC verification testing. - Prepare and propose a draft SHVC verification test plan. | V. Baroncini, Y.-K. Wang, Y. Ye (co-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

JCTVC-T1000 Meeting Report of the 20th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2015-05-29] (near next meeting)

Remains valid – not re-issued: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

(Remains valid, although from a prior meeting.)

JCTVC-T1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Update 2 of Encoder Description [C. Rosewarne (primary editor), B. Bross, M. Naccari, K. Sharman, G. J. Sullivan (co-editors)] (WG 11 N 15139) [2015-05-29] (near next meeting)

JCTVC-T1003 Verification test plan for format range extensions and interlaced video [C. Rosewarne, A. Tourapis, G. Barroux, M. Naccari (editors)] (WG 11 N 15140) [2015-03-20] (4 weeks)

Relative to S1003:

The ability to conduct the test as planned will depend on the availability of volunteered sponsorship to cover testing costs, toward which interested parties are requested to contribute. A lack of such sponsorship may delay the testing. Care is to be taken to ensure that the costs are not excessive, in consultation with the management of the JCT-VC and its parent bodies.

The final selection of the subset of the test sequences and bit rates to be tested remain to be selected by the test coordinator.

Remains valid – not reissued: JCTVC-S1004 HEVC Version 1 Conformance Testing Defect Report [T. Suzuki, W. Wan, G. J. Sullivan (editors)] (WG 11 N 15016) [2014-11-14] (3 weeks)

JCTVC-T1005 HEVC Screen Content Coding Draft Text 3 [R. Joshi, S. Liu, J. Xu, Y. Ye (editors)] (WG 11 N 15138 ISO/IEC PDAM) [2015-03-27] (5 weeks)

PDAM

Basic elements:

- IBC

- Adaptive colour transform

- Palette mode

- Adaptive MV resolution

Remains valid – not reissued: JCTVC-P1006 Common test conditions and software reference configurations for HEVC range extensions [D. Flynn, C. Rosewarne, K. Sharman (editors)]

JCTVC-T1007 SHVC Test Model 9 (SHM 9) Introduction and Encoder Description [J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela (editors)] (WG 11 N 15143) [2015-05-29] (near next meeting)

JCTVC-T1008 SHVC Conformance Testing Draft 2 [J. Boyce, A. K. Ramasubramonian] (WG 11 N 15158 ISO/IEC PDAM) [2015-03-27] (5 weeks)

PDAM

Remains valid – not updated JCTVC-Q1009 Common SHM Test Conditions and Software Reference Configurations [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

JCTVC-T1011 Reference Software for HEVC Format Range Extensions Draft 2 [F. Bossen, D. Flynn, K. Sühring, T. Suzuki (editors)] (WG 11 N 15146 ISO/IEC DAM) [2015-04-17] (8 weeks)

DAM

JCTVC-T1012 Conformance Testing for Improved HEVC Version 1 Testing and Format Range Extensions Draft 4 (WG 11 N 15156) [T. Suzuki, K. Kazui (editors)] [2015-03-27] (5 weeks)

PDAM (include enhancement & corrections of version 1 conformance testing).

JCTVC-T1013 Reference software for Scalable HEVC (SHVC) Extensions Draft 1 (WG 11 N 15150 ISO/IEC PDAM) [Y. He, V. Seregin (editors)] [2015-04-03] (6 weeks)

PDAM

JCTVC-T1014 Screen Content Coding Test Model 4 Encoder Description (SCM 4) [R. Joshi, J. Xu, R. Cohen, S. Liu, Z. Ma, Y. Ye (editors)] (WG 11 N 15144) [2015-05-29] (near next meeting)

JCTVC-T1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2015-03-13] (3 weeks)

(Using the modified GOP structure with temporal nesting for RA if the corresponding HM basis uses that.)

Not using global palette initialization.

Notes elsewhere on other aspects.

Remains valid – not re-issued: JCTVC-L1100 Common Test Conditions and Software Reference Configurations for HM [F. Bossen (editor)]

(Remains valid, although from a prior meeting.)

Note that regardless of preliminary CE plans established earlier in the meeting, such plans were not considered binding on final CE plans as reviewed in the closing plenary.

JCTVC-T1101 Description of Core Experiment 1 (CE1): Palette Mode Improvement [C. Gisquet, P. Lai, V. Seregin, X. Xiu (CE coordinators)] [2015-03-20] (4 weeks)

Category A: Tests from T0231 and T0233 (harmonization of copy-above and bypass grouping)

Category B: T0078 part II (removing dependency in index adjustment)

Category C: T0119 (reverse scan order) and T0174 (rotated scan)

Category D: T0133 (handling run lengths in relation to CU width)

Timeline: See section 7.3.

Test conditions CTC per T1015.

JCTVC-T1102 Description of Core Experiment 2 (CE2): Intra block copy memory access [K. Rapaka, C. Gisquet, J. Lainema, X. Xu (CE coordinators)] [2015-03-20] (4 weeks)

Category A: T0045/T0051/S0145 Signalling of IBC source blocks (without modifying constraints or decoder operation).

Category B: Constraints to reduce memory bandwidth

- B.1 Two-pass encoding decisions (T0051)

- B.2 Single-pass encoding decisions (T0045)

Category C: Avoiding references to bi-predicted regions (without signalling, R0188)

Timeline: Modified for additional milestone of bandwidth analysis module, CE software after 5 weeks.

Test conditions CTC per T1015.

Additional comparison point: Disabling deblocking adaptively at sequence or picture level

Additional test conditions: LD-P for subjective analysis.

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (starting meetings on the Tuesday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 8–8.5 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Friday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 7.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Wed. 13 – Wed. 21 Oct. 2015 22nd meeting under ITU-T auspices in Lucca, IT.

- Fri. 19 – Fri. 26 Feb. 2016 23rd meeting under WG 11 auspices in San Diego, US.

- Wed. 25 May – Wed. 1 June 2016 24th meeting under ITU-T auspices in Geneva, CH.

- …

The agreed document deadline for the 21st JCT-VC meeting is 9 June 2015. Plans for scheduling of agenda items within that meeting remain TBA.

XXX was thanked for the excellent hosting of the 21st meeting of the JCT-VC.

The JCT-VC meeting was closed at approximately XX00 hours on Fri. 26 June 2015.

# Annex A to JCT-VC report:
List of documents

# Annex B to JCT-VC report:
List of meeting participants

The participants of the nineteenth meeting of the JCT-VC, according to a sign-in sheet circulated during sessions (approximately XXX people in total), were as follows: