| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 36th Meeting: Gothenburg, SE, 6–12 July 2019 | Document: JCTVC-AJ_Notes_d2 |
| --- | --- |

| Title: | Meeting report of the 36th meeting of the Joint Collaborative Team on Video Coding (JCT-VC), Gothenburg, SE, 6–12 July 2019 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from chairs of JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen University Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs |  |  |

_____________________________

- Summary

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11 held its thirty-sixth meeting during 6–12 July 2019 at the Clarion Post Hotel in Gothenburg, SE. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 1.14 of this document.

The JCT-VC meeting began at approximately 0900 hours on Saturday 6 July 2019 with a half-day of meeting sessions. One additional meeting session was held at XXXX on XXday X July 2019, and …. The meeting was closed at approximately XXXX hours on XXday X July 2019. Approximately XX people attended the JCT-VC meeting, and X input documents and 7 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions, and the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information. Further work was performed on the specification of coding-independent code points related to video data. Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard was also conducted.

One primary goal of the meeting was to review the work that was performed in the interim period since the 35th JCT-VC meeting in producing:

- For HEVC conformance testing, Draft 11 of HEVC conformance for screen content coding (SCC) and non-intra high-throughput profiles

- For non-normative guidance on HEVC encoding practices, Update 10 of the HEVC Model (HM) 16 encoding algorithm description

- For video code points coordination, Draft 7 of usage of video signal type code points, and Draft 4 toward version 2 of technical report on usage of video signal type code points.

The other most important goals were to review the work on new SEI messages, encoder optimization work, and to review other technical input documents. Possible needs for corrections to the prior HEVC specification text were also considered.

The JCT-VC produced X output documents from the meeting (update):

- For HEVC conformance testing, Draft 11 of HEVC conformance for screen content coding (SCC) and non-intra high-throughput profiles

- For non-normative guidance on HEVC encoding practices, Update 10 of the HEVC Model (HM) 16 encoding algorithm description

- For video code points coordination, Draft 7 of usage of video signal type code points, and Draft 4 toward version 2 of technical report on usage of video signal type code points.

For the organization and planning of its future work, the JCT-VC established X "ad hoc groups" (AHGs) to progress the work on particular subject areas. The next four JCT-VC meetings were planned for Fri. 4 – Wed. 9 October 2019 under ITU-T SG 16 auspices in Geneva, CH, during Sat. 11 – Fri. 17 January 2020 under WG11 auspices in Brussels, BE, during Sat. 18 – Fri. 24 April 2020 under WG 11 auspices in Alpbach, AT, and during Thu 25 June – Wed 01 July 2020 under ITU-T SG16 auspices in Geneva, CH.

The document distribution site http://phenix.it-sudparis.eu/jct/ was used for distribution of all documents.

The reflector to be used for discussions by the JCT-VC and all of its AHGs is the JCT-VC reflector:
jct-vc@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see
https://lists.rwth-aachen.de/postorius/lists/jct-vc.lists.rwth-aachen.de/.

Administrative topics

Organization

The ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/ SC 29/ WG 11 Moving Picture Experts Group (MPEG). The parent bodies of the JCT-VC are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29/WG 11.

The Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T WP3/16 and ISO/IEC JTC 1/‌SC 29/‌WG 11 held its thirty-sixth meeting during 6–12 July 2019 at the Clarion Post Hotel in Gothenburg, SE. The JCT-VC meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

Meeting logistics

The JCT-VC meeting began at approximately 0900 hours on Saturday 6 July 2019 with a half-day of meeting sessions. One additional meeting session was held at XXXX on XXday X July 2019, and …. The meeting was closed at approximately XXXX hours on XXday X July 2019. Approximately XX people attended the JCT-VC meeting, and X input documents and 7 AHG reports were discussed. The meeting took place in a collocated fashion with a meeting of WG11 – one of the two parent bodies of the JCT-VC. The subject matter of the JCT-VC meeting activities consisted of work on the video coding standardization project known as High Efficiency Video Coding (HEVC) and its extensions, and the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information. Further work was performed on the specification of coding-independent code points related to video data. Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard was also conducted.

Some statistics are provided below for historical reference purposes:

- 1st "A" meeting (Dresden, 2010-04):		188 people, 40 input documents

- 2nd "B" meeting (Geneva, 2010-07):		221 people, 120 input documents

- 3rd "C" meeting (Guangzhou, 2010-10):		244 people, 300 input documents

- 4th "D" meeting (Daegu, 2011-01):		248 people, 400 input documents

- 5th "E" meeting (Geneva, 2011-03):		226 people, 500 input documents

- 6th "F" meeting (Turin, 2011-07):		254 people, 700 input documents

- 7th "G" meeting (Geneva, 2011-11)		284 people, 1000 input documents

- 8th "H" meeting (San Jose, 2012-02)		255 people, 700 input documents

- 9th "I" meeting (Geneva, 2012-04/05)		241 people, 550 input documents

- 10th "J" meeting (Stockholm, 2012-07)		214 people, 550 input documents

- 11th "K" meeting (Shanghai, 2012-10)		235 people, 350 input documents

- 12th "L" meeting (Geneva, 2013-01)		262 people, 450 input documents

- 13th "M" meeting (Incheon, 2013-04)		183 people, 450 input documents

- 14th "N" meeting (Vienna, 2013-07/08)		162 people, 350 input documents

- 15th "O" meeting (Geneva, 2013-10/11)		195 people, 350 input documents

- 16th "P" meeting (San José, 2014-01)		152 people, 300 input documents

- 17th "Q" meeting (Valencia, 2014-03/04)	126 people, 250 input documents

- 18th "R" meeting (Sapporo, 2014-06/07)		150 people, 350 input documents

- 19th "S" meeting (Strasbourg, 2014-10)		125 people, 300 input documents

- 20th "T" meeting (Geneva, 2015-02)		120 people, 200 input documents

- 21st "U" meeting (Warsaw, 2015-06)		91 people, 150 input documents

- 22nd "V" meeting (Geneva, 2015-10)		155 people, 75 input documents

- 23rd "W" meeting (San Diego, 2016-02)		159 people, 125 input documents

- 24th "X" meeting (Geneva, 2016-05/06)		162 people, 60 input documents

- 25th "Y" meeting (Chengdu, 2016-10)		93 people, 40 input documents

- 26th "Z" meeting (Geneva, 2017-01)		95 people, 30 input documents

- 27th "AA" meeting (Hobart, 2017-03/04)	76 people, 25 input documents

- 28th "AB" meeting (Turin, 2017-07)		71 people, 25 input documents

- 29th "AC" meeting (Macao, 2017-10)		107 people, 21 input documents

- 30th "AD" meeting (Gwangju, 2018-01)		85 people, 4 input documents

- 31st "AE" meeting (San Diego, 2018-04)	37 people, 11 input documents

- 32nd "AF" meeting (Ljubljana, 2018-07)	38 people, 8 input documents

- 33rd "AG" meeting (Macao, 2018-10)		32 people, 9 input documents

- 34th "AH" meeting (Marrakech, 2019-01)	34 people, 7 input documents

- 35th "AI" meeting (Geneva, 2019-03)		29 people, 4 input documents

- 36th "AJ" meeting (Gothenburg, 2019-07)	X people, X input documents

Information regarding logistics arrangements for the meeting had been provided via the email reflector jct-vc@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jctvc-site/2019_03_AI_Geneva/

Primary goals

One primary goal of the meeting was to review the work that was performed in the interim period since the thirty-third JCT-VC meeting in producing:

- For HEVC conformance testing, Draft 11 of HEVC conformance for screen content coding (SCC) and non-intra high-throughput profiles

- For non-normative guidance on HEVC encoding practices, Update 10 of the HEVC Model (HM) 16 encoding algorithm description

- For video code points coordination, Draft 7 of usage of video signal type code points, and Draft 4 toward version 2 of technical report on usage of video signal type code points.

The other most important goals were to review the work on new SEI messages, encoder optimization work, and to review other technical input documents. Possible needs for corrections to the prior HEVC specification text were also considered. A particular item is the need for correcting signalling of high throughput profiles.

Documents and document handling considerations

General

The documents of the JCT-VC meeting are listed in Annex A of this report. The documents can be found at http://phenix.it-sudparis.eu/jct/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is done using the keyword “Decision”, e.g., as follows:

- Decisions made by the group that affect the normative content of the draft standard are identified by prefixing the description of the decision with the string "Decision:".

- Decisions that affect the reference software but have no normative effect on the text are marked by the string "Decision (SW):".

- Decisions that fix a "bug" in the specification (an error, oversight, or messiness) are marked by the string "Decision (BF):".

- Decisions regarding things that correct the text to properly reflect the design intent, add supplemental remarks to the text, or clarify the text are marked by the string "Decision (Ed.):".

- Decisions regarding simplification or improvement of design consistency are marked by the string "Decision (Simp.):".

- Decisions regarding complexity reduction (in terms of processing cycles, memory capacity, memory bandwidth, line buffers, number of entropy-coding contexts, number of context-coded bins, etc.) … "Decision (Compl.):".

This meeting report is based primarily on notes taken by the chairs and projected for real-time review by the participants during the meeting discussions. The preliminary notes were also periodically circulated publicly by ftp and http during the meeting for information and coordination purposes. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report may not be uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Tuesday, 25 June 2019.

Non-administrative documents uploaded after 2359 hours in Paris/Geneva time Wednesday 26 June 2019 were to be considered "officially late". One contribution to this meeting, JCTVC-AJ0028, was registered and submitted late. It was an encoder-only proposal related to point cloud coding.

In some cases, contributions were revised after the initial version was uploaded. The contribution document archive website retains publicly-accessible prior versions in such cases. The timing of late document submissions for contributions is recorded in the list of documents in Annex B of this report and is also generally noted in the section discussing each contribution in this report.

Ad hoc group interim activity reports, CE summary results reports, break-out activity reports, and information documents containing the results of experiments requested during the meeting were not subject to the above-described deadline, as these are considered administrative report documents and they may not be possible to produce until after the availability of other input documents.

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when sufficient time for studying was given after the upload. Again, an exception is applied for AHG reports, CE summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions for this meeting.

It is noted that documents that are substantially revised after the initial upload are also a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This is especially a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the "comments" field on the web site can be used to indicate what is different in a revision (although this field has seldom been used and is often not checked by our participants).

"Placeholder" contribution documents that are basically empty of content, with perhaps only a brief abstract and some expression of an intent to provide a more complete submission as a revision, are considered unacceptable and were to be rejected in the document management system, as has been agreed since the third meeting. The initial uploads of such contribution documents are rejected as "placeholders" if they are uploaded without any significant content and are not corrected until after the upload deadline. Such “placeholder” cases did not occur at this meeting.

In some cases in recent history, a few contributions have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). Any such issues have been corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the chairs).

Some other errors may also have been noticed in other initial document uploads (wrong document numbers in headers, uploading of corrupted unreadable files, etc.) which have generally been sorted out in a reasonably timely fashion. The document web site contains an archive of each upload, along with a record of uploading times.

Outputs of the preceding meeting

The output documents of the previous meeting, particularly including the meeting report JCTVC-AI1000, the Draft 11 of HEVC conformance for screen content coding (SCC) and non-intra high-throughput profiles JCTVC-AI1004, the Update 10 of the HEVC Model (HM) 16 encoding algorithm description JCTVC-AI1002, the Usage of video signal type code points (Draft 7) JCTVC-AI1003, and the Draft 4 toward version 2 of technical report on usage of video signal type code points in JCTVC-AI1011 were approved.

The group was initially asked to review the prior meeting report for finalization. The meeting report was later approved without modification.

All output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

The chairs asked if there were any issues regarding potential mismatches between perceived technical content prior to adoption and later integration efforts. It was also asked whether there was adequate clarity of precise description of the technology in the associated proposal contributions.

It was remarked that, regarding software development efforts – for cases where "code cleanup" is a goal as well as integration of some intentional functional modification, it was emphasized that these two efforts should be conducted in separate integrations, so that it is possible to understand what is happening and to inspect the intentional functional modifications.

The need for establishing good communication with the software coordinators was also emphasized.

At some previous meetings, it had been remarked that in some cases the software implementation of adopted proposals revealed that the description that had been the basis of the adoption apparently was not precise enough, so that the software unveiled details that were not known before (except possibly for CE participants who had studied the software). Issues of combinations between different features (e.g., different adopted features) also tend to sometimes arise in the work. There should be time to study combinations of different adopted tools with more detail prior to adoption.

Attendance

The list of participants in the JCT-VC meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited by the Chairs as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the Chairs.

Agenda

The agenda for the JCT-VC meeting the meeting, for development of the High Efficiency Video Coding (HEVC) standard and its format range (RExt), scalability (SHVC), multi-view (MV-HEVC), 3D (3D-HEVC), screen content coding (SCC), and high-dynamic-range (HDR) extensions, and associated conformance test sets, reference software, verification testing, non-normative guidance information, and coding-independent code point specifications was as follows:

- Opening remarks and review of meeting logistics and communication practices

- IPR policy reminder and declarations

- Contribution document allocation

- Reports of ad hoc group activities

- Review of results of previous meeting

- Consideration of contributions and communications on project guidance

- Consideration of errata reports and needs for maintenance and enhancements of the HEVC standard and its associated conformance test specification and reference software

- Consideration of errata reports and needs for maintenance and enhancements of the specification of coding-independent code points for video signal type identification

- Consideration of proposals and preparations toward finalization of in-progress draft specifications of additional supplemental enhancement information metadata for the HEVC standard

- Consideration of errata reports and needs for maintenance and enhancements of supplemental enhancement information and video usability information metadata for the HEVC standard

- Consideration of proposed content and preparations toward finalization of in-progress draft content for a technical report on common combinations of video signal type code point identifiers

- Consideration of information contributions and non-normative guidance relevant to the HEVC standard

- Consideration of agreed related aspects of the AVC standard (esp. regarding supplemental enhancement information)

- Coordination activities relating to the work of the JCT-VC

- Approval of output documents and associated editing periods

- Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, meeting planning, refinement of expected standardization timelines, other planning issues

- Other business as appropriate for consideration

IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JCT-VC and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JCT-VC as necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jctvc-site (JCT-VC contribution templates)

- http://www.itu.int/ITU-T/studygroups/com16/jct-vc/index.html (JCT-VC general information and founding charter)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director's AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

"TSB has reported to the TSB Director's IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur's group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation."

The chairs invited participants to make any necessary verbal reports of previously-unreported IPR in draft standards under preparation, and opened the floor for such reports: No such verbal reports were made.

Software copyright disclaimer header reminder

It was noted that, as had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time, the HEVC reference software copyright license header language is the BSD license with a preceding sentence declaring that other contributor or third party rights, such as patent rights, may exist that are not granted by the license, as recorded in N10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the HEVC standard and its extensions, and for evaluating proposals for technology to be included in the design. After finalization of the draft, the software will be published by ITU-T and ISO/IEC as an example implementation of the HEVC standard and for use as the basis of products to promote adoption of the technology. [Also HDRTools and 360Lib and JM and other AVC codebases]

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

Communication practices

The documents for the meeting can be found at http://phenix.it-sudparis.eu/jct/. For the first two JCT-VC meetings, the JCT-VC documents had been made available at http://ftp3.itu.int/av-arch/jctvc-site, and documents for the first two JCT-VC meetings remain archived there as well. That site was also used for distribution of the contribution document template and circulation of drafts of this meeting report.

The JCT-VC email list is managed through the site https://lists.rwth-aachen.de/postorius/lists/jct-vc.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jct-vc@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JCT-VC participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages, and subscribers must respond adequately to basic inquiries regarding the nature of their interest in the work.

It was emphasized that usually discussions concerning CEs and AHGs should be performed using the JCT-VC email reflector.

Currently, JCT-VC is not running any CEs. When such CEs are conducted, CE internal discussions should primarily be concerned with organizational issues. Substantial technical issues that are not reflected by an original CE plan should be openly discussed on the reflector. Any new developments that are result of private communication cannot be considered to be the result of the CE.

For the headers and registrations of CE documents and AHG reports, email addresses of participants and contributors may be obscured or absent (and will be on request), although these will be available (in human readable format – possibly with some "obscurification") for primary CE coordinators and AHG chairs.

Terminology

Some terminology used in this report is explained below:

- 3D-HEVC: A set of extensions of HEVC that includes the combined coding of depth and texture information for 3D video coding.

- ACT: Adaptive colour transform.

- Additional Review: The stage of the ITU-T "alternative approval process" that follows a Last Call if substantial comments are received in the Last Call, during which a proposed revised text is available on the ITU web site for consideration as a candidate for final approval.

- AHG: Ad hoc group.

- AI: All-intra.

- AIF: Adaptive interpolation filtering.

- ALF: Adaptive loop filter.

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component).

- AMVP: Adaptive motion vector prediction.

- APS: Active parameter sets.

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR).

- AU: Access unit.

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: May refer either to block copy (see CPR or IBC) or backward compatibility. In the case of backward compatibility, this often refers to what is more formally called forward compatibility.

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BL: Base layer.

- BoG: Break-out group.

- BR: Bit rate.

- BV: Block vector (MV used for intra BC prediction, not a term used in the standard).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCP: Cross-component prediction.

- CD: Committee draft – a draft text of an international standard for the first formal ballot stage of the approval process in ISO/IEC – corresponding to a PDAM for amendment texts.

- CE: Core experiment – a coordinated experiment for which there is a draft design and associated test model software that have been established, e.g., as in experiments conducted after the 3rd or subsequent JCT-VC meeting and approved to be considered a CE by the group (see also SCE and SCCE, and TE).

- CGS: Colour gamut scalability (historically, also coarse-grained scalability).

- CL-RAS: Cross-layer random-access skip.

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector sometimes called a block vector, in a manner basically the same as motion-compensated prediction.

- Consent: A step taken in the ITU-T to formally move forward a text as a candidate for final approval (the primary stage of the ITU-T "alternative approval process").

- CTC: Common test conditions – a set of agreed conditions for coding experiments.

- CVS: Coded video sequence.

- DAM: Draft amendment – a draft text of an amendment to an international standard for the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DIS for complete texts.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DIS: Draft international standard – the second formal ballot stage of the approval process in ISO/IEC – corresponding to a DAM for amendment texts.

- DF: Deblocking filter.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element of AVC or HEVC).

- EL: Enhancement layer.

- ET: Encoding time.

- ETM: Experimental test model (design and software used for prior HDR/WCG coding experiments in MPEG).

- FDAM: Final draft amendment – a draft text of an amendment to an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDIS for complete texts.

- FDIS: Final draft international standard – a draft text of an international standard for the third formal ballot stage of the approval process in ISO/IEC – corresponding to an FDAM for amendment texts.

- HDR: High dynamic range – referring to video content having a brightness range that includes values greater than approximately 100 nits (often implicitly including WCG as well, since HDR video is typically also WCG video).

- HDR10: A term that refers to the single-layer coding of HDR/WCG video content using the HEVC Main 10 profile with a Y′CbCr 4:2:0 10 bit per sample colour representation with ITU-R BT.2020 colour primaries and the PQ transfer characteristics EOTF.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized in ITU-T as Rec. ITU-T H.265 and in ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – the draft reference software and its (non-normative) encoder algorithms used for HEVC experiments.

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (esp. 8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (esp. 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- IPCM: Intra pulse-code modulation (as in AVC and HEVC).

- JM: Joint model – the primary software codebase and associated (non-normative) encoding algorithms that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- Last Call: The stage of the ITU-T "alternative approval process" that follows Consent, during which a proposed text is available on the ITU web site for consideration as a candidate for final approval.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LM: Linear model.

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference pictures.

- MANE: Media-aware network element.

- MC: Motion compensation.

- MCTS: Motion-constrained tile set.

- MOS: Mean opinion score – a measurement of subjective video quality as reported by human test subjects.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JCT-VC).

- MV: Motion vector; alternatively, multiview.

- MV-HEVC: A set of extensions of HEVC using layered coding to enable the coding of video with multiple views or depth maps.

- NAL: Network abstraction layer (as in AVC and HEVC, contrast with VCL).

- NCL: Non-constant luminance, a type of colour difference representation.

- Nits: Candelas per square metre (cd/m2).

- NB: National body (usually used in reference to NBs of the WG 11 parent body).

- NSQT: Non-square quadtree.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g., light input to a camera) to output light (e.g., light emitted by a display).

- PCP: Parallelization of context processing.

- PDAM: Proposed draft amendment – a draft text of an amendment to an international standard for the first formal ballot stage of the ISO/IEC approval process – corresponding to a CD for complete texts.

- PDTR: Proposed draft technical report – the draft of a TR that is sent for a ballot in the ISO/IEC approval process.

- POC: Picture order count.

- PoR: Plan of record.

- PPS: Picture parameter set (as in AVC and HEVC).

- PQ: Perceptual quantization – the name given to an HDR EOTF curve specified in SMPTE ST 2084 and Rec. ITU-R BT.2100.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading.

- RASL: Random-access skipped leading.

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RExt: Format range extensions – a set of extensions of HEVC addressing high bit rate operation, high bit depths, and alternative chroma formats such as monochrome, 4:2:2, 4:4:4, high bit depths, and high throughput.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SCC: Screen content coding.

- SCE: Scalability core experiment (for SHVC).

- SCCE: Screen content core experiment (for SCC).

- SCM: Screen coding model (for SCC).

- SD: Slice data; alternatively, standard-definition.

- SDR: Standard dynamic range – referring to video content having a brightness range that would produce a maximum brightness of approximately 100 nits on a reference display under reference viewing conditions.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM (for SHVC).

- SHVC: Scalable high efficiency video coding – a set of extensions of HEVC that uses layered coding to enable the coding of supplemental pictures, quality enhancement layers, spatial resolution enhancement layers, and colour gamut enhancement layers.

- SIMD: Single instruction, multiple data.

- SPS: Sequence parameter set (as in AVC and HEVC).

- Supplement: In ITU-T terminology, a document that assists its readers by providing non-normative information and suggestions (sometimes considered a TR in ISO/IEC terminology).

- SVC: Scalable video coding, especially when referring to the associated extensions of AVC.

- TBA/TBD/TBP: To be announced/determined/presented.

- TE: Tool Experiment – a coordinated experiment conducted toward HEVC design at a more preliminary stage of work than those of CEs, e.g., as between the 1st and 2nd or 2nd and 3rd JCT-VC meetings, or a coordinated experiment conducted toward SHVC design between the 11th and 12th JCT-VC meetings.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TR: Technical report – e.g., a collection of non-normative suggestion guidance on appropriate technical practices (sometimes considered a “supplement” in ITU-T terminology).

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JCT-VC).

- VCL: Video coding layer (as in AVC and HEVC, contrast with NAL).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- WCG: Wide colour gamut – referring to video content having a colour gamut that includes colours substantially outside of the range of values that is representable using Rec. ITU-R BT.709.

- WD: Working draft – a term for a draft standard, especially one prior to its first ballot in the ISO/IEC approval process, although the term is sometimes used loosely to refer to a draft standard at any actual stage of parent-level approval processes.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - LCU: (formerly LCTU) largest coding unit (name formerly used for CTU before finalization of HEVC version 1).

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntaxError: Reference source not found within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

Liaison activity

The JCT-VC did not directly send or receive formal liaison communications at this meeting. However, there was an exchange of status and project information between the parent bodies.

Opening remarks and status of work items (not updated yet)

Opening remarks included:

- Meeting logistics, review of communication practices, attendance recording, and registration and badge pick-up reminder

- It was noted that number of contributions to this meeting has continued to be low relative to a few years ago.

Primary topic areas were noted as follows:

- HEVC text status [new edition recently done]: the 5th ed. for ITU had been published on 2018-05-11, the 3rd ed. for ISO/IEC had been published in 2017-10, and the 4th edition for ISO/IEC had an FDIS and a DoC issued at the last meeting.

  - The SEI manifest and prefix SEI messages, and some corrections, had been added in the ISO/IEC 4th ed. and were not yet in the ITU 5th edition.

  - DAM1 was issued in ISO/IEC at the January 2019 meeting, containing the annotated regions and fisheye video SEI messages (which were also not yet in ITU 5th edition).

  - Software for some of the SEI messages is also available – clarify on AR and FEV.

We need to decide whether to consent a new edition in ITU in March or wait until October (e.g., for the annotated regions SEI message and fisheye to mature). [Consent, not with Annot. Regions]

[Still working on AR and FEV]

For October, a new edition of software might also be possible

- AVC [new edition recently done]

  - In ISO/IEC, FDAM 1 for adding SEI messages was issued in Macao, but there was a chance to still integrate that into the 9th edition (updated text issued in Marrakech).

  - Some of that material now yet in the ITU edition - target new edition in ITU. Now? [Consent now.]

- Policies of ISO/IEC and possible consequences for JCT-VC

  - Standards editing

  - Rules for standards under ballot

- Screen content coding status

  - Software (bug fixes and code cleanup remain needed for the SCM to become a completely adequate replacement for the HM); issuing a new edition would be appropriate if this work converges.

  - Conformance – This is one of the top needs for work. In ISO/IEC, we issued a DAM ballot as an output of the Ljubljana meeting, but there was a delay in producing the output text, which had only been delivered during the Macao meeting, such that the DAM ballot will close on March 4 and the FDAM to be issued in March [skipping FDAM]. The new (2nd) edition of the basis text had been published in 2018-08. The new (3rd) edition in ITU-T had been consented at the Ljubljana meeting, Last Call closed 2018-10-13, and pre-publication occurred on 2018-11-27. Any need for updates/corrections?

  - Reference software – In the last approved version, there were errors in profile/level/constraint syntax for SCC in SCM. At some point, perhaps October 2019, we should approve a new version. We may wish to defer the next version until there is more to put in it.

- HDR

  - SEI/VUI has been specified in new editions

  - Two TRs have been published in ITU-T (when?), and both had also been published in ISO/IEC in 2018-08.

  - Reference software to be developed – software relating to HDR was currently in the HM separate from the SCM, plus there is a separate HDRTools library

- [working on 2nd version] A new TR on signalling combinations in practical use is under development, this is to be ISO/IEC 23091-4 in ISO/IEC and H.Sup.UVSTCP in ITU-T. PDTR was issued in Macao, was planned to become TR at this meeting – also ITU Approval (and update the reference to 23091-2 if feasible – try to do that) Another WD for a later revision of the TR was also issued, with a plan for approving that version around 2020-04.

- Improvement of test model texts and software manuals was encouraged

- It was noted that software support for the SEI messages is desirable. Together with HDRTools and 360Lib, we have software for experimentation with some SEI messages. The following items were noted to be desirable additionally.

  - Fisheye projection – not mature yet at last meeting

  - SEI manifest and prefix indication - available

  - Region-wise packing might be improved to illustrate the use of padding for a cubemap (some degree of support for the SEI message is available in the software)

  - MCTS extraction information nesting - available

  - Annotated regions (software available and has been updated recently)

- Experimental uses of the HM, SCM, SHM, and HTM reference software remain of interest

Key deliverables initially planned from this meeting:

- ITU-T consent for new edition on HEVC [or wait until October for AR and FEV]

- ITU-T consent for new edition on AVC [equivalent to ISO/IEC FDIS of last meeting]

- New draft for annotated regions and fisheye ? (DAM1 under ballot in ISO/IEC closing date 2019-10-03)

- TR (ed. 2) on code point usage PDTR

- New HM, SHM, and SCM document versions? HM17 with SCM integrated? Not expected. (code cleanup remains needed for this to become a completely adequate replacement for the HM)

A single meeting track was followed for the meeting discussions.

Scheduling of discussions

Scheduling: Generally, meeting time was scheduled during 0900–2000 hours, with coffee and lunch breaks as convenient. The meeting had been announced to start with AHG reports and then proceed with review of contributions. Ongoing scheduling refinements were announced on the group email reflector as needed.

Some particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Sat. 6 July, 1st day

  - 0900-1300 Opening remarks, status review, AHG report review

  - …

- …

Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized and categorized as follows. Some plenary sessions were chaired by both co-chairmen, and others by only one. Chairing of other discussions is noted for particular topics.

- AHG reports (7) (section 2)

- Project development status (2) (section 3)

- SEI messages (2) (section Error: Reference source not found)

- Code points (3) (section Error: Reference source not found)

- Software development (1) (section Error: Reference source not found)

- Outputs & planning: AHG plans, Conformance, Reference software, Verification testing, CTC (sections 5, , and 7)

NOTE – The number of contributions in each category, as shown in parenthesis above, may not be 100% precise.

Topics discussed in final wrap-up at the end of the meeting

Notes on potential remainders near the end of the meeting:

- Output preparations (see section 7 for the full list)

- Plans

  - AHGs

  - CEs – None.

  - OLSs to be produced by the parent bodies (status and project planning information exchanged between each other)

  - Reflectors (jct-vc) & sites (phenix and ftp3) to be used in future work

  - Meeting dates (next meeting to start XXday, X March 2019)

  - Doc deadline (next meeting deadline XXday X March 2019)

There were no requests to present any "TBP" contributions in the closing plenary.

AHG reports (7)

These reports were discussed Saturday 6 July 0900–XXXX (chaired by GJS and JRO), except as otherwise noted.

JCTVC-AJ0001 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

This document reports on the work of the JCT-VC ad hoc group on Project Management, including an overall status report on the project and the progress made during the interim period since the preceding meeting.

In the interim period since the 35th JCT-VC meeting, work towards finalizing the following (4) documents had been performed:

- For HEVC conformance testing, Draft 11 of HEVC conformance for screen content coding (SCC) and non-intra high-throughput profiles (not available)

- For non-normative guidance on HEVC encoding practices, Update 10 of the HEVC Model (HM) 16 encoding algorithm description

- For video code points coordination, Draft 7 of usage of video signal type code points, and Draft 4 toward version 2 of technical report on usage of video signal type code points.

The work of the JCT-VC overall had proceeded well in the interim period, although with relatively few input documents submitted to the current meeting (the lowest number ever at a JCT-VC meeting). Some discussion had been carried out on the group email reflector (which had approx. 1289 subscribers as of July 05, 2019), and all output documents from the preceding meeting had been produced.

The output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.it-sudparis.eu/jct/) or the ITU-based JCT-VC site (http://wftp3.itu.int/av-arch/jctvc-site/2019_07_AJ_Gothenburg/), particularly including the following:

- High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 10 (JCTVC-AI1002) [Posted 2019-06-30]

- Conformance Testing for HEVC Screen Content Coding (SCC) Extensions and Non-Intra High Throughput Profiles (Draft 11) (JCTVC-AI1004) [Posted 2019-XX-XX]

- Usage of video signal type code points (Draft 7) (JCTVC-AI1003) [Posted 2019-03-26]

- Draft 4 toward version 2 of technical report on usage of video signal type code points (JCTVC-AI1011) [Posted 2019-06-26, updated 2019-06-27]

The seven ad hoc groups had made progress, and reports from those activities had been submitted.

Software maintenance generally was progressing according to plans. Further action remains necessary for full integration including SCM tools as main branch.

Since the approval of software copyright header language at the March 2011 parent-body meetings, that topic seems to be resolved.

Released versions of the software are available on the SVN server at the following URL:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/version_number,
where version_number corresponds to one of the versions described below – e.g., HM-16.20.

Intermediate code submissions can be found on a variety of branches available at:
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/branches/branch_name,
where branch_name corresponds to a branch (eg., HM-16.20-dev).

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system (https://hevc.hhi.fraunhofer.de/trac/hevc). That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

The ftp site at ITU-T is used to exchange draft conformance testing bitstreams. The ftp site for downloading bitstreams is http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/.

A spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available in the same directory. It includes the list of bitstreams, codec features and settings, and status of verification.

9 input contributions to the current meeting (not counting the AHG reports) had been registered for consideration at the meeting. Most of these relate to errata reporting, non-normative improvements, and implementation.

A preliminary basis for the document subject allocation and meeting notes for the 36th meeting had been circulated to the participants by being announced in email, and was publicly available on the ITU-hosted ftp site (http://wftp3.itu.int/av-arch/jctvc-site/2019_07_AJ_Gothenburg/).

AJ0002

This document reports the work of the JCT-VC ad hoc group on (HEVC and AVC) test model editing and errata reporting (AHG2) between the 35th meeting in Geneva, CH (Mar. 2019) and the 36th meeting in Gothenburg, SE (Jul. 2019).

Update 10 of the encoder description was produced (JCTVC-AI1002), incorporating description of recently added pre-encoder filtering and updates to rate control.

Contribution JCTVC-AJ0021 HEVC errata: on the general decoding process is noted.

AI002x was about this:

Email regarding 3DV ATM

As you may recall from the previous meeting, there was a disparity between software and text in 3DV ATM:

depth_view_present_flag and texture_view_present_flag in the software were only available if profile_idc==ThreeDV_HIGH;

for the text, there was no profile_idc check, allowing the flags to be used for profile ThreeDV_EXTEND_HIGH.

To summarize:

the flags in question were added in JCT3V-B0147, and the EXTEND_HIGH profile was already an option in the software at that point in time.

http://phenix.int-evry.fr/jct2/doc_end_user/documents/2_Shanghai/wg11/JCT3V-B0147-v2.zip

The software and text seem to be completely different from the outset:

Software: num_views_minus1 is not restricted by profile_idc, but depth_view_present flag is restricted.

Text: num_views_minus1 and depth_view_present flag are restricted by profile_idc, but only at a higher level, and both by profile idc 138 (ThreeDV_HIGH).

It was commented that we need to make sure which codebase we were looking at.

K. Sharman studied this somewhat and found several versions of the software.

Try to contact Dong Tian?

The recommendations of the HEVC test model editing and errata reporting AHG are for JCT-VC to:

- Encourage the use of the issue tracker to report issues with the text of both the HEVC specification and the encoder description.

- Confirm resolutions of open tickets (if any) in the issue tracker and close them.

- Confirm behaviour of 3DV ATM versus the corresponding text.

- Review input documents identified above.

AJ0021 (like AI0022) has an errata report.

AJ0003

This report summarizes the activities of the AhG on HEVC HM, SCM, SHM, HTM and HDRTools software development and software technical evaluation that have taken place between the 35th and 36th JCT-VC meetings.

The current software model versions are:

- HM 16.20 (Sep. 2018)

- HM 16.20 + SCM 8.8 (Mar. 2018)

- SHM 12.4 (Jan. 2018)

- HTM 16.3 (Jul. 2018)

- HDRTools 0.18 (Sep. 2018)

- JM 19.0

- 3DV-ATM 15.0

The chairs have been considering using git for maintaining software.

AVC’s JM and 3DV-ATM can be found: https://vcgit.hhi.fraunhofer.de/jct-vc

The git repository also includes a merge request to update the code base to use cmake and fix incompatibilities with modern compilers.

There have not been any further releases of the HM reference software.

The chairs have been considering using git for maintaining software. HM and SCM have been ported but have not been made public yet.

There are several modifications still to be included:

- The adopted changes in JCTVC-Y0038 that include changes in the GOP settings, which require coordination with JVET for JEM development.

- Fisheye SEI: the status is unchanged from the previous meeting: a software patch had previously been provided to the software coordinators. It was reviewed just before the 35th JCTVC meeting and was significantly modified. The original author has been asked to verify the latest revision.

- Annotated regions SEI: A third revision of the patch is in the process of being reviewed.

- JCTVC-AG0026 (Random Access encoding with HM for video-based point cloud codec): Software was provided and reviewed in several rounds. There were concerns from the software coordinators regarding structure and interaction with interlace coding, which have not been resolved yet.

- A change to the default value of FULL_NBIT to ‘1’ to align test conditions. The impact of this was previously reported in JCTVC-AI0003.

- A bug has been reported on signed coding (which was recently moved into functions SyntaxElementWriter::xWriteSCode and the equivalent xReadSCode), where sign+magnitude was being coded, not 2’s complement. It is reported that the same bug exists in VTM, but not in JM.

The temporal filtering software had not yet been released in an HM release. This was expected to occur during the current meeting.

The HM document output of the previous meeting may also need a final revision to describe the parameter names to be found in the software.

The following SEI messages currently do not have any support:

- Fisheye video info (JCTVC-AH1012) (payloadType == 152)

- Motion-constrained tile sets extraction info nesting (159)

- SEI manifest (JCTVC-AG1005) (200)

- SEI prefix indication (JCTVC-AG1005) (201)

- Annotated regions (JCTVC-AH1012) (202)

The following SEI messages currently do not have control mechanisms to configure the encoder to form them (although there is code to put the messages in the bit-stream):

- Pan scan rectangle (payloadType == 2)

- Filler payload (3)

- User data registered ITU T T35 (4)

- User data unregistered (5)

- Scene info (9)

- Picture snapshot (15)

- Progressive refinement segments (16, 17)

- Film grain characteristics (19)

- Post filter hint (22)

- Deinterlace field identification (143)

- Content light level information (144)

- Dependent RAP indication (145)

- Coded region completion (146)

- Ambient viewing environment (148)

Example config files for using the provided software for the annotated regions SEI message would be desirable.

The following are persistent bug reports where study is encouraged:

- High level picture types: IRAP, RASL, RADL, STSA:

Tickets #1096, #1101, #1333, #1334, #1346.

- Rate-control and QP selection – numerous problems with multiple slices:
		Tickets #1314, #1338, #1339.

- Field-coding:
		Tickets #1145, #1153.

- Decoder picture buffer:

Tickets #1277, #1286, #1287, #1304.

- NoOutputOfPriorPicture processing:

Tickets #1335, #1336, #1393.

- Additional decoder checks:

Tickets #1367, #1383.

There have not been any further developments to SCM during this meeting cycle.

There have not been any further developments to SHM’s SHVC during this meeting cycle.

There have not been any updates to the HTM of MV-HEVC and 3D-HEVC.

There have not been any releases to the HDRTools software. However, the latest development branch includes the following changes:

- Proper namespace support and other fixes (DM)

- Support of possible integration with external software such as the VTM (DM)

- Cmake support (DM)

- Warning elimination for GCC v7.x/v8.x (DM/AT)

- Support of R12B/R12L formats (AT)

- Support of P3D65 YUV formats (AT)

- Generalization of 4:2:2 to 4:2:0 conversions (AT)

- Scaling filter fixes and addition of SHVC filtering (for upscaling/downscaling) (AT)

- LUT improvements as per Yonghye Kwon's suggestion (see Issue #3 on gitlab) (AT)

A new release is expected after coordination with the JVET software coordinators for a possible integration of HDRTools with the VTM software.

There have not been any updates to the JM, JSVM and JMVM software.

The AHG recommended to:

- Continue to develop reference software based on HM 16.20, HM 16.20 + SCM 8.8, SHM 12.4, HTM 16.3 and HDRTools 0.18 and improve their quality.

- Discuss moving from SVN to git for repositories.

- Test reference software more extensively outside of common test conditions.

- Add more conformance checks to the decoder to more easily identify non-conforming bit-streams, especially for profile and level constraints.

- Encourage people who are implementing HEVC based products to report all (potential) bugs that they are finding in that process.

- Encourage people to submit bit-streams that trigger bugs in the HM. Such bit-streams may also be useful for the conformance specification.

- Encourage people to submit configuration files that trigger bugs in HDRTools.

- Continue to investigate the merging of branches.

- Keep common test conditions aligned with JVET.

It was commented that integration of HDRTools into the HM would be desirable so that these would not need to be run separately.

Moving to git for all software was suggested, to enable using the same system as has been used in JVET. This was agreed. We should note that the SVN is no longer the place for updates, but the software should remain accessible there.

JCTVC-AJ0004 JCT-VC AHG report: Conformance test development (AHG4) [T. Suzuki, R. Joshi]

The ftp site at ITU-T is used to exchange bitstreams. The ftp site for downloading bitstreams is

http://wftp3.itu.int/av-arch/jctvc-site/bitstream_exchange/

The spreadsheet to summarize the status of bitstream exchange, conformance bitstream generation is available at this directory. It includes the list of bitstreams, codec features and settings, and status of verification.

[consider what else to copy in here]

The only updated section is about SCC conformance.

DAM ballot was passed, and FDAM was skipped. But the updated document for SCC conformance needs to be released. The updated document had not yet been released, and should be released during this meeting.

JCTVC-AJ0005 JCTVC AHG report: Test sequence material (AHG5) [T. Suzuki, V. Baroncini, E. François, P. Topiwala, S. Wenger]

[No significant update since the last meeting]

AJ0006

JCT-VC AHG 6 report: Report development for usage of video signal type code points

This report summarizes the activities of the AhG on report development for usage of video signal type code points that have taken place between the 35th and 36th JCT-VC meetings. Activities focused on work on text and diagrams of the draft output document JCTVC-AH1003 (Version 1 draft 6) intended for a Technical Report in ISO/IEC and Supplement document in ITU-T, and a follow-on draft JCTVC-AH1011 (Version 2 draft 3) for future changes to this Technical Report / Supplement.

The group worked on producing two output documents (JCTVC-AH1003 version 1 draft 6, and JCTVC-AH1011 version 2 draft 4) based on the 35th meeting discussions. For JCTVC-AH1003, The TR report version 1 draft had some minor edits to the output document and released soon after for ITU-T publication. For JCTVC-AH1011, It was agreed to include the proposed JCTVC-AH1021 format proposed by Apple in a new annex, JCTVC-AH1026 (ICTCP usage in the HDR-WCG table), and JCTVC-AH1022 (baseband topics in two additional tables) in the output document for the Version 2 of the TR (JCTVC-AI1011), whose planned approval timeline goal was to reach Final TR state by October ( allow for dates for ISO/IEC and ITU to align better) . Other edits were to improve the reference section, cite ST 2113 for P3 colorspaces, and make editorial refinements to make the document more readable.

The AHG had two teleconferences, on June 19th and June 25th. Notes from each call are listed below:

Call 1 ( June 19th)

Attendees: Atsuro, Alexis, Gary, Sean, Lars, Yasser

- Style checks

  - Check titles for capitals in table headers  and section headers ( including System identifiers)- Done

  - Check CbCr notes throughout doc ( including figure)- Done

  - Remove comments in body of document- Done

  - Centering of cells on checkmarks-Done

  - Check in baseband table the order of Systems Identifiers tags with reference to main table.-checked

  - Revise table number in Annex-Done

- Add in Apple document reference

- Add in registry for coding equations for ICTCP

- For table 8 add in not in cell that not specified in ITU-R side - Done

- Send email on discussion if anything else need to be added to document.

Call 2  ( June 25th)

Attendees: Atsuro, Alexis, Chris S, Gary, Sean, Lars, Walt, Yasser

- Went over changes on draft of first call Most were approved with some discussion on the following

  - On table 8 , Atsuro brought up that for BT709_YCC and BT709_RGB that these in the smpte spec were limited to UHDTV1 (3840x2160) and up to 60 Hz. This was put in as a footnote for those cell entries.

  - Some discussion of colour volume, colorimetry, color space were discussed again because the definitions did not quite capture the it included colour space, intensity quantization and transfer function. We did not quite resolve this on the call so decided this could be changed at the meeting and integrated as an output document for that meeting.

i.      Action item for Chris Seeger to resolve this before the next meeting starts ( July 5th)

      - After the meeting Sean suggested : colour coding characteristics: Combination of colour gamut, colour primaries, dynamic range, transfer function, colour representation, video range, and chroma sample location. Still may need some emails to fully resolve this.  (YS: we would need to check if all terms are defined in the document. If not I would need a definition for each term)

  - Timetable- Group believe the document is mostly complete and should be a goal to get this approved for ITU revision by the October meeting (otherwise they will meet next year) and if possible go thru the commenting process on the ISO side at the same time to keep both groups publication process in alignment in alignment.  This means at the next meeting (Gothenburg) we should have a PDTR as an output (if the requesting process allows for this).

At the meeting, JCT-VC should review input contributions, resolve comments, and continue working on the version 2 text for approval.

On “colour coding characteristics” – to review candidate draft text during the meeting.

On the alternative transfer charactistics SEI message – if the report should say something about that, we should review candidate content – at the previous meeting we had the impression that it was not in widespread use.

JCTVC-AJ0007 JCT-VC AHG Report: Supplemental enhancement information (AHG7) [J. Boyce, C. Fogg, G. J. Sullivan, Y.-K. Wang]

This document summarizes the activity of AHG7: Supplemental enhancement information between the 35th meeting in Geneva, CH (Mar 2019) and the 36th meeting in Gothenburg, SE (July 2019).

There were no SEI-related JVET output documents from the prior meeting. An MPEG output document was prepared.

There was no email reflector discussion, which is to take place on the main JCT-VC reflector.

There are two SEI related input contributions, both proposing new SEI messages.

JCTVC-AJ0022

Presentation info SEI message proposal for 8K UHD coded as 4K applications	C. Fogg (MovieLabs)

JCTVC-AJ0024

NALU verification SEI message	P. Andrivon, E. François (InterDigital)

It was remarked that AJ0027 on pic_struct is also relevant.

An HM software update was provided to the software coordinators for annotated regions SEI message corresponding to the specification in JCTVC-AH1002, based on feedback from the software coordinators provided during the Geneva meeting in March.

Regarding the fisheye SEI message, it was noted at the Geneva meeting that software was needed for the projection equations in 360Lib. There has not been any such software provided during this period.

The AHG recommends the following:

Review input contributions

Consider status of the Fisheye SEI message, because of lack of SW for implementation of the fisheye projection equations

It was noted that a ballot is ongoing that includes the fisheye message, so the October meeting, which is after that ballot closes, would seem to be the appropriate time to evaluate the status of that effort.

Coffee break 1030.

Project development, status, and guidance (2)

General (1)

JCTVC-AJ0020 Deployment status of the HEVC standard [G. J. Sullivan (Microsoft)]

Reviewed Saturday 1100 (GJS).

In the discussion of the contribution, suggestions for future updates included:

- Information about bitstream analyzers and conformance test suites

- More information about usage in surveillance applications

- Information about video players (e.g., ffmpeg – what packages does it use inside of it?)

Errata reports (1)

JCTVC-AJ0021 HEVC errata: on the general decoding process [Y.-K. Wang (Futurewei)]

Reviewed briefly Saturday 1105 (GJS)

Follow up of AI0022 with some additional information but not exact spec text. This is supported in spirit.

JCTVC-AJ0023 On sRGB and sYCC [T. Suzuki (Sony)]

Reviewed Saturday 1130 (GJS)

This contribution proposes to change the text relating transfer_characteristics and matrix coefficients in regard to IEC 61966-2-1. It asserts that the semantics of transfer characteristics (Table E.4), and the semantics of matrix coefficients (Table E.5) need correction.

In an initial review, this report seemed to be correct.

It probably indicates a need for correction in AVC, HEVC, CICP, and JPEG XR when feasible.

We don’t plan to create amendments just for this, but should keep track of the issue and plan to correct it when feasible.

It was agreed to issue an errata output document and include this.

JCTVC-AJ0026 On HRD for splicing [Y. Sanchez, R. Skupin, K. Sühring, T. Schierl (HHI)]

Reviewed Saturday 1110 (GJS)

This contribution provides details of an asserted bug of the hypothetical reference decoder (HRD) model of VVC and HEVC, specifically when splicing. The fist aspect of this contribution proposes a bug-fix for the reported issue. In the second aspect it is proposed to add a flag to picture timing SEI message that it is asserted would help a splicer to easily identify pictures within the bitstream, at which seamless splicing can be achieved.

Document JVET-O0496 reports the same issue for VVC and proposes the same bug fix as the first aspect of this contribution.

The second aspect is something proposed for VVC, not for HEVC.

We should further study this, together with other errata reports.

Communication with parent bodies (X)

Technical contributions (2)

JCTVC-AJ0022 Presentation info SEI message proposal for 8K UHD coded as 4K applications [C. Fogg (MovieLabs)]

Reviewed Tuesday 9 July 1110 (GJS)

This proposal introduces two alternative presentation info SEI messages for AVC and HEVC (and future VVC) that can be used to indicate that the coded bitstream is intended for display at a level of service greater than indicated by the level and resolution signalled in the sequence parameter set. For example, this proposed SEI message could convey that the intended presentation of the content is 8K, while the bitstream conforms to level 5.x ("4K") and therefore can then be decoded by legacy level 5.x IRDs (integrated receiver decoders). In this use scenario, the decoded output pictures should be upscaled by an indicated factor (2x in this example), in an IRD display with HDMI 2.1 "8K" output, or by the display device itself.

No particular scaling mechanism is suggested in this proposal. Instead, only an indicator that the program or content conveyed within the bitstream is intended for a higher-resolution service than would otherwise be inferred from the sequence parameter set and level indicators.

Two alternative syntaxes are proposed for a potential SEI message.

| presentation_info( payloadSize ) { | Descriptor |
| --- | --- |
| presentation_info_cancel_flag | u(1) |
| if( !presentation_info_cancel_flag ) { |  |
| presentation_info_persistence_flag | u(1) |
| presentation_scaling_factor_numerator | ue(v) |
| presentation_scaling_factor_denominator | ue(v) |
| } |  |
| } |  |

| mastering_display_viewing_info( payloadSize ) { | Descriptor |
| --- | --- |
| mastering_display_viewing_info_cancel_flag | u(1) |
| if( !mastering_display_viewing_info_cancel_flag ) { |  |
| mastering_display_viewing_info_persistence_flag | u(1) |
| mastering_display_horizontal_viewing_angle | ue(v) |
| } |  |
| } |  |

The presenter said that for higher resolution video (e.g., 8K) the expected viewing angle is larger and that the camera operator uses less panning (depending on the observer to track the objects themselves). High frame rate was also said to be especially important when larger viewing angles are used (since the camera operator is not following the moving objects with a camera as much).

It was asked what a receiver/decoder would do with this information, as it would just use whatever display capability that it had. It was remarked that the second syntax seems closer to describing something usable to the receiver. The receiver/display also does not necessarily know the viewing angle of the user.

It was commented that it is not just viewing angle that governs the user experience. For example, convergence / viewing distance is also relevant.

The proponent said this was partly for identifying the service for which the content was authored rather than necessarily to facilitate local display adaptation. For example, it could be used for identifying whether the video bitstream should be selected instead of some other bitstream. It was commented that this might be something that should be handled outside of the bitstream.

The proponent said that it could be used, e.g., for automatic cropping if it indicated that a wider field of view was intended than what is supported in the receiving user environment.

There was discussion of whether this should be combined with mastering display colour information, and it was commented that having those as separate SEI messages seemed preferable. For MDCV and AVE, the intent was for there to be some local adaptation of the content if the viewing environment differed from that described in the SEI message.

It was commented that other kinds of intended viewing environment description may be good to study.

Further study was encouraged, including identifying other characteristics that may be helpful to convey, study of potential use cases and applications, and potential receiving-side adaptations in response to the information.

JCTVC-AJ0024 NALU verification SEI message [P. Andrivon, E. François (InterDigital)]

Discussed Tuesday 1145 (GJS)

This contribution relates to error resilience and error concealment at the elementary stream level. It is proposed to carry in an SEI message a hash value characterizing (a group of) NALUs so that corruption of the (group of) NALU may be detected prior to decoding of said (group of) NALUs. It is asserted that this proposition allows verifying the integrity of a (group of) NALUs or the entirety of the coded stream so that appropriate error-concealment or error resilience measures can be triggered when a corruption is detected.

Described phenomenon:

| coded_data_hash( payloadSize ) { |  |
| --- | --- |
| cdh_type | u(8) |
| cdh_nalu_type | u(8) |
| if( cdh_type == 0 ) |  |
| for(i = 0 ; i < 16 ; i++ ) |  |
| cdh_md5[ i ] | b(8) |
| if( cdh_type == 1 ) |  |
| cdh_crc | u(16) |
| } |  |

The syntax was said to be similar to that in the decoded picture hash SEI message, except for not including a checksum type. This other type could also be used in principle.

It was commented that in the case of the decoded picture hash SEI message, this was partly provided because it is difficult data to generate by a system layer – because its calculation involves the complete decoding process. The picture hash SEI message was partly designed just for internal purposes in testing decoders, rather than necessarily being something we expected to be used in deployed consumer services.

It was asked whether such issues should be handled at a system layer rather than being multiplexed into the video content.

It was asked what the system would do with this information, and the proponent said receiving systems could take some corrective action or ignore an SEI message if it can be identified as corrupted.

It was commented that SEI message data is typically repeated periodically, so errors would be detected/corrected eventually.

It was commented that we have generally focused on data losses rather than data corruption in our designs.

Further study was encouraged, including determining its relevance to system designs, identifying conditions under which data corruption might be experienced, and identifying particular applications and use cases.

JCTVC-AJ0025 GOP-based temporal filter setting for low-delay coding [J. Enhorn, P. Wennersten, R. Sjöberg (Ericsson)]

Discussed Tuesday 1231 (GJS).

This contribution proposes a configuration value change for low-delay coding when enabling GOP-based temporal filter from JCTVC-AI0023 in HM. The value of TemporalFilterStrengthFrame4 is proposed to be set to 0.40 instead of 0.95. The HM source code is not modified.

The new setting was reportedly tested for the LDB and LDP configurations and compared to use of HM-16.20 (i.e., compared to no temporal filtering at all). The average Y/U/V BD-rates are reported to be -1.1%/-1.0%/-1.4% (LDB) and -2.0%/-1.8%/-1.9% (LDP).

This can be compared to the -0.2%/-0.9%/-1.5% (LDB) and -1.4%/-1.8%/-2.2% (LDP) that was reported in JCTVC-AI0023.

It is proposed that the value for TemporalFilterStrengthFrame4 in the two configuration files encoder_lowdelay_P_main10.cfg and encoder_lowdelay_main10.cfg is set to 0.40.

It was agreed to include this in the 16.21 release, which had been planned but had not yet occurred.

The text of the test model document does not cover the low delay case, so it is not affected.

It was noted that the gain in RA CTC was ~5% when used with a QP setting change, and 4.5% without the QP setting change. (In the new report JVET-O0549, the corresponding gain was 3.5%, so the difference between HM and VTM is not affected by more than about 1% if the codecs are similarly configured to either use or not use such filtering.) More gain seemed to be shown for higher resolution test sequences. The proponent said that subjective effects had not been studied.

This remains not used in the CTC.

JCTVC-AJ0027 AHG7: on pic_struct [S. McCarthy, F. Pu, T. Lu, P. Yin, W. Husak, T. Chen (Dolby)]

Reviewed Saturday 1210 (GJS).

This contribution proposes modifications to existing picture timing SEI message syntax and syntax values in HEVC and AVC to enable repetition of an arbitrary number of frames. The existing HEVC and AVC picture timing SEI message syntax element, pic_struct, is defined to indicate whether a decoded and displayed picture should be repeated, and also whether a picture should be displayed as a frame or as one or more fields. In HEVC and AVC, pic_struct supports only frame doubling and tripling, which limits its usefulness for display of video on high scan rate displays, such as 120 Hz displays. In this contribution, it is proposed for HEVC and AVC to:

- define a new pic_struct value that indicates frame repetition; and

- add a new syntax element to indicate the number of frames to be repeated.

It is additionally proposed to add HEVC pic_struct values 9 through 12 to AVC to facilitate transcoding and consistency between HEVC and AVC.

Comments:

- We should not confuse this with display frame rate. It is not necessary for the display to correspond with what is expressed in a bitstream.

- HEVC has more values defined than AVC has defined (specifically values 9-12, which are field-oriented). These were put in before HEVC was finished. We have not previously extended the allowed values (although we briefly considered it – see JCTVC-X1005).

- The spec says to ignore reserved values of pic_struct.

- The way the new syntax element is proposed to be added into the existing message, and the location in which it is proposed to be added.

- The CpbDpbDelaysPresentFlag compounds the extensibility problem

- Can we really expect decoders to react appropriately?

- AVC may have different issues than HEVC

- Could there be a way to define a new SEI message that would do this? (It would probably need to be used with fixed_pic_rate_within_cvs_flag equal to 0 to avoid semantics conflicts.)

- Is it really useful/necessary to indicate repetitions explicitly? This has been a perennial question for this SEI message and the repetition functionality. The decoder-side adaptation to the local display frame rate is ultimately the decoder’s responsibility rather than the encoder’s responsibility.

JCTVC-AJ0028 Encoder-only Supplemental Motion Vector Estimation for Point cloud Coding content [V. Zakharchenko, J. Chen (Futurewei)]

Discussed 1250 Tuesday (GJS)

This contribution proposes an encoder-only motion estimation based on external information for point cloud content. The motion estimation is performed at the encoder side based on the supplementary information provided by the video-based point cloud compression engine. Source pictures generated by the video-based point cloud compression engine can be accompanied with an additional information that is generated during projection and atlas composition stage in the video-based point cloud compression solution. Samples in the selected picture are allocated based on 3d coordinates, and thus the 2d location can be inferred from the information that is present in coded point cloud bitstream.

The nature of the picture that contains projected point cloud atlas can’t guarantee temporal consistency in patch allocation. The method suggests introducing an additional start point for the motion estimation process to solve this problem. As the result, the precise motion vector can be generated and encoded, providing a good spatial candidate for motion vector candidate list construction for the subsequent coded units.

The method was reportedly tested under V-PCC TMC2 v6 RA, common test configurations. The average D1/D2/Y/U/V total BD-rates were reported to be −6.4%/−6.6%/−10.8%/−16.7%/−15.3% for the dynamic point cloud content.

It was proposed to adopt the proposed method into the HM software and keep the method disabled by default for HM CTC and enabled for V-PCC CTC only.

We already have some software in the HM for experimental highly multiview experimentation beyond what the standard can do.

This is actually not violating the syntax or decoding process of HEVC; it is just an encoder search scheme for use in this particular application. This scheme is used in MPEG VPCC testing already, and it was reported that it would helpful to incorporate it into the HM rather than distributing it as a patch.

This scheme was said to be useful whenever additional motion information could be fed to the encoder; it is not really customized for PCC.

Decision (SW): Adopt (16.22) and incorporate into HM 16 Update 11 algorithm description.

JCTVC-AJ0029 AHG7: indication of shutter angle for variable frame rate application [S. McCarthy, T. Lu, F. Pu, P. Yin, W. Husak]

TBP

Project planning

Joint meeting

[None.]

Text drafting and software quality

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text. Similarly, software coordinators have the discretion to evaluate contributed software for suitability in regard to proper code style, bugginess, etc., and to not integrate code that is determined inadequate in software quality.

Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in CEs).

Suggestions for future meetings included the following generally supported principles:

- No review of normative contributions without draft specification text

- HM text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

As general guidance, it was suggested to avoid usage of company names in document titles, software modules, etc., and not to describe a technology by using a company name. Also, core experiment responsibility descriptions should name individuals, not just companies. AHG reports and CE descriptions/summaries are considered to be the contributions of individuals, not companies.

General issues for CEs and TEs

Group coordinated experiments have been planned in previous work, although none were established at the current meeting. These may generally fall into one of two categories:

- "Core experiments" (CEs) are the experiments for which there is a draft design and associated test model software that have been established.

- "Tool experiments" (TEs) are the coordinated experiments on coding tools at a more preliminary stage of work than those of "core experiments".

A preliminary description of each experiment is to be approved at the meeting at which the experiment plan is established.

It is possible to define sub-experiments within particular CEs and TEs, for example designated as CEX.a, CEX.b, etc., for a CEX, where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the HM software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

CE descriptions need to be fully precise – this is intended as a method of enabling full study and testing of a specific technology. Greater discipline in terms of what can be established as a CE may be an approach to helping with such issues. CEs should be more focused on testing just a few specific things, and the description should precisely define what is intended to be tested (available by the end of the meeting when the CE plan is approved).

It was noted that sometimes there is a problem of needing to look up other referenced documents, sometimes through multiple levels of linked references, to understand what technology is being discussed in a contribution – and that this often seems to happen with CE documents. It was emphasized that we need to have some reasonably understandable basic description, within a document, of what it is talking about.

Software study can be a useful and important element of adequate study; however, software availability is not a proper substitute for document clarity.

Software shared for CE purposes needs to be available with adequate time for study. Software of CEs should be available early, to enable close study by cross-checkers (not just provided shortly before the document upload deadline).

The general agreed common conditions for single-layer coding efficiency experiments are as described in the output document JCTVC-AF1100.

The general timeline agreed for CEs was expected to be as follows: 3 weeks to obtain the software to be used as the basis of experimental feature integration, 1 more week to finalize the description and participation, 2 more weeks to finalize the software.

When a CE is planned, a deadline of four weeks after the meeting would be established for organizations to express their interest in participating in a CE to the CE coordinators and for finalization of the CE descriptions by the CE coordinator with the assistance and consensus of the CE participants.

Any change in the scope of what technology will be tested in a CE, beyond what is recorded in the meeting notes, requires discussion on the general JCT-VC reflector.

As a general rule, all CEs are expected to include software available to all participants of the CE, with software to be provided within two (calendar) weeks after the release of the relevant software basis (e.g. the SCM). Exceptions must be justified, discussed on the general JCT-VC reflector, and recorded in the abstract of the summary report.

Final CE descriptions shall clearly describe specific tests to be performed, not describe vague activities. Activities of a less specific nature are delegated to Ad Hoc Groups rather than designated as CEs.

Experiment descriptions should be written in a way such that it is understood as a JCT-VC output document (written from an objective "third party perspective", not a company proponent perspective – e.g. referring to methods as "improved", "optimized" etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in referenced documents that are also available in the JCT-VC document archive.

Those who proposed technology in the respective context (by this or the previous meeting) can propose a CE or CE sub-experiment. Harmonizations of multiple such proposals and minor refinements of proposed technology may also be considered. Other subjects would not be designated as CEs.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable to have more than just one proponent and one cross-checker.

It is strongly recommended to plan resources carefully and not waste time on CE work on technology that may have little or no apparent benefit – it is also within the responsibility of the CE coordinator to take care of this.

A summary report written by the coordinator (with the assistance of the participants) is expected to be provided to the subsequent meeting. The review of the status of the work on the CE at the meeting is expected to rely heavily on the summary report, so it is important for that report to be well-prepared, thorough, and objective.

A non-final CE plan document would be reviewed and given tentative approval during the meeting (with guidance expressed to suggest modifications to be made in a subsequent revision).

The CE description for each planned CE would be described in an associated output document numbered as, for example, JCTVC-X11xx for CExx, where "xx" is the CE number (xx = 01, 02, etc.). Final CE plans would be recorded as revisions of these documents.

It must be understood that the JCT-VC is not obligated to consider the test methodology or outcome of a CE as being adequate. Good results from a CE do not impose an obligation on the group to accept the result (e.g., if the expert judgment of the group is that further data is needed or that the test methodology was flawed).

Some agreements relating to CE activities have been established as follows:

- Only qualified JCT-VC members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting.

- All software, results, documents produced in the CE should be announced and made available to all CE participants in a timely manner.

- If combinations of proposals are intended to be tested in a CE, the precise description shall be available with the final CE description; otherwise it cannot be claimed to be part of the CE.

Alternative procedure for handling complicated feature adoptions

The following alternative procedure had been approved at a preceding meeting as a method to be applied for more complicated feature adoptions:

- Run CE + provide software + text, then, if successful,

- Adopt into HM, including refinements of software and text (both normative & non-normative); then, if successful,

- Adopt into WD and common conditions.

Of course, we have the freedom (e.g. for simple things) to skip step 2.

Common test conditions for HEVC Coding Experiments

No particular changes were noted w.r.t. the prior CTC for work within the current scope of JCT-VC. See the prior output documents JCTVC-AC1100 for HEVC test conditions, JCTVC-X1009 for SHVC test conditions, JCTVC-Z1015 for SCC test conditions., and JCTVC-Z1020 for HDR/WCG test conditions.

Software development planning

Software coordinators were asked to work out the detailed schedule for software updates with the proponents of adopted changes as applicable.

Any adopted proposals where necessary software is not delivered by the scheduled date in a timely manner may be rejected.

At a previous meeting (Sapporo, July 2014), it was noted that it should be relatively easy to add MV-HEVC capability to the SHVC software, and it was strongly suggested that this should be done. This remains desirable. Further study was encouraged to determine the appropriate approach to future software maintenance, especially in regard to alignment of 3D video software with the SHM software.

Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JCT-VC reflector (jct-vc@lists.rwth-aachen.de).

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| Test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Develop proposed improvements to the JCTVC-AB1002 HEVC Test Model 16 (HM 16) Update 9 of Encoder Description - Collect reports of errata for the HEVC and AVC specification and the published HDR-related technical reports. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), J.-R. Ohm, K. Sharman, G. J. Sullivan, A. Tourapis, Y.-K. Wang (vice-chairs) | N |
| Software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, and HDRTools software and their distribution. - Enable software support for recently standardized additional SEI messages. - Produce documentation of software usage for distribution with the software. - Prepare and deliver results, reporting templates, and anchor test results according to JCT-VC common conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 on HEVC and AVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), B. Li, K. Sharman, V. Seregin, G. Tech, A. Tourapis (vice-chairs) | N |
| Conformance test development (AHG4) (jct-vc@lists.rwth-aachen.de) - Study the requirements of HEVC conformance testing to ensure interoperability. - Collect errata reports and discuss potential work plans and testing methodology to further improve conformance testing specifications for HEVC and AVC. - Identify needs for HEVC conformance bitstreams with particular characteristics. - Collect, distribute, and maintain the bitstream exchange database and draft conformance bitstream test sets. | T. Suzuki (chair), R. Joshi (vice-chair) | N |
| Test sequence material (AHG5) (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material, especially focusing on new needs for HDR/WCG test material and corresponding SDR test material. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG3 and AHG6 regarding HDR/WCG testing. | T. Suzuki, V. Baroncini (co-chairs), E. François, P. Topiwala, S. Wenger (vice-chairs) | N |
| Report development for usage of video signal type code points (AHG6) (jct-vc@lists.rwth-aachen.de) - Produce the output draft text AI1011 and develop proposed improvements of its content - Study the industry usage of video signal type code points and identify the most common and important combinations of such code points (including study of the draft text AI1011). | Y. Syed, C. Fogg (co-chairs) | Tel. TBA (one or two calls with at least two weeks notice) |
| Supplemental enhancement information (AHG7) (jct-vc@lists.rwth-aachen.de) - Study the draft texts JCTVC-AE1005 [update] and JCTVC-AE1012 [update] for HEVC - Produce and study the draft texts JCTVC-AF1006 [update] for AVC. - Consider proposals for additional SEI message data and associated syntax and semantics specification. - Develop usage scenario descriptions and showcase demonstrations. - Coordinate with AHG3 for software support of SEI messages. | J. Boyce (chair), C. Fogg, H.-M. Oh, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |

Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production.

The need for a new item in the work programme for Q6/16 was noted, for additional SEI messages for AVC.

JCTVC-AI1000 Meeting Report of the 35th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2018-12-14] (near next meeting)

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

JCTVC-AI1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 10 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] (WG 11 N 1xxxx) [2019-05-31] (near next meeting)

This includes modification to describe the JCTVC-AI0023 temporal filtering.

The rate control model description should also be updated for JCTVC-AH0024 rate control.

JCTVC-AI1003 Usage of video signal type code points (Draft 7) [L. Borg, C. Fogg, W. Husak, C. Seeger, G. J. Sullivan, Y. Syed, A. Tourapis (editors)] (WG 11 TR N xxxxx) [2019-03-27] (no editing period)

No editing period.

DoCR by MPEG.

Updated reference to 23091-2.

JCTVC-AI1004 Conformance Testing for HEVC Screen Content Coding (SCC) Extensions and Non-Intra High Throughput Profiles (Draft 11) [R. Joshi, I. Moccagatta, G. J. Sullivan, T. Suzuki, J. Xu (editors)] (WG 11 FDAM N xxxx) [2019-04-30]

Editorial improvements were made.

A DoCR was also issued by WG11.

JCTVC-AG1005 Additional Supplemental Enhancement Information for HEVC (Draft 3) [J. Boyce, H.-M. Oh, G. J. Sullivan, A. Tourapis, Y.-K. Wang] (WG 11 preliminary draft for FDIS N xxxxx) [2018-xx-xx] (4 weeks)

No output

Draft DoCR N xxxx also to be produced by WG11.

This corresponds to content added in a draft for FDIS for HEVC in the WG 11 parent body.

Contains updated semantics for MCDV; Fix needed for typo in MCDV.

It should also contain the fix for the off-by-one error for MCTS extraction and any other errata aspects as noted.

This includes the following draft new SEI messages.

- Fisheye

- Manifest and prefix indication

Software work is needed, esp. for the new packing and projection (e.g. in 360Lib and HM).

JCTVC-AG1006 Additional Supplemental Enhancement Information for AVC (Draft 4) [C. Fogg, W. Husak, G. J. Sullivan, A. M. Tourapis, Y.-K. Wang] (WG 11 FDAM N xxxxx) [2018-12-14] (9 weeks)

No output

DoCR N xxxx also to be produced by WG11.

This includes the following draft SEI messages and notes aspects.

- We should mention in the WD that the CRI SEI message is in progress in ISO/IEC (WG 11 N 16675 of 2017-01) and it may need adjustments as with MDCV and CLL (which have been published in ITU-T) [Update]

- Content light level

- Content colour volume

- Omnidirectional video messages for AVC (not fisheye)

- Manifest and prefix indication

- Ambient viewing environment SEI message (see JCTVC-AF0021).

- Persistency scope signalling method (see JCTVC-AF0025)

- Updated semantics of mastering display colour volume

- Miscellaneous minor fixes

Although software was not available for the manifest and prefix indication, it was planned to proceed since this contains a number of other topics that are needed by other bodies.

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye (editors)] (WG 11 N 15778)

No output: JCTVC-AG1008

Remains valid – not updated: JCTVC-X1009 Common Test Conditions for SHVC [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

- JCTVC-AI1011 Draft 4 toward version 2 of technical report on usage of video signal type code points [L. Borg, C. Fogg, W. Husak, A. Ichigaya, C. Seeger, G. J. Sullivan, Y. Syed, A. Tourapis (editors)] (WG 11 WD 1 N xxxx) [2019-05-30] (X weeks)

The basis should be aligned with the final v1 output.

In the discussion it was noted that some of the items in the annex are also in the main body and suggested to move that information to the annex.

Other editorial refinements are also planned.

If the MXF designation for ICTCP is determined, it should be included.

Remains valid – not updated JCTVC-AE1012 Annotated regions and fisheye video information SEI messages for HEVC (Draft 2) (J. Boyce, Y.-K. Wang, G. J. Sullivan) N 17662 in WG 11 [2018-10-31] (3 weeks)

No output: JCTVC-Z1013

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 16049)

Remains valid – not updated: JCTVC-Z1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2017-02-17]

No output: JCTVC-Z1016 through JCTVC-Z1019

Remains valid – not updated: JCTVC-Z1020 Common Test Conditions for HDR/WCG Video Coding Experiments [E. François, J. Sole, J. Ström, P. Yin (editors)] [2017-02-17] (1 month)

Pending – JCTVC-AF1100 Common Test Conditions for HM Video Coding Experiments [K. Sharman, K. Sühring (editors)] [2017-11-30] (1 month)

Revision to be issued if needed for coordination.

Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (usually starting meetings on the Friday of the first week and closing it on the Tuesday or Wednesday of the second week of the SG 16 meeting – a total of 6–6.5 meeting days, although different next time due to unusual WG 11 meeting date alignment), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices when it meets (starting meetings on the Saturday prior to such meetings and closing it on the last day of the WG 11 meeting – a total of 6.5 meeting days).

Some specific future meeting plans (to be confirmed) were established as follows:

- Thu. 3 – Wed. 9 October 2019, 37th meeting under ITU-T SG 16 auspices in Geneva, CH.

- Sat. 11 – Fri. 17 January 2020, 38th meeting under WG11 auspices in Brussels, BE.

- [add]

The agreed document deadline for the 37th JCT-VC meeting is XXday XX Sept. 2019. Plans for scheduling of agenda items within that meeting remained TBA.

XXX was thanked for the excellent hosting and organization of the 36th meeting of the JCT-VC.

The JCT-VC meeting was provisionally closed at approximately XXXx hours on XXday, XX July 2019, pending any need for a further meeting session to be announced on the email reflector. No need for a further meeting session was later identified.

- Annex A to JCT-VC report:
List of documents

- Annex B to JCT-VC report:
List of meeting participants

The participants of the thirty-sixth meeting of the JCT-VC, according to a sign-in sheet circulated during the meeting sessions (approximately XX people in total), were as follows:

The definitions of PB and PU are tricky for a 64x64 intra luma CB when the prediction control information is sent at the 64x64 level but the prediction operation is performed on 32x32 blocks. The PB, PU, TB and TU definitions are also tricky in relation to chroma for the smallest block sizes with the 4:2:0 and 4:2:2 chroma formats. Double-checking of these definitions is encouraged.