| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0445 |
| --- | --- |

| Title: | Non-CE11: Low complexity parallel significance map coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Takeshi Tsukuba, Ikai Tomohiro 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Email: | joels@qualcomm.com ikai.tomohiro@sharp.co.jp |
| Source: | Qualcomm, SHARP Corporation |  |  |

_____________________________

# Abstract

In HM5.0, for large transforms, significance map context calculation is dependent on the number of non-zero coefficients in a neighborhood support. Up to 10 conditions have to be checked to establish the right neighborhood. This is due to the border conditions on the block and to allow for 2-bin parallel processing. This contribution reduces the number of conditions to check while allowing for a 3-bin parallel processing. The average BD-rate of the method is 0.1%.

# Introduction

In current test model (HM), a five-point support is used to define a context model to code significance map of the transform coefficients (Fig 1). The significance flags of the neighbours from the support are added. However, the neighbours can be located outside the current block. Also, some neighbours are not considered in some cases to allow for a 2-bin parallel processing. Therefore, up to 10 conditions might be required to add the appropriate neighboring coefficients. For a 32×32 TU, this is up to around 10000 checks.

Fig. 1. Dependency in significance context calculation within support.

# Technical Description

This proposal combines the methods of [1] and [2] to achieve a lower complexity significance map for large transforms and allowing for processing 3-bins in parallel.

JCTVC-H0095 [1] uses a fix context for high-frequency coefficients, i.e., those with (xC,yC) such that

(xC>>2) + (yC>>2) < 3*(TU size >> 4)

This is around 67% of the coefficients in a 32×32 TU and includes the sub-blocks at the bottom row and right column.

JCTVC-H0427 [2] uses a modified support (Fig. 2 and Fig. 3) that doesn’t include the coefficient located below the current coefficient in any case. To achieve a 3-bin parallelization,  additional checks are required for the coefficient at position (0, 0) (top-left corner of the sub-block) and the coefficient at position (2, 3) as shown by green colour on Fig. 3. This is the same number of additional checks as in HM5.0. Then, the total number of conditions is 5 for the low frequency coefficients, since there is no need to check whether coefficients are out of boundaries because of [1].

Considering current HM5.0 implementation (which might not be optimal in this part), this implies that the number of conditions for the proposed method for a 32×32 TU can be up to 32*32*(0.67 + 0.33*5) = 2375 conditions, as opposed to 10000 of the current method.

Fig. 2. Modified support for the parallel processing.

Fig. 3. Modified support and conditions for the three bins parallel processing.

# Simulation results

The method is implemented on top of HM5.0. Results are provided using common test conditions and they are summarised in the tables below. More detailed test data can be found in the accompanied excel  sheets.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.08% | 0.21% | 0.32% | 0.02% | 0.10% | 0.15% |
| Class B | 0.12% | 0.38% | 0.40% | 0.04% | 0.12% | 0.13% |
| Class C | 0.09% | 0.12% | 0.14% | 0.04% | 0.15% | 0.15% |
| Class D | 0.08% | 0.19% | 0.15% | 0.04% | 0.14% | 0.07% |
| Class E | 0.23% | 0.38% | 0.43% | 0.10% | 0.48% | 0.41% |
| Overall | 0.12% | 0.26% | 0.28% | 0.05% | 0.19% | 0.17% |
|  | 0.12% | 0.27% | 0.29% | 0.05% | 0.19% | 0.17% |
| Class F | 0.09% | 0.17% | 0.05% | 0.04% | 0.14% | 0.08% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A (8bit) | 0.06% | 0.11% | 0.01% | 0.08% | 0.19% | -0.18% |
| Class B | 0.13% | 0.30% | 0.27% | 0.10% | 0.28% | 0.16% |
| Class C | 0.08% | 0.00% | 0.14% | 0.09% | 0.20% | 0.03% |
| Class D | 0.09% | 0.04% | 0.33% | 0.04% | 0.15% | -0.12% |
| Class E |  |  |  |  |  |  |
| Overall | 0.10% | 0.13% | 0.21% | 0.08% | 0.21% | 0.00% |
|  | 0.10% | 0.11% | 0.24% | 0.08% | 0.19% | -0.05% |
| Class F | 0.05% | 0.08% | 0.06% | 0.10% | 0.01% | 0.11% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.13% | 0.35% | 0.41% | 0.05% | -0.35% | 0.14% |
| Class C | 0.11% | 0.07% | 0.16% | 0.07% | 0.32% | 0.02% |
| Class D | 0.09% | -0.16% | 0.09% | 0.00% | 0.53% | 0.00% |
| Class E | 0.09% | -0.14% | -0.67% | 0.13% | -0.20% | 0.29% |
| Overall | 0.11% | 0.06% | 0.07% | 0.06% | 0.07% | 0.10% |
|  | 0.10% | 0.12% | 0.04% | 0.06% | 0.03% | 0.12% |
| Class F | 0.02% | 0.60% | -0.19% | 0.03% | -1.08% | -0.30% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Conclusion

In this contribution, a scheme to allow 3-bins in parallel for the large transforms significance map while reducing the complexity is proposed. The average BD-rate is 0.1%.

# References

[1] T. Ikai, “CE11.1 Improvement of significant_coeff_flag context at high frequency area,” JCTVC-H0095, JCT-VC 8th Meeting, San Jose, USA, Feb. 2012.

[2] V. Seregin, J. Sole, M. Karczewicz, “Non-CE11: Support modification for parallel calculation of significant map contexts,” JCTVC-H0427, JCT-VC 8th Meeting, San Jose, USA, Feb. 2012.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).