| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: San José, US, 9–17 Jan. 2014 | Document: JCTVC-P0084 |
| --- | --- |

| Title: | Indication of SMPTE 2084 and 2085 and carriage of 2086 metadata in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg 4300 N. 1st Street San Jose, CA 95134 USA Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Harmonic Inc. and Movielabs Inc. |  |  |

_____________________________

# Abstract

The contribution proposes text changes for HEVC video usability information (VUI) to indicate: (1) SMPTE ST.2084 (Electro-Optical Transfer Function for High Dynamic Range Reference Displays) with a new transfer_characteristcs entry;  (2) direct XYZ and SMPTE ST.2085 (YDzDx Color-Difference Encoding for XYZ Signals) color spaces with a new matrix_coeffs entries; and a new SEI for (3) carriage of SMPTE ST.2086 (Mastering Display Color Volume Metadata for High Luminance and Wide Color Gamut Images) metadata.

# Introduction

Following the additions of the recent proposal [1] to add Rec. 2020 [2] VUI indicators to HEVC, the proposed text included to support SMPTE ST.2084 [3], SMPTE ST.2085 [4], and SMPTE ST.2086 [5] in section 2 of this document builds on top of version 3 of the latest HEVC editor’s draft [6]. Testing of some combinations is provided in [7][8][9].

## Suggested use

An HEVC coded bitstream with an EOTF specified in SMPTE ST.2084 (signalled by the proposed transfer_characteristcis=16) should have BitDepth >= 10 for all sample component planes.

Bitstreams encoding video in the XYZ or YDzDx “color spaces” are expected to have color_primaries = 2 (undefined), with the proposed matrix_coeffs=11 for XYZ and matrix_coeffs=12 for YDzDx.

Similar to direct RGB (a.k.a. GBR), which can already be signalled by matrix_coeffs=0, a directly coded XYZ bitstream (proposed matrix_coeffs=11) could utilize the inter-component residual prediction RExt tool activated by PPS element luma_chroma_prediction_enabled_flag=1, as described in:

- RExt test model [10] section 4.8

- RExt draft [6] section 7.4.9.12 (Luma-chroma prediction semantics)

- RExt draft [6] section 8.6.6 (Residual modification process for transform blocks using luma-chroma prediction)

- “Experiment 2” from its original proposal [11].

# Proposed Text

Only the most relevant sections from Annex E [6] are copied herein for reader convenience., highlighted in yellow. An companion document to this proposal in the same .zip archive is provided  (JCTVC-O1005_v3_AnnexE_with_SMPTE_2084_and_2085.docx) that shows these proposed additions as tracked changes in the full context of the annex copied from the most recent editor’s HEVC draft specification.

The isolated changes to Annex D are exclusively provided in section 2.3 below (no companion document is necessary).

## SMPTE 2084 (EOTF)

The following entry (#16) is added to table E-4 (Transfer Characteristics)

| 16 | V=c1+c2Ln1+c3Lnm m=25234096×128=78.84375 c1=c3-c2+1=34244096=0.8359375 c2=24134096×32=18.8515625 n=26104096×14=0.1593017578125 c3=23924096×32=18.6875 | Society of Motion Picture and Television Engineers ST.2084 for 10, 12, 14 or 16 bit systems |
| --- | --- | --- |

## XYZ and SMPTE 2085 (YDzDx)

[ Equations E-43 through E-50 are added to support the Table E-5 matrix_coef entries. ]

Otherwise (matrix_coeffs is equal to 11), the following equations apply:

Y   = Round( Y )		(E-43)

Cb = Round(  Z )		(E-44)

Cr = Round(  X )		(E-45)

–	Otherwise (matrix_coeffs is equal to 12), the following equations apply:

| E'Dz=c1E'Z-E'Y2.0 c1=27632800 | (E-46) |
| --- | --- |
| E'Dx=E'X-c2E'Y 2.0 c2=27412763 | (E-47) |

Where input E’X, E’Y and E’Z are in the range [0.0, 1.0] and output E’Dz and E’Dx are in the nominal range (-0.5, +0.5).

Y=INT1015 E'Y+511.5∙2n-10		(E-48)

Cb=INT1015 E'Dz+511.5∙2n-10		(E-49)

Cr=INT1015 E'Dz+511.5∙2n-10		(E-50)

“n” denotes the BitDepth of the quantized signal, where n is an integer equal to or greater than 10.

Table E-5 – Matrix coefficients [additions shown only]

| 11 | XYZ | CIE 1931 See Equations E-43 to E-45 |
| --- | --- | --- |
| 12 | YDzDx | Society of Motion Picture and Television Engineers ST.2085 YDzDx Color-Difference Encoding for XYZ Signals See Equations E-46 to E-50 |

## SMPTE 2086 (Mastering Display Color Volume)

D.2.29 Mastering display color volume SEI message syntax

| mastering_display_color_volume( payloadSize ) { | Descriptor |
| --- | --- |
| for( c = 0; c< 3; c++) { |  |
| display_primaries_x [ c ] | u(16) |
| display_primaries_y [ c ] | u(16) |
| } |  |
| white_point_x | u(16) |
| white_point_y | u(16) |
| max_display_mastering_luminance | u(32) |
| min_display_mastering_luminance | u(32) |
| } |  |

D.3.29 Mastering display color volume SEI message semantics

This SEI message provides metadata for specifying the color volume (the color primaries, white point, and luminance range) of the display that was used in mastering video content of the output picture colour samples. The metadata is specified as a set of values independent of any specific digital representation, and is applicable to three-color additive display systems. The mastering display is a three-color additive display and projection system that has been configured to the mastering color volume.

This metadata does not specify the measurement methodologies and procedures for capturing the parameters of the metadata as well as any description of the mastering environment. Additionally, this metadata does not specify any information on the color transformations that would be necessary to preserve the creative intent on displays with color volumes different from the mastering display color volume.

The mastering_display_color_volume metdata persists in output order until any of the following conditions are true:

–	A new CVS begins.

–	A picture in an access unit containing a tone mapping information SEI message with the same value of tone_map_id is output having PicOrderCntVal greater than PicOrderCnt( CurrPic ).

The following elements given in CIE xy-coordinates (display_primaries_x[c], display_primaries_y[c], white_point_x, white_point_y), are derived by scaling and quantizing the traditional fractional unsigned values between 1.0 and 0 to a 16-bit unsigned integer (u(16)) signalled in the mastering_display_color_volume SEI:

quantized_CE_xy_coordinate = Round(( CIE_fractional_xy_coordinate * 216 ) -1 )

display_primaries_x[c] specifies the quantized CIE x-coordinate of the mastering display target primary color c.

display_primaries_y[c] specifies the quantized CIE y-coordinate of the mastering display target primary color c.

The combined xy display primaries coordinate represents the ratio of each CIE Y (c=0), Z (c=1), and X (c=2) tristimulus value to the sum of the tristimulus values as specified in CIE 15:2004 “Calculation of chromaticity coordinates”.

white_point_x  specifies the quantized CIE chromatictiy x-coordinate of the mastering display target white point.

white_point_y  specifies the quantized CIE chromatictiy y-coordinate of the mastering display target white point.

max_display_mastering_luminance specifies the target maximum luminance of the mastering display  in units of candela per square metre.

min_display_mastering_luminance specifies the target minimum luminance of the mastering display  in units of candela per square metre.

When present, max_display_mastering_luminance shall be greater than or equal to min_display_mastering_luminance.

# References

[1] T. Suzuki, “Proposal to support UHDTV colorimetory”, JCTVC-J0577, 10th meeting of JCTVC, Stockholm, Sweden, 11-20 July 2012.

[2] ITU-R BT.2020 (2012), Parameter Values for ultra-high definition television systems for production and internal programme exchange

[3] Society of Motion Picture and Television Engineers ST.2085 (2014), Electro-Optical Transfer Function for High Dynamic Range Reference Display.

[4] Society of Motion Picture and Television Engineers ST.2085 (2014), YDzDx Color-Difference Encoding for XYZ Signals

[5] Society of Motion Picture and Television Engineers ST.2086 (2014), Mastering Display Color Volume Metadata for High Luminance and Wide Color Gamut Images.

[6] D. Flynn, J. Sole, G.J. Sullivan, T. Suzuki, “HEVC Range Extension Draft 5”, JCTVC-O1005, 17 December 2013.

[7] B. Mandel, C. Fogg, J. Helman, “High dynamic range video coding results,” JCTVC-O0101, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013.

[8] P. Yin, T. Lu, “Cross-check Report for JCTVC-O0101 on High Dynamic Range Video Coding Results,” JCTVC-O0366.

[9] B. Mandel, C. Fogg, J. Helman, “HDR simulation results for FruitStall and Oblivion test sequences”, JCTVC-Pxxxx., 16th Meeting: San José, US, 9–17 Jan. 2014.

[10] N. Naccari, C. Rosewarne, G. Sullivan, “HEVC Range extensions test model 1 encoder description”, JCTVC-O1013, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013.

[11] W. Pu, W-S. Kim, J. Chen, J. Sole, M. Karczewicz, “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4”,  JCTVC-O0202, 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013

# Patent rights declaration(s)

Movielabs, and Harmonic do not have any current or pending patent rights relating to the technology described in this contribution.