| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC-AB0026 |
| --- | --- |

| Title: | Omnidirectional media format SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Qualcomm Incorporated 5775 Morehouse Drive San Diego, CA 92130, USA | Tel: Email: | +1 858 651 8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes the following items related to omnidirectional video:

- A new SEI message for signalling of region-wise packing (RWP), and

- A new SEI message for signalling of sphere rotation.

It is asserted that the two new SEI messages are necessary for the first version of the Omnidirectional MediA Format (OMAF) to work properly.

It is suggested that the need of separate rotation signalling in addition to the coverage signalling is discussed at a joint meeting between JCT-VC and MPEG Systems (including at least experts of the OMAF AHG), and

- If it is concluded that separate rotation is not needed because of the existence of the coverage signalling, then

  - Change the omnidirectional projection indication SEI message syntax such that the syntax elements omni_projection_yaw_center, omni_projection_pitch_center, and omni_projection_roll_center are present regardless of the values of sub_geometry_flag.

  - Apply the same conclusion to OMAF, including removal of the projection orientation box, removal of the distinction between global and local axels, and alignment of other places (e.g., the coverage boxes) as needed.

- Otherwise, add an SEI message for rotation signalling.

# Background

## OMAF processing steps for generation of pictures before encoding in content production

Clause 7.2.2.1 of the latest OMAF draft specification text in MPEG document M40849 is as follows:

7.2.2.1 Relation of decoded pictures to global coordinate axes (informative)

Figure 7-1 illustrates the conversions from a spherical picture to a packed picture that can be used in content authoring and the corresponding conversions from a packed picture to a spherical picture to be rendered that can be used in a player.

Figure 7-1 – Example of processing stages to derive a packed picture from a spherical image or vice versa

The projection structure is along the global coordinate axes as illustrated in Figure 7-1a, when

- the equator of the equirectangular panorama picture is aligned with the X axis of the global coordinate axes,

- the Y axis of the equirectangular panorama picture is aligned with the Y axis of the global coordinate axes, and

- the Z axis of the global coordinate axes passes through the middle point of the equirectangular panorama picture.

The content authoring can include the following steps:

- The input images are stitched to generate a sphere picture on the unit sphere per the global coordinate axes as indicated in Figure 7-1a.

- The unit sphere is then rotated relative to the global coordinate axes, as indicated in Figure 7-1b. The amount of rotation is specified by the orientation angles indicated in the ProjectionOrientationBox. The local coordinate axes of the projection structure are the axes of the coordinate system that has been rotated as specified by ProjectionOrientationBox, when present. The absence of ProjectionOrientationBox indicates that the local coordinate axes are the same as the global coordinate axes.

- As illustrated in Figure 7-1c, the spherical picture on the rotated unit sphere is then converted to a two-dimensional projected picture, for example using the equirectangular projection specified in clause 5.2.1.

- Rectangular RWP can be applied to obtain a packed picture from the projected picture. One example of packing is depicted in Figure 7-1c and Figure 7-1d. The dashed rectangles in Figure 7-1c indicate the projected regions on a projected picture, and the respective areas in Figure 7-1d indicate the corresponding packed regions. In this example, projected regions 1 and 3 are horizontally downsampled, while projected region 2 is kept at its original resolution.

GlobalCoverageInformationBox can be used to indicate which part of the sphere is covered by the packed picture.

In order to map a sample location of a packed picture (such as that in Figure 7-1d) to a projection structure used in rendering (Figure 7-1a), the player perform sequential mappings in reverse order from Figure 7-1d to Figure 7-1a.

As can be seen, OMAF content production consists of the ordered steps of stitching, rotation, projection and RWP for generation of video pictures that are sent to the encoder for encoding.

RWP enables manipulations (resize, reposition, rotation, and mirroring) of any rectangular region of the projected picture. RWP can be used to generate an emphasis on a specific viewport orientation or circumvent weaknesses of projections such as oversampling towards the poles in ERP. The latter is depicted in the top of Figure 1 where the areas near the poles of the sphere video are reduced in resolution. The bottom of Figure 1 depicts an example for an emphasized viewport orientation.

Figure 1: Two RWP examples based on the ERP format.
(credit: Robert Skupin of HHI)

Rotation may be applied to improve coding efficiency or to move an object or a region to a desired position on the sphere, e.g., as discussed in JCTVC-Z0025.

## OMAF requirement of carrying rendering information in video bitstreams

In the OMAF HEVC media profiles under consideration in MPEG document N16826, the use of HEVC omnidirectional projection indication (OPI) SEI message and HEVC frame packing arrangement SEI message are mandated. The reason is to enable OMAF systems that require to access information that is needed for rendering, e.g., projection and frame packing arrangement, directly from the video elementary bitstream rather than from file format or DASH MPD, for security and content protection considerations.

However, when RWP and rotation are applied for generation of video pictures before encoding, information on RWP and rotation would also be needed for such OMAF systems to access directly from the video elementary bitstream. The most straightforward way of carrying such information in a video elementary bitstream is to use SEI messages, similarly as projection and frame packing arrangement.

## Discussion on rotation

On rotation, one may argue that this is already covered by the syntax elements omni_projection_yaw_center, omni_projection_pitch_center, and omni_projection_roll_center of the OPI SEI message. However, these syntax elements plus omni_projection_yaw_range and omni_projection_pitch_range of the OPI SEI message indicate the coverage (i.e., the position and size of the region covered by the bitstream) on the sphere, they do not cover the rotation. For example, if sub_geometry_flag of the OPI SEI message is equal to 0, none of the coverage syntax elements are present, but rotation may still be performed. Another issue is that, while the coverage may be fixed while the rotation can be different in different CVSs (a counter argument for this point is that it is possible to make coverage different in different CVS too).

In the context of OMAF, one may also argue that the rotation can anyway be merged into the coverage, and everything is based on one set of coordinate axes (i.e., the local coordinate axes are considered as the global coordinate axes while the original global coordinate axes can be forgotten about; the initial viewpoint metadata can be used for correct rendering anyway). However, this way, when certain rotation is applied for video, then the same amount of rotation needs to be also applied to audio and other media types, if present. Discuss: does this actually matter, since Clause 5.1 of OMAF mandates that the coordination systems for different media types were made aligned during content production?

# Proposal

## Joint meeting discussion on rotation signalling

It is suggested that the need of a separate rotation signalling is needed in addition to the coverage signalling, preferably at a joint meeting between JCT-VC and MPEG Systems (including at least experts of the OMAF AHG), and based on the conclusion, discuss and decide the following suggestion:

- If it is concluded that separate rotation is not needed now that we have the coverage signalling, then

  - In the OPI SEI message, change the syntax such that the syntax elements omni_projection_yaw_center, omni_projection_pitch_center, and omni_projection_roll_center are present regardless of the values of sub_geometry_flag. Note that the omni_projection_yaw_range and omni_projection_pitch_range would still be absent when sub_geometry_flag is equal to 0.

  - Apply the same conclusion to OMAF, including removal of the projection orientation box, removal of the distinction between global and local axels, and alignment of other places (e.g., the coverage boxes) as needed.

- Otherwise, add an SEI message for rotation signalling.

## Omnidirectional region-wise packing SEI message

### Syntax

| omni_region_wise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| omni_region_wise_packing_cancel_flag | u(1) |
| if( !omni_region_wise_packing_cancel_flag ) { |  |
| omni_region_wise_packing_persistence_flag | u(1) |
| rwp_reserved_zero_6bits | u(6) |
| num_packed_regions | u(8) |
| proj_picture_width | u(16) |
| proj_picture_height | u(16) |
| for( i = 0; i < num_packed_regions; i++ ) { |  |
| rwp_reserved_zero_4bits[ i ] | u(4) |
| packing_type[ i ] | u(4) |
| if( packing_type[ i ] = = 0 ) { |  |
| proj_reg_width[ i ] | u(16) |
| proj_reg_height[ i ] | u(16) |
| proj_reg_top[ i ] | u(16) |
| proj_reg_left[ i ] | u(16) |
| transform_type[ i ] | u(3) |
| rwp_reserved_zero_5bits[ i ] | u(5) |
| packed_reg_width[ i ] | u(16) |
| packed_reg_height[ i ] | u(16) |
| packed_reg_top[ i ] | u(16) |
| packed_reg_left[ i ] | u(16) |
| } |  |
| } |  |
| } |  |
| } |  |

### Semantics

The omnidirectional region-wise packing SEI message provides information to enable remapping of the colour samples of the output decoded pictures onto projected pictures. [Ed. (YK): Add definitions of "projected picture" and "packed picture" similarly as defined in the OMAF draft spec, and change "to enable remapping of the colour samples of the output decoded pictures " in the first sentence of the semantics of the OPI SEI message to "to enable remapping of the colour samples of the projected pictures".]

omni_region_wise_packing_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional region-wise packing SEI message in output order. omni_region_wise_packing_cancel_flag equal to 0 indicates that omnidirectional region-wise packing information follows.

omni_region_wise_packing_persistence_flag specifies the persistence of the omnidirectional region-wise packing SEI message for the current layer.

omni_region_wise_packing_persistence_flag equal to 0 specifies that the omnidirectional region-wise packing SEI message applies to the current decoded picture only.

Let picA be the current picture. omni_region_wise_packing_persistence_flag equal to 1 specifies that the omnidirectional region-wise packing SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an omnidirectional region-wise packing SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an omnidirectional projection indication SEI message with omni_projection_information_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the omnidirectional region-wise packing SEI message in decoding order, an omnidirectional region-wise packing SEI message with omni_region_wise_packing_persistence_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore omnidirectional region-wise packing SEI messages with omni_region_wise_packing_persistence_flag equal to 0 that do not follow, in decoding order, an omnidirectional projection indication SEI message with omni_projection_information_cancel_flag equal to 0 in the CLVS that applies to the current picture.

rwp_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_6bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_6bits[ i ].

num_packed_regions specifies the number of packed regions. The value of num_packed_regions shall be greater than 0.

proj_picture_width and proj_picture_height specify the width and height, respectively, of the projected picture. The value of proj_picture_width and proj_picture_height shall be both greater than 0.

rwp_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_4bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_4bits[ i ].

packing_type[ i ] specifies the type of region-wise packing. packing_type[ i ] equal to 0 indicates rectangular region-wise packing. Other values are reserved. The value of packing_type[ i ] shall be equal to 0 in this version of this Specification. Decoders shall allow values of packing_type[ i ] greater than 0 and shall ignore all omnidirectional region-wise packing SEI messages with packing_type[ i ] greater than 0 for any value of i.

proj_reg_width[ i ], proj_reg_height[ i ], proj_reg_top[ i ], and proj_reg_left[ i ] are indicated in units of luma samples in a projected picture with width and height equal to proj_picture_width and proj_picture_height, respectively.

proj_reg_width[ i ] specifies the width of the i-th projected region. proj_reg_width[ i ] shall be greater than 0.

proj_reg_height[ i ] specifies the height of the i-th projected region. proj_reg_height[ i ] shall be greater than 0.

proj_reg_top[ i ] and proj_reg_left[ i ] specify the top luma sample row and the left-most luma sample column, respectively, in the projected picture. The values of proj_reg_top[ i ] and proj_reg_left[ i ], shall be in the range from 0, inclusive, indicating the top-left corner of the projected picture, to proj_picture_height − 1, inclusive, and proj_picture_width − 1, inclusive, respectively.

The sum proj_reg_width[ i ] and proj_reg_left[ i ] shall be less than proj_picture_width. The sum of proj_reg_height[ i ] and proj_reg_top[ i ] shall be less than proj_picture_height.

When the projected picture is stereoscopic, proj_reg_width[ i ], proj_reg_height[ i ], proj_reg_top[ i ] and proj_reg_left[ i ] shall be such that the projected region identified by these fields is within a single constituent picture of the projected picture.

transform_type[ i ] specifies the rotation and mirroring that have been applied to the i-th projected region to map it to the packed picture before encoding. When transform_type[ i ] specifies both rotation and mirroring, rotation has been applied after mirroring in the region-wise packing from the projected picture to the packed picture before encoding. The values of transform_type[ i ] are specified in Table D. X:

Table D.X – transform_type[ i ] values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (counter-clockwise) |
| 3 | rotation by 180 degrees (counter-clockwise) after mirroring horizontally |
| 4 | rotation by 90 degrees (counter-clockwise) after mirroring horizontally |
| 5 | rotation by 90 degrees (counter-clockwise) |
| 6 | rotation by 270 degrees (counter-clockwise) after mirroring horizontally |
| 7 | rotation by 270 degrees (counter-clockwise) |

rwp_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_5bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_5bits[ i ].

packed_reg_width[ i ], packed_reg_height[ i ], packed_reg_top[ i ], and packed_reg_left[ i ] specify the width, height, the top luma sample row, and the left-most luma sample column, respectively, of the packed region in the packed picture.

Let packedPicWidth and packedPicHeight be the width and height of the packed picture, which has the same size as the conformance cropping window. The values of packed_reg_width[ i ], packed_reg_height[ i ], packed_reg_top[ i ], and packed_reg_left[ i ] are constrained as follows:

- packed_reg_width[ i ] and packed_reg_height[ i ] shall both be greater than 0.

- The values of packed_reg_top[ i ] and packed_reg_left[ i ] shall in the range from 0, inclusive, indicating the top-left corner luma sample of the packed picture, to packedPicHeight − 1, inclusive, and packedPicWidth − 1, inclusive, respectively.

- The sum of packed_reg_width[ i ] and packed_reg_left[ i ] shall be less than packedPicWidth.

- The sum of packed_reg_height[ i ] and packed_reg_top[ i ] shall be less than packedPicHeight.

- The rectangle specified by packed_reg_width[ i ], packed_reg_height[ i ], packed_reg_top[ i ], and packed_reg_left[ i ] shall be non-overlapping with the rectangle specified by packed_reg_width[ j ], packed_reg_height[ j ], packed_reg_top[ j ], and packed_reg_left[ j ] for any value of j in the range of 0 to i – 1, inclusive.

## Omnidirectional sphere rotation SEI message

### Syntax

| omni_sphere_rotation( payloadSize ) { | Descriptor |
| --- | --- |
| omni_sphere_rotation_cancel_flag | u(1) |
| if( !omni_sphere_rotation_cancel_flag ) { |  |
| omni_sphere_rotation_persistence_flag | u(1) |
| opr_reserved_zero_6bits | u(6) |
| omni_rotation_yaw | i(32) |
| omni_rotation_pitch | i(32) |
| omni_rotation_roll | i(32) |
| } |  |
| } |  |

### Semantics

The omnidirectional sphere rotation SEI message provides information to enable conversion from the local coordinate axes to the global coordinate axes. [Ed. (YK): Add definitions of "local sphere axes " and "global coordinate axes" similarly as defined in the OMAF draft spec.]

omni_sphere_rotation_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous omnidirectional sphere rotation SEI message in output order. omni_sphere_rotation_cancel_flag equal to 0 indicates that omnidirectional sphere rotation information follows.

omni_region_wise_packing_persistence_flag specifies the persistence of the omnidirectional sphere rotation SEI message for the current layer.

omni_sphere_rotation_persistence_flag equal to 0 specifies that the omnidirectional sphere rotation SEI message applies to the current decoded picture only.

Let picA be the current picture. omni_sphere_rotation_persistence_flag equal to 1 specifies that the omnidirectional sphere rotation SEI message persists for the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing an omnidirectional sphere rotation SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

When an omnidirectional projection indication SEI message with omni_projection_information_cancel_flag equal to 0 is not present in the CLVS that applies to the current picture and precedes the omnidirectional sphere rotation SEI message in decoding order, an omnidirectional sphere rotation SEI message with omni_sphere_rotation_persistence_flag equal to 0 shall not be present in the CLVS that applies to the current picture. Decoders shall ignore omnidirectional sphere rotation SEI messages with omni_sphere_rotation_persistence_flag equal to 0 that do not follow, in decoding order, an omnidirectional projection indication SEI message with omni_projection_information_cancel_flag equal to 0 in the CLVS that applies to the current picture.

opr_reserved_zero_6bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for rwp_reserved_zero_6bits[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore the value of rwp_reserved_zero_6bits[ i ].

omni_rotation_yaw, omni_rotation_pitch, and omni_rotation_roll specify the yaw, pitch, and roll angles, respectively, of the rotation that has been applied to the unit sphere at the content production side to convert the global coordinate axes to the local coordinate axes.

The value of omni_rotation_yaw shall be in the range of −180 * 216 to 180 *216 − 1, inclusive.

The value of omni_rotation_pitch shall be in the range of −90 * 216 to 90 * 216, inclusive.

The value of omni_rotation_roll shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive.

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the authors).