| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 28th Meeting: Torino, IT, 15–21 July 2017 | Document: JCTVC-AB0030 |
| --- | --- |

| Title: | Scaling support in HDRTools and observations |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Alexis M. Tourapis David Singer Yeping Su Apple Inc. 1 Infinite Loop Cupertino, CA 95014, USA | Tel: Email: | +1 408 228 7983 atourapis@apple.com singer@apple.com yeping_su@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution discusses additional enhancements introduced to the HDRTools package, mostly in relationship  to video resolution scaling. More specifically, HDRTools now support upscaling and downscaling of 2D video content at any ratio using filters such as bilinear, bicubic, nearest neighbor, windowed lanczos, hamming and hanning window, Sine Window, and Gaussian. Some observations with regards to the use of these filters, especially on HDR material, are also discussed.

# Introduction

The HDRTools software package [1] was originally developed to assist the High Dynamic Range (HDR) and Wide Colour Gamut (WCG) standardization activities undertaken by MPEG. However, development on the software continued beyond that point, with the inclusion of a variety of video processing and format conversion techniques that are not restricted to HDR/WCG material. The software supports techniques such as format conversion to and from a variety of production and delivery video formats, chroma downsampling and upsampling, filtering/denoising, supports several different transfer functions for SDR and HDR content as well as color spaces, and a variety of video quality metrics.

Amongst one of the more recent and requested extensions to the software was support of generalized scaling, with limited support of Lanczos scaling as well as a few other scaling methods [2]. Scaling was a topic of considerable interest especially for HDR/WCG material, since such material may have to be upscaled to 4K or downscaled to 1080p resolutions for a variety of reasons, such as display or delivery.

# New Scaling Methods in HDRTools

In the latest development version of HDRTools [3] several new scaling methods were introduced. These methods were implemented based on the rather thorough work presented during the development of the MPEG-4 AVC/H.264 and SVC video coding standards in the JVT group [4][5][6][7][8][9]. We would direct the readers to the excellent work done by G.J. Sullivan in [4][5] and A. Segall in [6][7], without  excluding several other important contributions by other authors. JVT-R006 [9] especially includes a very useful excel workbook that can be used by anyone to calculate filters targeting different scaling ratios. We have used this excel sheet as our reference to implement some of these methods also in HDRTools. More specifically, we have added support for the following scaling methods, which can be selected using the parameter ScalingMode (specified below in parenthesis) in the software:

- Nearest-Neighbor (0)

- Bilinear (2)

- Lanczos-Window (3)

- Bicubic (4)

- Hanning-Window (5)

- Hamming-Window (6)

- Sine-Window (7)

- Gaussian (8)

The previously specified linear filters that were used for chroma format conversion can also be used when the scaling ratio is equal to 2 or 0.5 as well by setting ScalingMode=1. HDRTools currently supports scaling from any resolution to any other resolution, higher or lower. No limitations in aspect ration are also placed, i.e. one may select to scale up the horizontal resolution, and scale down the vertical resolution if that was so desired. The computation of the filter coefficients for most of the above filters is done following the methodology also used in JVT-R006. The number of sidelobes can be specified in either the HDRTools configuration filter or at the command line. Currently the same number is used for both horizontal and vertical filtering, but that can be easily modified. Phase offsets are adaptively computed based on the scaling ratio (initial, explicit offsets cannot currently be specified). Scaling is automatically performed by specifying different output resolution parameters (width and height) than the ones for the input.

As implemented, scaling is currently performed only on the original content color representation. That is, if the signal is in a linear RGB floating point representation and downscaling or upscaling is specified then the scaling process will be applied in that domain regardless of the target representation. This means that if it is desirable to perform scaling in a different from the content’s color representation, the content should first be converted into that representation instead. Then, using this new file, perform the desired scaling followed by any final color representation conversion. For example, if we wanted to convert an OpenEXR video sequence that was in a linear floating point, BT.2020 RGB representation to a BT.2100 PQ YCbCr 4:2:0 representation, while also applying resolution scaling in BT.2100 PQ RGB, we would first need to convert the original source sequence to the BT.2100 PQ RGB representation without any scaling. Then, HDRTools will be run a second time using this new sequence as its input source, while also specifying the target resolution and BT.2100 PQ YCbCr 4:2:0 output target representation. Extending the software to permit scaling at different stages is possible, but would require considerable more work if it was so desired (e.g. for permitting a single run of the software and reducing both storage and runtime/complexity requirements). Regardless, we consider that even in this form, the software can already provide considerable flexibility to the user on how and where to perform scaling.

# Observations on Scaling for HDR material

Even though it was well accepted that scaling was an important aspect of video systems during the creation of the Technical Report on Conversion and Coding Practices for HDR/WCG Y’CbCr 4:2:0 Video with PQ Transfer Characteristics [10], this process was considered out of scope and thus omitted from that document. During the meeting though it was suggested that given observations on chroma scaling, special care should be taken when also scaling the entire image. Further investigation on scaling of HDR/WCG material was encouraged, but until now not thoroughly, if at all, tested. In this section, some preliminary results on scaling of HDR/WCG material are presented.

In particular, we have examined how some of the methods described in the previous section would perform when applied in different domains, i.e. in linear BT.2100 RGB, non-linear (PQ) BT.2100 RGB, PQ BT.2100 YCbCr 4:4:4, and PQ BT.2100 Y’CbCr 4:2:0. For our experiments, we have only looked at the Lanczos-Window and the Gaussian filter methods since other categories implemented tend to have some similar traits (at least in relationship to our observations).

More specifically, what we have found is that Brick-wall filters, such as the Lanczos-Window one, should be discouraged from use directly on linear HDR/WCG signals. This is because for such signals the contrast ratio especially near edges of objects can be considerably higher accentuating ringing artifacts and clipping. An illustration of this problem is shown in Figure 1, where a step function signal with a minimum brightness of 10cd/m2 and a maximum brightness of 400cd/m2 is filtered both in the linear and non-linear domain using a Lanczos-Window filter. It can be seen that applying such a filter can create considerable ripple artifacts in both cases. However, the artifacts are much more severe, in terms of subjective impact in the linear case since the ripples are much more significant relatively in the lower brightness values, thus having a more perceptual impact. In fact, in the example shown, the ripple ends up also exceeding the value 0, and introducing clipping artifacts. It should be pointed out that a value of 10cd/m2 is a relatively modest brightest value, and in this particular case the transition from this value to a value of 400cd/m2 has introduced very dark, and in fact completely black pixels. On the other hand, applying the same filter and scaling in the non-linear domain results in a significant boost in the bright area, with filtered values reaching as high as 550cd/ m2 . The effect is even more dramatic looking at a step function that has a maximum brightness of 4000cd/ m2 (Figure 2). In this scenario, after filtering, the brightest value can be boosted to a value close to 6800cd/ m2, essentially a 70% increase in brightness. This behavior is obviously an outcome of the characteristics of the PQ transfer function. We should point out, however, that at least in our experiments, and on the displays that we have tested (i.e. Sim2), the increase in brightness was not perceived as a problem. However, it is not yet clear what impact this may have on other displays, and especially if dynamic metadata for display mapping is considered.

As expected, these problems do not exist when using a Gaussian filter (Figure 3 and Figure 4). We can observe that when such a filter is applied in the linear domain, there is again though a more significant impact on the darker values (i.e. there is a more moderate proportional increase in brightness at those values) while brighter values are reduced proportionally less. That is not the case when processing in the non-linear domain, where the impact on the darker values is much smaller. Nevertheless, such filters also tend to considerably blur the content after filtering, which in some cases may not be desirable.

Figure 5 through Figure 22 show several example cases of using the Lanczos-Window filter on real images. Figure 5, Figure 13, Figure 21, and Figure 22 show the effect of this filter applied in the linear domain for BT.2020 and BT.709 content. In both cases the ringing artifact are quite significant. Zoomed in versions are provided in Figure 6 and Figure 14. On the other hand, such artifacts do not appear when applying the same filters in the non-linear domain (Figure 7, Figure 8, Figure 9, Figure 15, Figure 16, and Figure 17). Interestingly enough, the differences between RGB and YCbCr processing, as well as processing in 4:2:0 seemed very subtle (we actually anticipated some chroma leakage to occur in the 4:2:0 case). However, maybe the fact that we converted the content first using the luma adjustment method to 4:2:0 reduced potentially the risk of chroma leakage. Then again, maybe other content could exhibit such an issue. Figure 23 and Figure 24 show the same images downscaled in the linear domain using a Gaussian filter. The impact on sharpness seems quite obvious in Figure 25 and Figure 26.

Figure 1. Illustration of ringing artifacts on a step function with a minimum brightness of 10cd/m^2 and a maximum brightness of 400cd/m^2 using a Lanczos-Window filter

Figure 2. Illustration of ringing artifacts on a step function with a minimum brightness of 10cd/m^2 and a maximum brightness of 4000cd/m^2 using a Lanczos-Window filter

Figure 3. Filtering a step function with a minimum brightness of 10cd/m^2 and a maximum brightness of 400cd/m^2 using a Gaussian filter

Figure 4. Filtering a step function with a minimum brightness of 10cd/m^2 and a maximum brightness of 4000cd/m^2 using a Gaussian filter

Figure 5. Frame 0 of FireEater in BT.2020 color primaries scaled in the linear domain

Figure 6. Zoomed area in Figure 1

Figure 7. Frame 0 of FireEater in BT.2020 color primaries scaled in non linear RGB BT.2100

Figure 8. Frame 0 of FireEater in BT.2020 color primaries scaled in BT.2100 YCbCr 444

Figure 9. Frame 0 of FireEater in BT.2020 color primaries scaled in BT.2100 YCbCr 420

Figure 10. Zoomed area in Figure 7

Figure 11. Zoomed area in Figure 8

Figure 12. Zoomed area in Figure 9

Figure 13. Frame 0 of Market in BT.2020 color primaries scaled in the linear domain

Figure 14. Zoomed area in Figure 13

Figure 15. Frame 0 of Market in BT.2020 color primaries scaled in non linear RGB BT.2100

Figure 16. Frame 0 of FireEater in BT.2020 color primaries scaled in YCbCr BT.2100 4:4:4

Figure 17. Frame 0 of FireEater in BT.2020 color primaries scaled in YCbCr BT.2100 4:2:0

Figure 18. Zoomed area in Figure 15

Figure 19. Zoomed area in Figure 16

Figure 20. Zoomed area in Figure 17

Figure 21. Frame 0 of FireEater in BT.709 color primaries scaled in the linear domain

Figure 22. Frame 0 of Market in BT.709 color primaries scaled in the linear domain

Figure 23. Frame 0 of FireEater in BT.2020 color primaries scaled using a 
Gaussian filter in the Linear domain

Figure 24. Frame 0 of FireEater in BT.2020 color primaries scaled using a 
Gaussian filter in the Linear domain

Figure 25. Zoomed area in Figure 23

Figure 26. Zoomed area in Figure 24

# Conclusion

In this contribution we have presented some preliminary results about scaling of HDR/WCG content using the HDRTools package. We have observed that performing scaling in the linear domain using brick-wall type filters, such as the Lanczos filter, could potentially introduce severe ringing artifacts and should likely be avoided. Such filters seem to operate ok in our tests when applied in the non-linear domain. Conventional filters such as the Gaussian filter could be used in both domains, but tend to result in blurred images. It is expected that more advanced filters, e.g. hybrid/edge adaptive filters could achieve a considerably better performance, i.e. avoid the ringing artifacts introduced by brick-wall filters while retaining sharpness. We would encourage experts to contribute such filters to the MPEG and the Video Coding community in general.

# Referenced Publications

- HDRTools Master branch: https://gitlab.com/standards/HDRTools/

- A.M. Tourapis, T. Baar, D. Singer, and Y. Su, “HDRTools: Generalized Scaling and Tone Mapping Support”, JCTVC contribution, JCTVC-Z0042, January 2017, Geneva, Switzerland

- HDRTools development branch 0.17-dev: https://gitlab.com/standards/HDRTools/tree/0.17-dev

- Garry Sullivan, “Downsampling for Color Format Conversion”, JVT-I018, San Diego, CA, September, 2003

- Garry Sullivan, “Color format upconversion for video display”, JVT-I019, San Diego, CA, September, 2003

- A. Segall, “Study of Upsampling/Down-sampling for Spatial Scalability”, JVT-Q083, Nice, FR, 14-21 October 2005

- A. Segall, “Upsampling and Down-sampling for Spatial Scalability”, JVT-R070, Bankok, TH, 14-20 January, 2006

- Garry Sullivan and Shijun Sun, “Ad Hoc Report on Spatial Scalability Filters”, JVT-Q007, Nice, FR, 14-21 October 2005

- Garry Sullivan and Shijun Sun, “Ad Hoc Report on Spatial Scalability Filters”, JVT-R006, Bankok, TH, 14-20 January, 2006

- J. Samuelsson, C. Fogg, A. Norkin, A. Segall, J. Ström, G. J. Sullivan, P. Topiwala, and A. Tourapis , “Conversion and Coding Practices for HDR/WCG Y′CbCr 4:2:0 Video with PQ Transfer Characteristics (Draft 4)”, JCTVC contribution, JCTVC-Z1020, January 2017, Geneva, Switzerland

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The filter was computed based on the assumption that the content would be downscaled by 2x. The number of sidelobes was set to 6.