INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC1/SC29/WG11

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC1/SC29/WG11

MPEG2012/M23897

February 2012, San Jose, USA

| Source | Samsung Electronics Co. Ltd |
| --- | --- |
| Status | Contribution |
| Title | Samsung AL-FEC Performance on D1-FEC EE |
| Authors | Sunghee Hwang, Seho Myung, Hyunkoo Yang, Sungoh Hwang, Kyungmo Park, |

In m21539, we proposed two stages FEC coding scheme based on RS and Quasi-cyclic LDPC (QC-LDPC) codes.

In this contribution,

- We review the channel model and simulation parameters agreed for wired network.

- We compare performance of the proposed codes with ideal FEC codes.

- We provide simulation results for all agreed channel parameters.

Channel Model and Simulation Parameters

We evaluated performance of forward error correction (FEC) codes for MMT. The detailed specifications of the FEC codes are given in m21539. The channel model and simulation parameters as agreed in m21506 are summarized in this section.

Random + REIN Channel Model for Wired Network

- Burst Length: 8ms fixed time

- Random Packet Loss Rate (R_PLR): 0 ~ 16.7%

- Burst Packet Loss Rate (B_PLR): 10–2, 10–3 and 10–4

FEC Parameters

- Source Block Length (K): 50, 100, 200, 400, 800, 1600, 3200, 6400 and 32000

Note: K = 32000 is need for the case of large file delivery.

- Code Rate (K/(K+P)): 20/21, 10/11, 20/23 and 5/6

Recommended System Parameters

- Source Block Length(K) is the number of packets.

- Packet size: 1000 bytes

- Data Rate: 8Mbps

- Packet loss happens packet by packet

Simulation Results

In this section, we compare the performance of FEC codes proposed in m21539 and an ideal code based on “Noise Margin” at a quasi error-free (QEF) point.

- QEF: Packet Loss Rate (PLR) 10–6 after FEC decoding

- Noise Margin: Maximally required R_PLR to achieve QEF point

We summarize “Noise Margin” in below Tables in Section 2.1.

- The numbers in parentheses are estimated values.

- RS codes are used only when K is smaller than or equal to 400.

- For LDPC codes, the minimum value of K is 400.

- The PLR after FEC decoding when R_PLR = 0 are given in gray shaded regions.

NOTE: For some cases, especially when K is relatively small and B_PLR is relatively large, the “Noise Margin” could not be obtained because there were residual errors caused by the burst noise even when R_PLR is zero.

- Table 1. “Noise Margin” when B_PLR = 0

| K |  | 50 | 100 | 200 | 400 | 800 | 1600 | 3200 | 6400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR= 20/21 | Ideal | N/A | 0.5% | 1.1% | 1.7% | 2.4% | 2.9% | 3.4% | 3.7% |
|  | RS |  | 0.5% | 1.1% | 1.1% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 0.9% | (2.1%) | (2.9%) | (3.4%) | (3.7%) |
| CR= 10/11 | Ideal | 0.9% | 1.9% | 3.2% | 4.4% | 5.5% | 6.5% | 7.2% | 7.7% |
|  | RS | 0.9% | 1.9% | 3.2% | 3.2% | N/A |  |  |  |
|  | LDPC | n/a |  |  | 4.0% | (5.4%) | (6.4%) | 7.2% | (7.7%) |
| CR= 20/23 | Ideal | N/A | 3.8% | 5.6% | 7.4% | 8.8% | 9.9% | 10.7% | 11.3% |
|  | RS |  | 3.8% | 5.6% | 5.6% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 6.7% | 8.6% | (9.8%) | 10.6% | (11.4%) |
| CR= 5/6 | Ideal | 3.4% | 5.8% | 8.1% | 10.2% | 11.9% | 13.1% | 14.2% | 14.8% |
|  | RS | 3.4% | 5.8% | 8.1% | 8.1% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 9.6% | 11.8% | 13.0% | 14.1% | 14.8% |

- Table 2. “Noise Margin” when B_PLR = 10–4

| K |  | 50 | 100 | 200 | 400 | 800 | 1600 | 3200 | 6400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR= 20/21 | Ideal | N/A | 2.4e-4 | 1.7e-6 | 0.8% | 2.0% | 2.7% | 3.3% | 3.7% |
|  | RS |  | 2.4e-4 | 1.7e-6 | 0.6% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 0.4% | 1.8% | 2.7% | (3.3%) | (3.7%) |
| CR= 10/11 | Ideal | 2.4e-4 | 0.1% | 1.9% | 4.0% | 5.3% | 6.3% | 7.1% | 7.7% |
|  | RS | 2.4e-4 | 0.1% | 1.9% | 3.0% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 3.6% | 5.2% | 6.2% | 7.1% | 7.6% |
| CR= 20/23 | Ideal | N/A | 1.5% | 4.9% | 7.1% | 8.7% | 9.8% | 10.7% | 11.4% |
|  | RS |  | 1.5% | 4.9% | 5.5% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 6.5% | 8.5% | (9.8%) | 10.7% | 11.4% |
| CR= 5/6 | Ideal | 0.4% | 4.2% | 7.7% | 10.0% | 11.8% | 13.1% | 14.1% | 14.8% |
|  | RS | 0.4% | 4.2% | 7.7% | 8.0% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 9.5% | 11.6% | 13.0% | 14.0% | 14.8% |

- Table 3. “Noise Margin” when B_PLR = 10–3

| K |  | 50 | 100 | 200 | 400 | 800 | 1600 | 3200 | 6400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR= 20/21 | Ideal | N/A | 1.2e-3 | 3.7e-5 | 2.2e-6 | 1.2% | 2.3% | 3.0% | 3.5% |
|  | RS |  | 1.2e-3 | 3.7e-5 | 2.2e-6 | N/A |  |  |  |
|  | LDPC | N/A |  |  | 4.4e-6 | 1.1% | 2.2% | 3.0% | 3.4% |
| CR= 10/11 | Ideal | 1.2e-3 | 1.8e-5 | 0.6% | 3.0% | 4.8% | 6.0% | 6.9% | 7.5% |
|  | RS | 1.2e-3 | 1.8e-5 | 0.6% | 2.4% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 2.7% | 4.8% | 5.9% | 6.8% | 7.4% |
| CR= 20/23 | Ideal | N/A | 1.7e-5 | 3.3% | 6.3% | 8.3% | 9.5% | 10.5% | 11.3% |
|  | RS |  | 1.7e-5 | 3.3% | 5.1% | N/A |  |  |  |
|  | LDPC | N/A |  |  | (5.9%) | 8.1% | 9.4% | 10.5% | (11.2%) |
| CR= 5/6 | Ideal | 8.6e-6 | 1.8% | 6.4% | 9.4% | 11.5% | 12.8% | 13.9% | 14.7% |
|  | RS | 8.6e-6 | 1.8% | 6.4% | 7.8% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 9.0% | 11.1% | 12.8% | (13.9%) | 14.6% |

- Table 4. “Noise Margin” when B_PLR = 10–2

| K |  | 50 | 100 | 200 | 400 | 800 | 1600 | 3200 | 6400 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR= 20/21 | Ideal | N/A | 1.2e-2 | 2.9e-3 | 1.4e-3 | 8.1e-5 | 1.9e-6 | 0.9% | 1.7% |
|  | RS |  | 1.2e-2 | 2.9e-3 | 1.4e-3 | N/A |  |  |  |
|  | LDPC | N/A |  |  | 1.5e-3 | 2.1e-4 | 5.4e-6 | 0.9% | 1.7% |
| CR= 10/11 | Ideal | 1.2e-2 | 1.6e-3 | 4.0e-4 | 3.9e-6 | 1.6% | 3.6% | 5.0% | 5.9% |
|  | RS | 1.2e-2 | 1.6e-3 | 4.0e-4 | 3.8e-6 | N/A |  |  |  |
|  | LDPC | N/A |  |  | 1.6e-5 | 1.6% | 3.4% | 5.0% | 5.9% |
| CR= 20/23 | Ideal | N/A | 1.5e-3 | 3.7e-5 | 2.1% | 5.3% | 7.4% | 8.7% | 9.7% |
|  | RS |  | 1.5e-3 | 3.7e-5 | 1.9% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 1.8% | 5.3% | 7.3% | 8.7% | 9.7% |
| CR= 5/6 | Ideal | 7.8e-4 | 1.1e-4 | 0.2% | 5.7% | 8.8% | 10.9% | 12.3% | 13.4% |
|  | RS | 7.8e-4 | 1.1e-4 | 0.2% | 4.9% | N/A |  |  |  |
|  | LDPC | N/A |  |  | 5.3% | 8.7% | (10.7%) | (12.2%) | (13.2%) |

Conclusions

- Proposed FEC scheme provides good performance approaching that of ideal codes.

- The performance gap between QC-LDPC and Ideal codes are very small.

- When burst noise is expected, it is recommended using FEC codes with sufficiently large K or low code rate.

NOTE: If the target PLR of the application is low, we can use FEC codes with

relatively short block length without worrying about burst noise.

Annex

Code rate = 20/21

RS codes (K = 100, 200, 400 and 800)

QC-LDPC codes (K = 400, 800, 1600, 3200, 6400 and 32000)

Code rate = 10/11

RS codes (K = 50, 100, 200, 400 and 800)

QC-LDPC codes (K = 400, 800, 1600, 3200, 6400 and 32000)

Code rate = 20/23

RS codes (K = 100, 200, 400 and 800)

QC-LDPC codes (K = 400, 800, 1600, 3200, 6400 and 32000)

Code rate = 5/6

RS codes (K = 50, 100, 200, 400 and 800)

QC-LDPC codes (K = 400, 800, 1600, 3200, 6400 and 32000)