| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 8th Meeting: San José, CA, USA, 1–10 February, 2012 | Document: JCTVC-H0389 |
| --- | --- |

| Title: | New Class E videoconferencing source content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wonkap Jang, Jill Boyce, Danny Hong, Adeel Abbas 433 Hackensack Ave Hackensack, NJ 07601 USA | Tel: Email: | +1 201 478 6145 {wonkap, jill, danny, adeel}@vidyo.com |
| Source: | Vidyo, Inc. |  |  |

_____________________________

# Abstract

Some concerns have been raised about vertical trembling in the current Class E video sequences in the common test conditions.  As requested at the last meeting in Geneva, Vidyo is making available some new videoconferencing test sequences, for consideration to replace the Class E sequences.  3 sequences at 1280x720 resolution and 60 fps are provided.

Vidyo owns the content and upon request will authorize it for use by the JCT, ITU, and MPEG under the normal standards usage conditions.

# Proposed test sequences

The sequences were captured using a Sony EVI-HD7V model camera. All sequences are 1280x720 at 60 fps.  All sequences are 600 frames long.

Sample still images from each of the sequences are shown below.

- Johnny_1280x720_60

- KristenAndSara_1280x720_60

- FourPeople_1280x720_60

# Encoding Results

A table of encoding results using HM 5.0 is shown below, to compare with the current Class E sequences.

| AI-HE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | QPISlice | kbps | Y psnr | U psnr | V psnr |
| Johnny | 22 | 20485.81 | 44.14 | 48.24 | 48.85 |
|  | 27 | 11347.23 | 41.99 | 46.46 | 47.04 |
|  | 32 | 6478.12 | 39.67 | 44.64 | 45.25 |
|  | 37 | 3763.34 | 37.09 | 43.23 | 43.88 |
| KristenAndSara | 22 | 22578.48 | 44.52 | 47.68 | 48.43 |
|  | 27 | 13444.23 | 42.26 | 45.72 | 46.60 |
|  | 32 | 8119.34 | 39.68 | 43.85 | 44.84 |
|  | 37 | 4927.93 | 36.83 | 42.46 | 43.46 |
| FourPeople | 22 | 30714.96 | 43.98 | 47.10 | 48.22 |
|  | 27 | 18764.23 | 41.48 | 44.83 | 46.05 |
|  | 32 | 11505.15 | 38.65 | 42.89 | 44.08 |
|  | 37 | 7016.50 | 35.54 | 41.56 | 42.64 |
| Vidyo1 | 22 | 21465.29 | 44.95 | 47.63 | 48.58 |
|  | 27 | 12545.44 | 42.53 | 45.89 | 46.59 |
|  | 32 | 7473.71 | 39.83 | 44.20 | 44.77 |
|  | 37 | 4490.20 | 36.91 | 42.96 | 43.40 |
| Vidyo3 | 22 | 22292.51 | 44.91 | 48.87 | 48.86 |
|  | 27 | 13548.89 | 42.49 | 47.09 | 46.42 |
|  | 32 | 8360.97 | 39.71 | 45.80 | 44.27 |
|  | 37 | 5006.85 | 36.58 | 44.75 | 42.72 |
| Vidyo4 | 22 | 23685.42 | 44.92 | 48.98 | 49.18 |
|  | 27 | 13629.03 | 42.15 | 46.86 | 47.08 |
|  | 32 | 7781.89 | 39.31 | 45.04 | 45.21 |
|  | 37 | 4403.11 | 36.49 | 43.59 | 43.76 |

| LB-HE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | QPISlice | kbps | Y psnr | U psnr | V psnr |
| Johnny | 22 | 1543.04 | 43.02 | 48.06 | 48.59 |
|  | 27 | 426.43 | 41.25 | 46.48 | 47.08 |
|  | 32 | 190.18 | 39.21 | 44.90 | 45.49 |
|  | 37 | 99.72 | 36.80 | 43.57 | 44.16 |
| KristenAndSara | 22 | 2030.78 | 43.30 | 47.22 | 48.05 |
|  | 27 | 713.72 | 41.23 | 45.40 | 46.35 |
|  | 32 | 322.75 | 38.89 | 43.65 | 44.61 |
|  | 37 | 165.58 | 36.27 | 42.34 | 43.38 |
| FourPeople | 22 | 2280.42 | 42.66 | 46.46 | 47.71 |
|  | 27 | 887.66 | 40.39 | 44.44 | 45.77 |
|  | 32 | 435.96 | 37.80 | 42.65 | 43.92 |
|  | 37 | 226.96 | 34.94 | 41.48 | 42.62 |
| Vidyo1 | 22 | 2032.81 | 43.69 | 47.09 | 48.06 |
|  | 27 | 736.80 | 41.41 | 45.72 | 46.36 |
|  | 32 | 355.04 | 38.89 | 44.37 | 44.76 |
|  | 37 | 188.64 | 36.14 | 43.26 | 43.49 |
| Vidyo3 | 22 | 2712.95 | 43.56 | 48.67 | 48.36 |
|  | 27 | 885.95 | 41.24 | 47.17 | 46.42 |
|  | 32 | 417.61 | 38.66 | 45.96 | 44.42 |
|  | 37 | 222.81 | 35.70 | 45.12 | 43.01 |
| Vidyo4 | 22 | 2328.03 | 43.44 | 48.46 | 48.70 |
|  | 27 | 735.13 | 41.04 | 46.55 | 46.74 |
|  | 32 | 323.80 | 38.46 | 44.90 | 44.95 |
|  | 37 | 167.91 | 35.77 | 43.54 | 43.54 |

| AI-LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | QPISlice | kbps | Y psnr | U psnr | V psnr |
| Johnny | 22 | 22398.99 | 44.13 | 48.13 | 48.60 |
|  | 27 | 12144.76 | 41.93 | 46.29 | 46.81 |
|  | 32 | 6927.78 | 39.63 | 44.39 | 44.91 |
|  | 37 | 4039.33 | 37.14 | 42.93 | 43.38 |
| KristenAndSara | 22 | 24489.21 | 44.48 | 47.57 | 48.31 |
|  | 27 | 14207.38 | 42.13 | 45.55 | 46.33 |
|  | 32 | 8563.39 | 39.52 | 43.62 | 44.47 |
|  | 37 | 5216.17 | 36.73 | 42.15 | 42.97 |
| FourPeople | 22 | 32993.13 | 43.88 | 46.93 | 47.92 |
|  | 27 | 19688.59 | 41.27 | 44.63 | 45.69 |
|  | 32 | 12081.87 | 38.44 | 42.68 | 43.76 |
|  | 37 | 7402.44 | 35.44 | 41.31 | 42.29 |
| Vidyo1 | 22 | 22722.23 | 44.60 | 47.40 | 48.25 |
|  | 27 | 13275.23 | 42.14 | 45.86 | 46.49 |
|  | 32 | 7981.06 | 39.47 | 44.46 | 44.96 |
|  | 37 | 4837.26 | 36.62 | 43.45 | 43.76 |
| Vidyo3 | 22 | 23532.83 | 44.60 | 48.63 | 48.44 |
|  | 27 | 14383.61 | 42.14 | 47.08 | 46.40 |
|  | 32 | 8988.78 | 39.36 | 45.95 | 44.60 |
|  | 37 | 5458.04 | 36.26 | 45.05 | 43.30 |
| Vidyo4 | 22 | 24821.18 | 44.61 | 48.68 | 48.86 |
|  | 27 | 14270.80 | 41.85 | 46.89 | 47.05 |
|  | 32 | 8226.37 | 39.05 | 45.30 | 45.42 |
|  | 37 | 4732.80 | 36.26 | 44.13 | 44.17 |

| LB-LC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | QPISlice | kbps | Y psnr | U psnr | V psnr |
| Johnny | 22 | 1430.19 | 42.83 | 47.64 | 48.20 |
|  | 27 | 421.27 | 41.03 | 46.11 | 46.60 |
|  | 32 | 187.75 | 38.85 | 44.48 | 44.97 |
|  | 37 | 97.61 | 36.41 | 43.18 | 43.45 |
| KristenAndSara | 22 | 1974.47 | 43.05 | 46.80 | 47.60 |
|  | 27 | 707.73 | 40.89 | 45.05 | 45.75 |
|  | 32 | 321.49 | 38.46 | 43.25 | 44.20 |
|  | 37 | 164.82 | 35.79 | 42.00 | 42.77 |
| FourPeople | 22 | 2242.21 | 42.35 | 46.01 | 47.16 |
|  | 27 | 895.94 | 40.07 | 44.08 | 45.19 |
|  | 32 | 435.59 | 37.40 | 42.34 | 43.46 |
|  | 37 | 224.72 | 34.53 | 41.15 | 42.02 |
| Vidyo1 | 22 | 2142.52 | 43.35 | 46.79 | 47.78 |
|  | 27 | 788.16 | 41.11 | 45.53 | 46.25 |
|  | 32 | 383.42 | 38.62 | 44.33 | 44.81 |
|  | 37 | 211.74 | 35.91 | 43.33 | 43.61 |
| Vidyo3 | 22 | 2847.73 | 43.12 | 48.34 | 47.98 |
|  | 27 | 952.87 | 40.82 | 47.05 | 46.28 |
|  | 32 | 453.18 | 38.31 | 45.92 | 44.67 |
|  | 37 | 247.76 | 35.46 | 45.02 | 43.55 |
| Vidyo4 | 22 | 2419.55 | 43.11 | 48.16 | 48.42 |
|  | 27 | 785.52 | 40.77 | 46.52 | 46.79 |
|  | 32 | 352.19 | 38.27 | 45.06 | 45.14 |
|  | 37 | 190.42 | 35.66 | 43.87 | 43.84 |

# Access information

The sequences have been uploaded to the University of Hannover ftp site, ftp.tnt.uni-hannover.de , in the NewClassE directory, and can be accessed using the same login and password as for the other HEVC source sequences.

# Patent rights declaration(s)

Vidyo, Inc.  does not have any current or pending patent rights relating to the technology described in this contribution.